FAILURE MODES AND EFFECTS ANALYSIS

| Document Title | Failure Modes and Effects Analysis (FMEA) for ASYM-141366 manufacture (Step 1 of ASYM-140020) |
| --- | --- |
| Document Number | RA-140020030102.01 |
| Product / Lilly ID | ASYM-141366 manufacture (Step 1 of ASYM-140020/ LSN4182924) |
| Process Scope | SPPS process for ASYM-140851 manufacture (Step 1 of ASYM-140020/ LSN4182924); PPQ campaign |
| CMO Site | Asymchem Life Science (Tianjin) Co., Ltd.(No.265, South Avenue, Binhai New Area TEDA, Tianjin, 300462, China) |

Approval and Signatures

This FMEA must be reviewed and approved prior to PPQ completion as a minimum. All approvals must be in place before the PPQ campaign commences for initial pre-commercialisation FMEAs.

| Prepared by |  |  |  |
| --- | --- | --- | --- |
| Zhonghua Fu (Associate Director, CMMD) | Signature & Date |  |  |
| Reviewed by |  |  |  |
| Liang Li (Senior Director, CMMD) | Signature & Date |  |  |
| Zhongyao Zhang (Director, Production) | Signature & Date |  |  |
| Lu Li (Director, QA) | Signature & Date |  |  |
| Approved by |  |  |  |
| Mingyang Yang (Executive Director, QA) | Signature & Date |  |  |

This document is reviewed and approved by Eli Lilly and Company. Refer to signature page for EDMS signatures and dates for Eli Lilly and Company approval.

Version History

| Version | Approved Date | Major Changes from Previous Version / Justification |
| --- | --- | --- |
| 1.0 | The last signature date is the effective date | New document established for PPQ campaign. |

Table of contents

1. Introduction and Background	5

1.1 Risk Topic Description	5

1.2 Risk Assessment Scope	5

1.3 FMEA Objectives	5

1.4 Abbreviations and Definitions	6

2. FMEA Team — Personnel and Roles	7

3. FMEA Scoring Criteria	8

3.1 Severity of Harm — Impact on Product Quality / Patient Safety	8

3.2 Probability of Occurrence	8

3.3 Probability of Detection	9

4. Product CQA Linkage and Process Map	10

4.1 CQA List and Severity Assignment 	10

4.2 Process Map — Unit Operations	11

5. FMEA — Main Risk Assessment Table	15

6. Equipment Train Risk Assessment	93

7. Risk Summary and Action Plan	97

8. Risk Review Schedule 	118

9. Risk Assessment Conclusion	119

10. References	121

Appendix A — Notes on Electrostatic Risk Control	121

1. Introduction and Background

1.1 Risk Topic Description

For ASYM-140020 (Lilly code: LSN4182924), also known as Eloralintide Fragment L2 (referred to as ASYM 140020 in Asymchem) is an intermediate in the synthesis of Eloralintide drug substance. The manufacturing process of LSN4182924 (ASYM-140020) comprises two chemical steps: Step 1 performs solid-phase peptide synthesis on 2-CTC resin through repeated coupling-washing-deprotection cycles, followed by washing and drying; Step 2 is produced via soft cleavage, neutralization, concentration, washing, precipitation and drying to yield the product. The SPPS process is the first step of ASYM-140020 (Lilly code: LSN4182924) preparation. To support commercialization of Eloralintide fragment L2 LSN4182924/ CPo140020-03 manufacturing process, it is proposed to conduct process performance qualification (PPQ) for LSN4182924/CPo140020-03 manufacturing process in campaign CPo140020-03 at Asymchem Life Science (Tianjin) Co.,Ltd. (No. 265, South Avenue, Binhai New Area TEDA. (No. 265, South Avenue, Binhai New Area TEDA, Tianjin, China; hereinafter referred to as TJ4). The PPQ production is scheduled from October 2026 at TJ4. Three PPQ batches of Step 1 will be manufactured, followed by subsequent commercial batches. The purpose of this FMEA is to document the science and risk-based approach applied to assess batch execution of ASYM-141366 following the Master Operation Record (MOR #: MOR-TJ414002001.04). The FMEA identifies the risk level for each potential failure mode and develops a risk reduction plan for high-risk scenarios.

1.2 Risk Assessment Scope

This FMEA is applicable to the commercial manufacturing process of ASYM-141366 at Asymchem TJ4 site. It covers all process unit operations from RSMs/resin/reagent loading through to final packaging as defined in the MBR. Equipment qualification, computer system validation, analytical method transfer and cleaning validation are addressed in separate documents and are not within the scope of this FMEA.

This FMEA connects process failure modes directly to product quality. It is intended as a living document and will be reviewed and updated following significant process changes, deviations.

1.3 FMEA Objectives

- Identify all potential failure modes in the manufacturing process that could impact product quality.

- Assess the severity of potential harm, probability of occurrence, and probability of detection for each failure mode.

- Assign a risk level (High / Medium / Low) using the scoring matrix defined.

- Identify and assign mitigation actions for all High-risk items; document rationale for accepting Medium risk items where mitigation is not implemented.

- Provide a risk summary that can be used to prioritise pre-commercialisation activities and ongoing monitoring.

1.4 Abbreviations and Definitions

| Abbreviation | Definition |
| --- | --- |
| API | Active Pharmaceutical Ingredient |
| CQA | Critical Quality Attribute |
| CPP | Critical Process Parameter |
| CM | Contract Manufacturer |
| PLC | Programmable Logic Controller (process automation) |
| DIC | N,N'-Diisopropylcarbodiimide (coupling reagent) |
| DIPEA | N,N-Diisopropylethylamine (base) |
| DMF | N,N-Dimethylformamide (solvent) |
| FMEA | Failure Modes and Effects Analysis |
| GQP | Global Quality Procedure |
| ICH | International Council for Harmonisation |
| IPC | In-Process Control |
| IPAc | Isopropyl acetate |
| LOD/KF | Loss on Drying/ Karl Fischer water content |
| MBR | Master Batch Record |
| MOR | Master Operation Record |
| MOC | Material of Construction |
| NOR / PAR | Normal Operating Range / Proven Acceptable Range |
| OOS | Out-of-Specification |
| Oxyma | Ethyl cyano(hydroxyimino)acetate-2-oxime (coupling additive) |
| PPQ | Process Performance Qualification |
| QA | Quality Assurance |
| QbD | Quality by Design |
| QRM | Quality by Design |
| ROPP | Quality Risk Management |
| RSM | Regulatory Starting Material |
| RPN | Risk Priority Number (Severity × Occurrence × Detection — legacy; replaced by matrix approach) |
| SPPS | Solid Phase Peptide Synthesis |
| TS/MS | Technical Services / Manufacturing Science |
| PFD report | Eloralintide Fragment L2 Intermediate Global Process Flow Document (PFD) LSN4182924 |
| OOE | Out of Expectation |

2. FMEA Team — Personnel and Roles

The cross-functional team listed below participated in the FMEA. At minimum, representatives from Technical Operations, Production, Quality Assurance and Lilly APIEM TS/MS should be included.

| Department / Function | Name | Role in FMEA |
| --- | --- | --- |
| Technical Operations / TS/MS | Zhonghua Fu (Associate Director, CMMD) | Risk evaluation sponsor — organises and leads the risk assessment; drafts the FMEA report |
| Technical Operations / TS/MS | Liang Li (Senior Director, CMMD) | SME — participates in risk assessment; provides process expertise |
| Production / Operations | Zhongyao Zhang (Director, Production) | SME — participates in risk assessment; provides operational/manufacturing expertise |
| Quality Assurance | Mingyang Yang (Executive Director, QA) | Reviews and approves FMEA report; ensures GMP compliance and alignment with quality system |
| Quality Control | Haini Wang (Senior Director, QC) | Participates as required; provides analytical method and in-process testing expertise |
| Lilly APIEM TS/MS | Johan Granander (Sr. Principal Scientist TSMS-EM) | Reviews risk assessment; provides Lilly process and quality expectations |
| Lilly APIEM QA | Novie Atakiwe, (Sr QA Associate TSMS-EM) | Reviews and approves FMEA as required by Quality Agreement |

3. FMEA Scoring Criteria

Risk is evaluated using the Failure Mode and Effects Analysis (FMEA) methodology, consistent with ICH Q9. Each failure mode is scored for Severity of Harm, Probability of Occurrence, and Probability of Detection. These three scores are then combined using the risk matrix to derive an overall Risk Level (High / Medium / Low).

3.1 Severity of Harm — Impact on Product Quality / Patient Safety

Severity is assessed with respect to impact on product quality or the CQA. Severity scores are fixed and should not be adjusted by the existence of controls.

| Severity Level | Description | Patient / Quality Impact | Score |
| --- | --- | --- | --- |
| Severe | Definite impact to product quality which impacts patient safety, resulting in batch rejection (within Lilly control) or product recall. | Direct patient safety impact / product recall | 5 |
| Major | Potential impact on product quality which may impact patient safety; may result in batch rejection. | Probable patient safety impact / batch rejection | 4 |
| Moderate | Potential impact to product quality requiring investigation. Some business impact — potential to lose some processing capability. | Investigation required; processing capability at risk | 3 |
| Minor | Readily available information to support no product quality impact (Observation). Minor business impact — schedule adjustments required to avoid impact to processing capability. | No quality impact; minor schedule impact | 2 |
| Negligible | No potential impact to product quality. No operational impact or impact to processing capability. | No impact | 1 |

3.2 Probability of Occurrence

Occurrence reflects the likelihood that the failure mode will occur, taking into account the current state of process knowledge and the robustness of the control strategy as designed.

| Probability of Occurrence | Description | Score |
| --- | --- | --- |
| Frequent | Common, repeated, or occurring often. Failure almost certain to occur. | 5 |
| Probable | Likely to occur under normal operating conditions. | 4 |
| Occasional | Occurring at irregular or infrequent intervals. Possible but not expected routinely. | 3 |
| Remote | Unlikely, but can be expected to occur at some point over the product lifecycle. | 2 |
| Improbable | Unlikely to occur. Not expected in the life of the product. | 1 |

3.3 Probability of Detection

Detection reflects the likelihood that the current control strategy will detect a failure mode before it results in harm to the patient. A lower detection score (higher number) indicates weaker detection capability.

| Probability of Detection | Description | Score |
| --- | --- | --- |
| Remote | No known controls available or highly unlikely that current controls will detect a potential failure mode. | 5 |
| Low | Small likelihood that the current controls will detect a potential failure mode. | 4 |
| Moderate | Possible but not certain that the current controls will detect a potential failure mode. | 3 |
| High | Current controls are likely to detect a potential failure mode. | 2 |
| Very High | The ability of current controls to detect a potential failure mode has been demonstrated. Detection is essentially certain. | 1 |

3.4 Risk Level Matrix

The overall residual Risk Level is determined by combining the Severity score with the combined probability of harm (derived from Occurrence and Detection scores) using the matrix below.

3.4.1 Probability of Harm:

| Probability of Occurrence Probability of Detection | Score | Improbable (1) | Remote (2) | Occasional (3) | Probable (4) | Frequent (5) |
| --- | --- | --- | --- | --- | --- | --- |
| Remote | 5 | Remote (2) | Remote (2) | Occasional (3) | Probable (4) | Frequent (5) |
| Low | 4 | Improbable (1) | Remote (2) | Occasional (3) | Probable (4) | Probable (4) |
| Moderate | 3 | Improbable (1) | Remote (2) | Remote (2) | Occasional (3) | Occasional (3) |
| High | 2 | Improbable (1) | Improbable (1) | Remote (2) | Remote (2) | Remote (2) |
| Very High | 1 | Improbable (1) | Improbable (1) | Improbable (1) | Improbable (1) | Remote (2) |

3.4.2 Residual Risk Level:

| Severity Probability of Harm | Score | Low (1) | Minor (2) | Moderate (3) | Major (4) | Severe (5) |
| --- | --- | --- | --- | --- | --- | --- |
| Frequent | 5 | Low | Medium | High | High | High |
| Probable | 4 | Low | Medium | Medium | High | High |
| Occasional | 3 | Low | Low | Medium | Medium | High |
| Remote | 2 | Low | Low | Low | Medium | Medium |
| Improbable | 1 | Low | Low | Low | Low | Medium |

| Residual Risk Level Definition: | LOW — Acceptable; no action required | MEDIUM — Mitigation recommended or risk acceptance rationale required | HIGH — Mitigation REQUIRED before PPQ |
| --- | --- | --- | --- |

Mitigation steps must be implemented for failure modes with High residual risk before PPQ commencement. For Medium residual risk, mitigation steps may be implemented, or a rationale for accepting the risk must be documented and approved. All Low-risk failure modes are acceptable with current controls.

4. Product CQA Linkage and Process Map

4.1 CQA List and Severity Assignment

The Step 1 in-process intermediate ASYM-141366 is controlled only by IPCs (refer to Table 4.1) and has no CQAs. Thus Step 1 failure modes are assessed for their impact on the Step 1 IPCs, and consequently, through those IPCs, on the product ASYM-140020 CQAs listed below Table 4.2. The following CQAs have been identified for product ASYM-140020. Severity scores assigned to each CQA are used as the basis for scoring failure modes that impact these attributes. The link between process control elements and CQAs have been established via PAR studies and identification of CPPs (refer to Table 10.1) prior to the FMEA to ensure accurate scoring.

Table 4.1: Eloralintide Fragment LSN4182924/ASYM-140020 In-Process Controls (IPCs) for Step 1

| Step | Parameter | Type | Acceptance criteria | Test Methodology |
| --- | --- | --- | --- | --- |
| Step 1 SPPS | Pre-treatment water content | IPC | NMT 500 ppm | KF |
|  | Coupling completion(Glu14) | IPC | Pass (negative) | Bromophenol blue |
|  | Coupling completion (all AAs except Asn21 and Glu14 ) | IPC | Pass (negative) | Ninhydrin test |
|  | Leu12 Post deprotection wash piperidine | IPC | NMT 500 ppm | Chloranil test |
|  | Orn11 post coupling wash piperidine residue | IPC | NMT 500 ppm | Chloranil test |
|  | Drying endpoint | IPC | NMT 9% | LOD |

Table 4.2: The Severity Assignment on LSN4182924/ASYM-140020 CQAs

| CQA | Definition / Acceptance Criterion | Category | Severity Score | Rationale / Regulatory Basis |
| --- | --- | --- | --- | --- |
| Resin loading | 1.0~1.2 mmol/g | Identity | 5 | Affects batch yield and coupling performance, and serves as the dosing basis for all critical raw materials. Failure to meet the specified range results in incomplete coupling and elevated impurity levels. |
| Identity | Confirmed by HPLC/MS; conforms to reference standard | Identity | 5 | The identity is necessary for the quality of the next‑step product. Failure to meet identity results in batch rejection |
| Assay | NLT 80.0 (%w/w) | Chemical Purity | 5 | Critical to achieving target charging equivalent required in the subsequent steps Failure to meet assay results in batch rejection |
| Purity | NLT 90.0% area | Chemical Purity | 5 | Presence of related substances can affect the quality of the next‑step intermediate Failure to meet purity results in batch rejection |
| Total Impurities | NMT 10.0% area | Chemical Purity | 5 | Presence of impurities can affect the quality of the next‑step intermediate Failure to meet purity results in batch rejection |
| ASYM-152486 (L2 des-(Asn21-NMeAsn22)) | NMT 2.0% area | Chemical Purity | 5 | Presence of ASYM-152486 can affect the quality of the next‑step intermediate Failure to meet purity results in batch rejection |
| ASYM-153207 (L2 endo-Gly24) | NMT 0.5% area | Chemical Purity | 5 | Presence of ASYM-153207 can affect the quality of the next‑step intermediate Failure to meet purity results in batch rejection |
| Any Unspecified Impurity(Report individual impurities ≥ 0.10% by RRT) | NMT 0.5% area | Chemical Purity | 5 | Presence of unspecified Impurity can affect the quality of the next‑step intermediate Failure to meet purity results in batch rejection |

4.2 Process Map — Unit Operations

The table below defines the unit operations for the manufacturing process. Each step is linked to the CQAs it may affect, providing the framework for the FMEA in Section 5.

Table 4.3: The Unit Operations Description

| No. # | Unit Operation | Description | Equipment |
| --- | --- | --- | --- |
| 1 | Dispense | Accurately weigh and dispense amino acids, Oxyma pure, PyBOP and 2-CTC resin; package into labeled LDPE bags in a controlled glove box environment. | Glove box (TJ4S-1200-GB01, TJ4S-1200-GB02, TJ4S-1200-GB03, TJ4S-1200-GB04, TJ4S-1200-GB05). |
| 2 | Equipment confirmation & Recipe Edit & Control System | Perform pre-use equipment qualification and integrity check; edit and load the validated process recipe into the synthesis control system. | Peptide Solid Phase Synthesizer (TJ4S-1213-Sy15), Activation tank (TJ4S-1213-Sy15-R01) Glass-lined Reactor (TJ4-1213-R22) Agitation tank (TJ4S-1200-AT02) |
| 3 | Pre-treatment | Add DMF to activation tank start agitator and stir. Transfer the solution into synthesis reactor. | Peptide Solid Phase Synthesizer (TJ4S-1213-Sy15), Activation tank (TJ4S-1213-Sy15-R01) |
| 4 | Resin Swelling and wash | 1.Charge 2-CTC resin into synthesis reactor. Swell resin with DMF via spray ball. Drain to waste vessel. 2.Charge DMF to synthesis reactor for washing resin. Drain to waste vessel. | Peptide Solid Phase Synthesizer (TJ4S-1213-Sy15) |
| 5 | Loading of Amino Acid 1 (Fmoc-Gly-OH) | Charge amino acid 1 and DMF and DIPEA to activation tank for activation. Transfer activated solution to synthesis reactor. Perform coupling reaction. Wash resin with DMF. | Peptide Solid Phase Synthesizer (TJ4S-1213-Sy15), Activation tank (TJ4S-1213-Sy15-R01) |
| 6 | Capping | Charge DMF, MeOH and DIPEA to synthesis reactor. Perform capping reaction. Wash resin with DMF | Peptide Solid Phase Synthesizer (TJ4S-1213-Sy15) |
| 7 | Fmoc Deprotection （Phe23- Orn11 expect Asn21 and Leu12） | 1. Deprotection with 20% piperidine/DMF solution. 2. Wash resin with IPAc and DMF. | Peptide Solid Phase Synthesizer (TJ4S-1213-Sy15) |
| 8 | Fmoc Deprotection （Asn21 ） | 1. Deprotection with 20% piperidine/DMF solution. 2. Deprotection with 20% piperidine/DMF solution. 3. Wash resin with IPAc and DMF. | Peptide Solid Phase Synthesizer (TJ4S-1213-Sy15), Deprotection tank (TJ4S-1213-SPS01-AT01&TJ4S-1213-SPS01-AT02) |
| 9 | Fmoc Deprotection （Leu12 ） | 1. Deprotection with 20% piperidine/DMF solution. 2. Wash resin with IPAc. 3. Chloranil test piperidine NMT 500 ppm. 4.Re-swelling resin with DMF. | Peptide Solid Phase Synthesizer (TJ4S-1213-Sy15), Deprotection tank (TJ4S-1213-SPS01-AT01&TJ4S-1213-SPS01-AT02) |
| 10 | Amino Acid Activation& Coupling（Phe23） | 1. Charge DMF, amino acid, PyBOP, Oxyma and DIPEA to activation tank for activation. 2. Transfer activated solution to synthesis reactor. Perform coupling reaction. 3. Ninhydrin test IPC after coupling (coupling completion confirmation — negative). 4. Post coupling wash with DMF. 5. Repeat Fmoc De-protection Cycle. | Peptide Solid Phase Synthesizer (TJ4S-1213-Sy15), Activation tank (TJ4S-1213-Sy15-R01), Agitation tank (TJ4S-1200-AT02) |
| 11 | Amino Acid Activation& Coupling（Asn21） | 1. Charge DMF, amino acid, Oxyma and DIC to activation tank for activation. 2. Transfer activated solution to synthesis reactor. Perform coupling reaction. 3. Add additional Oxyma/DMF and DIC to synthesis reactor. 4. 5. Post coupling wash with DMF. 6. Capping | Peptide Solid Phase Synthesizer (TJ4S-1213-Sy15), Activation tank (TJ4S-1213-Sy15-R01), Agitation tank (TJ4S-1200-AT02) |
| 12 | Amino Acid Activation& Coupling（NMeAsn22- Orn11 except Phe23 and Asn21） | 1. Charge DMF, amino acid, Oxyma and DIC to activation tank for activation. 2. Transfer activated solution to synthesis reactor. Perform coupling reaction. 3. Bromophenol blue/ Ninhydrin test IPC after coupling (coupling completion confirmation — negative). 4. Post coupling wash with DMF. 5. Repeat Fmoc Deprotection Clycle. | Peptide Solid Phase Synthesizer (TJ4S-1213-Sy15), Activation tank (TJ4S-1213-Sy15-R01) |
| 13 | Capping (Asn21 applicable) | 1. Charge DMF, pyridine and acetic anhydride to Agitation tank for activation. 2. Transfer activated solution to synthesis reactor. Perform capping reaction. 3. Post capping wash with DMF. 4. Perform Fmoc De-protection Cycle. | Peptide Solid Phase Synthesizer (TJ4S-1213-Sy15), Glass-lined Reactor (TJ4-1213-R22) |
| 14 | Resin de-swelling and washing | 1.Charge IPAc to synthesizer for resin washing. 2.Charge toluene and MTBE to synthesizer for resin de-swelling and washing. Chloranil test piperidine NMT 500 ppm. | Peptide Solid Phase Synthesizer (TJ4S-1213-Sy15) |
| 15 | Drying and Discharge | 1.Dry wet resin to LOD ≤ 9%. Discharge Eloralintide Fragment L2 Step 1 ASYM-141366 into double LDPE bags. | Peptide Solid Phase Synthesizer (TJ4S-1213-Sy15) |
| 16 | Packaging / Final Sampling | Weigh, sample and package dried resin in antistatic LDPE-lined fibre drums and stored at 2–8 °C label and record the appearance. | Packaging area |

5. FMEA — Main Risk Assessment Table

5.1 Process Parameters

The Proven Acceptable Ranges (PARs) for main process parameters were established via the execution of designed experiments and PAR report <Refer to RPT-1194035>. For the full scale batch, some of these Process Parameters were all controlled under cGMP operations and were all within the PARs. The details are listed below.

| CONTROL PARAMETER | TARGET |  | PAR |  | Type |
| --- | --- | --- | --- | --- | --- |
| Resin swell |  |  |  |  |  |
| Pre-treatment DMF quantity | 5.21 kg/kg |  | N/A |  | non-CPP |
| Water content (IPC) | NMT 500 ppm |  | N/A |  | non-CPP |
| 2-CTC resin substitution degree | 1.0 – 1.2 mmol/g |  | N/A |  | non-CPP |
| DMF swelling quantity | 7.58 kg/kg |  | N/A |  | non-CPP |
| Swelling temperature | 25 °C |  | N/A |  | non-CPP |
| Swelling time | 35 min |  | N/A |  | non-CPP |
| DMF wash quantity | 4.74 kg/kg |  | N/A |  | non-CPP |
| RESIN LOADING and CAPPING |  |  |  |  |  |
| Activation – Gly24 quantity | 2.25 eq. |  | 2.12 – 2.54 eq. |  | non-CPP |
| Activation – DMF quantity | 4.98 kg/kg |  | 4.14 – 5.61 kg/kg |  | non-CPP |
| Activation – DIPEA quantity | 6.00 eq. |  | 5.24 – 6.69 eq. |  | CPP |
| Activation-Dissolution temperature | 25.0 °C |  | 17.0 – 33.8 °C |  | CPP |
| Activation – Dissolution time | 5 min |  | 0 – 66 min |  | non-CPP |
| Coupling – Total DMF quantity | 5.93 kg/kg |  | 4.97 – 6.68 kg/kg |  | non-CPP |
| Coupling – Reaction temperature | 25.0 °C |  | 17.2 – 32.7 °C |  | CPP |
| Coupling – Reaction time | NLT 180 min |  | 180 – 481 min |  | non-CPP |
| Coupling – DMF wash quantity | 3x NLT 7.58 kg/kg |  | 3x NLT 6.62kg/kg |  | non-CPP |
| Capping – Total DMF quantity | 8.53 kg/kg |  | 7.47 – 9.61 kg/kg |  | non-CPP |
| Capping – MeOH quantity | 10.00 eq. |  | 8.73 – 11.20 eq. |  | non-CPP |
| Capping – DIPEA quantity | 5.00 eq. |  | 4.36 – 5.60 eq. |  | non-CPP |
| Capping – Temperature | 25.0 °C |  | 13.0 – 36.8 °C |  | non-CPP |
| Capping – Time | 60 min |  | 15 – 122 min |  | non-CPP |
| Capping – DMF wash quantity | 3x NLT 7.58 kg/kg |  | 3x NLT 7.58 kg/kg |  | CPP |
| DEPROTECTION |  |  |  |  |  |
| Piperidine/DMF solution concentration | 20% v/v |  | 14.99 – 25.00% v/v |  | non-CPP |
| Piperidine/DMF solution quantity (Standard AA) | 7.45 kg/kg |  | 6.54 – 8.38 kg/kg |  | non-CPP |
| Piperidine/DMF solution quantity (Pre-Ser20 Treatment 1) | 7.45 kg/kg |  | 6.51 – 8.40 kg/kg |  | non-CPP |
| Piperidine/DMF solution quantity (Pre-Ser20 Treatment 2) | 9.68 kg/kg |  | 7.44 – 10.91 kg/kg |  | non-CPP |
| Deprotection temperature (Standard AA) | 25.0 °C |  | 17.0 – 34.8 °C |  | CPP |
| Deprotection temperature (Pre-Ser20) | 25.0 °C |  | 14.6 – 34.7 °C |  | CPP |
| Piperidine/DMF exposure time (Standard AA) | 120 min |  | 68 – 180 min |  | non-CPP |
| Piperidine/DMF exposure time (Pre-Ser20 Treatment 1) | 15 min |  | 6 – 68 min |  | CPP |
| Piperidine/DMF exposure time (Pre-Ser20 Treatment 2) | 30 min |  | 6 – 83 min |  | CPP |
| IPAc wash quantity | 4x NLT 8.73 kg/kg |  | 4x NLT 7.93 kg/kg |  | CPP |
| Chloranil Test (IPC) (Pre-Orn11 coupling) | Negative |  | N/A |  | non-CPP |
| DMF wash quantity (Standard AA) | 2x NLT 11.38 kg/kg |  | 2x NLT 9.96 kg/kg |  | non-CPP |
| DMF wash quantity (Pre-Ser20 coupling) | 2x NLT 12.32 kg/kg |  | 2x NLT 10.78 kg/kg |  | non-CPP |
| DMF wash quantity (Pre-Val17, Pre-Leu12 coupling) | 2x NLT 9.48 kg/kg |  | 2x NLT 8.29 kg/kg |  | non-CPP |
| ACTIVATION |  |  |  |  |  |
| AA quantity (Standard AA) | 2.00 eq. |  | 1.88 – 2.13 eq. |  | non-CPP |
| AA quantity (Phe23) | 2.00 eq. |  | 1.88 – 2.13 eq. |  | non-CPP |
| AA quantity (NMeAsn22, αMePhe15) | 1.75 eq. |  | 1.65 – 1.86 eq. |  | non-CPP |
| AA quantity (Asn21) | 2.50 eq. |  | 2.35 – 2.66 eq. |  | non-CPP |
| Oxyma quantity (Standard AA) | 2.00 eq. |  | 1.94 – 2.25 eq. |  | CPP |
| Oxyma quantity (Phe23) | 2.00 eq. |  | 1.88 – 2.26 eq. |  | non-CPP |
| Oxyma quantity (NMeAsn22, αMePhe15) | 1.80 eq. |  | 1.75 – 2.03 eq. |  | CPP |
| Oxyma quantity (Asn21) | 2.60 eq. |  | 2.50 – 2.93 eq. |  | CPP |
| Dissolution temperature (Standard AA) | 20.0 °C |  | 12.6 – 26.5 °C |  | CPP |
| Dissolution temperature (Phe23) | 20.0 °C |  | 12.0 – 29.5 °C |  | CPP |
| Dissolution temperature (NMeAsn22, αMePhe15) | 20.0 °C |  | 12.6 – 26.9 °C |  | CPP |
| Dissolution temperature (Asn21) | 20.0 °C |  | 12.2 – 27.9 °C |  | CPP |
| PyBOP quantity (Phe23 cycle) | 2.00 eq. |  | 1.87 – 2.25 eq. |  | non-CPP |
| DIPEA quantity (Phe23 cycle) | 3.00 eq. |  | 2.62 – 3.40 eq. |  | non-CPP |
| DIC quantity (Standard AA) | 2.20 eq. |  | 2.01 – 2.48 eq. |  | CPP |
| DIC quantity (NMeAsn22, αMePhe15) | 2.00 eq. |  | 1.75 – 2.27 eq. |  | non-CPP |
| DIC quantity (Asn21) | 2.90 eq. |  | 2.52 – 3.29 eq |  | non-CPP |
| Total activation DMF quantity (Standard AA) | 6.64 kg/kg |  | 5.84 – 7.46 kg/kg |  | non-CPP |
| Total activation DMF quantity (Phe23) | 4.74 kg/kg |  | 4.16 – 5.33 kg/kg |  | non-CPP |
| Total activation DMF quantity (NMeAsn22, αMePhe15) | 5.69 kg/kg |  | 5.00 – 6.41 kg/kg |  | non-CPP |
| Total activation DMF quantity (Asn21) | 5.69 kg/kg |  | 4.98 – 6.40 kg/kg |  | non-CPP |
| Activation temperature (Standard AA) | 20.0 °C |  | 12.6 – 26.5 °C |  | CPP |
| Activation temperature (Phe23) | 5.0 °C |  | -2.5 – 13.6 °C |  | CPP |
| Activation temperature (NMeAsn22, αMePhe15) | 20.0 °C |  | 12.6 – 26.9 °C |  | CPP |
| Activation temperature (Asn21) | 20.0 °C |  | 12.2 – 27.9 °C |  | CPP |
| Activation time (Standard AA) | 20 min |  | 0 – 80 min |  | non-CPP |
| Activation time (Phe23) | 20 min |  | 0 – 80 min |  | non-CPP |
| Activation time (NMeAsn22, αMePhe15) | 20 min |  | 0 – 80 min |  | non-CPP |
| Activation time (Asn21) | 20 min |  | 0 – 80 min |  | non-CPP |
| COUPLING |  |  |  |  |  |
| Total coupling DMF quantity (Standard AA) | 7.58 kg/kg |  | 6.67 – 8.53 kg/kg |  | non-CPP |
| Total coupling DMF quantity (Phe23) | 7.58 kg/kg |  | 4.99 – 8.54 kg/kg |  | non-CPP |
| Total coupling DMF quantity (NMeAsn22, αMePhe15) | 6.64 kg/kg |  | 5.83 – 7.49 kg/kg |  | non-CPP |
| Total coupling DMF quantity (Asn21) | 7.58 kg/kg |  | 6.65 – 8.55 kg/kg |  | non-CPP |
| Oxyma kicker charge quantity (Asn21 cycle) | 1.00 eq. |  | 0.94 – 1.13 eq. |  | non-CPP |
| DIC kicker charge quantity (Asn21 cycle) | 1.10 eq. |  | 0.96 – 1.26 eq. |  | non-CPP |
| Coupling temperature (Standard AA) | 25.0 °C |  | 18.6 – 28.4 °C |  | CPP |
| Coupling temperature (Phe23) | 25.0 °C |  | 17.4 – 30.3 °C |  | CPP |
| Coupling temperature (NMeAsn22, αMePhe15) | 25.0 °C |  | 17.3 – 28.3 °C |  | CPP |
| Coupling temperature (Asn21) | 25.0 °C |  | 17.2 – 30.2 °C |  | CPP |
| Coupling reaction time (Standard AA) | NLT 240 min |  | 240 – 600 min |  | non-CPP |
| Coupling reaction time (Phe23) | NLT 360 min |  | 360 –841 min |  | non-CPP |
| Coupling reaction time (NMeAsn22, αMePhe15) | NLT 360 min |  | 360 – 840 min |  | non-CPP |
| Total Coupling reaction time (Asn21) | NLT 780 min |  | 780 – 1680 min |  | non-CPP |
| Coupling reaction time (Glu14) | NLT 480 min |  | 480 – 1083 min |  | non-CPP |
| Colorimetric Test (IPC) (Ninhydrin for Phe23, NMeAsn22, Ser20 αMePhe15, Ala13-Orn11; Chloranil for Asn21; Bromophenol Blue for Glu14) | Negative(x2) |  | N/A |  | non-CPP |
| Post coupling DMF wash quantity (except Orn11) | 3x NLT 7.58 kg/kg |  | 3x NLT 6.62 kg/kg |  | non-CPP |
| CAPPING POST ASN21 COUPLING |  |  |  |  |  |
| Capping DMF quantity |  | 6.64 kg/kg |  | 5.80 – 7.47 kg/kg | non-CPP |
| Capping Acetic anhydride quantity |  | 1.25 eq. |  | 1.09 – 1.41 eq. | non-CPP |
| Capping Pyridine quantity |  | 1.50 eq. |  | 1.31 – 1.69 eq. | non-CPP |
| Capping reaction temperature |  | 25.0 °C |  | 12.3 – 36.8 °C | non-CPP |
| Capping reaction time |  | 60 min |  | 10 – 120 min | non-CPP |
| Post capping DMF wash quantity |  | 3x NLT 7.58 kg/kg |  | 3x NLT 6.75 kg/kg | CPP |
| DE-SWELL AND DRYING |  |  |  |  |  |
| IPAc wash quantity |  | 5x NLT 8.73 kg/kg |  | 5x NLT 8.73 kg/kg | CPP |
| Toluene wash quantity |  | 2x NLT 8.72 kg/kg |  | 2x NLT 8.72 kg/kg | CPP |
| MTBE wash quantity |  | 4x NLT 7.41 kg/kg |  | 4x NLT 7.41 kg/kg | CPP |
| Chloranil Test (IPC) |  | Negative |  | N/A | non-CPP |
| Drying jacket temperature |  | NMT 30.0 °C |  | NMT 42.0 °C | non-CPP |
| LOD IPC |  | NMT 9% w/w |  | N/A | non-CPP |

5.2 Main Risk Assessment Table

The table below presents the full FMEA for each process unit operation. Columns are: Step | Unit Operation | Failure Mode (Hazard) | Potential Effect | Yes / No for CQA impact | Severity (Sev) | Potential Cause(s) | Current Controls / Detection Methods | Occurrence (Occ) | Detection (Det) | Risk Level | Mitigation Action / Responsible Party | Residual Risk Level.

Instructions: For each failure mode, (1) assess Severity from the CQA impact; (2) assign Occurrence score reflecting current process knowledge; (3) assign Detection score reflecting current control strategy; (4) determine Risk Level from the matrix in Section 3.4; (5) define mitigation for all High items and document rationale for accepted Medium items

| Step # | Unit Operation | Failure Mode (Hazard) | Potential Effect on Product Quality | Yes/No for CQA impact | Sev | Potential Cause(s) | Current Controls / Detection Methods |  | Occ | Det | Risk Level | Mitigation Action / Responsible Party | Residual Risk |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1. Dispense |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1.1 | Dispense RSMs or Resin or Oxyma or PyBOP | Incorrect RSM , resin, Oxyma or PyBOP used (wrong material / wrong lot) | Wrong amino acid incorporated — incorrect peptide sequence; batch rejection | Yes | 5 | Material labelling error; dispensing error; insufficient identity check at receipt or dispensing | - Materials shall be sourced from approved qualified suppliers. - Supplier Certificates of Analysis (CoA) shall be verified upon material receipt, and incoming inspection and release shall be performed in accordance with approved specifications. - Affix labels equipped with QR codes, which contain the material name, control number, material batch number and project number. Confirmation shall be performed upon material receipt, and workshop operators shall scan the QR codes to complete material receiving operations. - Materials for different projects shall be stored in separate designated zones inside the plant, and labels shall be affixed to distinguish different types of solid materials. - Line clearance and material verification before dispensing - Only one material is present in the dispensing area to prevent dispensing errors. - Prior to dispensing, material identification is performed using a handheld Raman spectrometer to prevent incorrect material usage. |  | 2 | 2 | Medium | - All staging zones for different materials of the same project shall be fitted with labels for identification inside the plant. - Different materials within the same project shall be separated by red demarcation lines inside the plant. Production department | Low |
| 1.2 | Dispense RSMs or Resin or Oxyma or PyBOP | Out-of-specification RSM used (e.g. low assay, wrong substitution degree) | Reduced coupling efficiency; impurity formation; product fails specification | Yes | 5 | Supplier CoA limit wider than internal receipt limit; receipt testing not performed; specification gap; Long transit time; No thermal insulation provided during transportation. | - Receipt testing per approved specification - It’s forbidden to locating the rejected material together with the released material and quarantine material. The rejected material shall be held and locked in physically segregate area/room “rejected” - Dept. of supplying personnel confirms the material batches that meet the project's production requirements based on the material requisition application and prepares the material (confirmation information includes but is not limited to: materials have been released in accordance with current version quality specification of the applied project, the supplier of the materials is a qualified supplier or the project qualified supplier (if any) category, the materials are within the retest date and the status is released, distribution projects of locked material are consistent with the locked project, material packaging is sealed and label/ identification are completed, materials outer packaging cleaning, etc...）. - After completing the material preparation, the <CP/CK Project Material Distribution Label>（QF02265) should be affixed on each package of the material and confirm that the label is affixed on the correct material package, and distributed to corresponding projects based on project requirements. - Transport with thermal insulation covers to control temperature; transit time is controlled within 1 hour. - Upon material receipt, workshop personnel shall verify package integrity, contamination on outer packaging, material name, release status and other relevant information. |  | 2 | 2 | Medium | - Confirm receipt specification tighter than or equal to supplier CoA limits for all RSMs - Ensure substitution degree result documented and reviewed prior to batch start. QA&QC | Low |
| 1.3 | Dispense RSMs or Resin or Oxyma or PyBOP | Using an uncalibrated scale | Reduced coupling efficiency; impurity formation; product fails specification | Yes | 4 | Failure to check in time before use | - External Calibration: Calibration of scale is managed by CMS system and performed periodically every one year by a qualified calibration contractor. - Internal calibration: Designated calibration personnel who is trained and qualified according to internal SOP, check the scales every three months. - For scales used for production, which are checked with standard weight before use. - Operator shall confirm that the scale is within calibration and suitable for charge quantity before use. |  | 2 | 2 | Low | - For electronic balances that have expired calibration periods or are malfunctioning, affix labels and remove them from the production workshop. Production department | Low |
| 1.4 | Dispense RSMs or Resin or Oxyma or PyBOP | Introduce foreign substances | Wrong amino acid incorporated — incorrect peptide sequence; batch rejection | Yes | 5 | Cross-use of tools and inadequate cleaning; Damaged packaging; | - Dispensing is performed in a dedicated glove box for sensitive materials. - The glove box is thoroughly cleaned (internal surfaces, gloves, and transfer port) between materials, it is recorded in the equipment usage log. Refer to SOPs QM03024 and CMa-Movable-T402.03 for details. - All tools (scoops, spatulas, containers) are dedicated, cleaned, dried, and pre-purged with nitrogen before placing into the glove box. - All dispensing operations are performed within the glove box, with sealed operations. - Operators have received cleaning training and are able to clean the glove box in the correct manner. - Operators shall perform tool counts before and after the use of dispensing tools to ensure no items are left behind, and confirm the integrity of the tools. - The Material entry and exit ports must be sealed must be sealed and are prohibited from being exposed to the environment to prevent contamination, cross-contamination, and the dropping of foreign matter. - All LDPE bags are sourced from qualified suppliers and are used only after release. - Materials shall be transferred via airlock during incoming and outgoing to avoid cross-contamination during transportation. All materials shall be kept sealed throughout the transfer process to prevent cross-contamination. - Keep fully sealed during transportation. - Transport with thermal covers for temperature control; transit time is controlled within 1 hour. - Upon material receipt, workshop personnel shall verify package integrity, contamination on outer packaging, material name, release status and other relevant information. |  | 2 | 2 | Medium | - All materials shall be checked for intact outer packaging before dispensing to avoid the introduction of foreign contaminants. - Nitrogen shall pass through filters before entering the glovebox. - Materials transportation shall avoid rainy and snowy weather conditions. Production department | Low |
| 1.5 | Dispense RSMs or Resin or Oxyma or PyBOP | Charge calculated incorrectly in the Raw Material Summary Table | Reduced coupling efficiency; impurity formation; product fails specification | Yes | 5 | Calculation errors and inadequate rechecking | - Master operation record review and approval before processing. - Calculation check sign off and charge check sign off in log sheet by second person. |  | 2 | 1 | Medium | - The calculation sheets will complete validation before use. CMMD;QA | Low |
| 1.6 | Dispense RSMs or Resin or Oxyma or PyBOP | Incorrect dispensing of material quantity | Reduced coupling efficiency; impurity formation; product fails specification | Yes | 5 | Incorrect numerical calculation, operation without double verification, Misread numerical values | - During dispensing operations, material calculations shall be verified by two operators, who shall sign for confirmation in the batch records. - After dispensing, when scanning codes to split materials, verify whether the residual weight after splitting is consistent with the actual weight of remaining materials. |  | 2 | 1 | Medium | - Perform material balance calculation upon completion of dispensing for each batch, cross-check the issued, consumed and residual material quantities. Production department; CMMD | Low |
| 2. Equipment confirmation & Recipe Edit & Control System |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2.1 | Equipment confirmation | Equipment are not applicable for operation purpose | Product fails specification, batch rejection | Yes | 5 | The equipment is not suitable for the manufacturing process; Equipment cleaning status; Use of uncalibrated flowmeter; Use of uncalibrated temperature probes |  | - The equipment will be selected as per the manufacturing process and batch size. Full-scale batch production confirmed equipment suitability. - Equipment performance will be qualified via IQ, OQ and PQ before use. - Maintenance schedules are in place for reactor with servicing conducted every three months. - Level II cleaning before PPQ batch. After each batch during production, all equipment will be cleaned as the cleaning validation protocol. - Calibration of flowmeter and thermometer transmitters are managed by CMS system and performed periodically every one year by a qualified calibration contractor. - Operator shall confirm that all instruments (temperature probes & pressure gauge & mass flow meter) are within calibration due date before use and recorded in MOR.. | 1 | 1 | Medium | - All equipment will be pre-production cleaned and pre-rinsed with reaction solvent before use. Production department | Low |
| 2.2 | Recipe edit | Incorrect conversion of process parameters (e.g., solvent addition volume, stirring time, temperature set point) | Product fails specification, batch rejection | Yes | 5 | Misunderstanding of process requirements; Manual input error during recipe editing; Material hold up in transfer line |  | - SOP specifies recipe editing steps and parameter conversion rules. - Two engineers shall edit the electronic recipe separately for the same project. Compare the electronic recipes edited by the two engineers, confirm each difference between them one by one, and correct any errors. To ensure the electronic recipe to be run is reviewed and accurate. - Before the running, production personnel and process chemist shall conduct the final review of the electronic recipe, after the review is correct, they shall sign and date on the operation record, then execute after QA approval. - Provide recipe editing training with process parameter conversion examples. - Full scale batch confirmed current recipe functions correctly as process expectation. | 2 | 2 | Medium | - Dual-person verification of the recipe shall be performed by production operators and process chemists before each module runs. Signatures and dates shall be recorded on the operation record once the verification is confirmed correct. Production department; CMMD | Low |
| 2.3 | Recipe Edit & Control System | PLC system failure or recipe error during automated SPPS cycle | Process step executed with wrong parameters; incorrect reagent added; processing time exceeded; batch record discrepancy | Yes | 5 | PLC software fault; recipe version error; power failure; network outage during batch |  | - PLC system validated per CSV requirements; - Recipe version controlled - UPS power backup for PLC system. - Operator alarm notification on PLC fault;. - Recipe reviewed and approved before PPQ campaign start (per GAP analysis). - Only current recipe is unlocked. The retired versions are locked from operation team. | 2 | 1 | Medium | - Dual-person verification of the recipe version shall be conducted prior to equipment operation. - Current recipe is verified by full-scale batch. Any recipe change prior PV needs to be tested by solvent run on each individual module. Production department ; CMMD | Low |
| 2.4 | Control System | Temperature deviation in synthesis reactor or buffer tanks | Coupling efficiency reduction; side reactions; impurity profile change | Yes | 5 | Chiller/heating system failure; jacket fluid leak; temperature sensor malfunction Pump malfunction TCU and heat transfer capabilities not meeting the process requirement, recipe error, jacket fouling, valve failure, air locks. |  | - Reactor and buffer tanks jacketed and temperature-controlled via PLC. - High and low temperature alarms shall be configured. - Equipment qualification includes temperature uniformity testing PLC - Operator training before campaign. - The automated control system will confirm that the temperature has reached the process range before proceeding to the next step. - The temperature control system and equipment temperature accuracy have been qualified. - Maintenance schedules are in place for TCU, pump & reactor with servicing conducted every three months. - Any valve operational fault will trigger an alarm and halt the program automatically. - SOP specifies recipe editing steps and parameter conversion rules. - Two engineers shall edit the electronic recipe separately for the same project. Compare the electronic recipes edited by the two engineers, confirm each difference between them one by one, and correct any errors. To ensure the electronic recipe to be run is reviewed and accurate. - Before the running, production personnel and process chemist shall conduct the final review of the electronic recipe, after the review is correct, they shall sign and date on the Operation record. - Provide recipe editing training with process parameter conversion examples. - Calibration of temperature transmitters are managed by CMS system and performed periodically every one year by a qualified calibration contractor. - Check equipment status at the start of MOR. | 3 | 2 | Medium | - Review the data after batch completion to identify abnormal temperature adjustment durations or consistent increases across consecutive batches. - Sufficient spare parts for equipment are stocked to enable timely replacement in case of defects. Production Dept. | Low |
| 2.5 | Control System | Nitrogen pressure loss during transfer or during N₂-blanketing, Undercharge or overcharge of Nitrogen | Potential moisture ingress to process; electrostatic risk if flow rate uncontrolled; | Yes | 3 | N₂ supply interruption; regulator failure; line blockage; pressure safety valve activation |  | - The program monitors nitrogen pressure and valve status in real time, the nitrogen shutdown time is automatically recorded by the program. Nitrogen supply to synthesizer via PCV is connected to DCS. Nitrogen high/low alarms will trigger system alarms and then high high alarms or low low alarms will trigger interlock. - Nitrogen is self-supplied via liquid nitrogen towers. Two liquid nitrogen towers are configured, one for operation and one as standby. The liquid level is monitored in real time to enable timely replenishment when the liquid level is low. - Maintenance schedules are implemented for reactor equipment every three months. - Any malfunction of the regulating valve and N₂ supply interruption will trigger an alarm and halt the running process simultaneously. - Nitrogen passes through nitrogen filters (0.22μm) before entering the workshop to remove impurities. - Check valves are installed on pipelines to prevent materials in reactors from flowing back into the pipelines. | 2 | 1 | Low | - If valve malfunction prevents timely material transfer, temporary nitrogen pipelines shall be prepared to start the pump for material transfer or waste discharge. Production department | Low |
| 2.6 | Control System | Insufficient compressed air | Solvent transfer failure; Filtration Failure Filtration Failure; Prolonged static residence of untransferred materials leads to extended De-Fmoc reaction time and elevated impurity levels. | Yes | 3 | Equipment damage; regulator failure; line blockage |  | - Two units of equipment are configured in active/standby mode to prevent equipment outage. - Maintenance schedules are in place for reactor with servicing conducted every three months. | 2 | 1 | Low | - If insufficient compressed air prevents valves or pneumatic pumps from starting and results in delayed material transfer, temporary nitrogen pipelines or alternative compressed air pipelines shall be used to start the pumps for material transfer or waste discharge. Production department | Low |
| 2.7 | Control System | Inadequate draining, sieve plate blockage in synthesis reactor, | Incomplete drainage of waste solvent; dilution of next step reagent; poor wash efficiency; extended hold time | Yes | 3 | Resin fines accumulation; resin swelling beyond design; mechanical damage to sieve |  | - Sieve plate inspection completed before PPQ. - The swelling temperature and duration shall be set via the recipe; waste discharge will run automatically once the present time elapses. - After completion of each batch, the equipment shall be disassembled and cleaned, and the sieve plate shall be inspected by QA and the Production manager. | 2 | 2 | Low | - Waste discharge duration shall be monitored and recorded by operators in the manufacturing operation record. Tuning forks are used for auxiliary judgment, and the PLC system automatically records the waste discharge time. - The recorded waste discharge time shall be double-checked by two personnel. Slow drain triggers operator check. Production department; CMMD | Low |
| 2.8 | Control System | Agitation error | Incomplete reaction, reduced coupling efficiency; impurity formation | Yes | 3 | PLC software fault; Operational error; Mechanical; electrical failure; incorrect set-point, recipe error. |  | - The stirring time is automatically recorded by the program, and a program recipe filing system is established, where only authorized personnel are allowed to edit and modify it to avoid operational errors. - Full-time monitoring is required during agitator parameter adjustment; after parameters are set, patrol inspections shall be performed every 1 h ± 15 min - Pre campaign training of manufacturing personnel. - Maintenance schedules are in place for reactor with servicing conducted every three months. - Equipment maintenance and electronic systems are equipped with dual hard disks to guarantee the integrity of operational data. - If the process is interrupted due to equipment abnormality, the system can resume operation from the interrupted point after the abnormality is rectified. SOP specifies recipe editing steps and parameter conversion rules. Each abnormal agitator stop is individually recorded/logged. - Two engineers shall edit the electronic recipe separately for the same project. Compare the electronic recipes edited by the two engineers, confirm each difference between them one by one, and correct any errors. To ensure the electronic recipe to be run is reviewed and accurate. - Before the running, production personnel and process chemist shall conduct the final review of the electronic recipe, after the review is correct, they shall sign and date on the operation record. - Provide recipe editing training with process parameter conversion examples. | 1 | 2 | Low | - Sufficient spare parts for the equipment are stocked to enable timely replacement in case of faults. - Every abnormal stop of agitation shall be documented as an abnormal event. Production department | Low |
| 2.9 | Control System | Nitrogen purging at the lower port stopped and failed to start, with flow or high rate below the required value. | Incomplete reaction, reduced coupling efficiency; impurity formation; resin fragmentation | Yes | 3 | PLC software fault; Operational error；N₂ supply interruption; regulator failure; line blockage; recipe version error; |  | - The program monitors nitrogen flow rate and valve status in real time. - Pre campaign training of manufacturing personnel. - PLC system validated per CSV requirements; - Recipe version controlled UPS power backup for PLC system.. - Recipe reviewed and approved before PPQ campaign start (per GAP analysis). - Nitrogen is self-supplied via liquid nitrogen towers. Two liquid nitrogen towers are configured, one for operation and one as standby. The liquid level is monitored in real time to enable timely replenishment when the liquid level is low. - Maintenance schedules are implemented for reactor equipment every three months. - Any malfunction of the regulating valve and N₂ supply interruption will trigger an alarm and halt the running process simultaneously. - Nitrogen passes through nitrogen filters (0.22 μm) before entering the workshop to remove impurities. - Check valves are installed on pipelines to prevent materials in reactors from flowing back into the pipelines. - Nitrogen is supplied to the synthesizer via the Pressure Control Valve (PCV), and this nitrogen circuit is connected to the Distributed Control System (DCS). Nitrogen pressure high/low alarms will trigger system alarms; high-high or low-low pressure alarms will activate interlocks. Nitrogen flow rate will only trigger system alarms. The target nitrogen pressure for bubbling operation is 0.05 MPa, and the flow rate is 11±5 m³. The flow parameter only triggers an alarm; an alarm will be activated if the target flow rate is not reached within 180 seconds. The pressure alarm threshold and pressure interlock threshold are -0.05 MPa and 0.5 MPa respectively. | 1 | 2 | Low | - Dual-person verification of the recipe version shall be conducted prior to equipment operation. Production department; CMMD | Low |
| 3. Pre-treatment |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3.1 | Pre-treatment &Resin Washing | Solvent substitution / wrong solvent used for washing | Residual incompatible solvent remains in the resin bed, which impairs resin swelling. | Yes | 4 | Incorrect storage vessel selected on PLC; Recipe error |  | - Dedicated fixed pipelines for each solvent reduce cross-connection risk. - SOP specifies recipe editing steps and parameter conversion rules. - Two engineers shall edit the electronic recipe separately for the same project. Compare the electronic recipes edited by the two engineers, confirm each difference between them one by one, and correct any errors. To ensure the electronic recipe to be run is reviewed and accurate. - Before the running, production personnel and process chemist shall conduct the final review of the electronic recipe, after the review is correct, they shall sign and date on the Operation record. | 1 | 2 | Low | - All materials are transferred via dedicated pumps. Production department | Low |
| 4. Resin swelling and washing |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4.1 | Resin swelling&washing | Resin not adequately swelled before loading | Non-uniform resin bed; poor reagent penetration; reduced coupling efficiency; impurities | Yes | 4 | Swelling step omitted or insufficient DMF/solvent used; temperature deviation during swelling |  | - Swelling step defined in MBR with time, solvent volume and temperature PAR. - PLC recipe controlled swelling time after agitation start. - After completion of charging, and sweep with nitrogen until all DMF are purged from the downstream line of the flowmeter to vessel. - Calibration of flowmeters is managed by CMS system and performed periodically every one year by a qualified calibration contractor. - Dedicated fixed pipelines for each solvent reduce cross-connection risk. - Maintenance schedules are in place for reactor with servicing conducted every three months. - DMF charging is controlled by the recipe program. The system manages the feeding via interlock between calibrated mass flowmeters and valves. | 1 | 1 | Low | - Pressure tightness test shall be performed on pipelines prior to use to ensure no material leakage occurs. Production department | Low |
| 5. Activation |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5.1 | Activation | Incomplete Activation (low Activation efficiency) | Deletion peptide formed (amino acid missing from sequence); product fails purity/sequence specification | Yes | 5 | Insufficient & over reagent charge (PyBOP/Oxyma/DIPEA), (DIC/Oxyma); temperature outside PAR; Use of uncalibrated scale; valve failures leading to liquid drain or over charge from solvent line | - PLC automated control and data logging of activation parameters, temperature, activation time, liquid charging volumes and charging time are automatically logged by the PLC system; solid charging quantities and charging time are recorded manually. - Calibration of scale is managed by CMS system and performed periodically every one year by a qualified calibration contractor. - Designated calibration personnel who is trained and qualified according to internal SOP, check the scales every three months. - For scales used for production, which are checked with standard weight before use. |  | 2 | 1 | Medium | - Pressure tightness test shall be performed on pipelines prior to use to ensure no material leakage occurs. Production department | Low |
| 5.2 | Activation | DIPEA (Gly24) charge quantity outside PAR (over- or under-charge) (CPP excursion) | Coupling efficiency reduction or side reactions; impurity formation | Yes | 4 | PLC recipe error; Using an uncalibrated scale Pump malfunction, transfer pipeline rupture or blockage | - DIPEA charge quantity is defined as a CPP in MOR. - PLC automated addition with flow meter / load cell confirmation. - SOP specifies recipe editing steps and parameter conversion rules. - Two engineers shall edit the electronic recipe separately for the same project. Compare the electronic recipes edited by the two engineers, confirm each difference between them one by one, and correct any errors. To ensure the electronic recipe to be run is reviewed and accurate. - Before the running, production personnel and process chemist shall conduct the final review of the electronic recipe, after the review is correct, they shall sign and date on the Operation record. - Provide recipe editing training with process parameter conversion examples - External Calibration: Calibration of scale is managed by CMS system and performed periodically every one year by a qualified calibration contractor. - Internal calibration: Designated calibration personnel who is trained and qualified according to internal SOP, check the scales every three months. - DIPEA: Pump malfunctions trigger automatic feedback and alarms, and the program will halt simultaneously. - DIPEA charging rate is automatically controlled by the system. |  | 2 | 1 | Low | - The PAR has been confirmed reference to the PFD report <WI-61120>. - Prior to charging, material identification is performed using a handheld Raman spectrometer to prevent incorrect material usage. - DIPEA addition pump equipped with backup pump for timely replacement upon malfunction. - Spare addition lines available for immediate replacement upon malfunction; - The tubing shall be replaced upon completion of each amino acid reaction cycle. Production department | Low |
| 5.3 | Activation | DIC charge quantity outside RAR (over- or under-charge) (CPP excursion) | Coupling efficiency reduction or side reactions; impurity formation | Yes | 4 | PLC recipe error; Using an uncalibrated scale | - DIC charge quantity is defined as a CPP in MOR. - PLC automated addition with flow meter / load cell confirmation. - SOP specifies recipe editing steps and parameter conversion rules. - Two engineers shall edit the electronic recipe separately for the same project. Compare the electronic recipes edited by the two engineers, confirm each difference between them one by one, and correct any errors. To ensure the electronic recipe to be run is reviewed and accurate. - Before the running, production personnel and process chemist shall conduct the final review of the electronic recipe, after the review is correct, they shall sign and date on the Operation record. - Provide recipe editing training with process parameter conversion examples - External Calibration: Calibration of scale is managed by CMS system and performed periodically every one year by a qualified calibration contractor. - Internal calibration: Designated calibration personnel who is trained and qualified according to internal SOP, check the scales every three months. - DIC: Pump malfunctions trigger automatic feedback and alarms, and the program will halt simultaneously. - DIC charging rate is automatically controlled by the system. - The weight of pre-divided materials will be recalculated and verified prior to charging. |  | 2 | 1 | Low | - The PAR has been confirmed reference to the PFD report <WI-61120>. - Prior to charging, material identification is performed using a handheld Raman spectrometer to prevent incorrect material usage. - DIC addition pump equipped with backup pump for timely replacement upon malfunction. - Spare addition lines available for immediate replacement upon malfunction; - The tubing shall be replaced upon completion of each amino acid reaction cycle. Production department | Low |
| 5.4 | Activation | Amino acid/ PyBOP/charge quantity outside PAR (over- or under-charge) | Coupling efficiency reduction or side reactions; impurity formation | Yes | 4 | Dispensing error; charging bags, material loss due to spills, materials sticking to SPPS walls and top base, | - Two-person manual charge check for amino acids/Oxyma/ PyBOP. The weight of pre-divided materials will be recalculated and verified prior to charging. - Charging bags are applied, and tight sealing is maintained between charging ports and bags to minimize spillage risk. - Charging bags are transparent for visual inspection. - Charging bags are applied, and tight sealing is maintained between charging ports and bags to minimize spillage risk. - Any spillage shall trigger an abnormality investigation, and corresponding corrective actions will be implemented based on risk assessment results. |  | 1 | 1 | Low | - Solid charging is followed by solvent rinsing. Coverage testing has been performed to guarantee effective rinsing performance. Production department | Low |
| 5.5 | Activation | Activation failure (premature DIC/Oxyma, DIPEA, PyBOP oxyma/ DIPEA hydrolysis or incorrect activation pH/conditions) | Active ester not formed; amino acid not coupled; deletion sequence impurity | Yes | 5 | Water contamination of DMF or amino acid; | - Oxyma quantity defined to ensure complete OBt ester formation. - Both the DMF storage tank and activation tank are protected with nitrogen. - Charging bags are applied, and tight sealing is maintained between charging ports and bags to minimize contamination risk. - Supplier Certificates of Analysis (CoA) shall be verified upon material receipt, and incoming inspection and release shall be performed in accordance with approved specifications. Ensure water content complies with the quality specification. |  | 2 | 1 | Medium | - Check the integrity of packaging bags before and after charging to prevent contamination caused by bag breakage. - Dual-person confirmation is required upon complete transfer of activation solution, and the confirmation shall be documented in the MOR. Production department, CMMD | Low |
| 5.6 | Activation | Amino acid/Oxyma / PyBOP charge error | Active ester not formed; amino acid not coupled; deletion sequence impurity | Yes | 5 | Incorrect material charging | - Two-person manual charge check for amino acids/Oxyma/ PyBOP - Prior to dispensing, material identification is performed using a handheld Raman spectrometer to prevent incorrect material usage. |  | 1 | 1 | Medium | - Only materials for current charging may be stored within the same production area. Production Dept. | Low |
| 5.7 | Activation | Oxyma (NMeAsn22- Orn11) charge quantity outside PAR (over- or under-charge) (CPP excursion) | Coupling efficiency reduction or side reactions; impurity formation | Yes | 4 | Dispensing error; charging bags, material loss due to spills, materials sticking to SPPS walls and top base, | - Oxyma charge quantity is defined as a CPP in MOR. - Two-person manual charge check for Oxyma. The weight of pre-divided materials will be recalculated and verified prior to charging. - Charging bags are applied, and tight sealing is maintained between charging ports and bags to minimize spillage risk. - Charging bags are transparent for visual inspection. - Charging bags are applied, and tight sealing is maintained between charging ports and bags to minimize spillage risk. - Any spillage shall trigger an abnormality investigation, and corresponding corrective actions will be implemented based on risk assessment results. |  | 1 | 1 | Low | - The PAR has been confirmed reference to the PFD report <WI-61120>. - Solid charging is followed by solvent rinsing. Coverage testing has been performed to guarantee effective rinsing performance. Production department | Low |
| 5.8 | Activation | Dissolution (jacket) and Activation temperature and (Gly24- Orn11) outside PAR (over- or under-charge) (CPP excursion) | Deletion peptide formed (amino acid missing from sequence); product fails purity/sequence specification | Yes | 5 | Chiller / heating system failure; jacket fluid leak; temperature sensor malfunction; TCU and heat transfer capability not meeting the process requirement; recipe error; jacket fouling; valve failure; air locks | - The reactor and the activation tank are jacketed and temperature-controlled via PLC, with high and low temperature alarms and interlock. - The automated control system confirms that the temperature has reached the process range before proceeding to the next step. - Any valve or pump fault triggers an alarm and halts the program automatically. - Calibration of temperature transmitters is managed by CMS and performed every year. - Operators are trained to respond to temperature alarms and the response is documented in the MOR. |  | 3 | 2 | Medium | - The PAR has been confirmed reference to the PFD report < WI-61120 >. - Configure the recipe so that the step is automatically paused / held when the batch or jacket temperature deviates outside the NOR; - Verify temperature uniformity of the activation tank at the minimum working volume. - Recipe interlock/hold: confirm it is implemented and tested before the first PPQ batch. CMMD; Production department | Low |
| 6. Coupling |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6.1 | Coupling | IPC endpoint error | Incomplete reaction impact the purity | Yes | 5 | Report error; Test method edit error; Sample contamination; Human error | - Test method will be reviewed and approved to avoid errors before it is effective. - The IPC endpoint will be presented in the MOR. - Before sampling, the online sampler shall be rinsed twice with the current reaction system, and the third sample shall be taken for testing. This ensures that the system has been completely displaced by the current reaction system, preventing inaccurate detection. - The material from the sampler rinses is discarded and is not returned to the equipment, thereby avoiding cross-contamination. - The next step shall only be initiated after two consecutive passing IPC samples confirm the reaction endpoint. - Approved test records are available. |  | 2 | 1 | Medium | - The trained and qualified project personnel analyse the samples. - The analysis results need to be reviewed by the second person before be reported to ensure the accuracy of the results. - A maximum of two samples may be tested simultaneously in the sample processing room. If two samples are processed concurrently, physical isolation shall be implemented throughout the sample treatment process to prevent sample contaminatio - Perform testing using validated analytical methods - Operators shall receive relevant training - Use brand-new sample vials for each test. Production department; CMMD | Low |
| 6.2 | Coupling | Incomplete Coupling (low Coupling efficiency) | Deletion peptide formed (amino acid missing from sequence); product fails purity/sequence specification | Yes | 5 | Coupling time too short; temperature outside PAR; Incomplete transfer of activation solution | - Ninhydrin / Bromophenol Blue IPC after coupling (negative = complete coupling). The next step shall only be initiated after two consecutive passing IPC samples confirm the reaction endpoint. - PLC automated control and data logging of coupling temperature. |  | 1 | 1 | Medium | - After transfer is completed, a two-person check shall be performed by personnel and documented in the MOR. Production department | Low |
| 6.3 | Coupling | Coupling temperature (Gly24- Orn11) outside PAR (over- or under-charge) (CPP excursion) | Deletion peptide formed (amino acid missing from sequence); product fails purity/sequence specification | Yes | 5 | Chiller / heating system failure; jacket fluid leak; temperature sensor malfunction; TCU and heat transfer capability not meeting the process requirement; recipe error; jacket fouling; valve failure; air locks | - The reactor and the activation tank are jacketed and temperature-controlled via PLC, with high and low temperature alarms and interlock. - The automated control system confirms that the temperature has reached the process range before proceeding to the next step. - Any valve or pump fault triggers an alarm and halts the program automatically. - Calibration of temperature transmitters is managed by CMS and performed every year. - Operators are trained to respond to temperature alarms and the response is documented in the MOR. |  | 3 | 2 | Medium | - The PAR has been confirmed reference to the PFD report < WI-61120 >. - Configure the recipe so that the step is automatically paused / held when the batch or jacket temperature deviates outside the NOR; - Verify temperature uniformity of the activation tank at the minimum working volume. - Recipe interlock/hold: confirm it is implemented and tested before the first PPQ batch. CMMD; Production department | Low |
| 7. Capping |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.1 | Capping | DMF/MeOH /DIPEA outside PAR (over- or under-charge) | Uncapped sequences proceed to next coupling; accumulation of deletion sequence impurities | Yes | 5 | MOR recipe error; Using an uncalibrated scale; Calculation errors and inadequate rechecking; Incorrect numerical calculation, operation without double verification, Misread numerical values | - External Calibration: Calibration of scale is managed by CMS system and performed periodically every one year by a qualified calibration contractor. - Internal calibration: Designated calibration personnel who is trained and qualified according to internal SOP, check the scales every three months. - For scales used for production, which are checked with standard weight before use. - Operator shall confirm that the scale is within calibration and suitable for charge quantity before use. - Master operation record review and approval before processing. - Calculation check sign off and charge check sign off in log sheet by second person. - During dispensing operations, material calculations shall be verified by two operators, who shall sign for confirmation in the batch records. |  | 2 | 1 | Medium | - The calculation sheets will complete validation before use. - Dual-person verification is implemented during charging to guarantee accurate charging quantity. - Validated spreadsheet is used to calculate quantity. Production department; CMMD | Low |
| 7.2 | Capping | Incomplete capping of unreacted amines | Uncapped sequences proceed to next coupling; accumulation of deletion sequence impurities | Yes | 5 | Capping time too short; temperature outside PAR; Material hold up in transfer line; | - PLC automated control and data logging of capping temperature and time. - Transfer shall be performed by personnel. Upon completion of transfer, confirmation and signature are required. |  | 2 | 1 | Medium | - After transfer is completed, residual materials in pipelines shall be purged with nitrogen to minimize the impact of pipeline residues. - After transfer is completed, a two-person check shall be performed by personnel and documented in the MOR. Production department | Low |
| 7.3 | Capping | Excessive residual capping solution | Accumulation of deletion sequence impurities | Yes | 5 | Capping time too long; blocked sieve plate / drain | - After completion of each batch, the equipment shall be disassembled and cleaned, and the sieve plate shall be inspected by QA and the production manager. - Material pipeline is separate and dedicated. |  | 2 | 1 | Medium | - Waste discharge duration shall be monitored and recorded by operators in the manufacturing operation record. Tuning forks are used for auxiliary judgment, and the PLC system automatically records the waste discharge time. - The recorded waste discharge time shall be double-checked by two personnel. Slow drain triggers operator check - Sufficient spare parts for the equipment are stocked to enable timely replacement in case of faults. Production department | Low |
| 7.4 | Washing (Post- Capping) | Incomplete removal of reagents/by-products between steps | Carry-over of excess coupling reagents into next step; impurity formation | Yes | 5 | Insufficient wash volumes; blocked sieve plate / drain; incorrect solvent used; PLC malfunction during wash cycle | - The wash volumes and cycles defined in MOR. - Equipment maintenance and electronic systems are equipped with dual hard disks to guarantee the integrity of operational data. - The system can resume operation from the interrupted point. - After completion of each batch, the equipment shall be disassembled and cleaned, and the sieve plate shall be inspected by QA and the Production manager. - Material pipeline is separate and dedicated. |  | 2 | 1 | Medium | - Waste discharge duration shall be monitored and recorded by operators in the manufacturing operation record. Tuning forks are used for auxiliary judgment, and the PLC system automatically records the waste discharge time. - The recorded waste discharge time shall be double-checked by two personnel. Slow drain triggers operator check - Sufficient spare parts for the equipment are stocked to enable timely replacement in case of faults. Production department | Low |
| 7.5 | Washing (Post- Capping) | Solvent substitution / wrong solvent used for washing | Insufficient washing; impact on coupling in next step | Yes | 3 | Incorrect storage vessel selected on PLC; Recipe error | - Dedicated fixed pipelines for each solvent reduce cross-connection risk. - SOP specifies recipe editing steps and parameter conversion rules. - Two engineers shall edit the electronic recipe separately for the same project. Compare the electronic recipes edited by the two engineers, confirm each difference between them one by one, and correct any errors. To ensure the electronic recipe to be run is reviewed and accurate. - Before the running, production personnel and process chemist shall conduct the final review of the electronic recipe, after the review is correct, they shall sign and date on the Operation record. |  | 1 | 2 | Low | - All materials are transferred via dedicated pumps. Production Dept | Low |
| 7.6 | Washing (Post- Capping) | DMF（post- Gly24 and Asn21） wash quantity outside PAR (over- or under-charge) (CPP excursion) | Deletion peptide formed (amino acid missing from sequence); product fails purity/sequence specification | Yes | 5 | valve failures leading to liquid drain or over charge from solvent line | - PLC automated control and data logging of activation parameters, temperature, activation time, liquid charging volumes and charging time are automatically logged by the PLC system; solid charging quantities and charging time are recorded manually. - Calibration of scale is managed by CMS system and performed periodically every one year by a qualified calibration contractor. - Designated calibration personnel who is trained and qualified according to internal SOP, check the scales every three months. - For scales used for production, which are checked with standard weight before use. - The DMF activation total quantity, the DMF quantity and the DMF rinse quantity are identified as CPPs for this unit operation they are controlled within the PAR and NOR defined in the MOR, and the charged quantities are automatically logged by the PLC system and verified against the NOR. |  | 2 | 1 | Medium | - The PAR has been confirmed reference to the PFD report < WI-61120>. - DMF wash quantity of post- Gly24 and Asn21, as a CPP in MOR. - Pressure tightness test shall be performed on pipelines prior to use to ensure no material leakage occurs. CMMD&Production Production Dept | Low |
| 7.7 | Capping | Acetic anhydride/pyridine charge quantity outside PAR (over- or under-charge) | Uncapped sequences proceed to next coupling; accumulation of deletion sequence impurities | Yes | 5 | MOR recipe error; Using an uncalibrated scale; Calculation errors and inadequate rechecking; Incorrect numerical calculation, operation without double verification, Misread numerical values | - External Calibration: Calibration of scale is managed by CMS system and performed periodically every one year by a qualified calibration contractor. - Internal calibration: Designated calibration personnel who is trained and qualified according to internal SOP, check the scales every three months. - For scales used for production, which are checked with standard weight before use. - Operator shall confirm that the scale is within calibration and suitable for charge quantity before use. - Master operation record review and approval before processing. - Calculation check sign off and charge check sign off in log sheet by second person. - During dispensing operations, material calculations shall be verified by two operators, who shall sign for confirmation in the batch records. |  | 2 | 1 | Medium | - The calculation sheets will complete validation before use. - After charging is completed, residual materials in pipelines shall be purged with nitrogen to minimize the impact of pipeline residues. - Use a dedicated capping solution preparation reactor - Dual-person verification is implemented during charging to guarantee accurate charging quantity. - Validated spreadsheet is used to calculate quantity. Production department.; CMMD | Low |
| 8. Fmoc Deprotection |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8.1 | Fmoc Deprotection | Incomplete Fmoc deprotection | Fmoc group remains on chain; coupling of next amino acid blocked; sequence deletion impurity | Yes | 5 | Insufficient piperidine concentration; using uncalibrated flowmeter; short contact time; temperature outside PAR; poor mixing, material blockage in the transfer line | - 20% piperidine/DMF concentration defined in MOR. - Calibration of flowmeter is managed by CMS system and performed periodically every one year by a qualified calibration contractor. - PLC automated control of temperature and reaction time. - Maintenance schedules are in place for reactor with servicing conducted every three months. - Flowmeters shall complete external calibration once per year. - Continuous nitrogen sparging is applied during deprotection to prevent uneven mixing of the mixture. - The 20% piperidine charge and the total De-Fmoc time are identified as CPPs for this unit operation they are controlled within the PAR and NOR defined in the MOR, and the charged quantity and the De-Fmoc time are automatically logged by the PLC system. |  | 2 | 1 | Medium | - After the preparation of 20% piperidine solution, stir for at least 0.5 hours to ensure uniform mixing. Production department | Low |
| 8.2 | Fmoc Deprotection | Introduce foreign substances | Fmoc group remains on chain; coupling of next amino acid blocked; sequence truncation impurity | Yes | 4 | Cross-use of pipelines | Material pipeline is separate and dedicated. |  | 1 | 2 | Low | - Dedicated transfer pumps are used, and all pipelines and pumps are clearly labeled. Production department | Low |
| 8.3 | Fmoc Deprotection | Excessively high Piperidine residual above | Piperidine causes side reactions in subsequent steps; potential impurity formation | Yes | 3 | Insufficient washing volume or cycles; wash solvent quality issue; blocked filter/drain; Washing durations； | - Washing steps defined with DMF volumes and number of cycles in MOR and PAR. - The number of washing cycles is controlled via the recipe. - SOP specifies recipe editing steps and parameter conversion rules. - Two engineers shall edit the electronic recipe separately for the same project. Compare the electronic recipes edited by the two engineers, confirm each difference between them one by one, and correct any errors. To ensure the electronic recipe to be run is reviewed and accurate. Before the running, production personnel and process chemist shall conduct the final review of the electronic recipe, after the review is correct, they shall sign and date on the Operation record. |  | 2 | 1 | Low | - Waste discharge duration shall be monitored and recorded by operators in the manufacturing operation record. Tuning forks are used for auxiliary judgment, and the PLC system automatically records the waste discharge time. - The recorded waste discharge time shall be double-checked by two personnel. Slow drain triggers operator check. Production department & CMMD | Low |
| 8.4 | Fmoc Deprotection | Temperature excursion outside PAR (CPP excursion) | Incomplete Fmoc deprotection; Fmoc group remains on chain; coupling of the next amino acid blocked; sequence deletion impurity | Yes | 4 | Chiller / heating system failure; jacket fluid leak; temperature sensor malfunction; TCU and heat transfer capability not meeting the process requirement; recipe error; jacket fouling; valve failure; air locks | - The temperature is controlled by the recipe program with alarm functions and interlock; an alarm is triggered when the temperature reaches the upper or lower limits set in the recipe and operators respond and take corrective actions. - Deprotection temperature and time are automatically logged by the PLC. Before solvents enter the reactor, they pass through a heat exchanger to adjust the temperature to approximately 20°C. Calibration of temperature transmitters is managed by CMS and performed every year. |  | 3 | 2 | Medium | - The PAR has been confirmed reference to the PFD report < WI-61120>. - Verify temperature uniformity of the reactor at the minimum working volume. CMMD; Production department | Low |
| 8.5 | Fmoc Deprotection | Piperidine/DMF exposure time (Pre-Ser20 Treatment 1&2) outside PAR (CPP excursion) | Fmoc group remains on chain; coupling of next amino acid blocked; sequence deletion impurity | Yes | 5 | Insufficient piperidine concentration; using uncalibrated flowmeter; short contact time; temperature outside PAR; poor mixing, material blockage in the transfer line | - 20% piperidine/DMF concentration defined in MOR. - Calibration of flowmeter is managed by CMS system and performed periodically every one year by a qualified calibration contractor. - PLC automated control of temperature and reaction time. - Maintenance schedules are in place for reactor with servicing conducted every three months. - Flowmeters shall complete external calibration once per year. - Continuous nitrogen sparging is applied during deprotection to prevent uneven mixing of the mixture. - The 20% piperidine charge and the total De-Fmoc time are identified as CPPs for this unit operation they are controlled within the PAR and NOR defined in the MOR, and the charged quantity and the De-Fmoc time are automatically logged by the PLC system. |  | 2 | 1 | Medium | - The PAR has been confirmed reference to the PFD report < WI-61120>. - After the preparation of 20% piperidine solution, stir for at least 0.5 hours to ensure uniform mixing. Production department | Low |
| 9. Washing (Post-Deprotection and Post-Coupling) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 9.1 | Washing (Post-De-protection) | IPAC quantity excursion outside PAR (CPP excursion) | Carry-over of piperidine, impurity formation | Yes | 4 | Insufficient wash volumes; blocked sieve plate / drain; incorrect solvent used; PLC malfunction during wash cycle | - Washing IPAC quantity are defined as CPPs with NOR / PAR in the MOR. - The wash volumes and cycles defined in MOR. - Equipment maintenance and electronic systems are equipped with dual hard disks to guarantee the integrity of operational data. - The system can resume operation from the interrupted point. - After completion of each batch, the equipment shall be disassembled and cleaned, and the sieve plate shall be inspected by QA and the production manager. Material pipeline is separate and dedicated. |  | 2 | 2 | Low | - The PAR has been confirmed reference to the PFD report < WI-61120>. - Waste discharge duration shall be monitored and recorded by operators in the manufacturing operation record. Tuning forks are used for auxiliary judgment, and the PLC system automatically records the waste discharge time. - The recorded waste discharge time shall be double-checked by two personnel. Slow drain triggers operator check - Sufficient spare parts for the equipment are stocked to enable timely replacement in case of faults. Production department | Low |
| 9.2 | Washing(Post-De-protection) | Solvent substitution / wrong solvent used for washing | Impact on coupling in next step; Incomplete removal of deprotection residues | Yes | 4 | Incorrect storage vessel selected on PLC; Recipe error | - Dedicated fixed pipelines for each solvent reduce cross-connection risk. - SOP specifies recipe editing steps and parameter conversion rules. - Two engineers shall edit the electronic recipe separately for the same project. Compare the electronic recipes edited by the two engineers, confirm each difference between them one by one, and correct any errors. To ensure the electronic recipe to be run is reviewed and accurate. - Before the running, production personnel and process chemist shall conduct the final review of the electronic recipe, after the review is correct, they shall sign and date on the Operation record. |  | 1 | 2 | Low | - All materials are transferred via dedicated pumps. Production department | Low |
| 9.3 | Washing (Re-swelling) | Resin not adequately swelled before loading; | Non-uniform resin bed; poor reagent penetration; reduced coupling efficiency; impurities | Yes | 4 | Insufficient wash volumes; Incomplete removal of IPAc; blocked sieve plate / drain; incorrect solvent used; PLC malfunction during wash cycle | - The wash volumes and cycles defined in MOR. - Equipment maintenance and electronic systems are equipped with dual hard disks to guarantee the integrity of operational data. - The system can resume operation from the interrupted point. - After completion of each batch, the equipment shall be disassembled and cleaned, and the sieve plate shall be inspected by QA and the production manager. - Material pipeline is separate and dedicated. |  | 1 | 2 | Low | - Waste discharge duration shall be monitored and recorded by operators in the manufacturing operation record. Tuning forks are used for auxiliary judgment, and the PLC system automatically records the waste discharge time. - The recorded waste discharge time shall be double-checked by two personnel. Slow drain triggers operator check - Sufficient spare parts for the equipment are stocked to enable timely replacement in case of faults. Production department | Low |
| 9.4 | Washing (Re-swelling) | Solvent substitution / wrong solvent used for washing | Impact on coupling in next step; Incomplete removal of deprotection residues; Swelling failure. | Yes | 4 | Incorrect storage vessel selected on PLC; Recipe error | - Dedicated fixed pipelines for each solvent reduce cross-connection risk. - SOP specifies recipe editing steps and parameter conversion rules. - Two engineers shall edit the electronic recipe separately for the same project. Compare the electronic recipes edited by the two engineers, confirm each difference between them one by one, and correct any errors. To ensure the electronic recipe to be run is reviewed and accurate. - Before the running, production personnel and process chemist shall conduct the final review of the electronic recipe, after the review is correct, they shall sign and date on the Operation record. |  | 1 | 2 | Low | - All materials are transferred via dedicated pumps. Production department | Low |
| 9.5 | Washing (Post- Coupling) | Incomplete removal of reagents/by-products between steps | Carry-over of excess coupling reagents into next step; impurity formation | Yes | 4 | Insufficient wash volumes; blocked sieve plate / drain; incorrect solvent used; PLC malfunction during wash cycle | - The wash volumes and cycles defined in MOR. - Equipment maintenance and electronic systems are equipped with dual hard disks to guarantee the integrity of operational data. - The system can resume operation from the interrupted point. - After completion of each batch, the equipment shall be disassembled and cleaned, and the sieve plate shall be inspected by QA and the Production manager. - Material pipeline is separate and dedicated. |  | 2 | 2 | Low | - Waste discharge duration shall be monitored and recorded by operators in the manufacturing operation record. Tuning forks are used for auxiliary judgment, and the PLC system automatically records the waste discharge time. - The recorded waste discharge time shall be double-checked by two personnel. Slow drain triggers operator check - Sufficient spare parts for the equipment are stocked to enable timely replacement in case of faults. Production department | Low |
| 9.6 | Washing (Post- Coupling) | Solvent substitution / wrong solvent used for washing | Insufficient washing; impact on coupling in next step | Yes | 3 | Incorrect storage vessel selected on PLC; Recipe error | - Dedicated fixed pipelines for each solvent reduce cross-connection risk. - SOP specifies recipe editing steps and parameter conversion rules. - Two engineers shall edit the electronic recipe separately for the same project. Compare the electronic recipes edited by the two engineers, confirm each difference between them one by one, and correct any errors. To ensure the electronic recipe to be run is reviewed and accurate. - Before the running, production personnel and process chemist shall conduct the final review of the electronic recipe, after the review is correct, they shall sign and date on the Operation record. |  | 2 | 2 | Low | - All materials are transferred via dedicated pumps. Production department | Low |
| 10. Continuous coupling cycles (Repeat 4~9 expect 7 refer to Above) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 10.1 | Continuous coupling cycles | Missing a sequence - deprotection cycle omitted (Fmoc group not removed, so the next amino acid cannot couple) | Fmoc group remains on chain; coupling of next amino acid blocked; sequence truncation impurity | Yes | 5 | Recipe error the deprotection cycle is omitted from the electronic recipe, or the wrong module / cycle sequence is programmed | - SOP specifies recipe editing steps and parameter conversion rules. - Two engineers shall edit the electronic recipe separately for the same project. Compare the electronic recipes edited by the two engineers, confirm each difference between them one by one, and correct any errors. To ensure the electronic recipe to be run is reviewed and accurate. - Before the running, production personnel and process chemist shall conduct the final review of the electronic recipe, after the review is correct, they shall sign and date on the Operation record. |  | 2 | 1 | Medium | - Dual-person verification of the recipe shall be performed by production operators and process chemists before each module runs. Signatures and dates shall be recorded on the operation record once the verification is confirmed correct. Production department.; CMMD | Low |
| 10.2 | Continuous coupling cycles | Missing a sequence - activation / coupling cycle omitted (the programmed amino acid is not incorporated) | Deletion peptide | Yes | 5 | Recipe error - the activation / coupling cycle is omitted from the electronic recipe, the wrong amino acid is programmed, or the wrong module is selected | - SOP specifies recipe editing steps and parameter conversion rules. - Two engineers shall edit the electronic recipe separately for the same project. Compare the electronic recipes edited by the two engineers, confirm each difference between them one by one, and correct any errors. To ensure the electronic recipe to be run is reviewed and accurate. Before the running, production personnel and process chemist shall conduct the final review of the electronic recipe, after the review is correct, they shall sign and date on the Operation record. |  | 2 | 1 | Medium | - Dual-person verification of the recipe shall be performed by production operators and process chemists before each module runs. Signatures and dates shall be recorded on the operation record once the verification is confirmed correct. Production department /CMMD | Low |
| 10.3 | Continuous coupling cycles | Missing a sequence - capping cycle omitted (unreacted amines remain uncapped and accumulate as deletion sequences) | Accumulation of deletion sequence impurities | Yes | 5 | Recipe error the capping cycle is omitted from the electronic recipe, or the capping module is not enabled | - SOP specifies recipe editing steps and parameter conversion rules. - Two engineers shall edit the electronic recipe separately for the same project. Compare the electronic recipes edited by the two engineers, confirm each difference between them one by one, and correct any errors. To ensure the electronic recipe to be run is reviewed and accurate. - Before the running, production personnel and process chemist shall conduct the final review of the electronic recipe, after the review is correct, they shall sign and date on the Operation record. |  | 2 | 1 | Medium | - Dual-person verification of the recipe shall be performed by production operators and process chemists before each module runs. Signatures and dates shall be recorded on the operation record once the verification is confirmed correct. Production department.; CMMD | Low |
| 11. De-swell and washing |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 11.1 | Washing (IPAC) | Solvent substitution / wrong solvent used for washing | Excessive piperidine/ amino acid /Oxyma residue; Affect the TFA concentration in Step 2; Residual piperidine cleave the Fmoc product | Yes | 4 | Incorrect storage vessel selected on PLC; recipe error; | - Dedicated fixed pipelines for each solvent reduce cross-connection risk. - SOP specifies recipe editing steps and parameter conversion rules. - Two engineers shall edit the electronic recipe separately for the same project. Compare the electronic recipes edited by the two engineers, confirm each difference between them one by one, and correct any errors. To ensure the electronic recipe to be run is reviewed and accurate. - Before the running, production personnel and process chemist shall conduct the final review of the electronic recipe, after the review is correct, they shall sign and date on the Operation record. |  | 2 | 2 | Low | - All materials are transferred via dedicated pumps. Production department | Low |
| 11.2 | Washing (IPAC) | Incomplete removal piperidine/ amino acid /Oxyma | Impurity formation | Yes | 4 | Insufficient wash volumes; blocked sieve plate / drain; incorrect solvent used; PLC malfunction during wash cycle | - The wash volumes and cycles defined in MOR. - Equipment maintenance and electronic systems are equipped with dual hard disks to guarantee the integrity of operational data. - The system can resume operation from the interrupted point. - After completion of each batch, the equipment shall be disassembled and cleaned, and the sieve plate shall be inspected by QA and the Production manager. - Material pipeline is separate and dedicated. - External Calibration: Calibration of scale is managed by CMS system and performed periodically every one year by a qualified calibration contractor. |  | 2 | 2 | Low | - Waste discharge duration shall be monitored and recorded by operators in the manufacturing operation record. Tuning forks are used for auxiliary judgment, and the PLC system automatically records the waste discharge time. - The recorded waste discharge time shall be double-checked by two personnel. Slow drain triggers operator check - Sufficient spare parts for the equipment are stocked to enable timely replacement in case of faults. Production department | Low |
| 11.3 | Washing (IPA) | Incomplete resin deswelling | Excessively long drying time | Yes | 3 | Use of uncalibrated balance, MOR editing error; blocked sieve plate / drain; incorrect solvent used; PLC malfunction during wash cycle | - MOR will be reviewed and approved to avoid errors before it is effective. - External Calibration: Calibration of scale is managed by CMS system and performed periodically every one year by a qualified calibration contractor. - Internal calibration: Designated calibration personnel who is trained and qualified according to internal SOP, check the scales every three months. - For scales used for production, which are checked with standard weight before use. - Operator shall confirm that the scale is within calibration and suitable for charge quantity before use. |  | 1 | 1 | Low | - For electronic balances that have expired calibration periods or are malfunctioning, affix labels and remove them from the production workshop. Production department | Low |
| 11.4 | Washing (IPA) | IPA charge quantity outside PAR (over- or under-charge) (CPP excursion) | Excessive piperidine/ amino acid /Oxyma residue; Affect the TFA concentration in Step 2; Residual piperidine cleave the Fmoc product | Yes | 4 | Incorrect storage vessel selected on PLC; recipe error; | - Charge quantities defined in MOR - Dedicated fixed pipelines for each solvent reduce cross-connection risk. - SOP specifies recipe editing steps and parameter conversion rules. - Two engineers shall edit the electronic recipe separately for the same project. Compare the electronic recipes edited by the two engineers, confirm each difference between them one by one, and correct any errors. To ensure the electronic recipe to be run is reviewed and accurate. - Before the running, production personnel and process chemist shall conduct the final review of the electronic recipe, after the review is correct, they shall sign and date on the Operation record. |  | 2 | 2 | Low | - The PAR has been confirmed reference to the PFD report. < WI-61120>. - All materials are transferred via dedicated pumps. Production department | Low |
| 11.5 | Washing (Toluene) | Solvent substitution / wrong solvent used for washing | Excessive piperidine/ amino acid //Oxyma residue; Affect the TFA concentration in Step 2; Residual piperidine cleave the Fmoc product | Yes | 4 | Incorrect storage vessel selected on PLC; recipe error; | - Dedicated fixed pipelines for each solvent reduce cross-connection risk. - SOP specifies recipe editing steps and parameter conversion rules. - Two engineers shall edit the electronic recipe separately for the same project. Compare the electronic recipes edited by the two engineers, confirm each difference between them one by one, and correct any errors. To ensure the electronic recipe to be run is reviewed and accurate. - Before the running, production personnel and process chemist shall conduct the final review of the electronic recipe, after the review is correct, they shall sign and date on the Operation record. |  | 2 | 2 | Low | - All materials are transferred via dedicated pumps. Production department | Low |
| 11.6 | Washing (Toluene) | Incomplete removal piperidine/ amino acid /Oxyma | Impurity formation | Yes | 4 | Insufficient wash volumes; blocked sieve plate / drain; incorrect solvent used; PLC malfunction during wash cycle | - The wash volumes and cycles defined in MOR. - Equipment maintenance and electronic systems are equipped with dual hard disks to guarantee the integrity of operational data. - The system can resume operation from the interrupted point. - After completion of each batch, the equipment shall be disassembled and cleaned, and the sieve plate shall be inspected by QA and the Production manager. - Material pipeline is separate and dedicated. - External Calibration: Calibration of scale is managed by CMS system and performed periodically every one year by a qualified calibration contractor. |  | 2 | 2 | Low | - Waste discharge duration shall be monitored and recorded by operators in the manufacturing operation record. Tuning forks are used for auxiliary judgment, and the PLC system automatically records the waste discharge time. - The recorded waste discharge time shall be double-checked by two personnel. Slow drain triggers operator check - Sufficient spare parts for the equipment are stocked to enable timely replacement in case of faults. Production department | Low |
| 11.7 | Washing (Toluene) | Incomplete resin deswelling | Excessively long drying time | Yes | 3 | Use of uncalibrated balance, MOR editing error; blocked sieve plate / drain; incorrect solvent used; PLC malfunction during wash cycle | - MOR will be reviewed and approved to avoid errors before it is effective. - External Calibration: Calibration of scale is managed by CMS system and performed periodically every one year by a qualified calibration contractor. - Internal calibration: Designated calibration personnel who is trained and qualified according to internal SOP, check the scales every three months. - For scales used for production, which are checked with standard weight before use. - Operator shall confirm that the scale is within calibration and suitable for charge quantity before use. |  | 1 | 1 | Low | - For electronic balances that have expired calibration periods or are malfunctioning, affix labels and remove them from the production workshop. Production department | Low |
| 11.8 | Washing (Toluene) | Toluene charge quantity outside PAR (over- or under-charge) (CPP excursion) | Excessive piperidine/ amino acid /Oxyma residue; Affect the TFA concentration in Step 2; Residual piperidine cleave the Fmoc product | Yes | 4 | Incorrect storage vessel selected on PLC; recipe error; | - Charge quantities defined in MOR - Dedicated fixed pipelines for each solvent reduce cross-connection risk. - SOP specifies recipe editing steps and parameter conversion rules. - Two engineers shall edit the electronic recipe separately for the same project. Compare the electronic recipes edited by the two engineers, confirm each difference between them one by one, and correct any errors. To ensure the electronic recipe to be run is reviewed and accurate. - Before the running, production personnel and process chemist shall conduct the final review of the electronic recipe, after the review is correct, they shall sign and date on the Operation record. |  | 2 | 2 | Low | - The PAR has been confirmed reference to the PFD report. < WI-61120>. - All materials are transferred via dedicated pumps. Production department | Low |
| 11.9 | Washing (MTBE) | Solvent substitution / wrong solvent used for washing | Excessive piperidine residue; Affect the TFA concentration in Step 2; Residual piperidine cleave the Fmoc product | Yes | 4 | Incorrect storage vessel selected on PLC; recipe error; | - Dedicated fixed pipelines for each solvent reduce cross-connection risk. - SOP specifies recipe editing steps and parameter conversion rules. - Two engineers shall edit the electronic recipe separately for the same project. Compare the electronic recipes edited by the two engineers, confirm each difference between them one by one, and correct any errors. To ensure the electronic recipe to be run is reviewed and accurate. - Before the running, production personnel and process chemist shall conduct the final review of the electronic recipe, after the review is correct, they shall sign and date on the Operation record. |  | 2 | 2 | Low | - All materials are transferred via dedicated pumps. Production department | Low |
| 11.10 | Washing (MTBE) | Incomplete resin deswelling | Excessively long drying time | Yes | 3 | Insufficient wash volumes; blocked sieve plate / drain; incorrect solvent used; PLC malfunction during wash cycle | - The wash volumes and cycles defined in MOR. - Equipment maintenance and electronic systems are equipped with dual hard disks to guarantee the integrity of operational data. - The system can resume operation from the interrupted point. - After completion of each batch, the equipment shall be disassembled and cleaned, and the sieve plate shall be inspected by QA and the Production manager. - Material pipeline is separate and dedicated. |  | 1 | 1 | Low | - Waste discharge duration shall be monitored and recorded by operators in the manufacturing operation record. Tuning forks are used for auxiliary judgment, and the PLC system automatically records the waste discharge time. - The recorded waste discharge time shall be double-checked by two personnel. Slow drain triggers operator check - Sufficient spare parts for the equipment are stocked to enable timely replacement in case of faults. Production department | Low |
| 11.11 | Washing (MTBE) | Solvent substitution / wrong solvent used for washing | Deswelling Failure；Excessively long drying time | Yes | 4 | Incorrect storage vessel selected on PLC; labelling error on solvent drum; Use of uncalibrated scale; recipe error; | - Dedicated fixed pipelines for each solvent reduce cross-connection risk. - Calibration of scale is managed by CMS system and performed periodically every one year by a qualified calibration contractor. - Designated calibration personnel who is trained and qualified according to internal SOP, check the scales every month. - For scales used for production, which are checked with standard weight before use. - SOP specifies recipe editing steps and parameter conversion rules. - Two engineers shall edit the electronic recipe separately for the same project. Compare the electronic recipes edited by the two engineers, confirm each difference between them one by one, and correct any errors. To ensure the electronic recipe to be run is reviewed and accurate. - Before the running, production personnel and process chemist shall conduct the final review of the electronic recipe, after the review is correct, they shall sign and date on the Operation record. |  | 1 | 2 | Low | - All materials are transferred via dedicated pumps. Production department | Low |
| 11.12 | Washing (MTBE) | IPC endpoint error | Excessive piperidine; Affect the TFA concentration in Step 2；Residual piperidine cleave the Fmoc of product | Yes | 5 | Report error; Test method edit error; Sample contamination; Human error | - Test method will be reviewed and approved to avoid errors before it is effective. - The IPC endpoint will be presented in the MOR. - Before sampling, the online sampler shall be rinsed twice with the current reaction system, and the third sample shall be taken for testing. This ensures that the system has been completely displaced by the current reaction system, preventing inaccurate detection. - The material from the sampler rinses is discarded and is not returned to the equipment, thereby avoiding cross-contamination. - The next step shall only be initiated after two consecutive passing IPC samples confirm the reaction endpoint. |  | 2 | 1 | Medium | - The trained and qualified project personnel analyse the samples. - The analysis results need to be reviewed by the second person before be reported to ensure the accuracy of the results. - Only one sample should be handled for testing in IPC room. - Perform testing using validated analytical methods - Operators shall receive relevant training - Use brand-new sample vials for each test. Production department; CMMD | Low |
| 11.13 | Washing (MTBE) | MTBE charge quantity outside PAR (over- or under-charge) (CPP excursion) | Excessive piperidine/ amino acid /Oxyma residue; Affect the TFA concentration in Step 2; Residual piperidine cleave the Fmoc of product | Yes | 4 | Incorrect storage vessel selected on PLC; recipe error; | - Charge quantities defined in MOR - Dedicated fixed pipelines for each solvent reduce cross-connection risk. - SOP specifies recipe editing steps and parameter conversion rules. - Two engineers shall edit the electronic recipe separately for the same project. Compare the electronic recipes edited by the two engineers, confirm each difference between them one by one, and correct any errors. To ensure the electronic recipe to be run is reviewed and accurate. - Before the running, production personnel and process chemist shall conduct the final review of the electronic recipe, after the review is correct, they shall sign and date on the Operation record. |  | 2 | 2 | Low | - The PAR has been confirmed reference to the PFD report. < WI-61120>. - All materials are transferred via dedicated pumps. Production department | Low |
| 12. Drying and Discharge |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 12.1 | Drying | Insufficient drying | Failed LOD specification; impact on downstream cleavage yield | Yes | 3 | Drying time too short; dryer temperature deviation; sampling inhomogeneity and testing error | - Maintenance schedules are in place for reactor with servicing conducted every three months. - Drying temperature defined in MOR. - LOD IPC at end of drying to confirm specification. - The LOD test method shall be validated prior to PPQ execution. - Sampling shall be performed in accordance with SOP (QM03015) and double-checked by two operators during sampling. - The drying time before sampling for testing shall be specified in the MOR. |  | 2 | 1 | Low | - Spare vacuum pumps are available for immediate replacement upon detection of malfunctions. Production department | Low |
| 12.2 | Drying | Over-drying | Resin becomes brittle; fines generated; potential chemical degradation of protecting groups; thermal degradation of peptide resin | Yes | 2 | Dryer temperature too high; drying time extended beyond MOR specification; Continuous stirring may cause resin fragmentation. | - Met the IPC ( LOD NMT 9.0% ) then stop drying. - PLC temperature alarm on over-temperature; manual check procedure. - The drying process is controlled by recipes to ensure batch-to-batch consistency. - Temperature alarms are configured; operators will perform timely adjustments upon alarm activation. - The maximum drying temperature is limited to 30°C. - The stirring interval is controlled by the PLC system, and operators conduct monitoring during stirring. |  | 2 | 2 | Low | - The maximum drying duration is specified in the MOR. - The maximum drying temperature is limited to 30°C. CMMD&Production drtment | Low |
| 12.3 | Resin Discharge | Incomplete discharging | Product loss | No | 1 | Residual materials remain inside the reactor, resin adhesion to the reactor wall | - For collection, the material is transferred from the synthesizer to a glovebox (with a collection bag in place) using the stirrer. Residual material remaining in the synthesizer is then gathered with a dedicated rake. Sealed operations are performed during the collecting. - Training Operators on Standard Material Receiving Procedures. |  | 2 | 1 | Low | - Adopt dual confirmation via visual inspection and weighing. Production department | Low |
| 13. Final Packaging and Sampling |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13.1 | Packaging / Final Sampling | Sample not representative of batch (non-homogeneous sampling) | Release testing result does not represent true batch quality; potential OOE result missed | Yes | 4 | 1.Incorrect sampling location; 2.Insufficient mixing/blending prior to sampling; 3.Smpling from one end of tray only; 4. In the release test, unqualified raw materials were reported to be qualified due to personnel errors, such as CoA filling errors, calculation errors, etc | - Sampling procedure defined in MBR per APIEM-LD-243 - Multiple sampling locations specified; sampling plan validated - QC sampling performed by trained QC personnel; - Release test data will be reviewed by a second person. - The final data will be reviewed and released by QA. |  | 2 | 2 | Low | No additional mitigation required. | Low |

6. Equipment Train Risk Assessment

In addition to the process FMEA above, a specific equipment train risk assessment is performed to evaluate the suitability of the proposed manufacturing equipment for the PPQ campaign. Reference the ESA: RA-140020030101.01. The manufacturing equipment meets the production requirements for the PPQ phase.

| Equipment Unit | Critical Parameter | Specification / Value | Risk Assessment for New Equipment / Train |
| --- | --- | --- | --- |
| Synthesis Reactor（TJ4S-1213-Sy15） |  |  |  |
| Synthesis reactor | Material of Construction (MOC) | Hastelloy | MOC is compatible with all process reagents and solvents (DMF, IPAc, MTBE, Piperidine, DIPEA, DIC, Oxyma，PyBOP, MeOH, Toluene, Pyridine and acetic anhydride). No risk. |
| Synthesis reactor | Capacity | 3000 L | Adequate for 48 kg 2-CTC resin input (~576 L max batch volume). No risk. |
| Synthesis reactor | Stirring volume | Stirring volume range of equipment (at rest): 51.54 L | Min batch volume: 96 L. No risk. |
| Synthesis reactor | Agitator speed | 5-40 rpm | Reference agitation speed: 20±10 rpm Confirmed sufficient for resin/DMF mixing. Same with reference batch. No risk. |
| Synthesis reactor | Solvents (DMF, IPAc, MTBE) charging accuracy | Charging accuracy is ≤2 kg when the charging amount is 5~70 kg; Charging accuracy is within ±3% when the charging amount is ≥70 kg. | DMF charging amount: Min 90.01 kg IPAc charging amount: Min 406.47 kg MTBE charging amount: Min 345.01 kg Toluene charging amount: Min 406.47 kg No risk. |
| Synthesis reactor | Solvents (Toluene) charging accuracy | Charging accuracy is ≤2 kg when the charging amount is 2.00~300 kg; Charging accuracy is within ±3% | Toluene charging amount: 406.47 ~ 431.61 kg (The charging range each drum is between 2.00~ 170.00 kg) No risk. |
| Synthesis reactor | 20% Piperidine solution charging accuracy | Charging accuracy of 20% piperidine solution is ≤1 kg when the charging amount is 10~70 kg; Charging accuracy of 20% piperidine solution is within ±3% when the charging amount is ≥70 kg. | 20% Piperidine solution charging amount : 346.78 ~ 368.23 kg,， No risk. |
| Synthesis reactor | Solution charging speed | ≤ 40 L/min (0.53 m/s) via 40 mm pipeline | Within permitted range for electrostatic hazard elimination per site procedure. No electrostatic risk. |
| Synthesis reactor | Temperature control | Control range of temperature of the equipment capability: -4~53 ℃ | The control range of batch temperature from process requirement is within the control range of equipment capability |
| Synthesis reactor | Temperature measurement | Minimum volume of the inner temperature probe (at rest): 26.91 L | The minimum batch volume is within the minimum temperature sensing capability, ensuring good temperature monitoring. |
| Synthesis reactor | Activation solution transfer speed | ≤ 114 L/min (1.51 m/s) via 40 mm pipeline | Within permitted range for electrostatic hazard elimination. No electrostatic risk. |
| Synthesis reactor | Deprotection solution transfer | ≤ 57 L/min (0.76 m/s) via 40 mm pipeline | Within permitted range. No electrostatic risk. |
| Synthesis reactor | Drain to waste | Diaphragm pump, ~455 L / 15–20 min, 40 mm pipeline | Within permitted range. No electrostatic risk. |
| Synthesis reactor | Nitrogen protection/inertization/nitrogen sparging | The equipment is equipped with nitrogen protection, inertization and nitrogen sparging functions. The range of nitrogen flow rate in the workshop is 1.5~15 m³/h. | The nitrogen sparging flow rate range required by the process is within the nitrogen sparging flow rate range of the equipment. |
| Synthesis reactor | Sampling for IPC testing | The equipment is equipped with a sampling port. | No risk. |
| Synthesis reactor | 2-CTC resin charge | Charge via powder charging bag | No risk. |
| Synthesis reactor | Control system | PLC (validated) | PLC system will be used — validated; no risk. |
| Synthesis reactor | Spray ball | Yes — CIP spray ball installed | Used for cleaning; no risk for washing or rinsing. |
| Activation Tank (TJ4S-1213-Sy15-R01) |  |  |  |
| Activation tank | Amino acid charging | Manual charge via N₂-protected flexible isolator | Same system as reference batch. No risk. |
| Activation tank | Material of Construction (MOC) | Hastelloy | MOC is compatible with all process reagents and solvents (DMF, DIPEA, DIC, Oxyma，PyBOP). No risk. |
| Activation tank | Capacity | 1500 L | ~325 L Max batch volume. No risk. |
| Activation tank | Stirring volume | Stirring volume range of equipment (at rest): 84.4 L | Min batch volume: 144 L. No risk. |
| Activation tank | Agitator speed | 15-130 rpm | Reference agitation speed: 95±10 rpm Confirmed sufficient for resin/DMF mixing. Same with reference batch. No risk. |
| Activation tank | DMF charging accuracy | Charging accuracy is ≤2 kg when the charging amount is 5~70 kg; Charging accuracy is within ±3% when the charging amount is ≥70 kg. | DMF charging amount: Min 91.01 kg |
| Activation tank | DMF charging speed | ≤ 40 L/min via 40 mm pipeline | Within permitted electrostatic range. No risk. |
| Activation tank | Temperature control | Control range of temperature of the equipment capability: -6~54 ℃ | The control range of batch temperature from process requirement is within the control range of equipment capability |
| Activation tank | Temperature measurement | The temperature measurement of the equipment is from the bottom. | The minimum batch volume is within the minimum temperature sensing capability, ensuring good temperature monitoring. |
| Activation tank | Transfer to synthesis reactor | Fixed pipeline, N₂ pressure ≤ 0.15 MPa, ≤ 114 L/min | Within permitted electrostatic range. No risk. |
| Activation tank | Solid charge | Charge via powder charging bag | No risk. |
| Activation tank | Control system | PLC (validated) | PLC system will be used — validated; no risk. |
| Activation tank | Spray ball | Yes — CIP spray ball installed | Used for cleaning; no risk for washing or rinsing. |
| Activation Tank (TJ4S-1213-SPS01-AT01/ TJ4S-1213-SPS01-AT02) |  |  |  |
| Agitation tank | Material of Construction (MOC) | Stainless Steel | MOC is compatible with all process solvents (DMF, Piperidine). No risk. |
| Agitation tank | DMF charging speed | ≤ 60 L/min via 40 mm pipeline | Within permitted electrostatic range. No risk. |
| Agitation tank | Piperidine charging speed | ≤ 40 L/min via 40 mm pipeline | Within permitted range. No electrostatic risk. |
| Agitation tank | Agitator speed | 20~120 rpm | Piperidine soluble in DMF. No risk. |
| Agitation tank | Transfer to reactor | ≤ 114 L/min (1.51 m/s) via 40 mm pipeline | Within permitted range. No electrostatic risk. |
| Glass-lined Reactor (TJ4S-1213-R22) |  |  |  |
| Glass-lined Reactor | Material of Construction (MOC) | Glass-lined | MOC is compatible with all process reagents and solvents (pyridine, acetic anhydride and DMF,). No risk. |
| Glass-lined Reactor | Stirring volume | Stirring volume range of equipment (at rest): 82.30 L | Min batch volume: 288L. |
| Glass-lined Reactor | Agitator speed | 0~138 rpm | The range of agitation speed from process requirements: 95±10 rpm. No risk. |
| Glass-lined Reactor | Temperature measurement | Minimum volume of the inner temperature probe (at rest): 123.10 L | The minimum batch volume is within the minimum temperature sensing capability, ensuring good temperature monitoring. |
| Glass-lined Reactor | Solvents (DMF) charging accuracy | Charging accuracy is ±3.0% when the charging amount is 2.00~300 kg; | DMF charging amount: 44.14 ~ 272.77 kg No risk. |
| Glass-lined Reactor | Reagents (pyridine) charging accuracy | Charging accuracy is ±3.0% when the charging amount is 2.00~300 kg; | Pyridine charging amount: 6.25 ~ 6.63 kg No risk. |
| Glass-lined Reactor | Reagents (acetic anhydride) charging accuracy | Charging accuracy is ±3.0% when the charging amount is 2.00~300 kg; | Acetic anhydride charging amount: 6.71 ~ 7.13 kg No risk. |
| Glass-lined Reactor | Temperature control | Control range of temperature of the equipment capability: -2.9~112.3 ℃ | The control range of batch temperature from process requirement is within the control range of equipment capability |
| Glass-lined Reactor | Transfer to reactor | ≤ 114 L/min (1.51 m/s) via 40 mm pipeline | Within permitted range. No electrostatic risk. |
| Activation Tank (TJ4S-1200-AT02） |  |  |  |
| Agitation tank | Material of Construction (MOC) | Hastelloy | MOC is compatible with all process reagents and solvents (Oxyma/DMF). No risk. |
| Agitation tank | Stirring volume | Stirring volume range of equipment (at rest): 20.05 L | Min batch volume: 45.88 L. No risk. |
| Agitation tank | Agitator speed | Max:119.84 rpm | Oxyma soluble in DMF. No risk. |
| Agitation tank | Oxyma charge | Charge via powder charging bag | No risk. |
| Agitation tank | Solvents (DMF) charging accuracy | Charging accuracy is ±3.0% when the charging amount is 2.00~300 kg; | DMF charging amount: 43.50 ~ 47.50 kg No risk. |
| Agitation tank | Transfer to reactor | ≤ 114 L/min (1.51 m/s) via 40 mm pipeline | Within permitted range. No electrostatic risk. |

7. Risk Summary and Action Plan

The table below summarises all failure modes by risk level. High risk items require documented mitigation actions with owners and target completion dates. These must be resolved prior to PPQ campaign commencement. Medium risk items require either implementation of mitigation or documented risk acceptance rationale, approved by the appropriate level.

| Step # | Unit Operation | Failure Mode (Hazard) | Risk Level | Residual Risk | Status | Action / Owner / Milestone | Completion Evidence | Signature/Date |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| High Risk Items - Mitigation Required Before PPQ |  |  |  |  |  |  |  |  |  |
| Medium Risk Items — Mitigation Recommended or Risk Acceptance Required |  |  |  |  |  |  |  |  |  |
| 1.1 | Dispense RSMs or Resin or Oxyma or PyBOP | Incorrect RSM , resin, Oxyma or PyBOP used (wrong material / wrong lot) | Medium | Low | Closed | - All staging zones for different materials of the same project shall be fitted with labels for identification inside the plant. - Different materials within the same project shall be separated by red demarcation lines inside the plant. Owner: Production department |  |  |  |
| 1.2 | Dispense RSMs or Resin or Oxyma or PyBOP | Out-of-specification RSM used (e.g. low assay, wrong substitution degree) | Medium | Low | Closed | - Confirm receipt specification tighter than or equal to supplier CoA limits for all RSMs - Ensure substitution degree result documented and reviewed prior to batch start. Owner: QA&QC |  |  |  |
| 1.3 | Dispense RSMs or Resin or Oxyma or PyBOP | Using an uncalibrated scale | Low | Low | Closed | - For electronic balances that have expired calibration periods or are malfunctioning, affix labels and remove them from the production workshop. Production department |  |  |  |
| 1.4 | Dispense RSMs or Resin or Oxyma or PyBOP | Introduce foreign substances | Medium | Low | Closed | - All materials shall be checked for intact outer packaging before dispensing to avoid the introduction of foreign contaminants. - Nitrogen shall pass through filters before entering the glovebox. - Materials transportation shall avoid rainy and snowy weather conditions. Owner: Production department |  |  |  |
| 1.5 | Dispense RSMs or Resin or Oxyma or PyBOP | Charge calculated incorrectly in the Raw Material Summary Table | Medium | Low | Closed | - The calculation sheets will complete validation before use. Owner: CMMD;QA |  |  |  |
| 1.6 | Dispense RSMs or Resin or Oxyma or PyBOP | Incorrect dispensing of material quantity | Medium | Low | Open | - Perform material balance calculation upon completion of dispensing for each batch, cross-check the issued, consumed and residual material quantities. Production department; CMMD Due Date: Prior to product release |  |  |  |
| 2.1 | Equipment confirmation | Equipment are not applicable for operation purpose | Medium | Low | Open | - All equipment will be pre-production cleaned and pre-rinsed with reaction solvent before use. Owner: Production department Due Date: Prior to product release |  |  |  |
| 2.2 | Recipe edit | Incorrect conversion of process parameters (e.g., solvent addition volume, stirring time, temperature set point) | Medium | Low | Open | - Dual-person verification of the recipe shall be performed by production operators and process chemists before each module runs. Signatures and dates shall be recorded on the operation record once the verification is confirmed correct. Owner: Production department; CMMD Due Date: Prior to product release |  |  |  |
| 2.3 | Recipe Edit & Control System | PLC system failure or recipe error during automated SPPS cycle | Medium | Low | Open | - Dual-person verification of the recipe version shall be conducted prior to equipment operation. - Current recipe is verified by full-scale batch. Any recipe change prior PV needs to be tested by solvent run on each individual module. Owner: Production department ; CMMD Due Date: Prior to product release |  |  |  |
| 2.4 | Control System | Temperature deviation in synthesis reactor or buffer tanks | Medium | Low | Closed | - Review the data after batch completion to identify abnormal temperature adjustment durations or consistent increases across consecutive batches. - Sufficient spare parts for equipment are stocked to enable timely replacement in case of defects. Owner: Production department |  |  |  |
| 2.5 | Control System | Nitrogen pressure loss during transfer or during N₂-blanketing, Undercharge or overcharge of Nitrogen | Low | Low | Open | - If valve malfunction prevents timely material transfer, temporary nitrogen pipelines shall be prepared to start the pump for material transfer or waste discharge. Owner: Production department Due Date: Prior to product release |  |  |  |
| 2.6 | Control System | Insufficient compressed air | Low | Low | Open | - If insufficient compressed air prevents valves or pneumatic pumps from starting and results in delayed material transfer, temporary nitrogen pipelines or alternative compressed air pipelines shall be used to start the pumps for material transfer or waste discharge. Owner: Production department Due Date: Prior to product release |  |  |  |
| 2.7 | Control System | Inadequate draining, sieve plate blockage in synthesis reactor, | Low | Low | Open | - Waste discharge duration shall be monitored and recorded by operators in the manufacturing operation record. Tuning forks are used for auxiliary judgment, and the PLC system automatically records the waste discharge time. - The recorded waste discharge time shall be double-checked by two personnel. Slow drain triggers operator check. Owner: Production department; CMMD Due Date: Prior to product release |  |  |  |
| 2.8 | Control System | Agitation error | Low | Low | Open | - Sufficient spare parts for the equipment are stocked to enable timely replacement in case of faults. - Every abnormal stop of agitation shall be documented as an abnormal event. Owner: Production department Due Date: Prior to product release |  |  |  |
| 2.9 | Control System | Nitrogen purging at the lower port stopped and failed to start, with flow or high rate below the required value. | Low | Low | Open | - Dual-person verification of the recipe version shall be conducted prior to equipment operation. Owner: Production department; CMMD Due Date: Prior to product release |  |  |  |
| 3.1 | Pre-treatment &Resin Washing | Solvent substitution / wrong solvent used for washing | Low | Low | Open | - All materials are transferred via dedicated pumps. Owner: Production department Due Date: Prior to product release |  |  |  |
| 4.1 | Resin swelling&washing | Resin not adequately swelled before loading | Low | Low | Open | - Pressure tightness test shall be performed on pipelines prior to use to ensure no material leakage occurs. Owner: Production department Due Date: Prior to product release |  |  |  |
| 5.1 | Activation | Incomplete Activation (low Activation efficiency) | Medium | Low | Closed | - Pressure tightness test shall be performed on pipelines prior to use to ensure no material leakage occurs. Owner: Production department |  |  |  |
| 5.2 | Activation | DIPEA (Gly24) charge quantity outside PAR (over- or under-charge) (CPP excursion) | Low | Low | Closed | - The PAR has been confirmed reference to the PFD report <WI-61120>. - Prior to charging, material identification is performed using a handheld Raman spectrometer to prevent incorrect material usage. - DIPEA addition pump equipped with backup pump for timely replacement upon malfunction. - Spare addition lines available for immediate replacement upon malfunction; - The tubing shall be replaced upon completion of each amino acid reaction cycle. Owner: Production department |  |  |  |
| 5.3 | Activation | DIC charge quantity outside RAR (over- or under-charge) (CPP excursion) | Low | Low | Closed | - The PAR has been confirmed reference to the PFD report <WI-61120>. - Prior to charging, material identification is performed using a handheld Raman spectrometer to prevent incorrect material usage. - DIC addition pump equipped with backup pump for timely replacement upon malfunction. - Spare addition lines available for immediate replacement upon malfunction; - The tubing shall be replaced upon completion of each amino acid reaction cycle. Owner: Production department |  |  |  |
| 5.4 | Activation | Amino acid/ PyBOP/charge quantity outside PAR (over- or under-charge) | Low | Low | Open | - Solid charging is followed by solvent rinsing. Coverage testing has been performed to guarantee effective rinsing performance. Owner: Production department Due Date: Prior to product release |  |  |  |
| 5.5 | Activation | Activation failure (premature DIC/Oxyma, DIPEA, PyBOP oxyma/ DIPEA hydrolysis or incorrect activation pH/conditions) | Medium | Low | Closed | - Check the integrity of packaging bags before and after charging to prevent contamination caused by bag breakage. - Dual-person confirmation is required upon complete transfer of activation solution, and the confirmation shall be documented in the MOR. Owner: Production department, CMMD |  |  |  |
| 5.6 | Activation | Amino acid/Oxyma / PyBOP charge error | Medium | Low | Closed | - Only materials for current charging may be stored within the same production area. Owner: Production Dept. |  |  |  |
| 5.7 | Activation | Oxyma (NMeAsn22- Orn11) charge quantity outside PAR (over- or under-charge) (CPP excursion) | Low | Low | Open | - The PAR has been confirmed reference to the PFD report <WI-61120>. - Solid charging is followed by solvent rinsing. Coverage testing has been performed to guarantee effective rinsing performance. Owner: Production department Due Date: Prior to product release |  |  |  |
| 5.8 | Activation | Dissolution (jacket) and Activation temperature and (Gly24- Orn11) outside PAR (over- or under-charge) (CPP excursion) | Medium | Low | Open | - The PAR has been confirmed reference to the PFD report < WI-61120 >. - Configure the recipe so that the step is automatically paused / held when the batch or jacket temperature deviates outside the NOR; - Verify temperature uniformity of the activation tank at the minimum working volume. - Recipe interlock/hold: confirm it is implemented and tested before the first PPQ batch. Owner: CMMD; Production department Due Date: Prior to product release |  |  |  |
| 6.1 | Coupling | IPC endpoint error | Medium | Low | Closed | - The trained and qualified project personnel analyse the samples. - The analysis results need to be reviewed by the second person before be reported to ensure the accuracy of the results. - A maximum of two samples may be tested simultaneously in the sample processing room. If two samples are processed concurrently, physical isolation shall be implemented throughout the sample treatment process to prevent sample contaminatio - Perform testing using validated analytical methods - Operators shall receive relevant training - Use brand-new sample vials for each test. Owner: Production department; CMMD |  |  |  |
| 6.2 | Coupling | Incomplete Coupling (low Coupling efficiency) | Medium | Low | Closed | - After transfer is completed, a two-person check shall be performed by personnel and documented in the MOR. Production department |  |  |  |
| 6.3 | Coupling | Coupling temperature and (Gly24- Orn11) outside PAR (over- or under-charge) (CPP excursion) | Medium | Low | Open | - The PAR has been confirmed reference to the PFD report < WI-61120 >. - Configure the recipe so that the step is automatically paused / held when the batch or jacket temperature deviates outside the NOR; - Verify temperature uniformity of the activation tank at the minimum working volume. - Recipe interlock/hold: confirm it is implemented and tested before the first PPQ batch. Owner: CMMD; Production department Due Date: Prior to product release |  |  |  |
| 7.1 | Capping | DMF/MeOH /DIPEA outside PAR (over- or under-charge) | Medium | Low | Open | - The calculation sheets will complete validation before use. - Dual-person verification is implemented during charging to guarantee accurate charging quantity. - Validated spreadsheet is used to calculate quantity. Owner: Production department; CMMD Due Date: Prior to product release |  |  |  |
| 7.2 | Capping | Incomplete capping of unreacted amines | Medium | Low | Closed | - After transfer is completed, residual materials in pipelines shall be purged with nitrogen to minimize the impact of pipeline residues. - After transfer is completed, a two-person check shall be performed by personnel and documented in the MOR. Owner: Production department |  |  |  |
| 7.3 | Capping | Excessive residual capping solution | Medium | Low | Closed | - Waste discharge duration shall be monitored and recorded by operators in the manufacturing operation record. Tuning forks are used for auxiliary judgment, and the PLC system automatically records the waste discharge time. - The recorded waste discharge time shall be double-checked by two personnel. Slow drain triggers operator check - Sufficient spare parts for the equipment are stocked to enable timely replacement in case of faults. Owner: Production department |  |  |  |
| 7.4 | Washing (Post- Capping) | Incomplete removal of reagents/by-products between steps | Medium | Low | Closed | - Waste discharge duration shall be monitored and recorded by operators in the manufacturing operation record. Tuning forks are used for auxiliary judgment, and the PLC system automatically records the waste discharge time. - The recorded waste discharge time shall be double-checked by two personnel. Slow drain triggers operator check - Sufficient spare parts for the equipment are stocked to enable timely replacement in case of faults. Owner: Production department |  |  |  |
| 7.5 | Washing (Post- Capping) | Solvent substitution / wrong solvent used for washing | Low | Low | Closed | - All materials are transferred via dedicated pumps. Owner: Production Dept |  |  |  |
| 7.6 | Washing (Post- Capping) | DMF（post- Gly24 and Asn21） wash quantity outside PAR (over- or under-charge) (CPP excursion) | Medium | Low | Closed | - The PAR has been confirmed reference to the PFD report < WI-61120>. - DMF wash quantity of post- Gly24 and Asn21, as a CPP in MOR. - Pressure tightness test shall be performed on pipelines prior to use to ensure no material leakage occurs. Owner: CMMD&Production |  |  |  |
| 7.7 | Capping | Acetic anhydride/pyridine charge quantity outside PAR (over- or under-charge) | Medium | Low | Open | - The calculation sheets will complete validation before use. - After charging is completed, residual materials in pipelines shall be purged with nitrogen to minimize the impact of pipeline residues. - Use a dedicated capping solution preparation reactor - Dual-person verification is implemented during charging to guarantee accurate charging quantity. - Validated spreadsheet is used to calculate quantity. Owner: Production department.; CMMD Due Date: Prior to product release |  |  |  |
| 8.1 | Fmoc Deprotection | Incomplete Fmoc deprotection | Medium | Low | Closed | - After the preparation of 20% piperidine solution, stir for at least 0.5 hours to ensure uniform mixing. Owner: Production department |  |  |  |
| 8.2 | Fmoc Deprotection | Introduce foreign substances | Low | Low | Closed | - Dedicated transfer pumps are used, and all pipelines and pumps are clearly labelled. Owner: Production department |  |  |  |
| 8.3 | Fmoc Deprotection | Excessively high Piperidine residual above | Low | Low | Closed | - Waste discharge duration shall be monitored and recorded by operators in the manufacturing operation record. Tuning forks are used for auxiliary judgment, and the PLC system automatically records the waste discharge time. - The recorded waste discharge time shall be double-checked by two personnel. Slow drain triggers operator check. Owner:Production department & CMMD |  |  |  |
| 8.4 | Fmoc Deprotection | Temperature excursion outside PAR (CPP excursion) | Medium | Low | Closed | - The PAR has been confirmed reference to the PFD report < WI-61120>. - Verify temperature uniformity of the reactor at the minimum working volume. Owner: CMMD; Production department Due Date: Prior to product release |  |  |  |
| 8.5 | Fmoc Deprotection | Piperidine/DMF exposure time (Pre-Ser20 Treatment 1&2) outside PAR (CPP excursion) | Medium | Low | Closed | - The PAR has been confirmed reference to the PFD report < WI-61120>. - After the preparation of 20% piperidine solution, stir for at least 0.5 hours to ensure uniform mixing. Owner: Production department |  |  |  |
| 9.1 | Washing (Post-De-protection) | IPAC quantity excursion outside PAR (CPP excursion) | Low | Low | Closed | - The PAR has been confirmed reference to the PFD report < WI-61120>. - Waste discharge duration shall be monitored and recorded by operators in the manufacturing operation record. Tuning forks are used for auxiliary judgment, and the PLC system automatically records the waste discharge time. - The recorded waste discharge time shall be double-checked by two personnel. Slow drain triggers operator check。 - Sufficient spare parts for the equipment are stocked to enable timely replacement in case of faults. Owner: Production department |  |  |  |
| 9.2 | Washing(Post-De-protection) | Solvent substitution / wrong solvent used for washing | Low | Low | Closed | - All materials are transferred via dedicated pumps. Owner: Production department |  |  |  |
| 9.3 | Washing (Re-swelling) | Resin not adequately swelled before loading; | Low | Low | Closed | - Waste discharge duration shall be monitored and recorded by operators in the manufacturing operation record. Tuning forks are used for auxiliary judgment, and the PLC system automatically records the waste discharge time. - The recorded waste discharge time shall be double-checked by two personnel. Slow drain triggers operator check - Sufficient spare parts for the equipment are stocked to enable timely replacement in case of faults. Owner: Production department |  |  |  |
| 9.4 | Washing (Re-swelling) | Solvent substitution / wrong solvent used for washing | Low | Low | Closed | - All materials are transferred via dedicated pumps. Owner: Production department |  |  |  |
| 9.5 | Washing (Post- Coupling) | Incomplete removal of reagents/by-products between steps | Low | Low | Closed | - Waste discharge duration shall be monitored and recorded by operators in the manufacturing operation record. Tuning forks are used for auxiliary judgment, and the PLC system automatically records the waste discharge time. - The recorded waste discharge time shall be double-checked by two personnel. Slow drain triggers operator check - Sufficient spare parts for the equipment are stocked to enable timely replacement in case of faults. Owner: Production department |  |  |  |
| 9.6 | Washing (Post- Coupling) | Solvent substitution / wrong solvent used for washing | Low | Low | Closed | - All materials are transferred via dedicated pumps. Owner: Production department |  |  |  |
| 10.1 | Continuous coupling cycles | Missing a sequence - deprotection cycle omitted (Fmoc group not removed, so the next amino acid cannot couple) | Medium | Low | Closed | - Dual-person verification of the recipe shall be performed by production operators and process chemists before each module runs. Signatures and dates shall be recorded on the operation record once the verification is confirmed correct. Owner: Production department.; CMMD |  |  |  |
| 10.2 | Continuous coupling cycles | Missing a sequence - activation / coupling cycle omitted (the programmed amino acid is not incorporated) | Medium | Low | Closed | - Dual-person verification of the recipe shall be performed by production operators and process chemists before each module runs. Signatures and dates shall be recorded on the operation record once the verification is confirmed correct. Owner: Production department /CMMD |  |  |  |
| 10.3 | Continuous coupling cycles | Missing a sequence - capping cycle omitted (unreacted amines remain uncapped and accumulate as deletion sequences) | Medium | Low | Closed | - Dual-person verification of the recipe shall be performed by production operators and process chemists before each module runs. Signatures and dates shall be recorded on the operation record once the verification is confirmed correct. Owner: Production department.; CMMD |  |  |  |
| 11.1 | Washing (IPAC) | Solvent substitution / wrong solvent used for washing | Low | Low | Closed | - All materials are transferred via dedicated pumps. Owner:Production department |  |  |  |
| 11.2 | Washing (IPAC) | Incomplete removal piperidine/ amino acid /Oxyma | Low | Low | Closed | - Waste discharge duration shall be monitored and recorded by operators in the manufacturing operation record. Tuning forks are used for auxiliary judgment, and the PLC system automatically records the waste discharge time. - The recorded waste discharge time shall be double-checked by two personnel. Slow drain triggers operator check - Sufficient spare parts for the equipment are stocked to enable timely replacement in case of faults. Owner: Production department |  |  |  |
| 11.3 | Washing (IPA) | Incomplete resin deswelling | Low | Low | Closed | - For electronic balances that have expired calibration periods or are malfunctioning, affix labels and remove them from the production workshop. Owner: Production department |  |  |  |
| 11.4 | Washing (IPA) | IPA charge quantity outside PAR (over- or under-charge) (CPP excursion) | Low | Low | Closed | - The PAR has been confirmed reference to the PFD report. < WI-61120>. - All materials are transferred via dedicated pumps. Owner: Production department |  |  |  |
| 11.5 | Washing (Toluene) | Solvent substitution / wrong solvent used for washing | Low | Low | Closed | - All materials are transferred via dedicated pumps. Production department |  |  |  |
| 11.6 | Washing (Toluene) | Incomplete removal piperidine/ amino acid /Oxyma | Low | Low | Closed | - Waste discharge duration shall be monitored and recorded by operators in the manufacturing operation record. Tuning forks are used for auxiliary judgment, and the PLC system automatically records the waste discharge time. - The recorded waste discharge time shall be double-checked by two personnel. Slow drain triggers operator check - Sufficient spare parts for the equipment are stocked to enable timely replacement in case of faults. Owner: Production department |  |  |  |
| 11.7 | Washing (Toluene) | Incomplete resin deswelling | Low | Low | Closed | - For electronic balances that have expired calibration periods or are malfunctioning, affix labels and remove them from the production workshop. Owner: Production department |  |  |  |
| 11.8 | Washing (Toluene) | Toluene charge quantity outside PAR (over- or under-charge) (CPP excursion) | Low | Low | Closed | - The PAR has been confirmed reference to the PFD report. < WI-61120>. - All materials are transferred via dedicated pumps. Owner: Production department |  |  |  |
| 11.9 | Washing (MTBE) | Solvent substitution / wrong solvent used for washing | Low | Low | Closed | - All materials are transferred via dedicated pumps. Owner: Production department |  |  |  |
| 11.10 | Washing (MTBE) | Incomplete resin deswelling | Low | Low | Closed | - Waste discharge duration shall be monitored and recorded by operators in the manufacturing operation record. Tuning forks are used for auxiliary judgment, and the PLC system automatically records the waste discharge time. - The recorded waste discharge time shall be double-checked by two personnel. Slow drain triggers operator check - Sufficient spare parts for the equipment are stocked to enable timely replacement in case of faults. Owner: Production department |  |  |  |
| 11.11 | Washing (MTBE) | Solvent substitution / wrong solvent used for washing | Low | Low | Closed | - All materials are transferred via dedicated pumps. Owner: Production department |  |  |  |
| 11.12 | Washing (MTBE) | IPC endpoint error | Medium | Low | Closed | - The trained and qualified project personnel analyse the samples. - The analysis results need to be reviewed by the second person before be reported to ensure the accuracy of the results. - Only one sample should be handled for testing in IPC room. - Perform testing using validated analytical methods - Operators shall receive relevant training - Use brand-new sample vials for each test. Owner: Production department; CMMD |  |  |  |
| 11.13 | Washing (MTBE) | MTBE charge quantity outside PAR (over- or under-charge) (CPP excursion) | Low | Low | Closed | - The PAR has been confirmed reference to the PFD report. < WI-61120>. - All materials are transferred via dedicated pumps. Owner: Production department |  |  |  |
| 12.1 | Drying | Insufficient drying | Low | Low | Closed | - Spare vacuum pumps are available for immediate replacement upon detection of malfunctions. Production department |  |  |  |
| 12.2 | Drying | Over-drying | Low | Low | Closed | - The maximum drying duration is specified in the MOR. - The maximum drying temperature is limited to 30°C. Owner: CMMD&Production department |  |  |  |
| 12.3 | Resin Discharge | Incomplete discharging | Low | Low | Closed | - Adopt dual confirmation via visual inspection and weighing. Owner: Production department |  |  |  |
| Low Risk Items — Acceptable; No Mitigation Required |  |  |  |  |  |  |  |  |  |
| All | All steps | All remaining failure modes assessed as Low risk | Low | Low | Closed | - No additional action required. Maintain current controls and monitoring programme. |  |  |  |

According to the assessment above, there was no High risk items identified. For the medium risks identified in Table above, the residual risks will be reduced to low risk after implementation of risk mitigation actions. The proposed action items can be effectively implemented with no risk, and can be properly managed in accordance with our routine production practices. No other significant recommended actions are required.

8. Risk Control and Mitigation

According to risk assessment above, there are no High-risk items identified in the manufacture of ASYM-141366 (Step 1 of ASYM-140020 / LSN4182924) at Asymchem Life Science (Tianjin) Co., Ltd. (No.265, South Avenue, Binhai New Area TEDA, Tianjin, 300462, China). The proposed action items for residual medium risk items can be effectively implemented with no risk and can be properly managed in accordance with our routine production practices. No other significant recommended actions are required.

9. Risk Review Schedule

This FMEA will be reviewed at the intervals below, or when triggered by any of the conditions listed. Reviewers must confirm that the risk assessment remains current and accurately represents the control strategy.

| Review Trigger | Action Required |
| --- | --- |
| Periodic review (minimum every 3 years) | Full FMEA review; confirm risk levels remain current; update if process or controls have changed. |
| Significant process change (CPP or CQA-linked parameter) | Reassess affected failure modes; update FMEA; re-approve at appropriate level. |
| Critical or major deviation impacting product quality | Assess whether failure mode was captured in FMEA; if not, add and re-score; update controls. |
| Post-PPQ completion | Review FMEA against actual PPQ results; confirm all risk levels remain valid. |
| Annual Product Review (APR) findings | Review if APR identifies adverse trends or new failure modes not captured in current FMEA. |
| New regulatory requirement or guidance | Assess impact on FMEA scoring or CQA list; update as required. |

10. Risk Assessment Conclusion

According to the risk assessment above, the risk from batch execution for ASYM-141366 manufacture (Step 1 of LSN4182924 / ASYM-140020) is acceptable. The proposed action items can be effectively implemented with no risk and can be properly managed in accordance with our routine production practices. No other significant recommended actions are required.

Table 10.1: Eloralintide fragment L2 Critical Process Parameters (CPPs) in Step 1

| Parameter | Type | PAR | Unit |
| --- | --- | --- | --- |
| Resin loading - DIPEA quantity | CPP | 5.24 – 6.69 | eq |
| Resin loading - Dissolution temperature | CPP | 17.0 – 33.8 | °C |
| Resin loading – Coupling Reaction temperature | CPP | 17.2 – 32.7 | °C |
| Resin loading – Capping DMF wash quantity | CPP | 3x NLT 7.58 | kg/kg |
| Deprotection temperature (Standard AA) | CPP | 17.0 – 34.8 | °C |
| Deprotection temperature (Pre-Ser20) | CPP | 14.6 – 34.7 | °C |
| Piperidine/DMF exposure time (Pre-Ser20 Treatment 1) | CPP | 6 - 68 | min |
| Piperidine/DMF exposure time (Pre-Ser20 Treatment 2) | CPP | 6 - 83 | min |
| Post Deprotection IPAc wash quantity | CPP | 4x NLT 7.93 | kg/kg |
| Oxyma quantity (Standard AA) | CPP | 1.94 – 2.25 | eq |
| Oxyma quantity (NMeAsn22, αMePhe15) | CPP | 1.75 – 2.03 | eq |
| Oxyma quantity (Asn21) | CPP | 2.50 – 2.93 | eq |
| Dissolution jacket temperature (Standard AA) | CPP | 12.6 – 26.5 | °C |
| Dissolution jacket temperature (Phe23) | CPP | 12.0 – 29.5 | °C |
| Dissolution jacket temperature (NMeAsn22, αMePhe15) | CPP | 12.6 - 26.9 | °C |
| Dissolution jacket temperature (Asn21) | CPP | 12.2 – 27.9 | °C |
| DIC quantity (Standard AA) | CPP | 2.01 -2.48 | eq |
| Activation temperature (Standard AA) | CPP | 12.6 – 26.5 | °C |
| Activation temperature (Phe23) | CPP | - 2.5 – 13.6 | °C |
| Activation temperature (NMeAsn22, αMePhe15) | CPP | 12.6 - 26.9 | °C |
| Activation temperature (Asn21) | CPP | 12.2 – 27.9 | °C |
| Coupling temperature (Standard AA) | CPP | 18.6 – 28.4 | °C |
| Coupling temperature (Phe23) | CPP | 17.4 – 30.3 | °C |
| Coupling temperature (NMeAsn22, αMePhe15) | CPP | 17.3 – 28.3 | °C |
| Coupling temperature (Asn21) | CPP | 17.2 – 30.2 | °C |
| Post Asn21 capping DMF wash quantity | CPP | 3x NLT6.75 | kg/kg |
| IPAc wash quantity | CPP | 5x NLT 8.73 | Kg / kg |
| Toluene wash quantity | CPP | 2xNLT 8.72 | Kg / kg |
| MTBE wash quantity | CPP | 4x NLT 7.41 | Kg / kg |

11. References

| Title | Procedure Number |
| --- | --- |
| Eloralintide Fmoc-Fragment 2, LSN4182924, SPPS Risk Assessment and PAR Report | RPT-1194035 |
| MOR | MOR-TJ414002001.04 |
| Equipment Suitability Assessment | RA- 140020030101.01 |
| Cleaning Impact Assessment | CVP-1400200301.01 |
| Eloralintide Fragment L2 Intermediate Global Process Flow Document (PFD) LSN4182924 | WI-61120 (Lilly document number) |
| Process Performance Qualification Protocol for the Manufacturing Process of ASYM-141366 | PVP-1400200301.01 |
| Full scale Full scale Non-saleable Batch Summary Report for Manufacturing of ASYM-140020 | FSR-1400200301.01 |
| Risk Assessment Report for Cross-Contamination Control Strategy in LSN4182924,/ASYM-140020 production | RA-140020030103.01 |

Appendix A — Notes on Electrostatic Risk Control

For SPPS processes using flammable solvents (DMF, IPAc, Tol, MeOH and MTBE) with pneumatic transfer, electrostatic hazard control is a critical safety and process consideration. The CMO must have a documented electrostatic control programme in place. The following principles must be verified:

- All transfer pipelines are constructed from conductive material (stainless steel) and are electrically bonded and grounded

- Solvent flow velocities are maintained within permitted limits for the pipeline diameter, per site electrostatic control procedure (e.g. ≤ 1.0–1.5 m/s for insulating solvents, depending on pipe diameter — verify against site TWI)

- All vessels, drums, and hoses are electrically grounded before and during solvent transfer

- Personnel handling solvents wear antistatic clothing and footwear; grounding straps used where required

- Antistatic LDPE bags used for product packaging