Equipment Suitability Assessment for ASYM-141366 Manufacture (Step 1 of CPo140020-03) at Asymchem Life Science (Tianjin) Co., Ltd.

Document No. and Version No. RA-140020030101.01

| Prepared by |  |  |  |  |
| --- | --- | --- | --- | --- |
| Pengda Zhao (Project manager, CMMD) | Signature |  | Date |  |
| Reviewed by |  |  |  |  |
| Liang Li (Senior Director, CMMD) | Signature |  | Date |  |
| Zhongyao Zhang (Director, Production) | Signature |  | Date |  |
| Chengxue Zhu (Manager, QA) | Signature |  | Date |  |
| Approved by |  |  |  |  |
| Qi Wang (Senior Director, QA) | Signature |  | Date |  |

Table of Content

1.	Objective	3

2.	Scope	3

3.	Definition & Abbreviation	3

4.	Assessment Strategy	3

5.	Process Description	3

6.	Equipment List	15

7.	Suitability Assessment on Function & Capacity	17

8.	Compatibility Study Based on the Process Flow	36

9.	Suitability Assessment on Qualification Range and Accuracy of Instruments	76

10.	Other Considerations	79

11.	Conclusion	84

12.	Revision History	84

- Objective

This risk assessment is intended to verify the suitability for the equipment chosen for ASYM-141366 (step 1 of CPo140020-03) manufacture based on the designed process in Master Operation Record (MOR) of MOR-TJ414002001.04.

- Scope

The scope of this risk assessment covers the equipment involved in ASYM-141366 (step 1 of CPo140020-03) manufacture at Building 12 Module 13 at Asymchem Life Science (Tianjin) Co., Ltd. (referred to as Asymchem TJ4)

- Definition & Abbreviations

RT: Room temperature

DMF: N, N-Dimethylformamide

DIC: N, N'-Diisopropylcarbodiimide

IPAC: Isopropyl Acetate

DIPEA: N,N-Diisopropylethylamine

GL: Glass lined

SS: Stainless steel

316L: 316 Low carbon stainless steel

316SS: 316 Stainless steel

304SS: 304 Stainless steel

PTFE: Polytetrafluoroethylene

LDPE: Low-density polyethylene

NOR: Normal operation range

PAR: Proven acceptable range

MPa: Megapascals

HSS: High‑Speed Steel

PP: Polypropylene

C-276: Hastelloy

HC-22: Hastelloy C-22
PyBOP: Benzotriazole-1-yl-oxytripyrrolidinophosphonium hexafluorophosphate

- Assessment Strategy

A gap analysis method is applied for this risk assessment. The method is to compare the process requirement and the actual equipment characteristic to identify any gaps, followed by a justification for the suitability. ￼

- Process Description

Reaction scheme：

Scheme 1: Process synthetic route for ASYM-141366 (step 1 of CPo140020-03)

Process description：

| # |  | Process Description |  |  |
| --- | --- | --- | --- | --- |
| 1 | Dispense Material |  |  | Materials involved in dispensing: Amino acids, Oxyma, PyBop and 2-CTC Resin are dispensed using a glovebox (TJ4S-1200-GB01&TJ4S-1200-GB02&TJ4S-1200-GB03&TJ4S-1200-GB04&TJ4S-1200-GB05). The materials are dispensed into charging bags, which are then placed inside low-density polyethylene outer bags, with labels affixed. Subsequently, the bagged materials are placed into high-density polyethylene (HDPE) drums for temporary storage. Materials are as follows: 2-CTC Resin; Fmoc-Gly-OH.; Fmoc-L-Phe-OH; Fmoc-L-N-Me-Asn(Trt)-OH; Fmoc-L-Asn(Trt)-OH; Fmoc-L-Ser(tBu)-OH; Fmoc-L-Arg(Pbf)-OH; Fmoc-L-Val-OH; Fmoc-L-Leu-OH; Fmoc-L-α-Me-Phe-OH; Fmoc-L-Glu(OtBu)-OH·H2O; Fmoc-L-Ala-OH·H2O; Fmoc-L-Orn(Boc)-OH; Oxyma Pure; PyBop; |
| 2 | Prepare the solution |  |  | Prepare 20% piperidine in DMF (v/v%) solution: Add DMF to a Storage Tanks (TJ4S-1213-SPS01-AT01& TJ4S-1213-SPS01-AT02), then start the stirrer, and charge piperidine. Stir and store for later use |
| 3 | Pre-treatment |  |  | 1. Add DMF (200.00 kg - 300.00 kg) into activation tank (TJ4S-1213-Sy15-R01). Then transfer the solution into synthesis reactor (TJ4S-1213-Sy15). 2. Start the stirrer, bubble with nitrogen for 5±2 min, then stop the stirrer and nitrogen, drain. Sample the mixture for Water content analysis. Note: If Water content≤500ppm, proceed to Operation 4, if Water content>500 ppm, repeat Operation 3 until the moisture content is ≤500 ppm. |
| 4 | Fmoc-Gly-OH dissolve |  |  | 1. Add 4.25 V DMF (4.029 kg/kg) into the TJ4S-1213-Sy15-R01. Start the stirrer. 2. Then add 2.25 eq Fmoc-Gly-OH. Rinse reactor wall with 1.0 V DMF (0.948 kg/kg). Stir for at least 15 min. |
| 5 | Resin swelling |  |  | 1. Add 48.00 kg 2-CTC resin (1.0 eq.) into TJ4S-1213-Sy15. Rinse reactor wall with 7.0 V DMF (6.636 kg/kg). 2. Bubble with nitrogen for 5 min ± 2 min (target: 5min), then start the stirrer. Rinse reactor wall with 1.0 V DMF (0.948 kg/kg). 3. Adjust the mixture to 25±5 ℃ (target: 25 ℃). After stirring for 35±5min (target: 35 min) at 25±5 ℃ (target: 25 ℃), turn off the nitrogen, stop stirring, and drain until there is no obvious dripping. |
| 6 | Wash resin |  |  | 1. Add 5.0 V DMF (4.740 kg/kg) into synthesis reactor (TJ4S-1213-Sy15). 2. Bubble with nitrogen for 5±2 min (target: 5 min), then start the agitator. After stirring for 12±2 min (target: 12 min), stop stirring, turn off the nitrogen. Drain until there is no obvious drip. |
| 7 | Activate and transfer |  |  | 1. Adjust to 25±5 ℃ (target: 25 ℃), Add 6.0 eq. DIPEA into the TJ4S-1213-Sy15-R01 at 25±5℃ (target: 25℃). Reference addition time is 30±15 min. 2. Stir for 5±2 min after addition. Transfer the mixture in activation tank (TJ4S-1213-Sy15-R01) into TJ4S-1213-Sy15. Add 1.0 V DMF (0.948 kg/kg) into TJ4S-1213-Sy15-R01 to rinse the wall. 3. Bubble with nitrogen to the synthesis reactor (TJ4S-1213-Sy15) for 5±2 min, then start the agitator. 4. Transfer the mixture in activation tank (TJ4S-1213-Sy15-R01) into synthesis reactor (TJ4S-1213-Sy15) |
| 8 | Coupling |  |  | 1. Heat the mixture to 25±5 ℃. The mixture is allowed to react for 3 h±1 5min (target: 3 h) at 25±5 ℃ (target: 25 ℃). 2. Drain until there is no reaction mixture drip. Move to next operation. |
| 9 | Wash resin |  |  | 1. Add 8.0 V DMF (7.584 kg/kg) into TJ4S-1213-Sy15-R01. Bubble with nitrogen for 5±2 min, then start the stirrer. 2. After stirring for 5±2 min, stop stirring, turn off the nitrogen. Then Transfer the DMF into TJ4S-1213-Sy15. Bubble with nitrogen for 5±2 min, then start the stirrer. 3. After stirring for 5±2 min, stop stirring, turn off the nitrogen. Drain until there is no obvious drip. |
| 10 | Capping |  |  | 1. Add 8.0 V DMF (7.584 kg/kg) into TJ4S-1213-Sy15. Bubble with nitrogen for 5±2 min, then start the stirrer. 2. After stirring for 5±2 min, add 10.0 eq. Methanol. add 5.0 eq. DIPEA at a reference time 20±10 min at 15~25 ℃. 3. Adjust to 20~30 ℃，the mixture is allowed to react for 1 h±15 min (target: 1 h) at 20~30 ℃. Stop stirring and turn off the nitrogen. Drain until there is no reaction mixture drip. |
| 11 | Gly24 ~ Leu12, Wash resin |  |  | 1. Add 8.0 V DMF (7.584 kg/kg) into TJ4S-1213-Sy15-R01. Bubble with nitrogen for 5±2 min, then start the stirrer. 2. After stirring for 5±2 min, stop stirring, turn off the nitrogen. Then Transfer the mixture into TJ4S-1213-Sy15. Bubble with nitrogen for 5±2 min, then start the stirrer. 3. After stirring for 7±2 min, stop stirring, turn off the nitrogen. Drain until there is no obvious drip. Perform this operation 3 times. |
| 12 | prePhe23 ~ preOrn11 Excluding preSer20 De-Fmoc |  |  | 1. Add 8.0 V 20% Piperidine in DMF (7.448 kg/kg) into TJ4S-1213-Sy15. Bubble with nitrogen for 5±2 min, then start the stirrer. 2. Adjust to 25±5 ℃ (target: 25 ℃). After stirring for 2 h±15 min (target: 2 h) at 25±5 ℃ (target: 25 ℃), stop stirring, turn off the nitrogen. Drain until there is no obvious drip. |
| 13 | prePhe23 ~ preLeu12 Excluding preSer20, preOrn11 Post deFmoc washing |  |  | 1. Add 10.0 V Isopropyl Acetate (8.730 kg/kg) into TJ4S-1213-Sy15. Bubble with nitrogen for 5±2 min, then start the stirrer. 2. After stirring for 5±2 min, stop stirring, turn off the nitrogen. Drain until there is no obvious drip. Perform this operation 4 times. |
| 14 | prePhe23 ~ preOrn11 Re-swelling |  |  | 1. Add 12.0 V DMF (11.376 kg/kg) into TJ4S-1213-Sy15. Bubble with nitrogen for 5±2 min, then start the stirrer. 2. After stirring for 10±2 min, stop stirring, turn off the nitrogen. Drain until there is no obvious drip. Perform this operation 2 times. |
| 15 | Fmoc-L-Phe-OH activation and transfer |  |  | 1. Add 4.0 V DMF (3.792 kg/kg) into TJ4S-1213-Sy15-R01. Start the stirrer. Then add 2.0 eq. Fmoc-L-Phe-OH. Stir for at least 15 min. 2. Cool the mixture to 5±5 ℃. Maintain for next operation at 5±5 ℃ (target: 5 ℃). Add 2.0 eq. PyBOP and 2.0 eq. OxymaPure into TJ4S-1213-Sy15-R01 at 5±5 ℃ (target: 5 ℃), rinse the TJ4S-1213-Sy15-R01 with 1.0 V DMF (0.948 kg/kg). Add 3.0 eq. DIPEA into TJ4S-1213-Sy15-R01, the addition rate depends on temperature control. Activate for 20±5 min at 5±5 ℃ (target: 5 ℃). 3. Add 2.0 V DMF (1.896 kg/kg) into TJ4S-1213-Sy15, Start the agitator. 4. Transfer the mixture in TJ4S-1213-Sy15-R01 into TJ4S-1213-Sy15. Add 1.0 V DMF (0.948 kg/kg) into TJ4S-1213-Sy15-R01 to rinse the wall. Bubble with nitrogen to the synthesis reactor (TJ4S-1213-Sy15) for 5±2 min, Transfer the mixture in activation tank (TJ4S-1213-Sy15-R01) into synthesis reactor (TJ4S-1213-Sy15) |
| 16 | Phe23 Coupling |  |  | 1. Heat the mixture to 25±5 ℃. Start timing when temperature reaches 20 ℃. The mixture is allowed to react at 25±5 ℃ (target: 25 ℃). 2. After 4 h±15 min (target: 4 h), sample the mixture every 0.5~1 h for ninhydrin test analysis until two consecutive samples test as negative ( (Yellow to Light pink Report Negative= Complete coupling; Blue to Purple Positive= Incomplete coupling).). And the mixture should be stirred for coupling time (6 h ≤ Coupling time ≤ 14 h). 3. Stop stirring and turn off the nitrogen. Drain until there is no reaction mixture drip. Move to next operation. |
| 17 | Fmoc-L-N-Me-Asn(Trt)-OH activation and transfer |  |  | 1. Add 4 V DMF (3.792 kg/kg) into TJ4S-1213-Sy15-R01. Start the stirrer. Then add 1.75 eq. Fmoc-L-N-Me-Asn(Trt)-OH. Stir for at least 15 min. 2. Adjust the mixture to 20±5 ℃. Add 1.80 eq. Oxyma Pure into TJ4S-1213-Sy15-R01 at 20±5 ℃ (target: 20 ℃), rinse the TJ4S-1213-Sy15-R01 with 2.0 V DMF (1.896 kg/kg). Add 2.0 eq. DIC into TJ4S-1213-Sy15-R01, the addition rate depends on temperature control. Activate for 20±5 min at 20±5 ℃ (target: 20 ℃). 3. Transfer the mixture in TJ4S-1213-Sy15-R01 into TJ4S-1213-Sy15. Add 1.0 V DMF (0.948 kg/kg) into TJ4S-1213-Sy15-R01 to rinse the wall. Bubble with nitrogen to the synthesis reactor (TJ4S-1213-Sy15) for 5±2 min, then start the agitator. Transfer the mixture in activation tank (TJ4S-1213-Sy15-R01) into synthesis reactor (TJ4S-1213-Sy15) |
| 18 | NMeAsn22, aMePhe15 Coupling |  |  | 1. Heat the mixture to 25±5 ℃. The mixture is allowed to react at 25±5 ℃ (target: 25 ℃). 2. After 4 h±15 min (target：4 h), sample the mixture every 0.5~1 h for ninhydrin test analysis until two consecutive samples test as negative ( (Yellow to Light pink Report Negative= Complete coupling; Blue to Purple Positive= Incomplete coupling). 6h ≤ Coupling time ≤ 14 h). 3. Stop stirring and turn off the nitrogen. Drain until there is no reaction mixture drip. |
| 19 | Fmoc-L-Asn(Trt)-OH activation and transfer |  |  | 1. Add 4.0 V DMF (3.792 kg/kg) into TJ4S-1213-Sy15-R01. Start the stirrer. Then add 2.5 eq. Fmoc-L-Asn(Trt)-OH. Stir for at least 15 min. 2. Adjust the mixture to 20±5 ℃. Maintain for next operation at 20±5 ℃ (target: 20 ℃). Add 2.6 eq. Oxyma Pure into TJ4S-1213-Sy15-R01 at 20±5 ℃ (target: 20 ℃), rinse the TJ4S-1213-Sy15-R01 with 2.0 V DMF (1.896 kg/kg). Add 2.9 eq. DIC into TJ4S-1213-Sy15-R01, the addition rate depends on temperature control. Activate for 20±5 min at 20±5 ℃ (target: 20 ℃). 3. Transfer the mixture in TJ4S-1213-Sy15-R01 into TJ4S-1213-Sy15. Add 1.0 V DMF (0.948 kg/kg) into TJ4S-1213-Sy15-R01 to rinse the wall. Bubble with nitrogen to the synthesis reactor (TJ4S-1213-Sy15) for 5±2 min, then start the agitator. Transfer the mixture in activation tank (TJ4S-1213-Sy15-R01) into synthesis reactor (TJ4S-1213-Sy15) |
| 20 |  | Fmoc-L-Asn(Trt)-OH coupling | 1. Heat the mixture to 25±5 ℃. The mixture is allowed to react at 25±5 ℃ (target: 25 ℃). 2. Once the total reaction time has reached 8h±15min, add additional 1.0 eq. Oxyma Pure and 1.0 V DMF (0.948 kg/kg) solution (the mixture should be prepared in TJ4S-1200-AT02 just before use) and 1.1 eq. DIC. 3. After 5 h±15 min (target: 5 h). Stop stirring and turn off the nitrogen. Drain until there is no reaction mixture drip. |  |
| 22 |  | Prepare capping solution | 1. Add 6.0 V DMF (5.688 kg/kg) into TJ4S-1213-R22. Start the stirrer. 2. Then add 1.50 eq. Pyridine and 1.25 eq. Acetic Anhydride with PTFE and peristaltic pump. Adjust the mixture to 25±5 ℃. Maintain for next operation at 25±5℃ (target: 25 ℃). 3. Stir for at least 5 min after addition, Transfer the mixture in TJ4S-1213-R22 into TJ4S-1213-Sy15. Add 1.0 V DMF (0.948 kg/kg) into TJ4S-1213-R22 to rinse the wall. Bubble with nitrogen to the synthesis reactor (TJ4S-1213-Sy15) for 5±2 min, then start the agitator. Transfer the mixture in Glass-lined reactor (TJ4S-1213-R22) into synthesis reactor (TJ4S-1213-Sy15) |  |
| 23 | Capping |  |  | 1. Heat the mixture in synthesis reactor (TJ4S-1213-Sy15) to 25±5 ℃. 2. The mixture is allowed to react at 25±5 ℃ (target: 25 ℃) for 1 h±15 min (target: 1 h). 3. Stop stirring and turn off the nitrogen. Drain until there is no reaction mixture drip. Move to next operation. |
| 24 | Wash resin |  |  | 1. Add 8.0 V DMF (7.584 kg/kg) into TJ4S-1213-Sy15. Bubble with nitrogen for 5±2 min, then start the stirrer. After stirring for 7±2 min, stop stirring, turn off the nitrogen. Drain until there is no obvious drip. Perform this operation 3 times. |
| 25 | De-Fmoc |  |  | 1. Add 8.0 V 20% Piperidine in DMF (7.448 kg/kg) into the TJ4S-1213-Sy15. Bubble with nitrogen for 3±2 min, then start the stirrer, adjust to 25±5 ℃ (target: 25 ℃). 2. After stirring for 15 min at 25±5 ℃ (target: 25 ℃), stop stirring, turn off the nitrogen. Drain until there is no obvious drip. 3. Add 8.0 V 20% Piperidine in DMF (7.448 kg/kg) into TJ4S-1213-Sy15. Bubble with nitrogen for 3±2 min, then start the stirrer. 4. After stirring for 30±2 min at 25±5 ℃ (target: 25 ℃), stop stirring, turn off the nitrogen. Drain until there is no obvious drip. |
| 26 | Post deFmoc washing |  |  | 1. Add 10.0 V Isopropyl Acetate (8.730 kg/kg) into TJ4S-1213-Sy15. Bubble with nitrogen for 3±2 min, then start the stirrer. 2. After stirring for 5±2 min, stop stirring, turn off the nitrogen. Drain until there is no obvious drip. Perform this operation 4 times. |
| 27 | Ser20 ~ Ser21 Fmoc-L-Ser(tBu)-OH activation and transfer |  |  | 1. Add 5.0 V DMF (4.740 kg/kg) into TJ4S-1213-Sy15-R01. Start the stirrer. Then add 2.0 eq. Fmoc-L-Ser(tBu)-OH Stir for at least 15 min. 2. Adjust the mixture to 20±5 ℃. Add 2.0 eq OxymaPure into TJ4S-1213-Sy15-R01 at 20±5 ℃ (target: 20 ℃), rinse the TJ4S-1213-Sy15-R01 with 2.0 V DMF (1.896 kg/kg). Add 2.2 eq. DIC into TJ4S-1213-Sy15-R01, the addition rate depends on temperature control. Activate for 20±5 min at 20±5 ℃ (target: 20 ℃). 3. Transfer the mixture in TJ4S-1213-Sy15-R01 into TJ4S-1213-Sy15. Add 1.0 V DMF (0.948 kg/kg) into TJ4S-1213-Sy15-R01 to rinse the wall. Bubble with nitrogen to the synthesis reactor (TJ4S-1213-Sy15) for 5±2 min, then start the agitator. Transfer the mixture in activation tank (TJ4S-1213-Sy15-R01) into synthesis reactor (TJ4S-1213-Sy15) |
| 28 | Ser20 ~ Leu16, Ala13 ~ Orn11 Coupling |  |  | 1. Heat the mixture to 25±5 ℃. The mixture is allowed to react at 25±5 ℃ (target: 25 ℃). 2. After 3 h±15 min (target：3 h), sample the mixture every 0.5~1 h for ninhydrin test analysis until two consecutive samples test as negative ( (Yellow to Light pink Report Negative= Complete coupling; Blue to Purple Positive= Incomplete coupling).). And the mixture should be stirred for coupling time (4 h≤Coupling Time≤10 h). 3. Stop stirring and turn off the nitrogen. Drain until there is no reaction mixture drip. Move to next operation. |
| 29 | Fmoc-L-Arg(Pbf)-OH activation and transfer |  |  | 1. Add 5.0 V DMF (4.740 kg/kg) into TJ4S-1213-Sy15-R01. Start the stirrer. Then add 2.0 eq Fmoc-L-Arg(Pbf)-OH. Stir for at least 15 min. 2. Adjust the mixture to 20±5 ℃. Maintain for next operation at 20±5 ℃ (target: 20 ℃). Add 2.0 eq. Oxyma Pure into TJ4S-1213-Sy15-R01 at 20±5 ℃ (target: 20 ℃), rinse the TJ4S-1213-Sy15-R01 with 2.0 V DMF (1.896 kg/kg). Add 2.2 eq. DIC into TJ4S-1213-Sy15-R01, the addition rate depends on temperature control. Activate for 20±5 min at 20±5 ℃ (target: 20 ℃). 3. Transfer the mixture in TJ4S-1213-Sy15-R01 into TJ4S-1213-Sy15. Add 1.0 V DMF (0.948 kg/kg) into TJ4S-1213-Sy15-R01 to rinse the wall. Bubble with nitrogen to the synthesis reactor (TJ4S-1213-Sy15) for 5±2 min, then start the agitator. Transfer the mixture in activation tank (TJ4S-1213-Sy15-R01) into synthesis reactor (TJ4S-1213-Sy15) |
| 30 | Fmoc-L-Val-OH activation and transfer |  |  | 1. Add 5.0 V DMF (4.740 kg/kg) into TJ4S-1213-Sy15-R01. Start the stirrer. Then add 2.0 eq. Fmoc-L-Val-OH. Stir for at least 15 min. 2. Adjust the mixture to 20±5 ℃. Maintain for next operation at 20±5 ℃ (target: 20 ℃). Add 2.0 eq. Oxyma Pure into TJ4S-1213-Sy15-R01 at 20±5 ℃ (target: 20 ℃), rinse the TJ4S-1213-Sy15-R01 with 2.0 V DMF (1.896 kg/kg). Add 2.2 eq. DIC into TJ4S-1213-Sy15-R01, the addition rate depends on temperature control. Activate for 20±5 min at 20±5 ℃ (target: 20 ℃). 3. Transfer the mixture in TJ4S-1213-Sy15-R01 into TJ4S-1213-Sy15. Add 1.0 V DMF (0.948 kg/kg) into TJ4S-1213-Sy15-R01 to rinse the wall. Bubble with nitrogen to the synthesis reactor (TJ4S-1213-Sy15) for 5±2 min, then start the agitator. Transfer the mixture in activation tank (TJ4S-1213-Sy15-R01) into synthesis reactor (TJ4S-1213-Sy15) |
| 31 | Fmoc-L-Leu-OH activation and transfer |  |  | 1. Add 5.0 V DMF (4.740 kg/kg) into TJ4S-1213-Sy15-R01. Start the stirrer. Then add 2.0 eq. Fmoc-L-Leu-OH. Stir for at least 15 min. 2. Adjust the mixture to 20±5 ℃. Maintain for next operation at 20±5 ℃ (target: 20 ℃). Add 2.0 eq. OxymaPure into TJ4S-1213-Sy15-R01 at 20±5 ℃ (target: 20 ℃), rinse the TJ4S-1213-Sy15-R01 with 2.0 V DMF (1.896 kg/kg). Add 2.2 eq. DIC into TJ4S-1213-Sy15-R01, the addition rate depends on temperature control. Activate for 20±5 min at 20±5 ℃ (target: 20 ℃). 3. Transfer the mixture in TJ4S-1213-Sy15-R01 into TJ4S-1213-Sy15. Add 1.0 V DMF (0.948 kg/kg) into TJ4S-1213-Sy15-R01 to rinse the wall. Bubble with nitrogen to the synthesis reactor (TJ4S-1213-Sy15) for 5±2 min, then start the agitator. Transfer the mixture in activation tank (TJ4S-1213-Sy15-R01) into synthesis reactor (TJ4S-1213-Sy15) |
| 32 | Fmoc-L-α-Me-Phe-OH activation and transfer |  |  | 1. Add 4.0 V DMF (3.792 kg/kg) into TJ4S-1213-Sy15-R01. Start the stirrer. Then add 1.75 eq. Fmoc-L-α-Me-Phe-OH. Stir for at least 15 min. 2. Adjust the mixture to 20±5 ℃. Maintain for next operation at 20±5 ℃ (target: 20 ℃). Add 1.80 eq. OxymaPure into TJ4S-1213-Sy15-R01 at 20±5 ℃ (target: 20 ℃), rinse the TJ4S-1213-Sy15-R01 with 2.0 V DMF (1.896 kg/kg). Add 2.0 eq. DIC into TJ4S-1213-Sy15-R01, the addition rate depends on temperature control. Activate for 20±5 min at 20±5 ℃ (target: 20 ℃). 3. Transfer the mixture in TJ4S-1213-Sy15-R01 into TJ4S-1213-Sy15. Add 1.0 V DMF (0.948 kg/kg) into TJ4S-1213-Sy15-R01 to rinse the wall. Bubble with nitrogen to the synthesis reactor (TJ4S-1213-Sy15) for 5±2 min, then start the agitator. Transfer the mixture in activation tank (TJ4S-1213-Sy15-R01) into synthesis reactor (TJ4S-1213-Sy15) |
| 33 | Fmoc-L-α-Me-Phe-OH coupling |  |  | 1. Heat the mixture to 25±5 ℃. Start timing when temperature reaches 20 ℃. The mixture is allowed to react at 25±5 ℃ (target: 25 ℃). 2. After 4 h±15 min (target：4 h), sample the mixture every 0.5~1 h for ninhydrin test analysis until two consecutive samples test as negative ( (Yellow to Light pink Report Negative= Complete coupling; Blue to Purple Positive= Incomplete coupling).) and the mixture should be stirred for coupling time (6 h≤Coupling Time≤14 h). 3. Stop stirring and turn off the nitrogen. Drain until there is no reaction mixture drip. Move to next operation. |
| 34 | Fmoc-L-Glu(OtBu)-OH·H2O activation and transfer |  |  | 1. Add 5.0 V DMF (4.740 kg/kg) into TJ4S-1213-Sy15-R01. Start the stirrer. Then add 2.0 eq. Fmoc-L-Glu(OtBu)-OH·H2O. Stir for at least 15 min. 2. Adjust the mixture to 20±5 ℃. Maintain for next operation at 20±5 ℃ (target: 20 ℃). Add 2.0 eq. Oxyma Pure into TJ4S-1213-Sy15-R01 at 20±5 ℃ (target: 20 ℃), rinse the TJ4S-1213-Sy15-R01 with 2.0 V DMF (1.896 kg/kg). Add 2.2 eq. DIC into TJ4S-1213-Sy15-R01, the addition rate depends on temperature control. Activate for 20±5 min at 20±5 ℃ (target: 20 ℃). 3. Transfer the mixture in TJ4S-1213-Sy15-R01 into TJ4S-1213-Sy15. Add 1.0 V DMF (0.948 kg/kg) into TJ4S-1213-Sy15-R01 to rinse the wall. Bubble with nitrogen to the synthesis reactor (TJ4S-1213-Sy15) for 5±2 min, then start the agitator. Transfer the mixture in activation tank (TJ4S-1213-Sy15-R01) into synthesis reactor (TJ4S-1213-Sy15). |
| 35 | Fmoc-L-Glu(OtBu)-OH·H2O coupling |  |  | 1. Heat the mixture to 25±5 ℃. Start timing when temperature reaches 20 ℃. The mixture is allowed to react at 25±5 ℃ (target: 25 ℃). 2. After 6 h±15 min (target：6 h), sample the mixture every 0.5~1 h for bromophenol blue analysis until two consecutive samples test as negative (Yellow to Orange Report Negative = Complete coupling; Blue to Black Positive = Incomplete coupling) and the mixture should be stirred for coupling time (8 h ≤ Coupling Time ≤ 18 h). 3. Stop stirring and turn off the nitrogen. Drain until there is no reaction mixture drip. |
| 36 | Fmoc-L-Ala-OH·H2O activation and transfer |  |  | 1. Add 5.0 V DMF (4.740 kg/kg) into TJ4S-1213-Sy15-R01. Start the stirrer. Then add 2.0 eq. Fmoc-L-Ala-OH·H2O. Stir for at least 15 min. 2. Adjust the mixture to 20±5 ℃. Maintain for next operation at 20±5 ℃ (target: 20 ℃). Add 2.0 eq. Oxyma Pure into TJ4S-1213-Sy15-R01 at 20±5 ℃ (target: 20 ℃), rinse the TJ4S-1213-Sy15-R01 with 2.0 V DMF (1.896 kg/kg). Add 2.2 eq. DIC into TJ4S-1213-Sy15-R01, the addition rate depends on temperature control. Activate for 20±5 min at 20±5 ℃ (target: 20 ℃). 3. Transfer the mixture in TJ4S-1213-Sy15-R01 into TJ4S-1213-Sy15. Add 1.0 V DMF (0.948 kg/kg) into TJ4S-1213-Sy15-R01 to rinse the wall. Bubble with nitrogen to the synthesis reactor (TJ4S-1213-Sy15) for 5±2 min, then start the agitator. Transfer the mixture in activation tank (TJ4S-1213-Sy15-R01) into synthesis reactor (TJ4S-1213-Sy15) |
| 37 | Fmoc-L-Leu-OH activation and transfer |  |  | 1. Add 5.0 V DMF (4.740 kg/kg) into TJ4S-1213-Sy15-R01. Start the stirrer. Then add 2.0 eq. Fmoc-L-Leu-OH. Stir for at least 15 min. 2. Adjust the mixture to 20±5 ℃. Add 2.0 eq. Oxyma Pure into TJ4S-1213-Sy15-R01 at 20±5 ℃ (target: 20 ℃), rinse the TJ4S-1213-Sy15-R01 with 2.0 V DMF (1.896 kg/kg). Add 2.2 eq. DIC into TJ4S-1213-Sy15-R01, the addition rate depends on temperature control. Activate for 20±5 min at 20±5 ℃ (target: 20 ℃). 3. Transfer the mixture in TJ4S-1213-Sy15-R01 into TJ4S-1213-Sy15. Add 1.0 V DMF (0.948 kg/kg) into TJ4S-1213-Sy15-R01 to rinse the wall. Bubble with nitrogen to the synthesis reactor (TJ4S-1213-Sy15) for 5±2 min, then start the agitator. Transfer the mixture in activation tank (TJ4S-1213-Sy15-R01) into synthesis reactor (TJ4S-1213-Sy15) |
| 38 | Wash resin |  |  | 1. Add 10.0 V Isopropyl Acetate (8.730 kg/kg) into TJ4S-1213-Sy15. Bubble with nitrogen for 5±2 min, then start the stirrer. 2. After stirring for 5±2min, stop stirring, turn off the nitrogen. Drain until there is no obvious drip. 3. Perform this operation 10 times, Starting with the 10th wash, sample the isopropyl acetate washing solution during stirring and develop with chloranil, compare the color with the standard 500 ppm Piperidine in Isopropyl Acetate as reference solution. |
| 39 | Fmoc-L- Orn(Boc)-OH activation and transfer |  |  | 1. Add 5.0 V DMF (4.740 kg/kg) into TJ4S-1213-Sy15-R01. Start the stirrer. Then add 2.0 eq. Fmoc-L-Orn(Boc)-OH (0.909 kg/mol). Stir for at least 15 min. 2. Adjust the mixture to 20±5 ℃. Maintain for next operation at 20±5 ℃ (target: 20 ℃). Add 2.0 eq. Oxyma Pure into TJ4S-1213-Sy15-R01 at 20±5 ℃ (target: 20 ℃), rinse the TJ4S-1213-Sy15-R01 with 2.0 V DMF (1.896 kg/kg). Add 2.2 eq. DIC into TJ4S-1213-Sy15-R01, the addition rate depends on temperature control. Activate for 20±5 min at 20±5 ℃ (target: 20 ℃). 3. Transfer the mixture in TJ4S-1213-Sy15-R01 into TJ4S-1213-Sy15. Add 1.0 V DMF (0.948 kg/kg) into TJ4S-1213-Sy15-R01 to rinse the wall. Bubble with nitrogen to the synthesis reactor (TJ4S-1213-Sy15) for 5±2 min, then start the agitator. Transfer the mixture in activation tank (TJ4S-1213-Sy15-R01) into synthesis reactor (TJ4S-1213-Sy15) |
| 40 | Wash resin |  |  | 1. Add 10.0 v IPAC (8.730 kg/kg) into TJ4S-1213-Sy15. Bubble with nitrogen for 5±2 min, then start the stirrer. 2. After stirring for 10±2 min, stop stirring, turn off the nitrogen. 3. Drain until there is no obvious drip. Perform this operation 5 times. |
| 41 | Wash resin |  |  | 1. Add 10.0 V Toluene (8.720 kg/kg) into TJ4S-1213-Sy15 by insert tube and diaphragm pump. Bubble with nitrogen for 5±2 min, then start the stirrer. After stirring for 25±5 min, stop stirring, turn off the nitrogen. Drain until there is no obvious drip. 2. Add 10.0 V MTBE (7.410 kg/kg) into TJ4S-1213-Sy15. Bubble with nitrogen for 5±2 min, then start the stirrer. After stirring for 15±5 min, stop stirring, turn off the nitrogen. Drain until there is no obvious drip. 3. Add 10.0 V MTBE (7.410 kg/kg) into TJ4S-1213-Sy15. Bubble with nitrogen for 5±2 min, then start the stirrer. After stirring for 15±5 min, stop stirring, turn off the nitrogen. Drain until there is no obvious drip. |
| 42 | Wash resin |  |  | 1. Add 10.0 V Toluene (8.720 kg/kg) into TJ4S-1213-Sy15 by insert tube and diaphragm pump. Bubble with nitrogen for 5±2 min, then start the stirrer. After stirring for 25±5 min, stop stirring, turn off the nitrogen. Drain until there is no obvious drip. 2. Add 10.0 V MTBE (7.410 kg/kg) into TJ4S-1213-Sy15. Bubble with nitrogen for 5±2 min, then start the stirrer. After stirring for 15±5 min, stop stirring, turn off the nitrogen. Drain until there is no obvious drip. 3. Add 10.0 V MTBE (7.410 kg/kg) into TJ4S-1213-Sy15. Bubble with nitrogen for 5±2 min, then start the stirrer. After stirring for 15±5 min, stop stirring, turn off the nitrogen. Drain until there is no obvious drip. 4. Sample the MTBE washing solution during the second stirring and develop with chloranil, compare the color with the standard 500 ppm piperidine in MTBE solution as reference solution. |
| 43 | Drying |  |  | 1. Sweep with nitrogen above, nitrogen flow (11~15 m3/h target: 14 m3/h) and vacuum underneath at T≤30 ℃ by jacket. 2. After drying for 16 h±2 h, sample the solid for LOD analysis every 6±4 h until LOD≤10.0%. Note: After drying 6 h, begin to stir the solid for 2~5 min at a speed of 7±2 rpm (reference) every 4±1 h. |
| 44 | Collect |  |  | Put intermediate into double LDPE bags, number and weigh, inner layer of pharmaceutical grade LDPE powder feeding bags, and outer pharmaceutical grade LDPE bag, the desiccant between the two layers, purge the inside of the package with nitrogen, seal the in-process Intermediate in HDPE drums for temporary storage. |

Figure 1: Equipment Flow Chart for ASYM-141366 Manufacture (Step 1 of CPo140020-03) ：

- Equipment List

The equipment to be used are summarized in Table 6-1 and indicated in Figure 1 below.

Table 6-1: Equipment List for ASYM-141366 (step 1 of CPo140020-03)

| Equipment Name | Equipment Code | Intended Use | Equipment Size | MOC |
| --- | --- | --- | --- | --- |
| Glovebox | TJ4S-1200-GB01 TJ4S-1200-GB02 TJ4S-1200-GB03 TJ4S-1200-GB05 | Dispense amino acid, 2-CTC resin and Oxyma | 50 L | Inner surface, rigid connection piece: 316L Gasket: PTFE Sight glass:Glass Dispensing scoop: 304 SS Filter cartridge: PP Gloves: CSPE |
| Glovebox | TJ4S-1200-GB04 | Dispense amino acid, 2-CTC resin and Oxyma | 50 L | Inner surface, rigid connection piece: Halar Gasket: PTFE Sight glass:Glass Dispensing scoop: 304 SS Filter cartridge: PP Gloves: CSPE |
| Synthesis Reactor | TJ4S-1213-Sy15 | Solid-phase synthesis& Dry | 3000 L | Reaction vessel material: Hastelloy (C-276) Agitator material: Hastelloy (N10276) Sieve plate: HC-22 Nitrogen distribution piping: Hastelloy (C-276) Sight glass: Glass Transfer pump material: SS316+PTFE, Thermometer well material: Hastelloy (C-276) Pipeline material: 316L Sealing ring: PTFE Mass flow meter: SS Valve material: 304 & 316L Gasket Material: PTFE Waste discharge pump material: SS316+PTFE Mechanical seal: 304SS & PTFE & FFKM Gloves: CSPE |
| Activation Tank | TJ4S-1213-Sy15-R01 | Dissolve and activate | 1500 L | Tank material: Hastelloy (C-276) Agitator material: Hastelloy (C-276) Sight glass: Glass Thermometer well material: Hastelloy (C-276) Pipeline material: 316L Sealing ring: PTFE Mass flow meter: SS Valve material: 304 & 316L Waste discharge pump material: 316SS Peristaltic pump tubing: PTFE Mechanical seal: 304SS & PTFE & FFKM Gloves: CSPE |
| Storage Tanks | TJ4S-1213-SPS01-AT01 | Storage of Prepared De-Fmoc Solution | 3000 L | Inner surface, temperature probe, agitator, pipeline, transfer pump, mass flowmeter, valve: 304 SS Mechanical seal: 304SS & PTFE & FFKM Sight glass: Glass Valve, gasket, including manway gaskets and gaskets on all inlet valve manifolds: PTFE Mechanical seal: 304SS & PTFE & FFKM |
|  | TJ4S-1213-SPS01-AT02 | Storage of Prepared De-Fmoc Solution | 3000 L | Inner surface, temperature probe, agitator, pipeline, transfer pump, mass flowmeter, valve: 304 SS Mechanical seal: 304SS & PTFE & FFKM Sight glass: Glass Valve, gasket, including manway gaskets and gaskets on all inlet valve manifolds: PTFE Mechanical seal: 304SS & PTFE & FFKM |
| Glass-line Reactor | TJ4S-1213-R22 | Prepared Ac2O / Pyridine / DMF solution | 1000 L | Inner surface, valve, temperature probe, agitator: Glass-lined Mechanical seal: GL & PTFE & FFKM |
| Storage tank | TJ4S-1200-AT02 | Prepared OxymaPure/ DMF solution | 500 L | Materials of body: Hastelloy (C-276) Material of agitator: Hastelloy (C-276) |

- Suitability Assessment on Function & Capacity

The suitability assessment is based on the benchmark material CTC resin with a feeding amount of 48.00 kg (51.84 mol) input.

Assessment on function & capacity for Glove box

See Table 7-1

Table 7-1: Equipment Suitability Assessment on Function for Glove box

| S/N | Process Requirements |  | Equipment characteristics | Feasibility |  | Justifications |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Operations | Material dispensing, Closed operation is available. | Glovebox can meet dispensing operation requirement. The equipment is equipped with a dedicated transition chamber. Materials are transferred into the glove box in an isolated manner through the transition chamber. The entire dispensing process is fully closed and controlled to effectively prevent the materials from contacting ambient air and contaminants. | ☒Yes | ☐ No | This glove box enables fully enclosed and isolated operations throughout the entire dispensing process. The system continuously purges nitrogen to maintain an inert protective atmosphere, effectively isolating ambient air, moisture and extraneous foreign matter. The operational status of the equipment complies with process control requirements and GMP compliance standards. It state the qualified OBE level Ⅲ. |
| 2 | Process Design | Nitrogen protection/Inert: oxygen content ≤1.0%. | The equipment has the function of protection under nitrogen flow Control range of oxygen content of the equipment: ≤1.0%. | ☒Yes | ☐ No | The oxygen content control range of the equipment meets the process requirements. |

Assessment on function & capacity for Agitator tank TJ4S-1213-SPS01-AT01

See Table 7-2

Table 7-2: Equipment Suitability Assessment on Function & Capacity for Agitator tank TJ4S-1213-SPS01-AT01

| S/N | Process Requirements |  | Equipment Characteristics | Feasibility |  | Justifications |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Operations | Stirring, solvent addition (bulk), internal temperature control, transfer to downstream reactor | Agitator tank has the functions of Stirring, solvent addition (bulk), internal temperature control, and transfer to downstream reactor | ☒Yes | ☐ No | N/A |
| 2 | Stirring | N/A | Agitator type: double-layer propeller type The range of agitation speed of equipment capability: 20~120 rpm | ☒Yes | ☐ No | N/A |
| 3 | Stirring volume | Min batch volume: 500 L Max batch volume: 2300 L | Minimum volume of the stirrer can reach at rest: 308.7L; Minimum volume of the stirrer can reach at maximum speed: 289.2 L. Maximum working volume: 3000 L | ☒Yes | ☐ No | The solution preparation process is controlled by the DCS system. When the liquid volume in the tank drops below 5% (Approx. 300 L), operators initiate preparation via the DCS system. The total batch volume for each piperidine/DMF solution preparation ranges: 500~2000 L (The system automatically calculates the volumes of DMF and piperidine based on the target concentration of 20%, and computes the feeding mass accordingly.), and interlocks are installed between flowmeters and transfer valves. The valves automatically close to stop feeding once the set target value is reached. The set solution preparation limit is greater than the minimum stirrable volume of the equipment and less than the equipment volume of 3000 L. The equipment meets the process requirements |
| 4 | Temperature measurement | No requirements of temperature from process | Minimum volume of the thermometer tube can reach at rest: 416.9 L; Minimum volume of the thermometer tube can reach at maximum speed: 356.8L. | ☒Yes | ☐ No | Temperature regulated via heat exchanger. At a solvent flow rate of 12 m³/h, the solvent passes through the heat exchanger, with its temperature rising from -15 °C to 20 °C. At a solvent flow rate of 20 m³/h, the solvent flows through the heat exchanger, and its temperature drops from 45 °C to 33 °C. |
| 5 | Solution charging | DMF charging amount (DMF density: 0.948 kg/L): 400~1600 L ; (379.20~1516.80 kg) | Charging accuracy of DMF is within ±1.5% when the charging amount is more than 400 L. | ☒Yes | ☐ No | Based on charging accuracy calculation, the concentration of Piperidine/DMF mixed solution is controlled within 19.5%~20.5%. The NOR range is 15%~20%, which complies with process requirements. |
|  |  | Piperidine charging amount (Piperidine density: 0.862 kg/L): 100~400 L; (86.20~344.8 kg) | Charging accuracy of Piperidine is within ±1.5% when the charging amount is more than 100 L. | ☒Yes | ☐ No |  |
| 6 | Process Design | Nitrogen protection/Inert: oxygen content ≤1.0%. | The equipment has the function of protection under nitrogen flow or static pressure Control range of oxygen content of the equipment: ≤1.0% | ☒Yes | ☐ No | The oxygen content control range of the equipment meets the process requirements. |
|  |  | Liquid charge | Dedicated pipeline for charging | ☒Yes | ☐ No | N/A |

Note: Refers to equipment qualification document SVP-TJ4-392.01-002

Assessment on function & capacity for Agitator tank TJ4S-1213-SPS01-AT02

See Table 7-3

Table 7-3: Equipment Suitability Assessment on Function & Capacity for Agitator tank TJ4S-1213-SPS01-AT02

| S/N | Process Requirements |  | Equipment Characteristics | Feasibility |  | Justifications |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Operations | Stirring, solvent addition (bulk), internal temperature control, transfer to downstream reactor | Agitator tank has the functions of Stirring, solvent addition (bulk), internal temperature control, and transfer to downstream reactor | ☒Yes | ☐ No | N/A |
| 2 | Stirring | N/A | Agitator type: double-layer propeller type The range of agitation speed of equipment capability: 20~120 rpm | ☒Yes | ☐ No | N/A |
| 3 | Stirring volume | Min batch volume: 500 L Max batch volume: 2300 L | Minimum volume of the stirrer can reach at rest: 309.3 L; Minimum volume of the stirrer can reach at maximum speed: 317.1 L. Maximum working volume: 3000 L | ☒Yes | ☐ No | The solution preparation process is controlled by the DCS system. When the liquid volume in the tank drops below 5% (Approx. 300 L), operators initiate preparation via the DCS system. The total batch volume for each piperidine/DMF solution preparation ranges: 500~2000 L (The system automatically calculates the volumes of DMF and piperidine based on the target concentration of 20%, and computes the feeding mass accordingly.), and interlocks are installed between flowmeters and transfer valves. The valves automatically close to stop feeding once the set target value is reached. The set solution preparation limit is greater than the minimum stirrable volume of the equipment and less than the equipment volume of 3000 L. The equipment meets the process requirements |
| 4 | Temperature measurement | No requirements of temperature from process | Minimum volume of the thermometer tube can reach at rest: 376.1 L; Minimum volume of the thermometer tube can reach at maximum speed: 349.7 L | ☒Yes | ☐ No | Temperature regulated via heat exchanger. At a solvent flow rate of 12 m³/h, the solvent passes through the heat exchanger, with its temperature rising from -15 °C to 20 °C. At a solvent flow rate of 20 m³/h, the solvent flows through the heat exchanger, and its temperature drops from 45 °C to 33 °C. |
| 5 | Solution charging | DMF charging amount (DMF density: 0.948 kg/L): 400~1600 L; (379.20~1516.80 kg) | Charging accuracy of DMF is within ±1.5% when the charging amount is more than 400 L. | ☒Yes | ☐ No | Based on charging accuracy calculation, the concentration of Piperidine/DMF mixed solution is controlled within 19.5%~20.5%. The NOR range is 15%~20%, which complies with process requirements. |
|  |  | Piperidine charging amount (Piperidine density: 0.862 kg/L): 100~400 L; (86.20~344.8 kg) | Charging accuracy of Piperidine is within ±1.5% when the charging amount is more than 100 L. | ☒Yes | ☐ No |  |
| 6 | Process Design | Nitrogen protection/Inert: oxygen content ≤1.0%. | The equipment has the function of protection under nitrogen flow or static pressure Control range of oxygen content of the equipment: ≤1.0% | ☒Yes | ☐ No | The oxygen content control range of the equipment meets the process requirements. |
|  |  | Liquid charge | Dedicated pipeline for charging | ☒Yes | ☐ No | N/A |

Note: Refers to equipment qualification document SVP-TJ4-392.01-002

Assessment on function & capacity for 3000L Synthesizer Reactor TJ4S-1213-Sy15

See Table7-4

Table 7-4: Equipment Suitability Assessment on Function & Capacity for 3000L Synthesizer Reactor TJ4S-1213-Sy15

| S/N | Process Requirements |  | Equipment Characteristics | Feasibility |  | Justifications |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Operations | Cooling, heating, stirring, filtration, material discharging, fluidization with nitrogen bubbling, solid and liquid charges, drying and sampling. | Synthesis Reactor TJ4S-1213-Sy15 has the functions of cooling, heating, stirring, filtration, material discharging, and fluidization with nitrogen bubbling, solid and liquid charges, drying, and sampling. | ☒Yes | ☐ No | N/A |
|  |  | Rinse by DMF | Synthesis Reactor has rinsing function: spray balls. | ☒Yes | ☐ No | N/A |
| 2 | Stirring volume | Min batch volume 96 L Max batch volume 576 L | Minimum volume of the stirrer can reach (at rest): 51.54 L; Minimum volume of the stirrer can reach (at maximum speed) 51.84 L; Maximum working volume: 3000 L | ☒Yes | ☐ No | The min/max batch volume are within the stirring volume range of equipment capability |
| 3 | Temperature measurement | The min batch volume 96 L | Minimum volume of the inner temperature probe (at rest): 26.91 L Minimum volume of the temperature probe (at maximum speed): 33.03 L | ☒Yes | ☐ No | The min batch volume is within the min temperature sensing capability; good temperature monitoring can be expected |
| 4 | Stirring speed | Reference stirring speed 20 ± 10 rpm | Agitator type: Upper layer: 4-Blade Hydrofoil Pump Up; Lower layer: 2-Curved Blade Turbine The range of agitation speed of equipment capability:5~40 rpm | ☒Yes | ☐ No | The stirring speed range required by the process is within the stirring speed range of the equipment. |
| 5 | Solution Feeding | DMF minimum charging amount: 91.01 kg, add accuracy required: ±3%; DMF maximum charging amount: 546.05 kg, required charging accuracy: ±3% | When the DMF add volume of the equipment is ≥70 kg, the feeding accuracy is within ≤3% When the DMF add volume of the equipment is 5kg~70 kg, the feeding accuracy is ≤2kg | ☒Yes | ☐ No | The charging accuracy required by the process is within the control range of the equipment. Temperature regulated via heat exchanger. At a solvent flow rate of 12 m³/h, the solvent passes through the heat exchanger, with its temperature rising from -15 °C to 20 °C. At a solvent flow rate of 20 m³/h, the solvent flows through the heat exchanger, and its temperature drops from 45 °C to 33 °C. |
|  |  | MTBE add amount range: 345.01 kg ~ 366.35 kg, add accuracy required: ±3% | When the MTBE add volume of the equipment is ≥70 kg, the feeding accuracy is within ≤3% | ☒Yes | ☐ No |  |
|  |  | IPAC add amount range: 406.47 kg ~ 431.61 kg, add accuracy required: ±3% | When the IPAc add volume of the equipment is ≥70 kg, the feeding accuracy is within ≤3% | ☒Yes | ☐ No |  |
|  |  | PIP/DMF add amount range: 346.78 kg ~ 368.23 kg, add accuracy required: ±3% | When the PIP/DMF add volume of the equipment is ≥70 kg, the feeding accuracy is within ≤3% | ☒Yes | ☐ No |  |
|  |  | Toluene addition amount range: 406.47 kg ~ 431.61 kg, add accuracy required: ±3%; The charging range each drum is between 2.00~ 170.00 kg. | 2.00 kg≤m≤300.00 kg: ±3.0% | ☒Yes | ☐ No |  |
| 6 | Process Design | Control range of batch jacket temperature from the process requirement 15~30 °C | Control range of temperature of the equipment capability: -14~57 ℃ | ☒Yes | ☐ No | The control range of batch temperature from process requirement is within the control range of equipment capability |
|  |  | Control range of batch temperature from the process requirement 15~30 °C | Control range of temperature of the equipment capability: -4~53 ℃ | ☒Yes | ☐ No | The range of agitation speed from process requirements is within the range of agitation speed of equipment capability |
|  |  | Bubble with nitrogen flow requirement: 5 m3/h±2 m3/h (target: 5 m3/h) | Control range of nitrogen flow of the equipment capability: 1.5~15 m3/h Control nitrogen flow rate via linkage of flow meter and regulating valve. |  |  | The control range of batch temperature from process requirement is within the control range of equipment capability |
|  |  | Nitrogen protection / inertization / nitrogen sparging flow rate: 3~15 m³/h | The equipment is equipped with nitrogen blanketing/inertization and nitrogen sparging functions. The range of nitrogen flow rate in the workshop is 1.5-15 m³/h | ☒Yes | ☐ No |  |
|  |  | No requirements of pressure from process. | The pressure range of equipment for normal operation: -0.08~0.0807 MPa Control nitrogen flow rate via linkage of flow meter and regulating valve. | ☒Yes | ☐ No | N/A |
|  |  | Add Liquid | The equipment is equipped with three Dedicated pipeline feedings and two Liquid inlets | ☒Yes | ☐ No | N/A |
|  |  | Solid charge | Charge via Solid charge equipment: Powder charging bag | ☒Yes | ☐ No | N/A |
|  |  | Solution Transfer | The equipment is equipped with solution transfer lines and feeding ports | ☒Yes | ☐ No | N/A |
|  |  | Sampling for IPC testing (mixture) | The equipment is equipped with a mixture sampling device | ☒Yes | ☐ No | Sample with 316L or PTFE liquid sampling tube |
|  |  | Sampling for dry solid | The equipment is equipped with a solid sampling port | ☒Yes | ☐ No | N/A |
|  |  | Filtration until no liquid drips | The equipment is equipped with a sieve plate and has filtration function. A tuning fork level meter is installed to assist in determining the filtration end point. Sieve plate pore size : 50 μm Adopt high-strength composite sintered plate with thickness of 4.7 mm. Perforated plate thickness: 8mm. | ☒Yes | ☐ No | N/A |
|  |  | Nitrogen protection/Inert: oxygen content ≤1.0%. | The equipment has the function of protection under nitrogen flow or static pressure Control range of oxygen content of the equipment: ≤1.0% | ☒Yes | ☐ No | N/A |
|  |  | Filter until no liquid drips | The equipment is equipped with a sieve plate for filtration and a tuning fork level meter, which can be used to determine the filtration end point | ☒Yes | ☐ No | N/A |

Note: Refers to equipment qualification document VR-TJ4-334.01-065.02

Assessment on function & capacity for 1500 L Activation Tank TJ4S-1213-Sy15-R01

See Table 7-5

Table 7-5: Equipment Suitability Assessment on Function & Capacity for 1500 L Activation Tank TJ4S-1213-Sy15-R01

| S/N | Process Requirements |  | Equipment characteristics | Feasibility |  | Justifications |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Operations | Cooling, heating, stirring, and Solution Transfer | 1500 L Activation Tank TJ4S-1213-Sy15-R01 is equipped with temperature rising/falling, stirring, and material transfer | ☒Yes | ☐ No | N/A |
|  |  | Rinse by DMF | Activation tank has rinsing function: spray balls. | ☒Yes | ☐ No | N/A |
|  |  | Discharge from the TJ4S-1213-Sy15-R01 to the TJ4S-1213-Sy15 | Dedicated transfer line (316L). | ☒Yes | ☐ No | N/A |
| 2 | Stirring volume | Min batch volume 144 L Max batch volume 384 L | Minimum volume of the stirrer (at rest): 84.4 L Minimum volume of the stirrer (at maximum speed): 78.8 L Maximum working volume: 1500 L | ☒Yes | ☐ No | The min/max batch volume are within the stirring volume range of equipment capability. |
| 3 | Temperature measurement | Min batch volume 144 L | The equipment adopts bottom temperature measurement, which can achieve the minimum temperature control volume required by the process and provides excellent temperature monitoring. | ☒Yes | ☐ No | The equipment adopts bottom temperature measurement, which can achieve the minimum temperature control volume required by the process and provides excellent temperature monitoring. |
| 4 | Stirring speed | Reference stirring speed 95±10 rpm | Agitator type: Double-layer four-bladed inclined paddle; The rotational speed of equipment is 15~130 rpm | ☒Yes | ☐ No | The stirring speed range required by the process is within the stirring speed range of the equipment |
| 5 | Solution Feeding | DMF minimum charging amount: 91.01 kg, add accuracy required: ±3% DMF rinse the wall range: 43.50 kg ~ 47.50 kg, add accuracy required: ±2 kg | When the DMF add volume of the equipment is ≥70 kg, the feeding accuracy is within ≤3% When the DMF add volume of the equipment is 5kg~70 kg, the feeding accuracy is ±2 kg | ☒Yes | ☐ No | The equipment is equipped with Dedicated pipeline feeding, and the feeding accuracy required by the process is within the control range of the equipment. |
|  |  | DIC addition amount range: 7.53 kg ~ 19.85 kg, add accuracy required: ±3% | 2.00 kg ≤m≤300.00 kg: ±3.0% | ☒Yes | ☐ No |  |
|  |  | DIPEA addition amount range: 21.03 kg ~ 42.06 kg, add accuracy required: ±3% | 2.00 kg ≤m≤300.00 kg: ±3.0% | ☒Yes | ☐ No |  |
|  |  | Pyridine addition amount range: 6.25 kg ~ 6.63 kg, add accuracy required: ±3% | 2.00 kg ≤m≤300.00 kg: ±3.0% | ☒Yes | ☐ No | The equipment is equipped with feeding ports And the feeding accuracy required by the process is within the control range of the equipment by manual weighing |
|  |  | Ac2O addition amount range: 6.71 kg ~ 7.13 kg, add accuracy required: ±3% | 2.00 kg ≤m≤300.00 kg: ±3.0%/ | ☒Yes | ☐ No |  |
| 6 | Process Design | Control range of batch temperature from the process requirement 0~30 °C | Equipment system design range: -6~54 ℃ (independent heat exchange system, ethylene glycol) | ☒Yes | ☐ No | The temperature control range required by the process is within the temperature control capacity of the equipment. |
|  |  | Control range of batch jacket temperature from the process requirement 0~30 °C | Control range of temperature of the equipment capability: -14~58 ℃ | ☒Yes | ☐ No | The control range of batch jacket temperature from process requirement is within the control range of equipment capability |
|  |  | No requirements of pressure from process. | The pressure range of equipment for normal operation: 0~0.0807 MPa | ☒Yes | ☐ No | The pressure range from process requirement is within the control range of equipment capability |
|  |  | Nitrogen protection/Inert: oxygen content ≤1.0%.Nitrogen protection/Inert | The equipment has the function of protection under nitrogen flow or static pressure Control range of oxygen content of the equipment: ≤1.0%The equipment has the function of protection under nitrogen flow or static pressure | ☒Yes | ☐ No | N/A |
|  |  | Solid charge | The equipment is equipped with feeding port, Solid charge via powder feeding bag | ☒Yes | ☐ No | N/A |
|  |  | Add Liquid | The equipment is equipped with Dedicated pipeline feedings and three liquid inlets | ☒Yes | ☐ No | N/A |
|  |  | Transfer from the Activation Tank TJ4S-1213-Sy15-R01 to Synthesis Reactor TJ4S-1213-Sy15 | The equipment is equipped with dedicated pipeline | ☒Yes | ☐ No | N/A |
|  |  | Filter until no liquid drips | The equipment is equipped with a sieve plate for filtration and a tuning fork level meter, which can be used to determine the filtration end point | ☒Yes | ☐ No | N/A |

Note: Refers to equipment qualification document VR-TJ4-334.01-065.02

Assessment on function & capacity for solid charge funnel TJ4S-1213-R22

See Table 7-6

Table 7-6: Equipment Suitability Assessment on Function & Capacity for Glass-line reactor TJ4S-1213-R22

| S/N | Process Requirements |  | Equipment Characteristics | Feasibility |  | Justifications |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Operations | Cooling, heating, stirring, material loading. | Reactor TJ4S-1213-R22 has the functions of cooling, heating, stirring, and material loading. | ☒Yes | ☐ No | N/A |
|  |  | Rinse by DMF | Reactor has rinsing function: spray balls. | ☒Yes | ☐ No | N/A |
|  |  | Discharge from the TJ4S-1213- R22 to the TJ4S-1213-Sy15 | Dedicated transfer line (316L). | ☒Yes | ☐ No | N/A |
| 2 | Stirring volume | Min batch volume 288 L Max batch volume 336 L | Minimum volume of the stirrer can reach at rest: 82.30 L; Minimum volume of the stirrer can reach at maximum speed: 58.50 L. Maximum working volume: 1000 L | ☒Yes | ☐ No | The min/max batch volume are within the stirring volume range of equipment capability. |
| 3 | Stirring speed | The range of agitation speed from process requirements: 95±10 rpm | Agitator type: Impeller type; The range of agitation speed of equipment capability: 0~138 rpm | ☒Yes | ☐ No | The stirring speed range required by the process is within the stirring speed range of the equipment. |
| 5 | Temperature measurement | The min batch volume 288 L | Minimum volume of the thermometer tube can reach at rest: 123.10 L; Minimum volume of the thermometer tube can reach at maximum speed: 107.40 L | ☒Yes | ☐ No | The min batch volume is within the min temperature sending capability, a good temperature monitoring can be expected. |
| 6 | Solution charging | DMF charging amount: 44.14 kg ~ 272.77 kg, required charging accuracy: ±3% | 2.00 kg ≤m≤300.00 kg: ±3.0% | ☒Yes | ☐ No | The charging accuracy required by the process is within the control range of the equipment. |
| 7 | Process Design | Control range of batch temperature from the process requirement 20~30 °C | Control range of temperature of the equipment capability: -2.9~112.3 ℃ | ☒Yes | ☐ No | The control range of batch temperature from process requirement is within the control range of equipment capability |
|  |  | No requirements of pressure from process. | The pressure range of equipment for normal operation: -0.098~0.083 MPa | ☒Yes | ☐ No | The pressure range from process requirement is within the control range of equipment capability |
|  |  | Nitrogen protection/Inert: oxygen content ≤1.0%. | The equipment has the function of protection under nitrogen flow or static pressure Control range of oxygen content of the equipment: ≤1.0% | ☒Yes | ☐ No | N/A |
|  |  | Liquid charge | Dedicated pipeline for charging | ☒Yes | ☐ No | N/A |
|  |  | Solution transfer | The tubing between R22 and L2 synthesizer is independent of the tubing between R22 and L1 synthesizer. The transfer endpoint is determined by operator visual observation until no further liquid flow is observed. Switching between the two tubing lines is realized via manual tee and valves. Dedicated pneumatic pumps are used for material transfer, and dual-valves are equipped at the synthesizer feed inlet, which can effectively prevent transfer errors. Cleaning operations shall be performed for the equipment upon completion of each project. Project-switching can only be initiated after cleaning is completed. | ☒Yes | ☐ No | The solutions prepared in the reactor for L1 and L2 are identical. Independent transfer lines are applied for L1 and L2 to avoid transfer errors. Cleaning is performed upon completion of each project to effectively prevent cross-contamination. |

Note: Refers to equipment qualification document SVP-TJ4-334.01-046(TJ4-IOPQ-SC-AsymP3S 1.0.1-02.01.)

Assessment on function & capacity for 200L Glass-line Reactor TJ4S-1200-AT02

See Table 7-5

Table 7-5: Equipment Suitability Assessment on Function & Capacity for 500L Glass-line Reactor TJ4S-1200-AT02

| S/N | Process Requirements |  | Equipment characteristics | Feasibility |  | Justifications |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Operations | Stirring, material loading and solution transfer. | The 500 L Storage tank TJ4S-1200-AT02 is equipped with stirring, material loading and solution transfer | ☒Yes | ☐ No | N/A |
| 2 | Stirring volume | Min batch volume: 45.88 L Max batch volume: 50.11 L | Stirring Minimum volume of equipment (at rest): 20.05 L Stirring Minimum volume of equipment (at maximum speed): 17.75 L Maximum working volume: 500 L | ☒Yes | ☐ No | The Stirring volume required by the process is within the stirring volume range of equipment. |
| 4 | Stirring speed | N/A | Agitator type: Propeller type Qualified maximum agitating speed is 125 rpm | ☒Yes | ☐ No | N/A |
| 5 | Process Design | Nitrogen protection/Inert: oxygen content ≤1.0%. | The equipment has the function of protection under nitrogen flow or static pressure Control range of oxygen content of the equipment: ≤1.0% | ☒Yes | ☐ No | N/A |
|  |  | Solid charge | The equipment is equipped with feeding port, Solid charge via powder feeding bag | ☒Yes | ☐ No | N/A |
|  |  | DMF maximum charging amount:43.50 ~ 47.50 kg, required charging accuracy: ±3% | 2.00 kg ≤ m ≤ 300.00 kg: ± 3.0% | ☒Yes | ☐ No | The charging accuracy required by the process is within the control range of the equipment. |
|  |  | Add Liquid | The equipment is equipped with Dedicated pipeline feedings and three liquid inlets | ☒Yes | ☐ No | N/A |
|  |  | Solution transfer | The equipment is equipped with solution transfer lines and feeding ports | ☒Yes | ☐ No | N/A |

Note: Refers to equipment qualification document SVP-TJ4-170.01-068

Conclusion: The intended equipment can meet the process requirements on functions and capacity

- Compatibility Study Based on the Process Flow

See Table 8-1 ~ Table 8-7 below

Table 8-1: Equipment Compatibility and Corrosion Impact Study for Agitator tank TJ4S-1213-SPS01-AT01/ TJ4S-1213-SPS01-AT02

| Process steps |  | Reaction |  |  | Equipment Codes |  | TJ4S-1213-SPS01-AT01&TJ4S-1213-SPS01-AT02 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| S/N | Process description (indicate the equipment number, solvent/regent/material name) | Equipment (Accessories) Name and its MOC | Corrosion Probability | Corrosion Impact | Chemical Compatible, Risk and Rational |  | Data Source |
| 1 | Prepare 20%piperidine of DMF (v/v%) solution: Add DMF to a Storage Tanks (TJ4S-1213-SPS01-AT01& TJ4S-1213-SPS01-AT02), then start the stirrer, and charge piperidine. Stir for 30 min at RT, store at RT. | Inner surface, temperature probe, agitator, pipeline, transfer pump, mass flowmeter, valve: 304 SS Mechanical seal: 304SS & PTFE & FFKM Sight glass: Glass Valve, gasket, including manway gaskets and gaskets on all inlet valve manifolds: PTFE Mechanical seal: 304SS & PTFE & FFKM | No | No | ☒Yes | ☐ No | ☐ Asymchem R&D Lab: N/A ☒ Asymchem Production data, Lot Number: The system based on production experience of batch CPo140020-02-01-01~04, compatible for PTFE,316L, 304SS ☐ Supplier data : N/A ☒ Chemical Engineering Handbook & Page: page10-14~10-15、page10-22~10-23 ☒ *Handbook & Page: Page 1161-1164 ☒Other: SOP:QM01020 The piperidine is a basic substance and has stable properties. Do not react with materials such as GL, 304SS, PTFE, FFKM and glass at room temperature. The materials are compatible. |
|  |  |  |  |  | ☐ High ☐ Medium ☒ Low | Data sources indicate good compatibility |  |

*: Note: Handbook refers to < CORROSION RESISTANCE TABLES (Author: Philip A. Schweitzer, P.E. Published in 2004)>.

Table 8-2: Equipment Compatibility and Corrosion Impact Study for glovebox TJ4S-1200-GB01&TJ4S-1200-GB02& TJ4S-1200-GB03& TJ4S-1200-GB05

| Process steps |  | Sieving |  |  | Equipment Codes |  | TJ4S-1200-GB01&TJ4S-1200-GB02& TJ4S-1200-GB03& TJ4S-1200-GB05 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| S/N | Process description (indicate the equipment number, solvent/regent/material name) | Equipment (Accessories) Name and its MOC | Corrosion Probability | Corrosion Impact | Chemical Compatible, Risk and Rational |  | Data Source |
| 1 | Materials involved in dispensing: Amino acids, Oxyma, PyBop and 2-CTC Resin are dispensed using a glovebox (TJ4S-1200-GB01&TJ4S-1200-GB02&TJ4S-1200-GB03&TJ4S-1200-GB04&TJ4S-1200-GB05). The materials are dispensed into charging bags, which are then placed inside low-density polyethylene outer bags, with labels affixed. Subsequently, the bagged materials are placed into high-density polyethylene (HDPE) drums for temporary storage. Materials are as follows: 2-CTC Resin; Fmoc-Gly-OH.; Fmoc-L-Phe-OH; Fmoc-L-N-Me-Asn(Trt)-OH; Fmoc-L-Asn(Trt)-OH; Fmoc-L-Ser(tBu)-OH; Fmoc-L-Arg(Pbf)-OH; Fmoc-L-Val-OH; Fmoc-L-Leu-OH; Fmoc-L-α-Me-Phe-OH; Fmoc-L-Glu(OtBu)-OH·H2O; Fmoc-L-Ala-OH·H2O; Fmoc-L-Orn(Boc)-OH; Oxyma Pure; PyBop; Cleaning solvents: Purified water and methanol. | Inner surface, rigid connection piece: 316L Gasket: PTFE Sight glass:Glass 304 SS Filter cartridge: PP Gloves: CSPE | No | No | ☒Yes | ☐ No | ☐ Asymchem R&D Lab:N/A ☒ Asymchem Production data, Lot Number: CPo140020-02-01-01~04 ☐ Supplier data: N/A ☐ *Handbook & Page: N/A ☒ Other: Resins, amino acids and oxyma pure are relatively stable in solid state, and do not react with materials such as PC, stainless steel, and glass at room temperature. The dispensing operation has a short duration, CEPS and the area is cleaned during material changeover. The materials are compatible. |
|  |  |  |  |  | ☐ High ☐ Medium ☒ Low | Based on the compatibility test results |  |

Table 8-3: Equipment Compatibility and Corrosion Impact Study for glovebox TJ4S-1200-GB04

| Process steps |  | Sieving |  |  | Equipment Codes |  | TJ4S-1200-GB04 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| S/N | Process description (indicate the equipment number, solvent/regent/material name) | Equipment (Accessories) Name and its MOC | Corrosion Probability | Corrosion Impact | Chemical Compatible, Risk and Rational |  | Data Source |
| 1 | Materials involved in dispensing: Amino acids, Oxyma, PyBop and 2-CTC Resin are dispensed using a glovebox (TJ4S-1200-GB01&TJ4S-1200-GB02&TJ4S-1200-GB03&TJ4S-1200-GB04&TJ4S-1200-GB05). The materials are dispensed into charging bags, which are then placed inside low-density polyethylene outer bags, with labels affixed. Subsequently, the bagged materials are placed into high-density polyethylene (HDPE) drums for temporary storage. Materials are as follows: 2-CTC Resin; Fmoc-Gly-OH.; Fmoc-L-Phe-OH; Fmoc-L-N-Me-Asn(Trt)-OH; Fmoc-L-Asn(Trt)-OH; Fmoc-L-Ser(tBu)-OH; Fmoc-L-Arg(Pbf)-OH; Fmoc-L-Val-OH; Fmoc-L-Leu-OH; Fmoc-L-α-Me-Phe-OH; Fmoc-L-Glu(OtBu)-OH·H2O; Fmoc-L-Ala-OH·H2O; Fmoc-L-Orn(Boc)-OH; Oxyma Pure; PyBop; Cleaning solvents: Purified water and methanol. | Inner surface, rigid connection piece: Halar Gasket: PTFE Sight glass:Glass Dispensing scoop: 304 SS Filter cartridge: PP Gloves: CSPE | No | No | ☒Yes | ☐ No | ☐ Asymchem R&D Lab: N/A ☒ Asymchem Production data, Lot Number: CPo140020-02-01-01~04 ☐ Supplier data: N/A ☐ *Handbook & Page: N/A ☒ Other: Resins, amino acids and oxyma pure are relatively stable in solid state, and do not react with materials such as PC, stainless steel, and glass at room temperature. The dispensing operation has a short duration, CEPS and the area is cleaned during material changeover. The materials are compatible. |

Table 8-4: Equipment Compatibility and Corrosion Impact Study for TJ4S-1213-Sy15

| Process steps |  |  |  |  |  |  | Reaction & Work-up & Concentration & Crystallization |  |  |  |  |  |  |  |  |  | Equipment Codes |  |  |  |  |  | TJ4S-1213-Sy15 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S/N |  | Process description (indicate the equipment number, solvent/regent/material name) |  |  | Equipment (Accessories) Name and its MOC |  |  | Corrosion Probability |  |  |  | Corrosion Impact |  |  |  | Chemical Compatible, Risk and Rational |  |  |  |  |  | Data Source |  |  |
| 1. |  |  | Rinse: 1. Add DMF (200.00kg-300.00 kg) into activation tank (TJ4S-1213-Sy15-R01). Then transfer the solution into synthesis reactor (TJ4S-1213-Sy15). 2. Start the stirrer, bubble with nitrogen for 5±2min, then stop the stirrer and nitrogen, drain. Sample the mixture for Water content analysis. Note: In all the following operations, unless otherwise noted. The agitation speed of synthesis reactor is set as 20±10 rpm, the agitation speed of activation tank is set as 95±10 rpm, and the nitrogen flow rate is set to 5±2 m³/h. |  |  | Reaction vessel material: Hastelloy (C-276) Agitator material: Hastelloy (N10276) Sieve plate: HC-22 Nitrogen distribution piping: Hastelloy (C-276) Sight glass: Glass Transfer pump material: SS316+PTFE, Thermometer well material: Hastelloy (C-276) Pipeline material: 316L Sealing ring: PTFE Mass flow meter: SS Valve material: 304 & 316L Gasket Material: PTFE Waste discharge pump material: SS316+PTFE Mechanical seal: 304SS & PTFE & FFKM Gloves: CSPE |  |  |  |  | No |  |  |  | No |  |  | ☒Yes |  |  | ☐ No |  |  | ☐ Asymchem R&D Lab: N/A ☒ Asymchem Production data, Lot Number: CPo140020-02-01-01~04 ☐ Supplier data: N/A ☒ *Handbook & Page: The Pilot Plant Real Book & Page 10-14, 10-15, ☒ Other: C-276 exhibits excellent corrosion resistance to various organic acids, inorganic acids, alkalis and salts. The material demonstrates good corrosion resistance, including tolerance to organic solvents. The solvents are neutral, and the material compatibility is confirmed. |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ☐ High ☐ Medium ☒ Low |  |  | According to The Pilot Plant Real Book Page 10-14&10-15, Dimethylformamide is compatible with SS, 306 SS and PTFE; Based on Asymchem previous campaign production data and experience, Dimethylformamide is also compatible with PTFE, Glass, 316SS and SS |  |  |  |
| 2 |  |  | Resin swelling: 1. Add 48.00 kg 2-CTC resin (1.0 eq.) into TJ4S-1213-Sy15. Rinse reactor wall with 7.0 V DMF (6.636 kg/kg). 2. Bubble with nitrogen for 5min±2min (target: 5min), then start the stirrer. Rinse reactor wall with 1.0 V DMF (0.948 kg/kg). 3. Adjust the mixture to 25±5 ℃ (target: 25℃). After stirring for 35±5 min (target: 35 min) at 25±5 ℃ (target: 25 ℃), turn off the nitrogen, stop stirring, and drain until there is no obvious dripping. |  |  | Reaction vessel material: Hastelloy (C-276) Agitator material: Hastelloy (N10276) Sieve plate: HC-22 Nitrogen distribution piping: Hastelloy (C-276) Sight glass: Glass Transfer pump material: SS316+PTFE, Thermometer well material: Hastelloy (C-276) Pipeline material: 316L Sealing ring: PTFE Mass flow meter: SS Valve material: 304 & 316L Gasket Material: PTFE Waste discharge pump material: SS316+PTFE Mechanical seal: 304SS & PTFE & FFKM Gloves: CSPE |  |  |  |  | No |  |  |  | No |  |  | ☒Yes |  |  | ☐ No |  |  | ☐ Asymchem R&D Lab: N/A ☒ Asymchem Production data, Lot Number: CPo140020-02-01-01~04 ☐ Supplier data: N/A ☒ *Handbook & Page: The Pilot Plant Real Book Page 10-14&10-15 ☒ Other: C-276 exhibits excellent corrosion resistance to various organic acids, inorganic acids, alkalis and salts. The material demonstrates good corrosion resistance, including tolerance to organic solvents. The solvents are neutral, and the material compatibility is confirmed. |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ☐ High ☐ Medium ☒ Low |  |  | According to The Pilot Plant Real Book Page 10-14&10-15, Dimethylformamide is compatible with SS, 316L and PTFE; based on Asymchem previous campaign production data and experience, Dimethylformamide is also compatible with PTFE, Glass, 316L and SS |  |  |  |  |
| 3 |  |  | Wash resin 1. Add 5.0 V DMF (4.740 kg/kg) into synthesis reactor (TJ4S-1213-Sy15). 2. Bubble with nitrogen for 5±2 min (target: 5 min), then start the agitator. After stirring for 12±2 min (target: 12 min), stop stirring, turn off the nitrogen. Drain until there is no obvious drip. |  |  | Reaction vessel material: Hastelloy (C-276) Agitator material: Hastelloy (N10276) Sieve plate: HC-22 Nitrogen distribution piping: Hastelloy (C-276) Sight glass: Glass Transfer pump material: SS316+PTFE, Thermometer well material: Hastelloy (C-276) Pipeline material: 316L Sealing ring: PTFE Mass flow meter: SS Valve material: 304 & 316L Gasket Material: PTFE Waste discharge pump material: SS316+PTFE Mechanical seal: 304SS & PTFE & FFKM Gloves: CSPE |  |  |  |  | No |  |  |  | No |  |  | ☒Yes |  |  | ☐ No |  |  | ☐ Asymchem R&D Lab: N/A ☒ Asymchem Production data, Lot Number: CPo140020-02-01-01~04 ☐ Supplier data: N/A ☒ *Handbook & Page: The Pilot Plant Real Book & Page 10-14, 10-15, ☒ Other:C-276 exhibits excellent corrosion resistance to various organic acids, inorganic acids, alkalis and salts. The material demonstrates good corrosion resistance, including tolerance to organic solvents. The solvents are neutral, and the material compatibility is confirmed. |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ☐ High ☐ Medium ☒ Low |  |  | According to The Pilot Plant Real Book Page 10-14&10-15, Dimethylformamide is compatible with SS, 306 SS and PTFE. Based on Asymchem previous campaign production data and experience, DIPEA is also compatible with PTFE, Glass, SS and SS |  |  |  |
| 4 |  |  | Fmoc-Gly-OH Activate and transfer 1. Adjust to 25±5 ℃ (target: 25 ℃), Add 6.0 eq. DIPEA into the TJ4S-1213-Sy15-R01 at 25±5℃ (target: 25℃). Reference addition time is 30±15 min. 2. Stir for 5±2 min after addition. Transfer the mixture in activation tank (TJ4S-1213-Sy15-R01) into TJ4S-1213-Sy15. Add 1.0 V DMF (0.948 kg/kg) into TJ4S-1213-Sy15-R01 to rinse the wall. 3. Bubble with nitrogen to the synthesis reactor (TJ4S-1213-Sy15) for 5±2 min, then start the agitator. 4. Transfer the mixture in activation tank (TJ4S-1213-Sy15-R01) into synthesis reactor (TJ4S-1213-Sy15) |  |  | Reaction vessel material: Hastelloy (C-276) Agitator material: Hastelloy (N10276) Sieve plate: HC-22 Nitrogen distribution piping: Hastelloy (C-276) Sight glass: Glass Transfer pump material: SS316+PTFE, Thermometer well material: Hastelloy (C-276) Pipeline material: 316L Sealing ring: PTFE Mass flow meter: SS Valve material: 304 & 316L Gasket Material: PTFE Waste discharge pump material: SS316+PTFE Mechanical seal: 304SS & PTFE & FFKM Gloves: CSPE |  |  |  |  | No |  |  |  | No |  |  | ☒Yes |  |  | ☐ No |  |  | ☐ Asymchem R&D Lab: N/A ☒ Asymchem Production data, Lot Number: CPo140020-02-01-01~04 ☐ Supplier data: N/A ☐ *Handbook & Page: N/A ☒ Other: C-276 exhibits excellent corrosion resistance to various organic acids, inorganic acids, alkalis and salts. The material demonstrates good corrosion resistance, including tolerance to organic solvents. The solvents are neutral, and the material compatibility is confirmed. |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ☐ High ☐ Medium ☒ Low |  |  | According to The Pilot Plant Real Book Page 10-14&10-15, Dimethylformamide is compatible with SS, 306L SS and PTFE. Based on Asymchem previous campaign production data and experience |  |  |  |
| 5 |  |  | Fmoc-Gly-OH coupling 1. Heat the mixture to 25±5 ℃. The mixture is allowed to react for 3h±15 min (target: 3 h) at 25±5 ℃ (target: 25 ℃). 2. Drain until there is no reaction mixture drip. Move to next operation. |  |  | Reaction vessel material: Hastelloy (C-276) Agitator material: Hastelloy (N10276) Sieve plate: HC-22 Nitrogen distribution piping: Hastelloy (C-276) Sight glass: Glass Transfer pump material: SS316+PTFE, Thermometer well material: Hastelloy (C-276) Pipeline material: 316L Sealing ring: PTFE Mass flow meter: SS Valve material: 304 & 316L Gasket Material: PTFE Waste discharge pump material: SS316+PTFE Mechanical seal: 304SS & PTFE & FFKM Gloves: CSPE |  |  |  |  | No |  |  |  | No |  |  | ☒Yes |  |  | ☐ No |  |  | ☐ Asymchem R&D Lab: N/A ☒ Asymchem Production data, Lot Number: CPo140020-02-01-01~04 ☐ Supplier data: N/A ☐ *Handbook & Page: The Pilot Plant Real Book Page 10-14&10-15, ☒ Other: C-276 exhibits excellent corrosion resistance to various organic acids, inorganic acids, alkalis and salts.The material demonstrates good corrosion resistance, including tolerance to organic solvents. The solvents are neutral, and the material compatibility is confirmed. |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ☐ High ☐ Medium ☒ Low |  |  | According to The Pilot Plant Real Book Page 10-14&10-15, Dimethylformamide is compatible with SS, 306 SS and PTFE. Based on Asymchem previous campaign production data and experience |  |  |  |
| 6 |  |  | Capping: 1. Add 8.0 V DMF (7.584 kg/kg) into TJ4S-1213-Sy15. Bubble with nitrogen for 5±2 min, then start the stirrer. 2. After stirring for 5±2 min, weigh and add 10.0 eq. Methanol. Manually weigh and add 5.0 eq. DIPEA at a reference time 20 ±10 min at 15~25 ℃. 3. Adjust to 20~30 ℃，the mixture is allowed to react for 1 h±15 min (target: 1 h) at 20~30 ℃. Stop stirring and turn off the nitrogen. Drain until there is no reaction mixture drip. |  |  | Reaction vessel material: Hastelloy (C-276) Agitator material: Hastelloy (N10276) Sieve plate: HC-22 Nitrogen distribution piping: Hastelloy (C-276) Sight glass: Glass Transfer pump material: SS316+PTFE, Thermometer well material: Hastelloy (C-276) Pipeline material: 316L Sealing ring: PTFE Mass flow meter: SS Valve material: 304 & 316L Gasket Material: PTFE Waste discharge pump material: SS316+PTFE Mechanical seal: 304SS & PTFE & FFKM Gloves: CSPE |  |  |  |  | No |  |  |  | No |  |  | ☒Yes |  |  | ☐ No |  |  | ☐ Asymchem R&D Lab: N/A ☒ Asymchem Production data, Lot Number: CPo140020-02-01-01~04 ☐ Supplier data: N/A ☐ *Handbook & Page: N/A ☒ Other: C-276 exhibits excellent corrosion resistance to various organic acids, inorganic acids, alkalis and salts. The material demonstrates good corrosion resistance, including tolerance to organic solvents. The solvents are neutral, and the material compatibility is confirmed. |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ☐ High ☐ Medium ☒ Low |  |  | According to The Pilot Plant Real Book Page 10-14&10-15, Dimethylformamide is compatible with SS, 306 SS and PTFE. Based on Asymchem previous campaign production data and experience |  |  |  |
| 7 |  |  | Gly24 ~ Leu12, including Ac2O/Pyr capping Wash resin 1. Add 8.0 V DMF (7.584 kg/kg) into TJ4S-1213-Sy15-R01. Bubble with nitrogen for 5±2 min, then start the stirrer. 2. After stirring for 5±2 min, stop stirring, turn off the nitrogen. Then Transfer the mixture into TJ4S-1213-Sy15. Bubble with nitrogen for 5±2 min, then start the stirrer. 3. After stirring for 7±2 min, stop stirring, turn off the nitrogen. Drain until there is no obvious drip. Perform this operation 3 times.. |  |  | Reaction vessel material: Hastelloy (C-276) Agitator material: Hastelloy (N10276) Sieve plate: HC-22 Nitrogen distribution piping: Hastelloy (C-276) Sight glass: Glass Transfer pump material: SS316+PTFE, Thermometer well material: Hastelloy (C-276) Pipeline material: 316L Sealing ring: PTFE Mass flow meter: SS Valve material: 304 & 316L Gasket Material: PTFE Waste discharge pump material: SS316+PTFE Mechanical seal: 304SS & PTFE & FFKM Gloves: CSPE |  |  |  |  | No |  |  |  | No |  |  | ☒Yes |  |  | ☐ No |  |  | ☐ Asymchem R&D Lab: N/A ☒ Asymchem Production data, Lot Number: CPo140020-02-01-01~04 ☐ Supplier data: N/A ☐ *Handbook & Page: N/A ☒ Other: C-276 exhibits excellent corrosion resistance to various organic acids, inorganic acids, alkalis and salts.The material demonstrates good corrosion resistance, including tolerance to organic solvents.The solvents are neutral, and the material compatibility is confirmed. |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ☐ High ☐ Medium ☒ Low |  |  | According to The Pilot Plant Real Book Page 10-14&10-15, Dimethylformamide is compatible with SS, 306 SS and PTFE. Based on Asymchem previous campaign production data and experience |  |  |  |
| 8 |  |  | PrePhe23 ~ preOrn11 Excluding preSer20 De-Fmoc: 1. Add 8.0 V 20% Piperidine in DMF (7.448 kg/kg) into TJ4S-1213-Sy15. Bubble with nitrogen for 5±2 min, then start the stirrer. 2. Adjust to 25±5 ℃ (target: 25 ℃). After stirring for 2 h±15 min (target: 2 h) at 25±5℃ (target: 25 ℃), stop stirring, turn off the nitrogen. Drain until there is no obvious drip. |  |  | Reaction vessel material: Hastelloy (C-276) Agitator material: Hastelloy (N10276) Sieve plate: HC-22 Nitrogen distribution piping: Hastelloy (C-276) Sight glass: Glass Transfer pump material: SS316+PTFE, Thermometer well material: Hastelloy (C-276) Pipeline material: 316L Sealing ring: PTFE Mass flow meter: SS Valve material: 304 & 316L Gasket Material: PTFE Waste discharge pump material: SS316+PTFE Mechanical seal: 304SS & PTFE & FFKM Gloves: CSPE |  |  |  |  | No |  |  |  | No |  |  | ☒Yes |  |  | ☐ No |  |  | ☐ Asymchem R&D Lab: N/A ☒ Asymchem Production data, Lot Number: CPo140020-02-01-01~04 ☐ Supplier data: N/A ☒ *Handbook & Page: QM01020& Page 5/11 ☒ Other: C-276 exhibits excellent corrosion resistance to various organic acids, inorganic acids, alkalis and salts. The material demonstrates good corrosion resistance, including tolerance to organic solvents.The solvents are neutral, and the material compatibility is confirmed. |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ☐ High ☐ Medium ☒ Low |  |  | Based on Asymchem previous campaign production data and experience |  |  |  |
| 9 |  |  | prePhe23 ~ preLeu12 Excluding preSer20, preOrn11 Post deFmoc washing 1. . Add 10.0 V Isopropyl Acetate (8.730 kg/kg) into TJ4S-1213-Sy15. Bubble with nitrogen for 5±2 min, then start the stirrer. 2. After stirring for 5±2 min, stop stirring, turn off the nitrogen. Drain until there is no obvious drip. Perform this operation 4 times. |  |  | Reaction vessel material: Hastelloy (C-276) Agitator material: Hastelloy (N10276) Sieve plate: HC-22 Nitrogen distribution piping: Hastelloy (C-276) Sight glass: Glass Transfer pump material: SS316+PTFE, Thermometer well material: Hastelloy (C-276) Pipeline material: 316L Sealing ring: PTFE Mass flow meter: SS Valve material: 304 & 316L Gasket Material: PTFE Waste discharge pump material: SS316+PTFE Mechanical seal: 304SS & PTFE & FFKM Gloves: CSPE |  |  |  |  | No |  |  |  | No |  |  | ☒Yes |  |  | ☐ No |  |  | ☐ Asymchem R&D Lab: N/A ☒ Asymchem Production data, Lot Number: CPo140020-02-01-01~04 ☐ Supplier data: N/A ☒ *Handbook & Page: QM01020& Page 5/11 ☒ Other: C-276 exhibits excellent corrosion resistance to various organic acids, inorganic acids, alkalis and salts. The material demonstrates good corrosion resistance, including tolerance to organic solvents. The solvents are neutral, and the material compatibility is confirmed. |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ☐ High ☐ Medium ☒ Low |  |  | According to QM01020& Page 5/11 Isopropyl Acetate is compatible with SS, 306 SS and PTFE. Based on Asymchem previous campaign production data and experience Based on Asymchem previous campaign production data and experience |  |  |  |
| 10 |  |  | PrePhe23 ~ preOrn11 Re-swelling 1. Add 12.0 V DMF (11.376 kg/kg) into TJ4S-1213-Sy15. Bubble with nitrogen for 5±2 min, then start the stirrer. 2. After stirring for 10±2 min, stop stirring, turn off the nitrogen. Drain until there is no obvious drip. Perform this operation 2 times. |  |  | Reaction vessel material: Hastelloy (C-276) Agitator material: Hastelloy (N10276) Sieve plate: HC-22 Nitrogen distribution piping: Hastelloy (C-276) Sight glass: Glass Transfer pump material: SS316+PTFE, Thermometer well material: Hastelloy (C-276) Pipeline material: 316L Sealing ring: PTFE Mass flow meter: SS Valve material: 304 & 316L Gasket Material: PTFE Waste discharge pump material: SS316+PTFE Mechanical seal: 304SS & PTFE & FFKM Gloves: CSPE |  |  |  |  | No |  |  |  | No |  |  | ☒Yes |  |  | ☐ No |  |  | ☐ Asymchem R&D Lab: N/A ☒ Asymchem Production data, Lot Number: CPo140020-02-01-01~04 ☐ Supplier data: N/A ☒ *Handbook & Page: The Pilot Plant Real Book Page 10-14&10-15 ☒ Other: C-276 exhibits excellent corrosion resistance to various organic acids, inorganic acids, alkalis and salts.The material demonstrates good corrosion resistance, including tolerance to organic solvents.The solvents are neutral, and the material compatibility is confirmed. |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ☐ High ☐ Medium ☒ Low |  |  | According to The Pilot Plant Real Book Page 10-14&10-15, Dimethylformamide is compatible with SS, 306 SS and PTFE. Based on Asymchem previous campaign production data and experience |  |  |  |
| 11 |  |  | Phe23 Coupling: 1. Heat the mixture to 25±5 ℃. Start timing when temperature reaches 20 ℃. The mixture is allowed to react at 25±5 ℃ (target: 25 ℃). 2. After 4 h±15 min (target: 4 h), sample the mixture every 0.5~1 h for ninhydrin test analysis until two consecutive samples test as negative (Yellow to Light pink Report Negative= Complete coupling; Blue to Purple Positive= Incomplete coupling).). And the mixture should be stirred for coupling time (6 h ≤ Coupling time ≤ 14 h). 3. Stop stirring and turn off the nitrogen. Drain until there is no reaction mixture drip. Move to next operation. |  |  | Reaction vessel material: Hastelloy (C-276) Agitator material: Hastelloy (N10276) Sieve plate: HC-22 Nitrogen distribution piping: Hastelloy (C-276) Sight glass: Glass Transfer pump material: SS316+PTFE, Thermometer well material: Hastelloy (C-276) Pipeline material: 316L Sealing ring: PTFE Mass flow meter: SS Valve material: 304 & 316L Gasket Material: PTFE Waste discharge pump material: SS316+PTFE Mechanical seal: 304SS & PTFE & FFKM Gloves: CSPE |  |  |  |  | No |  |  |  | No |  |  | ☒Yes |  |  | ☐ No |  |  | ☐ Asymchem R&D Lab: N/A ☒ Asymchem Production data, Lot Number: CPo140020-02-01-01~04 ☐ Supplier data: N/A ☒ *Handbook & Page: The Pilot Plant Real Book Page 10-14&10-15 ☒ Other: C-276 exhibits excellent corrosion resistance to various organic acids, inorganic acids, alkalis and salts.The material demonstrates good corrosion resistance, including tolerance to organic solvents.The solvents are neutral, and the material compatibility is confirmed. |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ☐ High ☐ Medium ☒ Low |  |  | According to The Pilot Plant Real Book Page 10-14&10-15, Dimethylformamide is compatible with SS, 306 SS and PTFE. Based on Asymchem previous campaign production data and experience |  |  |  |
| 12 |  |  | NMeAsn22, aMePhe15 Coupling 1. Heat the mixture to 25±5 ℃. The mixture is allowed to react at 25±5 ℃ (target: 25 ℃). 2. After 4 h±15 min (target：4 h), sample the mixture every 0.5~1 h for ninhydrin test analysis until two consecutive samples test as negative (Yellow to Light pink Report Negative= Complete coupling; Blue to Purple Positive= Incomplete coupling). 6 h ≤ Coupling time ≤ 14 h). 3. Stop stirring and turn off the nitrogen. Drain until there is no reaction mixture drip. |  |  | Reaction vessel material: Hastelloy (C-276) Agitator material: Hastelloy (N10276) Sieve plate: HC-22 Nitrogen distribution piping: Hastelloy (C-276) Sight glass: Glass Transfer pump material: SS316+PTFE, Thermometer well material: Hastelloy (C-276) Pipeline material: 316L Sealing ring: PTFE Mass flow meter: SS Valve material: 304 & 316L Gasket Material: PTFE Waste discharge pump material: SS316+PTFE Mechanical seal: 304SS & PTFE & FFKM Gloves: CSPE |  |  |  |  | No |  |  |  | No |  |  | ☒Yes |  |  | ☐ No |  |  | ☐ Asymchem R&D Lab: N/A ☒ Asymchem Production data, Lot Number: CPo140020-02-01-01~04 ☐ Supplier data: N/A ☐ *Handbook & Page: The Pilot Plant Real Book Page 10-14&10-15 ☒ Other: C-276 exhibits excellent corrosion resistance to various organic acids, inorganic acids, alkalis and salts.The material demonstrates good corrosion resistance, including tolerance to organic solvents.The solvents are neutral, and the material compatibility is confirmed. |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ☐ High ☐ Medium ☒ Low |  |  | According to The Pilot Plant Real Book Page 10-14&10-15, Dimethylformamide is compatible with SS, 306 SS and PTFE; Based on Asymchem previous campaign production data and experience Dimethylformamide is also compatible with Glass, HSS and SS |  |  |  |
| 13 |  |  | Fmoc-L-Asn(Trt)-OH coupling: 1. Heat the mixture to 25±5 ℃. The mixture is allowed to react at 25±5 ℃ (target: 25 ℃). 2. Once the total reaction time has reached 8 h±15 min, add additional 1.0 eq. Oxyma Pure and 1.0 V DMF (0.948 kg/kg) solution (the mixture should be prepared in TJ4S-1200-AT02 just before use) and 1.1eq. DIC. 3. After 5 h±15 min (target: 5 h). Stop stirring and turn off the nitrogen. Drain until there is no reaction mixture drip. |  |  | Reaction vessel material: Hastelloy (C-276) Agitator material: Hastelloy (N10276) Sieve plate: HC-22 Nitrogen distribution piping: Hastelloy (C-276) Sight glass: Glass Transfer pump material: SS316+PTFE, Thermometer well material: Hastelloy (C-276) Pipeline material: 316L Sealing ring: PTFE Mass flow meter: SS Valve material: 304 & 316L Gasket Material: PTFE Waste discharge pump material: SS316+PTFE Mechanical seal: 304SS & PTFE & FFKM Gloves: CSPE |  |  |  |  | No |  |  |  | No |  |  | ☒Yes |  |  | ☐ No |  |  | ☐ Asymchem R&D Lab: N/A ☒ Asymchem Production data, Lot Number: CPo140020-02-01-01~04 ☐ Supplier data: N/A ☒ *Handbook & Page: The Pilot Plant Real Book Page 10-14&10-15, ☒ Other: C-276 exhibits excellent corrosion resistance to various organic acids, inorganic acids, alkalis and salts.The material demonstrates good corrosion resistance, including tolerance to organic solvents.The solvents are neutral, and the material compatibility is confirmed. |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ☐ High ☐ Medium ☒ Low |  |  | According to The Pilot Plant Real Book Page 10-14&10-15, Dimethylformamide is compatible with SS, 306 SS and PTFE; Based on Asymchem previous campaign production data and experience Dimethylformamide is also compatible with Glass, HSS and SS |  |  |  |
| 14 |  |  | Ac2O/Pyridine Capping 1. Heat the mixture in synthesis reactor (TJ4S-1213-Sy15) to 25±5 ℃. 2. The mixture is allowed to react at 25±5 ℃ (target: 25 ℃) for 1 h±15 min (target: 1 h). 3. Stop stirring and turn off the nitrogen. Drain until there is no reaction mixture drip. Move to next operation. |  |  | Reaction vessel material: Hastelloy (C-276) Agitator material: Hastelloy (N10276) Sieve plate: HC-22 Nitrogen distribution piping: Hastelloy (C-276) Sight glass: Glass Transfer pump material: SS316+PTFE, Thermometer well material: Hastelloy (C-276) Pipeline material: 316L Sealing ring: PTFE Mass flow meter: SS Valve material: 304 & 316L Gasket Material: PTFE Waste discharge pump material: SS316+PTFE Mechanical seal: 304SS & PTFE & FFKM Gloves: CSPE |  |  |  |  | No |  |  |  | No |  |  | ☒Yes |  |  | ☐ No |  |  | ☐ Asymchem R&D Lab: N/A ☒ Asymchem Production data, Lot Number: CPo140020-02-01-01~04 ☐ Supplier data: N/A ☒ *Handbook & Page: The Pilot Plant Real Book Page 10-14&10-15 ☒ Other: C-276 exhibits excellent corrosion resistance to various organic acids, inorganic acids, alkalis and salts.The material demonstrates good corrosion resistance, including tolerance to organic solvents.The solvents are neutral, and the material compatibility is confirmed. |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ☐ High ☐ Medium ☒ Low |  |  | According to The Pilot Plant Real Book Page 10-14&10-15, Dimethylformamide is compatible with SS, 316LPTFE.PTFE; Based on Asymchem previous campaign production data and experience, Dimethylformamide is also compatible with Glass, PTFE and SS |  |  |  |
| 15 |  |  | PreSer20 De-Fmoc: 1. Add 8.0 V 20% Piperidine in DMF (7.448 kg/kg) into the TJ4S-1213-Sy15. Bubble with nitrogen for 3±2 min, then start the stirrer, adjust to 25±5 ℃ (target: 25 ℃). 2. After stirring for 15 min at 25±5℃ (target: 25℃), stop stirring, turn off the nitrogen. Drain until there is no obvious drip. 3. Add 8.0 V 20% Piperidine in DMF (7.448 kg/kg) into TJ4S-1213-Sy15. Bubble with nitrogen for 3±2 min, then start the stirrer. 4. After stirring for 30±2 min at 25±5 ℃ (target: 25℃), stop stirring, turn off the nitrogen. Drain until there is no obvious drip. |  |  | Reaction vessel material: Hastelloy (C-276) Agitator material: Hastelloy (N10276) Sieve plate: HC-22 Nitrogen distribution piping: Hastelloy (C-276) Sight glass: Glass Transfer pump material: SS316+PTFE, Thermometer well material: Hastelloy (C-276) Pipeline material: 316L Sealing ring: PTFE Mass flow meter: SS Valve material: 304 & 316L Gasket Material: PTFE Waste discharge pump material: SS316+PTFE Mechanical seal: 304SS & PTFE & FFKM Gloves: CSPE |  |  |  |  | No |  |  |  | No |  |  | ☒Yes |  |  | ☐ No |  |  | ☐ Asymchem R&D Lab: N/A ☒ Asymchem Production data, Lot Number: CPo140020-02-01-01~04 ☐ Supplier data: N/A ☒ *Handbook & Page: The Pilot Plant Real Book Page Page 10-10&10-11, Page 10-14&10-15, Page 10-24&10-25, ☒ Other: C-276 exhibits excellent corrosion resistance to various organic acids, inorganic acids, alkalis and salts. The material demonstrates good corrosion resistance, including tolerance to organic solvents.The solvents are neutral, and the material compatibility is confirmed. |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ☐ High ☐ Medium ☒ Low |  |  | According to The Pilot Plant Real Book Page 10-22&10-23, piperidine is compatible with SS, 306 SS and PTFE. Based on Asymchem previous campaign production data and experience |  |  |  |
| 16 |  |  | PreSer20, Post deFmoc washing: 1. Add 10.0 V Isopropyl Acetate (8.730 kg/kg) into TJ4S-1213-Sy15. Bubble with nitrogen for 3±2 min, then start the stirrer. 2. After stirring for 5±2 min, stop stirring, turn off the nitrogen. Drain until there is no obvious drip. Perform this operation 4 times. |  |  | Reaction vessel material: Hastelloy (C-276) Agitator material: Hastelloy (N10276) Sieve plate: HC-22 Nitrogen distribution piping: Hastelloy (C-276) Sight glass: Glass Transfer pump material: SS316+PTFE, Thermometer well material: Hastelloy (C-276) Pipeline material: 316L Sealing ring: PTFE Mass flow meter: SS Valve material: 304 & 316L Gasket Material: PTFE Waste discharge pump material: SS316+PTFE Mechanical seal: 304SS & PTFE & FFKM Gloves: CSPE |  |  |  |  | No |  |  |  | No |  |  | ☒Yes |  |  | ☐ No |  |  | ☐ Asymchem R&D Lab: N/A ☒ Asymchem Production data, Lot Number: CPo140020-02-01-01~04 ☐ Supplier data: N/A ☒ *Handbook & Page: QM01020& Page 5/11 ☒ Other: C-276 exhibits excellent corrosion resistance to various organic acids, inorganic acids, alkalis and salts.The material demonstrates good corrosion resistance, including tolerance to organic solvents.The solvents are neutral, and the material compatibility is confirmed. |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ☐ High ☐ Medium ☒ Low |  |  | According to QM01020& Page 5/11 Isopropyl Acetate is compatible with SS, 306 SS and PTFE. Based on Asymchem previous campaign production data and experience Isopropyl Acetate and DMF are also compatible with Glass, PTFE and SS |  |  |  |
| 17 |  |  | Ser20 ~ Leu16, Ala13 ~ Orn11 Coupling: 1. Heat the mixture to 25±5 ℃. The mixture is allowed to react at 25±5 ℃ (target: 25 ℃). 2. After 3 h±15 min (target：3 h), sample the mixture every 0.5~1 h for ninhydrin test analysis until two consecutive samples test as negative (Yellow to Light pink Report Negative= Complete coupling; Blue to Purple Positive= Incomplete coupling).). And the mixture should be stirred for coupling time (4 h≤Coupling Time≤10 h). 3. Stop stirring and turn off the nitrogen. Drain until there is no reaction mixture drip. Move to next operation. |  |  | Reaction vessel material: Hastelloy (C-276) Agitator material: Hastelloy (N10276) Sieve plate: HC-22 Nitrogen distribution piping: Hastelloy (C-276) Sight glass: Glass Transfer pump material: SS316+PTFE, Thermometer well material: Hastelloy (C-276) Pipeline material: 316L Sealing ring: PTFE Mass flow meter: SS Valve material: 304 & 316L Gasket Material: PTFE Waste discharge pump material: SS316+PTFE Mechanical seal: 304SS & PTFE & FFKM Gloves: CSPE |  |  |  |  | No |  |  |  | No |  |  | ☒Yes |  |  | ☐ No |  |  | ☐ Asymchem R&D Lab:N/A ☒ Asymchem Production data, Lot Number: CPo140020-02-01-01~04 ☐ Supplier data: N/A ☒ *Handbook & Page: The Pilot Plant Real Book & Page 10-22, 10-23 ☒ Other: C-276 exhibits excellent corrosion resistance to various organic acids, inorganic acids, alkalis and salts. The material demonstrates good corrosion resistance, including tolerance to organic solvents.The solvents are neutral, and the material compatibility is confirmed. |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ☐ High ☐ Medium ☒ Low |  |  | According to The Pilot Plant Real Book Page 10-22&10-23, piperidine is compatible with SS, 306 SS and PTFE. Based on Asymchem previous campaign production data and experience |  |  |  |
| 18 |  |  | Fmoc-L-Glu(OtBu)-OH·H2O coupling: 1. Heat the mixture to 25±5 ℃. Start timing when temperature reaches 20 ℃. The mixture is allowed to react at 25±5℃ (target: 25 ℃). 2. After 6 h±15 min (target：6 h), sample the mixture every 0.5~1 h for bromophenol blue analysis until two consecutive samples test as negative (Yellow to Orange Report Negative = Complete coupling; Blue to Black Positive = Incomplete coupling) and the mixture should be stirred for coupling time (8 h ≤Coupling Time≤18 h). 3. Stop stirring and turn off the nitrogen. Drain until there is no reaction mixture drip. |  |  | Reaction vessel material: Hastelloy (C-276) Agitator material: Hastelloy (N10276) Sieve plate: HC-22 Nitrogen distribution piping: Hastelloy (C-276) Sight glass: Glass Transfer pump material: SS316+PTFE, Thermometer well material: Hastelloy (C-276) Pipeline material: 316L Sealing ring: PTFE Mass flow meter: SS Valve material: 304 & 316L Gasket Material: PTFE Waste discharge pump material: SS316+PTFE Mechanical seal: 304SS & PTFE & FFKM Gloves: CSPE |  |  |  |  | No |  |  |  | No |  |  | ☒Yes |  |  | ☐ No |  |  | ☐ Asymchem R&D Lab: N/A ☒ Asymchem Production data, Lot Number: CPo140020-02-01-01~04 ☐ Supplier data: N/A ☒ *Handbook & Page: The Pilot Plant Real Book Page 10-14&10-15 ☒ Other: C-276 exhibits excellent corrosion resistance to various organic acids, inorganic acids, alkalis and salts.The material demonstrates good corrosion resistance, including tolerance to organic solvents.The solvents are neutral, and the material compatibility is confirmed. |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ☐ High ☐ Medium ☒ Low |  |  | According to The Pilot Plant Real Book Page 10-14&10-15, Dimethylformamide is compatible with SS, 306 SS and PTFE. Based on Asymchem previous campaign production data and experience |  |  |  |
| 19 |  |  | PreOrn11 Post deFmoc washing 1. Add 10.0 V Isopropyl Acetate (8.730 kg/kg) into TJ4S-1213-Sy15. Bubble with nitrogen for 5±2 min, then start the stirrer. 2. After stirring for 5±2 min, stop stirring, turn off the nitrogen. Drain until there is no obvious drip. 3. Perform this operation 10 times, Starting with the 10th wash, sample the isopropyl acetate washing solution during stirring and develop with chloranil, compare the color with the standard 500 ppm Piperidine in Isopropyl Acetate as reference solution. |  |  | Reaction vessel material: Hastelloy (C-276) Agitator material: Hastelloy (N10276) Sieve plate: HC-22 Nitrogen distribution piping: Hastelloy (C-276) Sight glass: Glass Transfer pump material: SS316+PTFE, Thermometer well material: Hastelloy (C-276) Pipeline material: 316L Sealing ring: PTFE Mass flow meter: SS Valve material: 304 & 316L Gasket Material: PTFE Waste discharge pump material: SS316+PTFE Mechanical seal: 304SS & PTFE & FFKM Gloves: CSPE |  |  |  |  | No |  |  |  | No |  |  | ☒Yes |  |  | ☐ No |  |  | ☐ Asymchem R&D Lab: N/A ☒ Asymchem Production data, Lot Number: CPo140020-02-01-01~04 ☐ Supplier data: N/A ☒ *Handbook & Page: The Pilot Plant Real Book Page 10-14&10-15 ☒ Other: C-276 exhibits excellent corrosion resistance to various organic acids, inorganic acids, alkalis and salts.The material demonstrates good corrosion resistance, including tolerance to organic solvents.The solvents are neutral, and the material compatibility is confirmed. |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ☐ High ☐ Medium ☒ Low |  |  | According to The Pilot Plant Real Book Page 10-14&10-15, Dimethylformamide is compatible with SS, 306 SS and PTFE. Based on Asymchem previous campaign production data and experience |  |  |  |
| 20 |  |  | Orn11 Wash resin 1. Add 10.0 v IPAC (8.730kg/kg) into TJ4S-1213-Sy15. Bubble with nitrogen for 5±2 min, then start the stirrer. 2. After stirring for 10±2 min, stop stirring, turn off the nitrogen. 3. Drain until there is no obvious drip. Perform this operation 5 times. |  |  | Reaction vessel material: Hastelloy (C-276) Agitator material: Hastelloy (N10276) Sieve plate: HC-22 Nitrogen distribution piping: Hastelloy (C-276) Sight glass: Glass Transfer pump material: SS316+PTFE, Thermometer well material: Hastelloy (C-276) Pipeline material: 316L Sealing ring: PTFE Mass flow meter: SS Valve material: 304 & 316L Gasket Material: PTFE Waste discharge pump material: SS316+PTFE Mechanical seal: 304SS & PTFE & FFKM Gloves: CSPE |  |  |  |  | No |  |  |  | No |  |  | ☒Yes |  |  | ☐ No |  |  | ☐ Asymchem R&D Lab: N/A ☒ Asymchem Production data, Lot Number: CPo140020-02-01-01~04 ☐ Supplier data: N/A ☒ *Handbook & Page: QM01020& Page 5/11 ☒ Other: C-276 exhibits excellent corrosion resistance to various organic acids, inorganic acids, alkalis and salts. The material demonstrates good corrosion resistance, including tolerance to organic solvents. The solvents are neutral, and the material compatibility is confirmed. |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ☐ High ☐ Medium ☒ Low |  |  | According to QM01020& Page 5/11 Isopropyl Acetate is compatible with SS, 306 SS and PTFE. Based on Asymchem previous campaign production data and experience |  |  |  |
| 21 |  |  | Wash resin 1. Add 10.0 V Toluene (8.720 kg/kg) into TJ4S-1213-Sy15 by insert tube and diaphragm pump. Bubble with nitrogen for 5±2 min, then start the stirrer. After stirring for 25±5 min, stop stirring, turn off the nitrogen. Drain until there is no obvious drip. 2. Add 10.0 V MTBE (7.410 kg/kg) into TJ4S-1213-Sy15. Bubble with nitrogen for 5±2 min, then start the stirrer. After stirring for 15±5 min, stop stirring, turn off the nitrogen. Drain until there is no obvious drip. 3. Add 10.0 V MTBE (7.410 kg/kg) into TJ4S-1213-Sy15. Bubble with nitrogen for 5±2 min, then start the stirrer. After stirring for 15±5 min, stop stirring, turn off the nitrogen. Drain until there is no obvious drip. |  |  | Reaction vessel material: Hastelloy (C-276) Agitator material: Hastelloy (N10276) Sieve plate: HC-22 Nitrogen distribution piping: Hastelloy (C-276) Sight glass: Glass Transfer pump material: SS316+PTFE, Thermometer well material: Hastelloy (C-276) Pipeline material: 316L Sealing ring: PTFE Mass flow meter: SS Valve material: 304 & 316L Gasket Material: PTFE Waste discharge pump material: SS316+PTFE Mechanical seal: 304SS & PTFE & FFKM Gloves: CSPE |  |  |  |  | No |  |  |  | No |  |  | ☒Yes |  |  | ☒ No |  |  | ☐ Asymchem R&D Lab: N/A ☒ Asymchem Production data, Lot Number: CPo140020-02-01-02 ☐ Supplier data: N/A ☒ *Handbook & Page: QM01020& Page 5/11 ☒ Other: C-276 exhibits excellent corrosion resistance to various organic acids, inorganic acids, alkalis and salts.The material demonstrates good corrosion resistance, including tolerance to organic solvents.The solvents are neutral, and the material compatibility is confirmed. |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ☐ High ☐ Medium ☒ Low |  |  | According to QM01020 & Page 5/11 Methyl tert-Butyl Ether is compatible with SS, 306 SS and PTFE. Based on Asymchem previous campaign production data and experience Methyl tert-Butyl Ether is also compatible with Glass, HSS and SS |  |  |  |
| 22 |  | Wash resin 1. Add 10.0 V Toluene (8.720 kg/kg) into TJ4S-1213-Sy15 by insert tube and diaphragm pump. Bubble with nitrogen for 5±2 min, then start the stirrer. After stirring for 25±5 min, stop stirring, turn off the nitrogen. Drain until there is no obvious drip. 2. Add 10.0 V MTBE (7.410 kg/kg) into TJ4S-1213-Sy15. Bubble with nitrogen for 5±2 min, then start the stirrer. After stirring for 15±5 min, stop stirring, turn off the nitrogen. Drain until there is no obvious drip. 3. Add 10.0 V MTBE (7.410 kg/kg) into TJ4S-1213-Sy15. Bubble with nitrogen for 5±2 min, then start the stirrer. After stirring for 15±5 min, stop stirring, turn off the nitrogen. Drain until there is no obvious drip. 4. Sample the MTBE washing solution during the second stirring and develop with chloranil, compare the color with the standard 500 ppm piperidine in MTBE solution as reference solution. |  |  | Reaction vessel material: Hastelloy (C-276) Agitator material: Hastelloy (N10276) Sieve plate: HC-22 Nitrogen distribution piping: Hastelloy (C-276) Sight glass: Glass Transfer pump material: SS316+PTFE, Thermometer well material: Hastelloy (C-276) Pipeline material: 316L Sealing ring: PTFE Mass flow meter: SS Valve material: 304 & 316L Gasket Material: PTFE Waste discharge pump material: SS316+PTFE Mechanical seal: 304SS & PTFE & FFKM Gloves: CSPE |  |  | No |  |  |  | No |  |  |  | ☒Yes |  |  | ☐ No |  |  | ☐ Asymchem R&D Lab: N/A ☒ Asymchem Production data, Lot Number: CPo140020-02-01-01~04 ☒ Supplier data: QM01020& Page 5/11 ☐ *Handbook & Page: N/A ☒ Other: C-276 exhibits excellent corrosion resistance to various organic acids, inorganic acids, alkalis and salts. The material demonstrates good corrosion resistance, including tolerance to organic solvents. The solvents are neutral, and the material compatibility is confirmed. |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ☐ High ☐ Medium ☒ Low |  |  | According to QM01020& Page 5/11 Isopropyl Acetate is compatible with SS, 306 SS and PTFE. Based on Asymchem previous campaign production data and experience Isopropyl Acetate and DMF are also compatible with Glass, PTFE and SS |  |  |  |  |  |
| 23 |  |  |  | Drying: 1. Sweep with nitrogen above, nitrogen flow（11~15 m3/h target: 14 m3/h） and vacuum underneath at T≤30 ℃ by jacket. 2. After drying for 16 h±2 h, sample the solid for LOD analysis every 6±4 h until LOD≤10.0%. Note: After drying 6 h, begin to stir the solid for 2~5 min at a speed of 7±2 rpm (reference) every 4±1 h. |  |  | Reaction vessel material: Hastelloy (C-276) Agitator material: Hastelloy (N10276) Sieve plate: HC-22 Nitrogen distribution piping: Hastelloy (C-276) Sight glass: Glass Transfer pump material: SS316+PTFE, Thermometer well material: Hastelloy (C-276) Pipeline material: 316L Sealing ring: PTFE Mass flow meter: SS Valve material: 304 & 316L Gasket Material: PTFE Waste discharge pump material: SS316+PTFE Mechanical seal: 304SS & PTFE & FFKM Gloves: CSPE |  | No |  |  |  | No |  |  |  | ☒Yes |  |  | ☐ No |  |  | ☐ Asymchem R&D Lab: N/A ☒ Asymchem Production data, Lot Number: CPo140020-02-01-01~04 ☒ Supplier data: QM01020 & Page 5/11 ☐ *Handbook & Page: N/A ☒ Other: C-276 exhibits excellent corrosion resistance to various organic acids, inorganic acids, alkalis and salts. The material demonstrates good corrosion resistance, including tolerance to organic solvents. The solvents are neutral, and the material compatibility is confirmed. |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ☐ High ☐ Medium ☒ Low |  |  | According to QM01020& Page 5/11 Isopropyl Acetate is compatible with SS, 306 SS and PTFE. Based on Asymchem previous campaign production data and experience Isopropyl Acetate and DMF are also compatible with Glass, PTFE and SS |  |  |  |  |

*Chemical Engineering Handbook refers to < THE PILOT PLANT REAL BOOK A. Unique Handbook1 for the Chemical Process Industry Francis X. McConville>

Table 8-5: Equipment Compatibility and Corrosion Impact Study for Activation Vessel (TJ4S-1213-Sy15-R01)

| Process steps |  |  |  |  |  | Reaction & Work-up & Concentration & Crystallization |  |  |  |  |  |  |  |  | Equipment Codes |  |  |  |  | TJ4S-1213-Sy15-R01 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S/N |  |  | Process description (indicate the equipment number, solvent/regent/material name) |  |  | Equipment (Accessories) Name and its MOC |  |  | Corrosion Probability |  |  | Corrosion Impact |  |  | Chemical Compatible, Risk and Rational |  |  |  |  | Data Source |
| 1 |  | Pre-treatment 1. Add DMF (200.00kg-300.00 kg) into activation tank (TJ4S-1213-Sy15-R01). Then transfer the solution into synthesis reactor (TJ4S-1213-Sy15). 2. Start the stirrer, bubble with nitrogen for 5±2 min, then stop the stirrer and nitrogen, drain. Sample the mixture for Water content analysis. Note: In all the following operations, unless otherwise noted. The agitation speed of synthesis reactor is set as 20±10 rpm, the agitation speed of activation tank is set as 95±10 rpm, and the nitrogen flow rate is set to 5±2 m³/h. |  |  | Tank material: Hastelloy (C-276) Agitator material: Hastelloy (C-276) Sight glass: Glass Thermometer well material: Hastelloy (C-276) Pipeline material: 316L Sealing ring: PTFE Mass flow meter: SS Valve material: 304 & 316L Waste discharge pump material: 316SS Peristaltic pump tubing: PTFE Mechanical seal: 304SS & PTFE & FFKM Gloves: CSPE |  |  | No |  |  | No |  |  | ☒Yes |  | ☐ No |  |  | ☐ Asymchem R&D Lab: N/A ☒ Asymchem Production data, Lot Number: CPo140020-02-01-02 ☐ Supplier data: N/A ☒ *Handbook & Page: The Pilot Plant Real Book page10-14, 10-15 ☒ Other: C-276 exhibits excellent corrosion resistance to various organic acids, inorganic acids, alkalis and salts.The material demonstrates good corrosion resistance, including tolerance to organic solvents.The solvents are neutral, and the material compatibility is confirmed. |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | ☐ High ☐Medium ☒ Low |  | According to The Pilot Plant Real Book Page 10-14&10-15, Dimethylformamide is compatible with SS, 306 SS and PTFE; Based on Asymchem previous campaign production data and experience Dimethylformamide is also compatible with Glass, HSS and SS |  |  |  |  |
| 2 |  | Fmoc-Gly-OH dissolve: 1. Add 4.25 V DMF (4.029 kg/kg) into the TJ4S-1213-Sy15-R01. Start the stirrer. 2. Then add 2.25 eq Fmoc-Gly-OH. Rinse reactor wall with 1.0 V DMF (0.948 kg/kg). Stir for at least 15 min. |  |  | Tank material: Hastelloy (C-276) Agitator material: Hastelloy (C-276) Sight glass: Glass Thermometer well material: Hastelloy (C-276) Pipeline material: 316L Sealing ring: PTFE Mass flow meter: SS Valve material: 304 & 316L Waste discharge pump material: 316SS Peristaltic pump tubing: PTFE Mechanical seal: 304SS & PTFE & FFKM Gloves: CSPE |  |  | No |  |  | No |  |  | ☒Yes |  | ☐ No |  |  | ☐ Asymchem R&D Lab: N/A ☒ Asymchem Production data, Lot Number: CPo140020-02-01-02 ☐ Supplier data: N/A ☒ *Handbook & Page: The Pilot Plant Real Book Page& 10-14,10-15 ☒ Other: C-276 exhibits excellent corrosion resistance to various organic acids, inorganic acids, alkalis and salts.The material demonstrates good corrosion resistance, including tolerance to organic solvents.The solvents are neutral, and the material compatibility is confirmed. |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | ☒ High ☐ Medium ☒ Low |  | According to The Pilot Plant Real Book Page 10-14&10-15, Dimethylformamide is compatible with SS and PTFE; Based on Asymchem previous campaign production data and experience Dimethylformamide is also compatible with Glass, HSS and SS |  |  |  |  |
| 3 |  | Gly24 ~ Leu12, including Ac2O/Pyridine capping Wash resin 1. Add 8.0 V DMF (7.584 kg/kg) into TJ4S-1213-Sy15-R01. Bubble with nitrogen for 5±2 min, then start the stirrer. 2. After stirring for 5±2 min, stop stirring, turn off the nitrogen. Then Transfer the mixture into TJ4S-1213-Sy15. Bubble with nitrogen for 5±2 min, then start the stirrer. 3. After stirring for 7±2 min, stop stirring, turn off the nitrogen. Drain until there is no obvious drip. Perform this operation 3 times. |  |  | Tank material: Hastelloy (C-276) Agitator material: Hastelloy (C-276) Sight glass: Glass Thermometer well material: Hastelloy (C-276) Pipeline material: 316L Sealing ring: PTFE Mass flow meter: SS Valve material: 304 & 316L Waste discharge pump material: 316SS Peristaltic pump tubing: PTFE Mechanical seal: 304SS & PTFE & FFKM Gloves: CSPE |  |  | No |  |  | No |  |  | ☒Yes |  | ☐ No |  |  | ☐ Asymchem R&D Lab: N/A ☒ Asymchem Production data, Lot Number: CPo140020-02-01-02 ☐ Supplier data: N/A ☒ *Handbook & Page: The Pilot Plant Real Book Page &10-14, 10-15 ☒ Other: C-276 exhibits excellent corrosion resistance to various organic acids, inorganic acids, alkalis and salts.The material demonstrates good corrosion resistance, including tolerance to organic solvents.The solvents are neutral, and the material compatibility is confirmed. |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | ☐ High ☐ Medium ☒ Low |  | According to The Pilot Plant Real Book Page 10-14&10-15, DMF is compatible with SS and PTFE; Based on Asymchem previous campaign production data and experience Dimethylformamide is also compatible with Glass, HSS and SS |  |  |  |  |
| 4 |  | Fmoc-L-Phe-OH activation and transfer: 1. Add 4.0 V DMF (3.792 kg/kg) into TJ4S-1213-Sy15-R01. Start the stirrer. Then add 2.0 eq. Fmoc-L-Phe-OH. Stir for at least 15 min. 2. Cool the mixture to 5±5 ℃. Maintain for next operation at 5±5 ℃ (target: 5 ℃). Add 2.0 eq. PyBOP and 2.0 eq. OxymaPure into TJ4S-1213-Sy15-R01 at 5±5 ℃ (target: 5 ℃), rinse the TJ4S-1213-Sy15-R01 with 1.0 V DMF (0.948 kg/kg). Add 3.0 eq. DIPEA into TJ4S-1213-Sy15-R01, the addition rate depends on temperature control. Activate for 20±5 min at 5±5 ℃ (target: 5℃). 3. Add 2.0 V DMF (1.896 kg/kg) into TJ4S-1213-Sy15, Start the agitator. 4. Transfer the mixture in TJ4S-1213-Sy15-R01 into TJ4S-1213-Sy15. Add 1.0 V DMF (0.948 kg/kg) into TJ4S-1213-Sy15-R01 to rinse the wall. Bubble with nitrogen to the synthesis reactor (TJ4S-1213-Sy15) for 5±2 min, Transfer the mixture in activation tank (TJ4S-1213-Sy15-R01) into synthesis reactor (TJ4S-1213-Sy15) |  |  | Tank material: Hastelloy (C-276) Agitator material: Hastelloy (C-276) Sight glass: Glass Thermometer well material: Hastelloy (C-276) Pipeline material: 316L Sealing ring: PTFE Mass flow meter: SS Valve material: 304 & 316L Waste discharge pump material: 316SS Peristaltic pump tubing: PTFE Mechanical seal: 304SS & PTFE & FFKM Gloves: CSPE |  |  | No |  |  | No |  |  | ☒Yes |  | ☐ No |  |  | ☐ Asymchem R&D Lab: N/A ☒ Asymchem Production data, Lot Number: CPo140020-02-01-02 ☐ Supplier data: N/A ☒ *Handbook & Page: The Pilot Plant Real Book Page & 10-14, 10-15 ☒ Other: C-276 exhibits excellent corrosion resistance to various organic acids, inorganic acids, alkalis and salts.The material demonstrates good corrosion resistance, including tolerance to organic solvents.The solvents are neutral, and the material compatibility is confirmed. |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | ☐ High ☐ Medium ☒ Low |  | According to The Pilot Plant Real Book Page 10-14&10-15, DMF is compatible with SS and PTFE; Based on Asymchem previous campaign production data and experience DIPEA is also compatible with Glass, HSS and SS |  |  |  |  |
| 5 |  | Fmoc-L-N-Me-Asn(Trt)-OH activation and transfer: 1. Add 4 V DMF (3.792 kg/kg) into TJ4S-1213-Sy15-R01. Start the stirrer. Then add 1.75 eq. Fmoc-L-N-Me-Asn(Trt)-OH. Stir for at least 15 min. 2. Adjust the mixture to 20±5 ℃. Add 1.80 eq. Oxyma Pure into TJ4S-1213-Sy15-R01 at 20±5 ℃ (target: 20 ℃), rinse the TJ4S-1213-Sy15-R01 with 2.0 V DMF (1.896 kg/kg). Add 2.0 eq. DIC into TJ4S-1213-Sy15-R01, the addition rate depends on temperature control. Activate for 20±5 min at 20±5 ℃ (target: 20 ℃). 3. Transfer the mixture in TJ4S-1213-Sy15-R01 into TJ4S-1213-Sy15. Add 1.0 V DMF (0.948 kg/kg) into TJ4S-1213-Sy15-R01 to rinse the wall. Bubble with nitrogen to the synthesis reactor (TJ4S-1213-Sy15) for 5±2 min, then start the agitator. Transfer the mixture in activation tank (TJ4S-1213-Sy15-R01) into synthesis reactor (TJ4S-1213-Sy15) |  |  | Tank material: Hastelloy (C-276) Agitator material: Hastelloy (C-276) Sight glass: Glass Thermometer well material: Hastelloy (C-276) Pipeline material: 316L Sealing ring: PTFE Mass flow meter: SS Valve material: 304 & 316L Waste discharge pump material: 316SS Peristaltic pump tubing: PTFE Mechanical seal: 304SS & PTFE & FFKM Gloves: CSPE |  |  | No |  |  | No |  |  | ☒Yes |  | ☐ No |  |  | ☐ Asymchem R&D Lab: N/A ☒ Asymchem Production data, Lot Number: CPo140020-02-01-02 ☐ Supplier data: N/A ☒ *Handbook & Page: N/A ☒ Other: C-276 exhibits excellent corrosion resistance to various organic acids, inorganic acids, alkalis and salts.The material demonstrates good corrosion resistance, including tolerance to organic solvents.The solvents are neutral, and the material compatibility is confirmed. |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | ☐ High ☐ Medium ☒ Low |  | According to The Pilot Plant Real Book Page 10-14&10-15, DMF is compatible with SS and PTFE; Based on Asymchem previous campaign production data and experience DIC is also compatible with Glass, HSS and SS |  |  |  |  |
| 6 |  | Fmoc-L-Asn(Trt)-OH activation and transfer: 1. Add 4.0 V DMF (3.792 kg/kg) into TJ4S-1213-Sy15-R01. Start the stirrer. Then add 2.5 eq. Fmoc-L-Asn(Trt)-OH. Stir for at least 15 min. 2. Adjust the mixture to 20±5 ℃. Maintain for next operation at 20±5℃ (target: 20℃). Add 2.6 eq. Oxyma Pure into TJ4S-1213-Sy15-R01 at 20±5 ℃ (target: 20 ℃), rinse the TJ4S-1213-Sy15-R01 with 2.0V DMF (1.896 kg/kg). Add 2.9 eq. DIC into TJ4S-1213-Sy15-R01, the addition rate depends on temperature control. Activate for 20±5 min at 20±5 ℃ (target: 20 ℃). 3. Transfer the mixture in TJ4S-1213-Sy15-R01 into TJ4S-1213-Sy15. Add 1.0 V DMF (0.948 kg/kg) into TJ4S-1213-Sy15-R01 to rinse the wall. Bubble with nitrogen to the synthesis reactor (TJ4S-1213-Sy15) for 5±2 min, then start the agitator. Transfer the mixture in activation tank (TJ4S-1213-Sy15-R01) into synthesis reactor (TJ4S-1213-Sy15) |  |  | Tank material: Hastelloy (C-276) Agitator material: Hastelloy (C-276) Sight glass: Glass Thermometer well material: Hastelloy (C-276) Pipeline material: 316L Sealing ring: PTFE Mass flow meter: SS Valve material: 304 & 316L Waste discharge pump material: 316SS Peristaltic pump tubing: PTFE Mechanical seal: 304SS & PTFE & FFKM Gloves: CSPE |  |  | No |  |  | No |  |  | ☒Yes |  | ☐ No |  |  | ☐ Asymchem R&D Lab: N/A ☒ Asymchem Production data, Lot Number: CPo140020-02-01-02 ☐ Supplier data: N/A ☒ *Handbook & Page: N/A ☒ Other: C-276 exhibits excellent corrosion resistance to various organic acids, inorganic acids, alkalis and salts.The material demonstrates good corrosion resistance, including tolerance to organic solvents.The solvents are neutral, and the material compatibility is confirmed. |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | ☐ High ☐ Medium ☒ Low |  | According to The Pilot Plant Real Book Page 10-14&10-15, DMF is compatible with SS and PTFE; Based on Asymchem previous campaign production data and experience Dimethylformamide is also compatible with Glass, HSS and SS |  |  |  |  |
| 7 |  | Ser20 ~ Ser21 Fmoc-L-Ser(tBu)-OH activation and transfer 1. Add 5.0 V DMF (4.740 kg/kg) into TJ4S-1213-Sy15-R01. Start the stirrer. Then add 2.0 eq. Fmoc-L-Ser(tBu)-OH Stir for at least 15 min. 2. Adjust the mixture to 20±5℃. Add 2.0 eq OxymaPure into TJ4S-1213-Sy15-R01 at 20±5 ℃ (target: 20℃), rinse the TJ4S-1213-Sy15-R01 with 2.0 V DMF (1.896 kg/kg). Add 2.2 eq. DIC into TJ4S-1213-Sy15-R01, the addition rate depends on temperature control. Activate for 20±5 min at 20±5℃ (target: 20 ℃). 3. Transfer the mixture in TJ4S-1213-Sy15-R01 into TJ4S-1213-Sy15. Add 1.0 V DMF (0.948 kg/kg) into TJ4S-1213-Sy15-R01 to rinse the wall. Bubble with nitrogen to the synthesis reactor (TJ4S-1213-Sy15) for 5±2 min, then start the agitator. Transfer the mixture in activation tank (TJ4S-1213-Sy15-R01) into synthesis reactor (TJ4S-1213-Sy15) |  |  | Tank material: Hastelloy (C-276) Agitator material: Hastelloy (C-276) Sight glass: Glass Thermometer well material: Hastelloy (C-276) Pipeline material: 316L Sealing ring: PTFE Mass flow meter: SS Valve material: 304 & 316L Waste discharge pump material: 316SS Peristaltic pump tubing: PTFE Mechanical seal: 304SS & PTFE & FFKM Gloves: CSPE |  |  | No |  |  | No |  |  | ☒Yes |  | ☐ No |  |  | ☐ Asymchem R&D Lab: N/A ☒ Asymchem Production data, Lot Number: CPo140020-02-01-02 ☐ Supplier data: N/A ☒ *Handbook & Page: page10-14～10-15 ☒ Other: C-276 exhibits excellent corrosion resistance to various organic acids, inorganic acids, alkalis and salts.The material demonstrates good corrosion resistance, including tolerance to organic solvents.The solvents are neutral, and the material compatibility is confirmed. |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | ☐ High ☐ Medium ☒ Low |  | According to The Pilot Plant Real Book Page 10-14&10-15, DMF is compatible with SS and PTFE; Based on Asymchem previous campaign production data and experience Dimethylformamide is also compatible with Glass, HSS and SS |  |  |  |  |
| 8 | Fmoc-L-Arg(Pbf)-OH activation and transfer： 1. Add 5.0 V DMF (4.740 kg/kg) into TJ4S-1213-Sy15-R01. Start the stirrer. Then add 2.0 eq Fmoc-L-Arg(Pbf)-OH. Stir for at least 15 min. 2. Adjust the mixture to 20±5 ℃. Maintain for next operation at 20±5 ℃ (target: 20 ℃). Add 2.0 eq. Oxyma Pure into TJ4S-1213-Sy15-R01 at 20±5 ℃ (target: 20℃), rinse the TJ4S-1213-Sy15-R01 with 2.0 V DMF (1.896 kg/kg). Add 2.2 eq. DIC into TJ4S-1213-Sy15-R01, the addition rate depends on temperature control. Activate for 20±5 min at 20±5℃ (target: 20 ℃). 3. Transfer the mixture in TJ4S-1213-Sy15-R01 into TJ4S-1213-Sy15. Add 1.0 V DMF (0.948 kg/kg) into TJ4S-1213-Sy15-R01 to rinse the wall. Bubble with nitrogen to the synthesis reactor (TJ4S-1213-Sy15) for 5±2 min, then start the agitator. Transfer the mixture in activation tank (TJ4S-1213-Sy15-R01) into synthesis reactor (TJ4S-1213-Sy15) |  |  | Tank material: Hastelloy (C-276) Agitator material: Hastelloy (C-276) Sight glass: Glass Thermometer well material: Hastelloy (C-276) Pipeline material: 316L Sealing ring: PTFE Mass flow meter: SS Valve material: 304 & 316L Waste discharge pump material: 316SS Peristaltic pump tubing: PTFE Mechanical seal: 304SS & PTFE & FFKM Gloves: CSPE |  |  | No |  |  | No |  |  | ☒Yes |  |  |  | ☐ No |  | ☐ Asymchem R&D Lab: N/A ☒ Asymchem Production data, Lot Number: CPo140020-02-01-02 ☐ Supplier data: N/A ☒ *Handbook & Page: page10-14～10-15 ☒ Other: C-276 exhibits excellent corrosion resistance to various organic acids, inorganic acids, alkalis and salts.The material demonstrates good corrosion resistance, including tolerance to organic solvents.The solvents are neutral, and the material compatibility is confirmed. |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  | ☐ High ☐ Medium ☒ Low |  |  |  | According to The Pilot Plant Real Book Page 10-14&10-15, DMF is compatible with SS and PTFE; Based on Asymchem previous campaign production data and experience Dimethylformamide is also compatible with Glass, HSS and SS |  |  |  |
| 9 |  |  | Fmoc-L-Val-OH activation and transfer： 1. Add 5.0 V DMF (4.740 kg/kg) into TJ4S-1213-Sy15-R01. Start the stirrer. Then add 2.0 eq. Fmoc-L-Val-OH. Stir for at least 15 min. 2. Adjust the mixture to 20±5 ℃. Maintain for next operation at 20±5 ℃ (target: 20 ℃). Add 2.0 eq. Oxyma Pure into TJ4S-1213-Sy15-R01 at 20±5 ℃ (target: 20 ℃), rinse the TJ4S-1213-Sy15-R01 with 2.0 V DMF (1.896 kg/kg). Add 2.2 eq. DIC into TJ4S-1213-Sy15-R01, the addition rate depends on temperature control. Activate for 20±5 min at 20±5 ℃ (target: 20 ℃). 3. Transfer the mixture in TJ4S-1213-Sy15-R01 into TJ4S-1213-Sy15. Add 1.0 V DMF (0.948 kg/kg) into TJ4S-1213-Sy15-R01 to rinse the wall. Bubble with nitrogen to the synthesis reactor (TJ4S-1213-Sy15) for 5±2 min, then start the agitator. Transfer the mixture in activation tank (TJ4S-1213-Sy15-R01) into synthesis reactor (TJ4S-1213-Sy15) |  |  | Tank material: Hastelloy (C-276) Agitator material: Hastelloy (C-276) Sight glass: Glass Thermometer well material: Hastelloy (C-276) Pipeline material: 316L Sealing ring: PTFE Mass flow meter: SS Valve material: 304 & 316L Waste discharge pump material: 316SS Peristaltic pump tubing: PTFE Mechanical seal: 304SS & PTFE & FFKM Gloves: CSPE |  |  | No |  |  | No |  |  | ☒Yes |  |  | ☐ No |  | ☐ Asymchem R&D Lab: N/A ☒ Asymchem Production data, Lot Number: CPo140020-02-01-02 ☐ Supplier data: N/A ☒ *Handbook & Page: page10-14～10-15 ☒ Other: C-276 exhibits excellent corrosion resistance to various organic acids, inorganic acids, alkalis and salts.The material demonstrates good corrosion resistance, including tolerance to organic solvents.The solvents are neutral, and the material compatibility is confirmed. |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ☐ High ☐ Medium ☒ Low |  |  | According to The Pilot Plant Real Book Page 10-14&10-15, DMF is compatible with SS and PTFE; Based on Asymchem previous campaign production data and experience Dimethylformamide is also compatible with Glass, HSS and SS |  |  |
| 10 |  |  | Fmoc-L-Leu-OH activation and transfer： 1. Add 5.0 V DMF (4.740 kg/kg) into TJ4S-1213-Sy15-R01. Start the stirrer. Then add 2.0 eq. Fmoc-L-Leu-OH. Stir for at least 15 min. 2. Adjust the mixture to 20±5 ℃. Maintain for next operation at 20±5 ℃ (target: 20 ℃). Add 2.0 eq. OxymaPure into TJ4S-1213-Sy15-R01 at 20±5 ℃ (target: 20 ℃), rinse the TJ4S-1213-Sy15-R01 with 2.0 V DMF (1.896 kg/kg). Add 2.2 eq. DIC into TJ4S-1213-Sy15-R01, the addition rate depends on temperature control. Activate for 20±5 min at 20±5 ℃ (target: 20 ℃). 3. Transfer the mixture in TJ4S-1213-Sy15-R01 into TJ4S-1213-Sy15. Add 1.0 V DMF (0.948 kg/kg) into TJ4S-1213-Sy15-R01 to rinse the wall. Bubble with nitrogen to the synthesis reactor (TJ4S-1213-Sy15) for 5±2 min, then start the agitator. Transfer the mixture in activation tank (TJ4S-1213-Sy15-R01) into synthesis reactor (TJ4S-1213-Sy15) |  |  | Tank material: Hastelloy (C-276) Agitator material: Hastelloy (C-276) Sight glass: Glass Thermometer well material: Hastelloy (C-276) Pipeline material: 316L Sealing ring: PTFE Mass flow meter: SS Valve material: 304 & 316L Waste discharge pump material: 316SS Peristaltic pump tubing: PTFE Mechanical seal: 304SS & PTFE & FFKM Gloves: CSPE |  |  | No |  |  | No |  |  | ☒Yes |  |  | ☐ No |  | ☐ Asymchem R&D Lab: N/A ☒ Asymchem Production data, Lot Number: CPo140020-02-01-02 ☐ Supplier data: N/A ☒ *Handbook & Page: page10-14～10-15 ☒ Other: C-276 exhibits excellent corrosion resistance to various organic acids, inorganic acids, alkalis and salts.The material demonstrates good corrosion resistance, including tolerance to organic solvents.The solvents are neutral, and the material compatibility is confirmed. |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ☐ High ☐ Medium ☒ Low |  |  | According to The Pilot Plant Real Book Page 10-14&10-15, DMF is compatible with SS and PTFE; Based on Asymchem previous campaign production data and experience Dimethylformamide is also compatible with Glass, HSS and SS |  |  |
| 11 |  |  | Fmoc-L-α-Me-Phe-OH activation and transfer： 1. Add 4.0 V DMF (3.792 kg/kg) into TJ4S-1213-Sy15-R01. Start the stirrer. Then add 1.75 eq. Fmoc-L-α-Me-Phe-OH. Stir for at least 15 min. 2. Adjust the mixture to 20±5 ℃. Maintain for next operation at 20±5 ℃ (target: 20 ℃). Add 1.80 eq. OxymaPure into TJ4S-1213-Sy15-R01 at 20±5 ℃ (target: 20 ℃), rinse the TJ4S-1213-Sy15-R01 with 2.0 V DMF (1.896 kg/kg). Add 2.0 eq. DIC into TJ4S-1213-Sy15-R01, the addition rate depends on temperature control. Activate for 20±5 min at 20±5 ℃ (target: 20 ℃). 3. Transfer the mixture in TJ4S-1213-Sy15-R01 into TJ4S-1213-Sy15. Add 1.0 V DMF (0.948 kg/kg) into TJ4S-1213-Sy15-R01 to rinse the wall. Bubble with nitrogen to the synthesis reactor (TJ4S-1213-Sy15) for 5±2 min, then start the agitator. Transfer the mixture in activation tank (TJ4S-1213-Sy15-R01) into synthesis reactor (TJ4S-1213-Sy15) |  |  | Tank material: Hastelloy (C-276) Agitator material: Hastelloy (C-276) Sight glass: Glass Thermometer well material: Hastelloy (C-276) Pipeline material: 316L Sealing ring: PTFE Mass flow meter: SS Valve material: 304 & 316L Waste discharge pump material: 316SS Peristaltic pump tubing: PTFE Mechanical seal: 304SS & PTFE & FFKM Gloves: CSPE |  |  | No |  |  | No |  |  | ☒Yes |  |  | ☐ No |  | ☐ Asymchem R&D Lab: N/A ☒ Asymchem Production data, Lot Number: CPo140020-02-01-02 ☐ Supplier data: N/A ☒ *Handbook & Page: page10-14～10-15 ☒ Other: C-276 exhibits excellent corrosion resistance to various organic acids, inorganic acids, alkalis and salts.The material demonstrates good corrosion resistance, including tolerance to organic solvents.The solvents are neutral, and the material compatibility is confirmed. |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ☐ High ☐ Medium ☒ Low |  |  | According to The Pilot Plant Real Book Page 10-14&10-15, DMF is compatible with SS and PTFE; Based on Asymchem previous campaign production data and experience Dimethylformamide is also compatible with Glass, HSS and SS |  |  |
| 12 |  |  | Fmoc-L-Glu(OtBu)-OH·H2O activation and transfer: 1. Add 5.0 V DMF (4.740 kg/kg) into TJ4S-1213-Sy15-R01. Start the stirrer. Then add 2.0 eq. Fmoc-L-Glu(OtBu)-OH·H2O. Stir for at least 15 min. 2. Adjust the mixture to 20±5 ℃. Maintain for next operation at 20±5 ℃ (target: 20 ℃). Add 2.0 eq. Oxyma Pure into TJ4S-1213-Sy15-R01 at 20±5 ℃ (target: 20 ℃), rinse the TJ4S-1213-Sy15-R01 with 2.0 V DMF (1.896 kg/kg). Add 2.2 eq. DIC into TJ4S-1213-Sy15-R01, the addition rate depends on temperature control. Activate for 20±5 min at 20±5 ℃ (target: 20 ℃). 3. Transfer the mixture in TJ4S-1213-Sy15-R01 into TJ4S-1213-Sy15. Add 1.0 V DMF (0.948 kg/kg) into TJ4S-1213-Sy15-R01 to rinse the wall. Bubble with nitrogen to the synthesis reactor (TJ4S-1213-Sy15) for 5±2 min, then start the agitator. Transfer the mixture in activation tank (TJ4S-1213-Sy15-R01) into synthesis reactor (TJ4S-1213-Sy15). |  |  | Tank material: Hastelloy (C-276) Agitator material: Hastelloy (C-276) Sight glass: Glass Thermometer well material: Hastelloy (C-276) Pipeline material: 316L Sealing ring: PTFE Mass flow meter: SS Valve material: 304 & 316L Waste discharge pump material: 316SS Peristaltic pump tubing: PTFE Mechanical seal: 304SS & PTFE & FFKM Gloves: CSPE |  |  | No |  |  | No |  |  | ☒Yes |  |  | ☐ No |  | ☐ Asymchem R&D Lab: N/A ☒ Asymchem Production data, Lot Number: CPo140020-02-01-02 ☐ Supplier data: N/A ☒ *Handbook & Page: page10-14～10-15 ☒ Other: C-276 exhibits excellent corrosion resistance to various organic acids, inorganic acids, alkalis and salts.The material demonstrates good corrosion resistance, including tolerance to organic solvents.The solvents are neutral, and the material compatibility is confirmed. |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ☐ High ☐ Medium ☒ Low |  |  | According to The Pilot Plant Real Book Page 10-14&10-15, DMF is compatible with SS and PTFE; Based on Asymchem previous campaign production data and experience Dimethylformamide is also compatible with Glass, HSS and SS |  |  |
| 13 |  |  | Fmoc-L-Ala-OH·H2O. activation and transfer 1. Add 5.0 V DMF (4.740 kg/kg) into TJ4S-1213-Sy15-R01. Start the stirrer. Then add 2.0 eq. Fmoc-L-Ala-OH·H2O. Stir for at least 15 min. 2. Adjust the mixture to 20±5 ℃. Maintain for next operation at 20±5 ℃ (target: 20 ℃). Add 2.0 eq. Oxyma Pure into TJ4S-1213-Sy15-R01 at 20±5 ℃ (target: 20 ℃), rinse the TJ4S-1213-Sy15-R01 with 2.0 V DMF (1.896 kg/kg). Add 2.2 eq. DIC into TJ4S-1213-Sy15-R01, the addition rate depends on temperature control. Activate for 20±5 min at 20±5 ℃ (target: 20 ℃). 3. Transfer the mixture in TJ4S-1213-Sy15-R01 into TJ4S-1213-Sy15. Add 1.0 V DMF (0.948 kg/kg) into TJ4S-1213-Sy15-R01 to rinse the wall. Bubble with nitrogen to the synthesis reactor (TJ4S-1213-Sy15) for 5±2 min, then start the agitator. Transfer the mixture in activation tank (TJ4S-1213-Sy15-R01) into synthesis reactor (TJ4S-1213-Sy15) |  |  | Tank material: Hastelloy (C-276) Agitator material: Hastelloy (C-276) Sight glass: Glass Thermometer well material: Hastelloy (C-276) Pipeline material: 316L Sealing ring: PTFE Mass flow meter: SS Valve material: 304 & 316L Waste discharge pump material: 316SS Peristaltic pump tubing: PTFE Mechanical seal: 304SS & PTFE & FFKM Gloves: CSPE |  |  | No |  |  | No |  |  | ☒Yes |  |  | ☐ No |  | ☐ Asymchem R&D Lab: N/A ☒ Asymchem Production data, Lot Number: CPo140020-02-01-02 ☐ Supplier data: N/A ☒ *Handbook & Page: page10-14～10-15 ☒ Other: C-276 exhibits excellent corrosion resistance to various organic acids, inorganic acids, alkalis and salts.The material demonstrates good corrosion resistance, including tolerance to organic solvents.The solvents are neutral, and the material compatibility is confirmed. |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ☐ High ☐ Medium ☒ Low |  |  | According to The Pilot Plant Real Book Page 10-14&10-15, DMF is compatible with SS and PTFE; Based on Asymchem previous campaign production data and experience Dimethylformamide is also compatible with Glass, HSS and SS |  |  |
| 14 |  |  | Fmoc-L- Orn(Boc)-OH activation and transfer: 1. Add 5.0 V DMF (4.740 kg/kg) into TJ4S-1213-Sy15-R01. Start the stirrer. Then add 2.0 eq. Fmoc-L-Orn(Boc)-OH (0.909 kg/mol). Stir for at least 15 min. 2. Adjust the mixture to 20±5 ℃. Maintain for next operation at 20±5 ℃ (target: 20 ℃). Add 2.0 eq. Oxyma Pure into TJ4S-1213-Sy15-R01 at 20±5 ℃ (target: 20 ℃), rinse the TJ4S-1213-Sy15-R01 with 2.0 V DMF (1.896 kg/kg). Add 2.2 eq. DIC into TJ4S-1213-Sy15-R01, the addition rate depends on temperature control. Activate for 20±5 min at 20±5 ℃ (target: 20 ℃). 3. Transfer the mixture in TJ4S-1213-Sy15-R01 into TJ4S-1213-Sy15. Add 1.0 V DMF (0.948 kg/kg) into TJ4S-1213-Sy15-R01 to rinse the wall. Bubble with nitrogen to the synthesis reactor (TJ4S-1213-Sy15) for 5±2 min, then start the agitator. Transfer the mixture in activation tank (TJ4S-1213-Sy15-R01) into synthesis reactor (TJ4S-1213-Sy15) |  |  | Tank material: Hastelloy (C-276) Agitator material: Hastelloy (C-276) Sight glass: Glass Thermometer well material: Hastelloy (C-276) Pipeline material: 316L Sealing ring: PTFE Mass flow meter: SS Valve material: 304 & 316L Waste discharge pump material: 316SS Peristaltic pump tubing: PTFE Mechanical seal: 304SS & PTFE & FFKM Gloves: CSPE |  |  | No |  |  | No |  |  | ☒Yes |  |  | ☐ No |  | ☐ Asymchem R&D Lab: N/A ☒ Asymchem Production data, Lot Number: CPo140020-02-01-02 ☐ Supplier data: N/A ☒ *Handbook & Page: page10-14～10-15 ☒ Other: C-276 exhibits excellent corrosion resistance to various organic acids, inorganic acids, alkalis and salts.The material demonstrates good corrosion resistance, including tolerance to organic solvents.The solvents are neutral, and the material compatibility is confirmed. |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ☐ High ☐ Medium ☒ Low |  |  | According to The Pilot Plant Real Book Page 10-14&10-15, DMF is compatible with SS and PTFE; Based on Asymchem previous campaign production data and experience Dimethylformamide is also compatible with Glass, HSS and SS |  |  |

*Chemical Engineering Handbook refers to < THE PILOT PLANT REAL BOOK A. Unique Handbook1 for the Chemical Process Industry Francis X. McConville>

Table 8-6: Equipment Compatibility and Corrosion Impact Study for Storage Tank TJ4S-1200-AT02

| Process steps |  | Reaction & Work-up & Concentration & Crystallization |  |  | Equipment Codes |  | TJ4S-1200-AT02 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| S/N | Process description (indicate the equipment number, solvent/regent/material name) | Equipment (Accessories) Name and its MOC | Corrosion Probability | Corrosion Impact | Chemical Compatible, Risk and Rational |  | Data Source |
| 1 | 1. Heat the mixture to 25±5 ℃. The mixture is allowed to react at 25±5 ℃ (target: 25 ℃). 2. Once the total reaction time has reached 8 h±15 min, add additional 1.0 eq. Oxyma Pure and 1.0 V DMF (0.948 kg/kg) solution (the mixture should be prepared in TJ4S-1200-AT02 just before use) and 1.1 eq. DIC. 3. After 5 h±15 min (target: 5 h). Stop stirring and turn off the nitrogen. Drain until there is no reaction mixture drip. | Inner surface, valve, temperature probe, agitator: Glass-lined Mechanical seal: GL & PTFE & FFKM | No | No | ☒Yes | ☐ No | ☐ Asymchem R&D Lab: N/A ☒ Asymchem Production data, Lot Number: CPo140020-02-01-01~04 ☐ Supplier data: N/A ☒ *Handbook & Page: The Pilot Plant Real Book Page 10-14&10-15 ☒ Other: C-276 exhibits excellent corrosion resistance to various organic acids, inorganic acids, alkalis and salts.The material demonstrates good corrosion resistance, including tolerance to organic solvents.The solvents are neutral, and the material compatibility is confirmed. |
|  |  |  |  |  | ☐ High ☐ Medium ☒ Low | According to The Pilot Plant Real Book Page 10-14&10-15, Dimethylformamide is compatible with SS, 316LPTFE.PTFE; Based on Asymchem previous campaign production data and experience, Dimethylformamide is also compatible with Glass, PTFE and SS |  |

*Chemical Engineering Handbook refers to < THE PILOT PLANT REAL BOOK A. Unique Handbook1 for the Chemical Process Industry Francis X. McConville>

Table 8-7: Equipment Compatibility and Corrosion Impact Study for TJ4S-1213-R22

| Process steps |  | Reaction & Work-up & Concentration & Crystallization |  |  | Equipment Codes |  | TJ4S-1213-R22 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| S/N | Process description (indicate the equipment number, solvent/regent/material name) |  | Equipment (Accessories) Name and its MOC |  | Corrosion Probability | Corrosion Impact |  | Chemical Compatible, Risk and Rational |  |  |  | Data Source |  |
| 1 | Prepare capping solution： 1. Add 6.0 V DMF (5.688 kg/kg) into TJ4S-1213-R22. Start the stirrer. 2. Then add 1.50 eq. Pyridine and 1.25 eq. Acetic Anhydride with PTFE and peristaltic pump. Adjust the mixture to 25±5 ℃. Maintain for next operation at 25±5 ℃ (target: 25 ℃). 3. Stir for at least 5 min after addition, Transfer the mixture in TJ4S-1213-R22 into TJ4S-1213-Sy15. Add 1.0 V DMF (0.948 kg/kg) into TJ4S-1213-R22 to rinse the wall. Bubble with nitrogen to the synthesis reactor (TJ4S-1213-Sy15) for 5±2 min, then start the agitator. Transfer the mixture in Glass-lined reactor (TJ4S-1213-R22) into synthesis reactor (TJ4S-1213-Sy15) |  | Reactor (inner surface, temperature probe, agitator, sieve plate): GL Sight glass: Glass Gasket, including manway gaskets and gaskets on all inlet valve manifolds: PTFE Mass flow meter, pipeline, transfer pump, valve: 316L Mechanical seal: GL & PTFE & FFKM Transfer pump: 304SS |  | No | No |  | ☒Yes |  | ☐ No |  | ☐ Asymchem R&D Lab: N/A ☒ Asymchem Production data, Lot Number: CPo140020-02-01-02 ☐ Supplier data: N/A ☒ *Handbook & Page: The Pilot Plant Real Book Page Page 10-10&10-11, Page 10-14&10-15, Page 10-24&10-25, ☒ Other: C-276 exhibits excellent corrosion resistance to various organic acids, inorganic acids, alkalis and salts. The material demonstrates good corrosion resistance, including tolerance to organic solvents.The solvents are neutral, and the material compatibility is confirmed. |  |
|  |  |  |  |  | ☐ High ☐ Medium ☒ Low | According to The Pilot Plant Real Book Page 10-10&10-11, Page 10-14&10-15, Page 10-24&10-25, Acetic Anhydride, Pyridine and Dimethylformamide are compatible with 316L, Glass and PTFE. Based on Asymchem previous campaign production data and experience, Dimethylformamide is also compatible with Glass, PTFE and SS |  |

*: Note: *Chemical Engineering Handbook refers to < THE PILOT PLANT REAL BOOK A. Unique Handbook1 for the Chemical Process Industry Francis X. McConville>

Conclusion

The MOC of intended equipment is compatible with the raw and process materials, no potential corrosion risk is identified.

- Suitability Assessment on Qualification Range and Accuracy of Instruments

See Table 9-1 below

Table 9-1: Suitability Assessment on Qualification Range and Accuracy of Instruments

| Equipment Code | Parameter |  | Latest Qualified Time | Instrument ID | Qualified Range | Accuracy | MOR | NOR |  | PAR | ¦Low boundary of PAR- NOR¦≥Accuracy? | High boundary of PAR- NOR ≥Accuracy? |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TJ4S-1213-SPS01-AT01/ TJ4S-1213-SPS01-AT02 | Charging quantity |  | 06/24/2026 | EB52090L | *≥400 (379.20 kg) DMF | ±3% | DMF charging amount (DMF density: 0.948 kg/L): 400~1600 L ; (379.20~1516.80 kg); Piperidine charging amount (Piperidine density: 0.862 kg/L): 100~400 L; (86.20~344.8 kg); Based on charging accuracy calculation, the concentration of Piperidine/DMF mixed solution is controlled within 19.5%~20.5%. The NOR range is 15%~20%, which complies with process requirements. |  |  | Yes | Yes |
|  | Charging quantity |  | 06/22/2026 | EB52092L | *≥100 L (86.20 kg) Piperidine | ±3% |  |  |  | Yes | Yes |
| Glovebox | Dispensing quantity |  | 09/16/2025 | 20250822183 | 0.20 kg - 60.00 kg | *±3% | 7.37~77.33 kg | TBD |  | TBD | Yes | Yes |
| TJ4S-1213-Sy15 | Mass Flow Meter (Rotary) |  | 12/10/2025 | CYJ038 | 1.5~15 m³/h | N/A | 3.0 ~15 m3/h | TBD | TBD | TBD | TBD |
|  | Pressure Transmitter |  | 12/16/2025 | PJGAU1Z1908F | -0.08 ~0.08 MPa | 0.0008 MPa | The process has no pressure requirements. The equipment pressure is controlled by the DCS system with a set pressure value of 0.005 MPa. The pressure inside the reactor is automatically regulated via valves and pressure interlocks. |  |  | TBD | TBD |
|  | Temperature Transmitter |  | 12/15/2025 | 8805TRZ8 | -5~55 ℃ | 1.4 ℃ | 20-30 ℃ |  | TBD | TBD | TBD | TBD |
|  | Mass Flow Meter*1 | DMF | 11/16/2025 | EB52400 | 20≤m<70 kg | ≤ 2 kg | DMF: 43.50 ~ 47.50 kg | TBD | TBD | TBD | TBD |
|  |  |  |  |  | M ≥ 70 kg | ≤ 3 % | DMF: 88.28 ~ 93.74 kg |  |  |  |  |
|  |  |  |  |  |  |  | DMF: 220.69 ~ 234.35 kg |  |  |  |  |
|  |  |  |  |  |  |  | DMF: 308.97 ~ 328.09 kg |  |  |  |  |  |
|  |  |  |  |  |  |  | DMF: 353.11 ~ 374.95 kg |  |  |  |  |  |
|  |  |  |  |  |  |  | DMF: 529.67 ~ 562.43 kg |  |  |  |  |  |
|  | Mass Flow Meter*2 | MTBE | 11/16/2025 | EB52374 | ≥70 kg | ≤ 3 % | 345.02 ~ 366.35 kg |  | TBD | TBD | TBD | TBD |
|  | Mass Flow Meter*3 | IPAc | 11/16/2025 | EB52398 | ≥70 kg | ≤ 3 % | 406.47 ~ 431.61 kg |  | TBD | TBD | TBD | TBD |
|  | Mass Flow Meter*4 | 20% Piperidine in DMF Solution | 11/18/2026 | EB52396 | ≥70 kg | ≤ 3 % | 346.78 ~ 368.23 kg |  | TBD | TBD | TBD | TBD |
|  | **Charging quantity |  | 10/29/2025 | 2025062708 | 2.00 ≤m < 300.00 kg | ± 3.0% | DIC: 6.98 ~ 18.40 kg | TBD | TBD | TBD | TBD |
|  | **Charging quantity |  | 10/29/2025 | 2025062708 | 2.00 ≤m < 300.00 kg | ± 3.0% | Toluene: 345.01 ~ 366.35 kg | TBD | TBD | TBD | TBD |
|  | **Charging quantity |  | 10/29/2025 | 2025062708 | 2.00 ≤m < 300.00 kg | ± 3.0% | DIPEA: 19.50~ 41.41 kg | TBD | TBD | TBD | TBD |
|  | Agitation |  | N/A | N/A | 5~40 rpm | ± 2 rpm | 10~30 rpm | TBD | TBD | TBD | TBD |
| TJ4S-1213-Sy15-R01 | Temperature transmitter |  | 12/16/2025 | CYK11543 | -25~100 ℃ | 1.725 ℃ | 5-20 ℃ | TBD |  | TBD | TBD | TBD |
|  | Pressure Transmitter |  | 12/15/2025 | CYJ11916 | 0~0.08 MPa | 0.0004 MPa | The process has no pressure requirements. The equipment pressure is controlled by the DCS system with a set pressure value of 0.005 MPa. The pressure inside the reactor is automatically regulated via valves and pressure interlocks. |  |  | TBD | TBD |
|  | Agitation |  | N/A | N/A | 15~130 rpm | ±2 rpm | 80~100 rpm | TBD |  | TBD | TBD | TBD |
| TJ4S-1213- R22 | Charging quantity |  | 06/24/2026 | EB52089L | ≥70 kg | ±3.0% | 264.83 ~ 272.77 kg | TBD |  | TBD | TBD | TBD |
|  | Agitation |  | N/A | N/A | 0~138 rpm | ±2 rpm | 95±10 rpm | TBD |  | TBD | TBD | TBD |
|  | **Charging quantity |  | 03/11/2025 | 2026011512 | 2.00 ≤m < 300.00 kg | ± 3.0% | DMF: 43.50 ~ 281.21 kg | TBD | TBD | TBD | TBD |
|  | **Charging quantity |  | 03/11/2025 | 2026011512 | 2.00 ≤m < 300.00 kg | ± 3.0% | Pyridine: 5.97~ 6.33 kg Acetic anhydride: 6.42~6.82 kg |  | TBD | TBD | TBD | TBD |
|  | Internal Temperature |  | 01/05/2026 | CYI101282 | -2.9~112.3 ℃ | ±1.35 ℃ | 20~30 ℃ | TBD |  | TBD | TBD | TBD |
|  | Pressure |  | 12/14/2025 | CYJ11903 | -0.098~0.083 MPa | ±0.0009 MPa | The process has no pressure requirements. The equipment pressure is controlled by the DCS system with a set pressure value of 0.005 MPa. The pressure inside the reactor is automatically regulated via valves and pressure interlocks. |  |  | Yes | Yes |

Note: Calculation of temperature Accuracy = Signal transform accuracy + sensor accuracy = 0.5%* range of gauge + [0.6 °C +0.010*testing temperature] (based on Grade C gauge);

Calculation of pressure Accuracy = 0.5%* range of gauge; Agitator speed accuracy = ±2r/min.

1) High boundary of PAR- High boundary of MOR ≥Accuracy，so the risk is relatively low.

2) The over temperature alarm limit for this operation is 14 ° C, Low boundary of PAR- Over temperature alarm limit ≥Accuracy，so the risk is relatively low.

Note: * Note: Weighing shall be carried out in batches, and the weighing range for each batch shall be kept within the qualified ranges. Each bag shall be subpackaged within a weight range of 1.00~25.00

** Note: The charging range each drum is between 2.00~ 170.00 kg.

Conclusion: The normal operation ranges of the process are within the qualified ranges and can meet the process requirements.

- Other Considerations

See Table 10-1~ Table 10-3 below

Table 10-1: Preventative Maintenance Review

| Equipment Code | Equipment component | MOC | Preventative Maintenance |
| --- | --- | --- | --- |
| TJ4S-1213-Sy15, TJ4S-1213-Sy15-R01 | Gasket on the manhole | PTFE | Disassembly cleaning will be performed for each batch of the equipment. After each disassembly cleaning, the integrity of the gasket on the manhole will be checked, and the relevant check results will be filled in the corresponding cleaning and checking record of the equipment. The final check shall be conducted by QA. |
|  | Mechanic seals | 316L &PTFE &FFKM(O-ring) | The equipment department conducts an overall inspection once a year. Check the wear of the moving ring, check the sealing surface, and re-adjust the tightening moving ring as necessary. |
|  | Sealing ring | Outer coated FEP core silicone rubber | The sealing ring of sealing joint between sieve plate and reactor body, sealing joint at material receiving port Visual inspection shall be conducted during each disassembly and cleaning. Pressure tightness test shall be performed after reassembly. |
|  | Gasket between reactor and valve | PTFE | Replace when equipment is disassembled for cleaning, and the relevant check results will be filled in the corresponding cleaning and checking record of the equipment. |
| TJ4S-1213-R22 | Ensure the integrity of the equipment. | GL | 1. For glass-lined equipment, the equipment department uses an enamel wall thickness check once a year. Regular inspection to the glass-lined equipment internal surface, if the GL was damage or rupture/crack, the check should also include the iron exposure detection. 2. Visual inspection of the integrity of the enamel during the cleaning. See detail in Enamel equipment inner surface stain control procedure<DS-GMP-023.01>. |
|  | Mechanical seal | GL&PTFE &FFKM(O-ring) | The equipment department conducts an overall inspection once a year. Check the wear of the moving ring, check the sealing surface, and re-adjust the tightening moving ring as necessary. |

Conclusion:

According to the Asymchem SOP required, all accessories/consumables are required to be unblemished and usable before the equipment is installed. The risk of using these accessories/consumables is assessed as low.

Table 10-2: Compatibility Matrix for Equipment MOC and Batch Mixtures

| Material Name | Compatible with C-276 | Compatible with PTFE | Compatible with SS | Compatible with Glass | Compatible with LDPE | Compatible with 316L | Compatible with HC-22 | Compatible with 304 SS | Compatible references |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reaction mixture | Yes | Yes | Yes | Yes | No contact | Yes | Yes | No contact | Based on production experience of batch CPo140020-02-01-01~04; Handbook & Page: Page 1161-1164; SOP:QM01020 |
| Activation mixture | Yes | Yes | Yes | Yes | No contact | Yes | Yes | No contact |  |
| 20% Piperidine in DMF solution | Yes | Yes | Yes | Yes | No contact | Yes | Yes | No contact | Based on production experience of batch CPo140020-02-01-01~04;; Chemical Engineering Handbook & Page: page10-14~10-15, page10-22~10-23; Handbook & Page: Page 1161-1164 SOP:QM01020 |
| Capping solution | Yes | Yes | Yes | Yes | No contact | Yes | Yes | Yes | Based on production experience of batch CPo140020-02-01-01~04; Handbook & Page: Page 41-44 & Page 2721-2724 & Page 1161-1164; SOP:QM01020 |
| ASYM-141366 intermediate | Yes | Yes | Yes | No contact | Yes | Yes | Yes | No contact | Based on production experience of batch CPo140020-02-01-01~04; |

Conclusion:

The selected MOC of accessories/components is suitable for process needs. The inner surface, temperature probe and agitator is C-276 and it is suitable for process needs. Other accessories are suitable for process needs. The risk of using the equipment is assessed as low.

Table 10-3: Compatibility Matrix for Equipment MOC and Raw Materials

| Material Name | Compatible with C-276 | Compatible with PTFE | Compatible with SS | Compatible with Glass | Compatible with LDPE | Compatible with HSS | Compatible with Polycarbonate | Compatible with PharMed polyolefin | Compatible references |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Fmoc-Gly-OH | Yes | Yes | Yes | No contact | Yes | Yes | Yes | No contact | Based on production experience of batch CPo140020-02-01-01~04; Resins, amino acids and oxyma pure are relatively stable in solid state, and do not react with materials such as LDPE, stainless steel, PTFE, PC, PP, CEPS and glass at room temperature. |
| Fmoc-L-Phe-OH | Yes | Yes | Yes | No contact | Yes | Yes | Yes | No contact |  |
| Fmoc-N-Me-Asn(Trt)-OH | Yes | Yes | Yes | No contact | Yes | Yes | Yes | No contact |  |
| Fmoc-L-Asn(Trt)-OH | Yes | Yes | Yes | No contact | Yes | Yes | Yes | No contact |  |
| Fmoc-L-Ser(tBu)-OH | Yes | Yes | Yes | No contact | Yes | Yes | Yes | No contact |  |
| Fmoc-L-Arg(Pbf)-OH | Yes | Yes | Yes | No contact | Yes | Yes | Yes | No contact |  |
| Fmoc-L-Val-OH | Yes | Yes | Yes | No contact | Yes | Yes | Yes | No contact |  |
| Fmoc-L-Leu-OH | Yes | Yes | Yes | No contact | Yes | Yes | Yes | No contact |  |
| Fmoc-L-α-Me-Phe-OH | Yes | Yes | Yes | No contact | Yes | Yes | Yes | No contact |  |
| Fmoc-L-Glu(OtBu)-OH·H2O | Yes | Yes | Yes | No contact | Yes | Yes | Yes | No contact |  |
| Fmoc-L-Ala-OH.H2O | Yes | Yes | Yes | No contact | Yes | Yes | Yes | No contact |  |
| Fmoc-Orn(Boc)-OH | Yes | Yes | Yes | No contact | Yes | Yes | Yes | No contact |  |
| DIPEA | Yes | Yes | Yes | No contact | No contact | Yes | No contact | Yes |  |
| Oxyma pure | Yes | Yes | Yes | No contact | Yes | Yes | Yes | No contact |  |
| PyBOP | Yes | Yes | Yes | No contact | Yes | Yes | Yes | No contact |  |
| DIC | Yes | Yes | Yes | Yes | No contact | Yes | No contact | No contact | Based on production experience of batch CPo140020-02-01-01~04; |
| Methanol | Yes | Yes | Yes | No contact | No contact | Yes | No contact | Yes |  |
| Acetic anhydride | Yes | Yes | Yes | No contact | No contact | Yes | No contact | Yes |  |
| DMF | Yes | Yes | Yes | Yes | No contact | Yes | No contact | Yes |  |
| IPAC | Yes | Yes | Yes | Yes | No contact | Yes | No contact | Yes |  |
| MTBE | Yes | Yes | Yes | Yes | No contact | Yes | No contact | No contact |  |
| Pyridine | Yes | Yes | No contact | No contact | No contact | No contact | No contact | Yes |  |
| Toluene | Yes | Yes | Yes | Yes | No contact | Yes | No contact | No contact |  |

Conclusion: The selected MOC of accessories/component is suitable for process needs. The risk of using these accessories/consumables is assessed as low. The Material Compatibility Assessment Report refer to TR-1400200303.01.

Peristaltic pump tubing shows poor compatibility with DIC, pyridine and acetic anhydride upon prolonged contact; however, such incompatibility risk can be mitigated under short-term contact. Therefore, the peristaltic pump tubing shall be replaced after each use. Since the material contact duration during each charging does not exceed 2 hours, short-term contact reduces the risks of material degradation and leaching, and the residual risk is low.

Conclusion

Based on above assessment, it can be concluded that the equipment train at building 12, model 13 at Asymchem TJ4 is suitable for ASYM-141366 (Step 1 of CPo140020-03) manufacture

Revision History

| Doc. No. and version | Document Changes | Justification | Effective Date |
| --- | --- | --- | --- |
| RA-140020030101.01 | N/A | The first version | The last signature date is the effective date. |