Cleaning Validation Protocol

of ASYM-141366

(Batch Size: 105.63 - 278.62 kg (Output))

At Asymchem Life Science (Tianjin) Co., Ltd.

(No.265, South Avenue, Binhai New Area TEDA, Tianjin, 300462, China)

| Prepared by |  |  |
| --- | --- | --- |
| Zhonghua Fu (Associate Director, CMMD) | Signature &Date |  |
| Reviewed by |  |  |
| Liang Li (Senior Director, CMMD) | Signature &Date |  |
| Zhongyao Zhang (Director, Production) | Signature &Date |  |
| Jiwei Cai (Director, QC) | Signature &Date |  |
| Lu Li (Director, QA) | Signature &Date |  |
| Approved by |  |  |
| Xinyi Zhu (Executive Director, CMMD) | Signature &Date |  |
| Ruijun Shi (Vice Chief Engineer, Production) | Signature &Date |  |
| Haini Wang (Senior Director, QC) | Signature &Date |  |
| Mingyang Yang (Executive Director, QA) | Signature &Date |  |
| Final Approved by |  |  |
| Shenghui Yu (Vice General Manager, QA) | Signature &Date |  |

- Revision History

| Protocol No. | Effective Date | Document Changes | Justification |
| --- | --- | --- | --- |
| CVP-1400200301.01 |  | The first version | N/A |
| Note 1: The document shall only take effect after the training is completed, and the effective date will be manually filled in by designated personnel with signature and date once confirmed |  |  |  |

`

Table of Content

Revision History	4

1.	Purpose	6

2.	Scope	6

3.	Responsibility	8

4.	Procedure support	8

5.	Cleaning Validation Strategy	8

6.	Execution procedure	13

6.1.	Summary of equipment train used in the production	13

6.2.	Pre-production equipment cleaning check	14

6.3.	Batch to batch flushing to mitigate the risk in the high risk equipment	15

6.4.	Selection of the target residue compounds	17

6.5.	Cleaning frequency	19

6.5.1	Cleaning frequency for within campaign in-situ minor cleaning	19

6.5.2	Cleaning frequency for dis-assemble major cleaning	20

6.6.	Allowable residue limit determination	21

6.6.1	Allowable residue limit for within-campaign in-situ minor cleaning	21

6.6.2	Allowable residue limit for dis-assemble major cleaning	21

6.6.3	Rationale for Exemption of Microbial/Bioburden Assessment for SPPS	22

6.7.	Equipment cleaning evaluation based on the risk analysis	23

6.8.	Cleaning method	29

6.9.	Sampling plan and method	31

6.10.	Test method validation and recovery study	33

6.11.	Dirty Hold Time Study	35

6.12.	Clean Hold Time	36

7	Product distribution plan during the cleaning validation	36

8	Impurity profile summary and impurity trending analysis	36

9	Amendment of protocol	37

10	Deviation and Change Handling	37

11	Action taken when out-of-limit result found	37

12	Summary and Conclusion	38

13	Revalidation	38

14	References	38

Annex 1: Cleaning method list	40

Annex  2: P&ID diagram for the swab and rinse sampling locations	40

Annex  3: Deviation record	40

Annex  4: Change record	40

Annex 5: The definition of abbreviation	40

- Purpose

The purpose of this cleaning validation protocol is to ensure that the cleaning program for all the material-contact equipment used for manufacturing ASYM-141366 (CPo140020-03, Step 1) are in place and that product contamination is minimized.

- Scope

This protocol is applicable for cleaning validation of the equipment used for manufacturing step 1 in-process intermediate ASYM-141366 according to the defined process (see PVP-1400200301 <Process Performance Qualification Protocol of ASYM-141366> (current version)) in the defined equipment train (Fig.1) and the defined batch size (105.63 ~ 278.62 kg) in Building 12 Module 13 at Asymchem Life Science (Tianjin) Co., Ltd (Address: No. 265, South Avenue, Binhai New Area TEDA, Tianjin, 300462, China, referred to as Asymchem TJ4). The cleaning validation will be performed following the approved cleaning method (CMa-TJ414002001<Equipment Cleaning Method > (current version), CMb-TJ414002001<Equipment Cleaning Method > (current version)). The cleaning validation will cover at least three continuous successful cleaning verification cycles.

The production building/module/equipment used for the manufacturing of the in-process intermediate ASYM-141366 are non-dedicated.

To sum up, the equipment and cleaning information involved in this Cleaning Validation Protocol is listed in Table 1. Detail evaluation of each specific part are further determined in separate section.

Table 1 The equipment and cleaning information involved in this Cleaning Validation Protocol

| Equipment name | Equipment ID | Cleaning Process | Cleaning frequency |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Batch to batch flush | In-situ minor cleaning | Disassemble major cleaning |
| Peptide Solid Phase Synthesizer | TJ4S-1213-Sy15 | Refer to CMa-TJ414002001<Equipment Cleaning Method > (current version), CMb-TJ414002001<Equipment Cleaning Method > (current version) which are evaluated during cleaning process design and development | Not applicable | Not applicable | After every batch |
|  | TJ4S-1213-Sy15-R01 |  |  |  |  |
| Glass-lined Reactor | TJ4S-1213-R22 |  | After each batch | Not applicable | After end of production |
| Agitation tank | TJ4S-1200-AT02 |  | After each batch | Not applicable | After end of production |

Fig. 1, The defined equipment train diagram and cleaning boundary

- Responsibility

The following responsibilities are assigned to the relevant departments and as listed in Table 3-1.

Table 3-1, Functions and Responsibilities

| Function | Responsible |
| --- | --- |
| Project Manager or associate director | Prepare the cleaning validation protocol and report |
| QA engineer/ manager/director | Release the equipment after satisfactory cleaning Review the cleaning validation protocol and report |
| Production Manager/ director | Organize the Cleaning and collect the rinse/reflux sample |
| Director/ senior director/Associate Chief Engineer, Production | Review the cleaning validation protocol and report |
| Analytical department | Perform the swab sampling Analysis and reporting the analytical results to QA |
| Senior Director, QC | Approve the cleaning validation protocol and report |
| Executive Director, Chemical Macromolecule Division | Approve the cleaning validation protocol and report |
| VGM, Analysis of CMMD | Approve the cleaning validation protocol and report |
| The head of Production management department / | Approve the cleaning validation protocol and report |
| The head of Quality management department | Approve the cleaning validation protocol and report |

All the employees who participate in this cleaning validation will be trained on this protocol, the corresponding cleaning method and SOPs outlined in section 4 of this protocol. Responsible personnel shall enter his/her name and sign in <Sign in table> (QF06006) and attach this table to the cleaning validation report.

- Procedure support

The corresponding SOPs are listed in Table 4-1.

Table 4-1, Corresponding SOP related to the cleaning validation, Equipment Cleaning, sampling and testing

| Title | SOP version #/SOP |
| --- | --- |
| Cleaning Validation Master Plan | CVMP-01 (current version) |
| Equipment cleaning management procedure for GMP production | DS-GMP-003 (current version) |
| Cleaning Validation | DS-GMP-043 (current version/) |
| OOS Result Investigation | QA-015 (current version/) |
| ATR result investigation | QA-032 (current version) |
| Sampling and Testing Procedure for cleaning verification | AC-053 (current version/) |
| Production Equipment Cleaning manual | QM03024 (current version) |
| Minimum lot size, rinse volume, reflux volume and internal surface area of equipment | QM05004 (current version) |
| Equipment to be swab-tested and the sampling point diagram | QM04005 (current version) |

- Cleaning Validation Strategy

According to Asymchem’s SOP <Cleaning Validation> DS-GMP-043 (current version), the life cycle of cleaning validation includes three stages:

- Stage 1: Cleaning process design and development, according to the pharmacology, toxicology, solubility and other information obtained in the process of product development, cleaning process is designed, transferred, scaled-up, and then determined. The detail information for Cleaning process design and development refer to separate documentation TR-1400200310 < LSN4182924 (ASYM-140020) Step 1 Cleaning Process Design and Development Report>.

- Stage 2: Cleaning process validation to ensure that the cleaning process developed and designed can repeatedly produce the desired results. This document Cleaning Validation Protocol CVP-1400200301.01 is intended to demonstrate that the approved cleaning procedures consistently remove process residues from product-contact equipment used in the manufacture of ASYM-141366 Step 1 and maintain equipment in a state suitable for subsequent manufacture.

- Stage 3: Continued cleaning process verification, to ensure that the key parameters and cleaning effect of the cleaning process are continuously monitored, and the cleaning effect is regularly confirmed to ensure that the cleaning process is in a controlled status. The protocol for Continued cleaning process verification will be separately raised after Cleaning process validation defined in this document during subsequent routine commercial production.

The cleaning process validation execution will be performed from the first commercial manufacturing campaign supporting Process Performance Qualification activities. Cleaning validation data will be generated from three consecutive cleaning cycles performed under routine operating conditions by trained production personnel using approved manufacturing and cleaning procedures. Successful completion of three consecutive cleaning validation runs without unexplained failures, deviations impacting cleaning effectiveness, or out-of-limit analytical results will constitute successful cleaning process qualification. For cleaning validation strategy diagram within Asymchem, see Fig. 2.

Fig. 2. General cleaning validation strategy

For definition of cleaning criteria:

The cleaning criteria for equipment at each level have been defined in DS-GMP-003<Equipment cleaning management procedure for GMP productions (DS)>. There are three levels of cleaning check, including Level 0, Level I, and Level II.

Level 0 is further classified into two scenarios: Level 0/A and Level 0/B (batch-to-batch flush).

- Level 0/A: The equipment can be successively used between batches without any treatment.

- Level 0/B (batch-to-batch flush): The equipment will be simply flushed in-between batches prior to use in the next batch per the process tolerance requirement.

Level I: Visual check on the inner surface of the equipment body after cleaning to confirm it is free of any material residue under adequate light. Equipment cleaning is performed in accordance with Equipment Cleaning Method CMb-TJ414002001 and CMa-TJ414002001. Level I visual inspection shall be carried out on equipment under dry condition upon completion of final nitrogen drying. Lighting conditions and inspection aids are specified, and designated inspection points are defined. All cleaning and inspection activities are documented in the cleaning records, which capture lighting conditions, inspection aids, designated inspection points and equipment components. The inspection results are signed off by the QA personnel.

Level II: Quantitative determination of the target residue on the equipment surface to ensure carryover of the target residue NMT Allowable/Acceptable Residue Limit (ARL) calculated based on health data. Results shall be presented in numerical form, rather than relying solely on a Pass/Fail assessment.

For cleaning frequency:

- In-situ minor cleaning

Within-campaign in-situ minor cleaning: To avoid the following batches being contaminated with residues and degradants from the previous batch, the equipment will be in-situ cleaned (un-disassemble) at defined batch frequency. This validation includes 3 rounds of cleaning and quantitative residue testing (Level I check followed by Level II quantitative test). After completion of the 3 rounds of cleaning validation, the equipment shall be cleaned per the validated cleaning method and batch frequency without extra residue testing

Based on the cleaning frequency assessment, within campaign in-situ minor cleaning is not involved during manufacture of ASYM-141366 Step 1. The Solid Phase Synthesizer reactor has a complex internal structure, and peptide resin exhibits poor solubility. So, Solid Phase Synthesizer (TJ4S-1213-Sy15) will be disassemble major cleaned after every batch, instead of within campaign in-situ minor cleaning.

Batch to batch flush: Equipment with higher risk will be flushed after each batch as per Level 0/B procedure.

Based on the cleaning frequency assessment, batch to batch flush will be performed for key equipment pyridine/acetic anhydride/DMF solution preparation reactor (TJ4S-1213-R22) and Oxyma/DMF solution preparation tank (TJ4S-1200-AT02). The pyridine/acetic anhydride/DMF solution preparation reactor (TJ4S-1213-R22) and Oxyma/DMF solution preparation tank (TJ4S-1200-AT02) will be batch to batch flushed after every batch.

- Disassemble Major cleaning：

- Pre-production cleaning: To ensure that there is no cross contamination from previous production, and before starting the production activity, all the equipment shall be cleaned to meet the defined ARL (Allowable Residual Limit) or MACO (Maximum Allowable Carry Over) in terms of the previous product in each piece of equipment of the equipment train.

- Within Campaign cleaning: To further eliminate the possible impact to subsequent batches from the traceable residual after designed in-situ minor cleaning (un-disassembled), the equipment will be disassembled cleaned at defined batch frequency. Considering the in-situ minor cleaning is validated in three cycles, after run the forth cycle, the next in-situ cleaning may be at risk, so the within campaign disassemble major cleaning is planned at this point to refresh the equipment train’s cleanliness. Hence the maximum continuous batches before disassemble major cleaning is defined as: 4n (n stands for the maximum batches for in-situ minor cleaning).

Based on the cleaning frequency assessment, disassemble major cleaning will be performed after each batch, so the 4n in-situ minor cleaning frequency is not applicable. The solid-phase synthesis reactor has a complex internal structure, and peptide resin exhibits poor solubility. So, Solid Phase Synthesizer (TJ4S-1213-Sy15) will be disassemble major cleaned after every batch.

- End of production cleaning: In order to prevent the next product from being contaminated from residue of this product remaining on the equipment, at the end of the production, the equipment will be cleaned following the post-production cleaning method—disassemble cleaning. Disassemble cleaning method will be verified by testing the residue in each piece of equipment separately by sampling from the worst-case location. Three rounds of cleaning verification consists of cleaning validation.

According to above cleaning validation strategy (including cleaning criteria and cleaning frequency), for the Step 1 manufacture of ASYM-140020, the cleaning frequency and criteria for each key equipment have been evaluated based on existing data and historical production experience, the detail risk assessment refer to the specific part within this cleaning validation protocol, and the brief information has been summarized as follow in Table 5-1:

- The pyridine/acetic anhydride/DMF solution preparation reactor (TJ4S-1213-R22) and Oxyma/DMF solution preparation tank (TJ4S-1200-AT02) will be batch to batch flushed after each batch as per Level 0/B criteria. They were assessed as having no target residue (more detail see the Section 6.5), hence, chemical residue testing is not required, only perform the Level 0/B verification.

- Solid Phase Synthesizer (TJ4S-1213-Sy15) will be disassemble major cleaned after every batch. Therefore, it does not involve in-situ minor cleaning.

Table 5-1, The basic cleaning information for the equipment

| Equipment name | Equipment ID | Cleaning frequency | Cleaning effectiveness verification | Residue limits and basis |
| --- | --- | --- | --- | --- |
| Peptide Solid Phase Synthesizer | TJ4S-1213-Sy15 TJ4S-1213-Sy15-R01 | Disassemble cleaning after every batch | - Level I visual check after disassemble cleaning - Level II chemical residue test after disassemble cleaning, including Rinse sample and Swab sample | The lower between 10ppm and the calculated allowable residue limit (ARL) based on the health data of compound |
| Glass-lined Reactor | TJ4S-1213-R22 | Batch to batch flushing after each batch | Level 0/B visual check after flush by pre-defined solvent | Only for preparing Ac2O/Pyridine /DMF solution, rinse thoroughly with DMF followed by water; the risk of residual carryover remains low, no need for chemical residue test. |
|  |  | Disassemble cleaning after end of production | Level I visual check after disassemble cleaning |  |
| Agitation tank | TJ4S-1200-AT02 | Batch to batch flushing after each batch | Level 0/B visual check after flush by pre-defined solvent | Only for preparing Oxyma Pure /DMF solution, rinse thoroughly with DMF followed by water; the risk of residual carryover remains low, no need for level II test., no need for chemical residue test. |
|  |  | Disassemble cleaning after end of production | Level I visual check after disassemble cleaning |  |

In addition to consider the production arrangement, total 21 continuous batches are planned in the first commercial manufacturing campaign supporting Process Performance Qualification activities, including 3 PPQ batches (CPo140020-03-01-02, CPo140020-03-01-03, CPo140020-03-01-04) and 18 commercial batches starting from October 2026. Hence, the scope of this cleaning validation has been specific determined, refer to Fig. 3.

Fig. 3, Solid Phase Synthesizer (TJ4S-1213-Sy15) cleaning validation strategy

Any changes in the cleaning approach post-validation, will be assessed for any impact on the validated cleaning method, and if needed, the cleaning method will be re-validated.

- Execution procedure

- Summary of equipment train used in the production

The equipment train, intended for CPo140020-03 Step 1, is listed in Table 6-1.

Table 6-1, Intended equipment train in CPo140020-03

| Step No. |  | Equipment name |  | Equipment ID |  | Size | MOC | Function |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  | Peptide Solid Phase Synthesizer |  | TJ4S-1213-Sy15 |  | 3000 L | Hastelloy | Peptide synthesis, Activate, Drying |
|  |  | Activation tank2 |  | TJ4S-1213-Sy15-R01 |  | 1500 L | Hastelloy |  |
|  |  | Glass-lined Reactor |  | TJ4S-1213-R22 |  | 1000 L | Glass-lined | Pyridine/acetic anhydride DMF solution |
|  | Agitation tank | TJ4S-1200-AT02 | 500 L | Hastelloy | Oxyma/DMF solution preparation |

Note2: The Activation Tank (TJ4S-1213-Sy15-R01) is part of the Peptide Solid Phase Synthesizer (TJ4S-1213-Sy15) and is cleaned together with the entire unit. This document will discuss the complete Peptide Solid Phase Synthesizer (TJ4S-1213-Sy15).

Table 6-2, Auxiliary equipment in CPo140020-03 Step1

| Equipment name(ID) | Auxiliary equipment | Function | Cleaning Cycle |
| --- | --- | --- | --- |
| Peptide Solid Phase Synthesizer (TJ4S-1213-Sy15) | Solid sampler (TJ4S-1213-Sy15-Sam01) | Sampling | Perform Level 0/B cleaning after each sampling within batch. Cleaning after production or between batches with Peptide Solid Phase Synthesizer (TJ4S-1213-Sy15) |
|  | Receiving Rake (TJ4S-1213-Sy15-Ra01) | Receiving | Cleaning after production or between batches with Peptide Solid Phase Synthesizer (TJ4S-1213-Sy15) |
|  | In-line Sampler | Sampling |  |
|  | Glove box | Receiving |  |
| Activation tank (TJ4S-1213-Sy15-R01) | Metering pump (TJ4S-1213-Sy15-P04) | Add DIPEA | Cleaning after production or between batches with Peptide Solid Phase Synthesizer (TJ4S-1213-Sy15) |
|  | Metering pump (TJ4S-1213-Sy15-P03) | Add DIC |  |
|  | System Pump (TJ4S-1213-Sy15-P02) | From TJ4S-1213-Sy15-R01 to TJ4S-1213-Sy15 |  |
| Glass-lined Reactor (TJ4S-1213-R22) | Diaphragm Pump (TJ4S-1200-PDP040) | Add pyridine/acetic anhydride/DMF solution | Perform Level 0/B cleaning between batches with Glass-lined Reactor (TJ4S-1213-R22) Perform Level I Cleaning after production with Glass-lined Reactor (TJ4S-1213-R22) |
| Agitation tank (TJ4S-1200-AT02) | Diaphragm Pump (TJ4S-1200-PDP039) | Add Oxyma/DMF solution | Perform Level 0/B cleaning between batches with Agitation tank (TJ4S-1200-AT02)) Perform Level I Cleaning after production |
| Pipeline | 140020-step01-P02 | From TJ4S-1213-R22 to TJ4S-1213-Sy15 | Perform Level 0/B cleaning between batches with Glass-lined Reactor (TJ4S-1213-R22) Perform Level I Cleaning separately after production |
|  | 140020-step01-P01 | From TJ4S-1200-AT02 to TJ4S-1213-Sy15 | Perform Level 0/B cleaning between batches with Agitation tank (TJ4S-1200-AT02) Perform Level I Cleaning separately after production |

- Pre-production equipment cleaning check

Before using the equipment for manufacture, all the equipment shall be cleaned. The details of previous product produced in each piece of equipment in the planned equipment train, the calculated MACO (Maximum Allowable Carry Over) of the target residue compound (absolute data), and the corresponding ARL (Allowable Residual Limit, relative data which is calculated from MACO and minimum batch size of next product) are listed in Table 6-3. The relative limit of target residue compound shall be either NMT 10 ppm or the calculated ARL (Allowable Residual Limit), whichever is lower.

Within the campaign (ASYM-141366 into ASYM-141366), the quality-based limit of 10ppm is applicable, and a health-based limit is not needed.

The glass-lined reactor TJ4S-1213-R22 was previously used for manufacturing ASYM-146362. A quality-based limit of 10 ppm applies, and no health-based limit is required.

Table 6-3, Calculated MACO and ARL

| Equipment ID | Prev. Product | Prev. product health data | Next product | Next product health data | MACO of prev. product | Next product MBS (min. batch size) | Calculated ARL | Calculated ARL in the equipment release compared to 10 ppm which is lower |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TJ4S-1213-Sy15 | ASYM-124852/ YL005318 and ASYM-140020 | OEL:10-100 ug/m3 | ASYM-141366 | MTDDnext= 1000 mg/day | 1056.3 mg | 105.63 kg | ARL=PDE/ MTDDnext =OEL×V/MTDDnext =10 μg/m3×10 m3/day/1000 mg/day =100 ppm>10 ppm | NMT 10 ppm |
| TJ4S-1213-R22 | ASYM-146362 | OEL: 1 ug/m3 | ASYM-141366 | MTDDnext= 1000 mg/day | 1056.3 mg | 105.63 kg | ARL=PDE/ MTDDnext =OEL×V/MTDDnext =1 μg/m3×10 m3/day/1000 mg/day =10 ppm | NMT 10 ppm |
| TJ4S-1200-AT02 | N/A3 | N/A3 | ASYM-141366 | MTDDnext= 1000 mg/day | N/A3 | 105.63kg | N/A3 | N/A3 |

Note 3：Before TJ4S-1200-AT02 is used for ASYM-141366 production, the storage tank was mainly used to prepare solutions such as DMF/methanol, with extremely low residual risk and therefore no target substances.

The calculation of MACO and ARL following the formula as specified in DS-GMP-003 < Equipment cleaning management procedure for GMP production> (current version), for example:

ARL=PDEMTDDnext

MACO=ARL×MBS=PDE×MBSMTDDnext

The acronyms in the above equations are listed in Attachment 5. The release result of the equipment train will summarized into <Equipment Training release checklist> (QF03282).

- Batch to batch flushing to mitigate the risk in the high risk equipment

The equipment with high risk shall be cleaned after each batch per Level 0/B criteria to mitigate the risk per step 1 process tolerance, as summarized in Table 6-4. According to the assessment in Table 6-4, the equipment TJ4S-1213-R22 and equipment TJ4S-1200-AT02 show high risks, respectively, thus they will be subjected to batch-to-batch flushing. The batch-to-batch cleaning method shall be defined in < CMb-TJ414002001> (current version).

Table 6-4, Risk assessment on batch to batch flushing

| Step No. | Equipment name | Equipment ID | Function | Risk assessment | Conclusion |
| --- | --- | --- | --- | --- | --- |
| 1 | Peptide Solid Phase Synthesizer | TJ4S-1213-Sy15 (3000L) | Amino acid activation, Peptide synthesis, Activate, Drying | This equipment is critical for solid-phase synthesis. The solid-phase synthesis reactor has a complex internal structure, and peptidyl resin exhibits poor solubility, making it hard to be cleaned completely by batch to batch flush. Inadequate cleaning will directly compromise the product quality of subsequent batches. In the solid-phase synthesis reactor, 2-CTC resin undergoes repeated cycles of coupling, washing, deprotection and re-washing, last drying to produce the peptidyl resin in-process intermediate ASYM-141366. Incomplete cleaning within campaign may lead to formation of additional long-chain impurities during the reaction cycles of the next batch. The reactor shall be disassembled for thorough cleaning upon completion of each batch to ensure no solid peptide resin residues remain, performing disassemble major cleaning every batch, so no need batch to batch flush. | Disassembled major cleaning is performed for every batch; therefore, batch-to-batch flush is not required. |
|  | Glass-lined Reactor | TJ4S-1213-R22 (1000L) | Pyridine/acetic anhydride DMF solution preparation | This equipment is used to prepare pyridine/acetic anhydride DMF solution. During the continuous production, the residual pyridine and acetic anhydride in the reactor may lead to the accumulation of impurities. pyridine and acetic anhydride are easily removed using DMF. Based on above, it is low risk to perform batch to batch flushing. Therefore, after each batch is completed, the Glass-lined Reactor needs to perform batch to batch flush by spray ball (coverage test qualification refer to SVP-TJ4-101.12-052). | After each batch is completed, the Glass-lined Reactor needs to conduct batch to batch flush |
|  | Agitation tank | TJ4S-1200-AT02 (500L) | Oxyma/DMF solution preparation |  | This equipment is used to prepare the Oxyma/DMF solution. During the continuous production, due to long interval between batches, dissolved Oxyma may lead to the accumulation of impurities. If residual material from previous batch remains uncleaned in the reactor, it will be incorporated into subsequent batch, and residues may accumulate inside the reactor over successive batches. Oxyma is easily removed using DMF Based on above, it is low risk to perform batch to batch flushing. Therefore, after each batch is completed, the Agitation tank needs to conduct batch to batch flush by spray ball (coverage test qualification refer to SVP-TJ4-170.01-068). |  | After each batch is completed, the Agitation tank needs to conduct batch to batch flush |

- Selection of the target residue compounds

As per the process of Step 1, the following materials are selected as the target residue compound for testing, see Table 6-5. And the rationale for target residue compound selection of each equipment is listed in Table 6-6.

Table 6-5, Target residue compound selected for testing

| Target Compound | Asymchem code | Customer code | Structure | Batch size kg | Solubility data |
| --- | --- | --- | --- | --- | --- |
| N-Fmoc-N’-Boc-L-ornithyl-L-leucyl-L-alanyl-L-γ-tBu-glutaminyl-α-methylphenylalanyl-L-leucyl-L-valyl-N’-(2,2,4,6,7-pentamethyldihydrobenzofuran-5-sulfonyl)-L-arginyl-tBu-L-seryl-tBu-L-seryl-N-triphenylmethyl-L-asparagyl-N-methyl-N’-triphenylmethyl-asparagyl-phenylalanyl-glycine | ASYM-140020 | LSN4182924 |  | 105.63 ~278.62 | 1 g in 5 ml DMF at 23~27 ℃ |
| (2S)-2-(9H-Fluoren-9-ylmethoxycarbonylamino)-5-[(2-methylpropan-2-yl)oxycarbonylamino]pentanoic acid (Fmoc-L-Orn(Boc)-OH) | ASYM-124852/ YL005318 | N/A |  | 105.63 ~278.62 | 1 g in 8 ml DMF at 23~27 ℃ |

Table 6-6, Rationale of target residue compound selection

| Step No. | Equipment name | Equipment ID | Function | Target residue | Rationale of target residue compound selection |
| --- | --- | --- | --- | --- | --- |
| 1 | Peptide Solid Phase Synthesizer | TJ4S-1213-Sy15 (3000L) | Amino acid activation, Peptide synthesis, Fmoc deprotection, Wash resin, Drying | ASYM-140020 and ASYM-124852 | Combining the process characteristics of the solid-phase synthesis reactor (including synthesizer and activation tank) and the subsequent soft cleavage process route of ASYM-141366 peptidyl resin, the peptide fragments bound to CTC resin are easily dissociated from the resin carrier in advance, forming the free solid product ASYM-140020, which had been proved by shaking the peptidyl resin ASYM-141366 in DMF solvent, it was demonstrated that the filtrate was mainly detected of the cleavage product ASYM-140020, ASYM-140020 becomes the main pollutant on the equipment surface. ASYM-141366 peptidyl resin is a resin bonded product, and the resin particles are easy to remove by washing; ASYM-140020 exists as fine solid particles, which are more liable to adsorb onto equipment surfaces and present greater cleaning difficulty. Material drying is the final unit operation carried out in the solid-phase synthesis reactor. No additional solvents are introduced in this step, and the in-process product ASYM-140020 demonstrates favorable stability under drying conditions, such that residues on equipment surfaces are not prone to spontaneous degradation or elimination. In consideration of all the above factors, ASYM-140020 is specified as the target residue for cleaning validation of this equipment. |
|  |  |  |  |  | The activation tank is exclusively used for amino acid activation. The activated amino acid solution is transferred to the next synthesizer unit via a dedicated pipeline, followed by washing and the next amino acid activation cycle. The tank does not come into contact with resin throughout the entire process. A total of 10 amino acids are used in this equipment. After the activation and transfer of each amino acid is completed, the tank is washed three times with DMF, which has good solubility for amino acids. The peptide solid-phase synthesizer is cleaned as a whole. ASYM-124852 was additionally selected as another target residue because it represents the final activated amino acid processed within the activation tank immediately prior to completion of the synthesis sequence. Due to its repeated exposure to activation and transfer operations and its potential presence within transfer lines and activation system components, it represents an additional worst-case cleaning challenge. ASYM-124852 is stable in the cleaning solvent. Therefore, ASYM-124852 and ASYM-140020 are selected as target residues. |
|  | Glass-lined Reactor | TJ4S-1213-R22 (1000L) | Pyridine/acetic anhydride DMF solution Preparation | N/A | In this reactor, DMF is used to release pyridine and acetic anhydride. Pyridine is a small nitrogen-containing organic base of low molecular weight. It exhibits good solubility in water and polar solvents and has no hydrophobic long chains, so it does not form polymer/gel-like deposits that are difficult to elute. Acetic anhydride hydrolyzes to acetic acid on contact with water; acetic acid is also a small-molecule species and will not form such hard-to-elute deposits. Both pyridine and acetic anhydride can be readily removed with the selected cleaning solvents. Therefore, it is not necessary to quantitatively test the target residue. |
|  | Agitation tank | TJ4S-1200-AT02 (500L) | Oxyma/DMF solution preparation | N/A | In this Agitation tank, DMF is used to free the material Oxyma in DMF and form a clear solution. Oxyma is a small-molecule additive with good solubility in polar solvents such as DMF and water. It has no hydrophobic long alkyl chains and does not tend to adsorb strongly onto equipment surfaces or form poorly soluble polymer/gel deposits. The residual Oxyma can be efficiently purged and removed using the defined cleaning solvents. Therefore, it is considered easy to clean. |

- Cleaning frequency

    - Cleaning frequency for within campaign in-situ minor cleaning

The equipment cleaning frequency and justification is further indicated in Table 6-7. The cleaning frequency has been established based on process knowledge, equipment design, residue characteristics and manufacturing experience.

Table 6-7, Rationale of the equipment batch cleaning frequency

| Step No. | Equipment name | Equipment ID | Function | Target residue | Cleaning Frequency | Rationale of Cleaning Frequency |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Peptide Solid Phase Synthesizer | TJ4S-1213-Sy15 (3000L) | Peptide synthesis, Activate, Drying | ASYM-140020 and ASYM-124852 | Dis-assemble major cleaning every batch | This equipment is critical for solid-phase synthesis. The solid-phase synthesis reactor has a complex internal structure, and peptide resin exhibits poor solubility, making it hard to rinse completely. Inadequate cleaning will directly compromise the product quality of subsequent batches. In the solid-phase synthesis reactor, 2-CTC resin undergoes repeated cycles of coupling, washing, de-protection and re-washing to produce the peptidyl resin in-process intermediate ASYM-141366. Incomplete inter-batch cleaning will lead to formation of additional long-chain impurities during the reaction cycles of the next batch. Based on past production experience of full-scale non-saleable batches, the reactor shall be disassembled for thorough cleaning upon completion of each batch to ensure no solid peptide resin residues remain, performing disassemble major cleaning every batch, so no need to conduct within in-situ minor cleaning. |
|  | Glass-lined Reactor | TJ4S-1213-R22 (1000L) | Pyridine/acetic anhydride DMF solution | N/A | Batch-to-batch flush (Level 0/B); no in-situ minor cleaning; disassemble major cleaning at end of production | Based on scientific knowledge, pyridine and acetic anhydride are stable in DMF and the reaction reactor is nitrogen protected, so there will be no introduction of water. During the continuous production, after process mixture in this reactor proceed to downstream, the next batch shall go into the reactor. All residues in reactor shall be dissolved and distributed into the next batch and no residue shall stay in the reactor for several batches. Pyridine/acetic anhydride can dissolve in DMF at any ratio and batch to batch DMF. Based on above, it is low risk to perform the flushing every batch. Therefore, only batch to batch flush is required; in-situ minor cleaning is not necessary. |
|  | Agitation tank | TJ4S-1200-AT02 (500L) | Oxyma/DMF solution preparation | N/A | Batch-to-batch flush (Level 0/B); no in-situ minor cleaning; disassemble major cleaning at end of production | The equipment is only used for dissolve Oxyma. Based on scientific knowledge, Oxyma is stable in DMF and the reactor is nitrogen protected. During the continuous production, after process mixture in this reactor proceed to downstream, the next batch shall go into the reactor. All of residue in reactor shall be dissolved and distributed into the next batch and no residue shall stay in the reactor for several batch. Oxyma has good solubility in DMF. Based on above, it is low risk to perform the flushing every batch. Therefore, only batch to batch flush is required; in-situ minor cleaning is not necessary. |

Note4: For Projects CPo140019-03 and CPo140020-03, Equipment TJ4S-1213-R22 was used to prepare the pyridine/acetic anhydride solution in DMF. The equipment is flushed between batches and subsequently disassembled for a major cleaning at the end of production.

    - Cleaning frequency for dis-assemble major cleaning

Disassemble major cleaning shall be carried out after campaign production completion and switching to next product. The disassemble major cleaning method for end of production is same with disassemble major cleaning method for within campaign, hence, disassemble major cleaning will be directly performed after each campaign once the within campaign disassemble major cleaning validation completion.

- Allowable residue limit determination

- Allowable residue limit for within-campaign in-situ minor cleaning

Based on above risk assessment about the within-campaign cleaning frequency, within-campaign in-situ cleaning isn’t applicable, hence this part isn’t applicable within this cleaning validation protocol.

- Allowable residue limit for dis-assemble major cleaning

After production of one batch has ended, the equipment shall be cleaned following the end of production cleaning method: disassemble cleaning, all the equipment shall be quantitatively tested to verify as meeting the defined ARL (or MACO) in terms of the previous product in each piece of equipment, the target compound relative residue criteria shall be the stricter level between 10 ppm (which is Asymchem’s internal acceptance criteria defined in DS-GMP-003 <Equipment cleaning management procedure for GMP production> (current version)) and calculated allowable residual limit (ARL). The calculation of MACO and ARL following the formula as specified in DS-GMP-003 <Equipment cleaning management procedure for GMP production> (current version).

Considering the within-campaign disassemble cleaning frequency is defined as cleaned after every batch for peptide solid phase synthesizer, and the target of selected equipment is listed in Table 6-8, and the calculation based on established healthy data is listed in Table 6-9.

Table 6-8, Toxicological data of the equipment selected for chemical residue test

| Step No. | Equipment name | Equipment ID | Function | Target residue | Toxicological data |
| --- | --- | --- | --- | --- | --- |
| 1 | Peptide Solid Phase Synthesizer | TJ4S-1213-Sy15(3000L) | Amino acid activation, Peptide synthesis, Fmoc deprotection, Wash resin, Drying | ASYM-140020 | OEL:10-100ug/m3 |
|  |  |  |  | ASYM-124852 | OEL:10-100ug/m3 |

Table 6-9, Calculated MACO and ARL

| Equipment ID | Prev. Product | Prev. product health data/ | Next product/ | Next product MTDD | MACO of prev. product | Next product min. batch size） | Calculated AR | Calculated ARL, compared to 10 ppm, which is lower |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TJ4S-1213-Sy15 | ASYM-140020 | OEL:10-100ug/m3 | ASYM-140020 | MTDDnext =1000mg | 1056.3 mg | 105.63 kg | ARL=PDE/ MTDDnext =OEL×V/MTDDnext =10μg/m3×10m3/day/1000mg/day =100ppm>10ppm | NMT 10ppm |
|  | ASYM-124852 | OEL:10-100ug/m3 | ASYM-140020 | MTDDnext =1000mg | 1056.3 mg | 105.63 kg | ARL=PDE/ MTDDnext =OEL×V/MTDDnext =10μg/m3×10m3/day/1000mg/day =100ppm>10ppm | NMT 10ppm |

Table 6-10, The calculated actual swab limit and rinse limit

| Step No. | Equipment ID | Target residue (Asymchem compound code) | Equipment material | Equipment internal surface area /Weight range and type of rinsing & refluxing solution | After Campaign |  |  | Between Campaign |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Equipment minimum output (e.g., refer to QM05004) | ARL | ARL of per cm2/100cm2 area/per unit volume | Minimum batch size of next product | ARL | ARL of per cm2/100cm2 area/per unit volume |
| Step 1 | 3000L TJ4S-1213-Sy15 | ASYM-140020 | Hastelloy | ☒Swab： 208185cm2 ☐Reflux： N/A ☒Rinse：DMF 105~115kg (111~121L) | 7.5 kg | 10 | Swab：36.03μg/100cm2 Rinse： 0.00062~0.00067mg/mL | 105.63kg | 10 | Swab：507.39μg/100cm2 Rinse： 0.00867~0.00950mg/mL |
|  |  | ASYM-124852 | Hastelloy | ☒Swab：208185cm2 ☐Reflux： N/A ☒Rinse：DMF 105~115kg (111~121L) | 7.5 kg | 10 | Swab：36.03μg/100cm2 Rinse： 0.00062~0.00067mg/mL | 105.63kg | 10 | Swab：507.39μg/100cm2 Rinse： 0.00867~0.00950mg/mL |

- Rationale for Exemption of Microbial/Bioburden Assessment for SPPS

Bioburden/microbial and endotoxin contamination were assessed as low risk Through evaluation of the anhydrous synthesis process, equipment washing with water followed by rinsing with DMF/MTBE and nitrogen drying, as well as downstream processes and control measures. The detailed assessment refers to the document RA-140020030103 < Risk Assessment Report for Contamination Control Strategy in LSN4182924/ ASYM-140020 production>.

- Equipment cleaning evaluation based on the risk analysis

- Assessment of the equipment

As per the residue material solubility, each equipment has a separately designed cleaning method, see Annex 1. All the equipment shall be cleaned in a defined interval according to cleaning method <CMb-TJ414002001 > (current version). Not all the equipment shall be swabbed, rinsed or refluxed to sample for residue test. Only the equipment that is confirmed as higher contamination risk shall be swabbed, rinsed or refluxed to sample for the residue test after cleaning.

The risk assessment is based on the Risk Ranking and Filtering method according to ICH Q9 <Quality Risk Management>, ASM-027 <Application of risk management tools> and QAP-005 <Risk Management Policy>. The selected method to determine worst case equipment -- risk ranking and filtering method refer to Asymchem’s SOP <Cleaning Validation> DS-GMP-043 (current version), detail assessment of worst case equipment is shown as follow:

The standard of the assessment is set as indicated in Table 6-11, and the standard risk grade is laid down in the four typical aspects with regard to the direct product contact equipment: material of construction, smoothness of the surface, complexity of the equipment construction, function of the equipment. In each of the four aspects, the risk grade is set in different circumstances. Basically, the parameters of GL reactor with homogenous reaction are set as 1.

Table 6-11, Standard risk ranking in the risk assessment

| Material of construction of direct product contact/ (A) | Stainless steel (SS), Hastelloy/ |  |  | Glass-lined (GL) |  |  |  | PTFE, PE, Halar |  |  | Polypropylene (PP) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Risk ranking by score | 0.8 |  |  | 1 |  |  |  | 2 |  |  | 3 |  |
| Smoothness of the surface (direct product contact) (B) | Flat smooth |  |  | Curve smooth |  |  |  | Irregular geometry smooth |  |  | Rough |  |
| Risk ranking by score | 0.5 |  |  | 1 |  |  |  | 2 |  |  | 3 |  |
| Complexity of the equipment construction (C) | Single geometry space |  |  | 2 or 3 continuous geometry space |  |  |  | Multi-continuous geometry spac |  |  | Complex irregular geometry space |  |
| Risk ranking by score | 1 |  |  | 1.5 |  |  |  | 2 |  |  | 3 |  |
| Function of the equipment | Prepare inorganic solution | Prepare organic solution | Homo-genous workup |  | Homo-genous reaction | Hetero-geneous workup | Hetero-geneous reaction |  | Centrifuge or filter | Only solid material contact (drying or material charging) |  | Crystallization, recrystallization |
| Risk ranking by score | 0.5 | 0.6 | 0.8 |  | 1 | 1.5 | 1.8 |  | 2.5 | 2.8 |  | 3 |

Based on the above established standard, risk assessment is performed on all the equipment used in the process. The result is listed in Table 6-12.The risk score is calculated by multiplying the grade score of A, B, C and D. And A stands for grade score of Material of construction of direct product contact, B stands for Smoothness of the direct product contact surface, C stands for Complexity of the equipment construction, D stands for Function of the equipment. After obtaining the risk score (gray background), the equipment with the same residue target, the same cleaning frequency, is kept under one group for comparison, and the highest score equipment shall be selected for the residue test as worst case. After risk filtering in Section 6.5, the Solid Phase Synthesizer (TJ4S-1213-Sy15) is cleaned by disassembly after each batch with highest score, while the Glass-lined Reactor (TJ4S-1213-R22) and Agitation tank (TJ4S-1200-AT02) do not involve chemical residue testing. Hence, Peptide Solid Phase Synthesizer TJ4S-1213-Sy15 is selected as the typical and representative equipment for chemical residue test.

Table 6-12 Risk Assessment Based on Risk Ranking and Filtering method

| Step No. | Target residue compound | Equipment type | Equipment ID | Size | MOC | Function | Risk Assessment (in terms of direct product contact) |  |  |  |  | Risk filtering |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Material of construction | Smoothness of the surface | Complexity of the equipment construction | Function of the equipment | Risk score |  |
|  |  |  |  |  |  |  | A | B | C | D |  |  |
| 1 | ASYM-140020 | Peptide Solid Phase Synthesizer | TJ4S-1213-Sy15 | 3000 L | Hastelloy | Peptide synthesis, Activate, Drying | 0.8 | 2 | 2 | 2.8 | 8.96 | Typical and Representative |
|  | N/A | Glass-lined Reactor | TJ4S-1213-R22 | 1000 L | Glass-lined | Pyridine/acetic anhydride DMF solution | 1 | 1 | 1 | 0.6 | 0.6 | Not Typical and Representative |
|  | N/A | Agitation tank | TJ4S-1200-AT02 | 500 L | Hastelloy | Oxyma/DMF solution preparation | 0.8 | 1 | 1 | 0.6 | 0.48 | Not Typical and Representative |

* Risk score=A×B×C×D

‡ Direct material contact, and material is easy to stick on the equipment, or the equipment is of complex structure is evaluated as “higher risk” in the risk ranking

† If one equipment experienced multi-purpose in the process, the highest risk score shall be considered

§In terms of the same residue target equipment, if final risk score is the same, the more complex structured equipment shall be considered as the typical and representative; and if two equal equipment were included for the same process (for example, two SS centrifuge machines are used for the isolation of the same product), either one can be considered typical and representative.

After the above risk ranking and filtering, the following equipment in Table 6-13, assessed as typical and representative equipment shall be rinse sampled or swab sampled for testing after equipment cleaning.

Table 6-13, Typical and representative equipment after Risk Ranking and Filtering

| Target residue | Cleaning frequency | Equipment type | Equipment ID | Size/Inner surface area | MOC | Function |
| --- | --- | --- | --- | --- | --- | --- |
| ASYM-140020 | Every batch | Peptide Solid Phase Synthesizer | TJ4S-1213-Sy15 | 3000 L | Hastelloy | Peptide synthesis, Activate, Drying |

The Peptide Solid Phase Synthesizer (TJ4S-1213-Sy15) has been identified as the worst-case equipment based on the following: it is in direct contact with peptide-bearing resin throughout the entire synthesis process, representing the longest product contact duration; the equipment has complex geometry with internal features that limit cleaning accessibility; internal structures (such as sieve plates, dead legs and transfer pathways) have a high residue retention potential; and the equipment is exposed to the highest quantity of process-related residues. All of the above factors have been considered, and this equipment received the highest overall score.

- Assessment of the spare parts

During the cleaning, the equipment parts such as gaskets, valves, tees, cross, sight glass, U-bend, condenser and reducers, etc. shall be cleaned together with the corresponding equipment.

For the within-campaign in-situ minor cleaning, at the defined intervals in the production, the equipment including the accessories shall be in-situ cleaned and the residue shall be measured by swab samples or solvent flush / rinse or other method (as defined in the cleaning method), without disassemble, before sampling for testing.

- Assessment of the pipelines

For the pipeline connections in the equipment train, it is fixed for the dedicated purpose in the process, there are several scenarios:

For the pipelines used for solvent delivery, it only contacts the solvent without other materials/intermediate/products. The solvents in this process are stable, so there is no solvent decomposition. After each addition, purge with nitrogen to ensure that all solvents are added. When at rest, the pipelines are in a closed state, so the cleaning risk level is low.

The pipelines used for the process solution transfer are cleaned together with the equipment with solvent rinses / flushes, so the risk level is low in terms of the same step in the same project, since pipelines are dedicated to each commercial project. In addition, if the production interval for ASYM-140020 will be long in future, the cleaned pipelines also can be assigned to other projects or disposable, then new pipelines will be engaged in next ASYM-140020 manufacturing.

The flexible hose is disposable for project, it will be discarded after campaign.

- Assessment of the sampling tools

The glass sampler is disposable and will not be cleaned.

For sampling of the process system in the peptide solid-phase synthesizer, samples will be collected under vacuum through an in-line sampler (stainless steel sampling line + glass collection bottle). Prior to each sampling, the sampling line needs to be purged with nitrogen to blow the system contents back into the process system, followed by vacuum sampling. The first two collected samples are discarded, and the third collected sample is sent for testing. After each sampling use, the glass collection bottle is not cleaned; a new glass collection bottle is replaced for the next sampling. During disassembly and cleaning, the in-line sampler (stainless steel sampling line + glass collection bottle) will be cleaned together with the peptide solid-phase synthesizer. Alternatively, sampling of the process system in the peptide solid-phase synthesizer can be performed directly using a disposable PTFE tube and a disposable syringe.

For sampling of solid materials in the solid-phase synthesizer, a stainless steel solid sampler is typically used. The stainless steel sampler shall be cleaned after each sampling operation. During disassembly and cleaning, the stainless steel solid sampler shall be cleaned together with the peptide solid-phase synthesizer. A detailed risk assessment for the cleaning of sampling tools is presented in Table 6-14.

Table 6-14,  Risk evaluation to sampling tool cleaning

| No | Tool | Criticality assessment |  | Residual possibility assessment |  |  | Level of risk | Control measure |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Impact on product quality | Criticality | Surface Assessment: Stainless Steel (SS), Plastic (P), Glass (G), Smooth (SM), Rough (R) | Clean Difficulty | Possible cause |  |  |
| 1 | Thief | Direct contact with the material. The residue will bring potential cross-contamination if reused. | Critical | SS, SM | Easy to clean as the surface is smooth | Low | Low | Flush/swab the surface completely, visual Inspection |

- Assessment of the operation aids

For the process aids, for example, the scoop, rake, pump and secondary protection bucket, the cleaning method is separately provided in the cleaning method document CMa-TJ414002001 <Equipments Cleaning Method> (current version) and CMb-TJ414002001 <Equipments Cleaning Method> (current version). Considering the MOC of these operation aids is SS & GL, smooth surface, simple in the structure, be removable cleaned and direct visual inspection, so the material residue is low risk.

- Assessment of the filter consumables

In the manufacturing of ASYM-140020, the gloves, latex tube, PTFE pipe and filter cartridge are used which are directly contact with specific material. From which, the gloves, latex tube and filter cartridge, will be disposed after cleaning together with the relevant equipment (Peptide Solid Phase Synthesizer, reactor, liquid filter).

In the SPPS process, there is 10 μm stainless steel filter before the solvent entering the production building from tank farm. This filter is exclusively used for filtering fresh solvents from the tank farm. Preventive maintenance is performed every 6 months (refer to QM03025<Equipment and facility maintenance manual>), including cleaning and integrity testing. Since it does not contact with peptide-related materials, the possibility of cross-contamination transfer is extremely low, thus the risk level is determined as low. so the risk level is low in the project.

- Cleaning method

The Level 0/B batch to batch flush cleaning is included in CMb-TJ414002001<Equipments Cleaning Method> (current version). The disassemble major cleaning method is included in CMa-TJ414002001< Equipments Cleaning Method> (current version).

The cleaning method for the worst-case equipment, the peptide solid phase synthesizer, have been practiced in full scale non-saleable batch, and achieved the cleaning purpose.

The critical cleaning parameters that need to be controlled and recorded in each validation run are shown in the table below.

Table 6-15, Cleaning Parameter Collection for Peptide Solid Phase Synthesizer

| Equipment type | Equipment ID | Cleaning frequency | Cleaning solvent | Cleaning method | Cleaning temp. | Cleaning solvent qty. | Cleaning time | Stir time | Stir speed | N2 Flow | Soak times | Repeat times |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Pre-defined | Pre-defined | Pre-defined | Pre-defined | Pre-defined | Pre-defined | Pre-defined | Pre-defined |
| Peptide solid phase synthesizer | TJ4S-1213-Sy15 TJ4S-1213-Sy15-R01 | Disassemble major cleaning After every batch | DMF | Automatic cleaning | Undefined | 290~310kg | 3min | 15min | 20~30rpm | 7~13 m3/h | NLT 0.5h | 2 |
|  |  |  | Purified water |  | Undefined | 290~310kg | 3min | 15min | 20~30rpm | 7~13 m3/h | Not applicable | 3 |
|  |  |  | Purified water | Rinse | Undefined | 105~115kg | Undefined | Not applicable | Not applicable | Not applicable | Not applicable | Not applicable |
|  |  |  | Purified water | Wipe | Undefined | 29.0~31.0kg | Undefined | Not applicable | Not applicable | Not applicable | Not applicable | Not applicable |
|  |  |  | DMF | Automatic cleaning | Undefined | 105~115kg | 3min | 15min | 20~30rpm | 7~13 m3/h | Not applicable | 1 |
|  |  |  | MTBE |  | Undefined | 170~190kg | 3min | 15min | 20~30rpm | 7~13 m3/h | Not applicable | 1 |

Table 6-16, Summary of Equipment Cleaning Parameters for Agitation Tank

| Equipment type | Equipment No. | Cleaning frequency | Cleaning solution | Cleaning method | Cleaning solution quantity |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Preset value |
| Glass-lined Reactor | TJ4S-1213-R22 | Disassemble major cleaning After end of production | DMF | Rinse | 170~180kg |
|  |  |  | Purified water | Rinse | 160~170kg |
|  |  |  | Purified water | Wipe | 100~110kg |
|  |  | Batch to batch flush After each batch | DMF | Flush | 170~180kg |
| Agitation tank | TJ4S-1200-AT02 | Disassemble major cleaning After end of production | DMF | Rinse | 95~105kg |
|  |  |  | Purified water | Rinse | 100~110kg |
|  |  |  | Purified water | Wipe | 20~25kg |
|  |  | Batch to batch flush After each batch | DMF | Flush | 90~110kg |

- Sampling plan and method

- Swab sampling for cleaning validation

- Safety

Wear powder-free gloves and goggles; Extreme care should be taken to prevent swab contamination during handling.

- Materials/Tools:

- Solvent: For the target compound, use the solvent prescribed in the specific cleaning method.

- For flat surfaces, use a frame draw the sampling area.

- An appropriate cuvette.

- Swab: Use approved swabs (Supplier: Texwipe; Part number: TX715; material: polyester; Swab head: length 25.7 mm, width 13 mm, thickness 4.2 mm; Swab handle: length 101.8 mm, width 5.2 mm, thickness 3 mm), in this validation non-dust cloth sponge swab is selected for active ingredients sampling (Fig. 4).

| Fig. 4, Texwipe TX715 polyester swab | Fig.5 Swab instruction over the sampling surface (10 cm*10 cm) |
| --- | --- |

- Procedure:

Press the swab head hard onto the sampling surface at about a 45° angle between the swab and the sampling surface and make sure that the swab surface is closely in contact with the sampling surface, and then swab the sampling surface slowly and steadily. Use one side of the swab to swab (swab sampling diagram as Fig. 5), then turn over the swab and change the swabbing direction to make it perpendicular to the previous swabbing direction (swab sampling diagram as Fig. 5). The swabbing process should cover the entire sampling surface. There should be no gaps in the sampling surface that have not been swabbed.

- Swab sampling location determination

The selected sampling location for each type of equipment was assessed as a general guideline at Asymchem, for detail, see <Equipment to be swab-tested and the sampling points diagram> (QM04005 current edition).

The sampling points are selected based on the document RA-TJ42024002.11<Risk Assessment Report for Sampling Point Locations of Production Equipment>, which is based on the risk ranking and filtering method according to ICH Q9, ASM-027<Application of Risk Management Tools> and QAP-005<Risk Management Policy>. Location selection considered complexity of the equipment construction, MOC of direct product contact part, smoothness of the surface, exposure to peptide-bearing resin and possibility of visual check. The selected locations therefore provide a conservative assessment of overall cleaning effectiveness. The specific sample location during the cleaning validation is summarized in the following Table 6-17.

Table 6-17,  the sample locations for the selected typical equipment

| Equipment I.D. | Sample location 1 | Sample location 2 | Sample location 3 | Sample location 4 |
| --- | --- | --- | --- | --- |
| Peptide Solid Phase Synthesizer (TJ4S-1213-Sy15) | Agitator blade | Inner surface | Side outlet | Sampling glove box |

And further illustrated as P&ID diagram in Annex  2.

- Rinse sampling for cleaning validation

- Safety: Wear gloves and goggles

- Tools: An appropriate sampling bottle

- Rinse sampling plan: Sampling plan includes sampling location, sample size, test items, sample number, sample method, frequency and testing plan, referring to Table 6-18.

- Rinse samples shall be collected only after completion of the equipment cleaning and successful QA visual check inspection.

- The sampling solvent will be that which is referred to in the cleaning method document.

- Choose the lowest outlet of the equipment to let rinse solvent out.

- Gather enough amount from the mid-part of the solution stream from the outlet of the equipment in a sampling bottle.

Table 6-18, Rinse and Swab sampling plan

| Equipment name | Sampling Location | Detail Location Description | Sampling method | Sample size | Sampling position P&ID map number | Test item | Number of Sample |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Peptide Solid Phase Synthesizer (TJ4S-1213-Sy15) | Upper expansion valve port of reactor | After washing the activation tank, an appropriate amount of eluent in the middle section is received from the upper expansion valve port of the synthesis reactor by running the transfer program and sent for testing | Rinsing sampling | ~500ml | Sampling location Diagram for Peptide Solid Synthesizer | ASYM-140020 and ASYM-124852 residue | 1 |
|  | In-line sampler5 | After cleaning the activation tank, a suitable volume of eluate is collected via the synthesis reactor’s in-line sampler by executing the transfer program and submitted for testing. | Rinsing sampling | ~500ml |  |  | 1 |
|  | Agitator | Sampling area of inner surface of 100 cm² agitator blade (bottom surface centre of agitator blade) | Swab sampling | 100 cm2 |  |  | 1 |
|  | Inner-surface of the synthesis reactor | Sampling area of 100 cm2 inner surface (close to the lower edge of inner surface) | Swab sampling | 100 cm2 |  |  | 1 |
|  | Side discharging outlet | Sampling area of inner surface of 100 cm2 side outlet (close to the outer edge of the side outlet | Swab sampling | 100 cm2 |  |  | 1 |
|  | Sampling glove box | Sampling area of inner surface of 100 cm2 (between left and right edges of the chamber Bottom) | Swab sampling | 100 cm2 |  |  | 1 |

Note 5: For samples collected via the in-line sampler, data collection will only be performed for the three validation rounds,no further testing will be conducted thereafter.

- Test method validation and recovery study

The cleaning validation studies are based on the swabbing technique and rinse technique. The validation studies include swabbing and rinsing of the main surface area as well as the hard-to-clean locations, both of which are carried out for the target compounds.

The validation results of system suitability, specificity, linearity, accuracy, limit of quantification and solution stability of method AM-140020-015 and AM-140020-014 all met the acceptance criteria as specified in validation protocol AM-140020-014-VP-01&02&03(validation report:AM-140020-014-VR-01&02&03) and AM-140020-015-VP-01&02(validation report:AM-140020-015-VR-01&02) , and the maximum injections between the two bracketing standard is 6. The limit of quantification solution, reference standard solution and swab sample solution were stable at room temperature for 24h. The method AM-140020-015 and AM-140020-014 have been successfully validated by QC laboratory of Asymchem Life Science (TJ4 site), and could meet the detection requirements.

After wipe sampling, seal the sampling bottle and place it in the pending-testing area. After the sample solution has been tested, seal it and place it at room temperature in the pending-disposal area.

Testing methods for target compounds are listed in Table 6-19.

Table 6-19,  Test methed of the corresponding target in the cleaning validation

| No. | Target | Test method | LOQ for Swab & Rinse | Rinse Limit | Swab Limit |
| --- | --- | --- | --- | --- | --- |
| 1 | ASYM-140020 | AM-140020-015 | 0.0006 mg/mL | 0.00867~0.00950 mg/mL | 507.39μg /100cm2 |
| 2 | ASYM-124852 | AM-140020-014 | 0.00013 mg/mL | 0.00867~0.00950 mg/mL | 507.39μg/100cm2 |

All the test methods shall be validated before performing the cleaning validation. The information of method validation is listed in Table 6-20.

Table 6-20,  Information of method validation

| Method Number | Method transfer/validation protocol no. | Method transfer/validation report no. | Qualified Lab |
| --- | --- | --- | --- |
| AM-140020-015 | AM-140020-015-VP | AM-140020-015-VR | Dept. of Quality Control Lab |
| AM-140020-014 | AM-140020-014-VP | AM-140020-014-VR | Dept. of Quality Control Lab |

Recovery study

According to the requirement of SOP:AC-053 <Sampling and Testing Procedure for cleaning verification>, for the rinsing cleaning: The recovery study on recent 10 compounds of the worst solubility for Asymchem TJ4 has been finished, and all the results are more than 50%, so the 50% is defined as the criteria for Asymchem TJ4 (refer to TJ4-SR-2026-002< Study Report for Rinsing Recovery >). The criteria is applicable to ASYM-14002- project. All swab and rinse results will be corrected for recovery.

The analytical method validation experiments demonstrated that the selected analytical methods are capable of quantitatively detecting the target residues at levels meeting the established acceptance criteria. Target-specific methods were developed based on the established acceptance criteria and specific equipment information, and the LOQ concentrations were determined. The validation results confirmed that the LOQs meet the intended acceptance criteria.

Table 6-21, Recovery study results

| Target residue | MOC | Recovery | RSD |
| --- | --- | --- | --- |
| ASYM-140020 | Hastelloy | 2% level: 98% | 8% |
|  |  | 100% level: 91% | 1% |
|  |  | 200% level: 90% | 0% |
| ASYM-124852 | Hastelloy | 1% level: 90% | 4% |
|  |  | 100% level: 96% | 3% |
|  |  | 200% level: 96% | 2% |

- Dirty Hold Time Study

Dirty Hold time: The period and conditions of storage of equipment before cleaning. The purpose of Dirty Hold Time qualification is to demonstrate that residue ageing does not adversely affect the ability of the validated cleaning process to achieve defined visual and chemical acceptance criteria. The proposed DHT represents the maximum anticipated operational delay between batch completion and cleaning initiation. Validation data generated at this condition will therefore represent a worst-case challenge to the cleaning process.

Based on process historical knowledge and PAR study (Refer to TR-1400200306 <Technical Summary for LSN4182924 (ASYM-140020) Step 1 PAR Study> (current version), TR-1400200307< Technical Summary for LSN4182924 (ASYM-140020) Step 2 PAR Study >), the in-process intermediate ASYM-141366 is stable at 2~8oC for 7 months and cleavage product ASYM-140020 is stable at 43oC for 21d. Since the solid is stable at certain days and all of equipment/auxiliaries are at closed status at rest (solvent evaporation is minimal, which has minimal impact on equipment cleaning), the dirty hold time is currently set in Table 6-22 as a guideline.

Pyridine/ acetic anhydride DMF solution and Oxyma DMF solution are stable at room temperature under nitrogen blanket based on common chemical experience. Since the solution is stable at certain days and all of equipment/auxiliaries are at closed status at rest (solvent evaporation is minimal, which has minimal impact on equipment cleaning), the dirty hold time is currently set in Table 6-22 as a guideline.

Table 6-22 Suggested current dirty hold time

| Equipment ID | Suggested dirty hold time |
| --- | --- |
| TJ4S-1213-Sy15 | 7 days |
| TJ4S-1213-Sy15-R01 | 14 days |
| TJ4S-1213-R22 | 14 days |
| TJ4S-1200-AT02 | 14 days |

As more data from subsequent campaigns is collected, the dirty hold time shall be evaluated based on the production data, and an updated project specific dirty hold time shall be proposed. DHT qualification will be performed as part of the cleaning validation, where cleaning will be executed under the proposed maximum DHT condition and residue levels will be tested. If the validation results meet the acceptance criteria, this will demonstrate with actual data that ageing does not adversely affect cleaning effectiveness. This will be specifically discussed in the cleaning validation report.

For the equipment dirty hold time between batches during the continuous production, all the data for the dirty hold time will be summarized in the cleaning validation report after the cleaning validation, and a specific dirty hold time for this project will be proposed in the report.

During PPQ batch and subsequent commercial manufacturing, manufacturing data shall be collected to further justify the dirty hold time. Should this time limit be exceeded, an investigation shall be initiated and an assessment shall be conducted.

For dirty hold time data collected during manufacturing, two types of dirty hold time data are collected: One is the time between the equipment last use and the start time of cleaning, another is time between the equipment last use and next consecutive use without cleaning.

Two key hold times shall be picked from the subsequent production, that is: the 3rd longest dirty hold time and the 1st longest dirty hold time, further conclude below.

  - If the dirty hold time is equal or less than the 3rd longest dirty hold time, it is regarded that the equipment was safe and no need cleaning, the equipment could be used in the later continuous production.

  - If the “hold time” is demonstrated/verified for one or two times during cleaning validation or subsequent commercial production, the hold time is regarded as alert dirty hold time (still safe since this hold time was demonstrated in the previous batches) and shall pay close attention to the impurity profile of the finished product. Any atypical impurity or OOS to the material specification shall be investigated.

  - If the “hold time” is never demonstrated/verified during cleaning validation or subsequent commercial production, the hold time is regarded as risk dirty hold time. Then shall evaluate the impurity profile of the finished product. Any atypical impurity or OOS to the material specifications should be investigated. If this hold time is proved acceptable, then this dirty hold time should be updated per this new outcome.

- Clean Hold Time

Clean Hold Time: The time between cleaning and equipment reuse, prior to additional cleaning.

The clean hold time is defined as NMT 6 months in the GMP facility after the equipment is cleaned and dried Considering the environment control and equipment storage is same between the GMP facility in TJ4 site and the GMP facility in the site for CHT validation, hence the CHT with NMT 6 months is applicable for the GMP facility in TJ4 site after equipment cleaning (refers to VR-180.01-02.03 <Validation Report of Clean Hold Time (6 months)>). If outside of this time, the equipment shall be cleaned again before next use.

- Product distribution plan during the cleaning validation

During the cleaning validation, the impacted batch involved in the cleaning validation shall be released according to the agreement between Asymchem and customer before the cleaning validation is executed completely. However, there will be interim cleaning validation report(s) to summarize the current cleaning validation results and indicate no impact on the product quality.

Justification for concurrent cleaning validation shall be approved by the site quality head.

QA shall release the TJ4S-1213-Sy15 equipment for the next batch only after the visual and analytical results of the run meet the acceptance criteria.

The interim cleaning validation report shall be approved prior to release of the Step 2 product.

- Impurity profile summary and impurity trending analysis

To further demonstrate the cleaning validation strategy feasible, the impurity profile summary from COA release test results will be collected and impurity trending analysis will be conducted by using control chart. Also, the corresponding impurity evaluation information will be summarized in the cleaning validation report. Cleaning validation forms one component of the overall contamination-control strategy. Mechanisms for continuously assuring cleaning effectiveness, including review of product quality data, impurity trend analysis, deviation investigations, change control and periodic assessment of cleaning process performance.

- Amendment of protocol

Any amendments of protocol during execution of protocol will shall be evaluated and approved.

- Deviation and Change Handling

- Any deviations found during execution of the cleaning validation shall be recorded and investigated according to QA-010 <Deviation Investigation> (current version). Any result exceeding an established acceptance criterion shall be considered a cleaning validation failure unless investigation demonstrates an assignable cause unrelated to cleaning effectiveness. Failure investigations shall be conducted in accordance with QA-010 <Deviation Investigation> (current version), covering root cause analysis, impact assessment and corrective actions. The investigation shall clearly distinguish between failure causes related to cleaning effectiveness and those unrelated to cleaning effectiveness (e.g., analytical method error, sampling error, instrument malfunction, etc.). If an assignable cause unrelated to cleaning effectiveness cannot be demonstrated, the failure shall be considered a cleaning process failure, and an assessment shall be performed to determine whether re-cleaning, re-validation or other corrective actions are required. Failures and their investigation conclusions shall be documented in a deviation report and assessed prior to protocol closure.

- Any changes executed during the cleaning validation shall be recorded and approved according to QA-006<change control> (current version).

- All copies of deviation investigation and changes control shall be attached to cleaning validation report. The information summary of deviation and changes control shall be summarized in Annex 3 and Annex 4. Any deviation or change shall be formally assessed for its impact on protocol validity, data integrity and cleaning process qualification prior to protocol closure. The assessment shall involve the Quality department and be documented in a deviation report or change control record. The assessment shall clearly determine whether the deviation or change affects the cleaning process qualification status and whether corrective actions or additional verification are required. Prior to protocol closure, it shall be confirmed that all deviations and changes have been closed or assessed.

- Action taken when out-of-limit result found

- When the result of residue testing is out of limit, the results shall be recorded and investigation shall be performed according to QA-010 <Deviation Investigation> (current version).

- If any significant unknown impurity peaks are found after comparing the blank of rinsing with sample solution, each unknown impurity shall be calculated as the target residual compound, respectively, and list the test results of all the unknown impurities and fill the maximum result for each unknown impurity in the corresponding cleaning verification test request.

When performing analytical quantitative testing using chromatography method, if the maximum result of a single unknown impurity is greater than the acceptable residue limit, laboratory investigation shall be initiated according to QA-010 <Deviation Investigation> (current version).If no assignable cause is found in the laboratory investigation, production investigation shall be initiated.

- Summary and Conclusion

When the protocol and related tests are finished, the relative validation report shall be completeted in time. The final report shall include the following contents:

- Approval page

- Evaluation of test results

- Conclusion / comments

- Deviation/Changes control list and copies

Successful completion of this protocol will provide documented evidence that the approved cleaning process consistently removes process residues from equipment used in the manufacture of ASYM-140851 Step 1 and maintains equipment in a condition suitable for subsequent manufacture. Successful completion of three consecutive cleaning validation runs meeting all predefined acceptance criteria will establish the qualified state of the cleaning process and support implementation into routine commercial manufacture.

- Revalidation

Revalidation shall be considered under the following circumstances:

- Equipment change or major modification to existing equipment;

- Cleaning procedure change;

- Cleaning solvent change；

- Evidence of cleaning failures (batch failures, cross contamination, routine monitoring);

- Major change on the process.

- References

- Cleaning Validation Master Plan (CVMP-01)

- Cleaning Process Design and Development Report

- Approved cleaning methods (CMa-TJ414002001, CMb-TJ414002001)

- OEB Assessment Report

- Analytical Method Validation Report

- Validation Report of Clean Hold Time (VR-180.01-02.03)

- Relevant SOPs (DS-GMP-003<Equipment cleaning management procedure for GMP production (DS)>, DS-GMP-043<Cleaning Validation (DS)>, QA-010<Deviation Investigation>, QA-006<Change Control>, QAP-005<Quality Risk Management Policy>)

- Process Validation Master Plan for  Manufacturing of LSN4182924/ASYM-140020 (VP-14002001)

- EU GMP Volume 4 Annex 15: Qualification and Validation

- ICH Q7 Good Manufacturing Practice Guide for Active Pharmaceutical Ingredients

- ICH Q9(R1) Quality Risk Management

- FDA Guidance for Industry: Process Validation, General Principles and Practices

- EMA Guideline on Setting Health-Based Exposure Limits for Use in Risk Identification in the Manufacture of Different Medicinal Products in Shared Facilities

Annex 1: Cleaning method list

| Step | Cleaning Method No. | Attachment No. |
| --- | --- | --- |
| 1 | CMa-TJ414002001 (current version) | Attachment 1 for CVP-1400200301.01 |
| 1 | CMb-TJ414002001 (current version) | Attachment 2 for CVP-1400200301.01 |

Annex  2: P&ID diagram for the swab and rinse sampling locations

| Equipment | P&ID number | Attachment No. |
| --- | --- | --- |
| Peptide solid phase synthesizer | Sampling location Diagram for Peptide Solid Synthesizer | Attachment 3 for CVP-1400200301.01 |

Annex  3: Deviation record

| Step/Affected Batches | Deviation Description | Root Cause | Date | Quality impact | Deviation Document No. |
| --- | --- | --- | --- | --- | --- |
| - | - | - | - | - | - |
| - | - | - | - | - | - |

Annex  4: Change record

| Step/Affected Batches | Change description | Date | Quality impact | Change Document No. |
| --- | --- | --- | --- | --- |
| - | - | - | - | - |
| - | - | - | - | - |

Annex 5: The definition of abbreviation

| Abbreviation | Description |
| --- | --- |
| ARL | Allowable Residue Limit |
| MACO | Maximum Acceptable Carry Over |
| MOC | Material of Construction |
| MBS | When Minimum batch size of next product(s) is known, MBS is selected as Minimum batch size of next product(s) When Minimum batch size of next product(s) is unknown, MBS is selected as Minimum batch size of cleaned equipment |
| MTDDnext | Standard maximum therapeutic daily dose of the next product If the next product is not daily dose, it needs transfer into daily dose. |
| MTDDprevious | Standard minimum therapeutic daily dose of the previous product. If the previous product is not daily dose, it needs transfer into daily dose. |
| SF | Safety factor According to observation and statistics of clinical pharmacology, toxicology and application, there will be no pharmacological reaction of 1/1000 MTDD for very sensitive bodies. For safety, take 1000 as safety factor for calculation is acceptable. |
| PDE | Permitted Daily Exposure. |
| OOS | Out of specification |
| ATR | Atypical result |
| NMT | No more than |