MOR Number and Ver /MOR编号和版本号： MOR-TJ414002001.04

Production Site Address/生产厂区地址：

Asymchem Life Science (Tianjin) Co., Ltd. (No.265, South Avenue, Binhai New Area TEDA, Tianjin, 300462, China) (Referred to as TJ4)

凯莱英医药集团（天津）股份有限公司（中国天津市滨海新区经济技术开发区南大街265号）（简称TJ4厂区）

Master Operation Record For

Fmoc-L2-SS peptidyl resin/ASYM-141366

Synthesis Route

合成路线

Code list for reagents, critical RM, RSM and in-process intermediate /product

试剂、原料和过程产物/产品的代码表

| Material Name/物料名称 | Asymchem Code/凯莱英代码 | Client Code/客户代码 | Control Number/内部控制号 |
| --- | --- | --- | --- |
| 2-CTC resin/2-氯-三苯甲基氯树脂 | N/A | N/A | YL005205 |
| OxymaPure/ 2-肟氰乙酸乙酯 | N/A | N/A | YL005180 |
| PyBOP/1H-苯并三唑-1-基氧三吡咯烷基六氟磷酸盐 | N/A | N/A | YL002855 |
| DIPEA/N,N-二异丙基乙胺 | N/A | N/A | YL000555 |
| DIC/N,N’-二异丙基碳二亚胺 | N/A | N/A | YL003564 |
| Fmoc-Gly-OH/芴甲氧羰基-甘氨酸 | ASYM-138200 | N/A | YL004846 |
| Fmoc-L-Phe-OH/芴甲氧羰基-L-苯丙氨酸 | ASYM-138203 | N/A | YL004840 |
| Fmoc-L-N-Me-Asn(Trt)-OH /Fmoc-N-甲基-N’-三苯甲基-天冬氨酰胺 | ASYM-138520 | N/A | YL008617 |
| Fmoc-L-Asn(Trt)-OH/Fmoc-N-三苯甲基-L-天冬酰胺 | ASYM-138519 | N/A | YL004843 |
| Fmoc-L-Ser(tBu)-OH/芴甲氧羰基-叔丁基-L-丝氨酸 | ASYM-132732 | N/A | YL003428 |
| Fmoc-L-Arg(Pbf)-OH/Fmoc-Pbf-精氨酸 | ASYM-138518 | N/A | YL003424 |
| Fmoc-L-Val-OH/芴甲氧羰基-L-缬氨酸 | ASYM-138202 | N/A | YL003110 |
| Fmoc-L-Leu-OH/芴甲氧羰基-L-亮氨酸 | ASYM-138201 | N/A | YL003425 |
| Fmoc-L-α-Me-Phe-OH/(S)-2-((((9H-芴-9-基)甲氧基)羰基)氨基)-2-甲基-3 -苯基丙酸 | ASYM-138210 | N/A | YL008272 |
| Fmoc-L-Glu(OtBu)-OH·H2O /芴甲氧羰基-L-谷氨酸-γ-叔丁酯一水合物 | ASYM-138390 | N/A | YL005741 |
| Fmoc-L-Ala-OH·H2O/芴甲氧羰基-L-丙氨酸一水合物 | ASYM-138204 | N/A | YL006160 |
| Fmoc-L-Orn(Boc)-OH/N-Fmoc-N'-Boc-L-鸟氨酸 | ASYM-124852 | N/A | YL005318 |
| DMF/N,N-二甲基甲酰胺 | N/A | N/A | YL000336 |
| Isopropyl acetate/乙酸异丙酯 | N/A | N/A | YL000932 |
| Acetic anhydride/乙酸酐 | N/A | N/A | SJ000213 |
| Pyriine/吡啶 | N/A | N/A | YL000617 |
| Toluene/甲苯 | N/A | N/A | YL000026 |
| MTBE/甲基叔丁基醚 | N/A | N/A | YL000557 |
| In-process intermediate/过程中间体ASYM-141366 | ASYM-141366 | N/A | ZJ007269 |

Operation Record for 141366

Master Operations Record (MOR) Approvals /MOR批准	7

Operations Record (OR) Issue /OR 签发	8

In-process intermediate Information/过程中间体信息	9

Deviation & Change Log/生产偏差及变更记录	10

Incident Log/事件调查登记	11

Parameter change log in recipe/配方参数修改登记表	14

Equipment Information List/设备使用清单	16

Material Information/原料信息	32

Analytical Method/分析方法	63

Recipe Code against the Unit Operation/基于单元操作的配方命名	64

Operation Instructions/操作指令	65

Process Safety information /工艺安全信息	67

Ins.1/操作1：Dispense/分装	68

1.1.	Dispense 2-CTC resin/分装2-氯-三苯甲基氯树脂	68

1.2.	Dispense Fmoc-Gly-OH/分装芴甲氧羰基-甘氨酸	70

1.3.	Dispense Fmoc-L-Phe-OH/分装芴甲氧羰基-L-苯丙氨酸	72

1.4.	Dispense PyBOP/分装1H-苯并三唑-1-基氧三吡咯烷基六氟磷酸盐	74

1.5.	Dispense OxymaPure/分装2-肟氰乙酸乙酯	76

1.6.	Dispense Fmoc-L-N-Me-Asn(Trt)-OH/分装Fmoc-N-甲基-N’-三苯甲基-天冬氨酰胺	78

1.7.	Dispense OxymaPure/分装2-肟氰乙酸乙酯	80

1.8.	Dispense Fmoc-L-Asn(Trt)-OH/分装Fmoc-N-三苯甲基-L-天冬酰胺	82

1.9.	Dispense OxymaPure/分装2-肟氰乙酸乙酯	84

1.10.	Dispense OxymaPure/分装2-肟氰乙酸乙酯	86

1.11.	Dispense Fmoc-L-Ser(tBu)-OH/分装芴甲氧羰基-叔丁基-L-丝氨酸	88

1.12.	Dispense OxymaPure/分装2-肟氰乙酸乙酯	90

1.13.	Dispense Fmoc-L-Ser(tBu)-OH/分装芴甲氧羰基-叔丁基-L-丝氨酸	92

1.14.	Dispense OxymaPure/分装2-肟氰乙酸乙酯	94

1.15.	Dispense Fmoc-L-Arg(Pbf)-OH/分装Fmoc-Pbf-精氨酸	96

1.16.	Dispense OxymaPure/分装2-肟氰乙酸乙酯	98

1.17.	Dispense Fmoc-L-Val-OH/分装芴甲氧羰基-L-缬氨酸	100

1.18.	Dispense OxymaPure/分装2-肟氰乙酸乙酯	102

1.19.	Dispense Fmoc-L-Leu-OH/分装芴甲氧羰基-L-亮氨酸	104

1.20.	Dispense OxymaPure/分装2-肟氰乙酸乙酯	106

1.21.	Dispense Fmoc-L-α-Me-Phe-OH/分装(S)-2-((((9H-芴-9-基)甲氧基)羰基)氨基)-2-甲基-3 -苯基丙酸	108

1.22.	Dispense OxymaPure/分装2-肟氰乙酸乙酯	110

1.23.	Dispense Fmoc-L-Glu(OtBu)-OH·H2O/分装芴甲氧羰基-L-谷氨酸-γ-叔丁酯一水合物	112

1.24.	Dispense OxymaPure/分装2-肟氰乙酸乙酯	114

1.25.	Dispense Fmoc-L-Ala-OH·H2O/分装芴甲氧羰基-L-丙氨酸一水合物	116

1.26.	Dispense OxymaPure/分装2-肟氰乙酸乙酯	118

1.27.	Dispense Fmoc-L-Leu-OH/分装芴甲氧羰基-L-亮氨酸	120

1.28.	Dispense OxymaPure/分装2-肟氰乙酸乙酯	122

1.29.	Dispense Fmoc-L-Orn(Boc)-OH/分装N-Fmoc-N'-Boc-L-鸟氨酸	124

1.30.	Dispense OxymaPure/分装2-肟氰乙酸乙酯	126

Ins.2/操作2：Equipment confirmation & RecipeConfirmation /设备执行和配方执行	128

2.1.	Equipment confirmation/设备执行	128

Ins.2/操作2：Equipment confirmation & RecipeConfirmation /设备执行和配方执行	129

2.1.Equipment confirmation/设备执行	129

2.2.	Recipe confirmation/配方执行	130

2.3.	Tank confirmation/罐区执行	130

2.4.	Run Recipe /配方运行	132

Ins.3/操作3：Synthesis/合成	133

3.0.	Pre-treatment/预处理	133

3.1.	Resin swelling/树脂溶胀	140

3.2.	（1）Fmoc-Gly-OH activation /芴甲氧羰基-甘氨酸活化	147

3.3.	（1）Fmoc-Gly-OH coupling/芴甲氧羰基-甘氨酸偶联	156

3.4.	Capping/封端	163

3.5.	（2）Deprotection/脱保护	176

3.6.	（2）Fmoc-L-Phe-OH activation/芴甲氧羰基-L-苯丙氨酸活化	186

Signature &Date/签名及日期：                       	186

3.7.	（2）Fmoc-L-Phe-OH coupling/芴甲氧羰基-L-苯丙氨酸偶联	198

3.8.	（3）Fmoc- L-N-Me-Asn(Trt)-OH deprotection/Fmoc-N-甲基-N’-三苯甲基-天冬氨酰胺脱保护	208

3.9.	（3）Fmoc-L-N-Me-Asn(Trt)-OH activation/ Fmoc-N-甲基-N’-三苯甲基-L-天冬氨酰胺活化	217

3.10.	（3）Fmoc-L-N-Me-Asn(Trt)-OH coupling/Fmoc-N-甲基-N’-三苯甲基-L-天冬氨酰胺偶联	227

3.11.	（4）Deprotection/脱保护	236

3.12.	（4）Fmoc-L-Asn(Trt)-OH activation/Fmoc-N-三苯甲基-L-天冬酰胺活化	246

CAUTION/注意：DMF/N，N-二甲基甲酰胺：Irritation of eyes, skin and nose. May cause nausea. /刺激眼睛、皮肤和鼻子。可能引起恶心。	246

Confirm that the above content has been acknowledged./确认已知悉上述内容。	246

Signature &Date/签名及日期：____________________	246

3.13.	（4）Fmoc-L-Asn(Trt)-OH coupling/Fmoc-N-三苯甲基-L-天冬酰胺偶联	256

3.14.	Prepare capping solution/配封端液	274

3.15 Capping/封端	282

3.16.	（5）Deprotection/脱保护	291

3.17.	（5）Fmoc-L-Ser(tBu)-OH activation/芴甲氧羰基-叔丁基-L-丝氨酸活化	304

3.18.	（5）Fmoc-L-Ser(tBu)-OH coupling/芴甲氧羰基-叔丁基-L-丝氨酸偶联	314

3.19.	（6）Deprotection/脱保护	322

3.20.	（6）Fmoc-L-Ser(tBu)-OH activation/芴甲氧羰基-叔丁基-L-丝氨酸活化	331

3.21.	（6）Fmoc-L-Ser(tBu)-OH coupling/芴甲氧羰基-叔丁基-L-丝氨酸偶联	341

3.22.	（7）Deprotection/脱保护	349

3.23.	（7）Fmoc-L-Arg(Pbf)-OH activation/Fmoc-Pbf-精氨酸活化	358

3.24.	（7）Fmoc-L-Arg(Pbf)-OH coupling/ Fmoc-Pbf-精氨酸偶联	369

3.25.	（8）Fmoc-L-Val-OH deprotection/芴甲氧羰基-L-缬氨酸脱保护	378

3.26.	（8）Fmoc-L-Val-OH activation/芴甲氧羰基-L-缬氨酸活化	387

3.27.	（8）Fmoc-L-Val-OH coupling/芴甲氧羰基-L-缬氨酸偶联	397

3.28.	（9）Deprotection/脱保护	405

3.29.	（9）Fmoc-L-Leu-OH activation/芴甲氧羰基-L-亮氨酸活化	414

3.30.	（9）Fmoc-L-Leu-OH coupling/芴甲氧羰基-L-亮氨酸偶联	423

3.31.	（10）Deprotection/脱保护	433

3.32.	（10）Fmoc-L-α-Me-Phe-OH activation/(S)-2-((((9H-芴-9-基)甲氧基)羰基)氨基)-2-甲基-3 -苯基丙酸活化	443

3.33.	 （10）Fmoc-L-α-Me-Phe-OH coupling /(S)-2-((((9H-芴-9-基)甲氧基)羰基)氨基)-2-甲基-3 -苯基丙酸偶联	453

3.34.	（11）Deprotection/脱保护	463

3.35.	（11）Fmoc-L-Glu(OtBu)-OH·H2O activation/芴甲氧羰基-L-谷氨酸-γ-叔丁酯一水合物活化	472

3.36.	（11）Fmoc-L-Glu(OtBu)-OH·H2O coupling/芴甲氧羰基-L-谷氨酸-γ-叔丁酯一水合物偶联	481

3.37.	（11）Deprotection/脱保护	491

3.38.	（12）Fmoc-L-Ala-OH·H2O activation/芴甲氧羰基-L-丙氨酸一水合物活化	501

3.39.	（12）Fmoc-L-Ala-OH·H2O coupling/芴甲氧羰基-L-丙氨酸一水合物偶联	511

3.40.	（13）Deprotection/脱保护	520

3.41.	（13）Fmoc-L-Leu-OH activation/芴甲氧羰基-L-亮氨酸活化	529

3.42.	（13）Fmoc-L-Leu-OH coupling/芴甲氧羰基-L-亮氨酸偶联	540

3.43.	Deprotection/脱保护	550

3.44.	Wash resin/树脂洗涤	559

3.45.	（14）Deprotection/脱保护	562

3.46.	（14）Fmoc-L- Orn(Boc)-OH activation/N-Fmoc-N'-Boc-L-鸟氨酸活化	566

3.47.	（14）Fmoc-L- Orn(Boc)-OH coupling/N-Fmoc-N'-Boc-L-鸟氨酸偶联	575

3.48.	Wash resin/树脂洗涤	596

3.49.	Dry/烘料	604

Ins.4/操作4：Collect/收料& Analyze/检测	606

Appendix/附录	610

1. Supplemental Record/附加页	610

2. Process flow chart /工艺流程图	613

3. English abbreviations /英文缩写表	615

Post-batch Approvals/操作完毕执行	616

Version Instructions/版本说明	617

Master Operations Record (MOR) Approvals /MOR批准

Authored by/编写者:

| Author & Date/撰写人及日期: | {{businessauthorsign}} |
| --- | --- |

Approved By/批准:

| Project Director & Date /项目主管及日期: | {{projectmanager}} |
| --- | --- |

| Production Director & Date /生产部主管及日期: | {{processerdirectorsign}} |
| --- | --- |

| QA project Manager & Date /QA项目经理及日期: | {{qa}} |
| --- | --- |

Client Approval & Date/客户审批及日期:      {{customersign}}

(The last signature date on this page is the effective date. /本页最后一个签字日期即为生效日期)

Operations Record (OR) Issue /OR 签发

Batch Number/生产批号:

Approved by QA/QA批准人:

Date/日期:

In-process intermediate Information/过程中间体信息

Expected yield/预期收率：60~130%

Calculation of theoretical output weight/理论产量计算：

Theoretical peptidyl resin output/理论肽树脂产出重量 = resin weight/树脂重量+ resin weight/树脂重量× degree of substitution/取代度 ×net MW gain 中间体分子量增量÷ 1000 kg/mol] =           kg +

kg×         mol/kg × 2787.03 g/mol ÷ 1000 kg/mol =            kg

Calculation of expected gain output weight/预期产量计算：

Expected output =Theoretical output × Expected yield

预期产量=理论肽树脂重量×预期收率=          * 60%  ~          * 130% =              ~

kg

Actual output/实际产量：

Physical output of peptidyl resin weight/实际肽树脂产量：             kg

Calculation of weight yield /实际收率计算方法:

Weight yield 增重收率= (Actual output 实际产量/Theoretical output理论产量)×100% =(__________kg/__________kg) ×100% =             %

According to the operation record, the Lot# of the in-process intermediate is assigned as/

根据操作记录，该批过程中间体的批号为：

Deviation & Change Log/生产偏差及变更记录

The corresponding deviations and changes related to this batch should be recorded in the following Table:

与本次相关的偏差和变更应该在下表中记录：

| No. / 序号 | Operation Instruction. No./ 操作指令编号 | Deviation report No. / 偏差报告编号 | No. / 序号 | Operation Instruction No. / 操作指令编号 | Change file No. / 变更文件编号 |
| --- | --- | --- | --- | --- | --- |
| 1 |  |  | 1 |  |  |
| 2 |  |  | 2 |  |  |
| 3 |  |  | 3 |  |  |
| 4 |  |  | 4 |  |  |
| 5 |  |  | 5 |  |  |
| 6 |  |  | 6 |  |  |
| 7 |  |  | 7 |  |  |
| 8 |  |  | 8 |  |  |
| 9 |  |  | 9 |  |  |

Incident Log/事件调查登记

The corresponding incident related to this batch should be recorded in the following Table: /与本次相关的异常事件应该在下表中记录：

| No. / 序号 | Operation Instruction.No./操作指令编号 | Incident No./事件编号 | Incident brief description/事件简述 | Filled by/Date 填写人/日期 |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |

Parameter change log in recipe/配方参数修改登记表

| No./ 序号 | Instruction./ 操作号 | Current parameter/ 当前参数 | Updated parameter/ 更新后的参数 | Revision justification and reference document (if any)/ 修改原因以及文件索引（如有） | Performed by/Date 执行人/日期 | Checked by/Date 核实人/日期 |
| --- | --- | --- | --- | --- | --- | --- |

| No./ 序号 | Instruction./ 操作号 | Current parameter/ 当前参数 | Updated parameter/ 更新后的参数 | Revision justification and reference document (if any)/ 修改原因以及文件索引（如有） | Performed by/Date 执行人/日期 | Checked by/Date 核实人/日期 |
| --- | --- | --- | --- | --- | --- | --- |

Equipment Information List/设备使用清单

Pre-batch Equipment Confirmation Checklist/每次生产前设备检查执行表

| Facility and Module/厂房和车间 | Operation Instruction No. 操作指令 编号 | Equipment/ 设备名称 | Function/ 用途 | Level of cleacing check/清洁检测核查级别 | Performed by & Date/ 执行人及日期 | Checked by & Date/ 核实人及日期 |
| --- | --- | --- | --- | --- | --- | --- |
| Building 12, Module 13/ 12号楼13车间 | 2~3 | Peptide Solid Phase Synthesizer/ 多肽固相合成仪 Equipment # 设备编号: TJ4S-1213-Sy15 Size /设备大小:3000 L MOC :Hastelloy/材质：哈氏合金 | Peptide Solid /固相合成& Dry/烘料 | Level Ⅰ Level Ⅱ Cleaning record number/清洁记录编号： ___________________________ |  |  |
|  |  | Activation tank /激活罐 Equipment # 设备编号: TJ4S-1213-Sy15-R01 Size /设备大小:1500 L MOC :Stainless Steel/材质：不锈钢 | Activate/活化& Dissolve/溶解 | Level Ⅰ Level Ⅱ Cleaning record number/清洁记录编号： ___________________________ |  |  |
| Building 12, Module 13/ 12号楼13车间 | 3 | Agitator tank/ 搅拌罐 Equipment # 设备编号: TJ4S-1213-SPS01-AT01 Size /设备大小:3000 L MOC :Stainless Steel/材质：不锈钢 | Prepare piperidine/DMF solution/ 配哌啶/DMF溶液 | Level 0 Level Ⅰ Cleaning record number/清洁记录编号： ___________________________ |  |  |
| Building 12, Module 13/ 12号楼13车间 | 3 | Agitator tank / 搅拌罐 Equipment # 设备编号: TJ4S-1213-SPS01-AT02 Size /设备大小:3000 L MOC :Stainless Steel/材质：不锈钢 | Prepare piperidine/DMF solution/ 配哌啶/DMF溶液 | Level 0 Level Ⅰ Cleaning record number/清洁记录编号： ___________________________ |  |  |
| Building 12, Module 13/ 12号楼13车间 | 3.13.4 | Agitator tank/ 搅拌罐 Equipment # 设备编号: TJ4S-1200-AT02 Size /设备大小:500 L MOC : Hastelloy/材质：哈式合金 | Prepared OxymaPure/ DMF solution/配2-肟氰乙酸乙酯的N,N-二甲基甲酰胺溶液 | Level 0 Level Ⅰ Cleaning record number/清洁记录编号： ___________________________ |  |  |
| Building 12, Module 13/ 12号楼13车间 | 3.13.4 | Reactor/ 反应釜 Equipment # 设备编号: TJ4S-1213-R22 Size /设备大小:1000 L MOC : Glass lined /材质：搪玻璃 | Prepared Ac₂O / Pyridine/DMF solution/配乙酸酐 / 吡啶/N,N-二甲基甲酰胺溶液 | Level 0 Level Ⅰ Cleaning record number/清洁记录编号： ___________________________ |  |  |
| Building 12/ 12号楼 | 1 | Glove box/手套箱 Equipment # 设备编号: _________ MOC : Stainless Steel /材质：不锈钢 | Subpackage/分装 | Level Ⅰ |  |  |
| Building 12/ 12号楼 | 1 | Glove box/手套箱 Equipment # 设备编号: _________ MOC : Stainless Steel /材质：不锈钢 | Subpackage/分装 | Level Ⅰ |  |  |
| Building 12/ 12号楼 | 1 | Glove box/手套箱 Equipment # 设备编号: _________ MOC : Stainless Steel /材质：不锈钢 | Subpackage/分装 | Level Ⅰ |  |  |
| Building 12/ 12号楼 | 1 | Glove box/手套箱 Equipment # 设备编号: _________ MOC : Stainless Steel /材质：不锈钢 | Subpackage/分装 | Level Ⅰ |  |  |
| Building 12/ 12号楼 | 3.2.1 | Metering pump/ 计量泵 Equipment # 设备编号: | DIPEA addition /加N,N-二异丙基乙胺 | Level Ⅰ |  |  |
|  |  | Pipeline hose/管路 _Equipment #设备编号:_________ MOC : Stainless Steel /材质：不锈钢 | DIPEA addition /加N,N-二异丙基乙胺 | Level Ⅰ |  |  |
| Building 12, Module 13/ 12号楼 | 3.4.2 | Peristaltic pump/ 蠕动泵 Equipment # 设备编号: | MeOH addition /加甲醇 | Level 0 |  |  |
|  |  | Stainless steel hose/ 不锈钢软管 Equipment # 设备编号: | Methanol addition/加甲醇 | Level Ⅰ |  |  |
| Building 12, Module 13/ 12号楼13车间 | 3.4.3 | Peristaltic pump/ 蠕动泵 Equipment # 设备编号: | DIPEA addition /加N,N-二异丙基乙胺 | Level 0 |  |  |
|  |  | Single use flexible hose/一次性加料管路 _Equipment #设备编号:_________ | DIPEA addition /加N,N-二异丙基乙胺 | Level Ⅰ |  |  |
| Building 12, Module 13/ 12号楼13车间 | 3.6.8 | Metering pump/ 计量泵 Equipment # 设备编号: | DIPEA addition /加N,N-二异丙基乙胺 | Level Ⅰ Level Ⅱ Cleaning record number/清洁记录编号： ___________________________ |  |  |
|  |  | Pipeline hose/管路 _Equipment #设备编号:_________ MOC : Stainless Steel /材质：不锈钢 | DIPEA addition /加N,N-二异丙基乙胺 | Level Ⅰ |  |  |
| Building 12, Module 13/ 12号楼13车间 | 3.9.7 | Metering pump/ 计量泵 Equipment # 设备编号: | DIC addition /加N,N-二异丙基碳二亚胺 | Level Ⅰ |  |  |
|  |  | Pipeline hose/管路 Equipment #设备编号:_________ MOC : Stainless Steel /材质：不锈钢 | DIC addition /加N,N-二异丙基碳二亚胺 | Level Ⅰ |  |  |
| Building 12, Module 13/ 12号楼13车间 | 3.12.7 | Metering pump/ 计量泵 Equipment # 设备编号: | DIC addition /加N,N-二异丙基碳二亚胺 | Level Ⅰ |  |  |
|  |  | Pipeline hose/管路 Equipment #设备编号:_________ MOC : Stainless Steel /材质：不锈钢 | DIC addition /加N,N-二异丙基碳二亚胺 | Level Ⅰ |  |  |
| Building 12, Module 13/ 12号楼13车间 | 3.13.5 | Diaphragm Pump/隔膜泵 Equipment #设备编号: _________ | DMF addition /加N,N-二甲基甲酰胺 | Level Ⅰ |  |  |
|  |  | Flexible hose/软管 Equipment #设备编号:_________ MOC : Stainless Steel /材质：不锈钢 | DMF addition /加N,N-二甲基甲酰胺 | Level Ⅰ |  |  |
| Building 12, Module 13/ 12号楼13车间 | 3.13.7 | Diaphragm pump/ 隔膜泵 Equipment # 设备编号: | OxymaPure/ DMF addition/加2-肟氰乙酸乙酯的N,N-二甲基甲酰胺溶液 | Level Ⅰ |  |  |
|  |  | Pipeline/ 管路 Equipment # 设备编号: 140020-step01-P01 MOC : Stainless Steel /材质：不锈钢 | OxymaPure/ DMF addition/加2-肟氰乙酸乙酯的N,N-二甲基甲酰胺溶液 | Level Ⅰ |  |  |
| Building 12, Module 13/ 12号楼13车间 | 3.13.7 | Peristaltic pump/ 蠕动泵 Equipment # 设备编号: | DIC addition /加N,N-二异丙基碳二亚胺 | Level 0 |  |  |
|  |  | Single use flexible hose/ 一次性加料管路 Equipment # 设备编号: | DIC addition /加N,N-二异丙基碳二亚 | Released for single use/一次性使用放行 |  |  |
| Building 12, Module 13/ 12号楼13车间 | 3.14.1 | Diaphragm Pump/隔膜泵 Equipment #设备编号: _________ | DMF addition /加N,N-二甲基甲酰胺 | Level Ⅰ |  |  |
|  |  | Flexible hose/软管 Equipment #设备编号:_________ MOC : Stainless Steel /材质：不锈钢 | DMF addition /加N,N-二甲基甲酰胺 | Level Ⅰ |  |  |
| Building 12, Module 13/ 12号楼13车间 | 3.14.3 | Peristaltic pump/ 蠕动泵 Equipment # 设备编号: | Ac₂O addition /加乙酸酐 | Level 0 |  |  |
|  |  | Single use flexible hose/ 一次性加料管路 Equipment # 设备编号: | Ac₂O addition /加乙酸酐 | Released for single use/一次性使用放行 |  |  |
| Building 12, Module 13/ 12号楼13车间 | 3.14.2 | Peristaltic pump/ 蠕动泵 Equipment # 设备编号: | Pyridine addition /加吡啶 | Level 0 |  |  |
|  |  | Single use flexible hose/ 一次性加料管路 Equipment # 设备编号: | Pyridine addition /加吡啶 | Released for single use/一次性使用放行 |  |  |
| Building 12, Module 13/ 12号楼13车间 | 3.15.1 | Diaphragm pump/ 隔膜泵 Equipment # 设备编号: | Capping solution addition/加封端液 | Level 0 Level Ⅰ Cleaning record number/清洁记录编号： ___________________________ |  |  |
|  |  | Pipeline/ 管路 Equipment # 设备编号: 140020-step01-P02 | Capping solution addition/加封端液 | Level Ⅰ |  |  |
| Building 12, Module 13/ 12号楼13车间 | 3.17.7 | Metering pump/ 计量泵 Equipment # 设备编号: | DIC addition /加N,N-二异丙基碳二亚胺 | Level Ⅰ |  |  |
|  |  | Pipeline hose/管路 Equipment #设备编号:_________ MOC : Stainless Steel /材质：不锈钢 | DIC addition /加N,N-二异丙基碳二亚胺 | Level Ⅰ |  |  |
| Building 12, Module 13/ 12号楼13车间 | 3.20.7 | Metering pump/ 计量泵 Equipment # 设备编号: | DIC addition /加N,N-二异丙基碳二亚胺 | Level Ⅰ |  |  |
|  |  | Pipeline hose/管路 Equipment #设备编号:_________ MOC : Stainless Steel /材质：不锈钢 | DIC addition /加N,N-二异丙基碳二亚胺 | Level Ⅰ |  |  |
| Building 12, Module 13/ 12号楼13车间 | 3.23.7 | Metering pump/ 计量泵 Equipment # 设备编号: | DIC addition /加N,N-二异丙基碳二亚胺 | Level Ⅰ |  |  |
|  |  | Pipeline hose/管路 Equipment #设备编号:_________ MOC : Stainless Steel /材质：不锈钢 | DIC addition /加N,N-二异丙基碳二亚胺 | Level Ⅰ |  |  |
| Building 12, Module 13/ 12号楼13车间 | 3.26.7 | Metering pump/ 计量泵 Equipment # 设备编号: | DIC addition /加N,N-二异丙基碳二亚胺 | LevelⅠ |  |  |
|  |  | Pipeline hose/管路 Equipment #设备编号:_________ MOC : Stainless Steel /材质：不锈钢 | DIC addition /加N,N-二异丙基碳二亚胺 | Level Ⅰ |  |  |
| Building 12, Module 13/ 12号楼13车间 | 3.29.7 | Metering pump/ 计量泵 Equipment # 设备编号: | DIC addition /加N,N-二异丙基碳二亚胺 | Level Ⅰ |  |  |
|  |  | Pipeline hose/管路 Equipment #设备编号:_________ MOC : Stainless Steel /材质：不锈钢 | DIC addition /加N,N-二异丙基碳二亚胺 | Level Ⅰ |  |  |
| Building 12, Module 13/ 12号楼13车间 | 3.32.7 | Metering pump/ 计量泵 Equipment # 设备编号: | DIC addition /加N,N-二异丙基碳二亚胺 | Level Ⅰ |  |  |
|  |  | Pipeline hose/管路 Equipment #设备编号:_________ MOC : Stainless Steel /材质：不锈钢 | DIC addition /加N,N-二异丙基碳二亚胺 | Level Ⅰ |  |  |
| Building 12, Module 13/ 12号楼13车间 | 3.35.7 | Metering pump/ 计量泵 Equipment # 设备编号: | DIC addition /加N,N-二异丙基碳二亚胺 | Level Ⅰ |  |  |
|  |  | Pipeline hose/管路 Equipment #设备编号:_________ MOC : Stainless Steel /材质：不锈钢 | DIC addition /加N,N-二异丙基碳二亚胺 | Level Ⅰ |  |  |
| Building 12, Module 13/ 12号楼13车间 | 3.38.7 | Metering pump/ 计量泵 Equipment # 设备编号: | DIC addition /加N,N-二异丙基碳二亚胺 | Level Ⅰ |  |  |
|  |  | Pipeline hose/管路 Equipment #设备编号:_________ MOC : Stainless Steel /材质：不锈钢 | DIC addition /加N,N-二异丙基碳二亚胺 | Level Ⅰ |  |  |
| Building 12, Module 13/ 12号楼13车间 | 3.41.7 | Metering pump/ 计量泵 Equipment # 设备编号: | DIC addition /加N,N-二异丙基碳二亚胺 | Level Ⅰ |  |  |
|  |  | Pipeline hose/管路 Equipment #设备编号:_________ MOC : Stainless Steel /材质：不锈钢 | DIC addition /加N,N-二异丙基碳二亚胺 | Level Ⅰ |  |  |
| Building 12, Module 13/ 12号楼13车间 | 3.46.7 | Metering pump/ 计量泵 Equipment # 设备编号: | DIC addition /加N,N-二异丙基碳二亚胺 | Level Ⅰ |  |  |
|  |  | Pipeline hose/管路 Equipment #设备编号:_________ MOC : Stainless Steel /材质：不锈钢 | DIC addition /加N,N-二异丙基碳二亚胺 | Level Ⅰ |  |  |
| Building 12, Module 13/ 12号楼13车间 | 3.47.7/ 3.47.9/3.48.1 | Metering pump/ 计量泵 Equipment # 设备编号: | Toluene addition /加甲苯 | Level Ⅰ |  |  |
|  |  | Pipeline hose/管路 Equipment #设备编号:_________ MOC : Stainless Steel /材质：不锈钢 | Toluene addition /加甲苯 | Level Ⅰ |  |  |

Note：The operator should fill this table before the equipment is used/

备注：此表格应该由操作者在使用设备前进行填写。

Material Information/原料信息

Raw Material Summary Table/原料使用汇总表

Limiting material:/基准物料*：

| Operation Instruction. No. 操作指令编号 | Raw Material Name 原料名称 | Average resin Substitution/取代度 (mmol/g)** | Standard Batch Quantity(kg) 标准投料量 | Total loading Moles /投料总摩尔量（mol）** | Weighing accuracy/ 称量精确度 | Acceptable weighing range/ 可接受称量范围（kg） | Actual quantity Charged 实际投料量 （kg） |
| --- | --- | --- | --- | --- | --- | --- | --- |
| *1.1/3.1.1 | 2-CTC resin/2-氯-三苯甲基氯树脂 Average resin substitution/平均树脂取代度 = mmol/g |  | 48.00 |  | ±0.2% | 47.90~48.10 |  |
|  |  |  | Entered by & Date/ 填写人及日期： Reviewed by& Date / 复核人及日期： |  |  |  |  |

*The material is the basis of the other material input in one step, that is, this material is deemed as 1.0 eq. Other material amounts are calculated based on this basis. /该物料是其他物料投料的基础，也就是说，如果该物料为1.0eq.，则其他物料的数量以该物料为基础进行计算。

**The Average resin substitution and Total loading moles are calculated by using the QA approved validated spreadsheet in batch record attachment.

Limiting material mole (1.00 eq) = Total loading moles (mol)/ 限量物料摩尔数（1.00 eq）= 总上载摩尔数（mol）= ____________ (mol)

Limiting material weight (W) = Actual quantity charged (kg)/ 限量物料重量（W）= 实际投料量（kg）= ____________ (kg)

Limiting material volume (Vol) = Actual quantity charged (L)/ 限量物料体积（Vol）= 实际投料量（L）= ____________ (L)

The acceptance criteria of substitution for 2-CTC resin is 1.0–1.2 mmol/g / 2 -氯三苯甲基氯树脂的取代度合格标准为：1.0–1.2 mmol/g。

Non-Limiting material:/非基准物料

For below table. /对于下表：

* Standard Batch Qty标准投料量=Limiting material quantity/基准物料数量×Qty Ratio to limiting material/与基准物料质量比；

Acceptable weighing range可接受称量范围=Standard Batch Qty标准投料量×（1±Weighing accuracy称量精确度）

* For materials that need to be converted with assay, need to indicate in parentheses after the Standard Batch Qty. value that the value means the quantity converted with assay. /对于需要进行含量折算的物料，需要在标准投料量数值后加括号注明该数值是指折算含量后的投料量。

** The meaning of the value should be the same as the value of ‘Standard Batch Qty.’. For materials that need to be converted with assay, need to indicate in parentheses after the ‘Acceptable Weighing Range’ value that the value means the quantity converted with assay. /数值的含义应该与“标准投料量”一列中数值的含义相同，对于需要进行含量折算的物料，需要在“可接受范围”数值后，加括号注明该数值是指折算含量后的投料量。

| Operation Instruction No. 操作指令编号 | Raw Material Name 原料名称 | MW 原料分子量 (g/mol) | Density 密度（g/cm3） | Volume Ratio to limiting material/ 与基准原料体积比 | Mol Ratio to limiting material/ 与基准原料摩尔比 | Qty Ratio to limiting material/ 与基准原料质量比 | Standard Batch Qty 标准投料量（kg） | Weighing accuracy 称量精确度 | Acceptable weighing range 可接受称量范围（kg） | Qty Charged/ 实际投料量 （kg） |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3.0.1 | DMF N,N-二甲基甲酰胺 | 73.10 | 0.948 | - | - | - | 200-300 | N/A | - |  |
| 3.0.1 | DMF N,N-二甲基甲酰胺 | 73.10 | 0.948 | - | - | - | 200-300 | N/A | - |  |
| 3.1.2 | DMF/N,N-二甲基甲酰胺 | 73.10 | 0.948 | 7 | - | 6.636 | 318.53 | ±3.0% | 308.97-328.09 |  |
| 3.1.4 | DMF/N,N-二甲基甲酰胺 | 73.10 | 0.948 | 1 | - | 0.948 | 45.50 | ±2 kg | 43.50-47.50 |  |
| 3.1.7 | DMF/N,N-二甲基甲酰胺 | 73.10 | 0.948 | 5 | - | 4.740 | 227.52 | ±3.0% | 220.69-234.35 |  |
| 3.2.1 | DMF N,N-二甲基甲酰胺 | 73.10 | 0.948 | 4.25 | - | 4.029 | 193.39 | ±3.0% | 187.59-199.19 |  |
| 1.2/3.2.2 | Fmoc-Gly-OH/芴甲氧羰基-甘氨酸 | 297.31 | - | - | 2.25 | - |  | ±3.0% |  |  |
| 3.2.3 | DMF/N,N-二甲基甲酰胺 | 73.10 | 0.948 | 1 | - | 0.948 | 45.50 | ±2 kg | 43.50-47.50 |  |
| 3.2.4 | DIPEA/N,N-二异丙基乙胺 | 129.25 | 0.742 | - | 6.00 | - | - | ±3.0% |  |  |
| （1）Fmoc-Gly-OH/芴甲氧羰基-甘氨酸 |  |  |  |  |  |  |  |  |  |  |
| 3.3.2 | DMF/N,N-二甲基甲酰胺 | 73.10 | 0.948 | 1 | - | 0.948 | 45.50 | ±2 kg | 43.50-47.50 |  |
| 3.3.6 | DMF/N,N-二甲基甲酰胺 | 73.10 | 0.948 | 8 | - | 7.584 | 364.03 | ±3.0% | 353.11-374.95 |  |
| 3.4.1 | DMF/N,N-二甲基甲酰胺 | 73.10 | 0.948 | 9 | - | 8.532 | 409.54 | ±3.0% | 397.25-421.83 |  |
| 3.4.2 | Methanol/甲醇 | 32.04 | - | - | 10.00 | - |  | ±3.0% |  |  |
| 3.4.3 | DIPEA/N,N-二异丙基乙胺 | 129.24 | 0.742 | - | 5.00 | - |  | ±3.0% |  |  |
| 3.4.6 | DMF/N,N-二甲基甲酰胺 | 73.10 | 0.948 | 9 | - | 8.532 | 409.54 (each wash/单次) | ±3.0% | 397.25-421.83 | See below/见下行 |
|  |  | Qty Charged/实际投料量（kg）：1st： kg；2nd： kg；3rd： kg； |  |  |  |  |  |  |  |  |
| （2）Fmoc-L-Phe-OH/芴甲氧羰基- L-苯丙氨酸 |  |  |  |  |  |  |  |  |  |  |
| 3.5.1 | 20% (V/V) piperidine/ DMF solution/20%（V/V）哌啶/DMF溶液 | - | 0.931 | 8 | - | 7.448 | 357.50 | ±3.0% | 346.77-368.23 |  |
| 3.5.5 | IPAC/乙酸异丙酯 | 102.13 | 0.873 | 10 | - | 8.730 | 419.04 (each wash/单次) | ±3.0% | 406.47-431.61 | See below/见下行 |
|  |  | Qty Charged/实际投料量（kg）：1st： kg；2nd： kg；3rd： kg；4th： kg |  |  |  |  |  |  |  |  |
| 3.5.6 | DMF/N,N-二甲基甲酰胺 | 73.10 | 0.948 | 12 | - | 11.376 | 546.05 | ±3.0% | 529.67-562.43 |  |
| 3.5.8 | DMF/N,N-二甲基甲酰胺 | 73.10 | 0.948 | 12 | - | 11.376 | 546.05 | ±3.0% | 529.67-562.43 |  |
| 3.6.1 | DMF/N,N-二甲基甲酰胺 | 73.10 | 0.948 | 4 | - | 3.792 | 182.02 | ±3.0% | 176.56-187.48 |  |
| 1.3/3.6.2 | Fmoc-L-Phe-OH/芴甲氧羰基-L-苯丙氨酸 | 387.43 | - | - | 2.00 | - |  | ±3.0% |  |  |
| 1.4/3.6.5 | PyBOP/1H-苯并三唑-1-基氧三吡咯烷基六氟磷酸盐 | 520.39 | - | - | 2.00 | - |  | ±3.0% |  |  |
| 1.5/3.6.6 | OxymaPure/2-肟氰乙酸乙酯 | 142.11 | - | - | 2.00 | - |  | ±3.0% |  |  |
| 3.6.7 | DMF/N,N-二甲基甲酰胺 | 73.10 | 0.948 | 1 | - | 0.948 | 45.50 | ±2 kg | 43.50-47.50 |  |
| 3.6.8 | DIPEA/N,N-二异丙基乙胺 | 129.24 | 0.742 | - | 3.00 | - |  | ±3.0% |  |  |
| 3.7.1 | DMF/N,N-二甲基甲酰胺 | 73.10 | 0.948 | 2 | - | 1.896 | 91.01 | ±3.0% | 88.28-93.74 |  |
| 3.7.3 | DMF/N,N-二甲基甲酰胺 | 73.10 | 0.948 | 1 | - | 0.948 | 45.50 | ±2 kg | 43.50-47.50 |  |
| 3.7.7 | DMF/N,N-二甲基甲酰胺 | 73.10 | 0.948 | 8 | - | 7.584 | 364.03 (each wash/单次) | ±3.0% | 353.11-374.95 | See below/见下行 |
|  |  | Qty Charged/实际投料量（kg）：1st： kg；2nd： kg；3rd： kg； |  |  |  |  |  |  |  |  |
| （3）Fmoc-L-N-Me-Asn(Trt)-OH /Fmoc-N-甲基-N’-三苯甲基-天冬氨酰胺 |  |  |  |  |  |  |  |  |  |  |
| 3.8.1 | 20% (V/V) piperidine/DMF solution/20%（V/V）哌啶/DMF溶液 | - | 0.931 | 8 | - | 7.448 | 357.50 | ±3.0% | 346.77-368.23 |  |
| 3.8.5 | IPAC/乙酸异丙酯 | 102.13 | 0.873 | 10 | - | 8.730 | 419.04 (each wash/单次) | ±3.0% | 406.47-431.61 | See below/见下行 |
|  |  | Qty Charged/实际投料量（kg）：1st： kg；2nd： kg；3rd： kg；4th： kg； |  |  |  |  |  |  |  |  |
| 3.8.6 | DMF/N,N-二甲基甲酰胺 | 73.10 | 0.948 | 12 | - | 11.376 | 546.05 | ±3.0% | 529.67-562.43 |  |
| 3.8.8 | DMF/N,N-二甲基甲酰胺 | 73.10 | 0.948 | 12 | - | 11.376 | 546.05 | ±3.0% | 529.67-562.43 |  |
| 3.9.1 | DMF/N,N-二甲基甲酰胺 | 73.10 | 0.948 | 4 | - | 3.792 | 182.02 | ±3.0% | 176.56-187.48 |  |
| 1.6/3.9.2 | Fmoc-L-N-Me-Asn(Trt)-OH /Fmoc-N-甲基-N’-三苯甲基-L-天冬氨酰胺 | 610.70 | - | - | 1.75 | - |  | ±3.0% |  |  |
| 1.7/3.9.5 | OxymaPure/2-肟氰乙酸乙酯 | 142.11 | - | - | 1.80 | - |  | ±3.0% |  |  |
| 3.9.6 | DMF/N,N-二甲基甲酰胺 | 73.10 | 0.948 | 2 | - | 1.896 | 91.01 | ±3.0% | 88.28-93.74 |  |
| 3.9.7 | DIC/N,N'-二异丙基碳二酰亚胺 | 126.20 | 0.815 | - | 2.00 | - |  | ±3.0% |  |  |
| 3.10.2 | DMF/N,N-二甲基甲酰胺 | 73.10 | 0.948 | 1 | - | 0.948 | 45.50 | ±2 kg | 43.50-47.50 |  |
| 3.10.6 | DMF/N,N-二甲基甲酰胺 | 73.10 | 0.948 | 8 | - | 7.584 | 364.03 (each wash/单次) | ±3.0% | 353.11-374.95 | See below/见下行 |
|  |  | Qty Charged/实际投料量（kg）：1st： kg；2nd： kg；3rd： kg； |  |  |  |  |  |  |  |  |
| （4）Fmoc-L-Asn(Trt)-OH/Fmoc-N-三苯甲基-L-天冬酰胺 |  |  |  |  |  |  |  |  |  |  |
| 3.11.1 | 20% (V/V) piperidine/DMF solution/20%（V/V）哌啶/DMF溶液 | - | 0.931 | 8 | - | 7.448 | 357.50 | ±3.0% | 346.77-368.23 |  |
| 3.11.5 | IPAC/乙酸异丙酯 | 102.13 | 0.873 | 10 | - | 8.730 | 419.04 (each wash/单次) | ±3.0% | 406.47-431.61 | See below/见下行 |
|  |  | Qty Charged/实际投料量（kg）：1st： kg；2nd： kg；3rd： kg；4th： kg； |  |  |  |  |  |  |  |  |
| 3.11.6 | DMF/N,N-二甲基甲酰胺 | 73.10 | 0.948 | 12 | - | 11.376 | 546.05 | ±3.0% | 529.67-562.43 |  |
| 3.11.8 | DMF/N,N-二甲基甲酰胺 | 73.10 | 0.948 | 12 | - | 11.376 | 546.05 | ±3.0% | 529.67-562.43 |  |
| 3.12.1 | DMF/N,N-二甲基甲酰胺 | 73.10 | 0.948 | 4 | - | 3.792 | 182.02 | ±3.0% | 176.56-187.48 |  |
| 1.8/3.12.2 | Fmoc-L-Asn(Trt)-OH/Fmoc-N-三苯甲基-L-天冬酰胺 | 596.67 | - | - | 2.50 | - |  | ±3.0% |  |  |
| 1.9/3.12.5 | OxymaPure/2-肟氰乙酸乙酯 | 142.11 | - | - | 2.60 | - |  | ±3.0% |  |  |
| 3.12.6 | DMF/N,N-二甲基甲酰胺 | 73.10 | 0.948 | 2 | - | 1.896 | 91.01 | ±3.0% | 88.28-93.74 |  |
| 3.12.7 | DIC/N,N'-二异丙基碳二酰亚胺 | 126.20 | 0.815 | - | 2.90 | - |  | ±3.0% |  |  |
| 3.13.2 | DMF/N,N-二甲基甲酰胺 | 73.10 | 0.948 | 1 | - | 0.948 | 45.50 | ±2 kg | 43.50-47.50 |  |
| 3.13.5 | DMF/N,N-二甲基甲酰胺 | 73.10 | 0.948 | 1 | - | 0.948 | 45.50 | ±2 kg | 43.50-47.50 |  |
| 1.10/3.13.6 | OxymaPure/2-肟氰乙酸乙酯 | 142.11 | - | - | 1.00 | - |  | ±3.0% |  |  |
| 3.13.7 | DIC/N,N'-二异丙基碳二酰亚胺 | 126.20 | 0.815 | - | 1.10 | - |  | ±3.0% |  |  |
| 3.13.8 | DMF/N,N-二甲基甲酰胺 | 73.10 | 0.948 | 8 | - | 7.584 | 364.03 (each wash/单次) | ±3.0% | 353.11-374.95 | See below/见下行 |
|  |  | Qty Charged/实际投料量（kg）：1st： kg；2nd： kg；3rd： kg； |  |  |  |  |  |  |  |  |
| 3.14.1 | DMF/N,N-二甲基甲酰胺 | 73.10 | 0.948 | 6 | - | 5.688 | 273.02 | ±3.0% | 264.83-281.21 |  |
| 3.14.2 | Pyridine/吡啶 | 79.10 | - | - | 1.50 | - |  | ±3.0% |  |  |
| 3.14.3 | Acetic anhydride/乙酸酐 | 102.09 | - | - | 1.25 | - |  | ±3.0% |  |  |
| 3.15.2 | DMF/N,N-二甲基甲酰胺 | 73.10 | 0.948 | 1 | - | 0.948 | 45.50 | ±2kg | 43.50-47.50 |  |
| 3.15.6 | DMF/N,N-二甲基甲酰胺 | 73.10 | 0.948 | 8 | - | 7.584 | 364.03 (each wash/单次) | ±3.0% | 353.11-374.95 | See below/见下行 |
|  |  | Qty Charged/实际投料量（kg）：1st： kg；2nd： kg；3rd： kg； |  |  |  |  |  |  |  |  |
| （5）Fmoc-L-Ser(tBu)-OH/芴甲氧羰基-叔丁基-L-丝氨酸 |  |  |  |  |  |  |  |  |  |  |
| 3.16.1 | 20% (V/V) piperidine/DMF solution/20%（V/V）哌啶/DMF溶液 | - | 0.931 | 8 | - | 7.448 | 357.50 | ±3.0% | 346.77-368.23 |  |
| 3.16.4 | 20% (V/V) piperidine/DMF solution/20%（V/V）哌啶/DMF溶液 | - | 0.931 | 8 | - | 7.448 | 357.50 | ±3.0% | 346.77-368.23 |  |
| 3.16.8 | IPAC/乙酸异丙酯 | 102.13 | 0.873 | 10 | - | 8.730 | 419.04(each wash/单次) | ±3.0% | 406.47-431.61 | See below/见下行 |
|  |  | Qty Charged/实际投料量（kg）：1st： kg；2nd： kg；3rd： kg；4th： kg； |  |  |  |  |  |  |  |  |
| 3.16.9 | DMF/N,N-二甲基甲酰胺 | 73.10 | 0.948 | 12 | - | 11.376 | 546.05 | ±3.0% | 529.67-562.43 |  |
| 3.16.11 | DMF/N,N-二甲基甲酰胺 | 73.10 | 0.948 | 12 | - | 11.376 | 546.05 | ±3.0% | 529.67-562.43 |  |
| 3.17.1 | DMF/N,N-二甲基甲酰胺 | 73.10 | 0.948 | 5 | - | 4.740 | 227.52 | ±3.0% | 220.69-234.34 |  |
| 1.11/3.17.2 | Fmoc-L-Ser(tBu)-OH/芴甲氧羰基-叔丁基-L-丝氨酸 | 383.44 | - | - | 2.00 | - |  | ±3.0% |  |  |
| 1.12/3.17.5 | OxymaPure/2-肟氰乙酸乙酯 | 142.11 | - | - | 2.00 | - |  | ±3.0% |  |  |
| 3.17.6 | DMF/N,N-二甲基甲酰胺 | 73.10 | 0.948 | 2 | - | 1.896 | 91.01 | ±3.0% | 88.28~93.74 |  |
| 3.17.7 | DIC/N,N'-二异丙基碳二酰亚胺 | 126.20 | 0.815 | - | 2.20 | - |  | ±3.0% |  |  |
| 3.18.2 | DMF/N,N-二甲基甲酰胺 | 73.10 | 0.948 | 1 | - | 0.948 | 45.50 | ±2 kg | 43.50-47.50 |  |
| 3.18.6 | DMF/N,N-二甲基甲酰胺 | 73.10 | 0.948 | 8 | - | 7.584 | 364.03 (each wash/单次) | ±3.0% | 353.11-374.95 | See below/见下行 |
|  |  | Qty Charged/实际投料量（kg）：1st： kg；2nd： kg；3rd： kg； |  |  |  |  |  |  |  |  |
| （6）Fmoc-L-Ser(tBu)-OH/芴甲氧羰基-叔丁基-L-丝氨酸 |  |  |  |  |  |  |  |  |  |  |
| 3.19.1 | 20% (V/V) piperidine/DMF solution/20%（V/V）哌啶/DMF溶液 | - | 0.931 | 8 | - | 7.448 | 357.50 | ±3.0% | 346.77-368.23 |  |
| 3.19.5 | IPAC/乙酸异丙酯 | 102.13 | 0.873 | 10 | - | 8.730 | 419.04(each wash/单次) | ±3.0% | 406.47-431.61 | See below/见下行 |
|  |  | Qty Charged/实际投料量（kg）：1st： kg；2nd： kg；3rd： kg；4th： kg； |  |  |  |  |  |  |  |  |
| 3.19.6 | DMF/N,N-二甲基甲酰胺 | 73.10 | 0.948 | 12 | - | 11.376 | 546.05 | ±3.0% | 529.67-562.43 |  |
| 3.19.8 | DMF/N,N-二甲基甲酰胺 | 73.10 | 0.948 | 12 | - | 11.376 | 546.05 | ±3.0% | 529.67-562.43 |  |
| 3.20.1 | DMF/N,N-二甲基甲酰胺 | 73.10 | 0.948 | 5 | - | 4.740 | 227.52 | ±3.0% | 220.69-234.34 |  |
| 1.13/3.20.2 | Fmoc-L-Ser(tBu)-OH/芴甲氧羰基-叔丁基-L-丝氨酸 | 383.44 | - | - | 2.00 | - |  | ±3.0% |  |  |
| 1.14/3.20.5 | OxymaPure/2-肟氰乙酸乙酯 | 142.11 | - | - | 2.00 | - |  | ±3.0% |  |  |
| 3.20.6 | DMF/N,N-二甲基甲酰胺 | 73.10 | 0.948 | 2 | - | 1.896 | 91.01 | ±3.0% | 88.28-93.74 |  |
| 3.20.7 | DIC/N,N'-二异丙基碳二酰亚胺 | 126.20 | 0.815 | - | 2.20 | - |  | ±3.0% |  |  |
| 3.21.2 | DMF/N,N-二甲基甲酰胺 | 73.10 | 0.948 | 1 | - | 0.948 | 45.50 | ±2 kg | 43.50-47.50 |  |
| 3.21.6 | DMF/N,N-二甲基甲酰胺 | 73.10 | 0.948 | 8 | - | 7.584 | 364.03 (each wash/单次) | ±3.0% | 353.11-374.95 | See below/见下行 |
|  |  | Qty Charged/实际投料量（kg）：1st： kg；2nd： kg；3rd： kg； |  |  |  |  |  |  |  |  |
| （7）Fmoc-L-Arg(Pbf)-OH/Fmoc-Pbf-精氨酸 |  |  |  |  |  |  |  |  |  |  |
| 3.22.1 | 20% (V/V) piperidine/DMF solution/20%（V/V）哌啶/DMF溶液 | - | 0.931 | 8 | - | 7.448 | 357.50 | ±3.0% | 346.77-368.23 |  |
| 3.22.5 | IPAC/乙酸异丙酯 | 102.13 | 0.873 | 10 | - | 8.730 | 419.04 (each wash/单次) | ±3.0% | 406.47-431.61 | See below/见下行 |
|  |  | Qty Charged/实际投料量（kg）：1st： kg；2nd： kg；3rd： kg；4th： kg； |  |  |  |  |  |  |  |  |
| 3.22.7 | DMF/N,N-二甲基甲酰胺 | 73.10 | 0.948 | 12 | - | 11.376 | 546.05 | ±3.0% | 529.67-562.43 |  |
| 3.22.8 | DMF/N,N-二甲基甲酰胺 | 73.10 | 0.948 | 12 | - | 11.376 | 546.05 | ±3.0% | 529.67-562.43 |  |
| 3.23.1 | DMF/N,N-二甲基甲酰胺 | 73.10 | 0.948 | 5 | - | 4.740 | 227.52 | ±3.0% | 220.69-234.34 |  |
| 1.15/3.23.2 | Fmoc-L-Arg(Pbf)-OH/Fmoc-Pbf-精氨酸 | 648.77 | - | - | 2.00 | - |  | ±3.0% |  |  |
| 1.16/3.23.5 | OxymaPure/2-肟氰乙酸乙酯 | 142.11 | - | - | 2.00 | - |  | ±3.0% |  |  |
| 3.23.6 | DMF/N,N-二甲基甲酰胺 | 73.10 | 0.948 | 2 | - | 1.896 | 91.01 | ±3.0% | 88.28-93.74 |  |
| 3.23.7 | DIC/N,N'-二异丙基碳二酰亚胺 | 126.20 | 0.815 | - | 2.20 | - |  | ±3.0% |  |  |
| 3.24.2 | DMF/N,N-二甲基甲酰胺 | 73.10 | 0.948 | 1 | - | 0.948 | 45.50 | ±2 kg | 43.50-47.50 |  |
| 3.24.6 | DMF/N,N-二甲基甲酰胺 | 73.10 | 0.948 | 8 | - | 7.584 | 364.03 (each wash/单次) | ±3.0% | 353.11-374.95 | See below/见下行 |
|  |  | Qty Charged/实际投料量（kg）：1st： kg；2nd： kg；3rd： kg； |  |  |  |  |  |  |  |  |
| （8）Fmoc-L-Val-OH/芴甲氧羰基-L-缬氨酸 |  |  |  |  |  |  |  |  |  |  |
| 3.25.1 | 20% (V/V) piperidine/DMF solution/20%（V/V）哌啶/DMF溶液 | - | 0.931 | 8 | - | 7.448 | 357.50 | ±3.0% | 346.77-368.23 |  |
| 3.25.5 | IPAC/乙酸异丙酯 | 102.13 | 0.873 | 10 | - | 8.730 | 419.04 (each wash/单次) | ±3.0% | 406.47-431.61 | See below/见下行 |
|  |  | Qty Charged/实际投料量（kg）：1st： kg；2nd： kg；3rd： kg；4th： kg； |  |  |  |  |  |  |  |  |
| 3.25.6 | DMF/N,N-二甲基甲酰胺 | 73.10 | 0.948 | 12 | - | 11.376 | 546.05 | ±3.0% | 529.67-562.43 |  |
| 3.25.8 | DMF/N,N-二甲基甲酰胺 | 73.10 | 0.948 | 12 | - | 11.376 | 546.05 | ±3.0% | 529.67-562.43 |  |
| 3.26.1 | DMF/N,N-二甲基甲酰胺 | 73.10 | 0.948 | 5 | - | 4.740 | 227.52 | ±3.0% | 220.69-234.34 |  |
| 1.17/3.26.2 | Fmoc-L-Val-OH/芴甲氧羰基-L-缬氨酸 | 339.39 | - | - | 2.00 | - |  | ±3.0% |  |  |
| 1.18/3.26.5 | OxymaPure/2-肟氰乙酸乙酯 | 142.11 | - | - | 2.00 | - |  | ±3.0% |  |  |
| 3.26.6 | DMF/N,N-二甲基甲酰胺 | 73.10 | 0.948 | 2 | - | 1.896 | 91.01 | ±3.0% | 88.28-93.74 |  |
| 3.26.7 | DIC/N,N'-二异丙基碳二酰亚胺 | 126.20 | 0.815 | - | 2.20 | - |  | ±3.0% |  |  |
| 3.27.2 | DMF/N,N-二甲基甲酰胺 | 73.10 | 0.948 | 1 | - | 0.948 | 45.50 | ±2 kg | 43.50-47.50 |  |
| 3.27.6 | DMF/N,N-二甲基甲酰胺 | 73.10 | 0.948 | 8 | - | 7.584 | 364.03 (each wash/单次) | ±3.0% | 353.11-374.95 | See below/见下行 |
|  |  | Qty Charged/实际投料量（kg）：1st： kg；2nd： kg；3rd： kg； |  |  |  |  |  |  |  |  |
| （9）Fmoc-L-Leu-OH/芴甲氧羰基-L-亮氨酸 |  |  |  |  |  |  |  |  |  |  |
| 3.28.1 | 20% (V/V) piperidine/DMF solution/20%（V/V）哌啶/DMF溶液 | - | 0.931 | 8 | - | 7.448 | 357.50 | ±3.0% | 346.77-368.23 |  |
| 3.28.5 | IPAC/乙酸异丙酯 | 102.13 | 0.873 | 10 | - | 8.730 | 419.04 (each wash/单次) | ±3.0% | 406.47-431.61 | See below/见下行 |
|  |  | Qty Charged/实际投料量（kg）：1st： kg；2nd： kg；3rd： kg；4th： kg； |  |  |  |  |  |  |  |  |
| 3.28.6 | DMF/N,N-二甲基甲酰胺 | 73.10 | 0.948 | 12 | - | 11.376 | 546.05 | ±3.0% | 529.67-562.43 |  |
| 3.28.8 | DMF/N,N-二甲基甲酰胺 | 73.10 | 0.948 | 12 | - | 11.376 | 546.05 | ±3.0% | 529.67-562.43 |  |
| 3.29.1 | DMF/N,N-二甲基甲酰胺 | 73.10 | 0.948 | 5 | - | 4.740 | 227.52 | ±3.0% | 220.69-234.34 |  |
| 1.19/3.29.2 | Fmoc-L-Leu-OH/芴甲氧羰基-L-亮氨酸 | 353.41 | - | - | 2.00 | - |  | ±3.0% |  |  |
| 1.20/3.29.5 | OxymaPure/2-肟氰乙酸乙酯 | 142.11 | - | - | 2.00 | - |  | ±3.0% |  |  |
| 3.29.6 | DMF/N,N-二甲基甲酰胺 | 73.10 | 0.948 | 2 | - | 1.896 | 91.01 | ±3.0% | 88.28-93.74 |  |
| 3.29.7 | DIC/N,N'-二异丙基碳二酰亚胺 | 126.20 | 0.815 | - | 2.20 | - |  | ±3.0% |  |  |
| 3.30.2 | DMF/N,N-二甲基甲酰胺 | 73.10 | 0.948 | 1 | - | 0.948 | 45.50 | ±2 kg | 43.50-47.50 |  |
| 3.30.6 | DMF/N,N-二甲基甲酰胺 | 73.10 | 0.948 | 8 | - | 7.584 | 364.03 (each wash/单次) | ±3.0% | 353.11-374.95 | See below/见下行 |
|  |  | Qty Charged/实际投料量（kg）：1st： kg；2nd： kg；3rd： kg； |  |  |  |  |  |  |  |  |
| （10）Fmoc-L-α-Me-Phe-OH/(S)-2-((((9H-芴-9-基)甲氧基)羰基)氨基)-2-甲基-3 -苯基丙酸 |  |  |  |  |  |  |  |  |  |  |
| 3.31.1 | 20% (V/V) piperidine/DMF solution/20%（V/V）哌啶/DMF溶液 | - | 0.931 | 8 | - | 7.448 | 357.50 | ±3.0% | 346.77-368.23 |  |
| 3.31.5 | IPAC/乙酸异丙酯 | 102.13 | 0.873 | 10 | - | 8.730 | 419.04 (each wash/单次) | ±3.0% | 406.47-431.61 | See below/见下行 |
|  |  | Qty Charged/实际投料量（kg）：1st： kg；2nd： kg；3rd： kg；4th： kg； |  |  |  |  |  |  |  |  |
| 3.31.6 | DMF/N,N-二甲基甲酰胺 | 73.10 | 0.948 | 12 | - | 11.376 | 546.05 | ±3.0% | 529.67-562.43 |  |
| 3.31.8 | DMF/N,N-二甲基甲酰胺 | 73.10 | 0.948 | 12 | - | 11.376 | 546.05 | ±3.0% | 529.67-562.43 |  |
| 3.32.1 | DMF/N,N-二甲基甲酰胺 | 73.10 | 0.948 | 4 | - | 3.792 | 182.02 | ±3.0% | 176.56-187.48 |  |
| 1.21/3.32.2 | Fmoc-L-α-Me-Phe-OH/(S)-2-((((9H-芴-9-基)甲氧基)羰基)氨基)-2-甲基-3 -苯基丙酸 | 401.45 | - | - | 1.75 | - |  | ±3.0% |  |  |
| 1.22/3.32.5 | OxymaPure/2-肟氰乙酸乙酯 | 142.11 | - | - | 1.80 | - |  | ±3.0% |  |  |
| 3.32.6 | DMF/N,N-二甲基甲酰胺 | 73.10 | 0.948 | 2 | - | 1.896 | 91.01 | ±3.0% | 88.28-93.74 |  |
| 3.32.7 | DIC/N,N'-二异丙基碳二酰亚胺 | 126.20 | 0.815 | - | 2.00 | - |  | ±3.0% |  |  |
| 3.33.2 | DMF/N,N-二甲基甲酰胺 | 73.10 | 0.948 | 1 | - | 0.948 | 45.50 | ±2 kg | 43.50-47.50 |  |
| 3.33.6 | DMF/N,N-二甲基甲酰胺 | 73.10 | 0.948 | 8 | - | 7.584 | 364.03 (each wash/单次) | ±3.0% | 353.11-374.95 | See below/见下行 |
|  |  | Qty Charged/实际投料量（kg）：1st： kg；2nd： kg；3rd： kg； |  |  |  |  |  |  |  |  |
| （11）Fmoc-L-Glu(OtBu)-OH·H2O /芴甲氧羰基-L-谷氨酸-γ-叔丁酯一水合物 |  |  |  |  |  |  |  |  |  |  |
| 3.34.1 | 20% (V/V) piperidine/DMF solution/20%（V/V）哌啶/DMF溶液 | - | 0.931 | 8 | - | 7.448 | 357.50 | ±3.0% | 346.77-368.23 |  |
| 3.34.5 | IPAC/乙酸异丙酯 | 102.13 | 0.873 | 10 | - | 8.730 | 419.04 (each wash/单次) | ±3.0% | 406.47-431.61 | See below/见下行 |
|  |  | Qty Charged/实际投料量（kg）：1st： kg；2nd： kg；3rd： kg；4th： kg； |  |  |  |  |  |  |  |  |
| 3.34.6 | DMF/N,N-二甲基甲酰胺 | 73.10 | 0.948 | 12 | - | 11.376 | 546.05 | ±3.0% | 529.67-562.43 |  |
| 3.34.8 | DMF/N,N-二甲基甲酰胺 | 73.10 | 0.948 | 12 | - | 11.376 | 546.05 | ±3.0% | 529.67-562.43 |  |
| 3.35.1 | DMF/N,N-二甲基甲酰胺 | 73.10 | 0.948 | 5 | - | 4.740 | 227.52 | ±3.0% | 220.69-234.34 |  |
| 1.23/3.35.2 | Fmoc-L-Glu(OtBu)-OH·H2O /芴甲氧羰基-L-谷氨酸-γ-叔丁酯一水合物 | 443.49 | - | - | 2.00 | - |  | ±3.0% |  |  |
| 1.24/3.35.5 | OxymaPure/2-肟氰乙酸乙酯 | 142.11 | - | - | 2.00 | - |  | ±3.0% |  |  |
| 3.35.6 | DMF/N,N-二甲基甲酰胺 | 73.10 | 0.948 | 2 | - | 1.896 | 91.01 | ±3.0% | 88.28-93.74 |  |
| 3.35.7 | DIC/N,N'-二异丙基碳二酰亚胺 | 126.20 | 0.815 | - | 2.20 | - |  | ±3.0% |  |  |
| 3.36.2 | DMF/N,N-二甲基甲酰胺 | 73.10 | 0.948 | 1 | - | 0.948 | 45.50 | ±2 kg | 43.50-47.50 |  |
| 3.36.6 | DMF/N,N-二甲基甲酰胺 | 73.10 | 0.948 | 8 | - | 7.584 | 364.03 (each wash/单次) | ±3.0% | 353.11-374.95 | See below/见下行 |
|  |  | Qty Charged/实际投料量（kg）：1st： kg；2nd： kg；3rd： kg； |  |  |  |  |  |  |  |  |
| （12）Fmoc-L-Ala-OH·H2O/芴甲氧羰基-L-丙氨酸一水合物 |  |  |  |  |  |  |  |  |  |  |
| 3.37.1 | 20% (V/V) piperidine/DMF solution/20%（V/V）哌啶/DMF溶液 | - | 0.931 | - | 8 | 7.448 | 357.50 | ±3.0% | 346.77-368.23 |  |
| 3.37.5 | IPAC/乙酸异丙酯 | 102.13 | 0.873 | - | 10 | 8.730 | 419.04 (each wash/单次) | ±3.0% | 406.47-431.61 | See below/见下行 |
|  |  | Qty Charged/实际投料量（kg）：1st： kg；2nd： kg；3rd： kg；4th： kg； |  |  |  |  |  |  |  |  |
| 3.37.6 | DMF/N,N-二甲基甲酰胺 | 73.10 | 0.948 | 12 | - | 11.376 | 546.05 | ±3.0% | 529.67-562.43 |  |
| 3.37.8 | DMF/N,N-二甲基甲酰胺 | 73.10 | 0.948 | 12 | - | 11.376 | 546.05 | ±3.0% | 529.67-562.43 |  |
| 3.38.1 | DMF/N,N-二甲基甲酰胺 | 73.10 | 0.948 | - | 5 | 4.740 | 227.52 | ±3.0% | 220.69-234.34 |  |
| 1.25/3.38.2 | Fmoc-L-Ala-OH·H2O/芴甲氧羰基-L-丙氨酸一水合物 | 329.35 | - | - | 2.00 | - |  | ±3.0% |  |  |
| 1.26/3.38.5 | OxymaPure/2-肟氰乙酸乙酯 | 142.11 | - | - | 2.00 | - |  | ±3.0% |  |  |
| 3.38.6 | DMF/N,N-二甲基甲酰胺 | 73.10 | 0.948 | 2 | - | 1.896 | 91.01 | ±3.0% | 88.28-93.74 |  |
| 3.38.7 | DIC/N,N'-二异丙基碳二酰亚胺 | 126.20 | 0.815 | - | 2.20 | - |  | ±3.0% |  |  |
| 3.39.2 | DMF/N,N-二甲基甲酰胺 | 73.10 | 0.948 | 1 | - | 0.948 | 45.50 | ±2 kg | 43.50-47.50 |  |
| 3.39.5 | DMF/N,N-二甲基甲酰胺 | 73.10 | 0.948 | 8 | - | 7.584 | 364.03 (each wash/单次) | ±3.0% | 353.11-374.95 | See below/见下行 |
|  |  | Qty Charged/实际投料量（kg）：1st： kg；2nd： kg；3rd： kg； |  |  |  |  |  |  |  |  |
| （13）Fmoc-L-Leu-OH/芴甲氧羰基-L-亮氨酸 |  |  |  |  |  |  |  |  |  |  |
| 3.40.1 | 20% (V/V) piperidine/DMF solution/20%（V/V）哌啶/DMF溶液 | - | 0.931 | 8 | - | 7.448 | 357.50 | ±3.0% | 346.77-368.23 |  |
| 3.40.5 | IPAC/乙酸异丙酯 | 102.13 | 0.873 | 10 | - | 8.730 | 419.04 (each wash/单次) | ±3.0% | 406.47-431.61 | See below/见下行 |
|  |  | Qty Charged/实际投料量（kg）：1st： kg；2nd： kg；3rd： kg；4th： kg； |  |  |  |  |  |  |  |  |
| 3.40.6 | DMF/N,N-二甲基甲酰胺 | 73.10 | 0.948 | 12 | - | 11.376 | 546.05 | ±3.0% | 529.67-562.43 |  |
| 3.40.8 | DMF/N,N-二甲基甲酰胺 | 73.10 | 0.948 | 12 | - | 11.376 | 546.05 | ±3.0% | 529.67-562.43 |  |
| 3.41.1 | DMF/N,N-二甲基甲酰胺 | 73.10 | 0.948 | 5 | - | 4.740 | 227.52 | ±3.0% | 220.69-234.34 |  |
| 1.27/3.41.2 | Fmoc-L-Leu-OH/芴甲氧羰基-L-亮氨酸 | 353.41 | - | - | 2.00 | - |  | ±3.0% |  |  |
| 1.28/3.41.5 | OxymaPure/2-肟氰乙酸乙酯 | 142.11 | - | - | 2.00 | - |  | ±3.0% |  |  |
| 3.41.6 | DMF/N,N-二甲基甲酰胺 | 73.10 | 0.948 | 2 | - | 1.896 | 91.01 | ±3.0% | 88.28-93.74 |  |
| 3.41.7 | DIC/N,N'-二异丙基碳二酰亚胺 | 126.20 | 0.815 | - | 2.20 | - |  | ±3.0% |  |  |
| 3.42.2 | DMF/N,N-二甲基甲酰胺 | 73.10 | 0.948 | 1 | - | 0.948 | 45.50 | ±2 kg | 43.50-47.50 |  |
| 3.42.6 | DMF/N,N-二甲基甲酰胺 | 73.10 | 0.948 | 8 | - | 7.584 | 364.03 (each wash/单次) | ±3.0% | 353.11-374.95 | See below/见下行 |
|  |  | Qty Charged/实际投料量（kg）：1st： kg；2nd： kg；3rd： kg； |  |  |  |  |  |  |  |  |
| De-Fmoc/脱保护 |  |  |  |  |  |  |  |  |  |  |
| 3.43.1 | 20% (V/V) piperidine/DMF solution/20%（V/V）哌啶/DMF溶液 | - | 0.931 | 8 | - | 7.448 | 357.50 | ±3.0% | 346.77-368.23 |  |
| 3.43.5 | IPAC/乙酸异丙酯 | 102.13 | 0.873 | 10 | - | 8.730 | 419.04 (each wash/单次) | ±3.0% | 406.47-431.61 | See below/见下行 |
|  |  | Qty Charged/实际投料量（kg）：1st： kg；2nd： kg；3rd： kg；4th： kg； 5th： kg；6th： kg；7th： kg；8th： kg；9th： kg；10th： kg； |  |  |  |  |  |  |  |  |
| 3.44.1 | IPAC/乙酸异丙酯 | 102.13 | 0.873 | 10 | - | 8.730 | 419.04 (each wash/单次) | ±3.0% | 406.47-431.61 | See below/见下行 |
|  |  | Qty Charged/实际投料量（kg）：1st： kg；2nd： kg； |  |  |  |  |  |  |  |  |
| （14）Fmoc-L-Orn(Boc)-OH/N-Fmoc-N'-Boc-L-鸟氨酸 |  |  |  |  |  |  |  |  |  |  |
| 3.45.1 | DMF/N,N-二甲基甲酰胺 | 73.10 | 0.948 | 12 | - | 11.376 | 546.05 | ±3.0% | 529.67-562.43 |  |
| 3.45.3 | DMF/N,N-二甲基甲酰胺 | 73.10 | 0.948 | 12 | - | 11.376 | 546.05 | ±3.0% | 529.67-562.43 |  |
| 3.46.1 | DMF/N,N-二甲基甲酰胺 | 73.10 | 0.948 | 5 | - | 4.740 | 227.52 | ±3.0% | 220.69-234.34 |  |
| 1.29/3.46.2 | Fmoc-L-Orn(Boc)-OH/N-Fmoc-N'-Boc-L-鸟氨酸 | 454.52 | - | - | 2.00 | - |  | ±3.0% |  |  |
| 1.30/3.46.5 | OxymaPure/2-肟氰乙酸乙酯 | 142.11 | - | - | 2.00 | - |  | ±3.0% |  |  |
| 3.46.6 | DMF/N,N-二甲基甲酰胺 | 73.10 | 0.948 | 2 | - | 1.896 | 91.01 | ±3.0% | 88.28-93.74 |  |
| 3.46.7 | DIC/N,N'-二异丙基碳二酰亚胺 | 126.20 | 0.815 | - | 2.20 | - |  | ±3.0% |  |  |
| 3.47.2 | DMF/N,N-二甲基甲酰胺 | 73.10 | 0.948 | 1 | - | 0.948 | 45.50 | ±2 kg | 43.50-47.50 |  |
| 3.47.6 | IPAC/乙酸异丙酯 | 102.13 | 0.873 | 10 | - | 8.730 | 419.04 (each wash/单次) | ±3.0% | 406.47-431.61 | See below/见下行 |
|  |  | Qty Charged/实际投料量（kg）：1st： kg；2nd： kg；3rd： kg；4th： kg； 5th： kg； |  |  |  |  |  |  |  |  |
| 3.47.7 | Toluene/甲苯 | 92.14 | 0.872 | 10 | - | 8.720 | 418.56 | ±3.0% | 406.00-431.12 |  |
| 3.47.8 | MTBE/甲基叔丁基醚 | 88.15 | 0.741 | 10 | - | 7.410 | 355.68 | ±3.0% | 345.01-366.35 | See below/见下行 |
|  |  | Qty Charged/实际投料量（kg）：1st： kg；2nd： kg； |  |  |  |  |  |  |  |  |
| 3.47.9 | Toluene/甲苯 | 92.14 | 0.872 | 10 | - | 8.720 | 418.56 | ±3.0% | 406.00-431.12 |  |
| 3.47.10 | MTBE/甲基叔丁基醚 | 88.15 | 0.741 | 10 | - | 7.410 | 355.68 (each wash/单次) | ±3.0% | 345.01-366.35 |  |
| 3.47.11 | MTBE/甲基叔丁基醚 | 88.15 | 0.741 | 10 | - | 7.410 | 355.68 (each wash/单次) | ±3.0% | 345.01-366.35 |  |
| 3.48.1 | Toluene/甲苯 | 92.14 | 0.872 | 10 | - | 8.720 | 418.56 | ±3.0% | 406.00-431.12 |  |
| 3.48.2 | MTBE/甲基叔丁基醚 | 88.15 | 0.741 | 10 | - | 7.410 | 355.68 (each wash/单次) | ±3.0% | 345.01-366.35 |  |
| 3.48.3 | MTBE/甲基叔丁基醚 | 88.15 | 0.741 | 10 | - | 7.410 | 355.68 (each wash/单次) | ±3.0% | 345.01-366.35 |  |

Recorded by& Date/记录人及日期(Chemist or engineer/相关技术人员)：

Checked by& Date/核实人及日期(Chemist or engineer/相关技术人员)：

Checked by QA & Date/QA核实人及日期：

(Chemist or engineer/相关技术人员)

Analytical Method/分析方法

- Sampling and test list of in-process intermediate /过程中间体取样检测列表

| Instruction No./ 指令编号 | Sample Type/ 样品检测类型 |  |  | Sample source description 样品来源描述 | Analytical test/ 检测项目 | Criterion of acceptability/ 合格标准 | Analytical method# / 分析方法编号 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | IPC test | FIO test | Release testing |  |  |  |  |
| 3.0.1 | X | - | - | DMF rinse solution/DMF淋洗液 | Water content/ 水分 | ≤500 ppm | AM-YL000336-004 |
|  | X | - | - | Coupling-stage resin/偶联阶段树脂 | Resin color/树脂颜色 | Negative(the resin is yellow to orange)/ 阴性(树脂黄色至橙色) | AM-140020-029 |
|  |  |  |  |  | Resin color/树脂颜色 |  | AM-140020-031 |
|  |  |  |  |  | 比色 |  | AM-140020-030 |
| 3.49.1 | X | - | - | Solid/烘料固体 | LOD/干燥失重 | ≤9% | AM-140020-016 |

Analytical report numbering method: project No. + step No. + batch No. + Ins.No. + Sampling No. in this Instruction, as CPo1234-01-01-01-09-01

所有分析报告的编号方式为项目号＋步数＋批次＋操作指令数＋指令中的第几次取样，如CPo1234-01-01-01-09-01

Confirm in MOR the decimal places of the IPC test acceptance criteria is consistent with the analytical method.

执行MOR中的IPC检测可接受标准的小数位数和分析方法的小数位数一致。

| Analytical Laboratory Manager & Date / 分析实验室经理签字及日期 | {{analysis}} |
| --- | --- |

Recipe Code against the Unit Operation/基于单元操作的配方命名

| Instruction No. /指令编号 | Recipe usage/配方用途 | Recipe name/配方命名 | Batch name/批次命名 |
| --- | --- | --- | --- |
| 3 | Synthesis/合成 | 14002001M01V02 | CPo140020 01 |

Recipe name: project number: "CCl#" + step No. +M+serial No. + V + version No. /配方命名： CCl#+步骤号+M+序列号+ V +2位版本号

Batch name: project number " + step No.+ batch No. / 批次命名：项目号+步骤号+批次号

Operation Instructions/操作指令

Operations Instruction 操作指令

Instructions/说明： - Key operation/关键操作 (refer to DS-QA-001参考 DS-QA-001): The CPP (if any), CIPCs (if any) will be indicated with ★in the MOR./对于关键工艺参数（如有），关键过程控制参数（如有）将在MOR中★用进行标记。 - In each operation instruction, the process parameter should be defined in the format of target with allowed operation range. /在操作指令处参数实例处一律以操作目标值加上允许操作范围的方式体现。 - Chinese version will prevail in this document. Where applicable, English translation is added to aid comprehension by reviewer whose native language is not Chinese. /本记录以中文为准，英文翻译是用于帮助非中文的审阅者理解的。 - In this document, some non-process operations, such as check as equipment readiness, hose connection, starting material, auxiliary solution preparation, may be performed in advance when appropriate. /在本记录中，非工艺流程操作，例如设备准备，管道连接，物料准备，辅助溶液配制可以在合适的时候提前操作。 - Various confirmations/各类执行 - Before material charging and dispensing, confirm the material is within the retest date/ the expiry date, Confirm whether the weight is adequate for use in the current charging. /加物料或分装物料前，确认物料在复检日期/有效期之内，确认重量是否满足本次投料使用。 - Before solid material charging, confirm/加固体物料前，执行： - No non-related material / equipment / instrument / tools / pipeline / document / record / label in the intended area /目标区域无不相关物料/设备/仪器/工具/管路/文件/记录/标签 - The outer packages are removed. If the outer package cannot be removed, unpack material without damaging the package bag, and before charging, check no package fragment and other extraneous matter in material. /外包装移除；如果外包装不能移除，但在不破坏包装的情况下打开包装袋，投料前以及过程中同时检查执行物料中无包装碎片以及异物 - Before drummed liquid material charging, visual check there is no other extraneous matter in material./加桶装液体物料前，应目视执行桶内无异物。 - After solid material charging, confirm/固体物料投料之后，确认： - The corresponding leftover material has been properly sealed and transferred to the staging area/相应的剩余物料已经恰当密封，转移到了暂存区域 - The waste packaging material has been sealed and transferred to the corresponding waste staging area. /产生的包材废物已经密封并转移到了相应的废物暂存区域 - The operation area/equipment surface has been cleaned/操作区域以及设备表面已经清洁 - Key raw materials, in-process products and products for PPQ and commercial production should be weighed after the weighing implements are checked using the standard weights. For the balance/scale with the maximum capacity of greater than 40 kg, check with the standard weight of 20 kg; for the balance/scale with maximum capacity is not more than 40 kg, check with the standard weight of 1/2 maximum capacity. /用于工艺性能执行和商业化生产的主原料、过程产物以及产品的称量前，应使用标准砝码对称量器具进行检查，量程大于40 kg的电子秤，采用20 kg的标准砝码进行检查；量程不大于40 kg的电子秤，采用最大量程1/2的标准砝码进行检查。 - The agitating speed control refers to the control in a normal steady process status exclude the operation-switch stage or interruption for some observation/sampling operation et.al. /搅拌速度控制指正常工艺稳定状态下的搅拌速度，不包括操作间的切换阶段或者为观察或者取样等操作为目的的扰动。 - The temperature control has priority to the material charging rate control from safety consideration，the material charging rate can further slow-down or stop to a temperature-controllable status. /从安全角度考虑，相比加料速度的控制，温度控制具有优先权，物料加料速度可以进一步放慢或者停止使温度处于可控状态。 - 8-10 L purified water should be discharged at the point of use before use. /纯化水每次使用前，应先从用水点放出约8-10 L水。 - Check the cleanness of glass sample bottle prior to use and rinse the sample bottle with reaction/workup solvent or mixture. /使用玻璃取样瓶前需要检查是否清洁，同时使用反应/后处理溶剂或体系润洗取样瓶。 - In the Master Operation Record (MOR), the responsibility of roles are defined as follows:/在该主操作记录中，对于各类角色职责定义如下： - Performed by: The person who executes the synthesis procedure. For manual operations (e.g., material charging, sampling), the person performing the operation by manual is the performer; For automated operations (e.g., automated peptide synthesis cycle operation), it is the person who initiates the automatic operation of the on-site monitoring program and is also the account login person during the automatic operation stage of the program. 执行人：为现场实际执行合成工序作业的人员。对于手动投料的操作（如投料、取样），操作人员即为执行人；对于程序自动运行的操作（如多肽自动合成循环操作等），执行人为当班现场监控程序自动运行的人员，也是程序自动运行阶段的账号登录人员。 - Transcribed by: The person who is responsible for transcribing electronic data automatically generated by equipment onto the paper batch operation record. Since computerized systems operate at high speed, the Transcriber can transcribe relevant information from the original electronic data such as audit trails and transcribe it into the batch operation record. Shift personnel shall complete the transcription of data required by the batch record for production operations that have been completed during their shift. 转录人：负责将设备自动化电子数据誊写至纸版批操作记录的人员。因计算机化系统运行速率较高，转录人可依据审计追踪等原始电子数据源，将相关信息摘抄至批操作记录中。当班人员应完成本班涉及到的已完成生产操作中，批记录中要求的数据摘抄工作。 - Checked by: The second person who witness the activity (such as operation, data, and record) is performed correctly through supervising on the site. 核实人：由第二人通过现场亲眼目击确证相应的活动（如操作，数据和记录的）被完全正确地执行的人员。

Process Safety information /工艺安全信息

- 2-CTC resin: OEB3, resin can not swell for too long time and avoid excessive drying caused by resin water absorption deactivation. / CTC树脂OEB3，树脂不可溶胀过长时间以及避免过量的抽干引起树脂吸水失活。 - It is necessary to confirm that De-Fmoc of resin is completed before activation. The addition of DIPEA requires attention to the temperature increase of mixture. /需要执行肽树脂脱Fmoc洗涤进行完毕再开始活化氨基酸。滴加N，N-二异丙基乙胺过程需要注意体系温度会升高。 - OxymaPure is highly toxic, high-energy compound. /2-肟氰乙酸乙酯是高毒，高能化合物。 - PyBOP is high energy compounds, impact sensitive, skin sensitizing. /1H-苯并三唑-1-基氧三吡咯烷基六氟磷酸盐是高能化合物，撞击敏感性，皮肤致敏。 - DIC is a key EHS concern chemical, highly toxic, skin sensitizing, respiratory sensitizing. /N,N’二异丙基碳二亚胺为EHS重点关注化学品，高毒，皮肤致敏，呼吸致敏。 - IPAC/乙酸异丙酯： Vapors irritate eyes and respiratory tract; high concentrations can be anesthetic. Liquid irritates eyes but causes no serious injury; may cause dermatitis; no serious effects if swallowed. /蒸汽会刺激眼睛和呼吸道；高浓度的蒸汽可能是麻醉剂。液体刺激眼睛，但不会造成严重伤害；可能会引起皮炎；吞食不会造成严重影响。 - DMF/N，N-二甲基甲酰胺： Irritation of eyes, skin and nose. May cause nausea. /刺激眼睛、皮肤和鼻子。可能引起恶心。 - piperidine哌啶：Strong local irritant and may cause permanent injury after short exposure to small amounts. /强烈的局部刺激，短时间少量接触可能造成永久性伤害。 - DIPEA/N,N-二异丙基乙胺：It can enter the body through inhalation, ingestion and skin absorption. It is strongly irritating to the eyes, skin, mucous membranes and upper respiratory tract. Inhalation of high-concentration vapors may cause inflammation, edema and spasm of the larynx/bronchi, even chemical pneumonia and pulmonary edema. Exposure may cause symptoms such as burning sensation, cough, headache, nausea and vomiting. /可通过吸入、食入、皮肤吸收侵入人体，对眼、皮肤、黏膜和上呼吸道有强刺激性，吸入高浓度蒸气会引发喉/支气管炎症水肿、痉挛，甚至化学性肺炎、肺水肿，接触后会出现烧灼感、咳嗽、头痛、恶心呕吐等症状 - MTBE/甲基叔丁基醚： Inhalation: May cause dizziness or suffocation. Contact may irritate or burn eyes or skin. May be harmful if swallowed. /吸入:可引起头晕或窒息。接触可能刺激或灼伤眼睛或皮肤。吞食可能有害。 - Tol/甲苯：Vapors irritate eyes and upper respiratory tract; cause dizziness, headache, anesthesia, respiratory arrest. Liquid irritates eyes and causes drying of skin. If aspirated, causes coughing, gagging, distress, and rapidly developing pulmonary edema. If ingested causes vomiting, griping, diarrhea, depressed respiration. 蒸气刺激眼睛和上呼吸道；引起头晕、头痛、麻醉、呼吸停止。液体刺激眼睛，导致皮肤干燥。如果抽吸，会导致咳嗽、咯咯声、痛苦和迅速发展的肺水肿。如果摄入会引起呕吐、抓痒、腹泻、呼吸抑制 - MeOH/甲醇: Exposure to excessive vapor causes eye irritation, head- ache, fatigue and drowsiness. High concentrations can produce central nervous system depression and optic nerve damage. 50,000 ppm will probably cause death in 1 to 2 hrs. Can be absorbed through skin. Swallowing may cause death or eye damage. /接触过多的蒸汽会引起眼睛刺激、头痛、疲劳和困倦。高浓度可导致中枢神经系统抑制和视神经损伤。50000 ppm可能会在1到2小时内导致死亡。可通过皮肤吸收。吞咽可能导致死亡或眼睛损伤 - ASYM-141366：OEB3, exposure control level≤10ug/m3. /OEB3，暴露控制水平≤10ug/m3

| Ins.1/操作1：Dispense/分装 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Wear nitrile rubber gloves and 3M 6000 series Full-face piece respirator, 6006CN cartridges, Particulate filter, White disposable chemical protective clothing. 戴丁腈橡胶手套、3M 6000系列全面罩, 6006CN滤毒盒, 颗粒物滤棉, 白色防化服。 |  |  |  |  |  |
| - Dispense 2-CTC resin/分装2-氯-三苯甲基氯树脂 |  |  |  |  |  |
| Before material dispensing, confirm the material is within the retest date/ the expiry date. Confirm whether the weight is adequate for use in the current dispensing. /物料分装前，执行物料在复检日期/有效期之内，确认重量是否满足本次分装使用。 Before solid material dispensing, confirm/固体物料分装前，确认： - No non-related material / equipment / instrument / tools / pipeline / document / record / label in the intended area. /目标区域无不相关物料/设备/仪器/工具/管路/文件/记录/标签。 - The outer packages are removed. If the outer package cannot be removed, but unpack material without damaging the package bag, and before dispensing, check no package fragment and other extraneous matter in material. /外包装移除；如果外包装不能移除，但在不破坏包装的情况下打开包装袋，分装前以及过程中同时检查执行物料中无包装碎片以及异物。 Performed by & Date/执行人及日期： |  |  |  |  |  |
| Dispense 2-CTC resin. /分装 48.00 kg 2-氯-三苯甲基氯树脂（Acceptable weighing range/可接受称量范围： 47.90 kg ~ 48.10 kg）。 For the Standard Batch Quantity, see Page 32~62 of 624 the information in "Raw Material Summary Table" corresponding to each unit operation/对于批标准投料量，参照Page 32~62 of 624“原料汇总表”中对应的每个单元操作。 Control No./内部控制号：YL005205 Scale ID. No./秤编号： Lot number of LDPEpowder feeding bag/低密度聚乙烯投料袋批号： Glovebox number /手套箱编号： |  |  |  |  |  |
| Key raw materials, in-process products and products for PPQ and commercial production should be weighed after the weighing implements are checked using the standard weights. For the balance/scale with the maximum capacity of greater than 40 kg, check with the standard weight of 20 kg; for the balance/scale with maximum capacity is not more than 40 kg, check with the standard weight of 1/2 maximum capacity. /用于工艺性能执行和商业化生产的主原料、过程产物以及产品的称量前，应使用标准砝码对称量器具进行检查，量程大于40 kg的电子秤，采用20 kg的标准砝码进行检查；量程不大于40 kg的电子秤，采用最大量程1/2的标准砝码进行检查。 Checking record for standard weights/标准砝码检查记录： |  |  |  |  |  |
| Weight No. /砝码编号 | Weight/ 砝码重量（kg） | Reading/ 示重 | Less than half of verification scale interval/ 小于检定分度值的一半 | Performed by & Date/ 执行人及日期 | Checked by & Date/ 核实人及日期 |
|  |  |  | Y N |  |  |

| 1.1. Dispense 2-CTC resin/分装2-氯三苯甲基氯树脂 |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ins. No. for material use/分装物料计划用于的操作 | Lot number/批号 | Charging Qty/投入的质量 |  |  |  |  |  |  | Total resin Mole/总摩尔量(mol)* | Performed by & Date/ 执行人及日期 | Checked by & Date/ 核实人及日期 |
|  |  | Gross weight/毛重 |  | Tare weight /皮重 |  | Net weight/净重 | No. /编号 | Substitution/取代度(mmol/g) |  |  |  |
| 3.1.1 |  |  |  |  |  |  | / |  |  |  |  |
|  |  |  |  |  |  |  | / |  |  |  |  |
|  |  |  |  |  |  |  | / |  |  |  |  |
|  |  |  |  |  |  |  | / |  |  |  |  |
|  |  |  |  |  |  |  | / |  |  |  |  |
|  |  |  |  |  |  |  | / |  |  |  |  |
|  |  |  |  |  |  |  | / |  |  |  |  |
|  |  |  |  |  |  |  | / |  |  |  |  |
|  |  |  |  |  |  |  | / |  |  |  |  |
|  |  |  |  |  |  |  | / |  |  |  |  |
|  |  |  |  |  |  |  | / |  |  |  |  |
|  |  |  |  |  |  |  | / |  |  |  |  |
|  |  |  |  |  |  |  | / |  |  |  |  |
|  | Total weight/合计重量 |  |  |  |  |  | - |  |  |  |  |
|  | Total No. of bags/合计袋数 |  |  |  |  |  | - |  |  |  |  |
| Begin/开始 |  |  | End/结束 |  |  |  |  |  |  |  |  |
| Note/备注：Average resin substitution and total loading moles are calculated using the QA-approved validated spreadsheet included in the batch record attachment. /平均树脂取代度及总上载摩尔数通过批记录附件中 QA 批准的已验证电子表格计算。 |  |  |  |  |  |  |  |  |  |  |  |
| After solid material dispensing, confirm/固体物料分装之后，确认： - The corresponding leftover material has been properly sealed and transferred to the staging area. /相应的剩余物料已经恰当密封，转移到了暂存区域。 - The waste packaging material has been sealed and transferred to the corresponding waste staging area. /产生的包材废物已经密封并转移到了相应的废物暂存区域。 - The operation area/equipment surface has been cleaned. /操作区域以及设备表面已经清洁。 Checked by & Date/核实人及日期： |  |  |  |  |  |  |  |  |  |  |  |

| - Dispense Fmoc-Gly-OH/分装芴甲氧羰基-甘氨酸 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Before material dispensing, confirm the material is within the retest date/ the expiry date, Confirm whether the weight is adequate for use in the current dispensing. /物料分装前，执行物料在复检日期/有效期之内，确认重量是否满足本次分装使用。 Before solid material dispensing, confirm/固体物料分装前，确认： - No non-related material / equipment / instrument / tools / pipeline / document / record / label in the intended area. /目标区域无不相关物料/设备/仪器/工具/管路/文件/记录/标签。 - The outer packages are removed. If the outer package cannot be removed, but unpack material without damaging the package bag, and before dispensing, check no package fragment and other extraneous matter in material. /外包装移除；如果外包装不能移除，但在不破坏包装的情况下打开包装袋，分装前以及过程中同时检查执行物料中无包装碎片以及异物。 Checked by & Date/核实人及日期： |  |  |  |  |  |
| Dispense Fmoc-Gly-OH. /分装 kg芴甲氧羰基-甘氨酸（Acceptable weighing range/可接受称量范围： kg ~ kg）。 For the Standard Batch Quantity, see Page 32~62 of 624 the information in "Raw Material Summary Table" corresponding to each unit operation/对于批标准投料量，参照Page 32~62 of 624“原料汇总表”中对应的每个单元操作。 Control No./内部控制号：YL004846 Scale ID. No./秤编号： Lot number of LDPE powder feeding bag/低密度聚乙烯粉末投料袋批号： Glovebox number /手套箱编号： |  |  |  |  |  |
| Key raw materials, in-process products and products for PPQ and commercial production should be weighed after the weighing implements are checked using the standard weights. For the balance/scale with the maximum capacity of greater than 40 kg, check with the standard weight of 20 kg; for the balance/scale with maximum capacity is not more than 40 kg, check with the standard weight of 1/2 maximum capacity. /用于工艺性能执行和商业化生产的主原料、过程产物以及产品的称量前，应使用标准砝码对称量器具进行检查，量程大于40 kg的电子秤，采用20 kg的标准砝码进行检查；量程不大于40 kg的电子秤，采用最大量程1/2的标准砝码进行检查。 Checking record for standard weights/标准砝码检查记录： |  |  |  |  |  |
| Weight No. /砝码编号 | Weight/ 砝码重量（kg） | Reading/ 示重 | Less than half of verification scale interval/ 小于检定分度值的一半 | Performed by & Date/ 执行人及日期 | Checked by & Date/ 核实人及日期 |
|  |  |  | Y N |  |  |

| 1.2. Dispense Fmoc-Gly-OH/分装芴甲氧羰基-甘氨酸 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ins. No. for material use/分装物料计划用于的操作 | Lot number/批号 | Charging Qty/投入的质量 |  |  |  | No. /编号 | Performed by & Date/ 执行人及日期 | Checked by & Date/ 核实人及日期 |
|  |  | Gross weight/毛重 | Tare weight /皮重 |  | Net weight/ 净重 |  |  |  |
| 3.2.2 |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  | Total weight/合计重量 |  |  |  |  | - |  |  |
|  | Total No. of bags/合计袋数 |  |  |  |  | - |  |  |
| Begin/开始 |  | End/结束 |  |  |  |  |  |  |
| After solid material dispensing, confirm/固体物料分装之后，确认： - The corresponding leftover material has been properly sealed and transferred to the staging area. /相应的剩余物料已经恰当密封，转移到了暂存区域。 - The waste packaging material has been sealed and transferred to the corresponding waste staging area. /产生的包材废物已经密封并转移到了相应的废物暂存区域。 - The operation area/equipment surface has been cleaned. /操作区域以及设备表面已经清洁。 Ckecked by & Date/核实人及日期： |  |  |  |  |  |  |  |  |

| - Dispense Fmoc-L-Phe-OH/分装芴甲氧羰基-L-苯丙氨酸 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Before material dispensing, confirm the material is within the retest date/ the expiry date, Confirm whether the weight is adequate for use in the current dispensing. /物料分装前，执行物料在复检日期/有效期之内，确认重量是否满足本次分装使用。 Before solid material dispensing, confirm/固体物料分装前，执行： - No non-related material / equipment / instrument / tools / pipeline / document / record / label in the intended area. /目标区域无不相关物料/设备/仪器/工具/管路/文件/记录/标签。 - The outer packages are removed. If the outer package cannot be removed, but unpack material without damaging the package bag, and before dispensing, check no package fragment and other extraneous matter in material. /外包装移除；如果外包装不能移除，但在不破坏包装的情况下打开包装袋，分装前以及过程中同时检查执行物料中无包装碎片以及异物。 Performed by & Date/执行人及日期： |  |  |  |  |  |
| Key raw materials, in-process products and products for PPQ and commercial production should be weighed after the weighing implements are checked using the standard weights. For the balance/scale with the maximum capacity of greater than 40 kg, check with the standard weight of 20 kg; for the balance/scale with maximum capacity is not more than 40 kg, check with the standard weight of 1/2 maximum capacity. /用于工艺性能执行和商业化生产的主原料、过程产物以及产品的称量前，应使用标准砝码对称量器具进行检查，量程大于40 kg的电子秤，采用20 kg的标准砝码进行检查；量程不大于40 kg的电子秤，采用最大量程1/2的标准砝码进行检查。 Checking record for standard weights/标准砝码检查记录： |  |  |  |  |  |
| Weight No. /砝码编号 | Weight/ 砝码重量（kg） | Reading/ 示重 | Less than half of verification scale interval/ 小于检定分度值的一半 | Performed by & Date/ 执行人及日期 | Checked by & Date/ 核实人及日期 |
|  |  |  | Y N |  |  |
| Dispense Fmoc-L-Phe-OH. /分装 kg芴甲氧羰基-L-苯丙氨酸（Acceptable weighing range/可接受称量范围： kg ~ kg）。 For the Standard Batch Quantity, see Page 32~62 of 624 the information in "Raw Material Summary Table" corresponding to each unit operation/对于批标准投料量，参照Page 32~62 of 624“原料汇总表”中对应的每个单元操作。 Control No./内部控制号：YL004840 Scale ID. No./秤编号： Glovebox number /手套箱编号： Lot number of LDPE bag/低密度聚乙烯投料袋批号： |  |  |  |  |  |

| 1.3. Dispense Fmoc-L-Phe-OH/分装芴甲氧羰基-L-苯丙氨酸 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ins. No. for material use/分装物料计划用于的操作 | Lot number/批号 | Charging Qty/投入的质量 |  |  |  | No. /编号 | Performed by & Date/ 执行人及日期 | Checked by & Date/ 核实人及日期 |
|  |  | Gross weight/毛重 | Tare weight /皮重 |  | Net weight/ 净重 |  |  |  |
| 3.6.2 |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  | Total weight/合计重量 |  |  |  |  | - |  |  |
|  | Total No. of bags/合计袋数 |  |  |  |  | - |  |  |
| Begin/开始 |  | End/结束 |  |  |  |  |  |  |
| After solid material dispensing, confirm/固体物料分装之后，确认： - The corresponding leftover material has been properly sealed and transferred to the staging area. /相应的剩余物料已经恰当密封，转移到了暂存区域。 - The waste packaging material has been sealed and transferred to the corresponding waste staging area. /产生的包材废物已经密封并转移到了相应的废物暂存区域。 - The operation area/equipment surface has been cleaned./操作区域以及设备表面已经清洁。 Checked by & Date/核实人及日期： |  |  |  |  |  |  |  |  |

- Dispense PyBOP/分装1H-苯并三唑-1-基氧三吡咯烷基六氟磷酸盐

Before material dispensing, confirm the material is within the retest date/ the expiry date, Confirm whether the weight is adequate for use in the current dispensing. /物料分装前，执行物料在复检日期/有效期之内，确认重量是否满足本次分装使用。 Before solid material dispensing, confirm/固体物料分装前，确认： - No non-related material / equipment / instrument / tools / pipeline / document / record / label in the intended area. /目标区域无不相关物料/设备/仪器/工具/管路/文件/记录/标签； - The outer packages are removed. If the outer package cannot be removed, but unpack material without damaging the package bag, and before dispensing, check no package fragment and other extraneous matter in material. /外包装移除；如果外包装不能移除，但在不破坏包装的情况下打开包装袋，分装前以及过程中同时检查执行物料中无包装碎片以及异物。 Checked by & Date/核实人及日期：

Dispense PyBOP. /分装 kg 1H-苯并三唑-1-基氧三吡咯烷基六氟磷酸盐（Acceptable weighing range/可接受称量范围： kg ~ kg）。 For the Standard Batch Quantity, see Page 32~62 of 624 the information in "Raw Material Summary Table" corresponding to each unit operation/对于批标准投料量，参照Page 32~62 of 624“原料汇总表”中对应的每个单元操作。 Control No./内部控制号：YL002855 Scale ID. No./秤编号： Glovebox number /手套箱编号： Lot number of LDPE powder feeding bag/低密度聚乙烯粉末投料袋批号： Glovebox number /手套箱编号：

| 1.4.Dispense PyBOP/分装1H-苯并三唑-1-基氧三吡咯烷基六氟磷酸盐 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ins. No. for material use/分装物料计划用于的操作 | Lot number/批号 | Charging Qty/投入的质量 |  |  |  | No. /编号 | Performed by & Date/ 执行人及日期 | Checked by & Date/ 核实人及日期 |
|  |  | Gross weight/毛重 | Tare weight /皮重 |  | Net weight/ 净重 |  |  |  |
| 3.6.5 |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  | Total weight/合计重量 |  |  |  |  | - |  |  |
|  | Total No. of bags/合计袋数 |  |  |  |  | - |  |  |
| Begin/开始 |  | End/结束 |  |  |  |  |  |  |
| After solid material dispensing, confirm/固体物料分装之后，确认： - The corresponding leftover material has been properly sealed and transferred to the staging area. /相应的剩余物料已经恰当密封，转移到了暂存区域。 - The waste packaging material has been sealed and transferred to the corresponding waste staging area. /产生的包材废物已经密封并转移到了相应的废物暂存区域。 - The operation area/equipment surface has been cleaned. /操作区域以及设备表面已经清洁。 Checked by & Date/核实人及日期： |  |  |  |  |  |  |  |  |

- Dispense OxymaPure/分装2-肟氰乙酸乙酯

Before material dispensing, confirm the material is within the retest date/ the expiry date, Confirm whether the weight is adequate for use in the current dispensing. /物料分装前，执行物料在复检日期/有效期之内，确认重量是否满足本次分装使用。 Before solid material dispensing, confirm/固体物料分装前，执行： - No non-related material / equipment / instrument / tools / pipeline / document / record / label in the intended area. /目标区域无不相关物料/设备/仪器/工具/管路/文件/记录/标签； - The outer packages are removed. If the outer package cannot be removed, but unpack material without damaging the package bag, and before dispensing, check no package fragment and other extraneous matter in material. /外包装移除；如果外包装不能移除，但在不破坏包装的情况下打开包装袋，分装前以及过程中同时检查执行物料中无包装碎片以及异物。 Performed by & Date/执行人及日期：

Dispense OxymaPure. /分装 kg 2-肟氰乙酸乙酯（Acceptable weighing range/可接受称量范围： kg ~ kg）。 For the Standard Batch Quantity, see Page 32~62 of 624 the information in "Raw Material Summary Table" corresponding to each unit operation/对于批标准投料量，参照Page 32~62 of 624“原料汇总表”中对应的每个单元操作。 Control No./内部控制号：YL005180 Scale ID. No./秤编号： Glovebox number /手套箱编号： Lot number of LDPE bag/低密度聚乙烯投料袋批号：

| 1.5. Dispense OxymaPure/分装2-肟氰乙酸乙酯 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ins. No. for material use/分装物料计划用于的操作 | Lot number/批号 | Charging Qty/投入的质量 |  |  |  | No. /编号 | Performed by & Date/ 执行人及日期 | Checked by & Date/ 核实人及日期 |
|  |  | Gross weight/毛重 | Tare weight /皮重 |  | Net weight/ 净重 |  |  |  |
| 3.6.6 |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  | Total weight/合计重量 |  |  |  |  | - |  |  |
|  | Total No. of bags/合计袋数 |  |  |  |  | - |  |  |
| Begin/开始 |  | End/结束 |  |  |  |  |  |  |
| After solid material dispensing, confirm/固体物料分装之后，确认： - The corresponding leftover material has been properly sealed and transferred to the staging area. /相应的剩余物料已经恰当密封，转移到了暂存区域。 - The waste packaging material has been sealed and transferred to the corresponding waste staging area. /产生的包材废物已经密封并转移到了相应的废物暂存区域。 - The operation area/equipment surface has been cleaned. /操作区域以及设备表面已经清洁。 Performed by & Date/执行人及日期： |  |  |  |  |  |  |  |  |

| - Dispense Fmoc-L-N-Me-Asn(Trt)-OH/分装Fmoc-N-甲基-N’-三苯甲基-天冬氨酰胺 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Before material dispensing, confirm the material is within the retest date/ the expiry date, Confirm whether the weight is adequate for use in the current dispensing. /物料分装前，执行物料在复检日期/有效期之内，确认重量是否满足本次分装使用。 Before solid material dispensing, confirm/固体物料分装前，执行： - No non-related material / equipment / instrument / tools / pipeline / document / record / label in the intended area. /目标区域无不相关物料/设备/仪器/工具/管路/文件/记录/标签。 - The outer packages are removed. If the outer package cannot be removed, but unpack material without damaging the package bag, and before dispensing, check no package fragment and other extraneous matter in material. /外包装移除；如果外包装不能移除，但在不破坏包装的情况下打开包装袋，分装前以及过程中同时检查执行物料中无包装碎片以及异物。 Performed by & Date/执行人及日期： |  |  |  |  |  |
| Key raw materials, in-process products and products for PPQ and commercial production should be weighed after the weighing implements are checked using the standard weights. For the balance/scale with the maximum capacity of greater than 40 kg, check with the standard weight of 20 kg; for the balance/scale with maximum capacity is not more than 40 kg, check with the standard weight of 1/2 maximum capacity. /用于工艺性能执行和商业化生产的主原料、过程产物以及产品的称量前，应使用标准砝码对称量器具进行检查，量程大于40 kg的电子秤，采用20 kg的标准砝码进行检查；量程不大于40 kg的电子秤，采用最大量程1/2的标准砝码进行检查。 Checking record for standard weights/标准砝码检查记录： |  |  |  |  |  |
| Weight No. /砝码编号 | Weight/ 砝码重量（kg） | Reading/ 示重 | Less than half of verification scale interval/ 小于检定分度值的一半 | Performed by & Date/ 执行人及日期 | Checked by & Date/ 核实人及日期 |
|  |  |  | Y N |  |  |
| Dispense Fmoc-L-N-Me-Asn(Trt)-OH. /分装 kg Fmoc-N-甲基-N’-三苯甲基-天冬氨酰胺（Acceptable weighing range/可接受称量范围： kg ~ kg）。 For the Standard Batch Quantity, see Page 32~62 of 624 the information in "Raw Material Summary Table" corresponding to each unit operation/对于批标准投料量，参照Page 32~62 of 624“原料汇总表”中对应的每个单元操作。 Control No./内部控制号： YL008617 Scale ID. No./秤编号： Glovebox number /手套箱编号： Lot number of LDPE bag/低密度聚乙烯投料袋批号： |  |  |  |  |  |

| 1.6. Dispense Fmoc-L-N-Me-Asn(Trt)-OH/分装Fmoc-N-甲基-N’-三苯甲基-天冬氨酰胺 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ins. No. for material use/分装物料计划用于的操作 | Lot number/批号 | Charging Qty/投入的质量 |  |  |  | No. /编号 | Performed by & Date/ 执行人及日期 | Checked by & Date/ 核实人及日期 |
|  |  | Gross weight/毛重 | Tare weight /皮重 |  | Net weight/ 净重 |  |  |  |
| 3.9.2 |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  | Total weight/合计重量 |  |  |  |  | - |  |  |
|  | Total No. of bags/合计袋数 |  |  |  |  | - |  |  |
| Begin/开始 |  | End/结束 |  |  |  |  |  |  |
| After solid material dispensing, confirm/固体物料分装之后，确认： - The corresponding leftover material has been properly sealed and transferred to the staging area./相应的剩余物料已经恰当密封，转移到了暂存区域。 - The waste packaging material has been sealed and transferred to the corresponding waste staging area. /产生的包材废物已经密封并转移到了相应的废物暂存区域。 - The operation area/equipment surface has been cleaned./操作区域以及设备表面已经清洁。 Performed by & Date/执行人及日期： |  |  |  |  |  |  |  |  |

- Dispense OxymaPure/分装2-肟氰乙酸乙酯

Before material dispensing, confirm the material is within the retest date/ the expiry date, Confirm whether the weight is adequate for use in the current dispensing. /物料分装前，执行物料在复检日期/有效期之内，确认重量是否满足本次分装使用。 Before solid material dispensing, confirm/固体物料分装前，确认： - No non-related material / equipment / instrument / tools / pipeline / document / record / label in the intended area. /目标区域无不相关物料/设备/仪器/工具/管路/文件/记录/标签； - The outer packages are removed. If the outer package cannot be removed, but unpack material without damaging the package bag, and before dispensing, check no package fragment and other extraneous matter in material. /外包装移除；如果外包装不能移除，但在不破坏包装的情况下打开包装袋，分装前以及过程中同时检查执行物料中无包装碎片以及异物。 Checked by & Date/核实人及日期：

Dispense OxymaPure. /分装 kg 2-肟氰乙酸乙酯（Acceptable weighing range/可接受称量范围： kg ~ kg）。 For the Standard Batch Quantity, see Page 32~62 of 624 the information in "Raw Material Summary Table" corresponding to each unit operation/对于批标准投料量，参照Page 32~62 of 624“原料汇总表”中对应的每个单元操作。 Control No./内部控制号：YL005180 Scale ID. No./秤编号： Lot number of LDPE powder feeding bag/低密度聚乙烯粉末投料袋批号： Glovebox number /手套箱编号：

| 1.7. Dispense OxymaPure/分装2-肟氰乙酸乙酯 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ins. No. for material use/分装物料计划用于的操作 | Lot number/批号 | Charging Qty/投入的质量 |  |  |  | No. /编号 | Performed by & Date/ 执行人及日期 | Checked by & Date/ 核实人及日期 |
|  |  | Gross weight/毛重 | Tare weight /皮重 |  | Net weight/ 净重 |  |  |  |
| 3.9.5 |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  | Total weight/合计重量 |  |  |  |  | - |  |  |
|  | Total No. of bags/合计袋数 |  |  |  |  | - |  |  |
| Begin/开始 |  | End/结束 |  |  |  |  |  |  |
| After solid material dispensing, confirm/固体物料分装之后，确认： - The corresponding leftover material has been properly sealed and transferred to the staging area. /相应的剩余物料已经恰当密封，转移到了暂存区域。 - The waste packaging material has been sealed and transferred to the corresponding waste staging area. /产生的包材废物已经密封并转移到了相应的废物暂存区域。 - The operation area/equipment surface has been cleaned/操作区域以及设备表面已经清洁。 Checked by & Date/核实人及日期： |  |  |  |  |  |  |  |  |

| - Dispense Fmoc-L-Asn(Trt)-OH/分装Fmoc-N-三苯甲基-L-天冬酰胺 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Before material dispensing, confirm the material is within the retest date/ the expiry date, Confirm whether the weight is adequate for use in the current dispensing. /物料分装前，执行物料在复检日期/有效期之内，确认重量是否满足本次分装使用。 Before solid material dispensing, confirm/固体物料分装前，确认： - No non-related material / equipment / instrument / tools / pipeline / document / record / label in the intended area. /目标区域无不相关物料/设备/仪器/工具/管路/文件/记录/标签； - The outer packages are removed. If the outer package cannot be removed, but unpack material without damaging the package bag, and before dispensing, check no package fragment and other extraneous matter in material. /外包装移除；如果外包装不能移除，但在不破坏包装的情况下打开包装袋，分装前以及过程中同时检查执行物料中无包装碎片以及异物。 Checked by & Date/核实人及日期： |  |  |  |  |  |
| Key raw materials, in-process products and products for PPQ and commercial production should be weighed after the weighing implements are checked using the standard weights. For the balance/scale with the maximum capacity of greater than 40 kg, check with the standard weight of 20 kg; for the balance/scale with maximum capacity is not more than 40 kg, check with the standard weight of 1/2 maximum capacity. /用于工艺性能执行和商业化生产的主原料、过程产物以及产品的称量前，应使用标准砝码对称量器具进行检查，量程大于40 kg的电子秤，采用20 kg的标准砝码进行检查；量程不大于40 kg的电子秤，采用最大量程1/2的标准砝码进行检查。 Checking record for standard weights/标准砝码检查记录： |  |  |  |  |  |
| Weight No. /砝码编号 | Weight/ 砝码重量（kg） | Reading/ 示重 | Less than half of verification scale interval/ 小于检定分度值的一半 | Performed by & Date/ 执行人及日期 | Checked by & Date/ 核实人及日期 |
|  |  |  | Y N |  |  |
| Dispense Fmoc-L-Asn(Trt)-OH. /分装 kg Fmoc-N-三苯甲基-L-天冬酰胺（Acceptable weighing range/可接受称量范围： kg ~ kg）。 For the Standard Batch Quantity, see Page 32~62 of 624 the information in "Raw Material Summary Table" corresponding to each unit operation/对于批标准投料量，参照Page 32~62 of 624“原料汇总表”中对应的每个单元操作。 Control No./内部控制号： YL004843 Scale ID. No./秤编号： Lot number of LDPE powder feeding bag/低密度聚乙烯粉末投料袋批号： Glovebox number /手套箱编号： |  |  |  |  |  |

| 1.8. Dispense Fmoc-L-Asn(Trt)-OH/分装Fmoc-N-三苯甲基-L-天冬酰胺 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ins. No. for material use/分装物料计划用于的操作 | Lot number/批号 | Charging Qty/投入的质量 |  |  |  | No. /编号 | Performed by & Date/ 执行人及日期 | Checked by & Date/ 核实人及日期 |
|  |  | Gross weight/毛重 | Tare weight /皮重 |  | Net weight/ 净重 |  |  |  |
| 3.12.2 |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  | Total weight/合计重量 |  |  |  |  | - |  |  |
|  | Total No. of bags/合计袋数 |  |  |  |  | - |  |  |
| Begin/开始 |  | End/结束 |  |  |  |  |  |  |
| After solid material dispensing, confirm/固体物料分装之后，确认： - The corresponding leftover material has been properly sealed and transferred to the staging area./相应的剩余物料已经恰当密封，转移到了暂存区域。 - The waste packaging material has been sealed and transferred to the corresponding waste staging area. /产生的包材废物已经密封并转移到了相应的废物暂存区域。 - The operation area/equipment surface has been cleaned/操作区域以及设备表面已经清洁。 Checked by & Date/核实人及日期： |  |  |  |  |  |  |  |  |

- Dispense OxymaPure/分装2-肟氰乙酸乙酯

Before material dispensing, confirm the material is within the retest date/ the expiry date, Confirm whether the weight is adequate for use in the current dispensing. /物料分装前，执行物料在复检日期/有效期之内，确认重量是否满足本次分装使用。 Before solid material dispensing, confirm/固体物料分装前，确认： - No non-related material / equipment / instrument / tools / pipeline / document / record / label in the intended area. /目标区域无不相关物料/设备/仪器/工具/管路/文件/记录/标签； - The outer packages are removed. If the outer package cannot be removed, but unpack material without damaging the package bag, and before dispensing, check no package fragment and other extraneous matter in material. /外包装移除；如果外包装不能移除，但在不破坏包装的情况下打开包装袋，分装前以及过程中同时检查执行物料中无包装碎片以及异物。 Checked by & Date/核实人及日期：

Dispense OxymaPure. /分装 kg 2-肟氰乙酸乙酯（Acceptable weighing range/可接受称量范围： kg ~ kg）。 For the Standard Batch Quantity, see Page 32~62 of 624 the information in "Raw Material Summary Table" corresponding to each unit operation/对于批标准投料量，参照Page 32~62 of 624“原料汇总表”中对应的每个单元操作。 Control No./内部控制号：YL005180 Scale ID. No./秤编号： Lot number of LDPE powder feeding bag/低密度聚乙烯粉末投料袋批号： Glovebox number /手套箱编号：

| 1.9. Dispense OxymaPure/分装2-肟氰乙酸乙酯 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ins. No. for material use/分装物料计划用于的操作 | Lot number/批号 | Charging Qty/投入的质量 |  |  |  | No. /编号 | Performed by & Date/ 执行人及日期 | Checked by & Date/ 核实人及日期 |
|  |  | Gross weight/毛重 | Tare weight /皮重 |  | Net weight/ 净重 |  |  |  |
| 3.12.5 |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  | Total weight/合计重量 |  |  |  |  | - |  |  |
|  | Total No. of bags/合计袋数 |  |  |  |  | - |  |  |
| Begin/开始 |  | End/结束 |  |  |  |  |  |  |
| After solid material dispensing, confirm/固体物料分装之后，确认： - The corresponding leftover material has been properly sealed and transferred to the staging area. /相应的剩余物料已经恰当密封，转移到了暂存区域。 - The waste packaging material has been sealed and transferred to the corresponding waste staging area. /产生的包材废物已经密封并转移到了相应的废物暂存区域。 - The operation area/equipment surface has been cleaned/操作区域以及设备表面已经清洁。 Checked by & Date/核实人及日期： |  |  |  |  |  |  |  |  |

- Dispense OxymaPure/分装2-肟氰乙酸乙酯

Before material dispensing, confirm the material is within the retest date/ the expiry date, Confirm whether the weight is adequate for use in the current dispensing. /物料分装前，执行物料在复检日期/有效期之内，确认重量是否满足本次分装使用。 Before solid material dispensing, confirm/固体物料分装前，确认： - No non-related material / equipment / instrument / tools / pipeline / document / record / label in the intended area./目标区域无不相关物料/设备/仪器/工具/管路/文件/记录/标签； - The outer packages are removed. If the outer package cannot be removed, but unpack material without damaging the package bag, and before dispensing, check no package fragment and other extraneous matter in material. /外包装移除；如果外包装不能移除，但在不破坏包装的情况下打开包装袋，分装前以及过程中同时检查执行物料中无包装碎片以及异物。 Checked by & Date/核实人及日期：

Dispense OxymaPure. /分装 kg 2-肟氰乙酸乙酯（Acceptable weighing range/可接受称量范围： kg ~ kg）。 For the Standard Batch Quantity, see Page 32~62 of 624 the information in "Raw Material Summary Table" corresponding to each unit operation/对于批标准投料量，参照Page 32~62 of 624“原料汇总表”中对应的每个单元操作。 Control No./内部控制号：YL005180 Scale ID. No./秤编号： Lot number of LDPE powder feeding bag/低密度聚乙烯粉末投料袋批号： Glovebox number /手套箱编号：

| 1.10. Dispense OxymaPure/分装2-肟氰乙酸乙酯 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ins. No. for material use/分装物料计划用于的操作 | Lot number/批号 | Charging Qty/投入的质量 |  |  |  | No. /编号 | Performed by & Date/ 执行人及日期 | Checked by & Date/ 核实人及日期 |
|  |  | Gross weight/毛重 | Tare weight /皮重 |  | Net weight/ 净重 |  |  |  |
| 3.13.6 |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  | Total weight/合计重量 |  |  |  |  | - |  |  |
|  | Total No. of bags/合计袋数 |  |  |  |  | - |  |  |
| Begin/开始 |  | End/结束 |  |  |  |  |  |  |
| After solid material dispensing, confirm/固体物料分装之后，确认： - The corresponding leftover material has been properly sealed and transferred to the staging area./相应的剩余物料已经恰当密封，转移到了暂存区域。 - The waste packaging material has been sealed and transferred to the corresponding waste staging area. /产生的包材废物已经密封并转移到了相应的废物暂存区域。 - The operation area/equipment surface has been cleaned/操作区域以及设备表面已经清洁。 Checked by & Date/核实人及日期： |  |  |  |  |  |  |  |  |

| - Dispense Fmoc-L-Ser(tBu)-OH/分装芴甲氧羰基-叔丁基-L-丝氨酸 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Before material dispensing, confirm the material is within the retest date/ the expiry date, Confirm whether the weight is adequate for use in the current dispensing. /物料分装前，执行物料在复检日期/有效期之内，确认重量是否满足本次分装使用。 Before solid material dispensing, confirm/固体物料分装前，确认： - No non-related material / equipment / instrument / tools / pipeline / document / record / label in the intended area./目标区域无不相关物料/设备/仪器/工具/管路/文件/记录/标签； - The outer packages are removed. If the outer package cannot be removed, but unpack material without damaging the package bag, and before dispensing, check no package fragment and other extraneous matter in material. /外包装移除；如果外包装不能移除，但在不破坏包装的情况下打开包装袋，分装前以及过程中同时检查执行物料中无包装碎片以及异物。 Checked by & Date/核实人及日期： |  |  |  |  |  |
| Key raw materials, in-process products and products for PPQ and commercial production should be weighed after the weighing implements are checked using the standard weights. For the balance/scale with the maximum capacity of greater than 40 kg, check with the standard weight of 20 kg; for the balance/scale with maximum capacity is not more than 40 kg, check with the standard weight of 1/2 maximum capacity. /用于工艺性能执行和商业化生产的主原料、过程产物以及产品的称量前，应使用标准砝码对称量器具进行检查，量程大于40 kg的电子秤，采用20 kg的标准砝码进行检查；量程不大于40 kg的电子秤，采用最大量程1/2的标准砝码进行检查。 Checking record for standard weights/标准砝码检查记录： |  |  |  |  |  |
| Weight No. /砝码编号 | Weight/ 砝码重量（kg） | Reading/ 示重 | Less than half of verification scale interval/ 小于检定分度值的一半 | Performed by & Date/ 执行人及日期 | Checked by & Date/ 核实人及日期 |
|  |  |  | Y N |  |  |
| Dispense Fmoc-L-Ser(tBu)-OH. /分装 kg芴甲氧羰基-叔丁基-L-丝氨酸（Acceptable weighing range/可接受称量范围： kg ~ kg） For the Standard Batch Quantity, see Page 32~62 of 624 the information in "Raw Material Summary Table" corresponding to each unit operation/对于批标准投料量，参照Page 32~62 of 624“原料汇总表”中对应的每个单元操作。 Control No./内部控制号：YL003428 Scale ID. No./秤编号： Lot number of LDPE powder feeding bag/低密度聚乙烯粉末投料袋批号： Glovebox number /手套箱编号： |  |  |  |  |  |

| 1.11. Dispense Fmoc-L-Ser(tBu)-OH/分装芴甲氧羰基-叔丁基-L-丝氨酸 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ins. No. for material use/分装物料计划用于的操作 | Lot number/批号 | Charging Qty/投入的质量 |  |  |  | No. /编号 | Performed by & Date/ 执行人及日期 | Checked by & Date/ 核实人及日期 |
|  |  | Gross weight/毛重 | Tare weight /皮重 |  | Net weight/ 净重 |  |  |  |
| 3.17.2 |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  | Total weight/合计重量 |  |  |  |  | - |  |  |
|  | Total No. of bags/合计袋数 |  |  |  |  | - |  |  |
| Begin/开始 |  | End/结束 |  |  |  |  |  |  |
| After solid material dispensing, confirm/固体物料分装之后，确认： - The corresponding leftover material has been properly sealed and transferred to the staging area/相应的剩余物料已经恰当密封，转移到了暂存区域。 - The waste packaging material has been sealed and transferred to the corresponding waste staging area. /产生的包材废物已经密封并转移到了相应的废物暂存区域。 - The operation area/equipment surface has been cleaned/操作区域以及设备表面已经清洁。 Checked by & Date/核实人及日期： |  |  |  |  |  |  |  |  |

- Dispense OxymaPure/分装2-肟氰乙酸乙酯

Before material dispensing, confirm the material is within the retest date/ the expiry date, Confirm whether the weight is adequate for use in the current dispensing. /物料分装前，执行物料在复检日期/有效期之内，确认重量是否满足本次分装使用。 Before solid material dispensing, confirm/固体物料分装前，确认： - No non-related material / equipment / instrument / tools / pipeline / document / record / label in the intended area /目标区域无不相关物料/设备/仪器/工具/管路/文件/记录/标签； - The outer packages are removed. If the outer package cannot be removed, but unpack material without damaging the package bag, and before dispensing, check no package fragment and other extraneous matter in material. /外包装移除；如果外包装不能移除，但在不破坏包装的情况下打开包装袋，分装前以及过程中同时检查执行物料中无包装碎片以及异物 Checked by & Date/核实人及日期：

Dispense OxymaPure. /分装 kg 2-肟氰乙酸乙酯（Acceptable weighing range/可接受称量范围： kg ~ kg）。 For the Standard Batch Quantity, see Page 32~62 of 624 the information in "Raw Material Summary Table" corresponding to each unit operation/对于批标准投料量，参照Page 32~62 of 624“原料汇总表”中对应的每个单元操作。 Control No./内部控制号：YL005180 Scale ID. No./秤编号： Lot number of LDPE powder feeding bag/低密度聚乙烯粉末投料袋批号： Glovebox number /手套箱编号：

| 1.12. Dispense OxymaPure/分装2-肟氰乙酸乙酯 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ins. No. for material use/分装物料计划用于的操作 | Lot number/批号 | Charging Qty/投入的质量 |  |  |  | No. /编号 | Performed by & Date/ 执行人及日期 | Checked by & Date/ 核实人及日期 |
|  |  | Gross weight/毛重 | Tare weight /皮重 |  | Net weight/ 净重 |  |  |  |
| 3.17.5 |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  | Total weight/合计重量 |  |  |  |  | - |  |  |
|  | Total No. of bags/合计袋数 |  |  |  |  | - |  |  |
| Begin/开始 |  | End/结束 |  |  |  |  |  |  |
| After solid material dispensing, confirm/固体物料分装之后，确认： - The corresponding leftover material has been properly sealed and transferred to the staging area. /相应的剩余物料已经恰当密封，转移到了暂存区域。 - The waste packaging material has been sealed and transferred to the corresponding waste staging area. /产生的包材废物已经密封并转移到了相应的废物暂存区域。 - The operation area/equipment surface has been cleaned./操作区域以及设备表面已经清洁。 Checked by & Date/核实人及日期： |  |  |  |  |  |  |  |  |

| - Dispense Fmoc-L-Ser(tBu)-OH/分装芴甲氧羰基-叔丁基-L-丝氨酸 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Before material dispensing, confirm the material is within the retest date/ the expiry date, Confirm whether the weight is adequate for use in the current dispensing. /物料分装前，执行物料在复检日期/有效期之内，确认重量是否满足本次分装使用。 Before solid material dispensing, confirm/固体物料分装前，确认： - No non-related material / equipment / instrument / tools / pipeline / document / record / label in the intended area. /目标区域无不相关物料/设备/仪器/工具/管路/文件/记录/标签； - The outer packages are removed. If the outer package cannot be removed, but unpack material without damaging the package bag, and before dispensing, check no package fragment and other extraneous matter in material. /外包装移除；如果外包装不能移除，但在不破坏包装的情况下打开包装袋，分装前以及过程中同时检查执行物料中无包装碎片以及异物。 Checked by & Date/核实人及日期： |  |  |  |  |  |
| Key raw materials, in-process products and products for PPQ and commercial production should be weighed after the weighing implements are checked using the standard weights. For the balance/scale with the maximum capacity of greater than 40 kg, check with the standard weight of 20 kg; for the balance/scale with maximum capacity is not more than 40 kg, check with the standard weight of 1/2 maximum capacity. /用于工艺性能执行和商业化生产的主原料、过程产物以及产品的称量前，应使用标准砝码对称量器具进行检查，量程大于40 kg的电子秤，采用20 kg的标准砝码进行检查；量程不大于40 kg的电子秤，采用最大量程1/2的标准砝码进行检查。 Checking record for standard weights/标准砝码检查记录： |  |  |  |  |  |
| Weight No. /砝码编号 | Weight/ 砝码重量（kg） | Reading/ 示重 | Less than half of verification scale interval/ 小于检定分度值的一半 | Performed by & Date/ 执行人及日期 | Checked by & Date/ 核实人及日期 |
|  |  |  | Y N |  |  |
| Dispense Fmoc-L-Ser(tBu)-OH. /分装 kg芴甲氧羰基-叔丁基-L-丝氨酸（Acceptable weighing range/可接受称量范围： kg ~ kg）。 For the Standard Batch Quantity, see Page 32~62 of 624 the information in "Raw Material Summary Table" corresponding to each unit operation/对于批标准投料量，参照Page 32~62 of 624“原料汇总表”中对应的每个单元操作。 Control No./内部控制号：YL003428 Scale ID. No./秤编号： Lot number of LDPE powder feeding bag/低密度聚乙烯粉末投料袋批号： Glovebox number /手套箱编号： |  |  |  |  |  |

| 1.13. Dispense Fmoc-L-Ser(tBu)-OH/分装芴甲氧羰基-叔丁基-L-丝氨酸 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ins. No. for material use/分装物料计划用于的操作 | Lot number/批号 | Charging Qty/投入的质量 |  |  |  | No. /编号 | Performed by & Date/ 执行人及日期 | Checked by & Date/ 核实人及日期 |
|  |  | Gross weight/毛重 | Tare weight /皮重 |  | Net weight/ 净重 |  |  |  |
| 3.20.2 |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  | Total weight/合计重量 |  |  |  |  | - |  |  |
|  | Total No. of bags/合计袋数 |  |  |  |  | - |  |  |
| Begin/开始 |  | End/结束 |  |  |  |  |  |  |
| After solid material dispensing, confirm/固体物料分装之后，确认： - The corresponding leftover material has been properly sealed and transferred to the staging area./相应的剩余物料已经恰当密封，转移到了暂存区域。 - The waste packaging material has been sealed and transferred to the corresponding waste staging area. /产生的包材废物已经密封并转移到了相应的废物暂存区域。 - The operation area/equipment surface has been cleaned./操作区域以及设备表面已经清洁。 Checked by & Date/核实人及日期： |  |  |  |  |  |  |  |  |

- Dispense OxymaPure/分装2-肟氰乙酸乙酯

Before material dispensing, confirm the material is within the retest date/ the expiry date, Confirm whether the weight is adequate for use in the current dispensing. /物料分装前，执行物料在复检日期/有效期之内，确认重量是否满足本次分装使用。 Before solid material dispensing, confirm/固体物料分装前，确认： - No non-related material / equipment / instrument / tools / pipeline / document / record / label in the intended area. /目标区域无不相关物料/设备/仪器/工具/管路/文件/记录/标签； - The outer packages are removed. If the outer package cannot be removed, but unpack material without damaging the package bag, and before dispensing, check no package fragment and other extraneous matter in material. /外包装移除；如果外包装不能移除，但在不破坏包装的情况下打开包装袋，分装前以及过程中同时检查执行物料中无包装碎片以及异物。 Checkmed by & Date/核实人及日期：

Dispense OxymaPure. /分装 kg 2-肟氰乙酸乙酯（Acceptable weighing range/可接受称量范围： kg ~ kg）。 For the Standard Batch Quantity, see Page 32~62 of 624 the information in "Raw Material Summary Table" corresponding to each unit operation/对于批标准投料量，参照Page 32~62 of 624“原料汇总表”中对应的每个单元操作。 Control No./内部控制号：YL005180 Scale ID. No./秤编号： Lot number of LDPE powder feeding bag/低密度聚乙烯粉末投料袋批号： Glovebox number /手套箱编号：

| 1.14. Dispense OxymaPure/分装2-肟氰乙酸乙酯 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ins. No. for material use/分装物料计划用于的操作 | Lot number/批号 | Charging Qty/投入的质量 |  |  |  | No. /编号 | Performed by & Date/ 执行人及日期 | Checked by & Date/ 核实人及日期 |
|  |  | Gross weight/毛重 | Tare weight /皮重 |  | Net weight/ 净重 |  |  |  |
| 3.20.5 |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  | Total weight/合计重量 |  |  |  |  | - |  |  |
|  | Total No. of bags/合计袋数 |  |  |  |  | - |  |  |
| Begin/开始 |  | End/结束 |  |  |  |  |  |  |
| After solid material dispensing, confirm/固体物料分装之后，确认： - The corresponding leftover material has been properly sealed and transferred to the staging area. /相应的剩余物料已经恰当密封，转移到了暂存区域。 - The waste packaging material has been sealed and transferred to the corresponding waste staging area. /产生的包材废物已经密封并转移到了相应的废物暂存区域。 - The operation area/equipment surface has been cleaned. /操作区域以及设备表面已经清洁。 Checked by & Date人及日期： |  |  |  |  |  |  |  |  |

| - Dispense Fmoc-L-Arg(Pbf)-OH/分装Fmoc-Pbf-精氨酸 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Before material dispensing, confirm the material is within the retest date/ the expiry date, Confirm whether the weight is adequate for use in the current dispensing. /物料分装前，执行物料在复检日期/有效期之内，确认重量是否满足本次分装使用。 Before solid material dispensing, confirm/固体物料分装前，确认： - No non-related material / equipment / instrument / tools / pipeline / document / record / label in the intended area. /目标区域无不相关物料/设备/仪器/工具/管路/文件/记录/标签； - The outer packages are removed. If the outer package cannot be removed, but unpack material without damaging the package bag, and before dispensing, check no package fragment and other extraneous matter in material. /外包装移除；如果外包装不能移除，但在不破坏包装的情况下打开包装袋，分装前以及过程中同时检查执行物料中无包装碎片以及异物。 Checked by & Date/核实人及日期： |  |  |  |  |  |
| Key raw materials, in-process products and products for PPQ and commercial production should be weighed after the weighing implements are checked using the standard weights. For the balance/scale with the maximum capacity of greater than 40 kg, check with the standard weight of 20 kg; for the balance/scale with maximum capacity is not more than 40 kg, check with the standard weight of 1/2 maximum capacity. /用于工艺性能执行和商业化生产的主原料、过程产物以及产品的称量前，应使用标准砝码对称量器具进行检查，量程大于40 kg的电子秤，采用20 kg的标准砝码进行检查；量程不大于40 kg的电子秤，采用最大量程1/2的标准砝码进行检查。 Checking record for standard weights/标准砝码检查记录： |  |  |  |  |  |
| Weight No. /砝码编号 | Weight/ 砝码重量（kg） | Reading/ 示重 | Less than half of verification scale interval/小于检定分度值的一半 | Performed by & Date/ 执行人及日期 | Checked by & Date/ 核实人及日期 |
|  |  |  | Y N |  |  |
| Dispense Fmoc-L-Arg(Pbf)-OH. /分装 kg Fmoc-Pbf-精氨酸（Acceptable weighing range/可接受称量范围： kg ~ kg）。 For the Standard Batch Quantity, see Page 32~62 of 624 the information in "Raw Material Summary Table" corresponding to each unit operation/对于批标准投料量，参照Page 32~62 of 624“原料汇总表”中对应的每个单元操作。 Control No./内部控制号：YL003424 Scale ID. No./秤编号： Lot number of LDPE powder feeding bag/低密度聚乙烯粉末投料袋批号： Glovebox number /手套箱编号： |  |  |  |  |  |

| 1.15. Dispense Fmoc-L-Arg(Pbf)-OH/分装Fmoc-Pbf-精氨酸 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ins. No. for material use/分装物料计划用于的操作 | Lot number/批号 | Charging Qty/投入的质量 |  |  |  | No. /编号 | Performed by & Date/ 执行人及日期 | Checked by & Date/ 核实人及日期 |
|  |  | Gross weight/毛重 | Tare weight /皮重 |  | Net weight/ 净重 |  |  |  |
| 3.23.2 |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  | Total weight/合计重量 |  |  |  |  | - |  |  |
|  | Total No. of bags/合计袋数 |  |  |  |  | - |  |  |
| Begin/开始 |  | End/结束 |  |  |  |  |  |  |
| After solid material dispensing, confirm/固体物料分装之后，确认： - The corresponding leftover material has been properly sealed and transferred to the staging area. /相应的剩余物料已经恰当密封，转移到了暂存区域。 - The waste packaging material has been sealed and transferred to the corresponding waste staging area. /产生的包材废物已经密封并转移到了相应的废物暂存区域。 - The operation area/equipment surface has been cleaned. /操作区域以及设备表面已经清洁。 Checked by & Date/核实人及日期： |  |  |  |  |  |  |  |  |

- Dispense OxymaPure/分装2-肟氰乙酸乙酯

Before material dispensing, confirm the material is within the retest date/ the expiry date, Confirm whether the weight is adequate for use in the current dispensing. /物料分装前，执行物料在复检日期/有效期之内，确认重量是否满足本次分装使用。 Before solid material dispensing, confirm/固体物料分装前，确认： - No non-related material / equipment / instrument / tools / pipeline / document / record / label in the intended area. /目标区域无不相关物料/设备/仪器/工具/管路/文件/记录/标签； - The outer packages are removed. If the outer package cannot be removed, but unpack material without damaging the package bag, and before dispensing, check no package fragment and other extraneous matter in material. /外包装移除；如果外包装不能移除，但在不破坏包装的情况下打开包装袋，分装前以及过程中同时检查执行物料中无包装碎片以及异物。 Checked by & Date/核实人及日期：

Dispense OxymaPure. /分装 kg 2-肟氰乙酸乙酯（Acceptable weighing range/可接受称量范围： kg ~ kg）。 For the Standard Batch Quantity, see Page 32~62 of 624 the information in "Raw Material Summary Table" corresponding to each unit operation/对于批标准投料量，参照Page 32~62 of 624“原料汇总表”中对应的每个单元操作。 Control No./内部控制号：YL005180 Scale ID. No./秤编号： Lot number of LDPE powder feeding bag/低密度聚乙烯粉末投料袋批号： Glovebox number /手套箱编号：

| 1.16. Dispense OxymaPure/分装2-肟氰乙酸乙酯 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ins. No. for material use/分装物料计划用于的操作 | Lot number/批号 | Charging Qty/投入的质量 |  |  |  | No. /编号 | Performed by & Date/ 执行人及日期 | Checked by & Date/ 核实人及日期 |
|  |  | Gross weight/毛重 | Tare weight /皮重 |  | Net weight/ 净重 |  |  |  |
| 3.23.5 |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  | Total weight/合计重量 |  |  |  |  | - |  |  |
|  | Total No. of bags/合计袋数 |  |  |  |  | - |  |  |
| Begin/开始 |  | End/结束 |  |  |  |  |  |  |
| After solid material dispensing, confirm/固体物料分装之后，确认： - The corresponding leftover material has been properly sealed and transferred to the staging area. /相应的剩余物料已经恰当密封，转移到了暂存区域。 - The waste packaging material has been sealed and transferred to the corresponding waste staging area. /产生的包材废物已经密封并转移到了相应的废物暂存区域。 - The operation area/equipment surface has been cleaned. /操作区域以及设备表面已经清洁。 Checked by & Date/核实人及日期： |  |  |  |  |  |  |  |  |

| - Dispense Fmoc-L-Val-OH/分装芴甲氧羰基-L-缬氨酸 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Before material dispensing, confirm the material is within the retest date/ the expiry date, Confirm whether the weight is adequate for use in the current dispensing. /物料分装前，执行物料在复检日期/有效期之内，确认重量是否满足本次分装使用。 Before solid material dispensing, confirm/固体物料分装前，确认： - No non-related material / equipment / instrument / tools / pipeline / document / record / label in the intended area. /目标区域无不相关物料/设备/仪器/工具/管路/文件/记录/标签； - The outer packages are removed. If the outer package cannot be removed, but unpack material without damaging the package bag, and before dispensing, check no package fragment and other extraneous matter in material. /外包装移除；如果外包装不能移除，但在不破坏包装的情况下打开包装袋，分装前以及过程中同时检查执行物料中无包装碎片以及异物。 Checked by & Date/核实人及日期： |  |  |  |  |  |
| Key raw materials, in-process products and products for PPQ and commercial production should be weighed after the weighing implements are checked using the standard weights. For the balance/scale with the maximum capacity of greater than 40 kg, check with the standard weight of 20 kg; for the balance/scale with maximum capacity is not more than 40 kg, check with the standard weight of 1/2 maximum capacity. /用于工艺性能执行和商业化生产的主原料、过程产物以及产品的称量前，应使用标准砝码对称量器具进行检查，量程大于40 kg的电子秤，采用20 kg的标准砝码进行检查；量程不大于40 kg的电子秤，采用最大量程1/2的标准砝码进行检查。 Checking record for standard weights/标准砝码检查记录： |  |  |  |  |  |
| Weight No. /砝码编号 | Weight/ 砝码重量（kg） | Reading/ 示重 | Less than half of verification scale interval/ 小于检定分度值的一半 | Performed by & Date/ 执行人及日期 | Checked by & Date/ 核实人及日期 |
|  |  |  | Y N |  |  |
| Dispense Fmoc-L-Val-OH. /分装 kg芴甲氧羰基-L-缬氨酸（Acceptable weighing range/可接受称量范围： kg ~ kg）。 For the Standard Batch Quantity, see Page 32~62 of 624 the information in "Raw Material Summary Table" corresponding to each unit operation/对于批标准投料量，参照Page 32~62 of 624“原料汇总表”中对应的每个单元操作。 Control No./内部控制号：YL003110 Scale ID. No./秤编号： Lot number of LDPE powder feeding bag/低密度聚乙烯粉末投料袋批号： Glovebox number /手套箱编号： |  |  |  |  |  |

| 1.17. Dispense Fmoc-L-Val-OH/分装芴甲氧羰基-L-缬氨酸 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ins. No. for material use/分装物料计划用于的操作 | Lot number/批号 | Charging Qty/投入的质量 |  |  |  | No. /编号 | Performed by & Date/ 执行人及日期 | Checked by & Date/ 核实人及日期 |
|  |  | Gross weight/毛重 | Tare weight /皮重 |  | Net weight/ 净重 |  |  |  |
| 3.26.2 |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  | Total weight/合计重量 |  |  |  |  | - |  |  |
|  | Total No. of bags/合计袋数 |  |  |  |  | - |  |  |
| Begin/开始 |  | End/结束 |  |  |  |  |  |  |
| After solid material dispensing, confirm/固体物料分装之后，确认： - The corresponding leftover material has been properly sealed and transferred to the staging area. /相应的剩余物料已经恰当密封，转移到了暂存区域。 - The waste packaging material has been sealed and transferred to the corresponding waste staging area. /产生的包材废物已经密封并转移到了相应的废物暂存区域。 - The operation area/equipment surface has been cleaned. /操作区域以及设备表面已经清洁。 Checked by & Date/核实人及日期： |  |  |  |  |  |  |  |  |

- Dispense OxymaPure/分装2-肟氰乙酸乙酯

Before material dispensing, confirm the material is within the retest date/ the expiry date, Confirm whether the weight is adequate for use in the current dispensing. /物料分装前，执行物料在复检日期/有效期之内，确认重量是否满足本次分装使用。 Before solid material dispensing, confirm/固体物料分装前，确认： - No non-related material / equipment / instrument / tools / pipeline / document / record / label in the intended area. /目标区域无不相关物料/设备/仪器/工具/管路/文件/记录/标签； - The outer packages are removed. If the outer package cannot be removed, but unpack material without damaging the package bag, and before dispensing, check no package fragment and other extraneous matter in material. /外包装移除；如果外包装不能移除，但在不破坏包装的情况下打开包装袋，分装前以及过程中同时检查执行物料中无包装碎片以及异物。 Checked by & Date/核实人及日期：

Dispense OxymaPure. /分装 kg 2-肟氰乙酸乙酯（Acceptable weighing range/可接受称量范围： kg ~ kg）。 For the Standard Batch Quantity, see Page 32~62 of 624 the information in "Raw Material Summary Table" corresponding to each unit operation/对于批标准投料量，参照Page 32~62 of 624“原料汇总表”中对应的每个单元操作。 Control No./内部控制号：YL005180 Scale ID. No./秤编号： Lot number of LDPE powder feeding bag/低密度聚乙烯粉末投料袋批号： Glovebox number /手套箱编号：

| 1.18. Dispense OxymaPure/分装2-肟氰乙酸乙酯 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ins. No. for material use/分装物料计划用于的操作 | Lot number/批号 | Charging Qty/投入的质量 |  |  |  | No. /编号 | Performed by & Date/ 执行人及日期 | Checked by & Date/ 核实人及日期 |
|  |  | Gross weight/毛重 | Tare weight /皮重 |  | Net weight/ 净重 |  |  |  |
| 3.26.5 |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  | 7 |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  | Total weight/合计重量 |  |  |  |  | - |  |  |
|  | Total No. of bags/合计袋数 |  |  |  |  | - |  |  |
| Begin/开始 |  | End/结束 |  |  |  |  |  |  |
| After solid material dispensing, confirm/固体物料分装之后，确认： - The corresponding leftover material has been properly sealed and transferred to the staging area./相应的剩余物料已经恰当密封，转移到了暂存区域。 - The waste packaging material has been sealed and transferred to the corresponding waste staging area. /产生的包材废物已经密封并转移到了相应的废物暂存区域。 - The operation area/equipment surface has been cleaned./操作区域以及设备表面已经清洁。 Checked by & Date/核实人及日期： |  |  |  |  |  |  |  |  |

| - Dispense Fmoc-L-Leu-OH/分装芴甲氧羰基-L-亮氨酸 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Before material dispensing, confirm the material is within the retest date/ the expiry date, Confirm whether the weight is adequate for use in the current dispensing. /物料分装前，执行物料在复检日期/有效期之内，确认重量是否满足本次分装使用。 Before solid material dispensing, confirm/固体物料分装前，确认： - No non-related material / equipment / instrument / tools / pipeline / document / record / label in the intended area. /目标区域无不相关物料/设备/仪器/工具/管路/文件/记录/标签； - The outer packages are removed. If the outer package cannot be removed, but unpack material without damaging the package bag, and before dispensing, check no package fragment and other extraneous matter in material. /外包装移除；如果外包装不能移除，但在不破坏包装的情况下打开包装袋，分装前以及过程中同时检查执行物料中无包装碎片以及异物。 Checked by & Date/核实人及日期： |  |  |  |  |  |
| Key raw materials, in-process products and products for PPQ and commercial production should be weighed after the weighing implements are checked using the standard weights. For the balance/scale with the maximum capacity of greater than 40 kg, check with the standard weight of 20 kg; for the balance/scale with maximum capacity is not more than 40 kg, check with the standard weight of 1/2 maximum capacity. /用于工艺性能执行和商业化生产的主原料、过程产物以及产品的称量前，应使用标准砝码对称量器具进行检查，量程大于40 kg的电子秤，采用20 kg的标准砝码进行检查；量程不大于40 kg的电子秤，采用最大量程1/2的标准砝码进行检查。 Checking record for standard weights/标准砝码检查记录： |  |  |  |  |  |
| Weight No. /砝码编号 | Weight/ 砝码重量（kg） | Reading/ 示重 | Less than half of verification scale interval/ 小于检定分度值的一半 | Performed by & Date/ 执行人及日期 | Checked by & Date/ 核实人及日期 |
|  |  |  | Y N |  |  |
| Dispense Fmoc-L-Leu-OH. /分装 kg芴甲氧羰基-L-亮氨酸（Acceptable weighing range/可接受称量范围： kg ~ kg） For the Standard Batch Quantity, see Page 32~62 of 624 the information in "Raw Material Summary Table" corresponding to each unit operation/对于批标准投料量，参照Page 32~62 of 624“原料汇总表”中对应的每个单元操作。 Control No./内部控制号：YL003425 Scale ID. No./秤编号： Lot number of LDPE powder feeding bag/低密度聚乙烯粉末投料袋批号： Glovebox number /手套箱编号： |  |  |  |  |  |

| 1.19. Dispense Fmoc-L-Leu-OH/分装芴甲氧羰基-L-亮氨酸 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ins. No. for material use/分装物料计划用于的操作 | Lot number/批号 | Charging Qty/投入的质量 |  |  |  | No. /编号 | Performed by & Date/ 执行人及日期 | Checked by & Date/ 核实人及日期 |
|  |  | Gross weight/毛重 | Tare weight /皮重 |  | Net weight/ 净重 |  |  |  |
| 3.29.2 |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  | Total weight/合计重量 |  |  |  |  | - |  |  |
|  | Total No. of bags/合计袋数 |  |  |  |  | - |  |  |
| Begin/开始 |  | End/结束 |  |  |  |  |  |  |
| After solid material dispensing, confirm/固体物料分装之后，确认： - The corresponding leftover material has been properly sealed and transferred to the staging area/相应的剩余物料已经恰当密封，转移到了暂存区域。 - The waste packaging material has been sealed and transferred to the corresponding waste staging area. /产生的包材废物已经密封并转移到了相应的废物暂存区域。 - The operation area/equipment surface has been cleaned/操作区域以及设备表面已经清洁。 Checked by & Date/核实人及日期： |  |  |  |  |  |  |  |  |

- Dispense OxymaPure/分装2-肟氰乙酸乙酯

Before material dispensing, confirm the material is within the retest date/ the expiry date, Confirm whether the weight is adequate for use in the current dispensing. /物料分装前，执行物料在复检日期/有效期之内，确认重量是否满足本次分装使用。 Before solid material dispensing, confirm/固体物料分装前，确认： - No non-related material / equipment / instrument / tools / pipeline / document / record / label in the intended area. /目标区域无不相关物料/设备/仪器/工具/管路/文件/记录/标签； - The outer packages are removed. If the outer package cannot be removed, but unpack material without damaging the package bag, and before dispensing, check no package fragment and other extraneous matter in material. /外包装移除；如果外包装不能移除，但在不破坏包装的情况下打开包装袋，分装前以及过程中同时检查执行物料中无包装碎片以及异物。 Checked by & Date/核实人及日期：

Dispense OxymaPure. /分装 kg 2-肟氰乙酸乙酯（Acceptable weighing range/可接受称量范围： kg ~ kg）。 For the Standard Batch Quantity, see Page 32~62 of 624 the information in "Raw Material Summary Table" corresponding to each unit operation/对于批标准投料量，参照Page 32~62 of 624“原料汇总表”中对应的每个单元操作。 Control No./内部控制号：YL005180 Scale ID. No./秤编号： Lot number of LDPE powder feeding bag/低密度聚乙烯粉末投料袋批号： Glovebox number /手套箱编号：

| 1.20. Dispense OxymaPure/分装2-肟氰乙酸乙酯 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ins. No. for material use/分装物料计划用于的操作 | Lot number/批号 | Charging Qty/投入的质量 |  |  |  | No. /编号 | Performed by & Date/ 执行人及日期 | Checked by & Date/ 核实人及日期 |
|  |  | Gross weight/毛重 | Tare weight /皮重 |  | Net weight/ 净重 |  |  |  |
| 3.29.3 |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  | Total weight/合计重量 |  |  |  |  | - |  |  |
|  | Total No. of bags/合计袋数 |  |  |  |  | - |  |  |
| Begin/开始 |  | End/结束 |  |  |  |  |  |  |
| After solid material dispensing, confirm/固体物料分装之后，确认： - The corresponding leftover material has been properly sealed and transferred to the staging area. /相应的剩余物料已经恰当密封，转移到了暂存区域。 - The waste packaging material has been sealed and transferred to the corresponding waste staging area. /产生的包材废物已经密封并转移到了相应的废物暂存区域。 - The operation area/equipment surface has been cleaned. /操作区域以及设备表面已经清洁。 Checked by & Date/核实人及日期： |  |  |  |  |  |  |  |  |

| - Dispense Fmoc-L-α-Me-Phe-OH/分装(S)-2-((((9H-芴-9-基)甲氧基)羰基)氨基)-2-甲基-3 -苯基丙酸 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Before material dispensing, confirm the material is within the retest date/ the expiry date, Confirm whether the weight is adequate for use in the current dispensing. /物料分装前，执行物料在复检日期/有效期之内，确认重量是否满足本次分装使用。 Before solid material dispensing, confirm/固体物料分装前，执行： - No non-related material / equipment / instrument / tools / pipeline / document / record / label in the intended area. /目标区域无不相关物料/设备/仪器/工具/管路/文件/记录/标签； - The outer packages are removed. If the outer package cannot be removed, but unpack material without damaging the package bag, and before dispensing, check no package fragment and other extraneous matter in material. /外包装移除；如果外包装不能移除，但在不破坏包装的情况下打开包装袋，分装前以及过程中同时检查执行物料中无包装碎片以及异物。 Performed by & Date/执行人及日期： |  |  |  |  |  |
| Key raw materials, in-process products and products for PPQ and commercial production should be weighed after the weighing implements are checked using the standard weights. For the balance/scale with the maximum capacity of greater than 40 kg, check with the standard weight of 20 kg; for the balance/scale with maximum capacity is not more than 40 kg, check with the standard weight of 1/2 maximum capacity. /用于工艺性能执行和商业化生产的主原料、过程产物以及产品的称量前，应使用标准砝码对称量器具进行检查，量程大于40 kg的电子秤，采用20 kg的标准砝码进行检查；量程不大于40 kg的电子秤，采用最大量程1/2的标准砝码进行检查。 Checking record for standard weights/标准砝码检查记录： |  |  |  |  |  |
| Weight No. /砝码编号 | Weight/ 砝码重量（kg） | Reading/ 示重 | Less than half of verification scale interval/ 小于检定分度值的一半 | Performed by & Date/ 执行人及日期 | Checked by & Date/ 核实人及日期 |
|  |  |  | Y N |  |  |
| Dispense Fmoc-L-α-Me-Phe-OH. /分装 kg(S)-2-((((9H-芴-9-基)甲氧基)羰基)氨基)-2-甲基-3 -苯基丙酸（Acceptable weighing range/可接受称量范围： kg ~ kg） For the Standard Batch Quantity, see Page 32~62 of 624 the information in "Raw Material Summary Table" corresponding to each unit operation/对于批标准投料量，参照Page 32~62 of 624“原料汇总表”中对应的每个单元操作。 Control No/内部控制号： YL008272 Scale ID. No./秤编号： Lot number of LDPE powder feeding bag/低密度聚乙烯粉末投料袋批号： Glovebox number /手套箱编号： |  |  |  |  |  |

| 1.21. Dispense Fmoc-L-α-Me-Phe-OH/分装(S)-2-((((9H-芴-9-基)甲氧基)羰基)氨基)-2-甲基-3 -苯基丙酸 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ins. No. for material use/分装物料计划用于的操作 | Lot number/批号 | Charging Qty/投入的质量 |  |  |  | No. /编号 | Performed by & Date/ 执行人及日期 | Checked by & Date/ 核实人及日期 |
|  |  | Gross weight/毛重 | Tare weight /皮重 |  | Net weight/ 净重 |  |  |  |
| 3.32.2 |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  | Total weight/合计重量 |  |  |  |  | - |  |  |
|  | Total No. of bags/合计袋数 |  |  |  |  | - |  |  |
| Begin/开始 |  | End/结束 |  |  |  |  |  |  |
| After solid material dispensing, confirm/固体物料分装之后，确认： - The corresponding leftover material has been properly sealed and transferred to the staging area. /相应的剩余物料已经恰当密封，转移到了暂存区域。 - The waste packaging material has been sealed and transferred to the corresponding waste staging area. /产生的包材废物已经密封并转移到了相应的废物暂存区域。 - The operation area/equipment surface has been cleaned. /操作区域以及设备表面已经清洁。 Checked by & Date/核实人及日期： |  |  |  |  |  |  |  |  |

- Dispense OxymaPure/分装2-肟氰乙酸乙酯

Before material dispensing, confirm the material is within the retest date/ the expiry date, Confirm whether the weight is adequate for use in the current dispensing. /物料分装前，执行物料在复检日期/有效期之内，确认重量是否满足本次分装使用。 Before solid material dispensing, confirm/固体物料分装前，确认： - No non-related material / equipment / instrument / tools / pipeline / document / record / label in the intended area. /目标区域无不相关物料/设备/仪器/工具/管路/文件/记录/标签； - The outer packages are removed. If the outer package cannot be removed, but unpack material without damaging the package bag, and before dispensing, check no package fragment and other extraneous matter in material. /外包装移除；如果外包装不能移除，但在不破坏包装的情况下打开包装袋，分装前以及过程中同时检查执行物料中无包装碎片以及异物。 Checked by & Date/核实人及日期：

Dispense OxymaPure. /分装 kg 2-肟氰乙酸乙酯（Acceptable weighing range/可接受称量范围： kg ~ kg）。 For the Standard Batch Quantity, see Page 32~62 of 624 the information in "Raw Material Summary Table" corresponding to each unit operation/对于批标准投料量，参照Page 32~62 of 624“原料汇总表”中对应的每个单元操作。 Control No./内部控制号：YL005180 Scale ID. No./秤编号： Lot number of LDPE powder feeding bag/低密度聚乙烯粉末投料袋批号： Glovebox number /手套箱编号：

| 1.22. Dispense OxymaPure/分装2-肟氰乙酸乙酯 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ins. No. for material use/分装物料计划用于的操作 | Lot number/批号 | Charging Qty/投入的质量 |  |  |  | No. /编号 | Performed by & Date/ 执行人及日期 | Checked by & Date/ 核实人及日期 |
|  |  | Gross weight/毛重 | Tare weight /皮重 |  | Net weight/ 净重 |  |  |  |
| 3.32.5 |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  | Total weight/合计重量 |  |  |  |  | - |  |  |
|  | Total No. of bags/合计袋数 |  |  |  |  | - |  |  |
| Begin/开始 |  | End/结束 |  |  |  |  |  |  |
| After solid material dispensing, confirm/固体物料分装之后，确认： - The corresponding leftover material has been properly sealed and transferred to the staging area. /相应的剩余物料已经恰当密封，转移到了暂存区域。 - The waste packaging material has been sealed and transferred to the corresponding waste staging area. /产生的包材废物已经密封并转移到了相应的废物暂存区域。 - The operation area/equipment surface has been cleaned. /操作区域以及设备表面已经清洁。 Checked by & Date/核实人及日期： |  |  |  |  |  |  |  |  |

| - Dispense Fmoc-L-Glu(OtBu)-OH·H2O/分装芴甲氧羰基-L-谷氨酸-γ-叔丁酯一水合物 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Before material dispensing, confirm the material is within the retest date/ the expiry date, Confirm whether the weight is adequate for use in the current dispensing. /物料分装前，执行物料在复检日期/有效期之内，确认重量是否满足本次分装使用。 Before solid material dispensing, confirm/固体物料分装前，确认： - No non-related material / equipment / instrument / tools / pipeline / document / record / label in the intended area. /目标区域无不相关物料/设备/仪器/工具/管路/文件/记录/标签； - The outer packages are removed. If the outer package cannot be removed, but unpack material without damaging the package bag, and before dispensing, check no package fragment and other extraneous matter in material. /外包装移除；如果外包装不能移除，但在不破坏包装的情况下打开包装袋，分装前以及过程中同时检查执行物料中无包装碎片以及异物。 Checked by & Date/核实人及日期： |  |  |  |  |  |
| Key raw materials, in-process products and products for PPQ and commercial production should be weighed after the weighing implements are checked using the standard weights. For the balance/scale with the maximum capacity of greater than 40 kg, check with the standard weight of 20 kg; for the balance/scale with maximum capacity is not more than 40 kg, check with the standard weight of 1/2 maximum capacity. /用于工艺性能执行和商业化生产的主原料、过程产物以及产品的称量前，应使用标准砝码对称量器具进行检查，量程大于40 kg的电子秤，采用20 kg的标准砝码进行检查；量程不大于40 kg的电子秤，采用最大量程1/2的标准砝码进行检查。 Checking record for standard weights/标准砝码检查记录： |  |  |  |  |  |
| Weight No. /砝码编号 | Weight/ 砝码重量（kg） | Reading/ 示重 | Less than half of verification scale interval/ 小于检定分度值的一半 | Performed by & Date/ 执行人及日期 | Checked by & Date/ 核实人及日期 |
|  |  |  | Y N |  |  |
| Dispense Fmoc-L-Glu(OtBu)-OH·H2O. /分装 kg芴甲氧羰基-L-谷氨酸-γ-叔丁酯一水合物（Acceptable weighing range/可接受称量范围： kg ~ kg）。 For the Standard Batch Quantity, see Page 32~62 of 624 the information in "Raw Material Summary Table" corresponding to each unit operation/对于批标准投料量，参照Page 32~62 of 624“原料汇总表”中对应的每个单元操作。 Control No./内部控制号：YL005741 Scale ID. No./秤编号： Lot number of LDPE powder feeding bag/低密度聚乙烯粉末投料袋批号： Glovebox number /手套箱编号： |  |  |  |  |  |

| 1.23. Dispense Fmoc-L-Glu(OtBu)-OH·H2O/分装芴甲氧羰基-L-谷氨酸-γ-叔丁酯一水合物 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ins. No. for material use/分装物料计划用于的操作 | Lot number/批号 | Charging Qty/投入的质量 |  |  |  | No. /编号 | Performed by & Date/ 执行人及日期 | Checked by & Date/ 核实人及日期 |
|  |  | Gross weight/毛重 | Tare weight /皮重 |  | Net weight/ 净重 |  |  |  |
| 3.35.2 |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  | Total weight/合计重量 |  |  |  |  | - |  |  |
|  | Total No. of bags/合计袋数 |  |  |  |  | - |  |  |
| Begin/开始 |  | End/结束 |  |  |  |  |  |  |
| After solid material dispensing, confirm/固体物料分装之后，确认： - The corresponding leftover material has been properly sealed and transferred to the staging area./相应的剩余物料已经恰当密封，转移到了暂存区域。 - The waste packaging material has been sealed and transferred to the corresponding waste staging area. /产生的包材废物已经密封并转移到了相应的废物暂存区域。 - The operation area/equipment surface has been cleaned. /操作区域以及设备表面已经清洁。 Checked by & Date/核实人及日期： |  |  |  |  |  |  |  |  |

- Dispense OxymaPure/分装2-肟氰乙酸乙酯

Before material dispensing, confirm the material is within the retest date/ the expiry date, Confirm whether the weight is adequate for use in the current dispensing. /物料分装前，执行物料在复检日期/有效期之内，确认重量是否满足本次分装使用。 Before solid material dispensing, confirm/固体物料分装前，执行： - No non-related material / equipment / instrument / tools / pipeline / document / record / label in the intended area. /目标区域无不相关物料/设备/仪器/工具/管路/文件/记录/标签； - The outer packages are removed. If the outer package cannot be removed, but unpack material without damaging the package bag, and before dispensing, check no package fragment and other extraneous matter in material. /外包装移除；如果外包装不能移除，但在不破坏包装的情况下打开包装袋，分装前以及过程中同时检查执行物料中无包装碎片以及异物。 Performed by & Date/执行人及日期：

Dispense OxymaPure. /分装 kg 2-肟氰乙酸乙酯（Acceptable weighing range/可接受称量范围： kg ~ kg）。 For the Standard Batch Quantity, see Page 32~62 of 624 the information in "Raw Material Summary Table" corresponding to each unit operation/对于批标准投料量，参照Page 32~62 of 624“原料汇总表”中对应的每个单元操作。 Control No./内部控制号：YL005180 Scale ID. No./秤编号： Lot number of LDPE powder feeding bag/低密度聚乙烯粉末投料袋批号： Glovebox number /手套箱编号：

| 1.24. Dispense OxymaPure/分装2-肟氰乙酸乙酯 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ins. No. for material use/分装物料计划用于的操作 | Lot number/批号 | Charging Qty/投入的质量 |  |  |  | No. /编号 | Performed by & Date/ 执行人及日期 | Checked by & Date/ 核实人及日期 |
|  |  | Gross weight/毛重 | Tare weight /皮重 |  | Net weight/ 净重 |  |  |  |
| 3.35.5 |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  | Total weight/合计重量 |  |  |  |  | - |  |  |
|  | Total No. of bags/合计袋数 |  |  |  |  | - |  |  |
| Begin/开始 |  | End/结束 |  |  |  |  |  |  |
| After solid material dispensing, confirm/固体物料分装之后，确认： - The corresponding leftover material has been properly sealed and transferred to the staging area. /相应的剩余物料已经恰当密封，转移到了暂存区域。 - The waste packaging material has been sealed and transferred to the corresponding waste staging area. /产生的包材废物已经密封并转移到了相应的废物暂存区域。 - The operation area/equipment surface has been cleaned. /操作区域以及设备表面已经清洁。 Checked by & Date/核实人及日期： |  |  |  |  |  |  |  |  |

| - Dispense Fmoc-L-Ala-OH·H2O/分装芴甲氧羰基-L-丙氨酸一水合物 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Before material dispensing, confirm the material is within the retest date/ the expiry date, Confirm whether the weight is adequate for use in the current dispensing. /物料分装前，执行物料在复检日期/有效期之内，确认重量是否满足本次分装使用。 Before solid material dispensing, confirm/固体物料分装前，确认： - No non-related material / equipment / instrument / tools / pipeline / document / record / label in the intended area. /目标区域无不相关物料/设备/仪器/工具/管路/文件/记录/标签； - The outer packages are removed. If the outer package cannot be removed, but unpack material without damaging the package bag, and before dispensing, check no package fragment and other extraneous matter in material. /外包装移除；如果外包装不能移除，但在不破坏包装的情况下打开包装袋，分装前以及过程中同时检查执行物料中无包装碎片以及异物。 Checked by & Date/核实人及日期： |  |  |  |  |  |
| Key raw materials, in-process products and products for PPQ and commercial production should be weighed after the weighing implements are checked using the standard weights. For the balance/scale with the maximum capacity of greater than 40 kg, check with the standard weight of 20 kg; for the balance/scale with maximum capacity is not more than 40 kg, check with the standard weight of 1/2 maximum capacity. /用于工艺性能执行和商业化生产的主原料、过程产物以及产品的称量前，应使用标准砝码对称量器具进行检查，量程大于40 kg的电子秤，采用20 kg的标准砝码进行检查；量程不大于40 kg的电子秤，采用最大量程1/2的标准砝码进行检查。 Checking record for standard weights/标准砝码检查记录： |  |  |  |  |  |
| Weight No. /砝码编号 | Weight/ 砝码重量（kg） | Reading/ 示重 | Less than half of verification scale interval/ 小于检定分度值的一半 | Performed by & Date/ 执行人及日期 | Checked by & Date/ 核实人及日期 |
|  |  |  | Y N |  |  |
| Dispense Fmoc-L-Ala-OH·H2O. /分装 kg芴甲氧羰基-L-丙氨酸一水合物（Acceptable weighing range/可接受称量范围： kg ~ kg）。 For the Standard Batch Quantity, see Page 32~62 of 624 the information in "Raw Material Summary Table" corresponding to each unit operation/对于批标准投料量，参照Page 32~62 of 624“原料汇总表”中对应的每个单元操作。 Control No./内部控制号：YL006160 Scale ID. No./秤编号： Lot number of LDPE powder feeding bag/低密度聚乙烯粉末投料袋批号： Glovebox number /手套箱编号： |  |  |  |  |  |

| 1.25. Dispense Fmoc-L-Ala-OH·H2O/分装芴甲氧羰基-L-丙氨酸一水合物 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ins. No. for material use/分装物料计划用于的操作 | Lot number/批号 | Charging Qty/投入的质量 |  |  |  | No. /编号 | Performed by & Date/ 执行人及日期 | Checked by & Date/ 核实人及日期 |
|  |  | Gross weight/毛重 | Tare weight /皮重 |  | Net weight/ 净重 |  |  |  |
| 3.38.2 |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  | Total weight/合计重量 |  |  |  |  | - |  |  |
|  | Total No. of bags/合计袋数 |  |  |  |  | - |  |  |
| Begin/开始 |  | End/结束 |  |  |  |  |  |  |
| After solid material dispensing, confirm/固体物料分装之后，确认： - The corresponding leftover material has been properly sealed and transferred to the staging area. /相应的剩余物料已经恰当密封，转移到了暂存区域。 - The waste packaging material has been sealed and transferred to the corresponding waste staging area. /产生的包材废物已经密封并转移到了相应的废物暂存区域。 - The operation area/equipment surface has been cleaned. /操作区域以及设备表面已经清洁。 Checked by & Date/核实人及日期： |  |  |  |  |  |  |  |  |

- Dispense OxymaPure/分装2-肟氰乙酸乙酯

Before material dispensing, confirm the material is within the retest date/ the expiry date, Confirm whether the weight is adequate for use in the current dispensing. /物料分装前，执行物料在复检日期/有效期之内，确认重量是否满足本次分装使用。 Before solid material dispensing, confirm/固体物料分装前，确认： - No non-related material / equipment / instrument / tools / pipeline / document / record / label in the intended area. /目标区域无不相关物料/设备/仪器/工具/管路/文件/记录/标签； - The outer packages are removed. If the outer package cannot be removed, but unpack material without damaging the package bag, and before dispensing, check no package fragment and other extraneous matter in material. /外包装移除；如果外包装不能移除，但在不破坏包装的情况下打开包装袋，分装前以及过程中同时检查执行物料中无包装碎片以及异物。 Checked by & Date/核实人及日期：

Dispense OxymaPure. /分装 kg 2-肟氰乙酸乙酯（Acceptable weighing range/可接受称量范围： kg ~ kg）。 For the Standard Batch Quantity, see Page 32~62 of 624 the information in "Raw Material Summary Table" corresponding to each unit operation/对于批标准投料量，参照Page 32~62 of 624“原料汇总表”中对应的每个单元操作。 Control No./内部控制号：YL005180 Scale ID. No./秤编号： Lot number of LDPE powder feeding bag/低密度聚乙烯粉末投料袋批号： Glovebox number /手套箱编号：

| 1.26. Dispense OxymaPure/分装2-肟氰乙酸乙酯 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ins. No. for material use/分装物料计划用于的操作 | Lot number/批号 | Charging Qty/投入的质量 |  |  |  | No. /编号 | Performed by & Date/ 执行人及日期 | Checked by & Date/ 核实人及日期 |
|  |  | Gross weight/毛重 | Tare weight /皮重 |  | Net weight/ 净重 |  |  |  |
| 3.38.5 |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  | Total weight/合计重量 |  |  |  |  | - |  |  |
|  | Total No. of bags/合计袋数 |  |  |  |  | - |  |  |
| Begin/开始 |  | End/结束 |  |  |  |  |  |  |
| After solid material dispensing, confirm/固体物料分装之后，确认： - The corresponding leftover material has been properly sealed and transferred to the staging area. /相应的剩余物料已经恰当密封，转移到了暂存区域。 - The waste packaging material has been sealed and transferred to the corresponding waste staging area. /产生的包材废物已经密封并转移到了相应的废物暂存区域。 - The operation area/equipment surface has been cleaned. /操作区域以及设备表面已经清洁。 Checked by & Date/核实人及日期： |  |  |  |  |  |  |  |  |

| - Dispense Fmoc-L-Leu-OH/分装芴甲氧羰基-L-亮氨酸 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Before material dispensing, confirm the material is within the retest date/ the expiry date, Confirm whether the weight is adequate for use in the current dispensing. /物料分装前，执行物料在复检日期/有效期之内，确认重量是否满足本次分装使用。 Before solid material dispensing, confirm/固体物料分装前，确认： - No non-related material / equipment / instrument / tools / pipeline / document / record / label in the intended area. /目标区域无不相关物料/设备/仪器/工具/管路/文件/记录/标签； - The outer packages are removed. If the outer package cannot be removed, but unpack material without damaging the package bag, and before dispensing, check no package fragment and other extraneous matter in material. /外包装移除；如果外包装不能移除，但在不破坏包装的情况下打开包装袋，分装前以及过程中同时检查执行物料中无包装碎片以及异物 。 Checked by & Date/核实人及日期： |  |  |  |  |  |
| Key raw materials, in-process products and products for PPQ and commercial production should be weighed after the weighing implements are checked using the standard weights. For the balance/scale with the maximum capacity of greater than 40 kg, check with the standard weight of 20 kg; for the balance/scale with maximum capacity is not more than 40 kg, check with the standard weight of 1/2 maximum capacity. /用于工艺性能执行和商业化生产的主原料、过程产物以及产品的称量前，应使用标准砝码对称量器具进行检查，量程大于40 kg的电子秤，采用20 kg的标准砝码进行检查；量程不大于40 kg的电子秤，采用最大量程1/2的标准砝码进行检查。 Checking record for standard weights/标准砝码检查记录： |  |  |  |  |  |
| Weight No. /砝码编号 | Weight/ 砝码重量（kg） | Reading/ 示重 | Less than half of verification scale interval/ 小于检定分度值的一半 | Performed by & Date/ 执行人及日期 | Checked by & Date/ 核实人及日期 |
|  |  |  | Y N |  |  |
| Dispense Fmoc-L-Leu-OH. /分装 kg芴甲氧羰基-L-亮氨酸（Acceptable weighing range/可接受称量范围： kg ~ kg）。 For the Standard Batch Quantity, see Page 32~62 of 624 the information in "Raw Material Summary Table" corresponding to each unit operation/对于批标准投料量，参照Page 32~62 of 624“原料汇总表”中对应的每个单元操作。 Control No./内部控制号：YL003425 Scale ID. No./秤编号： Lot number of LDPE powder feeding bag/低密度聚乙烯粉末投料袋批号： Glovebox number /手套箱编号： |  |  |  |  |  |

| 1.27. Dispense Fmoc-L-Leu-OH/分装芴甲氧羰基-L-亮氨酸 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ins. No. for material use/分装物料计划用于的操作 | Lot number/批号 | Charging Qty/投入的质量 |  |  |  | No. /编号 | Performed by & Date/ 执行人及日期 | Checked by & Date/ 核实人及日期 |
|  |  | Gross weight/毛重 | Tare weight /皮重 |  | Net weight/ 净重 |  |  |  |
| 3.41.2 |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  | Total weight/合计重量 |  |  |  |  | - |  |  |
|  | Total No. of bags/合计袋数 |  |  |  |  | - |  |  |
| Begin/开始 |  | End/结束 |  |  |  |  |  |  |
| After solid material dispensing, confirm/固体物料分装之后，确认： - The corresponding leftover material has been properly sealed and transferred to the staging area. /相应的剩余物料已经恰当密封，转移到了暂存区域。 - The waste packaging material has been sealed and transferred to the corresponding waste staging area. /产生的包材废物已经密封并转移到了相应的废物暂存区域。 - The operation area/equipment surface has been cleaned. /操作区域以及设备表面已经清洁。 Checked by & Date/核实人及日期： |  |  |  |  |  |  |  |  |

- Dispense OxymaPure/分装2-肟氰乙酸乙酯

Before material dispensing, confirm the material is within the retest date/ the expiry date, Confirm whether the weight is adequate for use in the current dispensing. /物料分装前，执行物料在复检日期/有效期之内，确认重量是否满足本次分装使用。 Before solid material dispensing, confirm/固体物料分装前，确认： - No non-related material / equipment / instrument / tools / pipeline / document / record / label in the intended area. /目标区域无不相关物料/设备/仪器/工具/管路/文件/记录/标签； - The outer packages are removed. If the outer package cannot be removed, but unpack material without damaging the package bag, and before dispensing, check no package fragment and other extraneous matter in material. /外包装移除；如果外包装不能移除，但在不破坏包装的情况下打开包装袋，分装前以及过程中同时检查执行物料中无包装碎片以及异物。 Checked by & Date/核实人及日期：

Dispense OxymaPure. /分装 kg 2-肟氰乙酸乙酯（Acceptable weighing range/可接受称量范围： kg ~ kg）。 For the Standard Batch Quantity, see Page 32~62 of 624 the information in "Raw Material Summary Table" corresponding to each unit operation/对于批标准投料量，参照Page 32~62 of 624“原料汇总表”中对应的每个单元操作。 Control No./内部控制号：YL005180 Scale ID. No./秤编号： Lot number of LDPE powder feeding bag/低密度聚乙烯粉末投料袋批号： Glovebox number /手套箱编号：

| 1.28. Dispense OxymaPure/分装2-肟氰乙酸乙酯 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ins. No. for material use/分装物料计划用于的操作 | Lot number/批号 | Charging Qty/投入的质量 |  |  |  | No. /编号 | Performed by & Date/ 执行人及日期 | Checked by & Date/ 核实人及日期 |
|  |  | Gross weight/毛重 | Tare weight /皮重 |  | Net weight/ 净重 |  |  |  |
| 3.41.5 |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  | Total weight/合计重量 |  |  |  |  | - |  |  |
|  | Total No. of bags/合计袋数 |  |  |  |  | - |  |  |
| Begin/开始 |  | End/结束 |  |  |  |  |  |  |
| After solid material dispensing, confirm/固体物料分装之后，确认： - The corresponding leftover material has been properly sealed and transferred to the staging area. /相应的剩余物料已经恰当密封，转移到了暂存区域。 - The waste packaging material has been sealed and transferred to the corresponding waste staging area. /产生的包材废物已经密封并转移到了相应的废物暂存区域。 - The operation area/equipment surface has been cleaned. /操作区域以及设备表面已经清洁。 Checked by & Date/核实人及日期： |  |  |  |  |  |  |  |  |

| - Dispense Fmoc-L-Orn(Boc)-OH/分装N-Fmoc-N'-Boc-L-鸟氨酸 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Before material dispensing, confirm the material is within the retest date/ the expiry date, Confirm whether the weight is adequate for use in the current dispensing. /物料分装前，执行物料在复检日期/有效期之内，确认重量是否满足本次分装使用。 Before solid material dispensing, confirm/固体物料分装前，确认： - No non-related material / equipment / instrument / tools / pipeline / document / record / label in the intended area. /目标区域无不相关物料/设备/仪器/工具/管路/文件/记录/标签； - The outer packages are removed. If the outer package cannot be removed, but unpack material without damaging the package bag, and before dispensing, check no package fragment and other extraneous matter in material. /外包装移除；如果外包装不能移除，但在不破坏包装的情况下打开包装袋，分装前以及过程中同时检查执行物料中无包装碎片以及异物。 Checked by & Date/核实人及日期： |  |  |  |  |  |
| Key raw materials, in-process products and products for PPQ and commercial production should be weighed after the weighing implements are checked using the standard weights. For the balance/scale with the maximum capacity of greater than 40 kg, check with the standard weight of 20 kg; for the balance/scale with maximum capacity is not more than 40 kg, check with the standard weight of 1/2 maximum capacity. /用于工艺性能执行和商业化生产的主原料、过程产物以及产品的称量前，应使用标准砝码对称量器具进行检查，量程大于40 kg的电子秤，采用20 kg的标准砝码进行检查；量程不大于40 kg的电子秤，采用最大量程1/2的标准砝码进行检查。 Checking record for standard weights/标准砝码检查记录： |  |  |  |  |  |
| Weight No. /砝码编号 | Weight/ 砝码重量（kg） | Reading/ 示重 | Less than half of verification scale interval/ 小于检定分度值的一半 | Performed by & Date/ 执行人及日期 | Checked by & Date/ 核实人及日期 |
|  |  |  | Y N |  |  |
| Dispense Fmoc-L-Orn(Boc)-OH. /分装 kg N-Fmoc-N'-Boc-L-鸟氨酸（Acceptable weighing range/可接受称量范围： kg ~ kg）。 For the Standard Batch Quantity, see Page 32~62 of 624 the information in "Raw Material Summary Table" corresponding to each unit operation/对于批标准投料量，参照Page 32~62 of 624“原料汇总表”中对应的每个单元操作。 Control No./内部控制号：YL005318 Scale ID. No./秤编号： Lot number of LDPE powder feeding bag/低密度聚乙烯粉末投料袋批号： Glovebox number /手套箱编号： |  |  |  |  |  |

| 1.29. Dispense Fmoc-L-Orn(Boc)-OH/分装N-Fmoc-N'-Boc-L-鸟氨酸 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ins. No. for material use/分装物料计划用于的操作 | Lot number/批号 | Charging Qty/投入的质量 |  |  |  | No. /编号 | Performed by & Date/ 执行人及日期 | Checked by & Date/ 核实人及日期 |
|  |  | Gross weight/毛重 | Tare weight /皮重 |  | Net weight/ 净重 |  |  |  |
| 3.46.2 |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  | Total weight/合计重量 |  |  |  |  | - |  |  |
|  | Total No. of bags/合计袋数 |  |  |  |  | - |  |  |
| Begin/开始 |  | End/结束 |  |  |  |  |  |  |
| After solid material dispensing, confirm/固体物料分装之后，确认： - The corresponding leftover material has been properly sealed and transferred to the staging area. /相应的剩余物料已经恰当密封，转移到了暂存区域。 - The waste packaging material has been sealed and transferred to the corresponding waste staging area. /产生的包材废物已经密封并转移到了相应的废物暂存区域。 - The operation area/equipment surface has been cleaned. /操作区域以及设备表面已经清洁。 Checked by & Date/核实人及日期： |  |  |  |  |  |  |  |  |

- Dispense OxymaPure/分装2-肟氰乙酸乙酯

Before material dispensing, confirm the material is within the retest date/ the expiry date, Confirm whether the weight is adequate for use in the current dispensing. /物料分装前，执行物料在复检日期/有效期之内，确认重量是否满足本次分装使用。 Before solid material dispensing, confirm/固体物料分装前，确认： - No non-related material / equipment / instrument / tools / pipeline / document / record / label in the intended area. /目标区域无不相关物料/设备/仪器/工具/管路/文件/记录/标签； - The outer packages are removed. If the outer package cannot be removed, but unpack material without damaging the package bag, and before dispensing, check no package fragment and other extraneous matter in material. /外包装移除；如果外包装不能移除，但在不破坏包装的情况下打开包装袋，分装前以及过程中同时检查执行物料中无包装碎片以及异物。 Checked by & Date/核实人及日期：

Dispense OxymaPure. /分装 kg 2-肟氰乙酸乙酯（Acceptable weighing range/可接受称量范围： kg ~ kg）。 For the Standard Batch Quantity, see Page 32~62 of 624 the information in "Raw Material Summary Table" corresponding to each unit operation/对于批标准投料量，参照Page 32~62 of 624“原料汇总表”中对应的每个单元操作。 Control No./内部控制号：YL005180 Scale ID. No./秤编号： Lot number of LDPE powder feeding bag/低密度聚乙烯粉末投料袋批号： Glovebox number /手套箱编号：

| 1.30. Dispense OxymaPure/分装2-肟氰乙酸乙酯 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ins. No. for material use/分装物料计划用于的操作 | Lot number/批号 | Charging Qty/投入的质量 |  |  |  | No. /编号 | Performed by & Date/ 执行人及日期 | Checked by & Date/ 核实人及日期 |
|  |  | Gross weight/毛重 | Tare weight /皮重 |  | Net weight/ 净重 |  |  |  |
| 3.46.5 |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  |  |  |  |  |  | / |  |  |
|  | Total weight/合计重量 |  |  |  |  | - |  |  |
|  | Total No. of bags/合计袋数 |  |  |  |  | - |  |  |
| Begin/开始 |  | End/结束 |  |  |  |  |  |  |
| After solid material dispensing, confirm/固体物料分装之后，确认： - The corresponding leftover material has been properly sealed and transferred to the staging area. /相应的剩余物料已经恰当密封，转移到了暂存区域。 - The waste packaging material has been sealed and transferred to the corresponding waste staging area. /产生的包材废物已经密封并转移到了相应的废物暂存区域。 - The operation area/equipment surface has been cleaned. /操作区域以及设备表面已经清洁。 Checked by & Date/核实人及日期： |  |  |  |  |  |  |  |  |

Ins.2/操作2：Equipment confirmation & RecipeConfirmation /设备确认和配方确认

- Equipment confirmation/设备确认

Before using, confirm peptide solid phase synthesizer (referred as synthesis reactor) oxygen content≤1.0%. 使用前确认多肽固相合成仪合成釜(以下简称合成釜)氧气含量≤1.0%。 3000 L Peptide solid phase synthesizer (referred as synthesis reactor)/多肽固相合成仪合成釜(以下简称合成釜)：TJ4S-1213-Sy15 Oxygen detector ID. No. /氧气检测器编号： Oxygen content/氧气含量= 设备检查表文件编号/Equipment Checklist Document Number： Checked by & Date核实人及日期：

Before using, confirm the synthesis reactor TJ4S-1213-Sy15’s calibration information is within valid date. 使用前确认合成釜TJ4S-1213-Sy15的仪表在校准有效期内。 Checked by & Date/核实人及日期： Before using, confirm that cleaning of the synthesis reactor TJ4S-1213-Sy15 has been completed/使用前，确认合成釜TJ4S-1213-Sy15清洁已完成 Checked by & Date/核实人及日期：

Before using, confirm solid-phase peptide synthesizer activation tank (referred as TJ4S-1213-Sy15-R01) oxygen content ≤1.0%. 使用前确认多肽固相合成仪激活罐(以下简称激活罐)氧气含量≤1.0%。 1500 L Solid-peptide peptide synthesizer activation tank (referred as activation tank) /多肽固相合成仪激活罐(以下简称激活罐)： TJ4S-1213-Sy15-R01 Oxygen detector ID. No. /氧气检测器编号： Oxygen content/氧气含量= 设备检查表文件编号/Equipment Checklist Document Number： Checked by & Date/核实人及日期：

2.1. Equipment confirmation/设备确认

Before using, confirm the activation tank TJ4S-1213-Sy15-R01’s calibration information is within valid date. 使用前确认激活罐TJ4S-1213-Sy15-R01的仪表在校准有效期内。 Checkmed by &Date/核实人及日期： Before using, confirm that cleaning of the activation tank TJ4S-1213-Sy15- R01 has been completed/ 使用前，确认激活罐TJ4S-1213-Sy15- R01清洁已完成 Checked by & Date/核实人及日期：

Ins.2/操作2：Equipment confirmation & RecipeConfirmation /设备执行和配方执行

2.1.Equipment confirmation/设备确认

Before using, confirm hastelloy agitator tank oxygen content≤1.0%. 使用前确认哈氏合金搅拌罐氧气含量≤1.0%。 500 L hastelly agitator tank/ 哈氏合金搅拌罐：TJ4S-1200-AT02 Oxygen detector ID. No. /氧气检测器编号： Oxygen content/氧气含量= 设备检查表文件编号/Equipment Checklist Document Number： Checked by & Date核实人及日期：

Before using, confirm the agitator tank TJ4S-1200-AT02’s calibration information is within valid date. 使用前执行搅拌罐TJ4S-1200-AT02上的仪表在校准有效期内。 Checked by & Date/核实人及日期：

Before using, confirm glass-lined reactor (referred as TJ4S-1213-R22) oxygen content ≤ 1.0%. 使用前执行搪玻璃反应釜氧气含量≤1.0%。 1000 L glass-lined reactor (referred as TJ4S-1213-R22) /搪玻璃反应釜： TJ4S-1213-R22 Oxygen detector ID. No. /氧气检测器编号： Oxygen content/氧气含量= 设备检查表文件编号/Equipment Checklist Document Number： Checked by & Date/核实人及日期：

Before using, confirm Glass-lined reactor TJ4S-1213-R22’s calibration information is within valid date. 使用前确认搪玻璃反应釜TJ4S-1213- R22上的仪表在校准有效期内。 Checked by &Date/核实人及日期：

- Recipe confirmation/配方确认

The recipe is edited and established on the synthesis equipment according to the requirements of Ins.3. 配方是根据指令3的工艺操作要求在合成设备上编辑和建立的。 Confirm synthesis recipe. /执行已经建立的合成配方： 1.Double editing / 双人编辑： Two engineers shall edit the electronic recipe separately for the same project. 由两名工程师对同一项目分别进行电子配方编辑 2.Consistency comparison / 一致性比对： Compare the electronic recipes edited by the two engineers, confirm each difference between them one by one, and correct any errors. To ensure the electronic recipe to be run is reviewed and accurate. 将双人编辑后的电子配方进行比对，对两个电子配方的不同点逐一进行执行，对编辑有误的电子配方进行纠正，确保在即将运行的电子配方是经过复核且准确的。 Performed by & Date/执行人及日期： Checked by & Date/核实人及日期： Recipe name /配方名称：14002001M01V02 Batch name /批次命名：CPo140020 01

Confirmed by & Date/确认人及日期：

Checked by & Date/核实人及日期：

- Tank confirmation/罐区确认

The 20% v/v piperidine/DMF solution is prepared and the bottom valve of agitator tank TJ4S-1213-SPS01-AT01、TJ4S-1213-SPS01-AT02 is opened. 20% v/v哌啶/DMF溶液完成配制，搅拌罐TJ4S-1213-SPS01-AT01、TJ4S-1213-SPS01-AT02底阀已开启； Confirmed by & Date/确认人及日期： Checked by & Date/核实人及日期： The preparation record for 20% v/v Piperidine Record details /配制20% v/v哌啶记录详见：

Confirm that the DMF supply system from tank farm is supplied. /确认罐区N,N-二甲基甲酰胺储罐已准备就绪； Confirmed by & Date/确认人及日期： Checked by & date/核实人及日期：

Confirm that the IPAc supply system from tank farm is supplied. /确认罐区乙酸异丙酯储罐供应系统已准备就绪； Confirmed by & Date/确认人及日期： Checked by & date/核实人及日期：

Confirm that the MTBE supply system from tank farm is supplied. /确认罐区甲基叔丁基醚储罐供应系统已准备就绪； Confirmed by & Date/确认人及日期： Checked by & date/核实人及日期：

| - Run Recipe /配方运行 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 2.4.1 Invoke synthesis recipe. /调用已经建立的合成配方： Recipe name /配方名称：14002001M01V02 Batch name /批次命名：CPo140020 01 Move to Modules /调用模块：3.0 Performed by & Date/执行人及日期： Checked by & Date/核实人及日期： |  |  |  |  |  |
| 2.4.2 Review of electronic recipe/配方复核 When reviewing an electronic recipe, the items to be checked include but are not limited to the recipe name, instructional operational logic, key process parameters, alarm parameters, and other parameters that may affect product quality. This is to ensure the electronic recipe meets the requirements for project production and GMP management./ 复核电子配方时，复核的内容包括但不限于配方名称、指令运行逻辑、关键工艺参数、报警参数及其他可能影响产品质量的参数，以确保该电子配方能够满足项目生产及 GMP 管理的要求。 Final review / 最终复核： Before the running, production personnel and process chemist shall conduct the final review of the electronic recipe. After the review is correct, they shall sign and date on the Operation Record (OR). 正式运行前，生产人员与工艺技术人员需共同对电子配方进行最终复核；复核无误后，在操作记录（OR）上签名并注明日期。 Performed by & Date/执行人及日期： Checked by & Date/核实人及日期： |  |  |  |  |  |
| 2.4.3 Confirm the synthetic recipe is selected correctly. And the peptide solid phase synthesizer is ready for use. The instrument is ready for all work. /执行合成配方选用正确，对固相合成仪各项工作准备就绪。 Before the electronic recipe is run, it is necessary to review the key information such as the name, version, and batch number of the electronic recipe to be run. 电子配方运行前，需要对即将运行的电子配方的名称、版本、运行批号等关键信息进行复核。 If the electronic recipe is locked before running, it possible that the electronic recipe cannot be run directly. After confirming that the information is correct, it can be unlocked and run. 若运行前电子配方处于锁定状态，可能无法直接运行该电子配方，可在执行信息无误后进行解锁后，正式运行该电子配方。 Click run button and start synthesis. /点击运行，开始合成。 Confirmed by & Date/确认人及日期： Checked by & Date/核实人及日期： |  |  |  |  |  |
| Ins.3/操作3：Synthesis/合成 |  |  |  |  |  |
| - Pre-treatment/预处理 |  |  |  |  |  |
| CAUTION/注意：DMF/N，N-二甲基甲酰胺：Irritation of eyes, skin and nose. May cause nausea. /刺激眼睛、皮肤和鼻子。可能引起恶心。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |
| 3.0 (1) DMF addition and water content testing /检测含水： Add 200.00 - 300.00 kg DMF via spray ball into activation tank (TJ4S-1213-Sy15-R01). Start the agitator of activation tank to 95±10 rpm (target: 95 rpm). Stir for 5±2 min (target 5 min). Then transfer the solution into synthesis reactor (TJ4S-1213-Sy15). Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h). Bubble with nitrogen for 5±2 min (target 5 min). Start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm). Stir for 5±2 min (target 5 min), turn off the agitation and nitrogen. Sample the mixture for Water content analysis. Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point. If Water content >500 ppm, repeat Module 3.0 until Water content ≤500 ppm. If Water content ≤500 ppm, move to next operation. 通过淋洗球向激活罐（TJ4S-1213-Sy15-R01）中加入200.00 - 300.00 kg N,N-二甲基甲酰胺。开启激活罐搅拌转速至95±10 rpm（目标: 95 rpm），搅拌5±2 min (目标：5 min)。转移至合成釜（TJ4S-1213-Sy15）中，开启合成釜下口鼓氮气，氮气流速开至5±2 m³/h (目标流速:5 m³/h)，鼓氮气5±2 min (目标：5 min)后。开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm)，搅拌5±2 min (目标：5 min)，停止搅拌，关闭氮气取样检测体系水分，通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。若水分＞500 ppm，则重复进行模块3.0直至水分≤500 ppm；若水分≤500 ppm，则进行下一操作。 |  |  |  |  | Times/次数 |
|  |  |  |  |  | 1 |
| Add 200.00 - 300.00 kg DMF by spray balls into activation tank TJ4S-1213-Sy15-R01/通过淋洗球向激活罐TJ4S-1213-Sy15-R01中加入200.00 – 300.00 kg N,N-二甲基甲酰胺。 |  |  |  |  |  |
| Rinse with DMF/N,N-二甲基甲酰胺淋洗 |  | Performed by & Date/执行人及日期： |  |  |  |
| Rinse with DMF/ N,N-二甲基甲酰胺淋洗 | Begin/开始： |  | End/结束： | Qty/重量： |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |

| 3.0. Pre-treatment/预处理 |  |  |  |
| --- | --- | --- | --- |
| Start the agitator of activation tank to 95±10 rpm (target: 95 rpm). Stir for 5 min±2 min (target: 5 min). /开启激活罐搅拌转速至95±10 rpm (目标转速: 95 rpm)。搅拌5 min±2 min（目标时间:5 min） |  |  |  |
| Pre-treatment/预处理 | Time point/时间节点 | Wash /洗涤 |  |
| Stir /搅拌 | Performed by & Date/执行人及日期： |  |  |
| Stir /搅拌 | Begin/开始： | End/结束： |  |
| Transcribed by & Date /转录人及日期： |  |  |  |
| Then transfer the solution into synthesis reactor TJ4S-1213-Sy15. Confirm complete transfer./转移至固相合成釜TJ4S-1213-Sy15中。 |  |  |  |
| Transfer/转移 | Begin/开始： |  | End/结束： |
|  | Performed by & Date/执行人及日期： |  |  |
|  | Checked by & Date/核实人及日期： |  |  |
| Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h). Bubble with nitrogen for 5±2 min (target 5 min). Start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm). Stir for 5 min±2 min (target: 5 min). Stop the stirring and turn off the nitrogen. /开启合成釜下口鼓氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min (目标：5 min)后。开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm)，搅拌5 min±5 min（目标时间:5 min）停止搅拌，关闭氮气。 |  |  |  |
| Bubble/鼓氮气 | Performed by & Date/执行人及日期： |  |  |
| Bubble/鼓氮气 | Begin/开始： | End/结束： |  |
| Transcribed by & Date /转录人及日期： |  |  |  |
| Stir /搅拌 | Begin/开始： | End/结束： |  |
|  | Performed by & Date/执行人及日期： |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |

| 3.0. Pre-treatment/预处理 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Standard sampling method/标准取样方法 |  |  |  |  |  |
| Approximately 10~20 ml of the mixture is withdrawn from the reactor via a dedicated in-line sampler and submit for analysis testing. During sampling, should wear nitrile gloves and 3M 6000 series Full-face piece respirator, 6006CN cartridges. 通过在线取样器从反应釜内抽取约取体系约10~20ml，直接送样检测。取样过程中，应该戴丁腈手套、3M 6000系列全面罩, 6006CN滤毒盒。 |  |  |  |  |  |
| Sampling number/ 取样编号 | Sampling time/ 取样时间 | Sampled by & Date/取样人及日期 | Water content/水分 |  | Result filled by &Date/ 结果填写人及日期 |
| CPo140020- -01- -3.0-01 |  |  |  |  |  |
| Drain until there is no drip, the operator visually confirms the draining end point. If KF≤500 ppm, move to next operation. If KF>500 ppm, repeat Module 3.0 until KF≤500 ppm./ 通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。若含水≤500 ppm，则进行下一操作；若含水＞500 ppm，则重复进行模块3.0直至含水≤500 ppm : |  |  |  |  |  |
| Drain waste after sampling./取样后排废 |  |  |  |  |  |
| Filter/过滤 | Begin/开始： |  |  | End/结束： |  |
|  | Performed by & Date/执行人及日期： |  |  |  |  |
|  | Checked by & Date/核实人及日期： |  |  |  |  |
| Confirms Water content ≤500 ppm. /确认水分≤500 ppm。 Confirms Water conten t≤500 ppm, move to Modules 3.1-3.43. /确认水分≤500 ppm。调用模块：3.1-3.43。 KF>500ppm, move to Module 3.0(2) /含水>500ppm，进行模块3.0(2)。 Confirmed by & Date/确认人及日期： Checked by & Date/核实人及日期： |  |  |  |  |  |
| Invoke synthesis recipe. /调用已经建立的合成配方： Recipe name /配方名称：14002001M01V02 Batch name /批次命名：CPo140020 01 Move to modules 3.1-3.43/调用3.1-3.43模块； Move to modules 3.0(2)/调用3.0(2)模块； Performed by & Date/执行人及日期： Checked by & Date/核实人及日期： |  |  |  |  |  |

| 3.0. Pre-treatment/预处理 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 3.0 (2) DMF addition and water content testing /检测含水： Add 200.00 - 300.00 kg DMF via spray ball into activation tank (TJ4S-1213-Sy15-R01). Start the agitator of activation tank to 95±10 rpm (target: 95 rpm). Stir for 5±2 min (target 5 min). Then transfer the solution into synthesis reactor (TJ4S-1213-Sy15). Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h). Bubble with nitrogen for 5±2 min (target 5 min). Start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm). Stir for 5±2 min (target 5 min), turn off the agitation and nitrogen. Sample the mixture for Water content analysis. Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point. If Water content >500 ppm, repeat Module 3.0 until Water content ≤500 ppm. If Water content ≤500 ppm, move to next operation. 通过淋洗球向激活罐（TJ4S-1213-Sy15-R01）中加入200.00 - 300.00 kg N,N-二甲基甲酰胺。开启激活罐搅拌转速至95±10 rpm（目标: 95 rpm），搅拌5±2 min (目标：5 min)。转移至合成釜（TJ4S-1213-Sy15）中，开启合成釜下口鼓氮气，氮气流速开至5±2 m³/h (目标流速:5 m³/h)，鼓氮气5±2 min (目标：5 min)后。开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm)，搅拌5±2 min (目标：5 min)，停止搅拌，关闭氮气取样检测体系水分，通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。若水分＞500 ppm，则重复进行模块3.0直至水分≤500 ppm；若水分≤500 ppm，则进行下一操作。 |  |  |  |  | Times/次数 |
|  |  |  |  |  | 1 |
| Add 200.00 - 300.00 kg DMF by spray balls into activation tank TJ4S-1213-Sy15-R01/通过淋洗球向激活罐TJ4S-1213-Sy15-R01中加入200.00 – 300.00 kg N,N-二甲基甲酰胺。 |  |  |  |  |  |
| Rinse with DMF/N,N-二甲基甲酰胺淋洗 |  | Performed by & Date/执行人及日期： |  |  |  |
| Rinse with DMF/ N,N-二甲基甲酰胺淋洗 | Begin/开始： |  | End/结束： | Qty/重量： |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |

| 3.0. Pre-treatment/预处理 |  |  |  |
| --- | --- | --- | --- |
| Start the agitator of activation tank to 95±10 rpm (target: 95 rpm). Stir for 5 min±2 min (target: 5 min). /开启激活罐搅拌转速至95±10 rpm (目标转速: 95 rpm)。搅拌5 min±2 min（目标时间:5 min）。 |  |  |  |
| Pre-treatment/预处理 | Time point/时间节点 |  | Wash /洗涤 |
| Stir /搅拌 | Performed by & Date/执行人及日期： |  |  |
| Stir /搅拌 | Begin/开始： | End/结束： |  |
| Transcribed by & Date /转录人及日期： |  |  |  |
| Then transfer the solution into synthesis reactor TJ4S-1213-Sy15. Confirm complete transfer./转移至固相合成釜TJ4S-1213-Sy15.中。 |  |  |  |
| Transfer/转移 | Begin/开始： |  | End/结束： |
|  | Performed by & Date/执行人及日期： |  |  |
|  | Checked by & Date/核实人及日期： |  |  |
| Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h). Bubble with nitrogen for 5±2 min (target 5 min). Start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm). Stir for 5 min±2 min (target: 5 min). Stop the stirring and turn off the nitrogen. /开启合成釜下口鼓氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min (目标时间：5 min)后。开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm)，搅拌5 min±5 min（目标时间:5 min）停止搅拌，关闭氮气。 |  |  |  |
| Bubble/鼓氮气 | Performed by & Date/执行人及日期： |  |  |
| Bubble/鼓氮气 | Begin/开始： | End/结束： |  |
| Transcribed by & Date /转录人及日期： |  |  |  |
| Stir /搅拌 | Begin/开始： |  | End/结束： |
|  | Performed by & Date/执行人及日期： |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |

| 3.0. Pre-treatment/预处理 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 2nd water content IPC Standard sampling method/第二次IPC水分标准取样方法 |  |  |  |  |  |  |
| Approximately 10~20 ml of the mixture is withdrawn from the reactor via a dedicated in-line sampler and submit for analysis testing. During sampling, should wear nitrile gloves and 3M 6000 series Full-face piece respirator, 6006CN cartridges. 通过在线取样器从反应釜内抽取约取体系约10~20ml，直接送样检测。取样过程中，应该戴丁腈手套、3M 6000系列全面罩, 6006CN滤毒盒。 |  |  |  |  |  |  |
| 2nd water content IPC Sampling number/ 第二次水分取样编号 | Sampling time/ 取样时间 |  | Sampled by & Date/取样人及日期 | Water content/水分 |  | Result filled by &Date/ 结果填写人及日期 |
| CPo140020- -01- -3.0-02 |  |  |  |  |  |  |
| Drain until there is no drip, the operator visually confirms the draining end point.If KF ≤500 ppm, move to next operation. If KF>500 ppm, repeat Module 3.0 until KF≤500 ppm./ 通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。若含水≤500 ppm，则进行下一操作；若含水＞500 ppm，则重复进行模块3.0直至含水≤500 ppm : |  |  |  |  |  |  |
| Drain waste after sampling./取样后排废 |  |  |  |  |  |  |
| Filter/过滤 |  | Begin/开始： |  |  | End/结束： |  |
|  |  | Performed by & Date/执行人及日期： |  |  |  |  |
|  |  | Checked by & Date/核实人及日期： |  |  |  |  |
| Transcribed by & Date /转录人及日期: |  |  |  |  |  |  |
| Confirms Water content ≤500 ppm. /确认水分≤500 ppm。 Confirms Water conten t≤500 ppm, move to Modules 3.1-3.43. /确认水分≤500 ppm。调用模块：3.1-3.43。 Otherwise, repeat Module 3.0. /否则，重复模块3.0。（From/上接： ；To/下转： ）； Confirmed by & Date/确认人及日期： Checked by & Date/核实人及日期： |  |  |  |  |  |  |
| Invoke synthesis recipe. /调用已经建立的合成配方： Recipe name /配方名称：14002001M01V02 Batch name /批次命名：CPo140020 01 Move to modules 3.1-3.43/调用3.1-3.43模块； Move to modules 3.0/调用3.0模块； Confirmed by & Date/确认人及日期： Checked by & Date/核实人及日期： |  |  |  |  |  |  |

3.0. Pre-treatment/预处理

Confirm the synthetic recipe is selected correctly and the instrument is ready for work. /执行合成配方选用正确，仪器各项工作准备就绪。 Confirmed by & Date/确认人及日期： Checked by & Date/核实人及日期： Click run button and start synthesis. /点击运行，开始合成。 Confirmed by & Date/确认人及日期： Checked by & Date/核实人及日期：

| - Resin swelling/树脂溶胀 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CAUTION/注意：2-CTC resin: OEB3, resin can not swell too long time and avoid excessive drying caused by resin water absorption deactivation. / 2-氯三苯甲基氯树脂OEB3，树脂不可溶胀过长时间以及避免过量的抽干引起树脂吸水失活。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |  |  |
| 3.1.1 2-CTC resin addition/加2-氯三苯甲基氯树脂 Add 48.00 kg 2-CTC resin (1.00eq) (Acceptable weighing range：47.90 kg ~ 48.10 kg) with powder feeding bags into synthesis reactor (TJ4S-1213-Sy15). 通过投料袋向合成釜(TJ4S-1213-Sy15)中加入 48.00kg 2-氯三苯甲基氯树脂(1.00eq)（可接受称量范围： 47.90 kg ~ 48.10 kg）。 |  |  |  |  |  |  | Times/次数 |
|  |  |  |  |  |  |  | 1 |
| Before material charging, confirm the material is within the retest date/ the expiry date. /加物料前，执行物料在复检日期/有效期之内。 Before solid material charging, confirm/加固体物料前，确认： - Before operation performed, confirm there is no non-related material / equipment / instrument / tools / pipeline / document / record / label in the intended area. /操作前，执行目标区域无不相关物料/设备/仪器/工具/管路/文件/记录/标签； - The outer packages are removed. If the outer package cannot be removed, but unpack material without damaging the package bag, and before charging, check no package fragment and other extraneous matter in material. /外包装移除；如果外包装不能移除，但在不破坏包装的情况下打开包装袋，投料前以及过程中同时检查执行物料中无包装碎片以及异物。 Confirmed by & Date/核实人及日期： |  |  |  |  |  |  |  |
| Charge 2-CTC resin/加2-氯三苯甲基氯树脂 |  | Performed by & Date/执行人及日期： |  |  |  |  |  |
| Charge 2-CTC resin/ 加2-氯三苯甲基氯树脂 |  |  | Begin/开始： |  | End/结束： |  |  |
| Transcribed by & Date/转录人及日期： |  |  |  |  |  |  |  |
| 2-CTC resin confirmation/2-氯三苯甲基氯树脂执行：Control No./内部控制号：YL005205 |  |  |  |  |  |  |  |
| Check items/检查项 |  | Is it correct?/是否正确 |  | Recorded by & Date/ 记录人及日期 |  | Checked by & Date/ 核实人及日期 |  |
| Material name/物料名称 |  | Yes/是；No/否 |  |  |  |  |  |
| Project batch number/ 项目批次号 |  | Yes/是；No/否 |  |  |  |  |  |
| Operation number/ 操作号 |  | Yes/是；No/否 |  |  |  |  |  |
| Control No. /内部控制号 |  | Yes/是；No/否 |  |  |  |  |  |
| Raman Scan Confirmation/拉曼扫描确认 |  | Conforms/相符 Does not conform/不相符 |  |  |  |  |  |

| 3.1. Resin swelling/树脂溶胀 |  |  |  |  |
| --- | --- | --- | --- | --- |
| No. /编号 | Net weight/净重 | All in/全部加入 | Recorded by & Date/ 记录人及日期 | Checked by & Date/ 核实人及日期 |
| / |  | Yes/是；No/否 |  |  |
| / |  | Yes/是；No/否 |  |  |
| / |  | Yes/是；No/否 |  |  |
| / |  | Yes/是；No/否 |  |  |
| / |  | Yes/是；No/否 |  |  |
| / |  | Yes/是；No/否 |  |  |
| / |  | Yes/是；No/否 |  |  |
| / |  | Yes/是；No/否 |  |  |
| / |  | Yes/是；No/否 |  |  |
| / |  | Yes/是；No/否 |  |  |
| / |  | Yes/是；No/否 |  |  |
| / |  | Yes/是；No/否 |  |  |
| / |  | Yes/是；No/否 |  |  |
| / |  | Yes/是；No/否 |  |  |
| / |  | Yes/是；No/否 |  |  |
| / |  | Yes/是；No/否 |  |  |
| Total weight/合计 |  | Confirm the total net weight is within the input range/执行合计净重在投料范围内 | Recorded by & Date/记录人及日期 | Checked by & Date/ 核实人及日期 |
|  |  | Yes/是；No/否 |  |  |
| After solid material charging, confirm/固体物料投料之后，确认： - The corresponding leftover material has been properly sealed and transferred to the staging area. /相应的剩余物料已经恰当密封，转移到了暂存区域。 - The waste packaging material has been sealed and transferred to the corresponding waste staging area. /产生的包材废物已经密封并转移到了相应的废物暂存区域。 - The operation area/equipment surface has been cleaned. /操作区域以及设备表面已经清洁。 Checked by & Date/核实人及日期： |  |  |  |  |

| 3.1. Resin swelling/树脂溶胀 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CAUTION/注意：DMF/N，N-二甲基甲酰胺：Irritation of eyes, skin and nose. May cause nausea. /刺激眼睛、皮肤和鼻子。可能引起恶心。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |  |  |
| 3.1.2 Add Swelling solvent/加溶胀溶剂: Rinse synthesis reactor (TJ4S-1213-Sy15) wall by spray balls with 318.53 kg（Acceptable weighing range：308.97 kg ~ 328.09 kg）(7 V) DMF. 通过淋洗球向合成釜（TJ4S-1213-Sy15）中加入318.53 kg（可接受称量范围：308.97 kg ~ 328.09 kg） （7 V）N,N-二甲基甲酰胺淋洗釜壁。 |  |  |  |  |  |  | Times/次数 |
|  |  |  |  |  |  |  | 1 |
| Rinse with DMF/ N,N-二甲基甲酰胺淋洗 |  | Performed by & Date/执行人及日期： |  |  |  |  |  |
|  |  | Begin/开始： |  |  | End/结束： | Qty/重量： |  |
| Transcribed by & Date /转录人及日期: |  |  |  |  |  |  |  |
| 3.1.3 Resin swelling/树脂溶胀: Synthesis reactor (TJ4S-1213-Sy15) is bubbled with nitrogen for 5±2 min (target 5 min), then start the agitator to 20±10 rpm (target: 20 rpm). 开启合成釜（TJ4S-1213-Sy15）下口鼓氮气，氮气流速开至5±2m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min (目标：5 min)后开始搅拌，搅拌转速20±10rpm(目标转速: 20rpm)。 |  |  |  |  |  |  | Times/次数 |
|  |  |  |  |  |  |  | 1 |
| Bubble/鼓氮气 |  |  |  | Performed by & Date/执行人及日期： |  |  |  |
|  |  |  |  | Begin/开始： |  |  |  |
|  |  |  |  | The end time is documented in Ins3.1.6 /结束时间记录于3.1.6 |  |  |  |
| Transcribed by & Date /转录人及日期: |  |  |  |  |  |  |  |
| Stir /搅拌 |  |  | Performed by & Date/执行人及日期： |  |  |  |  |
|  |  |  | Begin/开始： |  |  |  |  |
|  |  |  | The end time is documented in Ins3.1.6 /结束时间记录于3.1.6 |  |  |  |  |
| Transcribed by & Date /转录人及日期: |  |  |  |  |  |  |  |

| 3.1. Resin swelling/树脂溶胀 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 3.1.4 Resin rinsing/树脂淋洗 Rinse reactor (TJ4S-1213-Sy15) wall with 45.50 kg (Acceptable weighing range：43.50 kg ~ 47.50 kg) (1 V) DMF. / 向合成釜（TJ4S-1213-Sy15）中加入45.50 kg（可接受称量范围：43.50 kg ~ 47.50 kg）（1 V）N,N-二甲基甲酰胺淋洗釜壁。 |  |  |  |  |  | Times/次数 |
|  |  |  |  |  |  | 1 |
| Rinse with DMF/ N,N-二甲基甲酰胺淋洗 |  | Performed by & Date/执行人及日期： |  |  |  |  |
|  |  | Begin/开始： | End/结束： |  | Qty/重量： |  |
| Transcribed by & Date /转录人及日期: |  |  |  |  |  |  |
| 3.1.5 Adjust temperature /调温 Adjust the mixture to 25±5 ℃ (target: 25 ℃). /将体系调温至25±5 ℃(目标：25 ℃)。 Over-temperature warning value/超温预警设置数值： Low limit/低限: 22 oC ，High limit/高限: 28oC |  |  |  |  |  | Times/次数 |
|  |  |  |  |  |  | 1 |
| Adjust to 25±5℃/调温至25±5℃ | Performed by & Date/执行人及日期： |  |  |  |  |  |
|  | Begin/开始： |  |  | End/结束： |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |

| 3.1. Resin swelling/树脂溶胀 |  |  |  |  |
| --- | --- | --- | --- | --- |
| 3.1.6 Resin swelling /树脂溶胀 After stirring with the speed of 20±10 rpm (target: 20 rpm). Continue stirring for 35±5 min (target: 35 min) at 25±5 ℃ (target: 25 ℃), stop stirring, turn off the nitrogen. Start draining the solution from synthesis reactor with diaphragm pump. The operator visually confirms the draining end point. 控温25±5 ℃(目标：25 ℃)，合成釜搅拌转速控制20±10 rpm (目标转速: 20 rpm)，继续搅拌35±5 min (目标35 min)后停止搅拌，关闭氮气。通过隔膜泵从合成釜中排出溶液。由操作人员目视确认排液终点。 |  |  |  | Times/次数 |
|  |  |  |  | 1 |
| Continue stirring for 35±5 min (target 35 min), stop stirring, turn off the nitrogen. /继续搅拌35±5 min (目标时间：35 min)后停止搅拌，关闭氮气。 |  |  |  |  |
| Stir /搅拌 |  | Performed by & Date/执行人及日期： |  |  |
|  |  | End/结束： |  |  |
|  |  | The start time is documented in Ins3.1.3 /开始时间记录于3.1.3 |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |
| Bubble/鼓氮气 | Performed by & Date/执行人及日期： |  |  |  |
|  | End/结束： |  |  |  |
|  | The start time is documented in Ins3.1.3 /开始时间记录于3.1.3 |  |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |
| Internal temperature/温度 |  | High values/最高值： | Low values/最低值： |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |
| Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point. /通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  |  |  |
| Filter/过滤 |  | Performed by & Date/执行人及日期： |  |  |
|  |  | Checked by & Date/核实人及日期： |  |  |
|  |  | Begin/开始： | End/结束： |  |

| 3.1. Resin swelling/树脂溶胀 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| CAUTION/注意：DMF/N，N-二甲基甲酰胺：Irritation of eyes, skin and nose. May cause nausea. /刺激眼睛、皮肤和鼻子。可能引起恶心。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |
| 3.1.7 Post swelling wash / 溶胀后洗涤 Add 227.52 kg (Acceptable weighing range：220.69 kg ~ 234.35 kg) (5 V) DMF into synthesis reactor (TJ4S-1213-Sy15). 向合成釜（TJ4S-1213-Sy15）中加入227.52 kg（可接受称量范围：220.69 kg ~ 234.35 kg） (5 V) N,N-二甲基甲酰胺。 |  |  |  |  | Times/次数 |
|  |  |  |  |  | 1 |
| DMF addition / 加N,N-二甲基甲酰胺 | Performed by & Date/执行人及日期： |  |  |  |  |
|  | Begin/开始： |  | End/结束： | Qty/重量： |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |
| 3.1.8 Post swelling wash / 溶胀后洗涤 Turn on the nitrogen of synthesis reactor (TJ4S-1213-Sy15) to 5±2 m³/h (target: 5 m³/h), bubble with nitrogen for 5±2 min (target: 5 min), then start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm). After stirring for 12±2 min (target: 12 min), stop stirring, turn off the nitrogen. Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point. 开启合成釜（TJ4S-1213-Sy15）下口鼓氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min (目标：5 min)后，开启合成釜(TJ4S-1213-Sy15)搅拌转速至20±10 rpm (目标转速: 20 rpm)，搅拌12±2 min（目标：12 min）后，停止搅拌，关闭氮气。通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  |  |  | Times/次数 |
|  |  |  |  |  | 1 |
| Bubble/鼓氮气 |  | Performed by & Date/执行人及日期： |  |  |  |
|  |  | Begin/开始： |  |  |  |
| Transcribed by & Date /转录人及日期: |  |  |  |  |  |

| 3.1. Resin swelling/树脂溶胀 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm). After stirring for 12±2 min (target: 12 min), stop stirring, turn off the nitrogen. /开启合成釜(TJ4S-1213-Sy15)搅拌转速至20±10 rpm (目标转速: 20 rpm)，搅拌12±2 min（目标：12 min）后，停止搅拌，关闭氮气 |  |  |  |  |  |  |
| Stir /搅拌 |  | Performed by & Date/执行人及日期： |  |  |  |  |
| Stir /搅拌 |  | Begin/开始： |  | End/结束： |  |  |
| Transcribed by & Date /转录人及日期: |  |  |  |  |  |  |
| Bubble/鼓氮气 |  | Performed by & Date/执行人及日期： |  |  |  |  |
| Bubble/鼓氮气 |  | End/结束： |  |  |  |  |
| Transcribed by & Date /转录人及日期: |  |  |  |  |  |  |
| Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point. /通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  |  |  |  |  |
| Filter/过滤 |  |  |  | Performed by & Date/执行人及日期： |  |  |
|  |  |  |  | Checked by & Date/核实人及日期： |  |  |
|  |  |  |  | Begin/开始： |  | End/结束： |

| - （1）Fmoc-Gly-OH activation /芴甲氧羰基-甘氨酸活化 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| CAUTION/注意：DMF/N，N-二甲基甲酰胺：Irritation of eyes, skin and nose. May cause nausea. /刺激眼睛、皮肤和鼻子。可能引起恶心。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |  |
| 3.2.1 DMF addition /加料N,N-二甲基甲酰胺 Add 193.39 kg (Acceptable weighing range：187.59 kg ~ 199.19 kg) (4.25 V, 4.029 kg/kg) DMF into the activation tank (TJ4S-1213-Sy15-R01). Start the agitator, Adjust to 25±5 ℃ (target: 25 ℃). 向激活罐（TJ4S-1213-Sy15-R01）中加入193.39 kg（可接受称量范围：187.59 kg ~ 199.19 kg）（4.25 V, 4.029 kg/kg）N，N-二甲基甲酰胺，开启搅拌，将体系调温至25±5 ℃(目标：25 ℃)。 |  |  |  |  |  | Times/次数 |
|  |  |  |  |  |  | 1 |
| DMF addition / 加N,N-二甲基甲酰胺 |  | Performed by & Date/执行人及日期： |  |  |  |  |
|  |  | Begin/开始： | End/结束： |  | Qty/重量： |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |
| Stir /搅拌 | Performed by & Date/执行人及日期： |  |  |  |  |  |
|  | Begin/开始： |  |  |  |  |  |
|  | The end time is documented in Ins3.2.5 /结束时间记录于3.2.5 |  |  |  |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |
| Adjust/调温 | Performed by & Date/执行人及日期： |  |  |  |  |  |
| Adjust/调温 | Begin/开始： |  |  | End/结束： |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |
| 3.2.2 Fmoc-Gly-OH addition /加芴甲氧羰基-甘氨酸 Add Fmoc-Gly-OH (2.25 eq.) with powder feeding bags into the activation tank (TJ4S-1213-Sy15-R01) at 25±5 ℃ (target: 25 ℃). 控温25±5 ℃(目标：25 ℃),通过投料袋向激活罐（TJ4S-1213-Sy15-R01）加入 kg芴甲氧羰基-甘氨酸（2.25 eq.）（Acceptable weighing range/可接受称量范围： kg ~ kg）。 |  |  |  |  |  | Times/次数 |
|  |  |  |  |  |  | 1 |

| 3.2. （1）Fmoc-Gly-OH activation /芴甲氧羰基-甘氨酸活化 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ★Note:The dissolution temperature is a CPP, and the dissolution of Fmoc-Gly-OH hold temperature is a CPP./ 备注：溶解温度是CPP，溶解芴甲氧羰基-甘氨酸维持温度是关键工艺参数。 The operator shall confirm that the dissolution of Fmoc-Gly-OH mixture temperature is within the Normal Operating Range (NOR) 【25±5 oC (target: 25 oC)】/操作人员须确认芴甲氧羰基-甘氨酸溶解体系温度处于正常操作范围（NOR）内【25±5℃(目标温度：25 ℃)】 Checked by & Date/核实人及日期： Over-temperature warning value/超温预警设置数值： Lower limit/低限: 22 oC ，High limit/高限: 28oC |  |  |  |  |  |
| Before material charging, confirm the material is within the retest date/ the expiry date. /加物料前，执行物料在复检日期/有效期之内。 Before solid material charging, confirm/加固体物料前，执行： - Before operation performed, confirm no non-related material / equipment / instrument / tools / pipeline / document / record / label in the intended area /操作前，执行目标区域无不相关物料/设备/仪器/工具/管路/文件/记录/标签； - The outer packages are removed. If the outer package cannot be removed, but unpack material without damaging the package bag, and before charging, check no package fragment and other extraneous matter in material. /外包装移除；如果外包装不能移除，但在不破坏包装的情况下打开包装袋，投料前以及过程中同时检查执行物料中无包装碎片以及异物 Performed by & Date/执行人及日期： |  |  |  |  |  |
| Prior to charging each amino acid, the operator shall perform a Raman scan to confirm the identity of the material. The Raman scan result shall be compared against the reference spectrum of the corresponding amino acid. Only materials that conform to the reference spectrum may be charged. If the Raman scan result does not conform, stop the operation, quarantine the material, and initiate a investigation per site SOP. Record the Raman scan result, operator signature, and date/time in the table below./ 每次投氨基酸前，操作人员应进行拉曼扫描以执行物料。拉曼扫描结果应与相应氨基酸的参考光谱进行比对。仅与参考光谱相符的物料方可投料。如拉曼扫描结果不相符，应停止操作，对物料进行隔离，并依据厂区 SOP 启动异常事件调查。在下表中记录拉曼扫描结果、操作人员签字及日期 / 时间。 |  |  |  |  |  |
| Fmoc-Gly-OH confirmation/芴甲氧羰基-甘氨酸执行：Control No./内部控制号：YL004846 |  |  |  |  |  |
| Charge Fmoc-Gly-OH /加芴甲氧羰基-甘氨酸 |  | Performed by & Date/执行人及日期： |  |  |  |
|  |  | Begin/开始： |  | End/结束： |  |
| Transcribed by & Date /转录人及日期: |  |  |  |  |  |
| Check items/检查项 | Is it correct?/是否正确 |  | Recorded by & Date/ 记录人及日期 |  | Checked by & Date/ 核实人及日期 |
| Material name/物料名称 | Yes/是；No/否 |  |  |  |  |
| Project batch number/ 项目批次号 | Yes/是；No/否 |  |  |  |  |
| Operation number/ 操作号 | Yes/是；No/否 |  |  |  |  |

| 3.2. （1）Fmoc-Gly-OH activation /芴甲氧羰基-甘氨酸活化 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Check items/检查项 |  |  |  | Is it correct?/是否正确 |  | Recorded by & Date/ 记录人及日期 |  | Checked by & Date/ 核实人及日期 |
| Control No./内部控制号 |  |  |  | Yes/是；No/否 |  |  |  |  |
| Raman Scan Confirmation/拉曼扫描执行 |  |  |  | Conforms/相符 Does not conform/不相符 |  |  |  |  |
| No. /编号 |  |  | Net weight/净重 | All in/全部加入 |  | Recorded by & Date/ 记录人及日期 |  | Checked by & Date/ 核实人及日期 |  |
| / |  |  |  | Yes/是；No/否 |  |  |  |  |  |
| / |  |  |  | Yes/是；No/否 |  |  |  |  |  |
| / |  |  |  | Yes/是；No/否 |  |  |  |  |  |
| / |  |  |  | Yes/是；No/否 |  |  |  |  |  |
| / |  |  |  | Yes/是；No/否 |  |  |  |  |  |
| / |  |  |  | Yes/是；No/否 |  |  |  |  |  |
| / |  |  |  | Yes/是；No/否 |  |  |  |  |  |
| / |  |  |  | Yes/是；No/否 |  |  |  |  |  |
| / |  |  |  | Yes/是；No/否 |  |  |  |  |  |
| / |  |  |  | Yes/是；No/否 |  |  |  |  |  |
| Total weight/合计 |  |  |  | Confirm the total net weight is within the input range/执行合计净重在投料范围内 |  | Recorded by & Date/记录人及日期 |  | Checked by & Date/ 核实人及日期 |  |
|  |  |  |  | Yes/是；No/否 |  |  |  |  |  |
| After solid material charging, confirm/固体物料投料之后，确认： - The corresponding leftover material has been properly sealed and transferred to the staging area. /相应的剩余物料已经恰当密封，转移到了暂存区域。 - The waste packaging material has been sealed and transferred to the corresponding waste staging area. /产生的包材废物已经密封并转移到了相应的废物暂存区域。 - The operation area/equipment surface has been cleaned. /操作区域以及设备表面已经清洁。 Checked by & Date/核实人及日期： |  |  |  |  |  |  |  |  |  |

| 3.2.（1）Fmoc-Gly-OH activation /芴甲氧羰基-甘氨酸活化 |  |  |  |  |
| --- | --- | --- | --- | --- |
| CAUTION/注意：DMF/N，N-二甲基甲酰胺：Irritation of eyes, skin and nose. May cause nausea. /刺激眼睛、皮肤和鼻子。可能引起恶心。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |
| 3.2.3 Resin rinsing/树脂淋洗: Rinse the port of Activation tank (TJ4S-1213-Sy15-R01) with 45.50 kg（Acceptable weighing range：43.50 kg ~ 47.50 kg）（1 V, 0.948 kg/kg）DMF. Continue agitation of activation tank to 95±10 rpm (target: 95 rpm). Maintain stirring for 1~45 min (target: 5 min). Confirm no visible solids and later use. 用45.50 kg（可接受称量范围：43.50 kg ~ 47.50 kg）（1 V）N,N-二甲基甲酰胺淋洗激活罐（TJ4S-1213-Sy15-R01）加料口。维持激活罐搅拌转速至95±10 rpm (目标转速: 95 rpm)，继续搅拌1~45 min (目标时间：5min)，确认无明显固体待用。 |  |  |  | Times/次数 |
|  |  |  |  | 1 |
| Rinse the port of the activation tank (TJ4S-1213-Sy15-R01) with 45.50 kg（1 V）DMF by spray balls. 通过淋洗球用45.50 kg N,N-二甲基甲酰胺（1 V）。 |  |  |  |  |
| Rinse with DMF/N,N-二甲基甲酰胺淋洗 | Performed by & Date/执行人及日期： |  |  |  |
| Rinse with DMF/ N,N-二甲基甲酰胺淋洗 | Begin/开始： | End/结束： | Qty/重量： |  |
| Transcribed by & Date /转录人及日期: |  |  |  |  |

| 3.2. （1）Fmoc-Gly-OH activation /芴甲氧羰基-甘氨酸活化 |  |  |
| --- | --- | --- |
| Stirring for NLT 15min(Target: 20min) and later use./搅拌至少15 min(目标时间：20 min)，待用。 |  |  |
| Stir /搅拌 | Performed by & Date/执行人及日期： |  |
|  | Finish time/完成时间： |  |
| Transcribed by & Date /转录人及日期： |  |  |
| CAUTION/注意：DIPEA/N,N-二异丙基乙胺：It can enter the body through inhalation, ingestion and skin absorption. It is strongly irritating to the eyes, skin, mucous membranes and upper respiratory tract. Inhalation of high-concentration vapors may cause inflammation, edema and spasm of the larynx/bronchi, even chemical pneumonia and pulmonary edema. Exposure may cause symptoms such as burning sensation, cough, headache, nausea and vomiting. /可通过吸入、食入、皮肤吸收侵入人体，对眼、皮肤、黏膜和上呼吸道有强刺激性，吸入高浓度蒸气会引发喉/支气管炎症水肿、痉挛，甚至化学性肺炎、肺水肿，接触后会出现烧灼感、咳嗽、头痛、恶心呕吐等症状 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Checked by & Date/核实人及日期： |  |  |
| 3.2.4 DIPEA addition /加N,N-二异丙基乙胺 Add DIPEA (6.00 eq.) into the activation tank (TJ4S-1213-Sy15-R01) at 【25±5 ℃ (target: 25 ℃) 】. Reference addition time is 30±15 min (target: 30 min). Actual addition rate is according to the temperature control. 控温【25±5 ℃(目标温度：25 ℃)】，向激活罐（TJ4S-1213-Sy15-R01）中滴加 kg N,N-二异丙基乙胺(6.00 eq.)（Acceptable weighing range/可接受称量范围： kg ~ kg），参考滴加时间30±15 min(目标：30 min)，实际加料速度以控温为主。 |  | Times/次数 |
|  |  | 1 |
| ★Note: DIPEA quantity is a CPP. /备注：N, N-二异丙基乙胺用量是关键工艺参数。 The operator shall confirm that DIPEA quantity is within the Normal Operating Range (NOR) 【5.82~6.18 eq.】/操作人员须确认N, N-二甲基甲酰胺活化总用量处于正常操作范围（NOR）内：【5.82~6.18 eq.】 ★After coupling is completed, the operator shall confirm that all Critical Process Parameters (CPPs) are within their respective Normal Operating Ranges (NOR)/ 偶联结束后，操作人员须确认所有关键工艺参数（CPP）均处于各自的正常操作范围（NOR）内 Checked by & Date/核实人及日期： |  |  |

3.2.（1）Fmoc-Gly-OH activation /芴甲氧羰基-甘氨酸活化

Before material charging, confirm the material is within the retest date/ the expiry date, confirm whether the weight is adequate for use in the current charging. /加物料前，执行物料在复检日期/有效期之内，确认重量是否满足本次投料使用。 Checked by & Date/核实人及日期：

Before drummed liquid material charging, visual check there is no other extraneous matter in material. /加桶装液体物料前，应目视执行桶内无异物。 Visual Examination of Foreign Matters for Drum Solvent/桶装溶剂异物目视检查: 1. Place the solvent drum in a well-lit area (or use a portable light source) to ensure adequate visibility of the liquid contents./ 将溶剂桶置于光线充足区域（或使用便携式光源），确保液体内容物可见性良好 2. Visually inspect the solvent through the drum bung opening or transparent sight glass for any visible foreign matters, including but not limited to particulates, fibers, sediments, or unusual turbidity./ 通过桶口或透明视镜目视检查溶剂中是否有可见异物，包括但不限于颗粒、纤维、沉淀物或异常浑浊。 3. Confirm that the solvent is clear and free of visible foreign matters before charging. If any foreign matter is observed, do not use the drum and follow the applicable handling procedure for subsequent disposition./ 投料前确认溶剂澄清且无可见异物。如发现任何异物，不得使用该桶，并依据处理流程进行后续处置。 Checked by & Date/核实人及日期：

Pump number/泵编号： Weighing of DIPEA/N,N-二异丙基乙胺称重： Control Number/内部控制号：YL000555 Pump number/泵编号： Scale ID. No./秤编号： Material charging pipe number/物料管路编号：

3.2.（1）Fmoc-Gly-OH activation /芴甲氧羰基-甘氨酸活化

Confirm liquid charging pipeline is released. /执行加料管路经过放行。

Checked by & Date/核实人及日期：

Charging of drummed solvent: verify that the pipeline connections remain consistent before and after material feeding, then record the weighing reading 桶装溶剂溶液称量时，确认投料前后管路连接一致，读取称量示数。 Performed by & Date/执行人及日期： Checked by & Date/核实人及日期：

If the target net weight≥the planned net weight, perform full drum charge; otherwise, perform partial (Non-full drum) charge. /当目标加料量≥计划加入净重，勾选进行整桶加料；否则勾选进行零头加料。 Note: Target net weight to be charged = Total target net weight – Total net weight charged. Expected tare weight of drum after charge = Gross weight of drum that to be charged - Target net weight to be charged. 还需加入净重目标值= 加料步骤目标总净重-已加入净重总量 加料完成后预计桶皮重=待加料桶毛重-还需加入净重目标值

| 3.2. （1）Fmoc-Gly-OH activation /芴甲氧羰基-甘氨酸活化 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Addition type加料式 | Lot number/批号 | Pre-charging qty calculation/投料前投料量计算 |  |  |  |  |  | Actual Charging Qty/实际投料量 |  |  |  |  |  |
|  |  | Total qty charged/已加入净重总量 |  | Target qty to be charged/还需加入净重目标值 | Gross qty of drum that to be charged/待加料桶毛重 | Expected net weight of drum after charge /加料完成后预计桶净重 | Expected tare weight of drum after charge/加料完成后预计桶皮重 | Before charging/ 投料前 |  | After charging/ 投料后 | Net weight/ 净重 | Performed by & Date/ 执行人及日期 | Checked by & Date/ 核实人及日期 |
| Full drum整桶 Non-full drum /非整桶 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Full drum整桶 Non-full drum /非整桶 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Total/合计 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DIPEA Addition/加N,N-二异丙基乙胺 |  |  | Begin/开始： |  |  |  |  |  | End/结束： |  |  |  |  |
| Transcribed by & Date/转录人及日期： |  |  |  |  |  |  |  |  |  |  |  |  |  |

| 3.2. （1）Fmoc-Gly-OH activation /芴甲氧羰基-甘氨酸活化 |  |  |  |  |
| --- | --- | --- | --- | --- |
| CAUTION/注意：DIPEA/N,N-二异丙基乙胺：It can enter the body through inhalation, ingestion and skin absorption. It is strongly irritating to the eyes, skin, mucous membranes and upper respiratory tract. Inhalation of high-concentration vapors may cause inflammation, edema and spasm of the larynx/bronchi, even chemical pneumonia and pulmonary edema. Exposure may cause symptoms such as burning sensation, cough, headache, nausea and vomiting. /可通过吸入、食入、皮肤吸收侵入人体，对眼、皮肤、黏膜和上呼吸道有强刺激性，吸入高浓度蒸气会引发喉/支气管炎症水肿、痉挛，甚至化学性肺炎、肺水肿，接触后会出现烧灼感、咳嗽、头痛、恶心呕吐等症状。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Checked by & Date/核实人及日期： |  |  |  |  |
| 3.2.5 Fmoc-Gly-OH activation /芴甲氧羰基-甘氨酸活化 At 25±5 ℃ (target: 25 ℃), Continue agitation of the activation tank (TJ4S-1213-Sy15-RT01) to 95±10 rpm (target: 95 rpm). Maintain stirring for 5±2 min (target 5 min) after addition, stop stirring. 控温25±5 ℃(目标：25 ℃)，维持激活罐（TJ4S-1213-Sy15-RT01）转速至95±10 rpm (目标转速: 95 rpm)，继续搅拌5±2 min (目标：5 min)后停止搅拌。 Over-temperature warning value/超温预警设置数值： Low limit/低限: 22 oC ，High limit/高限: 28oC |  |  | Times/次数 |  |
|  |  |  | 1 |  |
| Stir /搅拌 |  | Performed by & Date/执行人及日期： |  |  |
|  |  | The start time is documented in Ins3.1.3 /开始时间记录于3.2.1 |  |  |
|  |  | End/结束： |  |  |
| Transcribed by & Date /转录人及日期: |  |  |  |  |
| Internal temperature/温度 |  | High values/最高值： | Low values/最低值： |  |
| Transcribed by & Date /转录人及日期: |  |  |  |  |

| - （1）Fmoc-Gly-OH coupling/芴甲氧羰基-甘氨酸偶联 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 3.3.1 Activation mixture transfer /活化体系转移 Transfer the mixture in activation tank (TJ4S-1213-Sy15-R01) into synthesis reactor (TJ4S-1213-Sy15). 将激活罐（TJ4S-1213-Sy15-R01）中体系转移至合成釜（TJ4S-1213-Sy15）中。 |  |  |  |  |  | Times/次数 |
|  |  |  |  |  |  | 1 |
| Transfer/转移 |  | Performed by & Date/执行人及日期： |  |  |  |  |
|  |  | Checked by & Date/核实人及日期： |  |  |  |  |
|  |  | Begin/开始： |  | End/结束： |  |  |
| CAUTION/注意：DMF/N，N-二甲基甲酰胺：Irritation of eyes, skin and nose. May cause nausea. /刺激眼睛、皮肤和鼻子。可能引起恶心。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |  |
| 3.3.2 DMF addition /加N,N-二甲基甲酰胺 Add 45.50 kg (Acceptable weighing range：43.50 kg ~ 47.50 kg) (1 V, 0.948 kg/kg) DMF by spray balls into activation tank (TJ4S-1213-Sy15-R01) to rinse the wall. 通过淋洗球向激活罐（TJ4S-1213-Sy15-R01）中加入45.50 kg（可接受称量范围：43.50 kg ~ 47.50 kg） (1 V, 0.948 kg/kg) N,N-二甲基甲酰胺淋洗罐壁。 |  |  |  |  |  | Times/次数 |
|  |  |  |  |  |  | 1 |
| Rinse with DMF/ N,N-二甲基甲酰胺淋洗 | Performed by & Date/执行人及日期： |  |  |  |  |  |
|  | Begin/开始： |  | End/结束： |  | Qty/重量： |  |
| Transcribed by & Date /转录人及日期: |  |  |  |  |  |  |
| 3.3.3 DMF rinsing solution transfer / N,N-二甲基甲酰胺淋洗液转移 Turn on the nitrogen of synthesis reactor (TJ4S-1213-Sy15) to 5±2 m³/h (target: 5 m³/h), Bubble with nitrogen, after 5±2 min (target: 5 min), then start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm). Transfer the mixture in activation tank (TJ4S-1213-Sy15-R01) into synthesis reactor (TJ4S-1213-Sy15). 合成釜（TJ4S-1213-Sy15）先开启下口氮气，氮气流速开至5±2m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min (目标：5 min)后，开启成釜(TJ4S-1213-Sy15)搅拌转速至20±10 rpm (目标转速: 20 rpm)，将激活罐（TJ4S-1213-Sy15-R01）中体系转移至合成釜（TJ4S-1213-Sy15）中。 |  |  |  |  |  | Times/次数 |
|  |  |  |  |  |  | 1 |

| 3.3. （1）Fmoc-Gly-OH coupling/芴甲氧羰基-甘氨酸偶联 |  |  |  |
| --- | --- | --- | --- |
| Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h). Bubble with nitrogen for 5±2 min (target: 5 min).Start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm)./ 开启合成釜下口鼓氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min（目标时间：5 min）后,开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm) |  |  |  |
| Bubble/鼓氮气 | Performed by & Date/执行人及日期： |  |  |
| Bubble/鼓氮气 | Begin/开始： |  |  |
|  | The end time is documented in Ins 3.3.5/结束时间记录于操作3.3.5 |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |
| Stir /搅拌 | Performed by & Date/执行人及日期： |  |  |
| Stir /搅拌 | Begin/开始： |  |  |
|  | The end time is documented in Ins 3.3.5/结束时间记录于操作3.3.5 |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |
| Transfer the mixture (DMF) from the activation tank TJ4S-1213-Sy15-R01 to the synthesis reactor TJ4S-1213-Sy15./ 将激活罐TJ4S-1213-Sy15-R01中体系转移至合成釜TJ4S-1213-Sy15中。 |  |  |  |
| Transfer/转移 | Performed by & Date/执行人及日期： |  |  |
|  | Checked by & Date/核实人及日期： |  |  |
|  | Begin/开始： | End/结束： |  |
| 3.3.4 Adjust synthesis reactor /调节合成釜温度 Adjust synthesis reactor (TJ4S-1213-Sy15) mixture to 25±5 ℃ (target: 25 ℃). /将合成釜(TJ4S-1213-Sy15)体系调温至25±5 ℃(目标：25 ℃). |  |  | Times/次数 |
|  |  |  | 1 |
| Adjust/调温 | Performed by & Date/执行人及日期： |  |  |
| Adjust/调温 | Begin/开始： | End/结束： |  |
| Transcribed by & Date /转录人及日期： |  |  |  |

| 3.3. （1）Fmoc-Gly-OH coupling/芴甲氧羰基-甘氨酸偶联 |  |  |  |  |
| --- | --- | --- | --- | --- |
| 3.3.5 Fmoc-Gly-OH coupling /芴甲氧羰基-甘氨酸偶联 The mixture is allowed to react fo 【3 h±15 min (target: 3 h)】 at 【25±5 ℃ (target: 25 ℃)】. Stop stirring, turn off the nitrogen. Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point. 控温【25±5 ℃(目标温度：25 ℃)】，反应【3 h±15 min（目标时间：3 h）】后，停止搅拌，关闭氮气。通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 Note: If resin adherence is found at other times during the reaction, rinse the reactor with 0.50~5.00 kg DMF each time. /备注：偶联过程中其它时间发现贴壁，每次使用0.50~5.00 kg N,N-二甲基甲酰胺淋洗，记录见附加页 |  |  |  | Times/次数 |
|  |  |  |  | 1 |
| ★The total reaction Fmoc-Gly-OH coupling time：NLT 600min (Sourced from the holding time study in the PAR study report.)/芴甲氧羰基-甘氨酸偶联总的反应时间不超过600min(来源于 PAR 研究报告中稳定时间研究)。 |  |  |  |  |
| React/反应 |  | Performed by & Date/执行人及日期： |  |  |
|  |  | Begin/开始： | End/结束： |  |
|  |  | The completion time of the coupling reaction serves as the start time of the filtration operation in Ins 3.3.5./偶联反应结束时间即为 Ins 3.3.5 过滤操作的开始时间。 |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |
| ★After coupling is completed, the operator shall confirm that all Critical Process Parameters (CPP) are within their respective Normal Operating Ranges (NOR)/ 偶联结束后，操作人员须确认所有关键工艺参数（CPP）均处于各自的正常操作范围（NOR）内。 ★Note:The coupling temperature is a CPP, and the coupling IPC hold temperature is a CPP./ 备注：偶联温度是CPP，偶联IPC维持温度是关键工艺参数。 The operator shall confirm that the reaction mixture temperature is within the Normal Operating Range (NOR) :20~30℃/操作人员须执行反应体系温度处于正常操作范围（NOR）内：20.0~30.0oC Checked by & Date/核实人及日期： Over-temperature warning value/超温预警设置数值： Lower limit/低限: 22 oC ，High limit/高限: 28 oC |  |  |  |  |

| 3.3. （1）Fmoc-Gly-OH coupling/芴甲氧羰基-甘氨酸偶联 |  |  |  |
| --- | --- | --- | --- |
| Stir /搅拌 | Performed by & Date/执行人及日期： |  |  |
|  | End/结束： |  |  |
|  | The start time is documented in Ins 3.3.3/开始时间记录于操作3.3.3 |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |
| Bubble/鼓氮气 | Performed by & Date/执行人及日期： |  |  |
|  | End/结束： |  |  |
|  | The start time is documented in Ins 3.3.3/开始时间记录于操作3.3.3 |  |  |
| Transcribed by & Date /转录人及日期: |  |  |  |
| Internal temperature/内部温度 |  | High values/最高值： | Low values/最低值： |
| Transcribed by & Date /转录人及日期: |  |  |  |
| Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point. / 通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  |  |
| Filter/过滤 |  | Performed by & Date/执行人及日期： |  |
|  |  | Checked by & Date/核实人及日期： |  |
|  |  | Begin/开始： | End/结束： |

| 3.3. （1）Fmoc-Gly-OH coupling/芴甲氧羰基-甘氨酸偶联 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| CAUTION/注意：DMF/N，N-二甲基甲酰胺：Irritation of eyes, skin and nose. May cause nausea. /刺激眼睛、皮肤和鼻子。可能引起恶心。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |  |
| 3.3.6 Activation Tank Rinse/激活罐洗涤 Add 364.03 kg（Acceptable weighing range：353.11 kg ~ 374.95 kg） (8 V) DMF into the activation tank (TJ4S-1213-Sy15-R01) by spray ball. Circulate and clean the activating liquid transfer pipeline 1, liquid charging port cleaning pipeline 2 and solid charging port cleaning pipeline 3, clean every pipeline for 2±1 min. 通过淋洗球向激活罐（TJ4S-1213-Sy15-R01）中加入364.03 kg（可接受称量范围：353.11 kg ~ 374.95 kg）（8 V）N,N-二甲基甲酰胺，循环清洗激活液转移管路1，液体加料口清洗管路2和固体加料口清洗管路3，每条线路各清洗2±1 min。 |  |  |  |  |  | Times/次数 |
|  |  |  |  |  |  | 1 |
| DMF addition / 加N,N-二甲基甲酰胺 | Performed by & Date/执行人及日期： |  |  |  |  |  |
|  | Begin/开始： |  | End/结束： |  | Qty/重量： |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |
| Clean the activating liquid transfer pipeline 1/循环清洗激活液转移管路1 |  | Performed by & Date/执行人及日期： |  |  |  |  |
|  |  | Begin/开始： |  | End/结束： |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |
| liquid charging port cleaning pipeline 2,/液体加料口清洗管路2 |  | Performed by & Date/执行人及日期： |  |  |  |  |
|  |  | Begin/开始： |  | End/结束： |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |
| Solid charging port cleaning pipeline3 /固体加料口清洗管路3 |  | Performed by & Date/执行人及日期： |  |  |  |  |
|  |  | Begin/开始： |  | End/结束： |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |

| 3.3. （1）Fmoc-Gly-OH coupling/芴甲氧羰基-甘氨酸偶联 |  |  |  |  |
| --- | --- | --- | --- | --- |
| 3.3.7 Post-coupling wash./偶联后洗涤 Transfer the mixture in activation tank (TJ4S-1213-Sy15-R01) to synthesis reactor (TJ4S-1213-Sy15). Turn on the nitrogen of synthesis reactor (TJ4S-1213-Sy15) to 5±2 m³/h (target: 5 m³/h), bubble with nitrogen, after 5±2 min (target 5 min), start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm). After stirring for 7±2 min (target：7 min), stop stirring, turn off the nitrogen. Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point. 将激活罐（TJ4S-1213-Sy15-R01）中体系转入合成釜(TJ4S-1213-Sy15)，开启下口鼓氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min (目标：5 min)后，开启合成釜(TJ4S-1213-Sy15)搅拌转速至20±10 rpm (目标转速: 20 rpm)，搅拌7±2 min（目标：7 min）。停止搅拌，关闭氮气。通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  |  | Times/次数 |
|  |  |  |  | 1 |
| Transfer the liquid to the synthesis reactor (TJ4S-1213-Sy15) ./将激活罐(TJ4S-1213-Sy15)中体系转移至合成釜中 |  |  |  |  |
| Transfer/转移 | Performed by & Date/执行人及日期： |  |  |  |
|  | Checked by & Date/核实人及日期： |  |  |  |
|  | Begin/开始： |  | End/结束： |  |
| Bubble/鼓氮气 | Performed by & Date/执行人及日期： |  |  |  |
| Bubble/鼓氮气 |  | Begin/开始： |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |
| Stir /搅拌 | Performed by & Date/执行人及日期： |  |  |  |
| Stir /搅拌 | Begin/开始： |  | End/结束： |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |
| Bubble/鼓氮气 | Performed by & Date/执行人及日期： |  |  |  |
| Bubble/鼓氮气 |  | End/结束： |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |

| 3.3. （1）Fmoc-Gly-OH coupling/芴甲氧羰基-甘氨酸偶联 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point. / 通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  |  |  |  |  |
| Filter/过滤 |  | Performed by & Date/执行人及日期： |  |  |  |  |
|  |  | Checked by & Date/核实人及日期： |  |  |  |  |
|  |  | Begin/开始： |  | End/结束： |  |  |
| - Capping/封端 |  |  |  |  |  |  |
| CAUTION/注意：DMF/N，N-二甲基甲酰胺：Irritation of eyes, skin and nose. May cause nausea. /刺激眼睛、皮肤和鼻子。可能引起恶心。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |  |
| 3.4.1 DMF addition/加N,N-二甲基甲酰胺 Add 409.54 kg (Acceptable weighing range：397.25 kg ~ 421.83 kg) (9 V, 8.532 kg/kg) DMF into synthesis reactor (TJ4S-1213-Sy15) by spray ball. Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h). Bubble with nitrogen for 5±2 min (target: 5 min), start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm), then turn off the nitrogen. 通过淋洗球向合成釜（TJ4S-1213-Sy15）中加入409.54 kg（可接受称量范围：397.25 kg ~ 421.83 kg）（9 V, 8.532 kg/kg）N，N-二甲基甲酰胺，开启下口鼓氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min (目标：5 min)后，开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm)，关闭氮气。 |  |  |  |  |  | Times/次数 |
|  |  |  |  |  |  | 1 |
| DMF addition /加N,N-二甲基甲酰胺 | Performed by & Date/执行人及日期： |  |  |  |  |  |
|  | Begin/开始： |  | End/结束： |  | Qty/重量： |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |

| 3.4. Capping/封端 |  |  |
| --- | --- | --- |
| Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h). Bubble with nitrogen for 5±2 min (target: 5 min), start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm)./开启合成釜下口鼓氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5 min±2 min（目标时间：5 min）后，开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm)。 |  |  |
| Bubble/鼓氮气 | Performed by & Date/执行人及日期： |  |
| Bubble/鼓氮气 | Begin/开始： |  |
|  | The end time is documented in Ins3.4.5 /结束时间记录于3.4.5 |  |
| Transcribed by & Date /转录人及日期： |  |  |
| Stir /搅拌 | Performed by & Date/执行人及日期： |  |
| Stir /搅拌 | Begin/开始： |  |
|  | The end time is documented in Ins3.4.5 /结束时间记录于3.4.5 |  |
| Transcribed by & Date /转录人及日期： |  |  |
| CAUTION/注意：Methanol/甲醇: Exposure to excessive vapor causes eye irritation, head- ache, fatigue and drowsiness. High concentrations can produce central nervous system depression and optic nerve damage. 50,000 ppm will probably cause death in 1 to 2 hrs. Can be absorbed through skin. Swallowing may cause death or eye damage. /接触过多的蒸汽会引起眼睛刺激、头痛、疲劳和困倦。高浓度可导致中枢神经系统抑制和视神经损伤。50000 ppm可能会在1到2小时内导致死亡。可通过皮肤吸收。吞咽可能导致死亡或眼睛损伤。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |
| 3.4.2 Methanol addition/加甲醇 Add methanol (10.00 eq.) through stainless steel hose and peristaltic pump into synthesis reactor (TJ4S-1213-Sy15). Adjust to 20±5 ℃ (target 20 ℃). 通过不锈钢软管，蠕动泵，向合成釜（TJ4S-1213-Sy15）加入 kg 甲醇（Acceptable weighing range/可接受称量范围： kg ~ kg）(10.00 eq.)，调温至20±5 ℃（目标：20 ℃）。 |  | Times/次数 |
|  |  | 1 |
| Before material charging, confirm the material is within the retest date/ the expiry date. Confirm whether the weight is adequate for use in the current charging. /加物料前，执行物料在复检日期/有效期之内，确认重量是否满足本次投料使用。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Checked by & Date/核实人及日期： |  |  |

3.4. Capping/封端

Before drummed liquid material charging, visual check there is no other extraneous matter in material. /加桶装液体物料前，应目视执行桶内无异物。 Visual Examination of Foreign Matters for Drum Solvent/桶装溶剂异物目视检查: 1. Place the solvent drum in a well-lit area (or use a portable light source) to ensure adequate visibility of the liquid contents./ 将溶剂桶置于光线充足区域（或使用便携式光源），确保液体内容物可见性良好 2. Visually inspect the solvent through the drum bung opening or transparent sight glass for any visible foreign matters, including but not limited to particulates, fibers, sediments, or unusual turbidity./ 通过桶口或透明视镜目视检查溶剂中是否有可见异物，包括但不限于颗粒、纤维、沉淀物或异常浑浊。 3. Confirm that the solvent is clear and free of visible foreign matters before charging. If any foreign matter is observed, do not use the drum and follow the applicable handling procedure for subsequent disposition./ 投料前确认溶剂澄清且无可见异物。如发现任何异物，不得使用该桶，并依据处理流程进行后续处置。 Checked by & Date/核实人及日期：

Confirm liquid charging pipeline is released. /执行加料管路经过放行。

Checked by & Date/核实人及日期：

Pump number/泵编号： Weighing of methanol/甲醇称重： Control Number/内部控制号：YL000031 Scale ID. No./秤编号： Material charging pipe number/物料管路编号：

If the target net weight≥the planned net weight, perform full drum charge; otherwise, perform partial (Non-full drum) charge. /当目标加料量≥计划加入净重，勾选进行整桶加料；否则勾选进行零头加料。 Note: Target net weight to be charged = Total target net weight – Total net weight charged. Expected tare weight of drum after charge = Gross weight of drum that to be charged - Target net weight to be charged. 还需加入净重目标值= 加料步骤目标总净重-已加入净重总量 加料完成后预计桶皮重=待加料桶毛重-还需加入净重目标值

| 3.4. Capping/封端 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Addition type加料式 | Lot number/批号 | Pre-charging qty calculation/投料前投料量计算 |  |  |  |  |  | Actual Charging Qty/实际投料量 |  |  |  |  |  |
|  |  | Total qty charged/已加入净重总量 |  | Target qty to be charged/还需加入净重目标值 | Gross qty of drum that to be charged/待加料桶毛重 | Expected net weight of drum after charge /加料完成后预计桶净重 | Expected tare weight of drum after charge/加料完成后预计桶皮重 | Before charging/ 投料前 |  | After charging/ 投料后 | Net weight/ 净重 | Performed by & Date/ 执行人及日期 | Checked by & Date/ 核实人及日期 |
| Full drum整桶 Non-full drum /非整桶 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Full drum整桶 Non-full drum /非整桶 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Total/合计 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Methanol addition/加甲醇 |  |  | Begin/开始： |  |  |  |  |  | End/结束： |  |  |  |  |
| Transcribed by & Date/转录人及日期： |  |  |  |  |  |  |  |  |  |  |  |  |  |

| 3.4. Capping/封端 |  |  |  |
| --- | --- | --- | --- |
| Adjust the synthesis reactor (TJ4S-1213-Sy15) mixture to 20±5 ℃ (target: 20 ℃). /调温合成釜(TJ4S-1213-Sy15)体系温度至20±5 ℃ (目标温度：20 ℃) |  |  |  |
| Adjust/调温 | Performed by & Date/执行人及日期： |  |  |
| Adjust/调温 | Begin/开始： | End/结束： |  |
| Transcribed by & Date /转录人及日期： |  |  |  |
| CAUTION/注意：DIPEA/N,N-二异丙基乙胺：It can enter the body through inhalation, ingestion and skin absorption. It is strongly irritating to the eyes, skin, mucous membranes and upper respiratory tract. Inhalation of high-concentration vapors may cause inflammation, edema and spasm of the larynx/bronchi, even chemical pneumonia and pulmonary edema. Exposure may cause symptoms such as burning sensation, cough, headache, nausea and vomiting. /可通过吸入、食入、皮肤吸收侵入人体，对眼、皮肤、黏膜和上呼吸道有强刺激性，吸入高浓度蒸气会引发喉/支气管炎症水肿、痉挛，甚至化学性肺炎、肺水肿，接触后会出现烧灼感、咳嗽、头痛、恶心呕吐等症状。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |
| 3.4.3 DIPEA addition /加N,N-二异丙基乙胺 Add DIPEA (5.00 eq.) through stainless steel hose and peristaltic pump into synthesis reactor (TJ4S-1213-Sy15). DIPEA addition at a reference time 20±10 min (target：20 min) at【 20±5 ℃ (target: 20 ℃)】. 控温【20±5 ℃ (target：20 ℃)】，通过不锈钢软管，蠕动泵，向合成釜（TJ4S-1213-Sy15）加入 kg N,N-二异丙基乙胺（Acceptable weighing range/可接受称量范围： kg ~ kg）（5.00 eq.），以控温为主，参考加料时间20±10 min（目标时间：20 min）。 |  |  | Times/次数 |
|  |  |  | 1 |

| 3.4. Capping/封端 |  |  |
| --- | --- | --- |
| Before material charging, confirm the material is within the retest date/ the expiry date. Confirm whether the weight is adequate for use in the current charging. /加物料前，执行物料在复检日期/有效期之内，确认重量是否满足本次投料使用。 Checked by & Date/核实人及日期： |  |  |
| Before drummed liquid material charging, visual check there is no other extraneous matter in material. /加桶装液体物料前，应目视执行桶内无异物。 Visual Examination of Foreign Matters for Drum Solvent/桶装溶剂异物目视检查: 1. Place the solvent drum in a well-lit area (or use a portable light source) to ensure adequate visibility of the liquid contents./ 将溶剂桶置于光线充足区域（或使用便携式光源），确保液体内容物可见性良好 2. Visually inspect the solvent through the drum bung opening or transparent sight glass for any visible foreign matters, including but not limited to particulates, fibers, sediments, or unusual turbidity./ 通过桶口或透明视镜目视检查溶剂中是否有可见异物，包括但不限于颗粒、纤维、沉淀物或异常浑浊。 3. Confirm that the solvent is clear and free of visible foreign matters before charging. If any foreign matter is observed, do not use the drum and follow the applicable handling procedure for subsequent disposition./ 投料前确认溶剂澄清且无可见异物。如发现任何异物，不得使用该桶，并依据处理流程进行后续处置。 Checked by & Date/核实人及日期： |  |  |
| Confirm liquid charging pipeline is released. /执行加料管路经过放行。 |  |  |
| Checked by & Date/核实人及日期： |  |  |
| Pump number/泵编号： Weighing of DIPEA/N,N-二异丙基乙胺称重： Control Number/内部控制号：YL000555 Scale ID. No./秤编号： Material charging pipe number/物料管路编号： |  |  |
| Internal temperature/内部温度 | High values/最高值： | Low values/最低值： |
| Transcribed by & Date/转录人及日期： |  |  |
| If the target net weight≥the planned net weight, perform full drum charge; otherwise, perform partial (Non-full drum) charge. /当目标加料量≥计划加入净重，勾选进行整桶加料；否则勾选进行零头加料。 Note: Target net weight to be charged = Total target net weight – Total net weight charged. Expected tare weight of drum after charge = Gross weight of drum that to be charged - Target net weight to be charged. 还需加入净重目标值= 加料步骤目标总净重-已加入净重总量 加料完成后预计桶皮重=待加料桶毛重-还需加入净重目标值 |  |  |

| 3.4. Capping/封端 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Addition type加料式 | Lot number/批号 | Pre-charging qty calculation/投料前投料量计算 |  |  |  |  |  | Actual Charging Qty/实际投料量 |  |  |  |  |  |
|  |  | Total qty charged/已加入净重总量 |  | Target qty to be charged/还需加入净重目标值 | Gross qty of drum that to be charged/待加料桶毛重 | Expected net weight of drum after charge /加料完成后预计桶净重 | Expected tare weight of drum after charge/加料完成后预计桶皮重 | Before charging/ 投料前 |  | After charging/ 投料后 | Net weight/ 净重 | Performed by & Date/ 执行人及日期 | Checked by & Date/ 核实人及日期 |
| Full drum整桶 Non-full drum /非整桶 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Full drum整桶 Non-full drum /非整桶 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Total/合计 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DIPEA Addition/加N,N-二异丙基乙胺 |  |  | Begin/开始： |  |  |  |  |  | End/结束： |  |  |  |  |
| Transcribed by & Date/转录人及日期： |  |  |  |  |  |  |  |  |  |  |  |  |  |

| 3.4. Capping/封端 |  |  |  |
| --- | --- | --- | --- |
| 3.4.4 Adjust temperature. /调温 Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h). Synthesis reactor (TJ4S-1213-Sy15) is bubble with nitrogen after addition. Adjust to【25±5 ℃ (target: 25 ℃)】. 加入完毕后开启合成釜（TJ4S-1213-Sy15）下口鼓氮气，氮气流量开至5±2 m³/h (目标流量: 5 m³/h).，调温至【25±5 ℃(目标：25 ℃)】。 |  |  | Times/次数 |
|  |  |  | 1 |
| Over-temperature warning value/超温预警设置数值： Lower limit/低限: 22 oC ，High limit/高限: 28 °C |  |  |  |
| Bubble/鼓氮气 | Performed by & Date/执行人及日期： |  |  |
| Bubble/鼓氮气 | Begin/开始： |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |
| Adjust/调温 | Performed by & Date/执行人及日期： |  |  |
| Adjust/调温 | Begin/开始： | End/结束： |  |
| Transcribed by & Date /转录人及日期： |  |  |  |
| 3.4.5 Capping /封端 The mixture is allowed to react for【1 h±15 min (target: 1 h)】at【25±5 ℃ (target: 25 ℃)】. Stop stirring and turn off the nitrogen. Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point. 控温【25±5 ℃(目标：25 ℃)，反应【1 h±15 min(目标1 h)】。停止搅拌，关闭氮气，通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  | Times/次数 |
|  |  |  | 1 |
| React/反应&Stir /搅拌 | Performed by & Date/执行人及日期： |  |  |

| 3.4. Capping/封端 |  |  |  |  |
| --- | --- | --- | --- | --- |
| React/反应&Stir /搅拌 | End/结束： |  |  |  |
|  | The start time is documented in Ins3.4.1 /开始时间记录于3.4.1 |  |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |
| Bubble/鼓氮气 | Performed by & Date/执行人及日期： |  |  |  |
| Bubble/鼓氮气 | End/结束： |  |  |  |
|  | The start time is documented in Ins3.4.1 /开始时间记录于3.4.1 |  |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |
| Internal temperature/温度 | High values/最高值： |  | Low values/最低值： |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |
| Filter/过滤 | Performed by & Date/执行人及日期： |  |  |
|  | Checked by & Date/核实人及日期： |  |  |
|  | Begin/开始： | End/结束： |  |

| 3.4. Capping/封端 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| DMF/N，N-二甲基甲酰胺：Irritation of eyes, skin and nose. May cause nausea. /刺激眼睛、皮肤和鼻子。可能引起恶心。 |  |  |  |  |  |
| 3.4.6 Activation Tank Rinse/激活罐洗涤 Add 409.54 kg (Acceptable weighing range：397.25 kg ~421.83 kg) (9 V, 8.532 kg/kg) DMF into the activation tank (TJ4S-1213-Sy15-R01) by spray ball. Circulate and clean the activating liquid transfer pipeline 1, solid charging port cleaning pipeline 2 and liquid charging port cleaning pipeline 3, clean every pipeline for 2±1 min. 通过淋洗球向激活罐（TJ4S-1213-Sy15-R01）中加入409.54 kg（可接受称量范围：397.25 kg ~ 421.83 kg） (9 V, 8.532 kg/kg) N,N-二甲基甲酰胺，循环清洗激活液转移管路1，固体加料口清洗管路2和液体加料口清洗管路3，每条线路各清洗2±1 min。 |  |  |  |  | Times/次数 |
|  |  |  |  |  | 3 |
| ★Note: DMF quantity is a CPP./备注：N, N-二甲基甲酰胺用量是关键工艺参数。 The operator shall confirm that DMF quantity is within the Normal Operating Range (NOR) 【397.25~421.83 kg】/操作人员须确认N, N-二甲基甲酰胺活化总用量处于正常操作范围（NOR）内：【397.25~421.83kg】 ★After coupling is completed, the operator shall confirm that all Critical Process Parameters (CPPs) are within their respective Normal Operating Ranges (NOR)/ 偶联结束后，操作人员须确认所有关键工艺参数（CPP）均处于各自的正常操作范围（NOR）内 Checked by & Date/核实人及日期： |  |  |  |  |  |
| Operation name/ 操作名称 | Time point/时间节点 | Clean the activating liquid transfer pipeline 1/循环清洗激活液转移管路1 | Solid charging port cleaning pipeline /固体加料口清洗管路2 | liquid charging port cleaning pipeline 3,/液体加料口清洗管路3 |  |
|  |  | Wash /洗涤1 | Wash /洗涤2 | Wash /洗涤3 |  |
| DMF addition/ 加N,N-二甲基甲酰胺 | Performed by & Date/执行人及日期： |  |  |  |  |
| DMF addition/ 加N,N-二甲基甲酰胺 | Begin/开始 |  |  |  |  |
|  | End/结束 |  |  |  |  |
|  | Qty/重量 |  |  |  |  |
|  | Transcribed by & Date /转录人及日期： |  |  |  |  |
| Circulate and clean/循环清洗 | Performed by & Date/执行人及日期： |  |  |  |  |
| Circulate and clean/循环清洗 | Begin/开始 |  |  |  |  |
|  | End/结束 |  |  |  |  |
|  | Transcribed by & Date /转录人及日期： |  |  |  |  |

| 3.4. Capping/封端 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 3.4.7 Post-capping wash./封端后洗涤 Transfer the mixture in activation tank (TJ4S-1213-Sy15-R01) to synthesis reactor (TJ4S-1213-Sy15). Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h). Bubble with nitrogen, after 5±2 min (target 5 min), start the agitator. After stirring for 7±2 min (target：7 min), stop stirring, turn off the nitrogen. Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point. 将激活罐（TJ4S-1213-Sy15-R01）洗涤液转入合成釜 (TJ4S-1213-Sy15)。开启合成釜下口鼓氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min (目标：5 min)后开启搅拌，搅拌7±2 min（目标：7 min）后停止搅拌，关闭氮气。通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  |  |  | Times/次数 |
|  |  |  |  |  | 3 |
| Ttransfer the liquid to the synthesis reactor (TJ4S-1213-Sy15) ./将激活罐中体系转移至合成釜(TJ4S-1213-Sy15)中。 |  |  |  |  |  |
| Operation name/ 操作名称 | Time point/时间节点 | Wash /洗涤1 | Wash /洗涤2 | Wash /洗涤3 |  |
| Transfer/转移 | Performed by & Date/执行人及日期： |  |  |  |  |
|  | Checked by & Date/核实人及日期 |  |  |  |  |
|  | Begin/开始 |  |  |  |  |
|  | End/结束 |  |  |  |  |

| 3.4. Capping/封端 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h), Bubble with nitrogen for 5±2 min (target:5 min), Start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm).After stirring for 5±2 min (target: 5 min), stop stirring, turn off the nitrogen./ 开启合成釜下口鼓氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min（目标时间：5 min）后开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm)，搅拌10±2 min（目标时间：10 min）后，停止搅拌，关闭氮气. |  |  |  |  |
| Operation name/ 操作名称 | Time point/时间节点 | Wash /洗涤1 | Wash /洗涤2 | Wash /洗涤3 |
| Bubble/鼓氮气 | Performed by & Date/执行人及日期： |  |  |  |
| Bubble/鼓氮气 | Begin/开始 |  |  |  |
|  | End/结束 |  |  |  |
|  | Transcribed by & Date /转录人及日期： |  |  |  |
| Stir /搅拌 | Performed by & Date/执行人及日期： |  |  |  |
| Stir /搅拌 | Begin/开始 |  |  |  |
|  | End/结束 |  |  |  |
|  | Transcribed by & Date /转录人及日期： |  |  |  |
| Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point. / 通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  |  |  |
| Operation name/ 操作名称 | Time point/时间节点 | Wash /洗涤1 | Wash /洗涤2 | Wash /洗涤3 |
| Filter/过滤 | Performed by & Date/ 执行人及日期： |  |  |  |
|  | Checked by & Date/ 核实人及日期： |  |  |  |
|  | Begin/开始 |  |  |  |
|  | End/结束 |  |  |  |

| - （2）Deprotection/脱保护 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| CAUTION/注意：Piperidine哌啶：Strong local irritant and may cause permanent injury after short exposure to small amounts. /强烈的局部刺激，短时间少量接触可能造成永久性伤害。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |  |
| 3.5.1 Piperidine soulution addition /加哌啶溶液 Adjust the jacket temperature of synthesis reactor (TJ4S-1213-Sy15) to 【25±5oC (target: 25oC).】. Add 357.50 kg (Acceptable weighing range: 346.77 kg ~ 368.23 kg) (8 V)【20% (V/V) piperidine/DMF solution】 into the Synthesis reactor. 将合成釜(TJ4S-1213-Sy15)夹套调温至【25±5℃（目标温度：25℃）】。向合成釜中加入357.50 kg（可接受称量范围：346.77 kg ~ 368.23 kg）(8 V)【20%哌啶/DMF (V/V)】溶液。 |  |  |  |  |  | Times/次数 |
|  |  |  |  |  |  | 1 |
| Adjust/调温 |  | Performed by & Date/执行人及日期： |  |  |  |  |
| Adjust/调温 |  | Begin/开始： |  | Adjust/调温 |  |  |
| Transcribed by & Date/转录人及日期： |  |  |  |  |  |  |
| Confirm and record that the 20%Piperidine solution concentration is within the NOR range./ 确认并记录20%哌啶溶液浓度在正常操作范围内：17.50%~22.50% 20%Piperidine solution concentration confirmation/20%哌啶溶液浓度确认：____________ Confirm and record that the 20%Piperidine solution temperature is within the NOR range./ 确认并记录20%哌啶溶液温度在正常操作范围内：20 oC ~30 oC 20%Piperidine solution temperature confirmation/20%哌啶溶液温度确认：____________ Confirmed by & Date/确认人及日期： Checked by & Date/核实人及日期： |  |  |  |  |  |  |
| Piperidine addition/加哌啶溶液 | Performed by & Date/执行人及日期： |  |  |  |  |  |
| Piperidine addition/ 加哌啶溶液 | Begin/开始： |  | End/结束： |  | Qty/重量： |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |

| 3.5. （2）Deprotection/脱保护 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 3.5.2 Adjust temperature /调温 Turn on the nitrogen of Synthesis reactor (TJ4S-1213-Sy15) to 5±2 m³/h (target: 5 m³/h). Bubble with nitrogen for 5±2 min (target 5 min). Start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm). then start the stirrer, Adjust to 25±5 ℃ (target: 25 ℃）. 开启合成釜（TJ4S-1213-Sy15）下口鼓氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min (目标时间：5 min)后开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm)，调温至【25±5 ℃(目标温度：25 ℃)】。 |  |  |  | Times/次数 |
|  |  |  |  | 1 |
| Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h). Bubble with nitrogen for 5±2 min (target :5 min) 开启合成釜下口鼓氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min (目标时间：5 min) |  |  |  |  |
| Bubble/鼓氮气 | Performed by & Date/执行人及日期： |  |  |  |
| Bubble/鼓氮气 | Begin/开始： |  |  |  |
|  | The end time is documented in Ins 3.5.3/结束时间记录于3.5.3 |  |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |
| Start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm). /开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm) |  |  |  |  |
| Stir /搅拌 | Performed by & Date/执行人及日期： |  |  |  |
| Stir /搅拌 | Begin/开始： |  |  |  |
|  | The end time is documented in Ins 3.5.3/结束时间记录于3.5.3 |  |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |
| Adjust to 25±5 oC (target: 25 oC)./ 调温至25±5 ℃(目标温度：25 oC) |  |  |  |  |
| Adjust/调温 |  | Performed by & Date/执行人及日期： |  |  |
| Adjust/调温 |  | Begin/开始： | End/结束： |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |

| 3.5. （2）Deprotection/脱保护 |  |  |
| --- | --- | --- |
| 3.5.3 Fmoc-L-Phe-OH De-Fmoc/芴甲氧羰基-L-苯丙氨酸脱保护 After stirring for 2 h±15 min (target: 2 h) at 【25±5 ℃ (target: 25 ℃)】, stop stirring, turn off the nitrogen. Filter until nearly no liquid outflow. Start draining the solution from synthesis reactor with diaphragmgm pump. Drain until there is no drip, the operator visually confirms the draining end point. 控温【25±5 ℃(目标温度：25 ℃)】， 搅拌2 h±15 min（目标时间：2 h）后，停止搅拌，关闭氮气，滤除至基本通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 Over-temperature warning value/超温预警设置数值： Lower limit/低限: 22 oC ，High limit/高限: 28 oC |  | Times/次数 |
|  |  | 1 |
| ★Note: Deprotection temperature is a CPP./备注：脱保护的温度是关键工艺参数。 ★Confirm and record that the 20%Piperidine solution temperature is within the NOR range 20 oC ~30 oC ./ 确认并记录20%哌啶溶液温度在正常操作范围内：20 oC ~30 oC。 20%Piperidine solution temperature confirmation/20%哌啶溶液温度确认：____________ Confirmed by& Date/确认人及日期： Checked by & Date/核实人及日期： |  |  |
| De-Fmoc/脱Fmoc &Stir /搅拌 | Performed by & Date/执行人及日期： |  |
| De-Fmoc/脱Fmoc &Stir /搅拌 | End/结束： |  |
|  | The start time is documented in Ins3.5.2 /开始时间记录于3.5.2 |  |
| Transcribed by & Date /转录人及日期： |  |  |

| 3.5. （2）Deprotection/脱保护 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Bubble/鼓氮气 | Performed by & Date/执行人及日期： |  |  |  |
| Bubble/鼓氮气 | End/结束： |  |  |  |
|  | The start time is documented in Ins3.5.2 /开始时间记录于3.5.2 |  |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |
| Internal temperature/内部温度 |  | High values/最高值： |  | Low values/最低值： |
| Transcribed by & Date /转录人及日期： |  |  |  |  |
| Filter/过滤 |  | Performed by & Date/执行人及日期： |  |  |
|  |  | Checked by & Date/核实人及日期： |  |  |
|  |  | Begin/开始： | End/结束： |  |
| ★After De-Fmoc is completed, the operator shall confirm that all Critical Process Parameters (CPPs) are within their respective Normal Operating Ranges (NOR)/ 脱保护结束后，操作人员须确认所有关键工艺参数（CPP）均处于各自的正常操作范围（NOR）内 Checked by & Date/核实人及日期： |  |  |  |  |

| 3.5. （2）Deprotection/脱保护 |  |  |  |
| --- | --- | --- | --- |
| 3.5.4 Adjust synthesis reactor temperature /合成釜调温 Adjust the synthesis reactor (TJ4S-1213-Sy15) jacket to 【20±5 ℃ (target: 20 ℃)】. /将合成釜（TJ4S-1213-Sy15）夹套调温至【20±5 ℃（目标温度：20 ℃）】。 |  |  | Times/次数 |
|  |  |  | 1 |
| Adjust/调温 | Performed by & Date/执行人及日期： |  |  |
| Adjust/调温 | Begin/开始： | End/结束： |  |
| Transcribed by & Date /转录人及日期： |  |  |  |
| CAUTION/注意：IPAC/乙酸异丙酯： Vapors irritate eyes and respiratory tract; high concentrations can be anesthetic. Liquid irritates eyes but causes no serious injury; may cause dermatitis; no serious effects if swallowed. /蒸汽会刺激眼睛和呼吸道；高浓度的蒸汽可能是麻醉剂。液体刺激眼睛，但不会造成严重伤害；可能会引起皮炎；吞食不会造成严重影响。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |
| 3.5.5 Post-deprotection wash/脱保护后洗涤 Add 445.92 kg (Acceptable weighing range：432.54 ~ 459.30 kg) (9.29 kg/kg, 10.64 V) isopropyl acetate into synthesis reactor (TJ4S-1213-Sy15). Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h). Bubble with nitrogen for 5±2 min (target 5 min). Start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm). After stirring for 5±2 min (target 5 min), stop stirring, turn off the nitrogen. Start draining the solution from synthesis reactor with diaphragmgm pump. Drain until there is no drip, the operator visually confirms the draining end point. 向合成釜（TJ4S-1213-Sy15）中加入445.92 kg（可接受称量范围：432.54 ~ 459.30 kg）（9.29 kg/kg, 10.64 V）乙酸异丙酯，开启下口氮气，氮气流速开至5±2m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min (目标时间：5 min)后开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm)，搅拌5±2 min (目标时间：5 min)后，停止搅拌，关闭氮气。通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  | Times/次数 |
|  |  |  | 4 |
| ★Note: IPAC quantity is a CPP./备注：乙酸异丙酯用量是关键工艺参数。 ★The operator shall confirm that IPAC quantity is within the Normal Operating Range (NOR) 【432.54 kg~459.30 kg】/操作人员须确认乙酸异丙酯用量处于正常操作范围（NOR）内：【432.54 kg~459.30 kg】 Checked by & Date/核实人及日期： |  |  |  |

| 3.5. （2）Deprotection/脱保护 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Operation name/操作名称 | Time point/时间节点 | Wash /洗涤1 | Wash /洗涤2 | Wash /洗涤3 | Wash /洗涤4 |
| Add 419.04 kg IPAc (10 V) by spray balls into the synthesis reactor TJ4S-1213-Sy15./ 向合成釜TJ4S-1213-Sy15 中加419.04kg乙酸异丙酯（10 V)（Acceptable weighing range/可接受称量范围：406.47~431.61 kg） |  |  |  |  |  |
| IPAC addition/加乙酸异丙酯 | Performed by & Date/执行人及日期： |  |  |  |  |
| IPAC addition/加乙酸异丙酯 | Begin/开始 |  |  |  |  |
|  | End/结束 |  |  |  |  |
|  | Qty/重量 |  |  |  |  |
|  | Transcribed by & Date /转录人及日期: |  |  |  |  |
| Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h) Bubble with nitrogen for 5±2 min (target: 5 min). Start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm). Stir for 5±2 min (target: 5 min), stop stirring, turn off the nitrogen./ 开启合成釜下口鼓氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min（目标时间：5 min）后开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm)，搅拌5±2 min（目标时间：5 min）后，停止搅拌，关闭氮气。 |  |  |  |  |  |
| Bubble/鼓氮气 | Performed by & Date/执行人及日期： |  |  |  |  |
| Bubble/鼓氮气 | Begin/开始 |  |  |  |  |
|  | End/结束 |  |  |  |  |
|  | Transcribed by & Date /转录人及日期: |  |  |  |  |
| Stir /搅拌 | Performed by & Date/执行人及日期： |  |  |  |  |
| Stir /搅拌 | Begin/开始 |  |  |  |  |
|  | End/结束 |  |  |  |  |
|  | Transcribed by & Date/转录人及日期 |  |  |  |  |

| 3.5. （2）Deprotection/脱保护 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point./通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Filter/过滤 |  | Performed by & Date/ 执行人及日期： |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Checked by & Date/ 核实人及日期： |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Begin/开始 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | End/结束 |  |  |  |  |  |  |  |  |  |  |  |
| CAUTION/注意：DMF/N，N-二甲基甲酰胺：Irritation of eyes, skin and nose. May cause nausea. /刺激眼睛、皮肤和鼻子。可能引起恶心。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3.5.6 DMF addition/加N,N-二甲基甲酰胺 Add 546.05 kg (Acceptable weighing range: 529.67 kg ~ 562.43 kg) (12 V) DMF into synthesis reactor (TJ4S-1213-Sy15). 向合成釜（TJ4S-1213-Sy15）中加入546.05 kg（可接受称量范围：529.67 kg ~ 562.43 kg）（12 V）N,N-二甲基甲酰胺。 |  |  |  |  |  |  |  |  |  |  |  | Times/次数 |  |
|  |  |  |  |  |  |  |  |  |  |  |  | 1 |  |
| DMF addition / 加N,N-二甲基甲酰胺 |  | Performed by & Date/执行人及日期： |  |  |  |  |  |  |  |  |  |  |  |
| DMF addition / 加N,N-二甲基甲酰胺 |  | Begin/开始： |  |  | End/结束： |  |  |  |  | Qty/重量： |  |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3.5.7 Resin reswelling wash 1/溶胀后洗涤1 Turn on the nitrogen of synthesis reactor(TJ4S-1213-Sy15) to 5±2 m³/h (target: 5 m³/h). Bubble with nitrogen for 5±2 min (target: 5 min). Start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm). After stirring for 10±2 min (target: 10 min), stop stirring, turn off the nitrogen. Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point. 开启合成釜（TJ4S-1213-Sy15）下口氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min (目标时间：5 min)后，开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm)，搅拌10±2 min（目标时间：10 min）后，停止搅拌，关闭氮气。通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  |  |  |  |  |  |  |  |  |  | Times/次数 |  |
|  |  |  |  |  |  |  |  |  |  |  |  | 1 |  |
| 3.5. （2）Deprotection/脱保护 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h) for 5±2 min (target: 5 min), Start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm). Stir for 10 ±2min (target: 10 min). Stop stirring, turn off the nitrogen./ 开启合成釜下口鼓氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min（目标时间：5 min）后，开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm)，搅拌10±2 min（目标时间:10 min）后，停止搅拌，关闭氮气. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bubble/鼓氮气 |  |  |  | Performed by & Date/执行人及日期： |  |  |  |  |  |  |  |  |  |
| Bubble/鼓氮气 |  |  |  | Begin/开始： |  |  |  |  | End/结束： |  |  |  |  |
| Transcribed by & Date /转录人及日期: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Stir /搅拌 |  |  |  | Performed by & Date/执行人及日期： |  |  |  |  |  |  |  |  |  |
| Stir /搅拌 |  |  |  | Begin/开始： |  |  |  |  | End/结束： |  |  |  |  |
| Transcribed by & Date /转录人及日期: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point./通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Filter/过滤 |  |  |  | Performed by & Date/执行人及日期： |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Checked by & Date/核实人及日期： |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Begin/开始： |  |  |  | End/结束： |  |  |  |  |  |

| 3.5. （2）Deprotection/脱保护 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| CAUTION/注意：DMF/N，N-二甲基甲酰胺：Irritation of eyes, skin and nose. May cause nausea. /刺激眼睛、皮肤和鼻子。可能引起恶心。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |  |
| 3.5.8 DMF addition/加N,N-二甲基甲酰胺 Add 546.05 kg (Acceptable weighing range: 529.67 kg ~ 562.43 kg) (12 V) DMF into synthesis reactor (TJ4S-1213-Sy15). 向合成釜（TJ4S-1213-Sy15）中加入546.05 kg（可接受称量范围：529.67 kg ~ 562.43 kg）（12 V）N,N-二甲基甲酰胺。 |  |  |  |  |  | Times/次数 |
|  |  |  |  |  |  | 1 |
| DMF addition / 加N,N-二甲基甲酰胺 | Performed by & Date/执行人及日期： |  |  |  |  |  |
| DMF addition / 加N,N-二甲基甲酰胺 | Begin/开始： |  | End/结束： |  | Qty/重量： |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |
| 3.5.9 Resin reswelling wash 1/溶胀后洗涤2 Turn on the nitrogen of synthesis reactor (TJ4S-1213-Sy15) to 5±2 m³/h (target: 5 m³/h). Bubble with nitrogen for 5±2 min (target: 5 min). Start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm). After stirring for 10±2 min (target: 10 min), stop stirring, turn off the nitrogen. Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point. 开启合成釜（TJ4S-1213-Sy15）下口氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min (目标时间：5 min)后，开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm)，搅拌10±2 min（目标时间：10 min）后，停止搅拌，关闭氮气。通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  |  |  |  | Times/次数 |
|  |  |  |  |  |  | 1 |
| Bubble/鼓氮气 |  | Performed by & Date/执行人及日期： |  |  |  |  |
| Bubble/鼓氮气 |  | Begin/开始： |  | End/结束： |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |

| 3.5. （2）Deprotection/脱保护 |  |  |  |
| --- | --- | --- | --- |
| Stir /搅拌 | Performed by & Date/执行人及日期： |  |  |
| Stir /搅拌 | Begin/开始： |  | End/结束： |
| Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point./通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  |  |
| Filter/过滤 | Performed by & Date/执行人及日期： |  |  |
|  | Checked by & Date/核实人及日期： |  |  |
|  | Begin/开始： | End/结束： |  |

| - （2）Fmoc-L-Phe-OH activation/芴甲氧羰基-L-苯丙氨酸活化 |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CAUTION/注意：DMF/N，N-二甲基甲酰胺：Irritation of eyes, skin and nose. May cause nausea. /刺激眼睛、皮肤和鼻子。可能引起恶心。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |  |  |  |  |  |  |  |
| 3.6.1 Add DMF to the activation reactor/激活罐加N,N-二甲基甲酰胺 Add 182.02 kg (Acceptable weighing range: 176.56 kg ~ 187.48 kg) (4 V) DMF into activation tank (TJ4S-1213-Sy15-R01). Start the agitator of activation tank to 95±10 rpm (target: 95 rpm). 向激活罐（TJ4S-1213-Sy15-R01）中加182.02 kg（可接受称量范围：176.56 kg ~ 187.48 kg ）（4 V）N，N-二甲基甲酰胺。开启激活罐搅拌转速至95±10 rpm (目标转速: 95 rpm)。 |  |  |  |  |  |  |  |  |  |  | Times/次数 |  |
|  |  |  |  |  |  |  |  |  |  |  | 1 |  |
| Add 182.02kg DMF (Acceptable weighing range: 176.56 ~ 187.48 kg) (4 V) by spray balls into activation tank (TJ4S-1213-Sy15-R01).通过淋洗球向激活罐(TJ4S-1213-Sy15-R01)中加入182.02 kg N, N-二甲基甲酰胺（可接受称量范围：176.56~187.48 kg）(4 V)。 |  |  |  |  |  |  |  |  |  |  |  |  |
| DMF addition / 加N,N-二甲基甲酰胺 |  |  | Performed by & Date/执行人及日期： |  |  |  |  |  |  |  |  |  |
| DMF addition / 加N,N-二甲基甲酰胺 |  |  | Begin/开始： |  |  | End/结束： |  |  | Qty/重量： |  |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |  |  |  |  |  |
| Start the agitator of activation tank to 95±10 rpm (target: 95 rpm)./ 开启激活罐搅拌转速至95±10 rpm (目标转速: 95 rpm)。 |  |  |  |  |  |  |  |  |  |  |  |  |
| Stir /搅拌 |  | Performed by & Date/执行人及日期： |  |  |  |  |  |  |  |  |  |  |
| Stir /搅拌 |  | Begin/开始： |  |  |  |  |  |  |  |  |  |  |
|  |  | The end time is documented in Ins 3.6.9/结束时间记录于3.6.9 |  |  |  |  |  |  |  |  |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |  |  |  |  |  |
| 3.6.2 Fmoc-L-Phe-OH addition/加芴甲氧羰基-L-苯丙氨酸 Add Fmoc-L-Phe-OH (2.00 eq.) with powder feeding bags into activation tank (TJ4S-1213-Sy15-R01).通过投料袋向激活罐（TJ4S-1213-Sy15-R01）中加入 kg芴甲氧羰基-L-苯丙氨酸（Acceptable weighing range/可接受称量范围： kg ~ kg）(2.00 eq.)。 |  |  |  |  |  |  |  |  |  |  | Times/次数 |  |
|  |  |  |  |  |  |  |  |  |  |  | 1 |  |
| Before material charging, confirm the material is within the retest date/ the expiry date,Confirm whether the weight is adequate for use in the current charging. /加物料前，执行物料在复检日期/有效期之内，确认重量是否满足本次投料使用。 Before solid material charging, confirm/加固体物料前，确认： - Before performed, confirm there is no non-related material / equipment / instrument / tools / pipeline / document / record / label in the intended area /操作前，执行目标区域无不相关物料/设备/仪器/工具/管路/文件/记录/标签 - The outer packages are removed. If the outer package cannot be removed, but un-package material without damaging the package bag, and before charging, check no package fragment and other extraneous matter in material. /外包装移除；如果外包装不能移除，但在不破坏包装的情况下打开包装袋，投料前以及过程中同时检查执行物料中无包装碎片以及异物 Checked by & Date/核实人及日期： |  |  |  |  |  |  |  |  |  |  |  |  |
| 3.6. （2）Fmoc-L-Phe-OH activation/芴甲氧羰基-L-苯丙氨酸活化 |  |  |  |  |  |  |  |  |  |  |  |  |
| Prior to charging each amino acid, the operator shall perform a Raman scan to confirm the identity of the material. The Raman scan result shall be compared against the reference spectrum of the corresponding amino acid. Only materials that conform to the reference spectrum may be charged. If the Raman scan result does not conform, stop the operation, quarantine the material, and initiate a investigation per site SOP. Record the Raman scan result, operator signature, and date/time in the table below./ 每次投氨基酸前，操作人员应进行拉曼扫描以执行物料。拉曼扫描结果应与相应氨基酸的参考光谱进行比对。仅与参考光谱相符的物料方可投料。如拉曼扫描结果不相符，应停止操作，对物料进行隔离，并依据厂区 SOP 启动异常事件调查。在下表中记录拉曼扫描结果、操作人员签字及日期 / 时间。 |  |  |  |  |  |  |  |  |  |  |  |  |
| Charge Fmoc-L-Phe-OH /加芴甲氧羰基-L-苯丙氨酸 |  |  |  |  | Performed by & Date/执行人及日期： |  |  |  |  |  |  |  |
|  |  |  |  |  | Begin/开始： |  |  | End/结束： |  |  |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |  |  |  |  |  |
| Fmoc-L-Phe-OH confirmation/芴甲氧羰基-L-苯丙氨酸确认：Control No./内部控制号：YL004840 |  |  |  |  |  |  |  |  |  |  |  |  |
| Check items/检查项 |  | Is it correct?/是否正确 |  |  |  | Recorded by & Date/ 记录人及日期 |  |  | Checked by & Date/ 核实人及日期 |  |  |  |
| Material name/物料名称 |  | Yes/是；No/否 |  |  |  |  |  |  |  |  |  |  |
| Project batch number/ 项目批次号 |  | Yes/是；No/否 |  |  |  |  |  |  |  |  |  |  |
| Operation number/ 操作号 |  | Yes/是；No/否 |  |  |  |  |  |  |  |  |  |  |
| Control No./内部控制号 |  | Yes/是；No/否 |  |  |  |  |  |  |  |  |  |  |
| Raman Scan Confirmation/拉曼扫描执行 |  | Conforms/相符 Does not conform/不相符 |  |  |  |  |  |  |  |  |  |  |

| 3.6. （2）Fmoc-L-Phe-OH activation/芴甲氧羰基-L-苯丙氨酸活化 |  |  |  |  |
| --- | --- | --- | --- | --- |
| No. /编号 | Net weight/净重 | All in/全部加入 | Recorded by & Date/ 记录人及日期 | Checked by & Date/ 核实人及日期 |
| / |  | Yes/是；No/否 |  |  |
| / |  | Yes/是；No/否 |  |  |
| / |  | Yes/是；No/否 |  |  |
| / |  | Yes/是；No/否 |  |  |
| / |  | Yes/是；No/否 |  |  |
| / |  | Yes/是；No/否 |  |  |
| / |  | Yes/是；No/否 |  |  |
| / |  | Yes/是；No/否 |  |  |
| / |  | Yes/是；No/否 |  |  |

| 3.6. （2）Fmoc-L-Phe-OH activation/芴甲氧羰基-L-苯丙氨酸活化 |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No. /编号 |  | Net weight/净重 |  | All in/全部加入 |  |  | Recorded by & Date/ 记录人及日期 |  | Checked by & Date/ 核实人及日期 |  |  |
| / |  |  |  | Yes/是；No/否 |  |  |  |  |  |  |  |
| / |  |  |  | Yes/是；No/否 |  |  |  |  |  |  |  |
| / |  |  |  | Yes/是；No/否 |  |  |  |  |  |  |  |
| / |  |  |  | Yes/是；No/否 |  |  |  |  |  |  |  |
| / |  |  |  | Yes/是；No/否 |  |  |  |  |  |  |  |
| / |  |  |  | Yes/是；No/否 |  |  |  |  |  |  |  |
| / |  |  |  | Yes/是；No/否 |  |  |  |  |  |  |  |
| / |  |  |  | Yes/是；No/否 |  |  |  |  |  |  |  |
| / |  |  |  | Yes/是；No/否 |  |  |  |  |  |  |  |
| Total/合计 |  |  |  | Confirm the total net weight is within the input range/执行合计净重在投料范围内 |  |  | Recorded by & Date/记录人及日期 |  | Checked by & Date/ 核实人及日期 |  |  |
|  |  |  |  | Yes/是；No/否 |  |  |  |  |  |  |  |
| After solid material charging, confirm/固体物料投料之后，确认： - The corresponding leftover material has been properly sealed and transferred to the staging area/相应的剩余物料已经恰当密封，转移到了暂存区域。 - The waste packaging material has been sealed and transferred to the corresponding waste staging area. /产生的包材废物已经密封并转移到了相应的废物暂存区域。 - The operation area/equipment surface has been cleaned/操作区域以及设备表面已经清洁。 Checked by & Date/核实人及日期： |  |  |  |  |  |  |  |  |  |  |  |
| 3.6.3 Agitation of activation tank /激活罐搅拌 Continue agitation of activation tank to 95±10 rpm (target: 95 rpm). Maintain stirring for NLT 15 min. 维持激活罐搅拌转速至95±10 rpm (目标转速: 95 rpm),继续搅拌至少15 min。 |  |  |  |  |  |  |  |  |  | Times/次数 |  |
|  |  |  |  |  |  |  |  |  |  | 1 |  |
| Maintain &Stir /保持搅拌 |  |  |  |  | Performed by & Date/执行人及日期： |  |  |  |  |  |  |
|  |  |  |  |  | Begin/开始： |  |  |  |  |  |  |
|  |  |  |  |  | The end time is documented in Ins 3.6.9/结束时间记录于3.6.9 |  |  |  |  |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |  |  |  |  |

| 3.6. （2）Fmoc-L-Phe-OH activation/芴甲氧羰基-L-苯丙氨酸活化 |  |  |  |  |
| --- | --- | --- | --- | --- |
| 3.6.4 Cool the activation tank /激活罐降温 Cool the activation tank (TJ4S-1213-Sy15-R01) mixture to 【5±5 ℃ (target: 5 ℃)】. /将激活罐(TJ4S-1213-Sy15-R01)中体系降温至【5±5 ℃（目标温度：5 ℃）】。 |  |  |  | Times/次数 |
|  |  |  |  | 1 |
| Cool/降温 |  | Performed by & Date/执行人及日期： |  |  |
|  |  | Begin/开始： | End/结束： |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |
| CAUTION/注意：PyBOP is high energy compounds, impact sensitive, skin sensitizing. /1H-苯并三唑-1-基氧三吡咯烷基六氟磷酸盐是高能化合物，撞击敏感性，皮肤致敏。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |
| 3.6.5 PyBOP addition /加 1H-苯并三唑-1-基氧三吡咯烷基六氟磷酸盐 Add PyBOP (2.00 eq.) with powder feeding bags into activation tank (TJ4S-1213-Sy15-R01) at 5±5 ℃ (target: 5 ℃. 控温5±5 ℃（目标温度：5 ℃），通过投料袋向激活罐（TJ4S-1213-Sy15-R01）中加入 kg 1H-苯并三唑-1-基氧三吡咯烷基六氟磷酸盐（Acceptable weighing range/可接受称量范围： kg ~ kg）(2.00 eq.)。 |  |  |  | Times/次数 |
|  |  |  |  | 1 |
| Check and record that the solid charge port is free of solid accumulation from the previous step./ 检查并记录固体投料口无来自上一步骤的固体残留。 Checked by & Date/核实人及日期： |  |  |  |  |
| Before material charging, confirm the material is within the retest date/ the expiry date, Confirm whether the weight is adequate for use in the current charging. /加物料前，执行物料在复检日期/有效期之内，确认重量是否满足本次投料使用。 Before solid material charging, confirm/加固体物料前，确认： - Before performed, confirm there is no non-related material / equipment / instrument / tools / pipeline / document / record / label in the intended area /操作前，执行目标区域无不相关物料/设备/仪器/工具/管路/文件/记录/标签 - The outer packages are removed. If the outer package cannot be removed, but un-package material without damaging the package bag, and before charging, check no package fragment and other extraneous matter in material. /外包装移除；如果外包装不能移除，但在不破坏包装的情况下打开包装袋，投料前以及过程中同时检查执行物料中无包装碎片以及异物 Checked by & Date/核实人及日期： |  |  |  |  |
| PyBOP addition/加1H-苯并三唑-1-基氧三吡咯烷基六氟磷酸盐 |  | Performed by & Date/执行人及日期： |  |  |
|  |  | Begin/开始： | End/结束： |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |

| 3.6. （2）Fmoc-L-Phe-OH activation/芴甲氧羰基-L-苯丙氨酸活化 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PyBOP confirmation/1H-苯并三唑-1-基氧三吡咯烷基六氟磷酸盐执行：Control No./内部控制号：YL002855 |  |  |  |  |  |  |  |
| Check items/检查项 |  | Is it correct?/是否正确 |  | Recorded by & Date/ 记录人及日期 |  | Checked by & Date/ 核实人及日期 |  |
| Material name/物料名称 |  | Yes/是；No/否 |  |  |  |  |  |
| Project batch number/ 项目批次号 |  | Yes/是；No/否 |  |  |  |  |  |
| Operation number/ 操作号 |  | Yes/是；No/否 |  |  |  |  |  |
| Control No./内部控制号 |  | Yes/是；No/否 |  |  |  |  |  |
| Raman Scan Confirmation/拉曼扫描执行 |  | Conforms/相符 Does not conform/不相符 |  |  |  |  |  |
| No. /编号 | Net weight/净重 | All in/全部加入 |  | Recorded by & Date/ 记录人及日期 |  | Checked by & Date/ 核实人及日期 |  |
| / |  | Yes/是；No/否 |  |  |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |  |
| Total/合计 |  | Confirm the total net weight is within the input range/执行合计净重在投料范围内 |  | Recorded by & Date/记录人及日期 |  | Checked by & Date/ 核实人及日期 |  |
|  |  | Yes/是；No/否 |  |  |  |  |  |
| 3.6. （2）Fmoc-L-Phe-OH activation/芴甲氧羰基-L-苯丙氨酸活化 |  |  |  |  |  |  |  |
| After solid material charging, confirm/固体物料投料之后，确认： - The corresponding leftover material has been properly sealed and transferred to the staging area/相应的剩余物料已经恰当密封，转移到了暂存区域 - The waste packaging material has been sealed and transferred to the corresponding waste staging area. /产生的包材废物已经密封并转移到了相应的废物暂存区域 - The operation area/equipment surface has been cleaned/操作区域以及设备表面已经清洁 Checked by & Date/核实人及日期： |  |  |  |  |  |  |  |
| Internal temperature/内部温度 |  |  | High values/最高值： |  | Low values/最低值： |  |  |
| Transcribed by & Date/转录人及日期： |  |  |  |  |  |  |  |
| CAUTION/注意：OxymaPure is highly toxic, high-energy compound. /2-肟氰乙酸乙酯是高毒，高能化合物。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |  |  |
| 3.6.6 OxymaPure addition/ 2-肟氰乙酸乙酯加料 Add OxymaPure (2.00 eq.) with powder feeding bags into activation tank (TJ4S-1213-Sy15-R01) at 【5±5 ℃ (target: 5 ℃】. 控温【5±5 ℃（目标温度：5 ℃）】，通过投料袋向激活罐（TJ4S-1213-Sy15-R01）中加入 kg 2-肟氰乙酸乙酯（Acceptable weighing range/可接受称量范围： kg ~ kg）(2.00 eq.)。 |  |  |  |  |  |  | Times/次数 |
|  |  |  |  |  |  |  | 1 |
| Before material charging, confirm the material is within the retest date/ the expiry date,Confirm whether the weight is adequate for use in the current charging. /加物料前，执行物料在复检日期/有效期之内，确认重量是否满足本次投料使用。 Before solid material charging, confirm/加固体物料前，确认： - Before operation performed, confirm there is no non-related material / equipment / instrument / tools / pipeline / document / record / label in the intended area /操作前；目标区域无不相关物料/设备/仪器/工具/管路/文件/记录/标签 - The outer packages are removed. If the outer package cannot be removed, but un-package material without damaging the package bag, and before charging, check no package fragment and other extraneous matter in material. /外包装移除；如果外包装不能移除，但在不破坏包装的情况下打开包装袋，投料前以及过程中同时检查执行物料中无包装碎片以及异物 Checked by & Date/核实人及日期： |  |  |  |  |  |  |  |
| Check and record that the solid charge port is free of solid accumulation from the previous step./ 检查并记录固体投料口无来自上一步骤的固体残留。 Checked by & Date/核实人及日期： |  |  |  |  |  |  |  |
| OxymaPure additon/加2-肟氰乙酸乙酯 |  |  | Performed by & Date/执行人及日期： |  |  |  |  |
|  |  |  | Begin/开始： |  | End/结束： |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |

| 3.6. （2）Fmoc-L-Phe-OH activation/芴甲氧羰基-L-苯丙氨酸活化 |  |  |  |  |
| --- | --- | --- | --- | --- |
| OxymaPure confirmation/2-肟氰乙酸乙酯确认：Control No./内部控制号：YL005180 |  |  |  |  |
| Check items/检查项 |  | Is it correct?/是否正确 | Recorded by & Date/ 记录人及日期 | Checked by & Date/ 核实人及日期 |
| Material name/物料名称 |  | Yes/是；No/否 |  |  |
| Project batch number/ 项目批次号 |  | Yes/是；No/否 |  |  |
| Operation number/ 操作号 |  | Yes/是；No/否 |  |  |
| Control No./内部控制号 |  | Yes/是；No/否 |  |  |
| No. /编号 | Net weight/净重 | All in/全部加入 | Recorded by & Date/ 记录人及日期 | Checked by & Date/ 核实人及日期 |
| / |  | Yes/是；No/否 |  |  |
| / |  | Yes/是；No/否 |  |  |
| / |  | Yes/是；No/否 |  |  |
| / |  | Yes/是；No/否 |  |  |
| / |  | Yes/是；No/否 |  |  |
| / |  | Yes/是；No/否 |  |  |
| / |  | Yes/是；No/否 |  |  |
| / |  | Yes/是；No/否 |  |  |
| / |  | Yes/是；No/否 |  |  |
| / |  | Yes/是；No/否 |  |  |
| / |  | Yes/是；No/否 |  |  |
| / |  | Yes/是；No/否 |  |  |
| Total/合计 |  | Confirm the total net weight is within the input range/执行合计净重在投料范围内 | Recorded by & Date/记录人及日期 | Checked by & Date/ 核实人及日期 |
|  |  | Yes/是；No/否 |  |  |

| 3.6. （2）Fmoc-L-Phe-OH activation/芴甲氧羰基-L-苯丙氨酸活化 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| After solid material charging, confirm/固体物料投料之后，确认： - The corresponding leftover material has been properly sealed and transferred to the staging area/相应的剩余物料已经恰当密封，转移到了暂存区域。 - The waste packaging material has been sealed and transferred to the corresponding waste staging area. /产生的包材废物已经密封并转移到了相应的废物暂存区域。 - The operation area/equipment surface has been cleaned/操作区域以及设备表面已经清洁。 Checked by & Date/核实人及日期： |  |  |  |  |  |  |
| Internal temperature/内部温度 |  | High values/最高值： |  | Low values/最低值： |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |
| CAUTION/注意：DMF/N，N-二甲基甲酰胺：Irritation of eyes, skin and nose. May cause nausea. /刺激眼睛、皮肤和鼻子。可能引起恶心。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |  |
| 3.6.7 Rinse the activation tank /淋洗激活罐 Rinse the port of Activation tank (TJ4S-1213-Sy15-R01) with 45.50 kg（Acceptable weighing range：43.50 kg ~ 47.50 kg）（1 V）DMF. 用45.50 kg（可接受称量范围：43.50 kg ~ 47.50 kg）（1 V）N,N-二甲基甲酰胺淋洗激活罐（TJ4S-1213-Sy15-R01）加料口。 |  |  |  |  |  | Times/次数 |
|  |  |  |  |  |  | 1 |
| Rinse with DMF/ N,N-二甲基甲酰胺淋洗 | Performed by & Date/执行人及日期： |  |  |  |  |  |
| Rinse with DMF/ N,N-二甲基甲酰胺淋洗 | Begin/开始： |  | End/结束： |  | Qty/重量： |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |

| 3.6. （2）Fmoc-L-Phe-OH activation/芴甲氧羰基-L-苯丙氨酸活化 |  |
| --- | --- |
| DIPEA/N,N-二异丙基乙胺：It can enter the body through inhalation, ingestion and skin absorption. It is strongly irritating to the eyes, skin, mucous membranes and upper respiratory tract. Inhalation of high-concentration vapors may cause inflammation, edema and spasm of the larynx/bronchi, even chemical pneumonia and pulmonary edema. Exposure may cause symptoms such as burning sensation, cough, headache, nausea and vomiting. /可通过吸入、食入、皮肤吸收侵入人体，对眼、皮肤、黏膜和上呼吸道有强刺激性，吸入高浓度蒸气会引发喉/支气管炎症水肿、痉挛，甚至化学性肺炎、肺水肿，接触后会出现烧灼感、咳嗽、头痛、恶心呕吐等症状 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |
| 3.6.8 DIPEA addition /加N,N-二异丙基乙胺 Add DIPEA (3.00 eq.) into the activation tank (TJ4S-1213-Sy15-R01) at 【5±5 ℃ (target: 5 ℃)】. Reference addition time is 50±20 min (target 50 min). The addition rate depends on temperature control. 控温【5±5 ℃(目标：5 ℃)】，向激活罐（TJ4S-1213-Sy15-R01）中滴加 kg N,N-二异丙基乙胺（Acceptable weighing range/可接受称量范围： kg ~ kg）(3.00 eq.)，滴加参考时间50±20 min（目标时间50 min），以控温为主。 Pump number/泵编号： Scale ID. No./秤编号： Material charging pipe number/物料管路编号： | Times/次数 |
|  | 1 |
| Over-temperature warning value/超温预警设置数值： Lower limit/低限: 2 oC ，High limit/高限: 8oC |  |
| Before material charging, confirm the material is within the retest date/ the expiry date, Confirm whether the weight is adequate for use in the current charging. /加物料前，执行物料在复检日期/有效期之内，确认重量是否满足本次投料使用。 Checked by & Date/核实人及日期： |  |

| 3.6. （2）Fmoc-L-Phe-OH activation/芴甲氧羰基-L-苯丙氨酸活化 |  |  |  |
| --- | --- | --- | --- |
| Before drummed liquid material charging, visual check there is no other extraneous matter in material. /加桶装液体物料前，应目视执行桶内无异物。 Visual Examination of Foreign Matters for Drum Solvent/桶装溶剂异物目视检查: 1. Place the solvent drum in a well-lit area (or use a portable light source) to ensure adequate visibility of the liquid contents./ 将溶剂桶置于光线充足区域（或使用便携式光源），确保液体内容物可见性良好 2. Visually inspect the solvent through the drum bung opening or transparent sight glass for any visible foreign matters, including but not limited to particulates, fibers, sediments, or unusual turbidity./ 通过桶口或透明视镜目视检查溶剂中是否有可见异物，包括但不限于颗粒、纤维、沉淀物或异常浑浊。 3. Confirm that the solvent is clear and free of visible foreign matters before charging. If any foreign matter is observed, do not use the drum and follow the applicable handling procedure for subsequent disposition./ 投料前确认溶剂澄清且无可见异物。如发现任何异物，不得使用该桶，并依据处理流程进行后续处置。 Checked by & Date/核实人及日期： |  |  |  |
| Confirm liquid charging pipeline is released. /确认加料管路经过放行。 |  |  |  |
| Checked by & Date/核实人及日期： |  |  |  |
| Charging of drummed solvent: verify that the pipeline connections remain consistent before and after material feeding, then record the weighing reading 桶装溶剂溶液称量时，确认投料前后管路连接一致，读取称量示数。 Performed by & Date/执行人及日期： Checked by & Date/核实人及日期： |  |  |  |
| DIPEA /N,N-二异丙基乙胺：Control Number/内部控制号：YL000555 |  |  |  |
| Internal temperature/内部温度 |  | High values/最高值： | Low values/最低值： |
| Transcribed by & Date /转录人及日期： |  |  |  |
| If the target net weight≥the planned net weight, perform full drum charge; otherwise, perform partial (Non-full drum) charge. /当目标加料量≥计划加入净重，勾选进行整桶加料；否则勾选进行零头加料。 Note: Target net weight to be charged = Total target net weight – Total net weight charged. Expected tare weight of drum after charge = Gross weight of drum that to be charged - Target net weight to be charged. 还需加入净重目标值= 加料步骤目标总净重-已加入净重总量 加料完成后预计桶皮重=待加料桶毛重-还需加入净重目标值 |  |  |  |  |

3.6. （2）Fmoc-L-Phe-OH activation/芴甲氧羰基-L-苯丙氨酸活化

| Addition type加料式 | Lot number/批号 | Pre-charging qty calculation/投料前投料量计算 |  |  |  |  |  | Actual Charging Qty/实际投料量 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Total qty charged/已加入净重总量 |  | Target qty to be charged/还需加入净重目标值 | Gross qty of drum that to be charged/待加料桶毛重 | Expected net weight of drum after charge /加料完成后预计桶净重 | Expected tare weight of drum after charge/加料完成后预计桶皮重 | Before charging/ 投料前 |  | After charging/ 投料后 | Net weight/ 净重 | Performed by & Date/ 执行人及日期 | Checked by & Date/ 核实人及日期 |
| Full drum整桶 Non-full drum /非整桶 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Full drum整桶 Non-full drum /非整桶 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Total/合计 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DIPEA Addition/加N,N-二异丙基乙胺 |  |  | Begin/开始： |  |  |  |  |  | End/结束： |  |  |  |  |
| Transcribed by & Date/转录人及日期： |  |  |  |  |  |  |  |  |  |  |  |  |  |

| 3.6. （2）Fmoc-L-Phe-OH activation/芴甲氧羰基-L-苯丙氨酸活化 |  |  |  |  |
| --- | --- | --- | --- | --- |
| 3.6.9 Hold at temperature /保温 At 5±5 ℃ (target: 5 ℃). Continue agitation of activation tank to 95±10 rpm (target: 95 rpm). Maintain stirring for 20±5 min (target: 20 min) after addition, then stop stirring. 滴毕，控温5±5 ℃(目标温度：5 ℃)，维持激活罐搅拌转速至95±10 rpm (目标转速: 95 rpm)，继续搅拌20±5 min(目标时间：20 min)，停止搅拌。 |  |  | Times/次数 |
|  |  |  | 1 |
| Stir /搅拌 | Performed by & Date/执行人及日期： |  |  |
| Stir /搅拌 | End/结束 |  |  |
|  | The start time is documented in Ins3.6.1 /开始时间记录于3.6.1 |  |  |
| Transcribed by & Date/转录人及日期： |  |  |  |
| 3.6.9 Internal temperature/内部温度 | High values/最高值： | Low values/最低值： |  |
| Transcribed by & Date/转录人及日期： |  |  |  |
| CAUTION/注意：DMF/N，N-二甲基甲酰胺：Irritation of eyes, skin and nose. May cause nausea. /刺激眼睛、皮肤和鼻子。可能引起恶心。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |

| - （2）Fmoc-L-Phe-OH coupling/芴甲氧羰基-L-苯丙氨酸偶联 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3.7.1 Adjust the synthesis reactor temperature /合成釜调温 Adjust the synthesis reactor jacket temperature to 25±5 °C. Add 91.01 kg (Acceptable weighing range：88.28 ~ 93.74 kg) (2 V) DMF into synthesis reactor (TJ4S-1213-Sy15) at jacket 25±5 °C. 在合成釜（TJ4S-1213-Sy15）夹套调温25±5 °C，控温夹套温度25±5 °C下加入91.01 kg （可接受称量范围：88.28 ~ 93.74 kg）（2 V）N,N-二甲基甲酰胺。 |  |  |  |  |  |  | Times/次数 |
|  |  |  |  |  |  |  | 1 |
| Adjust temperature/调温 | Performed by & Date/执行人及日期： |  |  |  |  |  |  |
|  | Begin/开始： |  | End/结束： |  |  |  |  |
| Jacket temperature/温度 | High values/最高值： |  | Low values/最低值： |  |  |  |  |
| Add 91.01 kg (Acceptable weighing range：88.28 ~ 93.74 kg) (2 V) DMF into synthesis reactor (TJ4S-1213-Sy15)./合成釜（TJ4S-1213-Sy15）加入91.01 kg （可接受称量范围：88.28 ~ 93.74 kg）（2 V）N,N-二甲基甲酰胺。 |  |  |  |  |  |  |  |
| Rinse with DMF/N,N-二甲基甲酰胺淋洗 | Performed by & Date/执行人及日期： |  |  |  |  |  |  |
| Rinse with DMF/ N,N-二甲基甲酰胺淋洗 | Begin/开始： |  |  | End/结束： |  | Qty/重量： |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |
| Jacket temperature/夹套温度 |  | High values/最高值： |  |  | Low values/最低值： |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |

| 3.7. （2）Fmoc-L-Phe-OH coupling/芴甲氧羰基-L-苯丙氨酸偶联 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 3.7.2 Transfer the mixture /转移体系 Transfer the mixture in activation tank (TJ4S-1213-Sy15-R01) into synthesis reactor (TJ4S-1213-Sy15). 将激活罐（TJ4S-1213-Sy15-R01）中体系转移至合成釜（TJ4S-1213-Sy15）中。 |  |  |  |  |  |  | Times/次数 |
|  |  |  |  |  |  |  | 1 |
| ★Note:The coupling temperature is a CPP, and the coupling IPC hold temperature is a CPP./ 备注：偶联温度是CPP，偶联IPC维持温度是关键工艺参数。 The operator shall confirm that the reaction mixture temperature is within the Normal Operating Range (NOR) 【25±5oC (target: 25oC)】 /操作人员须执行反应体系温度处于正常操作范围（NOR）内【25±5oC (target: 25oC)】 Checked by & Date/核实人及日期： Over-temperature warning value/超温预警设置数值： Lower limit/低限: 22 oC ，High limit/高限: 28 oC |  |  |  |  |  |  |  |
| Transfer/转移 |  | Performed by & Date/执行人及日期： |  |  |  |  |  |
|  |  | Checked by & Date/核实人及日期： |  |  |  |  |  |
|  |  | Begin/开始： |  |  | End/结束： |  |  |
| CAUTION/注意：DMF/N，N-二甲基甲酰胺：Irritation of eyes, skin and nose. May cause nausea. /刺激眼睛、皮肤和鼻子。可能引起恶心。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |  |  |
| 3.7.3 DMF addition /加N,N-二甲基甲酰胺 Add 45.50 kg (Acceptable weighing range：43.50 kg ~ 47.50 kg) (1 V) DMF by spray balls into activation tank (TJ4S-1213-Sy15-R01) to rinse the wall. 通过淋洗球向激活罐（TJ4S-1213-Sy15-R01）中加入45.50 kg（可接受称量范围：43.50 kg ~ 47.50 kg） (1 V) N,N-二甲基甲酰胺淋洗罐壁。 |  |  |  |  |  |  | Times/次数 |
|  |  |  |  |  |  |  | 1 |
| Rinse with DMF/N,N-二甲基甲酰胺淋洗 |  |  | Performed by & Date/执行人及日期： |  |  |  |  |
| Rinse with DMF/ N,N-二甲基甲酰胺淋洗 | Begin/开始： |  |  | End/结束： |  | Qty/重量： |  |
| Transcribed by & Date /转录人及日期: |  |  |  |  |  |  |  |

| 3.7. （2）Fmoc-L-Phe-OH coupling/芴甲氧羰基-L-苯丙氨酸偶联 |  |  |  |  |
| --- | --- | --- | --- | --- |
| 3.7.4 Transfer /转移 Turn on the nitrogen of Synthesis reactor (TJ4S-1213-Sy15) to 5±2 m³/h (target: 5 m³/h). Bubble with nitrogen for 5±2 min (target: 5 min). Start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm). then start the agitator. Transfer the mixture in activation tank (TJ4S-1213-Sy15-R01) into synthesis reactor (TJ4S-1213-Sy15). 合成釜（TJ4S-1213-Sy15）先开启下口氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min (目标时间：5 min)后，开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm)，将激活罐（TJ4S-1213-Sy15-R01）中体系转移至合成釜（TJ4S-1213-Sy15）中。 |  |  |  | Times/次数 |
|  |  |  |  | 1 |
| Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h). Bubble with nitrogen for 5±2 min (target :5 min) 开启合成釜下口鼓氮气，氮气流速开至5±2m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min (目标时间：5 min)。 |  |  |  |  |
| Bubble/鼓氮气 | Performed by & Date/执行人及日期： |  |  |  |
| Bubble/鼓氮气 | Begin/开始： |  |  |  |
|  | The end time is documented in Ins3.7.6 /结束时间记录于3.7.6 |  |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |
| Start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm). /开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm)。 |  |  |  |  |
| Stir /搅拌 | Performed by & Date/执行人及日期： |  |  |  |
| Stir /搅拌 | Begin/开始： |  |  |  |
|  | The end time is documented in Ins3.7.6 /结束时间记录于3.7.6 |  |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |
| Transfer/转移 |  | Performed by & Date/执行人及日期： |  |  |
|  |  | Checked by & Date/核实人及日期： |  |  |
|  |  | Begin/开始： | End/结束： |  |
| 3.7.5 Adjust temperature /调温 Adjust the synthesis reactor (TJ4S-1213-Sy15) mixture to【25±5 ℃ (target: 25 ℃)】. /将合成釜（TJ4S-1213-Sy15）体系调温至【25±5 ℃(目标温度：25 ℃)】. |  |  |  | Times/次数 |
|  |  |  |  | 1 |

| 3.7. （2）Fmoc-L-Phe-OH coupling/芴甲氧羰基-L-苯丙氨酸偶联 |  |  |  |  |
| --- | --- | --- | --- | --- |
| ★Note:The coupling temperature is a CPP, and the coupling IPC hold temperature is a CPP./ 备注：偶联温度是CPP，偶联IPC维持温度是关键工艺参数。 The operator shall confirm that the reaction mixture temperature is within the Normal Operating Range (NOR) /操作人员须执行反应体系温度处于正常操作范围（NOR）内 ★Lower limit/低限: 13 oC ，High limit/高限: 37 oC Over-temperature warning value/超温预警设置数值： Lower limit/低限: 21 oC ，High limit/高限: 28 oC |  |  |  |  |
| Adjust/调温 | Performed by & Date/执行人及日期： |  |  |  |
| Adjust/调温 |  | Begin/开始： | End/结束： |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |
| 3.7.6 Fmoc-L-Phe-OH coupling/芴甲氧羰基-L-苯丙氨酸偶联 Start timing when temperature reaches 20 ℃. The mixture is allowed to react at【25±5 ℃ (target: 25 ℃)】. After 4 h±15 min (target：4 h), sample the mixture every 0.5~1 h for ninhydrin test analysis until two consecutive samples test as negative (Yellow to Light pink Report Negative= Complete coupling; Blue to Purple Positive= Incomplete coupling). And the mixture should be stirred for coupling time (6 h≤Coupling Time≤14 h). Stop stirring, turn off the nitrogen. Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point. 控温【25±5 oC（目标温度25 oC，20 oC开始计时）】反应4 h±15 min（目标：4 h）后，取样进行茚三酮检测，每隔0.5~1.0 h取样一次，直至连续两次取样结果为阴性（黄色至浅粉色，报告阴性=偶联完全；蓝色至紫色，阳性=偶联不完全）后，且反应时间满足（6 h≤反应时间≤14 h）后，停止搅拌，关闭氮气。通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 Note: If resin adherence is found at other times during the reaction, rinse the reactor with 0.50~5.00 kg DMF each time. /备注：偶联过程中其它时间发现贴壁，每次使用0.50~5.00 kg N,N-二甲基甲酰胺淋洗，记录见附加页 |  |  |  | Times/次数 |
|  |  |  |  | 1 |

| 3.7. （2）Fmoc-L-Phe-OH coupling/芴甲氧羰基-L-苯丙氨酸偶联 |  |  |  |  |
| --- | --- | --- | --- | --- |
| The total reaction Fmoc-L-Phe-OH coupling time：NMT 841 min (Sourced from the holding time study in the PAR study report.)/芴甲氧羰基- L-苯丙氨酸偶联总的反应时间不超过841min(来源于 PAR 研究报告中稳定时间研究)。 Over-temperature warning value/超温预警设置数值： Lower limit/低限: 21 oC ，High limit/高限: 28 oC |  |  |  |  |
| React/反应 | Performed by & Date/执行人及日期： |  |  |
| React/反应 |  | Begin/开始： | End/结束： |
| Transcribed by & Date /转录人及日期： |  |  |  |
| ★After coupling is completed, the operator shall confirm that all Critical Process Parameters (CPP) are within their respective Normal Operating Ranges (NOR)/ 偶联结束后，操作人员须确认所有关键工艺参数（CPP）均处于各自的正常操作范围（NOR）内 ★Note: The coupling temperature is a CPP, and the coupling IPC hold temperature is a CPP./备注：偶联温度是CPP，偶联IPC维持温度是关键工艺参数。 The operator shall confirm that the reaction mixture temperature is within the Normal Operating Range (NOR) :20~30℃ /操作人员须确认反应体系温度处于正常操作范围（NOR）内:20~30℃ □Yes/是 □No/否 Confirmed by & Date/确认人及日期： Checked by & Date/核实人及日期： |  |  |  |
| Internal temperature/内部温度 |  | High values/最高值： | Low values/最低值： |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |

| 3.7. （2）Fmoc-L-Phe-OH coupling/芴甲氧羰基-L-苯丙氨酸偶联 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Standard sampling method/标准取样方法 |  |  |  |  |  |  |
| Take 5-10 ml mixture and perform testing in accordance with the ninhydrin test record. During sampling, should wear nitrile gloves and 3M 6000 series Full-face piece respirator, 6006CN cartridges. 通过在线取样器取样5~10 ml按照茚三酮检测记录进行检测。取样过程中，应该戴丁腈手套、3M 6000系列全面罩, 6006CN滤毒盒。 |  |  |  |  |  |  |
| Sampling number/ 取样编号 |  | Sampling time/取样时间 | Sampled by & Date/取样人及日期 | Negative or Positive/阴性或阳性 | Result filled by &Date/结果填写人及日期 | Checked by & Date/核实人及日期 |
| CPo140020- -01- -3.7.6-01 |  |  |  | Negative /阴性 Positive /阳性 |  |  |
| CPo140020- -01- -3.7.6-02 |  |  |  | Negative /阴性 Positive /阳性 |  |  |
| CPo140020- -01- -3.7.6-03 |  |  |  | Negative /阴性 Positive /阳性 |  |  |
| CPo140020- -01- -3.7.6-04 |  |  |  | Negative /阴性 Positive /阳性 |  |  |
| CPo140020- -01- -3.7.6-05 |  |  |  | Negative /阴性 Positive /阳性 |  |  |
| CPo140020- -01- -3.7.6-06 |  |  |  | Negative /阴性 Positive /阳性 |  |  |
| CPo140020- -01- -3.7.6-07 |  |  |  | Negative /阴性 Positive /阳性 |  |  |
| CPo140020- -01- -3.7.6-08 |  |  |  | Negative /阴性 Positive /阳性 |  |  |
| Note:For a failing coupling test.Continue hourly sampling up to the NMT 841 min hold limit; if still positive, stop, notify PM/QA and raise a deviation; no re-coupling without an approved deviation.偶联反应检测不合格的处置规则:按每小时 1 次的频率继续取样检测，直至达到不超过（NMT）841 min的保温时限；若到达时限后检测仍为阳性，停止反应，通知项目经理（PM）/ 质量保证部门（QA）并发起偏差报告；未经偏差批准，不得进行补偶联操作。 |  |  |  |  |  |  |
| Confirm the end point of the reaction and move to following operation. /确认反应终点后进行下一步操作。 The acceptance criterion is two consecutive negative test results./连续两次检测结果为阴性即为合格。 Confirmed by & Date/确认人及日期： Checked by & Date/核实人及日期： |  |  |  |  |  |  |

| 3.7. （2）Fmoc-L-Phe-OH coupling/芴甲氧羰基-L-苯丙氨酸偶联 |  |  |  |
| --- | --- | --- | --- |
| Stop the stirrer and turn off the nitrogen. Start draining the solution from synthesis reactor with diaphragmgm pump. Drain until there is no drip, the operator visually confirms the draining end point./ 停止搅拌，关闭氮气，通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  |  |
| Stir /搅拌 | Performed by & Date/执行人及日期： |  |  |
| Stir /搅拌 | End/结束： |  |  |
|  | The start time is documented in Ins3.7.4 /开始时间记录于3.7.4 |  |  |
| Transcribed by & Date/转录人及日期： |  |  |  |
| Bubble/鼓氮气 | Performed by & Date/执行人及日期： |  |  |
| Bubble/鼓氮气 | End/结束： |  |  |
|  | The start time is documented in Ins3.7.4 /开始时间记录于3.7.4 |  |  |
| Transcribed by & Date/转录人及日期： |  |  |  |
| Internal temperature/内部温度 | High values/最高值： | Low values/最低值： |  |
| Transcribed by & Date/转录人及日期： |  |  |  |
| Filter/过滤 | Performed by & Date/执行人及日期： |  |  |
|  | Checked by & Date/核实人及日期： |  |  |
|  | Begin/开始： |  | End/结束： |

| 3.7. （2）Fmoc-L-Phe-OH coupling/芴甲氧羰基-L-苯丙氨酸偶联 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ★After coupling is completed, the operator shall confirm that all Critical Process Parameters (CPPs) are within their respective Normal Operating Ranges (NOR)/ 偶联结束后，操作人员须确认所有关键工艺参数（CPP）均处于各自的正常操作范围（NOR）内 Checked by & Date/核实人及日期： |  |  |  |  |  |  |
| CAUTION/注意：DMF/N，N-二甲基甲酰胺：Irritation of eyes, skin and nose. May cause nausea. /刺激眼睛、皮肤和鼻子。可能引起恶心。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |
| 3.7.7 Activation Tank Rinse/激活罐洗涤 Add 364.03 kg (Acceptable weighing range：353.11 kg ~ 374.95 kg) (8 V) DMF into the activation tank (TJ4S-1213-Sy15-R01). Circulate and clean the activating liquid transfer pipeline 1, liquid charging port cleaning pipeline 2 and solid charging port cleaning pipeline 3, clean every pipeline for 2±1 min. 向激活罐（TJ4S-1213-Sy15-R01）中加入364.03 kg（可接受称量范围：353.11 kg ~ 374.95 kg） (8 V) N,N-二甲基甲酰胺，循环清洗激活液转移管路1，液体加料口清洗管路2和固体加料口清洗管路3，每条线路各清洗2±1 min。 |  |  |  |  | Times/次数 |
|  |  |  |  |  | 3 |
| Operation name/ 操作名称 | Time point/时间节点 | Clean the activating liquid transfer pipeline 1/循环清洗激活液转移管路1 | Liquid charging port cleaning pipeline 2 /y液体加料口清洗管路2 | Solid charging port cleaning pipeline 3,/固体加料口清洗管路3 |  |
|  |  | Wash /洗涤1 | Wash /洗涤2 | Wash /洗涤3 |  |
| DMF addition/ 加N,N-二甲基甲酰胺 | Performed by & Date/执行人及日期： |  |  |  |  |
| DMF addition/ 加N,N-二甲基甲酰胺 | Begin/开始 |  |  |  |  |
|  | End/结束 |  |  |  |  |
|  | Qty/重量 |  |  |  |  |
|  | Transcribed by & Date /转录人及日期： |  |  |  |  |
| Circulate and clean/循环清洗 | Performed by & Date/执行人及日期： |  |  |  |  |

| 3.7. （2）Fmoc-L-Phe-OH coupling/芴甲氧羰基-L-苯丙氨酸偶联 |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operation name/ 操作名称 |  |  | Time point/时间节点 |  | Clean the activating liquid transfer pipeline 1/循环清洗激活液转移管路1 |  | Liquid charging port cleaning pipeline 2 /液体加料口清洗管路2 |  | Solid charging port cleaning pipeline 3,/固体加料口清洗管路3 |  |  |
|  |  |  |  |  | Wash /洗涤1 |  | Wash /洗涤2 |  | Wash /洗涤3 |  |  |
| Circulate and clean/循环清洗 |  |  | Begin/开始 |  |  |  |  |  |  |  |  |
|  |  |  | End/结束 |  |  |  |  |  |  |  |  |
|  |  |  | Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |  |
| 3.7.8 Post-coupling wash./偶联后洗涤 Transfer the mixture in activation tank (TJ4S-1213-Sy15-R01) to synthesis reactor (TJ4S-1213-Sy15). Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h). Bubble with nitrogen, after 5±2 min (target 5 min), Start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm). After stirring for 7±2 min (target：7 min), stop stirring, turn off the nitrogen. Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point. 将激活罐（TJ4S-1213-Sy15-R01）洗涤液转入合成釜 (TJ4S-1213-Sy15)。开启合成釜下口鼓氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min (目标时间：5 min)后开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm)，搅拌7±2 min（目标时间：7 min）后停止搅拌，关闭氮气。通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  |  |  |  |  |  |  |  |  | Times/次数 |
|  |  |  |  |  |  |  |  |  |  |  | 3 |
| Transfer the liquid form activation tank to the synthesis reactor (TJ4S-1213-Sy15) ./将激活罐中体系转移至合成釜(TJ4S-1213-Sy15)中。 |  |  |  |  |  |  |  |  |  |  |  |
| Operation name/ 操作名称 |  |  | Time point/时间节点 |  | Wash /洗涤1 |  | Wash /洗涤2 |  | Wash /洗涤3 |  |  |
| Transfer/转移 |  |  | Performed by & Date/执行人及日期： |  |  |  |  |  |  |  |  |
|  |  |  | Checked by & Date/核实人及日期 |  |  |  |  |  |  |  |  |
|  |  |  | Begin/开始 |  |  |  |  |  |  |  |  |
|  |  |  | End/结束 |  |  |  |  |  |  |  |  |

| 3.7. （2）Fmoc-L-Phe-OH coupling/芴甲氧羰基-L-苯丙氨酸偶联 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h), Bubble with nitrogen for 5±2 min (target:5 min), Start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm).After stirring for 7±2 min (target: 7 min), stop stirring, turn off the nitrogen./ 开启合成釜下口鼓氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min（目标时间：5 min）后开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm)，搅拌7±2 min（目标时间：7 min）后，停止搅拌，关闭氮气。 |  |  |  |  |  |
| Operation name/ 操作名称 | Time point/时间节点 | Wash /洗涤1 | Wash /洗涤2 | Wash /洗涤3 |  |
| Bubble/鼓氮气 | Performed by & Date/执行人及日期： |  |  |  |
| Bubble/鼓氮气 | Begin/开始 |  |  |  |
|  | End/结束 |  |  |  |
|  | Transcribed by & Date /转录人及日期： |  |  |  |
| Stir /搅拌 | Performed by & Date/执行人及日期： |  |  |  |  |
| Stir /搅拌 | Begin/开始 |  |  |  |  |
|  | End/结束 |  |  |  |  |
|  | Transcribed by & Date /转录人及日期： |  |  |  |  |
| Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point/通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  |  |  |  |
| Filter/过滤 | Performed by & Date/执行人及日期： |  |  |  |  |
|  | Checked by & Date/核实人及日期 |  |  |  |  |
|  | Begin/开始 |  |  |  |  |
|  | End/结束 |  |  |  |  |

| - （3）Fmoc- L-N-Me-Asn(Trt)-OH deprotection/Fmoc-N-甲基-N’-三苯甲基-天冬氨酰胺脱保护 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CAUTION/注意：Piperidine哌啶：Strong local irritant and may cause permanent injury after short exposure to small amounts. /强烈的局部刺激，短时间少量接触可能造成永久性伤害。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |  |  |
| 3.8.1 Add piperidine solution /加哌啶溶液 Adjust the jacket temperature of synthesis reactor (TJ4S-1213-Sy15) to 【25±5 oC (target: 25 oC)】. Add 357.50 kg (Acceptable weighing range: 346.77 ~ 368.23 kg) （8 V）【20% (V/V) piperidine/DMF solution】 into the Synthesis reactor (TJ4S-1213-Sy15). 将合成釜(TJ4S-1213-Sy15)夹套调温至【25±5 ℃（目标温度：25 ℃）】。向合成釜（TJ4S-1213-Sy15）中加入357.50 kg（可接受称量范围：346.77 ~ 368.23 kg）（8 V）【20%哌啶/DMF (V/V)】溶液。 |  |  |  |  |  |  | Times/次数 |
|  |  |  |  |  |  |  | 1 |
| Adjust/调温 |  | Performed by & Date/执行人及日期： |  |  |  |  |  |
| Adjust/调温 |  | Begin/开始： |  |  | Adjust/调温 |  |  |
| Transcribed by & Date/转录人及日期： |  |  |  |  |  |  |  |
| Confirm and record that the 20%Piperidine solution concentration is within the NOR range./ 确认并记录20%哌啶溶液浓度在正常操作范围内：17.50%~22.50% 20%Piperidine solution concentration confirmation/20%哌啶溶液浓度确认：____________ ★Confirm and record that the 20%Piperidine solution temperature is within the NOR range./ 确认并记录20%哌啶溶液温度在正常操作范围内：20 oC ~30 oC 20%Piperidine solution temperature confirmation/20%哌啶溶液温度确认：____________ Confirmed by & Date/确认人及日期： Checked by & Date/核实人及日期： |  |  |  |  |  |  |  |
| ★Note:The deprotection temperature is a CPP./备注：脱保护反应温度是关键工艺参数。 Checked by & Date/核实人及日期： |  |  |  |  |  |  |  |
| Piperidine addition/加哌啶溶液 |  |  | Performed by & Date/执行人及日期： |  |  |  |  |
| Piperidine addition/ 加哌啶溶液 | Begin/开始： |  |  | End/结束： |  | Qty/重量： |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |

| 3.8. （3）Fmoc- L-N-Me-Asn(Trt)-OH deprotection/Fmoc-N-甲基-N’-三苯甲基-天冬氨酰胺脱保护 |  |  |  |  |
| --- | --- | --- | --- | --- |
| 3.8.2 Adjust synthesis reactor temperature /合成釜调温 Turn on the nitrogen of Synthesis reactor (TJ4S-1213-Sy15) to 5±2 m³/h (target: 5 m³/h). Bubble with nitrogen for 5±2 min (target 5 min). Start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm). then start the stirrer, Adjust to 25±5 ℃ (target: 25 ℃）. 开启合成釜（TJ4S-1213-Sy15）下口鼓氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min (目标时间：5 min)后开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm)，调温至【25±5 ℃(目标温度：25 ℃)】。 |  |  |  | Times/次数 |
|  |  |  |  | 1 |
| Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h). Bubble with nitrogen for 5±2 min (target :5 min) 开启合成釜下口鼓氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min (目标时间：5 min) |  |  |  |  |
| Bubble/鼓氮气 |  | Performed by & Date/执行人及日期： |  |  |
| Bubble/鼓氮气 |  | Begin/开始： |  |  |
|  |  | The end time is documented in Ins 3.8.3/结束时间记录于3.8.3 |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |
| Start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm). /开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm) |  |  |  |  |
| Stir /搅拌 |  | Performed by & Date/执行人及日期： |  |  |
| Stir /搅拌 |  | Begin/开始： |  |  |
|  |  | The end time is documented in Ins 3.8.3/结束时间记录于3.8.3 |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |
| Adjust to 25±5 oC (target: 25 oC)./ 调温至25±5 ℃(目标温度：25 oC) |  |  |  |  |

| 3.8. （3）Fmoc- L-N-Me-Asn(Trt)-OH deprotection/Fmoc-N-甲基-N’-三苯甲基-天冬氨酰胺脱保护 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Adjust/调温 |  | Performed by & Date/执行人及日期： |  |  |  |  |
| Adjust/调温 |  | Begin/开始： | End/结束： |  |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |
| 3.8.3 Fmoc- L-N-Me-Asn(Trt)-OH deprotection/Fmoc-N-甲基-N’-三苯甲基-天冬氨酰胺脱保护 After stirring for 【2 h±15 min (target: 2 h) 】at 【25±5 ℃ (target: 25 ℃)】, stop stirring, turn off the nitrogen. Filter until nearly no liquid outflow. Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point. 控温【25±5 ℃(目标温度：25 ℃)】， 搅拌【2 h±15 min（目标时间：2 h）】后，停止搅拌，关闭氮气，滤除至基本通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 Over-temperature warning value/超温预警设置数值： Lower limit/低限: 22 oC ，High limit/高限: 28 oC |  |  |  |  |  | Times/次数 |
|  |  |  |  |  |  | 1 |
| ★Confirm and record that the 20%Piperidine solution temperature is within the NOR range 20 oC ~30 oC ./ 确认并记录20%哌啶溶液温度在正常操作范围内：20 oC ~30 oC。 20%Piperidine solution temperature confirmation/20%哌啶溶液温度确认：____________ Confirmed by& Date/确认人及日期： Checked by & Date/核实人及日期： |  |  |  |  |  |  |
| De-Fmoc/脱Fmoc &Stir /搅拌 |  |  | Performed by & Date/执行人及日期： |  |  |  |
| De-Fmoc/脱Fmoc &Stir /搅拌 |  |  | End/结束： |  |  |  |
|  |  |  | The start time is documented in Ins3.8.2 /开始时间记录于3.8.2 |  |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |

| 3.8. （3）Fmoc- L-N-Me-Asn(Trt)-OH deprotection/Fmoc-N-甲基-N’-三苯甲基-天冬氨酰胺脱保护 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Bubble/鼓氮气 | Performed by & Date/执行人及日期： |  |  |  |
| Bubble/鼓氮气 | End/结束： |  |  |  |
|  | The start time is documented in Ins3.8.2 /开始时间记录于3.8.2 |  |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |
| Internal temperature/内部温度 |  | High values/最高值： |  | Low values/最低值： |
| Transcribed by & Date /转录人及日期： |  |  |  |  |
| Filter/过滤 |  | Performed by & Date/执行人及日期： |  |  |
|  |  | Checked by & Date/核实人及日期： |  |  |
|  |  | Begin/开始： | End/结束： |  |
| ★After De-Fmoc is completed, the operator shall confirm that all Critical Process Parameters (CPPs) are within their respective Normal Operating Ranges (NOR)/ 脱保护结束后，操作人员须确认所有关键工艺参数（CPP）均处于各自的正常操作范围（NOR）内 Checked by & Date/核实人及日期： |  |  |  |  |

| 3.8. （3）Fmoc- L-N-Me-Asn(Trt)-OH deprotection/Fmoc-N-甲基-N’-三苯甲基-天冬氨酰胺脱保护 |  |  |  |  |
| --- | --- | --- | --- | --- |
| 3.8.4 Adjust temperature /调温 Adjust the synthesis reactor (TJ4S-1213-Sy15) jacket to 【20±5 ℃ (target: 20 ℃)】. /将合成釜（TJ4S-1213-Sy15）夹套调温至【20±5 ℃（目标温度：20 ℃）】。 |  |  |  | Times/次数 |
|  |  |  |  | 1 |
| Adjust/调温 |  | Performed by & Date/执行人及日期： |  |  |
| Adjust/调温 |  | Begin/开始： | End/结束： |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |
| CAUTION/注意：IPAC/乙酸异丙酯： Vapors irritate eyes and respiratory tract; high concentrations can be anesthetic. Liquid irritates eyes but causes no serious injury; may cause dermatitis; no serious effects if swallowed. /蒸汽会刺激眼睛和呼吸道；高浓度的蒸汽可能是麻醉剂。液体刺激眼睛，但不会造成严重伤害；可能会引起皮炎；吞食不会造成严重影响。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |
| 3.8.5 Post-deprotection wash/脱保护后洗涤 Add 445.92 kg (Acceptable weighing range：432.54 ~ 459.30 kg) (9.29 kg/kg, 10.64 V) isopropyl acetate into synthesis reactor (TJ4S-1213-Sy15). Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h). Bubble with nitrogen for 5±2 min (target 5 min). Start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm). After stirring for 5±2 min (target 5 min), stop stirring, turn off the nitrogen. Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point. 向合成釜（TJ4S-1213-Sy15）中加入445.92 kg（可接受称量范围：432.54 ~ 459.30 kg) (9.29 kg/kg, 10.64 V）乙酸异丙酯，开启下口氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min (目标时间：5 min)后开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm)，搅拌5±2 min (目标时间：5 min)后，停止搅拌，关闭氮气。通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  |  | Times/次数 |
|  |  |  |  | 4 |
| ★Note: IPAC quantity is a CPP./备注：乙酸异丙酯用量是关键工艺参数。 ★The operator shall confirm that IPAC quantity is within the Normal Operating Range (NOR) 【432.54 kg~459.30 kg】/操作人员须确认乙酸异丙酯用量处于正常操作范围（NOR）内：【432.54 kg~459.30 kg】 Checked by & Date/核实人及日期： |  |  |  |  |

| 3.8. （3）Fmoc- L-N-Me-Asn(Trt)-OH deprotection/Fmoc-N-甲基-N’-三苯甲基-天冬氨酰胺脱保护 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Operation name/操作名称 | Time point/时间节点 | Wash /洗涤1 | Wash /洗涤2 | Wash /洗涤3 | Wash /洗涤4 |
| Add 419.04 kg IPAc (10 V) by spray balls into the synthesis reactor (TJ4S-1213-Sy15)./ 向合成釜(TJ4S-1213-Sy15) 中加419.04 kg乙酸异丙酯（10 V)（Acceptable weighing range/可接受称量范围：406.47~431.61 kg）。 |  |  |  |  |  |
| IPAC addition/加乙酸异丙酯 | Performed by & Date/执行人及日期： |  |  |  |  |
| IPAC addition/加乙酸异丙酯 | Begin/开始 |  |  |  |  |
|  | End/结束 |  |  |  |  |
|  | Qty/重量 |  |  |  |  |
|  | Transcribed by & Date /转录人及日期: |  |  |  |  |
| Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h) Bubble with nitrogen for 5±2 min (target: 5 min). Start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm). Stir for 5±2 min (target: 5 min), stop stirring, turn off the nitrogen./ 开启合成釜下口鼓氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min（目标时间：5 min）后开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm)，搅拌5±2 min（目标时间：5 min）后，停止搅拌，关闭氮气。 |  |  |  |  |  |
| Bubble/鼓氮气 | Performed by & Date/执行人及日期： |  |  |  |  |
| Bubble/鼓氮气 | Begin/开始 |  |  |  |  |
|  | End/结束 |  |  |  |  |
|  | Transcribed by & Date /转录人及日期: |  |  |  |  |
| Stir /搅拌 | Performed by & Date/执行人及日期： |  |  |  |  |
| Stir /搅拌 | Begin/开始 |  |  |  |  |
|  | End/结束 |  |  |  |  |
|  | Transcribed by & Date/转录人及日期 |  |  |  |  |

| 3.8. （3）Fmoc- L-N-Me-Asn(Trt)-OH deprotection/Fmoc-N-甲基-N’-三苯甲基-天冬氨酰胺脱保护 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point./通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  |  |  |  |  |  |  |  |  |
| Filter/过滤 |  | Performed by & Date/ 执行人及日期： |  |  |  |  |  |  |  |  |
|  |  | Checked by & Date/ 核实人及日期： |  |  |  |  |  |  |  |  |
|  |  | Begin/开始 |  |  |  |  |  |  |  |  |
|  |  | End/结束 |  |  |  |  |  |  |  |  |
| CAUTION/注意：DMF/N，N-二甲基甲酰胺：Irritation of eyes, skin and nose. May cause nausea. /刺激眼睛、皮肤和鼻子。可能引起恶心。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |  |  |  |  |  |
| 3.8.6 DMF addition /加N,N-二甲基甲酰胺 Add 546.05 kg (Acceptable weighing range: 529.67 ~ 562.43 kg) (12 V) DMF into synthesis reactor (TJ4S-1213-Sy15). 向合成釜（TJ4S-1213-Sy15）中加入546.05 kg（可接受称量范围：529.67 ~ 562.43 kg）（12 V）N,N-二甲基甲酰胺。 |  |  |  |  |  |  |  |  | Times/次数 |  |
|  |  |  |  |  |  |  |  |  | 1 |  |
| DMF addition / 加N,N-二甲基甲酰胺 |  | Performed by & Date/执行人及日期： |  |  |  |  |  |  |  |  |
| DMF addition / 加N,N-二甲基甲酰胺 |  | Begin/开始： |  | End/结束： |  |  | Qty/重量： |  |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |  |  |  |
| 3.8.7 Resin reswelling wash 1/溶胀后洗涤1 Turn on the nitrogen of synthesis reactor(TJ4S-1213-Sy15) to 5±2 m³/h (target: 5 m³/h). Bubble with nitrogen for 5±2 min (target: 5 min). Start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm). After stirring for 10±2 min (target: 10 min), stop stirring, turn off the nitrogen. Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point. 开启合成釜（TJ4S-1213-Sy15）下口氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min (目标：5 min)后，开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm)，搅拌10±2 min（目标：10 min）后，停止搅拌，关闭氮气。通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  |  |  |  |  |  |  | Times/次数 |  |
|  |  |  |  |  |  |  |  |  | 1 |  |

| 3.8. （3）Fmoc- L-N-Me-Asn(Trt)-OH deprotection/Fmoc-N-甲基-N’-三苯甲基-天冬氨酰胺脱保护 |  |  |  |
| --- | --- | --- | --- |
| Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h) for 5±2 min (target: 5min). Start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm). Stir for 10±2 min (target: 10 min). Stop stirring, turn off the nitrogen./ 开启合成釜下口鼓氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min（目标时间：5 min）后，开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm)，搅拌10±2 min（目标时间:10 min）后，停止搅拌，关闭氮气. |  |  |  |  |
| Bubble/鼓氮气 | Performed by&Date/执行人及日期： |  |  |  |
| Bubble/鼓氮气 | Begin/开始： |  | End/结束： |  |
| Transcribed by&Date/转录人及日期： |  |  |  |  |
| Stir /搅拌 | Performed by&Date/执行人及日期： |  |  |  |
| Stir /搅拌 | Begin/开始： |  | End/结束： |  |
| Transcribed by&Date/转录人及日期： |  |  |  |  |
| Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point./通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  |  |  |
| Filter/过滤 | Performed by&Date/执行人及日期： |  |  |  |
|  | Checked by & Date/核实人及日期： |  |  |  |
|  | Begin/开始： | End/结束： |  |  |

| 3.8. （3）Fmoc- L-N-Me-Asn(Trt)-OH deprotection/Fmoc-N-甲基-N’-三苯甲基-天冬氨酰胺脱保护 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CAUTION/注意：DMF/N，N-二甲基甲酰胺：Irritation of eyes, skin and nose. May cause nausea. /刺激眼睛、皮肤和鼻子。可能引起恶心。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |  |  |
| 3.8.8 DMF addition /加N,N-二甲基甲酰胺 Add 546.05 kg (Acceptable weighing range: 529.67 ~ 562.43 kg) (12 V) DMF into synthesis reactor (TJ4S-1213-Sy15). 向合成釜（TJ4S-1213-Sy15）中加入546.05 kg（可接受称量范围：529.67 ~ 562.43 kg）（12 V）N,N-二甲基甲酰胺。 |  |  |  |  |  |  | Times/次数 |
|  |  |  |  |  |  |  | 1 |
| DMF addition / 加N,N-二甲基甲酰胺 |  | Performed by & Date/执行人及日期： |  |  |  |  |  |
| DMF addition / 加N,N-二甲基甲酰胺 |  | Begin/开始： |  | End/结束： |  | Qty/重量： |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |
| 3.8.9 Resin reswelling wash 2/溶胀后洗涤2 Turn on the nitrogen of synthesis reactor(TJ4S-1213-Sy15) to 5±2 m³/h (target: 5 m³/h). Bubble with nitrogen for 5±2 min (target: 5 min). Start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm). After stirring for 10±2 min (target: 10 min), stop stirring, turn off the nitrogen. Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point. 开启合成釜（TJ4S-1213-Sy15）下口氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min (目标：5 min)后，开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm)，搅拌10±2 min（目标：10 min）后，停止搅拌，关闭氮气。通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  |  |  |  |  | Times/次数 |
|  |  |  |  |  |  |  | 1 |
| Bubble/鼓氮气 |  |  | Performed by & Date/执行人及日期： |  |  |  |  |
| Bubble/鼓氮气 |  |  | Begin/开始： |  | End/结束： |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |

| 3.8. （3）Fmoc- L-N-Me-Asn(Trt)-OH deprotection/Fmoc-N-甲基-N’-三苯甲基-天冬氨酰胺脱保护 |  |  |  |
| --- | --- | --- | --- |
| Stir /搅拌 | Performed by & Date/执行人及日期： |  |  |
| Stir /搅拌 | Begin/开始： |  | End/结束： |
| Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point./通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  |  |
| Filter/过滤 | Performed by & Date/执行人及日期： |  |  |
|  | Checked by & Date/核实人及日期： |  |  |
|  | Begin/开始： | End/结束： |  |

| - （3）Fmoc-L-N-Me-Asn(Trt)-OH activation/ Fmoc-N-甲基-N’-三苯甲基-L-天冬氨酰胺活化 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CAUTION/注意：DMF/N，N-二甲基甲酰胺：Irritation of eyes, skin and nose. May cause nausea. /刺激眼睛、皮肤和鼻子。可能引起恶心。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |  |  |
| 3.9.1 Add DMF to the activation reactor/激活罐加N,N-二甲基甲酰胺 Add 182.02 kg (Acceptable weighing range: 176.56 ~ 187.48 kg) (4 V) DMF into activation tank (TJ4S-1213-Sy15-R01). Start the agitator of activation tank to 95±10 rpm (target: 95 rpm). 向激活罐（TJ4S-1213-Sy15-R01）中加入182.02 kg（可接受称量范围：176.56 kg ~ 187.48 kg ）（4 V）N，N-二甲基甲酰胺。开启激活罐搅拌转速至95±10 rpm (目标转速: 95 rpm)。 |  |  |  |  |  | Times/次数 |  |
|  |  |  |  |  |  | 1 |  |
| DMF addition / 加N,N-二甲基甲酰胺 |  |  |  | Performed by & Date/执行人及日期： |  |  |  |  |
| DMF addition / 加N,N-二甲基甲酰胺 |  |  |  | Begin/开始： | End/结束： | Qty/重量： |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |  |
| Start the agitator of activation tank to 95±10 rpm (target: 95 rpm)./ 开启激活罐搅拌转速至95±10 rpm (目标转速: 95 rpm)。 |  |  |  |  |  |  |  |  |
| Stir /搅拌 |  |  | Performed by & Date/执行人及日期： |  |  |  |  |  |
| Stir /搅拌 |  |  | Begin/开始： |  |  |  |  |  |
|  |  |  | The end time is documented in Ins 3.9.8/结束时间记录于3.9.8 |  |  |  |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |  |
| 3.9.2 Fmoc-L-N-Me-Asn(Trt)-OH addition /加Fmoc-N-甲基-N’-三苯甲基-L-天冬氨酰胺 Add Fmoc-L-N-Me-Asn(Trt)-OH (1.75 eq.) with powder feeding bags into activation tank (TJ4S-1213-Sy15-R01). 通过投料袋向激活罐（TJ4S-1213-Sy15-R01）中加入 kg Fmoc-N-甲基-N’-三苯甲基-L-天冬氨酰胺（Acceptable weighing range/可接受称量范围： kg ~ kg）(1.75 eq.)。 |  |  |  |  |  |  | Times/次数 |  |
|  |  |  |  |  |  |  | 1 |  |
| Before material charging, confirm the material is within the retest date/ the expiry date.Confirm whether the weight is adequate for use in the current charging. /加物料前，执行物料在复检日期/有效期之内，确认重量是否满足本次投料使用。 Before solid material charging, confirm/加固体物料前，确认： - Before performed, confirm there is no non-related material / equipment / instrument / tools / pipeline / document / record / label in the intended area /操作前，执行目标区域无不相关物料/设备/仪器/工具/管路/文件/记录/标签 - The outer packages are removed. If the outer package cannot be removed, but un-package material without damaging the package bag, and before charging, check no package fragment and other extraneous matter in material. /外包装移除；如果外包装不能移除，但在不破坏包装的情况下打开包装袋，投料前以及过程中同时检查执行物料中无包装碎片以及异物 Checked by & Date/核实人及日期： |  |  |  |  |  |  |  |  |

| 3.9. （3）Fmoc-L-N-Me-Asn(Trt)-OH activation/ Fmoc-N-甲基-N’-三苯甲基-L-天冬氨酰胺活化 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Prior to charging each amino acid, the operator shall perform a Raman scan to confirm the identity of the material. The Raman scan result shall be compared against the reference spectrum of the corresponding amino acid. Only materials that conform to the reference spectrum may be charged. If the Raman scan result does not conform, stop the operation, quarantine the material, and initiate a investigation per site SOP. Record the Raman scan result, operator signature, and date/time in the table below./ 每次投氨基酸前，操作人员应进行拉曼扫描以执行物料。拉曼扫描结果应与相应氨基酸的参考光谱进行比对。仅与参考光谱相符的物料方可投料。如拉曼扫描结果不相符，应停止操作，对物料进行隔离，并依据厂区 SOP 启动异常事件调查。在下表中记录拉曼扫描结果、操作人员签字及日期 / 时间。 |  |  |  |  |  |  |
| Fmoc-L-N-Me-Asn(Trt)-OH addition/ 加Fmoc-N-甲基-N’-三苯甲基-L-天冬氨酰胺 |  |  | Begin/开始： |  | End/结束： |  |
|  |  |  | Performed by & Date/执行人及日期： |  |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |
| Fmoc-L-N-Me-Asn(Trt)-OH confirmation/Fmoc-N-甲基-N’-三苯甲基-L-天冬氨酰胺确认：Control No./内部控制号：YL008617 |  |  |  |  |  |  |
| Check items/检查项 |  | Is it correct?/是否正确 |  | Recorded by & Date/ 记录人及日期 |  | Checked by & Date/ 核实人及日期 |
| Material name/物料名称 |  | Yes/是；No/否 |  |  |  |  |
| Project batch number/ 项目批次号 |  | Yes/是；No/否 |  |  |  |  |
| Operation number/ 操作号 |  | Yes/是；No/否 |  |  |  |  |
| Control No./内部控制号 |  | Yes/是；No/否 |  |  |  |  |
| Raman Scan Confirmation/拉曼扫描执行 |  | Conforms/相符 Does not conform/不相符 |  |  |  |  |
| No. /编号 | Net weight/净重 | All in/全部加入 |  | Recorded by & Date/ 记录人及日期 |  | Checked by & Date/ 核实人及日期 |
| / |  | Yes/是；No/否 |  |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |

| 3.9. （3）Fmoc-L-N-Me-Asn(Trt)-OH activation/ Fmoc-N-甲基-N’-三苯甲基-L-天冬氨酰胺活化 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| No. /编号 | Net weight/净重 | All in/全部加入 |  |  | Recorded by & Date/ 记录人及日期 | Checked by & Date/ 核实人及日期 |  |
| / |  | Yes/是；No/否 |  |  |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |  |
| Total/合计 |  | Confirm the total net weight is within the input range/执行合计净重在投料范围内 |  |  | Recorded by & Date/记录人及日期 | Checked by & Date/ 核实人及日期 |  |
|  |  | Yes/是；No/否 |  |  |  |  |  |
| After solid material charging, confirm/固体物料投料之后，确认： - The corresponding leftover material has been properly sealed and transferred to the staging area/相应的剩余物料已经恰当密封，转移到了暂存区域。 - The waste packaging material has been sealed and transferred to the corresponding waste staging area. /产生的包材废物已经密封并转移到了相应的废物暂存区域。 - The operation area/equipment surface has been cleaned/操作区域以及设备表面已经清洁。 Checked by & Date/核实人及日期： |  |  |  |  |  |  |  |
| 3.9.3 Agitation of activation tank /激活罐搅拌 Continue agitation of activation tank to 95±10 rpm (target: 95 rpm). Maintain stirring for at least 15 min. Confirm no visible solids and later use. 维持激活罐搅拌转速至95±10 rpm (目标转速: 95 rpm), 搅拌至少15 min，确认无明显固体待用。 |  |  |  |  |  |  | Times/次数 |
|  |  |  |  |  |  |  | 1 |
| Maintain &Stir /保持搅拌 |  |  | Performed by & Date/执行人及日期： |  |  |  |  |
|  |  |  | Begin/开始： |  |  |  |  |
|  |  |  | The end time is documented in Ins 3.9.8/结束时间记录于3.9.8 |  |  |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |
| Confirm no visible solid/确认无明显固体： |  |  |  | Confirmed by& Date /确认人及日期： |  |  |  |

| 3.9. （3）Fmoc-L-N-Me-Asn(Trt)-OH activation/ Fmoc-N-甲基-N’-三苯甲基-L-天冬氨酰胺活化 |  |  |  |  |
| --- | --- | --- | --- | --- |
| 3.9.4 Adjust activation tan temperature /激活罐调温 Adjust the activation tank (TJ4S-1213-Sy15-R01) mixture to 【20±5 ℃ (target 20 ℃) 】. /将激活罐(TJ4S-1213-Sy15-R01)中体系调温至【20±5 ℃（目标：20 ℃）】。 |  |  |  | Times/次数 |
|  |  |  |  | 1 |
| Adjust/调温 | Performed by & Date/执行人及日期： |  |  |  |
| Adjust/调温 |  | Begin/开始： | End/结束： |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |
| CAUTION/注意：OxymaPure is highly toxic, high-energy compound. /2-肟氰乙酸乙酯是高毒，高能化合物。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |
| 3.9.5 OxymaPure addition /加2-肟氰乙酸乙酯 Add OxymaPure (1.80 eq.)with powder feeding bags into activation tank (TJ4S-1213-Sy15-R01) at 【20±5 ℃ (target: 20 ℃】. 控温【20±5 ℃（目标温度：20℃）】，通过投料袋向激活罐（TJ4S-1213-Sy15-R01）中加入 kg 2-肟氰乙酸乙酯（Acceptable weighing range/可接受称量范围： kg ~ kg）(1.80 eq.)。 |  |  |  | Times/次数 |
|  |  |  |  | 1 |
| Before material charging, confirm the material is within the retest date/ the expiry date,Confirm whether the weight is adequate for use in the current charging. /加物料前，执行物料在复检日期/有效期之内，确认重量是否满足本次投料使用。 Before solid material charging, confirm/加固体物料前，确认： - Before operation performed, confirm there is no non-related material / equipment / instrument / tools / pipeline / document / record / label in the intended area /操作前；目标区域无不相关物料/设备/仪器/工具/管路/文件/记录/标签 - The outer packages are removed. If the outer package cannot be removed, but un-package material without damaging the package bag, and before charging, check no package fragment and other extraneous matter in material. /外包装移除；如果外包装不能移除，但在不破坏包装的情况下打开包装袋，投料前以及过程中同时检查执行物料中无包装碎片以及异物 Checked by & Date/核实人及日期： |  |  |  |  |
| Check and record that the solid charge port is free of solid accumulation from the previous step./ 检查并记录固体投料口无来自上一步骤的固体残留。 Checked by & Date/核实人及日期： |  |  |  |  |
| OxymaPure addition/加2-肟氰乙酸乙酯 |  | Performed by & Date/执行人及日期： |  |  |
|  |  | Begin/开始： | End/结束： |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |

| 3.9. （3）Fmoc-L-N-Me-Asn(Trt)-OH activation/ Fmoc-N-甲基-N’-三苯甲基-L-天冬氨酰胺活化 |  |  |  |  |
| --- | --- | --- | --- | --- |
| OxymaPure confirmation/2-肟氰乙酸乙酯确认：Control No./内部控制号：YL005180 |  |  |  |  |
| Check items/检查项 |  | Is it correct?/是否正确 | Recorded by & Date/ 记录人及日期 | Checked by & Date/ 核实人及日期 |
| Material name/物料名称 |  | Yes/是；No/否 |  |  |
| Project batch number/ 项目批次号 |  | Yes/是；No/否 |  |  |
| Operation number/ 操作号 |  | Yes/是；No/否 |  |  |
| Control No./内部控制号 |  | Yes/是；No/否 |  |  |
| No. /编号 | Net weight/净重 | All in/全部加入 | Recorded by & Date/ 记录人及日期 | Checked by & Date/ 核实人及日期 |
| / |  | Yes/是；No/否 |  |  |
| / |  | Yes/是；No/否 |  |  |
| / |  | Yes/是；No/否 |  |  |
| / |  | Yes/是；No/否 |  |  |
| / |  | Yes/是；No/否 |  |  |
| / |  | Yes/是；No/否 |  |  |
| / |  | Yes/是；No/否 |  |  |
| / |  | Yes/是；No/否 |  |  |
| / |  | Yes/是；No/否 |  |  |
| / |  | Yes/是；No/否 |  |  |
| / |  | Yes/是；No/否 |  |  |
| / |  | Yes/是；No/否 |  |  |
| Total/合计 |  | Confirm the total net weight is within the input range/执行合计净重在投料范围内 | Recorded by & Date/记录人及日期 | Checked by & Date/ 核实人及日期 |
|  |  | Yes/是；No/否 |  |  |

| 3.9. （3）Fmoc-L-N-Me-Asn(Trt)-OH activation/ Fmoc-N-甲基-N’-三苯甲基-L-天冬氨酰胺活化 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| After solid material charging, confirm/固体物料投料之后，确认： - The corresponding leftover material has been properly sealed and transferred to the staging area/相应的剩余物料已经恰当密封，转移到了暂存区域。 - The waste packaging material has been sealed and transferred to the corresponding waste staging area. /产生的包材废物已经密封并转移到了相应的废物暂存区域。 - The operation area/equipment surface has been cleaned/操作区域以及设备表面已经清洁。 Checked by & Date/核实人及日期： |  |  |  |  |  |  |
| Internal temperature/内部温度 |  | High values/最高值： |  | Low values/最低值： |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |
| CAUTION/注意：DMF/N，N-二甲基甲酰胺：Irritation of eyes, skin and nose. May cause nausea. /刺激眼睛、皮肤和鼻子。可能引起恶心。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |  |
| 3.9.6 Rinse activation tank /激活罐淋洗 Rinse the port of Activation tank (TJ4S-1213-Sy15-R01) with 91.01 kg DMF（Acceptable weighing range：88.28 ~ 93.74 kg）(2 V). 用91.01 kg N,N-二甲基甲酰胺淋（可接受称量范围：88.28 ~ 93.74 kg）(2 V)淋洗激活罐（TJ4S-1213-Sy15-R01）加料口。 |  |  |  |  |  | Times/次数 |
|  |  |  |  |  |  | 1 |
| Rinse with DMF/ N,N-二甲基甲酰胺淋洗 | Performed by & Date/执行人及日期： |  |  |  |  |  |
| Rinse with DMF/ N,N-二甲基甲酰胺淋洗 | Begin/开始： |  | End/结束： |  | Qty/重量： |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |

| 3.9. （3）Fmoc-L-N-Me-Asn(Trt)-OH activation/ Fmoc-N-甲基-N’-三苯甲基-L-天冬氨酰胺活化 |  |  |
| --- | --- | --- |
| DIC is a key EHS concern chemical, highly toxic, skin sensitizing, respiratory sensitizing. /N,N’二异丙基碳二亚胺为EHS重点关注化学品，高毒，皮肤致敏，呼吸致敏。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |
| 3.9.7 DIC addition /加N,N'-二异丙基碳二亚胺 Add DIC (2.00 eq.) into the activation tank (TJ4S-1213-Sy15-R01) at reference time of 10-30 min at 【20±5 ℃ (target: 20 ℃)】. The addition rate depends on temperature control. 控温【20±5 ℃ (目标温度：20 ℃)】，向激活罐（TJ4S-1213-Sy15-R01）中滴加 kg N,N'-二异丙基碳二亚胺（Acceptable weighing range/可接受称量范围： kg ~ kg）(2.00 eq.)（滴加参考时间10-30 min，以控温为主）。 Pump number/泵编号： Scale ID. No./秤编号： Material charging pipe number/物料管路编号： |  | Times/次数 |
|  |  | 1 |
| ★Note:The DIC addition temperature is a CPP./ 备注：DIC加料温度是关键控制参数。 Prior to DIC addition, the operator shall confirm that the reaction mixture temperature is within the Normal Operating Range (NOR) / N,N'-二异丙基碳二亚胺投料前，操作人员须执行反应体系温度处于正常操作范围（NOR）内: Checked by & Date/核实人及日期： ★Lower limit/低限: 15 oC ，High limit/高限: 25 oC Over-temperature warning value/超温预警设置数值： Lower limit/低限: 22 oC ，High limit/高限: 23 oC |  |  |
| Before material charging, confirm the material is within the retest date/ the expiry date, Confirm whether the weight is adequate for use in the current charging. /加物料前，执行物料在复检日期/有效期之内，确认重量是否满足本次投料使用。 Checked by & Date/核实人及日期： |  |  |

| 3.9. （3）Fmoc-L-N-Me-Asn(Trt)-OH activation/ Fmoc-N-甲基-N’-三苯甲基-天冬氨酰胺活化 |  |  |
| --- | --- | --- |
| Before drummed liquid material charging, visual check there is no other extraneous matter in material. /加桶装液体物料前，应目视执行桶内无异物。 Visual Examination of Foreign Matters for Drum Solvent/桶装溶剂异物目视检查: 1. Place the solvent drum in a well-lit area (or use a portable light source) to ensure adequate visibility of the liquid contents./ 将溶剂桶置于光线充足区域（或使用便携式光源），确保液体内容物可见性良好 2. Visually inspect the solvent through the drum bung opening or transparent sight glass for any visible foreign matters, including but not limited to particulates, fibers, sediments, or unusual turbidity./ 通过桶口或透明视镜目视检查溶剂中是否有可见异物，包括但不限于颗粒、纤维、沉淀物或异常浑浊。 3. Confirm that the solvent is clear and free of visible foreign matters before charging. If any foreign matter is observed, do not use the drum and follow the applicable handling procedure for subsequent disposition./ 投料前确认溶剂澄清且无可见异物。如发现任何异物，不得使用该桶，并依据处理流程进行后续处置。 Checked by & Date/核实人及日期： |  |  |
| Confirm liquid charging pipeline is released. /确认加料管路经过放行。 |  |  |
| Checked by & Date/核实人及日期： |  |  |
| Charging of drummed solvent: verify that the pipeline connections remain consistent before and after material feeding, then record the weighing reading 桶装溶剂溶液称量时，确认投料前后管路连接一致，读取称量示数。 Performed by & Date/执行人及日期： Checked by & Date/核实人及日期： |  |  |
| DIC /N,N’-二异丙基碳二亚胺：Control Number/内部控制号：YL003564 |  |  |
| Internal temperature/内部温度 | High values/最高值： | Low values/最低值： |
| Transcribed by & Date /转录人及日期： |  |  |
| If the target net weight≥the planned net weight, perform full drum charge; otherwise, perform partial (Non-full drum) charge. /当目标加料量≥计划加入净重，勾选进行整桶加料；否则勾选进行零头加料。 Note: Target net weight to be charged = Total target net weight – Total net weight charged. Expected tare weight of drum after charge = Gross weight of drum that to be charged - Target net weight to be charged. 还需加入净重目标值= 加料步骤目标总净重-已加入净重总量 加料完成后预计桶皮重=待加料桶毛重-还需加入净重目标值 |  |  |  |

| 3.9. （3）Fmoc-L-N-Me-Asn(Trt)-OH activation/ Fmoc-N-甲基-N’-三苯甲基-天冬氨酰胺活化 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Addition type加料式 |  | Lot number/批号 | Pre-charging qty calculation/投料前投料量计算 |  |  |  |  |  | Actual Charging Qty/实际投料量 |  |  |  |  |  |  |
|  |  |  | Total qty charged/已加入净重总量 |  | Target qty to be charged/还需加入净重目标值 | Gross qty of drum that to be charged/待加料桶毛重 | Expected net weight of drum after charge /加料完成后预计桶净重 | Expected tare weight of drum after charge/加料完成后预计桶皮重 | Before charging/ 投料前 |  | After charging/ 投料后 | Net weight/ 净重 | Performed by & Date/ 执行人及日期 | Checked by & Date/ 核实人及日期 |  |
| Full drum整桶 Non-full drum /非整桶 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Full drum整桶 Non-full drum /非整桶 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Total weight /合计重量 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DIC Addition/ N,N'-二异丙基碳二酰亚胺 |  |  |  | Begin/开始： |  |  |  |  |  | End/结束： |  |  |  |  |  |
| Transcribed by & Date/转录人及日期： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| 3.9. （3）Fmoc-L-N-Me-Asn(Trt)-OH activation/ Fmoc-N-甲基-N’-三苯甲基-天冬氨酰胺活化 |  |  |  |  |
| --- | --- | --- | --- | --- |
| 3.9.8 Fmoc-L-N-Me-Asn(Trt)-OH activation/ Fmoc-N-甲基-N’-三苯甲基-天冬氨酰胺活化 At 20±5 ℃ (target: 20 ℃). Continue agitation of activation tank to 95±10 rpm (target: 95 rpm). Maintain stirring for 20±5 min (target: 20 min) after addition, then stop stirring. 滴毕，控温20±5 ℃(目标温度：20 ℃)，维持激活罐搅拌转速至95±10 rpm (目标转速: 95 rpm)，继续搅拌20±5 min(目标时间：20 min)，停止搅拌。 |  |  | Times/次数 |
|  |  |  | 1 |
| Stir /搅拌 | Performed by & Date/执行人及日期： |  |  |
| Stir /搅拌 | End/结束 |  |  |
|  | The start time is documented in Ins3.9.1 /开始时间记录于3.9.1 |  |  |
| Transcribed by & Date/转录人及日期： |  |  |  |
| 3.9.8 Internal temperature/内部温度 | High values/最高值： | Low values/最低值： |  |
| Transcribed by & Date/转录人及日期： |  |  |  |

| - （3）Fmoc-L-N-Me-Asn(Trt)-OH coupling/Fmoc-N-甲基-N’-三苯甲基-L-天冬氨酰胺偶联 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 3.10.1 Activation mixture transfer /活化体系转移 Transfer the mixture in activation tank (TJ4S-1213-Sy15-R01) into synthesis reactor (TJ4S-1213-Sy15). 将激活罐（TJ4S-1213-Sy15-R01）中体系转移至合成釜（TJ4S-1213-Sy15）中。 |  |  |  |  |  |  | Times/次数 |
|  |  |  |  |  |  |  | 1 |
| ★Note:The coupling temperature is a CPP, and the coupling IPC hold temperature is a CPP./ 备注：偶联温度是CPP，偶联IPC维持温度是关键工艺参数。 The operator shall confirm that the reaction mixture temperature is within the Normal Operating Range (NOR) 【25±5oC (target: 25oC)】 /操作人员须执行反应体系温度处于正常操作范围（NOR）内【25±5oC (target: 25oC)】 Checked by & Date/核实人及日期： ★Lower limit/低限: 13 oC ，High limit/高限: 37 oC Over-temperature warning value/超温预警设置数值： Lower limit/低限: 21 oC ，High limit/高限: 28 oC |  |  |  |  |  |  |  |
| Transfer/转移 |  |  | Performed by & Date/执行人及日期： |  |  |  |  |
|  |  |  | Checked by & Date/核实人及日期： |  |  |  |  |
|  |  |  | Begin/开始： |  | End/结束： |  |  |
| CAUTION/注意：DMF/N，N-二甲基甲酰胺：Irritation of eyes, skin and nose. May cause nausea. /刺激眼睛、皮肤和鼻子。可能引起恶心。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |  |  |
| 3.10.2 DMF addition /加N,N-二甲基甲酰胺 Add 45.50 kg (Acceptable weighing range: 43.50 ~ 47.50 kg) (1 V) DMF by spray balls into activation tank (TJ4S-1213-Sy15-R01) to rinse the wall. 通过淋洗球向激活罐（TJ4S-1213-Sy15-R01）中加入45.50 kg（可接受称量范围：43.50 ~ 47.50 kg） (1 V) N,N-二甲基甲酰胺淋洗罐壁。 |  |  |  |  |  |  | Times/次数 |
|  |  |  |  |  |  |  | 1 |
| Rinse with DMF/ N,N-二甲基甲酰胺淋洗 |  | Performed by & Date/执行人及日期： |  |  |  |  |  |
|  |  | Begin/开始： |  | End/结束： |  | Qty/重量： |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |

| 3.10. （3）Fmoc-L-N-Me-Asn(Trt)-OH coupling/ Fmoc-N-甲基-N’-三苯甲基-L-天冬氨酰胺偶联 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 3.10.3 Transfer /转移 Turn on the nitrogen of Synthesis reactor (TJ4S-1213-Sy15) to 5±2 m³/h (target: 5 m³/h). Bubble with nitrogen for 5±2 min (target: 5 min). Start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm). then start the agitator. Transfer the mixture in activation tank (TJ4S-1213-Sy15-R01) into synthesis reactor (TJ4S-1213-Sy15). 合成釜（TJ4S-1213-Sy15）先开启下口氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min (目标：5 min)后，开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm)，将激活罐（TJ4S-1213-Sy15-R01）中体系转移至合成釜（TJ4S-1213-Sy15）中。 |  |  |  |  | Times/次数 |
|  |  |  |  |  | 1 |
| Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h). Bubble with nitrogen for 5±2 min (target :5 min) 开启合成釜下口鼓氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min (目标时间：5 min) |  |  |  |  |  |
| Bubble/鼓氮气 |  | Performed by & Date/执行人及日期： |  |  |  |
| Bubble/鼓氮气 |  | Begin/开始： |  |  |  |
|  |  | The end time is documented in Ins3.10.5 /结束时间记录于3.10.5 |  |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |
| Start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm). /开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm) |  |  |  |  |  |
| Stir /搅拌 |  | Performed by & Date/执行人及日期： |  |  |  |
| Stir /搅拌 |  | Begin/开始： |  |  |  |
|  |  | The end time is documented in Ins3.10.5 /结束时间记录于3.10.5 |  |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |
| Transfer/转移 |  |  | Performed by & Date/执行人及日期： |  |  |
|  |  |  | Checked by & Date/核实人及日期： |  |  |
| Transfer/转移 |  |  | Begin/开始： | End/结束： |  |
| 3.10.4 Adjust synthesis reactor temperature /合成釜调温 Adjust the synthesis reactor (TJ4S-1213-Sy15) mixture to【25±5 ℃ (target: 25 ℃)】. /将合成釜（TJ4S-1213-Sy15）体系调温至【25±5 ℃(目标温度：25 ℃)】. |  |  |  |  | Times/次数 |
|  |  |  |  |  | 1 |

| 3.10. （3）Fmoc-L-N-Me-Asn(Trt)-OH coupling/ Fmoc-N-甲基-N’-三苯甲基-L-天冬氨酰胺偶联 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Adjust/调温 | Performed by & Date/执行人及日期： |  |  |  |
| Adjust/调温 | Begin/开始： | End/结束： |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |
| 3.10.5 Fmoc-L-N-Me-Asn(Trt)-OH coupling/ Fmoc-N-甲基-N’-三苯甲基-L-天冬氨酰胺偶联 The mixture is allowed to react at【25±5 ℃ (target: 25 ℃)】. After 4 h±15 min (target: 4 h) (target：4 h), sample the mixture every 0.5~1 h for ninhydrin test analysis until two consecutive samples test as negative (Yellow to Light pink Report Negative= Complete coupling; Blue to Purple Positive= Incomplete coupling). And the mixture should be stirred for coupling time (6 h≤Coupling Time≤14 h). Stop stirring, turn off the nitrogen. Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point. 控温【25±5 ℃(目标温度：25 ℃)】反应，反应4 h±15 min（目标时间：4 h）后，取样进行茚三酮检测，每隔0.5~1.0 h取样一次，直至连续两次取样结果为阴性（黄色至浅粉色，报告阴性=偶联完全；蓝色至紫色，阳性=偶联不完全）后，且反应时间满足（6 h≤反应时间≤14 h）后，停止搅拌，关闭氮气。通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 Note: If resin adherence is found at other times during the reaction, rinse the reactor with 0.50~5.00 kg DMF each time. /备注：偶联过程中其它时间发现贴壁，每次使用0.50~5.00 kg N,N-二甲基甲酰胺淋洗，记录见附加页 |  |  | Times/次数 |
|  |  |  | 1 |

| 3.10. （3）Fmoc-L-N-Me-Asn(Trt)-OH coupling/ Fmoc-N-甲基-N’-三苯甲基-L-天冬氨酰胺偶联 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| The total reaction Fmoc-L- N-Me-Asn(Trt)-OH coupling time：NMT 840 min (Sourced from the holding time study in the PAR study report.)/Fmoc-N-甲基-N’-三苯甲基-L-天冬氨酰胺偶联总的反应时间不超过840min(来源于 PAR 研究报告中稳定时间研究)。 Over-temperature warning value/超温预警设置数值： Lower limit/低限: 22 oC ，High limit/高限: 28 oC |  |  |  |  |  |  |  |
| React/反应 |  |  | Performed by & Date/执行人及日期： |  |  |  |  |
|  |  |  | Begin/开始： |  | End/结束： |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |
| ★After coupling is completed, the operator shall confirm that all Critical Process Parameters (CPP) are within their respective Normal Operating Ranges (NOR)/ 偶联结束后，操作人员须确认所有关键工艺参数（CPP）均处于各自的正常操作范围（NOR）内 ★Note: The coupling temperature is a CPP, and the coupling IPC hold temperature is a CPP./备注：偶联温度是CPP，偶联IPC维持温度是关键工艺参数。 The operator shall confirm that the reaction mixture temperature is within the Normal Operating Range (NOR) :20~30℃ /操作人员须确认反应体系温度处于正常操作范围（NOR）内:20~30℃ □Yes/是 □No/否 Confirmed by & Date/确认人及日期： Checked by & Date/核实人及日期： |  |  |  |  |  |  |
| Internal temperature/内部温度 |  |  | High values/最高值： |  | Low values/最低值： |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |

| 3.10. （3）Fmoc-L-N-Me-Asn(Trt)-OH coupling/ Fmoc-N-甲基-N’-三苯甲基-L-天冬氨酰胺偶联 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Standard sampling method/标准取样方法 |  |  |  |  |  |  |
| Take 5-10 ml mixture and perform testing in accordance with the ninhydrin test record. During sampling, should wear nitrile gloves and 3M 6000 series Full-face piece respirator, 6006CN cartridges. 通过在线取样器取样5~10ml按照茚三酮检测记录进行检测。取样过程中，应该戴丁腈手套、3M 6000系列全面罩, 6006CN滤毒盒。 |  |  |  |  |  |  |
| Sampling number/ 取样编号 |  | Sampling time/取样时间 | Sampled by & Date/取样人及日期 | Negative or Positive/阴性或阳性 | Result filled by &Date/结果填写人及日期 | Checked by & Date/核实人及日期 |
| CPo140020- -01- -3.10.5-01 |  |  |  | Negative /阴性 Positive /阳性 |  |  |
| CPo140020- -01- -3.10.5-02 |  |  |  | Negative /阴性 Positive /阳性 |  |  |
| CPo140020- -01- -3.10.5-03 |  |  |  | Negative /阴性 Positive /阳性 |  |  |
| CPo140020- -01- -3.10.5-04 |  |  |  | Negative /阴性 Positive /阳性 |  |  |
| CPo140020- -01- -3.10.5-05 |  |  |  | Negative /阴性 Positive /阳性 |  |  |
| CPo140020- -01- -3.10.5-06 |  |  |  | Negative /阴性 Positive /阳性 |  |  |
| CPo140020- 01- -3.10.5-07 |  |  |  | Negative /阴性 Positive /阳性 |  |  |
| CPo140020- -01- -3.10.5-08 |  |  |  | Negative /阴性 Positive /阳性 |  |  |
| Note:For a failing coupling test.Continue hourly sampling up to the NMT 841 min hold limit; if still positive, stop, notify PM/QA and raise a deviation; no re-coupling without an approved deviation.偶联反应检测不合格的处置规则:按每小时 1 次的频率继续取样检测，直至达到不超过（NMT）841 min的保温时限；若到达时限后检测仍为阳性，停止反应，通知项目经理（PM）/ 质量保证部门（QA）并发起偏差报告；未经偏差批准，不得进行补偶联操作。 |  |  |  |  |  |  |  |
| Confirm the end point of the reaction and move to following operation. /确认反应终点后进行下一步操作。 The acceptance criterion is two consecutive negative test results./连续两次检测结果为阴性即为合格。 Confirmed by & Date/确认人及日期： Checked by & Date/核实人及日期： |  |  |  |  |  |  |

| 3.10. （3）Fmoc-L-N-Me-Asn(Trt)-OH coupling/ Fmoc-N-甲基-N’-三苯甲基-L-天冬氨酰胺偶联 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Stir /搅拌 |  |  | Performed by & Date/执行人及日期： |  |  |  |  |
| Stir /搅拌 |  |  | End/结束： |  |  |  |  |
|  |  |  | The start time is documented in Ins 3.10.3 /开始时间记录于3.10.3 |  |  |  |  |
| Transcribed by & Date/转录人及日期： |  |  |  |  |  |  |  |
| Bubble/鼓氮气 |  |  | Performed by & Date/执行人及日期： |  |  |  |  |
| Bubble/鼓氮气 |  |  | End/结束： |  |  |  |  |
|  |  |  | The start time is documented in Ins 3.10.3 /开始时间记录于3.10.3 |  |  |  |  |
| Transcribed by & Date/转录人及日期： |  |  |  |  |  |  |  |
| Internal temperature/内部温度 |  |  | High values/最高值： | Low values/最低值： |  |  |  |
| Transcribed by & Date/转录人及日期： |  |  |  |  |  |  |  |
| Stop the stirrer and turn off the nitrogen. Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point./ 停止搅拌，关闭氮气，通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  |  |  |  |  |  |
| Filter/过滤 |  |  | Performed by & Date/执行人及日期： |  |  |  |  |
|  |  |  | Checked by & Date/核实人及日期： |  |  |  |  |
|  |  |  | Begin/开始： |  |  | End/结束： |  |
| ★After coupling is completed, the operator shall confirm that all Critical Process Parameters (CPPs) are within their respective Normal Operating Ranges (NOR)/ 偶联结束后，操作人员须确认所有关键工艺参数（CPP）均处于各自的正常操作范围（NOR）内。 Checked by & Date/核实人及日期： |  |  |  |  |  |

| 3.10. （3）Fmoc-L-N-Me-Asn(Trt)-OH coupling/ Fmoc-N-甲基-N’-三苯甲基-L-天冬氨酰胺偶联 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| CAUTION/注意：DMF/N，N-二甲基甲酰胺：Irritation of eyes, skin and nose. May cause nausea. /刺激眼睛、皮肤和鼻子。可能引起恶心。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |  |
| 3.10.6 Activation Tank Rinse/激活罐洗涤 Add 364.03 kg (Acceptable weighing range：353.11 ~ 374.95 kg) (8 V) DMF into the activation tank (TJ4S-1213-Sy15-R01). Circulate and clean the activating liquid transfer pipeline 1, liquid charging port cleaning pipeline 2 and solid charging port cleaning pipeline 3, clean every pipeline for 2±1 min. 向激活罐（TJ4S-1213-Sy15-R01）中加入364.03 kg（可接受称量范围：353.11 ~ 374.95 kg） (8 V) N,N-二甲基甲酰胺，循环清洗激活液转移管路1，液体加料口清洗管路2和固体加料口清洗管路3，每条线路各清洗2±1 min。 |  |  |  |  | Times/次数 |  |
|  |  |  |  |  | 3 |  |
| Operation name/ 操作名称 |  | Time point/时间节点 | Clean the activating liquid transfer pipeline 1/循环清洗激活液转移管路1 | Liquid charging port cleaning pipeline 2 /液体加料口清洗管路2 | Solid charging port cleaning pipeline 3,/固体加料口清洗管路3 |  |
|  |  |  | Wash /洗涤1 | Wash /洗涤2 | Wash /洗涤3 |  |
| DMF addition/ 加N,N-二甲基甲酰胺 |  | Performed by & Date/执行人及日期： |  |  |  |  |
| DMF addition/ 加N,N-二甲基甲酰胺 |  | Begin/开始 |  |  |  |  |
|  |  | End/结束 |  |  |  |  |
|  |  | Qty/重量 |  |  |  |  |
|  |  | Transcribed by & Date /转录人及日期： |  |  |  |  |
| Circulate and clean/循环清洗 |  | Performed by & Date/执行人及日期： |  |  |  |  |

| 3.10. （3）Fmoc-L-N-Me-Asn(Trt)-OH coupling/ Fmoc-N-甲基-N’-三苯甲基-天冬氨酰胺偶联 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Operation name/ 操作名称 |  | Time point/时间节点 | Clean the activating liquid transfer pipeline 1/循环清洗激活液转移管路1 | Liquid charging port cleaning pipeline 2 /液体加料口清洗管路2 | Sloid charging port cleaning pipeline 3,/固体加料口清洗管路3 |  |
|  |  |  | Wash /洗涤1 | Wash /洗涤2 | Wash /洗涤3 |  |
| Circulate and clean/循环清洗 |  | Begin/开始 |  |  |  |  |
|  |  | End/结束 |  |  |  |  |
|  |  | Transcribed by & Date /转录人及日期： |  |  |  |  |
| 3.10.7 Post-coupling wash./偶联后洗涤 Transfer the mixture in activation tank (TJ4S-1213-Sy15-R01) to synthesis reactor (TJ4S-1213-Sy15). Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h). Bubble with nitrogen, after 5±2 min (target 5 min), Start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm). After stirring for 7±2 min (target：7 min), stop stirring, turn off the nitrogen. Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point. 将激活罐（TJ4S-1213-Sy15-R01）洗涤液转入合成釜 (TJ4S-1213-Sy15)。开启合成釜下口鼓氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min (目标时间：5 min)后开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm)，搅拌7±2 min（目标时间：7 min）后停止搅拌，关闭氮气。通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  |  |  |  | Times/次数 |
|  |  |  |  |  |  | 3 |
| Transfer the liquid form activation tank (TJ4S-1213-Sy15-R01) to the synthesis reactor (TJ4S-1213-Sy15) ./将激活罐(TJ4S-1213-Sy15-R01)中体系转移至合成釜(TJ4S-1213-Sy15)中。 |  |  |  |  |  |  |
| Operation name/ 操作名称 |  | Time point/时间节点 | Wash /洗涤1 | Wash /洗涤2 | Wash /洗涤3 |  |
| Transfer/转移 |  | Performed by & Date/执行人及日期： |  |  |  |  |
|  |  | Checked by & Date/核实人及日期 |  |  |  |  |
|  |  | Begin/开始 |  |  |  |  |
|  |  | End/结束 |  |  |  |  |

| 3.10. （3）Fmoc-L-N-Me-Asn(Trt)-OH coupling/ Fmoc-N-甲基-N’-三苯甲基-天冬氨酰胺偶联 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h), Bubble with nitrogen for 5±2 min (target: 5 min), Start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm).After stirring for 7±2 min (target: 7 min), stop stirring, turn off the nitrogen./ 开启合成釜下口鼓氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min（目标时间：5 min）后开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm)，搅拌7±2 min（目标时间：7 min）后，停止搅拌，关闭氮气。 |  |  |  |  |
| Operation name/ 操作名称 | Time point/时间节点 | Wash /洗涤1 | Wash /洗涤2 | Wash /洗涤3 |
| Bubble/鼓氮气 | Performed by & Date/执行人及日期： |  |  |  |
| Bubble/鼓氮气 | Begin/开始 |  |  |  |
|  | End/结束 |  |  |  |
|  | Transcribed by & Date /转录人及日期： |  |  |  |
| Stir /搅拌 | Performed by & Date/执行人及日期： |  |  |  |
| Stir /搅拌 | Begin/开始 |  |  |  |
|  | End/结束 |  |  |  |
|  | Transcribed by & Date /转录人及日期： |  |  |  |
| Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point/通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  |  |  |
| Filter/过滤 | Performed by & Date/执行人及日期： |  |  |  |
|  | Checked by & Date/核实人及日期 |  |  |  |
|  | Begin/开始 |  |  |  |
|  | End/结束 |  |  |  |

| - （4）Deprotection/脱保护 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CAUTION/注意：Piperidine哌啶：Strong local irritant and may cause permanent injury after short exposure to small amounts. /强烈的局部刺激，短时间少量接触可能造成永久性伤害。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |  |  |
| 3.11.1 Piperidine solution addition /加哌啶溶液 Adjust the jacket temperature of synthesis reactor (TJ4S-1213-Sy15) to 【25±5 oC (target: 25 oC). 】. Add 357.50 kg (Acceptable weighing range: 346.77 ~ 368.23 kg) (8 V)【20% (V/V) piperidine/DMF solution】 into the Synthesis reactor (TJ4S-1213-Sy15). 将合成釜(TJ4S-1213-Sy15)夹套调温至【25±5 ℃（目标温度：25 ℃）】。向合成釜（TJ4S-1213-Sy15）中加入357.50 kg（可接受称量范围：346.77 ~ 368.23 kg） (8 V)【20%哌啶/DMF (V/V)】溶液。 |  |  |  |  |  | Times/次数 |  |
|  |  |  |  |  |  | 1 |  |
| Adjust/调温 |  | Performed by & Date/执行人及日期： |  |  |  |  |  |
| Adjust/调温 |  | Begin/开始： |  | Adjust/调温 |  |  |  |
| Transcribed by & Date/转录人及日期： |  |  |  |  |  |  |  |
| Confirm and record that the 20%Piperidine solution concentration is within the NOR range./ 确认并记录20%哌啶溶液浓度在正常操作范围内：17.50%~22.50% 20%Piperidine solution concentration confirmation/20%哌啶溶液浓度确认：____________ Confirm and record that the 20%Piperidine solution temperature is within the NOR range./ 确认并记录20%哌啶溶液温度在正常操作范围内：20 oC ~30 oC 20%Piperidine solution temperature confirmation/20%哌啶溶液温度确认：____________ Confirmed by & Date/确认人及日期： Checked by & Date/核实人及日期： |  |  |  |  |  |  |  |
| Piperidine addition/加哌啶溶液 | Performed by & Date/执行人及日期： |  |  |  |  |  |  |
| Piperidine addition/ 加哌啶溶液 | Begin/开始： |  | End/结束： |  | Qty/重量： |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |

| 3.11. （4）Deprotection/脱保护 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 3.11.2 Adjust synthesis reactor temperature /合成釜调温 Turn on the nitrogen of Synthesis reactor (TJ4S-1213-Sy15) to 5±2 m³/h (target: 5 m³/h). Bubble with nitrogen for 5±2 min (target 5 min). Start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm). then start the stirrer, Adjust to 25±5 ℃ (target: 25 ℃）. 开启合成釜（TJ4S-1213-Sy15）下口鼓氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min (目标：5 min)后开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm)，调温至【25±5 ℃(目标温度：25 ℃)】。 |  |  |  | Times/次数 |
|  |  |  |  | 1 |
| Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h). Bubble with nitrogen for 5±2 min (target :5 min) 开启合成釜下口鼓氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min (目标时间：5 min) |  |  |  |  |
| Bubble/鼓氮气 |  | Performed by & Date/执行人及日期： |  |  |
| Bubble/鼓氮气 |  | Begin/开始： |  |  |
|  |  | The end time is documented in Ins 3.11.3/结束时间记录于3.11.3 |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |
| Start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm). /开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm) |  |  |  |  |  |
| Stir /搅拌 |  | Performed by & Date/执行人及日期： |  |  |
| Stir /搅拌 |  | Begin/开始： |  |  |
|  |  | The end time is documented in Ins 3.11.3/结束时间记录于3.11.3 |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |
| Adjust to 25±5 oC (target: 25 oC)./ 调温至25±5 ℃(目标温度：25 oC) |  |  |  |  |

| 3.11. （4）Deprotection/脱保护 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Adjust/调温 |  | Performed by & Date/执行人及日期： |  |  |  |  |  |
| Adjust/调温 |  | Begin/开始： | End/结束： |  |  |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |
| 3.11.3 Fmoc-L-Asn(Trt)-OH /Fmoc-N-三苯甲基-L-天冬酰胺脱保护 After stirring for 【2 h±15 min (target: 2 h) 】at 【25±5 ℃ (target: 25 ℃)】, stop stirring, turn off the nitrogen. Filter until nearly no liquid outflow. Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point. 控温【25±5 ℃(目标温度：25 ℃)】， 搅拌【2 h±15 min（目标时间：2 h）】后，停止搅拌，关闭氮气，滤除至基本通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 Over-temperature warning value/超温预警设置数值： Lower limit/低限: 22 oC ，High limit/高限: 28 oC |  |  |  |  | Times/次数 |  |
|  |  |  |  |  | 1 |  |
| ★Confirm and record that the 20%Piperidine solution temperature is within the NOR range 20 oC ~30 oC ./ 确认并记录20%哌啶溶液温度在正常操作范围内：20 oC ~30 oC。 20%Piperidine solution temperature confirmation/20%哌啶溶液温度确认：____________ Confirmed by& Date/确认人及日期： Checked by & Date/核实人及日期： |  |  |  |  |  |  |
| De-Fmoc/脱Fmoc &Stir /搅拌 |  |  | Performed by & Date/执行人及日期： |  |  |  |
| De-Fmoc/脱Fmoc &Stir /搅拌 |  |  | End/结束： |  |  |  |
|  |  |  | The start time is documented in Ins3.11.2 /开始时间记录于3.11.2 |  |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |

| 3.11. （4）Deprotection/脱保护 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ubble/鼓氮气 | Performed by & Date/执行人及日期：____________________ |  |  |
| Bubble/鼓氮气 | End/结束： |  |  |
|  | The start time is documented in Ins3.11.2 /开始时间记录于3.11.2 |  |  |
| Transcribed by & Date /转录人及日期：____________________ |  |  |  |
| Internal temperature/内部温度 |  | High values/最高值： |  | Low values/最低值： |
| Transcribed by & Date /转录人及日期：____________________ |  |  |  |  |
| Filter/过滤 |  | Performed by & Date/执行人及日期：____________________ |  |  |
|  |  | Checked by & Date/核实人及日期：____________________ |  |  |
|  |  | Begin/开始： | End/结束： |  |
| ★After De-Fmoc is completed, the operator shall confirm that all Critical Process Parameters (CPPs) are within their respective Normal Operating Ranges (NOR)/ 脱保护结束后，操作人员须确认所有关键工艺参数（CPP）均处于各自的正常操作范围（NOR）内 Checked by & Date/核实人及日期： |  |  |  |  |

| 3.11. （4）Deprotection/脱保护 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 3.11.4 Adjust synthesis reactor jacket temperature /合成釜夹套调温 Adjust the synthesis reactor (TJ4S-1213-Sy15) jacket to 【20±5 ℃ (target: 20 ℃)】. /将合成釜（TJ4S-1213-Sy15）夹套调温至【20±5 ℃（目标温度：20 ℃）】。 |  |  |  | Times/次数 |  |
|  |  |  |  | 1 |  |
| Adjust/调温 |  | Performed by & Date/执行人及日期：____________________ |  |  |  |
| Adjust/调温 |  | Begin/开始： | End/结束： |  |  |
| Transcribed by & Date /转录人及日期：____________________ |  |  |  |  |  |
| CAUTION/注意：IPAC/乙酸异丙酯： Vapors irritate eyes and respiratory tract; high concentrations can be anesthetic. Liquid irritates eyes but causes no serious injury; may cause dermatitis; no serious effects if swallowed. /蒸汽会刺激眼睛和呼吸道；高浓度的蒸汽可能是麻醉剂。液体刺激眼睛，但不会造成严重伤害；可能会引起皮炎；吞食不会造成严重影响。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |
| 3.11.5 Post-deprotection wash/脱保护后洗涤 Add 445.92 kg (Acceptable weighing range：432.54 ~ 4459.30 kg) (9.29 kg/kg, 10.64 V) isopropyl acetate into synthesis reactor (TJ4S-1213-Sy15). Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h). Bubble with nitrogen for 5±2 min (target 5 min). Start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm). After stirring for 5±2 min (target 5 min), stop stirring, turn off the nitrogen. Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point. 向合成釜（TJ4S-1213-Sy15）中加入445.92 kg（可接受称量范围：432.54 ~ 4459.30 kg）（9.29 kg/kg, 10.64 V）乙酸异丙酯，开启下口氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min (目标时间：5 min)后开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm)，搅拌5±2 min (目标时间：5 min)后，停止搅拌，关闭氮气。通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  |  | Times/次数 |  |
|  |  |  |  | 4 |  |
| ★Note: IPAC quantity is a CPP./备注：乙酸异丙酯用量是关键工艺参数。 ★The operator shall confirm that IPAC quantity is within the Normal Operating Range (NOR) 【432.54 kg~459.30 kg】/操作人员须确认乙酸异丙酯用量处于正常操作范围（NOR）内：【432.54 kg~459.30 kg】 Checked by & Date/核实人及日期： |  |  |  |  |  |

| 3.11. （4）Deprotection/脱保护 |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operation name/操作名称 |  |  | Time point/时间节点 |  | Wash /洗涤1 |  | Wash /洗涤2 |  | Wash /洗涤3 |  | Wash /洗涤4 |  |
| Add 419.04 kg IPAc (10 V) by spray balls into the synthesis reactor (TJ4S-1213-Sy15)./ 向合成釜(TJ4S-1213-Sy15)中加419.04 kg乙酸异丙酯（10 V)）（Acceptable weighing range/可接受称量范围：406.47~431.61 kg） |  |  |  |  |  |  |  |  |  |  |  |  |
| IPAC addition/加乙酸异丙酯 |  |  | Performed by & Date/执行人及日期： |  |  |  |  |  |  |  |  |  |
| IPAC addition/加乙酸异丙酯 |  |  | Begin/开始 |  |  |  |  |  |  |  |  |  |
|  |  |  | End/结束 |  |  |  |  |  |  |  |  |  |
|  |  |  | Qty/重量 |  |  |  |  |  |  |  |  |  |
|  |  |  | Transcribed by & Date /转录人及日期: |  |  |  |  |  |  |  |  |  |
| Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h). Bubble with nitrogen for 5±2 min (target: 5 min). Start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm). Stir for 5±2 min (target: 5 min), stop stirring, turn off the nitrogen./ 开启合成釜下口鼓氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min（目标时间：5 min）后开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm)，搅拌5±2 min（目标时间：5 min）后，停止搅拌，关闭氮气。 |  |  |  |  |  |  |  |  |  |  |  |  |
| Bubble/鼓氮气 |  |  | Performed by & Date/执行人及日期： |  |  |  |  |  |  |  |  |  |
| Bubble/鼓氮气 |  |  | Begin/开始 |  |  |  |  |  |  |  |  |  |
|  |  |  | End/结束 |  |  |  |  |  |  |  |  |  |
|  |  |  | Transcribed by & Date /转录人及日期: |  |  |  |  |  |  |  |  |  |
| Stir /搅拌 |  |  | Performed by & Date/执行人及日期： |  |  |  |  |  |  |  |  |  |
| Stir /搅拌 |  |  | Begin/开始 |  |  |  |  |  |  |  |  |  |
|  |  |  | End/结束 |  |  |  |  |  |  |  |  |  |
|  |  |  | Transcribed by & Date/转录人及日期 |  |  |  |  |  |  |  |  |  |

| 3.11. （4）Deprotection/脱保护 |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operation name/操作名称 |  |  | Time point/时间节点 |  | Wash /洗涤1 |  | Wash /洗涤2 | Wash /洗涤3 |  | Wash /洗涤4 |  |
| Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point./通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  |  |  |  |  |  |  |  |  |  |
| Filter/过滤 |  | Performed by & Date/ 执行人及日期： |  |  |  |  |  |  |  |  |  |
|  |  | Checked by & Date/ 核实人及日期： |  |  |  |  |  |  |  |  |  |
|  |  | Begin/开始 |  |  |  |  |  |  |  |  |  |
|  |  | End/结束 |  |  |  |  |  |  |  |  |  |
| CAUTION/注意：DMF/N，N-二甲基甲酰胺：Irritation of eyes, skin and nose. May cause nausea. /刺激眼睛、皮肤和鼻子。可能引起恶心。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |  |  |  |  |  |

| 3.11. （4）Deprotection/脱保护 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3.11.6 DMF addition /加N,N-二甲基甲酰胺 Add 546.05 kg (Acceptable weighing range: 529.67 kg ~ 562.43 kg) (12 V) DMF into synthesis reactor (TJ4S-1213-Sy15). 向合成釜（TJ4S-1213-Sy15）中加入546.05 kg（可接受称量范围：529.67 kg ~ 562.43 kg）（12 V）N,N-二甲基甲酰胺。 |  |  |  |  |  | Times/次数 |
|  |  |  |  |  |  | 1 |
| DMF addition / 加N,N-二甲基甲酰胺 | Performed by & Date/执行人及日期：____________________ |  |  |  |  |  |
| DMF addition / 加N,N-二甲基甲酰胺 | Begin/开始： |  | End/结束： |  | Qty/重量： |  |
| Transcribed by & Date /转录人及日期：____________________ |  |  |  |  |  |  |
| 3.11.7 Resin reswelling wash 1/溶胀洗涤1 Turn on the nitrogen of synthesis reactor (TJ4S-1213-Sy15) to 5±2 m³/h (target: 5 m³/h). Bubble with nitrogen for 5±2 min (target: 5 min). Start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm). After stirring for 10±2 min (target: 10 min), stop stirring, turn off the nitrogen. Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point.开启合成釜（TJ4S-1213-Sy15）下口氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min (目标：5 min)后，开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm)，搅拌10±2 min（目标：10 min）后，停止搅拌，关闭氮气。通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  |  |  |  | Times/次数 |
|  |  |  |  |  |  | 1 |
| Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h) for 5±2 min (target: 5 min), Start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm). Stir for 10±2 min (target: 10 min). Stop stirring, turn off the nitrogen./ 开启合成釜下口鼓氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min（目标时间：5 min）后，开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm)，搅拌10±2 min（目标时间:10 min）后，停止搅拌，关闭氮气. |  |  |  |  |  |  |  |
| Bubble/鼓氮气 |  |  | Performed by & Date/执行人及日期：____________________ |  |  |  |  |
| Bubble/鼓氮气 |  |  | Begin/开始： |  | End/结束： |  |  |
| Transcribed by & Date /转录人及日期: ____________________ |  |  |  |  |  |  |  |
| Stir /搅拌 |  |  | Performed by & Date/执行人及日期：____________________ |  |  |  |  |
| Stir /搅拌 |  |  | Begin/开始： |  | End/结束： |  |  |
| Transcribed by & Date /转录人及日期: ____________________ |  |  |  |  |  |  |  |

| 3.11. （4）Deprotection/脱保护 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point./通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  |  |  |  |  |  |  |
| Filter/过滤 |  |  | Performed by & Date/执行人及日期：____________________ |  |  |  |  |  |
|  |  |  | Checked by & Date/核实人及日期：____________________ |  |  |  |  |  |
|  |  |  | Begin/开始： |  | End/结束： |  |  |  |
| CAUTION/注意：DMF/N，N-二甲基甲酰胺：Irritation of eyes, skin and nose. May cause nausea. /刺激眼睛、皮肤和鼻子。可能引起恶心。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |  |  |  |
| 3.11.8 DMF addition/加N,N-二甲基甲酰胺 Add 546.05 kg (Acceptable weighing range: 529.67 kg ~ 562.43 kg) (12 V) DMF into synthesis reactor (TJ4S-1213-Sy15). 向合成釜（TJ4S-1213-Sy15）中加入546.05 kg（可接受称量范围：529.67 kg ~ 562.43 kg）（12 V）N,N-二甲基甲酰胺。 |  |  |  |  |  | Times/次数 |  |
|  |  |  |  |  |  | 1 |  |
| DMF addition / 加N,N-二甲基甲酰胺 |  | Performed by & Date/执行人及日期：____________________ |  |  |  |  |  |  |
| DMF addition / 加N,N-二甲基甲酰胺 |  | Begin/开始： |  | End/结束： |  | Qty/重量： |  |  |
| Transcribed by & Date /转录人及日期：____________________ |  |  |  |  |  |  |  |  |
| 3.11.9 Resin reswelling wash 1/溶胀洗涤2 Turn on the nitrogen of synthesis reactor (TJ4S-1213-Sy15) to 5±2 m³/h (target: 5 m³/h). Bubble with nitrogen for 5±2 min (target: 5 min). Start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm). After stirring for 10±2 min (target: 10 min), stop stirring, turn off the nitrogen. Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point. 开启合成釜（TJ4S-1213-Sy15）下口氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min (目标：5 min)后，开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm)，搅拌10±2 min（目标：10 min）后，停止搅拌，关闭氮气。通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  |  |  |  |  | Times/次数 |  |
|  |  |  |  |  |  |  | 1 |  |

| 3.11. （4）Deprotection/脱保护 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h) for 5±2 min (target: 5 min), Start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm). Stir for 10±2 min (target: 10 min). Stop stirring, turn off the nitrogen./ 开启合成釜下口鼓氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min（目标时间：5 min）后，开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm)，搅拌10±2 min（目标时间:10 min）后，停止搅拌，关闭氮气。 |  |  |  |
| Bubble/鼓氮气 | Performed by & Date/执行人及日期：____________________ |  |  |
| Bubble/鼓氮气 | Begin/开始： |  | End/结束： |
| Transcribed by & Date /转录人及日期: ____________________ |  |  |  |
| Stir /搅拌 | Performed by & Date/执行人及日期：____________________ |  |  |
| Stir /搅拌 | Begin/开始： |  | End/结束： |
| Transcribed by & Date /转录人及日期: ____________________ |  |  |  |
| Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point./通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  |  |
| Filter/过滤 | Performed by & Date/执行人及日期：____________________ |  |  |
|  | Checked by & Date/核实人及日期：____________________ |  |  |
|  | Begin/开始： | End/结束： |  |

| - （4）Fmoc-L-Asn(Trt)-OH activation/Fmoc-N-三苯甲基-L-天冬酰胺活化 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CAUTION/注意：DMF/N，N-二甲基甲酰胺：Irritation of eyes, skin and nose. May cause nausea. /刺激眼睛、皮肤和鼻子。可能引起恶心。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期：____________________ |  |  |  |  |  |  |  |
| 3.12.1 Add DMF to the activation reactor/激活罐加N,N-二甲基甲酰胺： Add 182.02 kg (Acceptable weighing range: 176.56 ~ 187.48 kg) (4 V) DMF into activation tank (TJ4S-1213-Sy15-R01). Start the agitator of activation tank to 95±10 rpm (target: 95 rpm). 向激活罐（TJ4S-1213-Sy15-R01）中加入182.02 kg（可接受称量范围：176.56 ~ 187.48 kg ）（4 V）N，N-二甲基甲酰胺。开启激活罐搅拌转速至95±10 rpm (目标转速: 95 rpm)。 |  |  |  |  |  | Times/次数 |  |
|  |  |  |  |  |  | 1 |  |
| Add 182.02 kg DMF (Acceptable weighing range: 176.56 ~ 187.48 kg) (4 V) by spray balls into activation tank (TJ4S-1213-Sy15-R01).通过淋洗球向激活罐(TJ4S-1213-Sy15-R01)中加入182.02 kg N, N-二甲基甲酰胺（可接受称量范围：176.56~187.48 kg）(4 V)。 |  |  |  |  |  |  |  |
| DMF addition / 加N,N-二甲基甲酰胺 |  | Performed by & Date/执行人及日期：____________________ |  |  |  |  |  |
| DMF addition / 加N,N-二甲基甲酰胺 |  | Begin/开始： | End/结束： | Qty/重量： |  |  |  |
| Transcribed by & Date /转录人及日期：____________________ |  |  |  |  |  |  |  |
| Start the agitator of activation tank to 95±10 rpm (target: 95 rpm)./ 开启激活罐搅拌转速至95±10 rpm (目标转速: 95 rpm)。 |  |  |  |  |  |  |  |
| Stir /搅拌 | Performed by & Date/执行人及日期：____________________ |  |  |  |  |  |  |
| Stir /搅拌 | Begin/开始： |  |  |  |  |  |  |
|  | The end time is documented in Ins 3.12.8/结束时间记录于3.12.8 |  |  |  |  |  |  |
| Transcribed by & Date /转录人及日期：____________________ |  |  |  |  |  |  |  |
| 3.12.2 Fmoc-L-Asn(Trt)-OH addition /加Fmoc-N-三苯甲基-L-天冬酰胺 Add Fmoc-L-Asn(Trt)-OH (2.50 eq.) with powder feeding bags into activation tank (TJ4S-1213-Sy15-R01).通过投料袋向激活罐（TJ4S-1213-Sy15-R01）中加入 kg Fmoc-N-三苯甲基-L-天冬酰胺（Acceptable weighing range/可接受称量范围： kg ~ kg）(2.50 eq.)。 |  |  |  |  |  | Times/次数 |  |
|  |  |  |  |  |  | 1 |  |

| 3.12. （4）Fmoc-L-Asn(Trt)-OH activation/Fmoc-N-三苯甲基-L-天冬酰胺活化 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Before material charging, confirm the material is within the retest date/ the expiry date,Confirm whether the weight is adequate for use in the current charging. /加物料前，执行物料在复检日期/有效期之内，确认重量是否满足本次投料使用。 Before solid material charging, confirm/加固体物料前，确认： - Before performed, confirm there is no non-related material / equipment / instrument / tools / pipeline / document / record / label in the intended area /操作前，执行目标区域无不相关物料/设备/仪器/工具/管路/文件/记录/标签 - The outer packages are removed. If the outer package cannot be removed, but un-package material without damaging the package bag, and before charging, check no package fragment and other extraneous matter in material. /外包装移除；如果外包装不能移除，但在不破坏包装的情况下打开包装袋，投料前以及过程中同时检查执行物料中无包装碎片以及异物 Checked by & Date/核实人及日期： |  |  |  |  |  |  |
| Prior to charging each amino acid, the operator shall perform a Raman scan to confirm the identity of the material. The Raman scan result shall be compared against the reference spectrum of the corresponding amino acid. Only materials that conform to the reference spectrum may be charged. If the Raman scan result does not conform, stop the operation, quarantine the material, and initiate a investigation per site SOP. Record the Raman scan result, operator signature, and date/time in the table below./ 每次投氨基酸前，操作人员应进行拉曼扫描以执行物料。拉曼扫描结果应与相应氨基酸的参考光谱进行比对。仅与参考光谱相符的物料方可投料。如拉曼扫描结果不相符，应停止操作，对物料进行隔离，并依据厂区 SOP 启动异常事件调查。在下表中记录拉曼扫描结果、操作人员签字及日期 / 时间。 |  |  |  |  |  |  |
| Fmoc-L-Asn(Trt)-OH addition/ 加Fmoc-N-三苯甲基-L-天冬酰胺 |  |  | Performed by & Date/执行人及日期：____________________ |  |  |  |
|  |  |  | Begin/开始： |  | End/结束： |  |
| Transcribed by & Date /转录人及日期：____________________ |  |  |  |  |  |  |
| Fmoc-L-Asn(Trt)-OH confirmation/ Fmoc-N-三苯甲基-L-天冬酰胺执行：Control No./内部控制号：YL004843 |  |  |  |  |  |  |
| Check items/检查项 |  | Is it correct?/是否正确 |  | Recorded by & Date/ 记录人及日期 |  | Checked by & Date/ 核实人及日期 |
| Material name/物料名称 |  | Yes/是；No/否 |  |  |  |  |
| Project batch number/ 项目批次号 |  | Yes/是；No/否 |  |  |  |  |
| Operation number/ 操作号 |  | Yes/是；No/否 |  |  |  |  |
| Control No./内部控制号 |  | Yes/是；No/否 |  |  |  |  |
| Raman Scan Confirmation/拉曼扫描执行 |  | Conforms/相符 Does not conform/不相符 |  |  |  |  |
| No. /编号 | Net weight/净重 | All in/全部加入 |  | Recorded by & Date/ 记录人及日期 |  | Checked by & Date/ 核实人及日期 |
| / |  | Yes/是；No/否 |  |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |

| 3.12. （4）Fmoc-L-Asn(Trt)-OH activation/Fmoc-N-三苯甲基-L-天冬酰胺活化 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| No. /编号 | Net weight/净重 | All in/全部加入 |  |  | Recorded by & Date/ 记录人及日期 | Checked by & Date/ 核实人及日期 |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |  |  |  |
| Total/合计 |  | Confirm the total net weight is within the input range/执行合计净重在投料范围内 |  |  | Recorded by & Date/记录人及日期 | Checked by & Date/ 核实人及日期 |  |  |  |
|  |  | Yes/是；No/否 |  |  |  |  |  |  |  |
| After solid material charging, confirm/固体物料投料之后，确认： - The corresponding leftover material has been properly sealed and transferred to the staging area/相应的剩余物料已经恰当密封，转移到了暂存区域。 - The waste packaging material has been sealed and transferred to the corresponding waste staging area. /产生的包材废物已经密封并转移到了相应的废物暂存区域。 - The operation area/equipment surface has been cleaned/操作区域以及设备表面已经清洁。 Checked by & Date/核实人及日期： |  |  |  |  |  |  |  |  |  |
| 3.12.3 Agitation of activation tank /激活罐搅拌 Continue agitation of activation tank to 95±10 rpm (target: 95 rpm). Maintain stirring for at least 15 min. Confirm no visible solids and later use. 维持激活罐搅拌转速至95±10 rpm (目标转速: 95 rpm), 搅拌至少15 min，确认无明显固体待用。 |  |  |  |  |  |  | Times/次数 |  |  |
|  |  |  |  |  |  |  | 1 |  |  |
| Maintain &Stir /保持搅拌 |  |  | Performed by & Date/执行人及日期： |  |  |  |  |  |  |
|  |  |  | Begin/开始： |  |  |  |  |  |  |
|  |  |  | The end time is documented in Ins 3.12.8/结束时间记录于3.12.8 |  |  |  |  |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |  |  |
| Confirm no visible solid/确认无明显固体： |  |  |  | Confirmed by& Date /确认人及日期： |  |  |  |  |

| 3.12. （4）Fmoc-L-Asn(Trt)-OH activation/Fmoc-N-三苯甲基-L-天冬酰胺活化 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 3.12.4 Adjust activation tank temperature /激活罐调温 Adjust the activation tank (TJ4S-1213-Sy15-R01) mixture to 【20±5 ℃ (target: 20 ℃)】. /将激活罐(TJ4S-1213-Sy15-R01)中体系调温至【20±5 ℃（目标温度：20 ℃）】。 |  |  |  | Times/次数 |  |  |
|  |  |  |  | 1 |  |  |
| Adjust/调温 | Performed by & Date/执行人及日期：____________________ |  |  |  |  |  |
|  | Begin/开始： | End/结束： |  |  |  |  |
| Transcribed by & Date /转录人及日期：____________________ |  |  |  |  |  |  |
| CAUTION/注意：OxymaPure is highly toxic, high-energy compound. /2-肟氰乙酸乙酯是高毒，高能化合物。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |  |
| 3.12.5 OxymaPure addition /加2-肟氰乙酸乙酯 Add OxymaPure (2.60 eq.) with powder feeding bags into activation tank (TJ4S-1213-Sy15-R01) at 【20±5 ℃ (target: 20 ℃】. 控温【20±5 ℃（目标温度：20 ℃）】，通过投料袋向激活罐（TJ4S-1213-Sy15-R01）中加入 kg 2-肟氰乙酸乙酯（Acceptable weighing range/可接受称量范围： kg ~ kg）(2.60 eq.)。 |  |  |  | Times/次数 |  |  |
|  |  |  |  | 1 |  |  |
| Before material charging, confirm the material is within the retest date/ the expiry date,Confirm whether the weight is adequate for use in the current charging. /加物料前，执行物料在复检日期/有效期之内，确认重量是否满足本次投料使用。 Before solid material charging, confirm/加固体物料前，确认： - Before operation performed, confirm there is no non-related material / equipment / instrument / tools / pipeline / document / record / label in the intended area /操作前；目标区域无不相关物料/设备/仪器/工具/管路/文件/记录/标签 - The outer packages are removed. If the outer package cannot be removed, but un-package material without damaging the package bag, and before charging, check no package fragment and other extraneous matter in material. /外包装移除；如果外包装不能移除，但在不破坏包装的情况下打开包装袋，投料前以及过程中同时检查执行物料中无包装碎片以及异物 Checked by & Date/核实人及日期： |  |  |  |  |  |  |
| Check and record that the solid charge port is free of solid accumulation from the previous step./ 检查并记录固体投料口无来自上一步骤的固体残留。 Checked by & Date/核实人及日期： |  |  |  |  |  |  |
| OxymaPure addition/加2-肟氰乙酸乙酯 |  | Performed by & Date/执行人及日期：____________________ |  |  |  |  |
|  |  | Begin/开始： | End/结束： |  |  |  |
| Transcribed by & Date /转录人及日期：____________________ |  |  |  |  |  |  |

| 3.12. （4）Fmoc-L-Asn(Trt)-OH activation/Fmoc-N-三苯甲基-L-天冬酰胺活化 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| OxymaPure confirmation/2-肟氰乙酸乙酯确认：Control No./内部控制号：YL005180 |  |  |  |  |  |
| Check items/检查项 |  | Is it correct?/是否正确 | Recorded by & Date/ 记录人及日期 | Checked by & Date/ 核实人及日期 |  |  |
| Material name/物料名称 |  | Yes/是；No/否 |  |  |  |  |
| Project batch number/ 项目批次号 |  | Yes/是；No/否 |  |  |  |  |
| Operation number/ 操作号 |  | Yes/是；No/否 |  |  |  |  |
| Control No./内部控制号 |  | Yes/是；No/否 |  |  |  |  |
| No. /编号 | Net weight/净重 | All in/全部加入 | Recorded by & Date/ 记录人及日期 | Checked by & Date/ 核实人及日期 |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |
| Total/合计 |  | Confirm the total net weight is within the input range/执行合计净重在投料范围内 | Recorded by & Date/记录人及日期 | Checked by & Date/ 核实人及日期 |  |  |
|  |  | Yes/是；No/否 |  |  |  |  |

| 3.12. （4）Fmoc-L-Asn(Trt)-OH activation/Fmoc-N-三苯甲基-L-天冬酰胺活化 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| After solid material charging, confirm/固体物料投料之后，确认： - The corresponding leftover material has been properly sealed and transferred to the staging area/相应的剩余物料已经恰当密封，转移到了暂存区域。 - The waste packaging material has been sealed and transferred to the corresponding waste staging area. /产生的包材废物已经密封并转移到了相应的废物暂存区域。 - The operation area/equipment surface has been cleaned/操作区域以及设备表面已经清洁。 Checked by & Date/核实人及日期： |  |  |  |  |  |
| CAUTION/注意：DMF/N，N-二甲基甲酰胺：Irritation of eyes, skin and nose. May cause nausea. /刺激眼睛、皮肤和鼻子。可能引起恶心。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |
| 3.12.6 Rinse activation tank /淋洗激活罐 Rinse the port of Activation tank (TJ4S-1213-Sy15-R01) with 91.01 kg（Acceptable weighing range：88.28 ~ 93.74 kg）（2 V）DMF. 用91.01 kg（可接受称量范围：88.28 ~ 93.74 kg）（2 V）N,N-二甲基甲酰胺淋洗激活罐（TJ4S-1213-Sy15-R01）加料口。 |  |  |  | Times/次数 |  |
|  |  |  |  | 1 |  |
| Rinse with DMF/ N,N-二甲基甲酰胺淋洗 | Performed by & Date/执行人及日期：____________________ |  |  |  |  |
| Rinse with DMF/ N,N-二甲基甲酰胺淋洗 | Begin/开始： | End/结束： | Qty/重量： |  |  |
| Transcribed by & Date /转录人及日期：____________________ |  |  |  |  |  |

| 3.12. （4）Fmoc-L-Asn(Trt)-OH activation/Fmoc-N-三苯甲基-L-天冬酰胺活化 |  |  |
| --- | --- | --- |
| CAUTION/注意：DIC is a key EHS concern chemical, highly toxic, skin sensitizing, respiratory sensitizing. /N,N’二异丙基碳二亚胺为EHS重点关注化学品，高毒，皮肤致敏，呼吸致敏。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |
| 3.12.7 DIC addition /加N,N'-二异丙基碳二亚胺 Add DIC(2.90 eq.) into the activation tank (TJ4S-1213-Sy15-R01) at 【20±5 ℃ (target: 20 ℃)】. Reference addition time is 10 - 30 min. The addition rate depends on temperature control. 控温【20±5 ℃(目标：20 ℃)】，向激活罐（TJ4S-1213-Sy15-R01）中滴加 kg N,N'-二异丙基碳二亚胺（Acceptable weighing range/可接受称量范围： kg ~ kg）(2.90 eq.)，滴加参考时间10 - 30 min，以控温为主。 Pump number/泵编号： Scale ID. No./秤编号： Material charging pipe number/物料管路编号： |  | Times/次数 |
|  |  | 1 |
| ★Note:The DIC addition temperature is a CPP./ 备注：N,N'-二异丙基碳二亚胺加料温度是关键控制参数。 Prior to DIC addition, the operator shall confirm that the reaction mixture temperature is within the Normal Operating Range (NOR) / N,N'-二异丙基碳二亚胺投料前，操作人员须执行反应体系温度处于正常操作范围（NOR）内: Checked by & Date/核实人及日期： |  |  |
| ★Lower limit/低限: 15 °C ，High limit/高限: 25 °C Over-temperature warning value/超温预警设置数值： Lower limit/低限: 17 °C ，High limit/高限: 23 °C |  |  |
| Before material charging, confirm the material is within the retest date/ the expiry date, Confirm whether the weight is adequate for use in the current charging. /加物料前，执行物料在复检日期/有效期之内，确认重量是否满足本次投料使用。 Checked by & Date/核实人及日期： |  |  |

| 3.12. （4）Fmoc-L-Asn(Trt)-OH activation/Fmoc-N-三苯甲基-L-天冬酰胺活化 |  |  |  |
| --- | --- | --- | --- |
| Before drummed liquid material charging, visual check there is no other extraneous matter in material. /加桶装液体物料前，应目视执行桶内无异物。 Visual Examination of Foreign Matters for Drum Solvent/桶装溶剂异物目视检查: 1. Place the solvent drum in a well-lit area (or use a portable light source) to ensure adequate visibility of the liquid contents./ 将溶剂桶置于光线充足区域（或使用便携式光源），确保液体内容物可见性良好 2. Visually inspect the solvent through the drum bung opening or transparent sight glass for any visible foreign matters, including but not limited to particulates, fibers, sediments, or unusual turbidity./ 通过桶口或透明视镜目视检查溶剂中是否有可见异物，包括但不限于颗粒、纤维、沉淀物或异常浑浊。 3. Confirm that the solvent is clear and free of visible foreign matters before charging. If any foreign matter is observed, do not use the drum and follow the applicable handling procedure for subsequent disposition./ 投料前确认溶剂澄清且无可见异物。如发现任何异物，不得使用该桶，并依据处理流程进行后续处置。 Checked by & Date/核实人及日期：____________________ |  |  |  |
| Confirm liquid charging pipeline is released. /确认加料管路经过放行。 |  |  |  |
| Checked by & Date/核实人及日期： ____________________ |  |  |  |
| Charging of drummed solvent: verify that the pipeline connections remain consistent before and after material feeding, then record the weighing reading 桶装溶剂溶液称量时，确认投料前后管路连接一致，读取称量示数。 Performed by & Date/执行人及日期： Checked by & Date/核实人及日期： |  |  |  |
| DIC /N,N’-二异丙基碳二亚胺：Control Number/内部控制号：YL003564 |  |  |  |
| Internal temperature/内部温度 |  | High values/最高值： | Low values/最低值： |
| Transcribed by & Date /转录人及日期： |  |  |  |
| If the target net weight≥the planned net weight, perform full drum charge; otherwise, perform partial (Non-full drum) charge. /当目标加料量≥计划加入净重，勾选进行整桶加料；否则勾选进行零头加料。 Note: Target net weight to be charged = Total target net weight – Total net weight charged. Expected tare weight of drum after charge = Gross weight of drum that to be charged - Target net weight to be charged. 还需加入净重目标值= 加料步骤目标总净重-已加入净重总量 加料完成后预计桶皮重=待加料桶毛重-还需加入净重目标值 |  |  |  |  |

| 3.12. （4）Fmoc-L-Asn(Trt)-OH activation/Fmoc-N-三苯甲基-L-天冬酰胺活化 |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Addition type加料式 | Lot number/批号 | Pre-charging qty calculation/投料前投料量计算 |  |  |  |  |  | Actual Charging Qty/实际投料量 |  |  |  |  |
|  |  | Total qty charged/已加入净重总量 | Target qty to be charged/还需加入净重目标值 |  | Gross qty of drum that to be charged/待加料桶毛重 | Expected net weight of drum after charge /加料完成后预计桶净重 | Expected tare weight of drum after charge/加料完成后预计桶皮重 | Before charging/ 投料前 | After charging/ 投料后 | Net weight/ 净重 | Performed by & Date/ 执行人及日期 | Checked by & Date/ 核实人及日期 |
| Full drum整桶 Non-full drum /非整桶 |  |  |  |  |  |  |  |  |  |  |  |  |
| Full drum整桶 Non-full drum /非整桶 |  |  |  |  |  |  |  |  |  |  |  |  |
| Total/合计 |  |  |  |  |  |  |  |  |  |  |  |  |
| DIC addition/加N,N'-二异丙基碳二亚胺 |  |  |  | Begin/开始： |  |  |  | End/结束： |  |  |  |  |
| Transcribed by & Date/转录人及日期：____________________ |  |  |  |  |  |  |  |  |  |  |  |  |

| 3.12. （4）Fmoc-L-Asn(Trt)-OH activation/Fmoc-N-三苯甲基-L-天冬酰胺活化 |  |  |  |  |
| --- | --- | --- | --- | --- |
| 3.12.8 Fmoc-L-Asn(Trt)-OH activation/Fmoc-N-三苯甲基-L-天冬酰胺活化 At 20±5 ℃ (target: 20 ℃). Continue agitation of activation tank to 95±10 rpm (target: 95 rpm). Maintain stirring for 20±5 min (target: 20 min) after addition, then stop stirring. 滴毕，控温20±5 ℃(目标温度：20 ℃)，维持激活罐搅拌转速至95±10 rpm (目标转速: 95 rpm)，继续搅拌20±5 min(目标时间：20 min), 停止搅拌。 |  |  |  | Times/次数 |
|  |  |  |  | 1 |
| Stir /搅拌 |  | Performed by & Date/执行人及日期： ____________________ |  |  |
| Stir /搅拌 |  | End/结束 |  |  |
|  |  | The start time is documented in Ins3.12.1 /开始时间记录于3.12.1 |  |  |
| Transcribed by & Date/转录人及日期：____________________ |  |  |  |  |
| 3.12.9 Internal temperature/内部温度 |  | High values/最高值： | Low values/最低值： |  |
| Transcribed by & Date/转录人及日期：____________________ |  |  |  |  |

| - （4）Fmoc-L-Asn(Trt)-OH coupling/Fmoc-N-三苯甲基-L-天冬酰胺偶联 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3.13.1 Activation mixture transfer /活化体系转移 Transfer the mixture in activation tank (TJ4S-1213-Sy15-R01) into synthesis reactor (TJ4S-1213-Sy15). 将激活罐（TJ4S-1213-Sy15-R01）中体系转移至合成釜（TJ4S-1213-Sy15）中。 |  |  |  |  |  |  |  | Times/次数 |
|  |  |  |  |  |  |  |  | 1 |
| Transfer/转移 |  |  | Performed by & Date/执行人及日期： |  |  |  |  |  |
|  |  |  | Checked by & Date/核实人及日期： |  |  |  |  |  |
|  |  |  | Begin/开始： |  |  | End/结束： |  |  |
| ★Note:The coupling temperature is a CPP, and the coupling IPC hold temperature is a CPP./ 备注：偶联温度是CPP，偶联IPC维持温度是关键工艺参数。 The operator shall confirm that the reaction mixture temperature is within the Normal Operating Range (NOR) 【25±5oC (target: 25oC)】 /操作人员须执行反应体系温度处于正常操作范围（NOR）内【25±5oC (target: 25oC)】 Checked by & Date/核实人及日期： Over-temperature warning value/超温预警设置数值： Lower limit/低限: 22 oC ，High limit/高限: 28 oC |  |  |  |  |  |  |  |  |
| CAUTION/注意：DMF/N，N-二甲基甲酰胺：Irritation of eyes, skin and nose. May cause nausea. /刺激眼睛、皮肤和鼻子。可能引起恶心。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |  |  |  |
| 3.13.2 DMF addition /加N,N-二甲基甲酰胺 Add 45.50 kg (Acceptable weighing range：43.50 kg ~ 47.50 kg) (1 V) DMF by spray balls into activation tank (TJ4S-1213-Sy15-R01) to rinse the wall. 通过淋洗球向激活罐（TJ4S-1213-Sy15-R01）中加入45.50 kg（可接受称量范围：43.50 kg ~ 47.50 kg） (1 V) N,N-二甲基甲酰胺淋洗罐壁。 |  |  |  |  |  |  |  | Times/次数 |
|  |  |  |  |  |  |  |  | 1 |
| Rinse with DMF/N,N-二甲基甲酰胺淋洗 |  |  |  | Performed by & Date/执行人及日期： |  |  |  |  |
| Rinse with DMF/ N,N-二甲基甲酰胺淋洗 |  | Begin/开始： |  |  | End/结束： |  | Qty/重量： |  |
| Transcribed by & Date /转录人及日期: |  |  |  |  |  |  |  |  |

| 3.13. （4）Fmoc-L-Asn(Trt)-OH coupling/Fmoc-N-三苯甲基-L-天冬酰胺偶联 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 3.13.3 Transfer /转移 Turn on the nitrogen of Synthesis reactor (TJ4S-1213-Sy15) to 5±2 m³/h (target: 5 m³/h). Bubble with nitrogen for 5±2 min (target: 5 min). Start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm). then start the agitator. Transfer the mixture in activation tank (TJ4S-1213-Sy15-R01) into synthesis reactor (TJ4S-1213-Sy15). 合成釜（TJ4S-1213-Sy15）先开启下口氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min (目标：5 min)后，开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm)，将激活罐（TJ4S-1213-Sy15-R01）中体系转移至合成釜（TJ4S-1213-Sy15）中。 |  |  |  |  |  | Times/次数 |  |
|  |  |  |  |  |  | 1 |  |
| Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h). Bubble with nitrogen for 5±2 min (target :5 min) 开启合成釜下口鼓氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min (目标时间：5 min) |  |  |  |  |  |  |
| Bubble/鼓氮气 |  | Performed by & Date/执行人及日期： |  |  |  |  |
| Bubble/鼓氮气 |  | Begin/开始： |  |  |  |  |
|  |  | The end time is documented in Ins3.13.5 /结束时间记录于3.13.7 |  |  |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |
| Start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm). /开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm) |  |  |  |  |  |  |
| Stir /搅拌 |  | Performed by & Date/执行人及日期： |  |  |  |  |
| Stir /搅拌 |  | Begin/开始： |  |  |  |  |
|  |  | The end time is documented in Ins3.13.5 /结束时间记录于3.13.7 |  |  |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |
| Transfer/转移 |  |  | Performed by & Date/执行人及日期： |  |  |  |
|  |  |  | Checked by & Date/核实人及日期： |  |  |  |
|  |  |  | Begin/开始： | End/结束： |  |  |
| 3.13.4 Adjust synthesis reactor temperature /合成釜调温 Adjust the synthesis reactor (TJ4S-1213-Sy15) mixture to【25±5 ℃ (target: 25 ℃)】. /将合成釜（TJ4S-1213-Sy15）体系调温至【25±5 ℃(目标温度：25 ℃)】. |  |  |  | Times/次数 |  |  |
|  |  |  |  | 1 |  |  |

| 3.13. （4）Fmoc-L-Asn(Trt)-OH coupling/Fmoc-N-三苯甲基-L-天冬酰胺偶联 |  |  |  |
| --- | --- | --- | --- |
| ★Note:The coupling temperature is a CPP, and the coupling IPC hold temperature is a CPP./ 备注：偶联温度是CPP，偶联IPC维持温度是关键工艺参数。 The operator shall confirm that the reaction mixture temperature is within the Normal Operating Range (NOR) /操作人员须执行反应体系温度处于正常操作范围（NOR）内 Checked by & Date/核实人及日期： ★Lower limit/低限: 13 oC ，High limit/高限: 37 oC Over-temperature warning value/超温预警设置数值： Lower limit/低限: 22 oC ，High limit/高限: 28 oC |  |  |  |  |  |
| Adjust/调温 |  | Performed by & Date/执行人及日期： |  |  |  |
| Adjust/调温 |  |  | Begin/开始： | End/结束： |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |

| 3.13. （4）Fmoc-L-Asn(Trt)-OH coupling/Fmoc-N-三苯甲基-L-天冬酰胺偶联 |  |  |
| --- | --- | --- |
| CAUTION/注意：DMF/N，N-二甲基甲酰胺：Irritation of eyes, skin and nose. May cause nausea. /刺激眼睛、皮肤和鼻子。可能引起恶心。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |
| 3.13.5 Preparation of Oxyma in DMF/ Oxyma/DMF溶液的配制 Add 45.50 kg DMF (Acceptable weighing range：43.50 ~ 47.50 kg) (1 V) with stainless steel hose and diaphragm pump into Agitator tank（TJ4S-1200-AT02）. Start the stirring. 通过不锈钢软管，隔膜泵向搅拌罐（TJ4S-1200-AT02）中加入45.50 kg N,N-二甲基甲酰胺(DMF) (1 V)（Acceptable weighing range/可接受称量范围： 43.50 ~ 47.50 kg），开启搅拌。 | Times/次数 |  |
|  | 1 |  |
| Before material charging, confirm the material is within the retest date/ the expiry date, Confirm whether the weight is adequate for use in the current charging. /加物料前，执行物料在复检日期/有效期之内，确认重量是否满足本次投料使用。 Checked by & Date/核实人及日期： |  |  |
| Before drummed liquid material charging, visual check there is no other extraneous matter in material. /加桶装液体物料前，应目视执行桶内无异物。 Visual Examination of Foreign Matters for Drum Solvent/桶装溶剂异物目视检查: 1. Place the solvent drum in a well-lit area (or use a portable light source) to ensure adequate visibility of the liquid contents./ 将溶剂桶置于光线充足区域（或使用便携式光源），确保液体内容物可见性良好 2. Visually inspect the solvent through the drum bung opening or transparent sight glass for any visible foreign matters, including but not limited to particulates, fibers, sediments, or unusual turbidity./ 通过桶口或透明视镜目视检查溶剂中是否有可见异物，包括但不限于颗粒、纤维、沉淀物或异常浑浊。 3. Confirm that the solvent is clear and free of visible foreign matters before charging. If any foreign matter is observed, do not use the drum and follow the applicable handling procedure for subsequent disposition./ 投料前确认溶剂澄清且无可见异物。如发现任何异物，不得使用该桶，并依据处理流程进行后续处置。 Checked by & Date/核实人及日期： |  |  |
| Confirm liquid charging pipeline is released. /执行加料管路经过放行。 |  |  |
| Performed by & Date/执行人及日期： |  |  |
| Pump number/泵编号： Weighing of DMF/ N,N-二甲基甲酰胺称重： Control Number/内部控制号：YL000336 Scale ID. No./秤编号： Material charging pipe number/物料管路编号： |  |  |

| 3.13. （4）Fmoc-L-Asn(Trt)-OH coupling/Fmoc-N-三苯甲基-L-天冬酰胺偶联 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Addition type加料式 |  | Lot number/批号 | Pre-charging qty calculation/投料前投料量计算 |  |  |  |  |  |  | Actual Charging Qty/实际投料量 |  |  |  |  |  |
|  |  |  | Total qty charged/已加入净重总量 | Target qty to be charged/还需加入净重目标值 |  | Gross qty of drum that to be charged/待加料桶毛重 | Expected net weight of drum after charge /加料完成后预计桶净重 | Expected tare weight of drum after charge/加料完成后预计桶皮重 |  | Before charging/ 投料前 |  | After charging/ 投料后 | Net weight/ 净重 | Performed by & Date/ 执行人及日期 | Checked by & Date/ 核实人及日期 |
| Full drum整桶 Non-full drum /非整桶 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Full drum整桶 Non-full drum /非整桶 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Total/合计 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DMF Addition/加N,N-二甲基甲酰胺 |  |  |  |  | Begin/开始： |  |  |  |  |  | End/结束： |  |  |  |  |
| Performed by & Date/执行人及日期 |  |  |  |  |  |  |  |  | Checked by & Date/核实人及日期： |  |  |  |  |  |  |
| Stir/搅拌 |  |  |  |  |  |  |  |  | Begin/开始： |  |  |  |  |  |  |
| Performed by & Date/执行人及日期： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| 3.13. （4）Fmoc-L-Asn(Trt)-OH coupling/Fmoc-N-三苯甲基-L-天冬酰胺偶联 |  |  |  |  |
| --- | --- | --- | --- | --- |
| CAUTION/注意：OxymaPure is highly toxic, high-energy compound. /2-肟氰乙酸乙酯是高毒，高能化合物。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |
| 3.13.6 Preparation of Oxyma in DMF/ Oxyma/DMF溶液的配制 Add OxymaPure (1.00 eq.) with powder feeding bags into Agitator tank（TJ4S-1200-AT02）. After added, maintain stirring for NLT 15 min. Confirm no visible solids and later use. (the mixture should be prepared just before use). 通过投料袋向搅拌罐（TJ4S-1200-AT02）中加入 kg 2-肟氰乙酸乙酯（OxymaPure）（Acceptable weighing range/可接受称量范围： kg ~ kg）(1.00 eq.)，加料后，搅拌至少15 min, 确认无明显固体待用（现配现用）。 Note: The pre-prepared OxymaPure/DMF solution will be used in section 3.13.7./ 配制的2-肟氰乙酸乙酯/DMF溶液将用于操作3.13.7. |  |  | Times/次数 |  |
|  |  |  | 1 |  |
| Before material charging, confirm the material is within the retest date/ the expiry date,Confirm whether the weight is adequate for use in the current charging. /加物料前，执行物料在复检日期/有效期之内，确认重量是否满足本次投料使用。 Before solid material charging, confirm/加固体物料前，确认： - Before operation performed, confirm there is no non-related material / equipment / instrument / tools / pipeline / document / record / label in the intended area /操作前；目标区域无不相关物料/设备/仪器/工具/管路/文件/记录/标签 - The outer packages are removed. If the outer package cannot be removed, but un-package material without damaging the package bag, and before charging, check no package fragment and other extraneous matter in material. /外包装移除；如果外包装不能移除，但在不破坏包装的情况下打开包装袋，投料前以及过程中同时检查执行物料中无包装碎片以及异物 Checked by & Date/核实人及日期： |  |  |  |  |
| Check and record that the solid charge port is free of solid accumulation from the previous step./ 检查并记录固体投料口无来自上一步骤的固体残留。 Checked by & Date/核实人及日期： |  |  |  |  |
| OxymaPure addition/加2-肟氰乙酸乙酯 | Performed by & Date/执行人及日期： |  |  |  |
|  | Begin/开始： | End/结束： |  |  |

| 3.13. （4）Fmoc-L-Asn(Trt)-OH coupling/Fmoc-N-三苯甲基-L-天冬酰胺偶联 |  |  |  |  |
| --- | --- | --- | --- | --- |
| OxymaPure confirmation/2-肟氰乙酸乙酯确认：Control No./内部控制号：YL005180 |  |  |  |  |
| Check items/检查项 |  | Is it correct?/是否正确 | Recorded by & Date/ 记录人及日期 | Checked by & Date/ 核实人及日期 |
| Material name/物料名称 |  | Yes/是；No/否 |  |  |
| Project batch number/ 项目批次号 |  | Yes/是；No/否 |  |  |
| Operation number/ 操作号 |  | Yes/是；No/否 |  |  |
| Control No./内部控制号 |  | Yes/是；No/否 |  |  |
| No. /编号 | Net weight/净重 | All in/全部加入 | Recorded by & Date/ 记录人及日期 | Checked by & Date/ 核实人及日期 |
| / |  | Yes/是；No/否 |  |  |
| / |  | Yes/是；No/否 |  |  |
| / |  | Yes/是；No/否 |  |  |
| / |  | Yes/是；No/否 |  |  |
| / |  | Yes/是；No/否 |  |  |
| / |  | Yes/是；No/否 |  |  |
| / |  | Yes/是；No/否 |  |  |
| / |  | Yes/是；No/否 |  |  |
| / |  | Yes/是；No/否 |  |  |
| / |  | Yes/是；No/否 |  |  |
| / |  | Yes/是；No/否 |  |  |
| / |  | Yes/是；No/否 |  |  |
| Total/合计 |  | Confirm the total net weight is within the input range/执行合计净重在投料范围内 | Recorded by & Date/记录人及日期 | Checked by & Date/ 核实人及日期 |
|  |  | Yes/是；No/否 |  |  |

| 3.13. （4）Fmoc-L-Asn(Trt)-OH coupling/Fmoc-N-三苯甲基-L-天冬酰胺偶联 |  |  |  |
| --- | --- | --- | --- |
| After solid material charging, confirm/固体物料投料之后，确认： - The corresponding leftover material has been properly sealed and transferred to the staging area/相应的剩余物料已经恰当密封，转移到了暂存区域。 - The waste packaging material has been sealed and transferred to the corresponding waste staging area. /产生的包材废物已经密封并转移到了相应的废物暂存区域。 - The operation area/equipment surface has been cleaned/操作区域以及设备表面已经清洁。 Performed by & Date/执行人及日期： |  |  |  |
| Stir/搅拌 |  | End/结束： |  |
| Performed by & Date/执行人及日期 |  | Checked by & Date/核实人及日期 |  |
| Solution No./溶液批号：CPo140020- -01- -3.13.4-01 |  |  |  |
| CAUTION/注意：DIC is a key EHS concern chemical, highly toxic, skin sensitizing, respiratory sensitizing. /N,N’二异丙基碳二亚胺为EHS重点关注化学品，高毒，皮肤致敏，呼吸致敏。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |
| 3.13.7 Fmoc-L-Asn(Trt)-OH coupling/Fmoc-N-三苯甲基-L-天冬酰胺偶联 The mixture is allowed to react at【25±5 ℃ (target: 25 ℃)】. After the total reaction time has reached 8 h±15 min (target: 8 h), turn off the nitrogen, add all of the OxymaPure / DMF solution (the mixture be prepared in Ins.3.13.4) with dedicated process lines and diaphragm pump and DIC (1.10 eq.) with PTFE and peristaltic pump. After 5 h±15 min (target: 5 h), the mixture should be stirred for total time (13 h≤total time≤28 h), stop stirring, turn off the nitrogen. Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point. 控温【25±5 oC（目标温度25 oC】反应至8 h±15 min（目标: 8 h），关闭氮气，通过工艺固定管路和隔膜泵补加配置的2-肟氰乙酸乙酯/DMF溶液（操作3.13.6配制的）和通过四氟管路和蠕动泵加入 __________kg N,N-二异丙基碳二亚胺（Acceptable weighing range/可接受称量范围： kg ~ kg）(1.10 eq.)，开启下口鼓氮气。补加操作结束后反应至5 h±15 min（目标: 5h），反应总时间满足（13 h≤反应总时间≤28 h）后停止搅拌，关闭氮气。通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 Note: If resin adherence is found at other times during the reaction, rinse the reactor with 0.50~5.00 kg DMF each time. /备注：偶联过程中其它时间发现贴壁，每次使用0.50~5.00 kg N,N-二甲基甲酰胺淋洗，记录见附加页 |  |  | Times/次数 |
|  |  |  | 1 |

| 3.13. （4）Fmoc-L-Asn(Trt)-OH coupling/Fmoc-N-三苯甲基-L-天冬酰胺偶联 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The total reaction Fmoc-L-Asn(Trt)-OH coupling time：NMT 840 min (Sourced from the holding time study in the PAR study report.)/Fmoc-N-三苯甲基-L-天冬氨酰胺偶联总的反应时间不超过840min(来源于 PAR 研究报告中稳定时间研究)。 Over-temperature warning value/超温预警设置数值： Lower limit/低限: 22 oC ，High limit/高限: 28 oC |  |  |  |  |  |
| React/反应 |  | Performed by & Date/执行人及日期： |  |  |  |
| React for 8 h/反应8 h | Begin/开始： |  |  | End/结束： |
| Internal temperature/内部温度 | High values/最高值： |  |  | Low values/最低值： |
| Transcribed by & Date /转录人及日期： |  |  |  |  |
| Nitrogen/氮气 |  | Performed by & Date/执行人及日期： |  |  |
| Nitrogen/氮气 |  |  | End/结束： |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |
| Confirm liquid charging pipeline is released. /执行加料管路经过放行。 |  |  |  |  |  |
| Performed by & Date/执行人及日期： |  |  |  |  |  |

| 3.13. （4）Fmoc-L-Asn(Trt)-OH coupling/Fmoc-N-三苯甲基-L-天冬酰胺偶联 |  |  |  |
| --- | --- | --- | --- |
| Transfer OxymaPure and DMF solution /转移2-肟氰乙酸乙酯的DMF溶液 | Performed by & Date/执行人及日期： |  |  |
|  | Checked by & Date/核实人及日期： |  |  |
|  | Begin/开始： | End/结束： |  |
| ★Note:The DIC addition temperature is a CPP./ 备注：DIC加料温度是关键控制参数。 Prior to DIC addition, the operator shall confirm that the reaction mixture temperature is within the Normal Operating Range (NOR) / N,N'-二异丙基碳二亚胺投料前，操作人员须执行反应体系温度处于正常操作范围（NOR）内: Checked by & Date/核实人及日期： ★Lower limit/低限: xx oC ，High limit/高限: xx oC Over-temperature warning value/超温预警设置数值： Lower limit/低限: 22 oC ，High limit/高限: 28 oC |  |  |  |
| Before material charging, confirm the material is within the retest date/ the expiry date, Confirm whether the weight is adequate for use in the current charging. /加物料前，执行物料在复检日期/有效期之内，确认重量是否满足本次投料使用。 Checked by & Date/核实人及日期： |  |  |  |
| Before drummed liquid material charging, visual check there is no other extraneous matter in material. /加桶装液体物料前，应目视执行桶内无异物。 Visual Examination of Foreign Matters for Drum Solvent/桶装溶剂异物目视检查: 1. Place the solvent drum in a well-lit area (or use a portable light source) to ensure adequate visibility of the liquid contents./ 将溶剂桶置于光线充足区域（或使用便携式光源），确保液体内容物可见性良好 2. Visually inspect the solvent through the drum bung opening or transparent sight glass for any visible foreign matters, including but not limited to particulates, fibers, sediments, or unusual turbidity./ 通过桶口或透明视镜目视检查溶剂中是否有可见异物，包括但不限于颗粒、纤维、沉淀物或异常浑浊。 3. Confirm that the solvent is clear and free of visible foreign matters before charging. If any foreign matter is observed, do not use the drum and follow the applicable handling procedure for subsequent disposition./ 投料前确认溶剂澄清且无可见异物。如发现任何异物，不得使用该桶，并依据处理流程进行后续处置。 Checked by & Date/核实人及日期： |  |  |  |

| 3.13. （4）Fmoc-L-Asn(Trt)-OH coupling/Fmoc-N-三苯甲基-L-天冬酰胺偶联 |  |  |  |  |
| --- | --- | --- | --- | --- |
| DIC /N,N’-二异丙基碳二亚胺：Control Number/内部控制号：YL003564 |  |  |  |  |
| DIPEA Addition/加N,N-二异丙基乙胺 |  | Begin/开始： |  | End/结束： |  |  |
| DIPEA Addition/加N,N-二异丙基乙胺 |  | Performed by & Date/执行人及日期： |  |  |  |  |
| Internal temperature/内部温度 | High values/最高值： |  | Low values/最低值： |  |  |  |  |
| Transcribed by & Date/转录人及日期： |  |  |  |  |  |  |  |
| If the target net weight≥the planned net weight, perform full drum charge; otherwise, perform partial (Non-full drum) charge. /当目标加料量≥计划加入净重，勾选进行整桶加料；否则勾选进行零头加料。 Note: Target net weight to be charged = Total target net weight – Total net weight charged. Expected tare weight of drum after charge = Gross weight of drum that to be charged - Target net weight to be charged. 还需加入净重目标值= 加料步骤目标总净重-已加入净重总量 加料完成后预计桶皮重=待加料桶毛重-还需加入净重目标值 |  |  |  |  |  |

| 3.13. （4）Fmoc-L-Asn(Trt)-OH coupling/Fmoc-N-三苯甲基-L-天冬酰胺偶联 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Addition type加料式 | Lot number/批号 | Pre-charging qty calculation/投料前投料量计算 |  |  |  |  | Actual Charging Qty/实际投料量 |  |  |  |  |
|  |  | Total qty charged/已加入净重总量 | Target qty to be charged/还需加入净重目标值 | Gross qty of drum that to be charged/待加料桶毛重 | Expected net weight of drum after charge /加料完成后预计桶净重 | Expected tare weight of drum after charge/加料完成后预计桶皮重 | Before charging/ 投料前 | After charging/ 投料后 | Net weight/ 净重 | Performed by & Date/ 执行人及日期 | Checked by & Date/ 核实人及日期 |
| Full drum整桶 Non-full drum /非整桶 |  |  |  |  |  |  |  |  |  |  |  |
| Full drum整桶 Non-full drum /非整桶 |  |  |  |  |  |  |  |  |  |  |  |
| Total/合计 |  |  |  |  |  |  |  |  |  |  |  |
| Stir/搅拌 |  |  |  |  |  | Begin/开始： |  |  |  |  |  |
| Performed by & Date/执行人及日期： |  |  |  |  |  |  |  |  |  |  |  |

| 3.13. （4）Fmoc-L-Asn(Trt)-OH coupling/Fmoc-N-三苯甲基-L-天冬酰胺偶联 |  |  |  |
| --- | --- | --- | --- |
| Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h).开启合成釜下口鼓氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h) |  |  |  |
| Bubble/鼓氮气 | Performed by & Date/执行人及日期： |  |  |  |
| Bubble/鼓氮气 | Begin/开始： |  |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |
| React/反应 | Begin/开始： |  | End/结束： |
| Transcribed by & Date /转录人及日期： |  |  |  |
| Internal temperature/内部温度 | High values/最高值： | Low values/最低值： |  |
| Transcribed by & Date /转录人及日期： |  |  |  |
| ★After coupling is completed, the operator shall confirm that all Critical Process Parameters (CPP) are within their respective Normal Operating Ranges (NOR)/ 偶联结束后，操作人员须确认所有关键工艺参数（CPP）均处于各自的正常操作范围（NOR）内 ★Note: The coupling temperature is a CPP, and the coupling IPC hold temperature is a CPP./备注：偶联温度是CPP，偶联IPC维持温度是关键工艺参数。 The operator shall confirm that the reaction mixture temperature is within the Normal Operating Range (NOR) :20~30℃ /操作人员须确认反应体系温度处于正常操作范围（NOR）内:20~30℃ □Yes/是 □No/否 Confirmed by & Date/确认人及日期： Checked by & Date/核实人及日期： |  |  |  |

| 3.13. （4）Fmoc-L-Asn(Trt)-OH coupling/Fmoc-N-三苯甲基-L-天冬酰胺偶联 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Confirm the end point of the reaction. /执行反应终点。 Performed by & Date/执行人及日期： Checked by & Date/核实人及日期： |  |  |  |  |
| Stop the stirrer and turn off the nitrogen. Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point./ 停止搅拌，关闭氮气，通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  |  |  |
| Stir /搅拌 | Performed by & Date/执行人及日期： |  |  |  |
| Stir /搅拌 | End/结束： |  |  |  |
|  | The start time is documented in Ins 3.13.3 /开始时间记录于3.13.3 |  |  |  |
| Transcribed by & Date/转录人及日期： |  |  |  |  |
| Bubble/鼓氮气 | Performed by & Date/执行人及日期： |  |  |  |
| Bubble/鼓氮气 | End/结束： |  |  |  |
|  | The start time is documented in Ins 3.13.3 /开始时间记录于3.13.3 |  |  |  |
| Transcribed by & Date/转录人及日期： |  |  |  |  |
| Internal temperature/内部温度 | High values/最高值： | Low values/最低值： |  |  |
| Transcribed by & Date/转录人及日期： |  |  |  |  |
| Filter/过滤 | Performed by & Date/执行人及日期： |  |  |  |
|  | Checked by & Date/核实人及日期： |  |  |  |
|  | Begin/开始： |  | End/结束： |  |
| ★After coupling is completed, the operator shall confirm that all Critical Process Parameters (CPPs) are within their respective Normal Operating Ranges (NOR)/ 偶联结束后，操作人员须确认所有关键工艺参数（CPP）均处于各自的正常操作范围（NOR）内 Checked by & Date/核实人及日期： |  |  |  |  |

| 3.13. （4）Fmoc-L-Asn(Trt)-OH coupling/Fmoc-N-三苯甲基-L-天冬酰胺偶联 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| CAUTION/注意：DMF/N，N-二甲基甲酰胺：Irritation of eyes, skin and nose. May cause nausea. /刺激眼睛、皮肤和鼻子。可能引起恶心。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |  |
| 3.13.8 Activation Tank Rinse/激活罐洗涤 Add 364.03 kg (Acceptable weighing range：353.11 ~ 374.95 kg) (8 V) DMF into the activation tank (TJ4S-1213-Sy15-R01). Circulate and clean the activating liquid transfer pipeline 1, solid charging port cleaning pipeline 2 and liquid charging port cleaning pipeline 3, clean every pipeline for 2±1 min. 向激活罐（TJ4S-1213-Sy15-R01）中加入364.03 kg（可接受称量范围：353.11 ~ 374.95 kg） (8 V) N,N-二甲基甲酰胺，循环清洗激活液转移管路1，固体加料口清洗管路2和液体加料口清洗管路3，每条线路各清洗2±1 min。 |  |  |  |  | Times/次数 |  |
|  |  |  |  |  | 3 |  |
| Operation name/ 操作名称 | Time point/时间节点 | Clean the activating liquid transfer pipeline 1/循环清洗激活液转移管路1 | Solid charging port cleaning pipeline 2 /固体加料口清洗管路2 | liquid charging port cleaning pipeline 3,/液体加料口清洗管路3 |  |  |
|  |  | Wash /洗涤1 | Wash /洗涤2 | Wash /洗涤3 |  |  |
| DMF addition/ 加N,N-二甲基甲酰胺 | Performed by & Date/执行人及日期： |  |  |  |  |  |
| DMF addition/ 加N,N-二甲基甲酰胺 | Begin/开始 |  |  |  |  |  |
|  | End/结束 |  |  |  |  |  |
|  | Qty/重量 |  |  |  |  |  |
|  | Transcribed by & Date /转录人及日期： |  |  |  |  |  |
| Circulate and clean/循环清洗 | Performed by & Date/执行人及日期： |  |  |  |  |  |
| Circulate and clean/循环清洗 | Begin/开始 |  |  |  |  |  |
|  | End/结束 |  |  |  |  |  |
|  | Transcribed by & Date /转录人及日期： |  |  |  |  |  |

| 3.13. （4）Fmoc-L-Asn(Trt)-OH coupling/Fmoc-N-三苯甲基-L-天冬酰胺偶联 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 3.13.9 Post-coupling wash./偶联后洗涤 Transfer the mixture in activation tank (TJ4S-1213-Sy15-R01) to synthesis reactor (TJ4S-1213-Sy15). Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h). Bubble with nitrogen, after 5±2 min (target 5 min), Start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm). After stirring for 7±2 min (target：7 min), stop stirring, turn off the nitrogen. Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point. 将激活罐（TJ4S-1213-Sy15-R01）洗涤液转入合成釜 (TJ4S-1213-Sy15)。开启合成釜下口鼓氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min (目标时间：5 min)后开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm)，搅拌7±2 min（目标时间：7 min）后停止搅拌，关闭氮气。通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  |  |  | Times/次数 |  |
|  |  |  |  |  | 3 |  |
| Transfer the liquid form activation tank to the synthesis reactor (TJ4S-1213-Sy15) ./将激活罐中体系转移至合成釜中。 |  |  |  |  |  |  |
| Operation name/ 操作名称 | Time point/时间节点 | Wash /洗涤1 | Wash /洗涤2 | Wash /洗涤3 |  |  |
| Transfer/转移 | Performed by & Date/执行人及日期： |  |  |  |  |  |
|  | Checked by & Date/核实人及日期 |  |  |  |  |  |
|  | Begin/开始 |  |  |  |  |  |
|  | End/结束 |  |  |  |  |  |

| 3.13. （4）Fmoc-L-Asn(Trt)-OH coupling/Fmoc-N-三苯甲基-L-天冬酰胺偶联 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h). Bubble with nitrogen for 5±2 min (target:5 min), Start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm).After stirring for 7±2 min (target: 7 min), stop stirring, turn off the nitrogen./ 开启合成釜下口鼓氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min（目标时间：5 min）后开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm)，搅拌7±2 min（目标时间：7 min）后，停止搅拌，关闭氮气。 |  |  |  |  |
| Operation name/ 操作名称 | Time point/时间节点 | Wash /洗涤1 | Wash /洗涤2 | Wash /洗涤3 |
| Bubble/鼓氮气 | Performed by & Date/执行人及日期： |  |  |  |
| Bubble/鼓氮气 | Begin/开始 |  |  |  |
|  | End/结束 |  |  |  |
|  | Transcribed by & Date /转录人及日期： |  |  |  |
| Stir /搅拌 | Performed by & Date/执行人及日期： |  |  |  |
| Stir /搅拌 | Begin/开始 |  |  |  |
|  | End/结束 |  |  |  |
|  | Transcribed by & Date /转录人及日期： |  |  |  |
| Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point/通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  |  |  |
| Filter/过滤 | Performed by & Date/执行人及日期： |  |  |  |
|  | Checked by & Date/核实人及日期 |  |  |  |
|  | Begin/开始 |  |  |  |
|  | End/结束 |  |  |  |

| - Prepare capping solution/配封端液 |  |
| --- | --- |
| CAUTION/注意：DMF/N，N-二甲基甲酰胺：Irritation of eyes, skin and nose. May cause nausea. /刺激眼睛、皮肤和鼻子。可能引起恶心。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |
| 3.14.1 Prepare capping solution/配封端液 Add 273.02 kg DMF (Acceptable weighing range：264.83 ~ 281.21 kg) (6 V) into glass-lined reactor (TJ4S-1213-R22) with stainless steel hose and diaphragm pump. Start the agitator.向搪玻璃釜(TJ4S-1213-R22)中通过不锈钢软管和隔膜泵加入 273.02 kg N，N-二甲基甲酰胺（Acceptable weighing range/可接受称量范围： 264.83 ~ 281.21 kg），开启搅拌。 Pump number/泵编号： Scale ID. No./秤编号： Material charging pipe number/物料管路编号： | Times/次数 |
|  | 1 |
| Before material charging, confirm the material is within the retest date/ the expiry date, Confirm whether the weight is adequate for use in the current charging. /加物料前，执行物料在复检日期/有效期之内，确认重量是否满足本次投料使用。 Checked by & Date/核实人及日期： |  |
| Before drummed liquid material charging, visual check there is no other extraneous matter in material. /加桶装液体物料前，应目视执行桶内无异物。 Visual Examination of Foreign Matters for Drum Solvent/桶装溶剂异物目视检查: 1. Place the solvent drum in a well-lit area (or use a portable light source) to ensure adequate visibility of the liquid contents./ 将溶剂桶置于光线充足区域（或使用便携式光源），确保液体内容物可见性良好 2. Visually inspect the solvent through the drum bung opening or transparent sight glass for any visible foreign matters, including but not limited to particulates, fibers, sediments, or unusual turbidity./ 通过桶口或透明视镜目视检查溶剂中是否有可见异物，包括但不限于颗粒、纤维、沉淀物或异常浑浊。 3. Confirm that the solvent is clear and free of visible foreign matters before charging. If any foreign matter is observed, do not use the drum and follow the applicable handling procedure for subsequent disposition./ 投料前确认溶剂澄清且无可见异物。如发现任何异物，不得使用该桶，并依据处理流程进行后续处置。 Checked by & Date/核实人及日期： |  |
| Confirm liquid charging pipeline is released. /确认加料管路经过放行。 |  |
| Checked by & Date/核实人及日期： |  |

| 3.14. Prepare capping solution/配封端液 |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Addition type加料式 | Lot number/批号 | Pre-charging qty calculation/投料前投料量计算 |  |  |  |  |  | Actual Charging Qty/实际投料量 |  |  |  |  |
|  |  | Total qty charged/已加入净重总量 | Target qty to be charged/还需加入净重目标值 |  | Gross qty of drum that to be charged/待加料桶毛重 | Expected net weight of drum after charge /加料完成后预计桶净重 | Expected tare weight of drum after charge/加料完成后预计桶皮重 | Before charging/ 投料前 | After charging/ 投料后 | Net weight/ 净重 | Performed by & Date/ 执行人及日期 | Checked by & Date/ 核实人及日期 |
| Full drum整桶 Non-full drum /非整桶 |  |  |  |  |  |  |  |  |  |  |  |  |
| Full drum整桶 Non-full drum /非整桶 |  |  |  |  |  |  |  |  |  |  |  |  |
| Total/合计 |  |  |  |  |  |  |  |  |  |  |  |  |
| DMF Addition/加N,N-二甲基甲酰胺 |  |  |  | Begin/开始： |  |  |  | End/结束： |  |  |  |  |
| Performed by & Date/执行人及日期 |  |  |  |  |  |  | Checked by & Date/核实人及日期： |  |  |  |  |  |
| Stir/搅拌 |  |  |  |  |  |  | Begin/开始： |  |  |  |  |  |
| Performed by & Date/执行人及日期： |  |  |  |  |  |  |  |  |  |  |  |  |

| 3.14. Prepare capping solution/配封端液 |  |  |
| --- | --- | --- |
| CAUTION/注意：Pyriine is highly irritating; capable of anesthetizing the central nervous system. Irritating to eyes and upper respiratory tract. /吡啶是强烈刺激性；能麻醉中枢神经系统。对眼及上呼吸道有刺激作用。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |
| 3.14.2 Pyridine addition /加吡啶 Add pyridine (1.50 eq.) with PTFE and peristaltic pump into glass-lined reactor (TJ4S-1213-R22). 通过四氟管路，蠕动泵，向玻璃釜(TJ4S-1213-R22).加入 kg吡啶（Acceptable weighing range/可接受称量范围： kg ~ kg）(1.50 eq.)。 | Times/次数 |  |
|  | 1 |  |
| Before material charging, confirm the material is within the retest date/ the expiry date, Confirm whether the weight is adequate for use in the current charging. /加物料前，执行物料在复检日期/有效期之内，确认重量是否满足本次投料使用。 Checked by & Date/核实人及日期： |  |  |
| Before drummed liquid material charging, visual check there is no other extraneous matter in material. /加桶装液体物料前，应目视执行桶内无异物。 Visual Examination of Foreign Matters for Drum Solvent/桶装溶剂异物目视检查: 1. Place the solvent drum in a well-lit area (or use a portable light source) to ensure adequate visibility of the liquid contents./ 将溶剂桶置于光线充足区域（或使用便携式光源），确保液体内容物可见性良好 2. Visually inspect the solvent through the drum bung opening or transparent sight glass for any visible foreign matters, including but not limited to particulates, fibers, sediments, or unusual turbidity./ 通过桶口或透明视镜目视检查溶剂中是否有可见异物，包括但不限于颗粒、纤维、沉淀物或异常浑浊。 3. Confirm that the solvent is clear and free of visible foreign matters before charging. If any foreign matter is observed, do not use the drum and follow the applicable handling procedure for subsequent disposition./ 投料前确认溶剂澄清且无可见异物。如发现任何异物，不得使用该桶，并依据处理流程进行后续处置。 Checked by & Date/核实人及日期： |  |  |
| Confirm liquid charging pipeline is released. /确认加料管路经过放行。 |  |  |
| Checked by & Date/确认人及日期： |  |  |
| Pump number/泵编号： Weighing of pyridine /吡啶称重： Control Number/内部控制号 YL000617 Scale ID. No./秤编号： Material charging pipe number/物料管路编号： |  |  |

| 3.14. Prepare capping solution/配封端液 |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Addition type加料式 | Lot number/批号 | Pre-charging qty calculation/投料前投料量计算 |  |  |  |  |  | Actual Charging Qty/实际投料量 |  |  |  |  |
|  |  | Total qty charged/已加入净重总量 | Target qty to be charged/还需加入净重目标值 |  | Gross qty of drum that to be charged/待加料桶毛重 | Expected net weight of drum after charge /加料完成后预计桶净重 | Expected tare weight of drum after charge/加料完成后预计桶皮重 | Before charging/ 投料前 | After charging/ 投料后 | Net weight/ 净重 | Performed by & Date/ 执行人及日期 | Checked by & Date/ 核实人及日期 |
| Full drum整桶 Non-full drum /非整桶 |  |  |  |  |  |  |  |  |  |  |  |  |
| Full drum整桶 Non-full drum /非整桶 |  |  |  |  |  |  |  |  |  |  |  |  |
| Total/合计 |  |  |  |  |  |  |  |  |  |  |  |  |
| Pyridine Addition/加吡啶 |  |  |  | Begin/开始： |  |  |  | End/结束： |  |  |  |  |
| Performed by & Date/执行人及日期 |  |  |  |  |  |  | Checked by & Date/核实人及日期： |  |  |  |  |  |
| Stir/搅拌 |  |  |  |  |  |  | Begin/开始： |  |  |  |  |  |
| Performed by & Date/执行人及日期： |  |  |  |  |  |  |  |  |  |  |  |  |

| 3.14. Prepare capping solution/配封端液 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CAUTION/注意：Acetic anhydride has a choking sour odour; it is lachrymatory and corrosive. /乙酸酐具有窒息性酸味，有催泪和腐蚀作用Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |  |  |
| 3.14.3 Acetic anhydride addition /加乙酸酐 Add acetic anhydride (1.25 eq.) with PTFE and peristaltic pump into glass-lined reactor (TJ4S-1213-R22). /通过四氟管路，蠕动泵，向玻璃釜(TJ4S-1213-R22)加入 kg乙酸酐（Acceptable weighing range/可接受称量范围： kg ~ kg）(1.25 eq.)。 Pump number/泵编号： Scale ID. No./秤编号： Material charging pipe number/物料管路编号： |  |  |  |  |  | Times/次数 |  |
|  |  |  |  |  |  | 1 |  |
| Before material charging, confirm the material is within the retest date/ the expiry date, Confirm whether the weight is adequate for use in the current charging. /加物料前，执行物料在复检日期/有效期之内，确认重量是否满足本次投料使用。 Checked by & Date/核实人及日期： |  |  |  |  |  |  |  |
| Confirm liquid charging pipeline is released. /确认加料管路经过放行。 |  |  |  |  |  |  |  |
| Checeked by & Date/核实人及日期： |  |  |  |  |  |  |  |
| Pump number/泵编号： Weighing of acetic anhydride/乙酸酐称重： Control Number/内部控制号 SJ000213 Scale ID. No./秤编号： Material charging pipe number/物料管路编号： |  |  |  |  |  |  |  |
| Ac2O Lot number/批号 | Charging Qty/投入的质量 |  |  | Performed by & Date/执行人及日期 | Checked by & Date/核实人及日期 |  |  |
|  | Before charging/ 投料前 | After charging/ 投料后 | Net weight/ 净重 |  |  |  |  |

| 3.14. Prepare capping solution/配封端液 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Ac2O Lot number/批号 | Charging Qty/投入的质量 |  |  | Performed by & Date/执行人及日期 |  | Checked by & Date/核实人及日期 |  |
|  | Before charging/ 投料前 | After charging/ 投料后 | Net weight/ 净重 |  |  |  |  |
| Total/合计 |  |  |  |  |  |  |  |
| Acetic anhydride addition/加乙酸酐 | Begin/开始： |  |  |  | End/结束： |  |  |

| 3.14. Prepare capping solution/配封端液 |  |  |  |  |
| --- | --- | --- | --- | --- |
| 3.14.4 Adjust mixture temperature /体系调温 Adjust the mixture in glass-lined reactor (TJ4S-1213-R22) to 25±5 ℃ (target: 25 ℃). /将玻璃釜(TJ4S-1213-R22)体系调温至25±5 ℃(目标：25 ℃). |  |  |  |  | Times/次数 |
|  |  |  |  |  | 1 |
| ★Lower limit/低限: 13 oC ，High limit/高限: 37 oC Over-temperature warning value/超温预警设置数值： Lower limit/低限: 23 oC ，High limit/高限: 28 oC |  |  |  |  |
| Adjust record/调温记录：工艺要求温度：20~30 ℃（目标温度25 ℃）， ☐ Temperature within the process range, no need to adjust the temperature/温度在工艺范围内，无需调温。 Performed by & Date/执行人及日期： ☐ T<20 oC, Over-temperature Alarm value/温度<20 oC，超温报警设置数值： High alarm value/高报: 28 oC ☐ T>30 oC, Over-temperature Alarm value/温度>30 oC，超温报警设置数值： Low alarm value/低报: 22 oC Over-temperature Alarm Set by &Date/超温报警设置人签字及日期： Over-temperature Alarm Checked by &Date/超温报警核实人签字及日期： |  |  |  |  |
| Adjust/调温 | Begin/开始： | End/结束： |  |  |
| Performed by & Date/执行人及日期： |  |  |  |  |
| Glass-lined Reactor (TJ4S-1213-R22), Batch temperature trend refer to attachment. 搪玻璃釜(TJ4S-1213-R22)内温度趋势曲线见附件 Performed by & Date/执行人及日期： |  |  |  |  |

| 3.14. Prepare capping solution/配封端液 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 3.14.5 Stirring /搅拌 Stir at 25±5 ℃ (target: 25 ℃）for at least 5 min. /将体系控温25±5 ℃(目标温度：25 ℃)搅拌至少5 min。 |  |  |  | Times/次数 |  |
|  |  |  |  | 1 |  |
| Over-temperature Alarm value/超温报警设置数值：Low alarm value/低报: 22 oC High alarm value/高报: 28 oC Over-temperature Alarm Set by &Date/超温报警设置人签字及日期： Over-temperature Alarm Checked by &Date/超温报警核实人签字及日期： |  |  |  |  |  |
| Stir /搅拌 | Performed by & Date/执行人及日期： |  |  |  |  |
| Stir/搅拌 | Begin/开始： |  | End/结束： |  |  |
| Performed by & Date/执行人及日期： |  | Checked by & Date/核实人及日期： |  |  |  |
| Glass-lined Reactor (TJ4S-1213-R22), Batch temperature trend refer to attachment. 搪玻璃釜(TJ4S-1213-R22)内温度趋势曲线见附件 Performed by & Date/执行人及日期： |  |  |  |  |  |

| 3.15 Capping/封端 |  |  |  |
| --- | --- | --- | --- |
| 3.15.1 Transfer /转移 Transfer the mixture with dedicated process lines and diaphragm pump in glass-lined reactor (TJ4S-1213-R22) into synthesis reactor (TJ4S-1213-Sy15). 通过工艺固定管路和隔膜泵将搪玻璃釜 (TJ4S-1213-R22)中体系转移至合成釜（TJ4S-1213-Sy15）中。 |  |  | Times/次数 |
|  |  |  | 1 |
| Confirm liquid charging pipeline is released. /执行加料管路经过放行。 |  |  |  |
| Performed by & Date/执行人及日期： |  |  |  |
| Pump number /泵编号： |  |  |  |
| Transfer/转移 | Performed by & Date/执行人及日期： |  |  |
|  | Checked by & Date/核实人及日期： |  |  |
| Transfer/转移 | Begin/开始： | End/结束： |  |
| CAUTION/注意：DMF/N，N-二甲基甲酰胺：Irritation of eyes, skin and nose. May cause nausea. /刺激眼睛、皮肤和鼻子。可能引起恶心。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |
| 3.15.2 DMF addition /加N,N-二甲基甲酰胺 Add 45.50 kg DMF (Acceptable weighing range：43.50 ~ 47.50 kg) (1 V) by spray balls into glass-lined reactor (TJ4S-1213-R22) to rinse the wall. 通过淋洗球向搪玻璃釜 (TJ4S-1213-R22)中加入45.50 kg N,N-二甲基甲酰胺(DMF) （Acceptable weighing range/可接受称量范围： 43.50 ~ 47.50 kg）淋洗釜壁。 Pump number/泵编号： Scale ID. No./秤编号： Material charging pipe number/物料管路编号： |  |  | Times/次数 |
|  |  |  | 1 |

| 3.15 Capping/封端 |
| --- |
| Before material charging, confirm the material is within the retest date/ the expiry date. Confirm whether the weight is adequate for use in the current charging. /加物料前，执行物料在复检日期/有效期之内，确认重量是否满足本次投料使用。 Checked by & Date/核实人及日期： |  |
| Before drummed liquid material charging, visual check there is no other extraneous matter in material. /加桶装液体物料前，应目视执行桶内无异物。 Visual Examination of Foreign Matters for Drum Solvent/桶装溶剂异物目视检查: 1. Place the solvent drum in a well-lit area (or use a portable light source) to ensure adequate visibility of the liquid contents./ 将溶剂桶置于光线充足区域（或使用便携式光源），确保液体内容物可见性良好 2. Visually inspect the solvent through the drum bung opening or transparent sight glass for any visible foreign matters, including but not limited to particulates, fibers, sediments, or unusual turbidity./ 通过桶口或透明视镜目视检查溶剂中是否有可见异物，包括但不限于颗粒、纤维、沉淀物或异常浑浊。 3. Confirm that the solvent is clear and free of visible foreign matters before charging. If any foreign matter is observed, do not use the drum and follow the applicable handling procedure for subsequent disposition./ 投料前确认溶剂澄清且无可见异物。如发现任何异物，不得使用该桶，并依据处理流程进行后续处置。 Checked by & Date/核实人及日期： |  |
| Confirm liquid charging pipeline is released. /确认加料管路经过放行。 |  |
| Checked by & Date/核实人及日期： |  |
| Pump number/泵编号： Weighing of DMF/N,N-二甲基甲酰胺称重： Control Number/内部控制号 YL000336 Scale ID. No./秤编号： Material charging pipe number/物料管路编号： |  |

| 3.15 Capping/封端 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Addition type加料式 |  | Lot number/批号 | Pre-charging qty calculation/投料前投料量计算 |  |  |  |  |  |  | Actual Charging Qty/实际投料量 |  |  |  |  |
|  |  |  | Total qty charged/已加入净重总量 |  | Target qty to be charged/还需加入净重目标值 | Gross qty of drum that to be charged/待加料桶毛重 | Expected net weight of drum after charge /加料完成后预计桶净重 | Expected tare weight of drum after charge/加料完成后预计桶皮重 |  | Before charging/ 投料前 | After charging/ 投料后 | Net weight/ 净重 | Performed by & Date/ 执行人及日期 | Checked by & Date/ 核实人及日期 |
| Full drum整桶 Non-full drum /非整桶 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Full drum整桶 Non-full drum /非整桶 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Total weight /合计重量 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Rinse/淋洗 |  |  |  | Begin/开始： |  |  |  |  | End/结束： |  |  |  |  |  |
| Transcribed by & Date/转录人及日期： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| 3.15 Capping/封端 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 3.15.3 Transfer /转移 Turn on the nitrogen of Synthesis reactor (TJ4S-1213-Sy15) to 5±2 m³/h (target: 5 m³/h). Bubble with nitrogen for 5±2 min (target: 5 min). Start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm). then start the agitator. Transfer the mixture in Glass-lined reactor (TJ4S-1213-R22) into synthesis reactor (TJ4S-1213-Sy15). 合成釜（TJ4S-1213-Sy15）先开启下口氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min (目标：5 min)后，开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm) 将搪玻璃釜 (TJ4S-1213-R22)中体系转移至合成釜（TJ4S-1213-Sy15）中。 |  |  |  |  | Times/次数 |  |
|  |  |  |  |  | 1 |  |
| Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h). Bubble with nitrogen for 5±2 min (target :5 min) 开启合成釜下口鼓氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min (目标时间：5 min) |  |  |  |  |
| Bubble/鼓氮气 | Performed by & Date/执行人及日期： |  |  |  |
| Bubble/鼓氮气 | Begin/开始： |  |  |  |
|  | The end time is documented in Ins 3.15.5 /结束时间记录于3.15.5 |  |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |
| Start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm). /开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm) |  |  |  |  |
| Stir /搅拌 | Performed by & Date/执行人及日期： |  |  |  |
| Stir /搅拌 | Begin/开始： |  |  |  |
|  | The end time is documented in Ins 3.15.5 /结束时间记录于3.15.5 |  |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |
| Transfer/转移 |  | Performed by & Date/执行人及日期： |  |  |
|  |  | Checked by & Date/核实人及日期： |  |  |
|  |  | Begin/开始： | End/结束： |  |

| 3.15 Capping/封端 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CAUTION/注意：DMF/N，N-二甲基甲酰胺：Irritation of eyes, skin and nose. May cause nausea. /刺激眼睛、皮肤和鼻子。可能引起恶心。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |
| 3.15.4 Adjust the synthesis reactor temperature /合成釜调温 Adjust the synthesis reactor (TJ4S-1213-Sy15) mixture to【25±5 ℃ (target: 25 ℃)】. /将合成釜（TJ4S-1213-Sy15）体系调温至【25±5 ℃(目标温度：25 ℃)】. |  |  |  |  |  | Times/次数 |  |
|  |  |  |  |  |  | 1 |  |
| Over-temperature warning value/超温预警设置数值： Lower limit/低限: 21 oC ，High limit/高限: 28 oC |  |  |  |  |  |  |
| Performed by & Date/执行人及日期： |  |  |  |  |  |  |
| Adjust/调温 | Begin/开始： |  | End/结束： |  |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |
| 3.15.5 capping / 封端 The mixture is allowed to react at 25±5 ℃ (target: 25 ℃) for 1 h±15 min (target:1 h). Stop stirring and turn off the nitrogen. Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point. 控温25±5 ℃(目标：25 ℃)，反应1 h±15 min后，停止搅拌，关闭氮气。通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  |  |  |  | Times/次数 |  |
|  |  |  |  |  |  | 1 |  |
| React/反应 |  |  |  | Performed by & Date/执行人及日期： |  |  |  |

| 3.15 Capping/封端 |  |  |
| --- | --- | --- |
| Over-temperature warning value/超温预警设置数值： Lower limit/低限: 23 oC ，High limit/高限: 28 oC |  |  |
| React/反应 | Begin/开始： | End/结束： |
| Transcribed by & Date /转录人及日期： |  |  |
| Process chemist confirms the total reaction/ Capping time did not exceed 0~2 h (Sourced from the holding time study in the PAR study report.) 工艺技术人员确认总的封端反应时间不超过0~2 h(来源于 PAR 研究报告中稳定时间研究)。 Checked by & Date/核实人及日期____________________ |  |  |

| 3.15 Capping/封端 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Stir /搅拌 |  | Performed by & Date/执行人及日期：____________________ |  |  |  |
| Stir /搅拌 |  | End/结束： |  |  |  |
|  |  | The start time is documented in Ins3.15.3 /开始时间记录于3.15.3 |  |  |  |
| Transcribed by & Date /转录人及日期：____________________ |  |  |  |  |  |
| Bubble/鼓氮气 |  | Performed by & Date/执行人及日期：____________________ |  |  |  |
| Bubble/鼓氮气 |  | End/结束： |  |  |  |
|  |  | The start time is documented in Ins3.15.3 /开始时间记录于3.15.3 |  |  |  |
| Transcribed by & Date /转录人及日期：____________________ |  |  |  |  |  |
| Internal temperature/内部温度 |  |  | High values/最高值： |  | Low values/最低值： |
| Transcribed by & Date /转录人及日期：____________________ |  |  |  |  |  |
| Filter/过滤 |  |  | Performed by & Date/执行人及日期 ____________________ |  |  |  |
|  |  |  | Checked by & Date/核实人及日期：____________________ |  |  |  |
|  |  |  | Begin/开始： | End/结束： |  |  |
| ★After coupling is completed, the operator shall confirm that all Critical Process Parameters (CPPs) are within their respective Normal Operating Ranges (NOR)/ 偶联结束后，操作人员须确认所有关键工艺参数（CPP）均处于各自的正常操作范围（NOR）内 Checked by & Date/核实人及日期： |  |  |  |  |  |  |
| CAUTION/注意：DMF/N，N-二甲基甲酰胺：Irritation of eyes, skin and nose. May cause nausea. /刺激眼睛、皮肤和鼻子。可能引起恶心。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |

| 3.15 Capping/封端 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 3.15.6 Synthesis Reactor Rinse / 合成釜洗涤 Charge 364.03 kg DMF addition into synthesis reactor (TJ4S-1213-Sy15). Bubble with nitrogen, after 5±2 min (target 5 min), start the agitator. After stirring for 7±2 min (target：7 min), stop stirring, turn off the nitrogen. Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point. 向合成釜 (TJ4S-1213-Sy15) 加入364.03 kg N，N-二甲基甲酰胺（Acceptable weighing range/可接受称量范围：353.11 ~374.95 kg）（8V）。开启合成釜下口鼓氮气5±2 min (目标：5 min)后开启搅拌，搅拌7±2 min（目标：7 min）后停止搅拌，关闭氮气。通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  |  |  | Times/次数 |  |
|  |  |  |  |  | 3 |  |
| After the liquid is filtered out of the synthesis reactor (TJ4S-1213-Sy15), add the liquid to the synthesis reactor (TJ4S-1213-Sy15) ./ 待合成釜排废完毕后加料进合成釜中 |  |  |  |  |  |  |
| Operation name/ 操作名称 | Time point/时间节点 | Wash /洗涤1 | Wash /洗涤2 | Wash /洗涤3 |  |  |
| DMF addition/ 加N,N-二甲基甲酰胺 | Performed by & Date/执行人及日期： |  |  |  |  |  |
| DMF addition / 加N,N-二甲基甲酰胺 | Begin/开始 |  |  |  |  |  |
|  | End/结束 |  |  |  |  |  |
|  | Qty/重量 |  |  |  |  |  |
|  | Transcribed by & Date /转录人及日期： |  |  |  |  |  |

| 3.15 Capping/封端 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h).Bubble with nitrogen for 5±2 min (target 5 min), start the agitator. After stirring for 7±2 min (target：7 min), stop stirring, turn off the nitrogen.开启合成釜下口鼓氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min (目标：5 min)后开启搅拌转速至20±10 rpm (目标转速: 20 rpm)，搅拌7±2 min（目标：7 min）后停止搅拌，关闭氮气。 |  |  |  |  |  |  |  |
| Operation name/ 操作名称 |  |  | Time point/时间节点 | Wash /洗涤1 | Wash /洗涤2 | Wash /洗涤3 |
| Bubble/鼓氮气 |  |  | Begin/开始 |  |  |  |
|  |  |  | End/结束 |  |  |  |
|  |  |  | Transcribed by & Date /转录人及日期： |  |  |  |
| Stir /搅拌 |  |  | Performed by & Date/执行人及日期： |  |  |  |
| Stir /搅拌 |  |  | Begin/开始 |  |  |  |
|  |  |  | End/结束 |  |  |  |
|  |  |  | Transcribed by & Date /转录人及日期： |  |  |  |
| Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point/通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点 |  |  |  |  |  |  |
| Filter/过滤 |  |  | Performed by & Date/执行人及日期： |  |  |  |
|  |  |  | Checked by & Date/核实人及日期 |  |  |  |
|  |  |  | Begin/开始 |  |  |  |
|  |  |  | End/结束 |  |  |  |

| 3.16. （5）Deprotection/脱保护 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CAUTION/注意：Piperidine哌啶：Strong local irritant and may cause permanent injury after short exposure to small amounts. /强烈的局部刺激，短时间少量接触可能造成永久性伤害。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3.16.1 Piperidine solution addition /加哌啶溶液 Adjust the jacket temperature of synthesis reactor (TJ4S-1213-Sy15) to 【25±5 oC (target: 25 oC). 】. Add 357.50 kg 【20% (V/V) piperidine/DMF(8 V)】 (Acceptable weighing range: 346.77 ~ 368.23 kg) solution into the Synthesis reactor (TJ4S-1213-Sy15). 将合成釜(TJ4S-1213-Sy15)夹套调温至【25±5 ℃（目标温度：25 ℃）】。向合成釜（TJ4S-1213-Sy15）中加入357.50 kg【20%哌啶/DMF (V/V)溶液(8 V)】（可接受称量范围：346.77 ~ 368.23 kg）。 |  |  |  |  |  |  |  |  |  | Times/次数 |  |
|  |  |  |  |  |  |  |  |  |  | 1 |  |
| Adjust/调温 |  |  |  |  |  | Performed by & Date/执行人及日期： |  |  |  |  |  |
| Adjust/调温 |  |  |  |  |  | Begin/开始： |  | Adjust/调温 |  |  |  |
| Transcribed by & Date/转录人及日期： |  |  |  |  |  |  |  |  |  |  |  |
| Confirm and record that the 20%Piperidine solution concentration is within the NOR range./ 确认并记录20%哌啶溶液浓度在正常操作范围内：17.50%~22.50% 20%Piperidine solution concentration confirmation/20%哌啶溶液浓度确认：____________ Confirm and record that the 20%Piperidine solution temperature is within the NOR range./ 确认并记录20%哌啶溶液温度在正常操作范围内：20 oC ~30 oC 20%Piperidine solution temperature confirmation/20%哌啶溶液温度确认：____________ Confirmed by & Date/确认人及日期： Checked by & Date/核实人及日期： |  |  |  |  |  |  |  |  |  |  |  |
| ★Note:The deprotection temperature is a CPP./备注：脱保护反应温度是关键工艺参数。 Checked by & Date/核实人及日期： |  |  |  |  |  |  |  |  |  |  |  |
| Piperidine addition/ 加哌啶溶液 |  |  | Performed by & Date/执行人及日期： |  |  |  |  |  |  |  |  |
| Piperidine addition/ 加哌啶溶液 |  |  | Begin/开始： |  |  |  | End/结束： |  | Qty/重量： |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |  |  |  |  |
| 3.16. （5）Deprotection/脱保护 |  |  |  |  |  |  |  |  |  |  |  |  |
| 3.16.2 Adjust synthesis reactor temperature /合成釜调温 Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h) for 3±2 min (target 3 min), Start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm). Adjust to 【25±5 ℃ (target: 25 ℃)】. 开启合成釜下口鼓氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气3±2 min (目标时间：3 min)后开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm)，调温至【25±5 ℃(目标温度：25 ℃)】。 |  |  |  |  |  |  |  |  |  | Times/次数 |  |
|  |  |  |  |  |  |  |  |  |  | 1 |  |
| Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h). Bubble with nitrogen for 3±2 min (target :3 min) 开启合成釜下口鼓氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气3±2 min (目标时间：3 min) |  |  |  |  |  |  |  |  |  |  |  |
| Bubble/鼓氮气 |  |  |  | Performed by & Date/执行人及日期： |  |  |  |  |  |  |  |
| Bubble/鼓氮气 |  |  |  | Begin/开始： |  |  |  |  |  |  |  |
|  |  |  |  | The end time is documented in Ins 3.16.3/结束时间记录于3.16.3 |  |  |  |  |  |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |  |  |  |  |
| Start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm). 开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm) |  |  |  |  |  |  |  |  |  |  |  |
| Stir /搅拌 |  |  |  | Performed by & Date/执行人及日期： |  |  |  |  |  |  |  |
| Stir /搅拌 |  |  |  | Begin/开始： |  |  |  |  |  |  |  |
|  |  |  |  | The end time is documented in Ins 3.16.3/结束时间记录于3.16.3 |  |  |  |  |  |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |  |  |  |  |
| Adjust to 25±5 oC (target: 25 oC)./ 调温至25±5 ℃(目标温度：25 oC) Over-temperature warning value/超温预警设置数值： Lower limit/低限: 22 oC ，High limit/高限: 28 oC |  |  |  |  |  |  |  |  |  |  |  |

| 3.16. （5）Deprotection/脱保护 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Adjust/调温 |  | Performed by & Date/执行人及日期： |  |  |  |
| Adjust/调温 |  | Begin/开始： | End/结束： |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |
| 3.16.3 Fmoc-L-Ser(tBu)-OH De-Fmoc/芴甲氧羰基-叔丁基-L-丝氨酸脱保护 After stirring for【15 ± 2 min (target: 15 min) at 25±5 ℃ (target: 25 ℃)】, stop stirring, turn off the nitrogen. Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point. 控温【25±5 ℃(目标：25 ℃)，搅拌15 ± 2 min（目标：15 min）】后，停止搅拌，关闭氮气。通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  |  | Times/次数 |  |
|  |  |  |  | 1 |  |
| ★Confirm and record that the 20%Piperidine solution temperature is within the NOR range 20 oC ~30 oC ./ 确认并记录20%哌啶溶液温度在正常操作范围内：20 oC ~30 oC。 20%Piperidine solution temperature confirmation/20%哌啶溶液温度确认：____________ Confirmed by& Date/确认人及日期： Checked by & Date/核实人及日期： |  |  |  |  |  |
| De-Fmoc/脱Fmoc &Stir /搅拌 |  | Performed by & Date/执行人及日期：____________________ |  |  |  |
| De-Fmoc/脱Fmoc &Stir /搅拌 |  | End/结束： |  |  |  |
|  |  | The start time is documented in Ins3.16.2 /开始时间记录于3.16.2 |  |  |  |
| Transcribed by & Date /转录人及日期：____________________ |  |  |  |  |  |

| 3.16. （5）Deprotection/脱保护 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bubble/鼓氮气 |  | Performed by & Date/执行人及日期：____________________ |  |  |  |  |
| Bubble/鼓氮气 |  | End/结束： |  |  |  |  |
|  |  | The start time is documented in Ins3.16.2 /开始时间记录于3.16.2 |  |  |  |  |
| Transcribed by & Date /转录人及日期：____________________ |  |  |  |  |  |  |
| Internal temperature/内部温度 |  | High values/高: |  | Low values/最低值： |  |  |
| Transcribed by & Date /转录人及日期：____________________ |  |  |  |  |  |  |
| Filter/过滤 |  | Performed by & Date/执行人及日期：____________________ |  |  |  |  |
|  |  | Checked by & Date/核实人及日期：____________________ |  |  |  |  |
|  |  | Begin/开始： |  | End/结束： |  |  |
| ★After De-Fmoc is completed, the operator shall confirm that all Critical Process Parameters (CPPs) are within their respective Normal Operating Ranges (NOR)/ 脱保护结束后，操作人员须确认所有关键工艺参数（CPP）均处于各自的正常操作范围（NOR）内 Checked by & Date/核实人及日期： |  |  |  |  |  |  |
| CAUTION/注意：Piperidine哌啶：Strong local irritant and may cause permanent injury after short exposure to small amounts. /强烈的局部刺激，短时间少量接触可能造成永久性伤害。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |  |
| 3.16.4 Piperidine solution addition /加哌啶溶液 Add 464.64 kg 【20% (V/V) piperidine/DMF(9.68 kg/kg, 10.40 V)】 (Acceptable weighing range: 450.70 ~ 478.58 kg) solution into the Synthesis reactor (TJ4S-1213-Sy15). 向合成釜（TJ4S-1213-Sy15）中加入464.64 kg【20%哌啶/DMF (V/V)溶液(9.68 kg/kg, 10.40 V)】（可接受称量范围：450.70 ~ 478.58 kg）。 |  |  |  |  |  |  | Times/次数 |
|  |  |  |  |  |  |  | 1 |

| 3.16. （5）Deprotection/脱保护 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Piperidine addition/ 加哌啶溶液 |  | Performed by & Date/执行人及日期：____________________ |  |  |  |  |  |  |
| Piperidine addition/ 加哌啶溶液 |  | Begin/开始： |  |  | End/结束： | Qty/重量： |  |  |
| Transcribed by & Date /转录人及日期：____________________ |  |  |  |  |  |  |  |  |
| 3.16.5 Adjust temperature /调温 Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h) for 3±2 min (target 3 min), Start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm). Adjust to 【25±5 ℃ (target: 25 ℃)】. 开启合成釜下口鼓氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气3±2 min (目标时间：3 min)后开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm)，调温至【25±5 ℃(目标温度：25 ℃)】。 |  |  |  |  |  |  |  | Times/次数 |
|  |  |  |  |  |  |  |  | 1 |
| Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h). Bubble with nitrogen for 3±2 min (target :3 min) 开启合成釜下口鼓氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气3±2 min (目标时间：3 min) |  |  |  |  |  |  |  |  |
| Bubble/鼓氮气 |  |  |  |  | Performed by & Date/执行人及日期：____________________ |  |  |  |
| Bubble/鼓氮气 |  |  |  |  | Begin/开始： |  |  |  |
|  |  |  |  |  | The end time is documented in Ins 3.16.6/结束时间记录于3.16.6 |  |  |  |
| Transcribed by & Date /转录人及日期：____________________ |  |  |  |  |  |  |  |  |
| Start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm). 开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm) |  |  |  |  |  |  |  |  |
| Stir /搅拌 |  |  |  | Performed by & Date/执行人及日期：____________________ |  |  |  |  |
|  |  |  |  | Begin/开始： |  |  |  |  |
|  |  |  |  | The end time is documented in Ins 3.16.6/结束时间记录于3.16.6 |  |  |  |  |
| Transcribed by & Date /转录人及日期：____________________ |  |  |  |  |  |  |  |  |

| 3.16. （5）Deprotection/脱保护 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Adjust to 25±5 oC (target: 25 oC)./ 调温至25±5 ℃(目标温度：25 oC) Over-temperature warning value/超温预警设置数值： Lower limit/低限: 22 oC ，High limit/高限: 28 oC |  |  |  |  |
| Adjust/调温 |  | Performed by & Date/执行人及日期：____________________ |  |  |
| Adjust/调温 |  | Begin/开始： | End/结束： |  |
| Transcribed by & Date /转录人及日期：____________________ |  |  |  |  |
| 3.16.6 Fmoc-L-Ser(tBu)-OH De-Fmoc/芴甲氧羰基-叔丁基-L-丝氨酸脱保护 After stirring for【30 ± 2 min (target: 30 min) at 25±5 ℃ (target: 25 ℃)】, stop stirring, turn off the nitrogen. Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point. 控温【25±5 ℃(目标：25 ℃)，搅拌30 ± 2 min（目标：30 min）】后，停止搅拌，关闭氮气。通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 Over-temperature warning value/超温预警设置数值： Lower limit/低限: 22 oC ，High limit/高限: 28 oC |  |  |  | Times/次数 |
|  |  |  |  | 1 |
| ★Confirm and record that the 20%Piperidine solution temperature is within the NOR range 20 oC ~30 oC ./ 确认并记录20%哌啶溶液温度在正常操作范围内：20 oC ~30 oC。 20%Piperidine solution temperature confirmation/20%哌啶溶液温度确认：____________ Confirmed by& Date/确认人及日期： Checked by & Date/核实人及日期： |  |  |  |  |
| De-Fmoc/脱Fmoc &Stir /搅拌 |  | Performed by & Date/执行人及日期：____________________ |  |  |
| De-Fmoc/脱Fmoc &Stir /搅拌 |  | End/结束： |  |  |
|  |  | The start time is documented in Ins3.16.5 /开始时间记录于3.16.5 |  |  |
| Transcribed by & Date /转录人及日期：____________________ |  |  |  |  |

| 3.16. （5）Deprotection/脱保护 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bubble/鼓氮气 |  |  | Performed by & Date/执行人及日期：____________________ |  |  |  |  |  |
| Bubble/鼓氮气 |  |  | End/结束： |  |  |  |  |  |
|  |  |  | The start time is documented in Ins3.16.5 /开始时间记录于3.16.5 |  |  |  |  |  |
| Transcribed by & Date /转录人及日期：____________________ |  |  |  |  |  |  |  |  |
| Internal temperature/内部温度 |  | High values/高: |  |  | Low values/最低值： |  |  |  |
| Transcribed by & Date /转录人及日期：____________________ |  |  |  |  |  |  |  |  |
| Filter/过滤 |  |  | Performed by & Date/执行人及日期：____________________ |  |  |  |  |  |
|  |  |  | Checked by & Date/核实人及日期：____________________ |  |  |  |  |  |
|  |  |  | Begin/开始： |  | End/结束： |  |  |  |
| ★After De Fmoc is completed, the operator shall confirm that all Critical Process Parameters (CPPs) are within their respective Normal Operating Ranges (NOR)/ 脱保护结束后，操作人员须确认所有关键工艺参数（CPP）均处于各自的正常操作范围（NOR）内 Checked by & Date/核实人及日期： |  |  |  |  |  |  |  |  |  |
| 3.16.7 Adust temperature /调温 Adjust the synthesis reactor (TJ4S-1213-Sy15) jacket to【20±5 ℃ (target: 20 ℃)】. /将合成釜（TJ4S-1213-Sy15）夹套调温至【20±5 ℃（目标温度：20 ℃）】。 |  |  |  |  |  |  |  | Times/次数 |  |
|  |  |  |  |  |  |  |  | 1 |  |
| Adjust/调温 |  |  |  |  | Performed by & Date/执行人及日期：____________________ |  |  |  |  |  |  |
| Adjust/调温 |  |  |  |  | Begin/开始： |  |  | End/结束： |  |  |  |
| Transcribed by & Date /转录人及日期：____________________ |  |  |  |  |  |  |  |  |  |  |  |

| 3.16. （5）Deprotection/脱保护 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CAUTION/注意：IPAC/乙酸异丙酯： Vapors irritate eyes and respiratory tract; high concentrations can be anesthetic. Liquid irritates eyes but causes no serious injury; may cause dermatitis; no serious effects if swallowed. /蒸汽会刺激眼睛和呼吸道；高浓度的蒸汽可能是麻醉剂。液体刺激眼睛，但不会造成严重伤害；可能会引起皮炎；吞食不会造成严重影响。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |  |  |  |
| 3.16.8 Washing after deprotection /脱保护后洗涤 Add 445.92 kg (Acceptable weighing range：432.54 ~ 459.30 kg) (9.29 kg/kg, 10.64 V) isopropyl acetate into synthesis reactor (TJ4S-1213-Sy15). Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h). Bubble with nitrogen for 3±2 min (target 3 min), then start the agitator to 20±10 rpm (target: 20 rpm). After stirring for 5±2 min (target 5 min), stop stirring, turn off the nitrogen. Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point. 向合成釜（TJ4S-1213-Sy15）中加入445.92 kg（可接受称量范围：432.54 ~ 459.30 kg) (9.29 kg/kg, 10.64 V）乙酸异丙酯，开启下口氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气3±2 min (目标：3 min)后开启搅拌转速至20±10 rpm (目标转速: 20 rpm)，搅拌5±2 min (目标时间：5 min)后，停止搅拌，关闭氮气。通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  |  |  |  |  |  | Times/次数 |
|  |  |  |  |  |  |  |  | 4 |
| ★Note: IPAC quantity is a CPP./备注：乙酸异丙酯用量是关键工艺参数。 ★The operator shall confirm that IPAC quantity is within the Normal Operating Range (NOR) 【432.54 kg~459.30 kg】/操作人员须确认乙酸异丙酯用量处于正常操作范围（NOR）内：【432.54 kg~459.30 kg】 Checked by & Date/核实人及日期： |  |  |  |  |  |  |  |  |
| Operation name/操作名称 |  | Time point/时间节点 | Wash /洗涤1 | Wash /洗涤2 | Wash /洗涤3 | Wash /洗涤4 |  |  |
| Add 419.04 kg IPAc (10 V) by spray balls into the synthesis reactor (TJ4S-1213-Sy15)./ 向合成釜(TJ4S-1213-Sy15) 中加419.04 kg乙酸异丙酯（10 V）（Acceptable weighing range/可接受称量范围：440.34~467.58 kg） |  |  |  |  |  |  |  |  |
| IPAC addition/加乙酸异丙酯 |  | Performed by & Date/执行人及日期： |  |  |  |  |  |  |
| IPAC addition/加乙酸异丙酯 |  | Begin/开始 |  |  |  |  |  |  |
|  |  | End/结束 |  |  |  |  |  |  |
|  |  | Qty/重量 |  |  |  |  |  |  |
|  |  | Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |

| 3.16. （5）Deprotection/脱保护 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Operation name/操作名称 |  | Time point/时间节点 | Wash /洗涤1 | Wash /洗涤2 | Wash /洗涤3 | Wash /洗涤4 |
| Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h) Bubble with nitrogen for 5±2 min (target: 5 min). turn off the nitrogen. Start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm). Stir for 5±2 min (target: 5 min). After stirring for 5±2 min (target: 5 min), stop stirring, turn off the nitrogen./ 开启合成釜下口鼓氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min（目标时间：5 min）后开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm)，搅拌5±2 min（目标时间：10 min）后，停止搅拌，关闭氮气 |  |  |  |  |  |  |
| Bubble/鼓氮气 |  | Performed by & Date/执行人及日期： |  |  |  |  |
| Bubble/鼓氮气 |  | Begin/开始 |  |  |  |  |
|  |  | End/结束 |  |  |  |  |
|  |  | Transcribed by & Date /转录人及日期： |  |  |  |  |
| Stir /搅拌 |  | Performed by & Date/执行人及日期： |  |  |  |  |
| Stir /搅拌 |  | Begin/开始 |  |  |  |  |
|  |  | End/结束 |  |  |  |  |
|  |  | Transcribed by & Date /转录人及日期： |  |  |  |  |
| Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point./通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  |  |  |  |  |
| Filter/过滤 |  | Performed by & Date/执行人及日期： |  |  |  |  |
|  |  | Checked by & Date/核实人及日期 |  |  |  |  |
|  |  | Begin/开始 |  |  |  |  |
|  |  | End/结束 |  |  |  |  |

| 3.16. （5）Deprotection/脱保护 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| CAUTION/注意：DMF/N，N-二甲基甲酰胺：Irritation of eyes, skin and nose. May cause nausea. /刺激眼睛、皮肤和鼻子。可能引起恶心。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |  |
| 3.16.9 Add DMF/加N,N-二甲基甲酰胺 Add 546.05 kg (Acceptable weighing range: 529.67 ~ 562.43 kg) (12 V) DMF by spray balls into synthesis reactor (TJ4S-1213-Sy15). 通过淋洗球向合成釜（TJ4S-1213-Sy15）中加入546.05 kg（可接受称量范围：529.67 ~ 562.43 kg）（12 V）N,N-二甲基甲酰胺。 |  |  |  |  |  | Times/次数 |
|  |  |  |  |  |  | 1 |
| DMF addition/ 加N,N-二甲基甲酰胺 | Performed by & Date/执行人及日期： |  |  |  |  |  |
| DMF addition/ 加N,N-二甲基甲酰胺 | Begin/开始： |  | End/结束： |  | Qty/重量： |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |
| 3.16.10 Resin reswelling wash 1/溶胀后洗涤1 Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h) .Bubble with nitrogen for 3±2 min (target: 3 min), then start the agitator to 20±10 rpm (target: 20 rpm). After stirring for 5±2 min (target 5 min), stop stirring, turn off the nitrogen. Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point. 开启合成釜（TJ4S-1213-Sy15）下口鼓氮气, 氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气3±2 min后（目标：3 min），开启搅拌转速至20±10 rpm (目标转速: 20 rpm)，搅拌5±2 min (目标：5 min)后，停止搅拌，关闭氮气。通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  |  |  |  | Times/次数 |
|  |  |  |  |  |  | 1 |
| Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h) for 5±2 min (target: 5 min), Start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm). Stir for 10±5 min (target: 10 min). After stirring for 10±2 min (target: 10 min), stop stirring, turn off the nitrogen./ 开启合成釜下口鼓氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min（目标时间：5 min）后，开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm)，搅拌10 ±2 min（目标时间:10 min）后，停止搅拌，关闭氮气 |  |  |  |  |  |  |
| Bubble/鼓氮气 |  | Performed by & Date/执行人及日期： |  |  |  |  |
| Bubble/鼓氮气 |  | Begin/开始： |  | End/结束： |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |

| 3.16. （5）Deprotection/脱保护 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Stir /搅拌 |  | Performed by & Date/执行人及日期： |  |  |  |  |  |
| Stir /搅拌 |  | Begin/开始： |  |  | End/结束： |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |
| Filter/过滤 |  | Performed by & Date/执行人及日期： |  |  |  |  |  |
|  |  | Checked by & Date/核实人及日期： |  |  |  |  |  |
|  |  | Begin/开始： |  | End/结束： |  |  |  |
| CAUTION/注意：DMF/N，N-二甲基甲酰胺：Irritation of eyes, skin and nose. May cause nausea. /刺激眼睛、皮肤和鼻子。可能引起恶心。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |  |  |
| 3.16.11 Add DMF/加N,N-二甲基甲酰胺 Add 546.05 kg (Acceptable weighing range: 529.67 ~ 562.43 kg) (12 V) DMF by spray balls into synthesis reactor (TJ4S-1213-Sy15). 通过淋洗球向合成釜（TJ4S-1213-Sy15）中加入546.05 kg（可接受称量范围：529.67 ~ 562.43 kg）（12 V）N,N-二甲基甲酰胺。 |  |  |  |  |  |  | Times/次数 |
|  |  |  |  |  |  |  | 1 |
| DMF addition/ 加N,N-二甲基甲酰胺 | Performed by & Date/执行人及日期：____________________ |  |  |  |  |  |  |
| DMF addition/ 加N,N-二甲基甲酰胺 | Begin/开始： |  | End/结束： |  |  | Qty/重量： |  |
| Transcribed by & Date /转录人及日期：____________________ |  |  |  |  |  |  |  |

| 3.16. （5）Deprotection/脱保护 |  |  |  |  |
| --- | --- | --- | --- | --- |
| 3.16.12 Resin reswelling wash 2/溶胀后洗涤2 Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h) .Bubble with nitrogen for 3±2 min (target: 3min), then start the agitator to 20±10 rpm (target: 20 rpm). After stirring for 5±2 min (target 5 min), stop stirring, turn off the nitrogen. Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point. 开启合成釜（TJ4S-1213-Sy15）下口鼓氮气, 氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气3±2 min后（目标：3 min），开启搅拌转速至20±10 rpm (目标转速: 20 rpm)，搅拌5±2 min (目标：5 min)后，停止搅拌，关闭氮气。通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  |  | Times/次数 |
|  |  |  |  | 1 |
| Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h) for 5±2 min (target: 5 min), Start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm). Stir for 10±5 min (target: 10 min). After stirring for 10±2 min (target: 10 min), stop stirring, turn off the nitrogen./ 开启合成釜下口鼓氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min（目标时间：5 min）后，开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm)，搅拌10±2 min（目标时间:10 min）后，停止搅拌，关闭氮气 |  |  |  |  |
| Bubble/鼓氮气 | Performed by & Date/执行人及日期：____________________ |  |  |  |
| Bubble/鼓氮气 | Begin/开始： |  | End/结束： |  |
| Transcribed by & Date /转录人及日期：____________________ |  |  |  |  |
| Stir /搅拌 | Performed by & Date/执行人及日期：____________________ |  |  |  |
| Stir /搅拌 | Begin/开始： |  | End/结束： |  |
| Transcribed by & Date /转录人及日期：____________________ |  |  |  |  |
| Filter/过滤 | Performed by & Date/执行人及日期：____________________ |  |  |  |
|  | Checked by & Date/核实人及日期：____________________ |  |  |  |
|  | Begin/开始： | End/结束： |  |  |

| - （5）Fmoc-L-Ser(tBu)-OH activation/芴甲氧羰基-叔丁基-L-丝氨酸活化 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| CAUTION/注意：DMF/N，N-二甲基甲酰胺：Irritation of eyes, skin and nose. May cause nausea. /刺激眼睛、皮肤和鼻子。可能引起恶心。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |
| 3.17.1 Add DMF to the activation reactor/激活罐加N,N-二甲基甲酰胺 Add 227.52 kg (Acceptable weighing range: 220.69 ~ 234.34 kg) (5 V) DMF into activation tank (TJ4S-1213-Sy15-R01). Start the agitator to 95±5 rpm (target: 95 rpm). 向激活罐（TJ4S-1213-Sy15-R01）中加入227.52 kg（可接受称量范围：220.69 ~ 234.34 kg ）（5 V）N，N-二甲基甲酰胺，开启搅拌转速至95±5 rpm (目标转速: 95 rpm)。 |  |  |  | Times/次数 |  |
|  |  |  |  | 1 |  |
| Add 227.52 kg (Acceptable weighing range: 220.69 ~ 234.34 kg) (5 V) DMF into activation tank (TJ4S-1213-Sy15-R01). Start the agitator to 95±5 rpm (target: 95 rpm)./向激活罐（TJ4S-1213-Sy15-R01）中加入227.52 kg（可接受称量范围：220.69 ~ 234.34 kg ）（5 V）N，N-二甲基甲酰胺。 |  |  |  |  |  |
| DMF addition/加N,N-二甲基甲酰胺 | Performed by & Date/执行人及日期： |  |  |  |  |
| DMF addition/加N,N-二甲基甲酰胺 | Begin/开始： | End/结束： | Qty/重量： |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |
| Start the agitator of activation tank to 95±5 rpm (target: 95 rpm)./ 开启激活罐搅拌转速至95±5 rpm (目标转速: 95 rpm)。 |  |  |  |  |  |
| Stir/搅拌 | Performed by & Date/执行人及日期： |  |  |  |  |
| Stir /搅拌 | Begin/开始： |  |  |  |  |
|  | The end time is documented in Ins 3.17.8/结束时间记录于3.17.8 |  |  |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |
| 3.17.2 Add Fmoc-L-Ser(tBu)-OH /加芴甲氧羰基-叔丁基-L-丝氨酸 Add Fmoc-L-Ser(tBu)-OH (2.00 eq.) with powder feeding bags into activation tank (TJ4S-1213-Sy15-R01). 通过投料袋向激活罐（TJ4S-1213-Sy15-R01）中加入 kg芴甲氧羰基-叔丁基-L-丝氨酸（Acceptable weighing range/可接受称量范围： kg ~ kg）(2.00 eq.)。 |  |  |  | Times/次数 |  |
|  |  |  |  | 1 |  |

| 3.17. （5）Fmoc-L-Ser(tBu)-OH activation/芴甲氧羰基-叔丁基-L-丝氨酸活化 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Before material charging, confirm the material is within the retest date/ the expiry date,Confirm whether the weight is adequate for use in the current charging. /加物料前，执行物料在复检日期/有效期之内，确认重量是否满足本次投料使用。 Before solid material charging, confirm/加固体物料前，确认： - Before operation performed, No non-related material / equipment / instrument / tools / pipeline / document / record / label in the intended area /操作前，目标区域无不相关物料/设备/仪器/工具/管路/文件/记录/标签； - The outer packages are removed. If the outer package cannot be removed, but unpack material without damaging the package bag, and before charging, check no package fragment and other extraneous matter in material. /外包装移除；如果外包装不能移除，但在不破坏包装的情况下打开包装袋，投料前以及过程中同时检查执行物料中无包装碎片以及异物 Checked by & Date/核实人及日期： |  |  |  |  |
| Prior to charging each amino acid, the operator shall perform a Raman scan to confirm the identity of the material. The Raman scan result shall be compared against the reference spectrum of the corresponding amino acid. Only materials that conform to the reference spectrum may be charged. If the Raman scan result does not conform, stop the operation, quarantine the material, and initiate a investigation per site SOP. Record the Raman scan result, operator signature, and date/time in the table below./ 每次投氨基酸前，操作人员应进行拉曼扫描以执行物料。拉曼扫描结果应与相应氨基酸的参考光谱进行比对。仅与参考光谱相符的物料方可投料。如拉曼扫描结果不相符，应停止操作，对物料进行隔离，并依据厂区 SOP 启动异常事件调查。在下表中记录拉曼扫描结果、操作人员签字及日期 / 时间。 |  |  |  |  |
| Fmoc-L- Ser(tBu)-OH addition/加芴甲氧羰基-叔丁基-L-丝氨酸 |  | Performed by & Date/执行人及日期： |  |  |
|  |  | Begin/开始： |  | End/结束： |
| Transcribed by & Date /转录人及日期： |  |  |  |  |
| Fmoc-L- Ser(tBu)-OH confirmation/芴甲氧羰基-叔丁基-L-丝氨酸确认：Control No./内部控制号YL003428 |  |  |  |  |
| Check items/检查项 | Is it correct?/是否正确 |  | Recorded by & Date/ 记录人及日期 | Checked by & Date/ 核实人及日期 |
| Material name/物料名称 | Yes/是；No/否 |  |  |  |
| Project batch number/ 项目批次号 | Yes/是；No/否 |  |  |  |
| Operation number/ 操作号 | Yes/是；No/否 |  |  |  |
| Control No./内部控制号 | Yes/是；No/否 |  |  |  |
| Raman Scan Confirmation/拉曼扫描执行 | Conforms/相符 Does not conform/不相符 |  |  |  |

| 3.17. （5）Fmoc-L-Ser(tBu)-OH activation/芴甲氧羰基-叔丁基-L-丝氨酸活化 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| No. /编号 |  | Net weight/净重 |  | All in/全部加入 | Recorded by & Date/ 记录人及日期 | Checked by & Date/ 核实人及日期 |
| / |  |  |  | Yes/是；No/否 |  |  |  |
| / |  |  |  | Yes/是；No/否 |  |  |  |
| / |  |  |  | Yes/是；No/否 |  |  |  |
| / |  |  |  | Yes/是；No/否 |  |  |  |
| / |  |  |  | Yes/是；No/否 |  |  |  |
| / |  |  |  | Yes/是；No/否 |  |  |  |
| / |  |  |  | Yes/是；No/否 |  |  |  |
| / |  |  |  | Yes/是；No/否 |  |  |  |
| / |  |  |  | Yes/是；No/否 |  |  |  |
| / |  |  |  | Yes/是；No/否 |  |  |  |
| / |  |  |  | Yes/是；No/否 |  |  |  |
| / |  |  |  | Yes/是；No/否 |  |  |  |
| Total/合计 |  |  |  | Confirm the total net weight is within the input range/执行合计净重在投料范围内 |  | Recorded by & Date/记录人及日期 | Checked by & Date/ 核实人及日期 |
|  |  |  |  | Yes/是；No/否 |  |  |  |
| After solid material charging, confirm/固体物料投料之后，确认： - The corresponding leftover material has been properly sealed and transferred to the staging area/相应的剩余物料已经恰当密封，转移到了暂存区域。 - The waste packaging material has been sealed and transferred to the corresponding waste staging area. /产生的包材废物已经密封并转移到了相应的废物暂存区域。 - The operation area/equipment surface has been cleaned/操作区域以及设备表面已经清洁。 Checked by & Date/核实人及日期： |  |  |  |  |  |  |  |

| 3.17. （5）Fmoc-L-Ser(tBu)-OH activation/芴甲氧羰基-叔丁基-L-丝氨酸活化 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3.17.3 Agitation of activation tank /激活罐搅拌 Continue agitation of activation tank to 95±10 rpm (target: 95 rpm). Maintain Stirring for NLT 15 min. Confirm no visible solids and later use. 维持激活罐搅拌转速至95±10 rpm (目标转速: 95 rpm)，继续搅拌至少15 min，确认无明显固体待用。 |  |  |  |  |  |  | Times/次数 |  |  |
|  |  |  |  |  |  |  | 1 |  |  |
| Stir /搅拌 |  | Performed by & Date/执行人及日期： |  |  |  |  |  |  |  |
| Maintain &Stir /保持搅拌 |  | Begin/开始： |  |  |  |  |  |  |  |
|  |  | The end time is documented in Ins 3.17.8/结束时间记录于3.17.8 |  |  |  |  |  |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |  |  |
| Confirm no visible solid/确认无明显固体： |  |  |  | Confirmed by& Date /确认人及日期： |  |  |  |  |  |
| 3.17.4 Adjust activation tan temperature /激活罐调温 Adjust the activation tank (TJ4S-1213-Sy15-R01) mixture to 【20±5 ℃ (target 20 ℃) 】. /将激活罐(TJ4S-1213-Sy15-R01)中体系调温至【20±5 ℃（目标：20 ℃）】。 |  |  |  |  |  |  |  |  | Times/次数 |  |  |  |
|  |  |  |  |  |  |  |  |  | 1 |  |  |  |
| Adjust/调温 |  |  |  | Performed by & Date/执行人及日期： |  |  |  |  |  |  |
| Adjust/调温 |  |  |  | Begin/开始： |  | End/结束： |  |  |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |  |  |  |
| CAUTION/注意：OxymaPure is highly toxic, high-energy compound. /2-肟氰乙酸乙酯是高毒，高能化合物。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |  |  |  |  |
| 3.17.5 Add OxymaPure /加2-肟氰乙酸乙酯 Add OxymaPure (2.00 eq.) with powder feeding bags into activation tank (TJ4S-1213-Sy15-R01) at 20±5 ℃ (target 20 ℃). 控温20±5 ℃（目标：20 ℃），通过投料袋向激活罐（TJ4S-1213-Sy15-R01）中加入 kg 2-肟氰乙酸乙酯（Acceptable weighing range/可接受称量范围： kg ~ kg）(2.00 eq.)。 |  |  |  |  |  |  | Times/次数 |  |  |
|  |  |  |  |  |  |  | 1 |  |  |

3.17. （5）Fmoc-L-Ser(tBu)-OH activation/芴甲氧羰基-叔丁基-L-丝氨酸活化

Before material charging, confirm the material is within the retest date/ the expiry date,Confirm whether the weight is adequate for use in the current charging. /加物料前，执行物料在复检日期/有效期之内，确认重量是否满足本次投料使用。 Before solid material charging, confirm/加固体物料前，确认： - Before operation performed, confirm there is no non-related material / equipment / instrument / tools / pipeline / document / record / label in the intended area /操作前，执行目标区域无不相关物料/设备/仪器/工具/管路/文件/记录/标签 - The outer packages are removed. If the outer package cannot be removed, but un-package material without damaging the package bag, and before charging, check no package fragment and other extraneous matter in material. /外包装移除；如果外包装不能移除，但在不破坏包装的情况下打开包装袋，投料前以及过程中同时检查执行物料中无包装碎片以及异物 Checked by & Date/核实人及日期：

Check and record that the solid charge port is free of solid accumulation from the previous step./ 检查并记录固体投料口无来自上一步骤的固体残留。 Checked by & Date/核实人及日期：

| 3.17. （5）Fmoc-L-Ser(tBu)-OH activation/芴甲氧羰基-叔丁基-L-丝氨酸活化 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Charge OxymaPure /加2-肟氰乙酸乙酯 |  |  |  |  | Performed by & Date/执行人及日期： |  |  |  |  |
|  |  |  |  |  | Begin/开始： |  | End/结束： |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |  |  |
| Internal temperature/温度 |  |  |  | High values/最高值： |  |  |  | Low values/最低值： |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |  |  |
| OxymaPure confirmation/2-肟氰乙酸乙酯确认：Control No./内部控制号：YL005180 |  |  |  |  |  |  |  |  |  |
| Check items/检查项 |  |  | Is it correct?/是否正确 |  |  | Recorded by & Date/ 记录人及日期 |  |  | Checked by & Date/ 核实人及日期 |
| Material name/物料名称 |  |  | Yes/是；No/否 |  |  |  |  |  |  |
| Project batch number/ 项目批次号 |  |  | Yes/是；No/否 |  |  |  |  |  |  |
| Check items/检查项 |  |  | Is it correct?/是否正确 |  |  | Recorded by & Date/ 记录人及日期 |  |  | Checked by & Date/ 核实人及日期 |  |
| Operation number/ 操作号 |  |  | Yes/是；No/否 |  |  |  |  |  |  |  |
| Control No./内部控制号 |  |  | Yes/是；No/否 |  |  |  |  |  |  |  |
| Raman Scan Confirmation/拉曼扫描执行 |  |  | Conforms/相符 Does not conform/不相符 |  |  |  |  |  |  |  |
| No. /编号 |  | Net weight/净重 | All in/全部加入 |  |  | Recorded by & Date/ 记录人及日期 |  |  | Checked by & Date/ 核实人及日期 |
| / |  |  | Yes/是；No/否 |  |  |  |  |  |  |
| / |  |  | Yes/是；No/否 |  |  |  |  |  |  |
| / |  |  | Yes/是；No/否 |  |  |  |  |  |  |
| / |  |  | Yes/是；No/否 |  |  |  |  |  |  |
| / |  |  | Yes/是；No/否 |  |  |  |  |  |  |
| / |  |  | Yes/是；No/否 |  |  |  |  |  |  |
| / |  |  | Yes/是；No/否 |  |  |  |  |  |  |

| 3.17. （5）Fmoc-L-Ser(tBu)-OH activation/芴甲氧羰基-叔丁基-L-丝氨酸活化 |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No. /编号 |  |  | Net weight/净重 | All in/全部加入 |  | Recorded by & Date/ 记录人及日期 |  |  | Checked by & Date/ 核实人及日期 |  |  |
| / |  |  |  | Yes/是；No/否 |  |  |  |  |  |  |  |
| / |  |  |  | Yes/是；No/否 |  |  |  |  |  |  |  |
| / |  |  |  | Yes/是；No/否 |  |  |  |  |  |  |  |
| / |  |  |  | Yes/是；No/否 |  |  |  |  |  |  |  |
| / |  |  |  | Yes/是；No/否 |  |  |  |  |  |  |  |
| Total/合计 |  |  |  | Confirm the total net weight is within the input range/执行合计净重在投料范围内 |  | Recorded by & Date/记录人及日期 |  |  | Checked by & Date/ 核实人及日期 |  |  |
|  |  |  |  | Yes/是；No/否 |  |  |  |  |  |  |  |
| After solid material charging, confirm/固体物料投料之后，确认： - The corresponding leftover material has been properly sealed and transferred to the staging area/相应的剩余物料已经恰当密封，转移到了暂存区域。 - The waste packaging material has been sealed and transferred to the corresponding waste staging area. /产生的包材废物已经密封并转移到了相应的废物暂存区域。 - The operation area/equipment surface has been cleaned/操作区域以及设备表面已经清洁。 Checked by & Date/核实人及日期： |  |  |  |  |  |  |  |  |  |  |  |
| CAUTION/注意：DMF/N，N-二甲基甲酰胺：Irritation of eyes, skin and nose. May cause nausea. /刺激眼睛、皮肤和鼻子。可能引起恶心。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |  |  |  |  |  |  |
| 3.17.6 Add DMF /加N,N-二甲基甲酰胺 Rinse the port of Activation tank (TJ4S-1213-Sy15-R01) with 91.01 kg（Acceptable weighing range：88.28 ~ 93.74 kg）（2 V）DMF. 用91.01 kg（可接受称量范围：88.28 ~ 93.74 kg）（2 V）N,N-二甲基甲酰胺淋洗激活罐（TJ4S-1213-Sy15-R01）加料口。 |  |  |  |  |  |  |  |  |  | Times/次数 |  |
|  |  |  |  |  |  |  |  |  |  | 1 |  |
| Rinse with DMF/ N,N-二甲基甲酰胺淋洗 |  |  |  | Performed by & Date/执行人及日期： |  |  |  |  |  |  |  |
|  |  |  |  | Begin/开始： |  | End/结束： |  |  | Qty/重量： |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |  |  |  |  |
| 3.17. （5）Fmoc-L-Ser(tBu)-OH activation/芴甲氧羰基-叔丁基-L-丝氨酸活化 |  |  |  |  |  |  |  |  |  |  |  |  |
| Stir /搅拌 |  |  |  | Performed by & Date/执行人及日期： |  |  |  |  |  |  |  |
| Stir /搅拌 |  |  |  | Begin/开始： |  |  | End/结束： |  |  |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |  |  |  |  |
| CAUTION/注意：DIC is a key EHS concern chemical, highly toxic, skin sensitizing, respiratory sensitizing. /N,N’二异丙基碳二亚胺为EHS重点关注化学品，高毒，皮肤致敏，呼吸致敏。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |  |  |  |  |  |  |
| 3.17.7 DIC addition /加N,N-二异丙基碳二亚胺 Add DIC (2.20 eq.) into the activation tank (TJ4S-1213-Sy15-R01) at reference time of 10-30 min at 20±5 ℃ (target: 20 ℃). The addition rate depends on temperature control. 控温20±5 ℃ (目标20 ℃)，向激活罐（TJ4S-1213-Sy15-R01）中滴加 kg N,N'-二异丙基碳二亚胺（Acceptable weighing range/可接受称量范围： kg ~ kg）(2.20 eq.)（滴加参考时间10-30 min，以控温为主）。 Pump number/泵编号： Scale ID. No./秤编号： Material charging pipe number/物料管路编号： |  |  |  |  |  |  |  |  |  |  | Times/次数 |
|  |  |  |  |  |  |  |  |  |  |  | 1 |
| ★Note:The DIC addition temperature is a CPP./ 备注：N,N'-二异丙基碳二亚胺加料温度是关键控制参数。 Prior to DIC addition, the operator shall confirm that the reaction mixture temperature is within the Normal Operating Range (NOR) / N,N'-二异丙基碳二亚胺投料前，操作人员须执行反应体系温度处于正常操作范围（NOR）内: ★Lower limit/低限: XoC ，High limit/高限: XoC Over-temperature warning value/超温预警设置数值： Lower limit/低限: 22 oC ，High limit/高限: 28 oC |  |  |  |  |  |  |  |  |  |  |  |
| Before material charging, confirm the material is within the retest date/ the expiry date, Confirm whether the weight is adequate for use in the current charging. /加物料前，执行物料在复检日期/有效期之内，确认重量是否满足本次投料使用。 Performed by & Date/执行人及日期： |  |  |  |  |  |  |  |  |  |  |  |

| 3.17. （5）Fmoc-L-Ser(tBu)-OH activation/芴甲氧羰基-叔丁基-L-丝氨酸活化 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Before drummed liquid material charging, visual check there is no other extraneous matter in material. /加桶装液体物料前，应目视执行桶内无异物。 Visual Examination of Foreign Matters for Drum Solvent/桶装溶剂异物目视检查: 1. Place the solvent drum in a well-lit area (or use a portable light source) to ensure adequate visibility of the liquid contents./ 将溶剂桶置于光线充足区域（或使用便携式光源），确保液体内容物可见性良好 2. Visually inspect the solvent through the drum bung opening or transparent sight glass for any visible foreign matters, including but not limited to particulates, fibers, sediments, or unusual turbidity./ 通过桶口或透明视镜目视检查溶剂中是否有可见异物，包括但不限于颗粒、纤维、沉淀物或异常浑浊。 3. Confirm that the solvent is clear and free of visible foreign matters before charging. If any foreign matter is observed, do not use the drum and follow the applicable handling procedure for subsequent disposition./ 投料前确认溶剂澄清且无可见异物。如发现任何异物，不得使用该桶，并依据处理流程进行后续处置。 Checked by & Date/核实人及日期： |  |  |  |
| Confirm liquid charging pipeline is released. /确认加料管路经过放行。 |  |  |  |  |
| Checked by & Date/核实人及日期： |  |  |  |  |
| Charging of drummed solvent: verify that the pipeline connections remain consistent before and after material feeding, then record the weighing reading 桶装溶剂溶液称量时，确认投料前后管路连接一致，读取称量示数。 Performed by & Date/执行人及日期： Checked by & Date/核实人及日期： |  |  |  |  |
| DIC /N,N’-二异丙基碳二亚胺：Control Number/内部控制号：YL003564 |  |  |  |  |
| Internal temperature/内部温度 |  | High values/最高值： | Low values/最低值： |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |
| If the target net weight≥the planned net weight, perform full drum charge; otherwise, perform partial (Non-full drum) charge. /当目标加料量≥计划加入净重，勾选进行整桶加料；否则勾选进行零头加料。 Note: Target net weight to be charged = Total target net weight – Total net weight charged. Expected tare weight of drum after charge = Gross weight of drum that to be charged - Target net weight to be charged. 还需加入净重目标值= 加料步骤目标总净重-已加入净重总量 加料完成后预计桶皮重=待加料桶毛重-还需加入净重目标值 |  |  |  |  |

| 3.17. （5）Fmoc-L-Ser(tBu)-OH activation/芴甲氧羰基-叔丁基-L-丝氨酸活化 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Addition type加料式 |  | Lot number/批号 | Pre-charging qty calculation/投料前投料量计算 |  |  |  |  |  |  | Actual Charging Qty/实际投料量 |  |  |  |  |
|  |  |  | Total qty charged/已加入净重总量 |  | Target qty to be charged/还需加入净重目标值 | Gross qty of drum that to be charged/待加料桶毛重 | Expected net weight of drum after charge /加料完成后预计桶净重 | Expected tare weight of drum after charge/加料完成后预计桶皮重 |  | Before charging/ 投料前 | After charging/ 投料后 | Net weight/ 净重 | Performed by & Date/ 执行人及日期 | Checked by & Date/ 核实人及日期 |
| Full drum整桶 Non-full drum /非整桶 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Full drum整桶 Non-full drum /非整桶 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Total weight /合计重量 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DIC Addition/加N,N'-二异丙基碳二亚胺 |  |  |  | Begin/开始： |  |  |  |  | End/结束： |  |  |  |  |  |
| Transcribed by & Date/转录人及日期： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| 3.17. （5）Fmoc-L-Ser(tBu)-OH activation/芴甲氧羰基-叔丁基-L-丝氨酸活化 |  |  |  |
| --- | --- | --- | --- |
| 3.17.8 Fmoc-L-Ser(tBu)-OH activation /芴甲氧羰基-叔丁基-L-丝氨酸活化 At 20±5 ℃ (target: 20 ℃). Continue agitation of activation tank to 95±10 rpm (target: 95 rpm). Maintain for NLT 15 min (Target：20 min) after addition, then stop stirring. 滴毕，控温20±5 ℃(目标温度：20 ℃)，维持激活罐搅拌转速至95±10 rpm (目标转速: 95 rpm),继续搅拌20±5 min（目标时间：20 min），停止搅拌。 |  |  | Times/次数 |
|  |  |  | 1 |
| Stir /搅拌 | Performed by & Date/执行人及日期： |  |  |
| Stir /搅拌 | End/结束 |  |  |
|  | The start time is documented in Ins 3.17.1 /开始时间记录于3.17.1 |  |  |
| Transcribed by & Date/转录人及日期： |  |  |  |
| Internal temperature/内部温度 | High values/最高值： | Low values/最低值： |  |
| Transcribed by & Date/转录人及日期： |  |  |  |

| - （5）Fmoc-L-Ser(tBu)-OH coupling/芴甲氧羰基-叔丁基-L-丝氨酸偶联 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 3.18.1 Activation mixture transfer /活化体系转移 Transfer the mixture in activation tank (TJ4S-1213-Sy15-R01) into synthesis reactor (TJ4S-1213-Sy15). 将激活罐（TJ4S-1213-Sy15-R01）中体系转移至合成釜（TJ4S-1213-Sy15）中。 |  |  |  |  |  | Times/次数 |  |
|  |  |  |  |  |  | 1 |  |
| ★Note:The coupling temperature is a CPP, and the coupling IPC hold temperature is a CPP./ 备注：偶联温度是CPP，偶联IPC维持温度是关键工艺参数。 The operator shall confirm that the reaction mixture temperature is within the Normal Operating Range (NOR) 【25±5oC (target: 25oC)】 /操作人员须执行反应体系温度处于正常操作范围（NOR）内【25±5oC (target: 25oC)】 Checked by & Date/核实人及日期： ★Lower limit/低限: 13 oC ，High limit/高限: 37 oC Over-temperature warning value/超温预警设置数值： Lower limit/低限: 21 oC ，High limit/高限: 28 oC |  |  |  |  |  |  |  |
| Transfer/转移 |  | Performed by & Date/执行人及日期： |  |  |  |  |  |
|  |  | Checked by & Date/核实人及日期： |  |  |  |  |  |
|  |  | Begin/开始： |  | End/结束： |  |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |
| CAUTION/注意：DMF/N，N-二甲基甲酰胺：Irritation of eyes, skin and nose. May cause nausea. /刺激眼睛、皮肤和鼻子。可能引起恶心。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |  |  |
| 3.18.2 Add DMF /加N,N-二甲基甲酰胺 Add 45.50 kg (Acceptable weighing range: 43.50 ~ 47.50 kg) (1 V) DMF by spray balls into activation tank (TJ4S-1213-Sy15-R01) to rinse the wall. 通过淋洗球向激活罐（TJ4S-1213-Sy15-R01）中加入45.50 kg（可接受称量范围：43.50 ~ 47.50 kg） (1 V) N,N-二甲基甲酰胺淋洗罐壁。 |  |  |  |  |  |  | Times/次数 |
|  |  |  |  |  |  |  | 1 |
| Rinse with DMF/ N,N-二甲基甲酰胺淋洗 | Performed by & Date/执行人及日期： |  |  |  |  |  |  |
| Rinse with DMF/ N,N-二甲基甲酰胺淋洗 | Begin/开始： |  | End/结束： |  | Qty/重量： |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |
| 3.18.3 Transfer /转移 Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h). Bubble with nitrogen for 5±2 min (target: 5 min), then start the agitator. Transfer the mixture in activation tank (TJ4S-1213-Sy15-R01) into synthesis reactor (TJ4S-1213-Sy15). 合成釜（TJ4S-1213-Sy15）开启下口氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min (目标：5 min)后，开启搅拌转速至20±10 rpm (目标转速: 20 rpm)，将激活罐（TJ4S-1213-Sy15-R01）中体系转移至合成釜（TJ4S-1213-Sy15）中。 |  |  |  |  |  |  | Times/次数 |
|  |  |  |  |  |  |  | 1 |

| 3.18. （5）Fmoc-L-Ser(tBu)-OH coupling/芴甲氧羰基-叔丁基-L-丝氨酸偶联 |  |  |  |
| --- | --- | --- | --- |
| Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h). Bubble with nitrogen for 5±2 min (target: 5 min), Start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm). Stir for 10 ±5 min (target: 10 min)./ 开启合成釜下口鼓氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min（目标时间：5 min）后开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm)，搅拌10±5 min (目标时间: 10 min)。 |  |  |  |  |
| Bubble/鼓氮气 |  |  | Performed by & Date/执行人及日期： |  |
| Bubble/鼓氮气 |  |  | Begin/开始： |  |
|  |  |  | The end time is documented in Ins3.18.5 /结束时间记录于3.18.5 |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |
| Stir/搅拌 |  |  | Performed by & Date/执行人及日期： |  |
| Stir /搅拌 |  |  | Begin/开始： |  |
|  |  |  | The end time is documented in Ins3.18.5 /结束时间记录于3.18.5 |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |
| Transfer the mixture (DMF) from the activation tank (TJ4S-1213-Sy15-R01) to the synthesis reactor (TJ4S-1213-Sy15). /激活罐(TJ4S-1213-Sy15-R01)中体系转移至合成釜(TJ4S-1213-Sy15)中。 |  |  |  |  |
| Transfer/转移 |  | Performed by & Date/执行人及日期： |  |  |
|  |  | Checked by & Date/核实人及日期： |  |  |
|  |  | Begin/开始： |  | End/结束： |

| 3.18.4 Adjust synthesis reactor temperature /合成釜调温 Adjust the synthesis reactor (TJ4S-1213-Sy15) mixture to【25±5 ℃(target：25 ℃)】. 合成釜(TJ4S-1213-Sy15)体系调温至【25±5 ℃(目标温度：25 ℃)】。 | Times/次数 |
| --- | --- |
|  | 1 |
| The total reaction Fmoc-L-Ser(tBu)-OH coupling time：NMT 840 min (Sourced from the holding time study in the PAR study report.)/ 芴甲氧羰基-叔丁基-L-丝氨酸偶联总的反应时间不超过840min(来源于 PAR 研究报告中稳定时间研究)。 Over-temperature warning value/超温预警设置数值： Lower limit/低限: 21 oC ，High limit/高限: 28 oC |  |

| 3.18. （5）Fmoc-L-Ser(tBu)-OH coupling/芴甲氧羰基-叔丁基-L-丝氨酸偶联 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Adjust/调温 |  | Performed by & Date/执行人及日期： |  |  |
| Adjust/调温 |  | Begin/开始： | End/结束： |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |
| 3.18.5 Fmoc-L-Ser(tBu)-OH coupling/ 芴甲氧羰基-叔丁基-L-丝氨酸偶联 The mixture is allowed to react at 25±5 ℃ (target: 25 ℃). After stirring for 3 h±15 min (target: 3 h), then sample the mixture every 0.5~1 h for ninhydrin test analysis until two consecutive samples test as negative (Yellow to Light pink Report Negative= Complete coupling; Blue to Purple Positive= Incomplete coupling). And the mixture should be stirred for coupling time (4 h≤Coupling Time≤10 h). Stop stirring, turn off the nitrogen. Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point. 控温25±5 ℃(目标：25 ℃)反应，反应3 h±15 min（目标：3 h）后，然后取样进行茚三酮检测，每隔0.5~1.0 h取样一次，直至连续两次取样结果为阴性（黄色至浅粉色，报告阴性=偶联完全；蓝色至紫色，阳性=偶联不完全）后，且反应时间满足（4h≤反应时间≤10h）后，停止搅拌，关闭氮气。通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 Note: If resin adherence is found at other times during the reaction, rinse the reactor with 0.50~5.00 kg DMF each time. /备注：偶联过程中其它时间发现贴壁，每次使用0.50~5.00 kg N,N-二甲基甲酰胺淋洗，记录见附加页 |  |  |  | Times/次数 |
|  |  |  |  | 1 |
| React/反应 |  | Performed by & Date/执行人及日期： |  |  |

| 3.18. （5）Fmoc-L-Ser(tBu)-OH coupling/芴甲氧羰基-叔丁基-L-丝氨酸偶联 |  |  |  |
| --- | --- | --- | --- |
| ★After coupling is completed, the operator shall confirm that all Critical Process Parameters (CPP) are within their respective Normal Operating Ranges (NOR)/ 偶联结束后，操作人员须确认所有关键工艺参数（CPP）均处于各自的正常操作范围（NOR）内 ★Note: The coupling temperature is a CPP, and the coupling IPC hold temperature is a CPP./备注：偶联温度是CPP，偶联IPC维持温度是关键工艺参数。 The operator shall confirm that the reaction mixture temperature is within the Normal Operating Range (NOR) :20~30℃ /操作人员须确认反应体系温度处于正常操作范围（NOR）内:20~30℃ □Yes/是 □No/否 Confirmed by & Date/确认人及日期： Checked by & Date/核实人及日期： |  |  |  |
| React/反应 |  | Begin/开始： | End/结束： |
| Transcribed by & Date /转录人及日期： |  |  |  |

| 3.18. （5）Fmoc-L-Ser(tBu)-OH coupling/芴甲氧羰基-叔丁基-L-丝氨酸偶联 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Standard sampling method/标准取样方法 |  |  |  |  |  |  |  |  |  |
| Take 5-10 ml mixture and perform testing in accordance with the Kaiser test record. During sampling, should wear nitrile gloves and 3M 6000 series Full-face piece respirator, 6006CN cartridges. 通过在线取样器取样5~10ml按照茚三酮检测记录进行检测。取样过程中，应该戴丁腈手套、3M 6000系列全面罩, 6006CN滤毒盒。 |  |  |  |  |  |  |  |  |  |
| Sampling number/ 取样编号 |  | Sampling time/取样时间 | Sampled by & Date/取样人及日期 |  | Negative or Positive/阳性或阴性 |  |  | Result filled by &Date/结果填写人及日期 | Checked by & Date/核实人及日期 |
| CPo140020- -01- -3.18.5-01 |  |  |  |  | Negative /阴性 Positive /阳性 |  |  |  |  |
| CPo140020- -01- -3.18.5-02 |  |  |  |  | Negative /阴性 Positive /阳性 |  |  |  |  |
| CPo140020- -01- -3.18.5-03 |  |  |  |  | Negative /阴性 Positive /阳性 |  |  |  |  |
| CPo140020- -01- -3.18.5-04 |  |  |  |  | Negative /阴性 Positive /阳性 |  |  |  |  |
| CPo140020- -01- -3.18.5-05 |  |  |  |  | Negative /阴性 Positive /阳性 |  |  |  |  |
| CPo140020- -01- -3.18.5-06 |  |  |  |  | Negative /阴性 Positive /阳性 |  |  |  |  |
| CPo140020- -01- -3.18.5-07 |  |  |  |  | Negative /阴性 Positive /阳性 |  |  |  |  |
| CPo140020- -01- -3.18.5-08 |  |  |  |  | Negative /阴性 Positive /阳性 |  |  |  |  |
| Note:For a failing coupling test.Continue hourly sampling up to the NMT 841 min hold limit; if still positive, stop, notify PM/QA and raise a deviation; no re-coupling without an approved deviation.偶联反应检测不合格的处置规则:按每小时 1 次的频率继续取样检测，直至达到不超过（NMT）841 min的保温时限；若到达时限后检测仍为阳性，停止反应，通知项目经理（PM）/ 质量保证部门（QA）并发起偏差报告；未经偏差批准，不得进行补偶联操作。 |  |  |  |  |  |  |  |  |  |
| Confirm the end point of the reaction and move to following operation. /执行反应终点后进行下一步操作。 The acceptance criterion is two consecutive negative test results./连续两次检测结果为阴性即为合格。 Confirmed by & Date/确认人及日期： Checked by & Date/核实人及日期： |  |  |  |  |  |  |  |  |  |
| Stir/搅拌 |  |  |  | Performed by & Date/执行人及日期： |  |  |  |  |  |
| Stir /搅拌 |  |  |  | End/结束： |  |  |  |  |  |
|  |  |  |  | The start time is documented in Ins 3.18.3 /开始时间记录于3.18.3 |  |  |  |  |  |
| Transcribed by & Date/转录人及日期： |  |  |  |  |  |  |  |  |  |
| Bubble/鼓氮气 |  |  |  | Performed by & Date/执行人及日期： |  |  |  |  |  |
| Bubble/鼓氮气 |  |  |  | End/结束： |  |  |  |  |  |
|  |  |  |  | The start time is documented in Ins 3.18.3 /开始时间记录于3.18.3 |  |  |  |  |  |
| Transcribed by & Date/转录人及日期： |  |  |  |  |  |  |  |  |  |
| Internal temperature/内部温度 |  |  |  | High values/最高值： |  |  | Low values/最低值： |  |  |
| Transcribed by & Date/转录人及日期： |  |  |  |  |  |  |  |  |  |
| Filter/过滤 |  |  | Performed by & Date/执行人及日期: |  |  |  |  |  |
|  |  |  | Checked by & Date/核实人及日期： |  |  |  |  |  |
|  |  |  | Begin/开始： |  | End/结束： |  |  |  |
| ★After coupling is completed, the operator shall confirm that all Critical Process Parameters (CPPs) are within their respective Normal Operating Ranges (NOR)/ 偶联结束后，操作人员须确认所有关键工艺参数（CPP）均处于各自的正常操作范围（NOR）内。 Checked by & Date/核实人及日期： |  |  |  |  |  |  |  |  |

| 3.18. （5）Fmoc-L-Ser(tBu)-OH coupling/芴甲氧羰基-叔丁基-L-丝氨酸偶联 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| CAUTION/注意：DMF/N，N-二甲基甲酰胺：Irritation of eyes, skin and nose. May cause nausea. /刺激眼睛、皮肤和鼻子。可能引起恶心。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |  |
| 3.18.6 Activation Tank Rinse/激活罐洗涤 Add 364.03 kg (Acceptable weighing range：353.11 ~ 374.95 kg) (8 V) DMF into the activation tank (TJ4S-1213-Sy15-R01). Circulate and clean the activating liquid transfer pipeline, liquid charging port cleaning pipeline and solid charging port cleaning pipeline. Clean every line wash for 2±1 min (target: 2 min). 向激活罐（TJ4S-1213-Sy15-R01）中加入364.03 kg（可接受称量范围：353.11 ~ 374.95 kg） (8 V) N,N-二甲基甲酰胺，循环清洗激活罐转移管路，液体加料口清洗管路和固体加料口清洗管路，每条线路各清洗2±1 min（目标时间：2 min）。 |  |  |  |  |  | Times/次数 |
|  |  |  |  |  |  | 3 |
| Operation name/ 操作名称 |  | Time point/时间节点 | Clean the activating liquid transfer pipeline 1/循环清洗激活液转移管路1 | Liquid charging port cleaning pipeline 2 /液体加料口清洗管路2 | Solid charging port cleaning pipeline 3,/固体加料口清洗管路3 |  |
|  |  |  | Wash /洗涤1 | Wash /洗涤2 | Wash /洗涤3 |  |
| DMF addition/ 加N,N-二甲基甲酰胺 |  | Performed by & Date/执行人及日期： |  |  |  |  |
| DMF addition/ 加N,N-二甲基甲酰胺 |  | Begin/开始 |  |  |  |  |
|  |  | End/结束 |  |  |  |  |
|  |  | Qty/重量 |  |  |  |  |
|  |  | Transcribed by & Date /转录人及日期： |  |  |  |  |

| 3.18. （5）Fmoc-L-Ser(tBu)-OH coupling/芴甲氧羰基-叔丁基-L-丝氨酸偶联 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Operation name/ 操作名称 |  | Time point/时间节点 | Clean the activating liquid transfer pipeline 1/循环清洗激活液转移管路1 | Liquid charging port cleaning pipeline 2 /液体加料口清洗管路2 | Solid charging port cleaning pipeline 3,/固体加料口清洗管路3 |  |  |
|  |  |  | Wash /洗涤1 | Wash /洗涤2 | Wash /洗涤3 |  |  |
| Circulate and clean/循环清洗 |  | Performed by & Date/执行人及日期： |  |  |  |  |  |
| Circulate and clean/循环清洗 |  | Begin/开始 |  |  |  |  |  |
|  |  | End/结束 |  |  |  |  |  |
|  |  | Transcribed by & Date /转录人及日期： |  |  |  |  |  |
| 3.18.7 Post-coupling wash./偶联后洗涤 Transfer the mixture in activation tank (TJ4S-1213-Sy15-R01) to synthesis reactor (TJ4S-1213-Sy15). Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h). Bubble with nitrogen for 5±2 min (target 5 min), start the agitator. After stirring for 7±2 min (target：7 min), stop stirring, turn off the nitrogen. Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point. 将激活罐（TJ4S-1213-Sy15-R01）洗涤液转入合成釜 (TJ4S-1213-Sy15)。开启合成釜下口鼓氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min (目标：5 min)后开启搅拌转速至20±10 rpm (目标转速: 20 rpm)，搅拌7±2 min（目标：7 min）后停止搅拌，关闭氮气。通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  |  |  |  | Times/次数 |  |
|  |  |  |  |  |  | 3 |  |
| Transfer the liquid to the synthesis reactor (TJ4S-1213-Sy15) ./将激活罐中体系转移至合成釜中。 |  |  |  |  |  |  |
| Operation name/ 操作名称 | Time point/时间节点 | Wash /洗涤1 | Wash /洗涤2 | Wash /洗涤3 |  |  |
| Transfer/转移 | Performed by & Date/执行人及日期： |  |  |  |  |  |
|  | Checked by & Date/核实人及日期： |  |  |  |  |  |
|  | Begin/开始 |  |  |  |  |  |
|  | End/结束 |  |  |  |  |  |
| 3.18. （5）Fmoc-L-Ser(tBu)-OH coupling/芴甲氧羰基-叔丁基-L-丝氨酸偶联 |  |  |  |  |  |  |  |
| Operation name/ 操作名称 |  | Time point/时间节点 | Wash /洗涤1 | Wash /洗涤2 | Wash /洗涤3 |  |  |
| Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h). Bubble with nitrogen for 5±2 min (target 5 min), start the agitator. After stirring for 7±2 min (target：7 min), stop stirring, turn off the nitrogen.开启合成釜下口鼓氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min (目标时间：5 min)后开启搅拌转速至20±10 rpm (目标转速: 20 rpm)，搅拌7±2 min（目标时间：7 min）后停止搅拌，关闭氮气。 |  |  |  |  |  |  |  |
| Bubble/鼓氮气 |  | Performed by & Date/执行人及日期： |  |  |  |  |  |
| Bubble/鼓氮气 |  | Begin/开始 |  |  |  |  |  |
|  |  | End/结束 |  |  |  |  |  |
|  |  | Transcribed by & Date /转录人及日期： |  |  |  |  |  |
| Stir /搅拌 |  | Performed by & Date/执行人及日期： |  |  |  |  |  |
| Stir /搅拌 |  | Begin/开始 |  |  |  |  |  |
|  |  | End/结束 |  |  |  |  |  |
|  |  | Transcribed by & Date /转录人及日期 |  |  |  |  |  |
| Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point/通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  |  |  |  |  |  |
| Filter/过滤 |  | Performed by & Date/执行人及日期： |  |  |  |  |
|  |  | Checked by & Date/核实人及日期： |  |  |  |  |
|  |  | Begin/开始 |  |  |  |  |
|  |  | End/结束 |  |  |  |  |

| 3.19. （6）Deprotection/脱保护 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CAUTION/注意：Piperidine哌啶：Strong local irritant and may cause permanent injury after short exposure to small amounts. /强烈的局部刺激，短时间少量接触可能造成永久性伤害。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |  |  |
| 3.19.1 Add piperidine solution /加哌啶溶液 Adjust the jacket temperature of synthesis reactor (TJ4S-1213-Sy15) to 【25±5 oC (target: 25 oC). 】Add 357.50 kg 【20% (V/V) piperidine/DMF (8 V)】 (Acceptable weighing range: 346.77 ~ 368.23 kg) solution into the Synthesis reactor (TJ4S-1213-Sy15). 将合成釜(TJ4S-1213-Sy15)夹套调温至【25±5 ℃（目标温度：25 ℃）】。向合成釜（TJ4S-1213-Sy15）中加入357.50 kg【20%哌啶/DMF (V/V)溶液(8 V)】（可接受称量范围：346.77 ~ 368.23 kg）。 |  |  |  |  |  |  | Times/次数 |
|  |  |  |  |  |  |  | 1 |
| Adjust/调温 |  |  | Performed by & Date/执行人及日期： |  |  |  |  |
| Adjust/调温 |  |  | Begin/开始： |  | Adjust/调温 |  |  |
| Transcribed by & Date/转录人及日期： |  |  |  |  |  |  |  |
| Confirm and record that the 20%Piperidine solution concentration is within the NOR range./ 确认并记录20%哌啶溶液浓度在正常操作范围内：17.50%~22.50% 20%Piperidine solution concentration confirmation/20%哌啶溶液浓度确认：____________ Confirm and record that the 20%Piperidine solution temperature is within the NOR range./ 确认并记录20%哌啶溶液温度在正常操作范围内：20 oC ~30 oC 20%Piperidine solution temperature confirmation/20%哌啶溶液温度确认：____________ Confirmed by & Date/确认人及日期： Checked by & Date/核实人及日期： |  |  |  |  |  |  |  |
| Piperidine addition/ 加哌啶溶液 |  | Performed by & Date/执行人及日期： |  |  |  |  |  |
| Piperidine addition/ 加哌啶溶液 |  | Begin/开始： |  | End/结束： |  | Qty/重量： |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |

| 3.19. （6）Deprotection/脱保护 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 3.19.2 Adjust synthesis reactor temperature /合成釜调温 Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h) for 5±2 min (target 5 min), Start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm). Stir for 10±5 min (target: 10 min). Adjust to 【25±5 ℃ (target: 25 ℃)】. 开启合成釜下口鼓氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min (目标时间：5 min)后开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm)，调温至【25±5 ℃(目标温度：25 ℃)】。 |  |  |  |  | Times/次数 |
|  |  |  |  |  | 1 |
| Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h). Bubble with nitrogen for 5±2 min (target :5 min) 开启合成釜下口鼓氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min (目标时间：5 min) |  |  |  |  |  |
| Bubble/鼓氮气 |  |  | Performed by & Date/执行人及日期： |  |  |
| Bubble/鼓氮气 |  |  | Begin/开始： |  |  |
|  |  |  | The end time is documented in Ins 3.19.3/结束时间记录于3.19.3 |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |
| Start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm). 开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm) |  |  |  |  |  |
| Stir /搅拌 |  | Performed by & Date/执行人及日期： |  |  |  |
| Stir /搅拌 |  | Begin/开始： |  |  |  |
|  |  | The end time is documented in Ins 3.19.3/结束时间记录于3.19.3 |  |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |
| Adjust to 25±5 oC (target: 25 oC)./ 调温至25±5 ℃(目标温度：25 oC) |  |  |  |  |  |
| Adjust/调温 |  |  | Performed by & Date/执行人及日期： |  |  |
| Adjust/调温 |  |  | Begin/开始： | End/结束： |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |

| 3.19. （6）Deprotection/脱保护 |  |  |  |  |
| --- | --- | --- | --- | --- |
| 3.19.3 Fmoc-L-Ser(tBu)-OH activation De-Fmoc/芴甲氧羰基-叔丁基-L-丝氨酸脱保护 After stirring for【2 h±15 min (target: 2 h) at 25±5 ℃ (target: 25 ℃)】, stop stirring, turn off the nitrogen. Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point. 控温【25±5 ℃(目标：25 ℃)，搅拌2 h±15 min（目标：2 h）】后，停止搅拌，关闭氮气。通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 Over-temperature warning value/超温预警设置数值： Lower limit/低限: 22 oC ，High limit/高限: 28 oC |  |  | Times/次数 |
|  |  |  | 1 |
| ★Confirm and record that the 20%Piperidine solution temperature is within the NOR range 20 oC ~30 oC ./ 确认并记录20%哌啶溶液温度在正常操作范围内：20 oC ~30 oC。 20%Piperidine solution temperature confirmation/20%哌啶溶液温度确认：____________ Confirmed by& Date/确认人及日期： Checked by & Date/核实人及日期： |  |  |  |
| De-Fmoc/脱Fmoc &Stir /搅拌 | Performed by & Date/执行人及日期： |  |  |
| De-Fmoc/脱Fmoc &Stir /搅拌 | End/结束： |  |  |
|  | The start time is documented in Ins3.19.2 /开始时间记录于3.19.2 |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |

| 3.19. （6）Deprotection/脱保护 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Bubble/鼓氮气 |  | Performed by & Date/执行人及日期： |  |  |  |  |
| Bubble/鼓氮气 |  | End/结束： |  |  |  |  |
|  |  | The start time is documented in Ins3.19.2 /开始时间记录于3.19.2 |  |  |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |
| Internal temperature/内部温度 |  | High values/最高值: |  | Low values/最低值： |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |
| Filter/过滤 |  | Performed by & Date/执行人及日期： |  |  |  |  |
|  |  | Checked by & Date/核实人及日期： |  |  |  |  |
|  |  | Begin/开始： |  | End/结束： |  |  |
| ★After De Fmoc is completed, the operator shall confirm that all Critical Process Parameters (CPPs) are within their respective Normal Operating Ranges (NOR)/ 脱保护结束后，操作人员须确认所有关键工艺参数（CPP）均处于各自的正常操作范围（NOR）内 Checked by & Date/核实人及日期： |  |  |  |  |  |  |
| 3.19.4 Adjust the synthesis reactor /合成釜调温 Adjust the synthesis reactor (TJ4S-1213-Sy15) jacket to【20±5 ℃ (target: 20 ℃)】. /将合成釜（TJ4S-1213-Sy15）夹套调温至【20±5 ℃（目标温度：20 ℃）】。 |  |  |  |  | Times/次数 |  |
|  |  |  |  |  | 1 |  |
| Adjust/调温 |  | Performed by & Date/执行人及日期： |  |  |  |  |
| Adjust/调温 |  | Begin/开始： |  | End/结束： |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |

| 3.19. （6）Deprotection/脱保护 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CAUTION/注意：IPAC/乙酸异丙酯： Vapors irritate eyes and respiratory tract; high concentrations can be anesthetic. Liquid irritates eyes but causes no serious injury; may cause dermatitis; no serious effects if swallowed. /蒸汽会刺激眼睛和呼吸道；高浓度的蒸汽可能是麻醉剂。液体刺激眼睛，但不会造成严重伤害；可能会引起皮炎；吞食不会造成严重影响。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |  |  |
| 3.19.5 Post-deprotection wash/脱保护后洗涤 Add 445.92 kg (Acceptable weighing range：432.54 ~ 459.30 kg) (9.29 kg/kg, 10.64 V V) isopropyl acetate into synthesis reactor (TJ4S-1213-Sy15). Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h). Bubble with nitrogen for 5±2 min (target 5 min), then start the agitator to 20±10 rpm (target: 20 rpm). After stirring for 5±2 min (target 5 min), stop stirring, turn off the nitrogen. Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point. 向合成釜（TJ4S-1213-Sy15）中加入445.92 kg（可接受称量范围：432.54 ~ 459.30 kg) (9.29 kg/kg, 10.64 V）乙酸异丙酯，开启下口氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min (目标：5 min)后开启搅拌转速至20±10 rpm (目标转速: 20 rpm)，搅拌5±2 min (目标：5 min)后，停止搅拌，关闭氮气。通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  |  |  |  |  | Times/次数 |
|  |  |  |  |  |  |  | 4 |
| ★Note: IPAC quantity is a CPP./备注：乙酸异丙酯用量是关键工艺参数。 ★The operator shall confirm that IPAC quantity is within the Normal Operating Range (NOR) 【432.54 kg~459.30 kg】/操作人员须确认乙酸异丙酯用量处于正常操作范围（NOR）内：【432.54 kg~459.30 kg】 Checked by & Date/核实人及日期： |  |  |  |  |  |  |  |
| Operation name/操作名称 |  | Time point/时间节点 | Wash /洗涤1 | Wash /洗涤2 | Wash /洗涤3 | Wash /洗涤4 |  |
| Add 419.04 kg IPAc (10 V) by spray balls into the synthesis reactor (TJ4S-1213-Sy15)./ 向合成釜(TJ4S-1213-Sy15) 中加419.04 kg乙酸异丙酯（10 V）（Acceptable weighing range/可接受称量范围：440.34~467.58kg） |  |  |  |  |  |  |  |
| IPAC addition/加乙酸异丙酯 |  | Performed by & Date/执行人及日期： |  |  |  |  |  |
| IPAC addition/加乙酸异丙酯 |  | Begin/开始 |  |  |  |  |  |
|  |  | End/结束 |  |  |  |  |  |
|  |  | Qty/重量 |  |  |  |  |  |
|  |  | Transcribed by & Date /转录人及日期： |  |  |  |  |  |

| 3.19. （6）Deprotection/脱保护 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h). Bubble with nitrogen for 5±2 min (target: 5 min). turn off the nitrogen. Start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm). Stir for 5±2 min (target: 5 min). After stirring for 5±2 min (target: 5 min), stop stirring, turn off the nitrogen./ 开启合成釜下口鼓氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min（目标时间：5 min）后开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm)，搅拌5±2 min（目标时间：10 min）后，停止搅拌，关闭氮气。 |  |  |  |  |  |
| Bubble/鼓氮气 | Performed by & Date/执行人及日期： |  |  |  |  |
| Bubble/鼓氮气 | Begin/开始 |  |  |  |  |
|  | End/结束 |  |  |  |  |
|  | Transcribed by & Date /转录人及日期： |  |  |  |  |
| Stir /搅拌 | Performed by & Date/执行人及日期： |  |  |  |  |
| Stir /搅拌 | Begin/开始 |  |  |  |  |
|  | End/结束 |  |  |  |  |
|  | Transcribed by & Date /转录人及日期： |  |  |  |  |
| Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point./通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  |  |  |  |
| Filter/过滤 | Performed by & Date/执行人及日期： |  |  |  |  |
|  | Checked by & Date/核实人及日期 |  |  |  |  |
|  | Begin/开始 |  |  |  |  |
|  | End/结束 |  |  |  |  |

| 3.19. （6）Deprotection/脱保护 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| CAUTION/注意：DMF/N，N-二甲基甲酰胺：Irritation of eyes, skin and nose. May cause nausea. /刺激眼睛、皮肤和鼻子。可能引起恶心。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |  |
| 3.19.6 DMF addition /加N,N-二甲基甲酰胺 Add 546.05 kg (Acceptable weighing range: 529.67 ~ 562.43 kg) (12 V) DMF by spray balls into synthesis reactor (TJ4S-1213-Sy15). 通过淋洗球向合成釜（TJ4S-1213-Sy15）中加入546.05 kg（可接受称量范围：529.67 ~ 562.43 kg）（12 V）N,N-二甲基甲酰胺。 |  |  |  |  |  | Times/次数 |
|  |  |  |  |  |  | 1 |
| DMF addition/ 加N,N-二甲基甲酰胺 | Performed by & Date/执行人及日期： |  |  |  |  |  |
| DMF addition/ 加N,N-二甲基甲酰胺 | Begin/开始： |  | End/结束： |  | Qty/重量： |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |
| 3.19.7 Resin reswelling wash 1/溶胀后洗涤1 Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h) .Bubble with nitrogen for 5±2 min (target: 5 min), then start the agitator to 20±10 rpm (target: 20 rpm). After stirring for 10±2 min (target 10 min), stop stirring, turn off the nitrogen. Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point. 开启合成釜（TJ4S-1213-Sy15）下口鼓氮气, 氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min后（目标：5 min），开启搅拌转速至20±10 rpm (目标转速: 20 rpm)，搅拌10±2 min (目标：10 min)后，停止搅拌，关闭氮气。通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  |  |  |  | Times/次数 |
|  |  |  |  |  |  | 1 |
| Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h) for 5±2 min (target: 5 min), Start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm). Stir for 10±5 min (target: 10 min). After stirring for 10±2 min (target: 10 min), stop stirring, turn off the nitrogen./ 开启合成釜下口鼓氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min（目标时间：5 min）后，开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm)，搅拌10±2 min（目标时间:10 min）后，停止搅拌，关闭氮气。 |  |  |  |  |  |  |
| Bubble/鼓氮气 |  | Performed by & Date/执行人及日期： |  |  |  |  |
| Bubble/鼓氮气 |  | Begin/开始： |  | End/结束： |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |

| 3.19. （6）Deprotection/脱保护 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Stir /搅拌 |  | Performed by & Date/执行人及日期： |  |  |  |  |  |
| Stir /搅拌 |  | Begin/开始： |  |  | End/结束： |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |
| Filter/过滤 |  | Performed by & Date/执行人及日期： |  |  |  |  |  |
|  |  | Checked by & Date/核实人及日期： |  |  |  |  |  |
|  |  | Begin/开始： |  | End/结束： |  |  |  |
| CAUTION/注意：DMF/N，N-二甲基甲酰胺：Irritation of eyes, skin and nose. May cause nausea. /刺激眼睛、皮肤和鼻子。可能引起恶心。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |  |  |
| 3.19.8 DMF addition /加N,N-二甲基甲酰胺 Add 546.05 kg (Acceptable weighing range: 529.67 ~ 562.43 kg) (12 V) DMF by spray balls into synthesis reactor (TJ4S-1213-Sy15). 通过淋洗球向合成釜（TJ4S-1213-Sy15）中加入546.05 kg（可接受称量范围：529.67 ~ 562.43 kg）（12 V）N,N-二甲基甲酰胺。 |  |  |  |  |  |  | Times/次数 |
|  |  |  |  |  |  |  | 1 |
| DMF addition/ 加N,N-二甲基甲酰胺 | Performed by & Date/执行人及日期： |  |  |  |  |  |  |
| DMF addition/ 加N,N-二甲基甲酰胺 | Begin/开始： |  | End/结束： |  |  | Qty/重量： |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |
| 3.19.9 Resin reswelling wash 2/溶胀后洗涤2 Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h). Bubble with nitrogen for 5±2 min (target: 5 min), then start the agitator to 20±10 rpm (target: 20 rpm). After stirring for 10±2 min (target 10 min), stop stirring, turn off the nitrogen. Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point. 开启合成釜（TJ4S-1213-Sy15）下口鼓氮气, 氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min后（目标：5 min），开启搅拌转速至20±10 rpm (目标转速: 20 rpm)，搅拌10±2 min (目标：10 min)后，停止搅拌，关闭氮气。通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  |  |  |  |  | Times/次数 |
|  |  |  |  |  |  |  | 1 |

| 3.19. （6）Deprotection/脱保护 |  |  |  |
| --- | --- | --- | --- |
| Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h) for 5±2 min (target: 5 min), Start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm). Stir for 10±5 min (target: 10 min). After stirring for 10±2 min (target: 10 min), stop stirring, turn off the nitrogen./ 开启合成釜下口鼓氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min（目标时间：5 min）后，开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm)，搅拌10±2 min（目标时间:10 min）后，停止搅拌，关闭氮气 |  |  |  |
| Bubble/鼓氮气 | Performed by & Date/执行人及日期： |  |  |
| Bubble/鼓氮气 | Begin/开始： |  | End/结束： |
| Transcribed by & Date /转录人及日期： |  |  |  |
| Stir /搅拌 | Performed by & Date/执行人及日期： |  |  |
| Stir /搅拌 | Begin/开始： |  | End/结束： |
| Transcribed by & Date /转录人及日期： |  |  |  |
| Filter/过滤 | Performed by & Date/执行人及日期： |  |  |
|  | Checked by & Date/核实人及日期： |  |  |
|  | Begin/开始： | End/结束： |  |

| - （6）Fmoc-L-Ser(tBu)-OH activation/芴甲氧羰基-叔丁基-L-丝氨酸活化 |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CAUTION/注意：DMF/N，N-二甲基甲酰胺：Irritation of eyes, skin and nose. May cause nausea. /刺激眼睛、皮肤和鼻子。可能引起恶心。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |  |  |  |  |  |  |
| 3.20.1 Add DMF to the activation reactor/激活罐加N,N-二甲基甲酰胺 Add 227.52 kg (Acceptable weighing range: 220.69 ~ 234.34 kg) (5 V) DMF into activation tank (TJ4S-1213-Sy15-R01). Start the agitator to 95±10 rpm (target: 95 rpm). 向激活罐（TJ4S-1213-Sy15-R01）中加入227.52 kg（可接受称量范围：220.69 ~ 234.34 kg ）（5 V）N，N-二甲基甲酰胺，开启搅拌转速至95±10 rpm (目标转速: 95 rpm)。 |  |  |  |  |  |  |  |  |  | Times/次数 |  |
|  |  |  |  |  |  |  |  |  |  | 1 |  |
| Add 227.52 kg (Acceptable weighing range: 220.69 ~ 234.34 kg) (5 V) DMF into activation tank (TJ4S-1213-Sy15-R01). Start the agitator to 95±10 rpm (target: 95 rpm)./向激活罐（TJ4S-1213-Sy15-R01）中加入227.52 kg（可接受称量范围：220.69 ~ 234.34 kg ）（5 V）N，N-二甲基甲酰胺。 |  |  |  |  |  |  |  |  |  |  |  |
| DMF addition/加N,N-二甲基甲酰胺 |  |  | Performed by & Date/执行人及日期： |  |  |  |  |  |  |  |  |
| DMF addition/加N,N-二甲基甲酰胺 |  |  | Begin/开始： | End/结束： |  |  |  | Qty/重量： |  |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |  |  |  |  |
| Start the agitator of activation tank to 95±10 rpm (target: 95 rpm)./ 开启激活罐搅拌转速至95±10 rpm (目标转速: 95 rpm)。 |  |  |  |  |  |  |  |  |  |  |  |
| Stir/搅拌 |  |  | Performed by & Date/执行人及日期： |  |  |  |  |  |  |  |  |
|  |  |  | Begin/开始： |  |  |  |  |  |  |  |  |
|  |  |  | The end time is documented in Ins 3.20.8/结束时间记录于3.20.8 |  |  |  |  |  |  |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |  |  |  |  |
| 3.20.2 Fmoc-Ser(tBu)-OH addition /加芴甲氧羰基-叔丁基-L-丝氨酸 Add Fmoc-L-Ser(tBu)-OH (2.00 eq.) with powder feeding bags into activation tank (TJ4S-1213-Sy15-R01). 通过投料袋向激活罐（TJ4S-1213-Sy15-R01）中加入 kg芴甲氧羰基-叔丁基-L-丝氨酸（Acceptable weighing range/可接受称量范围： kg ~ kg）(2.00 eq.)。 |  |  |  |  |  |  |  |  |  | Times/次数 |  |
|  |  |  |  |  |  |  |  |  |  | 1 |  |
| Before material charging, confirm the material is within the retest date/ the expiry date,Confirm whether the weight is adequate for use in the current charging. /加物料前，执行物料在复检日期/有效期之内，确认重量是否满足本次投料使用。 Before solid material charging, confirm/加固体物料前，确认： - Before operation performed, No non-related material / equipment / instrument / tools / pipeline / document / record / label in the intended area /操作前，目标区域无不相关物料/设备/仪器/工具/管路/文件/记录/标签； - The outer packages are removed. If the outer package cannot be removed, but unpack material without damaging the package bag, and before charging, check no package fragment and other extraneous matter in material. /外包装移除；如果外包装不能移除，但在不破坏包装的情况下打开包装袋，投料前以及过程中同时检查执行物料中无包装碎片以及异物 Checked by & Date/核实人及日期： |  |  |  |  |  |  |  |  |  |  |  |
| 3.20. （6）Fmoc-L-Ser(tBu)-OH activation/芴甲氧羰基-叔丁基-L-丝氨酸活化 |  |  |  |  |  |  |  |  |  |  |  |
| Prior to charging each amino acid, the operator shall perform a Raman scan to confirm the identity of the material. The Raman scan result shall be compared against the reference spectrum of the corresponding amino acid. Only materials that conform to the reference spectrum may be charged. If the Raman scan result does not conform, stop the operation, quarantine the material, and initiate a investigation per site SOP. Record the Raman scan result, operator signature, and date/time in the table below./ 每次投氨基酸前，操作人员应进行拉曼扫描以执行物料。拉曼扫描结果应与相应氨基酸的参考光谱进行比对。仅与参考光谱相符的物料方可投料。如拉曼扫描结果不相符，应停止操作，对物料进行隔离，并依据厂区 SOP 启动异常事件调查。在下表中记录拉曼扫描结果、操作人员签字及日期 / 时间。 |  |  |  |  |  |  |  |  |  |  |  |
| Fmoc-L- Ser(tBu)-OH addition/加芴甲氧羰基-叔丁基-L-丝氨酸 |  |  |  |  | Performed by & Date/执行人及日期： |  |  |  |  |  |  |
|  |  |  |  |  | Begin/开始： |  | End/结束： |  |  |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |  |  |  |  |
| Fmoc-L- Ser(tBu)-OH confirmation/芴甲氧羰基-叔丁基-L-丝氨酸执行：Control No./内部控制号YL003428 |  |  |  |  |  |  |  |  |  |  |  |
| Check items/检查项 |  |  | Is it correct?/是否正确 |  |  | Recorded by & Date/ 记录人及日期 |  | Checked by & Date/ 核实人及日期 |  |  |  |
| Material name/物料名称 |  |  | Yes/是；No/否 |  |  |  |  |  |  |  |  |
| Project batch number/ 项目批次号 |  |  | Yes/是；No/否 |  |  |  |  |  |  |  |  |
| Operation number/ 操作号 |  |  | Yes/是；No/否 |  |  |  |  |  |  |  |  |
| Control No./内部控制号 |  |  | Yes/是；No/否 |  |  |  |  |  |  |  |  |
| Raman Scan Confirmation/拉曼扫描执行 |  |  | Conforms/相符 Does not conform/不相符 |  |  |  |  |  |  |  |  |
| No. /编号 |  | Net weight/净重 | All in/全部加入 |  |  | Recorded by & Date/ 记录人及日期 |  | Checked by & Date/ 核实人及日期 |  |  |  |
| / |  |  | Yes/是；No/否 |  |  |  |  |  |  |  |  |
| / |  |  | Yes/是；No/否 |  |  |  |  |  |  |  |  |
| / |  |  | Yes/是；No/否 |  |  |  |  |  |  |  |  |
| / |  |  | Yes/是；No/否 |  |  |  |  |  |  |  |  |
| / |  |  | Yes/是；No/否 |  |  |  |  |  |  |  |  |
| / |  |  | Yes/是；No/否 |  |  |  |  |  |  |  |  |
| / |  |  | Yes/是；No/否 |  |  |  |  |  |  |  |  |

| 3.20. （6）Fmoc-L-Ser(tBu)-OH activation/芴甲氧羰基-叔丁基-L-丝氨酸活化 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| No. /编号 | Net weight/净重 | All in/全部加入 |  |  | Recorded by & Date/ 记录人及日期 | Checked by & Date/ 核实人及日期 |  |
| / |  | Yes/是；No/否 |  |  |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |  |
| Total/合计 |  | Confirm the total net weight is within the input range/执行合计净重在投料范围内 |  |  | Recorded by & Date/记录人及日期 | Checked by & Date/ 核实人及日期 |  |
|  |  | Yes/是；No/否 |  |  |  |  |  |
| After solid material charging, confirm/固体物料投料之后，确认： - The corresponding leftover material has been properly sealed and transferred to the staging area/相应的剩余物料已经恰当密封，转移到了暂存区域。 - The waste packaging material has been sealed and transferred to the corresponding waste staging area. /产生的包材废物已经密封并转移到了相应的废物暂存区域。 - The operation area/equipment surface has been cleaned/操作区域以及设备表面已经清洁。 Checkmed by & Date/核实人及日期： |  |  |  |  |  |  |  |
| 3.20.3 Agitation of activation tank /激活罐搅拌 Continue agitation of activation tank to 95±10 rpm (target: 95 rpm). Maintain stirring for at least 15 min. Confirm no visible solids and later use. 维持激活罐搅拌转速至95±10 rpm (目标转速: 95 rpm), 搅拌至少15 min，确认无明显固体待用。 |  |  |  |  |  |  | Times/次数 |
|  |  |  |  |  |  |  | 1 |
| Stir /搅拌 |  |  | Performed by & Date/执行人及日期： |  |  |  |  |
| Stir /搅拌 |  |  | Begin/开始： |  |  |  |  |
|  |  |  | The end time is documented in Ins 3.20.8/结束时间记录于3.20.8 |  |  |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |
| Confirm no visible solid/确认无明显固体： |  |  |  | Confirmed by& Date /确认人及日期： |  |  |  |

| 3.20. （6）Fmoc-L-Ser(tBu)-OH activation/芴甲氧羰基-叔丁基-L-丝氨酸活化 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3.20.4 Adjust activation tan temperature /激活罐调温 Adjust the activation tank (TJ4S-1213-Sy15-R01) mixture to 【20±5 ℃ (target 20 ℃) 】. /将激活罐(TJ4S-1213-Sy15-R01)中体系调温至【20±5 ℃（目标：20 ℃）】。 |  |  |  |  | Times/次数 |  |  |  |
|  |  |  |  |  | 1 |  |  |  |
| Adjust/调温 |  | Performed by & Date/执行人及日期： |  |  |  |  |  |
| Adjust/调温 |  | Begin/开始： |  | End/结束： |  |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |
| CAUTION/注意：OxymaPure is highly toxic, high-energy compound. /2-肟氰乙酸乙酯是高毒，高能化合物。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |  |  |
| 3.20.5 OxymaPure addition /加2-肟氰乙酸乙酯 Add OxymaPure (2.00 eq.)with powder feeding bags into activation tank (TJ4S-1213-Sy15-R01) at 【20±5 ℃ (target: 20 ℃】. 控温【20±5 ℃（目标温度：20℃）】，通过投料袋向激活罐（TJ4S-1213-Sy15-R01）中加入 kg 2-肟氰乙酸乙酯（Acceptable weighing range/可接受称量范围： kg ~ kg）(2.00 eq.)。 |  |  |  |  |  | Times/次数 |  |  |
|  |  |  |  |  |  | 1 |  |  |
| Before material charging, confirm the material is within the retest date/ the expiry date,Confirm whether the weight is adequate for use in the current charging. /加物料前，执行物料在复检日期/有效期之内，确认重量是否满足本次投料使用。 Before solid material charging, confirm/加固体物料前，确认： - Before operation performed, confirm there is no non-related material / equipment / instrument / tools / pipeline / document / record / label in the intended area /操作前；目标区域无不相关物料/设备/仪器/工具/管路/文件/记录/标签 - The outer packages are removed. If the outer package cannot be removed, but un-package material without damaging the package bag, and before charging, check no package fragment and other extraneous matter in material. /外包装移除；如果外包装不能移除，但在不破坏包装的情况下打开包装袋，投料前以及过程中同时检查执行物料中无包装碎片以及异物 Checked by & Date/核实人及日期： |  |  |  |  |  |  |  |  |
| Check and record that the solid charge port is free of solid accumulation from the previous step./ 检查并记录固体投料口无来自上一步骤的固体残留。 Checked by & Date/核实人及日期： |  |  |  |  |  |  |  |
| Charge OxymaPure /加2-肟氰乙酸乙酯 |  | Performed by & Date/执行人及日期： |  |  |  |  |  |
|  |  | Begin/开始： |  | End/结束： |  |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |

| 3.20. （6）Fmoc-L-Ser(tBu)-OH activation/芴甲氧羰基-叔丁基-L-丝氨酸活化 |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Internal temperature/温度 |  |  |  |  | High values/最高值： |  |  | Low values/最低值： |  |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |  |  |  |  |
| OxymaPure confirmation/2-肟氰乙酸乙酯确认：Control No./内部控制号：YL005180 |  |  |  |  |  |  |  |  |  |  |  |
| Check items/检查项 |  |  |  | Is it correct?/是否正确 |  |  | Recorded by & Date/ 记录人及日期 |  |  | Checked by & Date/ 核实人及日期 |  |
| Material name/物料名称 |  |  |  | Yes/是；No/否 |  |  |  |  |  |  |  |
| Project batch number/ 项目批次号 |  |  |  | Yes/是；No/否 |  |  |  |  |  |  |  |
| Operation number/ 操作号 |  |  |  | Yes/是；No/否 |  |  |  |  |  |  |  |  |  |
| Control No./内部控制号 |  |  |  | Yes/是；No/否 |  |  |  |  |  |  |  |  |  |
| Raman Scan Confirmation/拉曼扫描执行 |  |  |  | Conforms/相符 Does not conform/不相符 |  |  |  |  |  |  |  |  |  |
| No. /编号 |  |  | Net weight/净重 | All in/全部加入 |  |  | Recorded by & Date/ 记录人及日期 |  |  | Checked by & Date/ 核实人及日期 |  |
| / |  |  |  | Yes/是；No/否 |  |  |  |  |  |  |  |
| / |  |  |  | Yes/是；No/否 |  |  |  |  |  |  |  |
| / |  |  |  | Yes/是；No/否 |  |  |  |  |  |  |  |
| / |  |  |  | Yes/是；No/否 |  |  |  |  |  |  |  |
| / |  |  |  | Yes/是；No/否 |  |  |  |  |  |  |  |
| / |  |  |  | Yes/是；No/否 |  |  |  |  |  |  |  |
| / |  |  |  | Yes/是；No/否 |  |  |  |  |  |  |  |
| / |  |  |  | Yes/是；No/否 |  |  |  |  |  |  |  |
| / |  |  |  | Yes/是；No/否 |  |  |  |  |  |  |  |
| / |  |  |  | Yes/是；No/否 |  |  |  |  |  |  |  |
| Total/合计 |  |  |  | Confirm the total net weight is within the input range/执行合计净重在投料范围内 |  |  | Recorded by & Date/记录人及日期 |  |  | Checked by & Date/ 核实人及日期 |  |
|  |  |  |  | Yes/是；No/否 |  |  |  |  |  |  |  |

| 3.20. （6）Fmoc-L-Ser(tBu)-OH activation/芴甲氧羰基-叔丁基-L-丝氨酸活化 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| After solid material charging, confirm/固体物料投料之后，确认： - The corresponding leftover material has been properly sealed and transferred to the staging area/相应的剩余物料已经恰当密封，转移到了暂存区域。 - The waste packaging material has been sealed and transferred to the corresponding waste staging area. /产生的包材废物已经密封并转移到了相应的废物暂存区域。 - The operation area/equipment surface has been cleaned/操作区域以及设备表面已经清洁。 Checked by & Date/核实人及日期： |  |  |  |  |  |
| CAUTION/注意：DMF/N，N-二甲基甲酰胺：Irritation of eyes, skin and nose. May cause nausea. /刺激眼睛、皮肤和鼻子。可能引起恶心。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |
| 3.20.6 Rinse activation tank /激活罐淋洗 Rinse the port of Activation tank (TJ4S-1213-Sy15-R01) with 91.01 kg（Acceptable weighing range：88.28 kg ~ 93.74 kg）（2 V）DMF. Stir for at least 15 min(Target: 20 min). 用91.01 kg（可接受称量范围：88.28 kg ~ 93.74 kg）（2 V）N,N-二甲基甲酰胺淋洗激活罐（TJ4S-1213-Sy15-R01）加料口。搅拌至少15 min（目标时间：20 min）。 |  |  |  |  | Times/次数 |  |
|  |  |  |  |  | 1 |  |
| Rinse with DMF/ N,N-二甲基甲酰胺淋洗 | Performed by & Date/执行人及日期： |  |  |  |  |  |
| Rinse with DMF/ N,N-二甲基甲酰胺淋洗 | Begin/开始： | End/结束： |  | Qty/重量： |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |
| Stir /搅拌 | Performed by & Date/执行人及日期： |  |  |  |  |  |
| Stir /搅拌 | Begin/开始： |  | End/结束： |  |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |
| CAUTION/注意：DIC is a key EHS concern chemical, highly toxic, skin sensitizing, respiratory sensitizing. /N,N’二异丙基碳二亚胺为EHS重点关注化学品，高毒，皮肤致敏，呼吸致敏。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |

| 3.20. （6）Fmoc-L-Ser(tBu)-OH activation/芴甲氧羰基-叔丁基-L-丝氨酸活化 |  |  |
| --- | --- | --- |
| 3.20.7 DIC addition /加N,N'-二异丙基碳二亚胺 Add DIC into the activation tank (TJ4S-1213-Sy15-R01) at reference time of 10-30 min at 20±5 ℃ (target: 20 ℃). The addition rate depends on temperature control. 控温20±5 ℃ (目标20 ℃)，向激活罐（TJ4S-1213-Sy15-R01）中滴加 kg N,N'-二异丙基碳二亚胺（Acceptable weighing range/可接受称量范围： kg ~ kg）（滴加参考时间10-30 min，以控温为主）。 Pump number/泵编号： Scale ID. No./秤编号： Material charging pipe number/物料管路编号： | Times/次数 |
|  | 1 |
| ★Note:The DIC addition temperature is a CPP./ 备注：N,N'-二异丙基碳二亚胺加料温度是关键控制参数。 Prior to DIC addition, the operator shall confirm that the reaction mixture temperature is within the Normal Operating Range (NOR) / N,N'-二异丙基碳二亚胺投料前，操作人员须执行反应体系温度处于正常操作范围（NOR）内: Checked by & Date/核实人及日期： ★Lower limit/低限: 15oC ，High limit/高限: 25oC Over-temperature warning value/超温预警设置数值： Lower limit/低限: 17oC ，High limit/高限: 23oC |  |
| Before material charging, confirm the material is within the retest date/ the expiry date, Confirm whether the weight is adequate for use in the current charging. /加物料前，执行物料在复检日期/有效期之内，确认重量是否满足本次投料使用。 Checked by & Date/核实人及日期： |  |
| Before drummed liquid material charging, visual check there is no other extraneous matter in material. /加桶装液体物料前，应目视执行桶内无异物。 Visual Examination of Foreign Matters for Drum Solvent/桶装溶剂异物目视检查: 1. Place the solvent drum in a well-lit area (or use a portable light source) to ensure adequate visibility of the liquid contents./ 将溶剂桶置于光线充足区域（或使用便携式光源），确保液体内容物可见性良好 2. Visually inspect the solvent through the drum bung opening or transparent sight glass for any visible foreign matters, including but not limited to particulates, fibers, sediments, or unusual turbidity./ 通过桶口或透明视镜目视检查溶剂中是否有可见异物，包括但不限于颗粒、纤维、沉淀物或异常浑浊。 3. Confirm that the solvent is clear and free of visible foreign matters before charging. If any foreign matter is observed, do not use the drum and follow the applicable handling procedure for subsequent disposition./ 投料前确认溶剂澄清且无可见异物。如发现任何异物，不得使用该桶，并依据处理流程进行后续处置。 Checked by & Date/核实人及日期： |  |

| 3.20. （6）Fmoc-L-Ser(tBu)-OH activation/芴甲氧羰基-叔丁基-L-丝氨酸活化 |  |  |
| --- | --- | --- |
| Confirm liquid charging pipeline is released. /确认加料管路经过放行。 |  |  |
| Performed by & Date/执行人及日期： |  |  |
| Charging of drummed solvent: verify that the pipeline connections remain consistent before and after material feeding, then record the weighing reading 桶装溶剂溶液称量时，确认投料前后管路连接一致，读取称量示数。 Performed by & Date/执行人及日期： Checked by & Date/核实人及日期： |  |  |
| DIC /N,N’-二异丙基碳二亚胺：Control Number/内部控制号：YL003564 |  |  |
| Internal temperature/内部温度 | High values/最高值： | Low values/最低值： |
| Transcribed by & Date /转录人及日期： |  |  |
| If the target net weight≥the planned net weight, perform full drum charge; otherwise, perform partial (Non-full drum) charge. /当目标加料量≥计划加入净重，勾选进行整桶加料；否则勾选进行零头加料。 Note: Target net weight to be charged = Total target net weight – Total net weight charged. Expected tare weight of drum after charge = Gross weight of drum that to be charged - Target net weight to be charged. 还需加入净重目标值= 加料步骤目标总净重-已加入净重总量 加料完成后预计桶皮重=待加料桶毛重-还需加入净重目标值 |  |  |

| 3.20. （6）Fmoc-L-Ser(tBu)-OH activation/芴甲氧羰基-叔丁基-L-丝氨酸活化 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Addition type加料式 |  | Lot number/批号 | Pre-charging qty calculation/投料前投料量计算 |  |  |  |  |  |  | Actual Charging Qty/实际投料量 |  |  |  |  |
|  |  |  | Total qty charged/已加入净重总量 |  | Target qty to be charged/还需加入净重目标值 | Gross qty of drum that to be charged/待加料桶毛重 | Expected net weight of drum after charge /加料完成后预计桶净重 | Expected tare weight of drum after charge/加料完成后预计桶皮重 |  | Before charging/ 投料前 | After charging/ 投料后 | Net weight/ 净重 | Performed by & Date/ 执行人及日期 | Checked by & Date/ 核实人及日期 |
| Full drum整桶 Non-full drum /非整桶 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Full drum整桶 Non-full drum /非整桶 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Total weight /合计重量 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DIC Addition/加N,N'-二异丙基碳二亚胺 |  |  |  | Begin/开始： |  |  |  |  | End/结束： |  |  |  |  |  |
| Transcribed by & Date/转录人及日期： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| 3.20. （6）Fmoc-L-Ser(tBu)-OH activation/芴甲氧羰基-叔丁基-L-丝氨酸活化 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3.20.8 Fmoc-L-Ser(tBu)-OH activation/ 芴甲氧羰基-叔丁基-L-丝氨酸活化 At 20±5 ℃ (target: 20 ℃). Continue agitation of activation tank to 95±10 rpm (target: 95 rpm). Maintain for 20±5 min (target: 20 min) after addition, then stop stirring. /滴毕，控温20±5℃(目标温度：20 ℃)，维持激活罐搅拌转速至95±10 rpm (目标转速: 95 rpm),继续搅拌20±5 min（目标时间：20 min）, 停止搅拌。 |  |  |  |  |  |  |  |  | Times/次数 |  |  |
|  |  |  |  |  |  |  |  |  | 1 |  |  |
| Stir /搅拌 |  |  | Performed by & Date/执行人及日期： |  |  |  |  |  |  |  |  |
| Stir /搅拌 |  |  | End/结束 |  |  |  |  |  |  |  |  |
|  |  |  | The start time is documented in Ins3.20.1 /开始时间记录于3.20.1 |  |  |  |  |  |  |  |  |
| Transcribed by & Date/转录人及日期： |  |  |  |  |  |  |  |  |  |  |  |
| 3.20.8 Internal temperature/内部温度 |  |  | High values/最高值： |  |  | Low values/最低值： |  |  |  |  |  |
| Transcribed by & Date/转录人及日期： |  |  |  |  |  |  |  |  |  |  |  |
| - （6）Fmoc-L-Ser(tBu)-OH coupling/芴甲氧羰基-叔丁基-L-丝氨酸偶联 |  |  |  |  |  |  |  |  |  |  |  |
| 3.21.1 Activation mixture transfer /活化体系转移 Transfer the mixture in activation tank (TJ4S-1213-Sy15-R01) into synthesis reactor (TJ4S-1213-Sy15). 将激活罐（TJ4S-1213-Sy15-R01）中体系转移至合成釜（TJ4S-1213-Sy15）中。 |  |  |  |  |  |  |  | Times/次数 |  |  |  |
|  |  |  |  |  |  |  |  | 1 |  |  |  |
| ★Note:The coupling temperature is a CPP, and the coupling IPC hold temperature is a CPP./ 备注：偶联温度是CPP，偶联IPC维持温度是关键工艺参数。 The operator shall confirm that the reaction mixture temperature is within the Normal Operating Range (NOR) 【25±5oC (target: 25oC)】 /操作人员须执行反应体系温度处于正常操作范围（NOR）内【25±5oC (target: 25oC)】 Checked by & Date/核实人及日期： Over-temperature warning value/超温预警设置数值： Lower limit/低限: 21 oC ，High limit/高限: 28 oC |  |  |  |  |  |  |  |  |  |  |  |
| Transfer/转移 |  |  | Performed by & Date/执行人及日期： |  |  |  |  |  |  |  |  |
|  |  |  | Checked by & Date/核实人及日期： |  |  |  |  |  |  |  |  |
|  |  |  | Begin/开始： |  |  | End/结束： |  |  |  |  |  |
| CAUTION/注意：DMF/N，N-二甲基甲酰胺：Irritation of eyes, skin and nose. May cause nausea. /刺激眼睛、皮肤和鼻子。可能引起恶心。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |  |  |  |  |  |  |
| 3.21.2 DMF addition /加N,N-二甲基甲酰胺 Add 45.50 kg (Acceptable weighing range: 43.50 ~ 47.50 kg) (1 V) DMF by spray balls into activation tank (TJ4S-1213-Sy15-R01) to rinse the wall. 通过淋洗球向激活罐（TJ4S-1213-Sy15-R01）中加入45.50 kg（可接受称量范围：43.50 ~ 47.50 kg） (1 V) N,N-二甲基甲酰胺淋洗罐壁。 |  |  |  |  |  |  |  |  |  | Times/次数 |  |  |
|  |  |  |  |  |  |  |  |  |  | 1 |  |  |
| Rinse with DMF/ N,N-二甲基甲酰胺淋洗 |  | Performed by & Date/执行人及日期： |  |  |  |  |  |  |  |  |  |  |
|  |  | Begin/开始： |  |  | End/结束： |  |  | Qty/重量： |  |  |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |  |  |  |  |  |

| 3.21. （6）Fmoc-L-Ser(tBu)-OH coupling/芴甲氧羰基-叔丁基-L-丝氨酸偶联 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 3.21.3 Transfer /转移 Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h). Bubble with nitrogen for 5±2 min (target: 5 min), then start the agitator. Transfer the mixture in activation tank (TJ4S-1213-Sy15-R01) into synthesis reactor (TJ4S-1213-Sy15). 合成釜（TJ4S-1213-Sy15）开启下口氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min (目标：5 min)后，开启搅拌转速至20±10 rpm (目标转速: 20 rpm)，将激活罐（TJ4S-1213-Sy15-R01）中体系转移至合成釜（TJ4S-1213-Sy15）中。 |  |  |  |  | Times/次数 |  |
|  |  |  |  |  | 1 |  |
| Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h). Bubble with nitrogen for 5±2 min (target: 5min), Start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm). Stir for 10±5 min (target: 10 min)./ 开启合成釜下口鼓氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min（目标时间：5 min）后开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm)， |  |  |  |  |  |  |  |
| Bubble/鼓氮气 |  | Performed by & Date/执行人及日期： |  |  |  |  |  |
| Bubble/鼓氮气 |  | Begin/开始： |  |  |  |  |  |
|  |  | The end time is documented in Ins3.21.5 /结束时间记录于3.21.5 |  |  |  |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |
| Stir/搅拌 |  | Performed by & Date/执行人及日期： |  |  |  |  |  |
| Stir /搅拌 |  | Begin/开始： |  |  |  |  |  |
|  |  | The end time is documented in Ins3.21.5 /结束时间记录于3.21.5 |  |  |  |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |
| Transfer the mixture(DMF) from the activation tank (TJ4S-1213-Sy15-R01) to the synthesis reactor (TJ4S-1213-Sy15). /激活罐(TJ4S-1213-Sy15-R01)中体系转移至合成釜(TJ4S-1213-Sy15)中。 |  |  |  |  |  |  |  |
| Transfer/转移 |  |  | Performed by & Date/执行人及日期： |  |  |  |  |
|  |  |  | Checked by & Date/核实人及日期： |  |  |  |  |
|  |  |  | Begin/开始： | End/结束： |  |  |  |
| 3.21.4 Adjust synthesis reactor temperature /合成釜调温 Adjust the mixture in synthesis reactor (TJ4S-1213-Sy15) to【 25±5 oC (target: 25 oC)】. 合成釜(TJ4S-1213-Sy15)体系调温至【25±5 ℃(目标温度：25 ℃)】。 |  |  |  |  |  | Times/次数 |  |
|  |  |  |  |  |  | 1 |  |

| 3.21. （6）Fmoc-L-Ser(tBu)-OH coupling/芴甲氧羰基-叔丁基-L-丝氨酸偶联 |  |  |  |
| --- | --- | --- | --- |
| The total reaction Fmoc-L-Ser(tBu)-OH coupling time：NMT 840 min (Sourced from the holding time study in the PAR study report.)/ 芴甲氧羰基-叔丁基-L-丝氨酸偶联总的反应时间不超过840min(来源于 PAR 研究报告中稳定时间研究)。 Over-temperature warning value/超温预警设置数值： Lower limit/低限: 21 oC ，High limit/高限: 28 oC |  |  |  |
| Adjust/调温 | Performed by & Date/执行人及日期： |  |  |
| Adjust/调温 | Begin/开始： | End/结束： |  |
| Transcribed by & Date /转录人及日期： |  |  |  |
| 3.21.5 Fmoc-L-Ser(tBu)-OH coupling/ 芴甲氧羰基-叔丁基-L-丝氨酸偶联 The mixture is allowed to react at 25±5 ℃ (target: 25 ℃). After stirring for 3 h±15 min (target: 3 h), then sample the mixture every 0.5~1 h for ninhydrin test analysis until two consecutive samples test as negative (Yellow to Light pink Report Negative= Complete coupling; Blue to Purple Positive= Incomplete coupling). And the mixture should be stirred for coupling time (4 h≤Coupling Time≤10 h). Stop stirring, turn off the nitrogen. Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point. 控温25±5 ℃(目标：25 ℃)反应，反应3 h±15 min（目标：3 h）后，然后取样进行茚三酮检测，每隔0.5~1.0 h取样一次，直至连续两次取样结果为阴性（黄色至浅粉色，报告阴性=偶联完全；蓝色至紫色，阳性=偶联不完全）后，且反应时间满足（4h≤反应时间≤10h）后，停止搅拌，关闭氮气。通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 Note: If resin adherence is found at other times during the reaction, rinse the reactor with 0.50~5.00 kg DMF each time. /备注：偶联过程中其它时间发现贴壁，每次使用0.50~5.00 kg N,N-二甲基甲酰胺淋洗，记录见附加页 |  |  | Times/次数 |
|  |  |  | 1 |

| 3.21. （6）Fmoc-L-Ser(tBu)-OH coupling/芴甲氧羰基-叔丁基-L-丝氨酸偶联 |  |  |
| --- | --- | --- |
| ★After coupling is completed, the operator shall confirm that all Critical Process Parameters (CPP) are within their respective Normal Operating Ranges (NOR)/ 偶联结束后，操作人员须确认所有关键工艺参数（CPP）均处于各自的正常操作范围（NOR）内 ★Note: The coupling temperature is a CPP, and the coupling IPC hold temperature is a CPP./备注：偶联温度是CPP，偶联IPC维持温度是关键工艺参数。 The operator shall confirm that the reaction mixture temperature is within the Normal Operating Range (NOR) :20~30℃ /操作人员须确认反应体系温度处于正常操作范围（NOR）内:20~30℃ □Yes/是 □No/否 Confirmed by & Date/确认人及日期： Checked by & Date/核实人及日期： |  |  |
| React/反应 | Begin/开始： | End/结束： |
| Transcribed by & Date /转录人及日期： |  |  |

| 3.21. （6）Fmoc-L-Ser(tBu)-OH coupling/芴甲氧羰基-叔丁基-L-丝氨酸偶联 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Take 5-10 ml mixture and perform testing in accordance with the Kaiser test record. During sampling, should wear nitrile gloves and 3M 6000 series Full-face piece respirator, 6006CN cartridges. 通过在线取样器取样5~10ml按照茚三酮检测记录进行检测。取样过程中，应该戴丁腈手套、3M 6000系列全面罩, 6006CN滤毒盒。 |  |  |  |  |  |  |  |  |  |  |
| Sampling number/ 取样编号 |  | Sampling time/取样时间 |  |  | Sampled by & Date/取样人及日期 | Negative or Positive/阳性或阴性 |  |  | Result filled by &Date/结果填写人及日期 | Checked by & Date/核实人及日期 |
| CPo140020- -01- -3.21.5-01 |  |  |  |  |  | Negative /阴性 Positive /阳性 |  |  |  |  |
| CPo140020- -01- -3.21.5-02 |  |  |  |  |  | Negative /阴性 Positive /阳性 |  |  |  |  |
| CPo140020- -01- -3.21.5-03 |  |  |  |  |  | Negative /阴性 Positive /阳性 |  |  |  |  |
| CPo140020- -01- -3.21.5-04 |  |  |  |  |  | Negative /阴性 Positive /阳性 |  |  |  |  |
| CPo140020- -01- -3.21.5-05 |  |  |  |  |  | Negative /阴性 Positive /阳性 |  |  |  |  |
| CPo140020- -01- -3.21.5-06 |  |  |  |  |  | Negative /阴性 Positive /阳性 |  |  |  |  |
| CPo140020- -01- -3.21.5-07 |  |  |  |  |  | Negative /阴性 Positive /阳性 |  |  |  |  |
| CPo140020- -01- -3.21.5-08 |  |  |  |  |  | Negative /阴性 Positive /阳性 |  |  |  |  |
| Confirm the end point of the reaction and move to following operation. /执行反应终点后进行下一步操作。 The acceptance criterion is two consecutive negative test results./连续两次检测结果为阴性即为合格。 Confirmed by & Date/确认人及日期： Checked by & Date/核实人及日期： |  |  |  |  |  |  |  |  |  |  |
| Stir/搅拌 |  |  |  |  | Performed by & Date/执行人及日期： |  |  |  |  |  |  |
| Stir /搅拌 |  |  |  |  | End/结束： |  |  |  |  |  |  |
|  |  |  |  |  | The start time is documented in Ins 3.21.3 /开始时间记录于3.21.3 |  |  |  |  |  |  |
| Transcribed by & Date/转录人及日期： |  |  |  |  |  |  |  |  |  |  |  |
| Bubble/鼓氮气 |  |  |  |  | Performed by & Date/执行人及日期： |  |  |  |  |  |  |
| Bubble/鼓氮气 |  |  |  |  | End/结束： |  |  |  |  |  |  |
|  |  |  |  |  | The start time is documented in Ins 3.21.3 /开始时间记录于3.21.3 |  |  |  |  |  |  |
| Transcribed by & Date/转录人及日期： |  |  |  |  |  |  |  |  |  |  |  |
| Internal temperature/内部温度 |  |  | High values/最高值： |  |  |  | Low values/最低值： |  |  |  |  |
| Transcribed by & Date/转录人及日期： |  |  |  |  |  |  |  |  |  |  |  |
| Filter/过滤 |  |  | Performed by & Date/执行人及日期 |  |  |  |  |  |  |  |
|  |  |  | Checked by & Date/核实人及日期： |  |  |  |  |  |  |  |
|  |  |  | Begin/开始： |  |  |  | End/结束： |  |  |  |
| ★After coupling is completed, the operator shall confirm that all Critical Process Parameters (CPPs) are within their respective Normal Operating Ranges (NOR)/ 偶联结束后，操作人员须确认所有关键工艺参数（CPP）均处于各自的正常操作范围（NOR）内。 Checked by & Date/核实人及日期： |  |  |  |  |  |  |  |  |  |  |

| 3.21. （6）Fmoc-L-Ser(tBu)-OH coupling/芴甲氧羰基-叔丁基-L-丝氨酸偶联 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| CAUTION/注意：DMF/N，N-二甲基甲酰胺：Irritation of eyes, skin and nose. May cause nausea. /刺激眼睛、皮肤和鼻子。可能引起恶心。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |
| 3.21.6 Activation Tank Rinse/激活罐洗涤 Add 364.03 kg (Acceptable weighing range：353.11 ~ 374.95 kg) (8 V) DMF into the activation tank (TJ4S-1213-Sy15-R01). Circulate and clean the activating liquid transfer pipeline, liquid charging port cleaning pipeline and solid charging port cleaning pipeline. Clean every line wash for 2±1 min (target: 2 min). 向激活罐（TJ4S-1213-Sy15-R01）中加入364.03 kg（可接受称量范围：353.11 ~ 374.95 kg） (8 V) N,N-二甲基甲酰胺，循环清洗激活罐转移管路，液体加料口清洗管路和固体加料口清洗管路，每条线路各清洗2±1 min（目标：2 min）。 |  |  |  |  | Times/次数 |
|  |  |  |  |  | 3 |
| Operation name/ 操作名称 | Time point/时间节点 | Clean the activating liquid transfer pipeline 1/循环清洗激活液转移管路1 | Liquid charging port cleaning pipeline 2 /液体加料口清洗管路2 | Solid charging port cleaning pipeline 3,/固体加料口清洗管路3 |  |
|  |  | Wash /洗涤1 | Wash /洗涤2 | Wash /洗涤3 |  |
| DMF addition/ 加N,N-二甲基甲酰胺 | Performed by & Date/执行人及日期： |  |  |  |  |
| DMF addition/ 加N,N-二甲基甲酰胺 | Begin/开始 |  |  |  |  |
|  | End/结束 |  |  |  |  |
|  | Qty/重量 |  |  |  |  |
|  | Transcribed by & Date /转录人及日期： |  |  |  |  |
| Circulate and clean/循环清洗 | Performed by & Date/执行人及日期： |  |  |  |  |

| 3.21. （6）Fmoc-L-Ser(tBu)-OH coupling/芴甲氧羰基-叔丁基-L-丝氨酸偶联 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operation name/ 操作名称 |  | Time point/时间节点 |  | Clean the activating liquid transfer pipeline 1/循环清洗激活液转移管路1 | liquid charging port cleaning pipeline 2 /液体加料口清洗管路2 | solid charging port cleaning pipeline 3,/固体加料口清洗管路3 |  |  |
|  |  |  |  | Wash /洗涤1 | Wash /洗涤2 | Wash /洗涤3 |  |  |
| Circulate and clean/循环清洗 |  | Begin/开始 |  |  |  |  |  |  |
|  |  | End/结束 |  |  |  |  |  |  |
|  |  | Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |
| 3.21.7 Post-coupling wash./偶联后洗涤 Transfer the mixture in activation tank (TJ4S-1213-Sy15-R01) to synthesis reactor (TJ4S-1213-Sy15). Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h).Bubble with nitrogen for 5±2 min (target: 5 min), start the agitator. After stirring for 7±2 min (target：7 min), stop stirring, turn off the nitrogen. Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point. 将激活罐（TJ4S-1213-Sy15-R01）洗涤液转入合成釜 (TJ4S-1213-Sy15)。开启合成釜下口鼓氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min (目标时间：5 min)后开启搅拌转速至20±10 rpm (目标转速: 20 rpm)，搅拌7±2 min（目标时间：7 min）后停止搅拌，关闭氮气。通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  |  |  |  |  | Times/次数 |
|  |  |  |  |  |  |  | 3 |
| Transfer the liquid form activation tank (TJ4S-1213-Sy15-R01) to the synthesis reactor (TJ4S-1213-Sy15) ./将激活罐(TJ4S-1213-Sy15-R01)中体系转移至合成釜(TJ4S-1213-Sy15)中。 |  |  |  |  |  |  |  |  |
| Operation name/ 操作名称 |  | Time point/时间节点 | Wash /洗涤1 | Wash /洗涤2 | Wash /洗涤3 |  |  |  |
| Transfer/转移 |  | Performed by & Date/执行人及日期： |  |  |  |  |  |  |
|  |  | Checked by & Date/核实人及日期： |  |  |  |  |  |  |
|  |  | Begin/开始 |  |  |  |  |  |  |
|  |  | End/结束 |  |  |  |  |  |  |

| 3.21. （6）Fmoc-L-Ser(tBu)-OH coupling/芴甲氧羰基-叔丁基-L-丝氨酸偶联 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h). Bubble with nitrogen for 5±2 min (target 5 min), start the agitator. After stirring for 7±2 min (target：7 min), stop stirring, turn off the nitrogen.开启合成釜下口鼓氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min (目标：5 min)后开启搅拌转速至20±10 rpm (目标转速: 20 rpm)，搅拌7±2 min（目标：7 min）后停止搅拌，关闭氮气。 |  |  |  |  |
| Operation name/ 操作名称 | Time point/时间节点 | Wash /洗涤1 | Wash /洗涤2 | Wash /洗涤3 |
| Bubble/鼓氮气 | Performed by & Date/执行人及日期： |  |  |  |
| Bubble/鼓氮气 | Begin/开始 |  |  |  |
|  | End/结束 |  |  |  |
|  | Transcribed by & Date /转录人及日期： |  |  |  |
| Stir /搅拌 | Performed by & Date/执行人及日期： |  |  |  |
|  | Begin/开始 |  |  |  |
|  | End/结束 |  |  |  |
|  | Transcribed by & Date /转录人及日期 |  |  |  |
| Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point/通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  |  |  |
| Operation name/ 操作名称 | Time point/时间节点 | Wash /洗涤1 | Wash /洗涤2 | Wash /洗涤3 |
| Filter/过滤 | Performed by & Date/执行人及日期： |  |  |  |
|  | Checked by & Date/核实人及日期： |  |  |  |
|  | Begin/开始 |  |  |  |
|  | End/结束 |  |  |  |

| - （7）Deprotection/脱保护 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CAUTION/注意：Piperidine哌啶：Strong local irritant and may cause permanent injury after short exposure to small amounts. /强烈的局部刺激，短时间少量接触可能造成永久性伤害。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |  |  |
| 3.22.1 Piperidine soulution addition /加哌啶溶液 Adjust the jacket temperature of synthesis reactor (TJ4S-1213-Sy15) to 【25±5 oC (target: 25 oC). 】Add 357.50 kg 【20% (V/V) piperidine/DMF(8 V)】 (Acceptable weighing range: 346.77 ~ 368.23 kg) solution into the Synthesis reactor. 将合成釜(TJ4S-1213-Sy15)夹套调温至【25±5 ℃（目标温度：25 ℃）】。向合成釜中加入357.50 kg【20%哌啶/DMF (V/V)溶液(8 V)】（可接受称量范围：346.77 ~ 368.23 kg）。 |  |  |  |  |  |  | Times/次数 |
|  |  |  |  |  |  |  | 1 |
| Adjust/调温 |  |  | Performed by & Date/执行人及日期： |  |  |  |  |
| Adjust/调温 |  |  | Begin/开始： |  | Adjust/调温 |  |  |
| Transcribed by & Date/转录人及日期： |  |  |  |  |  |  |  |
| Confirm and record that the 20%Piperidine solution concentration is within the NOR range./ 确认并记录20%哌啶溶液浓度在正常操作范围内：17.50%~22.50% 20%Piperidine solution concentration confirmation/20%哌啶溶液浓度确认：____________ Confirm and record that the 20%Piperidine solution temperature is within the NOR range./ 确认并记录20%哌啶溶液温度在正常操作范围内：20 oC ~30 oC 20%Piperidine solution temperature confirmation/20%哌啶溶液温度确认：____________ Confirmed by & Date/确认人及日期： Checked by & Date/核实人及日期： |  |  |  |  |  |  |  |
| Piperidine addition/ 加哌啶溶液 |  | Performed by & Date/执行人及日期： |  |  |  |  |  |
| Piperidine addition/ 加哌啶溶液 |  | Begin/开始： |  | End/结束： |  | Qty/重量： |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |

| 3.22. （7）Ddeprotection/脱保护 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 3.22.2 Adjust temperature /调温 Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h) for 5±2 min (target 5 min), Start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm). Stir for 10±5 min (target: 10 min). Adjust to 【25±5 ℃ (target: 25 ℃)】. 开启合成釜下口鼓氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min (目标时间：5 min)后开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm)，调温至【25±5 ℃(目标温度：25 ℃)】。 |  |  |  | Times/次数 |  |
|  |  |  |  | 1 |  |
| Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h). Bubble with nitrogen for 5±2 min (target :5 min) 开启合成釜下口鼓氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min (目标时间：5 min) |  |  |  |  |  |
| Bubble/鼓氮气 |  |  | Performed by & Date/执行人及日期： |  |  |
| Bubble/鼓氮气 |  |  | Begin/开始： |  |  |
|  |  |  | The end time is documented in Ins 3.22.3/结束时间记录于3.22.3 |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |
| Start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm). 开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm) |  |  |  |  |  |
| Stir /搅拌 |  | Performed by & Date/执行人及日期： |  |  |  |
| Stir /搅拌 |  | Begin/开始： |  |  |  |
|  |  | The end time is documented in Ins 3.22.3/结束时间记录于3.22.3 |  |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |
| Adjust to 25±5 oC (target: 25 oC)./ 调温至25±5 ℃(目标温度：25 oC) |  |  |  |  |  |
| Adjust/调温 |  |  | Performed by & Date/执行人及日期： |  |  |
| Adjust/调温 |  |  | Begin/开始： | End/结束： |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |

| 3.22. （7）Ddeprotection/脱保护 |  |  |  |  |
| --- | --- | --- | --- | --- |
| 3.22.3 Fmoc-L-Arg(Pbf)-OH De-Fmoc /Fmoc-Pbf-精氨酸脱保护 After stirring for【2 h±15 min (target: 2 h) at 25±5 ℃ (target: 25 ℃)】, stop stirring, turn off the nitrogen. Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point. 控温【25±5 ℃(目标：25 ℃)，搅拌2 h±15 min（目标：2 h）】后，停止搅拌，关闭氮气。通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 Over-temperature warning value/超温预警设置数值： Lower limit/低限: 22 oC ，High limit/高限: 28 oC |  |  | Times/次数 |  |
|  |  |  | 1 |  |
| ★Confirm and record that the 20%Piperidine solution temperature is within the NOR range 20 oC ~30 oC ./ 确认并记录20%哌啶溶液温度在正常操作范围内：20 oC ~30 oC。 20%Piperidine solution temperature confirmation/20%哌啶溶液温度确认：____________ Confirmed by& Date/确认人及日期： Checked by & Date/核实人及日期： |  |  |  |  |
| De-Fmoc/脱Fmoc &Stir /搅拌 |  |  | Performed by & Date/执行人及日期： |  |
| De-Fmoc/脱Fmoc &Stir /搅拌 |  |  | End/结束： |  |
|  |  |  | The start time is documented in Ins3.22.2 /开始时间记录于3.22.2 |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |

| 3.22. （7）Ddeprotection/脱保护 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Bubble/鼓氮气 |  |  |  | Performed by & Date/执行人及日期： |  |  |  |
| Bubble/鼓氮气 |  |  |  | End/结束： |  |  |  |
|  |  |  |  | The start time is documented in Ins3.22.2 /开始时间记录于3.22.2 |  |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |
| Internal temperature/内部温度 |  | High values/最高值: |  | Low values/最低值： |  |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |
| Filter/过滤 |  | Performed by & Date/执行人及日期： |  |  |  |  |  |
|  |  | Checked by & Date/核实人及日期： |  |  |  |  |  |
|  |  | Begin/开始： |  | End/结束： |  |  |  |
| ★After De Fmoc is completed, the operator shall confirm that all Critical Process Parameters (CPPs) are within their respective Normal Operating Ranges (NOR)/ 脱保护结束后，操作人员须确认所有关键工艺参数（CPP）均处于各自的正常操作范围（NOR）内 Checked by & Date/核实人及日期： |  |  |  |  |  |  |  |
| 3.22.4 Adjust the synthesis reactor (TJ4S-1213-Sy15) jacket to【20±5 ℃ (target: 20 ℃)】. /将合成釜（TJ4S-1213-Sy15）夹套调温至【20±5 ℃（目标温度：20 ℃）】。 |  |  |  |  | Times/次数 |  |  |
|  |  |  |  |  | 1 |  |  |
| Adjust/调温 | Performed by & Date/执行人及日期： |  |  |  |  |  |  |
| Adjust/调温 | Begin/开始： |  |  | End/结束： |  |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |

| 3.22. （7）Ddeprotection/脱保护 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CAUTION/注意：IPAC/乙酸异丙酯： Vapors irritate eyes and respiratory tract; high concentrations can be anesthetic. Liquid irritates eyes but causes no serious injury; may cause dermatitis; no serious effects if swallowed. /蒸汽会刺激眼睛和呼吸道；高浓度的蒸汽可能是麻醉剂。液体刺激眼睛，但不会造成严重伤害；可能会引起皮炎；吞食不会造成严重影响。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |  |  |  |  |  |
| 3.22.5 Post-deprotection wash/脱保护后洗涤 Add 445.92 kg (Acceptable weighing range：432.54 ~ 459.30 kg）（9.29 kg/kg, 10.64 V) isopropyl acetate into synthesis reactor (TJ4S-1213-Sy15). Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h). Bubble with nitrogen for 5±2 min (target 5 min), then start the agitator to 20±10 rpm (target: 20 rpm). After stirring for 5±2 min (target 5 min), stop stirring, turn off the nitrogen. Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point. 向合成釜（TJ4S-1213-Sy15）中加入445.92 kg（可接受称量范围：432.54 ~ 459.30 kg）（9.29 kg/kg, 10.64 V）乙酸异丙酯，开启下口氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min (目标：5 min)后开启搅拌转速至20±10 rpm (目标转速: 20 rpm)，搅拌5±2 min (目标：5 min)后，停止搅拌，关闭氮气。通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  |  |  |  |  | Times/次数 |
|  |  |  |  |  |  |  | 4 |
| ★Note: IPAC quantity is a CPP./备注：乙酸异丙酯用量是关键工艺参数。 ★The operator shall confirm that IPAC quantity is within the Normal Operating Range (NOR) 【432.54 kg~459.30 kg】/操作人员须确认乙酸异丙酯用量处于正常操作范围（NOR）内：【432.54 kg~459.30 kg】 Checked by & Date/核实人及日期： |  |  |  |  |  |  |  |
| Operation name/操作名称 |  | Time point/时间节点 | Wash /洗涤1 | Wash /洗涤2 | Wash /洗涤3 | Wash /洗涤4 |  |
| Add 419.04 kg IPAc (10.0 V) by spray balls into the synthesis reactor (TJ4S-1213-Sy15)./ 向合成釜(TJ4S-1213-Sy15) 中加419.04 kg乙酸异丙酯（（10.0 V）（Acceptable weighing range/可接受称量范围：440.34~467.58kg）） |  |  |  |  |  |  |  |
| IPAC addition/加乙酸异丙酯 |  | Performed by & Date/执行人及日期： |  |  |  |  |  |
| IPAC addition/加乙酸异丙酯 |  | Begin/开始 |  |  |  |  |  |
|  |  | End/结束 |  |  |  |  |  |
|  |  | Qty/重量 |  |  |  |  |  |
|  |  | Transcribed by & Date /转录人及日期： |  |  |  |  |  |

| 3.22. （7）Deprotection/脱保护 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h). Bubble with nitrogen for 5±2 min (target: 5 min). turn off the nitrogen. Start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm). Stir for 5±2 min (target: 5 min). After stirring for 5±2 min (target: 5 min), stop stirring, turn off the nitrogen./ 开启合成釜下口鼓氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min（目标时间：5 min）后开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm)，搅拌5±2 min（目标时间：10 min）后，停止搅拌，关闭氮气。 |  |  |  |  |  |  |  |  |  |  |  |  |
| Bubble/鼓氮气 |  | Performed by & Date/执行人及日期： |  |  |  |  |  |  |  |  |  |  |
| Bubble/鼓氮气 |  | Begin/开始 |  |  |  |  |  |  |  |  |  |  |
|  |  | End/结束 |  |  |  |  |  |  |  |  |  |  |
|  |  | Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |  |  |  |
| Stir /搅拌 |  | Performed by & Date/执行人及日期： |  |  |  |  |  |  |  |  |
| Stir /搅拌 |  | Begin/开始 |  |  |  |  |  |  |  |  |
|  |  | End/结束 |  |  |  |  |  |  |  |  |
|  |  | Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |  |
| Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point./通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  |  |  |  |  |  |  |  |  |
| Filter/过滤 |  | Performed by & Date/执行人及日期： |  |  |  |  |  |  |  |  |
|  |  | Checked by & Date/核实人及日期： |  |  |  |  |  |  |  |  |
|  |  | Begin/开始 |  |  |  |  |  |  |  |  |
|  |  | End/结束 |  |  |  |  |  |  |  |  |

| 3.22. （7）Deprotection/脱保护 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| CAUTION/注意：DMF/N，N-二甲基甲酰胺：Irritation of eyes, skin and nose. May cause nausea. /刺激眼睛、皮肤和鼻子。可能引起恶心。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |  |  |  |
| 3.22.6 DMF addition /加N,N-二甲基甲酰胺 Add 546.05 kg (Acceptable weighing range: 529.67 ~ 562.43 kg) (12 V) DMF by spray balls into synthesis reactor (TJ4S-1213-Sy15). 通过淋洗球向合成釜（TJ4S-1213-Sy15）中加入546.05 kg（可接受称量范围：529.67 ~ 562.43 kg）（12 V）N,N-二甲基甲酰胺。 |  |  |  |  |  | Times/次数 |  |  |
|  |  |  |  |  |  | 1 |  |  |
| DMF addition/ 加N,N-二甲基甲酰胺 |  | Performed by & Date/执行人及日期： |  |  |  |  |  |  |
| DMF addition/ 加N,N-二甲基甲酰胺 |  | Begin/开始： | End/结束： | Qty/重量： |  |  |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |  |
| 3.22.7 Resin reswelling wash 1/溶胀后洗涤1 Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h). Bubble with nitrogen for 5±2 min (target: 5 min), then start the agitator to 20±10 rpm (target: 20 rpm). After stirring for 10±2 min (target: 10 min), stop stirring, turn off the nitrogen. Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point. 开启合成釜（TJ4S-1213-Sy15）下口鼓氮气, 氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min后（目标：5 min），开启搅拌转速至20±10 rpm (目标转速: 20 rpm)，搅拌10±2 min (目标：10 min)后，停止搅拌，关闭氮气。通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  |  |  |  | Times/次数 |  |
|  |  |  |  |  |  | 1 |  |

| 3.22. （7）Deprotection/脱保护 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h) for 5±2 min (target: 5min). Start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm). Stir for 10 ±5 min (target: 10 min). After stirring for 10±2 min (target: 10 min), stop stirring, turn off the nitrogen./ 开启合成釜下口鼓氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min（目标时间：5 min）后，开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm)，搅拌10±2 min（目标时间:10 min）后，停止搅拌，关闭氮气。 |  |  |  |  |  |  |  |  |
| Bubble/鼓氮气 |  |  | Performed by & Date/执行人及日期： |  |  |  |  |  |
| Bubble/鼓氮气 |  |  | Begin/开始： |  | End/结束： |  |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |  |
| Stir /搅拌 |  |  | Performed by & Date/执行人及日期： |  |  |  |  |  |
| Stir /搅拌 |  |  | Begin/开始： |  | End/结束： |  |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |  |
| Filter/过滤 |  |  | Performed by & Date/执行人及日期： |  |  |  |  |  |
|  |  |  | Checked by & Date/核实人及日期： |  |  |  |  |  |
|  |  |  | Begin/开始： |  | End/结束： |  |  |  |
| CAUTION/注意：DMF/N，N-二甲基甲酰胺：Irritation of eyes, skin and nose. May cause nausea. /刺激眼睛、皮肤和鼻子。可能引起恶心。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |  |  |  |
| 3.22.8 Resin reswelling wash 2/溶胀后洗涤2 Add 546.05 kg (Acceptable weighing range: 529.67 ~ 562.43 kg) (12 V) DMF by spray balls into synthesis reactor (TJ4S-1213-Sy15). 通过淋洗球向合成釜（TJ4S-1213-Sy15）中加入546.05 kg（可接受称量范围：529.67 ~ 562.43 kg）（12 V）N,N-二甲基甲酰胺。 |  |  |  |  |  |  |  | Times/次数 |
|  |  |  |  |  |  |  |  | 1 |
| DMF addition/ 加N,N-二甲基甲酰胺 |  |  | Performed by & Date/执行人及日期： |  |  |  |  |  |
| DMF addition/ 加N,N-二甲基甲酰胺 |  |  | Begin/开始： |  | End/结束： |  | Qty/重量： |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |  |

| 3.22. （7）Deprotection/脱保护 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 3.22.9 Resin reswelling wash 2/溶胀后洗涤2 Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h) .Bubble with nitrogen for 5±2 min (target: 5 min), then start the agitator to 20±10 rpm (target: 20 rpm). After stirring for 10±2 min (target 10 min), stop stirring, turn off the nitrogen. Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point. 开启合成釜（TJ4S-1213-Sy15）下口鼓氮气, 氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min后（目标时间：5 min），开启搅拌转速至20±10 rpm (目标转速: 20 rpm)，搅拌10±2 min (目标时间：10 min)后，停止搅拌，关闭氮气。通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  |  |  |  | Times/次数 |  |
|  |  |  |  |  |  | 1 |  |
| Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h) for 5±2 min (target: 5 min), Start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm). Stir for 10±5 min (target: 10 min). After stirring for 10±2 min (target: 10 min), stop stirring, turn off the nitrogen./ 开启合成釜下口鼓氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min（目标时间：5 min）后，开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm)，搅拌10±2 min（目标时间:10 min）后，停止搅拌，关闭氮气。 |  |  |  |  |  |  |  |
| Bubble/鼓氮气 |  |  | Performed by & Date/执行人及日期： |  |  |  |  |
| Bubble/鼓氮气 |  |  | Begin/开始： |  | End/结束： |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |
| Stir /搅拌 |  |  | Performed by & Date/执行人及日期： |  |  |  |  |
| Stir /搅拌 |  |  | Begin/开始： |  | End/结束： |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |
| Filter/过滤 |  |  | Performed by & Date/执行人及日期： |  |  |  |  |
|  |  |  | Checked by & Date/核实人及日期： |  |  |  |  |
|  |  |  | Begin/开始： |  | End/结束： |  |  |

| - （7）Fmoc-L-Arg(Pbf)-OH activation/Fmoc-Pbf-精氨酸活化 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| CAUTION/注意：DMF/N，N-二甲基甲酰胺：Irritation of eyes, skin and nose. May cause nausea. /刺激眼睛、皮肤和鼻子。可能引起恶心。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |  |  |
| 3.23.1 Add DMF to the activation reactor/激活罐加N,N-二甲基甲酰胺 Add 227.52 kg (Acceptable weighing range: 220.69 ~ 234.34 kg) (5 V) DMF into activation tank (TJ4S-1213-Sy15-R01). Start the agitator to 95±10 rpm (target: 95 rpm). 向激活罐（TJ4S-1213-Sy15-R01）中加入227.52 kg（可接受称量范围：220.69 ~ 234.34 kg ）（5 V）N，N-二甲基甲酰胺，开启搅拌转速至95±10 rpm (目标转速: 95 rpm)。 |  |  |  |  | Times/次数 |  |  |
|  |  |  |  |  | 1 |  |  |
| (Acceptable weighing range: 220.69 ~ 234.34 kg) (5 V) DMF into activation tank (TJ4S-1213-Sy15-R01). Start the agitator to 95±10 rpm (target: 95 rpm)./向激活罐（TJ4S-1213-Sy15-R01）中加入227.52 kg（可接受称量范围：220.69 ~ 234.34 kg ）（5 V）N，N-二甲基甲酰胺。 |  |  |  |  |  |  |  |
| DMF addition/加N,N-二甲基甲酰胺 | Performed by & Date/执行人及日期： |  |  |  |  |  |  |
| DMF addition/加N,N-二甲基甲酰胺 | Begin/开始： | End/结束： | Qty/重量： |  |  |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |
| Start the agitator of activation tank to 95±10 rpm (target: 95 rpm)./ 开启激活罐搅拌转速至95±10 rpm (目标转速: 95 rpm)。 |  |  |  |  |  |  |  |
| Stir/搅拌 |  | Performed by & Date/执行人及日期： |  |  |  |  |  |
| Stir /搅拌 |  | Begin/开始： |  |  |  |  |  |
|  |  | The end time is documented in Ins 3.23.8/结束时间记录于3.23.8 |  |  |  |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |
| 3.23.2 Fmoc-L-Arg(Pbf)-OH addition /加Fmoc-Pbf-精氨酸 Add Fmoc-L-Arg(Pbf)-OH (2.00 eq.) with powder feeding bags into activation tank (TJ4S-1213-Sy15-R01). 通过投料袋向激活罐（TJ4S-1213-Sy15-R01）中加入 kg Fmoc-Pbf-精氨酸（Acceptable weighing range/可接受称量范围： kg ~ kg）(2.00 eq.)。 |  |  |  | Times/次数 |  |
|  |  |  |  | 1 |  |

| 3.23. （7）Fmoc-L-Arg(Pbf)-OH activation/Fmoc-Pbf-精氨酸活化 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Before material charging, confirm the material is within the retest date/ the expiry date. Confirm whether the weight is adequate for use in the current charging. /加物料前，执行物料在复检日期/有效期之内，确认重量是否满足本次投料使用。 Before solid material charging, confirm/加固体物料前，确认： - Before operation performed, No non-related material / equipment / instrument / tools / pipeline / document / record / label in the intended area /操作前，目标区域无不相关物料/设备/仪器/工具/管路/文件/记录/标签； - The outer packages are removed. If the outer package cannot be removed, butunpack material without damaging the package bag, and before charging, check no package fragment and other extraneous matter in material. /外包装移除；如果外包装不能移除，但在不破坏包装的情况下打开包装袋，投料前以及过程中同时检查执行物料中无包装碎片以及异物 Checked by & Date/核实人及日期： |  |  |  |  |  |
| Prior to charging each amino acid, the operator shall perform a Raman scan to confirm the identity of the material. The Raman scan result shall be compared against the reference spectrum of the corresponding amino acid. Only materials that conform to the reference spectrum may be charged. If the Raman scan result does not conform, stop the operation, quarantine the material, and initiate a investigation per site SOP. Record the Raman scan result, operator signature, and date/time in the table below./ 每次投氨基酸前，操作人员应进行拉曼扫描以执行物料。拉曼扫描结果应与相应氨基酸的参考光谱进行比对。仅与参考光谱相符的物料方可投料。如拉曼扫描结果不相符，应停止操作，对物料进行隔离，并依据厂区 SOP 启动异常事件调查。在下表中记录拉曼扫描结果、操作人员签字及日期 / 时间。 |  |  |  |  |  |
| Charge Fmoc-L-Arg(Pbf)-OH / 加Fmoc-Pbf-精氨酸 |  | Performed by & Date/执行人及日期： |  |  |  |
| Charge Fmoc-L-Arg(Pbf)-OH / 加Fmoc-Pbf-精氨酸 |  | Begin/开始： |  | End/结束： |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |
| Fmoc-L-Arg(Pbf)-OH confirmation/ Fmoc-Pbf-精氨酸执行：Control No./内部控制号YL003424 |  |  |  |  |  |
| Check items/检查项 | Is it correct?/是否正确 |  | Recorded by & Date/ 记录人及日期 |  | Checked by & Date/ 核实人及日期 |
| Material name/物料名称 | Yes/是；No/否 |  |  |  |  |
| Project batch number/ 项目批次号 | Yes/是；No/否 |  |  |  |  |
| Operation number/ 操作号 | Yes/是；No/否 |  |  |  |  |
| Control No./内部控制号 | Yes/是；No/否 |  |  |  |  |
| Raman Scan Confirmation/拉曼扫描执行 | Conforms/相符 Does not conform/不相符 |  |  |  |  |

| 3.23. （7）Fmoc-L-Arg(Pbf)-OH activation/Fmoc-Pbf-精氨酸活化 |  |  |  |  |
| --- | --- | --- | --- | --- |
| No. /编号 | Net weight/净重 | All in/全部加入 | Recorded by & Date/ 记录人及日期 | Checked by & Date/ 核实人及日期 |
| / |  | Yes/是；No/否 |  |  |
| / |  | Yes/是；No/否 |  |  |
| / |  | Yes/是；No/否 |  |  |
| / |  | Yes/是；No/否 |  |  |
| / |  | Yes/是；No/否 |  |  |
| / |  | Yes/是；No/否 |  |  |
| / |  | Yes/是；No/否 |  |  |
| / |  | Yes/是；No/否 |  |  |
| / |  | Yes/是；No/否 |  |  |
| / |  | Yes/是；No/否 |  |  |
| / |  | Yes/是；No/否 |  |  |
| / |  | Yes/是；No/否 |  |  |
| Total/合计 |  | Confirm the total net weight is within the input range/执行合计净重在投料范围内 | Recorded by & Date/记录人及日期 | Checked by & Date/ 核实人及日期 |
|  |  | Yes/是；No/否 |  |  |
| After solid material charging, confirm/固体物料投料之后，确认： - The corresponding leftover material has been properly sealed and transferred to the staging area/相应的剩余物料已经恰当密封，转移到了暂存区域。 - The waste packaging material has been sealed and transferred to the corresponding waste staging area. /产生的包材废物已经密封并转移到了相应的废物暂存区域。 - The operation area/equipment surface has been cleaned/操作区域以及设备表面已经清洁。 Confirmed by & Date/核实人及日期： |  |  |  |  |

| 3.23. （7）Fmoc-L-Arg(Pbf)-OH activation/Fmoc-Pbf-精氨酸活化 |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3.23.3 Agitation of activation tank /激活罐搅拌 Continue agitation of activation tank to 95±10 rpm (target: 95 rpm). Maintain stirring for at least 15 min. Confirm no visible solids and later use. 维持激活罐搅拌转速至95±10 rpm (目标转速: 95 rpm), 搅拌至少15 min，确认无明显固体待用。 |  |  |  |  |  |  |  |  |  | Times/次数 |  |  |
|  |  |  |  |  |  |  |  |  |  | 1 |  |  |
| Stir /搅拌 |  | Performed by & Date/执行人及日期： |  |  |  |  |  |  |  |  |  |  |
| Stir /搅拌 |  | Begin/开始： |  |  |  |  |  |  |  |  |  |  |
|  |  | The end time is documented in Ins 3.23.8/结束时间记录于3.23.8 |  |  |  |  |  |  |  |  |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |  |  |  |  |  |
| Confirm no visible solid/确认无明显固体： |  |  |  | Confirmed by& Date /确认人及日期： |  |  |  |  |  |  |  |  |
| 3.23.4 Adjust activation tan temperature /激活罐调温 Adjust the activation tank (TJ4S-1213-Sy15-R01) mixture to 【20±5 ℃ (target 20 ℃) 】. /将激活罐(TJ4S-1213-Sy15-R01)中体系调温至【20±5 ℃（目标：20 ℃）】。 |  |  |  |  |  |  |  |  |  | Times/次数 |  |  |
|  |  |  |  |  |  |  |  |  |  | 1 |  |  |
| Adjust/调温 |  |  |  |  | Performed by & Date/执行人及日期： |  |  |  |  |  |  |  |
| Adjust/调温 |  |  |  |  | Begin/开始： |  |  | End/结束： |  |  |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |  |  |  |  |  |
| CAUTION/注意：OxymaPure is highly toxic, high-energy compound. /2-肟氰乙酸乙酯是高毒，高能化合物。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |  |  |  |  |  |  |  |
| 3.23.5 OxymaPure addition /加2-肟氰乙酸乙酯 Add OxymaPure (2.00 eq.) with powder feeding bags into activation tank (TJ4S-1213-Sy15-R01) at 20±5 ℃ (target 20 ℃). 控温20±5 ℃（目标：20 ℃），通过投料袋向激活罐（TJ4S-1213-Sy15-R01）中加入 kg 2-肟氰乙酸乙酯（Acceptable weighing range/可接受称量范围： kg ~ kg）(2.00 eq.)。 |  |  |  |  |  |  |  |  |  | Times/次数 |  |  |
|  |  |  |  |  |  |  |  |  |  | 1 |  |  |
| Check and record that the solid charge port is free of solid accumulation from the previous step./ 检查并记录固体投料口无来自上一步骤的固体残留。 Checked by & Date/核实人及日期： |  |  |  |  |  |  |  |  |  |  |  |  |
| 3.23. （7）Fmoc-L-Arg(Pbf)-OH activation/Fmoc-Pbf-精氨酸活化 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Before material charging, confirm the material is within the retest date/ the expiry date,Confirm whether the weight is adequate for use in the current charging. /加物料前，执行物料在复检日期/有效期之内，确认重量是否满足本次投料使用。 Before solid material charging, confirm/加固体物料前，确认： - Before operation performed, No non-related material / equipment / instrument / tools / pipeline / document / record / label in the intended area /操作前，目标区域无不相关物料/设备/仪器/工具/管路/文件/记录/标签； - The outer packages are removed. If the outer package cannot be removed, but unpack material without damaging the package bag, and before charging, check no package fragment and other extraneous matter in material. /外包装移除；如果外包装不能移除，但在不破坏包装的情况下打开包装袋，投料前以及过程中同时检查执行物料中无包装碎片以及异物 Checked by & Date/核实人及日期： |  |  |  |  |  |  |  |  |  |  |  |  |
| OxymaPure addition/加2-肟氰乙酸乙酯 |  |  |  |  | Performed by & Date/执行人及日期： |  |  |  |  |  |  |  |
|  |  |  |  |  | Begin/开始： |  |  | End/结束： |  |  |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |  |  |  |  |  |
| Internal temperature/温度 |  |  |  |  | High values/最高值： |  |  | Low values/最低值： |  |  |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |  |  |  |  |  |
| OxymaPure confirmation/2-肟氰乙酸乙酯确认：Control No./内部控制号：YL005180 |  |  |  |  |  |  |  |  |  |  |  |  |
| Check items/检查项 |  |  | Is it correct?/是否正确 |  |  |  | Recorded by & Date/ 记录人及日期 |  | Checked by & Date/ 核实人及日期 |  |  |  |
| Material name/物料名称 |  |  | Yes/是；No/否 |  |  |  |  |  |  |  |  |  |
| Project batch number/ 项目批次号 |  |  | Yes/是；No/否 |  |  |  |  |  |  |  |  |  |
| Check items/检查项 |  |  | Is it correct?/是否正确 |  |  |  | Recorded by & Date/ 记录人及日期 |  | Checked by & Date/ 核实人及日期 |  |  |  |  |
| Operation number/ 操作号 |  |  | Yes/是；No/否 |  |  |  |  |  |  |  |  |  |  |
| Control No./内部控制号 |  |  | Yes/是；No/否 |  |  |  |  |  |  |  |  |  |  |
| Raman Scan Confirmation/拉曼扫描执行 |  |  | Conforms/相符 Does not conform/不相符 |  |  |  |  |  |  |  |  |  |  |

| 3.23. （7）Fmoc-L-Arg(Pbf)-OH activation/Fmoc-Pbf-精氨酸活化 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| No. /编号 | Net weight/净重 | All in/全部加入 | Recorded by & Date/ 记录人及日期 | Checked by & Date/ 核实人及日期 |
| / |  | Yes/是；No/否 |  |  |
| / |  | Yes/是；No/否 |  |  |
| / |  | Yes/是；No/否 |  |  |
| / |  | Yes/是；No/否 |  |  |
| / |  | Yes/是；No/否 |  |  |
| / |  | Yes/是；No/否 |  |  |
| / |  | Yes/是；No/否 |  |  |
| / |  | Yes/是；No/否 |  |  |
| / |  | Yes/是；No/否 |  |  |
| / |  | Yes/是；No/否 |  |  |
| / |  | Yes/是；No/否 |  |  |
| / |  | Yes/是；No/否 |  |  |
| Total/合计 |  | Confirm the total net weight is within the input range/执行合计净重在投料范围内 | Recorded by & Date/记录人及日期 | Checked by & Date/ 核实人及日期 |
|  |  | Yes/是；No/否 |  |  |
| After solid material charging, confirm/固体物料投料之后，确认： - The corresponding leftover material has been properly sealed and transferred to the staging area/相应的剩余物料已经恰当密封，转移到了暂存区域。 - The waste packaging material has been sealed and transferred to the corresponding waste staging area. /产生的包材废物已经密封并转移到了相应的废物暂存区域。 - The operation area/equipment surface has been cleaned/操作区域以及设备表面已经清洁。 Checked by & Date/核实人及日期： |  |  |  |  |

| 3.23. （7）Fmoc-L-Arg(Pbf)-OH activation/Fmoc-Pbf-精氨酸活化 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| CAUTION/注意：DMF/N，N-二甲基甲酰胺：Irritation of eyes, skin and nose. May cause nausea. /刺激眼睛、皮肤和鼻子。可能引起恶心。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |
| 3.23.6 Rinse activation tank /激活罐淋洗 Rinse the port of Activation tank (TJ4S-1213-Sy15-R01) with 91.01 kg（Acceptable weighing range：88.28 ~ 93.74 kg）（2 V）DMF. 用91.01 kg（可接受称量范围：88.28 ~ 93.74 kg）（2 V）N,N-二甲基甲酰胺淋洗激活罐（TJ4S-1213-Sy15-R01）加料口。 |  |  |  | Times/次数 |  |
|  |  |  |  | 1 |  |
| Rinse with DMF/ N,N-二甲基甲酰胺淋洗 | Performed by & Date/执行人及日期： |  |  |  |  |
| Rinse with DMF/ N,N-二甲基甲酰胺淋洗 | Begin/开始： | End/结束： | Qty/重量： |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |
| CAUTION/注意：DIC is a key EHS concern chemical, highly toxic, skin sensitizing, respiratory sensitizing. /N,N’二异丙基碳二亚胺为EHS重点关注化学品，高毒，皮肤致敏，呼吸致敏。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |
| 3.23.7 DIC addition /加N,N'-二异丙基碳二亚胺 Add DIC into the activation tank (TJ4S-1213-Sy15-R01) at reference time of 10-30 min at 20±5 ℃ (target: 20 ℃). The addition rate depends on temperature control.控温20±5 ℃ (目标20 ℃)，向激活罐（TJ4S-1213-Sy15-R01）中滴加 kg N,N'-二异丙基碳二亚胺（Acceptable weighing range/可接受称量范围： kg ~ kg）（滴加参考时间10-30 min，以控温为主）。 Pump number/泵编号： Scale ID. No./秤编号： Material charging pipe number/物料管路编号： |  |  |  |  | Times/次数 |
|  |  |  |  |  | 1 |

| 3.23. （7）Fmoc-L-Arg(Pbf)-OH activation/Fmoc-Pbf-精氨酸活化 |  |
| --- | --- |
| ★Note:The DIC addition temperature is a CPP./ 备注：N,N'-二异丙基碳二亚胺加料温度是关键控制参数。 Prior to DIC addition, the operator shall confirm that the reaction mixture temperature is within the Normal Operating Range (NOR) / N,N'-二异丙基碳二亚胺投料前，操作人员须执行反应体系温度处于正常操作范围（NOR）内: ★Lower limit/低限: 15 oC ，High limit/高限: 25 oC Over-temperature warning value/超温预警设置数值： Lower limit/低限: 17 oC ，High limit/高限: 23 oC |
| Before material charging, confirm the material is within the retest date/ the expiry date, Confirm whether the weight is adequate for use in the current charging. /加物料前，执行物料在复检日期/有效期之内，确认重量是否满足本次投料使用。 Performed by & Date/执行人及日期： |
| Before drummed liquid material charging, visual check there is no other extraneous matter in material. /加桶装液体物料前，应目视执行桶内无异物。 Visual Examination of Foreign Matters for Drum Solvent/桶装溶剂异物目视检查: 1. Place the solvent drum in a well-lit area (or use a portable light source) to ensure adequate visibility of the liquid contents./ 将溶剂桶置于光线充足区域（或使用便携式光源），确保液体内容物可见性良好 2. Visually inspect the solvent through the drum bung opening or transparent sight glass for any visible foreign matters, including but not limited to particulates, fibers, sediments, or unusual turbidity./ 通过桶口或透明视镜目视检查溶剂中是否有可见异物，包括但不限于颗粒、纤维、沉淀物或异常浑浊。 3. Confirm that the solvent is clear and free of visible foreign matters before charging. If any foreign matter is observed, do not use the drum and follow the applicable handling procedure for subsequent disposition./ 投料前确认溶剂澄清且无可见异物。如发现任何异物，不得使用该桶，并依据处理流程进行后续处置。 Checked by & Date/核实人及日期： |

| 3.23. （7）Fmoc-L-Arg(Pbf)-OH activation/Fmoc-Pbf-精氨酸活化 |  |  |
| --- | --- | --- |
| Confirm liquid charging pipeline is released. /确认加料管路经过放行。 |  |  |
| Performed by & Date/执行人及日期： |  |  |
| Charging of drummed solvent: verify that the pipeline connections remain consistent before and after material feeding, then record the weighing reading 桶装溶剂溶液称量时，确认投料前后管路连接一致，读取称量示数。 Performed by & Date/执行人及日期： Checked by & Date/核实人及日期： |  |  |
| DIC /N,N’-二异丙基碳二亚胺：Control Number/内部控制号：YL003564 |  |  |
| Internal temperature/内部温度 | High values/最高值： | Low values/最低值： |
| Transcribed by & Date /转录人及日期： |  |  |
| If the target net weight≥the planned net weight, perform full drum charge; otherwise, perform partial (Non-full drum) charge. /当目标加料量≥计划加入净重，勾选进行整桶加料；否则勾选进行零头加料。 Note: Target net weight to be charged = Total target net weight – Total net weight charged. Expected tare weight of drum after charge = Gross weight of drum that to be charged - Target net weight to be charged. 还需加入净重目标值= 加料步骤目标总净重-已加入净重总量 加料完成后预计桶皮重=待加料桶毛重-还需加入净重目标值 |  |  |

| 3.23. （7）Fmoc-L-Arg(Pbf)-OH activation/Fmoc-Pbf-精氨酸活化 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Addition type加料式 | Lot number/批号 | Pre-charging qty calculation/投料前投料量计算 |  |  |  |  |  | Actual Charging Qty/实际投料量 |  |  |  |  |  |
|  |  | Total qty charged/已加入净重总量 |  | Target qty to be charged/还需加入净重目标值 | Gross qty of drum that to be charged/待加料桶毛重 | Expected net weight of drum after charge /加料完成后预计桶净重 | Expected tare weight of drum after charge/加料完成后预计桶皮重 | Before charging/ 投料前 |  | After charging/ 投料后 | Net weight/ 净重 | Performed by & Date/ 执行人及日期 | Checked by & Date/ 核实人及日期 |
| Full drum整桶 Non-full drum /非整桶 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Full drum整桶 Non-full drum /非整桶 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Total/合计 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DIC Addition/加N,N'-二异丙基碳二亚胺 |  |  | Begin/开始： |  |  |  |  |  | End/结束： |  |  |  |  |
| Transcribed by & Date/转录人及日期： |  |  |  |  |  |  |  |  |  |  |  |  |  |

| 3.23. （7）Fmoc-L-Arg(Pbf)-OH activation/Fmoc-Pbf-精氨酸活化 |  |  |  |
| --- | --- | --- | --- |
| 3.23.8 Fmoc-L-Arg(Pbf)-OH activation / Fmoc-Pbf-精氨酸活化 At 20±5℃ (target: 20 ℃), continue agitation of activation tank to 95±10 rpm (target: 95 rpm). Maintain for 20±5 min (target: 20 min) after addition, then stop stirring. 控温20±5 ℃(目标温度：20 ℃)，维持激活罐搅拌转速至95±10 rpm (目标转速: 95 rpm),继续搅拌20±5 min（目标时间：20 min）, 停止搅拌。 |  |  | Times/次数 |
|  |  |  | 1 |
| Stir /搅拌 | Performed by & Date/执行人及日期： |  |  |
| Stir /搅拌 | End/结束 |  |  |
|  | The start time is documented in Ins3.23.1 /开始时间记录于3.23.1 |  |  |
| Transcribed by & Date/转录人及日期： |  |  |  |
| Internal temperature/内部温度 | High values/最高值： | Low values/最低值： |  |
| Transcribed by & Date/转录人及日期： |  |  |  |

| - （7）Fmoc-L-Arg(Pbf)-OH coupling/ Fmoc-Pbf-精氨酸偶联 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 3.24.1 Activation mixture transfer /活化体系转移 Transfer the mixture in activation tank (TJ4S-1213-Sy15-R01) into synthesis reactor (TJ4S-1213-Sy15). 将激活罐（TJ4S-1213-Sy15-R01）中体系转移至合成釜（TJ4S-1213-Sy15）中。 |  |  |  |  |  | Times/次数 |
|  |  |  |  |  |  | 1 |
| ★Note:The coupling temperature is a CPP, and the coupling IPC hold temperature is a CPP./ 备注：偶联温度是CPP，偶联IPC维持温度是关键工艺参数。 The operator shall confirm that the reaction mixture temperature is within the Normal Operating Range (NOR) 【25±5oC (target: 25oC)】 /操作人员须执行反应体系温度处于正常操作范围（NOR）内【25±5oC (target: 25oC)】 Checked by & Date/核实人及日期： Over-temperature warning value/超温预警设置数值： Lower limit/低限: 21 oC ，High limit/高限: 28 oC |  |  |  |  |  |  |
| Transfer/转移 |  | Performed by & Date/执行人及日期： |  |  |  |  |
|  |  | Checked by & Date/核实人及日期： |  |  |  |  |
|  |  | Begin/开始： |  | End/结束： |  |  |
| CAUTION/注意：DMF/N，N-二甲基甲酰胺：Irritation of eyes, skin and nose. May cause nausea. /刺激眼睛、皮肤和鼻子。可能引起恶心。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |  |
| 3.24.2 DMF addition /加N,N-二甲基甲酰胺 Add 45.50 kg (Acceptable weighing range: 43.50 ~ 47.50 kg) (1 V) DMF by spray balls into activation tank (TJ4S-1213-Sy15-R01) to rinse the wall. 通过淋洗球向激活罐（TJ4S-1213-Sy15-R01）中加入45.50 kg（可接受称量范围：43.50 ~ 47.50 kg） (1 V) N,N-二甲基甲酰胺淋洗罐壁。 |  |  |  |  |  | Times/次数 |
|  |  |  |  |  |  | 1 |
| Rinse with DMF/ N,N-二甲基甲酰胺淋洗 | Performed by & Date/执行人及日期： |  |  |  |  |  |
| Rinse with DMF/ N,N-二甲基甲酰胺淋洗 | Begin/开始： |  | End/结束： |  | Qty/重量： |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |
| 3.24.3 Transfer /转移 Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h). Bubble with nitrogen for 5±2 min (target: 5 min), then start the agitator. Transfer the mixture in activation tank (TJ4S-1213-Sy15-R01) into synthesis reactor (TJ4S-1213-Sy15). 合成釜（TJ4S-1213-Sy15）开启下口氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min (目标：5 min)后，开启搅拌转速至20±10 rpm (目标转速: 20 rpm)，将激活罐（TJ4S-1213-Sy15-R01）中体系转移至合成釜（TJ4S-1213-Sy15）中。 |  |  |  |  |  | Times/次数 |
|  |  |  |  |  |  | 1 |

| 3.24. （7）Fmoc-L-Arg(Pbf)-OH coupling/ Fmoc-Pbf-精氨酸偶联 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h). Bubble with nitrogen for 5±2 min (target: 5 min), Start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm). Stir for 10±5 min (target: 10 min)./ 开启合成釜下口鼓氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min（目标时间：5 min）后开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm)，搅拌10±5 min (目标时间: 10 min)。. |  |  |  |  |
| Bubble/鼓氮气 | Performed by & Date/执行人及日期： |  |  |  |
| Bubble/鼓氮气 | Begin/开始： |  |  |  |
|  | The end time is documented in Ins3.24.5 /结束时间记录于3.24.5 |  |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |
| Stir/搅拌 | Performed by & Date/执行人及日期： |  |  |  |
| Stir /搅拌 | Begin/开始： |  |  |  |
|  | The end time is documented in Ins3.24.5 /结束时间记录于3.24.5 |  |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |
| Transfer the mixture (DMF) from the activation tank (TJ4S-1213-Sy15-R01) to the synthesis reactor (TJ4S-1213-Sy15). /激活罐(TJ4S-1213-Sy15-R01)中体系转移至合成釜(TJ4S-1213-Sy15)中。 |  |  |  |  |
| Transfer/转移 |  | Performed by & Date/执行人及日期： |  |  |
|  |  | Checked by & Date/核实人及日期： |  |  |
|  |  | Begin/开始： | End/结束： |  |
| 3.24.4 Adjust synthesis reactor temperature/合成釜调温 Adjust the mixture in synthesis reactor(TJ4S-1213-Sy15) to【 25±5 oC (target: 25 oC)】. 合成釜(TJ4S-1213-Sy15)体系调温至【25±5 ℃(目标温度：25 ℃)】。 |  |  |  | Times/次数 |
|  |  |  |  | 1 |
| The total reaction Fmoc-L-Ser(tBu)-OH coupling time：NMT 840 min (Sourced from the holding time study in the PAR study report.)/ 芴甲氧羰基-叔丁基-L-丝氨酸偶联总的反应时间不超过840min(来源于 PAR 研究报告中稳定时间研究)。 Over-temperature warning value/超温预警设置数值： Lower limit/低限: 21 oC ，High limit/高限: 28 oC |  |  |  |  |

| 3.24. （7）Fmoc-L-Arg(Pbf)-OH coupling/ Fmoc-Pbf-精氨酸偶联 |  |  |  |
| --- | --- | --- | --- |
| Adjust/调温 | Performed by & Date/执行人及日期： |  |  |
| Adjust/调温 | Begin/开始： | End/结束： |  |
| Transcribed by & Date /转录人及日期： |  |  |  |
| 3.24.5 Fmoc-L-Arg(Pbf)-OH coupling/ Fmoc-Pbf-精氨酸偶联 The mixture is allowed to react at 25±5 ℃ (target: 25 ℃). After stirring for 3 h±15 min (target: 3 h), then sample the mixture every 0.5~1 h for ninhydrin test analysis until two consecutive samples test as negative (Yellow to Light pink Report Negative= Complete coupling; Blue to Purple Positive= Incomplete coupling). And the mixture should be stirred for coupling time (4 h≤Coupling Time≤10 h). Stop stirring, turn off the nitrogen. Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point. 控温25±5 ℃(目标：25 ℃)反应，反应3 h±15 min（目标：3 h）后，然后取样进行茚三酮检测，每隔0.5~1.0 h取样一次，直至连续两次取样结果为阴性（黄色至浅粉色，报告阴性=偶联完全；蓝色至紫色，阳性=偶联不完全）后，且反应时间满足（4h≤反应时间≤10h）后，停止搅拌，关闭氮气。通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 Note: If resin adherence is found at other times during the reaction, rinse the reactor with 0.50~5.00 kg DMF each time. /备注：偶联过程中其它时间发现贴壁，每次使用0.50~5.00 kg N,N-二甲基甲酰胺淋洗，记录见附加页 |  |  | Times/次数 |
|  |  |  | 1 |
| React/反应 | Performed by & Date/执行人及日期： |  |  |

| 3.24. （7）Fmoc-L-Arg(Pbf)-OH coupling/ Fmoc-Pbf-精氨酸偶联 |  |  |
| --- | --- | --- |
| ★After coupling is completed, the operator shall confirm that all Critical Process Parameters (CPP) are within their respective Normal Operating Ranges (NOR)/ 偶联结束后，操作人员须确认所有关键工艺参数（CPP）均处于各自的正常操作范围（NOR）内 ★Note: The coupling temperature is a CPP, and the coupling IPC hold temperature is a CPP./备注：偶联温度是CPP，偶联IPC维持温度是关键工艺参数。 The operator shall confirm that the reaction mixture temperature is within the Normal Operating Range (NOR) :20~30℃ /操作人员须确认反应体系温度处于正常操作范围（NOR）内:20~30℃ □Yes/是 □No/否 Confirmed by & Date/确认人及日期： Checked by & Date/核实人及日期： |  |  |
| React/反应 | Begin/开始： | End/结束： |
| Transcribed by & Date /转录人及日期： |  |  |

| 3.24. （7）Fmoc-L-Arg(Pbf)-OH coupling/ Fmoc-Pbf-精氨酸偶联 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Standard sampling method/标准取样方法 |  |  |  |  |  |  |  |
| Take 5-10 ml mixture and perform testing in accordance with the Kaiser test record. During sampling, should wear nitrile gloves and 3M 6000 series Full-face piece respirator, 6006CN cartridges. 通过在线取样器取样5~10 ml按照茚三酮检测记录进行检测。取样过程中，应该戴丁腈手套、3M 6000系列全面罩, 6006CN滤毒盒。 |  |  |  |  |  |  |  |
| Sampling number/ 取样编号 | Sampling time/取样时间 |  | Sampled by & Date/取样人及日期 | Negative or Positive/阳性或阴性 |  | Result filled by &Date/结果填写人及日期 | Checked by & Date/核实人及日期 |
| CPo140020- -01- -3.24.5-01 |  |  |  | Negative /阴性 Positive /阳性 |  |  |  |
| CPo140020- -01- -3.24.5-02 |  |  |  | Negative /阴性 Positive /阳性 |  |  |  |
| CPo140020- -01- -3.24.5-03 |  |  |  | Negative /阴性 Positive /阳性 |  |  |  |
| CPo140020- -01- -3.24.5-04 |  |  |  | Negative /阴性 Positive /阳性 |  |  |  |
| CPo140020- -01- -3.24.5-05 |  |  |  | Negative /阴性 Positive /阳性 |  |  |  |
| CPo140020- -01- -3.24.5-06 |  |  |  | Negative /阴性 Positive /阳性 |  |  |  |
| CPo140020- -01- -3.24.5-07 |  |  |  | Negative /阴性 Positive /阳性 |  |  |  |
| CPo140020- -01- -3.24.5-08 |  |  |  | Negative /阴性 Positive /阳性 |  |  |  |
| Confirm the end point of the reaction and move to following operation. /执行反应终点后进行下一步操作。 The acceptance criterion is two consecutive negative test results./连续两次检测结果为阴性即为合格。 Confirmed by & Date/确认人及日期： Checked by & Date/核实人及日期： |  |  |  |  |  |  |  |
| Stir/搅拌 |  |  | Performed by & Date/执行人及日期： |  |  |  |  |  |
| Stir /搅拌 |  |  | End/结束： |  |  |  |  |  |
|  |  |  | The start time is documented in Ins 3.24.3 /开始时间记录于3.24.3 |  |  |  |  |  |
| Transcribed by & Date/转录人及日期： |  |  |  |  |  |  |  |  |
| Bubble/鼓氮气 |  |  | Performed by & Date/执行人及日期： |  |  |  |  |  |
| Bubble/鼓氮气 |  |  | End/结束： |  |  |  |  |  |
|  |  |  | The start time is documented in Ins 3.24.3 /开始时间记录于3.24.3 |  |  |  |  |  |
| Transcribed by & Date/转录人及日期： |  |  |  |  |  |  |  |  |
| Internal temperature/内部温度 |  | High values/最高值： |  |  | Low values/最低值： |  |  |  |
| Transcribed by & Date/转录人及日期： |  |  |  |  |  |  |  |  |

| 3.24. （7）Fmoc-L-Arg(Pbf)-OH coupling/ Fmoc-Pbf-精氨酸偶联 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Filter/过滤 |  | Begin/开始： |  |  | End/结束： |  |  |  |
|  |  | Performed by & Date/执行人及日期 |  |  |  |  |  |  |
|  |  | Checked by & Date/核实人及日期： |  |  |  |  |  |  |
| ★After coupling is completed, the operator shall confirm that all Critical Process Parameters (CPPs) are within their respective Normal Operating Ranges (NOR)/ 偶联结束后，操作人员须确认所有关键工艺参数（CPP）均处于各自的正常操作范围（NOR）内 Checked by & Date/核实人及日期： |  |  |  |  |  |  |  |  |
| CAUTION/注意：DMF/N，N-二甲基甲酰胺：Irritation of eyes, skin and nose. May cause nausea. /刺激眼睛、皮肤和鼻子。可能引起恶心。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |  |  |  |
| 3.24.6 Activation Tank Rinse/激活罐洗涤 Add 364.03 kg (Acceptable weighing range：353.11 ~ 374.95 kg) (8 V) DMF into the activation tank (TJ4S-1213-Sy15-R01). Circulate and clean the activating liquid transfer pipeline, liquid charging port cleaning pipeline and solid charging port cleaning pipeline. Clean every line wash for 2±1 min (target: 2 min). 向激活罐（TJ4S-1213-Sy15-R01）中加入364.03 kg（可接受称量范围：353.11 ~ 374.95 kg） (8 V) N,N-二甲基甲酰胺，循环清洗激活罐转移管路，液体加料口清洗管路和固体加料口清洗管路，每条线路各清洗2±1 min（目标：2 min）。 |  |  |  |  |  |  |  | Times/次数 |
|  |  |  |  |  |  |  |  | 3 |
| Operation name/ 操作名称 |  | Time point/时间节点 |  | Clean the activating liquid transfer pipeline 1/循环清洗激活液转移管路1 | Liquid charging port cleaning pipeline 2 /液体加料口清洗管路2 |  | Solid charging port cleaning pipeline 3,/固体加料口清洗管路3 |  |
|  |  |  |  | Wash /洗涤1 | Wash /洗涤2 |  | Wash /洗涤3 |  |
| DMF addition/ 加N,N-二甲基甲酰胺 |  | Performed by & Date/执行人及日期： |  |  |  |  |  |  |

| 3.24. （7）Fmoc-L-Arg(Pbf)-OH coupling/ Fmoc-Pbf-精氨酸偶联 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Operation name/ 操作名称 |  | Time point/时间节点 | Clean the activating liquid transfer pipeline 1/循环清洗激活液转移管路1 | Liquid charging port cleaning pipeline 2 /液体加料口清洗管路2 | Solid charging port cleaning pipeline 3,/固体加料口清洗管路3 |  |
|  |  |  | Wash /洗涤1 | Wash /洗涤2 | Wash /洗涤3 |  |
| DMF addition/ 加N,N-二甲基甲酰胺 |  | Begin/开始 |  |  |  |  |
|  |  | End/结束 |  |  |  |  |
|  |  | Qty/重量 |  |  |  |  |
|  |  | Transcribed by & Date /转录人及日期： |  |  |  |  |
| Circulate and clean/循环清洗 |  | Performed by & Date/执行人及日期： |  |  |  |  |
| Circulate and clean/循环清洗 |  | Begin/开始 |  |  |  |  |
|  |  | End/结束 |  |  |  |  |
|  |  | Transcribed by & Date /转录人及日期： |  |  |  |  |
| 3.24.7 Post-coupling wash./偶联后洗涤 Transfer the mixture in activation tank (TJ4S-1213-Sy15-R01) to synthesis reactor (TJ4S-1213-Sy15). Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h).Bubble with nitrogen for 5±2 min (target 5 min), start the agitator. After stirring for 7±2 min (target：7 min), stop stirring, turn off the nitrogen. Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point. 将激活罐（TJ4S-1213-Sy15-R01）洗涤液转入合成釜 (TJ4S-1213-Sy15)。开启合成釜下口鼓氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min (目标时间：5 min)后开启搅拌转速至20±10 rpm (目标转速: 20 rpm)，搅拌7±2 min（目标时间：7 min）后停止搅拌，关闭氮气。通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  |  |  | Times/次数 |  |
|  |  |  |  |  | 3 |  |

| 3.24. （7）Fmoc-L-Arg(Pbf)-OH coupling/ Fmoc-Pbf-精氨酸偶联 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Transfer the liquid from activation tank to the synthesis reactor (TJ4S-1213-Sy15) ./将激活罐中体系转移至合成釜中。 |  |  |  |  |  |  |  |
| Operation name/ 操作名称 |  | Time point/时间节点 | Wash /洗涤1 | Wash /洗涤2 |  | Wash /洗涤3 |  |
| Transfer/转移 |  | Performed by & Date/执行人及日期： |  |  |  |  |  |
|  |  | Checked by & Date/核实人及日期： |  |  |  |  |  |
|  |  | Begin/开始 |  |  |  |  |  |
|  |  | End/结束 |  |  |  |  |  |
| Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h).Bubble with nitrogen for 5±2 min (target 5 min), start the agitator. After stirring for 7±2 min (target：7 min), stop stirring, turn off the nitrogen.开启合成釜下口鼓氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min (目标：5 min)后开启搅拌转速至20±10 rpm (目标转速: 20 rpm)，搅拌7±2 min（目标：7 min）后停止搅拌，关闭氮气。 |  |  |  |  |  |  |  |  |  |
| Bubble/鼓氮气 |  |  | Performed by & Date/执行人及日期： |  |  |  |  |  |  |
| Bubble/鼓氮气 |  |  | Begin/开始 |  |  |  |  |  |  |
|  |  |  | End/结束 |  |  |  |  |  |  |
|  |  |  | Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |
| Stir /搅拌 |  |  | Performed by & Date/执行人及日期： |  |  |  |  |  |  |
| Stir /搅拌 |  |  | Begin/开始 |  |  |  |  |  |  |
|  |  |  | End/结束 |  |  |  |  |  |  |
|  |  |  | Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |

| 3.24. （7）Fmoc-L-Arg(Pbf)-OH coupling/ Fmoc-Pbf-精氨酸偶联 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Operation name/ 操作名称 |  | Time point/时间节点 | Wash /洗涤1 | Wash /洗涤2 | Wash /洗涤3 |
| Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point/通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  |  |  |  |
| Filter/过滤 |  | Performed by & Date/执行人及日期： |  |  |  |
|  |  | Checked by & Date/核实人及日期： |  |  |  |
|  |  | Begin/开始 |  |  |  |
|  |  | End/结束 |  |  |  |

| - （8）Fmoc-L-Val-OH deprotection/芴甲氧羰基-L-缬氨酸脱保护 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CAUTION/注意：Piperidine哌啶：Strong local irritant and may cause permanent injury after short exposure to small amounts. /强烈的局部刺激，短时间少量接触可能造成永久性伤害。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |  |
| 3.25.1 Piperidine soulution addition /加哌啶溶液 Adjust the jacket temperature of synthesis reactor (TJ4S-1213-Sy15) to 【25±5 oC (target: 25 oC). 】. Add 357.50 kg 【20% (V/V) piperidine/DMF(8 V)】 (Acceptable weighing range: 346.77 ~ 368.23 kg) solution into the Synthesis reactor (TJ4S-1213-Sy15). 将合成釜(TJ4S-1213-Sy15)夹套调温至【25±5 ℃（目标温度：25 ℃）】。向合成釜（TJ4S-1213-Sy15）中加入357.50 kg【20%哌啶/DMF (V/V)溶液(8 V)】（可接受称量范围：346.77 ~ 368.23 kg）。 |  |  |  |  |  | Times/次数 |
|  |  |  |  |  |  | 1 |
| Adjust/调温 |  | Performed by & Date/执行人及日期： |  |  |  |  |
| Adjust/调温 |  | Begin/开始： |  | Adjust/调温 |  |  |
| Transcribed by & Date/转录人及日期： |  |  |  |  |  |  |
| Confirm and record that the 20%Piperidine solution concentration is within the NOR range./ 确认并记录20%哌啶溶液浓度在正常操作范围内：17.50%~22.50% 20%Piperidine solution concentration confirmation/20%哌啶溶液浓度确认：____________ Confirm and record that the 20%Piperidine solution temperature is within the NOR range./ 确认并记录20%哌啶溶液温度在正常操作范围内：20 oC ~30 oC 20%Piperidine solution temperature confirmation/20%哌啶溶液温度确认：____________ Confirmed by & Date/确认人及日期： Checked by & Date/核实人及日期： |  |  |  |  |  |  |
| Piperidine addition/ 加哌啶溶液 | Performed by & Date/执行人及日期： |  |  |  |  |  |
| Piperidine addition/ 加哌啶溶液 | Begin/开始： |  | End/结束： |  | Qty/重量： |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |

| 3.25. （8）Deprotection/脱保护 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 3.25.2 Adjust temperature /调温 Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h) for 5±2 min (target 5 min), Start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm). Stir for 10±5 min (target: 10 min). Adjust to 【25±5 ℃ (target: 25 ℃)】. 开启合成釜下口鼓氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min (目标时间：5 min)后开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm)，搅拌10±5 min (目标时间: 10 min)。调温至【25±5 ℃(目标温度：25 ℃)】。 |  |  |  | Times/次数 |  |
|  |  |  |  | 1 |  |
| Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h). Bubble with nitrogen for 5±2 min (target :5 min) 开启合成釜下口鼓氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min (目标时间：5 min) |  |  |  |  |  |
| Bubble/鼓氮气 |  |  | Performed by & Date/执行人及日期： |  |  |
| Bubble/鼓氮气 |  |  | Begin/开始： |  |  |
|  |  |  | The end time is documented in Ins 3.25.3/结束时间记录于3.25.3 |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |
| Start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm). 开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm) |  |  |  |  |  |
| Stir /搅拌 |  |  |  | Performed by & Date/执行人及日期： |  |
| Stir /搅拌 |  |  |  | Begin/开始： |  |
|  |  |  |  | The end time is documented in Ins 3.25.3/结束时间记录于3.25.3 |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |
| Adjust to 25±5 oC (target: 25 oC)./ 调温至25±5 ℃(目标温度：25 oC) |  |  |  |  |  |

| 3.25. （8）Deprotection/脱保护 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Adjust/调温 |  | Performed by & Date/执行人及日期： |  |  |  |
| Adjust/调温 |  | Begin/开始： |  | End/结束： |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |
| 3.25.3 Fmoc-L-Val-OH De-Fmoc /芴甲氧羰基-L-缬氨酸脱保护 After stirring for【2 h±15 min (target: 2 h) at 25±5 ℃ (target: 25 ℃)】, stop stirring, turn off the nitrogen. Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point. 控温【25±5 ℃(目标：25 ℃)，搅拌2 h±15 min（目标：2 h）】后，停止搅拌，关闭氮气。通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 Over-temperature warning value/超温预警设置数值： Lower limit/低限: 22 oC ，High limit/高限: 28 oC |  |  |  |  |  | Times/次数 |  |
|  |  |  |  |  |  | 1 |  |
| ★Confirm and record that the 20%Piperidine solution temperature is within the NOR range 20 oC ~30 oC ./ 确认并记录20%哌啶溶液温度在正常操作范围内：20 oC ~30 oC。 20%Piperidine solution temperature confirmation/20%哌啶溶液温度确认：____________ Confirmed by& Date/确认人及日期： Checked by & Date/核实人及日期： |  |  |  |  |  |  |  |
| De-Fmoc/脱Fmoc &Stir /搅拌 |  |  |  | Performed by & Date/执行人及日期： |  |  |  |
| De-Fmoc/脱Fmoc &Stir /搅拌 |  |  |  | End/结束： |  |  |  |
|  |  |  |  | The start time is documented in Ins3.25.2 /开始时间记录于3.25.2 |  |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |

| 3.25. （8）Deprotection/脱保护 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Bubble/鼓氮气 |  |  | Performed by & Date/执行人及日期： |  |  |  |
| Bubble/鼓氮气 |  |  | End/结束： |  |  |  |
|  |  |  | The start time is documented in Ins3.25.2 /开始时间记录于3.25.2 |  |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |
| Internal temperature/内部温度 |  |  | High values/最高值: |  | Low values/最低值： |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |
| Filter/过滤 |  |  | Performed by & Date/执行人及日期： |  |  |  |
|  |  |  | Checked by & Date/核实人及日期： |  |  |  |
|  |  |  | Begin/开始： |  | End/结束： |  |
| ★After De Fmoc is completed, the operator shall confirm that all Critical Process Parameters (CPPs) are within their respective Normal Operating Ranges (NOR)/ 脱保护结束后，操作人员须确认所有关键工艺参数（CPP）均处于各自的正常操作范围（NOR）内 Checked by & Date/核实人及日期： |  |  |  |  |  |  |
| 3.25.4 Adjust synthesis reactor temperature /合成釜调温 Adjust the synthesis reactor (TJ4S-1213-Sy15) jacket to【20±5 ℃ (target: 20 ℃)】. /将合成釜（TJ4S-1213-Sy15）夹套调温至【20±5 ℃（目标温度：20 ℃）】。 |  |  |  |  | Times/次数 |  |
|  |  |  |  |  | 1 |  |
| Adjust/调温 |  |  | Performed by & Date/执行人及日期： |  |  |  |  |
| Adjust/调温 |  |  | Begin/开始： |  | End/结束： |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |

| 3.25. （8）Deprotection/脱保护 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CAUTION/注意：IPAC/乙酸异丙酯： Vapors irritate eyes and respiratory tract; high concentrations can be anesthetic. Liquid irritates eyes but causes no serious injury; may cause dermatitis; no serious effects if swallowed. /蒸汽会刺激眼睛和呼吸道；高浓度的蒸汽可能是麻醉剂。液体刺激眼睛，但不会造成严重伤害；可能会引起皮炎；吞食不会造成严重影响。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |  |  |  |
| 3.25.5 Post-deprotection wash/脱保护后洗涤 Add 445.92 kg (Acceptable weighing range：432.54 ~ 459.30 kg) (9.29 kg/kg, 10.64 V) isopropyl acetate into synthesis reactor (TJ4S-1213-Sy15). Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h). Bubble with nitrogen for 5±2 min (target 5 min), then start the agitator to 20±10 rpm (target: 20 rpm). After stirring for 5±2 min (target 5 min), stop stirring, turn off the nitrogen. Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point. 向合成釜（TJ4S-1213-Sy15）中加入445.92 kg（可接受称量范围：432.54 ~ 459.30 kg) (9.29 kg/kg, 10.64 V）乙酸异丙酯，开启下口氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min (目标：5 min)后开启搅拌转速至20±10 rpm (目标转速: 20 rpm)，搅拌5±2 min (目标：5 min)后，停止搅拌，关闭氮气。通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  |  |  |  |  | Times/次数 |  |
|  |  |  |  |  |  |  | 4 |  |
| ★Note: IPAC quantity is a CPP./备注：乙酸异丙酯用量是关键工艺参数。 ★The operator shall confirm that IPAC quantity is within the Normal Operating Range (NOR) 【432.54 kg~459.30 kg】/操作人员须确认乙酸异丙酯用量处于正常操作范围（NOR）内：【432.54 kg~459.30 kg】 Checked by & Date/核实人及日期： |  |  |  |  |  |  |  |  |
| Operation name/操作名称 | Time point/时间节点 | Wash /洗涤1 |  | Wash /洗涤2 | Wash /洗涤3 | Wash /洗涤4 |  |  |
| Add 419.04 kg IPAc (10 V) by spray balls into the synthesis reactor (TJ4S-1213-Sy15)./ 向合成釜(TJ4S-1213-Sy15) 中加419.04 kg乙酸异丙酯（（10 V）（Acceptable weighing range/可接受称量范围：440.34~467.58kg）） |  |  |  |  |  |  |  |  |
| IPAC addition/加乙酸异丙酯 | Performed by & Date/执行人及日期： |  |  |  |  |  |  |  |
| IPAC addition/加乙酸异丙酯 | Begin/开始 |  |  |  |  |  |  |  |
|  | End/结束 |  |  |  |  |  |  |  |
|  | Qty/重量 |  |  |  |  |  |  |  |
|  | Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |
| 3.25. （8）Deprotection/脱保护 |  |  |  |  |  |  |  |  |
| Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h) Bubble with nitrogen for 5±2 min (target: 5 min). turn off the nitrogen. Start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm). Stir for 5±2 min (target: 5 min). After stirring for 5±2 min (target: 5 min), stop stirring, turn off the nitrogen./ 开启合成釜下口鼓氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5± 2min（目标时间：5 min）后开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm)，搅拌5±2 min（目标时间：5 min）后，停止搅拌，关闭氮气 |  |  |  |  |  |  |  |  |
| Operation name/操作名称 | Time point/时间节点 |  | Wash /洗涤1 | Wash /洗涤2 | Wash /洗涤3 | Wash /洗涤4 |  |
| Bubble/鼓氮气 | Performed by & Date/执行人及日期： |  |  |  |  |  |  |
| Bubble/鼓氮气 | Begin/开始 |  |  |  |  |  |  |
|  | End/结束 |  |  |  |  |  |  |
|  | Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |
| Stir /搅拌 | Performed by & Date/执行人及日期： |  |  |  |  |  |  |
| Stir /搅拌 | Begin/开始 |  |  |  |  |  |  |
|  | End/结束 |  |  |  |  |  |  |
|  | Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |
| Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point./通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  |  |  |  |  |  |
| Filter/过滤 | Performed by & Date/执行人及日期： |  |  |  |  |  |  |
|  | Checked by & Date/核实人及日期 |  |  |  |  |  |  |
|  | Begin/开始 |  |  |  |  |  |  |
|  | End/结束 |  |  |  |  |  |  |

| 3.25. （8）Deprotection/脱保护 |  |  |  |  |
| --- | --- | --- | --- | --- |
| CAUTION/注意：DMF/N，N-二甲基甲酰胺：Irritation of eyes, skin and nose. May cause nausea. /刺激眼睛、皮肤和鼻子。可能引起恶心。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |
| 3.25.6 DMF addition/加N,N-二甲基甲酰胺 Add 546.05 kg (Acceptable weighing range: 529.67 ~ 562.43 kg) (12 V) DMF by spray balls into synthesis reactor (TJ4S-1213-Sy15). 通过淋洗球向合成釜（TJ4S-1213-Sy15）中加入546.05 kg（可接受称量范围：529.67 ~ 562.43 kg）（12 V）N,N-二甲基甲酰胺。 |  |  |  | Times/次数 |
|  |  |  |  | 1 |
| DMF addition/ 加N,N-二甲基甲酰胺 | Performed by & Date/执行人及日期： |  |  |  |
| DMF addition/ 加N,N-二甲基甲酰胺 | Begin/开始： | End/结束： | Qty/重量： |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |
| 3.25.7 Resin reswelling wash 1/溶胀后洗涤1 Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h) .Bubble with nitrogen for 5±2 min (target: 5 min), then start the agitator to 20±10 rpm (target: 20 rpm). After stirring for 10±2 min (target 10 min), stop stirring, turn off the nitrogen. Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point. 开启合成釜（TJ4S-1213-Sy15）下口鼓氮气, 氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min后（目标：5 min），开启搅拌转速至20±10 rpm (目标转速: 20 rpm)，搅拌10±2 min (目标：10 min)后，停止搅拌，关闭氮气。通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  |  | Times/次数 |
|  |  |  |  | 1 |
| Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h) for 5±2 min (target: 5 min), Start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm). Stir for 10 ±5 min (target: 10 min). After stirring for 10±2 min (target: 10 min), stop stirring, turn off the nitrogen./ 开启合成釜下口鼓氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min（目标时间：5 min）后，开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm)，搅拌10±2 min（目标时间:10 min）后，停止搅拌，关闭氮气 |  |  |  |  |

| 3.25. （8）Deprotection/脱保护 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Bubble/鼓氮气 |  | Performed by & Date/执行人及日期： |  |  |  |  |
| Bubble/鼓氮气 |  | Begin/开始： |  | End/结束： |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |
| Stir /搅拌 |  | Performed by & Date/执行人及日期： |  |  |  |  |
| Stir /搅拌 |  | Begin/开始： |  | End/结束： |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |
| Filter/过滤 |  | Performed by & Date/执行人及日期： |  |  |  |  |
|  |  | Checked by & Date/核实人及日期： |  |  |  |  |
|  |  | Begin/开始： |  | End/结束： |  |  |
| CAUTION/注意：DMF/N，N-二甲基甲酰胺：Irritation of eyes, skin and nose. May cause nausea. /刺激眼睛、皮肤和鼻子。可能引起恶心。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |  |
| 3.25.8 DMF addition/加N,N-二甲基甲酰胺 Add 546.05 kg (Acceptable weighing range: 529.67 ~ 562.43 kg) (12 V) DMF by spray balls into synthesis reactor (TJ4S-1213-Sy15). 通过淋洗球向合成釜（TJ4S-1213-Sy15）中加入546.05 kg（可接受称量范围：529.67 ~ 562.43 kg）（12 V）N,N-二甲基甲酰胺。 |  |  |  |  |  | Times/次数 |
|  |  |  |  |  |  | 1 |
| DMF addition/ 加N,N-二甲基甲酰胺 | Performed by & Date/执行人及日期： |  |  |  |  |  |
| DMF addition/ 加N,N-二甲基甲酰胺 | Begin/开始： |  | End/结束： |  | Qty/重量： |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |

| 3.25. （8）Deprotection/脱保护 |  |  |  |
| --- | --- | --- | --- |
| 3.25.9 Resin reswelling wash 2/溶胀后洗涤2 Turn on the nitrogen of synthesis(TJ4S-1213-Sy15) reactor to 5±2 m³/h (target: 5 m³/h) .Bubble with nitrogen for 5±2 min (target: 5 min), then start the agitator to 20±10 rpm (target: 20 rpm). After stirring for 10±2 min (target 10 min), stop stirring, turn off the nitrogen. Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point. 开启合成釜(TJ4S-1213-Sy15)下口鼓氮气, 氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min后（目标时间：5 min），开启搅拌转速至20±10 rpm (目标转速: 20 rpm)，搅拌10±2 min (目标时间：10 min)后，停止搅拌，关闭氮气。通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  | Times/次数 |
|  |  |  | 1 |
| Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h) for 5±2 min (target: 5 min), Start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm). Stir for 10±5 min (target: 10 min). After stirring for 10±2 min (target: 10 min), stop stirring, turn off the nitrogen./ 开启合成釜下口鼓氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min（目标时间：5 min）后，开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm)，搅拌10±2 min（目标时间:10 min）后，停止搅拌，关闭氮气 |  |  |  |
| Bubble/鼓氮气 | Performed by & Date/执行人及日期： |  |  |
| Bubble/鼓氮气 | Begin/开始： | End/结束： |  |
| Transcribed by & Date /转录人及日期： |  |  |  |
| Stir /搅拌 | Performed by & Date/执行人及日期： |  |  |
| Stir /搅拌 | Begin/开始： | End/结束： |  |
| Transcribed by & Date /转录人及日期： |  |  |  |
| Filter/过滤 | Performed by & Date/执行人及日期： |  |  |
|  | Checked by & Date/核实人及日期： |  |  |
|  | Begin/开始： | End/结束： |  |
| CAUTION/注意：DMF/N，N-二甲基甲酰胺：Irritation of eyes, skin and nose. May cause nausea. /刺激眼睛、皮肤和鼻子。可能引起恶心。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |

| - （8）Fmoc-L-Val-OH activation/芴甲氧羰基-L-缬氨酸活化 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 3.26.1 Add DMF to the activation reactor/激活罐加N,N-二甲基甲酰胺： Add 227.52 kg (Acceptable weighing range: 220.69 ~ 234.34 kg) (5 V) DMF into activation tank (TJ4S-1213-Sy15-R01). Start the agitator to 95±10 rpm (target: 95 rpm). 向激活罐（TJ4S-1213-Sy15-R01）中加入227.52 kg（可接受称量范围：220.69 ~ 234.34 kg ）（5 V）N，N-二甲基甲酰胺，开启搅拌转速至95±10 rpm (目标转速: 95 rpm)。 |  |  |  | Times/次数 |
|  |  |  |  | 1 |
| Add 227.52 kg (Acceptable weighing range: 220.69 ~ 234.34 kg) (5 V) DMF into activation tank (TJ4S-1213-Sy15-R01). Start the agitator to 95±10 rpm (target: 95 rpm)./向激活罐（TJ4S-1213-Sy15-R01）中加入227.52 kg（可接受称量范围：220.69 ~ 234.34 kg ）（5 V）N，N-二甲基甲酰胺。 |  |  |  |  |
| DMF addition/加N,N-二甲基甲酰胺 | Performed by & Date/执行人及日期： |  |  |  |
| DMF addition/加N,N-二甲基甲酰胺 | Begin/开始： | End/结束： | Qty/重量： |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |
| Start the agitator of activation tank to 95±10 rpm (target: 95 rpm)./ 开启激活罐搅拌转速至95±10 rpm (目标转速: 95 rpm)。 |  |  |  |  |
| Stir/搅拌 |  | Performed by & Date/执行人及日期： |  |  |
| Stir /搅拌 |  | Begin/开始： |  |  |
|  |  | The end time is documented in Ins 3.26.8/结束时间记录于3.26.8 |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |
| 3.26.2 Fmoc-L-Val-OH addition /加芴甲氧羰基-L-缬氨酸 Add Fmoc-L-Val-OH (2.00 eq.) with powder feeding bags into activation tank (TJ4S-1213-Sy15-R01). 通过投料袋向激活罐（TJ4S-1213-Sy15-R01）中加入 kg芴甲氧羰基-L-缬氨酸（Acceptable weighing range/可接受称量范围： kg ~ kg）(2.00 eq.)。 |  |  |  | Times/次数 |
|  |  |  |  | 1 |
| Before material charging, confirm the material is within the retest date/ the expiry date. Confirm whether the weight is adequate for use in the current charging. /加物料前，执行物料在复检日期/有效期之内，确认重量是否满足本次投料使用。 Before solid material charging, confirm/加固体物料前，确认： - Before operation performed, No non-related material / equipment / instrument / tools / pipeline / document / record / label in the intended area /操作前，目标区域无不相关物料/设备/仪器/工具/管路/文件/记录/标签； - The outer packages are removed. If the outer package cannot be removed, butunpack material without damaging the package bag, and before charging, check no package fragment and other extraneous matter in material. /外包装移除；如果外包装不能移除，但在不破坏包装的情况下打开包装袋，投料前以及过程中同时检查执行物料中无包装碎片以及异物 Confirmed by & Date/核实人及日期： |  |  |  |  |

| 3.26. （8）Fmoc-L-Val-OH activation/芴甲氧羰基-L-缬氨酸活化 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Prior to charging each amino acid, the operator shall perform a Raman scan to confirm the identity of the material. The Raman scan result shall be compared against the reference spectrum of the corresponding amino acid. Only materials that conform to the reference spectrum may be charged. If the Raman scan result does not conform, stop the operation, quarantine the material, and initiate a investigation per site SOP. Record the Raman scan result, operator signature, and date/time in the table below./ 每次投氨基酸前，操作人员应进行拉曼扫描以执行物料。拉曼扫描结果应与相应氨基酸的参考光谱进行比对。仅与参考光谱相符的物料方可投料。如拉曼扫描结果不相符，应停止操作，对物料进行隔离，并依据厂区 SOP 启动异常事件调查。在下表中记录拉曼扫描结果、操作人员签字及日期 / 时间。 |  |  |  |  |  |  |
| Fmoc-L-Val-OH / 加芴甲氧羰基-L-缬氨酸 |  |  | Performed by & Date/执行人及日期： |  |  |  |
|  |  |  | Begin/开始： |  | End/结束： |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |
| Fmoc-L-Val-OH confirmation/芴甲氧羰基-L-缬氨酸执行：Control No./内部控制号YL003110 |  |  |  |  |  |  |
| Check items/检查项 |  | Is it correct?/是否正确 |  | Recorded by & Date/ 记录人及日期 |  | Checked by & Date/ 核实人及日期 |
| Material name/物料名称 |  | Yes/是；No/否 |  |  |  |  |
| Project batch number/ 项目批次号 |  | Yes/是；No/否 |  |  |  |  |
| Operation number/ 操作号 |  | Yes/是；No/否 |  |  |  |  |
| Control No./内部控制号 |  | Yes/是；No/否 |  |  |  |  |
| Raman Scan Confirmation/拉曼扫描执行 |  | Conforms/相符 Does not conform/不相符 |  |  |  |  |
| No. /编号 | Net weight/净重 | All in/全部加入 |  | Recorded by & Date/ 记录人及日期 |  | Checked by & Date/ 核实人及日期 |
| / |  | Yes/是；No/否 |  |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |

| 3.26. （8）Fmoc-L-Val-OH activation/芴甲氧羰基-L-缬氨酸活化 |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No. /编号 | Net weight/净重 | All in/全部加入 |  |  |  | Recorded by & Date/ 记录人及日期 |  |  | Checked by & Date/ 核实人及日期 |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |  |  |  |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |  |  |  |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |  |  |  |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |  |  |  |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |  |  |  |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |  |  |  |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |  |  |  |  |  |  |
| Total/合计 |  | Confirm the total net weight is within the input range/执行合计净重在投料范围内 |  |  |  | Recorded by & Date/记录人及日期 |  |  | Checked by & Date/ 核实人及日期 |  |  |  |
|  |  | Yes/是；No/否 |  |  |  |  |  |  |  |  |  |  |
| After solid material charging, confirm/固体物料投料之后，确认： - The corresponding leftover material has been properly sealed and transferred to the staging area/相应的剩余物料已经恰当密封，转移到了暂存区域。 - The waste packaging material has been sealed and transferred to the corresponding waste staging area. /产生的包材废物已经密封并转移到了相应的废物暂存区域。 - The operation area/equipment surface has been cleaned/操作区域以及设备表面已经清洁。 Checked by & Date/核实人及日期： |  |  |  |  |  |  |  |  |  |  |  |  |
| 3.26.3 Agitation of activation tank /激活罐搅拌 Continue agitation of activation tank to 95±10 rpm (target: 95 rpm). Maintain stirring for at least 15 min. Confirm no visible solids and later use. 维持激活罐搅拌转速至95±10 rpm (目标转速: 95 rpm), 搅拌至少15 min，确认无明显固体待用。 |  |  |  |  |  |  |  |  |  |  |  |  |
| Stir /搅拌 |  |  | Performed by & Date/执行人及日期： |  |  |  |  |  |  |  |  |  |
| Stir /搅拌 |  |  | Begin/开始： |  |  |  |  |  |  |  |  |  |
|  |  |  | The end time is documented in Ins 3.26.8/结束时间记录于3.26.8 |  |  |  |  |  |  |  |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |  |  |  |  |  |
| Confirm no visible solid/确认无明显固体： |  |  |  |  | Confirmed by& Date /确认人及日期： |  |  |  |  |  |  |  |
| 3.26. （8）Fmoc-L-Val-OH activation/芴甲氧羰基-L-缬氨酸活化 |  |  |  |  |  |  |  |  |  |  |  |  |
| 3.26.4 Adjust activation tan temperature /激活罐调温 Adjust the activation tank (TJ4S-1213-Sy15-R01) mixture to 【20±5 ℃ (target 20 ℃) 】. /将激活罐(TJ4S-1213-Sy15-R01)中体系调温至【20±5 ℃（目标：20 ℃）】。 |  |  |  |  |  |  |  |  |  |  | Times/次数 |  |
|  |  |  |  |  |  |  |  |  |  |  | 1 |  |
| Adjust/调温 |  |  |  | Performed by & Date/执行人及日期： |  |  |  |  |  |  |  |
| Adjust/调温 |  |  |  | Begin/开始： |  |  | End/结束： |  |  |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |  |  |  |  |
| CAUTION/注意：OxymaPure is highly toxic, high-energy compound. /2-肟氰乙酸乙酯是高毒，高能化合物。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |  |  |  |  |  |  |
| 3.26.5 OxymaPure addition /加2-肟氰乙酸乙酯 Add OxymaPure(2.00 eq.) with powder feeding bags into activation tank (TJ4S-1213-Sy15-R01) at 20±5 ℃ (target 20 ℃). 控温20±5 ℃（目标：20 ℃），通过投料袋向激活罐（TJ4S-1213-Sy15-R01）中加入 kg 2-肟氰乙酸乙酯(2.00 eq.)（Acceptable weighing range/可接受称量范围： kg ~ kg） |  |  |  |  |  |  |  |  |  | Times/次数 |  |
|  |  |  |  |  |  |  |  |  |  | 1 |  |
| Check and record that the solid charge port is free of solid accumulation from the previous step./ 检查并记录固体投料口无来自上一步骤的固体残留。 Checked by & Date/核实人及日期： |  |  |  |  |  |  |  |  |  |  |  |
| Before material charging, confirm the material is within the retest date/ the expiry date,Confirm whether the weight is adequate for use in the current charging. /加物料前，执行物料在复检日期/有效期之内，确认重量是否满足本次投料使用。 Before solid material charging, confirm/加固体物料前，执行： - Before operation performed, No non-related material / equipment / instrument / tools / pipeline / document / record / label in the intended area /操作前，目标区域无不相关物料/设备/仪器/工具/管路/文件/记录/标签； - The outer packages are removed. If the outer package cannot be removed, but unpack material without damaging the package bag, and before charging, check no package fragment and other extraneous matter in material. /外包装移除；如果外包装不能移除，但在不破坏包装的情况下打开包装袋，投料前以及过程中同时检查执行物料中无包装碎片以及异物 Checked by & Date/核实人及日期： |  |  |  |  |  |  |  |  |  |  |  |
| Charge OxymaPure /加2-肟氰乙酸乙酯 |  |  |  | Performed by & Date/执行人及日期： |  |  |  |  |  |  |  |
| Charge OxymaPure /加2-肟氰乙酸乙酯 |  |  |  | Begin/开始： |  |  | End/结束： |  |  |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |  |  |  |  |
| 3.26. （8）Fmoc-L-Val-OH activation/芴甲氧羰基-L-缬氨酸活化 |  |  |  |  |  |  |  |  |  |  |  |  |
| OxymaPure confirmation/2-肟氰乙酸乙酯确认：Control No./内部控制号：YL005180 |  |  |  |  |  |  |  |  |  |  |  |
| Check items/检查项 |  | Is it correct?/是否正确 |  |  |  | Recorded by & Date/ 记录人及日期 |  | Checked by & Date/ 核实人及日期 |  |  |  |
| Material name/物料名称 |  | Yes/是；No/否 |  |  |  |  |  |  |  |  |  |
| Project batch number/ 项目批次号 |  | Yes/是；No/否 |  |  |  |  |  |  |  |  |  |
| Operation number/ 操作号 |  | Yes/是；No/否 |  |  |  |  |  |  |  |  |  |
| Control No./内部控制号 |  | Yes/是；No/否 |  |  |  |  |  |  |  |  |  |
| Raman Scan Confirmation/拉曼扫描执行 |  | Conforms/相符 Does not conform/不相符 |  |  |  |  |  |  |  |  |  |
| No. /编号 | Net weight/净重 | All in/全部加入 |  |  |  | Recorded by & Date/ 记录人及日期 |  | Checked by & Date/ 核实人及日期 |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |  |  |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |  |  |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |  |  |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |  |  |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |  |  |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |  |  |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |  |  |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |  |  |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |  |  |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |  |  |  |  |  |

| 3.26. （8）Fmoc-L-Val-OH activation/芴甲氧羰基-L-缬氨酸活化 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| No. /编号 | Net weight/净重 | All in/全部加入 |  | Recorded by & Date/ 记录人及日期 |  | Checked by & Date/ 核实人及日期 |
| / |  | Yes/是；No/否 |  |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |
| Total/合计 |  | Confirm the total net weight is within the input range/执行合计净重在投料范围内 |  | Recorded by & Date/记录人及日期 |  | Checked by & Date/ 核实人及日期 |
|  |  | Yes/是；No/否 |  |  |  |  |
| After solid material charging, confirm/固体物料投料之后，确认： - The corresponding leftover material has been properly sealed and transferred to the staging area/相应的剩余物料已经恰当密封，转移到了暂存区域。 - The waste packaging material has been sealed and transferred to the corresponding waste staging area. /产生的包材废物已经密封并转移到了相应的废物暂存区域。 - The operation area/equipment surface has been cleaned/操作区域以及设备表面已经清洁。 Checked by & Date/核实人及日期： |  |  |  |  |  |  |
| Internal temperature/内部温度 |  |  | High values/最高值： |  | Low values/最低值： |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |

| 3.26. （8）Fmoc-L-Val-OH activation/芴甲氧羰基-L-缬氨酸活化 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| CAUTION/注意：DMF/N，N-二甲基甲酰胺：Irritation of eyes, skin and nose. May cause nausea. /刺激眼睛、皮肤和鼻子。可能引起恶心。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |  |
| 3.26.6 Rinse activation tank /激活罐淋洗 Rinse the port of Activation tank (TJ4S-1213-Sy15-R01) with 91.01 kg（Acceptable weighing range：88.28 ~ 93.74 kg）（2 V）DMF. 用91.01 kg（可接受称量范围：88.28 ~ 93.74 kg）（2 V）N,N-二甲基甲酰胺淋洗激活罐（TJ4S-1213-Sy15-R01）加料口。 |  |  |  |  | Times/次数 |  |
|  |  |  |  |  | 1 |  |
| Rinse with DMF/ N,N-二甲基甲酰胺淋洗 | Performed by & Date/执行人及日期： |  |  |  |  |  |
| Rinse with DMF/ N,N-二甲基甲酰胺淋洗 | Begin/开始： | End/结束： |  | Qty/重量： |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |
| Stir /搅拌 | Performed by & Date/执行人及日期： |  |  |  |  |  |
| Stir /搅拌 | Begin/开始： |  | End/结束： |  |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |
| CAUTION/注意：DIC is a key EHS concern chemical, highly toxic, skin sensitizing, respiratory sensitizing. /N,N’二异丙基碳二亚胺为EHS重点关注化学品，高毒，皮肤致敏，呼吸致敏。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |  |
| 3.26.7 DIC addition /加N,N'-二异丙基碳二亚胺 Add DIC (2.20 eq.) into the activation tank (TJ4S-1213-Sy15-R01) at reference time of 10-30 min at 20±5 ℃ (target: 20 ℃). The addition rate depends on temperature control.控温20±5 ℃ (目标20 ℃)，向激活罐（TJ4S-1213-Sy15-R01）中滴加 kg N,N'-二异丙基碳二亚胺（Acceptable weighing range/可接受称量范围： kg ~ kg）(2.20 eq.)（滴加参考时间10-30 min，以控温为主）。 Pump number/泵编号： Scale ID. No./秤编号： Material charging pipe number/物料管路编号： |  |  |  |  |  | Times/次数 |
|  |  |  |  |  |  | 1 |

| 3.26. （8）Fmoc-L-Val-OH activation/芴甲氧羰基-L-缬氨酸活化 |  |
| --- | --- |
| ★Note:The DIC addition temperature is a CPP./ 备注：N,N'-二异丙基碳二亚胺加料温度是关键控制参数。 Prior to DIC addition, the operator shall confirm that the reaction mixture temperature is within the Normal Operating Range (NOR) / N,N'-二异丙基碳二亚胺投料前，操作人员须执行反应体系温度处于正常操作范围（NOR）内: ★Lower limit/低限: 15oC ，High limit/高限: 25oC Over-temperature warning value/超温预警设置数值： Lower limit/低限: 17oC ，High limit/高限: 23oC |  |
| Before material charging, confirm the material is within the retest date/ the expiry date, Confirm whether the weight is adequate for use in the current charging. /加物料前，执行物料在复检日期/有效期之内，确认重量是否满足本次投料使用。 Performed by & Date/执行人及日期： |  |
| Before drummed liquid material charging, visual check there is no other extraneous matter in material. /加桶装液体物料前，应目视执行桶内无异物。 Visual Examination of Foreign Matters for Drum Solvent/桶装溶剂异物目视检查: 1. Place the solvent drum in a well-lit area (or use a portable light source) to ensure adequate visibility of the liquid contents./ 将溶剂桶置于光线充足区域（或使用便携式光源），确保液体内容物可见性良好 2. Visually inspect the solvent through the drum bung opening or transparent sight glass for any visible foreign matters, including but not limited to particulates, fibers, sediments, or unusual turbidity./ 通过桶口或透明视镜目视检查溶剂中是否有可见异物，包括但不限于颗粒、纤维、沉淀物或异常浑浊。 3. Confirm that the solvent is clear and free of visible foreign matters before charging. If any foreign matter is observed, do not use the drum and follow the applicable handling procedure for subsequent disposition./ 投料前确认溶剂澄清且无可见异物。如发现任何异物，不得使用该桶，并依据处理流程进行后续处置。 Checked by & Date/核实人及日期： |  |

| 3.26. （8）Fmoc-L-Val-OH activation/芴甲氧羰基-L-缬氨酸活化 |  |  |
| --- | --- | --- |
| Confirm liquid charging pipeline is released. /确认加料管路经过放行。 |  |  |
| Checked by & Date/核实人及日期： |  |  |
| Charging of drummed solvent: verify that the pipeline connections remain consistent before and after material feeding, then record the weighing reading 桶装溶剂溶液称量时，确认投料前后管路连接一致，读取称量示数。 Performed by & Date/执行人及日期： Checked by & Date/核实人及日期： |  |  |
| DIC /N,N’-二异丙基碳二亚胺：Control Number/内部控制号：YL003564 |  |  |
| Internal temperature/内部温度 | High values/最高值： | Low values/最低值： |
| Transcribed by & Date /转录人及日期： |  |  |
| If the target net weight≥the planned net weight, perform full drum charge; otherwise, perform partial (Non-full drum) charge. /当目标加料量≥计划加入净重，勾选进行整桶加料；否则勾选进行零头加料。 Note: Target net weight to be charged = Total target net weight – Total net weight charged. Expected tare weight of drum after charge = Gross weight of drum that to be charged - Target net weight to be charged. 还需加入净重目标值= 加料步骤目标总净重-已加入净重总量 加料完成后预计桶皮重=待加料桶毛重-还需加入净重目标值 |  |  |

| 3.26. （8）Fmoc-L-Val-OH activation/芴甲氧羰基-L-缬氨酸活化 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Addition type加料式 | Lot number/批号 | Pre-charging qty calculation/投料前投料量计算 |  |  |  |  |  | Actual Charging Qty/实际投料量 |  |  |  |  |  |
|  |  | Total qty charged/已加入净重总量 |  | Target qty to be charged/还需加入净重目标值 | Gross qty of drum that to be charged/待加料桶毛重 | Expected net weight of drum after charge /加料完成后预计桶净重 | Expected tare weight of drum after charge/加料完成后预计桶皮重 | Before charging/ 投料前 |  | After charging/ 投料后 | Net weight/ 净重 | Performed by & Date/ 执行人及日期 | Checked by & Date/ 核实人及日期 |
| Full drum整桶 Non-full drum /非整桶 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Full drum整桶 Non-full drum /非整桶 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Total/合计 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DIC Addition/加N,N'-二异丙基碳二亚胺 |  |  | Begin/开始： |  |  |  |  |  | End/结束： |  |  |  |  |
| Transcribed by & Date/转录人及日期： |  |  |  |  |  |  |  |  |  |  |  |  |  |

| 3.26. （8）Fmoc-L-Val-OH activation/芴甲氧羰基-L-缬氨酸活化 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 3.26.8 Fmoc-L-Val-OH activation /芴甲氧羰基-L-缬氨酸活化 At 20±5 ℃ (target: 20 ℃). Continue agitation of activation tank to 95±10 rpm (target: 95 rpm). Maintain for NLT 15 min after addition（Target：20 min）./滴毕，控温20±5 ℃(目标温度：20 ℃)，维持激活罐搅拌转速至95±10 rpm (目标转速: 95 rpm),继续搅拌至少15 min（目标时间：20 min）。 |  |  |  |  |  |  |  |
| Stir /搅拌 |  | Performed by & Date/执行人及日期： |  |  |  |  |  |
| Stir /搅拌 |  | End/结束： |  |  |  |  |  |
|  |  | The start time is documented in Ins3.26.1 /开始时间记录于3.26.1 |  |  |  |  |  |
| Transcribed by & Date /转录人及日期: |  |  |  |  |  |  |  |
| Internal temperature/内部温度 |  |  | High values/最高值： |  | Low values/最低值： |  |  |
| Transcribed by & Date/转录人及日期： |  |  |  |  |  |  |  |
| - （8）Fmoc-L-Val-OH coupling/芴甲氧羰基-L-缬氨酸偶联 |  |  |  |  |  |  |  |
| 3.27.1 Activation mixture transfer /活化体系转移 Transfer the mixture in activation tank (TJ4S-1213-Sy15-R01) into synthesis reactor (TJ4S-1213-Sy15). 将激活罐（TJ4S-1213-Sy15-R01）中体系转移至合成釜（TJ4S-1213-Sy15）中。 |  |  |  |  |  | Times/次数 |  |
|  |  |  |  |  |  | 1 |  |
| ★Note:The coupling temperature is a CPP, and the coupling IPC hold temperature is a CPP./ 备注：偶联温度是CPP，偶联IPC维持温度是关键工艺参数。 The operator shall confirm that the reaction mixture temperature is within the Normal Operating Range (NOR) 【25±5oC (target: 25oC)】 /操作人员须执行反应体系温度处于正常操作范围（NOR）内【25±5oC (target: 25oC)】 Checked by & Date/核实人及日期： Over-temperature warning value/超温预警设置数值： Lower limit/低限: 21 oC ，High limit/高限: 28 oC |  |  |  |  |  |  |  |
| Transfer/转移 |  | Performed by & Date/执行人及日期： |  |  |  |  |  |
|  |  | Checked by & Date/核实人及日期： |  |  |  |  |  |
|  |  | Begin/开始： |  | End/结束： |  |  |  |
| CAUTION/注意：DMF/N，N-二甲基甲酰胺：Irritation of eyes, skin and nose. May cause nausea. /刺激眼睛、皮肤和鼻子。可能引起恶心。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |  |  |
| 3.27.2 DMF addition /加N,N-二甲基甲酰胺 Add 45.50 kg (Acceptable weighing range: 43.50 ~ 47.50 kg) (1 V) DMF by spray balls into activation tank (TJ4S-1213-Sy15-R01) to rinse the wall. 通过淋洗球向激活罐（TJ4S-1213-Sy15-R01）中加入45.50 kg（可接受称量范围：43.50 ~ 47.50 kg） (1 V) N,N-二甲基甲酰胺淋洗罐壁。 |  |  |  |  |  | Times/次数 |  |
|  |  |  |  |  |  | 1 |  |

| 3.27. （8）Fmoc-L-Val-OH coupling/芴甲氧羰基-L-缬氨酸偶联 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Rinse with DMF/ N,N-二甲基甲酰胺淋洗 |  |  | Performed by & Date/执行人及日期： |  |  |  |  |  |  |  |
| Rinse with DMF/ N,N-二甲基甲酰胺淋洗 |  |  | Begin/开始： |  | End/结束： |  | Qty/重量： |  |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |  |  |  |
| 3.27.3 Transfer /转移 Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h). Bubble with nitrogen for 5±2 min (target: 5 min), then start the agitator. Transfer the mixture in activation tank (TJ4S-1213-Sy15-R01) into synthesis reactor (TJ4S-1213-Sy15). 合成釜（TJ4S-1213-Sy15）开启下口氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min (目标：5 min)后，开启搅拌转速至20±10 rpm (目标转速: 20 rpm)，将激活罐（TJ4S-1213-Sy15-R01）中体系转移至合成釜（TJ4S-1213-Sy15）中。 |  |  |  |  |  |  |  |  | Times/次数 |  |
|  |  |  |  |  |  |  |  |  | 1 |  |
| Bubble/鼓氮气 |  |  |  |  | Performed by & Date/执行人及日期： |  |  |  |  |  |
| Bubble/鼓氮气 |  |  |  |  | Begin/开始： |  |  |  |  |  |
|  |  |  |  |  | The end time is documented in Ins3.27.5 /结束时间记录于3.27.5 |  |  |  |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |  |  |  |
| Stir/搅拌 |  |  |  |  | Performed by & Date/执行人及日期： |  |  |  |  |  |
| Stir /搅拌 |  |  |  |  | Begin/开始： |  |  |  |  |  |
|  |  |  |  |  | The end time is documented in Ins3.27.5 /结束时间记录于3.27.5 |  |  |  |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |  |  |  |
| Transfer the mixture(DMF) from the activation tank (TJ4S-1213-Sy15-R01) to the synthesis reactor (TJ4S-1213-Sy15). /激活罐(TJ4S-1213-Sy15-R01)中体系转移至合成釜(TJ4S-1213-Sy15)中。 |  |  |  |  |  |  |  |  |  |  |
| Transfer/转移 |  |  | Performed by & Date/执行人及日期： |  |  |  |  |  |  |  |
|  |  |  | Checked by & Date/核实人及日期： |  |  |  |  |  |  |  |
|  |  |  | Begin/开始： |  |  |  | End/结束： |  |  |  |
| 3.27.4 Adjust synthesis reactor temperature /合成釜调温 Adjust the mixture in synthesis reactor (TJ4S-1213-Sy15) to【 25±5 oC (target: 25 oC)】. 合成釜(TJ4S-1213-Sy15)体系调温至【25±5 ℃(目标温度：25 ℃)】。 |  |  |  |  |  |  |  |  | Times/次数 |  |
|  |  |  |  |  |  |  |  |  | 1 |  |

| 3.27. （8）Fmoc-L-Val-OH coupling/芴甲氧羰基-L-缬氨酸偶联 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The total reaction Fmoc-L-Val-OH coupling time：NMT 840 min (Sourced from the holding time study in the PAR study report.)/ 芴甲氧羰基-L-缬氨酸偶联总的反应时间不超过840min(来源于 PAR 研究报告中稳定时间研究)。 Over-temperature warning value/超温预警设置数值： Lower limit/低限: 21 oC ，High limit/高限: 28 oC |  |  |  |  |  |
| Adjust/调温 |  | Performed by & Date/执行人及日期： |  |  |  |
| Adjust/调温 |  | Begin/开始： | End/结束： |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |
| 3.27.5 Fmoc-L-Val-OH coupling/芴甲氧羰基-L-缬氨酸偶联 The mixture is allowed to react at 25±5 ℃ (target: 25 ℃). After stirring for 3 h±15 min (target: 3 h), then sample the mixture every 0.5~1 h for ninhydrin test analysis until two consecutive samples test as negative (Yellow to Light pink Report Negative= Complete coupling; Blue to Purple Positive= Incomplete coupling). And the mixture should be stirred for coupling time (4 h≤Coupling Time≤10 h). Stop stirring, turn off the nitrogen. Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point. 控温25±5 ℃(目标：25 ℃)反应，反应3 h±15 min（目标：3 h）后，然后取样进行茚三酮检测，每隔0.5~1.0 h取样一次，直至连续两次取样结果为阴性（黄色至浅粉色，报告阴性=偶联完全；蓝色至紫色，阳性=偶联不完全）后，且反应时间满足（ 4h≤反应时间≤10 h）后，停止搅拌，关闭氮气。通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 Note: If resin adherence is found at other times during the reaction, rinse the reactor with 0.50~5.00 kg DMF each time. /备注：偶联过程中其它时间发现贴壁，每次使用0.50~5.00 kg N,N-二甲基甲酰胺淋洗，记录见附加页 |  |  |  | Times/次数 |
|  |  |  |  | 1 |

| 3.27. （8）Fmoc-L-Val-OH coupling/芴甲氧羰基-L-缬氨酸偶联 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| React/反应 |  |  | Performed by & Date/执行人及日期： |  |  |  |  |  |
| ★After coupling is completed, the operator shall confirm that all Critical Process Parameters (CPP) are within their respective Normal Operating Ranges (NOR)/ 偶联结束后，操作人员须确认所有关键工艺参数（CPP）均处于各自的正常操作范围（NOR）内 ★Note: The coupling temperature is a CPP, and the coupling IPC hold temperature is a CPP./备注：偶联温度是CPP，偶联IPC维持温度是关键工艺参数。 The operator shall confirm that the reaction mixture temperature is within the Normal Operating Range (NOR) :20~30℃ /操作人员须确认反应体系温度处于正常操作范围（NOR）内:20~30℃ □Yes/是 □No/否 Confirmed by & Date/确认人及日期： Checked by & Date/核实人及日期： |  |  |  |  |  |  |  |  |
| React/反应 |  |  | Begin/开始： |  |  | End/结束： |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |  |
| Standard sampling method/标准取样方法 |  |  |  |  |  |  |  |  |
| Take 5-10 ml mixture and perform testing in accordance with the Kaiser test record. During sampling, should wear nitrile gloves and 3M 6000 series Full-face piece respirator, 6006CN cartridges. 通过在线取样器取样5~10 ml按照茚三酮检测记录进行检测。取样过程中，应该戴丁腈手套、3M 6000系列全面罩, 6006CN滤毒盒。 |  |  |  |  |  |  |  |  |
| Sampling number/ 取样编号 | Sampling time/取样时间 |  | Sampled by & Date/取样人及日期 | Negative or Positive/阳性或阴性 |  | Result filled by &Date/结果填写人及日期 | Checked by & Date/核实人及日期 |  |
| CPo140020- -01- -3.27.5-01 |  |  |  | Negative /阴性 Positive /阳性 |  |  |  |  |
| CPo140020- -01- -3.27.5-02 |  |  |  | Negative /阴性 Positive /阳性 |  |  |  |  |
| CPo140020- -01- -3.27.5-03 |  |  |  | Negative /阴性 Positive /阳性 |  |  |  |  |
| CPo140020- -01- -3.27.5-04 |  |  |  | Negative /阴性 Positive /阳性 |  |  |  |  |

| 3.27. （8）Fmoc-L-Val-OH coupling/芴甲氧羰基-L-缬氨酸偶联 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sampling number/ 取样编号 | Sampling time/取样时间 |  |  | Sampled by & Date/取样人及日期 | Negative or Positive/阳性或阴性 |  |  | Result filled by &Date/结果填写人及日期 | Checked by & Date/核实人及日期 |  |
| CPo140020- -01- -3.27.5-05 |  |  |  |  | Negative /阴性 Positive /阳性 |  |  |  |  |  |
| CPo140020- -01- -3.27.5-06 |  |  |  |  | Negative /阴性 Positive /阳性 |  |  |  |  |  |
| CPo140020- -01- -3.27.5-07 |  |  |  |  | Negative /阴性 Positive /阳性 |  |  |  |  |  |
| CPo140020- -01- -3.27.5-08 |  |  |  |  | Negative /阴性 Positive /阳性 |  |  |  |  |  |
| Confirm the end point of the reaction. /执行反应终点。 The acceptance criterion is two consecutive negative test results./连续两次检测结果为阴性即为合格。 Confirmed by & Date/确认人及日期： Checked by & Date/核实人及日期： |  |  |  |  |  |  |  |  |  |  |
| Stir/搅拌 |  |  |  | Performed by & Date/执行人及日期： |  |  |  |  |  |  |  |
| Stir /搅拌 |  |  |  | End/结束： |  |  |  |  |  |  |  |
|  |  |  |  | The start time is documented in Ins 3.27.3 /开始时间记录于3.27.3 |  |  |  |  |  |  |  |
| Transcribed by & Date/转录人及日期： |  |  |  |  |  |  |  |  |  |  |  |
| Bubble/鼓氮气 |  |  |  | Performed by & Date/执行人及日期： |  |  |  |  |  |  |  |
| Bubble/鼓氮气 |  |  |  | End/结束： |  |  |  |  |  |  |  |
|  |  |  |  | The start time is documented in Ins 3.27.3 /开始时间记录于3.27.3 |  |  |  |  |  |  |  |
| Transcribed by & Date/转录人及日期： |  |  |  |  |  |  |  |  |  |  |  |
| Internal temperature/内部温度 |  |  | High values/最高值： |  |  |  | Low values/最低值： |  |  |  |  |
| Transcribed by & Date/转录人及日期： |  |  |  |  |  |  |  |  |  |  |  |
| Filter/过滤 |  |  | Performed by & Date/执行人及日期： |  |  |  |  |  |  |  |
|  |  |  | Checked by & Date/核实人及日期： |  |  |  |  |  |  |  |
|  |  |  | Begin/开始： |  |  |  | End/结束： |  |  |  |
| ★After coupling is completed, the operator shall confirm that all Critical Process Parameters (CPPs) are within their respective Normal Operating Ranges (NOR)/ 偶联结束后，操作人员须确认所有关键工艺参数（CPP）均处于各自的正常操作范围（NOR）内 Checked by & Date/核实人及日期： |  |  |  |  |  |  |  |  |  |  |

| 3.27. （8）Fmoc-L-Val-OH coupling/芴甲氧羰基-L-缬氨酸偶联 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| CAUTION/注意：DMF/N，N-二甲基甲酰胺：Irritation of eyes, skin and nose. May cause nausea. /刺激眼睛、皮肤和鼻子。可能引起恶心。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |
| 3.27.6 Activation Tank Rinse/激活罐洗涤 Add 364.03 kg (Acceptable weighing range：353.11 ~ 374.95 kg) (8 V) DMF into the activation tank (TJ4S-1213-Sy15-R01). Circulate and clean the activating liquid transfer pipeline, liquid charging port cleaning pipeline and solid charging port cleaning pipeline. Clean every line wash for 2±1 min (target: 2 min). 向激活罐（TJ4S-1213-Sy15-R01）中加入364.03 kg（可接受称量范围：353.11 ~ 374.95 kg） (8 V) N,N-二甲基甲酰胺，循环清洗激活罐转移管路，液体加料口清洗管路和固体加料口清洗管路，每条线路各清洗2±1 min（目标：2 min）。 |  |  |  |  | Times/次数 |
|  |  |  |  |  | 3 |
| Operation name/ 操作名称 | Time point/时间节点 | Clean the activating liquid transfer pipeline 1/循环清洗激活液转移管路1 | Liquid charging port cleaning pipeline 2 /液体加料口清洗管路2 | Solid charging port cleaning pipeline 3,/固体加料口清洗管路3 |  |
|  |  | Wash /洗涤1 | Wash /洗涤2 | Wash /洗涤3 |  |
| DMF addition/ 加N,N-二甲基甲酰胺 | Performed by & Date/执行人及日期： |  |  |  |  |
| DMF addition/ 加N,N-二甲基甲酰胺 | Begin/开始 |  |  |  |  |
|  | End/结束 |  |  |  |  |
|  | Qty/重量 |  |  |  |  |
|  | Transcribed by & Date /转录人及日期： |  |  |  |  |
| Circulate and clean/循环清洗 | Performed by & Date/执行人及日期： |  |  |  |  |
| Circulate and clean/循环清洗 | Begin/开始 |  |  |  |  |
|  | End/结束 |  |  |  |  |
|  | Transcribed by & Date /转录人及日期： |  |  |  |  |

| 3.27. （8）Fmoc-L-Val-OH coupling/芴甲氧羰基-L-缬氨酸偶联 |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3.27.7 Post-coupling wash./偶联后洗涤 Transfer the mixture in activation tank (TJ4S-1213-Sy15-R01) to synthesis reactor (TJ4S-1213-Sy15). Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h).Bubble with nitrogen for 5±2 min (target 5 min), start the agitator. After stirring for 7±2 min (target：7 min), stop stirring, turn off the nitrogen. Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point. 将激活罐（TJ4S-1213-Sy15-R01）洗涤液转入合成釜 (TJ4S-1213-Sy15)。开启合成釜下口鼓氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min (目标时间：5 min)后开启搅拌转速至20±10 rpm (目标转速: 20 rpm)，搅拌7±2 min（目标时间：7 min）后停止搅拌，关闭氮气。通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  |  |  |  |  |  |  |  | Times/次数 |  |
|  |  |  |  |  |  |  |  |  |  | 3 |  |
| After the liquid is filtered out of the synthesis reactor (TJ4S-1213-Sy15) and transfer the liquid to the synthesis reactor (TJ4S-1213-Sy15) ./ 待合成釜排废完毕后将激活罐中体系转移至合成釜中。 |  |  |  |  |  |  |  |  |  |  |  |
| Operation name/ 操作名称 |  |  | Time point/时间节点 |  | Wash /洗涤1 |  | Wash /洗涤2 |  | Wash /洗涤3 |  |  |
| Transfer/转移 |  |  | Performed by & Date/执行人及日期： |  |  |  |  |  |  |  |  |
|  |  |  | Checked by & Date/核实人及日期： |  |  |  |  |  |  |  |  |
|  |  |  | Begin/开始 |  |  |  |  |  |  |  |  |
|  |  |  | End/结束 |  |  |  |  |  |  |  |  |
| Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h). Bubble with nitrogen for 5±2 min (target 5 min), start the agitator. After stirring for 7±2 min (target：7 min), stop stirring, turn off the nitrogen.开启合成釜下口鼓氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min (目标：5 min)后开启搅拌转速至20±10 rpm (目标转速: 20 rpm)，搅拌7±2 min（目标：7 min）后停止搅拌，关闭氮气。 |  |  |  |  |  |  |  |  |  |  |  |
| Bubble/鼓氮气 |  |  | Performed by & Date/执行人及日期： |  |  |  |  |  |  |  |  |
| Bubble/鼓氮气 |  |  | Begin/开始 |  |  |  |  |  |  |  |  |
|  |  |  | End/结束 |  |  |  |  |  |  |  |  |
|  |  |  | Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |  |

| 3.27. （8）Fmoc-L-Val-OH coupling/芴甲氧羰基-L-缬氨酸偶联 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Operation name/ 操作名称 | Time point/时间节点 | Wash /洗涤1 | Wash /洗涤2 | Wash /洗涤3 |
| Stir /搅拌 | Performed by & Date/执行人及日期： |  |  |  |
| Stir /搅拌 | Begin/开始 |  |  |  |
|  | End/结束 |  |  |  |
|  | Transcribed by & Date /转录人及日期： |  |  |  |
| Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point/通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  |  |  |
| Filter/过滤 | Performed by & Date/执行人及日期： |  |  |  |
|  | Checked by & Date/核实人及日期： |  |  |  |
|  | Begin/开始 |  |  |  |
|  | End/结束 |  |  |  |

| - （9）Deprotection/脱保护 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CAUTION/注意：Piperidine哌啶：Strong local irritant and may cause permanent injury after short exposure to small amounts. /强烈的局部刺激，短时间少量接触可能造成永久性伤害。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |  |
| 3.28.1 Add piperidine solution /加哌啶溶液 Adjust the jacket temperature of synthesis reactor (TJ4S-1213-Sy15) to 【25±5 oC (target: 25 oC)】. Add 357.50 kg 【20% (V/V) piperidine/DMF(8 V)】 (Acceptable weighing range: 346.77 ~ 368.23 kg) solution into the Synthesis reactor (TJ4S-1213-Sy15). 将合成釜(TJ4S-1213-Sy15)夹套调温至【25±5 ℃（目标温度：25 ℃）】。向合成釜（TJ4S-1213-Sy15）中加入357.50 kg【20%哌啶/DMF (V/V)溶液(8 V)】（可接受称量范围：346.77 ~ 368.23 kg）。 |  |  |  |  |  | Times/次数 |
|  |  |  |  |  |  | 1 |
| Adjust/调温 |  | Performed by & Date/执行人及日期： |  |  |  |  |
| Adjust/调温 |  | Begin/开始： |  | Adjust/调温 |  |  |
| Transcribed by & Date/转录人及日期： |  |  |  |  |  |  |
| Confirm and record that the 20%Piperidine solution concentration is within the NOR range./ 确认并记录20%哌啶溶液浓度在正常操作范围内：17.50%~22.50% 20%Piperidine solution concentration confirmation/20%哌啶溶液浓度确认：____________ Confirm and record that the 20%Piperidine solution temperature is within the NOR range./ 确认并记录20%哌啶溶液温度在正常操作范围内：20 oC ~30 oC 20%Piperidine solution temperature confirmation/20%哌啶溶液温度确认：____________ Confirmed by & Date/确认人及日期： Checked by & Date/核实人及日期： |  |  |  |  |  |  |
| Piperidine addition/ 加哌啶溶液 | Performed by & Date/执行人及日期： |  |  |  |  |  |
| Piperidine addition/ 加哌啶溶液 | Begin/开始： |  | End/结束： |  | Qty/重量： |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |

| 3.28. （9）Deprotection/脱保护 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 3.28.2 Adjust synthesis reactor temperature /合成釜调温 Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h) for 5±2 min (target 5 min), Start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm). Stir for 10±5 min (target: 10 min). Adjust to 【25±5 ℃ (target: 25 ℃)】. 开启合成釜下口鼓氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min (目标时间：5 min)后开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm)，搅拌10min±5 min (目标时间: 10 min)。调温至【25±5 ℃(目标温度：25 ℃)】。 |  |  |  |  | Times/次数 |
|  |  |  |  |  | 1 |
| Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h). Bubble with nitrogen for 5±2 min (target :5 min) 开启合成釜下口鼓氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min (目标时间：5 min)。 |  |  |  |  |  |
| Bubble/鼓氮气 |  | Performed by & Date/执行人及日期： |  |  |  |
| Bubble/鼓氮气 |  | Begin/开始： |  |  |  |
|  |  | The end time is documented in Ins 3.28.3/结束时间记录于3.28.3 |  |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |
| Start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm). 开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm) |  |  |  |  |  |
| Stir /搅拌 |  |  | Performed by & Date/执行人及日期： |  |  |
| Stir /搅拌 |  |  | Begin/开始： |  |  |
|  |  |  | The end time is documented in Ins 3.28.3/结束时间记录于3.28.3 |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |
| Adjust to 25±5 oC (target: 25 oC)./ 调温至25±5 ℃(目标温度：25 oC) |  |  |  |  |  |
| Adjust/调温 |  | Performed by & Date/执行人及日期： |  |  |  |
| Adjust/调温 |  | Begin/开始： |  | End/结束： |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |

| 3.28. （9）Deprotection/脱保护 |  |  |
| --- | --- | --- |
| 3.28.3 Fmoc-L-Leu-OH De-Fmoc /芴甲氧羰基-L-亮氨酸脱保护 After stirring for【2 h±15 min (target: 2 h) at 25±5 ℃ (target: 25 ℃)】, stop stirring, turn off the nitrogen. Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point. 控温【25±5 ℃(目标：25 ℃)，搅拌2 h±15 min（目标：2 h）】后，停止搅拌，关闭氮气。通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 Over-temperature warning value/超温预警设置数值： Lower limit/低限: 22 oC ，High limit/高限: 28 oC |  | Times/次数 |
|  |  | 1 |
| ★Confirm and record that the 20%Piperidine solution temperature is within the NOR range 20 oC ~30 oC ./ 确认并记录20%哌啶溶液温度在正常操作范围内：20 oC ~30 oC。 20%Piperidine solution temperature confirmation/20%哌啶溶液温度确认：____________ Confirmed by& Date/确认人及日期： Checked by & Date/核实人及日期： |  |  |
| De-Fmoc/脱Fmoc &Stir /搅拌 | Performed by & Date/执行人及日期： |  |
| De-Fmoc/脱Fmoc &Stir /搅拌 | End/结束： |  |
|  | The start time is documented in Ins3.28.2 /开始时间记录于3.28.2 |  |
| Transcribed by & Date /转录人及日期： |  |  |

| 3.28. （9）Deprotection/脱保护 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Bubble/鼓氮气 |  |  | Performed by & Date/执行人及日期： |  |  |  |  |
| Bubble/鼓氮气 |  |  | End/结束： |  |  |  |  |
|  |  |  | The start time is documented in Ins3.29.2 /开始时间记录于3.29.2 |  |  |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |
| Internal temperature/内部温度 |  |  | High values/最高值: |  | Low values/最低值： |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |
| Filter/过滤 |  |  | Performed by & Date/执行人及日期： |  |  |  |  |
|  |  |  | Checked by & Date/核实人及日期： |  |  |  |  |
|  |  |  | Begin/开始： |  | End/结束： |  |  |
| ★After De Fmoc is completed, the operator shall confirm that all Critical Process Parameters (CPPs) are within their respective Normal Operating Ranges (NOR)/ 脱保护结束后，操作人员须确认所有关键工艺参数（CPP）均处于各自的正常操作范围（NOR）内 Checked by & Date/核实人及日期： |  |  |  |  |  |  |  |
| 3.28.4 Adjust synthesis reactor temperature /合成釜调温 Adjust the synthesis reactor (TJ4S-1213-Sy15) jacket to【20±5 ℃ (target: 20 ℃)】. /将合成釜（TJ4S-1213-Sy15）夹套调温至【20±5 ℃（目标温度：20 ℃）】。 |  |  |  |  | Times/次数 |  |  |
|  |  |  |  |  | 1 |  |  |
| Adjust/调温 |  |  | Performed by & Date/执行人及日期： |  |  |  |  |  |
| Adjust/调温 |  |  | Begin/开始： |  | End/结束： |  |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |  |
| CAUTION/注意：IPAC/乙酸异丙酯： Vapors irritate eyes and respiratory tract; high concentrations can be anesthetic. Liquid irritates eyes but causes no serious injury; may cause dermatitis; no serious effects if swallowed. /蒸汽会刺激眼睛和呼吸道；高浓度的蒸汽可能是麻醉剂。液体刺激眼睛，但不会造成严重伤害；可能会引起皮炎；吞食不会造成严重影响。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |  |  |

| 3.28. （9）Deprotection/脱保护 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 3.28.5 Post-deprotection wash/脱保护后洗涤 Add 445.92 kg (Acceptable weighing range：432.54 ~ 459.30 kg) (9.29 kg/kg, 10.64 V) isopropyl acetate into synthesis reactor (TJ4S-1213-Sy15). Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h). Bubble with nitrogen for 5±2 min (target 5 min), then start the agitator to 20±10 rpm (target: 20 rpm). After stirring for 5±2 min (target 5 min), stop stirring, turn off the nitrogen. Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point. 向合成釜（TJ4S-1213-Sy15）中加入445.92 kg（可接受称量范围：432.54 ~ 459.30 kg) (9.29 kg/kg, 10.64 V）乙酸异丙酯，开启下口氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min (目标：5 min)后开启搅拌转速至20±10 rpm (目标转速: 20 rpm)，搅拌5±2 min (目标：5 min)后，停止搅拌，关闭氮气。通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  |  |  |  | Times/次数 |
|  |  |  |  |  |  | 4 |
| ★Note: IPAC quantity is a CPP./备注：乙酸异丙酯用量是关键工艺参数。 ★The operator shall confirm that IPAC quantity is within the Normal Operating Range (NOR) 【432.54 kg~459.30 kg】/操作人员须确认乙酸异丙酯用量处于正常操作范围（NOR）内：【432.54 kg~459.30 kg】 Checked by & Date/核实人及日期： |  |  |  |  |  |  |
| Operation name/操作名称 | Time point/时间节点 | Wash /洗涤1 | Wash /洗涤2 | Wash /洗涤3 | Wash /洗涤4 |  |
| Add 419.04 kg IPAc (10.0 V) by spray balls into the synthesis reactor (TJ4S-1213-Sy15)./ 向合成釜(TJ4S-1213-Sy15) 中加419.04 kg乙酸异丙酯（（10.0 V）（Acceptable weighing range/可接受称量范围：440.34~467.58 kg）） |  |  |  |  |  |  |
| IPAC addition/加乙酸异丙酯 | Performed by & Date/执行人及日期： |  |  |  |  |  |
| IPAC addition/加乙酸异丙酯 | Begin/开始 |  |  |  |  |  |
|  | End/结束 |  |  |  |  |  |
|  | Qty/重量 |  |  |  |  |  |
|  | Transcribed by & Date /转录人及日期： |  |  |  |  |  |

| 3.25. （8）Deprotection/脱保护 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h) Bubble with nitrogen for 5±2 min (target: 5 min). turn off the nitrogen. Start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm). Stir for 5±2 min (target: 5 min). After stirring for 5±2 min (target: 5min), stop stirring, turn off the nitrogen./ 开启合成釜下口鼓氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min（目标时间：5 min）后开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm)，搅拌5±2 min（目标时间：10 min）后，停止搅拌，关闭氮气 |  |  |  |  |  |  |
| Operation name/操作名称 | Time point/时间节点 | Wash /洗涤1 | Wash /洗涤2 | Wash /洗涤3 | Wash /洗涤4 |
| Bubble/鼓氮气 | Performed by & Date/执行人及日期： |  |  |  |  |
| Bubble/鼓氮气 | Begin/开始 |  |  |  |  |
|  | End/结束 |  |  |  |  |
|  | Transcribed by & Date /转录人及日期： |  |  |  |  |
| Stir /搅拌 | Performed by & Date/执行人及日期： |  |  |  |  |
| Stir /搅拌 | Begin/开始 |  |  |  |  |
|  | End/结束 |  |  |  |  |
|  | Transcribed by & Date /转录人及日期： |  |  |  |  |
| Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point./通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  |  |  |  |
| Filter/过滤 | Performed by & Date/执行人及日期： |  |  |  |  |
|  | Checked by & Date/核实人及日期 |  |  |  |  |
|  | Begin/开始 |  |  |  |  |
|  | End/结束 |  |  |  |  |

| 3.28. （9）Deprotection/脱保护 |  |  |  |  |
| --- | --- | --- | --- | --- |
| CAUTION/注意：DMF/N，N-二甲基甲酰胺：Irritation of eyes, skin and nose. May cause nausea. /刺激眼睛、皮肤和鼻子。可能引起恶心。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |
| 3.28.6 DMF addition/加N,N-二甲基甲酰胺 Add 546.05 kg (Acceptable weighing range: 529.67 ~ 562.43 kg) (12 V) DMF by spray balls into synthesis reactor (TJ4S-1213-Sy15). 通过淋洗球向合成釜（TJ4S-1213-Sy15）中加入546.05 kg（可接受称量范围：529.67 ~ 562.43 kg）（12 V）N,N-二甲基甲酰胺。 |  |  |  | Times/次数 |
|  |  |  |  | 1 |
| DMF addition/ 加N,N-二甲基甲酰胺 | Performed by & Date/执行人及日期： |  |  |  |
| DMF addition/ 加N,N-二甲基甲酰胺 | Begin/开始： | End/结束： | Qty/重量： |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |
| 3.28.7 Resin reswelling wash 1/溶胀后洗涤1 Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h) .Bubble with nitrogen for 5±2 min (target: 5 min), then start the agitator to 20±10 rpm (target: 20 rpm). After stirring for 10±2 min (target 10 min), stop stirring, turn off the nitrogen. Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point. 开启合成釜（TJ4S-1213-Sy15）下口鼓氮气, 氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min后（目标：5 min），开启搅拌转速至20±10 rpm (目标转速: 20 rpm)，搅拌10±2 min (目标：10 min)后，停止搅拌，关闭氮气。通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  |  | Times/次数 |
|  |  |  |  | 1 |
| Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h) for 5±2 min (target: 5 min), Start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm). Stir for 10±5 min (target: 10 min). After stirring for 10±2 min (target: 10 min), stop stirring, turn off the nitrogen./ 开启合成釜下口鼓氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min（目标时间：5 min）后，开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm)，搅拌10±2 min（目标时间:10 min）后，停止搅拌，关闭氮气 |  |  |  |  |

| 3.28. （9）Deprotection/脱保护 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Bubble/鼓氮气 |  | Performed by & Date/执行人及日期： |  |  |  |  |
| Bubble/鼓氮气 |  | Begin/开始： |  | End/结束： |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |
| Stir /搅拌 |  | Performed by & Date/执行人及日期： |  |  |  |  |
| Stir /搅拌 |  | Begin/开始： |  | End/结束： |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |
| Filter/过滤 |  | Performed by & Date/执行人及日期： |  |  |  |  |
|  |  | Checked by & Date/核实人及日期： |  |  |  |  |
|  |  | Begin/开始： |  | End/结束： |  |  |
| CAUTION/注意：DMF/N，N-二甲基甲酰胺：Irritation of eyes, skin and nose. May cause nausea. /刺激眼睛、皮肤和鼻子。可能引起恶心。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |  |
| 3.25.8 DMF addition/加N,N-二甲基甲酰胺 Add 546.05 kg (Acceptable weighing range: 529.67 ~ 562.43 kg) (12 V) DMF by spray balls into synthesis reactor (TJ4S-1213-Sy15). 通过淋洗球向合成釜（TJ4S-1213-Sy15）中加入546.05 kg（可接受称量范围：529.67 ~ 562.43 kg）（12 V）N,N-二甲基甲酰胺。 |  |  |  |  |  | Times/次数 |
|  |  |  |  |  |  | 1 |
| DMF addition/ 加N,N-二甲基甲酰胺 | Performed by & Date/执行人及日期： |  |  |  |  |  |
| DMF addition/ 加N,N-二甲基甲酰胺 | Begin/开始： |  | End/结束： |  | Qty/重量： |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |

| 3.28. （9）Deprotection/脱保护 |  |  |  |
| --- | --- | --- | --- |
| 3.28.9 Resin reswelling wash 2/溶胀后洗涤2 Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h) .Bubble with nitrogen for 5±2 min (target: 5 min), then start the agitator to 20±10 rpm (target: 20 rpm). After stirring for 10±2 min (target 10 min), stop stirring, turn off the nitrogen. Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point. 开启合成釜（TJ4S-1213-Sy15）下口鼓氮气, 氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min后（目标：5 min），开启搅拌转速至20±10 rpm (目标转速: 20 rpm)，搅拌10±2 min (目标：10 min)后，停止搅拌，关闭氮气。通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  | Times/次数 |
|  |  |  | 1 |
| Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h) for 5±2 min (target: 5 min), Start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm). Stir for 10±5 min (target: 10 min). After stirring for 10±2 min (target: 10 min), stop stirring, turn off the nitrogen./ 开启合成釜下口鼓氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min（目标时间：5 min）后，开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm)，搅拌10±2 min（目标时间:10 min）后，停止搅拌，关闭氮气 |  |  |  |
| Bubble/鼓氮气 | Performed by & Date/执行人及日期： |  |  |
| Bubble/鼓氮气 | Begin/开始： | End/结束： |  |
| Transcribed by & Date /转录人及日期： |  |  |  |
| Stir /搅拌 | Performed by & Date/执行人及日期： |  |  |
| Stir /搅拌 | Begin/开始： | End/结束： |  |
| Transcribed by & Date /转录人及日期： |  |  |  |
| Filter/过滤 | Performed by & Date/执行人及日期： |  |  |
|  | Checked by & Date/核实人及日期： |  |  |
|  | Begin/开始： | End/结束： |  |

| - （9）Fmoc-L-Leu-OH activation/芴甲氧羰基-L-亮氨酸活化 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| CAUTION/注意：DMF/N，N-二甲基甲酰胺：Irritation of eyes, skin and nose. May cause nausea. /刺激眼睛、皮肤和鼻子。可能引起恶心。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |
| 3.29.1 Add DMF to the activation reactor/激活罐加N,N-二甲基甲酰胺： Add 227.52 kg (Acceptable weighing range: 220.69 ~ 234.34 kg) (5 V) DMF into activation tank (TJ4S-1213-Sy15-R01). Start the agitator of activation tank to 95±10 rpm (target: 95 rpm). 向激活罐（TJ4S-1213-Sy15-R01）中加入227.52 kg（可接受称量范围：220.69 ~ 234.34 kg）（5 V）N，N-二甲基甲酰胺，开启激活罐搅拌转速至95±10 rpm (目标转速: 95 rpm)。 |  |  |  | Times/次数 |  |
|  |  |  |  | 1 |  |
| Add 182.02 kg DMF (Acceptable weighing range: 176.56 ~ 187.48 kg) (4 V) by spray balls into activation tank (TJ4S-1213-Sy15-R01).通过淋洗球向激活罐(TJ4S-1213-Sy15-R01)中加入182.02 kg N, N-二甲基甲酰胺（可接受称量范围：176.56~187.48 kg）(4 V)。 |  |  |  |  |  |
| DMF addition/加N,N-二甲基甲酰胺 | Performed by & Date/执行人及日期： |  |  |  |  |
| DMF addition/加N,N-二甲基甲酰胺 | Begin/开始： | End/结束： | Qty/重量： |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |
| Start the agitator of activation tank to 95±10 rpm (target: 95 rpm)./ 开启激活罐搅拌转速至95±10 rpm (目标转速: 95 rpm)。 |  |  |  |  |  |
| Stir/搅拌 | Performed by & Date/执行人及日期： |  |  |  |  |
| Stir/搅拌 | Begin/开始： |  |  |  |  |
|  | The end time is documented in Ins 3.29.8/结束时间记录于3.29.8 |  |  |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |
| 3.29.2 Fmoc-L-Leu-OH addition /加芴甲氧羰基-L-亮氨酸 Add Fmoc-L-Leu-OH (2.00 eq.) with powder feeding bags into activation tank (TJ4S-1213-Sy15-R01). 通过投料袋向激活罐（TJ4S-1213-Sy15-R01）中加入 kg芴甲氧羰基-L-亮氨酸（Acceptable weighing range/可接受称量范围： kg ~ kg）(2.00 eq.)。 |  |  |  |  | Times/次数 |  |
|  |  |  |  |  | 1 |  |

| 3.29. （9）Fmoc-L-Leu-OH activation/芴甲氧羰基-L-亮氨酸活化 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Before material charging, confirm the material is within the retest date/ the expiry date,Confirm whether the weight is adequate for use in the current charging. /加物料前，执行物料在复检日期/有效期之内，确认重量是否满足本次投料使用。 Before solid material charging, confirm/加固体物料前，确认： - Before performed, confirm there is no non-related material / equipment / instrument / tools / pipeline / document / record / label in the intended area /操作前，执行目标区域无不相关物料/设备/仪器/工具/管路/文件/记录/标签 - The outer packages are removed. If the outer package cannot be removed, but un-package material without damaging the package bag, and before charging, check no package fragment and other extraneous matter in material. /外包装移除；如果外包装不能移除，但在不破坏包装的情况下打开包装袋，投料前以及过程中同时检查执行物料中无包装碎片以及异物 Checked by & Date/核实人及日期： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Prior to charging each amino acid, the operator shall perform a Raman scan to confirm the identity of the material. The Raman scan result shall be compared against the reference spectrum of the corresponding amino acid. Only materials that conform to the reference spectrum may be charged. If the Raman scan result does not conform, stop the operation, quarantine the material, and initiate a investigation per site SOP. Record the Raman scan result, operator signature, and date/time in the table below./ 每次投氨基酸前，操作人员应进行拉曼扫描以执行物料。拉曼扫描结果应与相应氨基酸的参考光谱进行比对。仅与参考光谱相符的物料方可投料。如拉曼扫描结果不相符，应停止操作，对物料进行隔离，并依据厂区 SOP 启动异常事件调查。在下表中记录拉曼扫描结果、操作人员签字及日期 / 时间。 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Charge Fmoc-L- Leu-OH /加芴甲氧羰基-L-亮氨酸 |  |  |  |  |  |  |  |  |  |  | Performed by & Date/执行人及日期： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | Begin/开始： |  |  |  |  |  |  | End/结束： |  |  |  |  |  |  |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Fmoc-L- Leu-OH confirmation/芴甲氧羰基-L-亮氨酸执行：Control No./内部控制号YL003425 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Check items/检查项 |  |  |  |  |  | Is it correct?/是否正确 |  |  |  |  |  |  |  | Recorded by & Date/ 记录人及日期 |  |  |  |  |  | Checked by & Date/ 核实人及日期 |  |  |  |  |  |  |
| Material name/物料名称 |  |  |  |  |  | Yes/是；No/否 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Project batch number/ 项目批次号 |  |  |  |  |  | Yes/是；No/否 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Operation number/ 操作号 |  |  |  |  |  | Yes/是；No/否 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Control No./内部控制号 |  |  |  |  |  | Yes/是；No/否 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Raman Scan Confirmation/拉曼扫描执行 |  |  |  |  |  | Conforms/相符 Does not conform/不相符 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| No. /编号 |  |  | Net weight/净重 |  |  | All in/全部加入 |  |  |  |  |  |  |  | Recorded by & Date/ 记录人及日期 |  |  |  |  |  | Checked by & Date/ 核实人及日期 |  |  |  |  |  |  |
| / |  |  |  |  |  | Yes/是；No/否 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| / |  |  |  |  |  | Yes/是；No/否 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| / |  |  |  |  |  | Yes/是；No/否 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3.29. （9）Fmoc-L-Leu-OH activation/芴甲氧羰基-L-亮氨酸活化 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| No. /编号 |  |  | Net weight/净重 |  |  | All in/全部加入 |  |  |  |  |  |  |  | Recorded by & Date/ 记录人及日期 |  |  |  |  |  | Checked by & Date/ 核实人及日期 |  |  |  |  |  |  |
| / |  |  |  |  |  | Yes/是；No/否 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| / |  |  |  |  |  | Yes/是；No/否 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| / |  |  |  |  |  | Yes/是；No/否 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| / |  |  |  |  |  | Yes/是；No/否 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| / |  |  |  |  |  | Yes/是；No/否 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| / |  |  |  |  |  | Yes/是；No/否 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| / |  |  |  |  |  | Yes/是；No/否 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| / |  |  |  |  |  | Yes/是；No/否 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Total/合计 |  |  |  |  |  | Confirm the total net weight is within the input range/执行合计净重在投料范围内 |  |  |  |  |  |  |  | Recorded by & Date/记录人及日期 |  |  |  |  |  | Checked by & Date/ 核实人及日期 |  |  |  |  |  |  |
|  |  |  |  |  |  | Yes/是；No/否 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| After solid material charging, confirm/固体物料投料之后，确认： - The corresponding leftover material has been properly sealed and transferred to the staging area/相应的剩余物料已经恰当密封，转移到了暂存区域。 - The waste packaging material has been sealed and transferred to the corresponding waste staging area. /产生的包材废物已经密封并转移到了相应的废物暂存区域。 - The operation area/equipment surface has been cleaned/操作区域以及设备表面已经清洁。 Checked by & Date/核实人及日期： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3.29.3 Agitation of activation tank /激活罐搅拌 Continue agitation of activation tank to 95±10 rpm (target: 95 rpm). Maintain stirring for at least 15 min. Confirm no visible solids and later use. 维持激活罐搅拌转速至95±10 rpm (目标转速: 95 rpm), 搅拌至少15 min，确认无明显固体待用。 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Times/次数 |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |  |  |
| Stir /搅拌 |  |  |  |  |  |  |  | Performed by & Date/执行人及日期： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | End/结束： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | The end time is documented in Ins 3.29.8/结束时间记录于3.29.8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Confirm no visible solid/确认无明显固体： |  |  |  |  |  |  |  |  |  |  | Confirmed by& Date /确认人及日期： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3.29. （9）Fmoc-L-Leu-OH activation/芴甲氧羰基-L-亮氨酸活化 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3.29.4 Adjust activation tan temperature /激活罐调温 Adjust the activation tank (TJ4S-1213-Sy15-R01) mixture to 【20±5 ℃ (target: 20 ℃)】. /将激活罐(TJ4S-1213-Sy15-R01)中体系降温至【20±5 ℃（目标温度：20 ℃）】 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Times/次数 |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |  |  |
| Adjust/调温 |  |  |  |  |  |  |  |  |  | Performed by & Date/执行人及日期： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Adjust/调温 |  |  |  |  |  |  |  |  |  | Begin/开始： |  |  |  |  |  |  | End/结束： |  |  |  |  |  |  |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CAUTION/注意：OxymaPure is highly toxic, high-energy compound. /2-肟氰乙酸乙酯是高毒，高能化合物。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3.29.5 OxymaPure addition /加2-肟氰乙酸乙酯 Add OxymaPure (2.00 eq.) with powder feeding bags into activation tank (TJ4S-1213-Sy15-R01) at 20±5 ℃ (target 20 ℃). 控温20±5 ℃（目标温度：20 ℃），通过投料袋向激活罐（TJ4S-1213-Sy15-R01）中加入 kg 2-肟氰乙酸乙酯（Acceptable weighing range/可接受称量范围： kg ~ kg）(2.00 eq.)。 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Times/次数 |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |  |  |
| Before material charging, confirm the material is within the retest date/ the expiry date, Confirm whether the weight is adequate for use in the current charging. /加物料前，执行物料在复检日期/有效期之内，确认重量是否满足本次投料使用。 Before solid material charging, confirm/加固体物料前，确认： - Before performed, confirm there is no non-related material / equipment / instrument / tools / pipeline / document / record / label in the intended area /操作前，执行目标区域无不相关物料/设备/仪器/工具/管路/文件/记录/标签 - The outer packages are removed. If the outer package cannot be removed, but un-package material without damaging the package bag, and before charging, check no package fragment and other extraneous matter in material. /外包装移除；如果外包装不能移除，但在不破坏包装的情况下打开包装袋，投料前以及过程中同时检查执行物料中无包装碎片以及异物 Checked by & Date/核实人及日期： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Check and record that the solid charge port is free of solid accumulation from the previous step./ 检查并记录固体投料口无来自上一步骤的固体残留。 Checked by & Date/核实人及日期： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Charge OxymaPure /加2-肟氰乙酸乙酯 |  |  |  |  |  |  |  |  |  | Performed by & Date/执行人及日期： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  | Begin/开始： |  |  |  |  |  |  | End/结束： |  |  |  |  |  |  |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3.29. （9）Fmoc-L-Leu-OH activation/芴甲氧羰基-L-亮氨酸活化 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| OxymaPure confirmation/2-肟氰乙酸乙酯确认：Control No./内部控制号：YL005180 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Check items/检查项 |  |  |  |  |  | Is it correct?/是否正确 |  |  |  |  |  |  | Recorded by & Date/ 记录人及日期 |  |  |  |  |  | Checked by & Date/ 核实人及日期 |  |  |  |  |  |
| Material name/物料名称 |  |  |  |  |  | Yes/是；No/否 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Project batch number/ 项目批次号 |  |  |  |  |  | Yes/是；No/否 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Operation number/ 操作号 |  |  |  |  |  | Yes/是；No/否 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Control No./内部控制号 |  |  |  |  |  | Yes/是；No/否 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Raman Scan Confirmation/拉曼扫描执行 |  |  |  |  |  | Conforms/相符 Does not conform/不相符 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| No. /编号 |  |  | Net weight/净重 |  |  | All in/全部加入 |  |  |  |  |  |  | Recorded by & Date/ 记录人及日期 |  |  |  |  |  | Checked by & Date/ 核实人及日期 |  |  |  |  |  |
| / |  |  |  |  |  | Yes/是；No/否 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| / |  |  |  |  |  | Yes/是；No/否 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| / |  |  |  |  |  | Yes/是；No/否 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| / |  |  |  |  |  | Yes/是；No/否 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| / |  |  |  |  |  | Yes/是；No/否 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| / |  |  |  |  |  | Yes/是；No/否 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| / |  |  |  |  |  | Yes/是；No/否 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| / |  |  |  |  |  | Yes/是；No/否 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| / |  |  |  |  |  | Yes/是；No/否 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| / |  |  |  |  |  | Yes/是；No/否 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| / |  |  |  |  |  | Yes/是；No/否 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| / |  |  |  |  |  | Yes/是；No/否 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| 3.29. （9）Fmoc-L-Leu-OH activation/芴甲氧羰基-L-亮氨酸活化 |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Total/合计 |  |  | Confirm the total net weight is within the input range/执行合计净重在投料范围内 |  |  | Recorded by & Date/记录人及日期 |  |  | Checked by & Date/ 核实人及日期 |  |  |
|  |  |  | Yes/是；No/否 |  |  |  |  |  |  |  |  |
| After solid material charging, confirm/固体物料投料之后，确认： - The corresponding leftover material has been properly sealed and transferred to the staging area/相应的剩余物料已经恰当密封，转移到了暂存区域。 - The waste packaging material has been sealed and transferred to the corresponding waste staging area. /产生的包材废物已经密封并转移到了相应的废物暂存区域。 - The operation area/equipment surface has been cleaned/操作区域以及设备表面已经清洁。 Checked by & Date/核实人及日期： |  |  |  |  |  |  |  |  |  |  |  |
| Internal temperature/内部温度 |  |  |  | High values/最高值： |  |  | Low values/最低值： |  |  |  |  |  |
| Transcribed by & Date/转录人及日期： |  |  |  |  |  |  |  |  |  |  |  |  |
| CAUTION/注意：DMF/N，N-二甲基甲酰胺：Irritation of eyes, skin and nose. May cause nausea. /刺激眼睛、皮肤和鼻子。可能引起恶心。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |  |  |  |  |  |  |  |
| 3.29.6 Rinse activation tank /激活罐淋洗 Rinse the port of Activation tank (TJ4S-1213-Sy15-R01) with 91.01 kg（Acceptable weighing range：88.28 ~ 93.74 kg）（2 V）DMF. 用91.01 kg（可接受称量范围：88.28 ~ 93.74 kg）（2 V）N,N-二甲基甲酰胺淋洗激活罐（TJ4S-1213-Sy15-R01）加料口。 |  |  |  |  |  |  |  |  |  | Times/次数 |  |  |
|  |  |  |  |  |  |  |  |  |  | 1 |  |  |
| Rinse with DMF/ N,N-二甲基甲酰胺淋洗 |  |  | Performed by & Date/执行人及日期： |  |  |  |  |  |  |  |  |  |
| Rinse with DMF/ N,N-二甲基甲酰胺淋洗 |  |  | Begin/开始： |  |  | End/结束： |  |  | Qty/重量： |  |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |  |  |  |  |  |
| DIC is a key EHS concern chemical, highly toxic, skin sensitizing, respiratory sensitizing. /N,N’二异丙基碳二亚胺为EHS重点关注化学品，高毒，皮肤致敏，呼吸致敏。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |  |  |  |  |  |  |  |

| 3.29. （9）Fmoc-L-Leu-OH activation/芴甲氧羰基-L-亮氨酸活化 |  |  |
| --- | --- | --- |
| 3.29.7 Add DIC(2.20 eq.) into the activation tank (TJ4S-1213-Sy15-R01) at reference time of 10-30 min at 【20±5 ℃ (target: 20 ℃)】. The addition rate depends on temperature control. 控温【20±5 ℃ (目标温度20 ℃)】，向激活罐（TJ4S-1213-Sy15-R01）中滴加 kg N,N'-二异丙基碳二亚胺（Acceptable weighing range/可接受称量范围： kg ~ kg）(2.20 eq.)（滴加参考时间10-30 min，以控温为主）。 Pump number/泵编号： Scale ID. No./秤编号： Material charging pipe number/物料管路编号： |  | Times/次数 |
|  |  | 1 |
| ★Note:The DIC addition temperature is a CPP./ 备注：DIC加料温度是关键控制参数。 Prior to DIC addition, the operator shall confirm that the reaction mixture temperature is within the Normal Operating Range (NOR) / N,N-二异丙基碳二亚胺投料前，操作人员须执行反应体系温度处于正常操作范围（NOR）内: Checked by & Date/核实人及日期： ★Lower limit/低限: 15 oC，High limit/高限: 25 oC Over-temperature warning value/超温预警设置数值： Lower limit/低限: 17 oC，High limit/高限: 23 oC |  |  |
| Before material charging, confirm the material is within the retest date/ the expiry date, Confirm whether the weight is adequate for use in the current charging. /加物料前，执行物料在复检日期/有效期之内，确认重量是否满足本次投料使用。 Checked by & Date/核实人及日期： |  |  |

| 3.29. （9）Fmoc-L-Leu-OH activation/芴甲氧羰基-L-亮氨酸活化 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Before drummed liquid material charging, visual check there is no other extraneous matter in material. /加桶装液体物料前，应目视执行桶内无异物。 Visual Examination of Foreign Matters for Drum Solvent/桶装溶剂异物目视检查: 1. Place the solvent drum in a well-lit area (or use a portable light source) to ensure adequate visibility of the liquid contents./ 将溶剂桶置于光线充足区域（或使用便携式光源），确保液体内容物可见性良好 2. Visually inspect the solvent through the drum bung opening or transparent sight glass for any visible foreign matters, including but not limited to particulates, fibers, sediments, or unusual turbidity./ 通过桶口或透明视镜目视检查溶剂中是否有可见异物，包括但不限于颗粒、纤维、沉淀物或异常浑浊。 3. Confirm that the solvent is clear and free of visible foreign matters before charging. If any foreign matter is observed, do not use the drum and follow the applicable handling procedure for subsequent disposition./ 投料前确认溶剂澄清且无可见异物。如发现任何异物，不得使用该桶，并依据处理流程进行后续处置。 Checked by & Date/核实人及日期： |  |  |  |  |
| Confirm liquid charging pipeline is released. /确认加料管路经过放行。 |  |  |  |  |
| Checked by & Date/核实人及日期： |  |  |  |  |
| Charging of drummed solvent: verify that the pipeline connections remain consistent before and after material feeding, then record the weighing reading 桶装溶剂溶液称量时，确认投料前后管路连接一致，读取称量示数。 Performed by & Date/执行人及日期： Checked by & Date/核实人及日期： |  |  |  |  |  |
| DIC /N,N’-二异丙基碳二亚胺：Control Number/内部控制号：YL003564 |  |  |  |  |  |
| Internal temperature/内部温度 |  | High values/最高值： | Low values/最低值： |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |
| If the target net weight≥the planned net weight, perform full drum charge; otherwise, perform partial (Non-full drum) charge. /当目标加料量≥计划加入净重，勾选进行整桶加料；否则勾选进行零头加料。 Note: Target net weight to be charged = Total target net weight – Total net weight charged. Expected tare weight of drum after charge = Gross weight of drum that to be charged - Target net weight to be charged. 还需加入净重目标值= 加料步骤目标总净重-已加入净重总量 加料完成后预计桶皮重=待加料桶毛重-还需加入净重目标值 |  |  |  |

| 3.29. （9）Fmoc-L-Leu-OH activation/芴甲氧羰基-L-亮氨酸活化 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Addition type加料式 | Lot number/批号 | Pre-charging qty calculation/投料前投料量计算 |  |  |  |  |  | Actual Charging Qty/实际投料量 |  |  |  |  |  |
|  |  | Total qty charged/已加入净重总量 |  | Target qty to be charged/还需加入净重目标值 | Gross qty of drum that to be charged/待加料桶毛重 | Expected net weight of drum after charge /加料完成后预计桶净重 | Expected tare weight of drum after charge/加料完成后预计桶皮重 | Before charging/ 投料前 |  | After charging/ 投料后 | Net weight/ 净重 | Performed by & Date/ 执行人及日期 | Checked by & Date/ 核实人及日期 |
| Full drum整桶 Non-full drum /非整桶 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Full drum整桶 Non-full drum /非整桶 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Total/合计 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DIC Addition/加N,N'-二异丙基碳二亚胺 |  |  | Begin/开始： |  |  |  |  |  | End/结束： |  |  |  |  |
| Transcribed by & Date/转录人及日期： |  |  |  |  |  |  |  |  |  |  |  |  |  |

| 3.29. （9）Fmoc-L-Leu-OH activation/芴甲氧羰基-L-亮氨酸活化 |  |  |  |  |
| --- | --- | --- | --- | --- |
| 3.29.8 Fmoc-L-Leu-OH activation/芴甲氧羰基-L-亮氨酸活化 At 20±5 ℃ (target: 20 ℃). Continue agitation of activation tank to 95±10 rpm (target: 95 rpm). Maintain stirring for 20±5 min (target: 20 min) after addition. 滴毕，控温20±5 ℃(目标温度：20 ℃)，维持激活罐搅拌转速至95±10 rpm (目标转速: 95 rpm)，继续搅拌20±5 min(目标时间：20 min)。 |  |  |  |  | Times/次数 |  |
|  |  |  |  |  | 1 |  |
| Stir /搅拌 |  | Performed by & Date/执行人及日期： |  |  |  |
| Stir /搅拌 |  | End/结束 |  |  |  |
|  |  | The start time is documented in Ins3.29.1 /开始时间记录于3.29.1 |  |  |  |
| Transcribed by & Date/转录人及日期： |  |  |  |  |  |
| Internal temperature/内部温度 |  | High values/最高值： | Low values/最低值： |  |  |
| Transcribed by & Date/转录人及日期： |  |  |  |  |  |
| - （9）Fmoc-L-Leu-OH coupling/芴甲氧羰基-L-亮氨酸偶联 |  |  |  |  |
| 3.30.1 Activation mixture transfer /活化体系转移 Transfer the mixture in activation tank (TJ4S-1213-Sy15-R01) into synthesis reactor (TJ4S-1213-Sy15). 将激活罐（TJ4S-1213-Sy15-R01）中体系转移至合成釜（TJ4S-1213-Sy15）中。 |  |  |  |  | Times/次数 |
|  |  |  |  |  | 1 |
| ★Note:The coupling temperature is a CPP, and the coupling IPC hold temperature is a CPP./ 备注：偶联温度是CPP，偶联IPC维持温度是关键工艺参数。 The operator shall confirm that the reaction mixture temperature is within the Normal Operating Range (NOR) 【25±5oC (target: 25oC)】 /操作人员须执行反应体系温度处于正常操作范围（NOR）内【25±5oC (target: 25oC)】 Checked by & Date/核实人及日期： Over-temperature warning value/超温预警设置数值： Lower limit/低限: 21 oC，High limit/高限: 28 oC |  |  |  |  |  |  |

| 3.30. （9）Fmoc-L-Leu-OH coupling/芴甲氧羰基-L-亮氨酸偶联 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Transfer/转移 |  |  |  | Performed by & Date/执行人及日期： |  |  |  |  |  |  |  |
|  |  |  |  | Checked by & Date/核实人及日期： |  |  |  |  |  |  |  |
|  |  |  |  | Begin/开始： |  |  | End/结束： |  |  |  |  |
| CAUTION/注意：DMF/N，N-二甲基甲酰胺：Irritation of eyes, skin and nose. May cause nausea. /刺激眼睛、皮肤和鼻子。可能引起恶心。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |  |  |  |  |  |
| 3.30.2 DMF addition /加N,N-二甲基甲酰胺 Add 45.50 kg (Acceptable weighing range: 43.50 ~ 47.50 kg) (1 V) DMF by spray balls into activation tank (TJ4S-1213-Sy15-R01) to rinse the wall. 通过淋洗球向激活罐（TJ4S-1213-Sy15-R01）中加入45.50 kg（可接受称量范围：43.50 ~ 47.50 kg） (1 V) N,N-二甲基甲酰胺淋洗罐壁。 |  |  |  |  |  |  |  |  | Times/次数 |  |
|  |  |  |  |  |  |  |  |  | 1 |  |
| Rinse with DMF/ N,N-二甲基甲酰胺淋洗 |  | Performed by & Date/执行人及日期： |  |  |  |  |  |  |  |  |
| Rinse with DMF/ N,N-二甲基甲酰胺淋洗 |  | Begin/开始： |  | End/结束： |  |  | Qty/重量： |  |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |  |  |  |
| 3.30.3 Transfer /转移 Turn on the nitrogen of Synthesis reactor (TJ4S-1213-Sy15) to 5±2 m³/h (target: 5 m³/h). Bubble with nitrogen for 5±2 min (target: 5 min). Start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm). then start the agitator. Transfer the mixture in activation tank (TJ4S-1213-Sy15-R01) into synthesis reactor (TJ4S-1213-Sy15). 合成釜（TJ4S-1213-Sy15）先开启下口氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，，鼓氮气5±2 min (目标：5 min)后，开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm)，将激活罐（TJ4S-1213-Sy15-R01）中体系转移至合成釜（TJ4S-1213-Sy15）中。 |  |  |  |  |  |  |  |  | Times/次数 |  |
|  |  |  |  |  |  |  |  |  | 1 |  |
| Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h). Bubble with nitrogen for 5±2 min (target :5 min) 开启合成釜下口鼓氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min (目标时间：5 min) |  |  |  |  |  |  |  |  |  |  |
| Bubble/鼓氮气 |  |  | Performed by & Date/执行人及日期： |  |  |  |  |  |  |  |  |
| Bubble/鼓氮气 |  |  | Begin/开始： |  |  |  |  |  |  |  |  |
|  |  |  | The end time is documented in Ins3.30.5 /结束时间记录于3.30.5 |  |  |  |  |  |  |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |  |  |  |  |
| 3.30. （9）Fmoc-L-Leu-OH coupling/芴甲氧羰基-L-亮氨酸偶联 |  |  |  |  |  |  |  |  |  |
| Start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm). /开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm) |  |  |  |  |  |  |  |  |  |  |  |
| Stir /搅拌 |  |  | Performed by & Date/执行人及日期： |  |  |  |  |  |  |  |  |
| Stir /搅拌 |  |  | Begin/开始： |  |  |  |  |  |  |  |  |
|  |  |  | The end time is documented in Ins3.30.5 /结束时间记录于3.30.5 |  |  |  |  |  |  |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |  |  |  |  |
| Transfer/转移 |  |  | Performed by & Date/执行人及日期： |  |  |  |  |  |  |  |  |
|  |  |  | Checked by & Date/核实人及日期： |  |  |  |  |  |  |  |  |
|  |  |  | Begin/开始： |  |  |  | End/结束： |  |  |  |  |
| 3.30.4 Adjust synthesis reactor temperature /合成釜调温 Adjust the synthesis reactor (TJ4S-1213-Sy15) mixture to【25±5 ℃ (target: 25 ℃)】. /将合成釜（TJ4S-1213-Sy15）体系调温至【25±5 ℃(目标温度：25 ℃)】. |  |  |  |  |  |  |  |  | Times/次数 |  |
|  |  |  |  |  |  |  |  |  | 1 |  |
| The total reaction Fmoc-L-Leu-OH coupling time：NMT 840 min (Sourced from the holding time study in the PAR study report.)/ 芴甲氧羰基-L-亮氨酸偶联总的反应时间不超过840min(来源于 PAR 研究报告中稳定时间研究)。 Over-temperature warning value/超温预警设置数值： Lower limit/低限: 22 oC ，High limit/高限: 28 oC |  |  |  |  |  |  |  |  |  |  |
| Adjust/调温 |  |  |  | Performed by & Date/执行人及日期： |  |  |  |  |  |  |
| Adjust/调温 |  |  |  | Begin/开始： |  | End/结束： |  |  |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |  |  |  |

| 3.30. （9）Fmoc-L-Leu-OH coupling/芴甲氧羰基-L-亮氨酸偶联 |  |  |
| --- | --- | --- |
| 3.30.5 Fmoc-L-Leu-OH coupling/芴甲氧羰基-L-亮氨酸偶联 The mixture is allowed to react at 【25±5 ℃ (target: 25 ℃) 】. After stirring for 3 h±15 min (target: 3 h), then sample the mixture every 0.5~1 h for ninhydrin test analysis until two consecutive samples test as negative (Yellow to Light pink Report Negative= Complete coupling; Blue to Purple Positive= Incomplete coupling). And the mixture should be stirred for coupling time (4 h≤Coupling Time≤10 h). Stop stirring, turn off the nitrogen. Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point. 控温【25±5 oC（目标温度：25 oC）】反应，反应3 h±15 min（目标时间：3 h）后，然后取样进行茚三酮检测，每隔0.5~1.0 h取样一次，直至连续两次取样结果为阴性（黄色至浅粉色，报告阴性=偶联完全；蓝色至紫色，阳性=偶联不完全）后，且反应时间满足（4 h≤反应时间≤10 h）后，停止搅拌，关闭氮气。通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 Note: If resin adherence is found at other times during the reaction, rinse the reactor with 0.50~5.00 kg DMF each time. /备注：偶联过程中其它时间发现贴壁，每次使用0.50~5.00 kg N,N-二甲基甲酰胺淋洗，记录见附加页 |  | Times/次数 |
|  |  | 1 |

| 3.30 （9）Fmoc-L-Leu-OH coupling/芴甲氧羰基-L-亮氨酸偶联 |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ★After coupling is completed, the operator shall confirm that all Critical Process Parameters (CPP) are within their respective Normal Operating Ranges (NOR)/ 偶联结束后，操作人员须确认所有关键工艺参数（CPP）均处于各自的正常操作范围（NOR）内 ★Note: The coupling temperature is a CPP, and the coupling IPC hold temperature is a CPP./备注：偶联温度是CPP，偶联IPC维持温度是关键工艺参数。 The operator shall confirm that the reaction mixture temperature is within the Normal Operating Range (NOR) :20~30℃ /操作人员须确认反应体系温度处于正常操作范围（NOR）内:20~30℃ □Yes/是 □No/否 Confirmed by & Date/确认人及日期： Checked by & Date/核实人及日期： |  |  |  |  |  |  |  |  |  |  |  |  |
| React/反应 |  |  |  | Begin/开始： |  |  |  | End/结束： |  |  |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |  |  |  |  |  |
| 3.30. （9）Fmoc-L-Leu-OH coupling/芴甲氧羰基-L-亮氨酸偶联 |  |  |  |  |  |  |  |  |  |  |  |  |
| Internal temperature/内部温度 |  |  | High values/最高值： |  |  |  | Low values/最低值： |  |  |  |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |  |  |  |  |  |
| Standard sampling method/标准取样方法 |  |  |  |  |  |  |  |  |  |  |  |  |
| Take 5-10 ml mixture and perform testing in accordance with the ninhydrin test record. During sampling, should wear nitrile gloves and 3M 6000 series Full-face piece respirator, 6006CN cartridges. 通过在线取样器取样5~10 ml按照茚三酮检测记录进行检测。取样过程中，应该戴丁腈手套、3M 6000系列全面罩, 6006CN滤毒盒。 |  |  |  |  |  |  |  |  |  |  |  |  |
| Sampling number/ 取样编号 |  |  | Sampling time/取样时间 |  |  | Sampled by & Date/取样人及日期 | Negative or Positive/阴性或阳性 |  |  | Result filled by &Date/结果填写人及日期 | Checked by & Date/核实人及日期 |  |
| CPo140020- -01- -3.30.5-01 |  |  |  |  |  |  | Negative /阴性 Positive /阳性 |  |  |  |  |  |
| CPo140020- -01- -3.30.5-02 |  |  |  |  |  |  | Negative /阴性 Positive /阳性 |  |  |  |  |  |
| CPo140020- -01- -3.30.5-03 |  |  |  |  |  |  | Negative /阴性 Positive /阳性 |  |  |  |  |  |
| CPo140020- -01- -3.30.5-04 |  |  |  |  |  |  | Negative /阴性 Positive /阳性 |  |  |  |  |  |
| Sampling number/ 取样编号 |  |  | Sampling time/取样时间 |  |  | Sampled by & Date/取样人及日期 | Negative or Positive/阴性或阳性 |  |  | Result filled by &Date/结果填写人及日期 | Checked by & Date/核实人及日期 |  |
| CPo140020- -01- -3.30.5-05 |  |  |  |  |  |  | Negative /阴性 Positive /阳性 |  |  |  |  |  |
| CPo140020- -01- -3.30.5-06 |  |  |  |  |  |  | Negative /阴性 Positive /阳性 |  |  |  |  |  |
| CPo140020- -01- -3.30.5-07 |  |  |  |  |  |  | Negative /阴性 Positive /阳性 |  |  |  |  |  |
| CPo140020- -01- -3.30.5-08 |  |  |  |  |  |  | Negative /阴性 Positive /阳性 |  |  |  |  |  |
| Confirm the end point of the reaction. /执行反应终点。 The acceptance criterion is two consecutive negative test results./连续两次检测结果为阴性即为合格。 Confirmed by & Date/确认人及日期： Checked by & Date/核实人及日期： |  |  |  |  |  |  |  |  |  |  |  |  |
| Stop the stirrer and turn off the nitrogen. Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point./ 停止搅拌，关闭氮气，通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  |  |  |  |  |  |  |  |  |  |
| Stir /搅拌 |  |  |  |  | Performed by & Date/执行人及日期： |  |  |  |  |  |  |
| Stir /搅拌 |  |  |  |  | End/结束： |  |  |  |  |  |  |
|  |  |  |  |  | The start time is documented in Ins 3.30.3 /开始时间记录于3.30.3 |  |  |  |  |  |  |
| Transcribed by & Date/转录人及日期： |  |  |  |  |  |  |  |  |  |  |  |
| Bubble/鼓氮气 |  |  |  |  | Performed by & Date/执行人及日期： |  |  |  |  |  |  |
| Bubble/鼓氮气 |  |  |  |  | End/结束： |  |  |  |  |  |  |
|  |  |  |  |  | The start time is documented in Ins 3.30.3 /开始时间记录于3.30.3 |  |  |  |  |  |  |
| Transcribed by & Date/转录人及日期： |  |  |  |  |  |  |  |  |  |  |  |

| 3.30. （9）Fmoc-L-Leu-OH coupling/芴甲氧羰基-L-亮氨酸偶联 |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Internal temperature/内部温度 |  |  |  | High values/最高值： |  |  |  | Low values/最低值： |  |  |
| Transcribed by & Date/转录人及日期： |  |  |  |  |  |  |  |  |  |  |
| Filter/过滤 |  |  | Performed by & Date/执行人及日期： |  |  |  |  |  |  |  |
|  |  |  | Checked by & Date/核实人及日期： |  |  |  |  |  |  |  |
|  |  |  | Begin/开始： |  |  |  | End/结束： |  |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |  |  |  |
| ★After coupling is completed, the operator shall confirm that all Critical Process Parameters (CPPs) are within their respective Normal Operating Ranges (NOR)/ 偶联结束后，操作人员须确认所有关键工艺参数（CPP）均处于各自的正常操作范围（NOR）内 Checked by & Date/核实人及日期： |  |  |  |  |  |  |  |  |  |  |  |
| CAUTION/注意：DMF/N，N-二甲基甲酰胺：Irritation of eyes, skin and nose. May cause nausea. /刺激眼睛、皮肤和鼻子。可能引起恶心。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |  |  |  |  |  |  |
| 3.30.6 Activation Tank Rinse/激活罐洗涤 Add 364.03 kg (Acceptable weighing range：353.11 ~ 374.95 kg) (8 V) DMF into the activation tank (TJ4S-1213-Sy15-R01). Circulate and clean the activating liquid transfer pipeline 1, liquid charging port cleaning pipeline 2 and solid charging port cleaning pipeline 3. Clean every line wash for 2±1 min (target: 2 min). 向激活罐（TJ4S-1213-Sy15-R01）中加入364.03 kg（可接受称量范围：353.11 ~ 374.95 kg） (8 V) N,N-二甲基甲酰胺，循环清洗激活罐转移管路1，液体加料口清洗管路2和固体加料口清洗管路3，每条线路各清洗2±1 min（目标：2 min）。 |  |  |  |  |  |  |  |  |  | Times/次数 |  |
|  |  |  |  |  |  |  |  |  |  | 3 |  |
| Operation name/ 操作名称 |  | Time point/时间节点 |  |  | Clean the activating liquid transfer pipeline 1/循环清洗激活液转移管路1 | Liquid charging port cleaning pipeline 2 /液体加料口清洗管路2 |  |  | Solid charging port cleaning pipeline 3,/固体加料口清洗管路3 |  |  |
|  |  |  |  |  | Wash /洗涤1 | Wash /洗涤2 |  |  | Wash /洗涤3 |  |  |
| DMF addition/ 加N,N-二甲基甲酰胺 |  | Performed by & Date/执行人及日期： |  |  |  |  |  |  |  |  |  |

| 3.30. （9）Fmoc-L-Leu-OH coupling/芴甲氧羰基-L-亮氨酸偶联 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Operation name/ 操作名称 |  | Time point/时间节点 | Clean the activating liquid transfer pipeline 1/循环清洗激活液转移管路1 | Liquid charging port cleaning pipeline 2 /液体加料口清洗管路2 | Solid charging port cleaning pipeline 3,/固体加料口清洗管路3 |
|  |  |  | Wash /洗涤1 | Wash /洗涤2 | Wash /洗涤3 |
| DMF addition/ 加N,N-二甲基甲酰胺 |  | Begin/开始 |  |  |  |
|  |  | End/结束 |  |  |  |
|  |  | Qty/重量 |  |  |  |
|  |  | Transcribed by & Date /转录人及日期： |  |  |  |
| Circulate and clean/循环清洗 |  | Performed by & Date/执行人及日期： |  |  |  |
| Circulate and clean/循环清洗 |  | Begin/开始 |  |  |  |
|  |  | End/结束 |  |  |  |
|  |  | Transcribed by & Date /转录人及日期： |  |  |  |

| 3.30. （9）Fmoc-L-Leu-OH coupling/芴甲氧羰基-L-亮氨酸偶联 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 3.30.7 Post-coupling wash./偶联后洗涤 Transfer the mixture in activation tank (TJ4S-1213-Sy15-R01) to synthesis reactor (TJ4S-1213-Sy15). Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h). Bubble with nitrogen, after 5±2 min (target 5 min), Start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm). After stirring for 7±2 min (target：7 min), stop stirring, turn off the nitrogen. Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point. 将激活罐（TJ4S-1213-Sy15-R01）洗涤液转入合成釜 (TJ4S-1213-Sy15)。开启合成釜下口鼓氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min (目标时间：5 min)后开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm)，搅拌7±2 min（目标时间：7 min）后停止搅拌，关闭氮气。通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  |  |  |  | Times/次数 |
|  |  |  |  |  |  | 3 |
| Transfer the liquid form activation tank (TJ4S-1213-Sy15-R01) to the synthesis reactor (TJ4S-1213-Sy15) ./将激活罐(TJ4S-1213-Sy15-R01)中体系转移至合成釜(TJ4S-1213-Sy15)中。 |  |  |  |  |  |  |
| Operation name/ 操作名称 |  | Time point/时间节点 | Wash /洗涤1 | Wash /洗涤2 | Wash /洗涤3 |  |
| Transfer/转移 |  | Performed by & Date/执行人及日期： |  |  |  |  |
|  |  | Checked by & Date/核实人及日期： |  |  |  |  |
| Transfer/转移 |  | Begin/开始 |  |  |  |  |
|  |  | End/结束 |  |  |  |  |
|  |  | Transcribed by & Date /转录人及日期： |  |  |  |  |
| Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h), Bubble with nitrogen for 5±2 min (target:5 min), Start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm).After stirring for 7±2 min (target: 7 min), stop stirring, turn off the nitrogen./ 开启合成釜下口鼓氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min（目标时间：5 min）后开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm)，搅拌7±2 min（目标时间：7 min）后，停止搅拌，关闭氮气。 |  |  |  |  |  |  |
| Bubble/鼓氮气 |  | Performed by & Date/执行人及日期： |  |  |  |  |

| 3.30. （9）Fmoc-L-Leu-OH coupling/芴甲氧羰基-L-亮氨酸偶联 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Operation name/ 操作名称 |  | Time point/时间节点 | Wash /洗涤1 | Wash /洗涤2 | Wash /洗涤3 |
| Bubble/鼓氮气 |  | Begin/开始 |  |  |  |
|  |  | End/结束 |  |  |  |
|  |  | Transcribed by & Date /转录人及日期： |  |  |  |
| Stir /搅拌 |  | Performed by & Date/执行人及日期： |  |  |  |
| Stir /搅拌 |  | Begin/开始 |  |  |  |
|  |  | End/结束 |  |  |  |
|  |  | Transcribed by & Date /转录人及日期： |  |  |  |
| Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point/通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  |  |  |  |
| Filter/过滤 |  | Performed by & Date/执行人及日期： |  |  |  |
|  |  | Checked by & Date/核实人及日期： |  |  |  |
|  |  | Begin/开始 |  |  |  |
|  |  | End/结束 |  |  |  |

| - （10）Deprotection/脱保护 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| CAUTION/注意：Piperidine哌啶：Strong local irritant and may cause permanent injury after short exposure to small amounts. /强烈的局部刺激，短时间少量接触可能造成永久性伤害。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |  |
| 3.31.1 Add piperidine solution /加哌啶溶液 Adjust the jacket temperature of synthesis reactor (TJ4S-1213-Sy15) to 【25±5 oC (target: 25 oC)】.Add 357.50 kg (Acceptable weighing range: 346.77 ~ 368.23 kg) (8 V)【20% (V/V) piperidine/DMF solution】 into the Synthesis reactor (TJ4S-1213-Sy15). 将合成釜(TJ4S-1213-Sy15)夹套调温至【25±5 ℃（目标温度：25 ℃）】。向合成釜（TJ4S-1213-Sy15）中加入357.50 kg（可接受称量范围：346.77 ~ 368.23 kg）(8 V)【20%哌啶/DMF (V/V) 】溶液。 |  |  |  |  |  | Times/次数 |
|  |  |  |  |  |  | 1 |
| Adjust/调温 |  | Performed by & Date/执行人及日期： |  |  |  |  |
| Adjust/调温 |  | Begin/开始： |  | Adjust/调温 |  |  |
| Transcribed by & Date/转录人及日期： |  |  |  |  |  |  |
| Confirm and record that the 20%Piperidine solution concentration is within the NOR range./ 确认并记录20%哌啶溶液浓度在正常操作范围内：17.50%~22.50% 20%Piperidine solution concentration confirmation/20%哌啶溶液浓度确认：____________ Confirm and record that the 20%Piperidine solution temperature is within the NOR range./ 确认并记录20%哌啶溶液温度在正常操作范围内：20 oC ~30 oC 20%Piperidine solution temperature confirmation/20%哌啶溶液温度确认：____________ Confirmed by & Date/确认人及日期： Checked by & Date/核实人及日期： |  |  |  |  |  |  |
| Piperidine addition/ 加哌啶溶液 | Performed by & Date/执行人及日期： |  |  |  |  |  |
| Piperidine addition/ 加哌啶溶液 | Begin/开始： |  | End/结束： |  | Qty/重量： |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |

| 3.31. （10）Deprotection/脱保护 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 3.31.2 Adjust synthesis reactor temperature /合成釜调温 Turn on the nitrogen of Synthesis reactor (TJ4S-1213-Sy15) to 5±2 m³/h (target: 5 m³/h). Bubble with nitrogen for 5±2 min (target 5 min). Start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm). then start the stirrer, Adjust to 25±5 ℃ (target: 25 ℃）. 开启合成釜（TJ4S-1213-Sy15）下口鼓氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min (目标：5 min)后开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm)，调温至【25±5 ℃(目标温度：25 ℃)】。 |  |  |  |  | Times/次数 |
|  |  |  |  |  | 1 |
| Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h). Bubble with nitrogen for 5±2 min (target :5 min) 开启合成釜下口鼓氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min (目标时间：5 min) |  |  |  |  |  |
| Bubble/鼓氮气 |  |  | Performed by & Date/执行人及日期： |  |  |
| Bubble/鼓氮气 |  |  | Begin/开始： |  |  |
|  |  |  | The end time is documented in Ins 3.31.3/结束时间记录于3.31.3 |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |
| Start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm). /开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm) |  |  |  |  |  |
| Stir /搅拌 |  |  | Performed by & Date/执行人及日期： |  |  |
| Stir /搅拌 |  |  | Begin/开始： |  |  |
|  |  |  | The end time is documented in Ins 3.31.3/结束时间记录于3.31.3 |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |
| Adjust to 25±5 oC (target: 25 oC)./ 调温至25±5 ℃(目标温度：25 oC) |  |  |  |  |  |

| 3.31. （10）Deprotection/脱保护 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Adjust/调温 |  | Performed by & Date/执行人及日期： |  |  |
| Adjust/调温 |  | Begin/开始： | End/结束： |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |
| 3.31.3 Fmoc-L-α-Me-Phe-OH De-Fmoc/(S)-2-((((9H-芴-9-基)甲氧基)羰基)氨基)-2-甲基-3 -苯基丙酸脱保护 After stirring for 【2 h±15 min (target: 2 h) 】at 【25±5 ℃ (target: 25 ℃) 】, stop stirring, turn off the nitrogen. Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point. 控温【25±5 ℃(目标温度：25 ℃) 】，搅拌【2 h±15 min（目标时间：2 h）】后，停止搅拌，关闭氮气。通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 Over-temperature warning value/超温预警设置数值： Lower limit/低限: 22 oC ，High limit/高限: 28 oC |  |  |  | Times/次数 |
|  |  |  |  | 1 |
| ★Confirm and record that the 20%Piperidine solution temperature is within the NOR range 20 oC ~30 oC ./ 确认并记录20%哌啶溶液温度在正常操作范围内：20 oC ~30 oC。 20%Piperidine solution temperature confirmation/20%哌啶溶液温度确认：____________ Confirmed by& Date/确认人及日期： Checked by & Date/核实人及日期： |  |  |  |  |
| De-Fmoc/脱Fmoc &Stir /搅拌 |  | Performed by & Date/执行人及日期： |  |  |
| De-Fmoc/脱Fmoc &Stir /搅拌 |  | End/结束： |  |  |
|  |  | The start time is documented in Ins3.31.2 /开始时间记录于3.31.2 |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |

| 3.31. （10）Deprotection/脱保护 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Bubble/鼓氮气 |  | Performed by & Date/执行人及日期： |  |  |  |  |  |
| Bubble/鼓氮气 |  | End/结束： |  |  |  |  |  |
|  |  | The start time is documented in Ins3.31.2 /开始时间记录于3.31.2 |  |  |  |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |
| Internal temperature/内部温度 |  | High values/最高值: |  |  | Low values/最低值： |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |
| Filter/过滤 |  | Performed by & Date/执行人及日期： |  |  |  |  |  |
|  |  | Checked by & Date/核实人及日期： |  |  |  |  |  |
|  |  | Begin/开始： |  | End/结束： |  |  |  |
| ★After De-Fmoc is completed, the operator shall confirm that all Critical Process Parameters (CPPs) are within their respective Normal Operating Ranges (NOR)/ 脱保护结束后，操作人员须确认所有关键工艺参数（CPP）均处于各自的正常操作范围（NOR）内 Checked by & Date/核实人及日期： |  |  |  |  |  |  |  |
| Transcribed by & Date/转录人及日期： |  |  |  |  |  |  |  |
| 3.31.4 Adjust the synthesis reactor /合成釜调温 Adjust the synthesis reactor (TJ4S-1213-Sy15) jacket to 【20±5 ℃ (target: 20 ℃)】. /将合成釜（TJ4S-1213-Sy15）夹套调温至【20±5 ℃（目标温度：20 ℃）】。 |  |  |  |  |  | Times/次数 |  |
|  |  |  |  |  |  | 1 |  |
| Adjust/调温 |  | Performed by & Date/执行人及日期： |  |  |  |  |  |
| Adjust/调温 |  | Begin/开始： |  |  | End/结束： |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |

| 3.31. （10）Deprotection/脱保护 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CAUTION/注意：IPAC/乙酸异丙酯： Vapors irritate eyes and respiratory tract; high concentrations can be anesthetic. Liquid irritates eyes but causes no serious injury; may cause dermatitis; no serious effects if swallowed. /蒸汽会刺激眼睛和呼吸道；高浓度的蒸汽可能是麻醉剂。液体刺激眼睛，但不会造成严重伤害；可能会引起皮炎；吞食不会造成严重影响。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |  |  |
| 3.31.5 Post-deprotection wash/脱保护后洗涤 Add 445.92 kg (Acceptable weighing range：432.54 ~ 459.30 kg) (9.29 kg/kg, 10.64 V) isopropyl acetate into synthesis reactor (TJ4S-1213-Sy15). Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h). Bubble with nitrogen for 5±2 min (target 5 min). Start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm), Bubble with nitrogen for 5±2 min (target 5 min), then start the agitator. After stirring for 5±2 min (target 5 min), stop stirring, turn off the nitrogen. Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point. 向合成釜（TJ4S-1213-Sy15）中加入445.92 kg（可接受称量范围：432.54 ~ 459.30 kg) (9.29 kg/kg, 10.64 V）乙酸异丙酯，开启下口氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min (目标时间：5 min)后开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm)，搅拌5±2 min (目标时间：5 min)后，停止搅拌，关闭氮气。通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  |  |  |  |  | Times/次数 |
|  |  |  |  |  |  |  | 4 |
| ★Note: IPAC quantity is a CPP./备注：乙酸异丙酯用量是关键工艺参数。 ★The operator shall confirm that IPAC quantity is within the Normal Operating Range (NOR) 【432.54 kg~459.30 kg】/操作人员须确认乙酸异丙酯用量处于正常操作范围（NOR）内：【432.54 kg~459.30 kg】 Checked by & Date/核实人及日期： |  |  |  |  |  |  |  |
| Operation name/操作名称 |  | Time point/时间节点 | Wash /洗涤1 | Wash /洗涤2 | Wash /洗涤3 | Wash /洗涤4 |  |
| Add 419.04 kg IPAc (10 V) by spray balls into the synthesis reactor (TJ4S-1213-Sy15)./ 向合成釜(TJ4S-1213-Sy15) 中加419.04 kg乙酸异丙酯（10 V)）（Acceptable weighing range/可接受称量范围：406.47~431.61 kg） |  |  |  |  |  |  |  |
| IPAC addition/加乙酸异丙酯 |  | Performed by & Date/执行人及日期： |  |  |  |  |  |

| 3.31. （10）Deprotection/脱保护 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Operation name/操作名称 |  | Time point/时间节点 | Wash /洗涤1 | Wash /洗涤2 | Wash /洗涤3 | Wash /洗涤4 |
| IPAC addition/加乙酸异丙酯 |  | Begin/开始 |  |  |  |  |
|  |  | End/结束 |  |  |  |  |
|  |  | Qty/重量 |  |  |  |  |
|  |  | Transcribed by & Date /转录人及日期： |  |  |  |  |
| Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h) Bubble with nitrogen for 5±2 min (target: 5 min). Start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm). Stir for 5±2 min (target: 5 min), stop stirring, turn off the nitrogen./ 开启合成釜下口鼓氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min（目标时间：5 min）后开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm)，搅拌5±2 min（目标时间：5 min）后，停止搅拌，关闭氮气。 |  |  |  |  |  |  |
| Bubble/鼓氮气 |  | Performed by & Date/执行人及日期： |  |  |  |  |
| Bubble/鼓氮气 |  | Begin/开始 |  |  |  |  |
|  |  | End/结束 |  |  |  |  |
|  |  | Transcribed by & Date /转录人及日期： |  |  |  |  |
| Stir /搅拌 |  | Performed by & Date/执行人及日期： |  |  |  |  |
| Stir /搅拌 |  | Begin/开始 |  |  |  |  |
|  |  | End/结束 |  |  |  |  |
|  |  | Transcribed by & Date /转录人及日期： |  |  |  |  |
| Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point./通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  |  |  |  |  |

| 3.31. （10）Deprotection/脱保护 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operation name/操作名称 |  | Time point/时间节点 |  | Wash /洗涤1 | Wash /洗涤2 | Wash /洗涤3 |  | Wash /洗涤4 |  |
| Filter/过滤 |  | Performed by & Date/ 执行人及日期： |  |  |  |  |  |  |  |
|  |  | Checked by & Date/ 核实人及日期： |  |  |  |  |  |  |  |
|  |  | Begin/开始 |  |  |  |  |  |  |  |
|  |  | End/结束 |  |  |  |  |  |  |  |
| CAUTION/注意：DMF/N，N-二甲基甲酰胺：Irritation of eyes, skin and nose. May cause nausea. /刺激眼睛、皮肤和鼻子。可能引起恶心。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |  |  |  |  |
| 3.31.6 DMF addition /加N,N-二甲基甲酰胺 Add 546.05 kg (Acceptable weighing range: 529.67 ~ 562.43 kg) (12 V) DMF into synthesis reactor (TJ4S-1213-Sy15). 向合成釜（TJ4S-1213-Sy15）中加入546.05 kg（可接受称量范围：529.67 ~ 562.43 kg）（12 V）N,N-二甲基甲酰胺。 |  |  |  |  |  |  |  |  | Times/次数 |
|  |  |  |  |  |  |  |  |  | 1 |
| DMF addition/ 加N,N-二甲基甲酰胺 |  |  | Performed by & Date/执行人及日期： |  |  |  |  |  |  |
| DMF addition/ 加N,N-二甲基甲酰胺 |  |  | Begin/开始： |  | End/结束： |  | Qty/重量： |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |  |  |

| 3.31. （10）Deprotection/脱保护 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 3.31.7 Resin reswelling wash 1/溶胀后洗涤1 Turn on the nitrogen of synthesis reactor (TJ4S-1213-Sy15) to 5±2 m³/h (target: 5 m³/h). Bubble with nitrogen for 5±2 min (target: 5 min), Start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm). After stirring for 10±2 min (target: 10 min), stop stirring, turn off the nitrogen. Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point. 开启合成釜（TJ4S-1213-Sy15）下口鼓氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min (目标：5 min)后，开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm)，搅拌10±2 min（目标：10 min）后，停止搅拌，关闭氮气。通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  |  |  | Times/次数 |
|  |  |  |  |  | 1 |
| Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h) for 5±2 min (target: 5 min), Start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm). Stir for 10±2 min (target: 10 min). Stop stirring, turn off the nitrogen./ 开启合成釜下口鼓氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min（目标时间：5 min）后，开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm)，搅拌10±2 min（目标时间:10 min）后，停止搅拌，关闭氮气. |  |  |  |  |  |
| Bubble/鼓氮气 |  | Performed by & Date/执行人及日期： |  |  |  |
| Bubble/鼓氮气 |  | Begin/开始： |  | End/结束： |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |
| Stir /搅拌 |  | Performed by & Date/执行人及日期： |  |  |  |
| Stir /搅拌 |  | Begin/开始： |  | End/结束： |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |
| Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point./通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  |  |  |  |
| Filter/过滤 |  | Performed by & Date/执行人及日期： |  |  |  |
|  |  | Checked by & Date/核实人及日期： |  |  |  |
|  |  | Begin/开始： | End/结束： |  |  |

| 3.31. （10）Deprotection/脱保护 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CAUTION/注意：DMF/N，N-二甲基甲酰胺：Irritation of eyes, skin and nose. May cause nausea. /刺激眼睛、皮肤和鼻子。可能引起恶心。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |  |  |
| 3.31.8 DMF addition /加N,N-二甲基甲酰胺 Add 546.05 kg (Acceptable weighing range: 529.67 ~ 562.43 kg) (12 V) DMF into synthesis reactor (TJ4S-1213-Sy15). 向合成釜（TJ4S-1213-Sy15）中加入546.05 kg（可接受称量范围：529.67 ~ 562.43 kg）（12 V）N,N-二甲基甲酰胺。 |  |  |  |  |  |  | Times/次数 |
|  |  |  |  |  |  |  | 1 |
| DMF addition/ 加N,N-二甲基甲酰胺 |  | Performed by & Date/执行人及日期： |  |  |  |  |  |
| DMF addition/ 加N,N-二甲基甲酰胺 |  | Begin/开始： |  | End/结束： |  | Qty/重量： |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |
| 3.31.9 Resin reswelling wash 1/溶胀后洗涤2 Turn on the nitrogen of synthesis reactor (TJ4S-1213-Sy15) to 5±2 m³/h (target: 5 m³/h). Bubble with nitrogen for 5±2 min (target: 5 min), Start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm). After stirring for 10±2 min (target: 10 min), stop stirring, turn off the nitrogen. Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point. 合成釜（TJ4S-1213-Sy15）开启下口鼓氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min (目标：5 min)后，开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm)，搅拌10±2 min（目标时间：10 min）后，停止搅拌，关闭氮气。通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  |  |  |  |  | Times/次数 |
|  |  |  |  |  |  |  | 1 |
| Bubble/鼓氮气 |  |  | Performed by & Date/执行人及日期： |  |  |  |  |
| Bubble/鼓氮气 |  |  | Begin/开始： |  | End/结束： |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |

| 3.31. （10）Deprotection/脱保护 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Stir /搅拌 |  | Performed by & Date/执行人及日期： |  |  |
| Stir /搅拌 |  | Begin/开始： |  | End/结束： |
| Transcribed by & Date /转录人及日期： |  |  |  |  |
| Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point./通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  |  |  |
| Filter/过滤 |  | Performed by & Date/执行人及日期： |  |  |
|  |  | Checked by & Date/核实人及日期： |  |  |
|  |  | Begin/开始： | End/结束： |  |

| - （10）Fmoc-L-α-Me-Phe-OH activation/(S)-2-((((9H-芴-9-基)甲氧基)羰基)氨基)-2-甲基-3 -苯基丙酸活化 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| CAUTION/注意：DMF/N，N-二甲基甲酰胺：Irritation of eyes, skin and nose. May cause nausea. /刺激眼睛、皮肤和鼻子。可能引起恶心。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |
| 3.32.1 Add DMF to the activation reactor/激活罐加N,N-二甲基甲酰胺 Add 182.02 kg (Acceptable weighing range: 176.56 ~ 187.48 kg) (4 V) DMF into activation tank (TJ4S-1213-Sy15-R01). Start the agitator of activation tank to 95±10 rpm (target: 95 rpm). 向激活罐（TJ4S-1213-Sy15-R01）中加入182.02 kg（可接受称量范围：176.56 ~ 187.48 kg ）（4 V）N，N-二甲基甲酰胺。开启激活罐搅拌转速至95±10 rpm (目标转速: 95 rpm)。 |  |  |  |  | Times/次数 |
|  |  |  |  |  | 1 |
| Add 182.02 kg DMF (Acceptable weighing range: 176.56 ~ 187.48 kg) (4 V) by spray balls into activation tank (TJ4S-1213-Sy15-R01).通过淋洗球向激活罐(TJ4S-1213-Sy15-R01)中加入182.02 kg N, N-二甲基甲酰胺（可接受称量范围：176.56~187.48kg）(4 V)。 |  |  |  |  |  |
| DMF addition / 加N,N-二甲基甲酰胺 |  | Performed by & Date/执行人及日期： |  |  |  |
| DMF addition / 加N,N-二甲基甲酰胺 |  | Begin/开始： | End/结束： | Qty/重量： |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |
| Start the agitator of activation tank to 95±10 rpm (target: 95 rpm)./ 开启激活罐搅拌转速至95±10 rpm (目标转速: 95 rpm) |  |  |  |  |  |
| Stir /搅拌 | Performed by & Date/执行人及日期： |  |  |  |  |
| Stir /搅拌 | Begin/开始： |  |  |  |  |
|  | The end time is documented in Ins 3.32.8/结束时间记录于3.32.8 |  |  |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |

| 3.32. （10）Fmoc-L-α-Me-Phe-OH activation/(S)-2-((((9H-芴-9-基)甲氧基)羰基)氨基)-2-甲基-3 -苯基丙酸活化 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 3.32.2 Fmoc-L-α-Me-Phe-OH addition /加(S)-2-((((9H-芴-9-基)甲氧基)羰基)氨基)-2-甲基-3 -苯基丙酸 Add Fmoc-L-α-Me-Phe-OH (1.75 eq.) with powder feeding bags into activation tank (TJ4S-1213-Sy15-R01).通过投料袋向激活罐（TJ4S-1213-Sy15-R01）中加入 kg (S)-2-((((9H-芴-9-基)甲氧基)羰基)氨基)-2-甲基-3 -苯基丙酸（Acceptable weighing range/可接受称量范围： kg ~ kg）(1.75 eq.)。 |  |  |  |  |  |  | Times/次数 |
|  |  |  |  |  |  |  | 1 |
| Before material charging, confirm the material is within the retest date/ the expiry date,Confirm whether the weight is adequate for use in the current charging. /加物料前，执行物料在复检日期/有效期之内，确认重量是否满足本次投料使用。 Before solid material charging, confirm/加固体物料前，确认： - Before performed, confirm there is no non-related material / equipment / instrument / tools / pipeline / document / record / label in the intended area /操作前，执行目标区域无不相关物料/设备/仪器/工具/管路/文件/记录/标签 - The outer packages are removed. If the outer package cannot be removed, but un-package material without damaging the package bag, and before charging, check no package fragment and other extraneous matter in material. /外包装移除；如果外包装不能移除，但在不破坏包装的情况下打开包装袋，投料前以及过程中同时检查执行物料中无包装碎片以及异物 Checked by & Date/核实人及日期： |  |  |  |  |  |  |  |
| Prior to charging each amino acid, the operator shall perform a Raman scan to confirm the identity of the material. The Raman scan result shall be compared against the reference spectrum of the corresponding amino acid. Only materials that conform to the reference spectrum may be charged. If the Raman scan result does not conform, stop the operation, quarantine the material, and initiate a investigation per site SOP. Record the Raman scan result, operator signature, and date/time in the table below./ 每次投氨基酸前，操作人员应进行拉曼扫描以执行物料。拉曼扫描结果应与相应氨基酸的参考光谱进行比对。仅与参考光谱相符的物料方可投料。如拉曼扫描结果不相符，应停止操作，对物料进行隔离，并依据厂区 SOP 启动异常事件调查。在下表中记录拉曼扫描结果、操作人员签字及日期 / 时间。 |  |  |  |  |  |  |  |
| Charge Fmoc-L-α-Me-Phe-OH /加(S)-2-((((9H-芴-9-基)甲氧基)羰基)氨基)-2-甲基-3 -苯基丙酸 |  |  | Performed by & Date/执行人及日期： |  |  |  |  |
|  |  |  | Begin/开始： |  | End/结束： |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |
| Fmoc-L-α-Me-Phe-OH confirmation/(S)-2-((((9H-芴-9-基)甲氧基)羰基)氨基)-2-甲基-3 -苯基丙酸执行： Control No./内部控制号YL008272 |  |  |  |  |  |  |  |
| Check items/检查项 |  | Is it correct?/是否正确 |  | Recorded by & Date/ 记录人及日期 |  | Checked by & Date/ 核实人及日期 |  |
| Material name/物料名称 |  | Yes/是；No/否 |  |  |  |  |  |
| Project batch number/ 项目批次号 |  | Yes/是；No/否 |  |  |  |  |  |
| Operation number/ 操作号 |  | Yes/是；No/否 |  |  |  |  |  |
| Control No./内部控制号 |  | Yes/是；No/否 |  |  |  |  |  |
| Raman Scan Confirmation/拉曼扫描执行 |  | Conforms/相符 Does not conform/不相符 |  |  |  |  |  |
| 3.32. （10）Fmoc-L-α-Me-Phe-OH activation/(S)-2-((((9H-芴-9-基)甲氧基)羰基)氨基)-2-甲基-3 -苯基丙酸活化 |  |  |  |  |  |  |  |
| No. /编号 | Net weight/净重 | All in/全部加入 |  | Recorded by & Date/ 记录人及日期 |  | Checked by & Date/ 核实人及日期 |  |
| / |  | Yes/是；No/否 |  |  |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |  |
| Total/合计 |  | Confirm the total net weight is within the input range/执行合计净重在投料范围内 |  | Recorded by & Date/记录人及日期 |  | Checked by & Date/ 核实人及日期 |  |
|  |  | Yes/是；No/否 |  |  |  |  |  |
| After solid material charging, confirm/固体物料投料之后，确认： - The corresponding leftover material has been properly sealed and transferred to the staging area/相应的剩余物料已经恰当密封，转移到了暂存区域。 - The waste packaging material has been sealed and transferred to the corresponding waste staging area. /产生的包材废物已经密封并转移到了相应的废物暂存区域。 - The operation area/equipment surface has been cleaned/操作区域以及设备表面已经清洁。 Checked by & Date/核实人及日期： |  |  |  |  |  |  |  |

| 3.32. （10）Fmoc-L-α-Me-Phe-OH activation/(S)-2-((((9H-芴-9-基)甲氧基)羰基)氨基)-2-甲基-3 -苯基丙酸活化 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 3.32.3 Agitation of activation tank /激活罐搅拌 Continue agitation of activation tank to 95±10 rpm (target: 95 rpm). Maintain stirring for at least 15 min. Confirm no visible solids and later use. 维持激活罐搅拌转速至95±10 rpm (目标转速: 95 rpm), 搅拌至少15 min，确认无明显固体待用。 |  |  |  |  |  | Times/次数 |  |
|  |  |  |  |  |  | 1 |  |
| Stir /搅拌 | Performed by & Date/执行人及日期： |  |  |  |  |  |  |
|  | End/结束： |  |  |  |  |  |  |
|  | The end time is documented in Ins 3.32.8/结束时间记录于3.32.8 |  |  |  |  |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |
| Confirm no visible solid/确认无明显固体： |  |  | Confirmed by& Date /确认人及日期： |  |  |  |  |
| 3.32.4 Adjust activation tan temperature /激活罐调温 Adjust the activation tank (TJ4S-1213-Sy15-R01) mixture to 【20±5 ℃ (target: 20 ℃)】./将激活罐(TJ4S-1213-Sy15-R01)中体系调温至【20±5 ℃（目标温度：20 ℃）】。 |  |  |  |  | Times/次数 |  |  |
|  |  |  |  |  | 1 |  |  |
| Adjust/调温 |  | Performed by & Date/执行人及日期： |  |  |  |  |
| Adjust/调温 |  | Begin/开始： |  | End/结束： |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |
| CAUTION/注意：OxymaPure is highly toxic, high-energy compound. /2-肟氰乙酸乙酯是高毒，高能化合物。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |  |
| 3.32.5 OxymaPure addition /加2-肟氰乙酸乙酯 Add OxymaPure (1.80 eq.) with powder feeding bags into activation tank (TJ4S-1213-Sy15-R01) at【 20±5 ℃ (target 20 ℃) 】. 控温【20±5 ℃（目标：20 ℃）】，通过投料袋向激活罐（TJ4S-1213-Sy15-R01）中加入 kg 2-肟氰乙酸乙酯（Acceptable weighing range/可接受称量范围： kg ~ kg）(1.80 eq.)。 |  |  |  |  | Times/次数 |  |
|  |  |  |  |  | 1 |  |

| 3.32. （10）Fmoc-L-α-Me-Phe-OH activation/(S)-2-((((9H-芴-9-基)甲氧基)羰基)氨基)-2-甲基-3 -苯基丙酸活化 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Before material charging, confirm the material is within the retest date/ the expiry date,Confirm whether the weight is adequate for use in the current charging. /加物料前，执行物料在复检日期/有效期之内，确认重量是否满足本次投料使用。 Before solid material charging, confirm/加固体物料前，确认： - Before operation performed, confirm there is no non-related material / equipment / instrument / tools / pipeline / document / record / label in the intended area /操作前；目标区域无不相关物料/设备/仪器/工具/管路/文件/记录/标签 - The outer packages are removed. If the outer package cannot be removed, but un-package material without damaging the package bag, and before charging, check no package fragment and other extraneous matter in material. /外包装移除；如果外包装不能移除，但在不破坏包装的情况下打开包装袋，投料前以及过程中同时检查执行物料中无包装碎片以及异物 Checked by & Date/核实人及日期： |  |  |  |  |  |  |  |
| Check and record that the solid charge port is free of solid accumulation from the previous step./ 检查并记录固体投料口无来自上一步骤的固体残留。 Checked by & Date/核实人及日期： |  |  |  |  |  |  |  |
| Charge OxymaPure /加2-肟氰乙酸乙酯 |  |  |  | Performed by & Date/执行人及日期： |  |  |  |
|  |  |  |  | Begin/开始： |  | End/结束： |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |
| OxymaPure confirmation/2-肟氰乙酸乙酯确认：Control No./内部控制号：YL005180 |  |  |  |  |  |  |  |
| Check items/检查项 |  |  | Is it correct?/是否正确 |  | Recorded by & Date/ 记录人及日期 |  | Checked by & Date/ 核实人及日期 |
| Material name/物料名称 |  |  | Yes/是；No/否 |  |  |  |  |
| Project batch number/ 项目批次号 |  |  | Yes/是；No/否 |  |  |  |  |
| Check items/检查项 |  |  | Is it correct?/是否正确 |  | Recorded by & Date/ 记录人及日期 |  | Checked by & Date/ 核实人及日期 |  |
| Operation number/ 操作号 |  |  | Yes/是；No/否 |  |  |  |  |  |
| Control No./内部控制号 |  |  | Yes/是；No/否 |  |  |  |  |  |
| No. /编号 |  | Net weight/净重 | All in/全部加入 |  | Recorded by & Date/ 记录人及日期 |  | Checked by & Date/ 核实人及日期 |
| / |  |  | Yes/是；No/否 |  |  |  |  |
| / |  |  | Yes/是；No/否 |  |  |  |  |
| / |  |  | Yes/是；No/否 |  |  |  |  |
| / |  |  | Yes/是；No/否 |  |  |  |  |

| 3.32. （10）Fmoc-L-α-Me-Phe-OH activation/(S)-2-((((9H-芴-9-基)甲氧基)羰基)氨基)-2-甲基-3 -苯基丙酸活化 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| No. /编号 | Net weight/净重 | All in/全部加入 |  | Recorded by & Date/ 记录人及日期 |  | Checked by & Date/ 核实人及日期 |
| / |  | Yes/是；No/否 |  |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |
| Total/合计 |  | Confirm the total net weight is within the input range/执行合计净重在投料范围内 |  | Recorded by & Date/记录人及日期 |  | Checked by & Date/ 核实人及日期 |
|  |  | Yes/是；No/否 |  |  |  |  |
| After solid material charging, confirm/固体物料投料之后，确认： - The corresponding leftover material has been properly sealed and transferred to the staging area/相应的剩余物料已经恰当密封，转移到了暂存区域。 - The waste packaging material has been sealed and transferred to the corresponding waste staging area. /产生的包材废物已经密封并转移到了相应的废物暂存区域。 - The operation area/equipment surface has been cleaned/操作区域以及设备表面已经清洁。 Checked by & Date/核实人及日期： |  |  |  |  |  |  |
| Internal temperature/内部温度 |  |  | High values/最高值： |  | Low values/最低值： |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |

| 3.32. （10）Fmoc-L-α-Me-Phe-OH activation/(S)-2-((((9H-芴-9-基)甲氧基)羰基)氨基)-2-甲基-3 -苯基丙酸活化 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| CAUTION/注意：DMF/N，N-二甲基甲酰胺：Irritation of eyes, skin and nose. May cause nausea. /刺激眼睛、皮肤和鼻子。可能引起恶心。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |
| 3.32.6 Adjust activation tan temperature /激活罐调温 Rinse the port of Activation tank (TJ4S-1213-Sy15-R01) with 91.01 kg（Acceptable weighing range：88.28 ~ 93.74 kg）（2 V）DMF. 用91.01 kg（可接受称量范围：88.28 ~ 93.74 kg）（2 V）N,N-二甲基甲酰胺淋洗激活罐（TJ4S-1213-Sy15-R01）加料口。 |  |  |  | Times/次数 |  |
|  |  |  |  | 1 |  |
| Rinse with DMF/ N,N-二甲基甲酰胺淋洗 | Performed by & Date/执行人及日期： |  |  |  |  |
| Rinse with DMF/ N,N-二甲基甲酰胺淋洗 | Begin/开始： | End/结束： | Qty/重量： |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |
| DIC is a key EHS concern chemical, highly toxic, skin sensitizing, respiratory sensitizing. /N,N’二异丙基碳二亚胺为EHS重点关注化学品，高毒，皮肤致敏，呼吸致敏。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |
| 3.32.7 DIC addition /加N,N'-二异丙基碳二亚胺 Add DIC (2.00 eq.) into the activation tank (TJ4S-1213-Sy15-R01) at reference time of 10-30 min at 【20±5 ℃ (target: 20 ℃) 】. The addition rate depends on temperature control. Stir for 20±5 min (target: 20 min) after addition. 控温【20±5 ℃ (目标温度20 ℃)】，向激活罐（TJ4S-1213-Sy15-R01）中滴加 kg N,N'-二异丙基碳二亚胺（Acceptable weighing range/可接受称量范围： kg ~ kg）(2.00 eq.)（滴加参考时间10-30 min，以控温为主），滴加完毕后搅拌20±5 min（目标时间：20 min）。 Pump number/泵编号： Scale ID. No./秤编号： Material charging pipe number/物料管路编号： |  |  |  |  | Times/次数 |
|  |  |  |  |  | 1 |

| 3.32. （10）Fmoc-L-α-Me-Phe-OH activation/(S)-2-((((9H-芴-9-基)甲氧基)羰基)氨基)-2-甲基-3 -苯基丙酸活化 |  |
| --- | --- |
| ★Note:The DIC addition temperature is a CPP./ 备注：DIC加料温度是关键控制参数。 Prior to DIC addition, the operator shall confirm that the reaction mixture temperature is within the Normal Operating Range (NOR) / N,N-二异丙基碳二亚胺投料前，操作人员须执行反应体系温度处于正常操作范围（NOR）内: Checked by & Date/核实人及日期： ★Lower limit/低限: 15 oC，High limit/高限: 25 oC Over-temperature warning value/超温预警设置数值： Lower limit/低限: 17 oC，High limit/高限: 23 oC |  |
| Before material charging, confirm the material is within the retest date/ the expiry date, Confirm whether the weight is adequate for use in the current charging. /加物料前，执行物料在复检日期/有效期之内。确认重量是否满足本次投料使用。 Performed by & Date/执行人及日期： |  |
| Before drummed liquid material charging, visual check there is no other extraneous matter in material. /加桶装液体物料前，应目视执行桶内无异物。 Visual Examination of Foreign Matters for Drum Solvent/桶装溶剂异物目视检查: 1. Place the solvent drum in a well-lit area (or use a portable light source) to ensure adequate visibility of the liquid contents./ 将溶剂桶置于光线充足区域（或使用便携式光源），确保液体内容物可见性良好 2. Visually inspect the solvent through the drum bung opening or transparent sight glass for any visible foreign matters, including but not limited to particulates, fibers, sediments, or unusual turbidity./ 通过桶口或透明视镜目视检查溶剂中是否有可见异物，包括但不限于颗粒、纤维、沉淀物或异常浑浊。 3. Confirm that the solvent is clear and free of visible foreign matters before charging. If any foreign matter is observed, do not use the drum and follow the applicable handling procedure for subsequent disposition./ 投料前确认溶剂澄清且无可见异物。如发现任何异物，不得使用该桶，并依据处理流程进行后续处置。 Checked by & Date/核实人及日期： |  |  |
| Confirm liquid charging pipeline is released. /确认加料管路经过放行。 |  |  |
| Checked by & Date/核实人及日期： |  |  |

| 3.32. （10）Fmoc-L-α-Me-Phe-OH activation/(S)-2-((((9H-芴-9-基)甲氧基)羰基)氨基)-2-甲基-3 -苯基丙酸活化 |  |  |
| --- | --- | --- |
| Charging of drummed solvent: verify that the pipeline connections remain consistent before and after material feeding, then record the weighing reading 桶装溶剂溶液称量时，确认投料前后管路连接一致，读取称量示数。 Performed by & Date/执行人及日期： Checked by & Date/核实人及日期： |  |  |  |
| DIC /N,N-二异丙基碳二亚胺：Control Number/内部控制号：YL003564 |  |  |  |
| Internal temperature/内部温度 |  | High values/最高值： | Low values/最低值： |
| Transcribed by & Date /转录人及日期： |  |  |  |
| If the target net weight≥the planned net weight, perform full drum charge; otherwise, perform partial (Non-full drum) charge. /当目标加料量≥计划加入净重，勾选进行整桶加料；否则勾选进行零头加料。 Note: Target net weight to be charged = Total target net weight – Total net weight charged. Expected tare weight of drum after charge = Gross weight of drum that to be charged - Target net weight to be charged. 还需加入净重目标值= 加料步骤目标总净重-已加入净重总量 加料完成后预计桶皮重=待加料桶毛重-还需加入净重目标值 |  |  |  |

| 3.32. （10）Fmoc-L-α-Me-Phe-OH activation/(S)-2-((((9H-芴-9-基)甲氧基)羰基)氨基)-2-甲基-3 -苯基丙酸活化 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Addition type加料式 | Lot number/批号 | Pre-charging qty calculation/投料前投料量计算 |  |  |  |  |  | Actual Charging Qty/实际投料量 |  |  |  |  |  |
|  |  | Total qty charged/已加入净重总量 |  | Target qty to be charged/还需加入净重目标值 | Gross qty of drum that to be charged/待加料桶毛重 | Expected net weight of drum after charge /加料完成后预计桶净重 | Expected tare weight of drum after charge/加料完成后预计桶皮重 | Before charging/ 投料前 |  | After charging/ 投料后 | Net weight/ 净重 | Performed by & Date/ 执行人及日期 | Checked by & Date/ 核实人及日期 |
| Full drum整桶 Non-full drum /非整桶 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Full drum整桶 Non-full drum /非整桶 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Total/合计 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DIC Addition/加N,N'-二异丙基碳二亚胺 |  |  | Begin/开始： |  |  |  |  |  | End/结束： |  |  |  |  |
| Transcribed by & Date/转录人及日期： |  |  |  |  |  |  |  |  |  |  |  |  |  |

| 3.32. （10）Fmoc-L-α-Me-Phe-OH activation/(S)-2-((((9H-芴-9-基)甲氧基)羰基)氨基)-2-甲基-3 -苯基丙酸活化 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3.32.8 Fmoc-L-α-Me-Phe-OH activation/(S)-2-((((9H-芴-9-基)甲氧基)羰基)氨基)-2-甲基-3 -苯基丙酸活化 At 20±5 ℃ (target: 20 ℃), continue agitation of activation tank to 95±10 rpm (target: 95 rpm). Maintain stirring for 20±5 min (target: 20 min) after addition. 滴毕，控温20±5 ℃(目标温度：20 ℃)，维持激活罐搅拌转速至95±10 rpm (目标转速: 95 rpm)，继续搅拌20±5 min(目标时间：20 min)。 |  |  |  |  |  |  |  |  |  |  |  |  |  | Times/次数 |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |  |  |
| Stir /搅拌 |  |  |  |  |  | Performed by & Date/执行人及日期： |  |  |  |  |  |  |  |  |  |  |
| Stir /搅拌 |  |  |  |  |  | End/结束 |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | The start time is documented in Ins3.32.1 /开始时间记录于3.32.1 |  |  |  |  |  |  |  |  |  |  |
| Transcribed by & Date/转录人及日期： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Internal temperature/内部温度 |  |  |  |  |  |  | High values/最高值： |  |  |  |  | Low values/最低值： |  |  |  |  |
| Transcribed by & Date/转录人及日期： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CAUTION/注意：DMF/N，N-二甲基甲酰胺：Irritation of eyes, skin and nose. May cause nausea. /刺激眼睛、皮肤和鼻子。可能引起恶心。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - （10）Fmoc-L-α-Me-Phe-OH coupling /(S)-2-((((9H-芴-9-基)甲氧基)羰基)氨基)-2-甲基-3 -苯基丙酸偶联 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3.33.1 Activation mixture transfer /活化体系转移 Transfer the mixture in activation tank (TJ4S-1213-Sy15-R01) into synthesis reactor (TJ4S-1213-Sy15). 将激活罐（TJ4S-1213-Sy15-R01）中体系转移至合成釜（TJ4S-1213-Sy15）中。 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Times/次数 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |  |
| ★Note:The coupling temperature is a CPP, and the coupling IPC hold temperature is a CPP./ 备注：偶联温度是CPP，偶联IPC维持温度是关键工艺参数。 The operator shall confirm that the reaction mixture temperature is within the Normal Operating Range (NOR) 【25±5oC (target: 25oC)】 /操作人员须执行反应体系温度处于正常操作范围（NOR）内【25±5oC (target: 25oC)】 Checked by & Date/核实人及日期： Over-temperature warning value/超温预警设置数值： Lower limit/低限: 21 oC ，High limit/高限: 28 oC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3.33. （10）Fmoc-L-α-Me-Phe -OH coupling/(S)-2-((((9H-芴-9-基)甲氧基)羰基)氨基)-2-甲基-3 -苯基丙酸偶联 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Transfer/转移 |  |  |  |  |  |  |  | Performed by & Date/执行人及日期： |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | Checked by & Date/核实人及日期： |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | Begin/开始： |  | End/结束： |  |  |  |  |  |  |
| CAUTION/注意：DMF/N，N-二甲基甲酰胺：Irritation of eyes, skin and nose. May cause nausea. /刺激眼睛、皮肤和鼻子。可能引起恶心。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3.33.2 DMF addition /加N,N-二甲基甲酰胺 Add 45.50 kg (Acceptable weighing range: 43.50 ~ 47.50 kg) (1 V) DMF by spray balls into activation tank (TJ4S-1213-Sy15-R01) to rinse the wall. 通过淋洗球向激活罐（TJ4S-1213-Sy15-R01）中加入45.50 kg（可接受称量范围：43.50 ~ 47.50 kg） (1 V) N,N-二甲基甲酰胺淋洗罐壁。 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Times/次数 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |  |
| Rinse with DMF/ N,N-二甲基甲酰胺淋洗 |  |  |  | Performed by & Date/执行人及日期： |  |  |  |  |  |  |  |  |  |  |  |  |
| Rinse with DMF/ N,N-二甲基甲酰胺淋洗 |  |  |  | Begin/开始： |  |  |  | End/结束： |  |  |  | Qty/重量： |  |  |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3.33.3 Transfer /转移 Turn on the nitrogen of Synthesis reactor (TJ4S-1213-Sy15) to 5±2 m³/h (target: 5 m³/h). Bubble with nitrogen for for 5±2 min (target: 5 min). Start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm). Transfer the mixture in activation tank (TJ4S-1213-Sy15-R01) into synthesis reactor (TJ4S-1213-Sy15). 合成釜（TJ4S-1213-Sy15）先开启下口氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min (目标：5 min)后，开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm)，将激活罐（TJ4S-1213-Sy15-R01）中体系转移至合成釜（TJ4S-1213-Sy15）中。 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Times/次数 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |  |
| Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h). Bubble with nitrogen for 5±2 min (target :5 min) 开启合成釜下口鼓氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min (目标时间：5 min) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bubble/鼓氮气 |  |  |  | Performed by & Date/执行人及日期： |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bubble/鼓氮气 |  |  |  | Begin/开始： |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | The end time is documented in Ins3.30.5 /结束时间记录于3.30.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3.33. （10）Fmoc-L-α-Me-Phe -OH coupling/(S)-2-((((9H-芴-9-基)甲氧基)羰基)氨基)-2-甲基-3 -苯基丙酸偶联 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm). /开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Stir /搅拌 |  |  |  | Performed by & Date/执行人及日期： |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Stir /搅拌 |  |  |  | Begin/开始： |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | The end time is documented in Ins3.33.5 /结束时间记录于3.33.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Transfer/转移 |  |  | Performed by & Date/执行人及日期： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Checked by & Date/核实人及日期： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Begin/开始： |  |  |  |  |  |  |  | End/结束： |  |  |  |  |  |  |
| 3.33.4 Adjust synthesis reactor temperature /合成釜调温 Adjust the synthesis reactor (TJ4S-1213-Sy15) mixture to 【25±5 ℃ (target: 25 ℃) 】. /将合成釜（TJ4S-1213-Sy15）体系调温至【25±5 ℃(目标：25 ℃)】. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Times/次数 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |  |
| The total reaction Fmoc-L-Leu-OH coupling time：NMT 840 min (Sourced from the holding time study in the PAR study report.)/ 芴甲氧羰基-L-亮氨酸偶联总的反应时间不超过840min(来源于 PAR 研究报告中稳定时间研究)。 Over-temperature warning value/超温预警设置数值： Lower limit/低限: 22 oC ，High limit/高限: 28 oC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Adjust/调温 |  |  | Performed by & Date/执行人及日期： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Adjust/调温 |  |  | Begin/开始： |  |  |  |  |  |  |  | End/结束： |  |  |  |  |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| 3.33. （10）Fmoc-L-α-Me-Phe -OH coupling/(S)-2-((((9H-芴-9-基)甲氧基)羰基)氨基)-2-甲基-3 -苯基丙酸偶联 |  |  |
| --- | --- | --- |
| 3.33.5 Fmoc-L-α-Me-Phe -OH coupling/(S)-2-((((9H-芴-9-基)甲氧基)羰基)氨基)-2-甲基-3 -苯基丙酸偶联 Start timing when temperature reaches 20 ℃. The mixture is allowed to react at 【25±5 ℃ (target: 25 ℃) 】. After stirring for 4 h±15 min (target: 4 h), then sample the mixture every 0.5~1 h for ninhydrin test analysis until two consecutive samples test as negative (Yellow to Light pink Report Negative= Complete coupling; Blue to Purple Positive= Incomplete coupling). And the mixture should be stirred for coupling time (6 h≤Coupling Time≤14 h). Stop stirring, turn off the nitrogen. Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point. 控温【25±5 ℃(目标：25 ℃, 20oC开始计时) 】反应，反应4 h±15 min（目标：4 h）后，然后取样进行茚三酮检测，每隔0.5~1.0 h取样一次，直至连续两次取样结果为阴性（黄色至浅粉色，报告阴性=偶联完全；蓝色至紫色，阳性=偶联不完全）后，且反应时间满足（6 h≤反应时间≤14 h）后，停止搅拌，关闭氮气。通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 Note: If resin adherence is found at other times during the reaction, rinse the reactor with 0.50~5.00 kg DMF each time. /备注：偶联过程中其它时间发现贴壁，每次使用0.50~5.00 kg N,N-二甲基甲酰胺淋洗，记录见附加页 |  | Times/次数 |
|  |  | 1 |

| 3.33. （10）Fmoc-L-α-Me-Phe -OH coupling/(S)-2-((((9H-芴-9-基)甲氧基)羰基)氨基)-2-甲基-3 -苯基丙酸偶联 |  |  |  |
| --- | --- | --- | --- |
| ★After coupling is completed, the operator shall confirm that all Critical Process Parameters (CPP) are within their respective Normal Operating Ranges (NOR)/ 偶联结束后，操作人员须确认所有关键工艺参数（CPP）均处于各自的正常操作范围（NOR）内 ★Note: The coupling temperature is a CPP, and the coupling IPC hold temperature is a CPP./备注：偶联温度是CPP，偶联IPC维持温度是关键工艺参数。 The operator shall confirm that the reaction mixture temperature is within the Normal Operating Range (NOR) :20~30℃ /操作人员须确认反应体系温度处于正常操作范围（NOR）内:20~30℃ □Yes/是 □No/否 Confirmed by & Date/确认人及日期： Checked by & Date/核实人及日期： |  |  |  |
| React/反应 |  | Performed by & Date/执行人及日期： |  |
| React/反应 |  | Begin/开始： | End/结束： |
| Transcribed by & Date /转录人及日期： |  |  |  |

| 3.33. （10）Fmoc-L-α-Me-Phe -OH coupling/(S)-2-((((9H-芴-9-基)甲氧基)羰基)氨基)-2-甲基-3 -苯基丙酸偶联 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Internal temperature/内部温度 |  |  | High values/最高值： |  |  | Low values/最低值： |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |  |
| Standard sampling method/标准取样方法 |  |  |  |  |  |  |  |  |
| Take 5-10 ml mixture and perform testing in accordance with the ninhydrin test record. During sampling, should wear nitrile gloves and 3M 6000 series Full-face piece respirator, 6006CN cartridges. 通过在线取样器取样5~1 0ml按照茚三酮检测记录进行检测。取样过程中，应该戴丁腈手套、3M 6000系列全面罩, 6006CN滤毒盒。 |  |  |  |  |  |  |  |  |
| Sampling number/ 取样编号 |  | Sampling time/取样时间 |  | Sampled by & Date/取样人及日期 | Negative or Positive/阴性或阳性 |  | Result filled by &Date/结果填写人及日期 | Checked by & Date/核实人及日期 |
| CPo140020- -01- -3.33.5-01 |  |  |  |  | Negative /阴性 Positive /阳性 |  |  |  |
| CPo140020- -01- -3.33.5-02 |  |  |  |  | Negative /阴性 Positive /阳性 |  |  |  |
| CPo140020- -01- -3.33.5-03 |  |  |  |  | Negative /阴性 Positive /阳性 |  |  |  |
| CPo140020- -01- -3.33.5-04 |  |  |  |  | Negative /阴性 Positive /阳性 |  |  |  |
| CPo140020- -01- -3.33.5-05 |  |  |  |  | Negative /阴性 Positive /阳性 |  |  |  |
| CPo140020- -01- -3.33.5-06 |  |  |  |  | Negative /阴性 Positive /阳性 |  |  |  |
| CPo140020- -01- -3.33.5-07 |  |  |  |  | Negative /阴性 Positive /阳性 |  |  |  |
| CPo140020- -01- -3.33.5-08 |  |  |  |  | Negative /阴性 Positive /阳性 |  |  |  |
| Confirm the end point of the reaction. /执行反应终点。 The acceptance criterion is two consecutive negative test results./连续两次检测结果为阴性即为合格。 Confirmed by & Date/确认人及日期： Checked by & Date/核实人及日期： |  |  |  |  |  |  |  |  |

| 3.33. （10）Fmoc-L-α-Me-Phe -OH coupling/(S)-2-((((9H-芴-9-基)甲氧基)羰基)氨基)-2-甲基-3 -苯基丙酸偶联 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Stir /搅拌 |  | Performed by & Date/执行人及日期： |  |  |
| Stir /搅拌 |  | End/结束： |  |  |
|  |  | The start time is documented in Ins 3.33.3 /开始时间记录于3.33.3 |  |  |
| Transcribed by & Date/转录人及日期： |  |  |  |  |
| Bubble/鼓氮气 |  | Performed by & Date/执行人及日期： |  |  |
| Bubble/鼓氮气 |  | End/结束： |  |  |
|  |  | The start time is documented in Ins 3.33.3 /开始时间记录于3.33.3 |  |  |
| Transcribed by & Date/转录人及日期： |  |  |  |  |
| Internal temperature/内部温度 |  | High values/最高值： | Low values/最低值： |  |
| Transcribed by & Date/转录人及日期： |  |  |  |  |
| Stop the stirrer and turn off the nitrogen. Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point./ 停止搅拌，关闭氮气，通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  |  |  |
| Filter/过滤 |  | Performed by & Date/执行人及日期： |  |  |
|  |  | Checked by & Date/核实人及日期： |  |  |
|  |  | Begin/开始： |  | End/结束： |
| ★After coupling is completed, the operator shall confirm that all Critical Process Parameters (CPPs) are within their respective Normal Operating Ranges (NOR)/ 偶联结束后，操作人员须确认所有关键工艺参数（CPP）均处于各自的正常操作范围（NOR）内 Checked by & Date/核实人及日期： |  |  |  |  |
| CAUTION/注意：DMF/N，N-二甲基甲酰胺：Irritation of eyes, skin and nose. May cause nausea. /刺激眼睛、皮肤和鼻子。可能引起恶心。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |

| 3.33. （10）Fmoc-L-α-Me-Phe -OH coupling/(S)-2-((((9H-芴-9-基)甲氧基)羰基)氨基)-2-甲基-3 -苯基丙酸偶联 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 3.33.6 Activation Tank Rinse/激活罐洗涤 Add 364.03 kg (Acceptable weighing range：353.11 ~ 374.95 kg) (8 V) DMF into the activation tank (TJ4S-1213-Sy15-R01). Circulate and clean the activating liquid transfer pipeline 1, liquid charging port cleaning pipeline 2 and solid charging port cleaning pipeline 3. Clean every line wash for 2±1 min (target: 2 min). 向激活罐（TJ4S-1213-Sy15-R01）中加入364.03 kg（可接受称量范围：353.11 ~ 374.95 kg） (8 V) N,N-二甲基甲酰胺，循环清洗激活罐转移管路1，液体加料口清洗管路2和固体加料口清洗管路3，每条线路各清洗2±1 min（目标：2 min）。 |  |  |  |  | Times/次数 |
|  |  |  |  |  | 3 |
| Operation name/ 操作名称 | Time point/时间节点 | Clean the activating liquid transfer pipeline 1/循环清洗激活液转移管路1 | Liquid charging port cleaning pipeline 2 /液体加料口清洗管路2 | Solid charging port cleaning pipeline 3,/固体加料口清洗管路3 |  |
|  |  | Wash /洗涤1 | Wash /洗涤2 | Wash /洗涤3 |  |
| DMF addition/ 加N,N-二甲基甲酰胺 | Performed by & Date/执行人及日期： |  |  |  |  |
| DMF addition/ 加N,N-二甲基甲酰胺 | Begin/开始 |  |  |  |  |
|  | End/结束 |  |  |  |  |
|  | Qty/重量 |  |  |  |  |
|  | Transcribed by & Date /转录人及日期： |  |  |  |  |
| Circulate and clean/循环清洗 | Performed by & Date/执行人及日期： |  |  |  |  |
| Circulate and clean/循环清洗 | Begin/开始 |  |  |  |  |
|  | End/结束 |  |  |  |  |
|  | Transcribed by & Date /转录人及日期： |  |  |  |  |

| 3.33. （10）Fmoc-L-α-Me-Phe -OH coupling/(S)-2-((((9H-芴-9-基)甲氧基)羰基)氨基)-2-甲基-3 -苯基丙酸偶联 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 3.33.7 Post-coupling wash./偶联后洗涤 Transfer the mixture in activation tank (TJ4S-1213-Sy15-R01) to synthesis reactor (TJ4S-1213-Sy15). Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h). Bubble with nitrogen, after 5±2 min (target 5 min), start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm). After stirring for 7±2 min (target：7 min), stop stirring, turn off the nitrogen. Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point. 将激活罐（TJ4S-1213-Sy15-R01）洗涤液转入合成釜 (TJ4S-1213-Sy15)。开启合成釜下口鼓氮气，，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min (目标时间：5 min)后开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm)，搅拌7±2 min（目标时间：7 min）后停止搅拌，关闭氮气。通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  |  |  |  | Times/次数 |
|  |  |  |  |  |  | 3 |
| Transfer the liquid form activation tank (TJ4S-1213-Sy15-R01) to the synthesis reactor (TJ4S-1213-Sy15) ./将激活罐(TJ4S-1213-Sy15-R01)中体系转移至合成釜(TJ4S-1213-Sy15)中。 |  |  |  |  |  |  |
| Operation name/ 操作名称 |  | Time point/时间节点 | Wash /洗涤1 | Wash /洗涤2 | Wash /洗涤3 |  |
| Transfer/转移 |  | Performed by & Date/执行人及日期： |  |  |  |  |
|  |  | Checked by & Date/核实人及日期： |  |  |  |  |
|  |  | Begin/开始 |  |  |  |  |
|  |  | End/结束 |  |  |  |  |

| 3.33. （10）Fmoc-L-α-Me-Phe -OH coupling/(S)-2-((((9H-芴-9-基)甲氧基)羰基)氨基)-2-甲基-3 -苯基丙酸偶联 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h), Bubble with nitrogen for 5±2 min (target:5 min), Start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm).After stirring for 7±2 min (target: 7 min), stop stirring, turn off the nitrogen./ 开启合成釜下口鼓氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min（目标时间：5 min）后开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm)，搅拌7±2 min（目标时间：7 min）后，停止搅拌，关闭氮气。 |  |  |  |  |  |  |
| Operation name/ 操作名称 |  | Time point/时间节点 | Wash /洗涤1 | Wash /洗涤2 | Wash /洗涤3 |  |
| Bubble/鼓氮气 |  | Performed by & Date/执行人及日期： |  |  |  |  |
| Bubble/鼓氮气 |  | Begin/开始 |  |  |  |
|  |  | End/结束 |  |  |  |
|  |  | Transcribed by & Date /转录人及日期： |  |  |  |
| Stir /搅拌 |  | Performed by & Date/执行人及日期： |  |  |  |
| Stir /搅拌 |  | Begin/开始 |  |  |  |
|  |  | End/结束 |  |  |  |
|  |  | Transcribed by & Date /转录人及日期： |  |  |  |
| Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point/通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  |  |  |  |
| Filter/过滤 |  | Performed by & Date/执行人及日期： |  |  |  |
|  |  | Checked by & Date/核实人及日期： |  |  |  |
|  |  | Begin/开始 |  |  |  |
|  |  | End/结束 |  |  |  |

| - （11）Deprotection/脱保护 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| CAUTION/注意：Piperidine哌啶：Strong local irritant and may cause permanent injury after short exposure to small amounts. /强烈的局部刺激，短时间少量接触可能造成永久性伤害。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |  |
| 3.34.1 Piperidine solution addition /加哌啶溶液 Adjust the jacket temperature of synthesis reactor (TJ4S-1213-Sy15) to 【25±5 oC (target: 25 oC).】. Add 357.50 kg (Acceptable weighing range: 346.77 ~ 368.23 kg) 【20% (V/V) piperidine/DMF solution 】into the Synthesis reactor (TJ4S-1213-Sy15). 将合成釜(TJ4S-1213-Sy15)夹套调温至【25±5 ℃（目标温度：25 ℃）】。向合成釜（TJ4S-1213-Sy15）中加入357.50 kg（可接受称量范围：346.77 ~ 368.23 kg）【20%哌啶/DMF (V/V) 】溶液。 |  |  |  |  |  | Times/次数 |
|  |  |  |  |  |  | 1 |
| Adjust/调温 |  | Performed by & Date/执行人及日期： |  |  |  |  |
| Adjust/调温 |  | Begin/开始： |  | Adjust/调温 |  |  |
| Transcribed by & Date/转录人及日期： |  |  |  |  |  |  |
| Confirm and record that the 20%Piperidine solution concentration is within the NOR range./ 确认并记录20%哌啶溶液浓度在正常操作范围内：17.50%~22.50% 20%Piperidine solution concentration confirmation/20%哌啶溶液浓度确认：____________ Confirm and record that the 20%Piperidine solution temperature is within the NOR range./ 确认并记录20%哌啶溶液温度在正常操作范围内：20 oC ~30 oC 20%Piperidine solution temperature confirmation/20%哌啶溶液温度确认：____________ Confirmed by & Date/确认人及日期： Checked by & Date/核实人及日期： |  |  |  |  |  |  |
| piperidine addition/ 加哌啶溶液 | Performed by&Date/执行人及日期： |  |  |  |  |  |
| Piperidine addition/ 加哌啶溶液 | Begin/开始： |  | End/结束： |  | Qty/重量： |  |
| Transcribed by&Date/转录人及日期： |  |  |  |  |  |  |

| 3.34. （11）Deprotection/脱保护 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 3.34.2 Adjust synthesis reactor temperature /合成釜调温 Turn on the nitrogen of Synthesis reactor (TJ4S-1213-Sy15) to 5±2 m³/h (target: 5 m³/h). Bubble with nitrogen for 5±2 min (target: 5 min), Start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm)., adjust to 【25±5 ℃ (target: 25 ℃) 】. 合成釜（TJ4S-1213-Sy15）开启下口鼓氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min后（目标：5 min），开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm)，调温至【25±5 ℃(目标温度：25 ℃) 】。 |  |  |  |  | Times/次数 |
|  |  |  |  |  | 1 |
| Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h). Bubble with nitrogen for 5±2 min (target :5 min) 开启合成釜下口鼓氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min (目标时间：5 min) |  |  |  |  |  |
| Bubble/鼓氮气 |  |  | Performed by & Date/执行人及日期： |  |  |
| Bubble/鼓氮气 |  |  | Begin/开始： |  |  |
|  |  |  | The end time is documented in Ins 3.34.3/结束时间记录于3.34.3 |  |  |
| Transcribed by&Date/转录人及日期： |  |  |  |  |  |
| Start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm). /开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm) |  |  |  |  |  |
| Stir /搅拌 |  | Performed by&Date/执行人及日期： |  |  |  |
| Stir /搅拌 |  | Begin/开始： |  |  |  |
|  |  | The end time is documented in Ins 3.34.3/结束时间记录于3.34.3 |  |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |
| Adjust to 25±5 oC (target: 25 oC)./ 调温至25±5 ℃(目标温度：25 oC) |  |  |  |  |  |

| 3.34. （11）Deprotection/脱保护 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Adjust/调温 |  | Performed by&Date/执行人及日期： |  |  |
| Adjust/调温 |  | Begin/开始： | End/结束： |  |
| Transcribed by&Date/转录人及日期： |  |  |  |  |
| 3.34.3 Fmoc-L-Glu(OtBu)-OH·H2O De-Fmoc /芴甲氧羰基-L-谷氨酸-γ-叔丁酯一水合物脱保护 After stirring for 【2 h±15 min (target: 2 h) 】 at 【25±5 ℃ (target: 25 ℃) 】, stop stirring, turn off the nitrogen. Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point. 控温【25±5 ℃(目标温度：25 ℃) 】，搅拌【2 h±15 min（目标：2 h）】后，停止搅拌，关闭氮气。通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 Over-temperature warning value/超温预警设置数值： Lower limit/低限: 22 oC ，High limit/高限: 28 oC |  |  |  | Times/次数 |
|  |  |  |  | 1 |
| ★Confirm and record that the 20%Piperidine solution temperature is within the NOR range 20 oC ~30 oC ./ 确认并记录20%哌啶溶液温度在正常操作范围内：20 oC ~30 oC。 20%Piperidine solution temperature confirmation/20%哌啶溶液温度确认：____________ Confirmed by& Date/确认人及日期： Checked by & Date/核实人及日期： |  |  |  |  |
| De-Fmoc/脱Fmoc &Stir /搅拌 |  | Performed by&Date/执行人及日期： |  |  |
| De-Fmoc/脱Fmoc &Stir /搅拌 |  | End/结束： |  |  |
|  |  | The start time is documented in Ins3.34.2 /开始时间记录于3.34.2 |  |  |
| Transcribed by&Date/转录人及日期： |  |  |  |  |

| 3.34. （11）Deprotection/脱保护 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Bubble/鼓氮气 |  | Performed by & Date/执行人及日期： |  |  |  |  |  |
| Bubble/鼓氮气 |  | End/结束： |  |  |  |  |  |
|  |  | The start time is documented in Ins3.34.2 /开始时间记录于3.34.2 |  |  |  |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |
| Internal temperature/内部温度 |  | High values/最高值: |  |  | Low values/最低值： |  |  |
| Transcribed by&Date/转录人及日期： |  |  |  |  |  |  |  |
| Filter/过滤 |  | Performed by&Date/执行人及日期： |  |  |  |  |  |
|  |  | Checked by & Date/核实人及日期： |  |  |  |  |  |
|  |  | Begin/开始： |  | End/结束： |  |  |  |
| ★After De-Fmoc is completed, the operator shall confirm that all Critical Process Parameters (CPPs) are within their respective Normal Operating Ranges (NOR)/ 脱保护结束后，操作人员须确认所有关键工艺参数（CPP）均处于各自的正常操作范围（NOR）内 Checked by & Date/核实人及日期： |  |  |  |  |  |  |  |
| 3.34.4 Adjust synthesis reactor temperature /合成釜调温 Adjust the synthesis reactor (TJ4S-1213-Sy15) jacket to【 20±5 ℃ (target: 20 ℃) 】. /将合成釜（TJ4S-1213-Sy15）夹套调温至【20±5 ℃（目标温度：20 ℃）】。 |  |  |  |  |  | Times/次数 |  |  |
|  |  |  |  |  |  | 1 |  |  |
| Adjust/调温 |  | Performed by & Date/执行人及日期： |  |  |  |  |  |  |
| Adjust/调温 |  | Begin/开始： |  |  | End/结束： |  |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |  |

| 3.34. （11）Deprotection/脱保护 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CAUTION/注意：IPAC/乙酸异丙酯： Vapors irritate eyes and respiratory tract; high concentrations can be anesthetic. Liquid irritates eyes but causes no serious injury; may cause dermatitis; no serious effects if swallowed. /蒸汽会刺激眼睛和呼吸道；高浓度的蒸汽可能是麻醉剂。液体刺激眼睛，但不会造成严重伤害；可能会引起皮炎；吞食不会造成严重影响。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |  |  |
| 3.34.5 Post-deprotection wash/脱保护后洗涤 Add 445.92 kg (Acceptable weighing range：432.54 ~ 459.30 kg) (9.29 kg/kg, 10.64 V) isopropyl acetate into synthesis reactor (TJ4S-1213-Sy15). Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h). Bubble with nitrogen for 5±2 min (target 5 min). Start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm). After stirring for 5±2 min (target 5 min), stop stirring, turn off the nitrogen. Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point. 向合成釜（TJ4S-1213-Sy15）中加入445.92 kg（可接受称量范围：432.54 ~ 459.30 kg) (9.29 kg/kg, 10.64 V）乙酸异丙酯，开启下口氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min (目标时间：5 min)后开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm)，搅拌5±2 min (目标时间：5 min)后，停止搅拌，关闭氮气。通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  |  |  |  |  | Times/次数 |
|  |  |  |  |  |  |  | 4 |
| ★Note: IPAC quantity is a CPP./备注：乙酸异丙酯用量是关键工艺参数。 ★The operator shall confirm that IPAC quantity is within the Normal Operating Range (NOR) 【432.54 kg~459.30 kg】/操作人员须确认乙酸异丙酯用量处于正常操作范围（NOR）内：【432.54 kg~459.30 kg】 Checked by & Date/核实人及日期： |  |  |  |  |  |  |  |
| Add 419.04 kg IPAc (10 V) by spray balls into the synthesis reactor TJ4S-1213-Sy15./ 向合成釜TJ4S-1213-Sy15 中加419.04 kg乙酸异丙酯（10 V)（Acceptable weighing range/可接受称量范围：406.47~431.61 kg） |  |  |  |  |  |  |  |
| Operation name/操作名称 |  | Time point/时间节点 | Wash /洗涤1 | Wash /洗涤2 | Wash /洗涤3 | Wash /洗涤4 |  |
| IPAC addition/加乙酸异丙酯 |  | Performed by & Date/执行人及日期： |  |  |  |  |  |

| 3.34. （11）Deprotection/脱保护 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Operation name/操作名称 |  | Time point/时间节点 | Wash /洗涤1 | Wash /洗涤2 | Wash /洗涤3 | Wash /洗涤4 |
| IPAC addition/加乙酸异丙酯 |  | Begin/开始 |  |  |  |  |
|  |  | End/结束 |  |  |  |  |
|  |  | Qty/重量 |  |  |  |  |
|  |  | Transcribed by & Date /转录人及日期： |  |  |  |  |
| Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h). Bubble with nitrogen for 5±2 min (target: 5 min). Start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm). Stir for 5±2 min (target: 5 min), stop stirring, turn off the nitrogen./ 开启合成釜下口鼓氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min（目标时间：5 min）后开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm)，搅拌5±2 min（目标时间：5 min）后，停止搅拌，关闭氮气。 |  |  |  |  |  |  |
| Bubble/鼓氮气 |  | Performed by & Date/执行人及日期： |  |  |  |  |
| Bubble/鼓氮气 |  | Begin/开始 |  |  |  |  |
|  |  | End/结束 |  |  |  |  |
|  |  | Transcribed by & Date /转录人及日期： |  |  |  |  |
| Stir /搅拌 |  | Performed by & Date/执行人及日期： |  |  |  |  |
| Stir /搅拌 |  | Begin/开始 |  |  |  |  |
|  |  | End/结束 |  |  |  |  |
|  |  | Transcribed by & Date /转录人及日期： |  |  |  |  |
| Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point./通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  |  |  |  |  |

| 3.34. （11）Deprotection/脱保护 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operation name/操作名称 |  | Time point/时间节点 |  | Wash /洗涤1 | Wash /洗涤2 | Wash /洗涤3 |  | Wash /洗涤4 |  |
| Filter/过滤 |  | Performed by & Date/执行人及日期： |  |  |  |  |  |  |  |
|  |  | Checked by & Date/核实人及日期 |  |  |  |  |  |  |  |
|  |  | Begin/开始 |  |  |  |  |  |  |  |
|  |  | End/结束 |  |  |  |  |  |  |  |
| CAUTION/注意：DMF/N，N-二甲基甲酰胺：Irritation of eyes, skin and nose. May cause nausea. /刺激眼睛、皮肤和鼻子。可能引起恶心。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |  |  |  |  |
| 3.34.6 DMF addition/加N,N-二甲基甲酰胺 Add 546.05 kg (Acceptable weighing range: 529.67 ~ 562.43 kg) (12 V) DMF into synthesis reactor (TJ4S-1213-Sy15). 向合成釜（TJ4S-1213-Sy15）中加入546.05 kg（可接受称量范围：529.67 ~ 562.43 kg）（12 V）N,N-二甲基甲酰胺。 |  |  |  |  |  |  |  |  | Times/次数 |
|  |  |  |  |  |  |  |  |  | 1 |
| DMF addition/ 加N,N-二甲基甲酰胺 |  |  | Performed by&Date/执行人及日期： |  |  |  |  |  |  |
| DMF addition/ 加N,N-二甲基甲酰胺 |  |  | Begin/开始： |  | End/结束： |  | Qty/重量： |  |  |
| Transcribed by&Date/转录人及日期： |  |  |  |  |  |  |  |  |  |

| 3.34. （11）Deprotection/脱保护 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 3.34.7 Resin reswelling wash 1/溶胀后洗涤1 Turn on the nitrogen of synthesis reactor (TJ4S-1213-Sy15) to 5±2 m³/h (target: 5 m³/h). Bubble with nitrogen for 5±2 min (target: 5 min). Start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm). After stirring for 10±2 min (target: 10 min), stop stirring, turn off the nitrogen. Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point. 合成釜（TJ4S-1213-Sy15）开启下口鼓氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min (目标时间：5 min)后，开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm)，搅拌10±2 min（目标时间：10 min）后，停止搅拌，关闭氮气。通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  |  |  |  | Times/次数 |
|  |  |  |  |  |  | 1 |
| Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h) for 5±2 min (target: 5 min). Start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm). Stir for 10±2 min (target: 10 min). Stop stirring, turn off the nitrogen./ 开启合成釜下口鼓氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min（目标时间：5 min）后，开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm)，搅拌10±2 min（目标时间:10 min）后，停止搅拌，关闭氮气. |  |  |  |  |  |  |
| Bubble/鼓氮气 |  | Performed by&Date/执行人及日期： |  |  |  |  |
| Bubble/鼓氮气 |  | Begin/开始： |  | End/结束： |  |  |
| Transcribed by&Date/转录人及日期： |  |  |  |  |  |  |
| Stir /搅拌 |  | Performed by&Date/执行人及日期： |  |  |  |  |
| Stir /搅拌 |  | Begin/开始： |  | End/结束： |  |  |
| Transcribed by&Date/转录人及日期： |  |  |  |  |  |  |
| Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point./通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  |  |  |  |  |
| Filter/过滤 |  |  | Performed by&Date/执行人及日期： |  |  |  |
|  |  |  | Checked by & Date/核实人及日期： |  |  |  |
|  |  |  | Begin/开始： |  | End/结束： |  |

| 3.34. （11）Deprotection/脱保护 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| CAUTION/注意：DMF/N，N-二甲基甲酰胺：Irritation of eyes, skin and nose. May cause nausea. /刺激眼睛、皮肤和鼻子。可能引起恶心。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |
| 3.34.8 DMF addition/加N,N-二甲基甲酰胺 Add 546.05 kg (Acceptable weighing range: 529.67 ~ 562.43 kg) (12 V) DMF into synthesis reactor (TJ4S-1213-Sy15). 向合成釜（TJ4S-1213-Sy15）中加入546.05 kg（可接受称量范围：529.67 ~ 562.43 kg）（12 V）N,N-二甲基甲酰胺。 |  |  |  |  | Times/次数 |
|  |  |  |  |  | 1 |
| DMF addition/ 加N,N-二甲基甲酰胺 |  | Performed by&Date/执行人及日期： |  |  |  |
| DMF addition/ 加N,N-二甲基甲酰胺 |  | Begin/开始： | End/结束： | Qty/重量： |  |
| Transcribed by&Date/转录人及日期： |  |  |  |  |  |
| 3.34.9 Resin reswelling wash 2/溶胀后洗涤2 Turn on the nitrogen of synthesis reactor (TJ4S-1213-Sy15) to 5±2 m³/h (target: 5 m³/h). Bubble with nitrogen for 5±2 min (target: 5 min). Start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm). After stirring for 10±2 min (target: 10 min), stop stirring, turn off the nitrogen. Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point. 合成釜（TJ4S-1213-Sy15）开启下口鼓氮气, 氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min (目标：5 min)后，开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm)，搅拌10±2 min（目标：10 min）后，停止搅拌，关闭氮气。通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  |  |  | Times/次数 |
|  |  |  |  |  | 1 |

| 3.34. （11）Deprotection/脱保护 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Bubble/鼓氮气 |  |  | Performed by&Date/执行人及日期： |  |  |  |
| Bubble/鼓氮气 |  |  | Begin/开始： |  | End/结束： |  |
| Transcribed by&Date/转录人及日期： |  |  |  |  |  |  |
| Stir /搅拌 |  |  | Performed by&Date/执行人及日期： |  |  |  |
| Stir /搅拌 |  |  | Begin/开始： |  | End/结束： |  |
| Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point./通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  |  |  |  |  |
| Transcribed by&Date/转录人及日期： |  |  |  |  |  |  |
| Filter/过滤 |  | Performed by&Date/执行人及日期： |  |  |  |  |
|  |  | Checked by & Date/核实人及日期： |  |  |  |  |
|  |  | Begin/开始： |  | End/结束： |  |  |
| - （11）Fmoc-L-Glu(OtBu)-OH·H2O activation/芴甲氧羰基-L-谷氨酸-γ-叔丁酯一水合物活化 |  |  |  |  |  |  |
| CAUTION/注意：DMF/N，N-二甲基甲酰胺：Irritation of eyes, skin and nose. May cause nausea. /刺激眼睛、皮肤和鼻子。可能引起恶心。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |  |
| 3.35.1 Add DMF to the activation reactor/激活罐加N,N-二甲基甲酰胺： Add 227.52 kg (Acceptable weighing range: 220.69 ~234.34 kg) (5 V) DMF into activation tank (TJ4S-1213-Sy15-R01). Start the agitator of activation tank to 95±10 rpm (target: 95 rpm). 向激活罐（TJ4S-1213-Sy15-R01）中加入227.52 kg（可接受称量范围：220.69 kg ~ 234.34 kg ）（5 V）N，N-二甲基甲酰胺。开启激活罐搅拌转速至95±10 rpm (目标转速: 95 rpm)。 |  |  |  |  |  | Times/次数 |
|  |  |  |  |  |  | 1 |
| Add 227.52 kg DMF (Acceptable weighing range: 220.69 ~ 234.34 kg) (5 V) by spray balls into activation tank (TJ4S-1213-Sy15-R01).通过淋洗球向激活罐(TJ4S-1213-Sy15-R01)中加入227.52 kg N, N-二甲基甲酰胺（可接受称量范围：220.69~234.34 kg）(5 V)。 |  |  |  |  |  |  |

| 3.35. （11）Fmoc-L-Glu(OtBu)-OH·H2O activation/芴甲氧羰基-L-谷氨酸-γ-叔丁酯一水合物活化 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DMF addition / 加N,N-二甲基甲酰胺 |  | Performed by & Date/执行人及日期： |  |  |  |  |  |
| DMF addition / 加N,N-二甲基甲酰胺 |  | Begin/开始： |  | End/结束： |  | Qty/重量： |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |
| Start the agitator of activation tank to 95±10 rpm (target: 95 rpm)./ 开启激活罐搅拌转速至95±10 rpm (目标转速: 95 rpm)。 |  |  |  |  |  |  |  |
| Stir /搅拌 | Performed by & Date/执行人及日期： |  |  |  |  |  |  |
|  | Begin/开始： |  |  |  |  |  |  |
|  | The end time is documented in Ins 3.35.8/结束时间记录于3.35.8 |  |  |  |  |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |
| 3.35.2 Fmoc-L-Glu(OtBu)-OH·H2O addition /加芴甲氧羰基-L-谷氨酸-γ-叔丁酯一水合物 Add Fmoc-L-Glu(OtBu)-OH·H2O (2.00 eq.) with powder feeding bags into activation tank (TJ4S-1213-Sy15-R01). 通过投料袋向激活罐（TJ4S-1213-Sy15-R01）中加入 kg芴甲氧羰基-L-谷氨酸-γ-叔丁酯一水合物（Acceptable weighing range/可接受称量范围： kg ~ kg）(2.00 eq.)。 |  |  |  |  |  |  | Times/次数 |
|  |  |  |  |  |  |  | 1 |
| Before material charging, confirm the material is within the retest date/ the expiry date,Confirm whether the weight is adequate for use in the current charging. /加物料前，执行物料在复检日期/有效期之内，确认重量是否满足本次投料使用。 Before solid material charging, confirm/加固体物料前，确认： - Before performed, confirm there is no non-related material / equipment / instrument / tools / pipeline / document / record / label in the intended area /操作前，执行目标区域无不相关物料/设备/仪器/工具/管路/文件/记录/标签 - The outer packages are removed. If the outer package cannot be removed, but un-package material without damaging the package bag, and before charging, check no package fragment and other extraneous matter in material. /外包装移除；如果外包装不能移除，但在不破坏包装的情况下打开包装袋，投料前以及过程中同时检查执行物料中无包装碎片以及异物 Checked by & Date/核实人及日期： |  |  |  |  |  |  |  |
| Charge Fmoc-L-Glu(OtBu)-OH·H2O /加芴甲氧羰基-L-谷氨酸-γ-叔丁酯一水合物 |  |  | Performed by & Date/执行人及日期： |  |  |  |  |
|  |  |  | Begin/开始： |  | End/结束： |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |
| Fmoc-L-Glu(OtBu)-OH·H2O confirmation/芴甲氧羰基-L-谷氨酸-γ-叔丁酯一水合物执行：Control No./内部控制号YL005741 |  |  |  |  |  |  |  |

| 3.35. （11）Fmoc-L-Glu(OtBu)-OH·H2O activation/芴甲氧羰基-L-谷氨酸-γ-叔丁酯一水合物活化 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Check items/检查项 |  | Is it correct?/是否正确 | Recorded by & Date/ 记录人及日期 | Checked by & Date/ 核实人及日期 |
| Material name/物料名称 |  | Yes/是；No/否 |  |  |
| Project batch number/ 项目批次号 |  | Yes/是；No/否 |  |  |
| Operation number/ 操作号 |  | Yes/是；No/否 |  |  |
| Control No./内部控制号 |  | Yes/是；No/否 |  |  |
| Raman Scan Confirmation/拉曼扫描执行 |  | Conforms/相符 Does not conform/不相符 |  |  |
| No. /编号 | Net weight/净重 | All in/全部加入 | Recorded by & Date/ 记录人及日期 | Checked by & Date/ 核实人及日期 |
| / |  | Yes/是；No/否 |  |  |
| / |  | Yes/是；No/否 |  |  |
| / |  | Yes/是；No/否 |  |  |
| / |  | Yes/是；No/否 |  |  |
| / |  | Yes/是；No/否 |  |  |
| / |  | Yes/是；No/否 |  |  |
| / |  | Yes/是；No/否 |  |  |
| / |  | Yes/是；No/否 |  |  |
| / |  | Yes/是；No/否 |  |  |
| / |  | Yes/是；No/否 |  |  |
| / |  | Yes/是；No/否 |  |  |
| / |  | Yes/是；No/否 |  |  |
| Total/合计 |  | Confirm the total net weight is within the input range/执行合计净重在投料范围内 | Recorded by & Date/记录人及日期 | Checked by & Date/ 核实人及日期 |
|  |  | Yes/是；No/否 |  |  |

| 3.35. （11）Fmoc-L-Glu(OtBu)-OH·H2O activation/芴甲氧羰基-L-谷氨酸-γ-叔丁酯一水合物活化 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| After solid material charging, confirm/固体物料投料之后，确认： - The corresponding leftover material has been properly sealed and transferred to the staging area/相应的剩余物料已经恰当密封，转移到了暂存区域。 - The waste packaging material has been sealed and transferred to the corresponding waste staging area. /产生的包材废物已经密封并转移到了相应的废物暂存区域。 - The operation area/equipment surface has been cleaned/操作区域以及设备表面已经清洁。 Checked by & Date/核实人及日期： |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3.35.3 Agitation of activation tank /激活罐搅拌 Continue agitation of activation tank to 95±10 rpm (target: 95 rpm). Maintain stirring for at least 15 min. Confirm no visible solids and later use. 维持激活罐搅拌转速至95±10 rpm (目标转速: 95 rpm), 搅拌至少15 min，确认无明显固体待用。 |  |  |  |  |  |  |  |  |  |  |  | Times/次数 |  |
|  |  |  |  |  |  |  |  |  |  |  |  | 1 |  |
| Stir /搅拌 |  |  |  | Performed by & Date/执行人及日期： |  |  |  |  |  |  |  |  |  |
|  |  |  |  | End/结束： |  |  |  |  |  |  |  |  |  |
|  |  |  |  | The end time is documented in Ins 3.35.8/结束时间记录于3.35.8 |  |  |  |  |  |  |  |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Confirm no visible solid/确认无明显固体： |  |  |  |  |  | Confirmed by& Date /确认人及日期： |  |  |  |  |  |  |  |
| 3.35.4 Adjust activation tan temperature /激活罐调温 Adjust the activation tank (TJ4S-1213-Sy15-R01) mixture to 【20±5 ℃ (target: 20 ℃)】. /将激活罐(TJ4S-1213-Sy15-R01)中体系调温至【20±5 ℃（目标温度：20 ℃）】。 |  |  |  |  |  |  |  |  |  |  | Times/次数 |  |  |
|  |  |  |  |  |  |  |  |  |  |  | 1 |  |  |
| Adjust/调温 |  |  |  |  | Performed by&Date/执行人及日期： |  |  |  |  |  |  |  |
|  |  |  |  |  | Begin/开始： |  |  |  | End/结束： |  |  |  |
| Transcribed by&Date/转录人及日期： |  |  |  |  |  |  |  |  |  |  |  |  |
| CAUTION/注意：OxymaPure is highly toxic, high-energy compound. /2-肟氰乙酸乙酯是高毒，高能化合物。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |  |  |  |  |  |  |  |
| 3.35.5 OxymaPure addition /加2-肟氰乙酸乙酯 Add OxymaPure (2.00 eq.) with powder feeding bags into activation tank (TJ4S-1213-Sy15-R01) at 【20±5 ℃ (target 20 ℃) 】. 控温20±5 ℃（目标温度：20 ℃）】，通过投料袋向激活罐（TJ4S-1213-Sy15-R01）中加入 kg 2-肟氰乙酸乙酯（Acceptable weighing range/可接受称量范围： kg ~ kg）(2.00 eq.)。 |  |  |  |  |  |  |  |  |  |  | Times/次数 |  |
|  |  |  |  |  |  |  |  |  |  |  | 1 |  |
| 3.35. （11）Fmoc-L-Glu(OtBu)-OH·H2O activation/芴甲氧羰基-L-谷氨酸-γ-叔丁酯一水合物活化 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Before material charging, confirm the material is within the retest date/ the expiry date,Confirm whether the weight is adequate for use in the current charging. /加物料前，执行物料在复检日期/有效期之内，确认重量是否满足本次投料使用。 Before solid material charging, confirm/加固体物料前，确认： - Before operation performed, confirm there is no non-related material / equipment / instrument / tools / pipeline / document / record / label in the intended area /操作前；目标区域无不相关物料/设备/仪器/工具/管路/文件/记录/标签 - The outer packages are removed. If the outer package cannot be removed, but un-package material without damaging the package bag, and before charging, check no package fragment and other extraneous matter in material. /外包装移除；如果外包装不能移除，但在不破坏包装的情况下打开包装袋，投料前以及过程中同时检查执行物料中无包装碎片以及异物 Checked by & Date/核实人及日期： |  |  |  |  |  |  |  |  |  |  |  |  |
| Check and record that the solid charge port is free of solid accumulation from the previous step./ 检查并记录固体投料口无来自上一步骤的固体残留。 Checked by & Date/核实人及日期： |  |  |  |  |  |  |  |  |  |  |  |  |
| Charge OxymaPure /加2-肟氰乙酸乙酯 |  |  |  |  | Performed by & Date/执行人及日期： |  |  |  |  |  |  |  |
|  |  |  |  |  | Begin/开始： |  |  | End/结束： |  |  |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |  |  |  |  |  |
| OxymaPure confirmation/2-肟氰乙酸乙酯确认：Control No./内部控制号：YL005180 |  |  |  |  |  |  |  |  |  |  |  |  |
| Check items/检查项 |  |  | Is it correct?/是否正确 |  |  |  | Recorded by & Date/ 记录人及日期 |  |  | Checked by & Date/ 核实人及日期 |  |  |
| Material name/物料名称 |  |  | Yes/是；No/否 |  |  |  |  |  |  |  |  |  |
| Project batch number/ 项目批次号 |  |  | Yes/是；No/否 |  |  |  |  |  |  |  |  |  |
| Operation number/ 操作号 |  |  | Yes/是；No/否 |  |  |  |  |  |  |  |  |  |  |
| Control No./内部控制号 |  |  | Yes/是；No/否 |  |  |  |  |  |  |  |  |  |  |
| No. /编号 |  | Net weight/净重 | All in/全部加入 |  |  |  | Recorded by & Date/ 记录人及日期 |  |  | Checked by & Date/ 核实人及日期 |  |  |
| / |  |  | Yes/是；No/否 |  |  |  |  |  |  |  |  |  |
| / |  |  | Yes/是；No/否 |  |  |  |  |  |  |  |  |  |
| / |  |  | Yes/是；No/否 |  |  |  |  |  |  |  |  |  |
| / |  |  | Yes/是；No/否 |  |  |  |  |  |  |  |  |  |

| 3.35. （11）Fmoc-L-Glu(OtBu)-OH·H2O activation/芴甲氧羰基-L-谷氨酸-γ-叔丁酯一水合物活化 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No. /编号 |  | Net weight/净重 | All in/全部加入 |  | Recorded by & Date/ 记录人及日期 |  | Checked by & Date/ 核实人及日期 |
| / |  |  | Yes/是；No/否 |  |  |  |  |
| / |  |  | Yes/是；No/否 |  |  |  |  |
| / |  |  | Yes/是；No/否 |  |  |  |  |
| / |  |  | Yes/是；No/否 |  |  |  |  |
| / |  |  | Yes/是；No/否 |  |  |  |  |
| / |  |  | Yes/是；No/否 |  |  |  |  |
| / |  |  | Yes/是；No/否 |  |  |  |  |
| / |  |  | Yes/是；No/否 |  |  |  |  |
| Total/合计 |  |  | Confirm the total net weight is within the input range/执行合计净重在投料范围内 |  | Recorded by & Date/记录人及日期 |  | Checked by & Date/ 核实人及日期 |
|  |  |  | Yes/是；No/否 |  |  |  |  |
| After solid material charging, confirm/固体物料投料之后，确认： - The corresponding leftover material has been properly sealed and transferred to the staging area/相应的剩余物料已经恰当密封，转移到了暂存区域。 - The waste packaging material has been sealed and transferred to the corresponding waste staging area. /产生的包材废物已经密封并转移到了相应的废物暂存区域。 - The operation area/equipment surface has been cleaned/操作区域以及设备表面已经清洁。 Checked by & Date/核实人及日期： |  |  |  |  |  |  |  |
| Internal temperature/内部温度 |  |  | High values/最高值： |  | Low values/最低值： |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |
| CAUTION/注意：DMF/N，N-二甲基甲酰胺：Irritation of eyes, skin and nose. May cause nausea. /刺激眼睛、皮肤和鼻子。可能引起恶心。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |  |  |

| 3.35. （11）Fmoc-L-Glu(OtBu)-OH·H2O activation/芴甲氧羰基-L-谷氨酸-γ-叔丁酯一水合物活化 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 3.35.6 Rinse activation tank /激活罐淋洗 Rinse the port of Activation tank (TJ4S-1213-Sy15-R01) with 91.01 kg（Acceptable weighing range：88.28 ~ 93.74 kg）（2 V）. 用91.01 kg（可接受称量范围：88.28 ~ 93.74 kg）（2 V）N,N-二甲基甲酰胺淋洗激活罐（TJ4S-1213-Sy15-R01）加料口。 |  |  |  |  |  | Times/次数 |  |
|  |  |  |  |  |  | 1 |  |
| Rinse with DMF/ N,N-二甲基甲酰胺淋洗 | Performed by&Date/执行人及日期： |  |  |  |  |  |  |
| Rinse with DMF/ N,N-二甲基甲酰胺淋洗 | Begin/开始： | End/结束： | Qty/重量： |  |  |  |  |
| Transcribed by&Date/转录人及日期： |  |  |  |  |  |  |  |
| DIC is a key EHS concern chemical, highly toxic, skin sensitizing, respiratory sensitizing. /N,N’二异丙基碳二亚胺为EHS重点关注化学品，高毒，皮肤致敏，呼吸致敏。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |  |  |
| 3.35.7 DIC addition /加N,N'-二异丙基碳二亚胺 Add DIC (2.20 eq.) into the activation tank (TJ4S-1213-Sy15-R01) at reference time of 10-30 min at 【20±5 ℃ (target: 20 ℃) 】. The addition rate depends on temperature control. Stir for 20±5 min (target: 20 min) after addition. 控温【20±5 ℃ (目标温度：20 ℃) 】，向激活罐（TJ4S-1213-Sy15-R01）中滴加 kg N,N'-二异丙基碳二亚胺（Acceptable weighing range/可接受称量范围： kg ~ kg）(2.20 eq.)（滴加参考时间10-30 min，以控温为主），滴加完毕后搅拌20±5 min（目标时间：20 min）。 Pump number/泵编号： Scale ID. No./秤编号： Material charging pipe number/物料管路编号： |  |  |  |  |  | Times/次数 |  |
|  |  |  |  |  |  | 1 |  |

| 3.35. （11）Fmoc-L-Glu(OtBu)-OH·H2O activation/芴甲氧羰基-L-谷氨酸-γ-叔丁酯一水合物活化 |  |  |  |  |
| --- | --- | --- | --- | --- |
| ★Note:The DIC addition temperature is a CPP./ 备注：N,N'-二异丙基碳二亚胺加料温度是关键控制参数。 Prior to DIC addition, the operator shall confirm that the reaction mixture temperature is within the Normal Operating Range (NOR) / N,N-二异丙基碳二亚胺投料前，操作人员须执行反应体系温度处于正常操作范围（NOR）内: Checked by & Date/核实人及日期： ★Lower limit/低限: 15oC，High limit/高限: 25 oC Over-temperature warning value/超温预警设置数值： Lower limit/低限: 17 oC，High limit/高限: 23oC |  |  |  |  |  |
| Before material charging, confirm the material is within the retest date/ the expiry date. Confirm whether the weight is adequate for use in the current charging. /加物料前，执行物料在复检日期/有效期之内。确认重量是否满足本次投料使用。 Checked by & Date/核实人及日期： |  |  |  |  |  |
| Before drummed liquid material charging, visual check there is no other extraneous matter in material. /加桶装液体物料前，应目视执行桶内无异物。 Visual Examination of Foreign Matters for Drum Solvent/桶装溶剂异物目视检查: 1. Place the solvent drum in a well-lit area (or use a portable light source) to ensure adequate visibility of the liquid contents./ 将溶剂桶置于光线充足区域（或使用便携式光源），确保液体内容物可见性良好 2. Visually inspect the solvent through the drum bung opening or transparent sight glass for any visible foreign matters, including but not limited to particulates, fibers, sediments, or unusual turbidity./ 通过桶口或透明视镜目视检查溶剂中是否有可见异物，包括但不限于颗粒、纤维、沉淀物或异常浑浊。 3. Confirm that the solvent is clear and free of visible foreign matters before charging. If any foreign matter is observed, do not use the drum and follow the applicable handling procedure for subsequent disposition./ 投料前确认溶剂澄清且无可见异物。如发现任何异物，不得使用该桶，并依据处理流程进行后续处置。 Checked by & Date/核实人及日期： |  |  |  |  |
| Confirm liquid charging pipeline is released. /确认加料管路经过放行。 |  |  |  |  |
| Checked by & Date/核实人及日期： |  |  |  |  |
| Charging of drummed solvent: verify that the pipeline connections remain consistent before and after material feeding, then record the weighing reading 桶装溶剂溶液称量时，确认投料前后管路连接一致，读取称量示数。 Performed by & Date/执行人及日期： Checked by & Date/核实人及日期： |  |  |  |  |
| DIC /N,N’-二异丙基碳二亚胺：Control Number/内部控制号：YL003564 |  |  |  |  |
| Internal temperature/内部温度 |  |  | High values/最高值： | Low values/最低值： |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |

3.35. （11）Fmoc-L-Glu(OtBu)-OH·H2O activation/芴甲氧羰基-L-谷氨酸-γ-叔丁酯一水合物活化

If the target net weight≥the planned net weight, perform full drum charge; otherwise, perform partial (Non-full drum) charge. /当目标加料量≥计划加入净重，勾选进行整桶加料；否则勾选进行零头加料。 Note: Target net weight to be charged = Total target net weight – Total net weight charged. Expected tare weight of drum after charge = Gross weight of drum that to be charged - Target net weight to be charged. 还需加入净重目标值= 加料步骤目标总净重-已加入净重总量 加料完成后预计桶皮重=待加料桶毛重-还需加入净重目标值

| 3.35. （11）Fmoc-L-Glu(OtBu)-OH·H2O activation/芴甲氧羰基-L-谷氨酸-γ-叔丁酯一水合物活化 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Addition type加料式 | Lot number/批号 | Pre-charging qty calculation/投料前投料量计算 |  |  |  |  |  | Actual Charging Qty/实际投料量 |  |  |  |  |  |
|  |  | Total qty charged/已加入净重总量 |  | Target qty to be charged/还需加入净重目标值 | Gross qty of drum that to be charged/待加料桶毛重 | Expected net weight of drum after charge /加料完成后预计桶净重 | Expected tare weight of drum after charge/加料完成后预计桶皮重 | Before charging/ 投料前 |  | After charging/ 投料后 | Net weight/ 净重 | Performed by & Date/ 执行人及日期 | Checked by & Date/ 核实人及日期 |
| Full drum整桶 Non-full drum /非整桶 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Full drum整桶 Non-full drum /非整桶 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Total/合计 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DIC Addition/加N,N'-二异丙基碳二亚胺 |  |  | Begin/开始： |  |  |  |  |  | End/结束： |  |  |  |  |
| Transcribed by & Date/转录人及日期： |  |  |  |  |  |  |  |  |  |  |  |  |  |

| 3.35. （11）Fmoc-L-Glu(OtBu)-OH·H2O activation/芴甲氧羰基-L-谷氨酸-γ-叔丁酯一水合物活化 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 3.35.8 Fmoc-L-Glu(OtBu)-OH·H2O activation/芴甲氧羰基-L-谷氨酸-γ-叔丁酯一水合物活化 At 5±5 ℃ (target: 5 ℃), continue agitation of activation tank to 95±10 rpm (target: 95 rpm). Maintain stirring for 20±5 min (target: 20 min) after addition, stop stirring. 滴毕，控温5±5 ℃(目标温度：5 ℃)，维持激活罐搅拌转速至95±10 rpm (目标转速: 95 rpm)，继续搅拌20±5 min(目标时间：20 min) ，停止搅拌。 |  |  |  |  | Times/次数 |
|  |  |  |  |  | 1 |
| Stir /搅拌 |  | Performed by & Date/执行人及日期： |  |  |  |
| Stir /搅拌 |  | End/结束 |  |  |  |
|  |  | The start time is documented in Ins3.35.1 /开始时间记录于3.35.1 |  |  |  |
| Transcribed by & Date/转录人及日期： |  |  |  |  |  |
| Internal temperature/内部温度 |  | High values/最高值： |  | Low values/最低值： |  |
| Transcribed by & Date/转录人及日期： |  |  |  |  |  |
| 3.36. （11）Fmoc-L-Glu(OtBu)-OH·H2O coupling/芴甲氧羰基-L-谷氨酸-γ-叔丁酯一水合物偶联 |  |  |  |  |  |
| 3.36.1 Activation mixture transfer /活化体系转移 Transfer the mixture in activation tank (TJ4S-1213-Sy15-R01) into synthesis reactor (TJ4S-1213-Sy15). 将激活罐（TJ4S-1213-Sy15-R01）中体系转移至合成釜（TJ4S-1213-Sy15）中。 |  |  |  |  | Times/次数 |
|  |  |  |  |  | 1 |
| ★Note:The coupling temperature is a CPP, and the coupling IPC hold temperature is a CPP./ 备注：偶联温度是CPP，偶联IPC维持温度是关键工艺参数。 The operator shall confirm that the reaction mixture temperature is within the Normal Operating Range (NOR) 【25±5oC (target: 25oC)】 /操作人员须执行反应体系温度处于正常操作范围（NOR）内【25±5oC (target: 25oC)】 Checked by & Date/核实人及日期： Over-temperature warning value/超温预警设置数值： Lower limit/低限: 22 oC ，High limit/高限: 28 oC |  |  |  |  |  |
| Transfer/转移 | Performed by&Date/执行人及日期： |  |  |  |  |
|  | Checked by & Date/核实人及日期： |  |  |  |  |
|  | Begin/开始： |  | End/结束： |  |  |

| 3.36. （11）Fmoc-L-Glu(OtBu)-OH·H2O coupling/芴甲氧羰基-L-谷氨酸-γ-叔丁酯一水合物偶联 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CAUTION/注意：DMF/N，N-二甲基甲酰胺：Irritation of eyes, skin and nose. May cause nausea. /刺激眼睛、皮肤和鼻子。可能引起恶心。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |  |  |
| 3.36.2 DMF addition /加N,N-二甲基甲酰胺 Add 45.50 kg (Acceptable weighing range: 43.50 ~ 47.50 kg) (1 V) DMF by spray balls into activation tank (TJ4S-1213-Sy15-R01) to rinse the wall. 通过淋洗球向激活罐（TJ4S-1213-Sy15-R01）中加入45.50 kg（可接受称量范围：43.50 ~ 47.50 kg） (1 V) N,N-二甲基甲酰胺淋洗罐壁。 |  |  |  |  |  | Times/次数 |  |
|  |  |  |  |  |  | 1 |  |
| Rinse with DMF/ N,N-二甲基甲酰胺淋洗 |  | Performed by&Date/执行人及日期： |  |  |  |  |  |  |
| Rinse with DMF/ N,N-二甲基甲酰胺淋洗 |  | Begin/开始： | End/结束： | Qty/重量： |  |  |  |  |
| Transcribed by&Date/转录人及日期： |  |  |  |  |  |  |  |  |
| 3.36.3 Transfer /转移 Turn on the nitrogen of Synthesis reactor (TJ4S-1213-Sy15) to 5±2 m³/h (target: 5 m³/h). Bubble with nitrogen for 5±2 min (target: 5 min), start the agitator of synthesis reactor to 20±10rpm (target: 20 rpm). Transfer the mixture in activation tank (TJ4S-1213-Sy15-R01) into synthesis reactor (TJ4S-1213-Sy15). 合成釜（TJ4S-1213-Sy15）先开启下口氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min (目标时间：5 min)后，开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm)，将激活罐（TJ4S-1213-Sy15-R01）中体系转移至合成釜（TJ4S-1213-Sy15）中。 |  |  |  |  |  | Times/次数 |  |  |
|  |  |  |  |  |  | 1 |  |  |
| Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h). Bubble with nitrogen for 5±2 min (target: 5 min) 开启合成釜下口鼓氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min (目标时间：5 min) |  |  |  |  |  |  |  |  |
| Bubble/鼓氮气 |  | Performed by & Date/执行人及日期： |  |  |  |  |  |  |
| Bubble/鼓氮气 |  | Begin/开始： |  |  |  |  |  |  |
|  |  | The end time is documented in Ins3.36.5 /结束时间记录于3.36.5 |  |  |  |  |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |  |

| 3.36. （11）Fmoc-L-Glu(OtBu)-OH·H2O coupling/芴甲氧羰基-L-谷氨酸-γ-叔丁酯一水合物偶联 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm). /开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm) |  |  |  |  |  |  |
| Stir /搅拌 |  | Performed by & Date/执行人及日期： |  |  |  |  |
| Stir /搅拌 |  | Begin/开始： |  |  |  |  |
|  |  | The end time is documented in Ins3.36.5 /结束时间记录于3.36.5 |  |  |  |  |
| Transcribed by&Date/转录人及日期： |  |  |  |  |  |  |
| Transfer/转移 |  |  | Performed by&Date/执行人及日期： |  |  |
|  |  |  | Checked by & Date/核实人及日期： |  |  |
|  |  |  | Begin/开始： | End/结束： |  |
| 3.36.4 Adjust synthesis reactor temperature /合成釜调温 Adjust the synthesis reactor (TJ4S-1213-Sy15) mixture to【25±5 ℃ (target: 25 ℃)】. /将合成釜（TJ4S-1213-Sy15）体系调温至【25±5 ℃(目标温度：25 ℃)】. |  |  |  |  | Times/次数 |
|  |  |  |  |  | 1 |
| The total reaction Fmoc-L-Leu-OH coupling time：NMT 840 min (Sourced from the holding time study in the PAR study report.)/ 芴甲氧羰基-L-亮氨酸偶联总的反应时间不超过840min(来源于 PAR 研究报告中稳定时间研究)。 Over-temperature warning value/超温预警设置数值： Lower limit/低限: 22 oC ，High limit/高限: 28 oC |  |  |  |  |  |
| Adjust/调温 |  |  | Performed by&Date/执行人及日期： |  |  |
| Adjust/调温 |  |  | Begin/开始： | End/结束： |  |
| Transcribed by&Date/转录人及日期： |  |  |  |  |  |

| 3.36. （11）Fmoc-L-Glu(OtBu)-OH·H2O coupling/芴甲氧羰基-L-谷氨酸-γ-叔丁酯一水合物偶联 |  |  |
| --- | --- | --- |
| 3.36.5 Fmoc-L-Glu(OtBu)-OH·H2O coupling/芴甲氧羰基-L-谷氨酸-γ-叔丁酯一水合物偶联 Start timing when temperature reaches 20 ℃.The mixture is allowed to react at 【25±5 ℃ (target: 25 ℃) 】. After 6 h±15 min (target: 6 h), sample the mixture every 0.5~1 h for bromophenol blue test analysis until two consecutive samples test as negative (Yellow to Orange Report Negative = Complete coupling; Blue to Black Positive = Incomplete coupling), and the mixture should be stirred for coupling time (8 h≤Coupling Time≤18 h). Stop stirring, turn off the nitrogen. Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point. 控温【25±5 oC（目标温度25 oC，20 oC开始计时）】，反应 6 h±15 min（目标：6 h）后，取样进行溴酚蓝检测，每隔0.5-1.0 h取样一次，直至连续两次取样结果为阴性（黄色至橙色报告为阴性=完全偶联；蓝色至黑色为阳性=未完全偶联）后同时反应时间满足（8 h≤反应时间≤18 h）后，停止搅拌，关闭氮气。通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 Note: If resin adherence is found at other times during the reaction, rinse the reactor with 0.50~5.00 kg DMF each time. /备注：偶联过程中其它时间发现贴壁，每次使用0.50~5.00 kg N,N-二甲基甲酰胺淋洗，记录见附加页 |  | Times/次数 |
|  |  | 1 |

| 3.36. （11）Fmoc-L-Glu(OtBu)-OH·H2O coupling/芴甲氧羰基-L-谷氨酸-γ-叔丁酯一水合物偶联 |  |  |  |
| --- | --- | --- | --- |
| ★After coupling is completed, the operator shall confirm that all Critical Process Parameters (CPP) are within their respective Normal Operating Ranges (NOR)/ 偶联结束后，操作人员须确认所有关键工艺参数（CPP）均处于各自的正常操作范围（NOR）内 ★Note: The coupling temperature is a CPP, and the coupling IPC hold temperature is a CPP./备注：偶联温度是CPP，偶联IPC维持温度是关键工艺参数。 The operator shall confirm that the reaction mixture temperature is within the Normal Operating Range (NOR) :20~30℃ /操作人员须确认反应体系温度处于正常操作范围（NOR）内:20~30℃ □Yes/是 □No/否 Confirmed by & Date/确认人及日期： Checked by & Date/核实人及日期： |  |  |  |
| React/反应 |  | Performed by&Date/执行人及日期： |  |
| React/反应 |  | Begin/开始： | End/结束： |
| Transcribed by&Date/转录人及日期： |  |  |  |

| 3.36. （11）Fmoc-L-Glu(OtBu)-OH·H2O coupling/芴甲氧羰基-L-谷氨酸-γ-叔丁酯一水合物偶联 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Internal temperature/内部温度 |  |  | High values/最高值： |  |  | Low values/最低值： |  |  |
| Transcribed by&Date/转录人及日期： |  |  |  |  |  |  |  |  |
| Standard sampling method/标准取样方法 |  |  |  |  |  |  |  |  |
| Take 5-10 ml mixture and perform testing in accordance with the ninhydrin test record. During sampling, should wear nitrile gloves and 3M 6000 series Full-face piece respirator, 6006CN cartridges. 通过在线取样器取样5~10 ml按照茚三酮检测记录进行检测。取样过程中，应该戴丁腈手套、3M 6000系列全面罩, 6006CN滤毒盒。 |  |  |  |  |  |  |  |  |
| Sampling number/ 取样编号 |  | Sampling time/取样时间 |  | Sampled by & Date/取样人及日期 | Negative or Positive/阴性或阳性 |  | Result filled by &Date/结果填写人及日期 | Checked by & Date/核实人及日期 |
| CPo140020- -01- -3.36.5-01 |  |  |  |  | Negative /阴性 Positive /阳性 |  |  |  |
| CPo140020- -01- -3.36.5-02 |  |  |  |  | Negative /阴性 Positive /阳性 |  |  |  |
| CPo140020- -01- -3.36.5-03 |  |  |  |  | Negative /阴性 Positive /阳性 |  |  |  |
| CPo140020- -01- -3.36.5-04 |  |  |  |  | Negative /阴性 Positive /阳性 |  |  |  |
| CPo140020- -01- -3.36.5-05 |  |  |  |  | Negative /阴性 Positive /阳性 |  |  |  |
| CPo140020- -01- -3.36.5-06 |  |  |  |  | Negative /阴性 Positive /阳性 |  |  |  |
| CPo140020- -01- -3.36.5-07 |  |  |  |  | Negative /阴性 Positive /阳性 |  |  |  |
| CPo140020- -01- -3.36.5-08 |  |  |  |  | Negative /阴性 Positive /阳性 |  |  |  |
| Confirm the end point of the reaction. /执行反应终点。 The acceptance criterion is two consecutive negative test results./连续两次检测结果为阴性即为合格。 Confirmed by & Date/确认人及日期： Checked by & Date/核实人及日期： |  |  |  |  |  |  |  |  |

| 3.36. （11）Fmoc-L-Glu(OtBu)-OH·H2O coupling/芴甲氧羰基-L-谷氨酸-γ-叔丁酯一水合物偶联 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Stop the stirrer and turn off the nitrogen. Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point./ 停止搅拌，关闭氮气，通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  |  |  |  |
| Stir /搅拌 |  | Performed by & Date/执行人及日期： |  |  |  |
| Stir /搅拌 |  | End/结束： |  |  |  |
|  |  | The start time is documented in Ins 3.36.3 /开始时间记录于3.36.3 |  |  |  |
| Transcribed by & Date/转录人及日期： |  |  |  |  |  |
| Bubble/鼓氮气 |  |  | Performed by & Date/执行人及日期： |  |  |
| Bubble/鼓氮气 |  |  | End/结束： |  |  |
|  |  |  | The start time is documented in Ins 3.36.3 /开始时间记录于3.36.3 |  |  |
| Transcribed by & Date/转录人及日期： |  |  |  |  |  |
| Internal temperature/内部温度 |  | High values/最高值： |  |  | Low values/最低值： |
| Transcribed by & Date/转录人及日期： |  |  |  |  |  |
| Filter/过滤 |  |  | Performed by&Date/执行人及日期： |  |  |
|  |  |  | Checked by & Date/核实人及日期： |  |  |
|  |  |  | Begin/开始： | End/结束 |  |
| ★After coupling is completed, the operator shall confirm that all Critical Process Parameters (CPPs) are within their respective Normal Operating Ranges (NOR)/ 偶联结束后，操作人员须确认所有关键工艺参数（CPP）均处于各自的正常操作范围（NOR）内 Checked by & Date/核实人及日期： |  |  |  |  |  |
| CAUTION/注意：DMF/N，N-二甲基甲酰胺：Irritation of eyes, skin and nose. May cause nausea. /刺激眼睛、皮肤和鼻子。可能引起恶心。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |

| 3.36. （11）Fmoc-L-Glu(OtBu)-OH·H2O coupling/芴甲氧羰基-L-谷氨酸-γ-叔丁酯一水合物偶联 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 3.36.6 Activation Tank Rinse/激活罐洗涤 Add 364.03 kg (Acceptable weighing range：353.11 ~ 374.95 kg) (8 V) DMF into the activation tank (TJ4S-1213-Sy15-R01). Circulate and clean the activating liquid transfer pipeline, liquid charging port cleaning pipeline and solid charging port cleaning pipeline. Clean every line wash for 2±1 min (target: 2 min). 向激活罐（TJ4S-1213-Sy15-R01）中加入364.03 kg（可接受称量范围：353.11 ~ 374.95 kg） (8 V) N,N-二甲基甲酰胺，循环清洗激活罐转移管路，液体加料口清洗管路和固体加料口清洗管路，每条线路各清洗2±1 min（目标：2 min）。 |  |  |  |  |  | Times/次数 |
|  |  |  |  |  |  | 3 |
| Operation name/ 操作名称 |  | Time point/时间节点 | Clean the activating liquid transfer pipeline 1/循环清洗激活液转移管路1 | Liquid charging port cleaning pipeline 2 /液体加料口清洗管路2 | Solid charging port cleaning pipeline 3,/固体加料口清洗管路3 |  |
|  |  |  | Wash /洗涤1 | Wash /洗涤2 | Wash /洗涤3 |  |
| DMF addition/ 加N,N-二甲基甲酰胺 |  | Performed by & Date/执行人及日期： |  |  |  |  |
| DMF addition/ 加N,N-二甲基甲酰胺 |  | Begin/开始 |  |  |  |  |
|  |  | End/结束 |  |  |  |  |
|  |  | Qty/重量 |  |  |  |  |
|  |  | Transcribed by & Date /转录人及日期： |  |  |  |  |
| Circulate and clean/循环清洗 |  | Performed by & Date/执行人及日期： |  |  |  |  |
| Circulate and clean/循环清洗 |  | Begin/开始 |  |  |  |  |
|  |  | End/结束 |  |  |  |  |
|  |  | Transcribed by & Date /转录人及日期： |  |  |  |  |

| 3.36. （11）Fmoc-L-Glu(OtBu)-OH·H2O coupling/芴甲氧羰基-L-谷氨酸-γ-叔丁酯一水合物偶联 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 3.36.7 Post-coupling wash./偶联后洗涤 Transfer the mixture in activation tank (TJ4S-1213-Sy15-R01) to synthesis reactor (TJ4S-1213-Sy15). Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h). Bubble with nitrogen, after 5±2 min (target 5 min). Start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm). After stirring for 7±2 min (target：7 min), stop stirring, turn off the nitrogen. Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point. 将激活罐（TJ4S-1213-Sy15-R01）洗涤液转入合成釜 (TJ4S-1213-Sy15)。开启合成釜下口鼓氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min (目标时间：5 min)后开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm)，搅拌7±2 min（目标时间：7 min）后停止搅拌，关闭氮气。通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  |  |  |  | Times/次数 |
|  |  |  |  |  |  | 3 |
| Transfer the liquid form activation tank (TJ4S-1213-Sy15-R01) to the synthesis reactor (TJ4S-1213-Sy15) ./将激活罐(TJ4S-1213-Sy15-R01)中体系转移至合成釜(TJ4S-1213-Sy15)中。 |  |  |  |  |  |  |
| Operation name/ 操作名称 |  | Time point/时间节点 | Wash /洗涤1 | Wash /洗涤2 | Wash /洗涤3 |  |
| Transfer/转移 |  | Performed by & Date/执行人及日期： |  |  |  |  |
|  |  | Checked by & Date/核实人及日期： |  |  |  |  |
|  |  | Begin/开始 |  |  |  |  |
|  |  | End/结束 |  |  |  |  |
| Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h), Bubble with nitrogen for 5±2 min (target:5 min). Start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm). After stirring for 7±2 min (target: 7 min), stop stirring, turn off the nitrogen./ 开启合成釜下口鼓氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min（目标时间：5 min）后开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm)，搅拌7±2 min（目标时间：7 min）后，停止搅拌，关闭氮气。 |  |  |  |  |  |  |

| 3.36. （11）Fmoc-L-Glu(OtBu)-OH·H2O coupling/芴甲氧羰基-L-谷氨酸-γ-叔丁酯一水合物偶联 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Operation name/ 操作名称 |  | Time point/时间节点 | Wash /洗涤1 | Wash /洗涤2 | Wash /洗涤3 |
| Bubble/鼓氮气 |  | Performed by & Date/执行人及日期： |  |  |  |
| Bubble/鼓氮气 |  | Begin/开始 |  |  |  |
|  |  | End/结束 |  |  |  |
|  |  | Transcribed by & Date /转录人及日期： |  |  |  |
| Stir /搅拌 |  | Performed by & Date/执行人及日期： |  |  |  |
| Stir /搅拌 |  | Begin/开始 |  |  |  |
|  |  | End/结束 |  |  |  |
|  |  | Transcribed by & Date /转录人及日期： |  |  |  |
| Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point/通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  |  |  |  |
| Filter/过滤 |  | Performed by & Date/执行人及日期： |  |  |  |
|  |  | Checked by & Date/核实人及日期： |  |  |  |
|  |  | Begin/开始 |  |  |  |
|  |  | End/结束 |  |  |  |

| - （12）Deprotection/脱保护 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CAUTION/注意：Piperidine哌啶：Strong local irritant and may cause permanent injury after short exposure to small amounts. /强烈的局部刺激，短时间少量接触可能造成永久性伤害。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |  |  |  |
| 3.37.1 Add piperidine solution/加哌啶溶液 Adjust the jacket temperature of synthesis reactor (TJ4S-1213-Sy15) to 【25±5 oC (target: 25 oC)】. Add 357.50 kg (Acceptable weighing range: 346.77 ~ 368.23 kg) 【20% (V/V) piperidine/DMF solution】 into the Synthesis reactor (TJ4S-1213-Sy15). 将合成釜(TJ4S-1213-Sy15)夹套调温至【25±5 ℃（目标温度：25 ℃）】。向合成釜（TJ4S-1213-Sy15）中加入357.50 kg（可接受称量范围：346.77 ~ 368.23 kg）【20%哌啶/DMF (V/V)溶液】。 |  |  |  |  |  | Times/次数 |  |  |
|  |  |  |  |  |  | 3 |  |  |
| Adjust/调温 |  | Performed by & Date/执行人及日期： |  |  |  |  |  |  |
| Adjust/调温 |  | Begin/开始： |  | Adjust/调温 |  |  |  |  |
| Transcribed by & Date/转录人及日期： |  |  |  |  |  |  |  |  |
| Confirm and record that the 20%Piperidine solution concentration is within the NOR range./ 确认并记录20%哌啶溶液浓度在正常操作范围内：17.50%~22.50% 20%Piperidine solution concentration confirmation/20%哌啶溶液浓度确认：____________ Confirm and record that the 20%Piperidine solution temperature is within the NOR range./ 确认并记录20%哌啶溶液温度在正常操作范围内：20 oC ~30 oC 20%Piperidine solution temperature confirmation/20%哌啶溶液温度确认：____________ Confirmed by & Date/确认人及日期： Checked by & Date/核实人及日期： |  |  |  |  |  |  |  |  |
| Add Piperidine addition/ 加哌啶溶液 |  | Performed by & Date /执行人及日期： |  |  |  |  |  |
| Add Piperidine addition/ 加哌啶溶液 |  | Begin/开始： |  | End/结束： |  | Qty/重量： |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |

| 3.37. （12）Deprotection/脱保护 |  |  |
| --- | --- | --- |
| 3.37.2 Adjust synthesis reactor temperature /合成釜调温 Turn on the nitrogen of Synthesis reactor (TJ4S-1213-Sy15) to 5±2 m³/h (target: 5 m³/h). Bubble with nitrogen for 5±2 min (target 5 min). Start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm). then start the stirrer, adjust to 25±5 ℃ (target: 25 ℃）. 开启合成釜（TJ4S-1213-Sy15）下口鼓氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min (目标：5 min)后开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm)，调温至【25±5 ℃(目标温度：25 ℃)】。 |  | Times/次数 |
|  |  | 3 |
| Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h). Bubble with nitrogen for 5±2 min (target :5 min) 开启合成釜下口鼓氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min (目标时间：5 min) |  |  |
| Bubble/鼓氮气 | Performed by & Date/执行人及日期： |  |
| Bubble/鼓氮气 | Begin/开始： |  |
|  | The end time is documented in Ins 3.37.3/结束时间记录于3.37.3 |  |
| Transcribed by & Date /转录人及日期: |  |  |
| Start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm). /开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm) |  |  |
| Stir /搅拌 | Performed by & Date/执行人及日期： |  |
| Stir /搅拌 | Begin/开始： |  |
|  | The end time is documented in Ins 3.37.3/结束时间记录于3.37.3 |  |
| Transcribed by & Date /转录人及日期: |  |  |
| Adjust to 25±5 oC (target: 25 oC)./ 调温至25±5 ℃(目标温度：25 oC) |  |  |

| 3.37. （12）Deprotection/脱保护 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Adjust/调温 |  | Performed by & Date/执行人及日期： |  |  |
|  |  | Begin/开始： | End/结束： |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |
| 3.37.3 Deprotection of Fmoc-L-Ala-OH·H2O/芴甲氧羰基-L-丙氨酸一水合物脱保护 After stirring for 【2 h±15 min (target: 2 h) 】at 【25±5 ℃ (target: 25 ℃) 】, stop stirring, turn off the nitrogen. Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point. 控温【25±5 ℃(目标：25 ℃) 】，搅拌【2 h±15 min（目标：2 h）】后，停止搅拌，关闭氮气。通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 Over-temperature warning value/超温预警设置数值： Lower limit/低限: 22 oC ，High limit/高限: 28 oC |  |  |  | Times/次数 |
|  |  |  |  | 3 |
| ★Confirm and record that the 20%Piperidine solution temperature is within the NOR range 20 oC ~30 oC ./ 确认并记录20%哌啶溶液温度在正常操作范围内：20 oC ~30 oC。 20%Piperidine solution temperature confirmation/20%哌啶溶液温度确认：____________ Confirmed by& Date/确认人及日期： Checked by & Date/核实人及日期： |  |  |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |
| De-Fmoc/脱Fmoc &Stir /搅拌 | Performed by & Date/执行人及日期： |  |  |  |
| De-Fmoc/脱Fmoc &Stir /搅拌 | End/结束： |  |  |  |
|  | The start time is documented in Ins3.37.2 /开始时间记录于3.37.2 |  |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |

| 3.37. （12）Deprotection/脱保护 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Bubble/鼓氮气 |  |  | Performed by & Date/执行人及日期： |  |
| Bubble/鼓氮气 |  |  | End/结束： |  |
|  |  |  | The start time is documented in Ins3.37.2 /开始时间记录于3.37.2 |  |
| Transcribed by & Date /转录人及日期: |  |  |  |  |
| Internal temperature/内部温度 |  | High values/最高值: |  | Low values/最低值： |
| Transcribed by & Date /转录人及日期: |  |  |  |  |
| Filter/过滤 | Performed by&Date/执行人及日期： |  |  |  |
|  | Checked by & Date/核实人及日期： |  |  |  |
|  | Begin/开始： |  |  | End/结束 |
| ★After De-Fmoc is completed, the operator shall confirm that all Critical Process Parameters (CPPs) are within their respective Normal Operating Ranges (NOR)/ 脱保护结束后，操作人员须确认所有关键工艺参数（CPP）均处于各自的正常操作范围（NOR）内 Checked by & Date/核实人及日期： |  |  |  |  |

| 3.37. （12）Deprotection/脱保护 |  |  |
| --- | --- | --- |
| Transcribed by & Date /转录人及日期: |  |  |
| 3.37.4 Synthesis reactor adjust temperature/合成釜调温 Adjust the synthesis reactor (TJ4S-1213-Sy15) jacket to 【20±5 ℃ (target: 20 ℃) 】. 将合成釜（TJ4S-1213-Sy15）夹套调温至【20±5 ℃（目标温度：20 ℃）】。 |  | Times/次数 |
|  |  | 3 |
| Adjust/调温 | Performed by & Date/执行人及日期： |  |
| Adjust/调温 | Begin/开始： |  |
| Transcribed by & Date /转录人及日期： |  |  |
| CAUTION/注意：IPAC/乙酸异丙酯： Vapors irritate eyes and respiratory tract; high concentrations can be anesthetic. Liquid irritates eyes but causes no serious injury; may cause dermatitis; no serious effects if swallowed. /蒸汽会刺激眼睛和呼吸道；高浓度的蒸汽可能是麻醉剂。液体刺激眼睛，但不会造成严重伤害；可能会引起皮炎；吞食不会造成严重影响。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |
| 3.37.5 Post-deprotection wash/脱保护后洗涤 Add 445.92 kg (Acceptable weighing range：432.54 ~ 459.30 kg) (9.29 kg/kg, 10.64 V) isopropyl acetate into synthesis reactor (TJ4S-1213-Sy15). Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h). Bubble with nitrogen for 5±2 min (target 5 min), start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm). After stirring for 5±2 min (target 5 min), stop stirring, turn off the nitrogen. Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point. 向合成釜（TJ4S-1213-Sy15）中加入445.92 kg（可接受称量范围：432.54 ~ 459.30 kg) (9.29 kg/kg, 10.64 V）乙酸异丙酯，开启下口氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min (目标时间：5 min)后开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm)，搅拌5±2 min (目标时间：5 min)后，停止搅拌，关闭氮气。通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  | Times/次数 |
|  |  | 4 |
| ★Note: IPAC quantity is a CPP./备注：乙酸异丙酯用量是关键工艺参数。 ★The operator shall confirm that IPAC quantity is within the Normal Operating Range (NOR) 【432.54 kg~459.30 kg】/操作人员须确认乙酸异丙酯用量处于正常操作范围（NOR）内：【432.54 kg~459.30 kg】 Checked by & Date/核实人及日期： |  |  |

| 3.37. （12）Deprotection/脱保护 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Operation name/操作名称 |  | Time point/时间节点 | Wash /洗涤1 | Wash /洗涤2 | Wash /洗涤3 | Wash /洗涤4 |
| Add 419.04 kg IPAc (10 V) by spray balls into the synthesis reactor (TJ4S-1213-Sy15)./ 向合成釜(TJ4S-1213-Sy15) 中加419.04 kg乙酸异丙酯（10 V)（Acceptable weighing range/可接受称量范围：406.47~431.61 kg）。 |  |  |  |  |  |  |
| IPAC addition/加乙酸异丙酯 |  | Performed by & Date/执行人及日期： |  |  |  |  |
| IPAC addition/加乙酸异丙酯 |  | Begin/开始 |  |  |  |  |
|  |  | End/结束 |  |  |  |  |
|  |  | Qty/重量 |  |  |  |  |
|  |  | Transcribed by & Date /转录人及日期： |  |  |  |  |
| Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h). Bubble with nitrogen for 5±2 min (target: 5 min). Start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm). Stir for 5±2 min (target: 5 min), stop stirring, turn off the nitrogen./ 开启合成釜下口鼓氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min（目标时间：5 min）后开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm)，搅拌5±2 min（目标时间：5 min）后，停止搅拌，关闭氮气。 |  |  |  |  |  |  |
| Bubble/鼓氮气 |  | Performed by & Date/执行人及日期： |  |  |  |  |
| Bubble/鼓氮气 |  | Begin/开始 |  |  |  |  |
|  |  | End/结束 |  |  |  |  |
|  |  | Transcribed by & Date /转录人及日期： |  |  |  |  |
| Stir /搅拌 |  | Performed by & Date/执行人及日期： |  |  |  |  |
| Stir /搅拌 |  | Begin/开始 |  |  |  |  |
|  |  | End/结束 |  |  |  |  |
|  |  | Transcribed by & Date /转录人及日期： |  |  |  |  |

| 3.37. （12）Deprotection/脱保护 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operation name/操作名称 | Time point/时间节点 |  | Wash /洗涤1 | Wash /洗涤2 | Wash /洗涤3 |  | Wash /洗涤4 |  |
| Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point./通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  |  |  |  |  |  |  |
| Filter/过滤 | Performed by & Date/执行人及日期： |  |  |  |  |  |  |  |
|  | Checked by & Date/核实人及日期： |  |  |  |  |  |  |  |
|  | Begin/开始 |  |  |  |  |  |  |  |
|  | End/结束 |  |  |  |  |  |  |  |
| DMF/N，N-二甲基甲酰胺：Irritation of eyes, skin and nose. May cause nausea. /刺激眼睛、皮肤和鼻子。可能引起恶心。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |  |  |  |
| 3.37.6 DMF addition /加N,N-二甲基甲酰胺 Add 546.05 kg (Acceptable weighing range: 529.67 ~ 562.43 kg) (12 V) DMF into synthesis reactor (TJ4S-1213-Sy15). 向合成釜（TJ4S-1213-Sy15）中加入546.05 kg（可接受称量范围：529.67 ~ 562.43 kg）（12 V）N,N-二甲基甲酰胺。 |  |  |  |  |  |  |  | Times/次数 |
|  |  |  |  |  |  |  |  | 1 |
| DMF addition/ 加N,N-二甲基甲酰胺 |  | Performed by&Date/执行人及日期： |  |  |  |  |  |  |
| DMF addition/ 加N,N-二甲基甲酰胺 |  | Begin/开始： |  | End/结束： |  | Qty/重量： |  |  |
| Transcribed by&Date/转录人及日期： |  |  |  |  |  |  |  |  |

| 3.37. （12）Deprotection/脱保护 |  |  |  |  |
| --- | --- | --- | --- | --- |
| 3.37.7 Resin reswelling wash 1/溶胀后洗涤1 Turn on the nitrogen of synthesis reactor(TJ4S-1213-Sy15) to 5±2 m³/h (target: 5 m³/h). Bubble with nitrogen for 5±2 min (target: 5 min). Start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm). After stirring for 10±2 min (target: 10 min), stop stirring, turn off the nitrogen. Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point. 开启合成釜（TJ4S-1213-Sy15）下口氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min (目标：5 min)后，开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm)，搅拌10±2 min（目标：10 min）后，停止搅拌，关闭氮气。通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  |  | Times/次数 |
|  |  |  |  | 1 |
| Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h) for 5±2 min (target: 5min). Start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm). Stir for 10±2 min (target: 10 min). Stop stirring, turn off the nitrogen./ 开启合成釜下口鼓氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min（目标时间：5 min）后，开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm)，搅拌10±2 min（目标时间:10 min）后，停止搅拌，关闭氮气. |  |  |  |  |
| Bubble/鼓氮气 | Performed by&Date/执行人及日期： |  |  |  |
| Bubble/鼓氮气 | Begin/开始： |  | End/结束： |  |
| Transcribed by&Date/转录人及日期： |  |  |  |  |
| Stir /搅拌 | Performed by&Date/执行人及日期： |  |  |  |
| Stir /搅拌 | Begin/开始： |  | End/结束： |  |
| Transcribed by&Date/转录人及日期： |  |  |  |  |
| Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point./通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  |  |  |
| Filter/过滤 | Performed by&Date/执行人及日期： |  |  |  |
|  | Checked by & Date/核实人及日期： |  |  |  |
|  | Begin/开始： | End/结束： |  |  |

| 3.37. （12）Deprotection/脱保护 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| CAUTION/注意：DMF/N，N-二甲基甲酰胺：Irritation of eyes, skin and nose. May cause nausea. /刺激眼睛、皮肤和鼻子。可能引起恶心。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |  |
| 3.37.8 DMF addition /加N,N-二甲基甲酰胺 Add 546.05 kg (Acceptable weighing range: 529.67 ~ 562.43 kg) (12 V) DMF into synthesis reactor (TJ4S-1213-Sy15). 向合成釜（TJ4S-1213-Sy15）中加入546.05 kg（可接受称量范围：529.67 ~ 562.43 kg）（12 V）N,N-二甲基甲酰胺。 |  |  |  |  |  | Times/次数 |
|  |  |  |  |  |  | 1 |
| DMF addition/ 加N,N-二甲基甲酰胺 | Performed by&Date/执行人及日期： |  |  |  |  |  |
| DMF addition/ 加N,N-二甲基甲酰胺 | Begin/开始： |  | End/结束： |  | Qty/重量： |  |
| Transcribed by&Date/转录人及日期： |  |  |  |  |  |  |
| 3.37.9 Resin reswelling wash 2/溶胀后洗涤2 Turn on the nitrogen of synthesis reactor(TJ4S-1213-Sy15) to 5±2 m³/h (target: 5 m³/h). Bubble with nitrogen for 5±2 min (target: 5 min). Start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm). After stirring for 10±2 min (target: 10 min), stop stirring, turn off the nitrogen. Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point. 开启合成釜（TJ4S-1213-Sy15）下口氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min (目标：5 min)后，开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm)，搅拌10±2 min（目标：10 min）后，停止搅拌，关闭氮气。通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。drainingdiaphragm |  |  |  |  |  | Times/次数 |
|  |  |  |  |  |  | 1 |
| Bubble/鼓氮气 |  | Performed by&Date/执行人及日期： |  |  |  |  |
| Bubble/鼓氮气 |  | Begin/开始： |  | End/结束： |  |  |
| Transcribed by&Date/转录人及日期： |  |  |  |  |  |  |

| 3.37. （12）Deprotection/脱保护 |  |  |
| --- | --- | --- |
| Stir /搅拌 | Performed by&Date/执行人及日期： |  |
| Stir /搅拌 | Begin/开始： | End/结束： |
| Transcribed by&Date/转录人及日期： |  |  |
| Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point./通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  |
| Filter/过滤 | Performed by&Date/执行人及日期： |  |
|  | Checked by & Date/核实人及日期： |  |
|  | Begin/开始： | End/结束： |

| - （12）Fmoc-L-Ala-OH·H2O activation/芴甲氧羰基-L-丙氨酸一水合物活化 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| CAUTION/注意：DMF/N，N-二甲基甲酰胺：Irritation of eyes, skin and nose. May cause nausea. /刺激眼睛、皮肤和鼻子。可能引起恶心。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |
| 3.38.1 Add DMF to the activation reactor/激活罐加N,N-二甲基甲酰胺 Add 227.52 kg (Acceptable weighing range: 220.69 ~ 234.34 kg) (5 V) DMF into activation tank (TJ4S-1213-Sy15-R01). Start the agitator of activation tank to 95±10 rpm (target: 95 rpm). 向激活罐（TJ4S-1213-Sy15-R01）中加入227.52 kg（可接受称量范围：220.69 ~ 234.34 kg ）（5 V） N，N-二甲基甲酰胺，Start the agitator of activation tank to 95±10 rpm (target: 95 rpm)。 |  |  |  |  | Times/次数 |
|  |  |  |  |  | 1 |
| Add 227.52 kg DMF (Acceptable weighing range: 220.69 ~ 234.34 kg) (5 V) by spray balls into activation tank (TJ4S-1213-Sy15-R01).通过淋洗球向激活罐(TJ4S-1213-Sy15-R01)中加入227.52 kg N, N-二甲基甲酰胺（可接受称量范围：220.69 ~ 234.34 kg）(5V)。 |  |  |  |  |  |
| DMF addition/ 加N,N-二甲基甲酰胺 |  | Performed by & Date /执行人及日期： |  |  |  |
| DMF addition/ 加N,N-二甲基甲酰胺 |  | Begin/开始： | End/结束： | Qty/重量： |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |
| Start the agitator of activation tank to 95±10 rpm (target: 95 rpm)./ 开启激活罐搅拌转速至95±10 rpm (目标转速: 95 rpm)。 |  |  |  |  |  |
| Stir /搅拌 |  | Performed by & Date /执行人及日期： |  |  |  |
| Stir /搅拌 |  | Begin/开始： |  |  |  |
|  |  | The end time is documented in Ins 3.38.8/结束时间记录于3.38.8 |  |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |
| 3.38.2 Fmoc-L-Ala-OH·H2O addition /加芴甲氧羰基-L-丙氨酸一水合物 Add Fmoc-L-Ala-OH·H2O (2.00 eq.) with powder feeding bags into activation tank (TJ4S-1213-Sy15-R01). 通过投料袋向激活罐（TJ4S-1213-Sy15-R01）中加入 kg芴甲氧羰基-L-丙氨酸一水合物（Acceptable weighing range/可接受称量范围： kg ~ kg）(2.00 eq.)。 |  |  |  |  | Times/次数 |
|  |  |  |  |  | 1 |

| 3.38. （12）Fmoc-L-Ala-OH·H2O activation/芴甲氧羰基-L-丙氨酸一水合物活化 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Before material charging, confirm the material is within the retest date/ the expiry date, confirm whether the weight is adequate for use in the current charging. /加物料前，执行物料在复检日期/有效期之内, 确认重量是否满足本次投料使用。 Before solid material charging, confirm/加固体物料前，确认： - Before operation performed, confirm there is no open operation in the operation room/area; No non-related material / equipment / instrument / tools / pipeline / document / record / label in the intended area /操作前，执行操作房间/区域无开放性操作；目标区域无不相关物料/设备/仪器/工具/管路/文件/记录/标签； - The outer packages are removed. If the outer package cannot be removed, but unpack material without damaging the package bag, and before charging, check no package fragment and other extraneous matter in material. /外包装移除；如果外包装不能移除，但在不破坏包装的情况下打开包装袋，投料前以及过程中同时检查执行物料中无包装碎片以及异物 Checked by & Date/核实人及日期： |  |  |  |  |  |  |  |
| Prior to charging each amino acid, the operator shall perform a Raman scan to confirm the identity of the material. The Raman scan result shall be compared against the reference spectrum of the corresponding amino acid. Only materials that conform to the reference spectrum may be charged. If the Raman scan result does not conform, stop the operation, quarantine the material, and initiate a investigation per site SOP. Record the Raman scan result, operator signature, and date/time in the table below./ 每次投氨基酸前，操作人员应进行拉曼扫描以执行物料。拉曼扫描结果应与相应氨基酸的参考光谱进行比对。仅与参考光谱相符的物料方可投料。如拉曼扫描结果不相符，应停止操作，对物料进行隔离，并依据厂区 SOP 启动异常事件调查。在下表中记录拉曼扫描结果、操作人员签字及日期 / 时间。 |  |  |  |  |  |  |  |
| Charge Fmoc-L-Ala-OH·H2O/加芴甲氧羰基-L-丙氨酸一水合物 |  |  |  | Performed by & Date/执行人及日期： |  |  |  |
|  |  |  |  | Begin/开始： |  | End/结束： |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |
| Fmoc-L-Ala-OH·H2O confirmation/芴甲氧羰基-L-丙氨酸一水合物执行：Control No./内部控制号YL006160 |  |  |  |  |  |  |  |
| Check items/检查项 |  |  | Is it correct?/是否正确 |  | Recorded by & Date/ 记录人及日期 |  | Checked by & Date/ 核实人及日期 |
| Material name/物料名称 |  |  | Yes/是；No/否 |  |  |  |  |
| Project batch number/ 项目批次号 |  |  | Yes/是；No/否 |  |  |  |  |
| Operation number/ 操作号 |  |  | Yes/是；No/否 |  |  |  |  |
| Control No./内部控制号 |  |  | Yes/是；No/否 |  |  |  |  |
| Raman Scan Confirmation/拉曼扫描执行 |  |  | Conforms/相符 Does not conform/不相符 |  |  |  |  |
| No. /编号 |  | Net weight/净重 | All in/全部加入 |  | Recorded by & Date/ 记录人及日期 |  | Checked by & Date/ 核实人及日期 |
| / |  |  | Yes/是；No/否 |  |  |  |  |
| / |  |  | Yes/是；No/否 |  |  |  |  |

| 3.38. （12）Fmoc-L-Ala-OH·H2O activation/芴甲氧羰基-L-丙氨酸一水合物活化 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| No. /编号 |  | Net weight/净重 | All in/全部加入 | Recorded by & Date/ 记录人及日期 | Checked by & Date/ 核实人及日期 |  |
| / |  |  | Yes/是；No/否 |  |  |  |
| / |  |  | Yes/是；No/否 |  |  |  |
| / |  |  | Yes/是；No/否 |  |  |  |
| / |  |  | Yes/是；No/否 |  |  |  |
| / |  |  | Yes/是；No/否 |  |  |  |
| / |  |  | Yes/是；No/否 |  |  |  |
| / |  |  | Yes/是；No/否 |  |  |  |
| / |  |  | Yes/是；No/否 |  |  |  |
| / |  |  | Yes/是；No/否 |  |  |  |
| / |  |  | Yes/是；No/否 |  |  |  |
| Total/合计 |  |  | Confirm the total net weight is within the input range/执行合计净重在投料范围内 | Recorded by & Date/记录人及日期 | Checked by & Date/ 核实人及日期 |  |
|  |  |  | Yes/是；No/否 |  |  |  |
| After solid material charging, confirm/固体物料投料之后，确认： - The corresponding leftover material has been properly sealed and transferred to the staging area/相应的剩余物料已经恰当密封，转移到了暂存区域。 - The waste packaging material has been sealed and transferred to the corresponding waste staging area. /产生的包材废物已经密封并转移到了相应的废物暂存区域。 - The operation area/equipment surface has been cleaned/操作区域以及设备表面已经清洁。 Checked by & Date/核实人及日期： |  |  |  |  |  |  |
| 3.38.3 Agitation of activation tank /激活罐搅拌 Continue agitation of activation tank to 95±10 rpm (target: 95 rpm). Maintain stirring for at least 15 min. Confirm no visible solids and later use. 维持激活罐搅拌转速至95±10 rpm (目标转速: 95 rpm), 搅拌至少15 min，确认无明显固体待用。 |  |  |  |  |  | Times/次数 |
|  |  |  |  |  |  | 1 |

| 3.38. （12）Fmoc-L-Ala-OH·H2O activation/芴甲氧羰基-L-丙氨酸一水合物活化 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stir /搅拌 |  | Performed by & Date/执行人及日期： |  |  |  |  |  |  |
|  |  | End/结束： |  |  |  |  |  |  |
|  |  | The end time is documented in Ins 3.38.8/结束时间记录于3.38.8 |  |  |  |  |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |  |
| Confirm no visible solid/确认无明显固体： |  |  |  | Confirmed by& Date /确认人及日期： |  |  |  |  |
| 3.38.4 Adjust the temperature of activation tank /激活罐调温 Adjust the mixture to 【20±5 ℃ (target 20 ℃) 】. /将体系调温至【20±5 ℃（目标：20 ℃）】。 |  |  |  |  |  |  | Times/次数 |  |
|  |  |  |  |  |  |  | 1 |  |
| Adjust/调温 |  |  |  | Performed by&Date/执行人及日期： |  |  |  |
| Adjust/调温 |  |  |  | Begin/开始： |  | End/结束： |  |
| Transcribed by&Date/转录人及日期： |  |  |  |  |  |  |  |
| CAUTION/注意：OxymaPure is highly toxic, high-energy compound. /2-肟氰乙酸乙酯是高毒，高能化合物。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |  |  |
| 3.38.5 OxymaPure addition /加2-肟氰乙酸乙酯 Add OxymaPure (2.00 eq.) with powder feeding bags into activation tank (TJ4S-1213-Sy15-R01) at 【20±5 ℃ (target 20 ℃) 】. 控温【20±5 ℃（目标温度：20 ℃）】，通过投料袋向激活罐（TJ4S-1213-Sy15-R01）中加入 kg 2-肟氰乙酸乙酯（Acceptable weighing range/可接受称量范围： kg ~ kg）(2.00 eq.)。 |  |  |  |  |  |  | Times/次数 |
|  |  |  |  |  |  |  | 1 |
| Before material charging, confirm the material is within the retest date/ the expiry date,Confirm whether the weight is adequate for use in the current charging. /加物料前，执行物料在复检日期/有效期之内，确认重量是否满足本次投料使用。 Before solid material charging, confirm/加固体物料前，确认： - Before operation performed, confirm there is no open operation in the operation room/area; No non-related material / equipment / instrument / tools / pipeline / document / record / label in the intended area /操作前，执行操作房间/区域无开放性操作；目标区域无不相关物料/设备/仪器/工具/管路/文件/记录/标签； - The outer packages are removed. If the outer package cannot be removed, but unpack material without damaging the package bag, and before charging, check no package fragment and other extraneous matter in material. /外包装移除；如果外包装不能移除，但在不破坏包装的情况下打开包装袋，投料前以及过程中同时检查执行物料中无包装碎片以及异物 Performed by & Date/核实人及日期： |  |  |  |  |  |  |  |
| Check and record that the solid charge port is free of solid accumulation from the previous step./ 检查并记录固体投料口无来自上一步骤的固体残留。 Checked by & Date/核实人及日期： |  |  |  |  |  |  |  |

| 3.38. （12）Fmoc-L-Ala-OH·H2O activation/芴甲氧羰基-L-丙氨酸一水合物活化 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Charge OxymaPure /加2-肟氰乙酸乙酯 |  |  | Begin/开始： |  | End/结束： |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |
| OxymaPure confirmation/2-肟氰乙酸乙酯确认：Control No./内部控制号：YL005180 |  |  |  |  |  |  |
| Check items/检查项 |  | Is it correct?/是否正确 |  | Recorded by & Date/ 记录人及日期 |  | Checked by & Date/ 核实人及日期 |
| Material name/物料名称 |  | Yes/是；No/否 |  |  |  |  |
| Project batch number/ 项目批次号 |  | Yes/是；No/否 |  |  |  |  |
| Check items/检查项 |  | Is it correct?/是否正确 |  | Recorded by & Date/ 记录人及日期 |  | Checked by & Date/ 核实人及日期 |  |
| Operation number/ 操作号 |  | Yes/是；No/否 |  |  |  |  |  |
| Control No./内部控制号 |  | Yes/是；No/否 |  |  |  |  |  |
| No. /编号 | Net weight/净重 | All in/全部加入 |  | Recorded by & Date/ 记录人及日期 |  | Checked by & Date/ 核实人及日期 |
| / |  | Yes/是；No/否 |  |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |

| 3.38. （12）Fmoc-L-Ala-OH·H2O activation/芴甲氧羰基-L-丙氨酸一水合物活化 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No. /编号 |  | Net weight/净重 | All in/全部加入 |  | Recorded by & Date/ 记录人及日期 |  | Checked by & Date/ 核实人及日期 |  |
| / |  |  | Yes/是；No/否 |  |  |  |  |  |
| / |  |  | Yes/是；No/否 |  |  |  |  |  |
| / |  |  | Yes/是；No/否 |  |  |  |  |  |
| Total/合计 |  |  | Confirm the total net weight is within the input range/执行合计净重在投料范围内 |  | Recorded by & Date/记录人及日期 |  | Checked by & Date/ 核实人及日期 |  |
|  |  |  | Yes/是；No/否 |  |  |  |  |  |
| After solid material charging, confirm/固体物料投料之后，确认： - The corresponding leftover material has been properly sealed and transferred to the staging area/相应的剩余物料已经恰当密封，转移到了暂存区域。 - The waste packaging material has been sealed and transferred to the corresponding waste staging area. /产生的包材废物已经密封并转移到了相应的废物暂存区域。 - The operation area/equipment surface has been cleaned/操作区域以及设备表面已经清洁。 Checked by & Date/核实人及日期： |  |  |  |  |  |  |  |  |
| Internal temperature/内部温度 |  |  |  | High values/最高值： |  | Low values/最低值： |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |  |
| CAUTION/注意：DMF/N，N-二甲基甲酰胺：Irritation of eyes, skin and nose. May cause nausea. /刺激眼睛、皮肤和鼻子。可能引起恶心。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |  |  |  |
| 3.38.6 Rinse activation tank/激活罐淋洗 Rinse the port of Activation tank (TJ4S-1213-Sy15-R01) with 91.01 kg（Acceptable weighing range：88.28 ~ 93.74 kg）（2 V）DMF. 用91.01 kg（可接受称量范围：88.28 ~ 93.74 kg）（2 V）N,N-二甲基甲酰胺淋洗激活罐（TJ4S-1213-Sy15-R01）加料口。 |  |  |  |  |  |  | Times/次数 |  |
|  |  |  |  |  |  |  | 1 |  |

| 3.38. （12）Fmoc-L-Ala-OH·H2O activation/芴甲氧羰基-L-丙氨酸一水合物活化 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Rinse with DMF/ N,N-二甲基甲酰胺淋洗 | Performed by&Date/执行人及日期： |  |  |  |  |
| Rinse with DMF/ N,N-二甲基甲酰胺淋洗 | Begin/开始： | End/结束： | Qty/重量： |  |  |
| Transcribed by&Date/转录人及日期： |  |  |  |  |  |
| DIC is a key EHS concern chemical, highly toxic, skin sensitizing, respiratory sensitizing. /N,N’二异丙基碳二亚胺为EHS重点关注化学品，高毒，皮肤致敏，呼吸致敏。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |
| 3.38.7 DIC addition /加N,N'-二异丙基碳二亚胺 Add DIC (2.20 eq.) into the activation tank (TJ4S-1213-Sy15-R01) at reference time of 10-30 min at 【20±5 ℃ (target: 20 ℃) 】. The addition rate depends on temperature control. Stir for 20±5 min (target: 20 min) after addition. 控温【20±5 ℃ (目标20 ℃) 】，向激活罐（TJ4S-1213-Sy15-R01）中滴加 kg N,N'-二异丙基碳二亚胺（Acceptable weighing range/可接受称量范围： kg ~ kg）(2.20 eq.)（滴加参考时间10-30 min，以控温为主），滴加完毕后搅拌20±5 min（目标：20 min）。 Pump number/泵编号： Scale ID. No./秤编号： Material charging pipe number/物料管路编号： |  |  |  |  | Times/次数 |
|  |  |  |  |  | 1 |
| ★Note:The DIC addition temperature is a CPP./ 备注：N,N'-二异丙基碳二亚胺加料温度是关键控制参数。 Prior to DIC addition, the operator shall confirm that the reaction mixture temperature is within the Normal Operating Range (NOR) N,N'-二异丙基碳二亚胺投料前，操作人员须执行反应体系温度处于正常操作范围（NOR）内: Checked by & Date/核实人及日期： ★Lower limit/低限: 15 oC ，High limit/高限: 25 oC Over-temperature warning value/超温预警设置数值： Lower limit/低限:17 oC ，High limit/高限: 23 oC |  |  |  |  |  |

| 3.38. （12）Fmoc-L-Ala-OH·H2O activation/芴甲氧羰基-L-丙氨酸一水合物活化 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Before material charging, confirm the material is within the retest date/ the expiry date. Confirm whether the weight is adequate for use in the current charging. /加物料前，执行物料在复检日期/有效期之内。确认重量是否满足本次投料使用。 Checked by & Date/核实人及日期： |  |  |  |  |
| Before drummed liquid material charging, visual check there is no other extraneous matter in material. /加桶装液体物料前，应目视执行桶内无异物。 Visual Examination of Foreign Matters for Drum Solvent/桶装溶剂异物目视检查: 1. Place the solvent drum in a well-lit area (or use a portable light source) to ensure adequate visibility of the liquid contents./ 将溶剂桶置于光线充足区域（或使用便携式光源），确保液体内容物可见性良好 2. Visually inspect the solvent through the drum bung opening or transparent sight glass for any visible foreign matters, including but not limited to particulates, fibers, sediments, or unusual turbidity./ 通过桶口或透明视镜目视检查溶剂中是否有可见异物，包括但不限于颗粒、纤维、沉淀物或异常浑浊。 3. Confirm that the solvent is clear and free of visible foreign matters before charging. If any foreign matter is observed, do not use the drum and follow the applicable handling procedure for subsequent disposition./ 投料前确认溶剂澄清且无可见异物。如发现任何异物，不得使用该桶，并依据处理流程进行后续处置。 Checked by & Date/核实人及日期： |  |  |  |  |
| Confirm liquid charging pipeline is released. /确认加料管路经过放行。 |  |  |  |  |
| Checked by & Date/核实人及日期： |  |  |  |  |
| Charging of drummed solvent: verify that the pipeline connections remain consistent before and after material feeding, then record the weighing reading 桶装溶剂溶液称量时，确认投料前后管路连接一致，读取称量示数。 Performed by & Date/执行人及日期： Checked by & Date/核实人及日期： |  |  |  |  |
| DIC addition /加N,N'-二异丙基碳二亚胺 |  |  | Performed by & Date/执行人及日期： |  |  |
| DIC /加N,N'-二异丙基碳二亚胺：Control Number/内部控制号：YL003564 |  |  |  |  |  |
| Performed by & Date/执行人及日期： |  |  |  |  |  |
| Internal temperature/内部温度 |  | High values/最高值： |  | Low values/最低值： |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |
| If the target net weight≥the planned net weight, perform full drum charge; otherwise, perform partial (Non-full drum) charge. /当目标加料量≥计划加入净重，勾选进行整桶加料；否则勾选进行零头加料。 Note: Target net weight to be charged = Total target net weight – Total net weight charged. Expected tare weight of drum after charge = Gross weight of drum that to be charged - Target net weight to be charged. 还需加入净重目标值= 加料步骤目标总净重-已加入净重总量 加料完成后预计桶皮重=待加料桶毛重-还需加入净重目标值 |  |  |  |  |  |

| 3.38. （12）Fmoc-L-Ala-OH·H2O activation/芴甲氧羰基-L-丙氨酸一水合物活化 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Addition type加料式 | Lot number/批号 | Pre-charging qty calculation/投料前投料量计算 |  |  |  |  |  | Actual Charging Qty/实际投料量 |  |  |  |  |  |
|  |  | Total qty charged/已加入净重总量 |  | Target qty to be charged/还需加入净重目标值 | Gross qty of drum that to be charged/待加料桶毛重 | Expected net weight of drum after charge /加料完成后预计桶净重 | Expected tare weight of drum after charge/加料完成后预计桶皮重 | Before charging/ 投料前 |  | After charging/ 投料后 | Net weight/ 净重 | Performed by & Date/ 执行人及日期 | Checked by & Date/ 核实人及日期 |
| Full drum整桶 Non-full drum /非整桶 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Full drum整桶 Non-full drum /非整桶 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Total/合计 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DIC Addition/加N,N'-二异丙基碳二亚胺 |  |  | Begin/开始： |  |  |  |  |  | End/结束： |  |  |  |  |
| Transcribed by & Date/转录人及日期： |  |  |  |  |  |  |  |  |  |  |  |  |  |

| 3.38. （12）Fmoc-L-Ala-OH·H2O activation/芴甲氧羰基-L-丙氨酸一水合物活化 |  |  |  |  |
| --- | --- | --- | --- | --- |
| 3.38.8 Fmoc-L-Ala-OH·H2O activation/芴甲氧羰基-L-丙氨酸一水合物活化 At 20±5 ℃ (target: 20 ℃). Continue agitation of activation tank to 95±10 rpm (target: 95 rpm). Maintain stirring for 20±5 min (target: 20 min) after addition, stop stirring. 滴毕，控温20±5 ℃(目标温度：20 ℃)，维持激活罐搅拌转速至95±10 rpm (目标转速: 95 rpm)，继续搅拌20±5 min(目标时间：20 min)，停止搅拌。 |  |  |  | Times/次数 |
|  |  |  |  | 1 |
| Stir /搅拌 |  | Performed by & Date/执行人及日期： |  |  |  |
| Stir /搅拌 |  | Begin/开始： |  |  |  |
|  |  | The start time is documented in Ins3.38.1 /开始时间记录于3.38.1 |  |  |  |
| Transcribed by & Date /转录人及日期: |  |  |  |  |  |
| Internal temperature/内部温度 |  | High values/最高值： | Low values/最低值： |  |  |
| Transcribed by & Date/转录人及日期： |  |  |  |  |  |

| - （12）Fmoc-L-Ala-OH·H2O coupling/芴甲氧羰基-L-丙氨酸一水合物偶联 |  |  |  |  |
| --- | --- | --- | --- | --- |
| 3.39.1 Activation mixture transfer/活化体系转移 Transfer the mixture in activation tank (TJ4S-1213-Sy15-R01) into synthesis reactor (TJ4S-1213-Sy15). 将激活罐（TJ4S-1213-Sy15-R01）中体系转移至合成釜（TJ4S-1213-Sy15）中。 |  |  | Times/次数 |  |
|  |  |  | 1 |  |
| ★Note:The coupling temperature is a CPP, and the coupling IPC hold temperature is a CPP./ 备注：偶联温度是CPP，偶联IPC维持温度是关键工艺参数。 The operator shall confirm that the reaction mixture temperature is within the Normal Operating Range (NOR) 【25±5oC (target: 25oC)】 /操作人员须执行反应体系温度处于正常操作范围（NOR）内【25±5oC (target: 25oC)】 Checked by & Date/核实人及日期： Over-temperature warning value/超温预警设置数值： Lower limit/低限: 22 oC ，High limit/高限: 28 oC |  |  |  |  |  |
| Transfer/转移 |  | Begin/开始： | End/结束： |  |  |
|  |  | Performed by & Date/执行人及日期： |  |  |  |
|  |  | Checked by & Date/核实人及日期： |  |  |  |
| CAUTION/注意：DMF/N，N-二甲基甲酰胺：Irritation of eyes, skin and nose. May cause nausea. /刺激眼睛、皮肤和鼻子。可能引起恶心。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |
| 3.39.2 DMF addition /加N,N-二甲基甲酰胺 Add 45.50 kg (Acceptable weighing range: 43.50 ~ 47.50 kg) (1 V) DMF by spray balls into activation tank (TJ4S-1213-Sy15-R01) to rinse the wall. 通过淋洗球向激活罐（TJ4S-1213-Sy15-R01）中加入45.50 kg（可接受称量范围：43.50 ~ 47.50 kg） (1 V) N,N-二甲基甲酰胺淋洗罐壁。 |  |  |  | Times/次数 |
|  |  |  |  | 1 |

| 3.39. （12）Fmoc-L-Ala-OH·H2O coupling/芴甲氧羰基-L-丙氨酸一水合物偶联 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Rinse with DMF/ N,N-二甲基甲酰胺淋洗 | Performed by&Date/执行人及日期： |  |  |  |  |  |  |  |
|  | Begin/开始： |  | End/结束： |  | Qty/重量： |  |  |  |
| Transcribed by&Date/转录人及日期： |  |  |  |  |  |  |  |  |
| 3.39.3 Transfer /转移 Turn on the nitrogen of Synthesis reactor (TJ4S-1213-Sy15) to 5±2 m³/h (target: 5 m³/h). Bubble with nitrogen for 5±2 min (target: 5 min). Start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm). then start the agitator. Transfer the mixture in activation tank (TJ4S-1213-Sy15-R01) into synthesis reactor (TJ4S-1213-Sy15). 合成釜（TJ4S-1213-Sy15）先开启下口氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，，鼓氮气5±2 min (目标：5 min)后，开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm)，将激活罐（TJ4S-1213-Sy15-R01）中体系转移至合成釜（TJ4S-1213-Sy15）中。 |  |  |  |  |  | Times/次数 |  |  |
|  |  |  |  |  |  | 1 |  |  |
| Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h). Bubble with nitrogen for 5±2 min (target :5 min) 开启合成釜下口鼓氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min (目标时间：5 min) |  |  |  |  |  |  |
| Bubble/鼓氮气 | Performed by & Date/执行人及日期： |  |  |  |  |  |  |
| Bubble/鼓氮气 | Begin/开始： |  |  |  |  |  |  |
|  | The end time is documented in Ins3.39.5 /结束时间记录于3.39.5 |  |  |  |  |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |
| Start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm). /开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm) |  |  |  |  |  |  |  |
| Stir /搅拌 | Performed by & Date/执行人及日期： |  |  |  |  |  |  |
| Stir /搅拌 | Begin/开始： |  |  |  |  |  |  |
|  | The end time is documented in Ins3.39.5 /结束时间记录于3.39.5 |  |  |  |  |  |  |
| Transcribed by&Date/转录人及日期： |  |  |  |  |  |  |  |
| Transfer/转移 |  | Performed by&Date/执行人及日期： |  |  |  |  |  |
|  |  | Checked by & Date/核实人及日期： |  |  |  |  |  |
|  |  | Begin/开始： |  | Transfer/转移 |  |  |  |

| 3.39. （12）Fmoc-L-Ala-OH·H2O coupling/芴甲氧羰基-L-丙氨酸一水合物偶联 |  |  |  |  |
| --- | --- | --- | --- | --- |
| 3.39.4 Adjust synthesis reactor temperature/合成釜调温 Adjust synthesis reactor (TJ4S-1213-Sy15) mixture to 【25±5 ℃ (target: 25 ℃) 】. 将合成釜（TJ4S-1213-Sy15）体系调温至【25±5 ℃(目标：25 ℃) 】. |  |  |  | Times/次数 |
|  |  |  |  | 1 |
| ★Note:The coupling temperature is a CPP, and the coupling IPC hold temperature is a CPP./ 备注：偶联温度是CPP，偶联IPC维持温度是关键工艺参数。 The total reaction Fmoc-L-Leu-OH coupling time：NMT 840 min (Sourced from the holding time study in the PAR study report.)/ 芴甲氧羰基-L-亮氨酸偶联总的反应时间不超过840min(来源于 PAR 研究报告中稳定时间研究)。 Over-temperature warning value/超温预警设置数值： Lower limit/低限: 22 oC ，High limit/高限: 28 oC |  |  |  |  |
| Adjust/调温 |  | Performed by&Date/执行人及日期： |  |  |
| Adjust/调温 |  | Begin/开始： | End/结束： |  |
| Transcribed by&Date/转录人及日期： |  |  |  |  |
| 3.39.5 Fmoc-L-Ala-OH·H2O coupling/芴甲氧羰基-L-丙氨酸一水合物偶联 The mixture is allowed to react at 25±5 ℃ (target: 25 ℃). After stirring for 3 h±15 min (target: 3 h), sample the mixture every 0.5~1 h for ninhydrin test analysis until two consecutive samples test as negative (Yellow to Light pink Report Negative= Complete coupling; Blue to Purple Positive= Incomplete coupling). And the mixture should be stirred for coupling time (4 h≤Coupling Time≤10 h). Stop stirring, turn off the nitrogen. Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point. 控温【25±5oC（目标温度25oC）】，反应3 h±15 min（目标：3 h）取样进行茚三酮检测，每隔0.5-1.0 h取样一次，直至连续两次取样结果为阴性（黄色至浅粉色，报告阴性=偶联完全；蓝色至紫色，阳性=偶联不完全）后,同时反应时间满足（4h≤反应时间≤10h）后，停止搅拌，关闭氮气。通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 Note: If resin adherence is found at other times during the reaction, rinse the reactor with 0.50~5.00 kg DMF each time. /备注：偶联过程中其它时间发现贴壁，每次使用0.50~5.00 kg N,N-二甲基甲酰胺淋洗，记录见附加页 |  |  |  | Times/次数 |
|  |  |  |  | 1 |
| 3.39. （12）Fmoc-L-Ala-OH·H2O coupling/芴甲氧羰基-L-丙氨酸一水合物偶联 |  |  |  |  |
| ★After coupling is completed, the operator shall confirm that all Critical Process Parameters (CPP) are within their respective Normal Operating Ranges (NOR)/ 偶联结束后，操作人员须确认所有关键工艺参数（CPP）均处于各自的正常操作范围（NOR）内 ★Note: The coupling temperature is a CPP, and the coupling IPC hold temperature is a CPP./备注：偶联温度是CPP，偶联IPC维持温度是关键工艺参数。 The operator shall confirm that the reaction mixture temperature is within the Normal Operating Range (NOR) :20~30℃ /操作人员须确认反应体系温度处于正常操作范围（NOR）内:20~30℃ □Yes/是 □No/否 Confirmed by & Date/确认人及日期： Checked by & Date/核实人及日期： |  |  |  |  |
| React/反应 |  | Performed by&Date/执行人及日期： |  |  |
| React/反应 |  | Begin/开始： | End/结束： |  |
| Transcribed by&Date/转录人及日期： |  |  |  |  |

| 3.39. （12）Fmoc-L-Ala-OH·H2O coupling/芴甲氧羰基-L-丙氨酸一水合物偶联 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Internal temperature/温度 |  |  | High values/最高值： |  |  | Low values/最低值： |  |  |
| Transcribed by&Date/转录人及日期： |  |  |  |  |  |  |  |  |
| Standard sampling method/标准取样方法 |  |  |  |  |  |  |  |  |
| Take 5-10 ml mixture and perform testing in accordance with the ninhydrin test record. During sampling, should wear nitrile gloves and 3M 6000 series Full-face piece respirator, 6006CN cartridges. 通过在线取样器取样5~10 ml按照茚三酮检测记录进行检测。取样过程中，应该戴丁腈手套、3M 6000系列全面罩, 6006CN滤毒盒。 |  |  |  |  |  |  |  |  |
| Sampling number/ 取样编号 |  | Sampling time/取样时间 |  | Sampled by & Date/取样人及日期 | Negative or Positive/阴性或阳性 |  | Result filled by &Date/结果填写人及日期 | Checked by & Date/核实人及日期 |
| CPo140020- -01- -3.39.5-01 |  |  |  |  | Negative /阴性 Positive /阳性 |  |  |  |
| CPo140020- -01- -3.39.5-02 |  |  |  |  | Negative /阴性 Positive /阳性 |  |  |  |
| CPo140020- -01- -3.39.5-03 |  |  |  |  | Negative /阴性 Positive /阳性 |  |  |  |
| CPo140020- -01- -3.39.5-04 |  |  |  |  | Negative /阴性 Positive /阳性 |  |  |  |
| CPo140020- -01- -3.39.5-05 |  |  |  |  | Negative /阴性 Positive /阳性 |  |  |  |
| CPo140020- -01- -3.39.5-06 |  |  |  |  | Negative /阴性 Positive /阳性 |  |  |  |
| CPo140020- -01- -3.39.5-07 |  |  |  |  | Negative /阴性 Positive /阳性 |  |  |  |
| CPo140020- -01- -3.39.5-08 |  |  |  |  | Negative /阴性 Positive /阳性 |  |  |  |
| Confirm the end point of the reaction. /执行反应终点。 The acceptance criterion is two consecutive negative test results./连续两次检测结果为阴性即为合格。 Confirmed by & Date/确认人及日期： Checked by & Date/核实人及日期： |  |  |  |  |  |  |  |  |

| 3.39. （12）Fmoc-L-Ala-OH·H2O coupling/芴甲氧羰基-L-丙氨酸一水合物偶联 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Stop the stirrer and turn off the nitrogen. Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point./ 停止搅拌，关闭氮气，通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  |  |  |
| Stir /搅拌 | Performed by & Date/执行人及日期： |  |  |  |
| Stir /搅拌 | End/结束： |  |  |  |
|  | The start time is documented in Ins 3.39.3 /开始时间记录于3.39.3 |  |  |  |
| Transcribed by & Date/转录人及日期： |  |  |  |  |
| Bubble/鼓氮气 | Performed by & Date/执行人及日期： |  |  |  |
| Bubble/鼓氮气 | End/结束： |  |  |  |
|  | The start time is documented in Ins 3.39.3 /开始时间记录于3.39.3 |  |  |  |
| Transcribed by & Date/转录人及日期： |  |  |  |  |
| Internal temperature/内部温度 |  | High values/最高值： |  | Low values/最低值： |
| Transcribed by & Date/转录人及日期： |  |  |  |  |
| Filter/过滤 | Performed by&Date/执行人及日期： |  |  |  |
|  | Checked by & Date/核实人及日期： |  |  |  |
|  | Begin/开始： |  | End/结束： |  |
| Transcribed by&Date/转录人及日期： |  |  |  |  |
| ★After coupling is completed, the operator shall confirm that all Critical Process Parameters (CPPs) are within their respective Normal Operating Ranges (NOR)/ 偶联结束后，操作人员须确认所有关键工艺参数（CPP）均处于各自的正常操作范围（NOR）内 Checked by & Date/核实人及日期： |  |  |  |  |

| 3.39. （12）Fmoc-L-Ala-OH·H2O coupling/芴甲氧羰基-L-丙氨酸一水合物偶联 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| CAUTION/注意：DMF/N，N-二甲基甲酰胺：Irritation of eyes, skin and nose. May cause nausea. /刺激眼睛、皮肤和鼻子。可能引起恶心。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |
| 3.39.6 Activation Tank Rinse/激活罐洗涤 Add 364.03 kg (Acceptable weighing range：353.11 ~ 374.95 kg) (8 V) DMF into the activation tank (TJ4S-1213-Sy15-R01). Circulate and clean the activating liquid transfer pipeline 1, liquid charging port cleaning pipeline 2 and solid charging port cleaning pipeline 3, clean every pipeline for 2±1 min. 向激活罐（TJ4S-1213-Sy15-R01）中加入364.03 kg（可接受称量范围：353.11 kg ~ 374.95 kg） (8 V) N,N-二甲基甲酰胺，循环清洗激活罐转移管路1，液体加料口清洗管路2和固体加料口清洗管路3，每条线路各清洗2±1 min（目标：2 min）。 |  |  |  |  | Times/次数 |
|  |  |  |  |  | 3 |
| Operation name/ 操作名称 | Time point/时间节点 | Clean the activating liquid transfer pipeline 1/循环清洗激活液转移管路1 | Liquid charging port cleaning pipeline 2 /液体加料口清洗管路2 | Solid charging port cleaning pipeline 3,/固体加料口清洗管路3 |  |
|  |  | Wash /洗涤1 | Wash /洗涤2 | Wash /洗涤3 |  |
| DMF addition/ 加N,N-二甲基甲酰胺 | Performed by & Date/执行人及日期： |  |  |  |  |
| DMF addition/ 加N,N-二甲基甲酰胺 | Begin/开始 |  |  |  |  |
|  | End/结束 |  |  |  |  |
|  | Qty/重量 |  |  |  |  |
|  | Transcribed by & Date /转录人及日期： |  |  |  |  |
| Circulate and clean/循环清洗 | Performed by & Date/执行人及日期： |  |  |  |  |
| Circulate and clean/循环清洗 | Begin/开始 |  |  |  |  |
|  | End/结束 |  |  |  |  |
|  | Transcribed by & Date /转录人及日期： |  |  |  |  |

| 3.39. （12）Fmoc-L-Ala-OH·H2O coupling/芴甲氧羰基-L-丙氨酸一水合物偶联 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 3.39.7 Post-coupling wash/偶联后洗涤 Transfer the mixture in activation tank (TJ4S-1213-Sy15-R01) to synthesis reactor (TJ4S-1213-Sy15). Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h). Bubble with nitrogen, after 5±2 min (target 5 min). Start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm). After stirring for 7±2 min (target：7 min), stop stirring, turn off the nitrogen. Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point. 将激活罐（TJ4S-1213-Sy15-R01）洗涤液转入合成釜 (TJ4S-1213-Sy15)。开启合成釜下口鼓氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min (目标时间：5 min)后开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm)，搅拌7±2 min（目标时间：7 min）后停止搅拌，关闭氮气。通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  |  |  |  | Times/次数 |
|  |  |  |  |  |  | 3 |
| Operation name/ 操作名称 |  | Time point/时间节点 | Wash /洗涤1 | Wash /洗涤2 | Wash /洗涤3 |  |
| Transfer the liquid form activation tank (TJ4S-1213-Sy15-R01) to the synthesis reactor (TJ4S-1213-Sy15) ./将激活罐(TJ4S-1213-Sy15-R01)中体系转移至合成釜(TJ4S-1213-Sy15)中。 |  |  |  |  |  |  |
| Transfer/转移 |  | Performed by & Date/执行人及日期： |  |  |  |  |
|  |  | Checked by & Date/核实人及日期： |  |  |  |  |
|  |  | Begin/开始 |  |  |  |  |
|  |  | End/结束 |  |  |  |  |
| Bubble/鼓氮气 |  | Performed by & Date/执行人及日期： |  |  |  |  |
| Bubble/鼓氮气 |  | Begin/开始 |  |  |  |  |
|  |  | End/结束 |  |  |  |  |
|  |  | Transcribed by & Date /转录人及日期： |  |  |  |  |

| 3.39. （12）Fmoc-L-Ala-OH·H2O coupling/芴甲氧羰基-L-丙氨酸一水合物偶联 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Operation name/ 操作名称 | Time point/时间节点 | Wash /洗涤1 | Wash /洗涤2 | Wash /洗涤3 |
| Stir /搅拌 | Performed by & Date/执行人及日期： |  |  |  |
| Stir /搅拌 | Begin/开始 |  |  |  |
|  | End/结束 |  |  |  |
|  | Transcribed by & Date /转录人及日期： |  |  |  |
| Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point/通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  |  |  |
| Filter/过滤 | Performed by & Date/执行人及日期： |  |  |  |
|  | Checked by & Date/核实人及日期： |  |  |  |
|  | Begin/开始 |  |  |  |
|  | End/结束 |  |  |  |

| - （13）Deprotection/脱保护 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| CAUTION/注意：Piperidine哌啶：Strong local irritant and may cause permanent injury after short exposure to small amounts. /强烈的局部刺激，短时间少量接触可能造成永久性伤害。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |  |
| 3.40.1 Add piperidine solution/加哌啶溶液 Adjust the jacket temperature of synthesis reactor (TJ4S-1213-Sy15) to 【25±5 oC (target: 25 oC). 】. Add 357.50 kg (Acceptable weighing range: 346.77 ~ 368.23 kg) (8 V)【20% (V/V) piperidine/DMF solution】 into the Synthesis reactor (TJ4S-1213-Sy15). 将合成釜(TJ4S-1213-Sy15)夹套调温至【25±5 ℃（目标温度：25 ℃）】。向合成釜（TJ4S-1213-Sy15）中加入357.50 kg（可接受称量范围：346.77 ~ 368.23 kg）(8 V)【20%哌啶/DMF (V/V)溶液】。 |  |  |  |  |  | Times/次数 |
|  |  |  |  |  |  | 1 |
| Adjust/调温 |  | Performed by & Date/执行人及日期： |  |  |  |  |
| Adjust/调温 |  | Begin/开始： |  | Adjust/调温 |  |  |
| Transcribed by & Date/转录人及日期： |  |  |  |  |  |  |
| Confirm and record that the 20%Piperidine solution concentration is within the NOR range./ 确认并记录20%哌啶溶液浓度在正常操作范围内：17.50%~22.50% 20%Piperidine solution concentration confirmation/20%哌啶溶液浓度确认：____________ Confirm and record that the 20%Piperidine solution temperature is within the NOR range./ 确认并记录20%哌啶溶液温度在正常操作范围内：20 oC ~30 oC 20%Piperidine solution temperature confirmation/20%哌啶溶液温度确认：____________ Confirmed by & Date/确认人及日期： Checked by & Date/核实人及日期： |  |  |  |  |  |  |
| piperidine addition/ 加哌啶溶液 | Performed by&Date/执行人及日期： |  |  |  |  |  |
| piperidine addition/ 加哌啶溶液 | Begin/开始： |  | End/结束： |  | Qty/重量： |  |
| Transcribed by&Date/转录人及日期： |  |  |  |  |  |  |

| 3.40. （13）Deprotection/脱保护 |  |  |  |  |
| --- | --- | --- | --- | --- |
| 3.40.2 Adjust synthesis reactor temperature /合成釜调温 Turn on the nitrogen of Synthesis reactor (TJ4S-1213-Sy15) to 5±2 m³/h (target: 5 m³/h). Bubble with nitrogen for 5±2 min (target 5 min). Start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm). then start the stirrer, Adjust to 25±5 ℃ (target: 25 ℃）. 开启合成釜（TJ4S-1213-Sy15）下口鼓氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min (目标：5 min)后开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm)，调温至【25±5 ℃(目标温度：25 ℃)】。 |  |  | Times/次数 |
|  |  |  | 1 |
| Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h). Bubble with nitrogen for 5±2 min (target :5 min) 开启合成釜下口鼓氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min (目标时间：5 min) |  |  |  |
| Bubble/鼓氮气 |  | Performed by&Date/执行人及日期： |  |  |
| Bubble/鼓氮气 |  | Begin/开始： |  |  |
|  |  | The end time is documented in Ins 3.40.3/结束时间记录于3.40.3 |  |  |
| Transcribed by&Date/转录人及日期： |  |  |  |  |
| Start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm). /开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm) |  |  |  |  |
| Stir /搅拌 |  | Performed by&Date/执行人及日期： |  |
| Stir /搅拌 |  | Begin/开始： |  |
|  |  | The end time is documented in Ins 3.40.3/结束时间记录于3.40.3 |  |
| Transcribed by&Date/转录人及日期： |  |  |  |
| Adjust to 25±5 oC (target: 25 oC)./ 调温至25±5 ℃(目标温度：25 oC) |  |  |  |  |

| 3.40. （13）Deprotection/脱保护 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Adjust/调温 |  | Performed by&Date/执行人及日期： |  |  |  |
| Adjust/调温 |  | Begin/开始： | End/结束： |  |  |
| Transcribed by&Date/转录人及日期： |  |  |  |  |  |
| 3.40.3 Deprotection of Fmoc-L-Leu-OH activation/芴甲氧羰基-L-亮氨酸脱保护 After stirring for 2 h±15 min (target: 2 h) at【 25±5 ℃ (target: 25 ℃) 】, stop stirring, turn off the nitrogen. Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point. 控温【25±5 ℃(目标：25 ℃) 】，搅拌2 h±15 min（目标：2 h）后，停止搅拌，关闭氮气。通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 Over-temperature warning value/超温预警设置数值： Lower limit/低限: 22 oC ，High limit/高限: 28 oC |  |  |  |  | Times/次数 |
|  |  |  |  |  | 1 |
| ★Confirm and record that the 20%Piperidine solution temperature is within the NOR range 20 oC ~30 oC ./ 确认并记录20%哌啶溶液温度在正常操作范围内：20 oC ~30 oC。 20%Piperidine solution temperature confirmation/20%哌啶溶液温度确认：____________ Confirmed by& Date/确认人及日期： Checked by & Date/核实人及日期： |  |  |  |  |  |
| De-Fmoc/脱Fmoc &Stir /搅拌 |  | Performed by&Date/执行人及日期： |  |  |  |
| De-Fmoc/脱Fmoc &Stir /搅拌 |  | End/结束： |  |  |  |
|  |  | The start time is documented in Ins3.40.2 /开始时间记录于3.40.2 |  |  |  |
| Transcribed by&Date/转录人及日期： |  |  |  |  |  |

| 3.40. （13）Deprotection/脱保护 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Bubble/鼓氮气 |  |  | Performed by & Date/执行人及日期： |  |  |  |
| Bubble/鼓氮气 |  |  | End/结束： |  |  |  |
|  |  |  | The start time is documented in Ins3.37.2 /开始时间记录于3.37.2 |  |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |
| Internal temperature/内部温度 |  |  | High values/最高值: | Low values/最低值： |  |  |
| Transcribed by&Date/转录人及日期： |  |  |  |  |  |  |
| Filter/过滤 |  |  | Performed by&Date/执行人及日期： |  |  |  |
|  |  |  | Checked by & Date/核实人及日期： |  |  |  |
|  |  |  | Begin/开始： | Filter/过滤 |  |  |
| ★After De-Fmoc is completed, the operator shall confirm that all Critical Process Parameters (CPPs) are within their respective Normal Operating Ranges (NOR)/ 脱保护结束后，操作人员须确认所有关键工艺参数（CPP）均处于各自的正常操作范围（NOR）内 Checked by & Date/核实人及日期： |  |  |  |  |  |  |
| 3.40.4 synthesis reactor adjust temperature/合成釜调温 Adjust the synthesis reactor (TJ4S-1213-Sy15) jacket to 【20±5 ℃ (target: 20 ℃) 】. /将合成釜（TJ4S-1213-Sy15）夹套调温至【20±5 ℃（目标温度：20 ℃）】。 |  |  |  |  | Times/次数 |  |  |
|  |  |  |  |  | 1 |  |  |
| Adjust/调温 | Performed by & Date/执行人及日期： |  |  |  |  |  |  |
| Adjust/调温 | Begin/开始： |  |  | End/结束： |  |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |
| CAUTION/注意：IPAC/乙酸异丙酯： Vapors irritate eyes and respiratory tract; high concentrations can be anesthetic. Liquid irritates eyes but causes no serious injury; may cause dermatitis; no serious effects if swallowed. /蒸汽会刺激眼睛和呼吸道；高浓度的蒸汽可能是麻醉剂。液体刺激眼睛，但不会造成严重伤害；可能会引起皮炎；吞食不会造成严重影响。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |  |  |

| 3.40. （13）Deprotection/脱保护 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 3.40.5 Post-deprotection wash/脱保护后洗涤 Add 445.92 kg (Acceptable weighing range：432.54 ~ 459.30 kg) (9.29 kg/kg, 10.64 V) isopropyl acetate into synthesis reactor (TJ4S-1213-Sy15). Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h). Bubble with nitrogen for 5±2 min (target 5 min), start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm). After stirring for 5±2 min (target 5 min), stop stirring, turn off the nitrogen. Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point. 向合成釜（TJ4S-1213-Sy15）中加入445.92 kg（可接受称量范围：432.54 ~ 459.30 kg) (9.29 kg/kg, 10.64 V）乙酸异丙酯，开启下口氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min (目标时间：5 min)后开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm)，搅拌5±2 min (目标时间：5 min)后，停止搅拌，关闭氮气。通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。drainingdiaphragm |  |  |  |  |  |  | Times/次数 |
|  |  |  |  |  |  |  | 4 |
| ★Note: IPAC quantity is a CPP./备注：乙酸异丙酯用量是关键工艺参数。 ★The operator shall confirm that IPAC quantity is within the Normal Operating Range (NOR) 【432.54 kg~459.30 kg】/操作人员须确认乙酸异丙酯用量处于正常操作范围（NOR）内：【432.54 kg~459.30 kg】 Checked by & Date/核实人及日期： |  |  |  |  |  |  |  |
| Operation name/操作名称 |  | Time point/时间节点 | Wash /洗涤1 | Wash /洗涤2 | Wash /洗涤3 | Wash /洗涤4 |  |
| Add 419.04 kg IPAc (10 V) by spray balls into the synthesis reactor (TJ4S-1213-Sy15)./ 向合成釜(TJ4S-1213-Sy15)中加419.04 kg乙酸异丙酯（10 V)）（Acceptable weighing range/可接受称量范围：406.47~431.61 kg） |  |  |  |  |  |  |  |
| IPAC addition/加乙酸异丙酯 |  | Performed by & Date/执行人及日期： |  |  |  |  |  |
| IPAC addition/加乙酸异丙酯 |  | Begin/开始 |  |  |  |  |  |
|  |  | End/结束 |  |  |  |  |  |
|  |  | Qty/重量 |  |  |  |  |  |
|  |  | Transcribed by & Date /转录人及日期： |  |  |  |  |  |

| 3.40. （13）Deprotection/脱保护 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Operation name/操作名称 | Time point/时间节点 | Wash /洗涤1 | Wash /洗涤2 | Wash /洗涤3 | Wash /洗涤4 |
| Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h). Bubble with nitrogen for 5±2 min (target: 5 min). Start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm). Stir for 5±2 min (target: 5 min), stop stirring, turn off the nitrogen./ 开启合成釜下口鼓氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min（目标时间：5 min）后开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm)，搅拌5±2 min（目标时间：5 min）后，停止搅拌，关闭氮气。 |  |  |  |  |  |
| Bubble/鼓氮气 | Performed by & Date/执行人及日期： |  |  |  |  |
| Bubble/鼓氮气 | Begin/开始 |  |  |  |  |
|  | End/结束 |  |  |  |  |
|  | Transcribed by & Date /转录人及日期： |  |  |  |  |
| Stir /搅拌 | Performed by & Date/执行人及日期： |  |  |  |  |
| Stir /搅拌 | Begin/开始 |  |  |  |  |
|  | End/结束 |  |  |  |  |
|  | Transcribed by & Date /转录人及日期： |  |  |  |  |
| Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point./通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  |  |  |  |
| Filter/过滤 | Performed by & Date/执行人及日期： |  |  |  |  |
|  | Checked by & Date/核实人及日期： |  |  |  |  |
|  | Begin/开始 |  |  |  |  |
|  | End/结束 |  |  |  |  |

| 3.40. （13）Deprotection/脱保护 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| DMF/N，N-二甲基甲酰胺：Irritation of eyes, skin and nose. May cause nausea. /刺激眼睛、皮肤和鼻子。可能引起恶心。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |  |
| 3.40.6 DMF addition /加N,N-二甲基甲酰胺 Add 546.05 kg (Acceptable weighing range: 529.67 ~ 562.43 kg) (12 V) DMF into synthesis reactor (TJ4S-1213-Sy15). 向合成釜（TJ4S-1213-Sy15）中加入546.05 kg（可接受称量范围：529.67 ~ 562.43 kg）（12 V）N,N-二甲基甲酰胺。 |  |  |  |  |  | Times/次数 |
|  |  |  |  |  |  | 1 |
| DMF addition/ 加N,N-二甲基甲酰胺 | Performed by&Date/执行人及日期： |  |  |  |  |  |
| DMF addition/ 加N,N-二甲基甲酰胺 | Begin/开始： |  | End/结束： |  | Qty/重量： |  |
| Transcribed by&Date/转录人及日期： |  |  |  |  |  |  |
| 3.40.7 Resin reswelling wash 1/溶胀后洗涤1 Turn on the nitrogen of synthesis reactor(TJ4S-1213-Sy15) to 5±2 m³/h (target: 5 m³/h). Bubble with nitrogen for 5±2 min (target: 5 min). Start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm). After stirring for 10±2 min (target: 10 min), stop stirring, turn off the nitrogen. Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point. 开启合成釜（TJ4S-1213-Sy15）下口氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min (目标：5 min)后，开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm)，搅拌10±2 min（目标：10 min）后，停止搅拌，关闭氮气。通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  |  |  |  | Times/次数 |
|  |  |  |  |  |  | 1 |
| Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h) for 5±2 min (target: 5 min), Start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm). Stir for 10±2 min (target: 10 min). Stop stirring, turn off the nitrogen./ 开启合成釜下口鼓氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min（目标时间：5 min）后，开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm)，搅拌10±2 min（目标时间:10 min）后，停止搅拌，关闭氮气。 |  |  |  |  |  |  |
| Bubble/鼓氮气 |  | Performed by&Date/执行人及日期： |  |  |  |  |
| Bubble/鼓氮气 |  | Begin/开始： |  | End/结束： |  |  |
| Transcribed by&Date/转录人及日期： |  |  |  |  |  |  |

| 3.40. （13）Deprotection/脱保护 |  |  |
| --- | --- | --- |
| Stir /搅拌 | Performed by&Date/执行人及日期： |  |
| Stir /搅拌 | Begin/开始： | End/结束： |
| Transcribed by&Date/转录人及日期： |  |  |
| Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point./通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  |
| Filter/过滤 | Performed by&Date/执行人及日期： |  |
|  | Checked by & Date/核实人及日期： |  |
|  | Begin/开始： | End/结束： |

| 3.40. （13）Deprotection/脱保护 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| CAUTION/注意：DMF/N，N-二甲基甲酰胺：Irritation of eyes, skin and nose. May cause nausea. /刺激眼睛、皮肤和鼻子。可能引起恶心。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |  |
| 3.40.8 DMF addition/加N,N-二甲基甲酰胺 Add 546.05 kg (Acceptable weighing range: 529.67 ~ 562.43 kg) (12 V) DMF into synthesis reactor (TJ4S-1213-Sy15). 向合成釜（TJ4S-1213-Sy15）中加入546.05 kg（可接受称量范围：529.67 ~ 562.43 kg）（12 V）N,N-二甲基甲酰胺。 |  |  |  |  |  | Times/次数 |
|  |  |  |  |  |  | 1 |
| DMF addition/ 加N,N-二甲基甲酰胺 | Performed by&Date/执行人及日期： |  |  |  |  |  |
| DMF addition/ 加N,N-二甲基甲酰胺 | Begin/开始： |  | End/结束： |  | Qty/重量： |  |
| Transcribed by&Date/转录人及日期： |  |  |  |  |  |  |
| 3.40.9 Resin reswelling wash 2/溶胀后洗涤2 Turn on the nitrogen of synthesis reactor (TJ4S-1213-Sy15) to 5±2 m³/h (target: 5 m³/h). Bubble with nitrogen for 5±2 min (target: 5 min). Start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm). After stirring for 10±2 min (target: 10 min), stop stirring, turn off the nitrogen. Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point. 开启合成釜（TJ4S-1213-Sy15）下口氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min (目标：5 min)后，开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm)，搅拌10±2 min（目标：10 min）后，停止搅拌，关闭氮气。通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  |  |  |  | Times/次数 |
|  |  |  |  |  |  | 1 |
| Bubble/鼓氮气 |  | Performed by&Date/执行人及日期： |  |  |  |  |
| Bubble/鼓氮气 |  | Begin/开始： |  | End/结束： |  |  |
| Transcribed by&Date/转录人及日期： |  |  |  |  |  |  |

| 3.40. （13）Deprotection/脱保护 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Stir /搅拌 |  | Performed by&Date/执行人及日期： |  |  |  |
| Stir /搅拌 |  | Begin/开始： |  | End/结束： |  |
| Transcribed by&Date/转录人及日期： |  |  |  |  |  |
| Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point./通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  |  |  |  |
| Filter/过滤 |  | Performed by&Date/执行人及日期： |  |  |  |
|  |  | Checked by & Date/核实人及日期： |  |  |  |
|  |  | Begin/开始： |  | End/结束： |  |
| Transcribed by&Date/转录人及日期： |  |  |  |  |  |
| - （13）Fmoc-L-Leu-OH activation/芴甲氧羰基-L-亮氨酸活化 |  |  |  |  |  |
| CAUTION/注意：DMF/N，N-二甲基甲酰胺：Irritation of eyes, skin and nose. May cause nausea. /刺激眼睛、皮肤和鼻子。可能引起恶心。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |
| 3.41.1 Add DMF to the activation reactor/激活罐加N,N-二甲基甲酰胺： Add 227.52 kg (Acceptable weighing range: 220.69 ~ 234.34 kg) (5 V) DMF into activation tank (TJ4S-1213-Sy15-R01). Start the agitator of activation tank to 95±10 rpm (target: 95 rpm). 向激活罐（TJ4S-1213-Sy15-R01）中加入227.52 kg（可接受称量范围：220.69 ~ 234.34 kg ）（5 V）N，N-二甲基甲酰胺，Start the agitator of activation tank to 95±10 rpm (target: 95 rpm)。 |  |  |  |  | Times/次数 |
|  |  |  |  |  | 1 |
| Add 227.52 kg DMF (Acceptable weighing range: 220.69 ~ 234.34 kg) (5 V) by spray balls into activation tank (TJ4S-1213-Sy15-R01).通过淋洗球向激活罐(TJ4S-1213-Sy15-R01)中加入227.52 kg N, N-二甲基甲酰胺（可接受称量范围：220.69 ~ 234.34 kg）(5V)。 |  |  |  |  |  |
| DMF addition/加N,N-二甲基甲酰胺 | Performed by&Date/执行人及日期： |  |  |  |  |
| DMF addition/加N,N-二甲基甲酰胺 | Begin/开始： |  | End/结束 |  |  |
| Transcribed by&Date/转录人及日期： |  |  |  |  |  |

| 3.41. （13）Fmoc-L-Leu-OH activation/芴甲氧羰基-L-亮氨酸活化 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Start the agitator of activation tank to 95±10 rpm (target: 95 rpm)./ 开启激活罐搅拌转速至95±10 rpm (目标转速: 95 rpm)。 |  |  |  |  |
| Stir/搅拌 | Performed by&Date/执行人及日期： |  |  |  |
| Stir/搅拌 | Begin/开始： |  |  |  |
|  | The end time is documented in Ins 3.41.8/结束时间记录于3.41.8 |  |  |  |
| Transcribed by&Date/转录人及日期： |  |  |  |  |
| 3.41.2 Add Fmoc-L-Leu-OH/加入芴甲氧羰基-L-亮氨酸 Add Fmoc-L-Leu-OH (2.00 eq.) with powder feeding bags into activation tank (TJ4S-1213-Sy15-R01). 通过投料袋向激活罐（TJ4S-1213-Sy15-R01）中加入 kg芴甲氧羰基-L-亮氨酸（Acceptable weighing range/可接受称量范围： kg ~ kg）(2.00 eq.)。 |  |  |  | Times/次数 |
|  |  |  |  | 1 |
| Before material charging, confirm the material is within the retest date/ the expiry date, confirm whether the weight is adequate for use in the current charging. /加物料前，执行物料在复检日期/有效期之内, 确认重量是否满足本次投料使用。 Before solid material charging, confirm/加固体物料前，确认： - Before operation performed, confirm there is no open operation in the operation room/area; No non-related material / equipment / instrument / tools / pipeline / document / record / label in the intended area /操作前，执行操作房间/区域无开放性操作；目标区域无不相关物料/设备/仪器/工具/管路/文件/记录/标签； - The outer packages are removed. If the outer package cannot be removed, but unpack material without damaging the package bag, and before charging, check no package fragment and other extraneous matter in material. /外包装移除；如果外包装不能移除，但在不破坏包装的情况下打开包装袋，投料前以及过程中同时检查执行物料中无包装碎片以及异物 Performed by & Date/核实人及日期： |  |  |  |  |
| Prior to charging each amino acid, the operator shall perform a Raman scan to confirm the identity of the material. The Raman scan result shall be compared against the reference spectrum of the corresponding amino acid. Only materials that conform to the reference spectrum may be charged. If the Raman scan result does not conform, stop the operation, quarantine the material, and initiate a investigation per site SOP. Record the Raman scan result, operator signature, and date/time in the table below./ 每次投氨基酸前，操作人员应进行拉曼扫描以执行物料。拉曼扫描结果应与相应氨基酸的参考光谱进行比对。仅与参考光谱相符的物料方可投料。如拉曼扫描结果不相符，应停止操作，对物料进行隔离，并依据厂区 SOP 启动异常事件调查。在下表中记录拉曼扫描结果、操作人员签字及日期 / 时间。 |  |  |  |  |
| Fmoc-L- Leu-OH addition /加芴甲氧羰基-L-亮氨酸 |  | Performed by & Date/执行人及日期： |  |  |
|  |  | Begin/开始： | End/结束： |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |

| 3.41. （13）Fmoc-L-Leu-OH activation/芴甲氧羰基-L-亮氨酸活化 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Fmoc-L- Leu-OH confirmation/芴甲氧羰基-L-亮氨酸执行：Control No./内部控制号YL003425 |  |  |  |  |
| Check items/检查项 |  | Is it correct?/是否正确 | Recorded by & Date/ 记录人及日期 | Checked by & Date/ 核实人及日期 |
| Material name/物料名称 |  | Yes/是；No/否 |  |  |
| Project batch number/ 项目批次号 |  | Yes/是；No/否 |  |  |
| Operation number/ 操作号 |  | Yes/是；No/否 |  |  |
| Control No./内部控制号 |  | Yes/是；No/否 |  |  |
| Raman Scan Confirmation/拉曼扫描执行 |  | Conforms/相符 Does not conform/不相符 |  |  |
| No. /编号 | Net weight/净重 | All in/全部加入 | Recorded by & Date/ 记录人及日期 | Checked by & Date/ 核实人及日期 |
| / |  | Yes/是；No/否 |  |  |
| / |  | Yes/是；No/否 |  |  |
| / |  | Yes/是；No/否 |  |  |
| / |  | Yes/是；No/否 |  |  |
| / |  | Yes/是；No/否 |  |  |
| / |  | Yes/是；No/否 |  |  |
| / |  | Yes/是；No/否 |  |  |
| / |  | Yes/是；No/否 |  |  |
| / |  | Yes/是；No/否 |  |  |
| / |  | Yes/是；No/否 |  |  |

| 3.41. （13）Fmoc-L-Leu-OH activation/芴甲氧羰基-L-亮氨酸活化 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| No. /编号 | Net weight/净重 | All in/全部加入 |  |  | Recorded by & Date/ 记录人及日期 | Checked by & Date/ 核实人及日期 |
| / |  | Yes/是；No/否 |  |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |
| Total/合计 |  | Confirm the total net weight is within the input range/执行合计净重在投料范围内 |  |  | Recorded by & Date/记录人及日期 | Checked by & Date/ 核实人及日期 |
|  |  | Yes/是；No/否 |  |  |  |  |
| After solid material charging, confirm/固体物料投料之后，确认： - The corresponding leftover material has been properly sealed and transferred to the staging area/相应的剩余物料已经恰当密封，转移到了暂存区域。 - The waste packaging material has been sealed and transferred to the corresponding waste staging area. /产生的包材废物已经密封并转移到了相应的废物暂存区域。 - The operation area/equipment surface has been cleaned/操作区域以及设备表面已经清洁。 Checked by & Date/核实人及日期： |  |  |  |  |  |  |
| 3.41.3 Agitation of activation tank /激活罐搅拌 Continue agitation of activation tank to 95±10 rpm (target: 95 rpm). Maintain stirring for at least 15 min. Confirm no visible solids and later use. 维持激活罐搅拌转速至95±10 rpm (目标转速: 95 rpm), 搅拌至少15 min，确认无明显固体待用。 |  |  |  |  |  |  |
| Stir /搅拌 |  |  | Performed by & Date/执行人及日期： |  |  |  |
|  |  |  | End/结束： |  |  |  |
|  |  |  | The end time is documented in Ins 3.41.8/结束时间记录于3.41.8 |  |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |
| Confirm no visible solid/确认无明显固体： |  |  |  | Confirmed by& Date /确认人及日期： |  |  |

| 3.41. （13）Fmoc-L-Leu-OH activation/芴甲氧羰基-L-亮氨酸活化 |  |  |  |  |
| --- | --- | --- | --- | --- |
| 3.41.4 Adjust the temperature of activation tank /激活罐调温 Adjust the mixture to 【20±5 ℃ (target 20 ℃) 】. 将体系调温至【20±5 ℃（目标：20 ℃）】。 |  |  | Times/次数 |  |
|  |  |  | 1 |  |
| Adjust/调温 |  | Performed by&Date/执行人及日期： |  |  |
| Adjust/调温 |  | Begin/开始： | End/结束： |  |
| Transcribed by&Date/转录人及日期： |  |  |  |  |
| CAUTION/注意：OxymaPure is highly toxic, high-energy compound. /2-肟氰乙酸乙酯是高毒，高能化合物。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |
| 3.41.5 OxymaPure addition /加2-肟氰乙酸乙酯 Add OxymaPure (2.00 eq.) with powder feeding bags into activation tank (TJ4S-1213-Sy15-R01) at 20±5 ℃ (target 20 ℃). 控温20±5 ℃（目标：20 ℃），通过投料袋向激活罐（TJ4S-1213-Sy15-R01）中加入 kg 2-肟氰乙酸乙酯（Acceptable weighing range/可接受称量范围： kg ~ kg）(2.00 eq.)。 |  |  |  | Times/次数 |
|  |  |  |  | 1 |
| Before material charging, confirm the material is within the retest date/ the expiry date,Confirm whether the weight is adequate for use in the current charging. /加物料前，执行物料在复检日期/有效期之内，确认重量是否满足本次投料使用。 Before solid material charging, confirm/加固体物料前，确认： - Before operation performed, confirm there is no open operation in the operation room/area; No non-related material / equipment / instrument / tools / pipeline / document / record / label in the intended area /操作前，执行操作房间/区域无开放性操作；目标区域无不相关物料/设备/仪器/工具/管路/文件/记录/标签； - The outer packages are removed. If the outer package cannot be removed, but unpack material without damaging the package bag, and before charging, check no package fragment and other extraneous matter in material. /外包装移除；如果外包装不能移除，但在不破坏包装的情况下打开包装袋，投料前以及过程中同时检查执行物料中无包装碎片以及异物 Performed by & Date/核实人及日期： |  |  |  |  |
| Check and record that the solid charge port is free of solid accumulation from the previous step./ 检查并记录固体投料口无来自上一步骤的固体残留。 Checked by & Date/核实人及日期： |  |  |  |  |

| 3.41. （13）Fmoc-L-Leu-OH activation/芴甲氧羰基-L-亮氨酸活化 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Charge OxymaPure /加2-肟氰乙酸乙酯 |  |  | Begin/开始： |  | End/结束： |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |
| OxymaPure confirmation/2-肟氰乙酸乙酯确认：Control No./内部控制号：YL005180 |  |  |  |  |  |  |
| Check items/检查项 |  | Is it correct?/是否正确 |  | Recorded by & Date/ 记录人及日期 |  | Checked by & Date/ 核实人及日期 |
| Material name/物料名称 |  | Yes/是；No/否 |  |  |  |  |
| Project batch number/ 项目批次号 |  | Yes/是；No/否 |  |  |  |  |
| Check items/检查项 |  | Is it correct?/是否正确 |  | Recorded by & Date/ 记录人及日期 |  | Checked by & Date/ 核实人及日期 |
| Operation number/ 操作号 |  | Yes/是；No/否 |  |  |  |  |
| Control No./内部控制号 |  | Yes/是；No/否 |  |  |  |  |
| No. /编号 | Net weight/净重 | All in/全部加入 |  | Recorded by & Date/ 记录人及日期 |  | Checked by & Date/ 核实人及日期 |
| / |  | Yes/是；No/否 |  |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |

| 3.41. （13）Fmoc-L-Leu-OH activation/芴甲氧羰基-L-亮氨酸活化 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| No. /编号 | Net weight/净重 | All in/全部加入 |  | Recorded by & Date/ 记录人及日期 |  | Checked by & Date/ 核实人及日期 |
| / |  | Yes/是；No/否 |  |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |
| Total/合计 |  | Confirm the total net weight is within the input range/执行合计净重在投料范围内 |  | Recorded by & Date/记录人及日期 |  | Checked by & Date/ 核实人及日期 |
|  |  | Yes/是；No/否 |  |  |  |  |
| After solid material charging, confirm/固体物料投料之后，确认： - The corresponding leftover material has been properly sealed and transferred to the staging area/相应的剩余物料已经恰当密封，转移到了暂存区域。 - The waste packaging material has been sealed and transferred to the corresponding waste staging area. /产生的包材废物已经密封并转移到了相应的废物暂存区域。 - The operation area/equipment surface has been cleaned/操作区域以及设备表面已经清洁。 Checked by & Date/核实人及日期： |  |  |  |  |  |  |
| Internal temperature/内部温度 |  |  | High values/最高值： |  | Low values/最低值： |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |

| 3.41. （13）Fmoc-L-Leu-OH activation/芴甲氧羰基-L-亮氨酸活化 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CAUTION/注意：DMF/N，N-二甲基甲酰胺：Irritation of eyes, skin and nose. May cause nausea. /刺激眼睛、皮肤和鼻子。可能引起恶心。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |  |  |  |
| 3.41.6 Rinse activation tank /激活罐淋洗 Rinse the port of Activation tank (TJ4S-1213-Sy15-R01) with 91.01 kg（Acceptable weighing range：88.28 ~ 93.74 kg）（2 V）DMF. 用91.01 kg（可接受称量范围：88.28 ~ 93.74 kg）（2 V）N,N-二甲基甲酰胺淋洗激活罐（TJ4S-1213-Sy15-R01）加料口。 |  |  |  |  |  |  | Times/次数 |  |
|  |  |  |  |  |  |  | 1 |  |
| Rinse with DMF/ N,N-二甲基甲酰胺淋洗 | Performed by&Date/执行人及日期： |  |  |  |  |  |  |  |
| Rinse with DMF/ N,N-二甲基甲酰胺淋洗 | Begin/开始： |  |  | End/结束： |  | Qty/重量： |  |  |
| Transcribed by&Date/转录人及日期： |  |  |  |  |  |  |  |  |
| CAUTION/注意：DIC is a key EHS concern chemical, highly toxic, skin sensitizing, respiratory sensitizing. /N,N’二异丙基碳二亚胺为EHS重点关注化学品，高毒，皮肤致敏，呼吸致敏。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |  |  |  |
| 3.41.7 DIC addition /加N,N'-二异丙基碳二亚胺 Add DIC (2.20 eq.) into the activation tank (TJ4S-1213-Sy15-R01) at reference time of 10-30 min at【 20±5 ℃ (target: 20 ℃) 】. The addition rate depends on temperature control. 控温【20±5 ℃ (目标20 ℃) 】，向激活罐（TJ4S-1213-Sy15-R01）中滴加 kg N,N'-二异丙基碳二亚胺（Acceptable weighing range/可接受称量范围： ~ kg）（滴加参考时间10-30 min，以控温为主）(2.20 eq.)。 Pump number/泵编号： Scale ID. No./秤编号： Material charging pipe number/物料管路编号： |  |  |  |  |  |  |  | Times/次数 |
|  |  |  |  |  |  |  |  | 1 |
| 3.41. （13）Fmoc-L-Leu-OH activation/芴甲氧羰基-L-亮氨酸活化 |  |  |  |  |  |  |  |  |
| ★Note:The DIC addition temperature is a CPP./ 备注：N,N'-二异丙基碳二亚胺加料温度是关键控制参数。 Prior to DIC addition, the operator shall confirm that the reaction mixture temperature is within the Normal Operating Range (NOR) N,N'-二异丙基碳二亚胺投料前，操作人员须执行反应体系温度处于正常操作范围（NOR）内: Checked by & Date/核实人及日期： ★Lower limit/低限: 8 oC ，High limit/高限: 32 oC Over-temperature warning value/超温预警设置数值： Lower limit/低限:17 oC ，High limit/高限: 23 oC |  |  |  |  |  |  |  |  |
| Before material charging, confirm the material is within the retest date/ the expiry date. Confirm whether the weight is adequate for use in the current charging. /加物料前，执行物料在复检日期/有效期之内。确认重量是否满足本次投料使用。 Checked by & Date/核实人及日期： |  |  |  |  |  |  |  |  |
| Before drummed liquid material charging, visual check there is no other extraneous matter in material. /加桶装液体物料前，应目视执行桶内无异物。 Visual Examination of Foreign Matters for Drum Solvent/桶装溶剂异物目视检查: 1. Place the solvent drum in a well-lit area (or use a portable light source) to ensure adequate visibility of the liquid contents./ 将溶剂桶置于光线充足区域（或使用便携式光源），确保液体内容物可见性良好 2. Visually inspect the solvent through the drum bung opening or transparent sight glass for any visible foreign matters, including but not limited to particulates, fibers, sediments, or unusual turbidity./ 通过桶口或透明视镜目视检查溶剂中是否有可见异物，包括但不限于颗粒、纤维、沉淀物或异常浑浊。 3. Confirm that the solvent is clear and free of visible foreign matters before charging. If any foreign matter is observed, do not use the drum and follow the applicable handling procedure for subsequent disposition./ 投料前确认溶剂澄清且无可见异物。如发现任何异物，不得使用该桶，并依据处理流程进行后续处置。 Checked by & Date/核实人及日期： |  |  |  |  |  |  |  |  |
| Confirm liquid charging pipeline is released. /确认加料管路经过放行。 |  |  |  |  |  |  |  |  |
| Checked by & Date/核实人及日期： |  |  |  |  |  |  |  |  |
| Charging of drummed solvent: verify that the pipeline connections remain consistent before and after material feeding, then record the weighing reading 桶装溶剂溶液称量时，确认投料前后管路连接一致，读取称量示数。 Performed by & Date/执行人及日期： Checked by & Date/核实人及日期： |  |  |  |  |  |  |  |  |
| DIC addition /加N,N'-二异丙基碳二亚胺 |  |  |  | Performed by & Date/执行人及日期： |  |  |  |  |  |
| DIC /加N,N'-二异丙基碳二亚胺：Control Number/内部控制号：YL003564 |  |  |  |  |  |  |  |  |  |
| Internal temperature/温度 |  |  | High values/最高值： |  |  | Low values/最低值： |  |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |  |  |

3.41. （13）Fmoc-L-Leu-OH activation/芴甲氧羰基-L-亮氨酸活化

If the target net weight≥the planned net weight, perform full drum charge; otherwise, perform partial (Non-full drum) charge. /当目标加料量≥计划加入净重，勾选进行整桶加料；否则勾选进行零头加料。 Note: Target net weight to be charged = Total target net weight – Total net weight charged. Expected tare weight of drum after charge = Gross weight of drum that to be charged - Target net weight to be charged. 还需加入净重目标值= 加料步骤目标总净重-已加入净重总量 加料完成后预计桶皮重=待加料桶毛重-还需加入净重目标值

| 3.41. （13）Fmoc-L-Leu-OH activation/芴甲氧羰基-L-亮氨酸活化 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Addition type加料式 | Lot number/批号 | Pre-charging qty calculation/投料前投料量计算 |  |  |  |  |  | Actual Charging Qty/实际投料量 |  |  |  |  |  |
|  |  | Total qty charged/已加入净重总量 |  | Target qty to be charged/还需加入净重目标值 | Gross qty of drum that to be charged/待加料桶毛重 | Expected net weight of drum after charge /加料完成后预计桶净重 | Expected tare weight of drum after charge/加料完成后预计桶皮重 | Before charging/ 投料前 |  | After charging/ 投料后 | Net weight/ 净重 | Performed by & Date/ 执行人及日期 | Checked by & Date/ 核实人及日期 |
| Full drum整桶 Non-full drum /非整桶 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Full drum整桶 Non-full drum /非整桶 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Total/合计 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DIC Addition/加N,N'-二异丙基碳二亚胺 |  |  | Begin/开始： |  |  |  |  |  | End/结束： |  |  |  |  |
| Transcribed by & Date/转录人及日期： |  |  |  |  |  |  |  |  |  |  |  |  |  |

| 3.41. （13）Fmoc-L-Leu-OH activation/芴甲氧羰基-L-亮氨酸活化 |  |  |  |  |
| --- | --- | --- | --- | --- |
| 3.41.8 Fmoc-L-Leu-OH activation/芴甲氧羰基-L-亮氨酸活化 At 20±5 ℃ (target: 20 ℃). Continue agitation of activation tank to 95±10 rpm (target: 95 rpm). Maintain stirring for 20±5 min (target: 20 min) after addition, stop stirring. 滴毕，控温20±5 ℃(目标温度：20 ℃)，维持激活罐搅拌转速至95±10 rpm (目标转速: 95 rpm)，继续搅拌20±5 min(目标时间：20 min)， 停止搅拌。 |  |  |  | Times/次数 |
|  |  |  |  | 1 |
| Stir /搅拌 | Performed by & Date/执行人及日期： |  |  |  |  |
| Stir /搅拌 | End/结束： |  |  |  |  |
|  | The start time is documented in Ins3.41.1 /开始时间记录于3.41.1 |  |  |  |  |
| Transcribed by & Date/转录人及日期： |  |  |  |  |  |
| Internal temperature/内部温度 | High values/最高值： | Low values/最低值： |  |  |  |
| Transcribed by & Date/转录人及日期： |  |  |  |  |  |

| - （13）Fmoc-L-Leu-OH coupling/芴甲氧羰基-L-亮氨酸偶联 |  |  |  |  |
| --- | --- | --- | --- | --- |
| 3.42.1 Activation mixture transfer /活化体系转移 Transfer the mixture in activation tank (TJ4S-1213-Sy15-R01) into synthesis reactor (TJ4S-1213-Sy15). 将激活罐（TJ4S-1213-Sy15-R01）中体系转移至合成釜（TJ4S-1213-Sy15）中。 |  |  |  | Times/次数 |
|  |  |  |  | 1 |
| Transfer/转移 |  |  | Performed by&Date/执行人及日期： |  |  |
|  |  |  | Checked by & Date/核实人及日期： |  |  |
|  |  |  | Begin/开始： | End/结束： |  |
| ★Note:The coupling temperature is a CPP, and the coupling IPC hold temperature is a CPP./ 备注：偶联温度是CPP，偶联IPC维持温度是关键工艺参数。 The operator shall confirm that the reaction mixture temperature is within the Normal Operating Range (NOR) 【25±5oC (target: 25oC)】 /操作人员须执行反应体系温度处于正常操作范围（NOR）内【25±5oC (target: 25oC)】 Checked by & Date/核实人及日期： Over-temperature warning value/超温预警设置数值： Lower limit/低限: 21 oC ，High limit/高限: 28 oC |  |  |  |  |
| CAUTION/注意：DMF/N，N-二甲基甲酰胺：Irritation of eyes, skin and nose. May cause nausea. /刺激眼睛、皮肤和鼻子。可能引起恶心。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |

| 3.42. （13）Fmoc-L-Leu-OH coupling/芴甲氧羰基-L-亮氨酸偶联 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 3.42.2 DMF addition /加N,N-二甲基甲酰胺 Add 45.50 kg (Acceptable weighing range: 43.50 ~ 47.50 kg) (1 V) DMF by spray balls into activation tank (TJ4S-1213-Sy15-R01) to rinse the wall. 通过淋洗球向激活罐（TJ4S-1213-Sy15-R01）中加入45.50 kg（可接受称量范围：43.50 ~ 47.50 kg） (1 V) N,N-二甲基甲酰胺淋洗罐壁。 |  |  |  | Times/次数 |  |
|  |  |  |  | 1 |  |
| Rinse with DMF/ N,N-二甲基甲酰胺淋洗 | Performed by&Date/执行人及日期： |  |  |  |  |
| Rinse with DMF/ N,N-二甲基甲酰胺淋洗 | Begin/开始： | End/结束： | Qty/重量： |  |  |
| Transcribed by&Date/转录人及日期： |  |  |  |  |  |
| 3.42.3 Transfer /转移 Turn on the nitrogen of Synthesis reactor (TJ4S-1213-Sy15) to 5±2 m³/h (target: 5 m³/h). Bubble with nitrogen for 5±2 min (target: 5 min). Start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm). then start the agitator. Transfer the mixture in activation tank (TJ4S-1213-Sy15-R01) into synthesis reactor (TJ4S-1213-Sy15). 合成釜（TJ4S-1213-Sy15）先开启下口氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min (目标：5 min)后，开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm)，将激活罐（TJ4S-1213-Sy15-R01）中体系转移至合成釜（TJ4S-1213-Sy15）中。 |  |  |  | Times/次数 |  |
|  |  |  |  | 1 |  |
| Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h). Bubble with nitrogen for 5±2 min (target :5 min) 开启合成釜下口鼓氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min (目标时间：5 min) |  |  |  |  |  |
| Bubble/鼓氮气 |  | Performed by & Date/执行人及日期： |  |  |  |
| Bubble/鼓氮气 |  | Begin/开始： |  |  |  |
|  |  | The end time is documented in Ins3.42.5 /结束时间记录于3.42.5 |  |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |

| 3.42. （13）Fmoc-L-Leu-OH coupling/芴甲氧羰基-L-亮氨酸偶联 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm). /开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm) |  |  |  |  |
| Stir /搅拌 | Performed by & Date/执行人及日期： |  |  |  |
| Stir /搅拌 | Begin/开始： |  |  |  |
|  | The end time is documented in Ins3.42.5 /结束时间记录于3.42.5 |  |  |  |
| Transcribed by&Date/转录人及日期： |  |  |  |  |
| Transfer/转移 | Performed by&Date/执行人及日期： |  |  |  |
|  | Checked by & Date/核实人及日期： |  |  |  |
|  | Begin/开始： |  | End/结束： |  |
| 3.42.4 Adjust synthesis reactor temperature /合成釜调温 Adjust synthesis reactor (TJ4S-1213-Sy15) mixture to 【25±5 ℃ (target: 25 ℃) 】. 将合成釜（TJ4S-1213-Sy15）体系调温至【25±5 ℃(目标：25 ℃) 】. |  |  |  | Times/次数 |
|  |  |  |  | 1 |
| ★Note:The coupling temperature is a CPP, and the coupling IPC hold temperature is a CPP./ 备注：偶联温度是CPP，偶联IPC维持温度是关键工艺参数。 The total reaction Fmoc-L-Leu-OH coupling time：NMT 840 min (Sourced from the holding time study in the PAR study report.)/ 芴甲氧羰基-L-亮氨酸偶联总的反应时间不超过840min(来源于 PAR 研究报告中稳定时间研究)。 Over-temperature warning value/超温预警设置数值： Lower limit/低限: 22 oC ，High limit/高限: 28 oC |  |  |  |  |
| Adjust/调温 |  | Performed by&Date/执行人及日期： |  |  |
| Adjust/调温 |  | Begin/开始： | End/结束： |  |
| Transcribed by&Date/转录人及日期： |  |  |  |  |

| 3.42. （13）Fmoc-L-Leu-OH coupling/芴甲氧羰基-L-亮氨酸偶联 |  |
| --- | --- |
| 3.42.5 Fmoc-L-Leu-OH coupling/芴甲氧羰基-L-亮氨酸偶联 The mixture is allowed to react at 25±5 ℃ (target: 25 ℃). After stirring for 3 h±15 min (target: 3 h), sample the mixture every 0.5~1 h for ninhydrin test analysis until two consecutive samples test as negative (Yellow to Light pink Report Negative= Complete coupling; Blue to Purple Positive= Incomplete coupling). And the mixture should be stirred for coupling time (4 h≤Coupling Time≤10 h). Stop stirring, turn off the nitrogen. Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point. 控温【25±5 oC（目标温度25 oC）】，反应3 h±15 min（目标：3h）取样进行茚三酮检测，每隔0.5-1.0 h取样一次，直至连续两次取样结果为阴性（黄色至浅粉色，报告阴性=偶联完全；蓝色至紫色，阳性=偶联不完全）后,同时反应时间满足（4h≤反应时间≤10h）后，停止搅拌，关闭氮气。通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 Note: If resin adherence is found at other times during the reaction, rinse the reactor with 0.50~5.00 kg DMF each time. /备注：偶联过程中其它时间发现贴壁，每次使用0.50~5.00 kg N,N-二甲基甲酰胺淋洗，记录见附加页 | Times/次数 |
|  | 1 |

| 3.42. （13）Fmoc-L-Leu-OH coupling/芴甲氧羰基-L-亮氨酸偶联 |  |  |
| --- | --- | --- |
| ★After coupling is completed, the operator shall confirm that all Critical Process Parameters (CPP) are within their respective Normal Operating Ranges (NOR)/ 偶联结束后，操作人员须确认所有关键工艺参数（CPP）均处于各自的正常操作范围（NOR）内 ★Note: The coupling temperature is a CPP, and the coupling IPC hold temperature is a CPP./备注：偶联温度是CPP，偶联IPC维持温度是关键工艺参数。 The operator shall confirm that the reaction mixture temperature is within the Normal Operating Range (NOR) :20~30℃ /操作人员须确认反应体系温度处于正常操作范围（NOR）内:20~30℃ □Yes/是 □No/否 Confirmed by & Date/确认人及日期： Checked by & Date/核实人及日期： |  |  |
| React/反应 | Performed by&Date/执行人及日期： |  |
| React/反应 | Begin/开始： | End/结束： |
| Transcribed by&Date/转录人及日期： |  |  |

| 3.42. （13）Fmoc-L-Leu-OH coupling/芴甲氧羰基-L-亮氨酸偶联 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Internal temperature/内部温度 |  | High values/最高值： |  |  | Low values/最低值： |  |  |  |
| Transcribed by&Date/转录人及日期： |  |  |  |  |  |  |  |  |
| Standard sampling method/标准取样方法 |  |  |  |  |  |  |  |
| Take 5-10 ml mixture and perform testing in accordance with the ninhydrin test record. During sampling, should wear nitrile gloves and 3M 6000 series Full-face piece respirator, 6006CN cartridges. 通过在线取样器取样5~10 ml按照茚三酮检测记录进行检测。取样过程中，应该戴丁腈手套、3M 6000系列全面罩, 6006CN滤毒盒。 |  |  |  |  |  |  |  |
| Sampling number/ 取样编号 | Sampling time/取样时间 |  | Sampled by & Date/取样人及日期 | Negative or Positive/阴性或阳性 |  | Result filled by &Date/结果填写人及日期 | Checked by & Date/核实人及日期 |
| CPo140020- -01- -3.42.5-01 |  |  |  | Negative /阴性 Positive /阳性 |  |  |  |
| CPo140020- -01- -3.42.5-02 |  |  |  | Negative /阴性 Positive /阳性 |  |  |  |
| CPo140020- -01- -3.42.5-03 |  |  |  | Negative /阴性 Positive /阳性 |  |  |  |
| CPo140020- -01- -3.42.5-04 |  |  |  | Negative /阴性 Positive /阳性 |  |  |  |
| CPo140020- -01- -3.42.5-05 |  |  |  | Negative /阴性 Positive /阳性 |  |  |  |
| CPo140020- -01- -3.42.5-06 |  |  |  | Negative /阴性 Positive /阳性 |  |  |  |
| CPo140020- -01- -3.42.5-07 |  |  |  | Negative /阴性 Positive /阳性 |  |  |  |
| CPo140020- -01- -3.42.5-08 |  |  |  | Negative /阴性 Positive /阳性 |  |  |  |
| Confirm the end point of the reaction. /执行反应终点。 The acceptance criterion is two consecutive negative test results./连续两次检测结果为阴性即为合格。 Confirmed by & Date/确认人及日期： Checked by & Date/核实人及日期： |  |  |  |  |  |  |  |
| Stop the stirrer and turn off the nitrogen. Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point./ 停止搅拌，关闭氮气，通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  |  |  |  |  |  |
| Stir /搅拌 |  | Performed by & Date/执行人及日期： |  |  |  |  |  |
| Stir /搅拌 |  | End/结束： |  |  |  |  |  |
|  |  | The start time is documented in Ins 3.42.3 /开始时间记录于3.42.3 |  |  |  |  |  |
| Transcribed by & Date/转录人及日期： |  |  |  |  |  |  |  |

| 3.42. （13）Fmoc-L-Leu-OH coupling/芴甲氧羰基-L-亮氨酸偶联 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Bubble/鼓氮气 | Performed by & Date/执行人及日期： |  |  |  |
| Bubble/鼓氮气 | End/结束： |  |  |  |
|  | The start time is documented in Ins 3.42.3 /开始时间记录于3.42.3 |  |  |  |
| Transcribed by & Date/转录人及日期： |  |  |  |  |
| Internal temperature/内部温度 |  | High values/最高值： |  | Low values/最低值： |
| Transcribed by & Date/转录人及日期： |  |  |  |  |
| Filter/过滤 | Performed by&Date/执行人及日期： |  |  |  |
|  | Checked by & Date/核实人及日期： |  |  |  |
|  | Begin/开始： |  | End/结束： |  |
| Transcribed by&Date/转录人及日期： |  |  |  |  |
| ★After coupling is completed, the operator shall confirm that all Critical Process Parameters (CPPs) are within their respective Normal Operating Ranges (NOR)/ 偶联结束后，操作人员须确认所有关键工艺参数（CPP）均处于各自的正常操作范围（NOR）内 Checked by & Date/核实人及日期： |  |  |  |  |

| 3.42. （13）Fmoc-L-Leu-OH coupling/芴甲氧羰基-L-亮氨酸偶联 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| CAUTION/注意：DMF/N，N-二甲基甲酰胺：Irritation of eyes, skin and nose. May cause nausea. /刺激眼睛、皮肤和鼻子。可能引起恶心。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |  |
| 3.42.6 Activation Tank wash/激活罐洗涤 Add 364.03 kg (Acceptable weighing range：353.11 ~ 374.95 kg) (8 V) DMF into the activation tank (TJ4S-1213-Sy15-R01). Circulate and clean the activating liquid transfer pipeline 1, liquid charging port cleaning pipeline 2 and solid charging port cleaning pipeline 3, clean every pipeline for 2±1 min. g pipeline and liquid charging port cleaning pipeline. Clean every line wash for 2±1 min (target: 2 min). 向激活罐（TJ4S-1213-Sy15-R01）中加入364.03 kg（可接受称量范围：353.11 kg ~ 374.95 kg） (8 V) N,N-二甲基甲酰胺，循环清洗激活罐转移管路1，液体加料口清洗管路2和固体加料口清洗管路3，每条线路各清洗2±1 min（目标时间：2 min）。 |  |  |  |  |  | Times/次数 |
|  |  |  |  |  |  | 3 |
| Operation name/ 操作名称 |  | Time point/时间节点 | Clean the activating liquid transfer pipeline 1/循环清洗激活液转移管路1 | Liquid charging port cleaning pipeline 2 /液体加料口清洗管路2 | Solid charging port cleaning pipeline 3,/固体加料口清洗管路3 |  |
|  |  |  | Wash /洗涤1 | Wash /洗涤2 | Wash /洗涤3 |  |
| DMF addition/ 加N,N-二甲基甲酰胺 |  | Performed by & Date/执行人及日期： |  |  |  |  |
| DMF addition/ 加N,N-二甲基甲酰胺 |  | Begin/开始 |  |  |  |  |
|  |  | End/结束 |  |  |  |  |
|  |  | Qty/重量 |  |  |  |  |
|  |  | Transcribed by & Date /转录人及日期： |  |  |  |  |
| Circulate and clean/循环清洗 |  | Performed by & Date/执行人及日期： |  |  |  |  |
| Circulate and clean/循环清洗 |  | Begin/开始 |  |  |  |  |
|  |  | End/结束 |  |  |  |  |
|  |  | Transcribed by & Date /转录人及日期： |  |  |  |  |
| 3.42. （13）Fmoc-L-Leu-OH coupling/芴甲氧羰基-L-亮氨酸偶联 |  |  |  |  |  |  |
| 3.42.7 Post-coupling wash./偶联后洗涤 Transfer the mixture in activation tank (TJ4S-1213-Sy15-R01) to synthesis reactor (TJ4S-1213-Sy15). Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h). Bubble with nitrogen, after 5±2 min (target 5 min), Start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm). After stirring for 7±2 min (target：7 min), stop stirring, turn off the nitrogen. Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point. 将激活罐（TJ4S-1213-Sy15-R01）洗涤液转入合成釜 (TJ4S-1213-Sy15)。开启合成釜下口鼓氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min (目标时间：5 min)后开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm)，搅拌7±2 min（目标时间：7 min）后停止搅拌，关闭氮气。通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  |  |  |  | Times/次数 |
|  |  |  |  |  |  | 3 |
| Operation name/ 操作名称 |  | Time point/时间节点 | Wash /洗涤1 | Wash /洗涤2 | Wash /洗涤3 |  |
| Transfer the liquid form activation tank (TJ4S-1213-Sy15-R01) to the synthesis reactor (TJ4S-1213-Sy15) ./将激活罐(TJ4S-1213-Sy15-R01)中体系转移至合成釜(TJ4S-1213-Sy15)中。 |  |  |  |  |  |  |
| Transfer/转移 |  | Performed by & Date/执行人及日期： |  |  |  |  |
|  |  | Checked by & Date/核实人及日期： |  |  |  |  |
|  |  | Begin/开始 |  |  |  |  |
|  |  | End/结束 |  |  |  |  |
| Bubble/鼓氮气 |  | Performed by & Date/执行人及日期： |  |  |  |  |
| Bubble/鼓氮气 |  | Begin/开始 |  |  |  |  |
|  |  | End/结束 |  |  |  |  |
|  |  | Transcribed by & Date /转录人及日期： |  |  |  |  |

| 3.42. （13）Fmoc-L-Leu-OH coupling/芴甲氧羰基-L-亮氨酸偶联 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Operation name/ 操作名称 | Time point/时间节点 | Wash /洗涤1 | Wash /洗涤2 | Wash /洗涤3 |
| Stir /搅拌 | Performed by & Date/执行人及日期： |  |  |  |
| Stir /搅拌 | Begin/开始 |  |  |  |
|  | End/结束 |  |  |  |
|  | Transcribed by & Date /转录人及日期： |  |  |  |
| Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point/通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  |  |  |
| Filter/过滤 | Performed by & Date/执行人及日期： |  |  |  |
|  | Checked by & Date/核实人及日期： |  |  |  |
|  | Begin/开始 |  |  |  |
|  | End/结束 |  |  |  |

| - Deprotection/脱保护 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| CAUTION/注意：Piperidine哌啶：Strong local irritant and may cause permanent injury after short exposure to small amounts. /强烈的局部刺激，短时间少量接触可能造成永久性伤害。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |  |
| 3.43.1 Add piperidine solution/加哌啶哌溶液 Adjust the jacket temperature of synthesis reactor (TJ4S-1213-Sy15) to 【25±5 oC (target: 25 oC). 】.Add 357.50 kg (Acceptable weighing range: 346.77 ~ 368.23 kg) (8 V) 【20% (V/V) piperidine/DMF solution】 into the Synthesis reactor (TJ4S-1213-Sy15). 将合成釜(TJ4S-1213-Sy15)夹套调温至【25±5 ℃（目标温度：25 ℃）】。向合成釜（TJ4S-1213-Sy15）中加入357.50 kg（可接受称量范围：346.77 ~ 368.23 kg）(8 V)【20%哌啶/DMF (V/V)溶液】。 |  |  |  |  |  | Times/次数 |
|  |  |  |  |  |  | 1 |
| Adjust/调温 |  | Performed by & Date/执行人及日期： |  |  |  |  |
| Adjust/调温 |  | Begin/开始： |  | Adjust/调温 |  |  |
| Transcribed by & Date/转录人及日期： |  |  |  |  |  |  |
| Confirm and record that the 20%Piperidine solution concentration is within the NOR range./ 确认并记录20%哌啶溶液浓度在正常操作范围内：17.50%~22.50% 20%Piperidine solution concentration confirmation/20%哌啶溶液浓度确认：____________ Confirm and record that the 20%Piperidine solution temperature is within the NOR range./ 确认并记录20%哌啶溶液温度在正常操作范围内：20 oC ~30 oC 20%Piperidine solution temperature confirmation/20%哌啶溶液温度确认：____________ Confirmed by & Date/确认人及日期： Checked by & Date/核实人及日期： |  |  |  |  |  |  |
| piperidine addition/ 加哌啶溶液 | Performed by&Date/执行人及日期： |  |  |  |  |  |
| piperidine addition/ 加哌啶溶液 | Begin/开始： |  | End/结束： |  | Qty/重量： |  |
| Transcribed by&Date/转录人及日期： |  |  |  |  |  |  |

| 3.43. Deprotection/脱保护 |  |  |  |  |
| --- | --- | --- | --- | --- |
| 3.43.2 Adjust synthesis reactor temperature /合成釜调温 Turn on the nitrogen of Synthesis reactor (TJ4S-1213-Sy15) to 5±2 m³/h (target: 5 m³/h). Bubble with nitrogen for 5±2 min (target 5 min). Start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm). then start the stirrer. Adjust to 25±5 ℃ (target: 25 ℃）. 开启合成釜（TJ4S-1213-Sy15）下口鼓氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min (目标：5 min)后开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm)，调温至【25±5 ℃(目标温度：25 ℃)】。 |  |  |  | Times/次数 |
|  |  |  |  | 1 |
| Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h). Bubble with nitrogen for 5±2 min (target :5 min) 开启合成釜下口鼓氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min (目标时间：5 min) |  |  |  |  |
| Bubble/鼓氮气 |  |  | Performed by&Date/执行人及日期： |  |
| Bubble/鼓氮气 |  |  | Begin/开始： |  |
|  |  |  | The end time is documented in Ins 3.43.3/结束时间记录于3.43.3 |  |
| Transcribed by&Date/转录人及日期： |  |  |  |  |
| Start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm). /开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm) |  |  |  |  |
| Stir /搅拌 |  | Performed by&Date/执行人及日期： |  |  |
| Stir /搅拌 |  | Begin/开始： |  |  |
|  |  | The end time is documented in Ins 3.43.3/结束时间记录于3.43.3 |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |
| Adjust to 25±5 oC (target: 25 oC)./ 调温至25±5 ℃(目标温度：25 oC) |  |  |  |  |

| 3.43. Deprotection/脱保护 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Adjust/调温 |  |  | Performed by&Date/执行人及日期： |  |  |  |
| Adjust/调温 |  |  | Begin/开始： | End/结束： |  |  |
| Transcribed by&Date/转录人及日期： |  |  |  |  |  |  |
| 3.43.3 Fmoc-L- Orn(Boc)-OH deprotection /N-Fmoc-N'-Boc-L-鸟氨酸脱保护 After stirring for 【2 h±15 min (target: 2 h) 】at 【25±5 ℃ (target: 25 ℃) 】, stop stirring, turn off the nitrogen. Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point. 控温【25±5 ℃(目标：25 ℃) 】，搅拌【2 h±15 min（目标：2 h）】后，停止搅拌，关闭氮气。通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 Over-temperature warning value/超温预警设置数值： Lower limit/低限: 22 oC ，High limit/高限: 28 oC |  |  |  |  | Times/次数 |  |
|  |  |  |  |  | 1 |  |
| ★Confirm and record that the 20%Piperidine solution temperature is within the NOR range 20 oC ~30 oC ./ 确认并记录20%哌啶溶液温度在正常操作范围内：20 oC ~30 oC。 20%Piperidine solution temperature confirmation/20%哌啶溶液温度确认：____________ Confirmed by& Date/确认人及日期： Checked by & Date/核实人及日期： |  |  |  |  |  |  |
| Maintained and Stir/保温搅拌 |  |  | Performed by&Date/执行人及日期： |  |  |  |
| Maintained and Stir/保温搅拌 |  |  | End/结束： |  |  |  |
|  |  |  | The start time is documented in Ins3.43.2 /开始时间记录于3.43.2 |  |  |  |
| Transcribed by&Date/转录人及日期： |  |  |  |  |  |  |

| 3.43. Deprotection/脱保护 |  |  |  |
| --- | --- | --- | --- |
| Bubble/鼓氮气 |  | Performed by & Date/执行人及日期： |  |
| Bubble/鼓氮气 |  | End/结束： |  |
|  |  | The start time is documented in Ins3.43.2 /开始时间记录于3.42.2 |  |
| Transcribed by & Date /转录人及日期： |  |  |  |
| Internal temperature/内部温度 |  | High values/最高值: | Low values/最低值： |
| Transcribed by&Date/转录人及日期： |  |  |  |
| Filter/过滤 |  | Performed by&Date/执行人及日期： |  |
|  |  | Checked by & Date/核实人及日期： |  |
|  |  | Begin/开始： | End/结束： |
| Transcribed by&Date/转录人及日期： |  |  |  |
| ★After De-Fmoc is completed, the operator shall confirm that all Critical Process Parameters (CPPs) are within their respective Normal Operating Ranges (NOR)/ 脱保护结束后，操作人员须确认所有关键工艺参数（CPP）均处于各自的正常操作范围（NOR）内 Checked by & Date/核实人及日期： |  |  |  |

| 3.43. Deprotection/脱保护 |  |  |  |  |
| --- | --- | --- | --- | --- |
| 3.43.4 Synthesis reactor adjust temperature/合成釜调温 Adjust the synthesis reactor (TJ4S-1213-Sy15) jacket to 【20±5 ℃ (target: 20 ℃) 】. 将合成釜（TJ4S-1213-Sy15）夹套调温至【20±5 ℃（目标温度：20 ℃）】。 |  |  |  | Times/次数 |
|  |  |  |  | 1 |
| Adjust/调温 |  | Performed by & Date/执行人及日期： |  |  |
| Adjust/调温 |  | Begin/开始： | End/结束： |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |
| CAUTION/注意：IPAC/乙酸异丙酯： Vapors irritate eyes and respiratory tract; high concentrations can be anesthetic. Liquid irritates eyes but causes no serious injury; may cause dermatitis; no serious effects if swallowed. /蒸汽会刺激眼睛和呼吸道；高浓度的蒸汽可能是麻醉剂。液体刺激眼睛，但不会造成严重伤害；可能会引起皮炎；吞食不会造成严重影响。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |
| 3.43.5 Post-deprotection wash/脱保护后洗涤 Add 445.92 kg (Acceptable weighing range：432.54 ~ 459.30 kg) (9.29 kg/kg, 10.64 V) isopropyl acetate into synthesis reactor (TJ4S-1213-Sy15). Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h). Bubble with nitrogen for 5±2 min (target 5 min), start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm). After stirring for 5±2 min (target 5 min), stop stirring, turn off the nitrogen. Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point. 向合成釜（TJ4S-1213-Sy15）中加入445.92 kg（可接受称量范围：432.54 ~ 459.30 kg) (9.29 kg/kg, 10.64 V）乙酸异丙酯，开启下口氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min (目标时间：5 min)后开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm)，搅拌5±2 min (目标时间：5 min)后，停止搅拌，关闭氮气。通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  |  | Times/次数 |
|  |  |  |  | 10 |
| ★Note: IPAC quantity is a CPP./备注：乙酸异丙酯用量是关键工艺参数。 ★The operator shall confirm that IPAC quantity is within the Normal Operating Range (NOR) 【432.54 kg~459.30 kg】/操作人员须确认乙酸异丙酯用量处于正常操作范围（NOR）内：【432.54 kg~459.30 kg】 Checked by & Date/核实人及日期： |  |  |  |  |

| 3.43. Deprotection/脱保护 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Operation name/操作名称 |  | Time point/时间节点 | Wash /洗涤1 | Wash /洗涤2 | Wash /洗涤3 | Wash /洗涤4 | Wash /洗涤5 |
| Add 419.04 kg IPAc (10 V) by spray balls into the synthesis reactor (TJ4S-1213-Sy15)./ 向合成釜(TJ4S-1213-Sy15)中加419.04 kg乙酸异丙酯（10 V)）（Acceptable weighing range/可接受称量范围：406.47~431.61 kg） |  |  |  |  |  |  |  |
| IPAC addition/加乙酸异丙酯 |  | Performed by & Date/执行人及日期： |  |  |  |  |  |
| IPAC addition/加乙酸异丙酯 |  | Begin/开始 |  |  |  |  |  |
|  |  | End/结束 |  |  |  |  |  |
|  |  | Qty/重量 |  |  |  |  |  |
|  |  | Transcribed by & Date /转录人及日期： |  |  |  |  |  |
| Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h) Bubble with nitrogen for 5±2 min (target: 5 min). Start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm). Stir for 5±2 min (target: 5 min), stop stirring, turn off the nitrogen./ 开启合成釜下口鼓氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min（目标时间：5 min）后开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm)，搅拌5±2 min（目标时间：5min）后，停止搅拌，关闭氮气。 |  |  |  |  |  |  |  |
| Bubble/鼓氮气 |  | Performed by & Date/执行人及日期： |  |  |  |  |  |
| Bubble/鼓氮气 |  | Begin/开始 |  |  |  |  |  |
|  |  | End/结束 |  |  |  |  |  |
|  |  | Transcribed by & Date /转录人及日期： |  |  |  |  |  |
| Stir /搅拌 |  | Performed by & Date/执行人及日期： |  |  |  |  |  |
| Stir /搅拌 |  | Begin/开始 |  |  |  |  |  |
|  |  | End/结束 |  |  |  |  |  |
|  |  | Transcribed by & Date /转录人及日期： |  |  |  |  |  |

| 3.43. Deprotection/脱保护 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operation name/操作名称 |  | Time point/时间节点 |  | Wash /洗涤1 |  | Wash /洗涤2 |  | Wash /洗涤3 |  | Wash /洗涤4 |  | Wash /洗涤5 |  |
| Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point./通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Filter/过滤 |  | Performed by & Date/执行人及日期： |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Checked by & Date/核实人及日期： |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Begin/开始 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | End/结束 |  |  |  |  |  |  |  |  |  |  |  |

| 3.43. Deprotection/脱保护 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operation name/操作名称 |  | Time point/时间节点 |  | Wash /洗涤6 |  | Wash /洗涤7 |  | Wash /洗涤8 |  | Wash /洗涤9 |  | Wash /洗涤10 |  |
| Add 419.04 kg IPAc (10 V) by spray balls into the synthesis reactor (TJ4S-1213-Sy15)./ 向合成釜(TJ4S-1213-Sy15)中加419.04 kg乙酸异丙酯（10 V)）（Acceptable weighing range/可接受称量范围：406.47~431.61 kg） |  |  |  |  |  |  |  |  |  |  |  |  |  |
| IPAC addition/加乙酸异丙酯 |  | Performed by & Date/执行人及日期： |  |  |  |  |  |  |  |  |  |  |  |
| IPAC addition/加乙酸异丙酯 |  | Begin/开始 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | End/结束 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Qty/重量 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |  |  |  |  |
| Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h) Bubble with nitrogen for 5±2 min (target: 5 min). Start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm). Stir for 5±2 min (target: 5 min), stop stirring, turn off the nitrogen./ 开启合成釜下口鼓氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min（目标时间：5 min）后开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm)，搅拌5±2 min（目标时间：5 min）后，停止搅拌，关闭氮气。 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bubble/鼓氮气 |  | Performed by & Date/执行人及日期： |  |  |  |  |  |  |  |  |  |  |  |
| Bubble/鼓氮气 |  | Begin/开始 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | End/结束 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |  |  |  |  |
| Stir /搅拌 |  | Performed by & Date/执行人及日期： |  |  |  |  |  |  |  |  |  |  |  |
| Stir /搅拌 |  | Begin/开始 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | End/结束 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |  |  |  |  |

| 3.43. Deprotection/脱保护 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operation name/操作名称 |  | Time point/时间节点 |  | Wash /洗涤6 |  | Wash /洗涤7 |  | Wash /洗涤8 |  | Wash /洗涤9 |  | Wash /洗涤10 |  |
| Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point./通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Filter/过滤 |  | Performed by & Date/执行人及日期： |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Checked by & Date/核实人及日期： |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Begin/开始 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | End/结束 |  |  |  |  |  |  |  |  |  |  |  |

| 3.43. Deprotection/脱保护 |  |  |  |  |
| --- | --- | --- | --- | --- |
| 3.43.5 IPC/过程控制 Starting with the 10th wash, sample the isopropyl acetate washing solution during stirring and develop with chloranil, compare the color with the standard 500 ppm piperidine/IPAC solution as reference solution. If the color is darker than the reference solution, invoke modules 3.44 until the color of the washing solution’s chloranil test is lighter or consistent with the reference solution and. If the color of the washing solution’s chloranil test is lighter or matches with the reference solution and then invoke modules 3.45-3.47. 取样第10次乙酸异丙酯洗涤液，采用四氯苯醌显色，与标准500 ppm哌啶/乙酸异丙酯溶液参比液比色：若颜色重于参比液，调用3.44模块，直至洗涤液颜色轻于参比液或与参比液颜色一致；若颜色轻于参比液或与参比液一致，调用3.45-3.47模块。 |  |  |  | Times/次数 |
|  |  |  |  | N/A |
| Standard sampling method/标准取样方法 |  |  |  |  |
| Take ~10 mL IPAc washing solution for develop testing. During sampling, should wear nitrile gloves and 3M 6000 series Full-face piece respirator, 6006CN cartridges. 取乙酸异丙酯洗液~10mL，进行显色检测。取样过程中，应该戴丁腈手套、3M 6000系列全面罩, 6006CN滤毒盒。 |  |  |  |  |
| Sample and develop with chloranil. /取样检测四氯苯醌显色。 Sampling number/取样编号：CPo140020- -01- -3.43.5-01 Sampling time/取样时间： Sampled by & Date/取样人及日期： The color is lighter or consistent with the reference solution, invoke modules 3.45-3.47. /颜色轻于参比液或与参比液一致，调用3.45-3.47模块； Otherwise, invoke modules 3.44. /否则，调用3.44模块； Performed by & Date/执行人及日期： Checked by & Date/核实人及日期： |  |  |  |  |
| Invoke synthesis recipe. /调用已经建立的合成配方： Recipe name /配方名称：14002001M01V02 Batch name /批次命名：CPo140020 01 Move to Modules 3.44; /调用模块：3.44； Move to Modules 3.45; /调用模块：3.45-3.47； Performed by & Date/执行人及日期： Checked by & Date/核实人及日期： |  |  |  |  |
| - Wash resin/树脂洗涤 |  |  |  |  |
| CAUTION/注意：IPAC/乙酸异丙酯： Vapors irritate eyes and respiratory tract; high concentrations can be anesthetic. Liquid irritates eyes but causes no serious injury; may cause dermatitis; no serious effects if swallowed. /蒸汽会刺激眼睛和呼吸道；高浓度的蒸汽可能是麻醉剂。液体刺激眼睛，但不会造成严重伤害；可能会引起皮炎；吞食不会造成严重影响。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |
| 3.44.1 Post-deprotection wash/脱保护后洗涤 Add 419.04 kg (Acceptable weighing range：406.47 ~ 431.61 kg) (10 V) isopropyl acetate into synthesis reactor (TJ4S-1213-Sy15). Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h). Bubble with nitrogen for 5±2 min (target 5 min), start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm). After stirring for 5±2 min (target 5 min), stop stirring, turn off the nitrogen. Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point. 向合成釜（TJ4S-1213-Sy15）中加入419.04 kg（可接受称量范围：406.47 ~ 431.61 kg）（10 V）乙酸异丙酯，开启下口氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min (目标时间：5 min)后开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm)，搅拌5±2 min (目标时间：5 min)后，停止搅拌，关闭氮气。通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  |  | Times/次数 |
|  |  |  |  | 2 |
| Operation name/操作名称 | Time point/时间节点 | Wash /洗涤1 | Wash /洗涤2 |  |
| Add 419.04 kg IPAc (10 V) by spray balls into the synthesis reactor (TJ4S-1213-Sy15)./ 向合成釜(TJ4S-1213-Sy15) 中加419.04 kg乙酸异丙酯（10 V)）（Acceptable weighing range/可接受称量范围：406.47~431.61kg） |  |  |  |  |
| IPAC addition/加乙酸异丙酯 | Performed by&Date/执行人及日期 |  |  |  |
| IPAC addition/加乙酸异丙酯 | Begin/开始 |  |  |  |
|  | End/结束 |  |  |  |
|  | Qty/重量 |  |  |  |
|  | Transcribed by&Date/转录人及日期 |  |  |  |

| 3.44. Wash resin/树脂洗涤 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Operation name/操作名称 |  | Time point/时间节点 | Wash /洗涤1 |  | Wash /洗涤2 |  |
| Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h) Bubble with nitrogen for 5±2 min (target: 5 min). Start the agitator of synthesis reactor to 20±10rpm (target: 20 rpm). Stir for 5±2 min (target: 5 min), stop stirring, turn off the nitrogen./ 开启合成釜下口鼓氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min（目标时间：5 min）后开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm)，搅拌5±2 min（目标时间：5 min）后，停止搅拌，关闭氮气。 |  |  |  |  |  |  |
| Bubble/鼓氮气 |  | Performed by&Date/执行人及日期 |  |  |  |  |
| Bubble/鼓氮气 |  | Begin/开始 |  |  |  |  |
|  |  | End/结束 |  |  |  |  |
|  |  | Transcribed by&Date/转录人及日期 |  |  |  |  |
| Stir /搅拌 |  | Performed by&Date/执行人及日期 |  |  |  |  |
| Stir /搅拌 |  | Begin/开始 |  |  |  |  |
|  |  | End/结束 |  |  |  |  |
|  |  | Transcribed by&Date/转录人及日期 |  |  |  |  |
| Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point./通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  |  |  |  |  |
| Filter/过滤 |  | Performed by & Date/执行人及日期： |  |  |  |  |
|  |  | Checked by & Date/核实人及日期： |  |  |  |  |
|  |  | Begin/开始 |  |  |  |  |
|  |  | End/结束 |  |  |  |  |

| 3.44. Wash resin/树脂洗涤 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 3.44.1 Sample /取样 Sample the second isopropyl acetate washing solution during stirring and develop with chloranil, compare the color with the standard 500 ppm piperidine/IPAC solution as reference solution. If the color is darker than the reference solution, repeat invoke modules 3.44 until the color of the washing solution’s chloranil test is lighter or consistent with the reference solutionand. If the color of the washing solution’s chloranil test is lighter or consistent with the reference solution and then invoke modules 3.45-3.47. 取样第2次乙酸异丙酯洗涤液，采用四氯苯醌显色，与标准500ppm哌啶/乙酸异丙酯溶液参比液比色：若颜色重于参比液，重新调用3.44模块，直至洗涤液颜色轻于参比液或与参比液颜色一致；若颜色轻于参比液或与参比液一致，调用3.45-3.47模块。 |  |  |  |  | Times/次数 |
|  |  |  |  |  | N/A |
| Standard sampling method/标准取样方法 |  |  |  |  |  |
| Take ~10 ml IPAc washing solution for develop testing. During sampling, should wear nitrile gloves and 3M 6000 series Full-face piece respirator, 6006CN cartridges. 取乙酸异丙酯洗液~10mL，进行显色检测。取样过程中，应该戴丁腈手套、3M 6000系列全面罩, 6006CN滤毒盒。 |  |  |  |  |  |
| Sample and develop with chloranil. /取样检测四氯苯醌显色。 Sampling number/取样编号：CPo140020- -01- -3.44.1-01 Sampling time/取样时间： Sampled by & Date/取样人及日期： The color is lighter or consistent with the reference solution, invoke modules 3.45-3.47. /颜色轻于参比液或与参比液一致，调用3.45-3.47模块； Otherwise, invoke modules 3.44 again. /否则，重新调用3.44模块； Performed by & Date/执行人及日期： Checked by & Date/核实人及日期： |  |  |  |  |  |
| Invoke synthesis recipe. /调用已经建立的合成配方： Recipe name /配方名称：14002001M01V02 Batch name /批次命名：CPo140020 01 Move to Modules 3.44; /调用模块：3.44； Move to Modules 3.45-3.47; /调用模块：3.45-3.47； Performed by & Date/执行人及日期： Checked by & Date/核实人及日期： |  |  |  |  |  |
| - （14）Deprotection/脱保护 |  |  |  |  |  |
| CAUTION/注意：DMF/N，N-二甲基甲酰胺：Irritation of eyes, skin and nose. May cause nausea. /刺激眼睛、皮肤和鼻子。可能引起恶心。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |
| 3.45.1 DMF addition /加N,N-二甲基甲酰胺 Add 546.05 kg (Acceptable weighing range: 529.67 ~ 562.43 kg) (12 V) DMF into synthesis reactor (TJ4S-1213-Sy15). 向合成釜（TJ4S-1213-Sy15）中加入546.05 kg（可接受称量范围：529.67 ~ 562.43 kg）（12 V）N,N-二甲基甲酰胺。 |  |  |  |  | Times/次数 |
|  |  |  |  |  | 1 |
| DMF addition/加N,N-二甲基甲酰胺 |  | Performed by&Date/执行人及日期： |  |  |  |
| DMF addition/加N,N-二甲基甲酰胺 |  | Begin/开始： | End/结束： | Qty/重量： |  |
| Transcribed by&Date/转录人及日期： |  |  |  |  |  |
| CAUTION/注意：DMF/N，N-二甲基甲酰胺：Irritation of eyes, skin and nose. May cause nausea. /刺激眼睛、皮肤和鼻子。可能引起恶心。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |
| 3.45.2 Resin reswelling wash 1/溶胀后洗涤1 Turn on the nitrogen of synthesis reactor (TJ4S-1213-Sy15) to 5±2 m³/h (target: 5 m³/h). Bubble with nitrogen for 5±2 min (target: 5 min). Start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm). After stirring for 10±2 min (target: 10 min), stop stirring, turn off the nitrogen. Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point. 开启合成釜（TJ4S-1213-Sy15）下口氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min (目标时间：5 min)后，开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm)，搅拌10±2 min（目标时间：10 min）后，停止搅拌，关闭氮气。通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  |  |  | Times/次数 |
|  |  |  |  |  | 1 |

| 3.45. （14）Deprotection/脱保护 |  |  |  |
| --- | --- | --- | --- |
| Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h) for 5±2 min (target: 5 min), Start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm). Stir for 10±2 min (target: 10 min). Stop stirring, turn off the nitrogen./ 开启合成釜下口鼓氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min（目标时间：5 min）后，开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm)，搅拌10±2 min（目标时间:10 min）后，停止搅拌，关闭氮气。 |  |  |  |
| Bubble/鼓氮气 |  | Performed by&Date/执行人及日期： |  |
| Bubble/鼓氮气 |  | Begin/开始： | End/结束： |
| Transcribed by&Date/转录人及日期： |  |  |  |
| Stir /搅拌 |  | Performed by&Date/执行人及日期： |  |
| Stir /搅拌 |  | Begin/开始： | End/结束： |
| Transcribed by&Date/转录人及日期： |  |  |  |
| Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point./通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  |  |
| Filter/过滤 |  | Performed by&Date/执行人及日期： |  |
|  |  | Checked by & Date/核实人及日期： |  |
|  |  | Begin/开始： | End/结束： |

| 3.45. （14）Deprotection/脱保护 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CAUTION/注意：DMF/N，N-二甲基甲酰胺：Irritation of eyes, skin and nose. May cause nausea. /刺激眼睛、皮肤和鼻子。可能引起恶心。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |  |  |
| 3.45.3 DMF addition /加N,N-二甲基甲酰胺 Add 546.05 kg (Acceptable weighing range: 529.67 ~ 562.43 kg) (12 V) DMF into synthesis reactor (TJ4S-1213-Sy15). 向合成釜（TJ4S-1213-Sy15）中加入546.05 kg（可接受称量范围：529.67 ~ 562.43 kg）（12 V）N,N-二甲基甲酰胺。 |  |  |  |  |  |  | Times/次数 |
|  |  |  |  |  |  |  | 1 |
| DMF addition/ 加N,N-二甲基甲酰胺 |  | Performed by&Date/执行人及日期： |  |  |  |  |  |
| DMF addition/ 加N,N-二甲基甲酰胺 |  | Begin/开始： |  | End/结束： |  | Qty/重量： |  |
| Transcribed by&Date/转录人及日期： |  |  |  |  |  |  |  |
| 3.45.4 Resin reswelling wash 2/溶胀后洗涤2 Turn on the nitrogen of synthesis reactor(TJ4S-1213-Sy15) to 5±2 m³/h (target: 5 m³/h). Bubble with nitrogen for 5±2 min (target: 5 min). Start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm). After stirring for 10±2 min (target: 10 min), stop stirring, turn off the nitrogen. Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point. 开启合成釜（TJ4S-1213-Sy15）下口氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min (目标时间：5 min)后，开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm)，搅拌10±2 min（目标时间：10 min）后，停止搅拌，关闭氮气。通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  |  |  |  |  | Times/次数 |
|  |  |  |  |  |  |  | 1 |
| Bubble/鼓氮气 |  |  | Performed by&Date/执行人及日期： |  |  |  |  |
| Bubble/鼓氮气 |  |  | Begin/开始： |  | End/结束： |  |  |
| Transcribed by&Date/转录人及日期： |  |  |  |  |  |  |  |

| 3.45. （14）Deprotection/脱保护 |  |  |  |
| --- | --- | --- | --- |
| Stir /搅拌 |  | Performed by&Date/执行人及日期： |  |
| Stir /搅拌 |  | Begin/开始： | End/结束： |
| Transcribed by&Date/转录人及日期： |  |  |  |
| Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point./通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  |  |
| Filter/过滤 |  | Performed by&Date/执行人及日期： |  |
|  |  | Checked by & Date/核实人及日期： |  |
| Filter/过滤 |  | Begin/开始： | End/结束： |

| - （14）Fmoc-L- Orn(Boc)-OH activation/N-Fmoc-N'-Boc-L-鸟氨酸活化 |  |  |  |  |
| --- | --- | --- | --- | --- |
| CAUTION/注意：DMF/N，N-二甲基甲酰胺：Irritation of eyes, skin and nose. May cause nausea. /刺激眼睛、皮肤和鼻子。可能引起恶心。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |
| 3.46.1 Add DMF to the activation reactor/激活罐加N,N-二甲基甲酰胺 Add 227.52 kg (Acceptable weighing range: 220.69 ~ 234.34 kg) (5 V) DMF into activation tank (TJ4S-1213-Sy15-R01). Start the agitator of activation tank to 95±10 rpm (target: 95 rpm). 向激活罐（TJ4S-1213-Sy15-R01）中加入227.52 kg（可接受称量范围：220.69 ~ 234.34 kg ）（5 V）N，N-二甲基甲酰胺，Start the agitator of activation tank to 95±10 rpm (target: 95 rpm)。 |  |  |  | Times/次数 |
|  |  |  |  | 1 |
| Add 227.52 kg DMF (Acceptable weighing range: 220.69 ~ 234.34 kg) (5 V) by spray balls into activation tank (TJ4S-1213-Sy15-R01).通过淋洗球向激活罐(TJ4S-1213-Sy15-R01)中加入227.52 kg N, N-二甲基甲酰胺（可接受称量范围：220.69 ~ 234.34 kg）(5 V)。 |  |  |  |  |
| DMF addition/加N,N-二甲基甲酰胺 | Performed by&Date/执行人及日期： |  |  |  |
| DMF addition/加N,N-二甲基甲酰胺 | Begin/开始： | End/结束： | Qty/重量： |  |
| Transcribed by&Date/转录人及日期： |  |  |  |  |
| Start the agitator of activation tank to 95±10 rpm (target: 95 rpm)./ 开启激活罐搅拌转速至95±10 rpm (目标转速: 95 rpm)。 |  |  |  |  |
| Stir/搅拌 | Performed by&Date/执行人及日期： |  |  |  |
| Stir/搅拌 | Begin/开始： |  |  |  |
|  | The end time is documented in Ins 3.46.8/结束时间记录于3.46.8 |  |  |  |
| Transcribed by&Date/转录人及日期： |  |  |  |  |
| 3.46.2 Add Fmoc-L-Orn(Boc)-OH/ 加N-Fmoc-N'-Boc-L-鸟氨酸 Add Fmoc-L-Orn(Boc)-OH (2.00 eq.) with powder feeding bags into activation tank (TJ4S-1213-Sy15-R01). 通过投料袋向激活罐（TJ4S-1213-Sy15-R01）中加入 kg N-Fmoc-N'-Boc-L-鸟氨酸（Acceptable weighing range/可接受称量范围： kg ~ kg）(2.00 eq.)。 |  |  |  | Times/次数 |
|  |  |  |  | 1 |

| 3.46. （14）Fmoc-L- Orn(Boc)-OH activation/N-Fmoc-N'-Boc-L-鸟氨酸活化 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Before material charging, confirm the material is within the retest date/ the expiry date, confirm whether the weight is adequate for use in the current charging. /加物料前，执行物料在复检日期/有效期之内, 确认重量是否满足本次投料使用。 Before solid material charging, confirm/加固体物料前，确认： - Before operation performed, confirm there is no open operation in the operation room/area; No non-related material / equipment / instrument / tools / pipeline / document / record / label in the intended area /操作前，执行操作房间/区域无开放性操作；目标区域无不相关物料/设备/仪器/工具/管路/文件/记录/标签； - The outer packages are removed. If the outer package cannot be removed, but unpack material without damaging the package bag, and before charging, check no package fragment and other extraneous matter in material. /外包装移除；如果外包装不能移除，但在不破坏包装的情况下打开包装袋，投料前以及过程中同时检查执行物料中无包装碎片以及异物 Checked by & Date/核实人及日期： |  |  |  |  |  |
| Prior to charging each amino acid, the operator shall perform a Raman scan to confirm the identity of the material. The Raman scan result shall be compared against the reference spectrum of the corresponding amino acid. Only materials that conform to the reference spectrum may be charged. If the Raman scan result does not conform, stop the operation, quarantine the material, and initiate a investigation per site SOP. Record the Raman scan result, operator signature, and date/time in the table below./ 每次投氨基酸前，操作人员应进行拉曼扫描以执行物料。拉曼扫描结果应与相应氨基酸的参考光谱进行比对。仅与参考光谱相符的物料方可投料。如拉曼扫描结果不相符，应停止操作，对物料进行隔离，并依据厂区 SOP 启动异常事件调查。在下表中记录拉曼扫描结果、操作人员签字及日期 / 时间。 |  |  |  |  |  |
| Charge Fmoc-L-Orn(Boc)-OH /加 N-Fmoc-N'-Boc-L-鸟氨酸 |  | Performed by & Date/执行人及日期： |  |  |  |
|  |  | Begin/开始： |  | End/结束： |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |
| Fmoc-L-Orn(Boc)-OH confirmation/ N-Fmoc-N'-Boc-L-鸟氨酸执行：Control No./内部控制号YL005318 |  |  |  |  |  |
| Check items/检查项 | Is it correct?/是否正确 |  | Recorded by & Date/ 记录人及日期 |  | Checked by & Date/ 核实人及日期 |
| Material name/物料名称 | Yes/是；No/否 |  |  |  |  |
| Project batch number/ 项目批次号 | Yes/是；No/否 |  |  |  |  |
| Operation number/ 操作号 | Yes/是；No/否 |  |  |  |  |
| Control No./内部控制号 | Yes/是；No/否 |  |  |  |  |
| Raman Scan Confirmation/拉曼扫描执行 | Conforms/相符 Does not conform/不相符 |  |  |  |  |

| 3.46. （14）Fmoc-L- Orn(Boc)-OH activation/N-Fmoc-N'-Boc-L-鸟氨酸活化 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No. /编号 | Net weight/净重 | All in/全部加入 |  |  |  |  | Recorded by & Date/ 记录人及日期 |  |  | Checked by & Date/ 核实人及日期 |  |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |  |  |  |  |  |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |  |  |  |  |  |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |  |  |  |  |  |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |  |  |  |  |  |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |  |  |  |  |  |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |  |  |  |  |  |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |  |  |  |  |  |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |  |  |  |  |  |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |  |  |  |  |  |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |  |  |  |  |  |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |  |  |  |  |  |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |  |  |  |  |  |  |  |  |
| Total/合计 |  | Confirm the total net weight is within the input range/执行合计净重在投料范围内 |  |  |  |  | Recorded by & Date/记录人及日期 |  |  | Checked by & Date/ 核实人及日期 |  |  |  |  |
|  |  | Yes/是；No/否 |  |  |  |  |  |  |  |  |  |  |  |  |
| After solid material charging, confirm/固体物料投料之后，确认： - The corresponding leftover material has been properly sealed and transferred to the staging area/相应的剩余物料已经恰当密封，转移到了暂存区域。 - The waste packaging material has been sealed and transferred to the corresponding waste staging area. /产生的包材废物已经密封并转移到了相应的废物暂存区域。 - The operation area/equipment surface has been cleaned/操作区域以及设备表面已经清洁。 Checked by & Date/核实人及日期： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3.46.3 Agitation of activation tank /激活罐搅拌 Continue agitation of activation tank to 95±10 rpm (target: 95 rpm). Maintain stirring for at least 15 min. Confirm no visible solids and later use. 维持激活罐搅拌转速至95±10 rpm (目标转速: 95 rpm), 搅拌至少15 min，确认无明显固体待用。 |  |  |  |  |  |  |  |  |  |  |  |  | Times/次数 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |  |
| 3.46. （14）Fmoc-L- Orn(Boc)-OH activation/N-Fmoc-N'-Boc-L-鸟氨酸活化 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Maintain & Stir /保持搅拌 |  |  | Performed by & Date/执行人及日期： |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Begin/开始： |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | The end time is documented in Ins 3.46.8/结束时间记录于3.46.8 |  |  |  |  |  |  |  |  |  |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Confirm no visible solid/确认无明显固体： |  |  |  |  |  | Confirmed by& Date /确认人及日期： |  |  |  |  |  |  |  |  |
| 3.46.4 Adjust the temperature of activation tank /激活罐调温 Adjust the mixture to 【20±5 ℃ (target 20 ℃) 】. /将体系调温至【20±5 ℃（目标温度：20 ℃）】。 |  |  |  |  |  |  |  |  |  |  |  | Times/次数 |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | 1 |  |  |
| Adjust/调温 |  |  |  |  | Performed by&Date/执行人及日期： |  |  |  |  |  |  |  |  |  |
| Adjust/调温 |  |  |  |  | Begin/开始： |  |  |  | End/结束： |  |  |  |  |  |
| Transcribed by&Date/转录人及日期： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CAUTION/注意：OxymaPure is highly toxic, high-energy compound. /2-肟氰乙酸乙酯是高毒，高能化合物。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3.46.5 OxymaPure addition /加2-肟氰乙酸乙酯 Add OxymaPure (2.00 eq.) with powder feeding bags into activation tank (TJ4S-1213-Sy15-R01) at 【20±5 ℃ (target 20 ℃)】. 控温【20±5 ℃（目标：20 ℃）】，通过投料袋向激活罐（TJ4S-1213-Sy15-R01）中加入 kg 2-肟氰乙酸乙酯（Acceptable weighing range/可接受称量范围： kg ~ kg）(2.00 eq.)。 |  |  |  |  |  |  |  |  |  |  |  | Times/次数 |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | 1 |  |  |
| Before material charging, confirm the material is within the retest date/ the expiry date,Confirm whether the weight is adequate for use in the current charging. /加物料前，执行物料在复检日期/有效期之内，确认重量是否满足本次投料使用。 - Before operation performed, no non-related material / equipment / instrument / tools / pipeline / document / record / label in the intended area /操作前，目标区域无不相关物料/设备/仪器/工具/管路/文件/记录/标签； - The outer packages are removed. If the outer package cannot be removed, but unpack material without damaging the package bag, and before charging, check no package fragment and other extraneous matter in material. /外包装移除；如果外包装不能移除，但在不破坏包装的情况下打开包装袋，投料前以及过程中同时检查执行物料中无包装碎片以及异物 Performed by & Date/核实人及日期： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Check and record that the solid charge port is free of solid accumulation from the previous step./ 检查并记录固体投料口无来自上一步骤的固体残留。 Checked by & Date/核实人及日期： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Charge OxymaPure /加2-肟氰乙酸乙酯 |  |  |  |  | Begin/开始： |  |  |  | End/结束： |  |  |  |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3.46. （14）Fmoc-L- Orn(Boc)-OH activation/N-Fmoc-N'-Boc-L-鸟氨酸活化 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Internal temperature/温度 |  |  |  |  | High values/最高值： |  |  |  | Low values/最低值： |  |  |  |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| OxymaPure confirmation/2-肟氰乙酸乙酯确认：Control No./内部控制号：YL005180 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Check items/检查项 |  | Is it correct?/是否正确 |  |  |  |  | Recorded by & Date/ 记录人及日期 |  |  | Checked by & Date/ 核实人及日期 |  |  |  |  |
| Material name/物料名称 |  | Yes/是；No/否 |  |  |  |  |  |  |  |  |  |  |  |  |
| Project batch number/ 项目批次号 |  | Yes/是；No/否 |  |  |  |  |  |  |  |  |  |  |  |  |
| Check items/检查项 |  | Is it correct?/是否正确 |  |  |  |  | Recorded by & Date/ 记录人及日期 |  |  | Checked by & Date/ 核实人及日期 |  |  |  |  |
| Operation number/ 操作号 |  | Yes/是；No/否 |  |  |  |  |  |  |  |  |  |  |  |  |
| Control No./内部控制号 |  | Yes/是；No/否 |  |  |  |  |  |  |  |  |  |  |  |  |
| No. /编号 | Net weight/净重 | All in/全部加入 |  |  |  |  | Recorded by & Date/ 记录人及日期 |  |  | Checked by & Date/ 核实人及日期 |  |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |  |  |  |  |  |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |  |  |  |  |  |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |  |  |  |  |  |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |  |  |  |  |  |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |  |  |  |  |  |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |  |  |  |  |  |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |  |  |  |  |  |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |  |  |  |  |  |  |  |  |
| / |  | Yes/是；No/否 |  |  |  |  |  |  |  |  |  |  |  |  |

| 3.46. （14）Fmoc-L- Orn(Boc)-OH activation/N-Fmoc-N'-Boc-L-鸟氨酸活化 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| No. /编号 | Net weight/净重 | All in/全部加入 | Recorded by & Date/ 记录人及日期 | Checked by & Date/ 核实人及日期 |
| / |  | Yes/是；No/否 |  |  |
| / |  | Yes/是；No/否 |  |  |
| / |  | Yes/是；No/否 |  |  |
| Total/合计 |  | Confirm the total net weight is within the input range/执行合计净重在投料范围内 | Recorded by & Date/记录人及日期 | Checked by & Date/ 核实人及日期 |
|  |  | Yes/是；No/否 |  |  |
| After solid material charging, confirm/固体物料投料之后，确认： - The corresponding leftover material has been properly sealed and transferred to the staging area/相应的剩余物料已经恰当密封，转移到了暂存区域。 - The waste packaging material has been sealed and transferred to the corresponding waste staging area. /产生的包材废物已经密封并转移到了相应的废物暂存区域。 - The operation area/equipment surface has been cleaned/操作区域以及设备表面已经清洁。 Checked by & Date/核实人及日期： |  |  |  |  |

| 3.46. （14）Fmoc-L- Orn(Boc)-OH activation/N-Fmoc-N'-Boc-L-鸟氨酸活化 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| CAUTION/注意：DMF/N，N-二甲基甲酰胺：Irritation of eyes, skin and nose. May cause nausea. /刺激眼睛、皮肤和鼻子。可能引起恶心。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |
| 3.46.6 Rinse/淋洗 Rinse the port of Activation tank (TJ4S-1213-Sy15-R01) with 91.01 kg（Acceptable weighing range：88.28 ~ 93.74 kg）（2 V）DMF. Stir for at least 15 min. 用91.01 kg（可接受称量范围：88.28 ~ 93.74 kg）（2 V）N,N-二甲基甲酰胺淋洗激活罐（TJ4S-1213-Sy15-R01）加料口。搅拌至少15 min。 |  |  |  | Times/次数 |  |
|  |  |  |  | 1 |  |
| Rinse with DMF/ N,N-二甲基甲酰胺淋洗 | Performed by&Date/执行人及日期： |  |  |  |  |
| Rinse with DMF/ N,N-二甲基甲酰胺淋洗 | Begin/开始： | End/结束： | Qty/重量： |  |  |
| Transcribed by&Date/转录人及日期： |  |  |  |  |  |
| Stir /搅拌 | Performed by & Date/执行人及日期： |  |  |  |  |
| Stir for at least 15 min /搅拌至少15 min |  | Finish time/完成时间： |  |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |
| CAUTION/注意：DIC is a key EHS concern chemical, highly toxic, skin sensitizing, respiratory sensitizing. /N,N’二异丙基碳二亚胺为EHS重点关注化学品，高毒，皮肤致敏，呼吸致敏。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |
| 3.46.7 DIC addition /加N,N'-二异丙基碳二亚胺 Add DIC(2.20 eq.) into the activation tank (TJ4S-1213-Sy15-R01) at reference time of 10-30 min at 【20±5 ℃ (target: 20 ℃) 】. The addition rate depends on temperature control. 控温【20±5 ℃ (目标20 ℃) 】，向激活罐（TJ4S-1213-Sy15-R01）中滴加 kg N,N'-二异丙基碳二亚胺（Acceptable weighing range/可接受称量范围： kg ~ kg）(2.00 eq.)（滴加参考时间10-30 min，以控温为主）。 Pump number/泵编号： Scale ID. No./秤编号： Material charging pipe number/物料管路编号： |  |  |  |  | Times/次数 |
|  |  |  |  |  | 1 |
| 3.46. （14）Fmoc-L- Orn(Boc)-OH activation/N-Fmoc-N'-Boc-L-鸟氨酸活化 |  |  |  |  |  |
| Over-temperature warning value/超温预警设置数值： Lower limit/低限:17 oC ，High limit/高限: 23 oC |  |  |  |  |  |
| Before material charging, confirm the material is within the retest date/ the expiry date. /加物料前，执行物料在复检日期/有效期之内。 Performed by & Date/执行人及日期： |  |  |  |  |  |
| Before drummed liquid material charging, visual check there is no other extraneous matter in material. /加桶装液体物料前，应目视执行桶内无异物。 Visual Examination of Foreign Matters for Drum Solvent/桶装溶剂异物目视检查: 1. Place the solvent drum in a well-lit area (or use a portable light source) to ensure adequate visibility of the liquid contents./ 将溶剂桶置于光线充足区域（或使用便携式光源），确保液体内容物可见性良好 2. Visually inspect the solvent through the drum bung opening or transparent sight glass for any visible foreign matters, including but not limited to particulates, fibers, sediments, or unusual turbidity./ 通过桶口或透明视镜目视检查溶剂中是否有可见异物，包括但不限于颗粒、纤维、沉淀物或异常浑浊。 3. Confirm that the solvent is clear and free of visible foreign matters before charging. If any foreign matter is observed, do not use the drum and follow the applicable handling procedure for subsequent disposition./ 投料前确认溶剂澄清且无可见异物。如发现任何异物，不得使用该桶，并依据处理流程进行后续处置。 Checked by & Date/核实人及日期： |  |  |  |  |  |

| 3.46. （14）Fmoc-L- Orn(Boc)-OH activation/N-Fmoc-N'-Boc-L-鸟氨酸活化 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Confirm liquid charging pipeline is released. /确认加料管路经过放行。 |  |  |  |
| Checked by & Date/核实人及日期： |  |  |  |
| Charging of drummed solvent: verify that the pipeline connections remain consistent before and after material feeding, then record the weighing reading 桶装溶剂溶液称量时，确认投料前后管路连接一致，读取称量示数。 Performed by & Date/执行人及日期： Checked by & Date/核实人及日期： |  |  |  |  |
| DIC /N,N’-二异丙基碳二亚胺：Control Number/内部控制号：YL003564 |  |  |  |  |
| Internal temperature/内部温度 |  | High values/最高值： | Low values/最低值： |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |
| If the target net weight≥the planned net weight, perform full drum charge; otherwise, perform partial (Non-full drum) charge. /当目标加料量≥计划加入净重，勾选进行整桶加料；否则勾选进行零头加料。 Note: Target net weight to be charged = Total target net weight – Total net weight charged. Expected tare weight of drum after charge = Gross weight of drum that to be charged - Target net weight to be charged. 还需加入净重目标值= 加料步骤目标总净重-已加入净重总量 加料完成后预计桶皮重=待加料桶毛重-还需加入净重目标值 |  |  |  |  |  |

| 3.46. （14）Fmoc-L- Orn(Boc)-OH activation/芴N-Fmoc-N'-Boc-L-鸟氨酸活化 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Addition type加料式 | Lot number/批号 | Pre-charging qty calculation/投料前投料量计算 |  |  |  |  |  | Actual Charging Qty/实际投料量 |  |  |  |  |  |
|  |  | Total qty charged/已加入净重总量 |  | Target qty to be charged/还需加入净重目标值 | Gross qty of drum that to be charged/待加料桶毛重 | Expected net weight of drum after charge /加料完成后预计桶净重 | Expected tare weight of drum after charge/加料完成后预计桶皮重 | Before charging/ 投料前 |  | After charging/ 投料后 | Net weight/ 净重 | Performed by & Date/ 执行人及日期 | Checked by & Date/ 核实人及日期 |
| Full drum整桶 Non-full drum /非整桶 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Full drum整桶 Non-full drum /非整桶 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Total/合计 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DIC Addition/加N,N'-二异丙基碳二亚胺 |  |  | Begin/开始： |  |  |  |  |  | End/结束： |  |  |  |  |
| Transcribed by & Date/转录人及日期： |  |  |  |  |  |  |  |  |  |  |  |  |  |

| 3.46. （14）Fmoc-L- Orn(Boc)-OH activation/N-Fmoc-N'-Boc-L-鸟氨酸活化 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 3.46.8 Fmoc-L- Orn(Boc)-OH activation/N-Fmoc-N'-Boc-L-鸟氨酸活化 At 20±5 ℃ (target: 20 ℃). Continue agitation of activation tank to 95±10 rpm (target: 95 rpm). Maintain stirring for 20±5 min (target: 20 min) after addition, then stop stirring. 滴毕，控温20±5 ℃(目标温度：20 ℃)，维持激活罐搅拌转速至95±10 rpm (目标转速: 95 rpm)，继续搅拌20±5 min(目标时间：20 min) ，停止搅拌。 |  |  |  |  |  |  |
| Stir /搅拌 |  | Performed by & Date/执行人及日期： |  |  |  |  |
| Stir /搅拌 |  | End/结束： |  |  |  |  |
|  |  | The start time is documented in Ins3.46.1 /开始时间记录于3.46.1 |  |  |  |  |
| Transcribed by & Date /转录人及日期: |  |  |  |  |  |  |
| Internal temperature/内部温度 |  | High values/最高值： |  | Low values/最低值： |  |  |
| Transcribed by & Date /转录人及日期: |  |  |  |  |  |  |
| - （14）Fmoc-L- Orn(Boc)-OH coupling/N-Fmoc-N'-Boc-L-鸟氨酸偶联 |  |  |  |  |  |  |
| 3.47.1 Transfer/转移 Transfer the mixture in activation tank (TJ4S-1213-Sy15-R01) into synthesis reactor (TJ4S-1213-Sy15). 将激活罐（TJ4S-1213-Sy15-R01）中体系转移至合成釜（TJ4S-1213-Sy15）中。 |  |  |  |  |  | Times/次数 |
|  |  |  |  |  |  | 1 |
| ★Note:The coupling temperature is a CPP, and the coupling IPC hold temperature is a CPP./ 备注：偶联温度是CPP，偶联IPC维持温度是关键工艺参数。 The operator shall confirm that the reaction mixture temperature is within the Normal Operating Range (NOR) /操作人员须执行反应体系温度处于正常操作范围（NOR）内 Checked by & Date/核实人及日期： ★Lower limit/低限: 13 oC ，High limit/高限: 37 oC Over-temperature warning value/超温预警设置数值： Lower limit/低限: 21 oC ，High limit/高限: 28 oC |  |  |  |  |  |  |
| Transfer/转移 |  |  | Performed by&Date/执行人及日期： |  |  |  |
|  |  |  | Checked by & Date/核实人及日期： |  |  |  |
|  |  |  | Begin/开始： |  | End/结束： |  |

| 3.47. （14）Fmoc-L- Orn(Boc)-OH coupling/N-Fmoc-N'-Boc-L-鸟氨酸偶联 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CAUTION/注意：DMF/N，N-二甲基甲酰胺：Irritation of eyes, skin and nose. May cause nausea. /刺激眼睛、皮肤和鼻子。可能引起恶心。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |  |  |
| 3.47.2 DMF addition /加N,N-二甲基甲酰胺 Add 45.50 kg (Acceptable weighing range: 43.50 ~ 47.50 kg) (1 V) DMF by spray balls into activation tank (TJ4S-1213-Sy15-R01) to rinse the wall. 通过淋洗球向激活罐（TJ4S-1213-Sy15-R01）中加入45.50 kg（可接受称量范围：43.50 ~ 47.50 kg） (1 V) N,N-二甲基甲酰胺淋洗罐壁。 |  |  |  |  |  | Times/次数 |  |
|  |  |  |  |  |  | 1 |  |
| Rinse with DMF/ N,N-二甲基甲酰胺淋洗 |  |  | Performed by&Date/执行人及日期： |  |  |  |  |
| Rinse with DMF/ N,N-二甲基甲酰胺淋洗 |  |  | Begin/开始： | End/结束： | Qty/重量： |  |  |
| Transcribed by&Date/转录人及日期： |  |  |  |  |  |  |  |
| 3.47.3 Transfer /转移 Turn on the nitrogen of Synthesis reactor (TJ4S-1213-Sy15) to 5±2 m³/h (target: 5 m³/h). Bubble with nitrogen for 5±2 min (target: 5 min). Start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm), then start the agitator. Transfer the mixture in activation tank (TJ4S-1213-Sy15-R01) into synthesis reactor (TJ4S-1213-Sy15). 合成釜（TJ4S-1213-Sy15）先开启下口氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min (目标：5 min)后，开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm)，将激活罐（TJ4S-1213-Sy15-R01）中体系转移至合成釜（TJ4S-1213-Sy15）中。 |  |  |  |  |  | Times/次数 |  |
|  |  |  |  |  |  | 1 |  |
| Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h). Bubble with nitrogen for 5±2 min (target :5 min) 开启合成釜下口鼓氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min (目标时间：5 min) |  |  |  |  |  |  |  |
| Bubble/鼓氮气 |  |  | Performed by & Date/执行人及日期： |  |  |  |  |
| Bubble/鼓氮气 |  |  | Begin/开始： |  |  |  |  |
| Transcribed by & Date /转录人及日期： |  |  |  |  |  |  |  |

| 3.47. （14）Fmoc-L- Orn(Boc)-OH coupling/N-Fmoc-N'-Boc-L-鸟氨酸偶联 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm). /开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm) |  |  |  |  |  |
| Stir /搅拌 |  |  | Performed by & Date/执行人及日期： |  |  |
| Stir /搅拌 |  |  | Begin/开始： |  |  |
| Transcribed by&Date/转录人及日期： |  |  |  |  |  |
| Transfer/转移 |  | Performed by&Date/执行人及日期： |  |  |  |
|  |  | Checked by & Date/核实人及日期： |  |  |  |
|  |  | Begin/开始： |  | End/结束： |  |
| 3.47.4 Adjust synthesis reactor temperature /合成釜调温 Adjust synthesis reactor (TJ4S-1213-Sy15) mixture to 【25±5 ℃ (target: 25 ℃) 】. /将合成釜（TJ4S-1213-Sy15）体系调温至【25±5 ℃(目标：25 ℃) 】. |  |  |  |  | Times/次数 |
|  |  |  |  |  | 1 |
| ★Note:The coupling temperature is a CPP, and the coupling IPC hold temperature is a CPP./ 备注：偶联温度是CPP，偶联IPC维持温度是关键工艺参数。 The operator shall confirm that the reaction mixture temperature is within the Normal Operating Range (NOR) /操作人员须执行反应体系温度处于正常操作范围（NOR）内 ★Lower limit/低限: 13 oC ，High limit/高限: 37 oC Over-temperature warning value/超温预警设置数值： Lower limit/低限: 22 °C ，High limit/高限: 28 °C |  |  |  |  |  |
| Adjust/调温 |  | Performed by&Date/执行人及日期： |  |  |  |
| Adjust/调温 |  | Begin/开始： |  | End/结束： |  |
| Transcribed by&Date/转录人及日期： |  |  |  |  |  |

| 3.47. （14）Fmoc-L- Orn(Boc)-OH coupling/N-Fmoc-N'-Boc-L-鸟氨酸偶联 |  |
| --- | --- |
| 3.47.5 Fmoc-L- Orn(Boc)-OH coupling/N-Fmoc-N'-Boc-L-鸟氨酸偶联 Start timing when temperature reaches 20 ℃.The mixture is allowed to react at 25±5 ℃ (target: 25 ℃). After stirring for 3 h±15 min (target: 3 h), sample the mixture every 0.5~1 h for ninhydrin test analysis until two consecutive samples test as negative (Yellow to Light pink Report Negative= Complete coupling; Blue to Purple Positive= Incomplete coupling). And the mixture should be stirred for coupling time (4 h≤Coupling Time≤10 h). Stop stirring, turn off the nitrogen. Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point. 控温【25±5 oC（目标温度25 oC，20 oC开始计时）】，控温25±5 ℃(目标：25 ℃)，反应3 h±15 min（目标：3h）取样进行茚三酮检测，每隔0.5-1.0 h取样一次，直至连续两次取样结果为阴性（黄色至浅粉色，报告阴性=偶联完全；蓝色至紫色，阳性=偶联不完全）后,同时反应时间满足（4h≤反应时间≤10h）后，停止搅拌，关闭氮气。通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 Note: If resin adherence is found at other times during the reaction, rinse the reactor with 0.50~5.00 kg DMF each time. /备注：偶联过程中其它时间发现贴壁，每次使用0.50~5.00 kg N,N-二甲基甲酰胺淋洗，记录见附加页 | Times/次数 |
|  | 1 |

| 3.47. （14）Fmoc-L-Orn(Boc)-OH coupling/N-Fmoc-N'-Boc-L-鸟氨酸偶联 |  |  |
| --- | --- | --- |
| The PAR range of reaction time: (14) Fmoc-L-Orn(Boc)-OH coupling time is 4 –10 h.（14）N-Fmoc-N'-Boc-L-鸟氨酸偶联反应时间的工艺可接受范围：4～10 h Note：Definition of Coupling Time/备注：偶联反应开始时间定义 - Coupling starts at the time when stirring is initiated. /以搅拌启动时刻作为偶联反应的起始时间。 - Coupling Time ends upon completion of the specified coupling hold‑time or upon IPC sampling for coupling reaction completion check. 偶联反应时间的结束节点为：完成规定的偶联保温时长，或取样开展偶联反应终点 IPC 检测的时刻。 ★Note:The total quantity of DMF is a CPP./备注：DMF总用量是关键工艺参数。 ★Note:The coupling temperature is a CPP, and the coupling IPC hold temperature is a CPP./ 备注：偶联温度是CPP，偶联IPC维持温度是关键工艺参数。 The operator shall confirm that the reaction mixture temperature is within the Normal Operating Range (NOR) /操作人员须执行反应体系温度处于正常操作范围（NOR）内 Checked by & Date/核实人及日期： ★Lower limit/低限: 13 oC ，High limit/高限: 37 oC Over-temperature warning value/超温预警设置数值： Lower limit/低限: 22 oC ，High limit/高限: 28 oC |  |  |
| React/反应 | Performed by&Date/执行人及日期： |  |
| React/反应 | Begin/开始： | End/结束： |
| Transcribed by&Date/转录人及日期： |  |  |
| Total reaction/ (14) Fmoc-L-Orn(Boc)-OH coupling time = End of reaction - Start of reaction/ 总反应/（14）N-Fmoc-N'-Boc-L-鸟氨酸偶联时间计算=反应结束时间_____ - 反应开始时间_______= _ 。 Recorded by& Date/记录人及日期：___________________ Checked by & Date/核实人及日期____________________ Process chemist confirms the total reaction/ (14) Fmoc-L-Orn(Boc)-OH coupling time did not exceed 4~10 h (Sourced from the holding time study in the PAR study report.) 工艺技术人员确认总的反应时间不超过4 ~10 h(来源于 PAR 研究报告中稳定时间研究)。 Checked by & Date/核实人及日期____________________ |  |  |

| 3.47. （14）Fmoc-L- Orn(Boc)-OH coupling/N-Fmoc-N'-Boc-L-鸟氨酸偶联 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Internal temperature/内部温度 |  | High values/最高值： |  |  | Low values/最低值： |  |  |  |
| Transcribed by&Date/转录人及日期： |  |  |  |  |  |  |  |  |
| Standard sampling method/标准取样方法 |  |  |  |  |  |  |  |
| Take 5-10 ml mixture and perform testing in accordance with the ninhydrin test record. During sampling, should wear nitrile gloves and 3M 6000 series Full-face piece respirator, 6006CN cartridges. 通过在线取样器取样5~10ml按照茚三酮检测记录进行检测。取样过程中，应该戴丁腈手套、3M 6000系列全面罩, 6006CN滤毒盒。 |  |  |  |  |  |  |  |
| Sampling number/ 取样编号 | Sampling time/取样时间 |  | Sampled by & Date/取样人及日期 | Negative or Positive/阴性或阳性 |  | Result filled by &Date/结果填写人及日期 | Checked by & Date/核实人及日期 |
| CPo140020- -01- -3.47.5-01 |  |  |  | Negative /阴性 Positive /阳性 |  |  |  |
| CPo140020- -01- -3.47.5-02 |  |  |  | Negative /阴性 Positive /阳性 |  |  |  |
| CPo140020- -01- -3.47.5-03 |  |  |  | Negative /阴性 Positive /阳性 |  |  |  |
| CPo140020- -01- -3.47.5-04 |  |  |  | Negative /阴性 Positive /阳性 |  |  |  |
| Confirm the end point of the reaction. /执行反应终点。 Performed by & Date/执行人及日期： Checked by & Date/核实人及日期： |  |  |  |  |  |  |  |
| Stop the stirrer and turn off the nitrogen. Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point./ 停止搅拌，关闭氮气，通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  |  |  |  |  |  |
| Stir /搅拌 |  | Performed by & Date/执行人及日期： |  |  |  |  |  |
| Stir /搅拌 |  | End/结束： |  |  |  |  |  |
| Transcribed by & Date/转录人及日期： |  |  |  |  |  |  |  |
| Bubble/鼓氮气 |  | Performed by & Date/执行人及日期： |  |  |  |  |  |
| Bubble/鼓氮气 |  | End/结束： |  |  |  |  |  |
| Transcribed by & Date/转录人及日期： |  |  |  |  |  |  |  |

| 3.47. （14）Fmoc-L- Orn(Boc)-OH coupling/N-Fmoc-N'-Boc-L-鸟氨酸偶联 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Internal temperature/内部温度 |  | High values/最高值： |  | Low values/最低值： |  |
| Transcribed by & Date/转录人及日期： |  |  |  |  |  |
| Filter/过滤 | Performed by&Date/执行人及日期： |  |  |  |  |
|  | Checked by & Date/核实人及日期： |  |  |  |  |
|  | Begin/开始： |  | End/结束： |  |  |
| ★After coupling is completed, the operator shall confirm that all Critical Process Parameters (CPPs) are within their respective Normal Operating Ranges (NOR)/ 偶联结束后，操作人员须确认所有关键工艺参数（CPP）均处于各自的正常操作范围（NOR）内 Checked by & Date/核实人及日期： |  |  |  |  |  |
| CAUTION/注意：IPAC/乙酸异丙酯： Vapors irritate eyes and respiratory tract; high concentrations can be anesthetic. Liquid irritates eyes but causes no serious injury; may cause dermatitis; no serious effects if swallowed. /蒸汽会刺激眼睛和呼吸道；高浓度的蒸汽可能是麻醉剂。液体刺激眼睛，但不会造成严重伤害；可能会引起皮炎；吞食不会造成严重影响。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |  |  |
| 3.47.6 Synthesis reactor wash 1/合成釜洗涤1 Add 419.04 kg (Acceptable weighing range：406.47 ~ 431.61 kg) (10 V) isopropyl acetate into synthesis reactor (TJ4S-1213-Sy15). Bubble with nitrogen for 5±2 min (target 5 min), then start the agitator. After stirring for 10±2 min (target 10 min), stop stirring, turn off the nitrogen. Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point. 向合成釜（TJ4S-1213-Sy15）中加入419.04 kg（可接受称量范围：406.47 ~ 431.61 kg）（10 V）乙酸异丙酯，开启下口氮气，鼓氮气5±2 min (目标：5 min)后开启搅拌，搅拌10±2 min (目标：10 min)后，停止搅拌，关闭氮气。通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  |  |  | Times/次数 |  |
|  |  |  |  |  | 5 |  |

| 3.47. （14）Fmoc-L- Orn(Boc)-OH coupling/N-Fmoc-N'-Boc-L-鸟氨酸偶联 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Turn on the nitrogen of synthesis reactor to 5±2 m³/h (target: 5 m³/h), Bubble with nitrogen for 5±2 min (target:5 min), Start the agitator of synthesis reactor to 20±10 rpm (target: 20 rpm).After stirring for 10±2 min (target: 10 min), stop stirring, turn off the nitrogen./ 开启合成釜下口鼓氮气，氮气流速开至5±2 m³/h (目标流速: 5 m³/h)，鼓氮气5±2 min（目标时间：5 min）后开启固相合成釜搅拌转速至20±10 rpm (目标转速: 20 rpm)，搅拌10±2 min（目标时间：10 min）后，停止搅拌，关闭氮气。 |  |  |  |  |  |  |
| Operation name/操作名称 | Time point/时间节点 | Wash /洗涤1 | Wash /洗涤2 | Wash /洗涤3 | Wash /洗涤4 | Wash /洗涤5 |
| IPAC addition/加乙酸异丙酯 | Performed by&Date/执行人及日期 |  |  |  |  |  |
| IPAC addition/加乙酸异丙酯 | Begin/开始 |  |  |  |  |  |
|  | End/结束 |  |  |  |  |  |
|  | Qty/重量 |  |  |  |  |  |
|  | Transcribed by&Date/转录人及日期 |  |  |  |  |  |
| Bubble/鼓氮气 | Performed by & Date/执行人及日期： |  |  |  |  |  |
| Bubble/鼓氮气 | Begin/开始 |  |  |  |  |  |
|  | End/结束 |  |  |  |  |  |
|  | Transcribed by&Date/转录人及日期 |  |  |  |  |  |

| 3.47. （14）Fmoc-L- Orn(Boc)-OH coupling/N-Fmoc-N'-Boc-L-鸟氨酸偶联 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Operation name/操作名称 |  | Time point/时间节点 | Wash /洗涤1 | Wash /洗涤2 | Wash /洗涤3 | Wash /洗涤4 | Wash /洗涤5 |
| Stir /搅拌 |  | Performed by & Date/执行人及日期： |  |  |  |  |  |
| Stir /搅拌 |  | Begin/开始 |  |  |  |  |  |
|  |  | End/结束 |  |  |  |  |  |
|  |  | Transcribed by&Date/转录人及日期 |  |  |  |  |  |
| Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point/通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  |  |  |  |  |  |
| Filter/过滤 |  | Performed by & Date/执行人及日期： |  |  |  |  |  |
|  |  | Checked by & Date/核实人及日期： |  |  |  |  |  |
|  |  | Begin/开始 |  |  |  |  |  |
|  |  | End/结束 |  |  |  |  |  |

| 3.47. （14）Fmoc-L- Orn(Boc)-OH coupling/N-Fmoc-N'-Boc-L-鸟氨酸偶联 |  |  |
| --- | --- | --- |
| CAUTION/注意：Tol/甲苯：Vapors irritate eyes and upper respiratory tract; cause dizziness, headache, anesthesia, respiratory arrest. Liquid irritates eyes and causes drying of skin. If aspirated, causes coughing, gagging, distress, and rapidly developing pulmonary edema. If ingested causes vomiting, griping, diarrhea, depressed respiration. 蒸气刺激眼睛和上呼吸道；引起头晕、头痛、麻醉、呼吸停止。液体刺激眼睛，导致皮肤干燥。如果抽吸，会导致咳嗽、咯咯声、痛苦和迅速发展的肺水肿。如果摄入会引起呕吐、抓痒、腹泻、呼吸抑制。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |
| 3.47.7 Toluene addition/加甲苯 Add 418.56 kg (Acceptable weighing range: 406.00 ~ 431.12 kg) (10 V) toluene into synthesis reactor (TJ4S-1213-Sy15) by with insert tube and diaphragm pump. Bubble with nitrogen for 5±2 min (target 5 min), then start the agitator. After stirring for 25±5 min (target: 25 min), stop stirring, turn off the nitrogen. Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point. 通过插底管，隔膜泵，向合成釜（TJ4S-1213-Sy15）中加入418.56 kg甲苯（Acceptable weighing range/可接受称量范围： 406.00 ~431.12 kg）（10V），开启下口鼓氮气5±2 min (目标：5 min)后，开启搅拌，搅拌25±5 min（目标： 25 min）后，停止搅拌，关闭氮气。通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  | Times/次数 |
|  |  | 1 |
| Before material charging, confirm the material is within the retest date/ the expiry date. /加物料前，执行物料在复检日期/有效期之内。 Performed by & Date/执行人及日期： |  |  |
| Before drummed liquid material charging, visual check there is no other extraneous matter in material. /加桶装液体物料前，应目视执行桶内无异物。 Visual Examination of Foreign Matters for Drum Solvent/桶装溶剂异物目视检查: 1. Place the solvent drum in a well-lit area (or use a portable light source) to ensure adequate visibility of the liquid contents./ 将溶剂桶置于光线充足区域（或使用便携式光源），确保液体内容物可见性良好 2. Visually inspect the solvent through the drum bung opening or transparent sight glass for any visible foreign matters, including but not limited to particulates, fibers, sediments, or unusual turbidity./ 通过桶口或透明视镜目视检查溶剂中是否有可见异物，包括但不限于颗粒、纤维、沉淀物或异常浑浊。 3. Confirm that the solvent is clear and free of visible foreign matters before charging. If any foreign matter is observed, do not use the drum and follow the applicable handling procedure for subsequent disposition./ 投料前确认溶剂澄清且无可见异物。如发现任何异物，不得使用该桶，并依据处理流程进行后续处置。 Checked by & Date/核实人及日期： |  |  |

| 3.47. （14）Fmoc-L- Orn(Boc)-OH coupling/N-Fmoc-N'-Boc-L-鸟氨酸偶联 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Confirm liquid charging pipeline is released. /执行加料管路经过放行。 |  |  |  |
| Performed by & Date/执行人及日期： |  |  |  |
| Pump number/泵编号： Scale ID. No./秤编号： Material charging pipe number/物料管路编号： |  |  |  |
| Toluene/甲苯：Control Number/内部控制号：YL000026 |  |  |  |
| Internal temperature/内部温度 |  | High values/最高值： | Low values/最低值： |
| Transcribed by & Date /转录人及日期： |  |  |  |
| If the target net weight≥the planned net weight, perform full drum charge; otherwise, perform partial (Non-full drum) charge. /当目标加料量≥计划加入净重，勾选进行整桶加料；否则勾选进行零头加料。 Note: Target net weight to be charged = Total target net weight – Total net weight charged. Expected tare weight of drum after charge = Gross weight of drum that to be charged - Target net weight to be charged. 还需加入净重目标值= 加料步骤目标总净重-已加入净重总量 加料完成后预计桶皮重=待加料桶毛重-还需加入净重目标值 |  |  |  |  |

| 3.47. （14）Fmoc-L- Orn(Boc)-OH coupling/N-Fmoc-N'-Boc-L-鸟氨酸偶联 |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Addition type加料式 | Lot number/批号 | Pre-charging qty calculation/投料前投料量计算 |  |  |  |  | Actual Charging Qty/实际投料量 |  |  |  |  |
|  |  | Total qty charged/已加入净重总量 | Target qty to be charged/还需加入净重目标值 | Gross qty of drum that to be charged/待加料桶毛重 | Expected net weight of drum after charge /加料完成后预计桶净重 | Expected tare weight of drum after charge/加料完成后预计桶皮重 | Before charging/ 投料前 | After charging/ 投料后 | Net weight/ 净重 | Performed by & Date/ 执行人及日期 | Checked by & Date/ 核实人及日期 |
| Full drum整桶 Non-full drum /非整桶 |  |  |  |  |  |  |  |  |  |  |  |
| Full drum整桶 Non-full drum /非整桶 |  |  |  |  |  |  |  |  |  |  |  |
| Full drum整桶 Non-full drum /非整桶 |  |  |  |  |  |  |  |  |  |  |  |

| 3.47. （14）Fmoc-L- Orn(Boc)-OH coupling/N-Fmoc-N'-Boc-L-鸟氨酸偶联 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Addition type加料式 | Lot number/批号 | Pre-charging qty calculation/投料前投料量计算 |  |  |  |  |  | Actual Charging Qty/实际投料量 |  |  |  |  |  |
|  |  | Total qty charged/已加入净重总量 |  | Target qty to be charged/还需加入净重目标值 | Gross qty of drum that to be charged/待加料桶毛重 | Expected net weight of drum after charge /加料完成后预计桶净重 | Expected tare weight of drum after charge/加料完成后预计桶皮重 | Before charging/ 投料前 |  | After charging/ 投料后 | Net weight/ 净重 | Performed by & Date/ 执行人及日期 | Checked by & Date/ 核实人及日期 |
| Full drum整桶 Non-full drum /非整桶 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Full drum整桶 Non-full drum /非整桶 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Total/合计 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Toluene Addition/加甲苯 |  |  | Begin/开始： |  |  |  |  |  | End/结束： |  |  |  |  |
| Transcribed by & Date/转录人及日期： |  |  |  |  |  |  |  |  |  |  |  |  |  |

| 3.47. （14）Fmoc-L- Orn(Boc)-OH coupling/N-Fmoc-N'-Boc-L-鸟氨酸偶联 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Bubble/鼓氮气 | Performed by&Date/执行人及日期： |  |  |  |
| Bubble/鼓氮气 | Begin/开始： | End/结束： |  |  |
| Transcribed by&Date/转录人及日期： |  |  |  |  |
| Stir /搅拌 | Performed by&Date/执行人及日期： |  |  |  |
| Stir /搅拌 | Begin/开始： | End/结束： |  |  |
| Transcribed by&Date/转录人及日期： |  |  |  |  |
| Stop the stirrer and turn off the nitrogen. Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point./ 停止搅拌，关闭氮气，通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  |  |  |
| Filter/过滤 | Performed by&Date/执行人及日期： |  |  |  |
|  | Checked by & Date/核实人及日期： |  |  |  |
|  | Begin/开始： | End/结束： |  |  |
| Transcribed by&Date/转录人及日期： |  |  |  |  |
| CAUTION/注意MTBE/甲基叔丁基醚： Inhalation: May cause dizziness or suffocation. Contact may irritate or burn eyes or skin. May be harmful if swallowed. /吸入:可引起头晕或窒息。接触可能刺激或灼伤眼睛或皮肤。吞食可能有害。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |
| 3.47.8 Shirnk with MTBE/甲基叔丁基醚收缩 Add 355.68 kg (Acceptable weighing range: 345.01 ~ 366.35 kg) (10 V) MTBE into synthesis reactor (TJ4S-1213-Sy15). Bubble with nitrogen for 5±2 min (target 5 min), then start the agitator. After stirring for 15±5 min (target: 15 min), stop stirring, turn off the nitrogen. Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point. 向合成釜（TJ4S-1213-Sy15）中加入355.68 kg甲基叔丁基醚（Acceptable weighing range/可接受称量范围： 345.01 ~ 366.35 kg）（10 V），开启下口氮气5±2 min (目标：5 min)，开启搅拌15±5 min（目标：15 min）后，停止搅拌，关闭氮气。通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  |  | Times/次数 |
|  |  |  |  | 2 |

| 3.47. （14）Fmoc-L- Orn(Boc)-OH coupling/N-Fmoc-N'-Boc-L-鸟氨酸偶联 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Operation name/操作名称 |  |  | Time point/时间节点 | Wash /洗涤1 | Wash /洗涤2 |  |
| MTBE addition/ 加甲基叔丁基醚 |  |  | Performed by&Date/执行人及日期 |  |  |  |
| MTBE addition/ 加甲基叔丁基醚 |  |  | Begin/开始 |  |  |  |
|  |  |  | End/结束 |  |  |  |
|  |  |  | Qty/重量 |  |  |  |
|  |  |  | Transcribed by&Date/转录人及日期 |  |  |  |
| Bubble/鼓氮气 |  |  | Performed by & Date/执行人及日期： |  |  |  |
|  |  |  | Begin/开始： |  |  |  |
|  |  |  | End/结束： |  |  |  |
|  |  |  | Transcribed by&Date/转录人及日期 |  |  |  |
| Stir /搅拌 |  |  | Performed by&Date/执行人及日期： |  |  |  |
|  |  |  | Begin/开始 |  |  |  |
|  |  |  | End/结束 |  |  |  |
|  |  |  | Transcribed by&Date/转录人及日期： |  |  |  |

| 3.47. （14）Fmoc-L- Orn(Boc)-OH coupling/N-Fmoc-N'-Boc-L-鸟氨酸偶联 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Operation name/操作名称 | Time point/时间节点 | Wash /洗涤1 | Wash /洗涤2 |  |
| Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point/通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  |  |  |
| Filter/过滤 | Performed by&Date/执行人及日期： |  |  |  |
|  | Checked by & Date/核实人及日期： |  |  |  |
|  | Begin/开始： |  |  |  |
|  | End/结束： |  |  |  |
| CAUTION/注意:Tol/甲苯：Vapors irritate eyes and upper respiratory tract; cause dizziness, headache, anesthesia, respiratory arrest. Liquid irritates eyes and causes drying of skin. If aspirated, causes coughing, gagging, distress, and rapidly developing pulmonary edema. If ingested causes vomiting, griping, diarrhea, depressed respiration. 蒸气刺激眼睛和上呼吸道；引起头晕、头痛、麻醉、呼吸停止。液体刺激眼睛，导致皮肤干燥。如果抽吸，会导致咳嗽、咯咯声、痛苦和迅速发展的肺水肿。如果摄入会引起呕吐、抓痒、腹泻、呼吸抑制。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |  |
| 3.47.9 Toluene addition/加甲苯 Add 418.56 kg toluene into synthesis reactor (TJ4S-1213-Sy15) by insert tube and diaphragm pump. Bubble with nitrogen for 5±2 min (target 5 min), then start the agitator. After stirring for 25±5 min, (target: 25 min) stop stirring, turn off the nitrogen. Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point. 通过插底管，隔膜泵，向合成釜（TJ4S-1213-Sy15）中加入418.56 kg甲苯（Acceptable weighing range/可接受称量范围： 406.00 ~ 431.12 kg）（10 V），开启下口鼓氮气5±2 min (目标：5 min)后，开启搅拌，搅拌25±5 min（目标：25 min）后，停止搅拌，关闭氮气。通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  |  | Times/次数 |
|  |  |  |  | 1 |

| 3.47. （14）Fmoc-L- Orn(Boc)-OH coupling/N-Fmoc-N'-Boc-L-鸟氨酸偶联 |  |  |
| --- | --- | --- |
| Before material charging, confirm the material is within the retest date/ the expiry date. /加物料前，执行物料在复检日期/有效期之内。 Performed by & Date/执行人及日期： |  |  |
| Before drummed liquid material charging, visual check there is no other extraneous matter in material. /加桶装液体物料前，应目视执行桶内无异物。 Visual Examination of Foreign Matters for Drum Solvent/桶装溶剂异物目视检查: 1. Place the solvent drum in a well-lit area (or use a portable light source) to ensure adequate visibility of the liquid contents./ 将溶剂桶置于光线充足区域（或使用便携式光源），确保液体内容物可见性良好 2. Visually inspect the solvent through the drum bung opening or transparent sight glass for any visible foreign matters, including but not limited to particulates, fibers, sediments, or unusual turbidity./ 通过桶口或透明视镜目视检查溶剂中是否有可见异物，包括但不限于颗粒、纤维、沉淀物或异常浑浊。 3. Confirm that the solvent is clear and free of visible foreign matters before charging. If any foreign matter is observed, do not use the drum and follow the applicable handling procedure for subsequent disposition./ 投料前确认溶剂澄清且无可见异物。如发现任何异物，不得使用该桶，并依据处理流程进行后续处置。 Checked by & Date/核实人及日期： |  |  |
| Confirm liquid charging pipeline is released. /执行加料管路经过放行。 |  |  |
| Performed by & Date/执行人及日期： |  |  |
| Pump number/泵编号： Scale ID. No./秤编号： Material charging pipe number/物料管路编号： |  |  |
| Toluene/甲苯：Control Number/内部控制号：YL000026 |  |  |
| Internal temperature/内部温度 | High values/最高值： | Low values/最低值： |
| Transcribed by & Date /转录人及日期： |  |  |
| If the target net weight≥the planned net weight, perform full drum charge; otherwise, perform partial (Non-full drum) charge. /当目标加料量≥计划加入净重，勾选进行整桶加料；否则勾选进行零头加料。 Note: Target net weight to be charged = Total target net weight – Total net weight charged. Expected tare weight of drum after charge = Gross weight of drum that to be charged - Target net weight to be charged. 还需加入净重目标值= 加料步骤目标总净重-已加入净重总量 加料完成后预计桶皮重=待加料桶毛重-还需加入净重目标值 |  |  |  |

| 3.47. （14）Fmoc-L- Orn(Boc)-OH coupling/N-Fmoc-N'-Boc-L-鸟氨酸偶联 |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Addition type加料式 | Lot number/批号 | Pre-charging qty calculation/投料前投料量计算 |  |  |  |  | Actual Charging Qty/实际投料量 |  |  |  |  |
|  |  | Total qty charged/已加入净重总量 | Target qty to be charged/还需加入净重目标值 | Gross qty of drum that to be charged/待加料桶毛重 | Expected net weight of drum after charge /加料完成后预计桶净重 | Expected tare weight of drum after charge/加料完成后预计桶皮重 | Before charging/ 投料前 | After charging/ 投料后 | Net weight/ 净重 | Performed by & Date/ 执行人及日期 | Checked by & Date/ 核实人及日期 |
| Full drum整桶 Non-full drum /非整桶 |  |  |  |  |  |  |  |  |  |  |  |
| Full drum整桶 Non-full drum /非整桶 |  |  |  |  |  |  |  |  |  |  |  |
| Full drum整桶 Non-full drum /非整桶 |  |  |  |  |  |  |  |  |  |  |  |

| 3.47. （14）Fmoc-L- Orn(Boc)-OH coupling/N-Fmoc-N'-Boc-L-鸟氨酸偶联 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Addition type加料式 | Lot number/批号 | Pre-charging qty calculation/投料前投料量计算 |  |  |  |  |  | Actual Charging Qty/实际投料量 |  |  |  |  |  |
|  |  | Total qty charged/已加入净重总量 |  | Target qty to be charged/还需加入净重目标值 | Gross qty of drum that to be charged/待加料桶毛重 | Expected net weight of drum after charge /加料完成后预计桶净重 | Expected tare weight of drum after charge/加料完成后预计桶皮重 | Before charging/ 投料前 |  | After charging/ 投料后 | Net weight/ 净重 | Performed by & Date/ 执行人及日期 | Checked by & Date/ 核实人及日期 |
| Full drum整桶 Non-full drum /非整桶 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Full drum整桶 Non-full drum /非整桶 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Total/合计 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Toluene Addition/加甲苯 |  |  | Begin/开始： |  |  |  |  |  | End/结束： |  |  |  |  |
| Transcribed by & Date/转录人及日期： |  |  |  |  |  |  |  |  |  |  |  |  |  |

| 3.47. （14）Fmoc-L- Orn(Boc)-OH coupling/N-Fmoc-N'-Boc-L-鸟氨酸偶联 |  |  |  |
| --- | --- | --- | --- |
| Bubble/鼓氮气 |  | Performed by&Date/执行人及日期： |  |
| Bubble/鼓氮气 |  | Begin/开始： | End/结束： |
| Transcribed by&Date/转录人及日期： |  |  |  |
| Stir /搅拌 |  | Performed by&Date/执行人及日期： |  |
| Stir /搅拌 |  | Begin/开始： | End/结束： |
| Transcribed by&Date/转录人及日期： |  |  |  |
| Stop the stirrer and turn off the nitrogen. Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point./ 停止搅拌，关闭氮气，通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  |  |
| Filter/过滤 |  | Performed by&Date/执行人及日期： |  |
|  |  | Checked by & Date/核实人及日期： |  |
|  |  | Begin/开始： | End/结束： |
| Transcribed by&Date/转录人及日期： |  |  |  |
| CAUTION/注意MTBE/甲基叔丁基醚： Inhalation: May cause dizziness or suffocation. Contact may irritate or burn eyes or skin. May be harmful if swallowed. /吸入:可引起头晕或窒息。接触可能刺激或灼伤眼睛或皮肤。吞食可能有害。 Confirm that the above content has been acknowledged./确认已知悉上述内容。 Signature &Date/签名及日期： |  |  |  |

| 3.47. （14）Fmoc-L- Orn(Boc)-OH coupling/N-Fmoc-N'-Boc-L-鸟氨酸偶联 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3.47.10 Shrink with MTBE /甲基叔丁基醚收缩 Add 355.68 kg (Acceptable weighing range: 345.01 ~ 366.35 kg) (10 V) MTBE into synthesis reactor (TJ4S-1213-Sy15). Bubble with nitrogen for 5±2 min (target 5 min), then start the agitator. After stirring for 15±5 min (target: 15 min), stop stirring, turn off the nitrogen. Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point. 向合成釜（TJ4S-1213-Sy15）中加入355.68 kg甲基叔丁基醚（Acceptable weighing range/可接受称量范围： 345.01 ~ 366.35 kg）（10 V），开启下口氮气5±2 min (目标：5 min)，开启搅拌15±5 min（目标：15 min）后，停止搅拌，关闭氮气。通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  |  |  |  |  | Times/次数 |  |
|  |  |  |  |  |  |  | 1 |  |
| Add MTBE/ 加甲基叔丁基醚 |  |  | Performed by&Date/执行人及日期： |  |  |  |  |  |
|  |  |  | Begin/开始： |  | End/结束： |  | Qty/重量： |  |
| Transcribed by&Date/转录人及日期： |  |  |  |  |  |  |  |  |
| Bubble/鼓氮气 |  |  |  | Performed by&Date/执行人及日期： |  |  |  |  |
| Bubble/鼓氮气 |  |  |  | Begin/开始： |  | End/结束： |  |  |
| Transcribed by&Date/转录人及日期： |  |  |  |  |  |  |  |  |
| Stir /搅拌 |  |  |  | Performed by&Date/执行人及日期： |  |  |  |  |
| Stir /搅拌 |  |  |  | Begin/开始： |  | End/结束： |  |  |
| Transcribed by&Date/转录人及日期： |  |  |  |  |  |  |  |  |
| Stop the stirrer and turn off the nitrogen. Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point./ 停止搅拌，关闭氮气，通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  |  |  |  |  |  |  |
| Filter/过滤 |  |  |  | Performed by&Date/执行人及日期： |  |  |  |  |
|  |  |  |  | Checked by & Date/核实人及日期： |  |  |  |  |
|  |  |  |  | Begin/开始： |  | End/结束： |  |  |
| Transcribed by&Date/转录人及日期： |  |  |  |  |  |  |  |  |

| 3.47. （14）Fmoc-L- Orn(Boc)-OH coupling/N-Fmoc-N'-Boc-L-鸟氨酸偶联 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| MTBE/甲基叔丁基醚： Inhalation: May cause dizziness or suffocation. Contact may irritate or burn eyes or skin. May be harmful if swallowed. /吸入:可引起头晕或窒息。接触可能刺激或灼伤眼睛或皮肤。吞食可能有害。 |  |  |  |  |  |  |  |
| 3.47.11 Shrink with MTBE /甲基叔丁基醚收缩 Add 355.68 kg (Acceptable weighing range: 345.01 ~ 366.35 kg) (10 V) MTBE into synthesis reactor (TJ4S-1213-Sy15). Bubble with nitrogen for 5±2 min (target 5 min), then start the agitator. After stirring for 15±5 min (target: 15 min), stop stirring, turn off the nitrogen. Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point. 向合成釜（TJ4S-1213-Sy15）中加入355.68 kg甲基叔丁基醚（Acceptable weighing range/可接受称量范围： 345.01 ~ 366.35 kg）（10 V），开启下口氮气5±2 min (目标：5 min)，开启搅拌15±5 min（目标时间：15 min）后，停止搅拌，关闭氮气。通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  |  |  |  | Times/次数 |  |
|  |  |  |  |  |  | 1 |  |
| MTBE addition/ 加甲基叔丁基醚 |  | Performed by&Date/执行人及日期： |  |  |  |  |  |
|  |  | Begin/开始： |  | End/结束： |  | Qty/重量： |  |
| Transcribed by&Date/转录人及日期： |  |  |  |  |  |  |  |
| Bubble/鼓氮气 |  |  | Performed by&Date/执行人及日期： |  |  |  |  |
| Bubble/鼓氮气 |  |  | Begin/开始： |  | End/结束： |  |  |
| Transcribed by&Date/转录人及日期： |  |  |  |  |  |  |  |
| Stir /搅拌 |  |  | Performed by&Date/执行人及日期： |  |  |  |  |
| Stir /搅拌 |  |  | Begin/开始： |  | End/结束： |  |  |
| Transcribed by&Date/转录人及日期： |  |  |  |  |  |  |  |
| Stop the stirrer and turn off the nitrogen. Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point./ 停止搅拌，关闭氮气，通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  |  |  |  |  |  |
| Filter/过滤 |  |  | Performed by&Date/执行人及日期： |  |  |  |  |
|  |  |  | Checked by & Date/核实人及日期： |  |  |  |  |
|  |  |  | Begin/开始： |  | End/结束： |  |  |

| 3.47. （14）Fmoc-L- Orn(Boc)-OH coupling/N-Fmoc-N'-Boc-L-鸟氨酸偶联 |  |
| --- | --- |
| 3.47.12 IPC/过程控制 Sample the MTBE washing solution during the second stirring and develop with chloranil, compare the color with the standard 500 ppm piperidine/MTBE solution as reference solution. If the color is darker than the reference solution, invoke modules 3.48 until the color of the washing solution’s chloranil test is lighter or consistent with the reference solution. If the color is lighter or consistent with the reference solution, invoke modules 3.49. 最后一次操作，取洗涤液采用四氯苯醌显色，与标准500ppm哌啶甲基叔丁基醚溶液参比液比色：若颜色重于参比液，调用3.48模块，直至洗涤液显色的颜色轻于参比液或与参比液一致；若颜色轻于参比液或与参比液一致，调用3.49模块。 | Times/次数 |
|  | N/A |
| Standard sampling method/标准取样方法 |  |
| Take ~10 mL MTBE washing solution for develop testing. During sampling, should wear nitrile gloves and 3M 6000 series Full-face piece respirator, 6006CN cartridges. 取甲基叔丁基醚洗液~10mL，进行显色检测。取样过程中，应该戴丁腈手套、3M 6000系列全面罩, 6006CN滤毒盒。 |  |
| Sample and develop with chloranil. /取样检测四氯苯醌显色。 Sampling number/取样编号：CPo140020- -01- -3.47.12-01 Sampling time/取样时间： Sampled by & Date/取样人及日期： The color is lighter or consistent with the reference solution, invoke modules 3.49. /颜色轻于参比液或与参比液一致，调用3.49模块； Otherwise, invoke modules 3.48. /否则，调用3.48模块； Performed by & Date/执行人及日期： Checked by & Date/核实人及日期： Invoke synthesis recipe. /调用已经建立的合成配方： Recipe name /配方名称：14002001M01V02 Batch name /批次命名：CPo140020 01 Move to Modules 3.48/调用模块：3.48； Move to Modules 3.49/调用模块：3.49； Performed by & Date/执行人及日期： Checked by & Date/核实人及日期： |  |

| - Wash resin/树脂洗涤 |  |
| --- | --- |
| Tol/甲苯：Vapors irritate eyes and upper respiratory tract; cause dizziness, headache, anesthesia, respiratory arrest. Liquid irritates eyes and causes drying of skin. If aspirated, causes coughing, gagging, distress, and rapidly developing pulmonary edema. If ingested causes vomiting, griping, diarrhea, depressed respiration. 蒸气刺激眼睛和上呼吸道；引起头晕、头痛、麻醉、呼吸停止。液体刺激眼睛，导致皮肤干燥。如果抽吸，会导致咳嗽、咯咯声、痛苦和迅速发展的肺水肿。如果摄入会引起呕吐、抓痒、腹泻、呼吸抑制 |  |
| 3.48.1 Toluene addition/加甲苯 Add 418.56 kg (Acceptable weighing range: 406.00 ~ 431.12 kg) (10 V) toluene into synthesis reactor (TJ4S-1213-Sy15) by insert tube and diaphragm pump. Bubble with nitrogen for 5±2 min (target 5 min), then start the agitator. After stirring for 25±5 min (target: 25 min), stop stirring, turn off the nitrogen. Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point. 通过插底管，隔膜泵，向合成釜（TJ4S-1213-Sy15）中加入418.56 kg甲苯（Acceptable weighing range/可接受称量范围： 406.00 ~ 431.12 kg）（10 V），开启下口鼓氮气5±2 min (目标：5 min)后，开启搅拌，搅拌25±5 min（目标： 25 min）后，停止搅拌，关闭氮气。通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 | Times/次数 |
|  | 1 |
| Before material charging, confirm the material is within the retest date/ the expiry date. /加物料前，执行物料在复检日期/有效期之内。 Performed by & Date/执行人及日期： |  |
| Before drummed liquid material charging, visual check there is no other extraneous matter in material. /加桶装液体物料前，应目视执行桶内无异物。 Performed by & Date/执行人及日期： |  |
| Confirm liquid charging pipeline is released. /执行加料管路经过放行。 |  |
| Performed by & Date/执行人及日期： |  |
| Pump number/泵编号： Weighing of toluene/甲苯称重： AControl Number/内部控制号：YL000026 Scale ID. No./秤编号： Material charging pipe number/物料管路编号： |  |

| 3.48. Wash resin/树脂洗涤 |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Addition type加料式 | Lot number/批号 | Pre-charging qty calculation/投料前投料量计算 |  |  |  |  | Actual Charging Qty/实际投料量 |  |  |  |  |
|  |  | Total qty charged/已加入净重总量 | Target qty to be charged/还需加入净重目标值 | Gross qty of drum that to be charged/待加料桶毛重 | Expected net weight of drum after charge /加料完成后预计桶净重 | Expected tare weight of drum after charge/加料完成后预计桶皮重 | Before charging/ 投料前 | After charging/ 投料后 | Net weight/ 净重 | Performed by & Date/ 执行人及日期 | Checked by & Date/ 核实人及日期 |
| Full drum整桶 Non-full drum /非整桶 |  |  |  |  |  |  |  |  |  |  |  |
| Full drum整桶 Non-full drum /非整桶 |  |  |  |  |  |  |  |  |  |  |  |
| Full drum整桶 Non-full drum /非整桶 |  |  |  |  |  |  |  |  |  |  |  |

| 3.48. Wash resin/树脂洗涤 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Addition type加料式 | Lot number/批号 | Pre-charging qty calculation/投料前投料量计算 |  |  |  |  |  | Actual Charging Qty/实际投料量 |  |  |  |  |  |
|  |  | Total qty charged/已加入净重总量 |  | Target qty to be charged/还需加入净重目标值 | Gross qty of drum that to be charged/待加料桶毛重 | Expected net weight of drum after charge /加料完成后预计桶净重 | Expected tare weight of drum after charge/加料完成后预计桶皮重 | Before charging/ 投料前 |  | After charging/ 投料后 | Net weight/ 净重 | Performed by & Date/ 执行人及日期 | Checked by & Date/ 核实人及日期 |
| Full drum整桶 Non-full drum /非整桶 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Full drum整桶 Non-full drum /非整桶 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Total/合计 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Toluene Addition/加甲苯 |  |  | Begin/开始： |  |  |  |  |  | End/结束： |  |  |  |  |
| Transcribed by & Date/转录人及日期： |  |  |  |  |  |  |  |  |  |  |  |  |  |

| 3.48. Wash resin/树脂洗涤 |  |  |
| --- | --- | --- |
| Bubble/鼓氮气 | Performed by&Date/执行人及日期： |  |
| Bubble/鼓氮气 | Begin/开始： | End/结束： |
| Transcribed by&Date/转录人及日期： |  |  |
| Stir /搅拌 | Performed by&Date/执行人及日期： |  |
| Stir /搅拌 | Begin/开始： | End/结束： |
| Transcribed by&Date/转录人及日期： |  |  |
| Stop the stirrer and turn off the nitrogen. Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point./ 停止搅拌，关闭氮气，通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  |
| Filter/过滤 | Performed by&Date/执行人及日期： |  |
|  | Checked by & Date/核实人及日期： |  |
|  | Begin/开始： | End/结束： |

| 3.48. Wash resin/树脂洗涤 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| MTBE/甲基叔丁基醚： Inhalation: May cause dizziness or suffocation. Contact may irritate or burn eyes or skin. May be harmful if swallowed. /吸入:可引起头晕或窒息。接触可能刺激或灼伤眼睛或皮肤。吞食可能有害。 |  |  |  |  |  |  |
| 3.48.2 Shirnk with MTBE/甲基叔丁基醚收缩 Add 355.68 kg (Acceptable weighing range: 345.01 ~ 366.35 kg) (10 V) MTBE into synthesis reactor (TJ4S-1213-Sy15). Bubble with nitrogen for 5±2 min (target 5 min), then start the agitator. After stirring for 15±5 min (target: 15 min), stop stirring, turn off the nitrogen. Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point. 向合成釜（TJ4S-1213-Sy15）中加入355.68 kg甲基叔丁基醚（Acceptable weighing range/可接受称量范围： 345.01 ~ 366.35 kg）（10 V），开启下口氮气5±2 min (目标时间：5 min)，开启搅拌15±5 min（目标时间：15 min）后，停止搅拌，关闭氮气。通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  |  |  |  | Times/次数 |
|  |  |  |  |  |  | 1 |
| Add MTBE/ 加甲基叔丁基醚 | Performed by&Date/执行人及日期： |  |  |  |  |  |
|  | Begin/开始： |  | End/结束： |  | Qty/重量： |  |
| Transcribed by&Date/转录人及日期： |  |  |  |  |  |  |
| Bubble/鼓氮气 |  | Performed by&Date/执行人及日期： |  |  |  |  |
| Bubble/鼓氮气 |  | Begin/开始： |  | End/结束： |  |  |
| Transcribed by&Date/转录人及日期： |  |  |  |  |  |  |
| Stir /搅拌 |  | Performed by&Date/执行人及日期： |  |  |  |  |
| Stir /搅拌 |  | Begin/开始： |  | End/结束： |  |  |
| Transcribed by&Date/转录人及日期： |  |  |  |  |  |  |

| 3.48. Wash resin/树脂洗涤 |  |  |  |
| --- | --- | --- | --- |
| Filter/过滤 | Performed by&Date/执行人及日期： |  |  |
|  | Checked by & Date/核实人及日期： |  |  |
|  | Begin/开始： | End/结束： |  |
| MTBE/甲基叔丁基醚： Inhalation: May cause dizziness or suffocation. Contact may irritate or burn eyes or skin. May be harmful if swallowed. /吸入:可引起头晕或窒息。接触可能刺激或灼伤眼睛或皮肤。吞食可能有害。 |  |  |  |
| 3.48.3 Shrink with MTBE/甲基叔丁基醚收缩 Add 355.68 kg MTBE into synthesis reactor (TJ4S-1213-Sy15). Bubble with nitrogen for 5±2 min (target 5 min), then start the agitator. After stirring for 15±5 min (target: 15 min), stop stirring, turn off the nitrogen. Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point. 向合成釜（TJ4S-1213-Sy15）中加入355.68 kg甲基叔丁基醚（Acceptable weighing range/可接受称量范围： 345.01 ~ 366.35 kg）（10 V），开启下口氮气5±2 min (目标：5 min)，开启搅拌15±5 min（目标：15 min）后，停止搅拌，关闭氮气。通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  | Times/次数 |
|  |  |  | 1 |

| 3.48. Wash resin/树脂洗涤 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Add MTBE/ 加甲基叔丁基醚 | Performed by&Date/执行人及日期： |  |  |  |  |  |
|  | Begin/开始： |  | End/结束： |  | Qty/重量： |  |
| Transcribed by&Date/转录人及日期： |  |  |  |  |  |  |
| Bubble/鼓氮气 |  | Performed by&Date/执行人及日期： |  |  |  |  |
| Bubble/鼓氮气 |  | Begin/开始： |  | End/结束： |  |  |
| Transcribed by&Date/转录人及日期： |  |  |  |  |  |  |
| Stir /搅拌 |  | Performed by&Date/执行人及日期： |  |  |  |  |
| Stir /搅拌 |  | Begin/开始： |  | End/结束： |  |  |
| Transcribed by&Date/转录人及日期： |  |  |  |  |  |  |
| Stop the stirrer and turn off the nitrogen. Start draining the solution from synthesis reactor with diaphragm pump. Drain until there is no drip, the operator visually confirms the draining end point./ 停止搅拌，关闭氮气，通过隔膜泵从合成釜中排出溶液。排液至无液体滴出，由操作人员目视确认排液终点。 |  |  |  |  |  |  |
| Filter/过滤 |  | Performed by&Date/执行人及日期： |  |  |  |  |
|  |  | Checked by & Date/核实人及日期： |  |  |  |  |
|  |  | Begin/开始： |  | End/结束： |  |  |
| 3.48.4 Sample /取样 Sample the MTBE washing solution during the second stirring and develop with chloranil, compare the color with the standard 500 ppm piperidine/MTBE solution as reference solution. If the color is darker than the reference solution, invoke modules 3.48 until the color of the washing solution’s chloranil test is lighter or consistent with the reference solution. If the color is lighter or consistent with the reference solution, invoke modules 3.49. 最后一次操作，取洗涤液采用四氯苯醌显色，与标准500ppm哌啶甲基叔丁基醚溶液参比液比色：若颜色重于参比液，调用3.48模块，直至洗涤液显色的颜色轻于参比液或与参比液一致；若颜色轻于参比液或与参比液一致，调用3.49模块。 |  |  |  |  |  | Times/次数 |
|  |  |  |  |  |  | N/A |

3.48. Wash resin/树脂洗涤

Standard sampling method/标准取样方法

Take ~10 mL MTBE washing solution for develop testing. During sampling, should wear nitrile gloves and 3M 6000 series Full-face piece respirator, 6006CN cartridges. 取甲基叔丁基醚洗液~10mL，进行显色检测。取样过程中，应该戴丁腈手套、3M 6000系列全面罩, 6006CN滤毒盒。

Sample and develop with chloranil. /取样检测四氯苯醌显色。 Sampling number/取样编号：CPo140020- -01- -3.48.4-01 Sampling time/取样时间： Sampled by & Date/取样人及日期： The color is lighter or consistent with the reference solution, invoke modules 3.49. /颜色轻于参比液或与参比液一致，调用3.49模块； Otherwise, invoke modules 3.48. /否则，调用3.48模块； Performed by & Date/执行人及日期： Checked by & Date/核实人及日期： Invoke synthesis recipe. /调用已经建立的合成配方： Recipe name /配方名称：14002001M01V02 Batch name /批次命名：CPo140020 01 Move to Modules 3.48/调用模块：3.48； Move to Modules 3.49/调用模块：3.49； Performed by & Date/执行人及日期： Checked by & Date/核实人及日期：

| - Dry/烘料 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3.49.1 Dry /烘料 Sweep with nitrogen above, nitrogen flow（11~15 m3/h target: 14 m3/h） and vacuum underneath at T ≤30 ℃ by jacket. After drying for 16 h±2 h, sample the solid for LOD analysis every 6±4 h until LOD ≤10.0%. 夹套控温T ≤30 ℃，上口吹氮气（11~15 m3/h 目标，14 m3/h），下抽真空，烘料16 h±2 h后开始第一次取样检测LOD，之后每隔6±4 h 取样一次直至LOD ≤10.0%。 Note: After drying 6 h, begin to stir the solid for 2~5 min at a speed of 7±2 rpm (reference) every 4±1 h. 备注：烘料6h后开启搅拌，每4±1 h，搅拌2~5 min，参考搅拌转速7±2 rpm。 |  |  |  |  |  |  |  | Times/次数 |
|  |  |  |  |  |  |  |  | 1 |
| Dry/烘料 |  | Performed by&Date/执行人及日期： |  |  |  |  |  |  |
| Dry/烘料 |  | Begin/开始： |  |  |  | End/结束： |  |  |
| Transcribed by&Date/转录人及日期： |  |  |  |  |  |  |  |  |
| Jacket temperature/夹套温度 |  |  |  | High values/最高值： |  |  | Low values/低 |  |
| Transcribed by&Date/转录人及日期： |  |  |  |  |  |  |  |  |
| Stirring record/搅拌记录： |  |  |  |  |  |  |  |  |
| No./次数 | Begin/开始 |  | End/结束 |  | Agitator speed/转速 |  | Recorded by & Date/记录人及日期 |  |
| 1 |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |  |  |
| 7 |  |  |  |  |  |  |  |  |
| 8 |  |  |  |  |  |  |  |  |
| 9 |  |  |  |  |  |  |  |  |
| 10 |  |  |  |  |  |  |  |  |

| 3.49. Dry/烘料 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Standard sampling method/标准取样方法 |  |  |  |  |
| Take ~1 g solid and submit for analysis testing. During sampling, should wear nitrile rubber gloves and 3M 6000 series Full-face piece respirator, 6006CN cartridges. 取固体~1g，直接送样检测。取样过程中，应该戴丁腈橡胶手套、3M 6000系列全面罩, 6006CN滤毒盒。 |  |  |  |  |
| Sampling number/ 取样编号 | Sampling time/取样时间 | Sampled by & Date/取样人及日期 | LOD/干燥失重 | Result filled by &Date/结果填写人及日期 |
| CPo140020- -01- -3.49.1-01 |  |  |  |  |
| CPo140020- -01- -3.49.1-02 |  |  |  |  |
| CPo140020- -01- -3.49.1-03 |  |  |  |  |
| CPo140020- -01- -3.49.1-04 |  |  |  |  |
| Process chemist confirms the end point of the drying and move to following operation. 工艺技术人员确认烘料终点后进行下一步操作； Process chemist & Date/ 工艺技术人员签字及日期： Checked by & Date/核实人签字及日期： |  |  |  |  |

| Ins.4/操作4：Collect/收料& Analyze/检测 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Wear nitrile gloves and 3M 6000 series Full-face piece respirator, 6006CN cartridges, Particulate Drain, White disposable chemical protective clothing. 戴丁腈手套、3M 6000系列全面罩, 6006CN滤毒盒, 颗粒物滤棉, 白色防化服。 |  |  |  |  |  |  |  |
| Put the peptide resin into double LDPE bags, number and weigh, inner layer of pharmaceutical grade LDPE powder feeding bags, and outer pharmaceutical grade LDPE bag, the desiccant between the two layers, purge the inside of the package with nitrogen, seal the in-process product in HDPE drums for temporary storage. 将烘干后肽树脂放入双层低密度聚乙烯袋中，编号称重。内层药用级粉末投料袋，外层药用级低密度聚乙烯袋，两层之间加干燥剂，氮气吹扫包装内部，中间体密封在高密度聚乙烯桶中临时存放。 In-process intermediate lot number/过程中间体编号：CPo140020- -01- Lot number of inner pharmaceutical grade LDPE powder feeding bags /内层药用级低密度聚乙烯袋批号： Lot number of outer pharmaceutical grade LDPE bag/外层药用级低密度聚乙烯袋批号： Lot number of desiccant/干燥剂批号： Note: The solid in each bag ≤25 kg. If the project has special requirements, need to do risk assessment and get approval. 备注：每袋物料≤25Kg，若项目有特殊要求，需经过风险评估和审批。 |  |  |  |  |  |  |  |
| Key raw materials, in-process products and products for PPQ and commercial production should be weighed after the weighing implements are checked using the standard weights. For the balance/scale with the maximum capacity of greater than 40 kg, check with the standard weight of 20 kg; for the balance/scale with maximum capacity is not more than 40 kg, check with the standard weight of 1/2 maximum capacity. /用于工艺性能执行和商业化生产的主原料、过程产物以及产品的称量前，应使用标准砝码对称量器具进行检查，量程大于40 kg的电子秤，采用20 kg的标准砝码进行检查；量程不大于40 kg的电子秤，采用最大量程1/2的标准砝码进行检查。 Checking record for standard weights/标准砝码检查记录： |  |  |  |  |  |  |  |
| Weight No. 砝码编号 | Weight 砝码重量（kg） | Reading 示重 |  | Less than half of verification scale interval. / 小于检定分度值的一半 | Performed by & Date/执行人及日期 |  | Checked by & Date/核实人及日期 |
|  |  |  |  | Y N |  |  |  |
| Collect/收料（Time/时间） |  |  | Begin/开始： |  |  | End/结束： |  |

| Ins.4/操作4：Collect/收料& Analyze/检测 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Weighing of peptide resin /肽树脂称重： Scale ID. No./秤编号： |  |  |  |  |  |
| peptide resin lot number/ 肽树脂批号 | in-process intermediate Qty/ 过程中间体质量 |  |  | Performed by & Date/ 执行人及日期 | Checked by & Date/ 核实人及日期 |
|  | Tare weight/ 皮重 | Total weight/ 总重 | Net weight/ 净重 |  |  |
| CPo140020- -01- |  |  |  |  |  |
| Total/合计 |  |  |  |  |  |

| Ins.4/操作4：Collect/收料& Analyze/检测 |  |
| --- | --- |
| Appearance/外观： Inspected by and date / 检查人及日期： |  |
| Label type (quality form number and version)/标签类型（表格编号及版本号）： Label filling and pasting Performed by&Date/标签填写及粘贴人及日期： Label filling and pasting Checked by&Date/标签填写及粘贴核实人： |  |
| Synthesis completed, print the corresponding operation log and attach it to this MOR/合成结束，打印出相应的操作记录并作为此文件附件。 |  |
| 1 | 3000 L Peptide Solid Phase Synthesizer /多肽固相合成仪（TJ4S-1213-Sy15）Summary Report of Operation Records log /操作记录汇总报表附件编号：CPo140020-_______-01- Print and confirm that the parameters in the attachment meet the process requirements. / 打印并执行附件中各参数满足工艺要求。 Performed by & Date/执行人及日期：_______________________________ |
| 2 | 3000 L Peptide Solid Phase Synthesizer /多肽固相合成仪（TJ4S-1213-Sy15）Temperature log/温度记录附件编号：CPo140020-________-01-__________________________________ Print and confirm that the parameters in the attachment meet the process requirements. / 打印并执行附件中各参数满足工艺要求。 Performed by & Date/执行人及日期：_______________________________ |
| 3 | 3000 L Stainless steel agitator tank/不锈钢搅拌罐（TJ4S-1213-SPS01-AT01）的20% V/V piperidine/DMF solution preparation log number/哌啶/DMF溶液配制记录附件编号：CPo140020-________-01-________________________ Print and confirm that the parameters in the attachment meet the process requirements. / 打印并执行附件中各参数满足工艺要求。 Performed by & Date/执行人及日期：_______________________________ |

| Ins.4/操作4：Collect/收料& Analyze/检测 |  |  |  |
| --- | --- | --- | --- |
| Synthesis completed, print the corresponding operation log and attach it to this MOR/合成结束，打印出相应的操作记录并作为此文件附件。 |  |  |  |
| 4 | 3000 L Stainless steel agitator tank/不锈钢搅拌罐（TJ4S-1213-SPS01-AT02）的20% V/V piperidine/DMF solution preparation log number/哌啶/DMF溶液配制记录附件编号：CPo140020-________-01-________________________ Print and confirm that the parameters in the attachment meet the process requirements. / 打印并执行附件中各参数满足工艺要求。 Performed by & Date/执行人及日期：_______________________________ |  |  |
| 5 | 3000 L Peptide Solid Phase Synthesizer /多肽固相合成仪（TJ4S-1213-Sy15）Raw Material Summary Table/物料使用汇总表附件编号：CPo140020-________-01-________________________ Print and confirm that the parameters in the attachment meet the process requirements. / 打印并执行附件中各参数满足工艺要求。 Performed by & Date/执行人及日期：_______________________________ |  |  |
| Note/备注：1）Number rules/编号原则：project No. + step No. + batch No. + page No. + attachment sequence No. /项目号+步骤号+批次号+页码号+附件号，e.g. 如CPo1234-01-02-03-P01-01）; |  |  |  |
| Transfer the in-process intermediate to warehouse for temporary storage. During transfer, use a cabinet when transferring. 将过程中间体从 转移至 暂存。转移时使用保温箱 Storage condition: seal and avoid light, storage at 2~8℃. /贮藏条件：密封，避光，2~8℃保存。 |  |  |  |
| Package Qty./包装数： Confirmation of package/包装方式执行： HDPE drum/高密度聚乙烯桶 |  |  |  |
| Transfer begin Time/ 转移开始时间： |  | Transfer end Time/ 转移结束时间： | Transfer time/ 转移用时： |
| Recorded by & date/填写人及日期： |  |  |  |
| Confirm transfer time not exceed 1 h. /执行转移时间不超过1h。 Performed by & date/执行人及日期： |  |  |  |

Appendix/附录

1. Supplemental Record/附加页

If the tables provided in the Operations Instructions are not enough to record all information, you can record them in the following table. If there is still not enough record, project personnel should request <OR supplemental page> (QF05037) from QA Dept.

若操作指令部分的表格不够用，可以把信息记录到如下表格中。如仍有填写不尽的应到QA部门领取《OR附加页》（QF05037）进行记录。

Note/注：

- Be consistent with the recording requirements of the operating part of the OR text. For example, if there is weighing operation, need to record the Performed by& Date and Checked by & Date; if there is temperature, need to record the Entered by and Date (record in the Performed by& Date column)./和OR正文中操作部分的记录要求一致。例如有称重操作的，需要记录执行人及日期和复核人及日期；有温度的，需要记录记录人及日期(记录在执行人及日期栏)。

- IPC Sample/material number rule: Batch No. + Instruction No. + Sequent number, sequent number should take OR and other corresponding OR supplemental page into consideration. For example, the first sample/material number in instruction 12 of batch 5, step 3 of project CPo122345-01 is CPo122345-01-03-05-12-05. /IPC样品/物料编号原则：生产批号+操作指令号+顺序号。顺序号应考虑该操作指令内OR及其他相关OR附加页中已有的顺序号。例如：项目CPo122345-01第三步第五批第12个操作指令中产生的第5个样品/物料编号，其编号为：CPo122345-01-03-05-12-05。

From/上接：                         （Ins. No. & Page#/指令编号和页码）

| Operation Name/ 操作名称 | Operation Description /操作内容 Include: Process parameters (e.g. Temp., vacuum, pressure, pH) 包含：工艺参数（温度、真空度、压力、pH）等 | Time/ 时间 | Performed by& Date/ 执行人/日期 | Reviewed by & Date/ 复核人/日期 | Note/ 备注 |
| --- | --- | --- | --- | --- | --- |

To/下转：            （Ins. No. & Page#/指令编号和页码/Document No. of Supplemental page/OR附加页编号）

From/上接：                         （Ins. No. & Page#/指令编号和页码）

| Operation Name/ 操作名称 | Operation Description /操作内容 Include: Process parameters (e.g. Temp., vacuum, pressure, pH) 包含：工艺参数（温度、真空度、压力、pH）等 | Time/ 时间 | Performed by& Date/ 执行人/日期 | Reviewed by & Date/ 复核人/日期 | Note/ 备注 |
| --- | --- | --- | --- | --- | --- |

To/下转：            （Ins. No. & Page#/指令编号和页码/Document No. of Supplemental page/OR附加页编号）

2. Process flow chart /工艺流程图

English/英文：

Chinese/中文：

3. English abbreviations /英文缩写表

| English Abbreviation 英文缩写 | English Full Name 英文全名 | Chinese Name 中文名 |
| --- | --- | --- |
| IPC | In Process Control | 过程控制 |
| SDS | Safety Data Sheet | 安全数据表 |
| Qty | Quantity | 质量 |
| Ins. | Instruction | 指令 |
| MPa | Mega Pascal | 兆帕斯卡 |
| KF | Karl-Fischer | 卡尔费休 |
| LOD | Loss on Drying | 干燥失重 |
| MP | Melting Point | 熔点 |
| BP | Boiling Point | 沸点 |
| NMT (NGT) | No More Than (No greater Than) | 不大于，即小于等于 |
| NLT | No Less Than | 不小于，即大于等于 |

Post-batch Approvals/操作完毕执行

I have reviewed the above operations record and confirmed that it accurately represents how the in-process intermediate noted on the record was produced.

我本人已经阅读了以上的操作记录，并确信此记录准确地记述了记录上过程中间体的生产过程。

Production Director & Date/生产主管及日期:

Project Director & Date/项目主管及日期:

QA project manager & Date/QA项目经理及日期:

Version Instructions/版本说明

1. Reference scale-up process procedure: Scale-up Process Procedure of Tech- TJ414002001, version 4.0

参照放大工艺：Tech-TJ414002001放大生产工艺（第四版）

2. The MOR is written with the Standardization language (SOP DS-QA-001 <Batch operation record management procedure (DS)>. MOR是按照标准化语言撰写（SOP DS-QA-001《批操作记录管理规程(DS)》）。

3. This is 4th version of MOR- TJ414002001 MOR, so the contrast with the last:

此版本为MOR- TJ414002001MOR第四版，与上一版的对比说明如下：

Brief description/更改内容简要描述： - The batch record template has been updated. / 批记录的模板升级。 - The equipment flow chart has been updated. /设备流程图更新。 - Adjusted two equipment: One of the equipment for preparation of OxymaPure/DMF solution is planned change to TJ4S-1200-AT02 from TJ4S-1213-R21; b). Another equipment for preparation of Ac2O / Pyridine / DMF solution will be changed to TJ4S-1213-R22 from TJ4S-1213-Sy15-R01. /设备调整：1配制Oxyma/DMF溶液的设备从TJ4S-1213-R21调整为TJ4S-1200-AT02； 2配制 Ac2O / Pyridine / DMF的封端溶液设备从TJ4S-1213-Sy15-R01调整到 TJ4S-1213-R22。 - Post Ac2O capping DMF wash is changed to direct DMF feeding into the synthesis reactor (TJ4S-1213-Sy15). / Ac2O封端后的树脂DMF洗涤方式调整为直接从合成釜(TJ4S-1213-Sy15)加料洗涤。 - The output batch size has been changed to 145.12 – 271.65 kg/批量产出范围调整至145.12 – 271.65 kg； - The collection bags are replaced with pharmaceutical powder charging bags. / 收料阶段的内包装变为药用级别投料袋。 - Adjustment of Sample Processing Method for IPC Testing. /IPC检测的样品处理方式调整。 - Sample preparation for substitution degree testing shall be adjusted with reference to the validated sample handling procedure for IPC testing. /取代度检测的样品处理，参照验证后的IPC检测阶段处理方法进行调整。 - The acceptance criteria for the in-process product have been removed. Sampling for release testing is no longer required. Instead, process chemists shall verify the color and record the observations in the MOR. / 已移除中间体的质量标准，不再开展放行取样；由工艺技术员执行颜色，并将观测结果记录于 MOR 中。 - The stir until visually clear was changed to stirring for at least 15 min. /将搅拌至目视溶清调整为搅拌至少15 min。 - After adding acetic anhydride to the end-capping solution, stir for 5–10 minutes (target: 7 minutes), adjusting to at least 5 minutes of stirring. /将配制封端液加料乙酸酐后搅拌5~10 min（目标：7 min）调整为搅拌至少5 min。 - The sampling time has been changed from 12 h after drying to 16 h ± 2 h. /由烘料12 h后开始取样调整为16 h±2 h。 - The reference time for DIPEA addition was adjusted from 15±5 min (target: 5 min) to 20±10 min (target: 10 min) in capping operation. /封端操作中DIPEA的加料参考时间由15±5 min（目标：5 min）调整为20±10 min（目标：10 min）。 Details in Change file, Change No./以上更改详细内容见变更文件，编号： Change-140020030104.01

4. MOR numbering method: MOR-site code+product code + step No. + version#(2digits), as MOR-TJ3111234A01.01.

MOR编号方式为MOR-厂区代码+产品代码+步骤号+2位版本号，如MOR-TJ3111234A01.01