Process Performance Qualification Protocol

for the Manufacturing Process of ASYM-141366

(Project Code CCL# ASYM-141366)

at Asymchem Life Science (Tianjin) Co., Ltd.

(No.265, South Avenue, Binhai New Area TEDA, Tianjin, 300462, China)

| Prepared by |  |  |
| --- | --- | --- |
| Pengda Zhao (Associate Director, CMMD) | Signature &Date |  |
| Reviewed by |  |  |
| Liang Li (Senior Director, CMMD) | Signature &Date |  |
| Zhongyao Zhang (Director, Production) | Signature &Date |  |
| Jiwei Cai ( Director, QC) | Signature &Date |  |
| Yongle Liu (Director, Dept. of Analytical Research and Development) | Signature &Date |  |
| Lu Li (Director, QA) | Signature &Date |  |
| Approved by |  |  |
| Xinyi Zhu (Executive Director, CMMD) | Signature &Date |  |
| Ruijun Shi (Vice Chief Engineer, Production) | Signature &Date |  |
| Haini wang (Senior Director, QC) | Signature &Date |  |
| Ziyuan Zhang (Senior Director, Dept. of Analytical Research and Development) | Signature &Date |  |
| Mingyang Yang (Executive Director, QA) | Signature &Date |  |
| Final Approved by |  |  |
| Shenghui Yu (Vice General Manager, QA) | Signature &Date |  |

This document is reviewed and approved by Eli Lilly and Company. Refer to signature page for EDMS signatures and dates for Eli Lilly and Company approval.

Revision History

| Effective Date | Protocol and version No. | Document Changes | Change Justification |
| --- | --- | --- | --- |
|  | PVP-1400200301.01 | The first version | N/A |

Note：The document shall only take effect after the training is completed, and the effective date will be manually filled in by designated personnel with signature and date once confirmed.

Table of Content

Revision History	4

1.	Introduction	6

2.	Objective and Scope	9

3.	Validation Type and Strategy	9

4.	Manufacturing, release and stability testing site	10

5.	Functions and Responsibilities	10

6.	Batch Plan	11

7.	Qualified Process Equipment	12

8.	Process Chemistry Overview	17

9.	Critical Process Parameters and Critical Quality Attributes (CPPs and CQAs)	33

10.	Materials and Specifications	34

11.	Analytical Methods and Testing	39

12.	Batch Production Record (OR) Review	40

13.	Sample Retaining 	40

14.	Packaging and Storage	40

15.	Stability Test	41

16.	Homogeneity study	41

17.	ASYM-141366 Cleaning Validation	41

18.	Changes and Deviations	42

19.	Acceptance Criteria	42

20.	Summary Report	42

21.	Batch Release and Disposition	43

Appendix 1 Training SOP (Current Version)	44

Appendix 2 Referring document list	45

Appendix 3   IPCs Results Summary Table	46

Appendix 4   CPPs Results Summary Table	48

- Introduction

Compound N-Fmoc-N’-Boc-L-ornithyl-L-leucyl-L-alanyl-L-γ-tBu-glutaminyl-α-methylphenylalanyl-L-leucyl-L-valyl-N’-(2,2,4,6,7-pentamethyldihydrobenzofuran-5-sulfonyl)-L-arginyl-tBu-L-seryl-tBu-L-seryl-N-triphenylmethyl-L-asparagyl-N-methyl-N’-triphenylmethyl-asparagyl-phenylalanyl-glycine (Lilly product code: LSN4182924; Asymchem product code: ASYM-140020; Compound name: Eloralintide Fragment L2) is a peptide fragment s product. The synthetic process is provided by Eli Lilly and Company (Lilly). Asymchem Life Sciin-procesence (Tianjin) Co., Ltd. (production address: No.265, South Avenue, Binhai New Area TEDA, Tianjin, 300462, China, referred to as Asymchem TJ4) is selected as the intended commercial supplier. Asymchem product code: ASYM-141366 is a peptide on-resin in-process intermediate of the final product ASYM-140020, and the structure is shown in Figure 1.

Figure 1: Structure of ASYM-141366

The in-process intermediate ASYM-141366 is prepared by solid-phase synthesis using 2-Chlorotrityl chloride (2-CTC) resin as the raw material. This synthetic route uses Fmoc-Gly-OH, Fmoc-L-Phe-OH, Fmoc-L-N-Me-Asn(Trt)-OH, Fmoc-L-Asn(Trt)-OH, Fmoc-L-Ser(tBu)-OH, Fmoc-L-Arg(Pbf)-OH, Fmoc-L-Val-OH, Fmoc-L-Leu-OH, Fmoc-L-α-Me-Phe-OH, Fmoc-L-Glu(OtBu)-OH·H2O, Fmoc-L-Ala-OH·H2O and Fmoc-L-Orn(Boc)-OH as the proposed regulatory starting materials. The target compound is prepared by multiple cycles of solid-phase synthesis, washing, deswelling and drying. The synthetic route is described in Figure 2 below.

Figure 2: Synthetic route of ASYM-141366

The process was transferred from Lilly to Asymchem in August 2024, and the process was familiarized and further optimized for scaling up at Asymchem Life Science (Tianjin) Co., Ltd. (referred to as Asymchem TJ4) R&D laboratory. This Process Performance Qualification (PPQ) protocol has been developed as part of the overall process validation lifecycle strategy described in the applicable Validation Master Plan < Process Validation Plan for Manufacturing of LSN4182924 (ASYM-140020): VP-14002001.01>. Process knowledge obtained during development, technology transfer, scale-up studies, risk assessments and process characterization activities has been used to establish the manufacturing strategy and define the parameters requiring validation during PPQ.

The process described in this protocol represents the intended commercial manufacturing process for ASYM-141366 and PPQ execution is intended to demonstrate suitability of the process for routine commercial manufacture.

For current campaign CPo140020-03, the process performance qualification (PPQ) of ASYM-141366 Step 1 is planned to be started in October 2026 under cGMP conditions at Asymchem TJ4. ASYM-141366 is a peptide-on-resin in-process intermediate intended for further processing in downstream manufacturing operations. As the material is not an isolated API intermediate with an approved release specification, process validation is primarily based on demonstrated control of the manufacturing process, successful achievement of critical in-process controls and suitability for downstream processing.  For clarity, the compound code and company codes applied in ASYM-141366 production are listed in Table 1 and Table 2 respectively.

Table 1. Corresponding Asymchem and Customer compound codes

| Step | RSM /In-process intermediate | Chemical Name | Customer Material Code | Asymchem Code |
| --- | --- | --- | --- | --- |
| 1 | RSM | Fmoc-Gly-OH | N/A | ASYM-138200 |
| 1 | RSM | Fmoc-L-Phe-OH | N/A | ASYM-138203 |
| 1 | RSM | Fmoc-L-N-Me-Asn(Trt)-OH | N/A | ASYM-138520 |
| 1 | RSM | Fmoc-L-Asn(Trt)-OH | N/A | ASYM-138519 |
| 1 | RSM | Fmoc-L-Ser(tBu)-OH | N/A | ASYM-132732 |
| 1 | RSM | Fmoc-L-Arg(Pbf)-OH | N/A | ASYM-138518 |
| 1 | RSM | Fmoc-L-Val-OH | N/A | ASYM-138202 |
| 1 | RSM | Fmoc-L-Leu-OH | N/A | ASYM-138201 |
| 1 | RSM | Fmoc-L-α-Me-Phe-OH | N/A | ASYM-138210 |
| 1 | RSM | Fmoc-L-Glu(OtBu)-OH·H2O | N/A | ASYM-138390 |
| 1 | RSM | Fmoc-L-Ala-OH·H2O | N/A | ASYM-138204 |
| 1 | RSM | Fmoc-L-Orn(Boc)-OH | N/A | ASYM-124852 |
| 1 | In-process intermediate | N-Fmoc-N’-Boc-L-ornithyl-L-leucyl-L-alanyl-L-γ-tBu-glutaminyl-α-methylphenylalanyl-L-leucyl-L-valyl-N’-(2,2,4,6,7-penta methyldihydrobenzofuran-5-sulfonyl)-L-arginyl-tBu-L-seryl-tBu-L-seryl-N-triphenylmethyl-L-asparagyl-N-methyl-N’-triphenylmethyl-asparagyl-phenylalanyl-glycine on resin | N/A | ASYM-141366 |

Table 2. Abbreviation code of the related company name

| Company Name | Company Address | Abbreviation |
| --- | --- | --- |
| Eli Lilly and Company | Eli Lilly Kinsale Limited Dunderrow Kinsale Cork P17 NY71 Ireland | Lilly |
| Asymchem Life Science (Tianjin) Co., Ltd. | No.265, South Avenue, Binhai New Area TEDA, Tianjin, 300462, China | Asymchem TJ4 |
| Chengdu ZY Biochemical Technology Co., Ltd. | No.170 Qingpu Road, Shou'an Subdistrict, Pujiang County, Chengdu City, Sichuan Province, P.R.China | ZY Biochem |
| Asymchem Laboratories (Jilin) Co., Ltd. | No. 899, Xingqi Road, High-tech Industrial Zone, Dunhua, Jilin 133700, China | DH3 |
| Liaoning Asymchem Laboratories Co., Ltd | Yimatu Village, Yimatu town, Fu Mongolia county (Fluorine Industry Development Area), Fuxin, Liaoning, 123129, China | FX2 |
| Tianjin Nankai Hecheng S&T Co., Ltd. | No.969-6, Kaixuan Street, Dagang Petrochemical Industrial Park, Binhai New Area, Tianjin, P.R.China | Hecheng |
| Sunresin New Materials Co. Ltd. | No.135, jinye Road, Xi'an Hitech Industrial Development Zone | SUN RESIN |

- Objective and Scope

The objective of this protocol is to establish scientific and risk-based evidence which provides a high degree of assurance that the manufacture process of ASYM-141366 will consistently produce. ASYM-141366 meets its IPC acceptance criteria and is suitable for forward processing.

The scope of this PPQ protocol is applicable to ASYM-141366 manufacturing process, using 2-CTC Resin, Fmoc-Gly-OH, Fmoc-L-Phe-OH, Fmoc-L-N-Me-Asn(Trt)-OH, Fmoc-L-Asn(Trt)-OH, Fmoc-L-Ser(tBu)-OH, Fmoc-L-Arg(Pbf)-OH, Fmoc-L-Val-OH, Fmoc-L-Leu-OH, Fmoc-L-α-Me-Phe-OH, Fmoc-L-Glu(OtBu)-OH·H2O, Fmoc-L-Ala-OH·H2O and Fmoc-L-Orn(Boc)-OH as the proposed regulatory starting materials, with 2-CTC Resin as a critical raw material. Manufacturing will be performed at the defined commercial manufacturing scale of target 48.00 kg 2-CTC Resin per batch for the SPPS process in Building 12 at Asychem TJ4 site. This protocol covers manufacture of ASYM-141366 at commercial scale in the qualified equipment train identified within this document and includes evaluation of process performance against CPPs and CQAs. Cleaning validation, stability and supporting studies are performed alongside PPQ under separate protocols (Sections 16–18)

- Validation Type and Strategy

A prospective Process Performance Qualification (PPQ) approach will be used to demonstrate that the commercial manufacturing process for ASYM-141366 when operated within the established process parameters and control strategy, is capable of manufacturing material that reliably meets its predefined quality attributes, in-process requirements and release specifications. The PPQ will be executed using the intended commercial manufacturing process, qualified facility, utilities and equipment, approved manufacturing instructions, qualified analytical methods, approved raw materials and appropriately trained personnel.

The validation strategy has been established using a science- and risk-based assessment of process knowledge, process complexity, sources of variability and the relationship between material attributes/process parameters and product quality attributes. Based on the available development and manufacturing knowledge, process risk assessment and demonstrated process understanding, three consecutive PPQ batches manufactured at the intended commercial scale using the defined commercial equipment train are considered sufficient to provide evidence of process reproducibility and an initial state of control. All three PPQ batches will be manufactured in the same equipment train, in accordance with the approved Master Operation Record (refer to Appendix 2) and this PPQ protocol (PVP-1400200301.01).

- Manufacturing, release and stability testing site

The manufacturing, release testing and stability testing sites are indicated in Table 3 below.

Table 3. Manufacturing, release and stability testing site

| Step No. | Manufacturing site and address | Release testing site and address | Stability testing site and address |
| --- | --- | --- | --- |
| Step 1 | Asymchem TJ4 | Asymchem TJ4 | Asymchem TJ4 |
|  | No.265, South Avenue, Binhai New Area TEDA, Tianjin, 300462, China | No.265, South Avenue, Binhai New Area TEDA, Tianjin, 300462, China | No.265, South Avenue, Binhai New Area TEDA, Tianjin, 300462, China |

- Functions and Responsibilities

CMMD

- Responsible for overall planning and coordination of the PPQ;

- Drafting, review and approval of PPQ document;

- Monitoring the process implement, organization of PPQ activity.

Quality Assurance

- Review and approval of  PPQ documents;

- Ensuring all validation activities are in accordance with Asymchem procedure and cGMP standards;

- Organize evaluation of related change controls and deviations.

Production Department

- Responsible for review and approval of PPQ documents;

- Responsible for operation of production, cleaning of facility/equipment during PPQ;

- IPC sampling.

Quality Control

- Responsible for the review and approval of PPQ documents.

- Ensure that specifications and validated analytical methods are available and relevant analytical instruments are qualified;

- IPC testing, sampling and testing of product and intermediate, and equipment cleaning testing.

Lilly

- Review and approve validation documents per the Quality Agreement.

- Define the Validation Strategy and communicate APIEM process validation expectations.

- Provide technical oversight for critical quality attributes and process parameters

- Provide GMP oversight and ensure alignment with Lilly governing standards.

The personals who are executing the protocol shall enter his/her name in training signature table. The training signature record shall be attached report to indicate that the personnel are trained on the SOP in Appendix 1 (current version).

- Batch Plan

The planned batch information for PPQ is summarized in the table below.

Table 4. Batch information

| Step | Vessel size | Type of batches | Number of batches | Batch Input* | Batch Output* | Yield range |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 3000 L | PPQ batch 1 | CPo140020- 03-01-02 | 2-Chlorotrityl chloride resin 48.00 kg target (47.90 – 48.10 kg) | ASYM-141366 (105.63 kg - 278.62 kg) | 60% - 130% |
|  |  | PPQ batch 2 | CPo140020- 03-01-03 |  |  |  |
|  |  | PPQ batch 3 | CPo140020- 03-01-04 |  |  |  |

Note: * Input varies within a ± 0.2% range of the target input. The batch size (output) is calculated based on the input range and the expected yield range. The lowest input amount charged combined with the expected lowest yield shall give the output lower limitation, while the highest input amount charged combined with the expected highest yield shall give the output upper limitation. .

Process validation (PPQ) strategy: Figure 3 depicts the Eloralintide Fragment 2 PPQ batch strategy, three commercial-scale SPPS PPQ batches is based on the batch strategy shown in Figure 3. It is designed to demonstrate that Step 1 (SPPS) and Step 2 (cleavage), operated under routine commercial manufacturing conditions at Asymchem TJ4, consistently produce in-process intermediates that meet pre-established acceptance criteria. The design comprises: Step 1 (SPPS): three consecutive SPPS PPQ batches (SPPS PPQ1, PPQ2 and PPQ3) manufactured at commercial scale using qualified equipment, approved raw materials, approved master batch records (MBRs) and the validated raw material charge calculation spreadsheet. Step 2 (cleavage): three consecutive cleavage PPQ batches (Cleavage PPQ1, PPQ2 and PPQ3) manufactured at commercial scale using qualified equipment, approved raw materials, approved master batch records (MBRs) and the validated raw material charge calculation spreadsheet. Material input variability: the SPPS PPQ batches use different approved raw material sources. The three consecutive cleavage PPQ batches are derived from three SPPS batches (SPPS PPQ1, PPQ2 and PPQ3). Batch-related governance items are defined within Asymchem PPQ protocol(s), which require Lilly API EM Quality approval prior to execution.

Figure 3: Batch tree of the PPQ batches manufacture

- Qualified Process Equipment

All the major equipment to be used in this process performance qualification campaign has completed IQ, OQ and PQ (see Table 5). The equipment selected is fit for process requirement, referring to equipment suitability assessment <RA-140020030101.01> Equipment selection has been assessed through technology transfer and scale-up activities and is considered suitable for commercial manufacture of ASYM-141366 (reference equipment suitability documentation: RA-140020030101.01).

All critical utilities supporting manufacture of PPQ batches have been qualified and are maintained in a qualified state through approved monitoring and maintenance programs.

Any modification to qualified equipment, automation systems, utilities or process control systems during PPQ execution shall be managed through the approved change control system and assessed for validation impact.

Table 5. Equipment Plan

| Step No | Equipment | Size/L | MOC* | Equipment ID | Location | Stirrer | Function in the process | Equipment Characteristics | MOC (All components in direct contact must be listed) | Specific Validation Protocol &Report No. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step 1 | Peptide Solid Phase Synthesizer | 3000 L | Hastelloy (C-276) | TJ4S-1213-Sy15 | Building 12 Module 13 | Propeller type (4-blade inclined paddle + S-type) | Synthesis process of peptide resin & Dry | Minimum volume of the stirrer can reach (at rest): 51.54 L; Minimum volume of the inner temperature probe (at rest): 26.91 L Control range of temperature of the equipment capability: -14~57 oC; Control range of temperature of the equipment capability: -4~53 oC The pressure range of equipment for normal operation: -0.08~0.0807 MPa The range of agitation speed of equipment capability: 5~40 rpm | Hastelloy C-276 (inner surface, nitrogen distribution piping, thermometer well material, Valve, Shower ball) Hastelloy N10276 ( agitator) Hastelloy HC-22 (Sieve plate) Glass (sight glass) 316L ( Pipeline material ) SS316 & PTFE( Transfer pump material, Waste discharge pump material ) PTFE ( Sealing ring, Gasket, including manway gaskets and gaskets on all inlet valve manifolds ) NRL: Gloves SS ( Mass flow meter) | SVP-TJ4-334.01-089 |
|  | Activation tank | 1500 L | Hastelloy (C-276) | TJ4S-1213-Sy15-R01 | Building 12 Module 13 | Double-layer four-bladed inclined paddle type | Amino acid activation | Minimum volume of the stirrer (at rest): 84.4 L The equipment adopts bottom temperature measurement, which can achieve the minimum temperature control volume required by the process and provides excellent temperature monitoring Equipment system design range: -6~54 oC Control range of temperature of the equipment capability: -14~58 oC (independent heat exchange system, ethylene glycol) The pressure range of equipment for normal operation: 0~0.0807 MPa The range of agitation speed of equipment capability, 15~130 rpm | Hastelloy C-276 ( inner surface, agitator, thermometer well material, valve, shower ball ) Glass ( Sight glass) 316L ( pipeline material, ) 316L+PTFE ( waste discharge pump material) PTFE ( Sealing ring, Peristaltic pump tubing) SS (Mass flow mete, metering pump and pipeline material ) | SVP-TJ4-334.01-089 |
|  | Agitation tank | 500 L | Hastelloy (C-276) | TJ4S-1200-AT02 | Building 12 Module 13 | Propeller type (three blades impeller) | Prepared Oxyma Pure /DMF solution | Stirring Minimum volume of equipment (at rest): 20.05 L The range of agitation speed of equipment capability: 0-119.84 rpm | Hastelloy C-276 ( inner surface, agitator ) SS (Valve, Thermometer probe, Transfer pump material, Pipeline material ) PTFE (gasket, including manway gaskets and gaskets on all inlet valve manifolds ) | SVP-TJ4-170.01-068 |
|  | Glass-line Reactor | 1000 L | Glass-lined | TJ4S-1213-R22 | Building 12 Module 13 | Impeller type (three blades impeller) | Prepared Ac2O/Pyridine /DMF solution | Minimum volume of the stirrer can reach at rest: 82.30 L; Minimum volume of the thermometer tube can reach at rest: 123.10 L; The range of agitation speed of equipment capability: 0~138 rpm (Rotation speed accuracy: ±2 rpm) Control range of temperature of the equipment capability: -2.9~112.3 oC The pressure range of equipment for normal operation: -0.098~0.083 MPa | Glass-lined ( Inner surface, temperature probe, agitator) SS&PTFE ( Valve) GL & PTFE & FFKM ( Mechanical seal) | SVP-TJ4-101.12-052 |
|  | Agitation tank | 3000 L | SS 304L (S30408) | TJ4S-1213-SPS01-AT01 | Building 12 Module 13 | Double-layer propeller type | Prepared piperidine/ DMF solution | Minimum volume of the stirrer can reach at rest: 308.7 L; Minimum volume of the stirrer can reach at maximum speed: 289.2 L. Maximum working volume: 3000 L Minimum volume of the thermometer tube can reach at rest: 416.9 L; Minimum volume of the thermometer tube can reach at maximum speed: 356.8 L. | 304 SS (inner surface, temperature probe, agitator, pipeline, transfer pump, mass flowmeter, valve) Glass (sight glass) PTFE (gasket, including manway gaskets and gaskets on all inlet valve manifolds) 304SS & FFKM (mechanical seal ) | SVP-TJ4-392.01-002 |
|  | Agitation tank | 3000 L | SS 304L (S30408) | TJ4S-1213-SPS01-AT02 | Building 12 Module 13 | Double-layer propeller type | Prepared piperidine/ DMF solution | Minimum volume of the stirrer can reach at rest: 309.3 L; Minimum volume of the stirrer can reach at maximum speed: 317.1 L. Maximum working volume: 3000 L Minimum volume of the thermometer tube can reach at rest: 376.1 L; Minimum volume of the thermometer tube can reach at maximum speed: 349.7 L | 304 SS (inner surface, temperature probe, agitator, pipeline, transfer pump, mass flowmeter, valve) Glass (sight glass) PTFE (gasket, including manway gaskets and gaskets on all inlet valve manifolds) 304SS & FFKM (mechanical seal ) | SVP-TJ4-392.01-002 |

Note: The gauges related to equipment are managed by electronic system CMS (Calibration Management System) to ensure the gauges are calibrated and valid during PPQ manufacturing.
 * MOC = Material of Construction   S30408 = 30408 Stainless Steel C-276 = Hastelloy C-276; 316L= 316L Stainless Steel

- Process Chemistry Overview

The process of Step 1 is based on the Lilly technical package and has been developed and finalized via risk assessment and the execution of designed experiments. The proven acceptable range (PAR) for all of the process parameters has been established <Refer to RPT- (Lilly document number: RPT-1190340.) -and form the basis of this process performance qualification. Only process parameters determined through process characterization and risk assessment to impact product quality or process performance, are considered within PPQ success criteria. Other operational parameters remain controlled through the approved MOR and are evaluated for potential impact should any deviation occur. The process flow chart is shown in Figure 4.

- Process Flow Chart

Figure 4. The Process Flow Chart of ASYM-141366

- Process description

The process description is summarized in Table 6 below.

Table 6. Process description for the process performance qualification

| No. | Unit operation name | Process Description |
| --- | --- | --- |
|  | Dispense & Solvent confirmation | Materials involved in dispensing: 2-CTC Resin; Fmoc-Gly-OH.; Fmoc-L-Phe-OH; Fmoc-L-N-Me-Asn(Trt)-OH; Fmoc-L-Asn(Trt)-OH; Fmoc-L-Ser(tBu)-OH; Fmoc-L-Arg(Pbf)-OH; Fmoc-L-Val-OH; Fmoc-L-Leu-OH; Fmoc-L-α-Me-Phe-OH; Fmoc-L-Glu(OtBu)-OH·H2O; Fmoc-L-Ala-OH·H2O; Fmoc-L-Orn(Boc)-OH; Oxyma Pure; PyBop; 20.00% piperidine/DMF (V/V) solution confirmation: The 20.00% v/v Piperidine/DMF solution is prepared and the bottom valve of tank is opened. Confirmed that the DMF supply system from tank farm was supplied. Confirmed that the IPAC supply system from tank farm was supplied. Confirmed that the MTBE supply system from tank farm was prepared. |
|  | Pre-treatment | Add DMF (5.50 V, 5.21 kg/kg) into activation tank (TJ4S-1213-Sy15-R01). Then transfer the solution into synthesis reactor (TJ4S-1213-Sy15). - Start the agitator, bubble with nitrogen, then stop the agitator and nitrogen, drain. - Sample the mixture for Water content analysis. If Water content ≤ 500ppm, proceed to next operation if Water content >500 ppm, repeat, until the Water content is ≤ 500ppm. Note: In all the following operations, unless otherwise noted. The agitation speed of synthesis reactor is set as 20 ± 10 rpm, the agitation speed of activation tank is set as 95 ± 10 rpm, and the nitrogen flow rate is set to 5 ± 2 m³/h. |
|  | Add 2-CTC resin & Fmoc-Gly-OH dissolve | - Add 2-CTC (48.00 kg, 1.00 eq) resin into TJ4S-1213-Sy15. - Add DMF (4.25 V, 4.03 kg/kg) into the TJ4S-1213-Sy15-R01. Start the agitator - Adjust the activation tank (TJ4S-1213-Sy15-R01) jacket temperature to 25 ± 5 °C (target: 25 °C). - Add Fmoc-Gly-OH (2.25 eq). Rinse reactor wall with DMF (1.0 V, 0.95 kg/kg). - Stir for at least 15 minutes, Confirm no visible solids and later use. |
|  | Resin swelling & Resin wash | - Rinse synthesis reactor (TJ4S-1213-Sy15) wall with DMF (7.0 V, 6.64 kg/kg). - Bubble with nitrogen, then start the agitator. - Rinse reactor wall with DMF (1.0 V, 0.95 kg/kg). - Adjust the mixture to 25 ± 5 °C (target: 25 °C). - After stirring for 35 ± 5 min (target: 35 min) at 25 ± 5 °C (target: 25 °C), turn off the nitrogen, stop stirring, and drain until there is no obvious dripping. Note: In all of the following operations, unless otherwise noted. The agitation speed of synthesis reactor is set as 20±10 rpm (target: 20 rpm), the agitation speed of activation tank is set as 95±10 rpm (target: 95 rpm) and the nitrogen flow rate is set to 5.0 ± 2 m³/h (target: 5 m³/h). |
|  | Wash resin | - Add DMF (5.0 V, 4.74 kg/kg) into synthesis reactor (TJ4S-1213-Sy15). - Bubble with nitrogen for 5 ± 2 min (target: 5 min), then start the agitator. - Stirring for 12 ± 2 min (target: 12 min), stop stirring, turn off the nitrogen. Drain until there is no obvious drip. |
|  | Fmoc-Gly-OH Activate and transfer | - Add DIPEA (6.00 eq) into the activation tank (TJ4S-1213-Sy15-R01) at 25 ± 5 °C (target: 25 °C). - Stir for 5 ± 2 min (target: 5 min) after addition, stop stirring. - Transfer the mixture in activation tank (TJ4S-1213-Sy15-R01) into synthesis reactor (TJ4S-1213-Sy15) - Add DMF (1.0 V, 0.95 kg/kg) into TJ4S-1213-Sy15-R01 to rinse the wall. |
| - 5 | Fmoc-Gly-OH coupling | - Bubble with nitrogen to the synthesis reactor (TJ4S-1213-Sy15), then start the agitator. - Transfer the mixture in activation tank (TJ4S-1213-Sy15-R01) into synthesis reactor (TJ4S-1213-Sy15) - Adjust temperature to 25 ± 5 °C. - The mixture is allowed to react for 3.5 h ± 15 min (target: 3.5 h) at 25 ± 5 °C (target: 25 °C). - Stop the agitator, turn off the nitrogen. Drain until there is no drip. Note: If resin adherence is found at other times during the reaction, rinse the reactor with 0.50 ~ 5.00 kg DMF each time. |
|  | Wash activation tank and wash resin | - Add DMF (8.0 V, 7.58 kg/kg) into the activation tank (TJ4S-1213-Sy15-R01). Circulate and clean the activating liquid transfer pipeline, solid charging port cleaning pipeline and liquid charging port cleaning pipeline. - After the liquid is filtered out of the synthesis reactor. Transfer the mixture in activation tank (TJ4S-1213-Sy15-R01) to synthesis reactor (TJ4S-1213-Sy15). - Bubble with nitrogen, after stirring, turn off the nitrogen. Drain until there is no obvious drip. |
|  | Resin Capping | - Add DMF (9.0 V, 8.53 kg/kg) into synthesis reactor (TJ4S-1213-Sy15). - Bubble with nitrogen, then start the agitator. - Turn off the nitrogen, manually weigh and add Methanol (10.00 eq.) with stainless steel hose and peristaltic pump. - Adjust to 20 ± 5 °C (target: 20 °C). Add DIPEA (5.00 eq.) at 20 ± 5 °C with stainless steel hose and peristaltic pump at 20 ± 5 °C (target: 20 °C). - Bubble with nitrogen after addition. Adjust to 25 ± 5 °C (target: 25 °C), the mixture is allowed to react for 1 h ± 15 min (target: 1 h) at 25 ± 5 °C (target: 25 °C). - Stop stirring and turn off the nitrogen. Drain until there is no reaction mixture drip. |
|  | Wash resin | - Add 9.0 V DMF (8.53 kg/kg) into the activation tank (TJ4S-1213-Sy15-R01). Circulate and clean the activating liquid transfer pipeline, solid charging port cleaning pipeline and liquid charging port cleaning pipeline. - After the liquid is filtered out of the synthesis reactor. Transfer the mixture in activation tank (TJ4S-1213-Sy15-R01) to synthesis reactor (TJ4S-1213-Sy15). - Bubble with nitrogen, after stirring, turn off the nitrogen. Drain until there is no obvious drip - Note: Perform this operation 3 times. |
|  | De-Fmoc | - Add 20 % piperidine in DMF (8.0 V, 7.45 kg/kg) into TJ4S-1213-Sy15. - Bubble with nitrogen, then start the agitator. - Adjust to 25 ± 5 °C (target: 25 °C) and stir for 110 -130 min (target: 2 h) at 25 ± 5 °C (target: 25 °C). - Stop stirring, turn off the nitrogen. Drain until there is no obvious drip. |
| - 1 | Wash resin | - Adjust the jacket of synthesis reactor TJ4S-1213-Sy15 to 20±5 oC (target: 20 °C). - Add Isopropyl Acetate (10.0 V, 8.73 kg/kg) into TJ4S-1213-Sy15. - Bubble with nitrogen, after stirring, turn off the nitrogen. Drain until there is no obvious drip - Perform this operation 4 times. |
| - 1 | Re-swelling | - Add DMF (12.0 V, 11.38 kg/kg) into TJ4S-1213-Sy15. - Bubble with nitrogen, after stirring, turn off the nitrogen. Drain until there is no obvious drip. - Perform this operation 2 times. |
|  | Fmoc-L-Phe-OH activation and transfer | - Add DMF (4.0 V, 3.79 kg/kg) into TJ4S-1213-Sy15-R01. Start the agitator. Adjust the jacket temperature to 20±5 °C. - Add Fmoc-L-Phe-OH (2.00 eq.). Stir at least 15minutes at 20 ± 5 °C (target: 20 °C).. - Cool the mixture to 5 ± 5 °C. Maintain for next operation at 5 ± 5 °C (target: 5 °C). - Add PyBOP (2.00 eq.) and OxymaPure (2.00 eq.) into TJ4S-1213-Sy15-R01, rinse the TJ4S-1213-Sy15-R01 with DMF (1.0 V, 0.95 kg/kg). - Add DIPEA (3.00 eq.) into TJ4S-1213-Sy15-R01, the addition rate depends on temperature control. Activate for 20 ± 5 min at 5 ± 5 °C (target: 5 °C). - Add DMF (2.0 V, 1.89 kg/kg) into TJ4S-1213-Sy15, Start the agitator. - Transfer the mixture in TJ4S-1213-Sy15-R01 into TJ4S-1213-Sy15. Add DMF (1.0 V, 0.95 kg/kg) into TJ4S-1213-Sy15-R01 to rinse the wall. - Bubble with nitrogen to the synthesis reactor (TJ4S-1213-Sy15) for 5 ± 2 min (target: 5 min), then start the agitator. Transfer the mixture in activation tank (TJ4S-1213-Sy15-R01) into synthesis reactor (TJ4S-1213-Sy15) |
|  | Coupling | - Heat the mixture to 19 ~ 28 °C (target: 25 °C).. Start timing when temperature reaches 20 °C. The mixture is allowed to react at 19 ~ 28 °C (target: 25 °C). - After 4 h±15 min (target: 4 h), sample the mixture every 0.5~1 h for ninhydrin test analysis until two consecutive samples test as negative (Yellow to Light pink Report Negative= Complete coupling; Blue to Purple Positive= Incomplete coupling). The mixture should be stirred for coupling time (6.5 h ≤Coupling time ≤13.5 h). - Stop stirring and turn off the nitrogen. Drain until there is no reaction mixture drip. Note: If resin adherence is found at other times during the reaction, rinse the reactor with 0.50~5.00 kg DMF each time. |
|  | Wash resin | - Add DMF (8.0 V, 7.58 kg/kg) into the activation tank (TJ4S-1213-Sy15-R01). Circulate and clean the activating liquid transfer pipeline, solid charging port cleaning pipeline and liquid charging port cleaning pipeline. - After the liquid is filtered out of the synthesis reactor. Transfer the mixture in activation tank (TJ4S-1213-Sy15-R01) to synthesis reactor (TJ4S-1213-Sy15). - Bubble with nitrogen, after stirring, turn off the nitrogen. Drain until there is no obvious drip Note: Perform this operation 3 times. |
|  | De-Fmoc | - Add 20 % piperidine in DMF (8.0 V, 7.45 kg/kg) into TJ4S-1213-Sy15. - Bubble with nitrogen for 5 ± 2 min, then start the agitator. - Adjust to 25 ± 5 °C (target: 25 °C) and stir for 110 -130 min (target: 2 h) at 25 ± 5 °C (target: 25 °C). - Stop stirring, turn off the nitrogen. Drain until there is no obvious drip. |
|  | Wash resin | - Adjust the jacket of synthesis reactor TJ4S-1213-Sy15 to 20±5 °C (target: 20 °C). - Add Isopropyl Acetate (10.0 V, 8.73 kg/kg) into TJ4S-1213-Sy15. . - Bubble with nitrogen, after stirring, turn off the nitrogen. Drain until there is no obvious drip - Perform this operation 4 times. |
|  | Re-swelling | - Add DMF (12.0 V, 11.38 kg/kg)into TJ4S-1213-Sy15. - Bubble with nitrogen, after stirring, turn off the nitrogen. Drain until there is no obvious drip. Perform this operation 2 times. |
|  | Fmoc-L-N-Me-Asn(Trt)-OH activation and transfer | - Add DMF (4 V, 3.79 kg/kg) into TJ4S-1213-Sy15-R01. Start the agitator. - Adjust the jacket temperature to 20 ± 5 °C (target: 20 °C). - Add Fmoc-L-N-Me-Asn(Trt)-OH (1.75 eq). Stir at least 15 minutes at 20 ± 5 °C (target: 20 °C), Confirm no visible solids and later use. - Add OxymaPure (1.80 eq) into TJ4S-1213-Sy15-R01, rinse the TJ4S-1213-Sy15-R01 with DMF (2.0 V, 1.90 kg/kg). - Add DIC (2.00 eq.) at 20 ± 5 °C (target: 20 °C). Stir for 20 ± 5 min (target: 20 min) after addition at 20 ± 5 °C (target: 20 °C). - Transfer the mixture in TJ4S-1213-Sy15-R01 into TJ4S-1213-Sy15. Add DMF (1.0 V, 0.95 kg/kg) into TJ4S-1213-Sy15-R01 to rinse the wall. - Bubble with nitrogen to the synthesis reactor (TJ4S-1213-Sy15) for 5 ± 2 min (target: 5 min), then start the agitator. Transfer the mixture in activation tank (TJ4S-1213-Sy15-R01) into synthesis reactor (TJ4S-1213-Sy15) |
|  | Coupling | - Heat the mixture to 19 ~ 27 °C (target: 25 °C). The mixture is allowed to react at 19 ~ 27 °C (target: 25 °C). - After 4 h ± 15 min (target: 4 h), sample the mixture every 0.5 ~ 1 h for ninhydrin test analysis until two consecutive samples test as negative (Yellow to Light pink Report Negative= Complete coupling; Blue to Purple Positive= Incomplete coupling, 6.5 h ≤ Coupling time ≤ 13.5 h). - Stop stirring and turn off the nitrogen. Drain until there is no reaction mixture drip. |
|  | Wash resin | - Add DMF (8.0 V, 7.58 kg/kg) into the activation tank (TJ4S-1213-Sy15-R01). Circulate and clean the activating liquid transfer pipeline, solid charging port cleaning pipeline and liquid charging port cleaning pipeline. - After the liquid is filtered out of the synthesis reactor. Transfer the mixture in activation tank (TJ4S-1213-Sy15-R01) to synthesis reactor (TJ4S-1213-Sy15). - Bubble with nitrogen, after stirring, turn off the nitrogen. Drain until there is no obvious drip Note: Perform this operation 3 times. |
|  | De-Fmoc | - Add 20 % piperidine in DMF (8.0 V, 7.45 kg/kg) into TJ4S-1213-Sy15. - Bubble with nitrogen for 5 ± 2 min, then start the agitator. - Adjust to 25 ± 5 °C (target: 25 °C) and stir for 110 -130 min (target: 2 h) at 25 ± 5 °C (target: 25 °C). - Stop stirring, turn off the nitrogen. Drain until there is no obvious drip. |
|  | Wash resin | - Adjust the jacket of synthesis reactor TJ4S-1213-Sy15 to 20±5 °C (target: 20 °C). - Add Isopropyl Acetate (10.0 V, 8.73 kg/kg) into TJ4S-1213-Sy15 . - Bubble with nitrogen, after stirring, turn off the nitrogen. Drain until there is no obvious drip Perform this operation 4 times. |
|  | Re-swelling | - Add DMF (12.0 V, 11.38 kg/kg) into TJ4S-1213-Sy15. - Bubble with nitrogen, after stirring, turn off the nitrogen. Drain until there is no obvious drip Perform this operation 2 times. |
|  | Fmoc-L-Asn(Trt)-OH activation and transfer | - Add DMF (4.0 V, 3.79 kg/kg) into TJ4S-1213-Sy15-R01. Start the agitator. - Adjust the jacket temperature to 20 ± 5 °C (target: 20 °C). - Add Fmoc-L-Asn(Trt)-OH (2.50 eq). Stir at least 15 minutes at 20 ± 5 °C (target: 20 °C)., Confirm no visible solids and later use - Add Oxyma Pure (2.60 eq) into TJ4S-1213-Sy15-R01 at 20 ± 5 °C (target: 20 °C), rinse the TJ4S-1213-Sy15-R01 with DMF (2.0 V, 1.90 kg/kg). - Add DIC (2.90 eq) at 20±5 °C (target: 20 °C). The addition rate depends on temperature control. Stir for 20±5 min (target: 20 min) after addition at 20±5 °C (target: 20 °C). - Transfer the mixture in TJ4S-1213-Sy15-R01 into TJ4S-1213-Sy15. Add DMF (1.0 V, 0.95 kg/kg) into TJ4S-1213-Sy15-R01 to rinse the wall. - Bubble with nitrogen to the synthesis reactor (TJ4S-1213-Sy15) for 5 ± 2 min (target: 5 min), then start the agitator. Transfer the mixture in activation tank (TJ4S-1213-Sy15-R01) into synthesis reactor (TJ4S-1213-Sy15) |
|  | Fmoc-L-Asn(Trt)-OH coupling | - Heat the mixture to 19 ~ 28 °C (target: 25 °C) The mixture is allowed to react at 19 ~ 28 °C (target: 25 °C) - Once the total reaction time has reached 8 h ± 15 min, add additional OxymaPure (1.00 eq) and DMF (1.0 V, 0.95 kg/kg) solution (the mixture should be prepared in TJ4S-1200-AT02 just before use) with dedicated process lines and diaphragm pump and DIC (1.10 eq) with PTFE and peristaltic pump. After 5 h ± 15 min (target: 5 h), the mixture should be stirred for total time (13.5 h ≤ total time ≤ 27.5 h). - Stop stirring, turn off the nitrogen. Drain until there is no reaction mixture drip. Note: If resin adherence is found at other times during the reaction, rinse the reactor with 0.50~5.00 kg DMF each time. |
|  | Wash resin | - Add DMF (8.0 V, 7.58 kg/kg) into the activation tank (TJ4S-1213-Sy15-R01). Circulate and clean the activating liquid transfer pipeline, solid charging port cleaning pipeline and liquid charging port cleaning pipeline. - After the liquid is filtered out of the synthesis reactor. Transfer the mixture in activation tank (TJ4S-1213-Sy15-R01) to synthesis reactor (TJ4S-1213-Sy15). - Bubble with nitrogen, after stirring, turn off the nitrogen. Drain until there is no obvious drip Note: Perform this operation 3 times. |
|  | Prepare capping solution | - Add DMF (6.0 V, 5.69 kg/kg) into TJ4S-1213- R22. Start the agitator. - Add Pyridine (1.50 eq) with PTFE and peristaltic pump and Acetic Anhydride (1.25 eq) with PTFE and peristaltic pump. - Adjust the mixture to 25 ± 5 °C (target: 25 °C). Maintain for next operation at 25±5 °C (target: 25 °C). Stir for at least 5 minutes after addition, - Transfer the mixture in TJ4S-1213- R22 into TJ4S-1213-Sy15. Add DMF (1.0 V, 0.95 kg/kg) into TJ4S-1213- R22 to rinse the wall. - Bubble with nitrogen to the synthesis reactor (TJ4S-1213-Sy15) for 5 ± 2 min (target: 5 min), then start the agitator. Transfer the mixture in activation tank TJ4S-1213- R22 into synthesis reactor (TJ4S-1213-Sy15) |
|  | Capping | - Adjust the mixture in synthesis reactor (TJ4S-1213-Sy15) to 25 ± 5 °C (target: 25 °C). - The mixture is allowed to react at 25 ±5 °C (target: 25°C) for 1 h ± 15 min (target: 1 h). - Stop stirring and turn off the nitrogen. Drain until there is no reaction mixture drip. |
|  | Wash resin | - After the liquid is filtered out of the synthesis reactor. Add DMF (8.0 V, 7.58 kg/kg) into the synthesis reactor (TJ4S-1213-Sy15). - Bubble with nitrogen, after stirring, turn off the nitrogen. Drain until there is no obvious drip Note: Perform this operation 3 times. |
|  | De-Fmoc | - Add 20% piperidine in DMF (8.0 V, 7.45 kg/kg) into the TJ4S-1213-Sy15. - Bubble with nitrogen for 3 ± 2 min (target: 3 min), then start the agitator. Adjust to 25 ± 5 °C (target: 25 °C). After stirring for 11 - 36 min (target: 15 min) at 25 ± 5 °C (target: 25 °C). - Stop stirring, turn off the nitrogen. Drain until there is no obvious drip. - Add 20% piperidine in DMF (10.4 V, 9.68 kg) into TJ4S-1213-Sy15. - Bubble with nitrogen for 3 ± 2 min (target: 3 min), then start the agitator. After stirring for 20 - 51 min (target: 30 min) at 25 ± 5 °C (target: 25 °C). - Stop stirring, turn off the nitrogen. Drain until there is no obvious drip. |
|  | Wash resin | - Adjust the jacket of synthesis reactor TJ4S-1213-Sy15 to 20±5 °C (target: 20 °C). - Add Isopropyl Acetate (10.0 V, 8.73 kg/kg) into TJ4S-1213-Sy15. - Bubble with nitrogen, after stirring, turn off the nitrogen. Drain until there is no obvious drip Perform this operation 4 times. |
|  | Re-swelling | - Add DMF (13.0 V, 12.32 kg/kg) into TJ4S-1213-Sy15. - Bubble with nitrogen, after stirring, turn off the nitrogen. Drain until there is no obvious drip Perform this operation 2 times. |
|  | Fmoc-L-Ser(tBu)-OH activation and transfer | - Add DMF (5.0 V, 4.74 kg/kg) into TJ4S-1213-Sy15-R01. Start the agitator. - Adjust the jacket temperature to 20 ± 5 °C (target: 20 °C). - Add Fmoc-L-Ser(tBu)-OH (2.00 eq.). Stir at least 15 minutes at 20 ± 5 °C (target: 20 °C). Confirm no visible solids and later use. - Add OxymaPure (2.00 eq.) into TJ4S-1213-Sy15-R01), rinse the TJ4S-1213-Sy15-R01 with DMF (2.0 V, 1.90 kg/kg). - Add DIC (2.20 eq.) into TJ4S-1213-Sy15-R01 at 20 ± 5 °C (target: 20 °C, activate for 20±5 min (target: 20 min) at 20 ± 5 °C (target: 20 °C). - Transfer the mixture in TJ4S-1213-Sy15-R01 into TJ4S-1213-Sy15. Add DMF (1.0 V，0.94 kg/kg) into TJ4S-1213-Sy15-R01 to rinse the wall. - Bubble with nitrogen to the synthesis reactor (TJ4S-1213-Sy15) for 5 ± 2 min (target: 5 min), then start the agitator. Transfer the mixture in activation tank (TJ4S-1213-Sy15-R01) into synthesis reactor (TJ4S-1213-Sy15) |
|  | Fmoc-L-Ser(tBu)-OH Coupling | - Heat the mixture to 20 - 27 °C (target: 25 °C). The mixture is allowed to react at 20 - 27 °C (target: 25 °C). - After 3 h±15 min (target: 3 h), sample the mixture every 0.5~1 h for ninhydrin test analysis until two consecutive samples test as negative (colorless, react completely). And the mixture should be stirred for coupling time (4.5 h ≤ Coupling time ≤ 9.5 h). - Stop stirring and turn off the nitrogen. Drain until there is no reaction mixture drip. |
|  | Wash resin | - Add DMF (8.0 V, 7.58 kg/kg) into the activation tank (TJ4S-1213-Sy15-R01). Circulate and clean the activating liquid transfer pipeline, solid charging port cleaning pipeline and liquid charging port cleaning pipeline. - After the liquid is filtered out of the synthesis reactor. Transfer the mixture in activation tank (TJ4S-1213-Sy15-R01) to synthesis reactor (TJ4S-1213-Sy15). - Bubble with nitrogen, after stirring, turn off the nitrogen. Drain until there is no obvious drip Note: Perform this operation 3 times. |
|  | De-Fmoc | - Add 20 % piperidine in DMF (8.0 V, 7.45 kg/kg) into TJ4S-1213-Sy15. - Bubble with nitrogen for 5 ± 2 min, then start the agitator. - Adjust to 25 ± 5 °C (target: 25 °C) and stir for 110 -130 min (target: 2 h) at 25 ± 5 °C (target: 25 °C). - Stop stirring, turn off the nitrogen. Drain until there is no obvious drip. |
|  | Wash resin | - Adjust the jacket of synthesis reactor TJ4S-1213-Sy15 to 20±5 °C (target: 20 °C). - Add Isopropyl Acetate (10.0 V, 8.73 kg/kg)) into TJ4S-1213-Sy15. - Bubble with nitrogen, after stirring, turn off the nitrogen. Drain until there is no obvious drip Perform this operation 4 times. |
|  | Re-swelling | - Add DMF (12.0 V, 11.38 kg/kg) into TJ4S-1213-Sy15. - Bubble with nitrogen, after stirring, turn off the nitrogen. Drain until there is no obvious drip Perform this operation 2 times. |
|  | Fmoc-L-Ser(tBu)-OH activation and transfer | - Add DMF (5.0 V, 4.74 kg/kg) into TJ4S-1213-Sy15-R01. Start the agitator. - Adjust the jacket temperature to 20 ± 5 °C (target: 20 °C). - Add Fmoc-L-Ser(tBu)-OH (2.00 eq.). Stir at least 15 minutes at 20 ± 5 °C (target: 20 °C), Confirm no visible solids and later use. - Add OxymaPure (2.00 eq.) into TJ4S-1213-Sy15-R01), rinse the TJ4S-1213-Sy15-R01 with DMF (2.0 V, 1.90 kg/kg). - Add DIC (2.20 eq.) into TJ4S-1213-Sy15-R01 at 20 ± 5 °C (target: 20 °C, activate for 20±5 min (target: 20 min) at 20 ± 5 °C (target: 20 °C). - Transfer the mixture in TJ4S-1213-Sy15-R01 into TJ4S-1213-Sy15. Add DMF (1.0 V，0.94 kg/kg) into TJ4S-1213-Sy15-R01 to rinse the wall. - Bubble with nitrogen to the synthesis reactor (TJ4S-1213-Sy15) for 5 ± 2 min (target: 5 min), then start the agitator. Transfer the mixture in activation tank (TJ4S-1213-Sy15-R01) into synthesis reactor (TJ4S-1213-Sy15) |
|  | Fmoc-L-Ser(tBu)-OH Coupling | - Heat the mixture to 20 - 27 °C (target: 25 °C). The mixture is allowed to react at 20 - 27 °C (target: 25 °C). - After 3 h±15 min (target: 3 h), sample the mixture every 0.5~1 h for ninhydrin test analysis until two consecutive samples test as negative (Yellow to Light pink Report Negative= Complete coupling; Blue to Purple Positive= Incomplete coupling). And the mixture should be stirred for coupling time (4.5 h ≤ Coupling time ≤ 9.5 h). - Stop stirring and turn off the nitrogen. Drain until there is no reaction mixture drip. Note: If resin adherence is found at other times during the reaction, rinse the reactor with 0.50~5.00 kg DMF each time. |
|  | Wash resin | - Add DMF (8.0 V, 7.58 kg/kg) into the activation tank (TJ4S-1213-Sy15-R01). Circulate and clean the activating liquid transfer pipeline, solid charging port cleaning pipeline and liquid charging port cleaning pipeline. - After the liquid is filtered out of the synthesis reactor. Transfer the mixture in activation tank (TJ4S-1213-Sy15-R01) to synthesis reactor (TJ4S-1213-Sy15). - Bubble with nitrogen, after stirring, turn off the nitrogen. Drain until there is no obvious drip Note: Perform this operation 3 times. |
|  | De-Fmoc | - Add 20 % piperidine in DMF (8.0 V, 7.45 kg/kg) into TJ4S-1213-Sy15. - Bubble with nitrogen for 5 ± 2 min, then start the agitator. - Adjust to 25 ± 5 °C (target: 25 °C). And stir for 110 -130 min (target: 2 h) at 25 ± 5 °C (target: 25 °C). Stop stirring, turn off the nitrogen. Drain until there is no obvious drip. |
|  | Wash resin | - Adjust the jacket of synthesis reactor TJ4S-1213-Sy15 to 20±5 °C (target: 20 °C). - Add Isopropyl Acetate (10.0 V, 8.73 kg/kg) into TJ4S-1213-Sy15 . - Bubble with nitrogen, after stirring, turn off the nitrogen. Drain until there is no obvious drip Perform this operation 4 times. |
|  | Re-swelling | - Add DMF (12.0 V, 11.38 kg/kg) into TJ4S-1213-Sy15. - Bubble with nitrogen, after stirring, turn off the nitrogen. Drain until there is no obvious drip Perform this operation 2 times. |
|  | Fmoc-L-Arg(Pbf)-OH activation and transfer | - Add DMF (5.0 V, 4.74 kg/kg) into TJ4S-1213-Sy15-R01. Start the agitator. - Adjust the jacket temperature to 20 ± 5 °C (target: 20 °C). - Add Fmoc-L-Arg(Pbf)-OH (2.00 eq). Stir at least 15minutes at 20 ± 5 °C (target: 20 °C)., Confirm no visible solids and later use. - Add OxymaPure (2.00 eq.) into TJ4S-1213-Sy15-R01 at 20 ± 5 °C (target: 20 °C), rinse the TJ4S-1213-Sy15-R01 with DMF (2.0 V, 1.90 kg/kg) - Add DIC (2.20 eq) into TJ4S-1213-Sy15-R01. Activate for 20±5 min (target: 20 min) at 20 ± 5 °C (target: 20 °C). - Transfer the mixture in TJ4S-1213-Sy15-R01 into TJ4S-1213-Sy15. Add DMF (1.0 V, 0.94 kg/kg) into TJ4S-1213-Sy15-R01 to rinse the wall. - Bubble with nitrogen to the synthesis reactor (TJ4S-1213-Sy15) for 5 ± 2 min (target: 5 min), then start the agitator. Transfer the mixture in activation tank (TJ4S-1213-Sy15-R01) into synthesis reactor (TJ4S-1213-Sy15) |
|  | Coupling | - Heat the mixture to 20 - 27 °C (target: 25 °C). The mixture is allowed to react at 20 - 27 °C (target: 25 °C). - After 3 h±15 min (target: 3 h), sample the mixture every 0.5~1 h for ninhydrin test analysis until two consecutive samples test as negative (Yellow to Light pink Report Negative= Complete coupling; Blue to Purple Positive= Incomplete coupling). And the mixture should be stirred for coupling time (4.5 h ≤ Coupling time ≤ 9.5 h). - Stop stirring and turn off the nitrogen. Drain until there is no reaction mixture drip. Note: If resin adherence is found at other times during the reaction, rinse the reactor with 0.50~5.00 kg DMF each time. |
|  | Wash resin | - Add DMF (8.0 V, 7.58 kg/kg) into the activation tank (TJ4S-1213-Sy15-R01). Circulate and clean the activating liquid transfer pipeline, solid charging port cleaning pipeline and liquid charging port cleaning pipeline. - After the liquid is filtered out of the synthesis reactor. Transfer the mixture in activation tank (TJ4S-1213-Sy15-R01) to synthesis reactor (TJ4S-1213-Sy15). - Bubble with nitrogen, after stirring, turn off the nitrogen. Drain until there is no obvious drip Note: Perform this operation 3 times. |
|  | De-Fmoc | - Add 20 % piperidine in DMF (8.0 V, 7.45 kg/kg) into TJ4S-1213-Sy15. - Bubble with nitrogen for 5 ± 2 min, then start the agitator. - Adjust to 25 ± 5 °C (target: 25 °C). And stir for 110 -130 min (target: 2 h) at 25 ± 5 °C (target: 25 °C). - Stop stirring, turn off the nitrogen. Drain until there is no obvious drip. |
|  | Wash resin | - Adjust the jacket of synthesis reactor TJ4S-1213-Sy15 to 20±5 °C (target: 20 °C). - Add Isopropyl Acetate (10.0 V, 8.73 kg/kg) into TJ4S-1213-Sy15. - Bubble with nitrogen, after stirring, turn off the nitrogen. Drain until there is no obvious drip Perform this operation 4 times. |
|  | Re-swelling | - Add DMF (10.0 V, 9.48 kg/kg) into TJ4S-1213-Sy15. - Bubble with nitrogen, after stirring, turn off the nitrogen. Drain until there is no obvious drip Perform this operation 2 times. |
|  | Fmoc-L-Val-OH activation and transfer | - Add DMF (5.0 V, 4.74 kg/kg) into TJ4S-1213-Sy15-R01. Start the agitator. - Adjust the jacket temperature to 20 ± 5 °C (target: 20 °C). - Add Fmoc-L-Val-OH (2.00 eq). Stir at least 15minutes at 20 ± 5 °C (target: 20 °C)., Confirm no visible solids and later use. - Add OxymaPure (2.00 eq.) into TJ4S-1213-Sy15-R01), rinse the TJ4S-1213-Sy15-R01 with DMF (2.0 V, 1.90 kg/kg). - Add DIC (2.20 eq.) into TJ4S-1213-Sy15-R01 at 20 ± 5 °C (target: 20 °C, activate for 20±5 min (target: 20 min) at 20 ± 5 °C (target: 20 °C). - Transfer the mixture in TJ4S-1213-Sy15-R01 into TJ4S-1213-Sy15. Add DMF (1.0 V，0.94 kg/kg) into TJ4S-1213-Sy15-R01 to rinse the wall. - Bubble with nitrogen to the synthesis reactor (TJ4S-1213-Sy15) for 5 ± 2 min (target: 5 min), then start the agitator. Transfer the mixture in activation tank (TJ4S-1213-Sy15-R01) into synthesis reactor (TJ4S-1213-Sy15) |
|  | Coupling | - Heat the mixture to 20 - 27 °C (target: 25 °C). The mixture is allowed to react at 20 - 27 °C (target: 25 °C). - After 3 h±15 min (target: 3 h), sample the mixture every 0.5~1 h for ninhydrin test analysis until two consecutive samples test as negative (Yellow to Light pink Report Negative= Complete coupling; Blue to Purple Positive= Incomplete coupling. And the mixture should be stirred for coupling time (4.5 h ≤ Coupling time ≤ 9.5 h).. - Stop stirring and turn off the nitrogen. Drain until there is no reaction mixture drip. Note: If resin adherence is found at other times during the reaction, rinse the reactor with 0.50~5.00 kg DMF each time. |
|  | Wash resin | - Add DMF (8.0 V, 7.58 kg/kg) into the activation tank (TJ4S-1213-Sy15-R01). Circulate and clean the activating liquid transfer pipeline, solid charging port cleaning pipeline and liquid charging port cleaning pipeline. - After the liquid is filtered out of the synthesis reactor. Transfer the mixture in activation tank (TJ4S-1213-Sy15-R01) to synthesis reactor (TJ4S-1213-Sy15). - Bubble with nitrogen, after stirring, turn off the nitrogen. Drain until there is no obvious drip Note: Perform this operation 3 times. |
|  | De-Fmoc | - Add 20 % piperidine in DMF (8.0 V, 7.45 kg/kg) into TJ4S-1213-Sy15. - Bubble with nitrogen for 5 ± 2 min, then start the agitator. - Adjust to 25 ± 5 °C (target: 25 °C), and stir for 110 -130 min (target: 2 h) at 25 ± 5 °C (target: 25 °C). Stop stirring, turn off the nitrogen. Drain until there is no obvious drip. |
|  | Wash resin | - Adjust the jacket of synthesis reactor TJ4S-1213-Sy15 to 20±5 °C (target: 20 °C). - Add Isopropyl Acetate (10.0 V, 8.73 kg/kg) into TJ4S-1213-Sy15. - Bubble with nitrogen, after stirring, turn off the nitrogen. Drain until there is no obvious drip Perform this operation 4 times. |
|  | Re-swelling | - Add DMF (12.0 V, 11.38 kg/kg) into TJ4S-1213-Sy15. - Bubble with nitrogen, after stirring, turn off the nitrogen. Drain until there is no obvious drip. Perform this operation 2 times. |
|  | Fmoc-L-Leu-OH activation and transfer | - Add DMF (5.0 V, 4.74 kg/kg) into TJ4S-1213-Sy15-R01. Start the agitator. - Adjust the jacket temperature to 20 ± 5 °C (target: 20 °C). - Add Fmoc-L-Leu-OH (2.00 eq). Stir at least 15minutes at 20 ± 5 °C (target: 20 °C)., Confirm no visible solids and later use. - Add OxymaPure (2.00 eq.) into TJ4S-1213-Sy15-R01), rinse the TJ4S-1213-Sy15-R01 with DMF (2.0 V, 1.90 kg/kg).. - Add DIC (2.20 eq.) into TJ4S-1213-Sy15-R01 at 20 ± 5 °C (target: 20 °C, activate for 20±5 min (target: 20 min) at 20 ± 5 °C (target: 20 °C). - Transfer the mixture in TJ4S-1213-Sy15-R01 into TJ4S-1213-Sy15. Add DMF (1.0 V，0.94 kg/kg) into TJ4S-1213-Sy15-R01 to rinse the wall. - Bubble with nitrogen to the synthesis reactor (TJ4S-1213-Sy15) for 5 ± 2 min (target: 5 min), then start the agitator. Transfer the mixture in activation tank (TJ4S-1213-Sy15-R01) into synthesis reactor (TJ4S-1213-Sy15) |
|  | Coupling | - Heat the mixture to 20 - 27 °C (target: 25 °C). The mixture is allowed to react at 20 - 27 °C (target: 25 °C). - After 3 h±15 min (target: 3 h), sample the mixture every 0.5~1 h for ninhydrin test analysis until two consecutive samples test as negative (Yellow to Light pink Report Negative= Complete coupling; Blue to Purple Positive= Incomplete coupling). And the mixture should be stirred for coupling time (4.5 h ≤ Coupling time ≤ 9.5 h). - Stop stirring and turn off the nitrogen. Drain until there is no reaction mixture drip. Note: If resin adherence is found at other times during the reaction, rinse the reactor with 0.50~5.00 kg DMF each time. |
|  | Wash resin | - Add DMF (8.0 V, 7.58 kg/kg) into the activation tank (TJ4S-1213-Sy15-R01). Circulate and clean the activating liquid transfer pipeline, solid charging port cleaning pipeline and liquid charging port cleaning pipeline. - After the liquid is filtered out of the synthesis reactor. Transfer the mixture in activation tank (TJ4S-1213-Sy15-R01) to synthesis reactor (TJ4S-1213-Sy15). - Bubble with nitrogen, after stirring, turn off the nitrogen. Drain until there is no obvious drip Note: Perform this operation 3 times. |
|  | De-Fmoc | - Add 20 % piperidine in DMF (8.0 V, 7.45 kg/kg) into TJ4S-1213-Sy15. - Bubble with nitrogen for 5 ± 2 min, then start the agitator. - Adjust to 25 ± 5 °C (target: 25 °C), stir for 110 -130 min (target: 2 h) at 25 ± 5 °C (target: 25 °C). - Stop stirring, turn off the nitrogen. Drain until there is no obvious drip. |
|  | Wash resin | - Adjust the jacket of synthesis reactor TJ4S-1213-Sy15 to 20±5 °C (target: 20 °C). - Add Isopropyl Acetate (10.0 V, 8.73 kg/kg) into TJ4S-1213-Sy15. - Bubble with nitrogen, after stirring, turn off the nitrogen. Drain until there is no obvious drip Perform this operation 4 times. |
|  | Re-swelling | - Add DMF (12.0 V, 11.38 kg/kg) into TJ4S-1213-Sy15. - Bubble with nitrogen, then start the agitator. Stirring and turn off the nitrogen. Drain until there is no obvious drip. Perform this operation 2 times. |
|  | Fmoc-L-α-Me-Phe-OH activation and transfer | - Add DMF (4 V, 3.79 kg/kg) into TJ4S-1213-Sy15-R01. Start the agitator. - Adjust the jacket temperature to 20 ± 5 °C (target: 20 °C). - Add Fmoc-L-α-Me-Phe-OH (1.75 eq). Stir at least 15 minutes at 20 ± 5 °C (target: 20 °C). Confirm no visible solids and later use. - Add OxymaPure (1.80 eq) into TJ4S-1213-Sy15-R01 at 20 ± 5 °C (target: 20 °C), rinse the TJ4S-1213-Sy15-R01 with DMF (2.0 V, 1.90 kg/kg). - Add DIC (2.00 eq) into TJ4S-1213-Sy15-R01, the addition rate depends on temperature control. Activate for 20 ± 5 min (target: 20 min) at 20 ± 5 °C (target: 20 °C). - Transfer the mixture in TJ4S-1213-Sy15-R01 into TJ4S-1213-Sy15. Add DMF (1.0 V, 0.95 kg/kg) into TJ4S-1213-Sy15-R01 to rinse the wall. - Bubble with nitrogen to the synthesis reactor (TJ4S-1213-Sy15) for 5 ± 2 min (target: 5min), then start the agitator. Transfer the mixture in activation tank (TJ4S-1213-Sy15-R01) into synthesis reactor (TJ4S-1213-Sy15) |
|  | Coupling | - Heat the mixture to 19 ~ 27 °C (target: 25 °C). The mixture is allowed to react at 19 ~27 °C (target: 25 °C). - After 4 h±15 min (target: 4 h), sample the mixture every 0.5~1 h for ninhydrin test analysis until two consecutive samples test as negative (Yellow to Light pink Report Negative= Complete coupling; Blue to Purple Positive= Incomplete coupling). And the mixture should be stirred for coupling time (6.5 h ≤ Coupling time ≤13.5 h). - Stop stirring and turn off the nitrogen. Drain until there is no reaction mixture drip. Note: If resin adherence is found at other times during the reaction, rinse the reactor with 0.50~5.00 kg DMF each time. |
|  | Wash resin | - Add DMF (8.0 V, 7.58 kg/kg) into the activation tank (TJ4S-1213-Sy15-R01). Circulate and clean the activating liquid transfer pipeline, solid charging port cleaning pipeline and liquid charging port cleaning pipeline. - After the liquid is filtered out of the synthesis reactor. Transfer the mixture in activation tank (TJ4S-1213-Sy15-R01) to synthesis reactor (TJ4S-1213-Sy15). - Bubble with nitrogen, after stirring, turn off the nitrogen. Drain until there is no obvious drip Note: Perform this operation 3 times. |
|  | De-Fmoc | - Add 20 % piperidine in DMF (8.0 V, 7.45 kg/kg) into TJ4S-1213-Sy15 - Bubble with nitrogen for 5 ± 2 min, then start the agitator. - Adjust to 25 ± 5 °C (target: 25 °C), and stir for 110 -130 min (target: 2 h) at 25 ± 5 °C (target: 25 °C). - Stop stirring, turn off the nitrogen. Drain until there is no obvious drip. |
|  | Wash resin | - Adjust the jacket of synthesis reactor TJ4S-1213-Sy15 to 20±5 oC (target: 20 oC). - Add Isopropyl Acetate (10.0 V, 8.73 kg/kg) into TJ4S-1213-Sy15. - Bubble with nitrogen, after stirring, turn off the nitrogen. Drain until there is no obvious drip. Perform this operation 4 times. |
|  | Re-swelling | - Add DMF (12.0 V, 11.38 kg/kg) into TJ4S-1213-Sy15. - Bubble with nitrogen, after stirring, turn off the nitrogen. Drain until there is no obvious drip Perform this operation 2 times. |
|  | Fmoc-L-Glu(OtBu)-OH·H2O activation and transfer | - Add DMF (5.0 V, 4.74 kg/kg) into TJ4S-1213-Sy15-R01. Start the agitator. - Adjust the jacket temperature to 20 ± 5 °C (target: 20 °C). - Add Fmoc-L-Glu(OtBu)-OH·H2O (2.00 eq.). Stir at least 15minutes at 20 ± 5 °C (target: 20 °C). Confirm no visible solids and later use. - Add OxymaPure (2.00 eq.) into TJ4S-1213-Sy15-R01), rinse the TJ4S-1213-Sy15-R01 with DMF (2.0 V, 1.90 kg/kg).. - Add DIC (2.20 eq.) into TJ4S-1213-Sy15-R01 at 20 ± 5 °C (target: 20 °C, activate for 20±5 min (target: 20 min) at 20 ± 5 °C (target: 20 °C). - Transfer the mixture in TJ4S-1213-Sy15-R01 into TJ4S-1213-Sy15. Add DMF (1.0 V，0.94 kg/kg) into TJ4S-1213-Sy15-R01 to rinse the wall. - Bubble with nitrogen to the synthesis reactor (TJ4S-1213-Sy15) for 5 ± 2 min (target: 5 min), then start the agitator. Transfer the mixture in activation tank (TJ4S-1213-Sy15-R01) into synthesis reactor (TJ4S-1213-Sy15). |
|  | Coupling | - Heat the mixture to 20 - 27 °C (target: 25 °C). The mixture is allowed to react at 20 - 27 °C (target: 25 °C). - After 6 h ± 15 min (target: 6 h), sample the mixture every 0.5 ~ 1 h for bromophenol blue analysis until two consecutive samples test as negative (Yellow to Orange Report Negative= Complete coupling; Blue to Black Positive= Incomplete coupling). And the mixture should be stirred for coupling time (8.5 h ≤ Coupling time ≤ 17.5 h). - Stop stirring and turn off the nitrogen. Drain until there is no reaction mixture drip. Note: If resin adherence is found at other times during the reaction, rinse the reactor with 0.50~5.00 kg DMF each time. |
|  | Wash resin | - Add DMF (8.0 V, 7.58 kg/kg) into the activation tank (TJ4S-1213-Sy15-R01). Circulate and clean the activating liquid transfer pipeline, solid charging port cleaning pipeline and liquid charging port cleaning pipeline. - After the liquid is filtered out of the synthesis reactor. Transfer the mixture in activation tank (TJ4S-1213-Sy15-R01) to synthesis reactor (TJ4S-1213-Sy15). - Bubble with nitrogen, after stirring, turn off the nitrogen. Drain until there is no obvious drip Note: Perform this operation 3 times. |
|  | De-Fmoc | - Add 20 % piperidine in DMF (8.0 V, 7.45 kg/kg) into TJ4S-1213-Sy15. - Bubble with nitrogen for 5 ± 2 min, then start the agitator. - Adjust to 25 ± 5 °C (target: 25 °C), and stir for 110 -130 min (target: 2 h) at 25 ± 5 °C (target: 25 °C). - Stop stirring, turn off the nitrogen. Drain until there is no obvious drip. |
|  | Wash resin | - Adjust the jacket of synthesis reactor TJ4S-1213-Sy15 to 20±5 °C (target: 20 °C). - Add Isopropyl Acetate (10.0 V, 8.73 kg/kg) into TJ4S-1213-Sy15. - Bubble with nitrogen, after stirring, turn off the nitrogen. Drain until there is no obvious drip Perform this operation 4 times. |
|  | Re-swelling | - Add DMF (12.0 V, 11.38 kg/kg) into TJ4S-1213-Sy15. - Bubble with nitrogen, after stirring, turn off the nitrogen. Drain until there is no obvious drip Perform this operation 2 times. |
|  | Fmoc-L-Ala-OH·H2O activation and transfer | - Add DMF (5.0 V, 4.74 kg/kg) into TJ4S-1213-Sy15-R01. Start the agitator. - Adjust the jacket temperature to 20 ± 5 °C (target: 20 °C). - Add Fmoc-L-Ala-OH·H2O (2.00 eq). Stir at least 15 minutes at 20 ± 5 °C (target: 20 °C), Confirm no visible solids and later use. - Add OxymaPure (2.00 eq.) into TJ4S-1213-Sy15-R01), rinse the TJ4S-1213-Sy15-R01 with DMF (2.0 V, 1.90 kg/kg).. - Add DIC (2.20 eq.) into TJ4S-1213-Sy15-R01 at 20 ± 5 °C (target: 20 °C, activate for 20±5 min (target: 20 min) at 20 ± 5 °C (target: 20 °C).. - Transfer the mixture in TJ4S-1213-Sy15-R01 into TJ4S-1213-Sy15. Add DMF (1.0 V，0.94 kg/kg) into TJ4S-1213-Sy15-R01 to rinse the wall. - Bubble with nitrogen to the synthesis reactor (TJ4S-1213-Sy15) for 5 ± 2 min (target: 5 min), then start the agitator. Transfer the mixture in activation tank (TJ4S-1213-Sy15-R01) into synthesis reactor (TJ4S-1213-Sy15) |
|  | Coupling | - Heat the mixture to 20 - 27 °C (target: 25 °C). The mixture is allowed to react at 20 - 27 °C (target: 25 °C). - After 3 h±15 min (target: 3 h), sample the mixture every 0.5~1 h for ninhydrin test analysis until two consecutive samples test as negative (Yellow to Light pink Report Negative= Complete coupling; Blue to Purple Positive= Incomplete coupling). And the mixture should be stirred for coupling time (4.5 h ≤ Coupling time ≤ 9.5 h). - Stop stirring and turn off the nitrogen. Drain until there is no reaction mixture drip. Note: If resin adherence is found at other times during the reaction, rinse the reactor with 0.50~5.00 kg DMF each time. |
|  | Wash resin | - Add DMF (8.0 V, 7.58 kg/kg) into the activation tank (TJ4S-1213-Sy15-R01). Circulate and clean the activating liquid transfer pipeline, solid charging port cleaning pipeline and liquid charging port cleaning pipeline. - After the liquid is filtered out of the synthesis reactor. Transfer the mixture in activation tank (TJ4S-1213-Sy15-R01) to synthesis reactor (TJ4S-1213-Sy15). - Bubble with nitrogen, after stirring, turn off the nitrogen. Drain until there is no obvious drip Note: Perform this operation 3 times. |
|  | De-Fmoc | - Add 20 % piperidine in DMF (8.0 V, 7.45 kg/kg) into TJ4S-1213-Sy15. - Bubble with nitrogen for 5 ± 2 min, then start the agitator. - Adjust to 25 ± 5 °C (target: 25 °C), and stir for 110 -130 min (target: 2 h) at 25 ± 5 °C (target: 25 °C). - Stop stirring, turn off the nitrogen. Drain until there is no obvious drip. |
|  | Wash resin | - Adjust the jacket of synthesis reactor TJ4S-1213-Sy15 to 20±5 °C (target: 20 °C). - Add Isopropyl Acetate (10.0 V, 8.73 kg/kg) into TJ4S-1213-Sy15. - Bubble with nitrogen, after stirring, turn off the nitrogen. Drain until there is no obvious drip Perform this operation 4 times. |
|  | Re-swelling | - Add DMF (10.0 V, 9.48 kg/kg) into TJ4S-1213-Sy15. - Bubble with nitrogen, then start the agitator. Stirring and turn off the nitrogen. Drain until there is no obvious drip. Perform this operation 2 times. |
|  | Fmoc-L-Leu-OH activation and transfer | - Add DMF (5.0 V, 4.74 kg/kg) into TJ4S-1213-Sy15-R01. Start the agitator. - Adjust the jacket temperature to 20 ± 5 °C (target: 20 °C). - Add Fmoc-L-Leu-OH (2.00 eq). Stir at least 15 minutes at 20 ± 5 °C (target: 20 °C)., Confirm no visible solids and later use. - Add OxymaPure (2.00 eq.) into TJ4S-1213-Sy15-R01), rinse the TJ4S-1213-Sy15-R01 with DMF (2.0 V, 1.90 kg/kg). - Add DIC (2.20 eq.) into TJ4S-1213-Sy15-R01 at 20 ± 5 °C (target: 20 °C, activate for 20±5 min (target: 20 min) at 20 ± 5 °C (target: 20 °C).. - Transfer the mixture in TJ4S-1213-Sy15-R01 into TJ4S-1213-Sy15. Add DMF (1.0 V，0.94 kg/kg) into TJ4S-1213-Sy15-R01 to rinse the wall. - Bubble with nitrogen to the synthesis reactor (TJ4S-1213-Sy15) for 5 ± 2 min (target: 5 min), then start the agitator. Transfer the mixture in activation tank (TJ4S-1213-Sy15-R01) into synthesis reactor (TJ4S-1213-Sy15) |
|  | Coupling | - Heat the mixture to 20 - 27 °C (target: 25 °C). The mixture is allowed to react at 20 - 27 °C (target: 25 °C). - After 3 h±15 min (target: 3 h), sample the mixture every 0.5~1 h for ninhydrin test analysis until two consecutive samples test as negative (Yellow to Light pink Report Negative= Complete coupling; Blue to Purple Positive= Incomplete coupling). And the mixture should be stirred for coupling time (4.5 h ≤ Coupling time ≤ 9.5 h).. - Stop stirring and turn off the nitrogen. Drain until there is no reaction mixture drip. Note: If resin adherence is found at other times during the reaction, rinse the reactor with 0.50~5.00 kg DMF each time. |
|  | Wash resin | - Add DMF (8.0 V, 7.58 kg/kg) into the activation tank (TJ4S-1213-Sy15-R01). Circulate and clean the activating liquid transfer pipeline, solid charging port cleaning pipeline and liquid charging port cleaning pipeline. - After the liquid is filtered out of the synthesis reactor. Transfer the mixture in activation tank (TJ4S-1213-Sy15-R01) to synthesis reactor (TJ4S-1213-Sy15). - Bubble with nitrogen, after stirring, turn off the nitrogen. Drain until there is no obvious drip Note: Perform this operation 3 times. |
|  | De-Fmoc | - Add 20 % piperidine in DMF (8.0 V, 7.45 kg/kg) into TJ4S-1213-Sy15. - Bubble with nitrogen for 5 ± 2 min, then start the agitator. - Adjust to 25 ± 5 °C (target: 25 °C), and stir for 110 -130 min (target: 2 h) at 25 ± 5 °C (target: 25 °C). - Stop stirring, turn off the nitrogen. Drain until there is no obvious drip. |
|  | Wash resin | - Adjust the jacket of synthesis reactor TJ4S-1213-Sy15 to 20±5 °C (target: 20 °C). - Add Isopropyl Acetate (10.0 V, 8.73 kg/kg) into TJ4S-1213-Sy15. - Bubble with nitrogen, after stirring, turn off the nitrogen. Drain until there is no obvious drip. - Perform this operation 10 times. - Starting with the 10th wash, sample the isopropyl acetate washing solution during stirring and develop with chloranil, compare the color with the standard 500 ppm Piperidine in Isopropyl Acetate as reference solution. If the color is darker than the reference solution, repeat the IPAC wash twice. Sample again. Repeat until the color of the washing solution’s chloranil test is lighter or consistent with the reference solution and then |
|  | Re-swelling | - Add DMF (12.0 V, 11.38 kg/kg) into TJ4S-1213-Sy15. - Bubble with nitrogen, after stirring, turn off the nitrogen. Drain until there is no obvious drip. Perform this operation 2 times. |
|  | Fmoc-L- Orn(Boc)-OH coupling activation and transfer | - Add DMF (5.0 V, 4.74 kg/kg) into TJ4S-1213-Sy15-R01. Start the agitator. - Adjust the jacket temperature to 20 ± 5 °C (target: 20 °C). - Add Fmoc-L- Orn(Boc)-OH (2.00 eq.) coupling. Stir at least 15 minutes at 20 ± 5 °C (target: 20 °C), Confirm no visible solids and later use. - Add OxymaPure (2.00 eq.) into TJ4S-1213-Sy15-R01), rinse the TJ4S-1213-Sy15-R01 with DMF (2.0 V, 1.90 kg/kg). - Add DIC (2.20 eq.) into TJ4S-1213-Sy15-R01 at 20 ± 5 °C (target: 20 °C, activate for 20±5 min (target: 20 min) at 20 ± 5 °C (target: 20 °C). - Transfer the mixture in TJ4S-1213-Sy15-R01 into TJ4S-1213-Sy15. Add DMF (1.0 V，0.94 kg/kg) into TJ4S-1213-Sy15-R01 to rinse the wall. - Bubble with nitrogen to the synthesis reactor (TJ4S-1213-Sy15) for 5 ± 2 min (target: 5 min), then start the agitator. Transfer the mixture in activation tank (TJ4S-1213-Sy15-R01) into synthesis reactor (TJ4S-1213-Sy15) |
|  | Coupling | - Heat the mixture to 20 - 27 °C (target: 25°C). The mixture is allowed to react at 20 - 27 (target: 25 °C). - After 3 h ± 15 min (target: 3 h), sample the mixture every 0.5 ~ 1 h for ninhydrin test analysis until two consecutive samples test as negative (Yellow to Light pink Report Negative= Complete coupling; Blue to Purple Positive= Incomplete coupling). And the mixture should be stirred for coupling time (4.5 h ≤ Coupling time ≤ 9.5 h). - Stop stirring and turn off the nitrogen. Drain until there is no reaction mixture drip. Note: If resin adherence is found at other times during the reaction, rinse the reactor with 0.50~5.00 kg DMF each time. |
|  | Wash resin | - Add IPAC (11.0 V, 9.60 kg/kg) into TJ4S-1213-Sy15. - Bubble with nitrogen, after stirring, turn off the nitrogen. Drain until there is no obvious drip. Note: Perform this operation 5 times. |
|  | Wash resin | - Add Toluene (11.0 V, 9.59 kg/kg) into TJ4S-1213-Sy15 via dip tube and diaphragm pump. Bubble with nitrogen, after stirring, turn off the nitrogen. Drain until there is no obvious drip - Add MTBE (11.0 V, 8.15 kg/kg) into TJ4S-1213-Sy15. Bubble with nitrogen, after stirring, turn off the nitrogen. Drain until there is no obvious drip - Add MTBE (11.0 V, 8.15 kg/kg) into TJ4S-1213-Sy15. Bubble with nitrogen, after stirring, turn off the nitrogen. Drain until there is no obvious drip |
|  | Wash resin | - Add Toluene (11.0 V, 9.59 kg/kg) into TJ4S-1213-Sy15 by manually weighing via dip tube and diaphragm pump. Bubble with nitrogen, after stirring, turn off the nitrogen. Drain until there is no obvious drip. - Add MTBE (11.0 V, 8.15 kg/kg) into TJ4S-1213-Sy15. Bubble with nitrogen, after stirring, turn off the nitrogen. Drain until there is no obvious drip. - Add MTBE (11.0 V, 8.15 kg/kg) into TJ4S-1213-Sy15. Bubble with nitrogen, after stirring, turn off the nitrogen. Drain until there is no obvious drip. - Sample the MTBE washing solution during the second stirring and develop with chloranil, compare the color with the standard 500 ppm piperidine in MTBE solution as reference solution. If the color is darker than the reference solution, repeat Step 93 until the chloranil test color of the washing solution is lighter than or consistent with that of the reference solution. If the color is lighter or consistent with the reference solution, |
|  | Drying | Sweep with nitrogen, and vacuum underneath at T ≤ 30 oC by jacket. After drying for 16±2 h, sample the solid for LOD analysis until LOD ≤ 9% (w/w). |
|  | Collect | Put in-process intermediate ASYM-141366 into double LDPE bags, with the inner layer of pharmaceutical-grade LDPE powder feeding bags, number and weigh, desiccant between the two layers, purge the inside of the package with nitrogen, seal for temporary storage, store at 2-8 oC. |

- Critical Process Parameters and Critical Quality Attributes (CPPs and CQAs)

ASYM-141366 (peptide on-resin) is an in-process intermediate without a release specification. It is not isolated as a bulk material and is forward processed directly into the Step 2 soft cleavage. Therefore, no critical quality attributes (CQAs) are defined for ASYM-141366.

The quality of the Step 1 output is assured by (1) operating the critical process parameters (CPPs) within their proven acceptable ranges (PARs), and (2) in-process controls (IPCs) that confirm the peptide on-resin is suitable for forward processing to produce Step 2 product ASYM-140020 (LSN4182924) meeting their specifications.

The CPPs were identified based on laboratory experiments, development experience, previous campaign knowledge. The CPPs were classified according to their potential impact on the quality attributes of the Fragment L2. The CPPs, with their PARs, normal operating ranges and targets, are listed in Table 7. No critical in-process controls (CIPCs) are established for Step 1.

The IPCs defined in Table 9 shall be performed for each PPQ batch. Processing shall not proceed to the next operation until the IPC acceptance criterion is met (refer to Table 9 for actions in the event of failure). IPC results will be summarized in Appendix 3.

- Operating Ranges for Critical Process Parameters

Table 7: Eloralintide fragment L2 Critical Process Parameters (CPPs)

| Step | Parameter | Type | Target | PAR* | NOR (MOR) | Unit |
| --- | --- | --- | --- | --- | --- | --- |
| Step 1 SPPS | Resin loading - DIPEA quantity | CPP | 6.00 | 5.24 – 6.69 | 5.70 - 6.30 | eq |
|  | Resin loading - Dissolution temperature | CPP | 25.0 | 17.0 – 33.8 | 20 – 30 | °C |
|  | Resin loading – Coupling Reaction temperature | CPP | 25.0 | 17.2 – 32.7 | 20 – 30 | °C |
|  | Resin loading – Capping DMF wash quantity | CPP | NLT 7.58 | NLT 7.58 | 8.10 – 8.96 | kg/kg |
|  | Deprotection temperature (Standard AA) | CPP | 25.0 | 17.0 – 34.8 | 20 – 30 | °C |
|  | Deprotection temperature (Pre-Ser20) | CPP | 25.0 | 14.6 – 34.7 | 20 - 30 | °C |
|  | Piperidine/DMF exposure time (Pre-Ser20 Treatment 1) | CPP | 15 | 6 - 68 | 11 - 36 | min |
|  | Piperidine/DMF exposure time (Pre-Ser20 Treatment 2) | CPP | 30 | 6 - 83 | 20 – 51 | min |
|  | Post Deprotection IPAc wash quantity | CPP | NLT 8.73 | NLT 7.93 | 8.38 - 9.08 | kg/kg |
|  | Oxyma quantity (Standard AA) | CPP | 2.00 | 1.94 – 2.25 | 1.97 -2.06 | eq |
|  | Oxyma quantity (NMeAsn22, αMePhe15) | CPP | 1.80 | 1.75 – 2.03 | 1.78 – 1.85 | eq |
|  | Oxyma quantity (Asn21) | CPP | 2.60 | 2.50 – 2.93 | 2.52 – 2.68 | eq |
|  | Dissolution jacket temperature (Standard AA) | CPP | 20.0 | 12.6 – 26.5 | 15 – 25 | °C |
|  | Dissolution jacket temperature (Phe23) | CPP | 20.0 | 12.0 – 29.5 | 15 – 25 | °C |
|  | Dissolution jacket temperature (NMeAsn22, αMePhe15) | CPP | 20.0 | 12.6 - 26.9 | 15 - 25 | °C |
|  | Dissolution jacket temperature (Asn21) | CPP | 20.0 | 12.2 – 27.9 | 15 – 25 | °C |
|  | DIC quantity (Standard AA) | CPP | 2.20 | 2.01 -2.48 | 2.09 - 2.31 | eq |
|  | Activation temperature (Standard AA) | CPP | 20.0 | 12.6 – 26.5 | 15 – 25 | °C |
|  | Activation temperature (Phe23) | CPP | 5.0 | - 2.5 – 13.6 | 0 – 10 | °C |
|  | Activation temperature (NMeAsn22, αMePhe15) | CPP | 20.0 | 12.6 - 26.9 | 15 - 25 | °C |
|  | Activation temperature (Asn21) | CPP | 20.0 | 12.2 – 27.9 | 15 – 25 | °C |
|  | Coupling temperature (Standard AA) | CPP | 25.0 | 18.6 – 28.4 | 20 - 27 | °C |
|  | Coupling temperature (Phe23) | CPP | 25.0 | 17.4 – 30.3 | 19 - 28 | °C |
|  | Coupling temperature (NMeAsn22, αMePhe15) | CPP | 25.0 | 17.3 – 28.3 | 19 - 27 | °C |
|  | Coupling temperature (Asn21) | CPP | 25.0 | 17.2 – 30.2 | 19 - 28 | °C |
|  | Post Asn21 capping DMF wash quantity | CPP | NLT 7.58 | NLT 6.75 | 7.20 -7.96 | kg/kg |
|  | IPAc wash quantity | CPP | NLT 8.73 | NLT 8.73 | 9.22 – 9.98 | Kg / kg |
|  | Toluene wash quantity | CPP | NLT 8.72 | NLT 8.72 | 9.21 – 9.97 | Kg / kg |
|  | MTBE wash quantity | CPP | NLT 7.41 | NLT 7.41 | 7.82 – 8.48 | Kg / kg |

Note: * Non-bold font format: not defined as CPP parameter.

- Materials and Specifications

- Origin of Materials

No starting materials, raw materials, reagents or solvents used in the manufacture of ASYM-141366 as well as any of the intermediates are derived from human or animal sources. Asymchem will provide a BSE/TSE statement for each lot manufactured. All starting materials and critical raw materials used in PPQ batches shall be procured from approved suppliers and released against approved specifications prior to use.

- Raw Materials, Reagents and Solvents

All raw materials, solvents and reagents are sourced and controlled by Asymchem against current specifications. For details of the specifications, see: Material specifications for dedicated project RMS-TJ414002003 (current version)

- Regulatory Starting Material / Critical Raw Material

Fmoc-Gly-OH, Fmoc-L-Phe-OH, Fmoc-L-N-Me-Asn(Trt)-OH, Fmoc-L-Asn(Trt)-OH, Fmoc-L-Ser(tBu)-OH, Fmoc-L-Arg(Pbf)-OH, Fmoc-L-Val-OH, Fmoc-L-Leu-OH, Fmoc-L-α-Me-Phe-OH, Fmoc-L-Glu(OtBu)-OH·H2O, Fmoc-L-Ala-OH·H2O and Fmoc-L-Orn(Boc)-OH are identified as registration starting materials. 2-Chlorotrityl Chloride (2-CTC) resin is identified as critical raw material.

The vendors selected for the process performance qualification campaign are listed in Table 8.

Table 8. Suppliers of the starting material and critical material

| No. | Amino acid | Control No. | CAS | Specification No. | Supplier | Manufacturer |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 2-CTC resin | YL005205 | 42074-68-0 | RMS-TJ414002003 (current version) | Tianjin Nankai Hecheng S&T Co., Ltd. (Address: No.969-6, Kaixuan Street, Dagang Petrochemical Industrial Park, Binhai New Area, Tianjin, P.R.China ) | Tianjin Nankai Hecheng S&T Co., Ltd. (Address: No.969-6, Kaixuan Street, Dagang Petrochemical Industrial Park, Binhai New Area, Tianjin, P.R.China ) |
|  |  |  |  |  | Sunresin New Materials Co. Ltd. (Address: No.135 Jinye Road, High-Tech Zone) | Sunresin New Materials Co. Ltd., GaoLing (Address: No.999 Weiyangjiu Road, Gaoling District, Xi’an City, Shaanxi Province, P.R.China) |
| 2 | Fmoc-Gly-OH | YL004846 | 29022-11-5 | SP-138200-02 (current version) | Liaoning Asymchem Laboratories Co., Ltd (Address: Yimatu Village, Yimatu town, Fu Mongolia county (Fluorine Industry Development Area), Fuxin, Liaoning, 123129, China). | Liaoning Asymchem Laboratories Co., Ltd (Address: Yimatu Village, Yimatu town, Fu Mongolia county (Fluorine Industry Development Area), Fuxin, Liaoning, 123129, China). |
|  |  |  |  |  | Asymchem Laboratories (Jilin) Co., Ltd. (DH3) (Address: No. 899, Xingqi Road, High-tech Industrial Zone, Dunhua, Jilin 133700, China) | Asymchem Laboratories (Jilin) Co., Ltd. (DH3) (Address: No. 899, Xingqi Road, High-tech Industrial Zone, Dunhua, Jilin 133700, China) |
|  |  |  |  |  | Chengdu ZY Biochemical Technology Co., Ltd. (Address: No.170 Qingpu Road, Shou'an Subdistrict, Pujiang County, Chengdu City, Sichuan Province, P.R.China) | Chengdu ZY Biochemical Technology Co., Ltd. (Address: No.170 Qingpu Road, Shou'an Subdistrict, Pujiang County, Chengdu City, Sichuan Province, P.R.China) |
| 3 | Fmoc-L-Phe-OH | YL004840 | 35661-40-6 | SP-138203-02 (current version) | Liaoning Asymchem Laboratories Co., Ltd (Address: Yimatu Village, Yimatu town, Fu Mongolia county (Fluorine Industry Development Area), Fuxin, Liaoning, 123129, China). | Liaoning Asymchem Laboratories Co., Ltd (Address: Yimatu Village, Yimatu town, Fu Mongolia county (Fluorine Industry Development Area), Fuxin, Liaoning, 123129, China). |
|  |  |  |  |  | Asymchem Laboratories (Jilin) Co., Ltd. (DH3) (Address: No. 899, Xingqi Road, High-tech Industrial Zone, Dunhua, Jilin 133700, China) | Asymchem Laboratories (Jilin) Co., Ltd. (DH3) (Address: No. 899, Xingqi Road, High-tech Industrial Zone, Dunhua, Jilin 133700, China) |
|  |  |  |  |  | Chengdu ZY Biochemical Technology Co., Ltd. (Address: No.170 Qingpu Road, Shou'an Subdistrict, Pujiang County, Chengdu City, Sichuan Province, P.R.China) | Chengdu ZY Biochemical Technology Co., Ltd. (Address: No.170 Qingpu Road, Shou'an Subdistrict, Pujiang County, Chengdu City, Sichuan Province, P.R.China) |
| 4 | Fmoc-L-N-Me-Asn(Trt)-OH | YL008617 | 941296-80-6 | SP-138520-02 (current version) | Asymchem Laboratories (Jilin) Co., Ltd. (DH3) (Address: No. 899, Xingqi Road, High-tech Industrial Zone, Dunhua, Jilin 133700, China) | Asymchem Laboratories (Jilin) Co., Ltd. (DH3) (Address: No. 899, Xingqi Road, High-tech Industrial Zone, Dunhua, Jilin 133700, China) |
| 5 | Fmoc-L-Asn(Trt)-OH | YL004843 | 132388-59-1 | SP-138519-02 (current version) | Liaoning Asymchem Laboratories Co., Ltd (Address: Yimatu Village, Yimatu town, Fu Mongolia county (Fluorine Industry Development Area), Fuxin, Liaoning, 123129, China). | Liaoning Asymchem Laboratories Co., Ltd (Address: Yimatu Village, Yimatu town, Fu Mongolia county (Fluorine Industry Development Area), Fuxin, Liaoning, 123129, China). |
|  |  |  |  |  | Asymchem Laboratories (Jilin) Co., Ltd. (DH3) (Address: No. 899, Xingqi Road, High-tech Industrial Zone, Dunhua, Jilin 133700, China) | Asymchem Laboratories (Jilin) Co., Ltd. (DH3) (Address: No. 899, Xingqi Road, High-tech Industrial Zone, Dunhua, Jilin 133700, China) |
|  |  |  |  |  | Chengdu ZY Biochemical Technology Co., Ltd. (Address: No.170 Qingpu Road, Shou'an Subdistrict, Pujiang County, Chengdu City, Sichuan Province, P.R.China) | Chengdu ZY Biochemical Technology Co., Ltd. (Address: No.170 Qingpu Road, Shou'an Subdistrict, Pujiang County, Chengdu City, Sichuan Province, P.R.China) |
| 6 | Fmoc-L-Ser(tBu)-OH | YL003428 | 71989-33-8 | SP-132732-02 (current version) | Liaoning Asymchem Laboratories Co., Ltd (Address: Yimatu Village, Yimatu town, Fu Mongolia county (Fluorine Industry Development Area), Fuxin, Liaoning, 123129, China). | Liaoning Asymchem Laboratories Co., Ltd (Address: Yimatu Village, Yimatu town, Fu Mongolia county (Fluorine Industry Development Area), Fuxin, Liaoning, 123129, China). |
|  |  |  |  |  | Asymchem Laboratories (Jilin) Co., Ltd. (DH3) (Address: No. 899, Xingqi Road, High-tech Industrial Zone, Dunhua, Jilin 133700, China) | Asymchem Laboratories (Jilin) Co., Ltd. (DH3) (Address: No. 899, Xingqi Road, High-tech Industrial Zone, Dunhua, Jilin 133700, China) |
|  |  |  |  |  | Chengdu ZY Biochemical Technology Co., Ltd. (Address: No.170 Qingpu Road, Shou'an Subdistrict, Pujiang County, Chengdu City, Sichuan Province, P.R.China) | Chengdu ZY Biochemical Technology Co., Ltd. (Address: No.170 Qingpu Road, Shou'an Subdistrict, Pujiang County, Chengdu City, Sichuan Province, P.R.China) |
| 7 | Fmoc-L-Arg(Pbf)-OH | YL003424 | 154445-77-9 | SP-138518-02 (current version) | Asymchem Laboratories (Jilin) Co., Ltd. (DH3) (Address: No. 899, Xingqi Road, High-tech Industrial Zone, Dunhua, Jilin 133700, China) | Asymchem Laboratories (Jilin) Co., Ltd. (DH3) (Address: No. 899, Xingqi Road, High-tech Industrial Zone, Dunhua, Jilin 133700, China) |
|  |  |  |  |  | Chengdu ZY Biochemical Technology Co., Ltd. (Address: No.170 Qingpu Road, Shou'an Subdistrict, Pujiang County, Chengdu City, Sichuan Province, P.R.China) | Chengdu ZY Biochemical Technology Co., Ltd. (Address: No.170 Qingpu Road, Shou'an Subdistrict, Pujiang County, Chengdu City, Sichuan Province, P.R.China) |
| 8 | Fmoc-L-Val-OH | YL003110 | 68858-20-8 | SP-138518-02 (current version) | Liaoning Asymchem Laboratories Co., Ltd (Address: Yimatu Village, Yimatu town, Fu Mongolia county (Fluorine Industry Development Area), Fuxin, Liaoning, 123129, China). | Liaoning Asymchem Laboratories Co., Ltd (Address: Yimatu Village, Yimatu town, Fu Mongolia county (Fluorine Industry Development Area), Fuxin, Liaoning, 123129, China). |
|  |  |  |  |  | Asymchem Laboratories (Jilin) Co., Ltd. (DH3) (Address: No. 899, Xingqi Road, High-tech Industrial Zone, Dunhua, Jilin 133700, China) | Asymchem Laboratories (Jilin) Co., Ltd. (DH3) (Address: No. 899, Xingqi Road, High-tech Industrial Zone, Dunhua, Jilin 133700, China) |
|  |  |  |  |  | Chengdu ZY Biochemical Technology Co., Ltd. (Address: No.170 Qingpu Road, Shou'an Subdistrict, Pujiang County, Chengdu City, Sichuan Province, P.R.China) | Chengdu ZY Biochemical Technology Co., Ltd. (Address: No.170 Qingpu Road, Shou'an Subdistrict, Pujiang County, Chengdu City, Sichuan Province, P.R.China) |
| 9 | Fmoc-L-Leu-OH | YL003425 | 35661-60-0 | SP-138201-02 (current version) | Liaoning Asymchem Laboratories Co., Ltd (Address: Yimatu Village, Yimatu town, Fu Mongolia county (Fluorine Industry Development Area), Fuxin, Liaoning, 123129, China). | Liaoning Asymchem Laboratories Co., Ltd (Address: Yimatu Village, Yimatu town, Fu Mongolia county (Fluorine Industry Development Area), Fuxin, Liaoning, 123129, China). |
|  |  |  |  |  | Asymchem Laboratories (Jilin) Co., Ltd. (DH3) (Address: No. 899, Xingqi Road, High-tech Industrial Zone, Dunhua, Jilin 133700, China) | Asymchem Laboratories (Jilin) Co., Ltd. (DH3) (Address: No. 899, Xingqi Road, High-tech Industrial Zone, Dunhua, Jilin 133700, China) |
|  |  |  |  |  | Chengdu ZY Biochemical Technology Co., Ltd. (Address: No.170 Qingpu Road, Shou'an Subdistrict, Pujiang County, Chengdu City, Sichuan Province, P.R.China) | Chengdu ZY Biochemical Technology Co., Ltd. (Address: No.170 Qingpu Road, Shou'an Subdistrict, Pujiang County, Chengdu City, Sichuan Province, P.R.China) |
| 10 | Fmoc-L-α-Me-Phe-OH | YL008272 | 135944-05-7 | SP-138210-02 (current version) | Asymchem Laboratories (Jilin) Co., Ltd. (DH3) (Address: No. 899, Xingqi Road, High-tech Industrial Zone, Dunhua, Jilin 133700, China) | Asymchem Laboratories (Jilin) Co., Ltd. (DH3) (Address: No. 899, Xingqi Road, High-tech Industrial Zone, Dunhua, Jilin 133700, China) |
| 11 | Fmoc-L-Glu(OtBu)-OH·H2O | YL005741 | 204251-24-1 | SP-138390-02 (current version) | Asymchem Laboratories (Jilin) Co., Ltd. (DH3) (Address: No. 899, Xingqi Road, High-tech Industrial Zone, Dunhua, Jilin 133700, China) | Asymchem Laboratories (Jilin) Co., Ltd. (DH3) (Address: No. 899, Xingqi Road, High-tech Industrial Zone, Dunhua, Jilin 133700, China) |
|  |  |  |  |  | Chengdu ZY Biochemical Technology Co., Ltd. (Address: No.170 Qingpu Road, Shou'an Subdistrict, Pujiang County, Chengdu City, Sichuan Province, P.R.China) | Chengdu ZY Biochemical Technology Co., Ltd. (Address: No.170 Qingpu Road, Shou'an Subdistrict, Pujiang County, Chengdu City, Sichuan Province, P.R.China) |
| 12 | Fmoc-L-Ala-OH·H2O | YL006160 | 207291-76-7 | SP-138204-02 (current version) | Liaoning Asymchem Laboratories Co., Ltd (Address: Yimatu Village, Yimatu town, Fu Mongolia county (Fluorine Industry Development Area), Fuxin, Liaoning, 123129, China). | Liaoning Asymchem Laboratories Co., Ltd (Address: Yimatu Village, Yimatu town, Fu Mongolia county (Fluorine Industry Development Area), Fuxin, Liaoning, 123129, China). |
|  |  |  |  |  | Asymchem Laboratories (Jilin) Co., Ltd. (DH3) (Address: No. 899, Xingqi Road, High-tech Industrial Zone, Dunhua, Jilin 133700, China) | Asymchem Laboratories (Jilin) Co., Ltd. (DH3) (Address: No. 899, Xingqi Road, High-tech Industrial Zone, Dunhua, Jilin 133700, China) |
|  |  |  |  |  | Chengdu ZY Biochemical Technology Co., Ltd. (Address: No.170 Qingpu Road, Shou'an Subdistrict, Pujiang County, Chengdu City, Sichuan Province, P.R.China) | Chengdu ZY Biochemical Technology Co., Ltd. (Address: No.170 Qingpu Road, Shou'an Subdistrict, Pujiang County, Chengdu City, Sichuan Province, P.R.China) |
| 13 | Fmoc-L-Orn(Boc)-OH | YL005318 | 109425-55-0 | SP-124852-02 (current version) | Asymchem Laboratories (Jilin) Co., Ltd. (DH3) (Address: No. 899, Xingqi Road, High-tech Industrial Zone, Dunhua, Jilin 133700, China) | Asymchem Laboratories (Jilin) Co., Ltd. (DH3) (Address: No. 899, Xingqi Road, High-tech Industrial Zone, Dunhua, Jilin 133700, China) |
|  |  |  |  |  | Chengdu ZY Biochemical Technology Co., Ltd. (Address: No.170 Qingpu Road, Shou'an Subdistrict, Pujiang County, Chengdu City, Sichuan Province, P.R.China) | Chengdu ZY Biochemical Technology Co., Ltd. (Address: No.170 Qingpu Road, Shou'an Subdistrict, Pujiang County, Chengdu City, Sichuan Province, P.R.China) |

- Analytical Methods and Testing

- Analytical Methods Validation

The analytical methods were developed and validated at Asymchem TJ4. All analytical methods for RSM and critical raw materials are validated before material release, refer to Change-TJ420260080.01; All IPC methods are validated before PV start, refer to AM-YL000336-004-VP-01, AM-140020-029-VP-01, AM-140020-031-VP-01, AM-140020-030-VP-01 and AM-140020-016-VP-01. Asymchem shall follow the validated methods in the process performance qualification production.

| ethod Number/ | Method Description/ | Method transfer/validation protocol |  | Method transfer/validation report |  | Qualified Lab |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Protocol No. | Effective Date | Report No. | Approval Date |  |

- Sample plan for IPC

The IPC plan is developed by Asymchem and shall be used in the process performance qualification production.

The IPC tests are  defined below：

IPC: In Process Control - Operations shall not proceed to the next step until results meeting the acceptance criteria are obtained; IPC targets are specified in Table 9.

Table 9. The testing plan of IPCs

| No. | Process Step | Test | Analysis method | Acceptance Criteria | Action in the event of failure |
| --- | --- | --- | --- | --- | --- |
| 1 | End of equipment rinse prior to SPPS operation | KF | AM-YL000336-004 | ≤ 500 ppm | Recharge DMF for rinsing again until meeting the criteria |
| 2 | Coupling Reaction completion for each amino acid cycle (Phe23~NMeAsn22, Ser20~αMePhe15, Ala13~Orn11) | Colorimetric Test (Ninhydrin for coupling of Phe23~NMeAsn22, Ser20~αMePhe15, Ala13~Orn11) | AM-140020-029 | Negative | Continue stirring and sample the mixture for ninhydrin test analysis until two consecutive samples test as negative (the solution of sample is yellow to light pink). |
| 3 | Coupling Reaction completion for amino acid cycle (Glu14) | Colorimetric Test (Bromophenol blue for coupling of Glu14) | AM-140020-031 | Negative | Continue stirring and sample the mixture for bromophenol blue test until two consecutive samples test as negative (the resin of sample is yellow to orange) |
| 4 | Chloranil test (Leu12 Post deprotection wash piperidine residue.) | Chloranil test | AM-140020-030 | Compare piperidine level to the color of 500 ppm standard (negative) | Perform two washes with isopropyl acetate until the chloranil test of the washing solution shows a lighter color or matches the reference solution |
| 5 | Chloranil test (Orn11 post coupling wash piperidine residue. | Chloranil test. | AM-140020-030 | Compare piperidine level to the color of 500 ppm standard (negative) | Perform three sequential washes: 1 wash with toluene followed by 2 washes with MTBE, until the chloranil test of the second MTBE washing solution displays a lighter color or matches the reference solution. |
| 6 | During SPPS drying | LOD | AM-140020-016 | ≤ 9 % (w/w) | Continue drying and sample the solid for LOD analysis until LOD ≤ 9 % (w/w) |

Note: In method AM-140020-016, vacuum drying oven is used for the LOD testing, it is simple technique that require no validation or verification.

- Batch Production Record (OR) Review

Procedure: The CMMD, Production Department and Quality Assurance will review and approve each entry in all sections of the executed batch production record for the process performance qualification batches. Ensure that the information is correct, complete, and all appropriate approvals are performed.

Sample Retaining

ASYM-141366 is the peptide-on-resin in-process intermediate produced by the SPPS process in the PPQ batches and has no release specification. It shall be retained when a peptidyl resin stability study is required and sample retention is necessary.  Samples will be taken for the following purpose, refer to Table 10:

Table 10.  Sample Retains of ASYM-141366

| Sample | Sample quantity | Storage location | Storage condition. |
| --- | --- | --- | --- |
| Stability test * | ~ 125 g | QC (Asymchem TJ4) | Refer to Stability Study Protocol PSP-140020-2026-003.01 |

Note: *In-process intermediate ASYM-141366 from only one PPQ batch will be put into the stability chamber for stability.

- Packaging and Storage

The material shall be packaged according to the following description after release Packaging specifications are summarized in Table 11.

Inner packaging:

Double bagged with LDPE (low-density polyethylene) bags: the inner pharmaceutical‑grade LDPE powder charging bag is sealed with plastic clips and latch-closure seals. And the outer pharmaceutical‑grade LDPE bag is sealed with cable ties separately, desiccant was added between two layers (When Net weight ≤ 1kg，NLT desiccant ≥ 10% of Net weight；Net weight ＞ 1kg，desiccant =1% of Net weight+100g),. Purge the inside of the package with nitrogen, seal for temporary storage

Outer packaging:

HPDE drum with cover for storage.

Storage: seal and avoid light, Store at 2~8 °C.

Table11.  Packaging Specifications

| Packaging | Specification | Seal Type |
| --- | --- | --- |
| Primary package | Size: 30 L or 50 L Pharmaceutical-Grade Powder feeding bag | Each bag is sealed with a sealing clip |
| Secondary package | Size: 1100 L × 700 W × 0.09 T (mm) Pharmaceutical-Grade LDPE bag | Each bag is sealed with a cable tie |
| Outer packaging | Size: 50 L HPDE drum with cover for storage | Each drum is sealed with clips. |
| Desiccants | Size: W:78±4 mm L:150±7 mm Weight: 100 g Material: Silica Gel | desiccant placed between primary and secondary package |

- Stability Test

ASYM-141366 peptide-on-resin is an in-process intermediate without release specification that is further processed in the downstream soft cleavage step 2. To support the stable hold time of peptide on-resin under drying conditions and its extended storage, a stability study will be established on one PPQ batch. The stability monitoring plan is detailed in the ASYM-141366 stability protocol <Refer to PSP-140020-2026-002.01>. .

- Homogeneity study

ASYM-141366 peptide-on-resin is an in-process intermediate without release testing. A homogeneity study is not required for ASYM-141366. As ASYM-141366 is not an isolated bulk material and remains bound to the resin support, a conventional bulk homogeneity study is not considered applicable. Process consistency and uniformity are assured through control of the solid-phase synthesis process, including validated mixing capability of the reactor system, defined process parameters, in-process controls, and process characterisation knowledge. Therefore, a separate homogeneity study is not required for ASYM-141366.

- ASYM-141366 Cleaning Validation

Cleaning validation of the Step 1 equipment train is performed concurrently with the PPQ campaign under CVP-1400200301, in accordance with Asymchem's Cleaning Validation procedure (DS-GMP-043). The cleaning procedures, residue limits, and sampling and analytical methods are approved before PPQ starts. After each cleaning run, QA releases the equipment for reuse once results meet the protocol acceptance criteria. Step 1 batches can be released for further processing but any downstream step 2 fragment L2 batches manufactured must not be released before QA approves the interim cleaning validation report. Cleaning validation results are reported in the cleaning validation report

- Changes and Deviations

During the process performance qualification execution, any change or deviation occurring during the execution shall be handled based on the agreement <Quality argeement: QAA03550.V01> made between Asymchem and Lilly. Each event shall be assessed by Asymchem for its impact on process validation conclusions and summarized in the validation report. After the campaign, these changes and deviations shall be reviewed in the final PPQ report.

- Acceptance Criteria

ASYM-141366 (peptide on-resin) is an in-process intermediate without a release specification, and no CQAs are defined for Step 1. In accordance with the stepwise process validation strategy defined in the Validation Master Plan (VP-14002001.01), Step 1 PPQ will be considered successful when all of the following criteria are met for each of the three SPPS PPQ batches (CPo140020-03-01-02, CPo140020-03-01-03 and CPo140020-03-01-04):

- The batches are manufactured according to the approved Master Operation Record (MOR-TJ414002001.04) and this protocol, using the qualified equipment train in Table 5 and starting materials and critical raw materials from approved suppliers, released against approved specifications.

- All CPPs are maintained within the PARs defined in Table 7.

- All IPCs meet the acceptance criteria in Table 9.

- All deviations and changes during PPQ execution are handled under the Quality Agreement (QAA03550.V01). Each one is investigated and assessed as having no impact on product quality or on the validation conclusion.

- Any failure to meet a criterion shall be investigated under the deviation procedure. If the root cause is shown to be unrelated to the process, the process validation may continue by adding an additional batch once the issue has been remediated. The affected batch(es) are excluded from the PV data set but are discussed in the PV summary.

- Summary Report

Upon completion of three SPPS PPQ batches, a Step 1 PPQ summary report shall be prepared. In line with the stepwise process validation strategy (VP-14002001.01), the report does not depend on Step 2 PPQ being completed. It shall be reviewed and approved according to the Quality Agreement (QAA03550.V01) and shall include:

Validation documentation summary: this protocol, the Master Operation Record, the executed batch production records, and referenced documents with their version numbers, including the cleaning validation protocol and report numbers.

Batch genealogy and materials:

The lots and suppliers of starting materials and 2-CTC resin used in each PPQ batch;

The substitution values of each resin lot and the total resin moles used as the charge basis;

The Step 2 batches each PPQ batch is designated for.

Batch input, output and yield for each batch, compared with the ranges in Table 4.

CPP results for each batch, compared with the PARs (Appendix 4).

IPC results for each batch (Appendix 3), including any repeat operations allowed by the Master Operation Record.

Deviations, changes, OOS and ATR events, with their impact on the validation conclusion.

Conclusion on whether the process was executed reproducibly and in a state of control. CPP, IPC and yield results for the three batches shall be tabulated and compared side by side. Statistical evaluation of process variability and capability will be performed under ongoing process verification (OPV) once enough commercial batch data are available.

Cleaning validation, stability, characterization and other technical study results are reported under their own protocols and are not evaluated in this report. Release of ASYM-141366 for further processing does not depend on approval of the summary report (refer to Section 20).

- Batch Release and Disposition

ASYM-141366 (peptide on-resin) is an in-process intermediate without a release specification. It is not distributed outside the manufacturing site; it is forward processed into Step 2 soft cleavage (ASYM-1400020). The in-process controls in Table 6 take the place of release testing for this material (refer to VP-14002001.01).

Each PPQ batch of ASYM-141366 may be released for further processing when QA confirms all of the following:

IPCs: all IPCs in Table 9 have met their acceptance criteria.

Batch documentation: QA has reviewed the executed batch record sections available at the time of release. As a minimum, this covers material charges, CPP entries and IPC results. All CPPs were within their PARs, or any excursion has been recorded as a deviation.

Deviations: all deviations for the batch have been documented and assessed for impact on quality. No critical or major deviation remains open unless QA has approved a documented impact assessment that allows processing to continue.

Equipment cleaning: if the batch was manufactured in equipment just after a cleaning validation run, QA has approved the interim cleaning validation report for that run (refer to Section 17).

Packaging and storage: the material has been packaged, labelled and stored as described in Section 16. It will be processed further within the hold time supported by the stability data in Section 15.

Appendix 1 Training SOP (Current Version)

| Title | Procedure Number |
| --- | --- |
| ATR Result Investigation | QA-032 (Current version) |
| Batch operation record management procedure (DS) | DS-QA-001 (Current version) |
| Change Control | QA-006 (Current version) |
| Cleaning validation (DS) | DS-GMP-043 (Current version) |
| Deviation investigation | QA-010 (Current version) |
| Good record practice, good documentation practice, good review practice and document integrity | ASM-015 (Current version) |
| Material homogeneity study procedure (DS) | DS-GMP-036 (Current version) |
| OOS Result Investigation | QA-015 (Current version) |
| Process Validation (DS) | DS-GMP-035 (Current version) |
| Product Release (DS) | DS-QA-004 (Current version) |
| Receipt, Release and Distribution of Raw Material for Production(DS) | DS-MM-004 (Current version) |
| Sampling and testing for raw material and product (DS) | DS-AC-004 (Current version) |
| Sampling and Testing Procedure for Cleaning Test | AC-053 (Current version) |
| Stability Testing Management Procedure (DS) | DS-AC-003 (Current version) |
| Supplier Control | MM-046 (Current version) |
| Validation and qualification of facility, utility and equipment | GMP-026 (Current version) |

Appendix 2 Referring document list

| Item | Title | Document and version number |
| --- | --- | --- |
| 1 | Master Operation Record | MOR-TJ414002001.04 |
| 2 | Equipment Suitability Assessment for ASYM-140020 (Step 1) | RA-140020030101.01 |
| 3 | PAR report | RPT-1194035 (Lilly document number.) |
| 4 | Process Flow Document | WI-61120 (Lilly document number.) |
| 5 | Analytical Method Validation/Transfer/Verification Master Plan Protocol | PSP-140020-2026-001.01 |
| 6 | Process Validation Plan for Manufacturing of LSN4182924 (ASYM-140020) | VP-14002001.01 |
| 7 | The change of specifications for raw material amino acids in projectCPo140019-03, CPo140020-03 and CPo140021-03 | Change-TJ420260080.01 |
| 8 | Cleaning Validation Protocol of ASYM-141366 | CVP-1400200301.01 |
| 9 | ASYM-141366 stability study in LSN4182924/140020 project | PSP-140020-2026-003.01 |

Appendix 3   IPCs Results Summary Table

| No. | Sample Description | Test Item | Acceptance Criteria | CPo140020-03- 01-02 | CPo140020-03-01-03 | CPo140020-03-01-04 |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Water content after DMF washing before resin swelling | Water content | NMT 500 ppm |  |  |  |
| 2 | Ninhydrin test in amino acid coupling, Phe23 | Ninhydrin test | Negative |  |  |  |
| 3 | Ninhydrin test in amino acid coupling, Phe23 | Ninhydrin test | Negative |  |  |  |
| 4 | Ninhydrin test in amino acid coupling, NMeAsn22 | Ninhydrin test | Negative |  |  |  |
| 5 | Ninhydrin test in amino acid coupling, NMeAsn22 | Ninhydrin test | Negative |  |  |  |
| 6 | Ninhydrin test in amino acid coupling, Ser20 | Ninhydrin test | Negative |  |  |  |
| 7 | Ninhydrin test in amino acid coupling, Ser20 | Ninhydrin test | Negative |  |  |  |
| 8 | Ninhydrin test in amino acid coupling, Ser19 | Ninhydrin test | Negative |  |  |  |
| 9 | Ninhydrin test in amino acid coupling, Ser19 | Ninhydrin test | Negative |  |  |  |
| 10 | Ninhydrin test in amino acid coupling, Arg18 | Ninhydrin test | Negative |  |  |  |
| 11 | Ninhydrin test in amino acid coupling, Arg18 | Ninhydrin test | Negative |  |  |  |
| 12 | Ninhydrin test in amino acid coupling, Val17 | Ninhydrin test | Negative |  |  |  |
| 13 | Ninhydrin test in amino acid coupling, Val17 | Ninhydrin test | Negative |  |  |  |
| 14 | Ninhydrin test in amino acid coupling, Leu16 | Ninhydrin test | Negative |  |  |  |
| 15 | Ninhydrin test in amino acid coupling, Leu16 | Ninhydrin test | Negative |  |  |  |
| 16 | Ninhydrin test in amino acid coupling, αMePhe 15 | Ninhydrin test | Negative |  |  |  |
| 17 | Ninhydrin test in amino acid coupling, αMePhe 15 | Ninhydrin test | Negative |  |  |  |
| 18 | Bromophenol blue test in amino acid coupling, Glu14 | Bromophenol blue test | Negative |  |  |  |
| 19 | Bromophenol blue test in amino acid coupling, Glu14 | Bromophenol blue test | Negative |  |  |  |
| 20 | Ninhydrin test in amino acid coupling, Ala13 | Ninhydrin test | Negative |  |  |  |
| 21 | Ninhydrin test in amino acid coupling, Ala13 | Ninhydrin test | Negative |  |  |  |
| 22 | Ninhydrin test in amino acid coupling, Leu12 | Ninhydrin test | Negative |  |  |  |
| 23 | Ninhydrin test in amino acid coupling, Leu12 | Ninhydrin test | Negative |  |  |  |
| 24 | Chloranil test (Leu12 Post deprotection wash piperidine residue.) | Chloranil test | The color is lighter or consistent with the reference solution |  |  |  |
| 25 | Ninhydrin test in amino acid coupling, Orn11 | Ninhydrin test | Negative |  |  |  |
| 26 | Ninhydrin test in amino acid coupling, Orn11 | Ninhydrin test | Negative |  |  |  |
| 27 | Chloranil test (Orn11 post coupling wash piperidine residue. | Chloranil test | The color is lighter or consistent with the reference solution |  |  |  |
| 28 | LOD under drying | LOD | ≤ 9 % (w/w) |  |  |  |

Note: Upon completion of production, Asymchem shall summarize all test results and compile them into the report.

Appendix 4   CPPs Results Summary Table

| Step | Parameter | Type | Target | PAR | NOR(MOR) | Unit | CPo140020-03- 01-02 | CPo140020-03-01-03 | CPo140020-03-01-04 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step 1 SPPS | Resin loading - DIPEA quantity | CPP | 6.00 | 5.24 – 6.69 | 5.70 - 6.30 | eq |  |  |  |
|  | Resin loading - Dissolution temperature | CPP | 25.0 | 17.0 – 33.8 | 20 – 30 | °C |  |  |  |
|  | Resin loading – Coupling Reaction temperature | CPP | 25.0 | 17.2 – 32.7 | 20 – 30 | °C |  |  |  |
|  | Resin loading – Capping DMF wash quantity | CPP | NLT 7.58 | NLT 7.58 | 8.10 – 8.96 | kg/kg |  |  |  |
|  | Deprotection temperature (Standard AA) | CPP | 25.0 | 17.0 – 34.8 | 20 – 30 | °C |  |  |  |
|  | Deprotection temperature (Pre-Ser20) | CPP | 25.0 | 14.6 – 34.7 | 20 - 30 | °C |  |  |  |
|  | Piperidine/DMF exposure time (Pre-Ser20 Treatment 1) | CPP | 15 | 6 - 68 | 11 - 36 | min |  |  |  |
|  | Piperidine/DMF exposure time (Pre-Ser20 Treatment 2) | CPP | 30 | 6 - 83 | 20 – 51 | min |  |  |  |
|  | Post Deprotection IPAc wash quantity | CPP | NLT 8.73 | NLT 7.93 | 8.38 - 9.08 | kg/kg |  |  |  |
|  | Oxyma quantity (Standard AA) | CPP | 2.00 | 1.94 – 2.25 | 1.97 -2.06 | eq |  |  |  |
|  | Oxyma quantity (NMeAsn22, αMePhe15) | CPP | 1.80 | 1.75 – 2.03 | 1.78 – 1.85 | eq |  |  |  |
|  | Oxyma quantity (Asn21) | CPP | 2.60 | 2.50 – 2.93 | 2.52 – 2.68 | eq |  |  |  |
|  | Dissolution jacket temperature (Standard AA) | CPP | 20.0 | 12.6 – 26.5 | 15 – 25 | °C |  |  |  |
|  | Dissolution jacket temperature (Phe23) | CPP | 20.0 | 12.0 – 29.5 | 15 – 25 | °C |  |  |  |
|  | Dissolution jacket temperature (NMeAsn22, αMePhe15) | CPP | 20.0 | 12.6 - 26.9 | 15 - 25 | °C |  |  |  |
|  | Dissolution jacket temperature (Asn21) | CPP | 20.0 | 12.2 – 27.9 | 15 – 25 | °C |  |  |  |
|  | DIC quantity (Standard AA) | CPP | 2.20 | 2.01 -2.48 | 2.09 - 2.31 | eq |  |  |  |
|  | Activation temperature (Standard AA) | CPP | 20.0 | 12.6 – 26.5 | 15 – 25 | °C |  |  |  |
|  | Activation temperature (Phe23) | CPP | 5.0 | - 2.5 – 13.6 | 0 – 10 | °C |  |  |  |
|  | Activation temperature (NMeAsn22, αMePhe15) | CPP | 20.0 | 12.6 - 26.9 | 15 - 25 | °C |  |  |  |
|  | Activation temperature (Asn21) | CPP | 20.0 | 12.2 – 27.9 | 15 – 25 | °C |  |  |  |
|  | Coupling temperature (Standard AA) | CPP | 25.0 | 18.6 – 28.4 | 20 - 27 | °C |  |  |  |
|  | Coupling temperature (Phe23) | CPP | 25.0 | 17.4 – 30.3 | 19 - 28 | °C |  |  |  |
|  | Coupling temperature (NMeAsn22, αMePhe15) | CPP | 25.0 | 17.3 – 28.3 | 19 - 27 | °C |  |  |  |
|  | Coupling temperature (Asn21) | CPP | 25.0 | 17.2 – 30.2 | 19 - 28 | °C |  |  |  |
|  | Post Asn21 capping DMF wash quantity | CPP | NLT 7.58 | NLT6.75 | 7.20 -7.96 | kg/kg |  |  |  |
|  | IPAc wash quantity | CPP | NLT 8.73 | NLT 8.73 | 9.22 – 9.98 | Kg / kg |  |  |  |
|  | Toluene wash quantity | CPP | NLT 8.72 | NLT 8.72 | 9.21 – 9.97 | Kg / kg |  |  |  |
|  | MTBE wash quantity | CPP | NLT 7.41 | NLT 7.41 | 7.82 – 8.48 | Kg / kg |  |  |  |