☒ Batch to batch flush /批与批之间的冲洗     ☒ Within campaign in-situ minor cleaning /项目内原位小清洁

☐ Within campaign disassemble major cleaning /项目内拆卸大清洁     ☐ End of production cleaning /项目结束后清洁

☐ Equipment Qualification /设备确认  ☐ Other /其他_____N/A______

Cleaning Method No. and Version No./清洗方法编号及版本号：     CMb-TJ414002001.01

- Revision history/版本历史

| Version 版本号 | Effective date 生效日期 | Description of revision/修订描述 (Including reason, background, content etc./包括修订原因，背景，内容等） |
| --- | --- | --- |
| 01 | The effective date is defined as the last approval date/最后签字日期为生效日期 | CMb-TJ414002001.01 is the first version of equipment cleaning method between the batches and within batch for ASYM-140020 Step1 at TJ4 site. / CMb-TJ414002001.01表示ASYM-140020 项目在TJ4厂区生产的第1步生产设备的第一版批与批之间及批次内的清洗方案。 |

- Method/方法

| Equipment / 设备 | Usage/ 用途 | Cleaning method and frequency/清洁方式以及频率 |  | Level 0/B& Level I Cleaning verification/ Level 0/B& Level I清洁确认 | Level II cleaning verification / Level II清洁确认 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Name & I.D., Spec. / MOC/名称&编号，规格/材质 |  | Cleaning method description/ 清洁方法描述 | Cleaning frequency and assessment/ 清洁频率及其风险评估 |  | Sampling method/ 取样方法 | Target compound, ARL, testing method/ 目标物，接受标准，分析方法 |
| 1000L Glass-lined reactor (include pump and transfer pipeline) /1000L搪玻璃反应釜 (包含泵和其转移管路) Equipment # 设备编号: TJ4S-1213-R22 Moc/材质： Glass-lined/搪玻璃 | Prepare the solution/配溶液 | - Rinse inner surface of the reactor with 175±5 kg DMF, and then drain to waste liquid tank through pump and process transfer pipeline. /使用175±5 kg N, N-二甲基甲酰胺淋洗反应釜内表面，然后通过泵和工艺转移管路排至废液罐。 - Dry by compressed gas. /压缩气体吹干。 | Perform Level 0/B cleaning between batches. /批次间执行Level 0/B的清洗。 Note /备注: For Projects CPo140019-03 and CPo140020-03, The CMb-TJ414002001.01 cleaning method applies to the shared glass-lined reactor TJ4S-1213-R22 for Projects CPo140019-03 and CPo140020-03. Project CPo140019-03 will also require preparation of pyridine/acetic anhydride solution in DMF within this reactor. Given the consistency of the solution systems between the two projects, Glass-lined Reactor TJ4S-1213-R22 may be utilized for manufacturing of Project CPo140019-03 upon successful completion of the CMb-TJ414002001 cleaning procedure defined in this plan./对于CPo140019-03和CPo140020-03项目，CMb-TJ414002001.01清洁方法适用于CPo140019-03和CPo140020-03项目的共用搪玻璃反应器TJ4S-1213-R22。CPo140019-03项目需要在该反应器内制备DMF中的吡啶/乙酸酐溶液。鉴于两个项目之间溶液系统的一致性，在成功完成本计划中定义的CMb-TJ414002001清洁程序后，搪玻璃反应釜TJ4S-1213-R22可用于生产CPo140019-03项目。 | Level 0/B | N/A | N/A |
| 500L Agitation tank (include pump and transfer pipeline) /500L搅拌罐 (包含泵和其转移管路) Equipment # 设备编号: TJ4S-1200-AT02 Hastelloy/哈氏合金 | Prepare the solution/配溶液 | - The Agitation tank is rinsed with 100±10 kg DMF by spray ball, and then drain to waste liquid tank through pump and process transfer pipeline. /使用100±10kg N, N-二甲基甲酰胺通过淋洗球冲洗搅拌罐，然后通过泵和工艺转移管路排至废液罐。 - Dry by compressed gas. /压缩气体吹干。 | Perform Level 0/B cleaning between batches. /批次间执行Level 0/B的清洗。 | Level 0/B | N/A | N/A |
| Solid sampler/固体取样器 | Sampling for solid during drying stage /烘料阶段固体取样 | - Wipe with 10±5 kg purified water. /使用10±5 kg纯化水擦洗。 - Dry by compressed gas. /压缩气体吹干。 | Perform Level 0/B cleaning within batch. /批次内执行Level 0/B的清洗。 | Level 0/B | N/A | N/A |
| 50L Glove box/手套箱 MOC/材质: Halar –lined or 316L /衬哈或316L不锈钢 | Dispensing /分装 | - Wipe with DMF. /使用DMF擦洗。 - Wipe with MeOH. /使用甲醇擦洗。 - Dry by compressed gas. /压缩气体吹干。 - Change gloves./更换手套。 | Perform Level I cleaning when switching between different materials within batch. /批次内更换不同物料时执行Level I的清洗。 | Level I | Change gloves when switching within different batches, no need level II test./不同批次内需更换手套，无需进行level Ⅱ检测。 | N/A |
| 50L Glove box/手套箱 MOC/材质: Halar –lined or 316L /衬哈或316L不锈钢 | Dispensing/分装 | - Wipe with 10±5kg DMF. /使用10±5kg DMF擦洗。 - Wipe with 10±5kg MeOH. /使用10±5kg甲醇擦洗。 - Dry by compressed gas. /压缩气体吹干。 - Change gloves./更换手套。 | Perform Level I cleaning between batchs (next batch）. /批次间执行Level I的清洁（下一个批次）。 | Level I | Change gloves when switching between different batches, no need level II test./不同批次间需更换手套，无需进行level Ⅱ检测。 | N/A |
| Glass collection bottle/玻璃取样瓶 | Sampling/取样 | - After each sampling use, the glass collection bottle is not cleaned; a new glass collection bottle is replaced for the next sampling. /每次取样使用后，玻璃收集瓶不进行清洁；下次取样时更换新的玻璃取样瓶。 | N/A | N/A | N/A | N/A |

Instruction/说明：

- Define of different Cleaning verification class/不同清洁确认级别定义：

Level 0/B (Batch to batch flush): The equipment shall be simply flushed in-between batches prior to use. Process chemist or production manager/assistant engineer shall verify the equipment available after cleaning. / Level 0/B（批与批之间的冲洗）：设备在批次间连续使用前进行简单的冲洗。清洗后项目人或者生产经理/助理工程师应对设备状态进行确认。

Level I (in-situ minor cleaning): Visual check on the inner surface of the equipment body after cleaning to confirm it is free of any material residue under adequate light./Level I（原位小清洁）：设备清洗后，在足够的光源下对设备内表面的目视检查，确认无任何物料残留。

Level I (disassemble major cleaning): Visual check on the inner surface of the equipment body, components, pipelines as well as fittings after cleaning to confirm it is free of any material residue under adequate light./ Level I（拆卸大清洁）：设备清洗后，在足够的光源下对设备内表面，部件，管路和配件的目视检查，确认无任何物料残留。

- For batch to batch flush and the within campaign in-situ minor cleaning, the <Equipment Cleaning Method> should be numbered as “CMb-site code+product code+ Step No.+ version number (2 digits)”. e.g.: The first version of the equipment cleaning method of CPo1234-01 step2 is CMb-TJ211123402.01. /对于项目内批次间和步骤间的原位小清洗方案编号方式为：CMb-厂区代码+产品代码+步骤号+2位版本号，如CPo1234-01第2步第一个版本的设备清洗方法编号为CMb-TJ211123402.01；

For the within campaign disassemble major cleaning and the end of production cleaning, the <Equipment Cleaning Method> should be numbered as “CMa-site code+ product code + Step No.+ version number (2 digits)”. e.g.: The first version of the equipment cleaning method of CPo1234-01 step2 is CMa-TJ211123402.01./ 对于项目内的拆卸大清洁和生产结束后的清洗方案编号方式为：CMa-厂区代码+产品代码+步骤号+2位版本号，如CPo1234-01第2步第一个版本的设备清洗方法编号为CMa-TJ211123402.01；

For equipment qualification, the <Equipment Cleaning Method> should be numbered as “CMq-validation protocol / test run protocol number +version number”. e.g.: The first version of the equipment cleaning method of SVP-TJ1-101.01-001 is CMq- SVP-TJ1-101.01-010 v.1 /对于设备验证清洗方案编号方式为：CMq-验证文件编号+版本号，如验证文件SVP-TJ1-101.01-010使用设备清洗方法编号为CMq-SVP-TJ1-101.01-010 v.1。

For others, the <Equipment Cleaning Method> could be numbered as “CMa-usage-site+Sequence No.+version number”. E.g.: At TJ1, the first version of the equipment cleaning method of waste disposal is CMa-waste disposal-TJ101v.1 /对于其他的清洗方法，可以按照如下方式编号：CMa-用途-厂区+序号+版本号。如CMa-waste disposal-TJ101v.1代表天津1厂区2第一份用于三废处理的清洗方法。

- The selection of cleaning solvents/清洗溶剂的选择：

| 3.1 | Whether react with the equipment material/是否与设备材质发生反应 | ☐ Yes/是 ☒ No/否 |
| --- | --- | --- |
| 3.2 | Whether react with target material/是否与目标物发生反应 | ☐ Yes/是 ☒ No/否 |
| 3.3 | Whether target material dissolved well in the solvent/是否对目标物有较好的溶解性 | ☒ Yes/是 ☐ No/否 |
|  | The cleaning solvent does not affect the test method./清洗溶剂不影响测试方法的使用 | ☒ Yes/是 ☐ No/否 |
|  | Reference solubility data/溶解度参考数据 Target/ 目标物; Cleaning solvent/ 清洁溶剂; Solubility/溶解性 (1g target can soluble in XX ml XX solvent at XX oC / 1g 目标物在什么温度可溶于多少ml什么溶剂中). N/A; N/A; N/A. For equipment of complex structure, for example agitating dryer, the target should exhibit good solubility in the selected cleaning solvent (>1g/10ml) 对于结构复杂的设备，比如搅拌干燥器，目标物在选定的清洁溶剂中应该有较好的溶解性(>1g/10ml) There is an reference substance for the target清洁检测目标物是否有对照品 ☐ Y/是 reference substance lot number对照品批号____N/A_________☐ Currently no, please indicate when available/暂时没有，请提供对照品可以获得的日期：_______N/A_____ If reflux/rinse cleaning method is used, is the above solubility covered in the worst case of solubility in the recovery rate study? 如果使用了回流/淋洗的清洁方式，上面的溶解度数据是否包含在了最差溶解度情况下的回收率研究中？ ☐ Yes, no additional reflux recovery study is needed / 是，不需要额外的回流回收率研究； ☐ No, not covered, extend study of the worst case of solubility in the recovery rate should be done, responsibly person and target finish date / 不，没有包含，需要扩展最差溶解度情况下的回收率研究，负责人和目标完成时间：_______________________ Confirmed by Project analysis manager & date / 项目分析经理确认/日期：________________________________________________________ |

- Assessment of the detergent 清洁剂评估： No detergent will be used during the equipment cleaning. /设备清洗过程中将不会使用清洁剂。

Project Leader & Date / 项目主管签字及日期：

Production Manager & Date / 生产经理签字及日期：

QA Project Manager & Date / QA项目经理签字及日期：