ELI LILLY AND ASYMCHEM

API External Manufacturing (APIEM)

Gap Assessment and Risk Assessment

For Technology Transfer

| Document Title | Gap Assessment / Risk Assessment for Technology Transfer of LSN4182924 / ASYM-140020 from Full Scale Non-saleable Batch to PPQ Batches at Asymchem Life Science (Tianjin) Co., Ltd., |
| --- | --- |
| Document ID | GA-140020-0301.01 |
| Product / Lilly ID | LSN4182924 / ASYM-140020 |
| Sending Site (Current Process) | Asymchem Life Science (Tianjin) Co., Ltd |
| Receiving Site (CMO) | Asymchem Life Science (Tianjin) Co., Ltd |
| Commercialization Phase | Process Performance Qualification / Commercial production |

Approval and Signatures

This document requires review and approval by the roles listed below prior to commencement of process validation or PPQ activities at the Asymchem Life Science (Tianjin) Co., Ltd, (No.265, South Avenue, Binhai New Area TEDA, Tianjin, 300462, China) (Referred to as TJ4). Approval is required from both sending and receiving site representatives. Data Integrity review is also required.

| Prepared by |  |  |
| --- | --- | --- |
| Pengda Zhao (Associate Director, CMMD) | Signature &Date |  |
| Reviewed by |  |  |
| Liang Li (Senior Director, CMMD) | Signature &Date |  |
| Yongle Liu (Director, Dept. of Analytical Research and Development) | Signature &Date |  |
| Haini Wang (Senior Director, QC) | Signature &Date |  |
| Zhongyao Zhang (Director, Production) | Signature &Date |  |
| Lu Li (Director, QA) | Signature &Date |  |
| Approved by |  |  |
| Mingyang Yang (Executive Director, QA) | Signature &Date |  |

This document is reviewed and approved by Eli Lilly and Company. Refer to signature page for EDMS signatures and dates for Eli Lilly and Company approval.

Version History

| Version | Date | Summary of Changes |
| --- | --- | --- |
| 01 | The last signature date is defined as the effective date | The first version |

Abbreviations and Definitions	7

1. Report Summary	8

2. Scope and Purpose	9

3. Gap Assessment / Risk Assessment	10

3.1 Process Chemistry	10

3.2 Process Changes	11

3.3 Process Equipment	13

3.4 Specifications and Analytical Methods	17

3.4.1 Raw Material Specifications	17

3.4.2 In-Process Control (IPC) Items and Methods	22

3.4.3 Analytical Methods	24

3.5 Cleaning Validation	30

3.6 Technology Transfer Gap Assessment (by Area)	32

4. Summary Risk Mitigation / Action Plan	47

5. Raw Material Supplier Information	52

6. Personnel	54

7. Cleaning	54

8. Storage and Handling Conditions	55

9. Packaging Materials	55

10. Conclusion	55

11. References	57

12. Appendix I Risk Mitigation Measures/Action Plan Tracking Table	58

# Abbreviations and Definitions

The following abbreviations and definitions apply throughout this document.

| Abbreviation / Term | Definition |
| --- | --- |
| APIEM | API External Manufacturing (Eli Lilly) |
| AR | Analytical Research lab |
| cGMP | Current Good Manufacturing Practice |
| CM | Contract Manufacturer |
| PLC | Programmable Logic Controller |
| DCM | Dichloromethane |
| DIC | N,N'-Diisopropylcarbodiimide |
| DIPEA | Ethyldiisopropylamine |
| DMF | N,N-Dimethylformamide |
| DS | Drug Substance |
| EHS | Environment, Health and Safety |
| EA | Ethyl acetate |
| ESA | Equipment Suitability Analysis |
| Fmoc | 9-Fluorenylmethyloxycarbonyl |
| IPAc | Isopropyl acetate |
| IPC | In-Process Control |
| LDPE | Low density polyethylene |
| LOD | Loss on Drying |
| MOC | Material of Construction |
| MOR | Master Operation Record |
| MeOH | Methanol |
| MTBE | Methyl tert-butyl ether |
| Oxyma | Ethyl cyano(hydroxyimino)acetate-2-oxime |
| OEB | Occupational exposure band |
| OOS | Out of Specification |
| PFD | Process Flow Document |
| PHA | Process Hazard Analysis |
| PyBOP | Benzotriazol-1-yl-oxytripyrrolidinophosphonium hexafluorophosphate |
| PPQ | Process Performance Qualification |
| QA | Quality Assurance |
| QC | Quality Control |
| RA | Risk Assessment |
| RSM | Registered Starting Material |
| SPPS | Solid Phase Peptide Synthesis |
| SS | Stainless Steel |
| TS/MS | Technical Services / Manufacturing Science |
| TFA | Trifluoroacetic acid |
| UPLC | Ultra High-Performance Liquid Chromatography |
| VMP | Validation Master Plan |
| 2-CTC | 2_Chlorotrityl Chloride Resin |

# 1. Report Summary

Title of Report

Gap Assessment / Risk Assessment for Technology Transfer of LSN4182924 / ASYM-140020 from Full Scale Non-saleable Batch to PPQ Batches at Asymchem Life Science (Tianjin) Co., Ltd.,

Lilly Number/ Name

LSN4182924

Abstract

This report provides the detailed gap assessment / risk assessment to meet the requirements of the LSN4182924 / ASYM-140020 technology transfer. The assessment documents all identified gaps or differences across the scope areas, as well as the associated risks of these gaps on drug substance quality. The purpose of this assessment is to evaluate the readiness of the manufacturing process, analytical methods, equipment, facilities, utilities, personnel, contamination control strategy and quality systems for execution of PPQ activities associated with LSN4182924. The assessment further verifies that technology transfer activities, process changes, risk mitigations and prerequisite validation activities are appropriately controlled such that process performance, product quality and patient safety are not adversely impacted. There are no changes to Participant Team, Raw Material Supplier information. Only the following optimization items are identified, which have no impact on commercial manufacturing and product Critical Quality Attributes (CQAs): The identified adjustments are categorized as follows: - Process of Step 1. The Changes are grouped into five categories (For details of related changes, refer to Change-CPo140020-030104.01): - MOR documentation and GMP enhancements (Incident Log, dual-person verification, expanded pre-charging and visual-inspection checklists, and clarified roles and records); - Process parameter refinements (minimum dissolution, activation and mixing times, visual discharge/transfer endpoints, removal of the agitation-stop step, and formal CPP/NOR designation) - The equipment adjustment introduces process-related changes - Analytical method validation upgrade and adjustment of intermediate release test method. - Other adjustments. - Process of Step 2. The Changed documented are grouped into three categories (For details of related changes, refer to Change-CPo140020-030204.01): - MOR documentation and GMP enhancements (Incident Log, dual-person verification, expanded pre-charging and visual-inspection checklists, and clarified roles and records); - . Other process changes - .The packaging operation record updated. - Equipment adjustment for PV batches: The equipment (Prepared OxymaPure/DMF Solution) is changed from the Glass-lined reactor TJ4S-1213-R21 to the Agitation tank TJ4S-1200-AT02. The equipment (Prepared Capping Solution- Ac2O/ Pyridine) is changed from Activation tank TJ4S-1213-Sy15-R01 to the Glass-lined reactor TJ4S-1213-R22. - Raw Material & Specifications - Raw material control: Raw material specifications are refined and tightened to further strengthen upstream quality control. - In-Process Control (IPC) Items of Step 1 & Step 2: 1). The analytical methods (Ninhydrin test: AM-140020-029, Bromophenol blue test: AM-140020-031 and Chloranil test: AM-140020-030) during PPQ campaign has been validated. 2). The drying endpoint LOD acceptance criterion is tightened from 10.0% to 9% (w/w). 3). The In-process intermediate specifications has been removed. 4). The product Quality Specification for LSN4182924 / ASYM140020 has been changed (Relevant contents have been filled in Change-140020030205.01). All the changes described above have no impact on product CQAs, with an overall assessed risk level of low.

# 2. Scope and Purpose

The purpose of this report is to document the detailed gap assessment / risk assessment for the technology transfer of LSN4182924 / ASYM-140020 between full scale non-saleable batch and PPQ batches as well as the following commercial manufacturing at TJ4. The scope of this assessment includes evaluation of process readiness, analytical readiness, equipment suitability, contamination control readiness, validation readiness, quality system readiness, material readiness and completion status of prerequisite PPQ activities.

Table 1: Gap Assessment Team

The cross-functional team shown below has reviewed the ASYM-141366 and process from the sending site and receiving site.

| Role | Sending Site (Asymchem) | Receiving Site (Asymchem) |
| --- | --- | --- |
| Process chemist | Pengda Zhao, Associate Director, CMMD | Pengda Zhao, Associate Director, CMMD |
| Technical Reviewer | Liang Li, Senior Director, CMMD | Liang Li, Senior Director, CMMD |
| Process Engineering | Zhongyao Zhang, Director, Production | Zhongyao Zhang, Director, Production |
| Operations | Can Zhang, Associate Director, Production | Can Zhang, Associate Director, Production |
| Quality Assurance | Lu Li, Director, QA | Lu Li, Director, QA |
| Laboratories (QC) | Haini Wang, Senior Director, QC | Haini Wang, Senior Director, QC |
| Dept. of Analytical Research and Development (ARD) | Yongle Liu, Director, ARD | Yongle Liu, Director, ARD |

Table 2: Risk Scoring Table — Severity of Harm Impact on LSN4182924 / ASYM-140020 Quality

Risk classification follows the scoring scale below. The scoring is based on impact to the quality of LSN4182924 / ASYM-140020. Risk classification assumes no action or mitigation has been implemented.

| Risk Level | Severity of Harm — Impact on the quality of LSN4182924 / ASYM-140020 |
| --- | --- |
| Low | No or low correlation with CQA based on available data or extensive prior knowledge. |
| Medium | Potentially low correlation with CQA based on prior/scientific knowledge and failure of acceptance criteria is not expected; however, data is not available or studies are ongoing. |
| High | Parameter has strong correlation with CQA, may cause exceedance of CQA limit, either based on experimental data or prior/scientific knowledge. |

The summary table in Section 4 provides the consolidated action plan with owners and milestones. Technical risks, mitigations and completion milestones will also be agreed during project Manufacturability Reviews.

# 3. Gap Assessment / Risk Assessment

This section identifies differences or gaps to be addressed during transfer of this LSN4182924 / ASYM-140020 to Asymchem TJ4. For each area, the sending site status is documented, any challenges/gaps/differences at Asymchem TJ4 are described, a risk ranking is assigned per Table 2, and a mitigation plan or action is proposed.

## 3.1 Process Chemistry

3.1.1 The synthetic scheme for ASYM-141366 is shown below.

Scheme 1. The synthetic scheme for the ASYM-141366

The synthetic route of 141366 adopted at the Asymchem TJ4 remains unchanged. The compound ASYM-141366 is an in-process product for the synthesis of LSN4182924 /ASYM-140020, which is synthesized via solid-phase synthesis, with 2-CTC Resin as the starting material; Other amino acids are sequentially coupled via coupling reactions including Fmoc-Gly-OH, Fmoc-L-Phe-OH, Fmoc-L-N-Me-Asn(Trt)-OH, Fmoc-L-Asn(Trt)-OH, Fmoc-L-Ser(tBu)-OH, Fmoc-L-Arg(Pbf)-OH, Fmoc-L-Val-OH, Fmoc-L-Leu-OH, Fmoc-L-α-Me-Phe-OH, Fmoc-L-Glu(OtBu)-OH·H2O, Fmoc-L-Ala-OH·H2O and Fmoc-L-Orn(Boc)-OH. Also, the coupling reagents, namely Oxyma, PyBOP, DIC and DIPEA, remain unchanged throughout technology transfer.

3.1.2 The synthetic scheme for the LSN4182924 / ASYM-140020 is shown below.

Scheme 2, Synthetic scheme for LSN4182924 / ASYM-140020

The product LSN4182924 / ASYM-140020 is obtained by soft cleavage (cocktail of TFA/EA) from the 2-CTC resin.

3.2 Process Changes

Asymchem is pleased to present a development report detailing the optimized processes for Step1~2 in support of the 3rd campaign (API-8) PAR study and manufacture of LSN4182924 / ASYM-140020.

3.2.1 Process Changes for ASYM-141366 (Step 1)

The changes documented  are grouped into five categories: (1) MOR documentation and GMP enhancements (Incident Log, dual-person verification, expanded pre-charging and visual-inspection checklists, and clarified roles and records), which do not alter process steps or parameters; (2) process parameter refinements (minimum dissolution, activation and mixing times, visual discharge/transfer endpoints, removal of the agitation-stop step, and formal CPP/NOR designation), each supported by the PAR study and full-scale batch experience and kept within demonstrated ranges; (3) The equipment adjustment introduces process-related changes including: The equipment flow chart has been updated, and the post-capping DMF wash step using DMF is replaced by direct charging into the synthesis reactor TJ4S-1213-Sy15. The stirring time for the Ac₂O/Pyridine/DMF solvent mixture is adjusted to at least 5 min; (4) Analytical method validation upgrade and adjustment of intermediate release test method. Clear quantitative requirements are specified for both sample handling procedures and test result acceptance criteria for IPC testing (Document No. AM-140020-029, AM-140020-030, AM-140020-031), and the sample handling procedure for substitution degree testing also be adjusted. Lastly, the acceptance criteria for the in-process product have been removed. Sampling for release testing is no longer necessary. Instead, process chemists shall visually verify the material color and document the findings within the MOR. (5) Other adjustments. The output batch size has been changed to 105.63 kg - 278.62 kg, due to ensure the robustness and rigor of process‑validation data. During collection, the inner bags are changed to powder feeding bags, which allow direct charging in Step 2 of CPo140020-03 PPQ batches to avoid repackaging. Furthermore, there are additional adjustments, including: DIPEA is added over a reference duration of 20 ± 10 min (target: 20 min) during the MeOH capping step, The first sampling time for the dried material during drying is extended to 16 h ± 2h.

On the basis of the PAR, risk assessments, full-scale batch and solvent-run evidence, none of the changes adversely affects product CQAs, impurity profile, yield, cleaning, stability or PPQ success; the revised parameters and methods will be specifically monitored and trended across the three PPQ batches as the change-effectiveness check. For details of related changes, refer to Change-CPo140020-030104.01

3.2.2 The Process Changes for LSN4182924/ASYM-140020 (Step 2)

The changes documented are grouped into three categories. (1) MOR documentation and GMP enhancements (Incident Log, dual-person verification, expanded pre-charging and visual-inspection checklists, and clarified roles and records), which do not alter process steps or parameters; (2) Other process changes. The batch size range has been changed to 67.62 - 154.82 kg. A rinse operation has been added in the sodium chloride solution preparation stage; The settling time for liquid separation in the MOR is adjusted According to the actual production time of the full-scale and Demo batch; Pressure filtration may be performed directly without settling after the final transfer as appropriate in the Filter of the crystallization system: The storage and shipping temperature for product LSN4182924/ASYM140020 is adjusted to 2 ~ 8 °C. (3) The packaging operation record for ASYM140020 has been upgraded from MPR-TJ414002002.03 to MPR-TJ414002002.04.

Parameters also will be revised based on the PAR study. Based on the PAR, risk assessments, full-scale batch and solvent-run evidence, the proposed changes will not adversely affect product CQAs, impurity profile, yield, cleaning, stability or PPQ success. The revised parameters and methods will be specifically monitored and trended across the three PPQ batches as a change effectiveness check. For details of related changes, refer to Change-CPo140020-030204.01

3.3 Process Equipment

3.3.1 The process equipment for ASYM-141366 (Step 1)

The detailed information of MOR number and batch size for Step 1 between The Full Scale non-saleable batch and The PPQ & Commercial Batches is listed in table 3. The process equipment proposed for the PPQ campaign is listed in table 4 and compared with the Full Scale non-saleable batch equipment. The step 1 Equipment Suitability Analysis (ESA) has been completed and documented in Equipment Suitability Assessment Report < RA-140020030101.01>, in which all equipment suitability conclusion required for PPQ  have been clearly defined, and this document will be approved  prior to the PPQ batch.

Table 3: The Batch Gap of MOR Number and Batch Size for Step 1

| Batch Gap | The Full Scale non-saleable batch | The PPQ & Commercial Batches |
| --- | --- | --- |
| MOR | MOR-TJ414002001.03 | MOR-TJ414002001.04 |
| Batch size | 170.00 kg – 190.00 kg | 105.63 kg – 278.62 kg |

Table 4: The Step 1 Process Equipment Gap Comparison

| Intended Purpose | Full Scale non-saleable batch Equipment Code | Capacity | MOC* | PPQ Equipment Code | Capacity | MOC* | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Peptide synthesis, Fmoc deprotection, Wash resin, Drying | TJ4S-1213-Sy15 | 3000 L | Hastelloy C-276 | TJ4S-1213-Sy15 | 3000 L | Hastelloy C-276 | No gap — same equipment. |
| Activation tank | TJ4S-1213-Sy15-R01 | 1500 L | Hastelloy C-276 | TJ4S-1213-Sy15-R01 | 1500 L | Hastelloy C-276 | No gap — same equipment. |
| Storage of Prepared De-Fmoc Solution | TJ4S-1213-SPS01-AT01 | 3000 L | SS 304L (S30408) | TJ4S-1213-SPS01-AT01 | 3000 L | SS 304L (S30408) | No gap — same equipment. |
|  | TJ4S-1213-SPS01-AT02 | 3000 L | SS 304L (S30408) | TJ4S-1213-SPS01-AT02 | 3000 L | SS 304L (S30408) | No gap — same equipment |
| Prepared OxymaPure/ DMF Solution | TJ4S-1213-R21 | 200 L | Glass-lined | TJ4S-1200-AT02 | 500 L | Hastelloy C-276 | The equipment is changed from glass-lined reactor to stirred tank. The equipment working volume has been changed from 200 L to 500 L, and the primary raw material has been switched from Glass-lined to Hastelloy C-276. / The new equipment has good material compatibility, with minimum stirring volume compliant with process demands. |
| Prepared Capping (Ac2O/ Pyridine) Solution | TJ4S-1213-Sy15-R01 | 1500 L | Hastelloy C-276 | TJ4S-1213-R22 | 1000 L | Glass-lined | The equipment is changed from Activation Tank to glass-lined reactor. The equipment working volume has been changed from 1500 L to 1000 L, and the primary raw material has been switched from C-276 to Glass-lined./ The new equipment has excellent material compatibility, and its stirring volume meets the process requirements |

* MOC = Material of Construction   S30408 = 30408 Stainless Steel C-276 = Hastelloy

Conclusion: The input quantity remains unchanged, while the batch‑size value has been revised with an expanded range. The equipment (Peptide Synthesis, Fmoc Deprotection, Resin Wash, Drying, Activation Vessel, Storage of Prepared De-Fmoc Solution) used for Full scale non saleable batches and PPQ batches is identical. The equipment (Prepared OxymaPure/DMF Solution) is changed from the Glass-lined reactor TJ4S-1213-R21 to the Agitation tank TJ4S-1200-AT02. The equipment (Prepared Capping Solution- Ac2O/ Pyridine) is changed from Activation tank TJ4S-1213-Sy15-R01 to the the Glass-lined reactor TJ4S-1213-R21.The replaced equipment can satisfy the requirements for solution preparation in the process with regard to working volume and minimum agitation volume and complies with process requirements. Reference the Equipment Suitability Assessment report (RA-140020030101.01).

3.3.2 The Process Changes for LSN4182924/ASYM-140020 (Step 2)

The detailed information of MOR number and batch size for Step 2 between The Full Scale non-saleable batch and The PPQ & Commercial Batches is listed in table 5. The process equipment proposed for the PPQ campaign is listed in table 6 and compared with the Full Scale non-saleable batch equipment. The Step 2 Equipment Suitability Analysis (ESA) and report are in preparation and will be completed by Asymchem prior to the step 2 PPQ campaign.

Table 5: The Batch Gap of MOR Number and Batch Size for Step 2

| Batch Gap | The Full Scale non-saleable batch | The PPQ & Commercial Batches |
| --- | --- | --- |
| MOR | MOR-TJ414002002.03 | MOR-TJ414002002.04 |
| Batch size | 90 kg - 140 kg | 67.62 - 154.82 |

Table 6: The Process Equipment Proposed GAP for Step 2

| Intended Purpose | Full Scale non-saleable batch Equipment Code | Capacity | MOC* | PPQ Equipment Code | Capacity | MOC* | Comments / GAP Assessment |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Prepare neutralization solution & brine | TJ4S-1212-R05 | 1000 L | Glass-lined | TJ4S-1212-R05 | 1000 L | Glass-lined | No gap — same equipment. |
| Prepare cleavage solution | TJ4S-1212-R13 | 2000 L | Glass-lined | TJ4S-1212-R13 | 2000 L | Glass-lined | No gap — same equipment. |
| Cleavage & Filtration | TJ4S-1212-R09 | 2000 L | Glass-lined & Hastelloy C-22 | TJ4S-1212-R09 | 2000 L | Glass-lined & Hastelloy C-22 | No gap — same equipment |
| Neutralization & washing & Concentration | TJ4S-1213-R11 | 5000 L | Glass-lined | TJ4S-1213-R11 | 5000 L | Glass-lined | No gap — same equipment. |
| Precipitation | TJ4S-1213-R13 | 12500 L | Glass-lined | TJ4S-1213-R13 | 12500 L | Glass-lined | No gap — same equipment. |
| Filter & Dry | TJ4S-1212-FD05 | DN2000 | Hastelloy C-276 | TJ4S-1212-FD05 | DN2000 | Hastelloy C-276 | No gap — same equipment. |

* MOC = Material of Construction   C-276 = Hastelloy C-276   C-22 = Hastelloy C-22

Conclusion: The input range of Step 2 shall be adjusted based on equipment suitability. And the batch‑size value has been revised with an expanded range. Although the ESA report has not yet been finalized, given that the equipment used for Full scale non saleable batches and PPQ batches is identical, and its parameters meet process requirements, the residual risk associated with any outstanding ESA activities is low. The equipment remains identical; no gaps exist. Reference the Equipment Suitability Assessment report (RA-140020030201.01).

3.4 Specifications and Analytical Methods

### 3.4.1 Raw Material Specifications

The comparison of specifications and testing methods for raw materials used in the process between the Full Scale non-saleable batch and the PPQ campaign is shown in table 7.

Table 7: The Raw Material Specification Assessment

| Material Name | Full Scale non-saleable Batch Material No. | PPQ Material No. | Full Scale non-saleable batch Spec Document | PPQ Spec Document | Full Scale non-saleable batch QSV | PPQ QSV | Gaps and Justification |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2-CTC Resin | YL005205 | YL005205 | RMS-TJ414002002.05 | RMS-TJ414002003 | QAA03550 | QAA03550 | The specification test items and analytical methods are consistent; only the quality specification version differs. |
| Fmoc-Gly-OH | YL004846 | YL004846 | RMS-TJ414002002.05 | SP-138200-02 (current version) | QAA03550 | QAA03550 | The specification has been converted into independent specification documents |
| Fmoc-L-Phe-OH | YL004840 | YL004840 | RMS-TJ414002002.05 | SP-138203-02 (current version) | QAA03550 | QAA03550 | The specification has been converted into independent specification documents |
| Fmoc-L-N-Me-Asn(Trt)-OH | YL008617 | YL008617 | RMS-TJ414002002.05 | SP-138520-02 (current version) | QAA03550 | QAA03550 | The purity specification of Fmoc-L-N-Me-Asn(Trt)-OH has been adjusted from ≥98.0% to ≥99.0%. The specification of Des-Methyl impurity was adjusted from ≤0.2% to ≤0.1%. |
| Fmoc-L-Asn(Trt)-OH | YL004843 | YL004843 | RMS-TJ414002002.05 | SP-138519-02 (current version) | QAA03550 | QAA03550 | The specification has been converted into independent specification documents |
| Fmoc-L-Ser(tBu)-OH | YL003428 | YL003428 | RMS-TJ414002002.05 | SP-132732-02 (current version) | QAA03550 | QAA03550 | The specification has been converted into independent specification documents |
| Fmoc-L-Arg(Pbf)-OH | YL003424 | YL003424 | RMS-TJ414002002.05 | SP-138518-02 (current version) | QAA03550 | QAA03550 | The specification has been converted into independent specification documents |
| Fmoc-L-Val-OH | YL003110 | YL003110 | RMS-TJ414002002.05 | SP-138202-02 (current version) | QAA03550 | QAA03550 | Add detection of Fmoc-Leu-OH and Fmoc-Ile-OH in Fmoc-L-Val-OH, with specifications of ≤0.1% each. |
| Fmoc-L-Leu-OH | YL003425 | YL003425 | RMS-TJ414002002.05 | SP-138201-02 (current version) | QAA03550 | QAA03550 | The specification has been converted into independent specification documents. |
| Fmoc-L-α-Me-Phe-OH | YL008272 | YL008272 | RMS-TJ414002002.05 | SP-138210-02 (current version) | QAA03550 | QAA03550 | The purity specification of Fmoc-L-α-Me-Phe-OH has been adjusted from ≥98.0% to ≥99.0%. |
| Fmoc-L-Glu(OtBu)-OH·H2O | YL005741 | YL005741 | RMS-TJ414002002.05 | SP-138390-02 (current version) | QAA03550 | QAA03550 | The specification has been converted into independent specification documents |
| Fmoc-L-Ala-OH·H2O | YL006160 | YL006160 | RMS-TJ414002002.05 | SP-138204-02 (current version) | QAA03550 | QAA03550 | The specification has been converted into independent specification documents. |
| Fmoc-L-Orn(Boc)-OH | YL005318 | YL005318 | RMS-TJ414002002.05 | SP-124852-02 (current version) | QAA03550 | QAA03550 | The purity specification of Fmoc-L-Orn(Boc)-OH have been adjusted from ≥98.0% to ≥99.0%. |
| Oxyma pure | YL005180 | YL005180 | RMS-TJ414002002.05 | RMS-TJ414002003 | QAA03550 | QAA03550 | Purity ≥ 99.0%. No GAP; |
| N,N'-Diisopropylcarbodiimide | YL003564 | YL003564 | RMS-TJ414002002.05 | RMS-TJ414002003 | QAA03550 | QAA03550 | Purity ≥ 99.0%. No GAP |
| PyBop | YL002855 | YL002855 | RMS-TJ414002002.05 | RMS-TJ414002003 | QAA03550 | QAA03550 | Purity ≥ 99.0%. No GAP |
| Acetic anhydride | SJ000213 | SJ000213 | RMS-TJ414002002.05 | RMS-TJ414002003 | QAA03550 | QAA03550 | Purity ≥ 99.0%. No GAP |
| Pyridine | YL000617 | YL000617 | <Material Specification sheet >QM02018 | RMS-TJ414002003 | QAA03550 | QAA03550 | Purity ≥ 99.0%; Water content ≤0.2% w/w. No GAP |
| Methanol | YL000031 | YL000031 | <Material Specification sheet >QM02018 | RMS-TJ414002003 | QAA03550 | QAA03550 | Purity ≥ 99.0%; Water content ≤0.2% w/w. No GAP |
| Piperidine | YL001394 | YL001394 | <Material Specification sheet >QM02018 | RMS-TJ414002003 | QAA03550 | QAA03550 | Purity ≥ 97.0%; Water content ≤0.2% w/w. No GAP |
| Ethyldiisopropylamine | YL000555 | YL000555 | <Material Specification sheet >QM02018 | RMS-TJ414002003 | QAA03550 | QAA03550 | Purity ≥ 99.0%; Water content ≤0.1% w/w. No GAP |
| N,N-Dimethylformamide | YL000336 | YL000336 | <Material Specification sheet >QM02018 | RMS-TJ414002003 | QAA03550 | QAA03550 | Purity ≥ 99.0%; Water content ≤0.1% w/w. No GAP |
| Isopropyl acetate | YL000932 | YL000932 | <Material Specification sheet >QM02018 | RMS-TJ414002003 | QAA03550 | QAA03550 | Purity ≥ 99.0%; Water content ≤0.1% w/w. No GAP |
| Toluene | YL000026 | YL000026 | <Material Specification sheet >QM02018 | RMS-TJ414002003 | QAA03550 | QAA03550 | Purity ≥ 99.0%; Water content ≤0.1% w/w. No GAP |
| tert-Butyl methyl ether | YL000557 | YL000557 | <Material Specification sheet >QM02018 | RMS-TJ414002003 | QAA03550 | QAA03550 | Purity ≥ 99.0%; Water content ≤0.1% w/w. No GAP |
| Ethyl acetate | YL000025 | YL000025 | <Material Specification sheet >QM02018 | RMS-TJ414002003 | QAA03550 | QAA03550 | Purity ≥ 99.0%; Water content ≤0.1% w/w. No GAP |
| n-Heptane | YL000474 | YL000474 | <Material Specification sheet >QM02018 | RMS-TJ414002003 | QAA03550 | QAA03550 | Purity ≥ 99.0%; Water content ≤0.1% w/w. No GAP |
| Trifluoroacetic acid | YL000419 | YL000419 | <Material Specification sheet >QM02018 | RMS-TJ414002003 | QAA03550 | QAA03550 | Assay ≥ 98.0%; No GAP |
| Sodium chloride | YL000493 | YL000493 | RMS-TJ414002002.05 | RMS-TJ414002003 | QAA03550 | QAA03550 | No GAP |

Conclusion:

1. All amino acid specification (Fmoc-Gly-OH, Fmoc-L-Phe-OH, ,Fmoc-L-N-Me-Asn(Trt)-OH, Fmoc-L-Asn(Trt)-OH, Fmoc-L-Ser(tBu)-OH, Fmoc-L-Arg(Pbf)-OH, Fmoc-L-Val-OH, Fmoc-L-Leu-OH, Fmoc-L-α-Me-Phe-OH, Fmoc-L-Glu(OtBu)-OH·H2O, Fmoc-L-Ala-OH·H2O, Fmoc-L-Orn(Boc)-OH) have been converted into independent specification documents;

2. Tighten the purity specification for Fmoc-L-N-Me-Asn(Trt)-OH, Fmoc-L-α-Me-Phe-OH, and Fmoc-L-Orn(Boc)-OH, and add specific impurity detection to Fmoc-L-Val-OH; Tightening the demethylation impurity limit in Fmoc-L-N-Me-Asn(Trt)-OH ensures better quality of raw materials and has a favorable impact on downstream processes and finished products.

3. Material Specification for Dedicated Project will adjust the information according to the actual situation, adding specification for reagents. This change has no impact on product quality. Above all, this GAP does not affect the product quality.

### 3.4.2 In-Process Control (IPC) Items and Methods

a). The In-Process Control (IPC) Items for step 1 SPPS (ASYM-141366)

The comparison of IPC items and analytical methods between the Full Scale non-saleable batch and PPQ campaign is shown in table 8.

Table 8: The Step 1 SPPS IPC Items Gap Assessment

| Process Step |  | Test Item |  | Full Scale non-saleable Batch Criteria |  | PPQ Campaign Criteria |  | GAP Assessment / Method Reference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pre-treatment |  | Water content |  | ≤500 ppm |  | ≤500 ppm |  | No Gap. Method: AM-YL000336-004 |
| Amino acid coupling Completion (Phe23 - NMeAsn22, Ser20 - aMePhe15, Ala13 - Orn11) |  | Ninhydrin test |  | Negative (colorless or nearly colorless) |  | Negative (the solution of sample is yellow to light pink). |  | - The method used for PPQ increased the sample weight and optimized the types and amounts of color reagents added, making the detection more sensitive and meeting the detection requirements for process tracking. Method validation is in progress and shall be completed with sign‑off prior to PPQ production. - The criteria for judging the color of sample solution are different, the modified sample solution color is consistent with the method. Method: AM-140020-029 |
| Coupling Completion (Glu14) |  | Bromophenol blue |  | Negative (colorless or nearly colorless) |  | Negative (the resin of sample is yellow to orange) |  | - The method increased the sample weight and optimized the types and amounts of color reagents added, making the detection more sensitive and meeting the detection requirements for process tracking. Method validation is in progress and shall be completed with sign‑off prior to PPQ production. - The criteria for judging the resin color of sample are different, the modified color is consistent with the method. Method: AM-140020-031 |
| Chloranil test (Leu12 Post deprotection wash piperidine residue.) |  | Chloranil test |  | Negative |  | Negative |  | Clear quantitative requirements are specified for both sample‑handling procedures, and the criteria for test result evaluation are the same. Method validation is in progress and shall be completed with sign‑off prior to PPQ production. Method: Method: AM-140020-030 |
| Chloranil test (Orn11 post coupling wash piperidine residue. |  | Chloranil test |  | Negative |  | Negative |  | Clear quantitative requirements are specified for both sample‑handling procedures, and the criteria for test result evaluation are the same. Method validation is in progress and shall be completed with sign‑off prior to PPQ production. Method: Method: AM-140020-030 |
| Drying | LOD | ≤10.0% | ≤9%(w/w) | The drying endpoint LOD acceptance criterion is tightened from 10.0% to 9%(w/w) Method: AM-140020-016 |

Conclusion:

- The methods of Ninhydrin test (Phe23 -  NMeAsn22, Ser20 - aMePhe15, Ala13 - Orn11), Bromophenol blue (Glu14) for amino acid coupling completion and Chloranil test(Leu12 Post deprotection wash, Orn11 post coupling wash)

The analytical methods (Ninhydrin test: AM-140020-029, Bromophenol blue test: AM-140020 - 031 and Chloranil test: AM-140020-030) during PPQ campaign has been validated, and clear quantitative requirements will be defined for IPC sample‑handling procedures and test‑result acceptance criteria.

- The LOD Acceptance Criterion

Based on the PAR study, the drying endpoint LOD acceptance criterion is tightened from 10.0% to 9% (w/w). The tighter criterion provides an additional margin for residual solvent and moisture control, which supports product stability and downstream process consistency.

3. Other IPC items: No gaps

b).

The In-Process Control (IPC) Items for step 2 soft cleavage in LSN4182924 / ASYM-140020

The comparison of IPC items and analytical methods between the Full Scale non-saleable batch and PPQ campaign is shown in table 9.

Table 9: The Step 2 Soft Cleavage IPC Items Gap Assessment

| Process Step | Test Item | Full Scale non-saleable Batch Criteria | PPQ Campaign Criteria | GAP Assessment / Method Reference |
| --- | --- | --- | --- | --- |
| Separation before purified water wash | pH value | 5 - 7 | 5 - 7 | No Gap. Method: AM-140020-018 |
| Dry | KF | Two consecutive samples both ≤1.8% | Two consecutive samples both ≤1.8% | No Gap. Method: AM-140020-023 |

Conclusion: No gaps exist in the in-process control items or methods for step 2 soft cleavage of LSN4182924 / ASYM-140020 between the Full Scale non-saleable batch and PPQ campaign.

3.4.3 Analytical Methods

a). The In-Process Control (IPC) Method Validation

The completion status of method transfer/validation for step 1~2 IPC methods is shown in Table 10.

Table 10: The Summary of IPC Method Validation

| Step No | PPQ Batches Method | Validation Protocol | Validation experiment | Validation report |
| --- | --- | --- | --- | --- |
| Step1 | AM-YL000336-004.03 | AM-YL000336-004-VP-01.01 (Effective Date: 05/29/2026) | Completed | AM-YL000336-004-VR-01.01 (Effective Date: 09/08/2026) |
|  | Establish a dedicated method for PPQ batch: AM-140020-029.02 (Ninhydrin test) | AM-140020-029-VP-01.01 (Effective Date: 07/01/2026) | Completed | AM-140020-029-VR-01.01 (Effective Date: 09/10/2026) |
|  | Establish a dedicated method for PPQ batch: AM-140020-031.02 (Bromophenol blue) | AM-140020-031-VP-01.01 (Effective Date: 08/12/2026) | Completed | AM-140020-031-VR-01.01 (Effective Date: 09/03/2026) |
|  | Establish a dedicated method for PPQ batch: AM-140020-030.02 (Chloranil test) | AM-140020-030-VP-01.01 (Effective Date: 07/15/2026) | Completed | AM-140020-030-VR-01.01 (Effective Date: 09/07/2026) |
|  | AM-140020-016.05 | AM-140020-016-VP-01.01 (Effective Date: 07/08/2026) | Completed | AM-140020-016-VR-01.01 (Effective Date: 09/14/2026) |
| Step2 | AM-140020-018.03 | N/A | Completed | N/A |
|  | AM-140020-023.03 | AM-140020-023-VP-01.01 (Effective Date: 07/08/2026) | Completed | AM-140020-023-VR-01.01 ( 09/14/2026) |

Conclusion:

The validation protocol and experimental work for IPC test methods have been completed. Thus, there is no risk.

b). The in-process Intermediate Specification and Release Method Validation

The comparison of in-process intermediate specifications between the Full Scale non-saleable batch and PPQ campaigns is shown in Table 11.

Table 11: In-process intermediate Release Specification Comparison and Method Validation

| Test Items |  | Current spec for full scale |  | Proposed spec for PPQ |  | Validation Protocol | Validation experiment | Validation report |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Method | Acceptance Criteria | Method | Acceptance Criteria |  |  |  |
| Description | Appearance | AM-275 | Light yellow to yellow solid | N/A | N/A | N/A | N/A | N/A |

Conclusion: In-process intermediate specifications has been removed. Sampling for release testing is no longer necessary. Instead, process chemists shall visually verify the material color and document the findings within the MOR.

c).The Product Specification and Release Method Validation

The comparison of Product Quality Specifications for LSN4182924 / ASYM-140020 between the Full Scale non-saleable batch and PPQ campaigns is shown in table 12

Table 12: The Product Quality Specifications Comparison and Method Validation

| Test Item |  | Full Scale non-saleable batch |  |  | PPQ Campaigns |  |  |  | Validation Protocol | Validation experiment | Validation report |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Method | Acceptance Criteria |  | Method | Acceptance Criteria |  |  |  |  |  |
| Description | Appearance | AM-140020-006 | White to yellow solid |  | AM-140020-006 | White to yellow solid |  |  | N/A | N/A | N/A |
| Identification | Identification by UHPLC | AM-140020-001 | Conforms to the reference standard |  | AM-140020-001 | Conforms to the reference standard |  |  | AM-140020-001-VP-02.01 | Under client review | Completion prior to PV Step2 start |
|  | Identification by MS | AM-013 | Consistent with the exact mass |  | AM-140020-025 | Consistent with the exact mass |  |  | AM-140020-025-VP-01.01 | Completed | Completion prior to PV Step2 star |
| Tests | Purity and related impurities by UHPLC | AM-140020-001 | Purity | ≥ 90.0% | AM-140020-001 | Purity |  | ≥ 90.0% area | AM-140020-001-VP-02.01 | Under client review | Completion prior to PV Step2 start |
|  |  |  | Total impurities | ≤ 10.0% |  | Total impurities |  | ≤ 10.0% area | AM-140020-001-VP-02.01 | Under client review | Completion prior to PV Step2 star |
|  |  |  | Any individual impurities | ≤ 1.0% |  | Any individual impurities (Report individual impurities ≥ 0.10% by RRT) |  | ≤ 0.5% area | AM-140020-001-VP-02.01 | Under client review | Completion prior to PV Step2 star |
|  |  |  | Report individual impurities ≥ 0.10% by RRT |  |  | Individual specified impurities | ASYM-153207 (L2 endo-Gly24) | ≤ 0.5% Area | AM-140020-001-VP-02.01 | Under client review | Completion prior to PV Step2 star |
|  |  |  |  |  |  |  | ASYM-152486 (L2 des-(Asn21-NMeAsn22)) | ≤ 2.0% area | AM-140020-001-VP-02.01 | Under client review | Completion prior to PV Step2 star |
|  | Water content by KF | AM-140020-005 | ≤ 2.0% w/w |  | AM-140020-005 | ≤ 2.0% w/w |  |  | AM-140020-005-VP-02.01 | Completed | AM-140020-005-VR-02.01 (Completion prior to PV Step2 star) |
|  | Residual organic solvents by HS-GC | AM-140020-003 | ≤ 0.5% |  | AM-140020-003 | ≤ 0.5% |  |  | AM-140020-003-VP-02.01 | Completed | AM-140020-003-VR-02.01 (Effective Date: 09/28/2026) |
|  |  | AM-140020-002 | Methanol (MeOH) | Report | AM-140020-002 | Methanol (MeOH) |  | Report | AM-140020-002-VP-02.01 | Completed | AM-140020-002-VR-02.01(Completion prior to PV Step2 star) |
|  |  |  | Methyl tert-Butyl Ether (MTBE) | Report |  | Methyl tert-Butyl Ether (MTBE) |  | Report | AM-140020-002-VP-02.01 | Completed | AM-140020-002-VR-02.01(Completion prior to PV Step2 star) |
|  |  |  | Isopropyl acetate (IPAc) | ≤ 5000 ppm |  | Isopropyl acetate (IPAc) |  | ≤ 5000 | AM-140020-002-VP-02.01 | Completed | AM-140020-002-VR-02.01(Completion prior to PV Step2 star) |
|  |  |  | n-Heptane (n-Hep) | Report |  | n-Heptane (n-Hep) |  | Report | AM-140020-002-VP-02.01 | Completed | AM-140020-002-VR-02.01(Completion prior to PV Step2 star) |
|  |  |  | Toluene (Tol) | Report |  | Toluene (Tol) |  | Report | AM-140020-002-VP-02.01 | Completed | AM-140020-002-VR-02.01(Completion prior to PV Step2 star) |
|  |  |  | Ethyl acetate (EA) | Report |  | Ethyl acetate (EA) |  | Report | AM-140020-002-VP-02.01 | Completed | AM-140020-002-VR-02.01(Completion prior to PV Step2 star) |
|  |  |  | N,N-Dimethylformamide (DMF) | ≤ 880 ppm |  | N,N-Dimethylformamide (DMF) |  | ≤ 880 ppm | AM-140020-002-VP-02.01 | Completed | AM-140020-002-VR-02.01(Completion prior to PV Step2 star) |
|  |  |  | N, N-Diisopropylethylamine (DIPEA) | ≤ 1500 ppm |  | N, N-Diisopropylethylamine (DIPEA) |  | ≤ 1500 ppm | AM-140020-002-VP-02.01 | Completed | AM-140020-002-VR-02.01(Completion prior to PV Step2 star) |
|  |  | AM-140020-004 | Pyridine | ≤ 5000 | AM-140020-004 | Pyridine |  | ≤ 5000 | AM-140020-004-VP-02.01 | Completed | AM-140020-004-VR-02.01(Completion prior to PV Step2 star) |
|  |  |  | Piperidine | Report |  | Piperidine |  | Report | AM-140020-004-VP-02.01 | Completed | AM-140020-004-VR-02.01(Completion prior to PV Step2 star) |
| Assay |  | AM-140020-001 | ≥ 80.0% w/w and ≤ 110.0% w/w |  | AM-140020-001 | ≥ 80.0% w/w |  |  | AM-140020-001-VP-02.01 | Completed | Completion prior to PV Step2 star |

Conclusion:  The updated the analytical method of Identification changed from AM-013 to AM-140020-025, Which specified instrument parameters, solution preparations and analysis procedure can better provide guidance for analysis; Add specified impurity in Purity and related impurities by UHPLC test item: ASYM-153207 (L2 endo-Gly24): ≤ 0.5%; ASYM-152486 (L2 des-(Asn21-NMeAsn22)): ≤ 2.0% and Update the acceptance criteria of individual impurities from ≤1.0% to ≤ 0.5% are based on historical data and PAR study documents (TR-1400200307.01); description information; The change of adding the unit of the test items, and adding the requirement of “Report individual impurities ≥ 0.10% by RRT” to the item of Any individual impurities; The Assay of product has been changed from “≥ 80.0% w/w and ≤ 110.0% w/w” to “≥ 80.0% w/w”

The above changes are based on historical data and the PAR report, have no impact on product quality. These GAP do not affect the product quality.

d) Laboratory Status

Laboratory facilities and systems supporting Steps 1-2 IPC and PPQ testing have been reviewed. All critical analytical instruments are qualified (IQ/OQ/PQ completed) and within valid calibration periods. Laboratory environment (temperature, humidity, fume hoods) meets the requirements for the intended analytical procedures. Consumables including chromatography columns, sample vials, filters and solvents are available and released for use. Laboratory data acquisition systems are accessible and configured properly for data recording, electronic traceability and GMP compliance.

e) Analyst Qualification

All analysts assigned to perform Steps 1-2 IPC and PPQ testing have completed required GMP training, the training of methods and hands on practical exercises for the relevant test methods will be completed prior to usage of the methods. Analyst qualification assessments (proficiency tests) have been conducted. Only analysts with successfully passed qualification are authorized to perform the corresponding IPC and PPQ sample testing. Training records and qualification documentation are maintained on file. No unqualified personnel will conduct testing activities for PPQ campaign.

f) Reference standard qualification status

All reference standards used for Steps 1–2 IPC and PPQ testing have been qualified and are within their valid expiry periods. Sufficient inventory is available to support the entire PPQ campaign. The only exception is the dimer reference standard, which is currently under preparation. The preparation progress of this reference standard has been included in the action plan with a defined owner and target date, and does not affect PPQ initiation. No testing activities relying on this reference standard will be initiated until it is ready.

3.5 Cleaning Validation

Table 13: The Gap Analysis for Cleaning Validation

| Focus Area | Current Status | Gap Analysis | Risk |
| --- | --- | --- | --- |
| Current cleaning validation status | Step 1 CVP under revision; Step 2 CVP drafts pending review | CVPs not yet approved | Medium |
| Outstanding development activities | Solubility testing of target residues in candidate cleaning solvents has been completed | The cleaning frequency for Steps 2 requires further discussion. | Low |
| ARL/PDE strategy | All determined | No Gap | Low |
| Analytical method readiness | Cleaning detection method validation completed for step 1 Cleaning detection method development and validation completed based on methanol as the cleaning solvent for step 2 | As the cleaning solvent needs to be changed to process solvent, the cleaning detection methods for Steps 2 need to be re-developed and re-validated | Medium |
| Dirty hold time | All determined | No Gap | Low |
| Clean hold time | All determined | No Gap | Low |

Cleaning validation is a critical component of the contamination control strategy and PPQ readiness. The project consists of two steps. The Step 1 CVP is under revision, and the Step 2 CVP have been drafted and are pending review. Both two CVPs will be approved prior to PPQ execution. Cleaning validation will be executed concurrently with PPQ, and the CVR will be completed prior to product release. The outstanding development activities mainly involve the development and validation of cleaning detection methods for Steps 2 following the change of cleaning solvent. All gaps have been incorporated into the action plan with defined owners and target dates, and no adverse impact on PPQ execution or product quality is anticipated.

3.6 Technology Transfer Gap Assessment (by Area)

The table below provides the structured gap assessment across all key technology transfer areas. Risk classification follows Table 2.

Per Table 2, risk ranking assumes no mitigation has been implemented. A 'Low' ranking is assigned only where available data or extensive prior knowledge demonstrate no or low correlation with CQAs, independent of planned future activities; the listed activity (e.g., routine training, document update) is a procedural prerequisite under the established quality system for an unchanged process, not a control for a new hazard. Items that rely on not-yet-completed validation, qualification or assessment to control risk are ranked Medium and tracked in Section 4 until completed.

Table 14 The Structured Gap across All Key Technology Transfer Areas and Risk classification Assessment

| Area Evaluated | Sending Site (ASYM / Current Process) | Receiving Site Status — Challenges, Gaps or Differences | Risk to product Quality | Report Reference, Risk Ranking Rationale, Mitigation Plan and/or Action |
| --- | --- | --- | --- | --- |
| Due Diligence |  |  |  |  |
| Quality Due Diligence Assessment (QDDA) | Provides outcomes and conclusions of the QDDA with any gaps and CAPAs identified Lilly GQAAC pre-commercialization audit completed | Relevant personnel shall be present on-site to accompany auditors and respond to all relevant inquiries. Relevant documents and attachments shall be provided promptly upon request. Timely follow-up and confirmation shall be performed to verify resolution of all issues, preventing omissions in communication records. The QA organization shall provide the corrective action plan to the customer within 30 calendar days and timely update the customer on the progress of corrective actions. | Low | Risk ranking rationale: QDDA completed; the CAPAs are administrative and have no process/CQA impact; per Table 2 there is no or low correlation with CQA Risk ranking rationale: QA shall archive all relevant audit documents Corrective Actions and Preventive Actions Management (QA-009) |
| Facility |  |  |  |  |
| Facility Design, Capacity and Environmental Controls | Describe capacity adequacy, room classifications, process/material/waste flows, contamination control strategy (CCS) Any special temperature, humidity or light protection requirements Cross-contamination controls | The TJ4 site adopts cGMP-compliant facility design, with sufficient production space, qualified utility systems and production capacity to fully support the SPPS manufacturing activities. (SMF-TJ4-001) | Low | Risk ranking rationale: facility qualified and in routine use; no facility change; no CQA impact. Site Master File for Asymchem Life science (Tianjin) Co., Ltd (SMF-TJ4-001) Risk Assessment Report for Cross-contamination Control strategy in LSN4182924/ASYM-140020 production (RA-140020030103.01) |
| Raw material; in-process intermediate ASYM-141366 / LSN4182924 / product LSN4182924 / ASYM-140020 Storage | Storage temperature and humidity Room classification Light protection On-site or off-site Capacity | Materials shall be stored in accordance with relevant specifications to ensure they are maintained under appropriate storage conditions and safeguard product quality. The process intermediate will be stored in accordance with relevant specifications. | Low | Risk ranking rationale: storage follows existing qualified specifications; no change to storage conditions Storage condition monitoring of storage area and equipment (DS) (DS-MM-008) |
| Utilities |  |  |  |  |
| Water | Grade of water used in each process step Minimum water grade required by control strategy Water specifications | The purified water system shall be disinfected and sterilized in accordance with relevant procedures to reduce the risk of microbial contamination within the water system and ensure the water quality meets process requirements. | Low | Risk ranking rationale: existing qualified water system; no change; no CQA impact. Disinfection procedure of water system (DS) (DS-GMP-019) Procedure for in-house made water system monitoring and service instruction (DS) (DS-GMP-020) |
| Process Gases (Nitrogen.) | Filtered and monitored process gases used Nitrogen for SPPS synthesizer, storage tank inertion, transfer lines | The nitrogen system distribution, nitrogen pipeline filtration, replacement frequency of fittings and equipment maintenance shall be implemented in accordance with relevant procedures. | Low | Risk ranking rationale: existing qualified nitrogen system; no change; no CQA impact. Nitrogen Distribution System Management Procedure (GMP-285) |
| Heat Transfer Fluids / Plant Steam | Heat transfer fluids used in vessel jackets Qualified temperature range Expected variability around temperature set points | A utility system management SOP is established at the site to guarantee uninterrupted smooth production in case of system malfunctions. | Low | Risk ranking rationale: existing qualified utility system; no change; no CQA impact. Power supplying system management procedure in TJ4 site (DS) (DS-GMP-065) |
| Personnel and Safety |  |  |  |  |
| Batch Scheduling / Manufacturing Shifts | Describe shift schedule at sending site Confirm impact of shift schedule on hold times, contact time limits | Asymchem shift schedule (8 h/day, 6 days/week * 4 four-shift operation, rotating 24-hour shifts with personnel on duty every day) Pre-shift briefings are held before taking up posts, and shift handover discipline shall be implemented via on-site handover. | low | Risk ranking rationale: shift schedule unchanged; hold/contact times unchanged; no CQA impact. Requirements for PAR and NOR parameters are specified in batch records. Pre-production training will be conducted prior to manufacturing. |
| Training, Capability and Experience | How is MOR training performed and by whom? Do operators sign a log confirming participation? Experience level of operators with this process type? | Personnel training shall be completed prior to project initiation in accordance with process validation requirements. QA shall conduct training on key procedures prior to the start of validation production. All operators shall sign the training log to confirm their participation. | Low | Risk ranking rationale: routine GMP training under the established program; process unchanged; training is a procedural prerequisite, not a control for a new hazard. Executive procedure of production projects(DS) (DS-GMP-030) Employee training (ASM-003) |
| Safety / MSDS / Process Safety Information (PSI) | SDS / MSDS available for all materials? Process Safety Information (PSI) / GIPSM available? PHIS and PHA completed? Special equipment or set-up for safety purposes? | SDS documents is available for raw materials. PHA shall be completed prior to the execution of PPQ batches. And PHIS will be complete before commercial production | Medium | Procedure for Chemical Classification, Labelling and SDS Management (EHS-085-P01.V02) Process Hazard Analysis (PHA) Management Procedure (EHS-G-8.1-P02-M02.V01) |
| Cleaning and Sanitising |  |  |  |  |
| Cleaning and Sanitizing Agents | Sanitizing/cleaning agents used for equipment Product compatibility data with cleaning agents Cleaning validation carryover limits established | Appropriate cleaning agents shall be determined based on material properties in accordance with relevant SOPs; suitable carryover limits for cleaning validation shall be established according to equipment usage | Low | Risk ranking rationale: cleaning agents and carryover limits established; no change; no CQA impact. Facility disinfection procedure (DS) (DS-GMP-006) Equipment cleaning management procedure for GMP production (DS) (DS-GMP-003) |
| Cleaning Validation | Cleaning validation status for equipment dedicated to this product Carryover limits established based on product toxicology / PDE Analytical method for cleaning validation swab / rinse testing | Asymchem shall develop appropriate cleaning methods for cleaning equipment used in LSN4182924 /ASYM-140020 manufacturing during PPQ Cleaning testing activities shall be carried out in accordance with relevant cleaning procedures in Asymchem | Low | Equipment cleaning management procedure for GMP production (DS) (DS-GMP-003) Cleaning validation (DS) (DS-GMP-043) |
| Raw Materials and Registered Starting Materials |  |  |  |  |
| RSM / Critical Material Receipt and Storage | RSM receipt testing strategy (on receipt or just before use?) Sampling strategy (opening containers on receipt, or using supplier samples?) Temperature excursion assessment required? Sufficient storage capacity and alarm strategy? Light protection, humidity control, freeze/thaw areas? | The Asymchem establish standard operating procedures for the receipt, inspection, sampling, testing, disposal and distribution of production materials to ensure that raw materials remain under controlled supervision with complete and proper records. | Low | Risk ranking rationale: existing receipt SOP; no change to receipt strategy; no CQA impact. Receipt, Release and Distribution of Raw Material for Production(DS) (DS-MM-004) |
| RSM / Raw Material Vendor Qualification | Are all approved suppliers qualified per supplier qualification SOP? Vendor qualification strategy for any new suppliers CoA limits vs receipt limits — are supplier CoAs wider than Lilly receipt limits? | All suppliers have completed qualification assessment and are listed on the Approved Supplier List | Low | Risk ranking rationale: all suppliers qualified and on the approved list; no change; no CQA impact. Supplier Control (MM-046) |
| RSM / Critical Material Shipping | Air vs. sea shipment logistics Temperature-controlled shipping requirements Shipping qualification status Receipt testing strategy | Corresponding SOPs shall be followed in accordance with the different stages of materials | Low | Risk ranking rationale: existing shipping SOP; no change; no CQA impact. Transferring between sites(MM-020) Receipt, Release and Distribution of Raw Material for Production(DS) (DS-MM-004) |
| Raw Material Dispensing | Appropriate dispensing equipment available (scales, tolerances, significant digits)? Nitrogen protection or humidity control required during dispensing? Special dispensing environment required? | Asymchem has formulated relevant procedures to support the material dispensing environment, covering room clearance, dispensing room usage and daily management, inspection, calibration and operation of weighing instruments, and inert gas protection for process equipment. | Low | Risk ranking rationale: existing dispensing SOP and qualified equipment; no change; no CQA impact. Changeover in production area/room (DS) (DS-GMP-007) Sampling / Dispensing Room Management (DS) (DS-MM-016) Inert gas protection for processing equipment (GMP-272) |
| Process and Equipment |  |  |  |  |
| Equipment Qualification (IQ/OQ/PQ) | All manufacturing equipment qualified to current standards Maintenance/PM/calibration systems in place Equipment capabilities and measurement tolerances defined Sufficient qualified equipment available to meet production demand | Step 1 Equipment Suitability Analysis (ESA) Preliminary assessment has been completed, and the equipment meets the requirements of the production process. Step 2 Equipment Suitability Analysis (ESA) are in progress and will be completed prior to PPQ production for each step Equipment qualification status IQ/OQ/PQ Has completed | Low | Equipment suitability assessment (RA-140020030101.01/RA-140020030201.01) Facility/Equipment/Utility preventative maintenance management (DS) (DS-GMP-022) Validation and qualification of facility, utility and equipment (GMP-026) |
| Batch Size / Scale | Confirm no batch size increase between sending and receiving site If batch size changes: use Lilly SURI tool for scale-up risk identification Assess impact on sampling plan and processing time limits | The molar charging quantity of PPQ batches remains unchanged. | Low | Risk ranking rationale: no batch size change; no scale-up risk; no CQA impact. No change. |
| Vessel Agitation (Mixing / Agitation) | Vessel design adequate for raw material dissolution and processing Power/Volume (P/V) ratio experience Method of addition to vessel Safety implications of mixing/agitation | The agitator used for preparing Oxyma Pure/DMF solution has been changed from impeller type to propeller type. The agitator for prepared Ac₂O/Pyridine/DMF solution has been changed from double-layer four-bladed inclined paddle type to general impeller type. The relevant equipment suitability assessment has been completed | Low | Risk ranking rationale: agitator change assessed in the completed ESA; homogeneous mixtures, minimal impact; no CQA impact. Both the Oxyma/DMF solution and Ac₂O/pyridine/DMF solution form homogeneous mixtures after dissolution, and their uniformity is minimally affected by agitation types. |
| Vessel Sampling and Transfer | In-process sampling locations appropriate per APIEM-LD-243? Mode of product transfer (gravity / nitrogen / pump) Pressure / temperature monitoring and data historian links OOS audible alarms? | Sampling and sample handling procedures are specified within MOR and TSP documents. Sampling and testing operations shall be carried out in accordance with the relevant company sampling and testing procedures. The company has formulated relevant procedures to ensure the transfer, storage and testing of test samples are under control. Both the prepared Oxyma Pure/DMF solution and Ac₂O/Pyridine/DMF solution are transferred via fixed pipelines with pumps. Temperature, humidity and differential pressure monitoring systems shall be managed and used per relevant company SOPs to ensure stable and effective operation of the systems. | Low | Risk ranking rationale: existing sampling SOP and fixed pipelines; no change; no CQA impact. Sampling and testing for raw material and product (DS) (DS-AC-004) Analytical sample management (AC-019) Fixed pipelines reduce the risk of cross-contamination. Temperature, Humidity and Pressure Difference Monitoring System Management Procedure (GMP-331) |
| In-Process Filtration (non-API) | Filter locations, MOC, porosity Filter change frequency (per batch / PM) Emergency change-out process defined? | Filter replacement and maintenance shall be performed in accordance with relevant company procedures. Asymchem has established procedures for the change management of all production-related hardware and software. | Low | Risk ranking rationale: existing filtration SOP; no change; no CQA impact. Equipment and facility maintenance manual (QM03025) Processing aid materials management procedure （DS-MM-015） |
| API Process Filtration / Final Filtration | Is this product an injectable API requiring sterile filtration? Is filtration part of API bioburden control strategy? Filter integrity testing method and limits defined In-use times and filter sizes appropriate for batch size | The replacement of fluid filters shall be implemented in accordance with the relevant procedures for equipment component management. | Low | Risk ranking rationale: existing filtration SOP; no change; no CQA impact. Equipment fitting management procedure (DS-GMP-026) |
| API Processing / Hold Times | Data supporting multiple temperature cycles (freeze/thaw)? Processing time limits established Transport time from processing area to storage within validated limits Shipment qualification for all configurations used? Material dating aligned with manufacturing/shipping schedules? | Operational time requirements will be clearly specified in the MOR. Utility Systems / Equipment Damage / Impact of Deviations / Test Data / Analytical Data / Material Release / Other Force Majeure Factors Asymchem has established guidelines for deviation reporting and investigation to identify root causes of deviations, implement corrective and preventive actions, and drive continuous improvement of the quality system. | Low | Risk ranking rationale: hold times defined in the existing MOR; no change; no CQA impact. MOR-TJ414002001 MOR-TJ414002002 |
| Packaging / Final Sampling | Room classification for packaging area Open vs. closed (BIBO) packaging operations In-process controls, sampling method and volumes Glove box use — pressure and integrity evaluation Cleaning method for packaging area Is product milled or sieved? | Asymchem has established procedures for packaging and labelling of chemical products. Asymchem has established procedures for sampling and testing. Packaging material and finished drug product Asymchem has established relevant procedures for shared facility control management and contamination control strategy. | Low | Risk ranking rationale: existing packaging SOP; no change; no CQA impact. Finished Product Packaging and Labeling (DS) (DS-MM-010) Contamination control strategy (QAP-022) Control procedure of shared facility in API production. (DS-GMP-011) |
| QC & Contract Laboratories |  |  |  |  |
| Facility and Instrumentation | Appropriate laboratory equipment available for all DS, DP and raw material methods New instruments/methods required? | A dedicated analytical department and team are available with adequate lab facilities and equipment to cover all test methods of DS, Laboratory equipment are available for the 140020 commercial production and raw material methods | Low | Risk ranking rationale: all instruments qualified and calibrated; no change; no CQA impact. Site Master File for Asymchem Life science (Tianjin) Co., Ltd (SMF-TJ4-001) Buildings, facilities and equipment are subject to qualification and validation as required (GMP-026) |
| Analytical Methods | Methods transferred, validated/verified for DS, RSMs and raw materials Sample holding times validated? Product matrix/solvent compatibility with sample containers confirmed? Calibration ranges current? New methods required for cleaning validation? | Asymchem has relevant guidelines for the establishment, verification, validation and transfer of analytical methods, Validation will be performed for final products, intermediates, IPC and RSMs, excluding raw materials for PPQ Batches. Raw material Asymchem has established a standard procedure for the management of chemical intermediates to govern the sampling, testing, storage, release and distribution of chemical intermediates. Asymchem has SOPs and guiding documents related to sampling containers. No new instruments/methods required | Low | Analytical Method Establishment, Verification, Validation and Transfer (AC-002) Chemical intermediate management (DS) (DS-MM-003) Sampling and testing for raw material and product (DS) (DS-AC-004) Guideline for Sampling of Packaging Material (QM04024) |
| Reference Standards | Reference standards available for all methods New storage conditions required? Reference standard qualification completed? | Asymchem has established a standard procedure governing the sourcing, testing, storage, usage, re-testing and disposal of reference standards. And qualification of reference standards has been completed. | Low | Risk ranking rationale: reference standards qualified and within expiry; no change; no CQA impact. Standard Material Management (AC-020) Chemical Macromolecular Standard Material Management(DS) (DS-AC-016) |
| Stability Programm | Stability programm includes all required ICH conditions Stability chambers available and qualified Sufficient chamber capacity Stability data required to support tech transfer identified | Asymchem has established a management procedure for stability chambers to ensure compliance with all stability study requirements. Stability chambers are available and qualified Chamber capacity is sufficient Asymchem has established a management procedure for Stability study procedure | Low | Risk ranking rationale: stability program and chambers qualified; no change; no CQA impact. Stability chamber management procedure (AC-163) Stability testing management procedure(DS) (DS-AC-003) |
| Laboratory Personnel | Personnel trained and qualified for all analytical methods Lab shift structure compatible with in-process testing requirements | Training and qualification evaluation documents are available to guarantee analysts can run complex analytical instruments, maintain proper data management standards and prove their expertise in relevant fields. And all analytical methods will be trained and qualified before PPQ batches Asymchem has established an employee training management system to ensure all staff meet occupational competency requirements. Asymchem shift schedule (12 h / day, 4 days / week * four-shift operation | Low | Risk ranking rationale: routine analyst qualification under the established program; methods unchanged; qualification is a prerequisite, not a control for a new hazard. Training Manual for Physical and Chemical Analysis Laboratory(QM04019) Employee training (ASM-003) |
| Sample Preparation and Handling | Analytical methods contain detailed sample preparation and handling instructions Any matrices known to be difficult or prone to variability? | Sampling and testing operations shall be carried out in accordance with the relevant company sampling and testing procedures. The analytical methods include detailed instructions for sample preparation, sample handling as well as the testing procedures. | Low | Risk ranking rationale: methods contain detailed sample handling instructions; no change; no CQA impact. TSP-1400200305.01 TSP-1400200306.01 |
| Technical Studies |  |  |  |  |
| Elemental Impurities Assessment | Elemental impurities risk assessment completed per ICH Q3D? Local data generated for new equipment / site? Risk of elemental impurity presence in product evaluated? | Asymchem has clearly defined internal requirements for the application scope and limits of elemental impurities according to ICH Q3D | Low | Risk ranking rationale: ICH Q3D assessment; no new equipment introducing elemental risk; no or low CQA correlation. Guideline For Elemental Impurities (QM04187) |
| Nitrosamine Risk Assessment | Nitrosamine risk assessment completed per applicable regulatory guidance Nitrosamine levels adequate to ensure patient safety Risk of nitrosamine formation in product evaluated | Asymchem has established control strategies for nitrosamine impurities throughout the manufacturing process. | Low | N-Nitrosamine impurity control strategy (QAP-021) |
| Residual Solvent Assessment | Residual solvent risk assessment completed per ICH Q3C (R8) Solvents used in process evaluated for Class 1, 2, 3 classification Method available to test residual solvents in DS? | Asymchem conducts residual solvent risk assessment in accordance with the ICH Q3C guideline. Residual solvents in DS test methods are fully established and validated. | Low | Risk ranking rationale: ICH Q3C assessment; residual solvent methods validated; no change; no CQA impact. Analytical Method Establishment, Verification, Validation and Transfer (AC-002) |

# 4. Summary Risk Mitigation / Action Plan

The table 15 below lists all gaps, differences and associated actions identified in Section 3. Owner functions and milestone dates must be confirmed by the Technology Transfer Team at project initiation. This table should be reviewed at each project Manufacturability Review.

Table 15: List of All Gaps, Differences and Associated Actions Identified in Section 3

| Area Evaluated | Gap / Difference | Risk to product CQAs | Action Plan | Owner (Function) | Milestone |
| --- | --- | --- | --- | --- | --- |
| Due Diligence |  |  |  |  |  |
| N/A | N/A | N/A | N/A | N/A | N/A |
| Facility |  |  |  |  |  |
| N/A | N/A | N/A | N/A | N/A | N/A |
| Utilities |  |  |  |  |  |
| N/A | N/A | N/A | N/A | N/A | N/A |
| Personnel / Safety |  |  |  |  |  |
| PHA | No Gap PHA Checklist -CPo140020-03-01 has been completed. | Low | No Action | EHS | Completed |
| PHIS | No Gap Process Hazard Identification Summary Report for CPo140020-03 has been completed. | Low | No Action | EHS | Completed |
| Process and Equipment |  |  |  |  |  |
| Process (Step 1) | MOR documentation and GMP enhancements ((Incident Log, dual-person verification, expanded pre-charging and visual-inspection checklists, and clarified roles and records) | Low | This update was implemented upon joint discussion and confirmation among Production, Customer, QAN/A | CMMD | Assigned Owner: Pengda Zhao Due Date: 11/05/2026 Item status and Closure criteria: Change control sign-off |
| Process (Step 1) | process parameter refinements (minimum dissolution, activation and mixing times, visual discharge/transfer endpoints, removal of the agitation-stop step, and formal CPP/NOR designation) | Low | This update was implemented upon joint discussion and confirmation among Production, Customer, QAN/A | CMMD | Assigned Owner: Pengda Zhao Due Date: 11/05/2026 Item status and Closure criteria: Change control sign-off |
| Process (Step 1) | The equipment adjustment introduces process-related changes | low | Relevant contents have been filled in Change-CPo140020-030104.01 | CMMD | Assigned Owner: Pengda Zhao Due Date: 11/05/2026 Item status and Closure criteria: Change control sign-off |
| Process (Step 1) | Upgrade of analytical method validation and revision of the intermediate release test method. These revisions introduce MOR-related changes. | low | MOR-TJ414002001 | CMMD | Assigned Owner: Pengda Zhao Due Date: 11/05/2026 Item status and Closure criteria: Change control sign-off |
| Process (Step 1) | Other adjustments about process | low | Relevant contents have been filled in Change-CPo140020-030104.01 | CMMD | Assigned Owner: Pengda Zhao Due Date: 11/05/2026 Item status and Closure criteria: Change control sign-off |
| Process (Step 2) | MOR documentation and GMP enhancements ((Incident Log, dual-person verification, expanded pre-charging and visual-inspection checklists, and clarified roles and records) | Low | This update was implemented upon joint discussion and confirmation among Production, Customer, QAN/A | CMMD | Assigned Owner: Pengda Zhao Due Date: 11/20/2026 Item status and Closure criteria: Change control sign-off |
| Process (Step 2) | Other process changes | low | Relevant contents have been filled in Change-CPo140020-030204.01 | CMMD | Assigned Owner: Pengda Zhao Due Date: 11/20/2026 Item status and Closure criteria: Change control sign-off |
| Equipment (Step 1) | 1. The equipment for prepared Oxyma Pure/DMF solution has been changed from TJ4S-1213-R21 to TJ4S-1200-AT02; 2. The equipment for preparing Ac2O / Pyridine / DMF solution has been changed from TJ4S-1213-Sy15-R01 to TJ4S-1213-R22 | Low | 1. Equipment performance will be qualified via IQ, OQ and PQ before use. 2. Has been updated the process equipment flow diagram. 3. Prepared solutions are transferred through fixed process piping. 4. Correct labels shall display unique material names and project numbers for identification of solution types. 5. Personnel confirmation of complete transfer. 6. Two personnel (process chemist and operator) always monitor the operation. | CMMD | Assigned Owner: Zhongyao Zhang Due Date: 11/05/2026 Item status and Closure criteria: complete equipment re-validation and engineering re-assessment |
| ESA | The step 1 Equipment suitability assessment (RA-140020030101.01) is not yet finalized. | Medium | Finalize and sign off the Equipment suitability assessment report(RA-140020030101.01) | CMMD | Assigned Owner: Pengda Zhao Due Date: 11/05/2026 Item status and Closure criteria: ESA sign-off |
|  | The step 2 Equipment suitability assessment (RA-140020030201.01) is not yet finalized. | Medium | Finalize and sign off the Equipment suitability assessment report (RA-140020030201.01) | CMMD | signed Owner: Pengda Zhao Due Date: 11/20/2026 Item status and Closure criteria: ESA sign-off |
| Cleaning Validation |  |  |  |  |  |
| Cleaning Validation (Step 1) | Cleaning validation gaps identified for product-dedicated equipment: cleaning validation status, carryover limits based on toxicology/PDE, and analytical method for swab/rinse testing. | Medium | Develop appropriate cleaning methods during PPQ; establish toxicology/PDE-based carryover limits; perform cleaning testing per DS-GMP-003 and DS-GMP-043. | CMMD | Assigned Owner: Zhonghua Fu Due Date: 11/05/2026 Item status and Closure criteria: CVP sign-off |
| Cleaning Validation (Step 2) | Cleaning validation gaps identified for product-dedicated equipment: cleaning validation status, carryover limits based on toxicology/PDE, and analytical method for swab/rinse testing. | Medium | Develop appropriate cleaning methods during PPQ; establish toxicology/PDE-based carryover limits; perform cleaning testing per DS-GMP-003 and DS-GMP-043. | CMMD | Assigned Owner: Zhonghua Fu Due Date: 11/20/2026 Item status and Closure criteria: CVP sign-off |
| Analytical method readiness | As the cleaning solvent needs to be changed to process solvent, the cleaning detection methods for Steps 2 need to be re-developed and re-validated | Medium | Method development and validation will be initiated as soon as possible. | QC | Assigned Owner: Haini Wang Due Date: 11/20/2026 Item status and Closure criteria: complete the cleaning detect method validation |
| Materials & Specification |  |  |  |  |  |
| Raw material supplier | A new raw material supplier has been added. | Low | Relevant contents have been filled in Change-140020-0306.01 | CMMD | The change control is signed-off |
| Material Specification | 1. The purity specification of Fmoc-L-N-Me-Asn(Trt)-OH has been adjusted from ≥98.0% to ≥99.0%. The specification of Des-Methyl impurity was adjusted from ≤0.2% to ≤0.1%. 2. Add detection of Fmoc-Leu-OH and Fmoc-Ile-OH in Fmoc-L-Val-OH, with specifications of ≤0.1% each. 3. The purity specification of Fmoc-L-α-Me-Phe-OH and Fmoc-L-Orn(Boc)-OH have been adjusted from ≥98.0% to ≥99.0%. 4. The specification of N,N-Dimethylformamide, Piperidine, Methanol, Ethyldiisopropylamine, Ethyl acetate, tert-Butyl methyl ether, Pyridine, Toluene, Purified water, Heptane, Trifluoroacetic acid were added. | Low | Relevant contents have been filled in Change-140020-030205.01 | ARD | Assigned Owner: Yongle Liu Due Date: 11/20/2026 Item status and Closure criteria: Change control sign-off |
| Product Quality Specification | Analytical method of Identification changed from AM-013 to AM-140020-025 | Low | Relevant contents have been filled in Change-140020030205.01 | AR | The change control is signed-off |
|  | Add specified impurity in Purity and related impurities by UHPLC test item: ASYM-153207 (L2 endo-Gly24): ≤ 0.5%; ASYM-152486 (L2 des-(Asn21-NMeAsn22)): ≤ 2.0% |  |  |  |  |
|  | Update the acceptance criteria of individual impurities from ≤1.0% to ≤ 0.5% are based on historical data and PAR study documents |  |  |  |  |
|  | The change of adding the unit of the test items, and adding the requirement of “Report individual impurities ≥ 0.10% by RRT” to the item of Any individual impurities |  |  |  |  |
| QC & Contract Labs |  |  |  |  |  |
| IPC Methods | Clear quantitative requirements are specified for both sample handling procedures and test result acceptance criteria for IPC testing (Document No. AM-140020-029, AM-140020-031, AM-140020-030) | Low | The new method has been fully validated. | ARD | Completed |
| IPC Methods | The drying endpoint LOD acceptance criterion is tightened from 10.0% to 9% (w/w) | Low | Relevant contents have been filled in Change-CPo140020-030104.01 | CMMD | Assigned Owner: Pengda Zhao Due Date: 11/05/2026 Item status and Closure criteria: Change control sign-off |
| Sample Preparation and Handling | TSP is in preparation. | Low | The TSP to-do list: TSP-1400200305.01 TSP-1400200307.01 | CMMD | Assigned Owner: Pengda Zhao Step1：Due Date: 10/30//2026 Step2：Due Date: 11/14//2026 |
| Technical Studies |  |  |  |  |  |
| Stability Study | Stability testing for ASYM-141366; LSN4182924 / ASYM-140020 have been added for the PPQ Batches. | Low | Stability protocol shall be formulated and implemented. | ARD | After PPQ Batches |
| Contamination control readiness, | Contamination control readiness is in preparation. | Low | The contamination control readiness report RA-1400200301 will be completed prior to manufacturing. | Production | Assigned Owner: He Han Due Date: 10/30//2026 |
| Validation readiness | Validation readiness is in preparation. | Low | The process validation readiness checklist Cpv-CPo140020-030101 Cpv-CPo140020-030201 | CMMD | Assigned Owner: Pengda Zhao Step1：Due Date: 10/30//2026 Step2：Due Date: 11/20/2026 |
| Nitrosamine Risk Assessment | Risk assessment is being conducted through experiments. | Low | Conduct Nitrosamine Risk Assessment and Report Preparation | CMMD | Assigned Owner: Pengda Zhao Due Date: prior to product release Item status and Closure criteria: Completion of Nitrosamine Risk Assessment Report |

Note: *The due dates are established based on the current manufacturing schedule and shall not be later than the actual PPQ initiation date.

# 5. Raw Material Supplier Information

All suppliers of critical raw materials and primary packaging materials must be qualified in accordance with the Asymchem's Supplier Qualification (MM-046) and Evaluation procedure prior to their use in PPQ batches. The approved supplier list is provided below (table 16).

Table 16: The Approved Supplier List

| Raw Material Name | Material Code (PPQ) | Approved Supplier(s) | Supplier Classification | Supplier Qualification Document No. | Latest Audit Date |
| --- | --- | --- | --- | --- | --- |
| 2-CTC resin | YL005205 | Tianjin Nankai Hecheng S&T Co., Ltd. | Class 2 | ADT2026002.V01 | 01/13/2026 |
|  |  | Sunresin New Materials Co.Ltd., GaoLing | Class 2 | ADT2026001.V01 | 01/06/2026 |
| Fmoc-Gly-OH | YL004846 | Liaoning Asymchem Laboratories Co., Ltd (FX2). | Class 2 | FX2-IA-202501.01 | 12/16/2025 |
|  |  | Asymchem Laboratories (Jilin) Co., Ltd. (DH3) | Class 2 | DH3-IA-202501.01 | 10/29/2025 |
|  |  | Chengdu ZY Biochemical Technology Co., Ltd. | Class 2 | ADT2025043.V01 | 12/04/2025 |
| Fmoc-L-Phe-OH | YL004840 | Liaoning Asymchem Laboratories Co., Ltd (FX2). | Class 2 | FX2-IA-202501.01 | 12/16/2025 |
|  |  | Asymchem Laboratories (Jilin) Co., Ltd. (DH3) | Class 2 | DH3-IA-202501.01 | 10/29/2025 |
|  |  | Chengdu ZY Biochemical Technology Co., Ltd. | Class 2 | ADT2025043.V01 | 12/04/2025 |
| Fmoc-L-N-Me-Asn(Trt)-OH | YL008617 | Asymchem Laboratories (Jilin) Co., Ltd. (DH3) | Class 2 | DH3-IA-202501.01 | 10/29/2025 |
| Fmoc-L-Asn(Trt)-OH | YL004843 | Asymchem Laboratories (Jilin) Co., Ltd. (DH3) | Class 2 | DH3-IA-202501.01 | 10/29/2025 |
|  |  | Liaoning Asymchem Laboratories Co., Ltd (FX2) | Class 2 | FX2-IA-202501.01 | 12/16/2025 |
|  |  | Chengdu ZY Biochemical Technology Co., Ltd. | Class 2 | ADT2025043.V01 | 12/04/2025 |
| Fmoc-L-Ser(tBu)-OH | YL003428 | Liaoning Asymchem Laboratories Co., Ltd (FX2) | Class 2 | FX2-IA-202501.01 | 12/16/2025 |
|  |  | Asymchem Laboratories (Jilin) Co., Ltd. (DH3) | Class 2 | DH3-IA-202501.01 | 10/29/2025 |
|  |  | Chengdu ZY Biochemical Technology Co., Ltd. | Class 2 | ADT2025043.V01 | 12/04/2025 |
| Fmoc-L-Arg(Pbf)-OH | YL003424 | Asymchem Laboratories (Jilin) Co., Ltd. (DH3) | Class 2 | DH3-IA-202501.01 | 10/29/2025 |
|  |  | Chengdu ZY Biochemical Technology Co., Ltd. | Class 2 | ADT2025043.V01 | 12/04/2025 |
| Fmoc-L-Val-OH | YL003110 | Liaoning Asymchem Laboratories Co., Ltd (FX2) | Class 2 | FX2-IA-202501.01 | 12/16/2025 |
|  |  | Asymchem Laboratories (Jilin) Co., Ltd. (DH3) | Class 2 | DH3-IA-202501.01 | 10/29/2025 |
|  |  | Chengdu ZY Biochemical Technology Co., Ltd. | Class 2 | ADT2025043.V01 | 12/04/2025 |
| Fmoc-L-Leu-OH | YL003425 | Liaoning Asymchem Laboratories Co., Ltd (FX2) | Class 2 | FX2-IA-202501.01 | 12/16/2025 |
|  |  | Asymchem Laboratories (Jilin) Co., Ltd. (DH3) | Class 2 | DH3-IA-202501.01 | 10/29/2025 |
|  |  | Chengdu ZY Biochemical Technology Co., Ltd. | Class 2 | ADT2025043.V01 | 12/04/2025 |
| Fmoc-L-α-Me-Phe-OH | YL008272 | Asymchem Laboratories (Jilin) Co., Ltd. (DH3) | Class 2 | DH3-IA-202501.01 | 10/29/2025 |
| Fmoc-L-Glu(OtBu)-OH·H2O | YL005741 | Asymchem Laboratories (Jilin) Co., Ltd. (DH3) | Class 2 | DH3-IA-202501.01 | 10/29/2025 |
|  |  | Chengdu ZY Biochemical Technology Co., Ltd. | Class 2 | ADT2025043.V01 | 12/04/2025 |
| Fmoc-L-Ala-OH·H2O | YL006160 | Liaoning Asymchem Laboratories Co., Ltd (FX2) | Class 2 | FX2-IA-202501.01 | 12/16/2025 |
|  |  | Asymchem Laboratories (Jilin) Co., Ltd. (DH3) | Class 2 | DH3-IA-202501.01 | 10/29/2025 |
|  |  | Chengdu ZY Biochemical Technology Co., Ltd. | Class 2 | ADT2025043.V01 | 12/04/2025 |
| Fmoc-L-Orn(Boc)-OH | YL005318 | Asymchem Laboratories (Jilin) Co., Ltd. (DH3) | Class 2 | DH3-IA-202501.01 | 10/29/2025 |
|  |  | Chengdu ZY Biochemical Technology Co., Ltd. | Class 2 | ADT2025043.V01 | 12/04/2025 |
| ArmorFlex 114 bag with Fill Neck and Chargepoint Discharge | YQ000572 | ILC Dover Poland sp. z o.o. | Class 2 | ADT2026007.V01 | 03/11/2026 |

# 6. Personnel

All personnel involved in LSN4182924/ASYM-140020 (Step 1 and Step 2) manufacturing operation sampling, testing, handling, and documentation associated with manufacture of LSN4182924/ASYM-140020 (Step 1 and Step 2) during the PPQ campaign are trained in accordance with cGMP and GDP requirements prior to campaign start-up.

# 7. Cleaning

All equipment used for Full-scale non sale batch production was cleaned in accordance with the on-site cleaning procedures of the Asymchem. Inspection and QA release were completed prior to use. The synthesis reactor TJ4S-1213-Sy15 was cleaned per document CMa-Sy-TJ401.current; the agitator tank TJ4S-1213-SPS-AT01 & TJ4S-1213-SPS-AT02, and TJ4S-1200-AT02 were cleaned per document CMa-Moyable-TJ402.current; The glass-lined reactor TJ4S-1212-R05 and TJ4S-1212-R13 were cleaned per document CMa-Fixed-TJ404.current; The equipment of TJ4S-1212-R09, TJ4S-1212-R13, TJ4S-1213-R11, TJ4S-1213-R13, TJ4S-1213-R22 and TJ4S-1212-FD05 were cleaned per document CMa-Fixed-TJ402.current.

The cleaning validation will be conducted as the basis of specific product together with the PPQ batches and commercial batches. The cleaning validation protocol (CVP-1400200301 & CVP-1400200302) will also be prepared.

# 8. Storage and Handling Conditions

It is confirmed that the storage conditions applicable to in-process product 141366 should be storage at 2–8 °C and the final LSN4182924 / ASYM-140020 should be storage at -25 – -15 °C).  The storage conditions will be applied in the PPQ campaign as in the Full Scale non-saleable batch.

# 9. Packaging Materials

The Packaging Materials for LSN4182924/ASYM-140020, Primary is ArmorFlex 114 bag with Fill Neck and Chargepoint Discharge for packaging. Seal the bag using double pull buckle seals. Put it into aluminum foil bag, add pharmaceutical-grade silica gel desiccant between the outer bag of ArmorFlex 114 bag with Fill Neck and Chargepoint Discharge and the aluminum foil bag. Add 3 bags desiccant to each package. Heat seal the aluminum foil bags. And the Outer packaging is HDPE drum, seal with security seal.

It is confirmed that the packaging configuration will be used in the PPQ campaign as in the Full Scale non-saleable batch.

# 10. Conclusion

In summary, for the LSN4182924 / ASYM-140020 technology transfer. There are no changes to Participant Team, Raw Material Supplier information. Multiple optimization items and method adjustments have been identified across raw material control, process operation, equipment facility and analytical method domains, with no critical or major gaps impacting process consistency and product Critical Quality Attributes (CQAs) observed.

The identified adjustments are categorized as follows:

- Process of Step 1. The Changed documented are grouped into five categories (For details of related changes, refer to Change-CPo140020-030104.01):

- MOR documentation and GMP enhancements (Incident Log, dual-person verification, expanded pre-charging and visual-inspection checklists, and clarified roles and records);

- Process parameter refinements (minimum dissolution, activation and mixing times, visual discharge/transfer endpoints, removal of the agitation-stop step, and formal CPP/NOR designation)

- The equipment adjustment introduces process-related changes

- Analytical method validation upgrade and adjustment of intermediate release test method.

- Other adjustments.

- Process of Step 2. The Changed documented are grouped into three categories (For details of related changes, refer to Change-CPo140020-030204.01):

- MOR documentation and GMP enhancements (Incident Log, dual-person verification, expanded pre-charging and visual-inspection checklists, and clarified roles and records);

- . Other process changes

- .The packaging operation record updated.

- Equipment adjustment for PV batches: The equipment (Prepared OxymaPure/DMF Solution) is changed from the Glass-lined reactor TJ4S-1213-R21 to the Agitation tank TJ4S-1200-AT02. The equipment (Prepared Capping Solution- Ac2O/ Pyridine) is changed from Activation tank TJ4S-1213-Sy15-R01 to the the Glass-lined reactor TJ4S-1213-R22.

- Raw Material & Specifications

- Raw material control: Raw material specifications are refined and tightened to further strengthen upstream quality control.

- In-Process Control (IPC) Items of Step 1 & Step 2:

1). The analytical methods (Ninhydrin test: AM-140020-029, Bromophenol blue test: AM-140020-031 and Chloranil test: AM-140020-030) during PPQ campaign has been validated.

2). The drying endpoint LOD acceptance criterion is tightened from 10.0% to 9% (w/w).

3). The In-process intermediate specifications has been removed.

4). The product Quality Specification for LSN4182924 / ASYM140020 has been changed (Relevant contents have been filled in Change-140020030205.01).

All identified adjustments are process optimization, equipment enhancement or method reliability improvement measures, and do not alter the core synthetic route, critical process parameter ranges, IPC acceptance criteria or final product quality specifications. All these changes have no impact on CQAs. All action items have been incorporated into the technology transfer action plan, with each item assigned a defined owner, due date, status, and closure criteria, in accordance with ICH Q9 quality risk management requirements. All analytical method upgrades and validations will be completed prior to PPQ initiation. All outstanding items are managed through the Quality Management System (QMS) for centralized tracking. The corresponding tracking records and supporting evidence will be appended to this document. No adverse impact on PPQ execution or product quality is anticipated.

The remaining TBDs, including residual solvent reporting criteria and validation report effective dates, are captured in the action plan with owners and target dates, and will be completed prior to the relevant PPQ step or use of the method.

# 11. References

ICH Q3C (R8) — Residual Solvents

ICH Q3D — Elemental Impurities

ICH Q7 — Good Manufacturing Practice for APIs

Equipment Suitability Assessment Report: RA-140020030101.01

Equipment Suitability Assessment Report: RA-140020030201.01

Cleaning Impact Assessment: Reference document number: CMa-Sy-TJ401.01, CMa-Moyable-TJ402.03, CMa-Fixed-TJ404.01.

CMO Supplier Qualification Procedure: SOP-MM-046

# 12. Appendix I Risk Mitigation Measures/Action Plan Tracking Table

Table 17: The tracking table of mitigation measure/action

| Area Evaluated | Gap / Difference | Risk to product CQAs | Action Plan | Owner (Function) | Milestone* | Completion Evidence | Signature/Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Process and Equipment |  |  |  |  |  |  |  |
| Process (Step 1) | MOR documentation and GMP enhancements (Incident Log, dual-person verification, expanded pre-charging and visual-inspection checklists, and clarified roles and records) | Low | This update was implemented upon joint discussion and confirmation among Production, Customer, QA N/A | CMMD | Assigned Owner: Pengda Zhao Due Date: 11/05/2026 Item status and Closure criteria: Change control sign-off |  |  |
| Process (Step 1) | process parameter refinements (minimum dissolution, activation and mixing times, visual discharge/transfer endpoints, removal of the agitation-stop step, and formal CPP/NOR designation) | Low | This update was implemented upon joint discussion and confirmation among Production, Customer, QA N/A | CMMD | Assigned Owner: Pengda Zhao Due Date: 11/05/2026 Item status and Closure criteria: Change control sign-off |  |  |
| Process (Step 1) | The equipment adjustment introduces process-related changes | low | Relevant contents have been filled in Change-CPo140020-030104.01 | CMMD | Assigned Owner: Pengda Zhao Due Date: 11/05/2026 Item status and Closure criteria: Change control sign-off |  |  |
| Process (Step 1) | Upgrade of analytical method validation and revision of the intermediate release test method. These revisions introduce MOR-related changes. | low | MOR-TJ414002001 | CMMD | Assigned Owner: Pengda Zhao Due Date: 11/05/2026 Item status and Closure criteria: Change control sign-off |  |  |
| Process (Step 1) | Other adjustments about process | low | Relevant contents have been filled in Change-CPo140020-030104.01 | CMMD | Assigned Owner: Pengda Zhao Due Date: 11/05/2026 Item status and Closure criteria: Change control sign-off |  |  |
| Process (Step 2) | MOR documentation and GMP enhancements ((Incident Log, dual-person verification, expanded pre-charging and visual-inspection checklists, and clarified roles and records) | Low | This update was implemented upon joint discussion and confirmation among Production, Customer, QA N/A | CMMD | Assigned Owner: Pengda Zhao Due Date: 11/20/2026 Item status and Closure criteria: Change control sign-off |  |  |
| Process (Step 2) | Other process changes | low | Relevant contents have been filled in Change-CPo140020-030204.01 | CMMD | Assigned Owner: Pengda Zhao Due Date: 11/20/2026 Item status and Closure criteria: Change control sign-off |  |  |
| Equipment (Step 1) | 1. The equipment for prepared Oxyma Pure/DMF solution has been changed from TJ4S-1213-R21 to TJ4S-1200-AT02; 2. The equipment for preparing Ac2O / Pyridine / DMF solution has been changed from TJ4S-1213-Sy15-R01 to TJ4S-1213-R22 | Low | 1. Equipment performance will be qualified via IQ, OQ and PQ before use. 2. Has been updated the process equipment flow diagram. 3. Prepared solutions are transferred through fixed process piping. 4. Correct labels shall display unique material names and project numbers for identification of solution types. 5. Personnel confirmation of complete transfer. 6. Two personnel (process chemist and operator) always monitor the operation. | CMMD | Assigned Owner: Zhongyao Zhang Due Date: 11/05/2026 Item status and Closure criteria: complete equipment re-validation and engineering re-assessment |  |  |
| ESA | The step 1 Equipment suitability assessment (RA-140020030101.01) is not yet finalized. | Medium | Finalize and sign off the Equipment suitability assessment report(RA-140020030101.01) | CMMD | Assigned Owner: Pengda Zhao Due Date: 11/05/2026 Item status and Closure criteria: ESA sign-off |  |  |
|  | The step 2 Equipment suitability assessment (RA-140020030201.01) is not yet finalized. | Medium | Finalize and sign off the Equipment suitability assessment report (RA-140020030201.01) | CMMD | signed Owner: Pengda Zhao Due Date: 11/20/2026 Item status and Closure criteria: ESA sign-off |  |  |
| Cleaning Validation |  |  |  |  |  |  |  |
| Cleaning Validation (Step 1) | Cleaning validation gaps identified for product-dedicated equipment: cleaning validation status, carryover limits based on toxicology/PDE, and analytical method for swab/rinse testing. | Medium | Develop appropriate cleaning methods during PPQ; establish toxicology/PDE-based carryover limits; perform cleaning testing per DS-GMP-003 and DS-GMP-043. | CMMD | Assigned Owner: Zhonghua Fu Due Date: 11/05/2026 Item status and Closure criteria: CVP sign-off |  |  |
| Cleaning Validation (Step 2) | Cleaning validation gaps identified for product-dedicated equipment: cleaning validation status, carryover limits based on toxicology/PDE, and analytical method for swab/rinse testing. | Medium | Develop appropriate cleaning methods during PPQ; establish toxicology/PDE-based carryover limits; perform cleaning testing per DS-GMP-003 and DS-GMP-043. | CMMD | Assigned Owner: Zhonghua Fu Due Date: 11/20/2026 Item status and Closure criteria: CVP sign-off |  |  |
| Analytical method readiness | As the cleaning solvent needs to be changed to process solvent, the cleaning detection methods for Steps 2 need to be re-developed and re-validated | Medium | Method development and validation will be initiated as soon as possible. | QC | Assigned Owner: Haini Wang Due Date: 11/20/2026 Item status and Closure criteria: complete the cleaning detect method validation |  |  |
| Materials & Specification |  |  |  |  |  |  |  |
| Raw material supplier Material Specification | A new raw material supplier has been added. | Low | Relevant contents have been filled in Change-140020-0306.01 | CMMD | The change control is signed-off |  |  |
|  | 1. The purity specification of Fmoc-L-N-Me-Asn(Trt)-OH has been adjusted from ≥98.0% to ≥99.0%. The specification of Des-Methyl impurity was adjusted from ≤0.2% to ≤0.1%. 2. Add detection of Fmoc-Leu-OH and Fmoc-Ile-OH in Fmoc-L-Val-OH, with specifications of ≤0.1% each. 3. The purity specification of Fmoc-L-α-Me-Phe-OH and Fmoc-L-Orn(Boc)-OH have been adjusted from ≥98.0% to ≥99.0%. 4. The specification of N,N-Dimethylformamide, Piperidine, Methanol, Ethyldiisopropylamine, Ethyl acetate, tert-Butyl methyl ether, Pyridine, Toluene, Purified water, Heptane, Trifluoroacetic acid were added. | Low | Relevant contents have been filled in Change-140020-030205.01 | ARD | Assigned Owner: Yongle Liu Due Date: 11/20/2026 Item status and Closure criteria: Change control sign-off |  |  |
| Product Quality Specification | Analytical method of Identification changed from AM-013 to AM-140020-025 | Low | Relevant contents have been filled in Change-140020030205.01 | AR | The change control is signed-off |  |  |
|  | Add specified impurity in Purity and related impurities by UHPLC test item: ASYM-153207 (L2 endo-Gly24): ≤ 0.5%; ASYM-152486 (L2 des-(Asn21-NMeAsn22)): ≤ 2.0% |  |  |  |  |  |  |
|  | Update the acceptance criteria of individual impurities from ≤1.0% to ≤ 0.5% are based on historical data and PAR study documents |  |  |  |  |  |  |
|  | The change of adding the unit of the test items, and adding the requirement of “Report individual impurities ≥ 0.10% by RRT” to the item of Any individual impurities |  |  |  |  |  |  |
| QC & Contract Labs |  |  |  |  |  |  |  |
| IPC Methods | Clear quantitative requirements are specified for both sample handling procedures and test result acceptance criteria for IPC testing (Document No. AM-140020-029, AM-140020-031, AM-140020-030) | Low | The new method has been fully validated. | ARD | Completed |  |  |
| IPC Methods | The drying endpoint LOD acceptance criterion is tightened from 10.0% to 9% (w/w) | Low | Relevant contents have been filled in Change-CPo140020-030104.01 | CMMD | Assigned Owner: Pengda Zhao Due Date: 11/05/2026 Item status and Closure criteria: Change control sign-off |  |  |
| Sample Preparation and Handling | TSP is in preparation. | Low | The TSP to-do list: TSP-1400200305.01 TSP-1400200307.01 | CMMD | Assigned Owner: Pengda Zhao Step1：Due Date: 10/30//2026 Step2：Due Date: 11/14//2026 |  |  |
| Technical Studies |  |  |  |  |  |  |  |
| Stability Study | Stability testing for ASYM-141366; LSN4182924 / ASYM-140020 have been added for the PPQ Batches. | Low | Stability protocol shall be formulated and implemented. | ARD | After PPQ Batches |  |  |
| Contamination control readiness, | Contamination control readiness is in preparation. | Low | The contamination control readiness report RA-1400200301 will be completed prior to manufacturing. | Production | Assigned Owner: He Han Due Date: 10/30//2026 |  |  |
| Validation readiness | Validation readiness is in preparation. | Low | The process validation readiness checklist Cpv-CPo140020-030101 Cpv-CPo140020-030201 | CMMD | Assigned Owner: Pengda Zhao Step1：Due Date: 10/30//2026 Step2：Due Date: 11/20/2026 |  |  |
| Nitrosamine Risk Assessment | Risk assessment is being conducted through experiments. | Low | Conduct Nitrosamine Risk Assessment and Report Preparation | CMMD | Assigned Owner: Pengda Zhao Due Date: prior to product release Item status and Closure criteria: Completion of Nitrosamine Risk Assessment Report |  |  |

Note: *The due dates are established based on the current manufacturing schedule and shall not be later than the actual PPQ initiation date.