☐ Batch to batch flush /批与批之间的冲洗     ☐ Within campaign in-situ minor cleaning /项目内原位小清洁

☒ Within campaign disassemble major cleaning /项目内拆卸大清洁     ☒ End of production cleaning /项目结束后清洁

☐ Equipment Qualification /设备确认  ☐ Other /其他___N/A_____

Cleaning Method No. and Version No./清洗方法编号及版本号： CMa-TJ414002001.01

- Revision history/版本历史

| Version 版本号 | Effective date 生效日期 | Description of revision/修订描述 (Including reason, background, content etc./包括修订原因，背景，内容等） |
| --- | --- | --- |
| 01 | The effective date is defined as the last approval date/最后签字日期为生效日期 | CMa-TJ414002001.01 is the first version of equipment cleaning method for within-campaign major cleaning and after end of production for ASYM-140020 Step1 at TJ4. / CMa-TJ414002001.01表示ASYM-140020 项目在TJ4厂区生产的第1步生产设备的第一版项目内拆卸大清洁和项目结束后的清洗方案。 |

- Method/方法

| Equipment / 设备 | Usage/ 用途 | Cleaning method and frequency/清洁方式以及频率 |  | Level 0/B& Level I Cleaning verification/ Level 0/B& Level I清洁确认 | Level II cleaning verification / Level II清洁确认 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Name & I.D., Spec. / MOC/名称&编号，规格/材质 |  | Cleaning method description/ 清洁方法描述 | Cleaning frequency and assessment/ 清洁频率及其风险评估 |  | Sampling method/ 取样方法 | Target compound, ARL, testing method/ 目标物，接受标准，分析方法 |
| 3000L Peptide Solid Phase Synthesizer (include activation reactor TJ4S-1213-Sy15-R01 and transfer pipeline, pump, in-line sampler, rake sampling pipeline and glove box)) 多肽固相合成仪（包含激活罐TJ4S-1213-Sy15-R01和转移管路，泵、取样管路，在线取样器，耙子和手套箱） Equipment # 设备编号: TJ4S-1213-Sy15 Moc/材质： Hastelloy/哈氏合金 | Synthesis /合成 | Automatic cleaning/自动清洗: Invoke the cleaning method for equipment cleaning/调用清洗方法进行清洗: Run recipe: CMaTJ414002001A01 运行配方 : CMaTJ414002001A01 Automatic Cleaning procedure/自动清洗程序 - Add 300±10 kg DMF into activation reactor through pipeline. /通过N,N-二甲基甲酰胺加料管路向激活罐内加入300±10 kg N,N-二甲基甲酰胺。 - Cycle cleaning activation reactor and pipeline 3 min. /循环清洗激活罐及管路3 min. - Flush the synthesizer by rinsing ball with DMF that in activation reactor, purge the transfer pipe 1 min by nitrogen. /将激活罐内的N,N-二甲基甲酰胺通过转移管路和合成仪淋洗球淋洗合成仪，氮气吹扫转移管路1 min. - Start the stir and the nitrogen program of the bottom for 15 min, stir rotate speed 25±5 rpm, nitrogen flow rate 10±3 m3/h. Stop stirring and soak for not less than 0.5h. /开启合成仪搅拌和下口鼓氮气程序搅洗15min。搅拌转速25±5 rpm，氮气流量10±3 m3/h。关闭搅拌，泡洗至少0.5h。 - Drain the DMF in the synthesizer to the waste liquid tank through the waste pipe. /将合成仪内的N，N-二甲基甲酰胺通过排废管路排至废液罐。 - Repeat operation 1~5 once, total 2 times. /重复1~5操作1次，总共2次。 - Exhaust pneumatic diaphragm pump and pipeline, nitrogen blow dry pump and feeding pipeline. Purge time 20±5 min. /排尽新鲜溶剂气动隔膜泵及管路，氮气吹干新鲜溶剂泵及其加料管路，吹扫时间20±5 min。 - Add 300±10 kg purified water into activation reactor, run the automatic cleaning method, and cycle cleaning activation reactor and pipeline 3 min. /向激活罐内加入300±10 kg纯化水，继续运行清洗方法，循环清洗3 min。 - Flush the synthesizer by rinsing ball with purified water that in activation reactor, purge the transfer pipe 1min by nitrogen. /将激活罐内的纯化水通过转移管路和合成仪淋洗球淋洗合成仪，氮气吹扫转移管路1min。 - Start the stir and the nitrogen program of the bottom for 15 min, stir rotate speed 25±5 rpm, nitrogen flow rate 10±3 m3/h./开启合成仪搅拌和下口鼓氮气程序搅洗15min。搅拌转速25±5 rpm，氮气流量10±3m3/h。 - Drain the purified water in the synthesizer to the waste liquid tank through the waste pipe. /将合成仪内的纯化水通过排废管路排至废液罐。 - Repeat operation 8~11 twice, total 3 times. /重复8~11操作2次，总共3次。 Manual cleaning/手动清洁： - Use 110±5 kg purified water to rinse the inner surface of the synthesizer and the surface of the sieve plate until there is no resin residue and the activation tank and the to the solvent-free odor residue. If resin residue is still observed, repeat rinsing of the synthesizer internal surfaces and frit surface with 110±5 kg purified water until no resin residue remains, and rinse the activation vessel until no solvent odor is detected. Record the number of repetitions. /使用110±5 kg纯化水冲洗合成仪内表面、筛板表面至无树脂残留，以及激活罐至无溶剂气味残留,如仍有树脂残留，重复使用110±5 kg纯化水冲洗合成仪内表面、筛板表面至无树脂残留，以及激活罐至无溶剂气味残留。记录重复次数。 - Disassemble the nitrogen/exhaust gas lines and valves, rupture discs, pump, solid charging valves, other material charging valves, in-line sample, rake, DIPEA and DIC charging valves, and receiving gloveboxes of the activation tank and synthesis kettle to the smallest section, wipe them with 30.0±1.0 kg purified water until no material remains, and blow dry with compressed gas./将激活罐与合成釜的氮气/尾气管路及阀门、爆破片、泵、固体加料阀、其他物料加料阀，在线取样器、耙子、 DIPEA和DIC加料阀以及收料手套箱拆卸至最小节，使用30.0±1.0 kg纯化水擦拭至无物料残留，压缩气体吹干。 - Lift and rotate the synthesizer sieve plate into place.将合成仪筛板升起并旋转到位。 - Install the disassembled valve to the corresponding position. /将拆卸的阀门安装至对应位置。 Invoke the cleaning method for equipment cleaning/调用清洁方法进行溶剂置换： Run recipe/运行配方：CMaTJ414002001B01 - Rinse activation reactor with 110±5 kg DMF, cycle cleaning 3 min, and then transfer it to the synthetic reactor through the transfer pipeline, turn on the stirring and bottom nitrogen continue for 15 min stirring speed is 25±5 rpm, nitrogen flow rate is 10±3 m3/h, and then drain through the waste discharge pipeline. /使用110±5 kg N,N-二甲基甲酰胺淋洗激活罐，循环3min，后通过转移管路转移至合成仪内，开启搅拌和下口鼓氮气，持续15min，搅拌转速25±5 rpm，氮气流量10±3 m3/h，再通过排废管路排空。 - Rinse activation reactor with 180±10 kg MTBE, cycle cleaning 3 min, and then transfer it to the synthetic reactor through the transfer pipeline turn on the stirring and bottom nitrogen, continue for 15 min, and then drain through the waste discharge pipeline. Stirring speed is 25±5 rpm，nitrogen flow rate is 10±3 m3/h./使用180±10 kg甲基叔丁基醚淋洗激活罐，循环3 min，后通过转移管路转移至合成仪内，开启搅拌和下口鼓氮气，持续15 min，再通过排废管路排空。搅拌转速25±5 rpm，氮气流量10±3 m3/h。 - Dry the activation reactor and synthesizer include transfer pipeline by nitrogen. /氮气吹干激活罐和合成仪以及转移管路。 | Cleaning after production or between batches/项目结束后或批次间清洁 | Level I | - Swab with MeOH for Level II test/用甲醇擦拭取样进行Level II检测。 - Rinse the activation reactor with 110±5 kg DMF and then transfer it into synthesizer, sample the rinsing solvent for Level II test(two samples were taken from upper expansion valve port of the Peptide Solid Phase Synthesizer and via the in-line sampler, then submitted for testing separately)./使用110±5 kg N,N-二甲基甲酰胺淋洗激活罐后通过转移管路淋洗合成仪，取淋洗液取样检测Level Ⅱ残留（2个样品分别取自合成仪器的上展口阀口和和通过在线取样器，分别送样检测）。 Note /备注: For samples collected via the in-line sampler, data collection will only be performed for the three validation rounds; no further testing will be conducted thereafter./ 对于取样器取样，只是在验证3轮执行数据收集，后续不再检测。 | Target compound /目标物：ASYM-140020 Testing method/分析方法： AM-140020-015 Allowable criteria /接受标准： Cleaning after production/项目结束后清洁： ASYM-140020≤ARL/可接受残留限度 Swab/擦拭: ASYM-140020≤36.03μg/100cm2 (Recovery/回收率: 2% 1evel: 98%; 100% 1evel: 91%; 200% 1evel: 90%). Rinse/淋洗: ASYM-140020: ≤.0.00062~0.00067mg/mL (Recovery/回收率: 50% ) Cleaning between batches/批次间清洁: ASYM-140020: ≤10ppm Swab/擦拭: ASYM-140020≤507.39μg/100cm2 (Recovery/回收率: 2% 1evel: 98%; 100% 1evel: 91%; 200% 1evel: 90%). Rinse/淋洗: ASYM-140020: ≤.0.00867~0.00950mg/mL (Recovery/回收率: 50% ) Target compound /目标物： ASYM-124852/YL005318 Testing method/分析方法： AM-140020-014 Allowable criteria /接受标准： Cleaning after production/项目结束后清洁： ASYM-124852 (YL005318) ≤ARL/可接受残留限度 Swab/擦拭: ASYM-124852 (YL005318)≤36.03μg/100cm2 (Recovery/回收率: 1% 1evel: 90%; 100% 1evel: 96%; 200% 1evel: 96%). Rinse/淋洗: ASYM-124852 (YL005318): ≤.0.00062~0.00067mg/mL (Recovery/回收率: 50%) Cleaning between batches/批次间清洁: ASYM-124852 (YL005318) ≤10ppm Swab/擦拭: ASYM-124852 (YL005318)≤507.39μg/100cm2 (Recovery/回收率: 1% 1evel: 90%; 100% 1evel: 96%; 200% 1evel: 96%). Rinse/淋洗: ASYM-124852 (YL005318): ≤0.000867~0.000950mg/mL (Recovery/回收率: 50%) |
| 1000L Glass-lined reactor (include pump and transfer pipeline) /1000L搪玻璃反应釜 (包含泵和其转移管路) Equipment # 设备编号: TJ4S-1213-R22 Moc/材质： Glass-lined/搪玻璃 | Prepare Ac2O/Pyridine /DMF solution/配置乙酸酐/吡啶的N,N-二甲基甲酰胺的溶液 | - Rinse inner surface of the reactor with 175±5 kg DMF, and drain to waste liquid tank through pump and process transfer pipeline. /使用175±5 kg N, N-二甲基甲酰胺淋洗反应釜内表面，通过泵和工艺转移管路排至废液罐。 - Flush the reactor with 165±5 kg purified water./使用165±5 kg纯化水冲洗反应釜。 - Disassemble the fittings until inseparable, wipe fittings with 105±5 kg purified water until there is no material residue./将配件拆卸至最小节，使用105±5 kg纯化水擦洗配件至无可视物料残留。 - Dry by compressed gas. /压缩气体吹干。 | Cleaning after production /项目结束后清洁 | Level I | Only for preparing Ac2O/Pyridine /DMF solution, rinse thoroughly with DMF followed by water; the risk of residual carryover remains low, no need for level II test./仅涉及配置乙酸酐/吡啶的N,N-二甲基甲酰胺的溶液，先N,N-二甲基甲酰胺淋洗，再水冲洗，物料继续残留的风险很低，无需进行Level II 检测 | N/A |
| 500L Agitation tank (include pump and transfer pipeline) /500L搅拌罐 (包含泵和其转移管路) Equipment # 设备编号: TJ4S-1200-AT02 Moc/材质：Hastelloy/哈氏合金 | Prepare the Oxyma Pure /DMF solution,/ 配置2-肟氰乙酸乙酯的N,N-二甲基甲酰胺溶液 | - Rinse inner surface of the Agitation tank with 100±5 kg DMF and drain to waste liquid tank through pump and processtransfer pipeline. /使用100±5 kg N, N-二甲基甲酰胺淋洗搅拌罐内表面，通过泵和工艺转移管路排至废液罐。 - Rinse the stirring tank with 105±5 kg of purified water./用105±5 kg纯化水冲洗搅拌罐。 - Disassemble the fittings until inseparable, wipe fittings with 22.5±2.5 kg purified water until there is no material residue./将配件拆卸至最小节，使用22.5±2.5 kg纯化水擦洗配件至无可视物料残留。 - Dry by compressed gas. /压缩气体吹干。 | Cleaning after production /项目结束后清洁 | Level I | Only for preparing Oxyma Pure /DMF solution, rinse thoroughly with DMF followed by water; the risk of residual carryover remains low, no need for level II test./仅涉及配置2-肟氰乙酸乙酯的N,N-二甲基甲酰胺溶液，先N,N-二甲基甲酰胺淋洗，再水冲洗，物料继续残留的风险很低，无需进行Level II 检测 | N/A |
| Pump for fresh Solvent/新鲜溶剂泵 | Add DMF/加N,N-二甲基甲酰 | - Flush with 80±10kg purified water. /使用80±10kg 纯化水冲洗。 - Dry by compressed gas. /压缩气体吹干。 | Cleaning after production /项目结束后清洁 | Level I | Only solvent are used, no need for level II test./仅涉及溶剂使用，无需进行Level II 检测 | N/A |
| 50L Glove box手套箱/ MOC/材质: Halar –lined or 316L /衬哈或316L不锈钢 | Dispensing /分装 | - Wipe with 10±5kg DMF./使用 10±5kg DMF擦洗。 - Wipe with 10±5kg purified water./使用 10±5kg 纯化水擦洗。 - Dry by compressed gas. /压缩气体吹干。 | Cleaning after production /项目结束后清洁 | Level I | Change gloves when switch project, no need level II test./切换项目时更换手套，无需进行level Ⅱ检测。 | N/A |
| Solid sampler/固体取样器 | Sampling for solid /固体取样 | - Wipe with 20±5kg DMF. /使用20±5kg DMF擦洗。 - Wipe with 20±5kg purified water. /使用20±5kg纯化水擦洗。 Dry by compressed gas. /压缩气体吹干。 | Cleaning after production /项目结束后清洁 | Level I | N/A | N/A |
| Diaphragm pump/隔膜泵 Equipment-#设备编号： TJ4S-1200-PDP039 | Transfer Oxyma/DMF solution/转移Oxyma/DMF溶液 | - Rinse inner surface with 80±20kg DMF. /使用 80±20kg DMF冲洗。 - Flush with 150±20kg purified water. /使用150±20kg纯化水冲洗。 - Disassemble the fittings until inseparable, wipe fittings with 50±10kg purified water until there is no material residue. /将配件拆卸至最小节，使用50±10kg纯化水擦洗配件至无可视物料残留。 - Dry by compressed gas. /压缩气体吹干。 | Cleaning after production /项目结束后清洁 | Level I | N/A | N/A |
| Diaphragm pump /隔膜泵 Equipment-#设备编号： TJ4S-1200-PDP040 | Transfer capping solution/转移封端溶液 | - Rinse inner surface with 80±20kg DMF. /使用 80±20kg DMF冲洗。 - Flush with 150±20kg purified water. /使用150±20kg纯化水冲洗。 - Disassemble the fittings until inseparable, wipe fittings with 50±10kg purified water until there is no material residue. /将配件拆卸至最小节，使用50±10kg纯化水擦洗配件至无可视物料残留。 - Dry by compressed gas. /压缩气体吹干。 | Cleaning after production /项目结束后清洁 | Level I | N/A | N/A |
| Process transfer pipeline/ 工艺转移管路 MOC/材质 Stainless steel/不锈钢 | Transfer mixture/ 转移体系 | - Flush pipeline with 30±5kg purified water. /使用 30±5kg纯化水冲洗管路。 - Disassemble the fittings until inseparable, wipe each fittings with 10±5kg purified water until there is no material residue. /将配件拆卸至最小节，每根使用 10±5kg纯化水擦洗配件。 - Dry by compressed gas. /压缩气体吹干。 | Cleaning after production /项目结束后清洁 | Level I | N/A | N/A |
| 3000L Agitation tank (include pump and transfer pipeline)/3000L搅拌罐(包含泵和其转移管路) Equipment # 设备编号: TJ4S-1213-SPS01-AT01&TJ4S-1213-SPS01-AT02 MOC/材质：Glass-lined/搪玻璃 | Preparing solution of piperidine/DMF/配哌啶的DMF溶液 | - Reference the SOP QM03148, the de-protection solution preparation system is dedicated to the preparation of piperidine solution with low risk. Therefore, storage tanks that remain unused for an extended period (more than 6 months) shall be drained and treated, followed by two rinses with 200 kg DMF prior to next use./ 索引SOP QM03148脱保护配液系统专用于哌啶溶液的配制，风险较低。故对于长时间不使用（大于6个月）的储罐应排空处理，并使用200kg DMF淋洗2次，待下次使用。 | Cleaning after production /项目结束后清洁 | Level I | N/A | N/A |

Instruction/说明：

- Define of different Cleaning verification class/不同清洁确认级别定义：

Level 0/B (Batch to batch flush): The equipment shall be simply flushed in-between batches prior to use. Process chemist or production manager/assistant engineer shall verify the equipment available after cleaning. / Level 0/B（批与批之间的冲洗）：设备在批次间连续使用前进行简单的冲洗。清洗后项目人或者生产经理/助理工程师应对设备状态进行确认。

Level I (in-situ minor cleaning): Visual check on the inner surface of the equipment body after cleaning to confirm it is free of any material residue under adequate light./Level I（原位小清洁）：设备清洗后，在足够的光源下对设备内表面的目视检查，确认无任何物料残留。

Level I (disassemble major cleaning): Visual check on the inner surface of the equipment body, components, pipelines as well as fittings after cleaning to confirm it is free of any material residue under adequate light./ Level I（拆卸大清洁）：设备清洗后，在足够的光源下对设备内表面，部件，管路和配件的目视检查，确认无任何物料残留。

- For batch to batch flush and the within campaign in-situ minor cleaning, the <Equipment Cleaning Method> should be numbered as “CMb-site code+product code+ Step No.+ version number (2 digits)”. e.g.: The first version of the equipment cleaning method of CPo1234-01 step2 is CMb-TJ211123402.01. /对于项目内批次间和步骤间的原位小清洗方案编号方式为：CMb-厂区代码+产品代码+步骤号+2位版本号，如CPo1234-01第2步第一个版本的设备清洗方法编号为CMb-TJ211123402.01；

For the within campaign disassemble major cleaning and the end of production cleaning, the <Equipment Cleaning Method> should be numbered as “CMa-site code+ product code + Step No.+ version number (2 digits)”. e.g.: The first version of the equipment cleaning method of CPo1234-01 step2 is CMa-TJ211123402.01./ 对于项目内的拆卸大清洁和生产结束后的清洗方案编号方式为：CMa-厂区代码+产品代码+步骤号+2位版本号，如CPo1234-01第2步第一个版本的设备清洗方法编号为CMa-TJ211123402.01；

For equipment qualification, the <Equipment Cleaning Method> should be numbered as “CMq-validation protocol / test run protocol number +version number”. e.g.: The first version of the equipment cleaning method of SVP-TJ1-101.01-001 is CMq- SVP-TJ1-101.01-010 v.1 /对于设备验证清洗方案编号方式为：CMq-验证文件编号+版本号，如验证文件SVP-TJ1-101.01-010使用设备清洗方法编号为CMq-SVP-TJ1-101.01-010 v.1。

For others, the <Equipment Cleaning Method> could be numbered as “CMa-usage-site+Sequence No.+version number”. E.g.: At TJ1, the first version of the equipment cleaning method of waste disposal is CMa-waste disposal-TJ101v.1 /对于其他的清洗方法，可以按照如下方式编号：CMa-用途-厂区+序号+版本号。如CMa-waste disposal-TJ101v.1代表天津1厂区2第一份用于三废处理的清洗方法。

- The selection of cleaning solvents/清洗溶剂的选择：

| 3.1 | Whether react with the equipment material/是否与设备材质发生反应 | ☐ Yes/是 ☒ No/否 |
| --- | --- | --- |
| 3.2 | Whether react with target material/是否与目标物发生反应 | ☐ Yes/是 ☒ No/否 |
| 3.3 | Whether target material dissolved well in the solvent/是否对目标物有较好的溶解性 | ☒ Yes/是 ☐ No/否 |
|  | The cleaning solvent does not affect the test method./清洗溶剂不影响测试方法的使用 | ☒ Yes/是 ☐ No/否 |
|  | Reference solubility data/溶解度参考数据 Target/ 目标物; Cleaning solvent/ 清洁溶剂; Solubility/溶解性 (1g target can soluble in XX ml XX solvent at XX oC / 1g 目标物在什么温度可溶于多少ml什么溶剂中). ASYM-140020; DMF/N, N-二甲基甲酰胺; 1g target can soluble in 5 ml DMF solvent at 23~27oC / 1g 目标物在23~27℃可溶于5 ml N, N-二甲基甲酰胺溶剂中. ASYM-124852/YL005318; DMF/N, N-二甲基甲酰胺; 1g target can soluble in 8 ml DMF solvent at 23~27oC / 1g 目标物在23~27℃可溶于8ml N, N-二甲基甲酰胺溶剂中. For equipment of complex structure, for example agitating dryer, the target should exhibit good solubility in the selected cleaning solvent (>1g/10ml) 对于结构复杂的设备，比如搅拌干燥器，目标物在选定的清洁溶剂中应该有较好的溶解性(>1g/10ml) There is an reference substance for the target清洁检测目标物是否有对照品 ☒ Y/是 reference substance lot number对照品批号ASYM -140020： CPo140020-03-2472-024-01-RS____ ASYM-124852/ YL005318:TJ4-124852-251224-01-RS ☐ Currently no, please indicate when available/暂时没有，请提供对照品可以获得的日期：___N/A_________ If reflux/rinse cleaning method is used, is the above solubility covered in the worst case of solubility in the recovery rate study? 如果使用了回流/淋洗的清洁方式，上面的溶解度数据是否包含在了最差溶解度情况下的回收率研究中？ ☒ Yes, no additional reflux recovery study is needed / 是，不需要额外的回流回收率研究； ☐ No, not covered, extend study of the worst case of solubility in the recovery rate should be done, responsibly person and target finish date / 不，没有包含，需要扩展最差溶解度情况下的回收率研究，负责人和目标完成时间：_________N/A______________ Confirmed by Project analysis manager & date / 项目分析经理确认/日期：________________________________________________________ |

- Assessment of the detergent 清洁剂评估： No detergent will be used during the equipment cleaning. /设备清洗过程中将不会使用清洁剂。

- The reflux temperature range is for reference only, the actual reflux state is the control target. /此清洁方法中，回流温度范围仅供参考，具体以达到实际回流状态为准。

Project Leader & Date / 项目主管签字及日期：

Production Manager & Date / 生产经理签字及日期：

QA Project Manager & Date / QA项目经理签字及日期：